3GPP TSG RAN WG1 #116	                                                                                                 R1-2401433

Athens, Greece, Feb 26th – Mar 1st, 2024

Agenda item:		9.1.3.1

Source: 		Qualcomm Incorporated

Title: 			Additional study on CSI prediction

Document for:		Discussion and Decision

# Introduction

In this contribution, we discuss aspects related to AI/ML for CSI prediction using a single-sided model on the UE-side, including the potential system-level performance gain from CSI prediction, considerations related to the non-AI/ML benchmark schemes, and evaluation results of AI/ML-based prediction.

Study objectives with corresponding checkpoints in RAN#105 (Sept ’24): - CSI feedback enhancement [RAN1]: - For CSI compression (two-sided model), further study ways to: - Improve trade-off between performance and complexity/overhead - e.g., considering extending the spatial/frequency compression to spatial/temporal/frequency compression, cell/site specific models, CSI compression plus prediction (compared to Rel-18 non-AI/ML based approach), etc. - Alleviate/resolve issues related to inter-vendor training collaboration. while addressing other aspects requiring further study/conclusion as captured in the conclusions section of the TR 38.843. - For CSI prediction (one-sided model), further study performance gain over Rel-18 non-AI/ML based approach and associated complexity, while addressing other aspects requiring further study/conclusion as captured in the conclusions section of the TR 38.843 (e.g., cell/site specific model could be considered to improve performance gain). - Necessity and details of model Identification concept and procedure in the context of LCM [RAN2/RAN1] - CN/OAM/OTT collection of UE-sided model training data [RAN2/RAN1]: - For the FS_NR_AIML_Air study use cases, identify the corresponding contents of UE data collection - Analyse the UE data collection mechanisms identified during the FS_NR_AIML_Air (TR 38.843 section 7.2.1.3.2) study along with the implications and limitations of each of the methods - Model transfer/delivery [RAN2/RAN1]: - Determine whether there is a need to consider standardised solutions for transferring/delivering AI/ML model(s) considering at least the solutions identified during the FS_NR_AIML_Air study - Testability and interoperability [RAN4]: - Finalize the testing framework and procedure for one-sided models and further analyse the various testing options for two-sided models, in collaboration with RAN1, and including at least: - Relation to legacy requirements - Performance monitoring and LCM aspects considering use-case specifics - Generalization aspects - Static/non-static scenarios/conditions and propagation conditions for testing (e.g., CDL, field data, etc.) - UE processing capability and limitations - Post-deployment validation due to model change/drift - RAN5 aspects related to testability and interoperability to be addressed on a request basis NOTE: offline training is assumed for the purpose of this project. NOTE: the outcome of the study objectives should be captured in TR 38.843 for future reference. NOTE: Coordination with SA/SA WGs of the ongoing study/work as it may relate to their required work.

# Potential system-level performance gain from CSI prediction

In this section, we show the potential gain in system-level performance from on-time CSI availability. Consider the case where we have a 10 slot CSI-RS periodicity. In addition to this, assume we have a CSI delay from processing at UE and/or gNB side. The cumulative delay by which the stale CSI is utilized by the gNB for downlink scheduling will cause a system performance loss. By implementing a CSI prediction algorithm, at least some of this loss can be reclaimed. As the CSI delay increases, the greater the loss from stale CSI, and the scope of gain possible through the usage of a CSI prediction algorithm.

Figure 2-1 below show the loss in system performance for various UE speeds for the urban macro scenario, in terms of mean, edge and cell-center performance. The loss shown for each scheme is the percentage reduction in the user perceived throughput due to the delayed CSI under that scheme when compared to a benchmark scheme that assumes CSI feedback with no delay. We assumed a low offered load (< 20% resource utilization) with bursty traffic and Release 16 eType2 with Parameter Combination of 4. Note that the same compression (eType2 PC4) is applied to the benchmark scheme also to convey the CSI feedback.

Figure 2-1: Impact of Stale CSI on User Experience of Mean, Edge and Cell-center UEs (3–120 kmph, 11-444 Hz Doppler)

At high CSI-delays and high Doppler, potential performance gain from a good CSI-prediction algorithm can be over 50% at cell edge. Even at low Doppler, gains of 20-30% are possible around 20 slot delays.

At high CSI-delays and high Doppler, potential performance gain from a good CSI-prediction algorithm can be more than 50% at cell edge. Even at low Doppler, gains of 20-30% are possible around 20 slot delays.

# Non-AI/ML benchmark

There are mainly two kinds of non-AI/ML benchmarks used in Rel-18 discussions. Nearest historical CSI and non-AI/ML based CSI prediction. It is concluded in TR 38.843 [1] that the majority of sources observe performance gain using AI/ML based CSI prediction over nearest historical CSI. However, for AI/ML based CSI prediction over non-AI/ML based CSI prediction, the results are quite diverse. It may be due to the different settings used for non-AI/ML based benchmark.

AI/ML based CSI prediction outperforms the benchmark of the nearest historical CSI in general, where the majority of sources observe up to 10.6% gain in terms of mean UPT. for AI/ML based CSI prediction over non-AI/ML based CSI prediction, 3 sources observe 0.7%~7% gain while 2 sources observe performance loss of -0.1%~-17% in terms of mean UPT.

## Trade-off of non-AI/ML based CSI prediction

One of the typical non-AI/ML based CSI prediction benchmarks is autoregression. In this section, we discuss the trade-off of autoregression (AR) algorithm.

Figure 3-1:Autoregression prediction

AR needs to first calculate the autocorrelation matrix. After obtaining the correlation matrices, AR uses p historical observations to predict future channels. The key to the algorithm is to obtain accurate correlation matrices. There are at least two factors impacting the accuracy of the correlation matrices.

### The dimension of autocorrelation matrix

Assuming the prediction target is the channel matrix, there may be different choices for the autocorrelation computation. For example, the granularity at which the autocorrelation matrix is computed and maintained, and its size may be different in different implementations – e.g., whether it is specific to each frequency resource or port or a common one. Depending on such choices, the performance of the non-AI/ML benchmark may differ a lot.

### “Learning” window size for autocorrelation matrix calculation

Like the AI/ML cases, the autocorrelation matrix needs to be “learnt” using old historical observations. Unlike the AI/ML cases, this autocorrelation matrix “learning” can be online instead offline, a sliding window may be used to learn the autocorrelation matrix and keep updating it. Usually larger window size, more samples used, more accurate the autocorrelation matrices can be obtained, especially for autocorrelation matrices with large size. For the case of synthetic channels generated by channel models, it may be easier to have a large window and to get accurate autocorrelation matrices. However, considering more practical cases, a large window may not be feasible since the channel profiles may not be fixed for a long time. Results of smaller window size may also need to be considered.

Figure 3-2 shows the trade-off performance of autoregression prediction with UE speed 30 km/h and p = 4. The x axis is the learning window size, i.e., how many historical samples are used for autocorrelation matrix calculation. The y axis is the SGCS of the predicted eigenvectors of the first two layers. As shown in the figure, a large diverse performance can be observed. With a small learning window, the SCGS can be as high as -1.5 dB and with a larger learning window, the SGCS can go lower to about -10 dB.

Figure 3-2: Trade-off performance of autoregression prediction, UE speed 30 km/h, p = 4

The performance of non-AI/ML based CSI prediction benchmark can be diverse due to different trade-off choice in the algorithm parameters.

Companies to report the non-AI/ML CSI prediction performance together with the algorithm parameters (e.g., size of autocorrelation matrix, learning window size).

## Online learning of non-AI/ML based CSI prediction

As discussed in the previous section, the non-AI/ML based CSI prediction can use a sliding window to keep updating the intermediate parameters. In this case, when predicting the future channel, not only the last “inference” part, but also the “learning” part needs to be performed. When calculating the complexity of non-AI/ML based CSI prediction, both “inference” and “learning” part need to be taken into account.

Table 1: Calculations needed for non-AI/ML and AI/ML CSI prediction

|  | Updating the parameters | Using historical inputs samples to predict future sample (Inference) |
| --- | --- | --- |
| Non-AI/ML based CSI prediction | Needed | Needed |
| AI/ML CSI prediction | No need | Needed |

The complexity of online updating calculation needs to be taken into account for non-AI/ML based CSI prediction.

# AI/ML CSI prediction

## Model input/output

It was concluded in Rel-18 that for CSI prediction, raw channel and eigenvector inputs are considered for evaluations. In this section, we discuss the model input/output for AI/ML based CSI prediction.

Conclusion If the AI/ML based CSI prediction sub use case is to be selected as a sub use case, both of the following types of AI/ML model input are considered for evaluations: - Raw channel matrixes - Eigenvector(s) Agreement Capture the following high level observations for CSI prediction to section 6.2.2.8 of TR 38.843: - From the perspective of model input/output type, it is more beneficial in performance by considering raw channel matrix as the model input than precoding matrix

Although raw channel inputs/outputs may have a better performance compared to eigenvectors, there are several issues for raw channel inputs/outputs.

### Phase coherence

In order to train the AI/ML models for CSI prediction, a single shot input is not enough, and a time domain sequence of multiple slots is needed. To keep the time domain correlation within the time domain sequence, phase coherence is preferred. However, phase coherence across slots may not be maintained due to e.g., UL/DL switching. This poses implementation challenges as UE may have to guarantee phase coherence either via complicated hardware designs or via an algorithmic approach, which will increase UE implementation complexity. Any uncorrected phase may likely degrade prediction performance.

### Output size

If the full channel needs to be predicted, all the entries in the channel matrix need to be predicted. Even if low rank scenarios can be expected, the size cannot be reduced, which is not efficient.

If eigenvectors need to be predicted, during the low rank scenarios, only some eigenvectors need to be predicted instead of the whole set of eigenvectors. In this case, compared to the raw channel output case, the output size will be much smaller.

Raw channel inputs/outputs can bring larger implementation challenges than eigenvector inputs/outputs.

Although raw channel inputs/outputs may potentially have better prediction performance (related to AI/ML designs), it may bring larger implementation challenges. Based on these observations, we have the following proposal.

Whether model input/output uses raw channel or eigenvectors is up to UE implementation choice and should not impact the related signalling designs.

## Evaluation results

In this section, we show the results of AI/ML CSI prediction. Rel-18 evaluation assumption was used. The CSI-RS periodicity was set to 5 ms. For both non-AI/ML and AI/ML, 4 historical samples were used as input and output 1 predicted result. For each historical sample in the tracks, 4 Rx, 32 port, 50 RBs were assumed. Non-AI/ML used learning windows to keep updating the intermediate parameters with two learning window sizes. The size of small learning window is 25 historical samples. And the size of large learning window is 50 historical samples. Offline training was used for AI/ML CSI prediction. Eigenvectors were used as model input/output. Totally about 16k tracks were used for training.

Table 2: SGCS of non-AI/ML and AI/ML prediction results

| UE Speed | Nearest historical CSI | Non-AI/ML (small learning window) | Non-AI/ML (large learning window) | AI/ML prediction |
| --- | --- | --- | --- | --- |
| 30 km/h | 0.73 | 0.67 | 0.82 | 0.82 |
| 60 km/h | 0.63 | 0.61 | 0.82 | 0.68 |

# Conclusions

In this document, we have discussed aspects related to potential system-level performance gain, non-AI/ML benchmark, AI/ML CSI prediction. We have made the following observations and proposals:

- At high CSI-delays and high Doppler, potential performance gain from a good CSI-prediction algorithm can be more than 50% at cell edge. Even at low Doppler, gains of 20-30% are possible around 20 slot delays.

- The performance of non-AI/ML based CSI prediction benchmark can be diverse due to different trade-off choice.

- Raw channel inputs/outputs can bring larger implementation challenges than eigenvector inputs/outputs.

- Companies to report the non-AI/ML CSI prediction performance together with the algorithm parameters (e.g., size of autocorrelation matrix, learning window size).

The complexity of online updating calculation needs to be taken into account for non-AI/ML based CSI prediction.

Whether model input/output uses raw channel or eigenvectors is up to UE implementation choice and should not impact the related signalling designs.

# References

- TR 38.843 v18.0.0, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface” (Release 18), December 2023.

- RP-234039, “New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, Qualcomm (Moderator), 3GPP TSG RAN #102, December 2023.