3GPP TSG RAN WG1 #116                                                                                                       R1-2401434

Athens, Greece, Feb 26th – Mar 1st, 2024

Agenda item:		9.1.3.2

Source: 		Qualcomm Incorporated

Title: 			Additional study on CSI compression

Document for:		Discussion and Decision

# Introduction

The following study objectives related to CSI feedback enhancement were identified in the Release 19 work item description [2] :

Study objectives with corresponding checkpoints in RAN#105 (Sept ’24): - CSI feedback enhancement [RAN1]: - For CSI compression (two-sided model), further study ways to: - Improve trade-off between performance and complexity/overhead - e.g., considering extending the spatial/frequency compression to spatial/temporal/frequency compression, cell/site specific models, CSI compression plus prediction (compared to Rel-18 non-AI/ML based approach), etc. - Alleviate/resolve issues related to inter-vendor training collaboration. while addressing other aspects requiring further study/conclusion as captured in the conclusions section of the TR 38.843. - For CSI prediction (one-sided model), further study performance gain over Rel-18 non-AI/ML based approach and associated complexity, while addressing other aspects requiring further study/conclusion as captured in the conclusions section of the TR 38.843 (e.g., cell/site specific model could be considered to improve performance gain).

In this contribution, we discuss aspects related to the use case of CSI compression using two-sided model. Specifically, we discuss potential approaches to addressing the inter-vendor collaboration complexity issue, as well as approaches to improve the performance of CSI compression using localized models and temporal compression techniques including joint compression and prediction.

# Inter-vendor collaboration aspects

## Data-driven PMI mapping

The following discussion is in the context of UE feeding back CSI in the form of a preferred precoding matrix to a gNB. We use the term PMI mapping to denote the mapping between the precoding matrix indicator (PMI) in the CSI feedback payload (i.e., the output of the CSI generation part) and the precoding matrix it represents (i.e., the target CSI or input-CSI-NW / output-CSI-UE).

A CSI feedback scheme based on AI/ML is expected to provide a performance benefit primarily because the AI/ML scheme can be optimized to specific scenarios in a data-driven manner. In legacy CSI schemes such as Type I, Type II, or eType II, the PMI mapping is fully specified in the standard. In contrast, AI/ML schemes may optimize the PMI mapping based on the data distribution of precoding matrix instances in a specific scenario. Therefore, a data-driven PMI mapping cannot be pre-determined and standardized.

To achieve the benefit of data-driven optimization of the CSI feedback scheme, the PMI mapping (i.e., the mapping between the precoding matrix indicator (PMI) in the CSI feedback payload and the precoding matrix it represents) cannot be pre-determined and standardized.

Instead, the mapping has to be determined outside the standard, specific to each scenario. This leads to several related questions:

- Training: How can UE-side and NW-side achieve compatibility between the encoder and decoder models during the training stage in terms of the PMI mapping that they implement?

- Inference: How can UE-side and NW-side align the PMI mapping during CSI feedback?

- Testing: What should the scope of RAN4 performance testing be if the PMI mapping is not specified in the standard?

Requiring every pair of UE and NW vendors to collaborate offline to align the PMI mapping for each scenario may not be scalable. Such issues have been discussed during the Release 18 study and this was one of the identified reasons for the lack of RAN1 consensus on the recommendation of CSI compression for normative work in the Release 18 technical report TR 38.843 [1].

Even if the PMI mapping itself is not standardized, standards support is needed to enable vendors to achieve a common understanding of the mapping, both for training and inference.

Study potential specification support to enable vendors to achieve a common understanding of the PMI mapping (i.e., the mapping between the precoding matrix indicator (PMI) in the CSI feedback payload and the precoding matrix it represents), both for training and inference.

## Sequential training based on sharing PMI mapping information

The Release 18 study analysed different types of training collaboration. Among these, the Type 2 sequential and Type 3 separate training flavours allow one of the two sides to define the PMI mapping as part of an initial training step. Subsequently, the mapping is conveyed to the training entity of the other side to develop their model in a compatible manner sequentially. The mapping may be conveyed in different ways as discussed in the next section – for example, through a dataset exchange or gradient exchange mechanism. Type 1 training can also be interpreted as being within this framework in the sense that there is an initial training phase on one side and the PMI mapping is conveyed to the other side in the form of a reference AI/ML model. The model must be optimized, compiled and tested by the training entity of the other side before deployment on devices.

### Different approaches to sharing the PMI mapping

Once the PMI to precoding matrix mapping has been determined, there may be different ways to share it with another vendor for the purpose of enabling that vendor to develop a model compatible with the mapping. The following approaches can be used:

- Dataset sharing: In this approach, the mapping is represented in the form of a dataset comprising example pairs of CSI feedback payload (PMI) and the corresponding precoder. This corresponds to Type 3 training that was discussed in Release 18.

- Reference model sharing: In this case, the mapping is represented implicitly in the form of a reference AI/ML model that implements the mapping in an approximate manner. This corresponds to Type 1 training that was discussed in Release 18. The reference model may be an encoder or a decoder.

  - Reference encoder: If it is an encoder, then the model can be used to derive the CSI payload that would convey a given precoding matrix in a manner compatible with the mapping.

  - Reference decoder: If it is a decoder, then the model can be used to infer how a CSI payload will be interpreted by the network according to the PMI mapping.

- Application programming interface (API): This is another approach where the mapping is registered implicitly in the form of an API. An entity that needs to develop a model based on the PMI mapping can use the API to send an input precoder sample and retrieve the mapped payload output or vice versa. Another possibility is to use the API to send an activation and ground truth and obtain a gradient to perform gradient descent. This corresponds to Type 2 sequential training that was discussed in Release 18.

However, in all these cases, UE and NW vendors are required to coordinate the exchange of the dataset, gradient, or model in an offline training collaboration procedure.

Assuming NW-first training for the discussion, since each NW-side vendor may develop a different PMI mapping independently, each UE-side vendor would need to collaborate with each NW-side vendor to obtain the PMI mapping. Similarly, any NW-side vendor would need to collaborate with each UE-side vendor to provide the PMI mapping information. Without standards support, such pairwise collaboration may not be scalable across vendors.

A standardized solution to enable the sharing of the PMI mapping from the vendor that trains first to the vendor that trains second would be very useful. Since the PMI mapping defined by one NW-vendor is expected to be common for all UE-side vendors, one solution would be to register and store the PMI mapping in a central registry.

## A central registry for PMI mapping

In this section, we discuss the approach of two-sided model management based on a central registry to register the PMI mapping.

The PMI mapping defined by the NW-vendor during initial training needs to be provided to the UE-side training entity for the purpose of developing a compatible UE-side model. As shown in Figure 2-1, the NW-side entity that performs the initial training can define and register the PMI mapping at the registry. Subsequently, any UE-side vendor’s training entity can retrieve the mapping information from such a registry and train a compatible UE-side model based on this information. The NW-side vendor may optionally develop a different NW-side decoder model with further optimization before deploying on a gNB, while ensuring that it is still compatible with the registered PMI mapping.

Figure 2-1: Two-sided model operation based on central registry for PMI-precoder mapping

Such a discussion also applies to UE-first training. In this case, the UE-side vendor that performs the first training step can define and register the PMI mapping at the registry. Subsequently, any vendor can retrieve the mapping information from such a registry and train a compatible NW-side model based on this information.

The procedure to register and retrieve the mapping information can be standardized. It should be noted that the requirement is to exchange the mapping information between the central registry and a training entity, hence air-interface signalling specification is not expected for the transfer mechanism of the mapping information.

To summarize, on one extreme, the PMI-precoder mapping may be fully standardized (as in legacy CSI schemes), but this does not allow data-driven optimization using AI/ML. On the other extreme, if there is no standards support, then pairwise inter-vendor collaboration is needed for alignment. The middle ground could be to define standardized procedures to register/retrieve the PMI-precoder mapping using a central registry to facilitate alignment among vendors while still allowing data-driven optimization using AI/ML.

Results from the Release 18 AI/ML study on different types of sequential training showed the feasibility of the above approach. Specifically, a model trained in the second stage of sequential training based on the model trained in the first stage was shown to work in a compatible manner with the first-trained model. This observation holds for both the dataset exchange approach and the approach of using a reference model of the other side together with a freeze-and-train training technique using gradient exchange.

In practice, the performance of the dataset sharing approach may depend on the size of the dataset shared. An inadequate amount of data may result in poor performance. In contrast, if the reference decoder is shared or if an API is made available for gradient exchange, the UE-side training entity can use it to train the encoder model without being limited by the dataset size.

To address concerns related to inter-vendor training collaboration complexity, study the approach of 2-sided model training based on registering the mapping between the PMI payload and the preferred precoding matrix that it represents.

The procedure to enable a training entity to register and retrieve the PMI-precoder mapping information from the central registry can be standardized.

## Relation to RAN4 aspects

RAN4 has discussed different approaches to the testing procedure for CSI compression using two-sided models. The approaches of fully specified and partially specified decoder that were discussed in RAN4 resemble the approaches discussed above.

Figure 2-2: RAN4 testing framework for two-sided models

Specifically, the procedure discussed in RAN4 can be interpreted as a way to specify the PMI mapping in the RAN4 standards. Such specification may be in the form of a dataset, a reference encoder, or a reference decoder model. This is similar to the approaches discussed in Section 2.2.1 for registering the PMI mapping in a central registry.

Subsequently, in the RAN4 context, any UE and test equipment (TE) vendors can independently develop encoder and decoder models based on the standardized PMI mapping. Since the models are developed based on the same mapping, they are expected to be compatible with each other.

# Localized models

In this section, we discuss the approach of developing localized models, i.e., models developed specifically for use within a local region.

The work item description [2] has identified a study objective associated with performance improvement:

“For CSI compression (two-sided model), further study ways to:

- Improve trade-off between performance and complexity/overhead

  - e.g., considering extending the spatial/frequency compression to spatial/temporal/frequency compression, cell/site specific models, CSI compression plus prediction (compared to Rel-18 non-AI/ML based approach), etc.”

One of the aspects in the list of examples above is “cell/site specific models”. Due to spatial consistency, the channel samples collected from a local region or site are expected to be more correlated and hence more compressible as compared to the overall data from a much larger layout. Specifically, the distribution of precoders is expected to be sparser, and AI/ML techniques can exploit this to derive a more optimized PMI mapping to feedback the precoder information. Therefore, a model trained on the local training dataset and used for inference on a test dataset from the same local scenario is expected to perform well compared to a model that is trained on the global dataset.

Regarding the evaluation methodology for this aspect, we need to consider that the 3GPP spatial channel model uses a statistical approach to model the variability of channel characteristics across cells and locations. For example, the probability distribution used to derive the large and small scale parameters is homogeneous, i.e., it is typically not different between UEs in one cell and UEs in another cell. In contrast, in a real deployment, the channel characteristics may have even larger location-dependent variability due to other differences in the environment such as different terrain, blockers, etc. that are not fully captured by the channel model.

Channel characteristics in a real deployment are likely to have larger location-dependent variability as compared to the variability observed in a synthetic dataset derived using the spatial channel model.

We next present our evaluation results to quantify the gain from localized models. For this evaluation, we consider three cases for the training dataset –

- Case 1 (baseline - global dataset):

  - UE-side encoder and NW-side decoder are trained using a dataset from the full dense urban layout.

- Case 2 (local indoor dataset):

  - UE-side encoder and NW-side decoder are trained using data from UEs that are local to a small region within the layout: Indoor UEs at ground level dropped within a 25 m. by 25 m. region and associated to the same cell.

- Case 3 (local outdoor dataset):

  - UE-side encoder and NW-side decoder are trained using data from UEs that are local to a small region within the layout: Outdoor UEs dropped within a 25 m. by 25 m. region associated to the same cell.

Figure 3-1 shows the SGCS for layer 1 for the cases described above, when those models are used for inference on the local dataset. The SGCS is expressed as a loss metric in dB scale, i.e.,

SGCS (dB) = 10log 10(1-SGCS)

A lower value of this metric corresponds to better accuracy.

Figure 3-1: SGCS for inference on local scenario using models trained on global vs. local scenario

The result is shown for the case of a simple AI/ML model and a model with a complex structure. We make the following observations:

- The plots show that the models trained on local data outperform the models trained on global data when other aspects are the same.

- The plots also show that the simple model trained on local data can achieve performance that is very close to the complex model trained on the global data.

This points to two benefits of training on local data – the SGCS can improve for the same model complexity. Alternately, the same SGCS performance can be achieved using a much simpler AI/ML model.

Localized models, which are developed specifically for use within a local region, provide better performance compared to global models of comparable size trained using data from the entire layout.

Localized models provide similar performance with a smaller model complexity compared to global models.

Based on this discussion, we have the following proposals:

Study the performance improvement associated with localized models (models developed specifically for use within a local region) compared to global models.

For the evaluation of localized models, the local region could be defined as UEs dropped within a specific boundary (e.g., 25 m. by 25 m. square), with the same serving cell, and with the same indoor/outdoor state.

Study techniques and potential specification impact to enable the use of localized models to achieve the associated improvement in the performance-complexity tradeoff.

# Temporal domain compression

## Temporal compression sub-use cases

Release 18 study item studied the sub-use case of spatial-frequency domain CSI compression using two-sided model. The work item description [2] has identified a study objective associated with performance improvement:

“For CSI compression (two-sided model), further study ways to:

- Improve trade-off between performance and complexity/overhead

  - e.g., considering extending the spatial/frequency compression to spatial/temporal/frequency compression, cell/site specific models, CSI compression plus prediction (compared to Rel-18 non-AI/ML based approach), etc.”

Incorporating temporal domain into the problem is expected to open new avenues for performance gains from AI/ML. There may be different sub-use cases related to the temporal domain. They can be classified based on the following aspects:

- History at encoder: This aspect is about whether the UE makes use of its own past observations of the channel over an observation window to derive and compress the CSI feedback.

- History at decoder: This aspect is about whether the gNB makes use of prior CSI feedback reports. If yes, then this allows a UE to make the CSI report depend on a previous report, e.g., a differential report.

- Reporting CSI window: This aspect is about whether the report corresponds to one slot or to a CSI window with multiple slots, and whether any prediction is involved or not.

Based on these aspects, we identify the following flavours of the problem:

- Case 1: Compress present CSI and report

  - Case 1A: Report is a standalone report

  - Case 1B: Report is a differential report that depends on a prior report

- Case 2: Predict one future instance or multiple present/future instances of CSI, compress and report

  - Case 2A: Report is a standalone report

  - Case 2B: Report is a differential report that depends on a prior report

Among these, Case 1A has already been studied in Release 18. For Case 2, the prediction and compression may happen as separate operations or as a joint prediction and compression step. This is up to implementation.

Consider the following sub-use cases related to temporal domain CSI compression:

- Case 1: Compress present CSI and report.

  - Case 1A: Report is a standalone report.

  - Case 1B: Report is a differential report that depends on a prior report.

- Case 2: Predict one future instance or multiple present/future instances of CSI, compress them and report.

  - Case 2A: Report is a standalone report.

  - Case 2B: Report is a differential report that depends on a prior report.

## Evaluation methodology aspects

In this section, we discuss which scenarios are most suitable for studying temporal domain compression. Also, we propose an update to the format of the results table for capturing the results.

### Evaluation scenarios

The gain from jointly compressing and reporting multiple CSI instances depends on the amount of correlation in time-domain between the CSI samples. If there is medium/high correlation in time-domain (TD), we expect to achieve gain from jointly compressing multiple CSI instances in one or more reports compared to separately compressing and reporting each CSI sample. In turn, the performance in scenarios with low TD correlation is mainly limited by the accuracy of the prediction algorithm. The CSI TD correlation is inversely proportional to the UE speed and the CSI-RS periodicity.

Rel-18 evaluation methodology for the study of CSI prediction using ML [1] was focused on outdoor scenarios with medium/high Doppler (UE speeds 10 to 120 km/h) and 5ms CSI feedback periodicity. In particular, for outdoor Dense Urban scenarios, most of the channel realizations are LOS and the precoder can be easily compressed. In section 4.3.3.1, we present results showing that in scenarios with UE speed of 30 km/h or higher, the performance is limited by the prediction accuracy and CSI compression is not the bottleneck.

Evaluation scenarios considered in Rel-18 study on CSI prediction assume 100% outdoor UEs. For Dense Urban scenarios, outdoor UEs have mostly line-of-sight channels and CSI compression is not the performance bottleneck in the system.

For a sub-use case involving compression and prediction, in general the end-to-end performance is determined by both the compression and prediction accuracy. To quantify the difficulty of prediction, we study the impact of delayed CSI on SGCS between the current precoder sample Vt at time t and the nearest historical precoder Vt-1 at time (t-1). For compression, we compute the SGCS between precoder Vt and the reconstructed Vt assuming ML/legacy compression algorithm for medium sized CSI payload, e.g., ML compression assuming 128 bits per layer. By comparing these two SGCS values, one can classify the different scenarios to one of the following categories:

- Prediction-limited: The end-to-end performance is limited by the prediction accuracy and increasing the CSI payload size beyond a threshold and/or changing the compression algorithm have little or no impact on the performance.

  - These scenarios can be used for comparing different prediction algorithms (ML vs non-ML).

  - E.g., Outdoor scenarios with medium/high UE speeds (> 30 km/h) and typical CSI-RS periodicity (5ms).

- Compression-limited: The end-to-end performance is limited by the compression accuracy. That is, the compression algorithm (ML vs legacy) and CSI payload size have significant impact on the end-to-end performance.

  - These scenarios are best suited for showing how much of the TD correlation can be exploited by the compression algorithm in reducing reporting overhead or improving the performance for the same payload size.

  - E.g., Scenarios with mostly indoor UEs and very low UE speeds (3 km/h).

- Prediction-compression-limited: The end-to-end performance is limited by both the compression and prediction accuracy.

  - These scenarios can be used to demonstrate how much of the gain from ML compression of multiple TD samples translates to end-to-end performance gain after accounting for the prediction error.

  - E.g., Outdoor scenarios with low UE speeds (10 km/h) or scenarios with mostly indoor UEs and long CSI-RS periodicity.

For characterizing the benefits of ML CSF in compression across spatial/temporal/frequency domains, we need to consider scenarios which are either compression-limited or prediction-compression-limited. Prediction-limited scenarios have little utility in comparing different compression schemes and more suitable to be studied under the CSI prediction sub-agenda.

Scenarios where the end-to-end performance is either compression-limited or prediction-compression-limited are best suited for studying different compression algorithms. In these scenarios changing the compression algorithm and/or the CSI payload size translate to noticeable change in end-to-end SGCS.

For studying different compression algorithms, the evaluation scenarios should at least include cases where the end-to-end performance is either compression-limited or prediction-compression-limited.

Figure 4-1 Configuration of periodic and aperiodic burst of CSI-RS resources for Rel-18 etype-II codebook.

Figure 4-1 shows the setup for Rel-18 etypeII codebook, where the UE reports compressed representation of N4∈{1,2,4,8} precoder samples assuming one of the following CSI-RS configurations.

- Periodic (P) or semi-persistent (SP) CSI-RS resource with periodicity ‘d’, and CSI prediction window size is equal to d.N4 slots.

  - For CSI window with N4 >1, the reporting periodicity can be reduced relative to CSI-RS periodicity while minimizing the impact of delayed CSI on performance.

- Aperiodic (AP) CSI-RS burst consisting of K∈{4,8,12} resources with m∈{1,2} slot separation.

  - Dense CSI-RS resources in time-domain is configured for improving the CSI prediction and compression accuracy, especially for scenarios with high/medium Doppler.

Rel-18 etype-II codebook for medium/high Doppler allows configuring an aperiodic CSI-RS burst with resources every one or two slots, which helps in improving CSI prediction/compression accuracy especially for high UE speeds.

Like Rel-18 etype-II, when paired with a prediction algorithm, an ML algorithm for joint spatial/temporal/frequency compression can provide one of the following benefits.

- Reduce the performance degradation caused by delayed CSI.

- Reduce the CSI reporting overhead in terms of payload and/or reporting periodicity.

To evaluate the benefits of ML compression algorithms, we propose two scenario types:

- Indoor scenarios (mostly NLOS, low-speed UEs): For evaluating benefits of ML compression in reducing CSI reporting overhead.

  - E.g., Dense Urban with 80% indoor UEs, UE speed of 3 km/h, and periodic CSI-RS with 5ms periodicity.

- Outdoor scenarios (mostly LOS, medium/high-speed UEs): For evaluating benefits of ML in providing up-to-date CSI in case of medium/high Doppler

  - E.g., Dense Urban with 100% outdoor UEs, UE speeds of 10 km/h, and CSI-RS with 5ms periodicity

  - E.g., Dense Urban with 100% outdoor UEs, UE speeds > 30 km/h, and AP CSI-RS burst with m=1 or 2.

Study the benefits of ML compression over time/frequency/spatial domains in

  - Indoor scenarios having mostly NLOS and low-speed UEs (e.g., Dense Urban scenario with 80% indoor UEs)

  - Outdoor scenarios having mostly LOS UEs (e.g., Dense Urban with 100% outdoor UEs) and different UE speeds.

For outdoor scenarios with UE speeds of 30 km/h or higher, study the performance assuming an aperiodic CSI-RS burst with K resources which is scheduled periodically.

### Performance metric

In addition to the performance metrics considered in Rel-18 [1], we also need to account for the case where the baseline and the ML schemes have different reporting periodicities and/or report size, see Figure 4-2. For example, consider scheme #1 with CSI reporting periodicity of 2.d and CSI report size of X bits. While, in scheme #2 we have CSI reporting periodicity of 4.d and report size of Y bits. To have a fair comparison between the two schemes, we can consider the bitrate of reporting in terms of payload bits per unit time. Assuming CSI-RS period is d, then reporting bitrate for scheme #1 is X/(2.d).d = X/2 bits per CSI-RS period. Similarly, for scheme #2 the reporting bitrate is Y/4 bits per CSI-RS period.

Figure 4-2 Schemes with different reporting periodicities and report payload size.

For compressing schemes with different reporting periodicities and/or payload size, for fair comparison consider the reporting bitrate as the overhead metric, e.g., number of bits per CSI-RS period.

### Format of the results table

To capture the study results on joint prediction and compression, we can consider Rel-18 results Table-1 [1] as a starting point, as the main goal is to evaluate the ML compression performance. Additionally, we can modify the table by including the time-domain assumptions similar to Rel-18 results Table-6 [1]. In particular, the time domain assumptions include:

- UE speed (km/h)

- CSI feedback periodicity

- Observation window (number/distance)

- Prediction window (number/distance between prediction instances/distance from the last observation instance to the 1st prediction instance)

- Whether/how to adopt spatial consistency

Additionally, we need to include the description of the prediction method used with the benchmark and the one used with the ML algorithm. The description of the prediction method includes at least the following:

- Prediction input type (e.g., channel, or precoder).

- Prediction output type (e.g., channel, or precoder).

- Prediction method (e.g., Ideal, nearest-historical-CSI, ML prediction, or non-ML prediction).

For capturing the evaluations results on temporal domain compression, Rel-18 results Table-1 can be used as a starting point with the following modifications:

- Add the time domain assumptions (e.g., UE speed, CSI feedback periodicity, observation window, prediction window, …, etc.).

- Add the descriptions of the prediction algorithms for the baseline and the AI/ML algorithms (e.g., input type, output type, and prediction method.).

- If CSI feedback periodicity is different from the CSI-RS periodicity, the overhead is normalized and expressed in terms of reporting bitrate (e.g., bits per CSI-RS period).

## Evaluation results

This section includes system simulation results on the benefit of up-to-date CSI, analysis on the compression and delayed CSI impact for different Dense Urban datasets, and initial results on joint prediction and compression in two scenarios.

### Benefits of up-to-date CSI

In this section, we show the potential gain in system-level performance from up-to-date CSI availability. Consider the case where we have a 10 slot CSI-RS periodicity. In addition to this, assume we have a CSI delay from processing at UE and/or gNB side. The cumulative delay by which the stale CSI is utilized by the gNB for downlink scheduling will cause a system performance loss. By implementing a CSI prediction algorithm, at least some of this loss can be reclaimed. As the CSI delay increases, the greater the loss from stale CSI, and the scope of gain possible through the usage of a CSI prediction algorithm.

Figure  below show the loss in system performance for various UE speeds for the urban macro scenario, in terms of mean, edge and cell-center performance. The loss shown for each scheme is the percentage reduction in the user perceived throughput due to the delayed CSI under that scheme when compared to a benchmark scheme that assumes CSI feedback with no delay. We assumed a low offered load (< 20% resource utilization) with bursty traffic and Release 16 eType2 with Parameter Combination of 4. Note that the same compression (eType2 PC4) is applied to the benchmark scheme also to convey the CSI feedback.

Figure 4-3: Impact of Stale CSI on User Experience of Mean, Edge and Cell-center UEs (3–120 kmph, 11-444 Hz Doppler)

At high CSI-delays and high Doppler, potential performance gain from a good CSI-prediction algorithm can be over 50% at cell edge. Even at low Doppler, gains of 20-30% are possible around 20 slot delays.

At high CSI-delays and high Doppler, potential performance gain from a good CSI-prediction algorithm can be more than 50% at cell edge. Even at low Doppler, gains of 20-30% are possible around 20 slot delays.

### Trade-off between compression error and delay impact

As discussed earlier, for a given CSI payload range, we want to determine whether a given scenario is compression-limited, prediction-limited, or prediction-compression-limited. We compare the effect of delayed CSI and compression separately to quantify their impact on SGCS. If SGCS degradation from delayed CSI is much larger than degradation from compression, then a scenario is considered prediction-limited. Similarly, if SGCS degradation from compression is much larger than delayed CSI, then a scenario is considered compression limited. If their impact is comparable, then a scenario is considered prediction-compression-limited.

Figure 4-4 Illustration of how the impact of delayed CSI and compression on SGCS is computed

In the evaluations, we consider a CSI measurement window of 1 sample and a CSI prediction window of 1 sample. We compute the SGCS between the current precoder sample Vt and the nearest historical precoder Vt-T without compression, for CSI delay ‘T=1’, as illustrated in Figure 4-4. For the compression error, we consider ideal prediction (i.e., delay T=0), and compute the SGCS between Vt and the reconstructed Vt assuming a CSI payload of 128 bits.

Our results on the impact of delayed CSI and compression for four different scenarios are shown in Figure 4-5, where the SGCS is computed for layer 1 and the scenarios are described in Table 4-1. From the results, we observe that ‘DU-3km-5ms’ is a compression-limited scenario since the compression SGCS is much lower compared to SGCS from delayed CSI. On the other hand, for scenario ‘DU-30km-5ms’, the compression SGCS is much better than SGCS from delayed CSI, i.e., this is a prediction-limited scenario. For the other two scenarios, the performance will be limited be both prediction and compression, i.e., we have a prediction-compression-limited scenario.

Figure 4-5 The impact of delayed CSI and compression on SGCS in different Dense Urban scenarios

Table 4-1 Description of the different dense urban datasets

| Label | Outdoor UEs (%) | UE speed (km/h) | CSI-RS period (ms) | #subbands |
| --- | --- | --- | --- | --- |
| DU-10km-5ms | 100 | 10 | 5 | 13 |
| DU-30km-5ms | 100 | 30 | 5 | 12 |
| DU-3km-20ms | 20 | 3 | 20 | 13 |
| DU-3km-5ms | 20 | 3 | 5 | 13 |

The SGCS analysis on the impact of delayed CSI and ML compression error illustrates the following:

- Outdoor Dense Urban scenarios (mostly LOS UEs) with medium/high UE speed (>30 km/h) and CSI-RS periodicity of 5ms are prediction-limited.

- Indoor Dense Urban scenarios (mostly NLOS UEs) with CSI-RS periodicity of 5ms and UE speed 3km/h are compression-limited.

- Outdoor Dense Urban scenarios (mostly LOS UEs) with low UE speed (10 km/h) and CSI-RS periodicity of 5ms are prediction-compression-limited.

- Indoor Dense Urban scenarios (mostly NLOS UEs) with CSI-RS periodicity of 20ms and UE speed 3km/h are prediction-compression-limited.

### Initial results on joint compression and prediction (Case 2A)

In this section, we present results on ML performance for joint compression and prediction in two different scenarios, a prediction-limited scenario (dense urban with 100% outdoor UE and medium speed) and a compression-limited scenario (dense urban with 20% outdoor UEs and very low speed).

#### Scenario 1: Dense Urban with 100% outdoor UEs and medium speed

Figure 4-6 shows compression SGCS versus payload size for reporting one TD sample of layer 1 in a Dense Urban scenario with 100% outdoor UEs, UE speeds of 30km/h, and CSI-RS periodicity of 5ms. We compare the compression performance under ideal and realistic precoder prediction to illustrate that end-to-end performance is dominated by the prediction SGCS. We observe that reducing the payload size from 128 to 16 bits has limited impact on the end-to-end SGCS when realistic prediction algorithm is used. While, under ideal prediction SGCS improves with increasing the payload size. This confirms that this scenario is prediction-limited and may be less suited for comparing different compression algorithms.

Figure 4-6 Compression performance in SGCS under ideal and realistic prediction for scenario1.

For a Dense Urban scenario with 100% outdoor UEs, 5ms CSI periodicity, and UE speeds of 30km/h, the end-to-end SGCS performance is limited by the prediction accuracy and reducing the CSI payload from 128 bits to 16 bits leads to only a small degradation in end-to-end SGCS.

#### Scenario 2: Dense Urban with 80% indoor UEs and very low speed

To quantify the achievable gain from jointly compressing multiple TD samples, we present in Figure 4-8 the compression SGCS under ideal prediction for different CSI payloads normalized by number of TD samples (bits per TD sample). In this scenario, we consider a Dense Urban scenario with 80% indoor UEs, UE speeds of 3km/h, and CSI-RS periodicity of 5ms. In particular, we consider an observation/prediction window size of {1, 2, or 4} TD samples and compare their SGCS performance. The compression scheme under ideal and realistic prediction is illustrated in Figure 4-7.

Figure 4-7 Illustration of compression of multiple TD samples under ideal and realistic prediction algorithms.

From the results, we observe that compressing multiple TD samples outperforms the ML baseline of compressing 1 TD sample. The gain from compressing multiple TD samples can be realized as SGCS improvement for the same number of bits per TD sample. Alternatively, for the same total payload size per report, we can report multiple TD samples, e.g., 2 or 4 samples, at the cost of slight degradation in SGCS compared with 1 TD sample per report. The gains shown in this figure are upper bounds on the achievable gains with a realistic prediction algorithm.

SGCS performance for a compression scheme with ideal prediction can be used as an upper-bound on the achievable end-to-end SGCS when this scheme is used with a realistic prediction algorithm.

For the evaluation of temporal domain compression, include the ideal prediction scenario as one evaluation scenario for reference, i.e., a comparison of different temporal domain CSI compression schemes where the CSI prediction is ideal.

Figure 4-8 SGCS compression performance for different number of TD samples under ideal prediction for scenario2.

In Figure 4-9, we compare SGCS performance for compressing multiple TD samples under ideal prediction and realistic precoder prediction algorithm. From the results we observe a minor degradation in SGCS (< 0.35 dB) when comparing the ideal and realistic prediction algorithms. This indicates that for this scenario we can accurately estimate the TD samples in the prediction window and the end-to-end performance is mainly limited by the compression accuracy.

Figure 4-9 SGCS performance for compressing multiple TD samples under ideal and realistic prediction for scenario2.

For a Dense Urban scenario with 80% indoor UEs, 5ms CSI periodicity, and UE speeds of 3km/h, the end-to-end SGCS performance is limited by the compression accuracy. For joint prediction and compression of multiple TD sample, a minor degradation in end-to-end SGCS (< 0.35 dB) is observed when comparing the ideal and realistic prediction algorithms.

# Conclusions

In this document, we have discussed aspects related to AI/ML-based CSI compression using two-sided model. We have the following observations:

Observation 1:	To achieve the benefit of data-driven optimization of the CSI feedback scheme, the PMI mapping (i.e., the mapping between the precoding matrix indicator (PMI) in the CSI feedback payload and the precoding matrix it represents) cannot be pre-determined and standardized.

Observation 2:	Channel characteristics in a real deployment are likely to have larger location-dependent variability as compared to the variability observed in a synthetic dataset derived using the spatial channel model.

Observation 3:	Localized models, which are developed specifically for use within a local region, provide better performance compared to global models of comparable size trained using data from the entire layout.

Observation 4:	Localized models provide similar performance with a smaller model complexity compared to global models.

Observation 5:	Evaluation scenarios considered in Rel-18 study on CSI prediction assume 100% outdoor UEs. For Dense Urban scenarios, outdoor UEs have mostly line-of-sight channels and CSI compression is not the performance bottleneck in the system.

Observation 6:	Scenarios where the end-to-end performance is either compression-limited or prediction-compression-limited are best suited for studying different compression algorithms. In these scenarios changing the compression algorithm and/or the CSI payload size translate to noticeable change in end-to-end SGCS.

Observation 7:	Rel-18 etype-II codebook for medium/high Doppler allows configuring an aperiodic CSI-RS burst with resources every one or two slots, which helps in improving CSI prediction/compression accuracy especially for high UE speeds.

Observation 8:	At high CSI-delays and high Doppler, potential performance gain from a good CSI-prediction algorithm can be more than 50% at cell edge. Even at low Doppler, gains of 20-30% are possible around 20 slot delays.

Observation 9:	The SGCS analysis on the impact of delayed CSI and ML compression error illustrates the following:

- Outdoor Dense Urban scenarios (mostly LOS UEs) with medium/high UE speed (>30 km/h) and CSI-RS periodicity of 5ms are prediction-limited.

- Indoor Dense Urban scenarios (mostly NLOS UEs) with CSI-RS periodicity of 5ms and UE speed 3km/h are compression-limited.

- Outdoor Dense Urban scenarios (mostly LOS UEs) with low UE speed (10 km/h) and CSI-RS periodicity of 5ms are prediction-compression-limited.

- Indoor Dense Urban scenarios (mostly NLOS UEs) with CSI-RS periodicity of 20ms and UE speed 3km/h are prediction-compression-limited.

Observation 10:	For a Dense Urban scenario with 100% outdoor UEs, 5ms CSI periodicity, and UE speeds of 30km/h, the end-to-end SGCS performance is limited by the prediction accuracy and reducing the CSI payload from 128 bits to 16 bits leads to only a small degradation in end-to-end SGCS.

Observation 11:	SGCS performance for a compression scheme with ideal prediction can be used as an upper-bound on the achievable end-to-end SGCS when this scheme is used with a realistic prediction algorithm.

Observation 12:	For a Dense Urban scenario with 80% indoor UEs, 5ms CSI periodicity, and UE speeds of 3km/h, the end-to-end SGCS performance is limited by the compression accuracy. For joint prediction and compression of multiple TD sample, a minor degradation in end-to-end SGCS (< 0.35 dB) is observed when comparing the ideal and realistic prediction algorithms.

We have the following proposals:

Proposal 1:	Study potential specification support to enable vendors to achieve a common understanding of the PMI mapping (i.e., the mapping between the precoding matrix indicator (PMI) in the CSI feedback payload and the precoding matrix it represents), both for training and inference.

Proposal 2:	To address concerns related to inter-vendor training collaboration complexity, study the approach of 2-sided model training based on registering the mapping between the PMI payload and the preferred precoding matrix that it represents.

Proposal 3:	The procedure to enable a training entity to register and retrieve the PMI-precoder mapping information from the central registry can be standardized.

Proposal 4:	Study the performance improvement associated with localized models (models developed specifically for use within a local region) compared to global models.

Proposal 5:	For the evaluation of localized models, the local region could be defined as UEs dropped within a specific boundary (e.g., 25 m. by 25 m. square), with the same serving cell, and with the same indoor/outdoor state.

Proposal 6:	Study techniques and potential specification impact to enable the use of localized models to achieve the associated improvement in the performance-complexity tradeoff.

Proposal 7:	Consider the following sub-use cases related to temporal domain CSI compression:

- Case 1: Compress present CSI and report.

  - Case 1A: Report is a standalone report.

  - Case 1B: Report is a differential report that depends on a prior report.

- Case 2: Predict one future instance or multiple present/future instances of CSI, compress them and report.

  - Case 2A: Report is a standalone report.

  - Case 2B: Report is a differential report that depends on a prior report.

Proposal 8:	For studying different compression algorithms, the evaluation scenarios should at least include cases where the end-to-end performance is either compression-limited or prediction-compression-limited.

Proposal 9:	Study the benefits of ML compression over time/frequency/spatial domains in

- Indoor scenarios having mostly NLOS and low-speed UEs (e.g., Dense Urban scenario with 80% indoor UEs)

- Outdoor scenarios having mostly LOS UEs (e.g., Dense Urban with 100% outdoor UEs) and different UE speeds.

Proposal 10:	For outdoor scenarios with UE speeds of 30 km/h or higher, study the performance assuming an aperiodic CSI-RS burst with K resources which is scheduled periodically.

Proposal 11:	For compressing schemes with different reporting periodicities and/or payload size, for fair comparison consider the reporting bitrate as the overhead metric, e.g., number of bits per CSI-RS period.

Proposal 12:	For capturing the evaluations results on temporal domain compression, Rel-18 results Table-1 can be used as a starting point with the following modifications:

- Add the time domain assumptions (e.g., UE speed, CSI feedback periodicity, observation window, prediction window, …, etc.).

- Add the descriptions of the prediction algorithms for the baseline and the AI/ML algorithms (e.g., input type, output type, and prediction method.).

- If CSI feedback periodicity is different from the CSI-RS periodicity, the overhead is normalized and expressed in terms of reporting bitrate (e.g., bits per CSI-RS period).

Proposal 13:	For the evaluation of temporal domain compression, include the ideal prediction scenario as one evaluation scenario for reference, i.e., a comparison of different temporal domain CSI compression schemes where the CSI prediction is ideal.

# References

- TR 38.843 v18.0.0, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface” (Release 18), December 2023.

- RP-234039, “New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, Qualcomm (Moderator), 3GPP TSG RAN #102, December 2023.