# 3GPP TSG-RAN WG1 Meeting #116 															R1-2401435

Athens, Greece, Feb 26th – Mar 1st, 2024

Agenda item:	9.1.3.3

Source: 	Qualcomm Incorporated

Title: 	Other aspects of AI/ML model and data

Document for:	Discussion/Decision

# Introduction

The Rel-18 study on AI/ML Air Interface [1] has been concluded. The study has been captured into the Technical Report TR 38.843 [2]. Based on the Rel-18 study outcome, at RAN #102, a new Work Item on artificial intelligence/machine learning for NR air interface has been approved in [3], with the following objectives:

Objective of SI or Core part WI or Testing part WI Provide specification support for the following aspects: - AI/ML general framework for one-sided AI/ML models within the realm of what has been studied in the FS_NR_AIML_Air project [RAN2]: - Signalling and protocol aspects of Life Cycle Management (LCM) enabling functionality and model (if justified) selection, activation, deactivation, switching, fallback - Identification related signalling is part of the above objective - Necessary signalling/mechanism(s) for LCM to facilitate model training, inference, performance monitoring, data collection (except for the purpose of CN/OAM/OTT collection of UE-sided model training data) for both UE-sided and NW-sided models - Signalling mechanism of applicable functionalities/models - Beam management - DL Tx beam prediction for both UE-sided model and NW-sided model, encompassing [RAN1/RAN2]: - Spatial-domain DL Tx beam prediction for Set A of beams based on measurement results of Set B of beams (“BM-Case1”) - Temporal DL Tx beam prediction for Set A of beams based on the historic measurement results of Set B of beams (“BM-Case2”) - Specify necessary signalling/mechanism(s) to facilitate LCM operations specific to the Beam Management use cases, if any - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE NOTE: Strive for common framework design to support both BM-Case1 and BM-Case2 - Positioning accuracy enhancements, encompassing [RAN1/RAN2/RAN3]: - Direct AI/ML positioning: - (1st priority) Case 1: UE-based positioning with UE-side model, direct AI/ML positioning - (2nd priority) Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning - (1st priority) Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning - AI/ML assisted positioning - (2nd priority) Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning - (1st priority) Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning - Specify necessary measurements, signalling/mechanism(s) to facilitate LCM operations specific to the Positioning accuracy enhancements use cases, if any - Investigate and specify the necessary signalling of necessary measurement enhancements (if any) - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE for relevant positioning sub use cases - Core requirements for the above two use cases for AI/ML LCM procedures and UE features [RAN4]: - Specify necessary RAN4 core requirements for the above two use cases. - Specify necessary RAN4 core requirements for LCM procedures including performance monitoring. Study objectives with corresponding checkpoints in RAN#105 (Sept ’24): - CSI feedback enhancement [RAN1]: - For CSI compression (two-sided model), further study ways to: - Improve trade-off between performance and complexity/overhead - e.g., considering extending the spatial/frequency compression to spatial/temporal/frequency compression, cell/site specific models, CSI compression plus prediction (compared to Rel-18 non-AI/ML based approach), etc. - Alleviate/resolve issues related to inter-vendor training collaboration. while addressing other aspects requiring further study/conclusion as captured in the conclusions section of the TR 38.843. - For CSI prediction (one-sided model), further study performance gain over Rel-18 non-AI/ML based approach and associated complexity, while addressing other aspects requiring further study/conclusion as captured in the conclusions section of the TR 38.843 (e.g., cell/site specific model could be considered to improve performance gain). - Necessity and details of model Identification concept and procedure in the context of LCM [RAN2/RAN1] - CN/OAM/OTT collection of UE-sided model training data [RAN2/RAN1]: - For the FS_NR_AIML_Air study use cases, identify the corresponding contents of UE data collection - Analyse the UE data collection mechanisms identified during the FS_NR_AIML_Air (TR 38.843 section 7.2.1.3.2) study along with the implications and limitations of each of the methods - Model transfer/delivery [RAN2/RAN1]: - Determine whether there is a need to consider standardised solutions for transferring/delivering AI/ML model(s) considering at least the solutions identified during the FS_NR_AIML_Air study - Testability and interoperability [RAN4]: - Finalize the testing framework and procedure for one-sided models and further analyse the various testing options for two-sided models, in collaboration with RAN1, and including at least: - Relation to legacy requirements - Performance monitoring and LCM aspects considering use-case specifics - Generalization aspects - Static/non-static scenarios/conditions and propagation conditions for testing (e.g., CDL, field data, etc.) - UE processing capability and limitations - Post-deployment validation due to model change/drift - RAN5 aspects related to testability and interoperability to be addressed on a request basis NOTE: offline training is assumed for the purpose of this project. NOTE: the outcome of the study objectives should be captured in TR 38.843 for future reference. NOTE: Coordination with SA/SA WGs of the ongoing study/work as it may relate to their required work. Objective of Performance part WI - For Beam Management and Positioning Accuracy enhancement use cases, specify performance requirements and test cases for AI/ML LCM procedures (including performance monitoring) and UE features enabled by UE-sided models - Specify necessary performance requirements and tests (including metrics) for the above-mentioned use cases - Specify necessary test cases and performance requirements for LCM procedure, including performance monitoring.

In this contribution, for the sub-agenda “other aspects of AI/ML model and data”, we discuss the following aspect among the Rel-19 objectives.

Study objectives with corresponding checkpoints in RAN#105 (Sept ’24): - Necessity and details of model Identification concept and procedure in the context of LCM [RAN2/RAN1] - CN/OAM/OTT collection of UE-sided model training data [RAN2/RAN1]: - For the FS_NR_AIML_Air study use cases, identify the corresponding contents of UE data collection - Analyse the UE data collection mechanisms identified during the FS_NR_AIML_Air (TR 38.843 section 7.2.1.3.2) study along with the implications and limitations of each of the methods - Model transfer/delivery [RAN2/RAN1]: - Determine whether there is a need to consider standardised solutions for transferring/delivering AI/ML model(s) considering at least the solutions identified during the FS_NR_AIML_Air study

# Model identification concept and procedure in the context of LCM

The following regarding model identification has been agreed, and captured in TR 38.843 based on the Rel-18 study:

Study objectives with corresponding checkpoints in RAN#105 (Sept ’24): - Necessity and details of model Identification concept and procedure in the context of LCM [RAN2/RAN1]

Correspondingly, this section discusses model identification.

## Background

Two LCM flavors, namely functionality-based LCM and model-ID-based LCM have been discussed in the Rel-18 study. The boundary between the two is blurry in that:

From TR 38.843 ... From RAN1 perspective, an AI/ML model identified by a model ID may be logical, and how it maps to physical AI/ML model(s) may be up to implementation. ... For functionality/model-ID based LCM, once functionalities/models are identified, the same or similar procedures may be used for their activation, deactivation, switching, fallback, and monitoring. Model ID, if needed, can be used in a Functionality (defined in functionality-based LCM) for LCM operations.

Regardless of the naming, for the UE-side models (including the UE-part of two-sided models), the primary objective is to establish common understanding between NW and UE regarding the AI/ML capability/applicability at the UE-side and to facilitate UE-side model operations (either via model ID or transparently inside a functionality). In this regard, handling of NW-side additional conditions for UE-side models is a key discussion, of which model identification is one of the possible solutions.

Toward this end, the Rel-18 study concluded the following:

From TR 38.843 ... For an AI/ML-enabled feature/FG, additional conditions refer to any aspects that are assumed for the training of the model but are not a part of UE capability for the AI/ML-enabled feature/FG. It does not imply that additional conditions are necessarily specified. Additional conditions can be divided into two categories: NW-side additional conditions and UE-side additional conditions. Note: whether specification impact is needed is a separate discussion. For inference for UE-side models, to ensure consistency between training and inference regarding NW-side additional conditions (if identified), the following options can be taken as potential approaches (when feasible and necessary): - Model identification to achieve alignment on the NW-side additional condition between NW-side and UE-side - Model training at NW and transfer to UE, where the model has been trained under the additional condition - Information and/or indication on NW-side additional conditions is provided to UE - Consistency assisted by monitoring (by UE and/or NW, the performance of UE-side candidate models/functionalities to select a model/functionality) - Other approaches are not precluded - Note: the possibility that different approaches can achieve the same function is not denied ... For AI/ML model identification of UE-side or UE-part of two-sided models, model identification is categorized in the following types: - Type A: Model is identified to NW (if applicable) and UE (if applicable) without over-the-air signalling - The model may be assigned with a model ID during the model identification, which may be referred/used in over-the-air signalling after model identification. - Type B: Model is identified via over-the-air signalling, - Type B1: - Model identification initiated by the UE, and NW assists the remaining steps (if any) of the model identification - the model may be assigned with a model ID during the model identification - Type B2: - Model identification initiated by the NW, and UE responds (if applicable) for the remaining steps (if any) of the model identification - the model may be assigned with a model ID during the model identification - Note: This study does not imply that model identification is necessary. One example use case for Type B1 and B2 is model identification in model transfer from NW to UE. Another example is model identification with data collection related configuration(s) and/or indication(s) and/or dataset transfer. Note: Other example use cases are not precluded. Note: Offline model identification may be applicable for some of the example use cases. Once models are identified, at least for Type A, UE can indicate supported AI/ML model IDs for a given AI/ML-enabled Feature/FG in a UE capability report as starting point. Note: model identification using capability report is not precluded for type B1 and type B2. Model ID may or may not be globally unique, and different types of model IDs may be created for a single model for various LCM purposes. Note: Details can be studied in the WI phase.

Beyond what’s agreed during the Rel-18 study and captured into the TR, there were a lot more discussions and further elaborations during the Rel-18 study on this topic. Below is the latest (not agreed) proposal in the Feature Lead summary [11].

For inference for UE-side models, further study the following approaches (when feasible and necessary) to ensure consistency between training and inference regarding NW-side additional conditions (if identified): Approaches; Description; What is provided for inference; Model identification. Model identification to achieve alignment on the NW-side additional condition between NW-side and UE-side; Model identification is may be used to align information and/or indication on NW-side additional conditions is offline.; Model ID signaling; Type A. Model identification is may be used along with the other approaches listed in the rows below of the this current table.; Model ID signaling; Type A, B1, B2. Information and/or indication on NW-side additional conditions is provided to UE; NW provides assistance information/indication on NW-side additional conditions for both data collection for training and inference.; NW provides assistance information/indication on NW-side additional conditions for both data collection for training and inference.; None. NW provides data collection related configuration(s) and/or indication(s) and/or dataset transfer; Model ID signaling; NW provides model ID or UE/UE-side identifies new model(s) based on measurement configuration(s) or data collection-related configuration(s) Type A, B1, B2. Consistency assisted by monitoring (by UE and/or NW, the performance of UE-side candidate models/functionalities to select a model/functionality); NW provides monitoring related configuration(s)/procedure(s); Model ID signaling if model is identified, otherwise model selection by UE based on monitoring outcome; Model may be identified Type A, B1. Model training at NW and transfer to UE, where the model has been trained under the additional condition; Model is trained under the NW-side additional conditions. Model is transferred to UE to be used for inference.; Model ID signaling; Type B2. - Different options may have different pros/cons in - Engineering Isolation between NW side and UE side - Scalability and NW-side LCM burden - Specification effort - Protection to network’s proprietary implementation - UE implementation complexity - UE power consumption and latency - Over-the-air signaling overhead - Further study can be pursued in the WI phase.

With the above background in mind from the Rel-18 study, in the next section, we discuss model identification and other approaches for ensuring consistency between training and inference regarding NW-side additional conditions for UE-side models.

## Handling of NW-side additional conditions for UE-side models

In this section, we discuss the various options for ensuring consistency between training and inference regarding NW-side additional conditions for UE-side models.

- Model identification to achieve alignment on the NW-side additional condition between NW-side and UE-side

- Model training at NW and transfer to UE, where the model has been trained under the additional condition

- Information and/or indication on NW-side additional conditions is provided to UE

- Consistency assisted by monitoring (by UE and/or NW, the performance of UE-side candidate models/functionalities to select a model/functionality)

### Model identification

We note that model identification a) is not mutually exclusive with b)-d). That is, model identification may be used along with with b)-d). Therefore, in this section, we focus on a flavor of model identification where the NW-side additional conditions are aligned offline. The flavor where the NW-side additional conditions are aligned online overlaps with b)-d) and can be discussed as a part of b)-d).

Model identification with offline alignment of NW-side additional conditions

In this solution, NW-side additional condition is aligned offline between NW-side and UE-side for training, model(s) are identified referring to the alignment, and model IDs are used during inference. In this way, consistency between training and inference regarding NW-side additional conditions is ensured.

In general, a model identification may be initiated by the NW-side. During offline collaboration for alignment, NW-side can provide indication on NW-side additional conditions to the UE-side(s) in the form of a (logical) model ID. Therefore, the model ID may represent a dataset ID and/or may be associated with underlying NW-side additional condition(s). In particular, a model ID is NOT associated with any particular UE vendors or UE types; the same model ID may be used across different UE vendors, different UE types, and different physical models at a UE, as the model ID only represents the underlying NW-side additional condition(s).

As an example, let’s assume that NW side helps creating two datasets – dataset 1 for certain gNB beam codebooks and dataset 2 for certain other gNB beam codebooks. NW side can help creating the two datasets, as the NW side knows the beam codebooks and their beam shapes and therefore knows how to categorize the beam codebooks into two groups to create the two datasets. Each categorized dataset may be assigned an ID, say ID 1 and ID 2, which we may call a dataset ID or a model ID. The name is not important, but in RAN1 discussion we have called it a model ID. This is called “model identification”. This will allow the UE side(s) to develop models specific to each dataset. During UE capability, UE indicates the supported model IDs, and during inference, NW indicates the model ID that UE should use for inference.

In this approach, model identification is trivial; model identification is trivially accomplished by the NW-side assigning/providing a model ID to the UE-side(s). Associated configurations/conditions with the dataset may also be shared by the NW-side with the UE-side(s). Once the NW-side shares the dataset with associated configuration/conditions and an assigned model ID, it is up to the UE-side to develop physical model(s) to add the support of the (logical) model ID at the UE. There is no model identification/registration from the UE-side to the NW-side. As the model is identified without over-the-air signaling, it belongs to Model Identification Type A.

### Model training at NW and transfer to UE

One solution to ensure consistency between training and inference regarding NW-side additional conditions is to train a model at NW and transfer to UE. As the NW is aware of its own NW-side additional conditions, NW can train one or more models, each for specific NW-side additional conditions, and for inference, have the model that matches the inference time NW-side additional conditions transferred to UE, thereby ensuring consistency between training and inference.

During model transfer, model being transferred may be identified by a model ID. This belongs to model identification Type B2.

### Information and/or indication on NW-side additional conditions

In this solution, NW-side additional condition is aligned via over-the-air signaling from NW to UE for training. Models may or may not be identified depending on flavors, which we discuss below.

We can categorize this solution into the following three flavors.

Assistance Information on NW-side additional conditions is provided over-the-air to UE for both training and inference

As an example, to facilitate the UE-side model development, NW signals information on gNB beam codebook, such as information characterizing gNB beam shapes, to help UE side(s) collect and categorize the dataset(s). This will help the UE side(s) to categorize dataset accordingly and develop one or multiple models, where each model can be associated with particular value(s) of gNB beam codebook signaled from the NW. During inference, similarly, NW signals information on gNB beam codebook being currently used. As the UE already knows the association between its models and gNB codebook information, there is no need of model identification.

NW provides linkage between data collection and inference

In this flavor, a signaling is defined representing linkage between data collection time NW-side additional conditions and inference time NW-side additional conditions. As an example, a QCL notion could be extended to provide linkage between the beams used for inference and the beams used for training. Specifically, for the beams appearing in the measurement (Set B) and reporting (Set A) configurations for inference, a QCL linkage may be provided with the beams configured for data collection for training.

This is an attractive approach that minimizes disclosure of NW-side proprietary information while ensuring consistency between training and inference regarding NW-side additional conditions for UE-side models.

NW provides data collection related configuration(s) and/or indication(s) and/or dataset transfer

In this flavor, NW provides data collection related configuration(s) and/or indication(s) for training data collection. During inference, information that connects to training data configuration and/or indication is provided. For example, NW may indicate a data collection ID during the data collection period, and during inference, NW may provide a data collection ID pointing to one of the previous data collection instances.

In this flavor, a data collection ID or a dataset ID may be regarded as a (logical) model ID, and the NW’s signaling of a data collection ID or a dataset ID may be regarded as a trivial “model identification”.

### Consistency assisted by monitoring

It may also be possible to rely on performance monitoring to ensure consistency between training and inference regarding NW-side additional conditions.

As a toy example, let’s assume that NW side has two different gNB beam codebook configurations, CB1 and CB2. Let’s further assume that, without knowledge of gNB beam codebook configurations, UE-side develops multiple models, model 1, 2, 3, and 4, where each model is developed based on certain hypothetical gNB beam shape assumptions. Let’s assume that model 1 happens to work well for CB1 (perhaps because the hypothetical gNB beam shape assumed for model 1 training matches reasonably well with CB1), and model 2 happens to work well for CB2. Suppose that gNB is using CB1 during inference. Prior to inference, UE blindly tries all the 4 models and monitors their performance. UE finds out that model 1 works well. Then, UE proceeds with using model 1 during inference. Note here that there is no model identification; NW does not know the existence of the four models at UE, and the model numbers 1-4 are internal to the UE-side.

Obviously, this trial-and-error approach based on monitoring is inefficient, as it will incur overhead in terms of latency and UE power consumption. It’s also quite possible that none of the hypothetical gNB beam shapes UE used for model training matches with the actual gNB codebooks, in which case even the best model selected out of monitoring may have a poorer performance compared to the other solutions based on offline/over-the-air alignment or model transfer.

One potential approach to relieve the latency and power consumption overhead is to perform model identification after monitoring. That is, once the best performing model (model 1 in the above example) is determined via monitoring, UE may initiate model identification, letting NW know that UE has a model that works (reasonably) well under the NW-side additional condition (which is unknown at UE) and gets a model ID assigned. NW can record the association between the assigned model ID and the NW-side additional condition, so that whenever NW uses the same NW-side additional condition toward the UE, NW may configure the model ID for the UE. Therefore, the UE needs to go through the trial-and-error monitoring only once for the given NW-side additional condition. Furthermore, other UEs of the same type from the same vendor supporting the model can also update their capability to indicate the support of the model ID to the NW.

While the above post-monitoring model identification may relieve the latency and power consumption overhead, it comes with the downside of the NW-side bookkeeping of individual UE’s model IDs.

Overall, the monitoring-based approach does not seem attractive due to the above-mentioned downsides.

### Summary and further discussion

The following table summarizes the above discussions.

Proposal 1: Agree on the following table.

| Approaches | Description | What is provided for inference |
| --- | --- | --- |
| a) Model identification to achieve alignment on the NW-side additional condition between NW-side and UE-side | Model identification (Type A) is used to align information and/or indication on NW-side additional conditions offline. | Model ID signaling |
|  | Model identification (Type A) is used to align information and/or indication on NW-side additional conditions online. This option is covered by b)-d). | Model ID signaling |
| b) Information and/or indication on NW-side additional conditions is provided to UE | NW provides assistance information/indication on NW-side additional conditions for inference. |  |
|  | NW provides linkage between data collection and inference |  |
|  | NW provides data collection related configuration(s) and/or indication(s) and/or dataset transfer | Information that connects to training data configuration and/or indication and/or dataset May be in the form of dataset ID, configuration ID, model ID, etc. |
| c) Consistency assisted by monitoring (by UE and/or NW, the performance of UE-side candidate models/functionalities to select a model/functionality) | NW provides monitoring related configuration(s)/procedure(s). Further, UE may identify applicable model(s) based on monitoring outcome. | Model ID signaling if model is identified, otherwise model selection by UE based on monitoring outcome |
| d) Model training at NW and transfer to UE, where the model has been trained under the additional condition | Model is trained under the additional conditions. Model is transferred to UE to be used for inference. | Use the transferred model |

Proposal 2: NW-side additional conditions for UE-side models may be provided in the form of model identification, in the form of assistance information, via signaling a linkage between data collection and inference, via data collection related configurations, via dataset transfer, and/or via model transfer.

## Two-sided models

For two-sided models, the Rel-18 study on AI/ML Air Interface [1] has concluded in TR 38.843 [2] that

In CSI compression using two-sided model use case, in order to select a CSI generation model compatible with the CSI reconstruction model used by the gNB, the following aspect has been proposed: - Pairing information can be established based on model identification

## Need of model identification

The following regarding model identification has been agreed, and captured in TR 38.843 based on the Rel-18 study:

Study objectives with corresponding checkpoints in RAN#105 (Sept ’24): - Necessity and details of model Identification concept and procedure in the context of LCM [RAN2/RAN1]

Rel-18 study on AI/ML Air Interface concluded with various use cases of model identification.

... One example use case for Type B1 and B2 is model identification in model transfer from NW to UE. Another example is model identification with data collection related configuration(s) and/or indication(s) and/or dataset transfer. Note: Other example use cases are not precluded. Note: Offline model identification may be applicable for some of the example use cases. ... In CSI compression using two-sided model use case, in order to select a CSI generation model compatible with the CSI reconstruction model used by the gNB, the following aspect has been proposed: - Pairing information can be established based on model identification ... For inference for UE-side models, to ensure consistency between training and inference regarding NW-side additional conditions (if identified), the following options can be taken as potential approaches (when feasible and necessary): - Model identification to achieve alignment on the NW-side additional condition between NW-side and UE-side

Observation 1: Model identification may be used for the following purposes:

- To achieve alignment on the NW-side additional condition between NW-side and UE-side

- To identify models during and after model transfer

- Along with data collection related configuration(s) and/or indication(s) and/or dataset transfer

- For pairing of two-sided models

A single generalizable model may work reasonably well but may not be as good as a model that is specifically developed for a particular scenario/configuration/dataset. In such a scenario, a reasonable approach could be to use functionality-based LCM without model identification as a baseline, for which a relatively loose performance is required (e.g., via RAN4 test), and whenever need arises, to develop specific model(s) optimized for a particular scenario/configuration/dataset. A tighter performance target could be associated with the specific model via the validation performance during the offline model training, and the tighter performance could be ensured via model monitoring during inference operation.

Observation 2: Models developed and trained for a particular scenario/configuration/dataset may provide better performance than a generalized model that covers a wide range of scenarios/configurations/datasets.

Proposal 3: Target performance may be associated with a model and aligned between the NW side and UE side during model identification.

Proposal 4: Model identification provides means for (1) identifying a stronger performance during model identification for a scenario/configuration/dataset, (2) correspondingly set a stronger performance target for the scenario/configuration/dataset, (3) and monitoring for the stronger performance via model monitoring during inference.

Proposal 5: Model identification may be used for selected scenario/configuration/datasets for enhanced performance within functionality-based LCM.

Some companies have raised concerns on large scale deployment scalability of model identification, questioning the scalability of inter-vendor collaboration (i.e., M infra vendors and N UE vendors resulting in M*N collaborations). However, there is no such need of M*N inter-vendor collaboration. As discussed earlier, model identification may be initiated and unilaterally decided by the NW-side (e.g., a logical model associated with NW-side additional conditions, a logical model representing two-sided pairing in NW-first sequential training). Once a model is identified by the NW-side, the same model ID may be used across different UE vendors, different UE types, and different physical models at a UE, without involving collaboration with the NW-side. Therefore, there is no scalability concern.

Observation 3: Model identification requires minimal-to-none inter-vendor collaboration effort and does not have a scalability issue.

Regardless of the model identification type, once a model is identified, there is only minor specification impact for enabling models in functionality-based-LCM, mainly:

- UE indicating supported model IDs, e.g., in the UE capability

- NW configuring/signaling model ID for activation/switching/monitoring

Observation 4: Additional spec impact for model identification is minimal.

Considering mentioned use cases and various benefits of model identification, and considering minor additional specification impacts, it is natural to support model identification in specification.

Proposal 6: Support model identification in Rel-19 normative work.

# CN/OAM/OTT collection of UE-sided model training data

The following regarding the data collection for UE-side model training has been agreed, and captured in TR 38.843 based on the Rel-18 study:

##### 7.2.1.3.2 Data collection for UE-side model training The following proposals were discussed in RAN2: 1. UE collects and directly transfers training data to the Over-The-Top (OTT) server; 1a) OTT (TRansparent) 1b) OTT (non-TRansparent) 2. UE collects training data and transfers it to Core Network. Core Network transfers the training data to the OTT server. 3. UE collects training data and transfers it to OAM. OAM transfers the needed data to the OTT server. RAN2 did not study or analyse these proposals and did not agree to requirements or recommendations.

In the study item phase, RAN2 discussed different proposals and captured them as mentioned above. However, RAN2 could not analyze proposals, and thus could not provide its recommendation on data collection for UE-sided model training. Thus, the work item description (WID) “New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface” [RP - 234039] captured this for further study during the work item (as a continued study item from Rel-18). The WID captures the study objective as follows:

- CN/OAM/OTT collection of UE-sided model training data [RAN2/RAN1]: - For the FS_NR_AIML_Air study use cases, identify the corresponding contents of UE data collection - Analyse the UE data collection mechanisms identified during the FS_NR_AIML_Air (TR 38.843 section 7.2.1.3.2) study along with the implications and limitations of each of the methods

Note that the above study objective does not imply that the UE-sided model can be trained on the network side (e.g., CN or OAM), rather the RAN1/RAN2 discussion should be focused on data collection for model training on the UE side, considering the following

- Direct transfer of the collected data to the OTT server (in a transparent or non-transparent method)

- Transfer of the collected data to the OTT server (via CN or OAM)

Proposal 7: The RAN1/RAN2 discussion should be focused on data collection for model training on the UE side, considering the following

- Direct transfer of the collected data to the OTT server (in a 3GPP transparent or 3GPP non-transparent method)

- Transfer of the collected data to the OTT server (via CN or OAM).

Note that the input and output for a UE-side model is implementation specific. The actual inputs/outputs to/from the models used at the device may be different from the nominal inputs and outputs. For example, a model at UE may take auxiliary inputs such as SNR, Doppler, sensor measurements. Alternatively, the UE vendor may decide to develop one model for low SNR and another model for high SNR and switch among them transparently. Which auxiliary inputs to use and whether to develop one or multiple models is an implementation-specific decision per device/chipset development and cannot be pre-determined/standardized. As another example, a model at UE may use pre-processing and post-processing that may make the actual input/output to/from the model different from the nominal ones. What pre/post-processing should be used and what should be the actual inputs and outputs of a model is an implementation-specific decision per device/chipset development and cannot be pre-determined/standardized.

As mentioned above the actual input/output and side/auxiliary information are implementations-specific choices; and are proprietary information. Therefore, the data collected from/by a UE vendor should not be shared with other UE vendors, network vendors, operators, or third parties. To ensure the protection of the proprietary information. Therefore, any data collection solution that cannot ensure the protection of the proprietary information cannot be used for data collection for UE-side model training.

Observation 5: The actual input/output and side/auxiliary information for a UE-side model are implementations-specific choices and cannot be pre-determined/standardized.

Observation 6: The auxiliary/side information collected for the model development can be proprietary. Therefore, the data collected from/by a UE vendor should not be shared with other UE vendors, network vendors, operators, or third parties.

Proposal 8: A data collection method that cannot ensure the protection of the UE proprietary information cannot be used as data collection for UE-sided model training.

Note that during the runtime, which model(s) UE can run depends upon several UE conditions, e.g., UE power status, UE memory, the coexistence of different AI/ML features, the coexistence of different AI/ML features with non-AI/ML feature, and others. Therefore, for consistency between the training and inference of AI/ML functionalities/models, during the UE model development phase, the training entities may consider these runtime constraints at the UE. In our understanding, only the UE vendors can ensure this for UE models and infra-vendors can ensure this for infra models. Therefore, UE-side models can only be trained by UE vendors and network-side models can only be trained by network vendors, at least in the Rel-19 and foreseeable near future.

Observation 7: During the runtime, which model(s) UE can run depends upon several UE conditions, e.g., UE power status, UE memory, the coexistence of different AI/ML features, the coexistence of AI/ML features with non-AI/ML feature, and others.

Proposal 9: Considering the implementation-specific nature of the model input/output and auxiliary/side information, and considering the runtime constraints (as mentioned in observation 7), the UE-side model can only be trained by the UE vendor, at least in the Rel-19 and foreseeable near future.

# Model transfer/delivery

The following regarding model delivery/transfer has been agreed and captured in the TR through Rel-18 study:

From TR 38.843 ... Scenario/configuration specific (including site-specific configuration/channel conditions) models may provide performance benefits in some studied use cases (i.e., when a single model cannot generalize well to multiple scenarios/configurations/sites). - At least, when UE has limitation to store all related models, model delivery/transfer, if feasible, to UE may be beneficial, at the cost of overhead/latency associated with model delivery/transfer. - Note: On-device Finetuning/retraining, if feasible, of a single model may be an alternative to model delivery/transfer. - Note: a single model may generalize well in some studied use cases. - Note: Model transfer/delivery to UE may also face challenges, e.g., proprietary issues /burdens in some scenarios ... Table 4.3-1 introduces different options for model delivery/transfer to UE, training location, and model delivery/transfer format combinations for UE-side models and UE-part of two-sided models: Table 4.3-1: Model delivery/transfer cases Case; Model delivery/transfer; Model storage location; Training location. y; model delivery (if needed) over-the-top.; Outside 3GPP Network; UE-side / NW-side / neutral site. z1; model transfer in proprietary format.; 3GPP Network; UE-side / neutral site. z2; model transfer in proprietary format.; 3GPP Network; NW-side. z3; model transfer in open format.; 3GPP Network; UE-side / neutral site. z4; model transfer in open format of a known model structure at UE, i.e., an exact model structure as has been previously identified between NW and UE and for which the UE has explicitly indicated its support.; 3GPP Network; NW-side. z5; model transfer in open format of an unknown model structure at UE, i.e., any other model structure not covered in z4, including any model structure that is only partially known.; 3GPP Network; NW-side. Note: The definition of various Cases is only for the purpose of facilitating discussion and does not imply applicability, feasibility, entity mapping, architecture, signalling nor any prioritization.. When a model of a known structure at UE (e.g., Case z4) is transferred from the Network, the new model being identified (e.g., via Type B2) has the same structure as a previously identified model at the Network and UE. For model delivery/transfer to UE (for UE-side models and UE-part of two-sided models): - Model delivery/transfer to UE, if feasible, may be beneficial to handle scenario/configuration specific (including site-specific configuration/channel conditions) models (i.e., when a single model cannot generalize well to multiple scenarios/configurations/sites), to reduce the device storage requirement. - Model delivery/transfer to UE after offline compiling and/or testing may be friendlier from UE’s implementation point of view compared to the case without offline compiling and/or testing. On the other hand, the case without offline compiling and/or testing (that can update parameter with known model structure), may have benefit at least in terms of shorter model parameter update timescale. - Model transfer/delivery of an unknown structure at UE has more challenges related to feasibility (e.g. UE implementation feasibility) compared to delivery/transfer of a known structure at UE. - For model trained at network side, Case y (w/ NW-side training) and Case z2 may incur the burden of offline cross-vendor collaboration such as sending a model to the UE-side and/or compiling a model. - For model trained at UE side/neutral site, Case z1 and Case z3 may incur the burden of offline cross-vendor collaboration to send the trained model from the UE-side to the network, compared to Case y (w/ UE-side training) which does not have such burden. - Model storage at the 3GPP network, compared to storing the model outside the 3GPP network, may come with 3GPP network side burden on model maintenance/storage. - Proprietary design disclosure concern may arise from model training and/or model storage at the network side compared to other cases (such as case y with UE side training) which does not have such issue.

Considering devices’ capability and engineering practices, it is mandatory that, for any model delivered/transferred to the target UEs, the model (including its structure and parameters) should have been fully tested for the target UEs, and its support should have been indicated by the UE capability. Any model delivery/transfer that does not meet this requirement should be deprioritized.

Proposal 10: For any model delivered/transferred to the target UEs, the model (including its structure and parameters) should have been fully tested for the target UEs, and its support should have been indicated by the UE capability. Any model delivery/transfer that does not meet this requirement should be deprioritized.

# Conclusions

Proposal 1: Agree on the following table.

| Approaches | Description | What is provided for inference |
| --- | --- | --- |
| a) Model identification to achieve alignment on the NW-side additional condition between NW-side and UE-side | Model identification (Type A) is used to align information and/or indication on NW-side additional conditions offline. | Model ID signaling |
|  | Model identification (Type A) is used to align information and/or indication on NW-side additional conditions online. This option is covered by b)-d). | Model ID signaling |
| b) Information and/or indication on NW-side additional conditions is provided to UE | NW provides assistance information/indication on NW-side additional conditions for inference. |  |
|  | NW provides linkage between data collection and inference |  |
|  | NW provides data collection related configuration(s) and/or indication(s) and/or dataset transfer | Information that connects to training data configuration and/or indication and/or dataset May be in the form of dataset ID, configuration ID, model ID, etc. |
| c) Consistency assisted by monitoring (by UE and/or NW, the performance of UE-side candidate models/functionalities to select a model/functionality) | NW provides monitoring related configuration(s)/procedure(s). Further, UE may identify applicable model(s) based on monitoring outcome. | Model ID signaling if model is identified, otherwise model selection by UE based on monitoring outcome |
| d) Model training at NW and transfer to UE, where the model has been trained under the additional condition | Model is trained under the additional conditions. Model is transferred to UE to be used for inference. | Use the transferred model |

Proposal 2: NW-side additional conditions for UE-side models may be provided in the form of model identification, in the form of assistance information, via signaling a linkage between data collection and inference, via data collection related configurations, via dataset transfer, and/or via model transfer.

Observation 1: Model identification may be used for the following purposes:

- To achieve alignment on the NW-side additional condition between NW-side and UE-side

- To identify models during and after model transfer

- Along with data collection related configuration(s) and/or indication(s) and/or dataset transfer

- For pairing of two-sided models

Observation 2: Models developed and trained for a particular scenario/configuration/dataset may provide better performance than a generalized model that covers a wide range of scenarios/configurations/datasets.

Proposal 3: Target performance may be associated with a model and aligned between the NW side and UE side during model identification.

Proposal 4: Model identification provides means for (1) identifying a stronger performance during model identification for a scenario/configuration/dataset, (2) correspondingly set a stronger performance target for the scenario/configuration/dataset, (3) and monitoring for the stronger performance via model monitoring during inference.

Proposal 5: Model identification may be used for selected scenario/configuration/datasets for enhanced performance within functionality-based LCM.

Observation 3: Model identification requires minimal-to-none inter-vendor collaboration effort and does not have a scalability issue.

Observation 4: Additional spec impact for model identification is minimal.

Proposal 6: Support model identification in Rel-19 normative work.

Proposal 7: The RAN1/RAN2 discussion should be focused on data collection for model training on the UE side, considering the following

- Direct transfer of the collected data to the OTT server (in a 3GPP transparent or 3GPP non-transparent method)

- Transfer of the collected data to the OTT server (via CN or OAM).

Observation 5: The actual input/output and side/auxiliary information for a UE-side model are implementations-specific choices and cannot be pre-determined/standardized.

Observation 6: The auxiliary/side information collected for the model development can be proprietary. Therefore, the data collected from/by a UE vendor should not be shared with other UE vendors, network vendors, operators, or third parties.

Proposal 8: A data collection method that cannot ensure the protection of the UE proprietary information cannot be used as data collection for UE-sided model training.

Observation 7: During the runtime, which model(s) UE can run depends upon several UE conditions, e.g., UE power status, UE memory, the coexistence of different AI/ML features, the coexistence of AI/ML features with non-AI/ML feature, and others.

Proposal 9: Considering the implementation-specific nature of the model input/output and auxiliary/side information, and considering the runtime constraints (as mentioned in observation 7), the UE-side model can only be trained by the UE vendor, at least in the Rel-19 and foreseeable near future.

Proposal 10: For any model delivered/transferred to the target UEs, the model (including its structure and parameters) should have been fully tested for the target UEs, and its support should have been indicated by the UE capability. Any model delivery/transfer that does not meet this requirement should be deprioritized.

# References

- RP-213599, New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, RAN #94e, Dec. 2021

- TR 38.843 v18.0.0, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18), Dec. 2023

- RP-234039, New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, RAN #102, Dec. 2023

- R1-2210708, Summary#1 of General Aspects of AI/ML Framework, Moderator (Qualcomm), 3GPP TSG RAN WG1 Meeting #110-Bis-e

- R1-2213003, Revised final summary of General Aspects of AI/ML framework, Moderator (Qualcomm), 3GPP TSG RAN WG1 Meeting #111

- R1-2301868, Final summary of General Aspects of AI/ML Framework, Moderator (Qualcomm), 3GPP TSG-RAN WG1 Meeting #112

- R1-2304054, Final summary of General Aspects of AI/ML Framework, Moderator (Qualcomm), 3GPP TSG-RAN WG1 Meeting #112-bis-e

- R1-2306052, Final summary of General Aspects of AI/ML Framework, Moderator (Qualcomm), 3GPP TSG-RAN WG1 Meeting #113

- R1-2308292, Final summary of General Aspects of AI/ML Framework, Moderator (Qualcomm), 3GPP TSG-RAN WG1 Meeting #114

- R1-2310374, Final summary of General Aspects of AI/ML Framework, Moderator (Qualcomm), 3GPP TSG-RAN WG1 Meeting #114bis

- R1-2312407, Final summary of General Aspects of AI/ML Framework, Moderator (Qualcomm), 3GPP TSG-RAN WG1 Meeting #115