3GPP TSG RAN WG1 #116 	R1-2401437

Athens, Greece, Feb 26th – Mar 1st, 2024

Agenda item:	9.2.2

Source: 	Qualcomm Incorporated

Title: 	CSI enhancements for up to 128 ports and UE-assisted CJT with non-ideal TRP synchronization

Document for:	Discussion/Decision

# Introduction

In this contribution, we discuss the following two objectives of Rel-19 MIMO [1]:

… - Specify CSI support for up to 128 CSI-RS ports, targeting FR1 - Type-I codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks - Type-II codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks, without modifying any codebook parameter other than introducing additional values for the number of ports codebook parameter(s) - Extension of CRI(s)-based CSI reporting (CQI/PMI/RI calculated per CRI for ≥1 CRIs) for hybrid beamforming supporting up to a total of 128 CSI-RS ports across all resources, with up to 32 CSI-RS ports per resource, without new codebook design - Specify UE reporting enhancement for CJT deployments under non-ideal synchronization and backhaul, targeting FR1, both FDD and TDD - Inter-TRP time misalignment and frequency/phase offset measurement and reporting, assuming legacy CSI-RS design, with stand-alone aperiodic reporting on PUSCH …

# CSI enhancements for up to 128 ports

According to WID, there are three objectives: a, b, and c, for CSI enhancements with up to 128 ports. In our understanding, the purpose of objective a and b is to support single report with more than 32 ports, and the possibly up to 4 CSI-RS resources are irrelevant to CRI – they only take a same role as legacy single CSI-RS resource configured within a CSI-RS resource set. While for objective c, the focus is extending legacy single-CRI report to multi-CRI report (each CRI-associated report is still constrained by up to 32 ports).

In our understanding, in objective a and b, there is no intention to support UE reporting CRI (neither objective c’s multi-CRI, nor legacy single-CRI), or in other words, 0-bit CRI for objective a and b.

It is expected that the above is the common understanding. But to avoid any confusion, the following proposal is made to clarify the scope of objective a and b.

Proposal 1: For up to 128-port CSI, no CRI is reported when CSI report is configured with a codebook of more than 32 ports.

In the reminder of this contribution, we refer objective a and b as enhancing non-CRI report with more than 32 ports, and we refer objective c as introducing multi-CRI report while each CRI is associated with up to only 32 ports. In short, we use non-CRI report to refer objective a and b, while muti-CRI report to refer objective c.

## Specific enhancements for non-CRI report (objective a & b)

For objective a and b, the WID states “supporting up to a total of 128 CSI-RS ports”. However, there are many values from 32 to 128, WID does not specify which values are supported and which are not. To support and commercialize a new number of CSI-RS ports, it would require a lot of effort to implement and testing it from both UE vendors and gNB vendors. Therefore, RAN1 should only select a subset of number of total ports in the range of [33, 128].

Considering existing deployment never support 12 or 24 CSI-RS ports, it makes sense to prioritize supporting 64 and 128 ports as a baseline, and further study the necessity to support other values such as 48, 72, 96. In other words, it is recommended that RAN1 deprioritize entries with N1=12 and N2=3 in the following Table 1, as they are less likely to be deployed in reality.

Table 1. Possible new {N1, N2} value combinations

| N2 N1 | 8 | 12 | 16 |
| --- | --- | --- | --- |
| 2 | 32 (existing) | 48 | 64 |
| 3 | 48 | 72 | 96 |
| 4 | 64 | 96 | 128 |

With the above analysis, the following proposal is proposed.

Proposal 2: For up to 128-port CSI with non-CRI report, prioritize specifying 64 and 128 ports.

- FFS whether to support other values such as 48, 72, 96 ports.

For objective a and b, it is obvious that up to four 32-port CSI-RS resources can be configured to build up 128 CSI-RS ports. However, how to map the 128 ports CSI-RS to time-frequency grid, i.e., the CSI-RS pattern, are still open. In the next two sections, the CSI-RS pattern design is discussed for 64 and 128 ports respectively.

### CSI-RS pattern for 64 ports

In high level, more CSI-RS ports can be supported by extending legacy CSI-RS pattern in time, and/or frequency domain. In the following, 2 potential solutions to design the CSI-RS pattern for 64 ports are considered and compared:

- Solution 1: Reuse Rel-15 CSI-RS pattern. FDM 2 CSI-RS resources in frequency domain.

- Solution 2: Reuse Rel-15 CSI-RS pattern. TDM 2 CSI-RS resources in a same slot.

In the following, we illustrate the details of the 2 solutions.

Solution 1 (FDM) is very simple. It reuses existing 32-port CSI-RS pattern (i.e. row 16, 17, and 18 of Table 7.4.1.5.3-1 in TS 38.211) with frequency density 0.5. Network can configure two CSI-RS resources, one on even RBs and the other on odd RBs, as illustrated in Figure 1.

Figure 1. FDM (solution 1) to support 64-port CSI-RS in a same slot

Solution 2 (TDM) is very simple as well. As illustrated by Figure 2, network can configure two TDMed CSI-RS resources in the same slot.

Figure 2. TDM (solution 2) to support 64-port CSI-RS in a same slot

One should notice that it is important to confine the two CSI-RS resources in the same slot, to guarantee the phase coherence between ports 0-to-31 and 32-to-63. Otherwise, if the two CSI-RS resources are distributed in two different slots, it would require UE to maintain downlink Rx phase coherence across different slots. There are extra efforts needed in UE implementation, e.g., UE has to lock the RF/hardware setting such as LNA and AGC across slots, to make sure the Rx phase does not change across those slots.

Comparing between putting the two CSI-RS in a same v.s. different slot(s), we have the following observations.

- From CSI-RS overhead perspective, they are the same.

- From performance point of view, putting them in a same slot would achieve better channel estimation performance. If the two CSI-RS are in two different slots, the channel variation due to Doppler or FTL residual error will result selecting a precoder which does not match with the real channel in a slot, because the PDSCH transmission actually applying this precoder is transmitted in one single slot, not across two slots.

- From specification point of view, RAN1 need to discuss restrictions for UE to maintain Rx phase coherence across different DL slots similar to the discussion had in Rel-17 UL DMRS bundling for UL coverage enhancement.

- From UE implementation point of view, putting them in a same slot would reduce channel estimation latency, remove the necessity to do buffering for channel estimation, remove the necessity to lock RF/hardware setting across slots.

- Last but not least, the issue gets more complicated in intra-band CA scenario. For example, if specification support distributing two CSI-RS resources in two slots, in intra-band CA, the chain effect will require a UE to maintain Rx phase coherence for not only 2 slots but much longer, depends on how long the chain is configured by NW. As illustrated in Figure 3, if NW configure CSI-RS reports on multiple intra-band CCs and the 2-slot CSI-RS resources of those reports overlap, due to the fact that the UE will share the same RF/hardware (such as LNA and AGC) across intra-band CCs, the setting for the RF/hardware cannot change across the overlapped many slots, not just 2 slots to maintain Rx phase coherence. One could argue that NW can break the chain by not configure overlapping 2-slot CSI-RS resources across CC. But then it introduces configuration restriction to NW. If confining two resources in a single slot, such restriction can be avoided.

Figure 3. Illustration the chain effect for DL Rx phase coherence in intra-band CA.

Next, the performance of solution 1 (FDM) and solution 2 (TDM) is compared. Intuitively, one should expect solution 2 can have better performance because solution 2 can potentially use density 1 (RE/RB/port) for CSI-RS, while solution 1 can only use density 0.5. This expectation is confirmed with the channel estimation result in Figure 4, which shows that with doubled frequency density of solution 2 over solution 1, more than 1 dB improvement is observed for solution 2, in terms of channel estimation NMSE (normalized mean square error).

Figure 4. Channel estimation NMSE for 64-port CSI-RS with solution 1 (FDM) and solution 2 (TDM, double frequency density).

However, when channel estimation gap translates to throughput performance, the gap of throughput between solution 1 and 2 is reduced. With Type-I codebook based on estimated CSI-RS ports, the gap between solution 1 and 2 is less than 0.1 dB.

Figure 5. Throughput for 64-port CSI-RS with solution 1 (FDM) and solution 2 (TDM, double frequency density).

Of course, solution 2 (TDM) can also be configured with density 0.5, which is the same density as solution 1 (FDM). It is expected that solution 1 and 2 has almost the same performance in this case.

With the above analysis, we can see that solution 1 and 2 have similar performance and both are very simple to implement. Solution 2 is slightly preferred because it can offer network flexibility to configure either 0.5 or 1 density, which have potential benefit in large delay spread channels.

Proposal 3: For up to 128-port CSI with non-CRI report, support 64-port codebook by K=2 of 32-port CSI-RS resources TDMed in a same slot.

### CSI-RS pattern for 128 ports

For 128-port CSI-RS, network need to configure four 32-port CSI-RS resources. Due to the same reason of Rx phase coherence issue, we don’t support distribute the 4 CSI-RS resources in different slots. With the 4 CSI-RS resources confined in a same slot, the straightforward way is to use both solution 1 (FDM) and solution 2 (TDM) to put the 4 resources in the same slot, as illustrated in Figure 6.

Figure 6. Solution 1 + solution 2 (FDM+TDM) to support 128-port CSI-RS in a same slot.

With the above analysis, the following proposal is proposed.

Proposal 4: For up to 128-port CSI with non-CRI report, support 128-port codebook by K=4 of 32-port CSI-RS resources FDMed and TDMed in a same slot.

### Type-I codebook design

For the design Type-I codebook with more than 32 ports, before considering changing the codebook structure for any potential performance improvement, the following question need to be considered:

- As a low-hanging fruit, how much performance gain it can achieve by simply extending the Rel-15 Type-I codebook to larger {N1, N2} values, without any codebook structure change?

The above is a critical open question to answer. For the evolution of 5G eco-system, if there is already significant gain with the “low-hanging fruit”, this can be deployed by UE and gNB vendors as a baseline mode quickly, which will bring already significant gain to cell phone users. The possibility of fast commercialization of this baseline mode is a huge benefit that RAN1 should consider. Given the specification effort to standardize this baseline mode is trivial, in the meantime, RAN1 can use most of the TUs in subsequent meetings to study various enhancements which changes the legacy Type-I codebook structure with potential further improvement over the baseline mode. Those further enhancements can be defined as an advanced mode for Rel-19 Type-I codebook.

With the above motivation to define the baseline mode, we studied the performance of this baseline mode with system level simulations. The key simulation assumptions are listed as below.

| Parameters |  |
| --- | --- |
| DL precoder | Legacy Type-I single panel codebook with extended {N1, N2} O1 = 4, O2 = 4 (for N2 >=1) |
| Scenario | Umi ISD 200 |
| Carrier frequency | 6 GHz |
| Simulation BW | 20MHz (50 RBs) |
| gNB antenna array | (M,N,P)=(4, 16, 2) DH = 0.5λ, dV = 0.8λ, 256 ±45° x-pol antennas |
| UE ant | (M,N,P) = (1, 2, 2) 4 x-pol antennas with (0/+90 deg) |
| Network | 1 tier; 7 cells, 21 sectors, 8 users/cell |
| UE speed | Mixed indoor(80%)/outdoor(20%) UEs scenario: indoor 3km/h, outdoor 30km/h 100% outdoor UEs scenario: 3km/h |
| Traffic Models | Full-buffer |
| Modulation | Up to 256-QAM |
| MU-MIMO | Enabled |

In the following, the performance of 64-port and 128-port type-I codebook in baseline mode is compared with 32-port type-I codebook. As shown in the following tables, under different scenarios of indoor/outdoor with different {N1, N2}, significant gain can already be achieved with 64-port and 128-port type-I codebook in baseline mode, without any other enhancement, compared to 32-port type-I codebook.

| Antenna array with 16 columns and 4 rows of dual polarized antenna elements |  |  |  |
| --- | --- | --- | --- |
| MU-MIMO, Mixed indoor/outdoor UEs |  |  |  |
|  | Average TPUT | Cell center (95%-tile) | Cell edge (5%-tile) |
| 32 ports (N1,N2) = (4, 4) | 1x | 1x | 1x |
| 64 ports (N1,N2) = (8, 4) | 1.33x | 1.37x | 1.51x |
| 128 ports (N1,N2) = (16, 4) | 1.92x | 1.81x | 2.34x |

| Antenna array with 16 columns and 4 rows of dual polarized antenna elements |  |  |  |
| --- | --- | --- | --- |
| MU-MIMO, Mixed indoor/outdoor UEs |  |  |  |
|  | Average TPUT | Cell center (95%-tile) | Cell edge (5%-tile) |
| 32 ports (N1,N2) = (16, 1) | 1x | 1x | 1x |
| 64 ports (N1,N2) = (16, 2) | 1.39x | 1.37x | 2.77x |
| 128 ports (N1,N2) = (16, 4) | 1.62x | 1.46x | 3.87x |

| Antenna array with 16 columns and 4 rows of dual polarized antenna elements |  |  |  |
| --- | --- | --- | --- |
| MU-MIMO, 100% outdoor UEs |  |  |  |
|  | Average TPUT | Cell center (95%-tile) | Cell edge (5%-tile) |
| 32 ports (N1,N2) = (4, 4) | 1x | 1x | 1x |
| 64 ports (N1,N2) = (8, 4) | 1.50x | 1.30x | 1.64x |
| 128 ports (N1,N2) = (16, 4) | 2.44x | 1.58x | 2.45x |

| Antenna array with 16 columns and 4 rows of dual polarized antenna elements |  |  |  |
| --- | --- | --- | --- |
| MU-MIMO, 100% outdoor UEs |  |  |  |
|  | Average TPUT | Cell center (95%-tile) | Cell edge (5%-tile) |
| 32 ports (N1,N2) = (16, 1) | 1x | 1x | 1x |
| 64 ports (N1,N2) = (16, 2) | 1.18x | 1.13x | 1.34x |
| 128 ports (N1,N2) = (16, 4) | 1.34x | 1.31x | 1.17x |

Observation 1: 64-port and 128-port type-I codebook in baseline mode already achieve significant gain over 32-port type-I codebook.

Based on the above observation and the justification provided to fast commercialization of 64-port and 128-port Type-I CSI, the following in proposed.

Proposal 5: For up to 128-port CSI with non-CRI report,

- Specify a baseline mode as reusing Rel-15 legacy Type-I codebook structure by simply extending to larger {N1, N2} without any codebook structure change.

- Further support an advanced mode for potential enhancements of 64-port and 128-port Type-I codebook with structure change. FFS enhancement details.

  - Any change of codebook structure, if specified, should be justified with performance gain over the baseline mode and reasonable impact to UE/gNB implementation complexity.

### Type-II design

When introducing larger {N1, N2}, one direct impact is SD selection: Select L={2,4, or optionally 6} SD bases out of an orthogonal set of N1N2, with a report indication of log2CN1N2,L bits, where the largest value of N1N2 is increased from existing largest 16 (for 32-port), possibly to 64 (for 128-port).

Based on the encoding/decoding of SD selection indication (Clause 5.2.2.2.3 of TS 38.214, as extracted below), any combinatorial value C(x,y) with x=0,1,…,N1N2-1 and y=1,…,L should be stored on UE memory, e.g. as in Table 5.2.2.2.3-1 of TS 38.214. It is noted that the required memory a low-level memory with high cost, due to the dynamic nature of L1 report).

| Decoding |  |
| --- | --- |
| Encoding |  |

With N1N2 increased to 64, a huge table with 64 rows is expected for UE to encode SD selection indication.

Besides, large number of bits for a combinatorial value can be expected, e.g. log2C63,4=20 bits.

Proposal 6: For up to 128-port CSI with non-CRI report, for SD basis selection of Type-II codebook, study mechanisms for combinatorial coefficients table C(x,y) requiring less memory than x=63.

## Specific enhancements for multi-CRI report (objective c)

According to legacy CSI framework, for either periodic or aperiodic CSI report, all CSI parameters are RRC-configured.

- The dynamic characteristics of A-CSI report is realized via aperiodic triggering state, wherein, under any A-CSI triggering state, all triggered report(s) have RRC-configured parameter CSI-ReportConfig{…} with no dynamic signaling to change its report-content-related component parameters.

For multi-CRI report, we see no reason to break this principle of RRC-configured CSI parameter. Therefore, we propose the following.

Proposal 7: For up to 128-port CSI with multi-CRI report, support RRC-configured the number of CRIs (denoted as M) in a CSI report config. Do not support dynamically change of M via DCI or MAC-CE.

Then the remaining work of multi-CRI report is mainly about CSI information bits packing and dropping.

Proposal 8: For up to 128-port CSI with multi-CRI report, specify procedure for CSI info bits packing and dropping for multi-CRI report. FFS details.

## Enhancements for both non-CRI and multi-CRI reports

### CSI-RS resource config

Regarding the configuration of CSI-RS for a non-CRI report with up to 128 ports (objective a and b), as discussed in Section 2.1.1, one should notice that it is important to confine the CSI-RS resources in the same slot, to guarantee the phase coherence between CSI-RS ports located in different CSI-RS resources for a same report. Otherwise, if the CSI-RS resources are distributed in different slots, it would require UE to maintain downlink Rx phase coherence across different slots. This is a challenging requirement for UE to achieve. There are extra efforts needed in UE implementation, e.g., UE has to lock the RF/hardware setting such as LNA and AGC across slots, to make sure the Rx phase does not change across those slots.

Comparing between putting the two CSI-RS in a same v.s. different slot(s), we have the following observations.

- From CSI-RS overhead perspective, they are the same.

- From performance point of view, putting them in a same slot would achieve better channel estimation performance. If the two CSI-RS are in two different slots, the channel variation due to Doppler or FTL residual error will result selecting a precoder which does not match with the real channel in a slot, because the PDSCH transmission actually applying this precoder is transmitted in one single slot, not across two slots.

- From specification point of view, RAN1 need to discuss restrictions for UE to maintain Rx phase coherence across different DL slots similar to the discussion had in Rel-17 UL DMRS bundling for UL coverage enhancement.

- From UE implementation point of view, putting them in a same slot would reduce channel estimation latency, remove the necessity to do buffering for channel estimation, remove the necessity to lock RF/hardware setting across slots.

- Last but not least, the issue gets more complicated in intra-band CA scenario. For example, if specification support distributing two CSI-RS resources in two slots, in intra-band CA, the chain effect will require a UE to maintain Rx phase coherence for not only 2 slots but much longer, depends on how long the chain is configured by NW. As illustrated in Figure 7, if NW configure CSI-RS reports on multiple intra-band CCs and the 2-slot CSI-RS resources of those reports overlap, due to the fact that the UE will share the same RF/hardware (such as LNA and AGC) across intra-band CCs, the setting for the RF/hardware cannot change across the overlapped many slots, not just 2 slots to maintain Rx phase coherence. One could argue that NW can break the chain by not configure overlapping 2-slot CSI-RS resources across CC. But then it introduces configuration restriction to NW. If confining two resources in a single slot, such restriction can be avoided.

Figure 7. Illustration the chain effect for DL Rx phase coherence in intra-band CA.

Based on the above observations, the following is proposed.

Proposal 9: For up to 128-port CSI with non-CRI report (objective a & b), the configured multiple CSI-RS resources have to be confined within a slot.

In existing TS 38.214 (Clause 5.2.2.3.1), CSI-RS resources within a same resource set are required as

All CSI-RS resources within one set are configured with same density and same nrofPorts, except for the NZP CSI-RS resources used for interference measurement. The UE expects that all the CSI-RS resources of a resource set are configured with the same starting RB and number of RBs and the same cdm-type.

For simplicity, we think the above restrictions can generally be reused, with the following necessary changes

- For the purpose of FDMed CSI-RS resources (to support K=4 of 32-port CSI-RS resources confined within a same slot), different RB offsets are needed;

- For non-CRI report (objective a & b), the K>1 CSI-RS resources basically takes the same role as legacy single CSI-RS resource, therefore, it makes sense for them to have a single-QCL and a single EPRE offset, as defined by legacy for a single CSI-RS resource.

Proposal 10: For up to 128-port CSI with non-CRI report (objective a & b), support CMR configured as a set of K>1 (and K is up to 4) CSI-RS resources where each resource is with

- A same number of ports (up to 32);

- A same CDM type, and a same frequency density;

- A same number of RBs;

- A same starting RB with potential different RB offset (for FDM);

- A same QCL;

- A same EPRE offset (PDSCH-to-CSIRS and CSIRS-to-SSB).

As for multi-CRI report (objective c), the last two bullets of the above proposal do not apply.

Proposal 11: For up to 128-port CSI with multi-CRI report (objective c), support CMR configured as a set of K>1 (and K is up to 4) CSI-RS resources where each resource is with

- A same number of ports (up to 32);

- A same CDM type, and a same frequency density;

- A same number of RBs;

- A same starting RB with potential different RB offset (for FDM).

For IMR configuration, for non-CRI report (objective a & b), although the report is configured with multiple CMR resources, there is no motivation to configure multiple IMR resources. Therefore, the following is proposed.

Proposal 12: For up to 128-port CSI with non-CRI report (objective a & b), support only a single IMR configured for a report with K>1 CMRs.

### CPU / active resource counting, CSI processing time relaxation

Regarding the CSI processing timeline, given that Rel-19 increases number of CSI ports and/or number of CRIs in a CSI report, apparently UE would need more time to generate the CSI report. In the following, the details of CSI processing time extension are discussed.

For non-CRI CSI report (objective a and b), the increment in a CSI report is on the dimension of number of CSI-RS ports, which is increased from up to 32 ports to P ports where P can be either 64 or 128. One could check that the complexity of calculating a non-CRI CSI report is linearly scaled with the number of CSI-RS ports, because both the channel estimation complexity and the number of total hypotheses to select PMI/RI are linearly scale with the number of CSI-RS ports.

Since the complexity of CSI calculation is linearly scaled with number of CSI-RS ports, the following is proposed.

Proposal 13: For up to 128-port CSI with non-CRI report (objective a & b), linearly scale the CSI process time Z and Z’ by P/32, where P is the total # CSI-RS ports (across all CSI-RS resources) associated with the CSI report config.

For multi-CRI CSI report (objective c), the increment in a CSI report is on two dimensions, i.e., the number of CRIs in a report is increased from one to more than one, and the number of CSI-RS ports per CSI-RS resource is increased from 16 or 8 to up to 32. One should notice that, according to 38.214 “If 𝐾𝑠 = 2 CSI-RS resources are configured, each resource shall contain at most 16 CSI-RS ports. If 2 < 𝐾𝑠 ≤ 8 CSI-RS resources are configured, each resource shall contain at most 8 CSI-RS ports”. While in Rel-19, up to 32 CSI-RS ports per resource is allowed.

The extension of process time for multi-CRI report should take the two-dimensional increment of complexity into account. Therefore, we envision the processing time need to increase in two dimensions as well. Firstly, Z and Z’ should be linearly scaled with a factor based on the ratio of number of ports increment per resource, which is similar to previous proposal. Secondly, an additional processing time should be given to UE to handle the multiple CRIs in the report, which is unique for the new multi-CRI report. The additional time extension should be linearly scaled with M-1, where M is the number of CRIs that NW requesting in the report. “M-1” is due to the legacy CSI processing timeline already support a single CRI report.

With the above analysis, the following is proposed for processing timeline extension for a multi-CRI report.

Proposal 14: For multi-CRI report (i.e., objective c), extend CSI process time Z and Z’ as the following

- If Ks = 2:

  - Z= Q/16 * Z + (M-1) * Y, Z’= Q/16 * Z’ + (M-1) * Y,

- Else if 2 < Ks <= 8:

  - Z = Q/8 * Z + (M-1)  * Y, Z’ = Q/8 * Z’ + (M-1) * Y

where Q is the max number of CSI-RS ports allowed in a CSI-RS resource associated with the multi-CRI report, Ks is the number of CSI-RS resources configured for the report, M is the number of CRIs configured to be included in the report, Y is a number of OFDM symbols.

- FFS the value of Y.

For either non-CRI report (objective a & b), or multi-CRI report (objective c), with the total ports increased, computational resource budget counting such as CPU, active resource, and ports counting becomes a crucial to specify in Rel-19.

According to observations in Rel-18, companies may tend to think CPU/active resources/ports counting as last meeting trivial tasks to go through. However, what happened in Rel-18 was that many issues on this topic were identified but with not sufficient time to discussed thus left open. We don’t want to report the same “unintentional” mistake as in Rel-18. Therefore, the following is proposed.

Proposal 15: For up to 128-port CSI, RAN1 start the work to specify the procedure for CPU counting, active CSI-RS resource counting, and active CSI-RS ports counting no later than RAN1 #117 (the 3rd meeting of Rel-19).

Specifically for active CSI-RS resource / port counting, we identified one issue may exist for objective a & b, based on that a single CSI report is configured with K>1 CSI-RS resources. For instance, considering following three cases

- Case A: 4 non-CRI reports each with 1 resource of 32 ports;

- Case B: 1 non-CRI report with 128 ports, which is configured with 4 resources and 32 ports per resource;

- Case C: 1 multi-CRI report with 4 resources each with 32 ports.

In current UE capability framework of active resource & ports counting, all the three cases are count as 4 active resources with total 128 ports. However, the UE complexity to handle the three cases are different. The UE complexity to handle case A is higher than case B, because case A requires 4 independent interference estimations. furthermore, case A requires 4 independent evaluation/selection of CQI/RI/PMI. A UE can support case B may not be able to support case A. For case C, depends on how many CRIs is requested by gNB and how UE implements CRI selection, the complexity can vary from similar to case A to lower than case B.

However, existing UE capability framework on total CSI-RS resources & ports is not able to differentiate the above cases. New UE feature group is needed to differentiate them.

Proposal 16: For up to 128-port CSI, define mechanism in active resource and ports capability report to differentiate supporting multiple non-CRI reports, a single non-CRI report, and a single multi-CRI report, which are configured with the same number of CSI-RS resources and the same number of total ports.

## Interaction with NES

Network energy saving (NES) is a Rel-18 (and also Rel-19) study that has considerable impact on CSI.

From UE perspective, it is not desirable if we just separately define two sets of mechanisms for 128-port CSI and NES – reusing or leveraging already-existed mechanisms is generally beneficial from either standard effort perspective, or from UE/network implementation effort perspective.

Proposal 17: For up to 128-port CSI, consider the network energy saving (NES) aspects.

# UE-assisted CJT with non-ideal TRP synchronization

## Use case identification

According to WID, there are three candidate contents to assist CJT: (1) Inter-TRP timing offset/misalignment; (2) Inter-TRP frequency offset; (3) Inter-TRP phase offset. A first question to ask is, how could these reported contents possibly assist CJT transmission? In our understanding, use cases should be identified first, to figure out under what scenario, which one or more of the three candidate contents may be needed.

### Scenario: Inter-TRP phase coherence analysis for distributed TRPs

For distributed TRPs with each individual clock sources, due to the frequency / oscillator (XO) difference between the clocks of the TRPs, inter-TRP phase will drift away over time and corrupt the CJT precoder. For instance, transmissions from two TRPs adding up to each other, may change to counteracting each other.

Some examples are provided in Table 2, with XO drift (i.e. frequency offset) e=0.05 ppm and 1 ppb respectively. It is noted that ±0.05 ppm is a worst case according to a basic RAN4 requirements on macro-site BS [2]. Here phase coherence is defined as maintained within a threshold (e.g. 3° or 5°).

Regarding how long a time duration t can make sure phase change 2π⋅2efc⋅t≤ phase_thr (here 2e is used as inter-TRP frequency offset due to it is the worst case of ±e), it is observed in Table 2 that even for some lower FR1 bands, e=0.05ppm only results in symbol-level (< 0.5 msec) phase coherence – this may not be workable for CJT, since it means non-coherent phase even within a CJT-PDSCH transmission.

On the other side, ppb-level XO drift allows phase coherence with several to ten milliseconds.

Table 2. Time duration (in msec) for inter-TRP phase maintained within a threshold (phase_thr)

| phase_thr fc | e = 0.05 ppm |  | e = 1 ppb |  |
| --- | --- | --- | --- | --- |
|  | 3° | 5° | 3° | 5° |
| 2 GHz | 0.042 | 0.069 | 2.08 | 3.47 |
| 700 MHz | 0.119 | 0.198 | 5.95 | 9.92 |

Observation 2: To guarantee phase-coherence for distributed TRPs within at least millisecond-level, ppb-level (or better) XO drift error should be guaranteed for the TRPs.

Proposal 18: For UE-assisted CJT with non-ideal TRP synchronization, focus Rel-19 study on the scenario of ppb-level inter-TRP XO drift.

### Use case 1: To assist PDSCH precoding in supplementary to PMI

Nevertheless, even with millisecond-level phase coherence, it may not be practical for UE to report PMI such frequently e.g. every 5 or 10 msec.

According to the mathematical derivation in the Appendix, the received signal at UE (after FFT) is

yk,l=n=1Np=1Pnβn,pdk,lexp-j2πkΔfRn,p0c+τn0expj2πvn,pc+enfclT, where

- n denotes TRP index, and TRPn has propagation paths p=1,…,Pn;

- en denotes the inter-TRP XO drift of TRPn (relative to a reference TRP e.g. TRP1);

- τn denotes the inter-TRP timing offset/misalignment of TRPn (relative to a reference TRP e.g. TRP1);

- Rn,p0c and vn,p0cfc are the per-path propagation delay and Doppler, respectively;

- βn,p is determined by the attenuation coefficient of TRPn’s path p (detailed in Appendix);

- k,l denote frequency-domain (subcarrier) and time-domain (symbol) index respectively;

- dk,l is the modulated data transmitted on subcarrier k and symbol l;

- Other parameters and derivation procedures in Appendix.

And it is observed that

  - TRPs’ channel propagation delay and timing offset: Rn,p0c+τn0 are lumped up together, and cause a linear phase rotation in frequency-domain;

  - TRPs’ channel Doppler and XO drift: vn,pc+enfc are lumped up together, and cause a linear phase rotation in time-domain.

It is a common sense that for sTRP, channel Doppler (i.e. UE velocity) is the main factor to channel variation over time – according to Rel-18 study on Type-II-Doppler and TDCP.

From the above analysis/derivation, it can be observed that it is the joint impact of the channel Doppler (vn,pcfc for each propagation path) and the XO drift (enfc for each TRP) that contributes to CJT mTRP channel’s variation.

Observation 3: For CJT mTRP channel, the joint impact of Doppler (UE velocity) and TRPs’ XO drift determines channel variation over time.

The difference between channel Doppler and TRPs’ XO drift is per-propagation-path v.s. per-TRP.

- Channel variation due to per-propagation-path Doppler requires PMI updates;

- It is simpler to handle “channel” variation due to per-TRP XO drift: Per-TRP (inter-TRP) phase offset updates can work.

For CJT, we generally consider low mobility UE, and thus UE velocity is not the main contributor to channel variation, compared to TRPs’ XO drift. In such case, PMI can be updated less frequently than inter-TRP phase offset.

Observation 4: For CJT mTRP channel with low UE velocity (compared with TRPs’ XO drift), PMI updates can be expected less frequent than inter-TRP phase offset updates.

For instance, for low mobility UE with small Doppler spread, PMI updates periodicity can typically be e.g. 40 msec. Then within each PMI report periodicity, we can have “cheaper” operation to handle per-TRP XO drift: Inter-TRP phase offset report e.g. every 5 or 10 msec, and the reported phase offset is applied to PDSCH precoding on top of PMI.

An example is depicted in Figure 8:

- Denote the precoder according to reported CJT-PMI (at time t0) as WTRP1⋮WTRPn⋮;

- Denote inter-TRP (e.g. TRPn-to-1) phase offset as ϕn=ϕTRPn-ϕTRP1;

- After UE measures and reports the inter-TRP phase offset at time t1: ϕnt1=ϕTRPnt1-ϕTRP1t1, network applies precoder for CJT-PDSCH transmission as WTRP1⋮WTRPnexp-jϕnt1-ϕnt0⋮.

More details are provided in Clause 3.2.

Figure 8. Inter-TRP phase offset updates within a periodicity of CJT-PMI report

Proposal 19: For UE-assisted CJT with non-ideal TRP synchronization, study use case 1: UE reports inter-TRP phase offset, to assist PDSCH precoding on top of PMI.

### Use case 2: To guarantee SRS reciprocity (for TDD)

For TDD system, reciprocity-based channel sounding is based on the assumption HDL=HULT, and specially for CJT mTRP (e.g. 2 TRPs), it is

HTRP1DL,HTRP2DL=HTRP1ULHTRP2ULT=HTRP1ULT,HTRP2ULT      (1)

However, for distributed TRPs with each individual clock sources, there can be misalignment between the measured UL channel (via SRS) and the DL channel for a potentially PDSCH transmission.

- The measured UL channel (taking into account Tx and Rx phase):

HTRP1ULHTRP2UL=ψTRP1RxHTRP1ULψTRP2RxHTRP2ULψUETx      (2)

- The DL “channel” (including Tx and Rx phase) for PDSCH:

HTRP1DL,HTRP2DL=ψUERxHTRP1DLψTRP1Tx,HTRP2DLψTRP2Tx      (3)

In the above equations, Rx misalignment ψRx and Tx misalignment ψTx (for either TRPs or UE) comprises two terms: Timing offset/misalignment (denoted as τRx,τTx) and phase offset (denoted as ϕRx,ϕTx).

- ψRxk=expj2πkΔfτRx+jϕRx at subcarrier k

- ψTxk=exp-j2πkΔfτTx-jϕTx at subcarrier k

It is noted that timing offset (τRx or τTx) causes a linear phase rotation in frequency-domain (over subcarriers).

To enable reciprocity-based channel sounding, it should satisfy

HTRP1DL,HTRP2DL=θ⋅HTRP1ULHTRP2ULT      (4),

where θ is a TRP-common constant, which is OK to be unknown.

⟹ψUERxHTRP1DLψTRP1Tx,HTRP2DLψTRP2Tx=θ⋅ψTRP1RxHTRP1ULψTRP2RxHTRP2ULTψUETx

⟹HTRP1DL,HTRP2DLψTRP2TxψTRP1Tx=θψUETxψUERxψTRP1RxψTRP1Tx⋅HTRP1ULT,ψTRP2RxψTRP1RxHTRP2ULT=θ'⋅HTRP1ULT,ψTRP2RxψTRP1RxHTRP2ULT

Given Equation (1), we should let ψTRP2TxψTRP1Tx=ψTRP2RxψTRP1Rx⟹ψTRP1RxψTRP1Tx=ψTRP2RxψTRP2Tx

⟹τTRP1Rx+τTRP1Tx=τTRP2Rx+τTRP2TxϕTRP1Rx+ϕTRP1Tx=ϕTRP2Rx+ϕTRP2Tx

In order to satisfy the above synchronized timing and phase between TRPs, inter-TRP Rx-Tx timing offset and phase offset should 0, e.g. for 2-TRP case, the following should be eliminated

- Inter-TRP Rx-Tx timing offset: τTRP2to1=τTRP2Rx+τTRP2Tx-τTRP1Rx+τTRP1Tx=0;

- Inter-TRP Rx-Tx phase offset: ϕTRP2to1=ϕTRP2Rx+ϕTRP2Tx-ϕTRP1Rx+ϕTRP1Tx=0.

Observation 5: For reciprocity/SRS-based channel sounding to be workable for CJT in TDD system, inter-TRP Rx-Tx timing offset/misalignment and phase offset should be eliminated.

Some intuitive explanations are illustrated with the following example in Figure 9.

Figure 9. CJT-TDD reciprocity issue with non-ideal TRP synchronization: Different PDPs seen via UL and DL

For simplicity, in this example, single propagation path is assumed for both of the two TRPs.

Also for simplicity, here for one TRP, a same clock is assumed for both its Rx and Tx. For instance, τ1 denotes TRP1’s timing relative to genie, for both Rx and Tx; Similarly for τ2 (TRP2).

Note that all denotations in “gray” are genie, and are not known with explicit values by any TRP or UE, including

- τ1 and τ2, for TRP1 and TRP2 timing (both Rx and Tx) respectively;

- τoF1 and τoF2 (time of flight), denoting TRP1-UE and TRP2-UE propagation delay, respectively.

For the PDPs seen by UL and DL channel respectively:

- Channel estimation via UL (SRS, by network) has PDP with TRP2-to-1 delay as: τNetworkRx,TRP2to1=tTRP2Rx-tTRP1Rx=τoF2-τoF1+τ1-τ2=τoF2-τoF1-τ2-τ1, where tTRP1Rx and tTRP2Rx are based on TRP1’s and TRP2’s clock respectively. (Since ADC samples are processed coherently between the two TRPs, at network side.)

  - In this example, τNetworkRx,TRP2to1>0.

- Channel seen via DL (a potential CJT-PDSCH received by UE) has PDP with TRP2-to-1 delay as: τUERx,TRP2to1=τoF2-τoF1+τ1-τ2=τoF2-τoF1+τ2-τ1, different than channel estimated via UL.

  - In this example, τUERx,TRP2to1<0, different than τNetworkRx,TRP2to1.

Observation 6: The SRS reciprocity issue for CJT in TDD system caused by inter-TRP timing offset/misalignment, is due to different PDPs seen via UL and DL.

If UE can measure and report inter-TRP Rx-Tx timing offset and phase offset, these offsets can be eliminated by network synchronization e.g. sync TRP2 to TRP1. Note that

- The reported timing offset does not include channel propagation delay;

- The reported phase offset does not include channel phase (different than the use case 1 in Subclause 3.1.2).

Proposal 20: For UE-assisted CJT with non-ideal TRP synchronization, for TDD system, study use case 2: UE reports inter-TRP Rx-Tx timing offset and phase offset, wherein

- The reported inter-TRP timing offset does not include channel propagation delay;

- The reported inter-TRP phase offset does not include channel phase.

## FDD-typical: UE-assisted precoding on top of PMI

As analysed in Subclause 3.1.2, inter-TRP phase offset can assist CJT-PDSCH precoding on top of PMI, and detailed steps are given as following (refer to Figure 8):

- Step1: At time t0, UE measures N CSI-RSs (transmitted by N TRPs) and reports CJT-PMI.

  - Based on the reported PMI (denote the precoder as WTRP1⋮WTRPn⋮), network can derive inter-TRP phase offset (by averaging across all ports of a pair of TRPs).

    - Denote, for example, TRPn-to-1 phase offset at t0 as ϕnt0=ϕTRPnt0-ϕTRP1t0.

- Step2: At time t1, UE measures and reports inter-TRP phase offset: ϕnt1=ϕTRPnt1-ϕTRP1t1.

- Step3: Network applies precoder for CJT-PDSCH at time t>t1 as WTRP1⋮WTRPnexp-jϕnt1-ϕnt0⋮.

  - This is due to channel variation from t0 to t1 (assuming UE velocity is small)

HTRP1t1,…,HTRPnt1,…≈HTRP1t0expjϕTRP1t1-ϕTRP1t0,…,HTRPnt0expjϕTRPnt1-ϕTRPnt0,…=expjϕTRP1t1-ϕTRP1t0HTRP1t0,…,HTRPnt0expjϕnt1-ϕnt0,…

  - This precoder is assumed constant until the next inter-TRP phase offset update (e.g. t2) or PMI updates.

The next question is, whether the inter-TRP phase offset change is “predictable”?

It may depend on several factors, for example, (1) how stable TRPs’ XO drift en is over time (for how long can we assume it constant); (2) The per-path Doppler (even if UE velocity is constant, the inter-TRP frequency offset caused by it can variate over time, due to multi-path under each TRP).

An example is illustrated in Figure 10, for inter-TRP frequency offset assisted PDSCH precoding on top of {PMI and inter-TRP phase offset}.

The steps would be similar as the above step1 to step3 for inter-TRP phase offset report, with difference:

- Step1: (PMI report at t0 – same as above.)

- Step2: At time t1, UE measures and reports inter-TRP frequency offset: fnt1=fTRPnt1-fTRPnt1.

  - Optionally, in addition, UE may also report inter-TRP phase offset: ϕnt1=ϕTRPnt1-ϕTRP1t1.

- Step3: Network applies precoder for CJT-PDSCH at time t>t1 as WTRP1⋮WTRPnexp-jϕnt1-ϕnt0-j2πfnt1⋅t-t1⋮

  - HTRP1t,…,HTRPnt,…≈HTRP1t1expj2πfTRP1t1⋅t-t1,…,HTRPnt1expj2πfTRPnt1⋅t-t1,…=expj2πfTRP1t1⋅t-t1HTRP1t1,…,HTRPnt0expj2πfnt1⋅t-t1,…

Figure 10. Inter-TRP frequency offset report

Proposal 21: For UE-assisted CJT with non-ideal TRP synchronization, study under use case 1: UE reports inter-TRP frequency offset, to assist PDSCH precoding on top of PMI.

Regarding how the TRP specific XO drift can be modelled, we may refer to channel model.

38.901 channel model (NLOS path as an example, and LOS path is similar regarding Doppler):

It is observed that UE velocity is assumed constant over time. Similarly, since XO drift causes a similar impact on measured “channel” variation over time, it may reuse model mechanisms for UE velocity. For instance, with a term like:

expj2πrrx,n,mT⋅vc+eTRPfclT, for ray m of reflection cluster n, under each certain TRP

Regardless, we are also open to hear input from network side.

Proposal 22: For UE-assisted CJT with non-ideal TRP synchronization, study XO drift model for FR1.

- A baseline model can be, a constant ppb-level XO drift value added up to Doppler of all paths from a same TRP.

It should be noted that CMR for frequency offset and for phase offset measurement is different. While inter-TRP phase offset measurement can rely on one-shot single-port CSI-RSs from all TRPs, inter-TRP phase offset measurement needs CSI-RS bursts over time. – Here we denote one burst as CSI-RS occasions from one TRP, over time.

Proposal 23: For UE-assisted CJT with non-ideal TRP synchronization, study measurement resource for inter-TRP frequency offset report.

- Candidate 1: Legacy TRS;

- Candidate 2: Rel-18 TDCP CMR;

- Candidate 3: Rel-18 Type-II-Doppler CMR (aperiodic CSI-RS burst, and/or legacy periodic CSI-RS);

- Candidate 4: New design not precluded.

UE-assisted inter-TRP phase (and/or frequency) offset on top of PMI can also work for both FDD and TDD system (more typical for FDD, although).

It is noted that this phase offset includes channel phase – which is different from use case 2, as further discussed in the next Clause 3.3.

## TDD: UE-assisted TRP synchronization for SRS reciprocity

Different than FDD case, SRS reciprocity is TDD-specific issue. As analysed in SubClause 3.1.3, reciprocity only holds if a TRP-common Rx-Tx timing offset and phase offset are guaranteed. – Note that this timing offset and phase offset do not include channel propagation delay and channel phase.

To get the inter-TRP Rx-Tx timing/phase offset without channel delay and phase, we can consider an approach illustrated in Figure 11, with the following procedure:

- Step1: UE transmits one SRS to all TRPs (two TRPs in this example for simplicity).

  - Received raw data z1=ψTRP1RxhTRP1ULψUETx for TRP1; z2=ψTRP2RxhTRP2ULψUETx for TRP2, where

    - For Rx-side (either TRP or UE), ψRxk=expj2πkΔfτRx+jϕRx at subcarrier k

    - For Tx-side (either TRP or UE), ψTxk=exp-j2πkΔfτTx-jϕTx at subcarrier k

  - It is noted that SRS sequence details are ignored here (e.g. treat it as all “1” sequence for simplicity).

- Step2: Each TRP transmits a corresponding single-port CSI-RS to the UE, where each CSI-RS is precoded based on the corresponding received SRS (z1* and z2*)

  - The precoding of CSI-RS ensures that the propagation delay and phase of the channel is canceled out, and doesn’t impact y1, y2 as received by UE:

    - y1=ψUERxhTRP1DLψTRP1Txz1*=hTRP12ψUERxψTRP1TxψTRP1Rx*ψUETx*;

    - y2=ψUERxhTRP2DLψTRP2Txz2*=hTRP22ψUERxψTRP2TxψTRP2Rx*ψUETx*.

  - It is noted that CSI-RS sequence details are ignored here (e.g. treat it as all “1” sequence for simplicity).

- Step3: UE calculates y1*y2=hTRP12hTRP22ψTRP1RxψTRP1Tx*ψTRP2Rx*ψTRP2Tx

  - This ensures that the phase of UE’s Rx-Tx uncertainty is cancelled out, and the only remaining phase is the inter-TRP timing offset and phase offset:

    - τTRP2to1=τTRP2Rx+τTRP2Tx-τTRP1Rx+τTRP1Tx;

    - ϕTRP2to1=ϕTRP2Rx+ϕTRP2Tx-ϕTRP1Rx+ϕTRP1Tx.

  - The inter-TRP timing offset and phase offset (between the two TRPs: TRP1 and TRP2) are estimated by observations y1*ky2k across multiple subcarriers k=0,1,… over the entire bandwidth.

- Step4: The UE-estimated τTRP1to2,ϕTRP1to2 are reported to one of the TRPs to synchronize to the other TRP.

Figure 11. UE-assisted TRP synchronization for SRS reciprocity for CJT-TDD

Proposal 24: For UE-assisted CJT with non-ideal TRP synchronization, for TDD system, study UE reporting inter-TRP timing offset and phase offset, based on CSI-RS precoded according to SRS receiving, to cancel out the impact of channel propagation delay and channel phase.

# Conclusion

In this contribution, we discuss CSI enhancements for up to 128 ports, and UE-assisted CJT with non-ideal synchronization. Based on the observation:

Observation 1: 64-port and 128-port type-I codebook in baseline mode already achieve significant gain over 32-port type-I codebook.

Observation 2: To guarantee phase-coherence for distributed TRPs within at least millisecond-level, ppb-level (or better) XO drift error should be guaranteed for the TRPs.

Observation 3: For CJT mTRP channel, the joint impact of Doppler (UE velocity) and TRPs’ XO drift determines channel variation over time.

Observation 4: For CJT mTRP channel with low UE velocity (compared with TRPs’ XO drift), PMI updates can be expected less frequent than inter-TRP phase offset updates.

Observation 5: For reciprocity/SRS-based channel sounding to be workable for CJT in TDD system, inter-TRP Rx-Tx timing offset/misalignment and phase offset should be eliminated.

Observation 6: The SRS reciprocity issue for CJT in TDD system caused by inter-TRP timing offset/misalignment, is due to different PDPs seen via UL and DL.

We propose:

Proposal 1: For up to 128-port CSI, no CRI is reported when CSI report is configured with a codebook of more than 32 ports.

Proposal 2: For up to 128-port CSI with non-CRI report, prioritize specifying 64 and 128 ports.

- FFS whether to support other values such as 48, 72, 96 ports.

Proposal 3: For up to 128-port CSI with non-CRI report, support 64-port codebook by K=2 of 32-port CSI-RS resources TDMed in a same slot.

Proposal 4: For up to 128-port CSI with non-CRI report, support 128-port codebook by K=4 of 32-port CSI-RS resources FDMed and TDMed in a same slot.

Proposal 5: For up to 128-port CSI with non-CRI report,

- Specify a baseline mode as reusing Rel-15 legacy Type-I codebook structure by simply extending to larger {N1, N2} without any codebook structure change.

- Further support an advanced mode for potential enhancements of 64-port and 128-port Type-I codebook with structure change. FFS enhancement details.

  - Any change of codebook structure, if specified, should be justified with performance gain over the baseline mode and reasonable impact to UE/gNB implementation complexity.

Proposal 6: For up to 128-port CSI with non-CRI report, for SD basis selection of Type-II codebook, study mechanisms for combinatorial coefficients table C(x,y) requiring less memory than x=63.

Proposal 7: For up to 128-port CSI with multi-CRI report, support RRC-configured the number of CRIs (denoted as M) in a CSI report config. Do not support dynamically change of M via DCI or MAC-CE.

Proposal 8: For up to 128-port CSI with multi-CRI report, specify procedure for CSI info bits packing and dropping for multi-CRI report. FFS details.

Proposal 9: For up to 128-port CSI with non-CRI report (objective a & b), the configured multiple CSI-RS resources have to be confined within a slot.

Proposal 10: For up to 128-port CSI with non-CRI report (objective a & b), support CMR configured as a set of K>1 (and K is up to 4) CSI-RS resources where each resource is with

- A same number of ports (up to 32);

- A same CDM type, and a same frequency density;

- A same number of RBs;

- A same starting RB with potential different RB offset (for FDM);

- A same QCL;

- A same EPRE offset (PDSCH-to-CSIRS and CSIRS-to-SSB).

Proposal 11: For up to 128-port CSI with multi-CRI report (objective c), support CMR configured as a set of K>1 (and K is up to 4) CSI-RS resources where each resource is with

- A same number of ports (up to 32);

- A same CDM type, and a same frequency density;

- A same number of RBs;

- A same starting RB with potential different RB offset (for FDM).

Proposal 12: For up to 128-port CSI with non-CRI report (objective a & b), support only a single IMR configured for a report with K>1 CMRs.

Proposal 13: For up to 128-port CSI with non-CRI report (objective a & b), linearly scale the CSI process time Z and Z’ by P/32, where P is the total # CSI-RS ports (across all CSI-RS resources) associated with the CSI report config.

Proposal 14: For multi-CRI report (i.e., objective c), extend CSI process time Z and Z’ as the following

- If Ks = 2:

  - Z= Q/16 * Z + (M-1) * Y, Z’= Q/16 * Z’ + (M-1) * Y,

- Else if 2 < Ks <= 8:

  - Z = Q/8 * Z + (M-1)  * Y, Z’ = Q/8 * Z’ + (M-1) * Y

where Q is the max number of CSI-RS ports allowed in a CSI-RS resource associated with the multi-CRI report, Ks is the number of CSI-RS resources configured for the report, M is the number of CRIs configured to be included in the report, Y is a number of OFDM symbols.

- FFS the value of Y.

Proposal 15: For up to 128-port CSI, RAN1 start the work to specify the procedure for CPU counting, active CSI-RS resource counting, and active CSI-RS ports counting no later than RAN1 #117 (the 3rd meeting of Rel-19).

Proposal 16: For up to 128-port CSI, define mechanism in active resource and ports capability report to differentiate supporting multiple non-CRI reports, a single non-CRI report, and a single multi-CRI report, which are configured with the same number of CSI-RS resources and the same number of total ports.

Proposal 17: For up to 128-port CSI, consider the network energy saving (NES) aspects.

Proposal 18: For UE-assisted CJT with non-ideal TRP synchronization, focus Rel-19 study on the scenario of ppb-level inter-TRP XO drift.

Proposal 19: For UE-assisted CJT with non-ideal TRP synchronization, study use case 1: UE reports inter-TRP phase offset, to assist PDSCH precoding on top of PMI.

Proposal 20: For UE-assisted CJT with non-ideal TRP synchronization, for TDD system, study use case 2: UE reports inter-TRP Rx-Tx timing offset and phase offset, wherein

- The reported inter-TRP timing offset does not include channel propagation delay;

- The reported inter-TRP phase offset does not include channel phase.

Proposal 21: For UE-assisted CJT with non-ideal TRP synchronization, study under use case 1: UE reports inter-TRP frequency offset, to assist PDSCH precoding on top of PMI.

Proposal 22: For UE-assisted CJT with non-ideal TRP synchronization, study XO drift model for FR1.

- A baseline model can be, a constant ppb-level XO drift value added up to Doppler of all paths from a same TRP.

Proposal 23: For UE-assisted CJT with non-ideal TRP synchronization, study measurement resource for inter-TRP frequency offset report.

- Candidate 1: Legacy TRS;

- Candidate 2: Rel-18 TDCP CMR;

- Candidate 3: Rel-18 Type-II-Doppler CMR (aperiodic CSI-RS burst, and/or legacy periodic CSI-RS);

- Candidate 4: New design not precluded.

Proposal 24: For UE-assisted CJT with non-ideal TRP synchronization, for TDD system, study UE reporting inter-TRP timing offset and phase offset, based on CSI-RS precoded according to SRS receiving, to cancel out the impact of channel propagation delay and channel phase.

# References

- RP-234007, New WID: NR MIMO Phase 5, RAN#102, Dec.11th-15th, 2023

- 3GPP TS 38.104

# Appendix: Mathematical derivation of UE-received CJT transmission with TRP individual clock sources

For OFDM signal transmitted by N TRPs, n=1, …, N

- Tx baseband by TRP1:

x1t=l=0L-1k=0K-1dk,lexpj2πkΔft-lTrectt-lT+TCPT

Where symbol length T=TOFDM+TCP, and SCS Δf=1TOFDM.

- k is subcarrier index (frequency-domain);

- l is OFDM-symbol index (time-domain);

- dk,l is determined by the modulated data to convey;

- rect( ) function denotes a rectangle window with unit width in [0,1].

The above time or frequency parameters (T,TOFDM,TCP,Δf) are all based on TRP#1’s timing and oscillator (XO) – if for TRPn, denote the inter-TRP (TRPn-to-1) XO drift as en, e.g. en=0.1 ppm (±0.05 ppm) as a worst case according to a basic RAN4 requirements on Macro-site BS [2], we have for TRPn:

xnt=l=0L-1k=0K-1dk,lexpj2π1+enkΔft-τn0-lT1+enrectt-τn0-lT-TCP/1+enT/1+en

- where τn0 is the TRPn-to-1 timing offset at TRP1’s time 0; And τ10=0.

- Tx RF by TRP1:

x1RFt=x1texpj2πfct=l=0L-1k=0K-1dk,lexpj2πfc+kΔftexp-j2πkΔflT

Tx RF by TRPn:

xnRFt=xntexpj2π1+enfct-τn0+jϕnTx0=l=0L-1k=0K-1dk,lexpj2π1+enfc+kΔft-τn0exp-j2πkΔflTexpjϕnTx0

- where ϕnTx0 denotes the TRP#n-to-#1 Tx phase difference at TRP1’s time 0; And ϕ1Tx0=0.

It is noted that rect( ) window is neglected in the above equations for simplicity.

- Rx RF (sent over mTRP channel, each with multi-path propagation):

  - Each TRP’s multi-path propagation delay τn,ppatht=Rn,ptc, where Rn,pt=Rn,p0-vn,pt, by assuming delay is constant within a symbol (varying over symbols), while velocity is constant within a duration of L symbols.

    - vn,p=vUEcosθn,p, where θn,p is AoA for path p of TRPn.

yRFt=n=1Np=1Pnαn,pxnRFt-τn,ppatht=n=1Nαn,pl=0L-1k=0K-1dk,lexpj2π1+enfc+kΔft-Rn,ptc-τn0exp-j2πkΔflTexpjϕnTx0=n=1Np=1Pnαn,pl=0L-1k=0K-1dk,lexp-j2πkΔflTexpj2π1+enfc+kΔft-Rn,p0c+vn,pct-τn0expjϕnTx0=n=1Np=1Pnαn,pexp-j2π1+enfcRn,p0c+τn0expjϕnTx0l=0L-1k=0K-1dk,lexp-j2πkΔflTexpj2π1+enfc+kΔf1+vn,pctexp-j2π1+enkΔfRn,p0c+τn0=n=1Np=1Pnβn,pl=0L-1k=0K-1dk,lexp-j2πkΔflTexpj2π1+vn,pc+en+vn,pcenfc+kΔftexp-j2π1+enkΔfRn,p0c+τn0≈n=1Np=1Pnβn,pl=0L-1k=0K-1dk,lexpj2πfc+kΔftexp-j2πkΔflTexp-j2π1+enkΔfRn,p0c+τn0expj2πvn,pc+enfc+kΔft

  - where αn,p is the attenuation coefficient of path p of TRPn, and here we denote βn,p:=αn,pexp-j2π1+enfcRn,p0c+τn0expjϕnTx0 for simplicity.

It is noted that vn,pcen is neglected due to that it is small compared to either vn,pc or en.

- Rx baseband – down-converted with exp-j2πfct

  - Here for simplicity, ignore the timing and frequency offset between TRP1 and UE (i.e. assuming perfect DL tracking), since this does not impact inter-TRP timing/frequency offset.

yt=n=1Np=1Pnβn,Pl=0L-1k=0K-1dk,lexpj2πkΔftexp-j2πkΔflTexp-j2π1+enkΔfRn,p0c+τn0expj2πvn,pc+enfc+kΔft

- After ADC sampling (t→lT+k'Ts) – index k' corresponds time-domain signal

yk',l=n=1Np=1Pnβn,pk=0K-1dk,lexpj2πkΔflT+k'Tsexp-j2πkΔflTexp-j2π1+enkΔfRn,p0c+τn0expj2πvn,pc+enfc+kΔflT+k'Ts=n=1Np=1Pnβn,pk=0K-1dk,lexpj2πkΔfk'Tsexp-j2πkΔf1+enRn,p0c+τn0expj2πvn,pc+enfc+kΔflT+k'Ts,k'=0,1,…,K-1

It is noted that

  - Here symbol index l is denoted on yk',l, thus “l=0L-1” is removed;

  - The term “expj2πkΔfk'Ts” is basically IFFT, and will disappear after FFT at Rx side.

yk',l=n=1Np=1Pnβn,pk=0K-1dk,lexpj2πkΔfk'Tsexp-j2πkΔf1+enRn,p0c+τn0expj2πvn,pc+enfc+kΔfk'Tsexpj2πvn,pc+enfc+kΔflT≈n=1Np=1Pnβn,pk=0K-1dk,lexpj2πkΔfk'Tsexp-j2πkΔfRn,p0c+τn0expj2πvn,pc+enfclT

where we have some terms neglected:

  - The term “expj2πvn,pc+enfc+kΔfk'Ts” is associated with phase shift within an OFDM symbol (over samples k'Ts), which also causes ICI.

    - This impairment can be ignored if vn,pc+enfc≪Δf – from ICI perspective, Doppler shift should anyway be guaranteed much smaller than SCS (already been handled by Rel-15 waveform/numerology design).

  - The term “expj2πvn,pc+enkΔflT” is Doppler scaling effect (over bandwidth KΔf) – or interpreted from a different perspective, it is timing/delay changing over time LT.

    - This impairment can be ignored if bandwidth KΔf≪fc (carrier frequency) – generally true.

- After FFT (k'→k, frequency-domain signal)

yk,l=n=1Np=1Pnβn,pdk,lexp-j2πkΔfRn,p0c+τn0expj2πvn,pc+enfclT

It is observed that:

  - TRPs’ channel propagation delay and timing offset: Rn,p0c+τn0 are lumped up together, and cause a linear phase rotation in frequency-domain;

  - TRPs’ channel Doppler and XO drift: vn,pc+enfc are lumped up together, and cause a linear phase rotation in time-domain.