3GPP TSG RAN WG1 #116			R1-2401454

Athens, Greece, February 26th – March 1st, 2024

Agenda item:	9.6.3

Source: 	Qualcomm Incorporated

Title: 	LP-WUR Operation in Connected Mode

Document for:	Discussion and Decision

# Introduction

In [1], a new work item (WI) is approved for the low-power (LP) wakeup signal (WUS) and wakeup receiver (WUR) design for NR. Objectives of this WI are as follows. The RAN1 related scope includes three main topics:

- LP-WUS and LP-SS design

- LP-WUR operation in UE idle and inactive modes

- LP-WUR operation in UE connected mode.

In this paper, we discuss the design aspects for the LP-WUR operation in UE connected mode. The general LP-WUS/LP-SS design and idle/inactive mode procedure are discussed in our companion papers in [2] and [3].

4.1 Objective of SI or Core part WI or Testing part WI The objectives of the work item are the following: - To specify an LP-WUS design commonly applicable to both IDLE/INACTIVE and CONNECTED modes (RAN1, RAN4) - Specify OOK (OOK-1 and/or OOK-4) based LP-WUS with overlaid OFDM sequence(s) over OOK symbol - The LP-WUS design shall ensure that for IDLE/INACTIVE operation, the same information is delivered irrespective of LP-WUR type. The OFDM sequence can carry information. - At least duty-cycled monitoring of LP-WUS is supported - For IDLE/INACTIVE modes - Specify procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring (RAN2, RAN1, RAN3, RAN4) - Specify LP-SS with periodicity with Yms for LP-WUR, for synchronization and/or RRM for serving cell. (RAN1, RAN4) - LP-SS is based on OOK-1 and/or OOK-4 waveform with or without overlaid OFDM sequences. Further down selection between with and without overlaid OFDM sequences is to be done within WI. - Note: For LP-WUR that can receive existing PSS/SSS, existing PSS/SSS can be used for synchronization and RRM instead of LP-SS. - Y will be decided within WI. 320ms is the start point. - Specify further RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR, including the necessary conditions (RAN4, RAN2) - For CONNECTED mode, specify procedures to allow UE MR PDCCH monitoring triggered by LP-WUS including activation and deactivation procedure of LP-WUS monitoring (RAN2, RAN1) - Check in RAN#105 for potential TU adjustment in RAN2 - Note: In CONNECTED mode, UE MR ultra-deep sleep is not considered, and UE RRM/RLM/BFD/CSI measurements are performed by MR - Note: The target coverage of LP-WUS and LP-SS shall be the coverage of PUSCH for message3. - Note: The optimization of LP-WUS signal design for idle/inactive mode is prioritized over the optimization for connected mode. - Specify the necessary RAN4 core requirement(s) to support the feature (RAN4). - This objective is to be further refined in RAN#103 4.2 Objective of Performance part WI NOTE: Leave empty if the WI proposal does not contain a RAN performance part. - Specify the demodulation performance and test cases for LP-WUS/WUR[RAN4]. - Specify RRM measurement performance requirement and corresponding test cases for the LP-WUS/WUR[RAN4].

# Connected mode procedure

For connected mode, we consider the case in that CDRX is configured and LP-WUS is used to replace the Downlink Control Information with CRC scrambled by PS-RNTI (DCP) before the start of DRX OnDuration. The DRX cycles are either matched to periodicities of the underlying traffic if the traffic is periodic or transmissions of the traffic are aligned with DRX cycles. For the latter, a large DRX cycle is more preferrable from power saving perspective as long as the delay requirement of the traffic is satisfied. On top of the basic DCP function, the LP-WUS can also indicate the uncertain data arrival time, known as jitter, for traffic that has tight delay deadline. For this use case, the LP-WUS determines the start of PDCCH monitoring within the DRX OnDuration for the indicated UE group. When jitter is present in the traffic, the DRX OnDuration may be configured wider to cover the jitter range.

Proposal 1: Support using LP-WUS to replace DCP and to indicate the jitter delay when CDRX is configured.

When CDRX is configured, the LP-WUS is continuously monitored in a window starting before the OnDuration start. For the basic use case of LP-WUS replacing DCP, UE starts to monitor the PDCCH from the OnDuration start when the LP-WUS indicates the UE’s associated UE group (see Figure 1). A delay between the LP-WUS and the OnDuration start is configured to account for the MR wakeup time after LP-WUS detection. For the jitter delay indication use case, network starts to send the LP-WUS immediately after the traffic data becomes available for transmission. UE PDCCH monitoring starts after the first detected LP-WUS and is further delayed by the offset (see Figure 2). Then, the first PDCCH monitoring occasion for the UE to monitor may fall in the OnDuration. For both use cases, network may transmit LP-WUS in multiple occasions in the LP-WUS window and UE can stop detecting subsequential LP-WUS after the first detected LP-WUS.

Proposal 2: LP-WUS is monitored in continuous occasions within the LP-WUS window. PDCCH monitoring is triggered in one of the following two modes based on network configuration:

- Mode 1: PDCCH monitoring starts from the start of the DRX OnDuration

- Mode 2: PDCCH monitoring starts after the first detected LP-WUS by an offset.

Figure 1: LP-WUS is used to replace the DCP

Figure 2: LP-WUS indicates jitter delay

# Conclusions

In this contribution, we have provided the following observations and proposals:

Proposal 1: Support using LP-WUS to replace DCP and to indicate the jitter delay when CDRX is configured.

Proposal 2: LP-WUS is monitored in continuous occasions within the LP-WUS window. PDCCH monitoring is triggered in one of the following two modes based on network configuration:

- Mode 1: PDCCH monitoring starts from the start of the DRX OnDuration

- Mode 2: PDCCH monitoring starts after the first detected LP-WUS by an offset.

# References

- RP-234056, New WID on low-power Wake-up Signal and Receiver for NR

- R1-2401452, LP-WUS and LP-SS design, Qualcomm, February 2024

- R1-2401453, LP-WUR operation in idle and inactive modes, Qualcomm, February 2024