3GPP TSG RAN WG1 #116								        R1-2401455

Athens, Greece, February 26th – March 1st, 2024

Agenda item:	9.7.1

Source: 	Qualcomm Incorporated

Title: 	ISAC Deployment Scenarios

Document for:		Discussion

- Introduction

A RAN1-led study item has been agreed in RAN#102 for channel modeling for sensing, with the following objectives [1]

The focus of the study is to define channel modelling aspects to support object detection and/or tracking (as per the SA1 meaning in TS 22.137). The study should aim at a common modelling framework capable of detecting and/or tracking the following example objects and to enable them to be distinguished from unintended objects: - UAVs - Humans indoors and outdoors - Automotive vehicles (at least outdoors) - Automated guided vehicles (e.g. in indoor factories) - Objects creating hazards on roads/railways, with a minimum size dependent on frequency All six sensing modes should be considered (i.e. TRP-TRP bistatic, TRP monostatic, TRP-UE bistatic, UE-TRP bistatic, UE-UE bistatic, UE monostatic). Frequencies from 0.5 to 52.6 GHz are the primary focus, with the assumption that the modelling approach should scale to 100 GHz. (If significant problems are identified with scaling above 52.6 GHz, the range above 52.6 GHz can be deprioritized.) For the above use cases, sensing modes and frequencies: - Identify details of the deployment scenarios corresponding to the above use cases. - Define channel modelling details for sensing using 38.901 as a starting point, and taking into account relevant measurements, including: - modelling of sensing targets and background environment, including, for example (if needed by the above use cases), radar cross-section (RCS), mobility and clutter/scattering patterns; - spatial consistency. It will be discussed at RAN#105 whether to include additional study beyond channel modelling for ISAC.

The expected output is the modifications to TR38.901 for sensing aspects of channel modeling, to be completed in RAN#108 (June 2025), after 8.5 TUs of RAN1 work spanning 9 RAN1 meetings from RAN1#116 (current meeting) through RAN1#121 (May 2025). In this contribution we provide our approach on deployment scenarios for ISAC.

- ISAC deployment scenarios

  - Interaction/decoupling of deployment scenarios and channel model construction

In a companion contribution [2], we discuss in detail our approach towards ISAC channel modeling. A key idea from that discussion is to separate the modeling into two components: Modeling of the objects of interest for sensing – both target and non-target (e.g. background, clutter, spurious target, etc), and construction of the new channel model using those objects. We note that the same objects (e.g., humans) could be dropped into multiple scenarios (e.g., Indoor Hotspot, UMi, etc). We also note that there is already a rich family of scenarios developed across the TRs 38.901, 36.777 (UAV) and 37.885 (V2X), which can directly be reused for the sensing study. Thus, there is no compelling need to define new scenarios from scratch for ISAC. What is needed is to define the nature of the objects that are to be dropped into these scenarios, so that we can then create the new channel model that includes interactions with these objects.

The channel model construction is discussed in detail in [2], using an abstract modeling representation of the object in terms of one or more ‘scatter-points’, each assigned a location, velocity, orientation (LCS), and an LCS input and output angle dependent gain function G(φin,θin, φout, θout) which represents the gain that the object applies to a ray arriving at it along (azimuth, zenith) direction (φin,θin) and leaving it with (azimuth, elevation) angle (φout,θout). Large scale properties such as pathloss and LOS/NLOS-state for the links from object to other nodes (Tx, Rx or other objects) can be created exactly as in legacy TR38.901, as if the object was another Tx/Rx node placed in the corresponding scenario. Some extensions may be needed of the legacy TR38.901 in this connection: For example, the UMi scenario does not allow placing a UE closer than a certain minimum distance to the gNB, but we may still want to place an object very near to a UE or a gNB. So if we try to reuse the gNB-UE pathloss formula for a link from gNB to a very nearby object, or from UE to a very nearby object, we have to ensure that the formula is still applicable, or generalize the existing formula. As another example of a required extension, the UAV scenarios (TR36.777) are only defined for FR1 and need to be extended to FR2. However, on the whole, a large amount of reuse is possible here, of the equations and ideas from TR38.901. The details of the channel construction are provided in [2]. The conclusion from this is that the main additional effort needed to define the deployment scenarios is to define the gain function G() for various families of objects of interest, along with the procedures and relevant distributions for dropping the objects into the existing scenarios to model the relevant use-cases.

The key proposals along these lines in [2] are repeated here below for completeness and ease of reference in this contribution (although we do not also repeat them in the Conclusion (Section 3) of this contribution). In the rest of the contribution, we will then focus on various specific examples of gain functions.

Proposal 7[2]: Reuse existing scenarios from TRs 39.901, 37.885 and 36.777, and define modeling of physical objects (both sensing target(s) and non-target(s)) dropped into these scenarios. New scenarios may be defined only if specific use-cases are identified that cannot be covered by such reuse. Prioritize modeling of scatterers in existing scenarios over the design of new scenarios.

Proposal 8[2]: Define families of object types, and allow reuse of object types across multiple scenarios.

Proposal 9[2]: Define scenarios for NR UAV study in TR38.901 that leverage TR36.777 for FR1 and extend it to FR2, and apply Proposal 7 to (re)use these scenarios instead of TR36.777 for sensing study.

Proposal 10[2]: Define necessary extensions to TR37.885 scenarios for sensing in TR38.901, and use the resulting scenarios for sensing channel modeling described in TR38.901.

Proposal 11[2]: Model a small object as a single scatter-point applying a gain value G(φin,θin, φout, θout) to a ray that arrives at the scatter-point with (azimuth, elevation) angles (φin,θin) and leaves it with (azimuth, elevation) angles (φout,θout), wherein all the 4 angles are with respect to an LCS attached to the scatter-point.  The gain function G(.) allows an abstract modeling of a combination of reflection, refraction, and attenuation. LCS to GCS translation is part of ‘object drop procedure’, analogous to UE orientation being part of UE drop in TR38.901.

  - Single scatter-point definitions.

Objects that impact/interact with the over-the-air propagation will never be isolated points in reality. However, for small enough objects, at least for some studies it suffices to model them as a single scatter-point. This makes the modeling simpler, as detailed in [2], and easier to perform calibration of results across companies, while also still providing useful information about sensing performance. The assumptions and corresponding limitations of this modeling are:

- Object shape and micro motions between relative parts of the object are ignored. Thus, the approach is enough for studies related to detecting presence/location/velocity of the object, but not for detecting more complicated object properties such as shape and micro-doppler. However, even those can be studied by modelling the object as multiple scatter-points with (in general) different positions, velocities, orientations and gain functions.

- The object should be small enough and/or far enough away from the source (Tx) and destination (Rx) so that the plane-wave assumption is reasonable for a wave from the Tx to the object and from the object to the Rx. Pathloss changes due to difference in distance from the Tx/Rx to the object surface vs. the location of the single point representation of the object should be negligible.

Proposal 1: Consider cases where only a single scatter-point is dropped into the scenario. The drop may be random or may be at a specific location in the scenario layout, relative to other nodes (e.g., relative to gNB(s) and/or UE(s)).

    - Specular mirror/reflector scatter-point

Mirror reflection is a generalization of TR38.901 Section 7.6.8 on ground-reflection. As observed in [2], the ground is a large object introduced in the scene, but for a given pair of Tx and Rx node positions, only one point on the ground really matters, and the location of this point can be computed in a straightforward manner by geometry and Snell’s law (angles of incidence and reflection should be equal). The reflection is then completely characterized by that location and the direction of the normal to the surface (ground) at that location. The same concept can be used for any flat surface large or small (e.g., a wall instead of ground). The gain function is given by

G(φin,θin, φout, θout) = Gp(θin) * δ(θout - θin) * δ(φout - φin -180o),

where Gp(θin) is the scalar ‘mirror gain’ for each of the two polarizations ‘p’= parallel and perpendicular. Here δ() is the ‘indicator’ or Kronecker delta function (taking value of 1 when its argument is zero, and zero otherwise), which ensures that the gain is nonzero only along the output direction that corresponds to the input direction via the Snell’s law. Gp(θin) depends on the refractive index of the mirror material, via formulas analogous to 7.6-38 and 7.6-39 in TR38.901. Note that this describes an ideal specular two-sided mirror that completely absorbs all refractions into it. In the case of ground-reflection, the absorption may be a reasonable assumption since the ground is a large object. For any other practical mirror such as a wall or a vehicle, this is likely not the case, and there will be some attenuated ‘pass-through’ and possibly also some diffraction around the object. All this is idealized and ignored in the simplified definition above. A one-sided mirror which is opaque and non-reflective on the other side can be modeled by setting Gp(θin)=0 for all θin corresponding to arrivals at the opaque surface, i.e., θin ≥ 90o. This is illustrated in Figure 1.

Figure 1: Specular reflector scatter-point

Also note that even though this mirror is ‘perfectly specular’, it can still be used to model some amount of diffuse scattering, based on the angular spreads in the incoming and reflected clusters the mirror is influencing, as also detailed in [2]. Each incoming ray in the cluster has a slightly different angle of arrival, and thus a slightly different departure angle, but all those departing rays may still be coupled back into a cluster at the receiver. This is somewhat analogous to existing TR38.901 handling of the first cluster in LoS channel – in addition to adding the LoS specular ray, the first cluster AoA and AoD are also modified to align with the LoS direction, but each individual ray within that cluster has its own AoA and AoD.

    - Specular omnidirectional retro-reflector scatter-point

For monostatic sensing, the Tx and Rx are very close to each other. However, the ray from the Tx may be reflected into another direction and may never make it back to the collocated Rx. A retro-reflector is a special reflector that sends the reflection back in the direction of the incoming EM wave. Practically it may be quite difficult to ensure this behavior for each and every incoming direction (both azimuth and zenith). However, as an idealized construction, we can imagine a reflector behaving this way. This is illustrated in Figure 2. It could be useful to validate results from monostatic sensing, by providing benign scenarios in which multiple monostatic sensors placed around the object can all see a strong enough reflection from the object. The gain function is given by

G(φin,θin, φout, θout) = Gp(φin,θin) * δ(θout - θin) * δ(φout - φin)

Figure 2: Specular omnidirectional retro-reflector scatter-point behaviour for different incoming ray angles

The ‘monostatic gain’ Gp(φin,θin) can be used to control the gain as a function of Tx/Rx unit position and polarization p (parallel and perpendicular). In particular it can be set to constant=1 to make the gain independent of Tx/Rx position and polarization. It can also be set to 1 for certain incoming angles (φin,θin) and zero for other angles, to represent practical retro-reflectors that have reflective as well as opaque non-reflective surfaces, and only exhibit retro-reflector behavior on a limited set of incoming ray angles. This is somewhat analogous to the modeling of one-sided mirrors instead of 2-sided mirrors, as described in Section 2.2.1.

    - Pass-through attenuator scatter-point

Figure 3: Pass-through attenuator

This scatter-point is illustrated in Figure 3, with gain function given by

G(φin,θin, φout, θout) = Gp(φin,θin) * δ(θout – (180o – θin)) * δ(φout - φin -180o)

The existing TR38.901 Section 7.6.4 blockage model-B can in some sense be viewed to be a special case of this, where a detailed procedure is given to construct Gp(φin,θin) based on a model of the blocker as a rectangular screen with knife-edge diffraction around all the 4 edges. The screen adds an angle-dependent attenuation to all rays passing through it, but it does not change the ray angles (the rays just ‘pass through’ as shown in Figure 3). Thus, the physical dimensions of the rectangular screen object are abstracted away into the gain function Gp(φin,θin) and then the object can be represented as a single scatter-point, for all the rays that interact with the scatter-point. The blockage model-B further defines the geometric procedure to determine which rays interact with the scatter-point, based on the dimensions and location of the screen-blocker.

    - Specular Refractor/prism scatter-point

Figure 4: Refractor/prism scatter-point

This scatter-point is illustrated in Figure 4, and can be represented by

G(φin,θin, φout, θout) = Gp(φin,θin) * δ(θout – fθ(φin,θin)) * δ(φout - fφ(φin,θin))

The functions fθ and fφ abstract away the ‘bending behavior’ of the prism, capturing the emerging angle of the ray directed towards the prism at a given incident angle. These have to be computed by geometry and Snell’s law applied at all the surfaces at which refraction occurs. For a convex polyhedral prism, there will be only 2 such surfaces surfaces. With non-convex polyhedra the ray could emerge from the prism and then re-enter it, and further this could happen multiple times. It is still in principle possible to repeatedly apply Snell’s law at each surface of interaction and determine how the ray finally emerges from the object, and thus determine the functions fθ and fφ and also the corresponding gain function Gp. The implicit assumption here is that each incident angle only corresponds to one unique emergent angle. Note that the above gain function can also be seen as generalizing the previously described gain-functions of Sections 2.2.1, 2.2.2 and 2.2.3 – those gain-functions have specific choices of fθ and fφ (e.g., fθ(φin,θin) = 180o – θin and fφ(φin,θin) = φin +180o for Section 2.2.3, and fθ(φin,θin) = θin and fφ(φin,θin) = φin for Section 2.2.2)

    - Single-input multiple-output specular scatter-point

A scatter-point can simultaneously have more than one of the behaviors described thus far. For example, both reflection and refraction may happen at the surfaces of a prism. Figure 4 shows only the refractions, but we could similarly also consider reflection at the first incident surface using the one-sided mirror model of Section 2.2.1.  The resulting gain function can be expressed as a summation of the individual gain-functions. We could further also consider reflection at the second surface, which would then lead to the ray impinging on the internal face of a third surface, and the process can be continued until the ray emerges. For simplicity, this process could be limited to a finite number of surface interactions; this may be justified by assuming that the absorption loss would grow very large after multiple interactions and the ray may be too weak to require explicit modeling after that.

    - Diffuse scattering versions of the specular scatter-points

The gain functions defined thus far (in Sections 2.2.1-5) all involve the use of Kronecker delta functions δ() to enforce the property that each incoming ray angle corresponds to only one unique outgoing ray angle, or in case of Section 2.2.5, a finite set of outgoing angles (captured by a summation of terms involving different δ() functions). We could replace all these δ() functions with other ‘peaky’ pulse-shape functions p(x) that are ‘smoother’ than the Kronecker delta – they would still have peak at x=0, but would not be all zero for non-zero x, but instead generally decrease with increasing x. The decrease may be monotonic (e.g., with a gaussian pulseshape) or also locally non-monotonic (e.g., with a sinc function pulseshape). The width of the main peak at x=0 is a parameter that can be used to tune the reflector behavior to more specular (narrower width) or more diffuse (broader width). Each of the earlier gain functions further has a product of two Kronecker delta functions, one representing the outgoing azimuth and the other representing the outgoing elevation, as a function of the incoming azimuth and elevation angles. The pulse-shapes used for these two delta functions may be the same or different. The pulse shapes should be normalized so that the total energy emerging from the scatter point over all emergent angles equals the difference between the energy impinging the scatter point and the absorption loss.

  - Objects modelled using more than one scatter-point

    - Small convex polyhedral reflector

For a small convex polyhedron with all faces as specular mirrors, each face can be treated as a 1-sided mirror, represented by a single scatter-point as described in Section 2.2.1, where the opaque/non-reflecting surface of the 1-sided mirror is the inner one, and the reflecting surface is the outer one. Each of these scatter-points could be placed at the centroid of the corresponding face. This creates an object represented by F scatter-points, where F is the number of faces. For further simplicity, we could also instead place all those scatter-points at the centroid of the whole 3-dimensional object instead. In this case, the model can be thought of as a single scatter-point with a gain-function that sums the gain-functions of the individual one-sided mirror scatter-points. However each of these individual gain-functions is described in Section 2.2.1 relative to its own LCS, defined using the normal to the surface. Thus, it is still fully equivalent to thinking of this as F separate scatter-points that just happen to be collocated. The two approaches are illustrated in Figure 5

Figure 5: Polyhedral reflector as a collection of scatter-points

The convexity assumption ensures that a reflection from one face can never be incident on another face. The fact that the object is a polyhedron (3D) rather than a flat 2D surface as in Section 2.2.1, and is totally reflective (no absorption/refraction)  ensures that no ray can ever hit the non-reflective ‘inner’ surface of the constituent one-sided mirrors – since any incident ray is completely reflected and cannot penetrate into the object material.

The gain functions at the individual surfaces may be the same or different, and each could use Kronecker delta functions (giving a perfect specular reflection) or a smoother pulseshape giving a more diffuse reflection as described in Section 2.2.6.

If the polyhedron is resting on the ground, then the face resting on the ground cannot have incident rays if we assume (as is typically the case) that all the Tx and Rx nodes are above the ground. This should be taken care of in the procedure for determining the incoming and outgoing cluster angles at each scatter-point: The scatter-point representing the face resting on the ground should not have any incoming or outgoing clusters. This can be made part of the procedure for selecting the angles of the clusters that arrive at each scatter-point. Note that for the LoS specular ray, it will automatically follow from geometry: Rays arriving from above the ground will hit the non-reflecting side of the one-sided mirror representation of the face resting on the ground.

    - Large flat reflector surface

A large flat specular reflector is a generalization of ground reflection and has also been discussed in [2]: Even though the surface is very large, the reflection only happens at a specific point on the surface, which can be determined using geometry from the locations of the Tx, Rx and the surface. So, for any one Tx, Rx pair, it can be modeled just as a single point, as in Sec. 2.2.1, although the location, orientation, and gain function of this point will in general be different for different Tx/Rx pairs. Note that the surface does not have to be rectangular shaped for this, it can be any polygon or even a surface with curved and potentially non-convex boundary. The reflector scatter-point location can be determined assuming it is an infinite plane, and then it needs to be considered only if that location actually lies on the finite arbitrary-shaped surface.

This approach avoids having to discretize the large surface into a fine grid of individual scatter-points, and then model the interactions with all of these scatter points. Only some interactions will be relevant, and the geometry of reflection allows identifying and modeling only those scatter-points.

If the reflector is diffuse rather than specular (as in Section 2.2.6), then in general there will be a continuum of relevant scatter-points, which then forces us towards the approach based on the grid of scatter-points. To control the complexity, we may again define the ‘most relevant scatter point’ based on the peak/dominant reflection angle corresponding to the peak of the pulse-shape function defined in Section 2.2.6, and then define the grid in a small area centered around this ‘most relevant scatter point’. The dimensions of this area can depend on the width of the pulse-shape functions for both azimuth and elevation.

  - Potential mapping between physical objects and proposed simplified models

Table 1 below shows some potential applications of the simplified object models described in this contribution.

| Scenario | Object | Model |
| --- | --- | --- |
| InH/UMi/UMa | Human | Diffuse reflector polyhedron (e.g., parallelopiped) or scatter-point |
| UAV | UAV | Specular reflector scatter-point |
| InF | AGV | Specular reflector scatter-point or polyhedron depending on distance to relevant Tx/Rx nodes. |
| Highway | Vehicle | Specular reflector polyhedron |
| Uma | Trees/clutter | Diffuse reflector polyhedron or scatter-point depending on distance to relevant Tx/Rx nodes |
| Monostatic sensing in various scenarios, object ‘small’ relative to distances to Tx/Rx | Various (human, AGV, UAV, vehicle) | Omnidirectional retro-reflector |

Table 1: Potential mapping between physical objects and simplified object models.

Note that although we have described the forms of many gain functions, we have not yet completely specified them in the manner eventually needed for capture in TR38.901. For example, we have not explicitly specified the function Gp(φin,θin) for each of the polarizations ‘p’ that appears in many of the gain function equations. This needs to be worked out in more detail in upcoming meetings. As also noted in [2], the gain function in general is a 2x2 matrix including cross-polarization terms, in addition to the two diagonal terms represented by Gp(φin,θin) for the two polarizations ‘p’, and expressions for the cross-polarizations also need to be made explicit. In general, the gain function can be used to determine both monostatic and bistatic RCS of the object, and this can be matched to tables of RCS published in the literature for different types of objects, to determine the function in more detail. Note that the RCS and the gain-function could also include some random components, modeling the fact that different instances of the same type of object (e.g., two different cars of the same or different model, or two different trees or humans) will have slightly different gain functions. In simulations involving multiple instances of the objects, each object would get a different gain-function, assigned just like the large-scale parameters in the existing TR38.901, and then held constant as the small-scale fading evolved. Even if only a single instance is modeled, simulations across multiple drops of Tx/Rx nodes (e.g., gNBs and UEs) and the object would get different gain-functions for the object.

- Conclusion

We have discussed ISAC deployment scenarios and descriptions of simple objects that can be dropped into them for sensing channel modelling. We have the following proposals:

Proposal 1: Consider cases where only a single scatter-point is dropped into the scenario. The drop may be random or may be at a specific location in the scenario layout, relative to other nodes (e.g., relative to gNB(s) and/or UE(s)).

Proposal 2: Consider the simplified models of specific objects with gain-functions as proposed in this contribution, as building blocks to model actual sensing targets (such as humans, AGVs, UAVs, vehicles, hazard objects, etc) and clutter.

- References

- RP-234069, “New SID: Study on channel modelling for Integrated Sensing And Communication (ISAC) for NR,” Nokia, Nokia Shanghai Bell. RAN #102

- R1-2101456, “ISAC Channel modeling”, Qualcomm Incorporated, RAN1#116