3GPP TSG RAN WG1 #116	R1-2401460

Athens, Greece, February 26th – March 1st, 2024

Agenda item:	9.11.3

Source: 	Qualcomm Incorporated

Title: 	NR-NTN uplink capacity/throughput enhancement

Document for:	Discussion and Decision

# Background

In RAN#102, a new work item on NR NTN Phase 3 was agreed [1], including the following objective:

Uplink Capacity/Throughput Enhancement for FR1-NTN [RAN1, RAN2, RAN4]

- Study then specify, if beneficial, DFT-s-OFDM PUSCH enhancements via Orthogonal Cover Codes (OCC)

  - Determine the achievable capacity improvement to be targeted taking into account realistic impairments (e.g. Doppler, time variation, phase distortion, etc)

  - Specify necessary signalling, if needed

  - Update RF requirements accordingly, if needed

  - Note: The study can consider orthogonal cover codes across OFDM symbols, across slots, and/or within an OFDM symbol.

  - Note: the study phase is targeted to be completed by RAN#104

- Notes for this objective:

  - The enhancement is not targeting improvements/impacts of MU-MIMO capability

  - The enhancement is not targeted to PUSCH DMRS

  - No enhancement for initial access

  - Enhancements to PRACH are not in scope.

  - This feature may be applicable for UEs operating in terrestrial networks based on a common design

In this contribution we present our initial views on NR NTN uplink capacity enhancements.

# Motivation

In NTN systems, most of the users of the system will operate in low SNR conditions. To be able to close the link, repetitions are applied to the different physical channels.

Although the use of repetitions allows a single user to successfully deliver a transport block to the gNB, it comes at the cost of network resources: a low-SNR UE needing 32 repetitions to transmit a single transport block will use 32 times more network resources than a high-SNR UE needing a single repetition (assuming same bandwidth allocation). Equivalently, a network supporting a population of low-SNR UEs will be able to support 32 times less users than one of high-SNR UEs. This capacity reduction, however, is not a fundamental limitation, as we will see next.

Using the NR coding chain, under a sufficiently large number of repetitions, the UE will exhaust the circular buffer and the same coded bits will be transmitted multiple times. The repetition of a given bit does not affect the coding rate, but increases the SNR seen by that given bit. For instance, if the full circular buffer is repeated X times, the coding rate will be still 1/5 (rate of the mother code), but the SNR of each bit will be increased by 10log10 (X) dB. The same increase in SNR per bit can be achieved (without a degradation in performance under ideal conditions) by reducing the equivalent bandwidth of the UE by a factor of X and, therefore, increasing the multiplexing capability by a factor of X.

A similar insight can be derived theoretically from the Shannon capacity formula, albeit assuming idealized conditions (infinite codeword length, Gaussian codes, etc.). In low SNR it is possible to reduce the equivalent bandwidth of a single user without impacting its performance. From Shannon’s capacity formula, the achievable bitrate (in nats/s) is:

C=W log⁡1+PW N0

where P is the received signal power, N0 is the power spectral density of the noise, and W is the bandwidth.

At low SNR values, it is well known that the capacity does not depend on the bandwidth W and is asymptotically approximated by C= PN0. In practice, this means a user needing many repetitions (low SNR) would not be affected by a reduction in its bandwidth. Equivalently, the same amount of bandwidth (e.g. 180kHz) would be able to support more users. Adding more users to the system will increase the total throughput, since more power is added to the system (each user will be transmitting at maximum power).

Although the theoretical reasoning above is based on the bandwidth of a signal, they are also applicable to any technique that reduces the degrees of freedom of the channel (or orthogonal channel uses).

# Evaluation methodology

As stated in the work item [1], RAN1 shall study the benefits of applying OCC to PUSCH before proceeding to the normative phase.

Table 1 captures the proposed link level parameters for the evaluation methodology.

| Parameters | Values |
| --- | --- |
| Channel Model | NTN-TDL-C-Rural, 30 degree elevation angle |
| Doppler Spread | 5 Hz |
| Bandwidth | 180 kHz (1 PRB) |
| Repetitions | TBoMS with (N=)8 slots, Number of repetitions (K) = to be reported |
| DMRS | 2 DMRS per slot (config type 1, 8 orthogonal ports) |
| Modulation | QPSK |
| Channel coding scheme | LDPC |
| TBS | 184 |
| DMRS Bundling | Enabled over all TBoMS slots |
| Channel Estimation | LMMSE |
| SNR Range | -10 to -3 dB |
| Antenna configuration at Satellite | 1Rx |
| Antenna configuration at UE | 1Tx |
| Transmission mode | SISO |
| Transmission rank | 1 |
| Multiplexing order for OCC (M) | {1,2,4,6,8,12} (Note that 1 means no OCC) |

Table 1 Evaluation parameters for link level evaluation

| Impairments | Values |
| --- | --- |
| CFO | Uniformly distributed b/w ±0.1 ppm @ 2 GHz per UE |
| Timing offset | Uniformly distributed b/w +-1 μs per UE |
| Timing drift (based on DMRS bundling requirements from R1-2303606) | Uniformly distributed b/w +-1.15μss per UE |
| CFO Compensation | Realistic |

Table 2 Parameters for modelling impairments

Proposal 1: The assumptions in tables 1 and 2 are agreed for evaluation of OCC.

Unlike previous studies in NR NTN (e.g. Rel-18 uplink coverage enhancements), the application of OCC will not increase the performance of the system on a per-UE basis. Instead, the objective is to maximize the overall system throughput (or capacity) by packing more UEs in the same time-frequency resources while minimizing the degradation to individual UEs. In a simple example, applying an OCC with a spreading factor of M=2 would lead, in the best case, to doubling the network throughput while keeping the same per-UE performance as the scenario without OCC.

In order to assess the capacity benefits of OCC, we define the following metrics:

Average BLER for OCC:

Firstly, to evaluate the performance of multiple users using OCC, we consider average BLER across users as the metric for reliability. Let BLERi denote BLER of user i and let M denote the OCC multiplexing order such that i = 1,..,M . Given that all the users transmit same number of TBs (and all users experience identically distributed fading), taking the average BLER across users is a good performance metric. Formally, the average BLER is defined 1Mi=1MBLERi

Single user degradation:

As mentioned above, OCC will not improve the performance from a single user perspective: the best case scenario is where the performance of OCC and no-OCC is the same. Therefore, we propose to measure the degradation a single user would experience when OCC is used (with respect to a baseline where no OCC is applied) as a figure of merit.

We define this metric as follows:

- Measure the SNR x1 at which the single user BLER (no-OCC) is 10% (y1in Figure 1)

- Measure the SNR x2 required to achieve 10% BLER (y1) under OCC (x1 in Figure 1).

- The single user degradation is x2 - x1 dB. This degradation is to be reported for different values of multiplexing orders (M), OCC schemes, etc.

In the ideal case, introducing OCC would not degrade the performance from a single user perspective and the single user degradation would be 0dB.

Capacity gain:

This metric indicates the capacity gain (total throughput) when OCC is applied, taking into account the difference in reliability (BLER) of a system with no OCC to a system with OCC. This metric is defined as follows:

- Measure the SNR x1 at which the single user BLER (no-OCC) is 10% (y1in Fig. 1)

- At the same SNR value x1, measure the BLER y2 when applying OCC.

- The relative throughput gain is 1-y21-y1M, where M is the multiplexing order being used for OCC. This gain is to be reported for different values of multiplexing orders (M), OCC schemes, etc.

Fig. 1 depicts the key quantities used to derive the performance metrics.

Fig. 1: Depiction of data points for evaluation metrics. The reported metrics are x2 - x1 and 1-y21-y1M

Proposal 2: For the evaluation of OCC, the following metrics are agreed (xi, yi are defined in the figure above):

- Degradation at 10% BLER (in dB) vs single user (no-OCC):  x2 - x1 .

- Relative throughput gain at the SNR corresponding to single user 10% BLER: 1-y21-y1M.

As we will discuss in Section 4, there are different techniques to orthogonally multiplex UEs in the same time-frequency resources. Some of these techniques may result in changes to the transmitted signal which impact basic properties of the waveform. For example, applying a cover code in the frequency domain will, in principle, damage the single carrier structure of DFTs-OFDM, which will impact metrics such as cubic metric (CM), peak to average ratio (PAPR) or emissions. In some cases, the impact on the waveform may not only depend on the technique being used but also on the particular OCC codebook (e.g. Hadamard or DFT) and codeword being used. Therefore, RAN1 should study the impact of proposed OCC schemes to existing waveforms.

Proposal 3: For assessment on impact on RF requirements, PAPR / CM shall be reported for the different OCC schemes.

- FFS: reporting of OOBE / IBE.

# Techniques to be evaluated and initial assessment

In this section we discuss different domains over which OCC may be applied. OCC may be applied in the frequency domain i.e., across a sub-carrier level. OCC may also be applied in the time domain i.e., across symbols, groups of symbols, or slots. Additionally, the above techniques may be combined (e.g. similar to how DMRS combines a comb structure with time-domain OCC).

## Cross-symbol and cross-symbol cluster OCC

In cross-symbol OCC, the cover code is applied to OFDM symbols i.e., one symbol is repeated M total times, and a cover code is applied on top. This is shown in Fig. 2.

Fig. 2: Cross-symbol OCC with M=2

With respect to the impact of potential impairments on the orthogonality of this scheme, we make the following qualitative observations:

- Timing offset: Cross-symbol OCC is expected to be robust to timing offset, given that the timing offset is within the cyclic prefix. This is because each sub-carrier of each symbol is impacted by timing offset in the same manner, hence preserving the orthogonality of the cover code.

- Timing drift: Timing drift is expected to have little impact on cross-symbol OCC as long as there is pre-compensation every slot.

- Frequency Error: The impact of CFO will get worse as M increases on cross-symbol OCC. This is because each symbol within the OCC group experiences a phase ramp because of CFO, therefore impacting orthogonality. The higher the value of M, the higher the impact of the phase ramp (because of spreading in time) resulting in loss of orthogonality.

Emission considerations: Since OCC is done in time domain, the waveform per symbol does not change, hence Cross-symbol OCC is expected to have same emissions (IBE and OOB) as compared to case with no OCC.

A generalization of cross-symbol OCC is cross-symbol cluster OCC. In cross-symbol cluster OCC, the cover is applied to a group OFDM symbols (a symbol cluster) i.e., one cluster after doing OCC becomes M clusters. This is shown in Fig. 3. In addition to the spreading factor M, another parameter to be considered for this scheme is the cluster length L i.e., how many OFDM symbols are grouped together to form a cluster. Note that cross-symbol OCC is equivalent cross-symbol cluster OCC when symbol-cluster has only one symbol (L=1).

Fig. 3: Cross-symbol cluster OCC with M=2 and L=2

Resilience to impairments of cross-symbol cluster OCC:

- Timing offset and timing drift: Same performance impact as cross-symbol OCC.

- Frequency Error: In addition to the impact of M (as explained above for cross-symbol OCC), the value of the cluster length L will also affect the performance under frequency error: the larger the value of cluster length L, the larger the spread of symbols across clusters i.e., symbols from each cluster are farther away from each other and suffer from orthogonality loss resulting in performance degradation.

Emission considerations: Same as cross-symbol OCC.

## Cross-slot OCC

In cross-slot OCC, the cover code is applied to OFDM slots i.e, one slot is repeated M total times and the cover code is applied on top. This is shown in Fig. 4. Note that cross-slot OCC can be considered a case of cross-symbol cluster OCC where the duration of the cluster is equal to that of the slot duration e.g., cluster length of 14 symbols with OCC is equal to cross-slot OCC.

Fig. 4: Cross-slot OCC with M=2

Resilience to impairments of cross-slot OCC:

- Timing offset and timing drift: Same performance as cross-symbol OCC.

- Frequency Error: The impact of CFO is worse for slot-wise OCC. This is because spread is done slot-wise rather than symbol or cluster-wise. This means that phase degradation will be worse since the scale of spread in time is worse. Same argument for cluster-wise OCC for large values of L can be applied to explain performance degradation of cross-slot OCC.

Emission considerations:  Same as cross-symbol OCC.

## Intra-symbol pre-DFTS OCC: “comb-like” structure

In Intra-symbol pre-DFTs OCC, a comb-based OCC is applied to sub-carriers. This is shown in Fig. 5. The comb structure can be created by using oversampling property of DFT i.e., a repetition of sequence before doing DFT-s, will result in comb structure after performing DFTS.

Fig. 5: Intra-symbol pre-DFTS OCC with M=2

Resilience to impairments of intra-symbol pre-DFTs OCC:

- Timing offset: Intra-symbol pre DFTS OCC is expected to be robust to timing offset, given that the timing offset is within the cyclic prefix. Timing offset will result in phase difference between different sub-carriers within the same symbol. However, since this type of OCC is done per sub-carrier rather than across sub-carriers, this phase difference will have minimal impact of system performance.

- Timing drift: Timing drift is expected to have little impact on intra-symbol pre DFTS OCC as long as there is pre-compensation every slot.

- Frequency Error: Intra-symbol pre DFTS OCC is expected to be robust to CFO since it is done within a symbol and impact of CFO within a symbol is small given that allocated bandwidth is small enough (lesser PRB allocation)

Emission considerations: Since sub-carriers are power-boosted in case of intra-symbol pre DFTS OCC, there may be impact on in-band emissions for this scheme. This is because power from the edge-subcarriers may leak into the neighboring PRBs. Note that current DMRS already supports comb-like structure, so as long as the spreading factor M is moderate emissions should not be a concern.

## Intra-symbol FD-OCC

In Intra-symbol FD-OCC, OCC is applied to sub-carriers i.e., one sub-carrier is repeated M times with a cover code on top. This is shown in Fig. 6.

Fig. 6: Intra-symbol FD-OCC with M=2

Resilience to impairments of intra-symbol FD-OCC:

- Timing offset: Intra-symbol FD-OCC is expected to be impacted by timing offset. Timing offset will result in phase difference between different sub-carriers within the same symbol. This phase difference will result in loss of orthogonality which will degrade the system performance. Higher the value of M, higher will be the phase differences amongst the sub-carriers, higher will be the loss of orthogonality.

- Timing drift: Timing drift is expected to have little impact on intra-symbol pre DFTS OCC as long as there is pre-compensation every slot.

- Frequency Error: Intra-symbol FD-OCC is expected to be robust to CFO since it is done within a symbol and impact of CFO within a symbol is small given that allocated bandwidth is small enough (lesser PRB allocation)

Emission considerations: Since there may be abrupt phase changes per sub-carrier in case of intra-symbol FD-OCC, the generated waveform per symbol will differ as compared to case with no OCC. In addition, the single carrier waveform of DFT-s will be, in general, broken. These phase changes may impact out of band emissions (PAPR and CM) of the system. Additionally, as we increase the multiplexing order within a PRB before DFTS, we may have IBE issues as well like that of pre DFTS OCC.

Based on our initial assessment, two of the schemes above have clear drawbacks and may be excluded from evaluations in RAN1:

- Cross-slot OCC will be very sensitive to Doppler, since the spreading happens across at least 2ms. A frequency error of 0.1ppm (+/- 200Hz) will impact the orthogonality across slots.

- Intra-symbol FD-OCC will destroy the single-carrier property of DFTs-OFDM waveform, which is not desirable. For intra-symbol OCC, a frequency domain comb structure is preferred.

Therefore, we make the following proposal:

Proposal 4: RAN1 to study at least the following OCC techniques (including combinations of both):

- Cross-symbol and cross-symbol cluster OCC

- Intra-symbol pre-DFT-s OCC (comb structure)

# Preliminary evaluation results

In this section, we present evaluation metrics discussed in Section 3 for different types of OCC largely based on parameters from Tables 1 and 2, with some exceptions. One exception is that we use TBoMS over 20 slots (N=20) and (K=1). This TBoMS configuration is not allowed in the standard currently. Another exception is use of ideal CFO compensation i.e., we assume that CFO of each UE is accurately known at the receiver. This assumption was made to observe how much does the system lose in terms of orthogonality even if perfect CFO compensation was done. Also, note that DFTS PUSCH allows up to 8 orthogonal DMRS ports. Therefore, multiplexing order of 12 is not supported in standard. However, we still analyze M=12 case to get an estimate of the upper bound on OCC performance, assuming 12 orthogonal DMRS ports.

The results are presented in Table 3.

| For single user (no OCC), the BLER is 10% y1 at -7.7 dB x1 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Evaluation metrics | Multiplexing order (M) | Without impairments |  |  | With impairments |  |  |
|  |  | Intra symbol FD-OCC | Pre-DFTS Comb OCC | Cross-symbol OCC | Intra symbol FD-OCC | Pre-DFTS Comb OCC | Cross-symbol OCC |
| Single user degradation (dB) | 2 | 0 | 0 | 0 | 0.2 | 0.2 | 0.2 |
|  | 4 | 0 | 0 | 0 | 0.3 | 0.2 | 0.3 |
|  | 6 | 0.3 | 0.3 | 0.3 | 0.6 | 0.4 | 0.8 |
|  | 12 | 1 | 1 | 1 | 1.7 | 1 | 2.1 |
| Capacity gain | 2 | 2 | 2 | 2 | 1.97 | 1.97 | 1.97 |
|  | 4 | 4 | 4 | 4 | 3.92 | 3.94 | 3.92 |
|  | 6 | 5.9 | 5.9 | 5.9 | 5.79 | 5.83 | 5.76 |
|  | 12 | 11.2 | 11.2 | 11.2 | 10.1 | 11.2 | 10 |

Table 3: Evaluation metrics with different multiplexing orders

Observation 1: Based on preliminary evaluation results, the following is observed:

- Pre-DFT-s comb OCC is most robust to impairments.

- The loss of cross-symbol OCC is small for small values of M, but increases up to 2.1dB for multiplexing of 12 users.

- Systems with OCC can get a relative throughput gain of up to 6x with a single-user degradation of less than 1 dB.

# Potential specification impact

Regarding the specification impact of OCC, we make some preliminary assessment on the potential issues that RAN1 would need to discuss after the benefit of OCC is confirmed:

- Spreading factor(s) that are supported by the standard.

- Signaling of spreading factor (DG and CG)

- Signaling of OCC codeword to be used for a particular transmission.

- Determination of TBS when OCC is applied.

- Aspects related to UCI multiplexing.

- Resource allocation.

Proposal 5: RAN1 to consider the following aspects when developing specifications for OCC:

- Spreading factor(s) that are supported by the standard.

- Signaling of spreading factor (DG and CG)

- Signaling of OCC codeword to be used for a particular transmission.

- Determination of TBS when OCC is applied.

- Aspects related to UCI multiplexing.

- Resource allocation.

# Summary

In this contribution we presented our initial views on uplink capacity enhancements for NR NTN. We made the following observations and proposals:

| Parameters | Values |
| --- | --- |
| Channel Model | NTN-TDL-C-Rural, 30 degree elevation angle |
| Doppler Spread | 5 Hz |
| Bandwidth | 180 kHz (1 PRB) |
| Repetitions | TBoMS with (N=)8 slots, Number of repetitions (K) = to be reported |
| DMRS | 2 DMRS per slot (config type 1, 8 orthogonal ports) |
| Modulation | QPSK |
| Channel coding scheme | LDPC |
| TBS | 184 |
| DMRS Bundling | Enabled over all TBoMS slots |
| Channel Estimation | LMMSE |
| SNR Range | -10 to -3 dB |
| Antenna configuration at Satellite | 1Rx |
| Antenna configuration at UE | 1Tx |
| Transmission mode | SISO |
| Transmission rank | 1 |
| Multiplexing order for OCC (M) | {1,2,4,6,8,12} (Note that 1 means no OCC) |

Table 1 Evaluation parameters for link level evaluation

| Impairments | Values |
| --- | --- |
| CFO | Uniformly distributed b/w ±0.1 ppm @ 2 GHz per UE |
| Timing offset | Uniformly distributed b/w +-1 μs per UE |
| Timing drift (based on DMRS bundling requirements from R1-2303606) | Uniformly distributed b/w +-1.15μss per UE |
| CFO Compensation | Realistic |

Table 2 Parameters for modelling impairments

Proposal 1: The assumptions in tables 1 and 2 are agreed for evaluation of OCC.

Depiction of data points for evaluation metrics. The reported metrics are x2 - x1 and 1-y21-y1M

Proposal 2: For the evaluation of OCC, the following metrics are agreed (xi, yi are defined in the figure above):

- Degradation at 10% BLER (in dB) vs single user (no-OCC):  x2 - x1 .

- Relative throughput gain at the SNR corresponding to single user 10% BLER: 1-y21-y1M.

Proposal 3: For assessment on impact on RF requirements, PAPR / CM shall be reported for the different OCC schemes.

- FFS: reporting of OOBE / IBE.

Proposal 4: RAN1 to study at least the following OCC techniques (including combinations of both):

- Cross-symbol and cross-symbol cluster OCC

- Intra-symbol pre-DFT-s OCC (comb structure)

Observation 1: Based on preliminary evaluation results, the following is observed:

- Pre-DFT-s comb OCC is most robust to impairments.

- The loss of cross-symbol OCC is small for small values of M, but increases up to 2.1dB for multiplexing of 12 users.

- Systems with OCC can get a relative throughput gain of up to 6x with a single-user degradation of less than 1 dB.

Proposal 5: RAN1 to consider the following aspects when developing specifications for OCC:

- Spreading factor(s) that are supported by the standard.

- Signaling of spreading factor (DG and CG)

- Signaling of OCC codeword to be used for a particular transmission.

- Determination of TBS when OCC is applied.

- Aspects related to UCI multiplexing.

- Resource allocation.

# References

[1] RP-234078, New WID: Non-Terrestrial Networks (NTN) for NR Phase 3