3GPP TSG RAN WG1 #116	R1-2401461

Athens, Greece, February 26th – March 1st, 2024

Agenda item:	9.11.4

Source: 	Qualcomm Incorporated

Title: 	IOT-NTN uplink capacity/throughput enhancement

Document for:	Discussion and Decision

# Background

In RAN#102, a new work item on NR NTN Phase 3 was agreed [1], including the following objective:

- Support of Capacity enhancements for uplink

  - Study then specify, if beneficial, enhancements to enable multiplexing of multiple UEs (e.g. up to the min of 4 and the maximum allowed by the existing UL and DL signalling) in a single 3.75 kHz or 15 kHz subcarrier via orthogonal cover codes (OCC) for NPUSCH format 1 and NPRACH [RAN1, RAN2]

    - Multi-tone support for 15 kHz SCS should also be considered

Note: Impact of impairment shall be taken into account

  - Study and specify, if beneficial the following enhancements to reduce the necessary uplink and downlink signaling to complete an EDT transaction [RAN2]:

    - Msg3 transmission without msg1/RAR

    - Efficient delivery (reduced overhead) of msg4 / RRCEarlyDataComplete

In this contribution we present our initial views on OCC for IOT NTN.

# Motivation

In IOT NTN systems, most of the users of the system will operate in low SNR conditions. To be able to close the link, repetitions are applied to the different physical channels.

Although the use of repetitions allows a single user to successfully deliver a transport block to the eNB, it comes at the cost of network resources: a low-SNR UE needing 200ms to transmit a single transport block will use 10 times more network resources than a high-SNR UE needing 20ms (assuming same bandwidth allocation). Equivalently, a network supporting a population of low-SNR UEs will be able to support 10 times less users than one of high-SNR UEs. This capacity reduction, however, is not a fundamental limitation, as we will see next.

Using the NB-IoT coding chain, under a sufficiently large number of repetitions, the UE will exhaust the circular buffer and the same coded bits will be transmitted multiple times. The repetition of a given bit does not affect the coding rate, but increases the SNR seen by that given bit. For instance, if the full circular buffer is repeated X times, the coding rate will be still 1/3 (rate of the mother code), but the SNR of each bit will be increased by 10log10 (X) dB. The same increase in SNR per bit can be achieved (without a degradation in performance under ideal conditions) by reducing the equivalent bandwidth of the UE by a factor of X and, therefore, increasing the multiplexing capacity by a factor of X.

A similar insight can be derived theoretically from the Shannon capacity formula, albeit assuming idealized conditions (infinite codeword length, Gaussian codes, etc.). In low SNR conditions it is possible to reduce the equivalent bandwidth of a single user without impacting its performance. From Shannon’s capacity formula, the achievable bitrate (in nats/s) is:

C=W log⁡1+PW N0

where P is the received signal power, N0 is the power spectral density of the noise, and W is the bandwidth.

At low SNR values, it is well known that the capacity does not depend on the bandwidth W, and is asymptotically approximated by C= PN0. In practice, this means a user needing many repetitions (low SNR) would not be affected by a reduction in its bandwidth. Equivalently, the same amount of bandwidth (e.g. 3.75kHz or 15kHz) would be able to support more users. Adding more users to the system will increase the total throughput, since more power is added to the system (each user will be transmitting at maximum power).

Although the theoretical reasoning above is based on the bandwidth of a signal, they are also applicable to any technique that reduces the degrees of freedom of the channel (or orthogonal channel uses).

# NPUSCH capacity enhancements

## Evaluation methodology

As stated in the work item [1], RAN1 shall study the benefits of applying OCC to NPUSCH before proceeding to the normative phase.

Table 1 captures the proposed link level parameters for the evaluation methodology. Although the enhancements are applicable to any orbit type, we propose to focus the evaluations on GEO for simplicity.

| Parameters | Values |
| --- | --- |
| Channel Model | NTN-TDL-C-Rural, 30 degree elevation angle |
| Doppler Spread | 5 Hz |
| Bandwidth | Single tone (15 or 3.75kHz) |
| Repetitions / Number of RUs | To be reported |
| DMRS | For baseline: Per Rel-18 specs. For OCC: Declared by proponents. |
| Modulation | QPSK / BPSK |
| Channel coding scheme | LDPC |
| TBS | To be reported |
| Channel Estimation | LMMSE |
| SNR Range | Per TR 36.763 for GSO: For 3.75kHz SCS: Between -5.2 and 2.6dB For 15kHz SCS: Between -11.2 and -3.5dB |
| Antenna configuration at Satellite | 1Rx |
| Antenna configuration at UE | 1Tx |
| Transmission mode | SISO |
| Transmission rank | 1 |
| Multiplexing order for OCC (M) | Up to 4 |

Table 1 Evaluation parameters for link level evaluation of NPUSCH with OCC

| Impairments | Values |
| --- | --- |
| CFO | Uniformly distributed b/w ±0.1 ppm @ 2 GHz per UE |
| Timing offset | Uniformly distributed b/w +-1 μs per UE |
| Timing drift | None (GEO scenario) |
| CFO Compensation | Realistic |

Table 2 Parameters for modelling impairments of NPUSCH with OCC

Proposal 1: The assumptions in tables 1 and 2 are agreed for evaluation of NPUSCH with OCC.

The application of OCC will not increase the performance of the system on a per-UE basis. Instead, the objective is to maximize the overall system throughput (or capacity) by packing more UEs in the same time-frequency resources while minimizing the degradation to individual UEs. In a simple example, applying an OCC with a spreading factor of M=2 would lead, in the best case, to doubling the network throughput while keeping the same per-UE performance as the scenario without OCC.

In order to assess the capacity benefits of OCC, we define the following metrics:

Average BLER for OCC:

Firstly, to evaluate the performance of multiple users using OCC, we consider average BLER across users as the metric for reliability. Let BLERi denote BLER of user i and let M denote the OCC multiplexing order such that i = 1,..,M . Given that all the users transmit same number of TBs (and all users experience identically distributed fading), taking the average BLER across users is a good performance metric. Formally, the average BLER is defined 1Mi=1MBLERi

Single user degradation:

As mentioned above, OCC will not improve the performance from a single user perspective: the best case scenario is where the performance of OCC and no-OCC is the same. Therefore, we propose to measure the degradation a single user would experience when OCC is used (with respect to a baseline where no OCC is applied) as a figure of merit.

We define this metric as follows:

- Measure the SNR x1 at which the single user BLER (no-OCC) is 10% (y1in Figure 1)

- Measure the SNR x2 required to achieve 10% BLER (y1) under OCC (x1 in Figure 1).

- The single user degradation is x2 - x1 dB. This degradation is to be reported for different values of multiplexing orders (M), OCC schemes, etc.

In the ideal case, introducing OCC would not degrade the performance from a single user perspective and the single user degradation would be 0dB.

Capacity gain:

This metric indicates the capacity gain (total throughput) when OCC is applied. We compare reliability (BLER) of a system with no OCC to a system with OCC. This metric is defined as follows:

- Measure the SNR x1 at which the single user BLER (no-OCC) is 10% (y1in Fig. 1)

- At the same SNR value x1, measure the BLER y2 when applying OCC.

- The relative throughput gain is 1-y21-y1M, where M is the multiplexing order being used for OCC. This gain is to be reported for different values of multiplexing orders (M), OCC schemes, etc.

Figure 1 depicts the key quantities used to derive the performance metrics.

Figure 1: Depiction of data points for evaluation metrics. The reported metrics are x2 - x1 and 1-y21-y1M

Proposal 2: For the evaluation of OCC for NPUSCH, the following metrics are agreed (xi, yi are defined in the figure above):

- Degradation at 10% BLER (in dB) vs single user (no-OCC):  x2 - x1 .

- Relative throughput gain at the SNR corresponding to single user 10% BLER: 1-y21-y1M.

## Techniques to be evaluated

In this section we present our views on the techniques to be evaluated for OCC. Given we are focusing on single tone transmissions, all the options rely on orthogonal codes across time. If multi-tone is decided to be evaluated, codes in the frequency domain could be additionally considered.

### Cross-symbol OCC

Under cross-symbol OCC, consecutive symbols are repeated with a cover code applied on top, as depicted in Figure 2.

Figure 2: Cross-symbol OCC, spreading factor (M) of 2

Repeating the symbols back to back has the advantage of being more resilient to CFO, since the time spread of an OCC code is as small as possible. For instance, a spreading factor of 2 results in an equivalent subcarrier spacing of 1.875kHz, so degradation should be minimal as long as the CFO difference between UEs is significantly smaller than this value.

### Cross-slot / cross-RU OCC

In cross-slot or cross-RU OCC, the current structure of a slot (or RU) is maintained, after which the same slot (or RU) is repeated M-1 times with a cover code applied on top. Figure 3 depicts this operation, taking cross-slot as an example:

Figure 3: Cross-slot OCC, spreading factor (M) of 2

One disadvantage of cross-slot OCC (vs cross-symbol OCC) is that the two repeated symbols are now separated by 1 slot duration (2ms for the case of 3.75kHz SCS). In this case, the equivalent subcarrier spacing would be 500Hz. With a worst-case CFO of 0.1ppm at 2GHz bandwidth, the resulting differential CFO between two UEs will be up to 400Hz. Therefore, cross-slot OCC is expected to suffer from significant degradation in the presence of Doppler.

Observation 1: Cross-slot OCC is impacted by CFO, resulting in loss of orthogonality within an OCC codeword.

Based on the above, we make the following proposal:

Proposal 3: At least for single tone, consider cross-symbol OCC (OCC across consecutive OFDM symbols) as the baseline technique to be evaluated.

### Aspects related to DMRS design

One key difference in the design of OCC for NB-IoT and NR is that NB-IoT NPUSCH DMRS does not support orthogonal multiplexing of UEs. When designing the new DMRS structure, special attention should be paid to the following aspects:

- CFO pull-in range: The separation between DMRS symbols should allow to accurately estimate the CFO within the desired range. Having DMRS symbols very separated in time will lead to a wrap-around of the phase and would not allow to estimate the CFO accurately due to potential ambiguities. If the DMRS symbols are very close in time, then the accuracy of the CFO will be affected due to the smaller phase change between two symbols.

- Overhead: In general, the relative overhead of DMRS should be kept as low as possible.

Therefore, we make the following proposal:

Proposal 4: For DMRS with OCC, proponents to declare the DMRS structure that they assumed for the evaluation, accounting for the following aspects:

- Pull-in range of DMRS.

- Relative overhead of DMRS.

## Interaction with other features

In NB-IoT NTN, in addition to the RRC_CONNECTED data transfer, enhancements like EDT (including Rel-19 enhancements) and PUR may be used for short data transfer. In some cases, networks may operate with a majority of the UEs using these techniques. Therefore, it is proposed that NPUSCH OCC is studied in all these cases.

Proposal 5: RAN1 to study usage of NPUSCH OCC at least in the following scenarios:

- Connected mode dynamic grant.

- EDT

- PUR

- RACH-less EDT (R19)

# NPRACH capacity enhancements

## Evaluation Methodology

Table 3 and 4 capture the proposed link level parameters for the evaluation methodology of NPRACH with OCC.

| Parameters | Values |
| --- | --- |
| Channel Model | NTN-TDL-C-Rural, 30 degree elevation angle |
| Doppler Spread | 5 Hz |
| Bandwidth | Single tone, 3.75kHz |
| NPRACH Premable Format | Format 1 (CP symbol of the same length as other symbols) |
| Repetitions / Number of Preamble Repetition Units (PRUs) | To be reported |
| Modulation | QPSK / BPSK |
| Probability of Misdetection / False Alarm | 1% / 0.1% |
| SNR Range | Per TR 36.763 for GSO: For 3.75kHz SCS: Between -5.2 and 2.6dB |
| Antenna configuration at Satellite | 1Rx |
| Antenna configuration at UE | 1Tx |
| Multiplexing order for OCC (M) | Up to 4 |

Table 3 Evaluation parameters for link level evaluation of NPRACH with OCC

| Impairments | Values |
| --- | --- |
| CFO | Uniformly distributed b/w ±0.1 ppm @ 2 GHz per UE |
| Timing offset | Uniformly distributed b/w +-1 μs per UE |
| Timing drift | None (GEO scenario) |
| CFO Compensation | Realistic |

Table 4 Parameters for modelling impairments of NPRACH with OCC

Proposal 6: The assumptions in tables 3 and 4 are agreed for evaluation of NPRACH with OCC.

As with the NPUSCH case, the introduction of OCC will not improve the link performance of a single UE – it will just increase the number of available preambles and thus the NPRACH capacity. Therefore, in order to assess the performance improvements of OCC, we propose to declare the degradation at 1% missed detection rate for each multiplexing order. As long as this degradation is small, the uplink capacity of NPRACH will be increased without affecting the single user performance.

Proposal 7: For the evaluation of OCC for NPRACH, the following metric is agreed:

- Degradation at 1% missed detection rate (in dB) for each multiplexing order (M).

## Techniques to be evaluated

For multiplexing multiple UEs over the same NPRACH preamble, OCC-based multiplexing schemes may be applied at different granularities—e.g., across the symbols within a symbol-group, or, across different (e.g., adjacent) symbol groups.

### Cross-symbol OCC

In the figure below, we provide a schematic diagram of what a “cross-symbol” (within a symbol group) multiplexing structure may potentially look like—for example, M UEs may transmit using their orthogonal multiplexing signatures within a symbol group, and the base station will be tasked with de-multiplexing the transmissions on the given preamble (in the figure below, the preamble of interest is shaded in blue, with a starting subcarrier index of 6, and following the spec-compliant pseudo-random frequency-hopped pattern of resource allocation in time and frequency).

Note that a cross-symbol OCC is less sensitive to the impact of CFO than a cross-symbol group variant (discussed next), owing to the lesser physical time span of the orthogonal cover code: the longer the time-span of an OCC codeword, the more the loss of orthogonality due to CFO (in other words, the range of CFO that can be handled is inversely proportional to the physical time span of the OCC).

Figure 4: Cross-symbol OCC for NPRACH

### Cross-symbol group OCC

A cross-symbol group multiplexing architecture, for M=2 UEs, over two consecutive symbol groups, is depicted in the figure below.

Figure 5: Cross-symbol group OCC for NPRACH

To demonstrate the difficulty of performing cross-symbol group OCC for NPRACH multiplexing, let us take a closer look at the impact of CFO on the receiver.

The CFO can be thought of a phase ramp in time with a (worst case, as per the assumptions in Table 4) slope of  2πϵn, where ϵ=fo(=200 Hz)Δf=2003750=0.0533, where n is symbol index (assuming NPRACH Format 1, where the “CP symbol” is of the same length as the other symbols within a symbol group). For the CFO to not cause a loss of orthogonality over the block of n symbols, n should meet n≪ 12ϵ=18.76 symbols. Even in the simplest cross-symbol group OCC example, i.e., M=2 UEs multiplexed (e.g., using a length 2 Hadamard code) across adjacent symbol groups which are spaced 6 symbols apart—there would be a phase difference of 115 degrees and a maximum differential phase difference between two UEs of 230 degrees, severely impacting OCC demultiplexing performance. Further, for larger multiplexing orders (e.g., M=4 and beyond), the situation would only become more and more prohibitive.

Observation 2: Cross-symbol group OCC is impacted by CFO, resulting in loss of orthogonality across OCC codewords.

Proposal 8: Study at least the following technique for NPRACH multiplexing using OCC

- OCC across consecutive symbols (within a symbol group).

## Interaction with other features and specification impact

In order to maximize the capacity increase given by OCC in a legacy deployment, it is desirable to have legacy UEs and OCC capable UEs share the same set of NPRACH resources. To that end, RAN1 should investigate mechanisms for such a coexistence to be facilitated. As an example, in a cross-symbol OCC setup for NPRACH (as described in Section 4.2.1), a legacy UE may be thought of one that always transmits an OCC signature of an “all 1 OCC codeword”—therefore, OCC codebooks comprising the “all 1 codeword” may facilitate legacy UEs to transmit NPRACH over the same set of resources. However, aspects such as how to share the pool of NPRACH resources between new and legacy UEs, would need to be further studied.

Proposal 9: RAN1 to strive for allowing multiplexing of legacy UEs and OCC UEs in the same NPRACH resources

The designs that RAN1 develops for OCC-based NPRACH multiplexing should be as broadly applicable as possible, since NPRACH capacity may have the potential to become a bottleneck in any instance in which a NPRACH may be required to be transmitted. While (contention-based) legacy (4-step) initial access is the most canonical use case, important features for efficient small-data transmission, such as EDT, also warrant due consideration, particularly considering the “partitioned” NPRACH preamble space for EDT. Contention-free random access (e.g., PDCCH ordered NPRACH) is also an aspect that is worth studying, w.r.t the potential capacity benefits of OCC-based NPRACH multiplexing.

Proposal 10: RAN1 to study usage of NPRACH OCC at least in the following scenarios:

- Initial access

- EDT

- PDCCH order

- Connected mode CBRA

Currently, there isn’t a notion of an OCC signature associated with a UE’s transmission of a NPRACH preamble, which would need to be introduced for supporting OCC-based NPRACH. The introduction of an OCC-signature associated with an NPRACH preamble transmission may require studying any potential impact on the random access response (RAR), which would now need to address multiple UEs transmitting NPRACH on the same physical time and frequency resource.

Proposal 11: RAN1 to study the specification impact of NPRACH OCC on the RAR.

# Summary

In this contribution we presented our initial views on uplink capacity enhancements for IOT NTN. We made the following observations and proposals:

| Parameters | Values |
| --- | --- |
| Channel Model | NTN-TDL-C-Rural, 30 degree elevation angle |
| Doppler Spread | 5 Hz |
| Bandwidth | Single tone (15 or 3.75kHz) |
| Repetitions / Number of RUs | To be reported |
| DMRS | For baseline: Per Rel-18 specs. For OCC: Declared by proponents. |
| Modulation | QPSK / BPSK |
| Channel coding scheme | LDPC |
| TBS | To be reported |
| Channel Estimation | LMMSE |
| SNR Range | Per TR 36.763 for GSO: For 3.75kHz SCS: Between -5.2 and 2.6dB For 15kHz SCS: Between -11.2 and -3.5dB |
| Antenna configuration at Satellite | 1Rx |
| Antenna configuration at UE | 1Tx |
| Transmission mode | SISO |
| Transmission rank | 1 |
| Multiplexing order for OCC (M) | Up to 4 |

Table 1 Evaluation parameters for link level evaluation of NPUSCH with OCC

| Impairments | Values |
| --- | --- |
| CFO | Uniformly distributed b/w ±0.1 ppm @ 2 GHz per UE |
| Timing offset | Uniformly distributed b/w +-1 μs per UE |
| Timing drift | None (GEO scenario) |
| CFO Compensation | Realistic |

Table 2 Parameters for modelling impairments of NPUSCH with OCC

Proposal 1: The assumptions in tables 1 and 2 are agreed for evaluation of NPUSCH with OCC.

Depiction of data points for evaluation metrics. The reported metrics are x2 - x1 and 1-y21-y1M

Proposal 2: For the evaluation of OCC for NPUSCH, the following metrics are agreed (xi, yi are defined in the figure above):

- Degradation at 10% BLER (in dB) vs single user (no-OCC):  x2 - x1 .

- Relative throughput gain at the SNR corresponding to single user 10% BLER: 1-y21-y1M.

Observation 1: Cross-slot OCC is impacted by CFO, resulting in loss of orthogonality across OCC codewords.

Proposal 3: At least for single tone, consider cross-symbol OCC (OCC across consecutive OFDM symbols) as the baseline technique to be evaluated.

Proposal 4: For DMRS with OCC, proponents to declare the DMRS structure that they assumed for the evaluation, accounting for the following aspects:

- Pull-in range of DMRS.

- Relative overhead of DMRS.

Proposal 5: RAN1 to study usage of NPUSCH OCC at least in the following scenarios:

- Connected mode dynamic grant.

- EDT

- PUR

- RACH-less EDT (R19)

| Parameters | Values |
| --- | --- |
| Channel Model | NTN-TDL-C-Rural, 30 degree elevation angle |
| Doppler Spread | 5 Hz |
| Bandwidth | Single tone, 3.75kHz |
| NPRACH Premable Format | Format 1 (CP symbol of the same length as other symbols) |
| Repetitions / Number of Preamble Repetition Units (PRUs) | To be reported |
| Modulation | QPSK / BPSK |
| Probability of Misdetection / False Alarm | 1% / 0.1% |
| SNR Range | Per TR 36.763 for GSO: For 3.75kHz SCS: Between -5.2 and 2.6dB |
| Antenna configuration at Satellite | 1Rx |
| Antenna configuration at UE | 1Tx |
| Multiplexing order for OCC (M) | Up to 4 |

Table 3 Evaluation parameters for link level evaluation of NPRACH with OCC

| Impairments | Values |
| --- | --- |
| CFO | Uniformly distributed b/w ±0.1 ppm @ 2 GHz per UE |
| Timing offset | Uniformly distributed b/w +-1 μs per UE |
| Timing drift | None (GEO scenario) |
| CFO Compensation | Realistic |

Table 4 Parameters for modelling impairments of NPRACH with OCC

Proposal 6: The assumptions in tables 3 and 4 are agreed for evaluation of NPRACH with OCC.

Proposal 7: For the evaluation of OCC for NPRACH, the following metric is agreed:

- Degradation at 1% missed detection rate (in dB) for each multiplexing order (M).

Observation 2: Cross-symbol group OCC is impacted by CFO, resulting in loss of orthogonality across OCC codewords.

Proposal 8: Study at least the following technique for NPRACH multiplexing using OCC

- OCC across consecutive symbols (within a symbol group).

Proposal 9: RAN1 to strive for allowing multiplexing of legacy UEs and OCC UEs in the same NPRACH resources

Proposal 10: RAN1 to study usage of NPRACH OCC at least in the following scenarios:

- Initial access

- EDT

- PDCCH order

- Connected mode CBRA

Proposal 11: RAN1 to study the specification impact of NPRACH OCC on the RAR.

# References

[1] RP-234077, New WID: Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3