3GPP TSG-RAN WG1 #116			R1-2401477

Athens, Greece, Feb. 26th – Mar 1th, 2024

Agenda item:	9.1.3.1

Source:	NTPU

Title:	Discussion on CSI Prediction under AI/ML for NR Air-Interface

Document for:	Discussion and Decision

- Introduction

In RAN#102 meeting, a new Rel-19 WID on AI/ML for NR Air-Interface [1] was agreed. The WID includes several study objectives that address some outstanding issues to deepen understanding with a view to future normative work. In this contribution, we would discuss CSI Prediction for CSI feedback enhancement.

Study objectives with corresponding checkpoints in RAN#105 (Sept ’24): - CSI feedback enhancement [RAN1]: - For CSI prediction (one-sided model), further study performance gain over Rel-18 non-AI/ML based approach and associated complexity, while addressing other aspects requiring further study/conclusion as captured in the conclusions section of the TR 38.843 (e.g., cell/site specific model could be considered to improve performance gain).

- Evaluations

In [2], many companies provide simulation results and observations to evaluate the gain of CSI prediction using AI/ML. For performance gains based on non-AI/ML benchmarks, Squared Generalised Cosine Similarity (SGCS) and User Perceived Throughput (UPT) metrics are provided according to the benchmarks of nearest historical CSI and auto-regression/Kalman filter. The impact of modelling spatial consistency and the performance gains based on Normalized Mean Squared Error (NMSE) intermediated KPI have also been studied, but there is a lack of the corresponding observation in [2]. Spatial consistency is a characteristic of a channel model such as a large number of closely located users. To study complete performance gains, more complete observations may be required such as considering both spatial consistency procedure A and spatial consistency procedure B in [3]. On the other hand, observations based on the NMSE metric can also be considered. To evaluate AI/ML solutions more generally, we propose to study the impact on these two factors.

Proposal 1: RAN 1 is suggested to further study the impact of modelling spatial consistency based on both procedure A and procedure B, and to get more observations based on the performance gain of NMSE metric.

For generalization, observations on SGCS metrics at different UE speeds are provided, as well as observations on the approach of dataset mixing. To consider more different scenarios, we propose to study the impact on various deployment scenarios (e.g., 3D urban micro (UMi), 3D urban macro (UMa)), various carrier frequencies, and other aspects of scenarios.

Proposal 2: Study the impact on various deployment scenarios, various carrier frequencies, etc. for generalization evaluations.

​.

- Data Collection and Performance Monitoring

In [2], CSI prediction using UE-sided model is considered. Based on CSI prediction using UE-sided model, many companies discuss signaling and procedure used for data collection. The efficiency of data collection should be considered for the design signaling and procedure. For example, the data collection efficiency refers to the latency of the data collection process. Moreover, the data size for each data collection process may be one of the important factors in signalling design. Therefore, we propose to study the data characteristics of data collection to signaling design and procedure design.

Proposal 3: Study the data characteristics of data collection to signaling design and procedure design.

Based on CSI prediction using UE-sided model, three types of performance monitoring for functionality-based Life-Cycle Management (LCM) are considered.

Type 1: - UE calculates the performance metric(s) - UE reports performance monitoring output that facilitates functionality fallback decision at the network - Performance monitoring output details can be further defined - NW may configure threshold criterion to facilitate UE side performance monitoring (if needed). - NW makes decision(s) of functionality fallback operation (fallback mechanism to legacy CSI reporting). Type 2: - UE reports predicted CSI and/or the corresponding ground-truth - NW calculates the performance metrics. - NW makes decision(s) of functionality fallback operation (fallback mechanism to legacy CSI reporting). Type 3: - UE calculates the performance metric(s) - UE reports performance metric(s) to the NW - NW makes decision(s) of functionality fallback operation (fallback mechanism to legacy CSI reporting).

For type 1 performance monitoring, UE calculates the performance metric(s), generates performance monitoring output based on the corresponding performance metric(s), and reports the output to NW. NW then makes decision(s) of functionality fallback operation according to the received output from UE. For type 2 performance monitoring, UE reports predicted CSI and/or the corresponding ground-truth directly. NW then calculates the performance metrics, and makes NW makes decision(s) of functionality fallback operation according to the calculated performance metrics. For type 3 performance monitoring, UE calculates the performance metric(s), and reports it to NW. NW then makes decision(s) of functionality fallback operation according to the received performance metric(s) from UE. All three types of performance monitoring may require the calculation of performance metric(s). The performance metric(s) refers to NMSE, SGCS, …etc. However, the requirements for performance metrics may be different for each type of performance monitoring. Thus, we propose to study the appropriate performance metric for each type of performance monitoring separately.

Proposal 4: Study the appropriate performance metric for each type of performance monitoring separately.

- Conclusions

Proposal 1: RAN1 is suggested to further study the impact of modelling spatial consistency based on both procedure A and procedure B, and to get more observations performance gain based on performance gain of the NMSE metric.

Proposal 2: Study the impact on various deployment scenarios, various carrier frequencies, etc. for generalization evaluation.

Proposal 3: Study the data characteristics of data collection to signaling design and procedure design.

Proposal 4: Study the appropriate performance metric for each type of performance monitoring separately.

- Reference

- RP-234039, New WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface, Qualcomm

- RP-233946, TR 38.843 v2.0.1 Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface, Qualcomm Germany

- 3GPP TR 38.901 V17.1.0 (2024-01), Study on channel model for frequencies from 0.5 to 100 GHz