3GPP TSG-RAN WG1 Meeting #116	R1-2401479

Athens, Greece, February 26th – March 1st, 2024

Agenda Item:	9.1.3.3

Source:	NTPU

Title:	Discussion on functionality update, model identification, data collection and model transfer

Document for:	Discussion and Decision

Introduction

In RAN#102 meeting, the Rel-19 WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface was approved [1], which includes the following objective corresponding to the model identification/procedure, collection of UE-sided model training data, and model transfer/delivery:

Provide specification support for the following aspects: - AI/ML general framework for one-sided AI/ML models within the realm of what has been studied in the FS_NR_AIML_Air project [RAN2]: - Signalling and protocol aspects of Life Cycle Management (LCM) enabling functionality and model (if justified) selection, activation, deactivation, switching, fallback - Identification related signalling is part of the above objective - Necessary signalling/mechanism(s) for LCM to facilitate model training, inference, performance monitoring, data collection (except for the purpose of CN/OAM/OTT collection of UE-sided model training data) for both UE-sided and NW-sided models - Signalling mechanism of applicable functionalities/models … Study objectives with corresponding checkpoints in RAN#105 (Sept ’24): … - Necessity and details of model Identification concept and procedure in the context of LCM [RAN2/RAN1] - CN/OAM/OTT collection of UE-sided model training data [RAN2/RAN1]: - For the FS_NR_AIML_Air study use cases, identify the corresponding contents of UE data collection - Analyse the UE data collection mechanisms identified during the FS_NR_AIML_Air (TR 38.843 section 7.2.1.3.2) study along with the implications and limitations of each of the methods - Model transfer/delivery [RAN2/RAN1]: - Determine whether there is a need to consider standardised solutions for transferring/delivering AI/ML model(s) considering at least the solutions identified during the FS_NR_AIML_Air study …

In this contribution, we discuss the aspect of AI/ML model and data including functionality update, model identification, data collection (i.e., CN/OAM/OTT collection of UE-sided model training data), and model transfer/delivery.

Discussion

Functionality Update

In [2], Life Cycle Management (LCM) of AI/ML model and AI/ML functionality is characterized. AI/ML model refers to a data driven algorithm that applies AI/ML techniques to generate a set of outputs based on a set of inputs. AI/ML functionality refers to an AI/ML-enabled Feature/FG enabled by configuration(s), where configuration(s) is(are) supported based on conditions indicated by UE capability.

LCM includes common operations both for AI/ML model and AI/ML functionality, e.g., (1) functionality/model identification, (2) functionality/model selection, activation, deactivation, switching, and fallback operation and (3) functionality/model monitoring. However, we observe that “update” operation is specific for AI/ML model, i.e., model update in LCM, but is not captured for AI/ML functionality in LCM.

Observation 1: Model update is captured in LCM, while functionality update is not included in LCM.

However, we need to further clarify whether functionality update is required. If it is required, how to capture in LCM. According to [2], “After functionality identification, necessity, mechanisms, for UE to report updates on applicable functionality(es) among functionality(es) are studied, where the applicable functionalities may be a subset of all functionalities. Applicable functionalities can be reported by the UE”, the concept of functionality update was studied, and the UE can report the updates on applicable functionalities. Thus, we suggest RAN1 to further discuss how to capture the functionality update in LCM.

Proposal 1: RAN1 clarifies the requirement of functionality update and further discusses how to capture the functionality update in LCM.

Model Identification

Model identification (ID) is an important step in AI/ML model management and LCM. Model ID is used to identify an AI/ML model. For further study, we need to clarify whether the network can identify an AI/ML model via the corresponding model ID. Based on the description in [2], we observe the description contradiction on whether the network can identify an AI/ML model via the corresponding model ID. For further study, we need to clarify this issue first. For example, one description in [2], “In functionality-based LCM, network indicates activation/deactivation/fallback/switching of AI/ML functionality via 3GPP signalling (e.g., RRC, MAC-CE, DCI). Models may not be identified at the Network, and UE may perform model-level LCM. Whether and how much awareness/interaction NW should have about model-level LCM requires further study”, points out that models may not be identified at the network via the model IDs. For another example, another description in [2], “For AI/ML model identification, models are identified by model ID at the Network. UE indicates supported AI/ML models”, indicates that models are identified at the network via the model IDs.

Observation 2: Whether AI/ML models are identified at the network via the model IDs needs to be clarified.

However, it is possible that AI/ML models are NOT identified at the network via the model IDs in some conditions (e.g., functionality-based LCM), while AI/ML models are identified at the network via the model IDs in other conditions (e.g., for UE-side models and UE-parts of two-sided models). We would suggest RAN1 to revisit the descriptions in [2] and clarify the issue: whether AI/ML models are identified at the network via the model IDs. Furthermore, model identification can be applied in the functionality-based LCM. Thus, we further suggest RAN1 to clarify whether the “model identification” in one of the study objectives, “Necessity and details of model Identification concept and procedure in the context of LCM”, relates to functionality identification.

Proposal 2: RAN1 clarifies whether AI/ML models are identified at the network via the model IDs.

Proposal 3: RAN1 clarifies whether the “model identification” in one of the study objectives, “Necessity and details of model Identification concept and procedure in the context of LCM”, relates to functionality identification.

After AI/ML functionality identification, it is possible that the UE may report the updates on the applicable AI/ML functionalities. In addition, AI/ML functionality can be activated, deactivated, fallback and switched. Since UE may have one AI/ML model for the functionality, or UE may have multiple AI/ML models for the functionality, it is clear that the AI/ML models can be associated with the AI/ML functionality. Thus, we suggest RAN1 to discuss whether the associated model IDs for an AI/ML functionality will be impacted after the update, activation, deactivation, fallback or switch of the AI/ML functionality.

Proposal 4: RAN1 clarifies whether the associated model IDs for an AI/ML functionality will be impacted after the update, activation, deactivation, fallback or switch of the AI/ML functionality.

CN/OAM/OTT Collection of UE-Sided Model Training Data

One of the study objectives in [1] is “CN/OAM/OTT collection of UE-sided model training data”. For the FS_NR_AIML_Air study use cases (e.g., beam management, positioning accuracy enhancement, CSI feedback enhancement), the corresponding contents of UE data collection related to RAN1 may be the L1 metrics such as RSRP, RSRQ and SINR for difference reference signals (e.g., SSB, PRS and CSI-RS). However, considering RAN2’s discussion captured in [2], as below, only IP packets are transferred and read by CN/OAM/OTT.

The following proposals were discussed in RAN2: - UE collects and directly transfers training data to the Over-The-Top (OTT) server; 1a) OTT (3GPP transparent) 1b) OTT (non-3GPP transparent) - UE collects training data and transfers it to Core Network. Core Network transfers the training data to the OTT server. - UE collects training data and transfers it to OAM. OAM transfers the needed data to the OTT server.

Thus, the gap may be how to encapsulate the data with L1 metrics into an IP packet, which can be achieved via encapsulation and seems to be no RAN1 spec impact.

Proposal 5: No RAN1 spec impact on the study objective “CN/OAM/OTT collection of UE-sided model training data”.

Model Transfer/Delivery

To perform the AI/ML model for air interface, model training phase, model inference phase and model management phase are indispensable. According to [2], no matter functionality-based LCM or model-ID-based LCM, each phase operation (i.e., model training phase, model inference phase and model management phase) may need the information of AI/ML model (e.g., structure of the AI/ML model and/or coefficients used for the AI/ML model). In addition, due to the memory limitation on the UE side, the UE may not be able to store all AI/ML models and the corresponding coefficients to perform the model training, model inference and model management. Therefore, the applicable way to support the UE having memory limitation issues to perform the AI/ML model for air interface is to store AI/ML model(s) at the NW side and transfer to the UE when the UE needs to perform model training, model inference or model management.

Observation 3: Memory limitation may result in the UE being unable to store all AI/ML models to perform the AI/ML model for air interface.

Proposal 6: RAN1 at least supports model transfer from the NW side to the UE side.

Conclusions

In this contribution, we have the following observation and proposal:

Observation 1: Model update is captured in LCM, while functionality update is not included in LCM.

Observation 2: Whether AI/ML models are identified at the network via the model IDs needs to be clarified.

Observation 3: Memory limitation may result in the UE being unable to store all AI/ML models to perform the AI/ML model for air interface.

Proposal 1: RAN1 clarifies the requirement of functionality update and further discusses how to capture the functionality update in LCM.

Proposal 2: RAN1 clarifies whether AI/ML models are identified at the network via the model IDs.

Proposal 3: RAN1 clarifies whether the “model identification” in one of the study objectives, “Necessity and details of model Identification concept and procedure in the context of LCM”, relates to functionality identification.

Proposal 4: RAN1 clarifies whether the associated model IDs for an AI/ML functionality will be impacted after the update, activation, deactivation, fallback or switch of the AI/ML functionality.

Proposal 5: No RAN1 spec impact on the study objective “CN/OAM/OTT collection of UE-sided model training data”.

Proposal 6: RAN1 at least supports model transfer from the NW side to the UE side.

References

- [1] RP-234039, New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm (Moderator)

- [2] TR 38.843 V2.0.0, 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18), 2023-12