3GPP TSG RAN WG1 #116		R1-2400739

Athens, Greece, February 26th – March 1st, 2024

Agenda item:	9.4.2.4

Source:	Samsung

Title:	Considerations for Waveform characteristics of carrier-wave

Document for:	Discussion and decision

- Introduction

During RAN#102, The SID of “Study on solutions for Ambient IoT (Internet of Things) in NR” was approved [1] where the objectives for this is follows:

4 Objective 4.1 Objective of SI or Core part WI or Testing part WI <The general scope is omitted> The following objectives are set, within the General Scope: - Evaluation assumptions - Conclude at least the following aspects of design targets left to WGs in Clause 5 (RAN design targets) of TR 38.848 [RAN1]. - Clause 5.3: Applicable maximum distance target values(s) - Clause 5.6: Refine the definition of latency suitable for use in RAN WGs - Clause 5.8: 2D distribution of devices - Define necessary further evaluation assumptions of deployment scenarios for coverage and coexistence evaluations [RAN1, RAN4] - Identify basic blocks/components of possible Ambient IoT device architectures, taking into account state of the art implementations of low-power low-complexity devices which meet the RAN design target for power consumption and complexity. [RAN1] - Define link budget calculation for coverage, including whether/how to model carrier wave from node(s) inside or outside the connectivity topology. NOTE: Assessment performance of the design targets is within the study of feasibility and necessity of proposals in the following objectives, e.g. by inspection of reference implementations in the field, simulations, analytically. NOTE: strive to minimize evaluation cases in RAN1. - Study necessary and feasible solutions for Ambient IoT as prescribed in the General Scope, including decisions on which functions, procedures, etc. are needed and not needed, and ensuring at least the required functionalities in Section 6.2 of TR 38.848. Study of positioning in Rel-19 is RAN3-led, limited to functionalities which would have no, or minimal, specification impact (note: this does not imply any decision relating to WI creation). Study the feasibility and required functionalities for proximity determination (coordination with SA3 is required for privacy aspects). - RAN1-led: For the Ambient IoT DL and UL: - Frame structure, synchronization and timing, random access - Numerologies, bandwidths, and multiple access - Waveforms and modulations - Channel coding - Downlink channel/signal aspects - Uplink channel/signal aspects - Scheduling and timing relationships - Study necessary characteristics of carrier-wave waveform for a carrier wave provided externally to the Ambient IoT device, including for interference handling at Ambient IoT UL receiver, and at NR basestation. For Topology 2, no difference in physical layer design from Topology 1. <The rest is omitted>

- Characteristics for CW waveform provided externally

Operation band for A-IoT carrier wave

According to the SID [1], A-IoT system operates on FR1 licensed FDD spectrum. Considering the complexity of the device, it may be preferable to transmit the carrier wave to the device using the same frequency band as the tag’s uplink transmission, whether it be in an FDD uplink band or an FDD downlink band. However, regulatory restrictions on a CW node emitting the carrier wave and operational band limitations, such as the maximum EIRP that can be applied to the carrier wave, could make this simple approach challenging. For instance, if the gNB serves as the CW node, regulatory constraints might hinder it from emitting the carrier wave in an FDD uplink band. In this case, for the tag to perform uplink transmission in the FDD uplink band, a frequency shifter within the tag that supports a broad range of frequency shifts from the DL band to UL band may be necessary. Given the capability of the tag, employing such a high-performance frequency shifter might not be feasible. On the other hand, if the UE or a dedicate device serves as the CW node and emits the CW using an FDD uplink band, the maximum EIRP available to the device could be limited to 23 dBm. Consequently, the signal strength of CW may be diminished, leading to a reduction in the energy delivered to the tag. Thus, variable limitations on the carrier wave, such as available resources, band, and the maximum EIRP, are dependent on factors including the type of carrier wave node, the band being utilized, and etc. Therefore, as an initial step, it is necessary to consider which types of carrier wave nodes can be used and in which bands the carrier wave can be transmitted, and then to study the regulations and feasibility related to these aspects.

Proposal 1. Study the feasibility of the following alternatives including regulatory aspects:

- Carrier wave node:

  - Alt. 1: basestation

  - Alt. 2: UE

  - Alt. 3: a dedicated device

- Operation band:

  - Alt. 1: FDD UL band

  - Alt. 2: FDD DL band

- 2.2 Interference handling

Two approaches are being considered for generating the uplink signal at the tag in the A-IoT system: backscatter based and self-generation. The backscatter based approach involves reflecting an externally provided carrier wave to generate signals, which significantly reduces the complexity and power consumption required at the tag. The carrier wave serves as the source of the tag’s uplink signal, which means that the characteristics of the carrier wave waveform are imparted onto the tag’s uplink signal, thereby affecting the quality of it. If the waveform of the carrier wave is altered, it follows that the reflected signal from the tag will also exhibit similar distortion. Additionally, if carrier wave waveform occupies a wider bandwidth, the signal reflected by the tag will likely have a wider bandwidth as well. Figure 1 shows the spectrum of a signal reflected from a tag for a single-tone and multi-tone carrier wave. In the figure, 2 tones are used as carrier wave with 240 kHz frequency gap at 900MHz band. In addition, Manchester code, and ASK modulation are applied with the 60 kbps bit rate and 120 kHz signal bandwidth. If the frequency gap of the carrier waves is an integer multiple of signal bandwidth, interference caused by backscattering signaling of the other tone is about 13dB, which is big enough to abstract the information carried by backscattering signal from the targeted tone. . However, as also can be seen in the figure, this broadens the bandwidth of the reflected signal and makes it possible to expect increased interference accordingly. With more tones as carrier wave, a larger bandwidth will be occupied. Since the bit rate cannot be increased, the spectrum efficiency has significant degradation. Although the performance (e.g., coverage) may be increased by potentially combining potentially at receiver side, whether there will be gain or loss with the same total energy of CW at all frequencies needs to be study and evaluated, considering the impact of channel estimation and channel fading. Therefore, single-tone can be considered as a first start, and when it comes to multi-tone, justification of the needs and benefits need to be justified and more discussion on interference may be also necessary. Note that these also can vary with the modulation scheme at a tag (e.g., OOK, FSK, etc.).

Furthermore, achieving precise selective reflection of specific frequencies would pose a challenge for the backscattering component of the tag due to its inherent low-complexity limitation. The carrier wave should be designed with careful consideration for the tag’s ability to reflect signals. Note that the properties of the tag's uplink signal and the interference levels of the overall system are influenced by the characteristics of the carrier wave waveform. Hence, it is crucial to examine the impact of the carrier wave waveform's characteristics on the tag's uplink signal and determine the essential properties of the carrier wave waveform to reduce interference. Carrier wave also directly cause interference within the system as discussed in [2]. As illustrated in Figure 2 and Figure 3, carrier wave can cause interference to both tag or NR UE (@gNB or @ NR UE) for none co-located nodes for CW and gNB, interference to tag and NR UE interference(@reader/@gNB) for co-located node for CW and reader/gNB. Carrier wave design and study need to consider the potential interference and try to mitigate it.

Proposal 2. Sinusoid signal can be a starting point as the carrier wave waveform. Further study feasibility, necessity, and performance of other waveform and the characteristics (e.g., power spectral density, bandwidth, or etc.) that can minimize interference, taking into account the modulation scheme at a tag (e.g., OOK, FSK, or etc.).

Figure 1. Spectrum of Manchester code

Figure 2 Carrier wave to tag and NR UE interference (@gNB or @ NR UE) for none co-located node for CW and gNB

Figure 3 Carrier wave to tag and NR UE interference(@reader/@gNB) for co-located node for CW and reader/gNB

- 2.3 Others

Taking into account the types of tags, type 1 has a peak power consumption of ~ 1uW, and these tags operate based on backscattering for uplink transmission, which requires the carrier wave to be externally provided. Type 2, on the other hand, have a peak power consumption less than or equal to a few hundred uW and can either generate signals internally or rely on backscattering for uplink transmission. When a type-1 tag performs uplink transmission based on backscattering, it is essential that carrier wave is provided to a tag. If carrier wave is not provided properly at the time the tag intends to transmit, the tag may fail to perform data transmission successfully. In cases where Type 2 tags generate their own signals internally, the carrier wave can still be utilized for the tags, for example, by serving as a source for energy harvesting. In this case, it is worth noting that the carrier wave could potentially act as interference to Type 2 devices when they transmit the uplink signal while generating their own signals.

Given this, it may be necessary to have separate control signals for the carrier wave node. For type 1 tags, this would ensure that the carrier wave is provided at the appropriate timing to match the uplink transmission occasions. For type 2 tags, it would prevent the carrier wave from interfering with the tag’s uplink transmissions. The period, duration or frequency resource information of the carrier wave can be discussed first.

Proposal 3. Study the necessity and feasibility of control signals for the carrier wave node.

.

- Conclusion

In this contribution, we made the following observations and proposals:

Proposal 1. Study the feasibility of the following alternatives including regulatory aspects:

- Carrier wave node:

  - Alt. 1: basestation

  - Alt. 2: UE

  - Alt. 3: a dedicated device

- Operation band:

  - Alt. 1: FDD UL band

  - Alt. 2: FDD DL band

Proposal 2. Sinusoid signal can be a starting point as the carrier wave waveform. Further study feasibility, necessity, and performance of other waveform and the characteristics (e.g., power spectral density, bandwidth, or etc.) that can minimize interference, taking into account the modulation scheme at a tag (e.g., OOK, FSK, or etc.).

Proposal 3. Study the necessity and feasibility of control signals for the carrier wave node.

References:

- RP-234058 “New SID: Study on solutions for Ambient IoT (Internet of Things) in NR”

- R1-2400734 “Considerations for evaluation assumptions and results”