3GPP TSG RAN WG1 #116                              		                                       R1-2401488

Athens, Greece, February 26th – March 1st, 2024

Source:	Moderator (OPPO)

Title:	Summary on Rel-18 STxMP

Agenda Item:	8.1

Document for:	Discussion and Decision

# Introduction

This document summarizes remaining issues on Rel-18 STxMP proposed in company contributions of AI 8.1.

# Issues

A couple of preliminary remarks on the proposals/issues to be treated (hence included in the FL summaries for discussions):

- Proposals to introduce new additional feature or additional enhancement will not be treated.

- TPs to correct obvious typos or align RRC parameter names, they will not be treated and should be proposed to the respective spec editor(s) during the post-RAN1#116 draft CR review process. They are classified as “editorial TPs” hence handled by the spec editors (before the company CR phase starts).

## TP on PUCCH SFN in 38.213

ZTE, vivo and Samsung proposed TP to correct some description on the case when PUCCH with SFN scheme + PUCCH repetition number are configured.

- ZTE proposed the following TP to clarify how to apply the 1st/2nd TCI state on PUCCH with SFN + repetition (some change on top of TP in tdoc, some offline comments).

### 9.2.6 PUCCH repetition procedure <Unchanged parts are omitted> For NPUCCHrepeat>1, - the UE repeats the PUCCH transmission with the UCI over NPUCCHrepeat slots - if the UE is provided multipanelSfnScheme and apply-IndicatedTCIState = 'both', and when the PUCCH resource of the PUCCH transmission is applied with two TCI-State or TCI-UL-State, a repetition of the PUCCH transmission simultaneously uses first and second spatial domain filters corresponding to first and second TCI-State or TCI-UL-State - a repetition of the PUCCH transmission in each of the NPUCCHrepeat slots has a same number of consecutive symbols, as provided by nrofSymbols - a repetition of the PUCCH transmission in each of the NPUCCHrepeat slots has a same first symbol, as provided by startingSymbolIndex if subslotLengthForPUCCH is not provided; otherwise mod(startingSymbolIndex, subslotLengthForPUCCH) <Unchanged parts are omitted>

- Samsung proposed the following TP to correct the condition of mTRP TDM PUCCH repetition:

When a PUCCH resource used for repetitions of a PUCCH transmission by a UE includes - first and second spatial settings, or first and second sets of power control parameters, as described in [11, TS 38.321] and in clauses 7 and 7.2.1, or - first and second TCI-State or TCI-UL-State and the UE is not provided multipanelSfnScheme - if the UE is provided multipanelSfnScheme and apply-IndicatedTCIState = ‘both’, a repetition of the PUCCH transmission uses first and second spatial domain filters corresponding to first and second TCI-State or TCI-UL-State the UE - uses the first and second spatial settings or the first and second indicated TCI-State or TCI-UL-State, or the first and second sets of power control parameters, for first and second repetitions of the PUCCH transmission, respectively, when NPUCCHrepeat=2, - alternates between the first and second spatial settings or between the first and second indicated TCI-State or TCI-UL-State, or between the first and second sets of power control parameters, respectively, per NPUCCHswitch repetitions of the PUCCH transmission, where NPUCCHswitch=1 if mappingPattern = 'cyclicMapping'; else, NPUCCHswitch=2. A UE does not expect a PUCCH that is in response to a DCI format detection to overlap with any other PUCCH that does not satisfy the corresponding timing conditions in clause 9.2.5.

- vivo proposed the following TP to (1) clarify the condition of mTRP TDM PUCCH repetition and (2) clarify the description of PUCCH with SFN + repetition.

9.2.6 PUCCH repetition procedure -------------------------------------------Unchanged parts are omitted------------------------------------------- For NPUCCHrepeat>1, - the UE repeats the PUCCH transmission with the UCI over NPUCCHrepeat slots - if the UE is provided multipanelSfnScheme and apply-IndicatedTCIState = 'both', a repetition of the PUCCH transmission uses first and second spatial domain filters corresponding to first and second TCI-State or TCI-UL-State - a repetition of the PUCCH transmission in each of the NPUCCHrepeat slots has a same number of consecutive symbols, as provided by nrofSymbols - a repetition of the PUCCH transmission in each of the NPUCCHrepeat slots has a same first symbol, as provided by startingSymbolIndex if subslotLengthForPUCCH is not provided; otherwise mod(startingSymbolIndex, subslotLengthForPUCCH) - the UE is configured by interslotFrequencyHopping whether or not to perform frequency hopping for repetitions of the PUCCH transmission in different slots - if the UE is configured to perform frequency hopping for repetitions of a PUCCH transmission across slots and the UE is not provided pucch-DMRS-Bundling = 'enabled' - the UE performs frequency hopping per slot - the UE transmits the PUCCH starting from a first PRB, provided by startingPRB, in slots with even number and starting from a second PRB, provided by secondHopPRB, in slots with odd number. The slot indicated to the UE for the first repetition of the PUCCH transmission has number 0 and each subsequent slot until the UE transmits the PUCCH in NPUCCHrepeat slots is counted regardless of whether or not the UE transmits the PUCCH in the slot - the UE does not expect to be configured to perform frequency hopping for a repetition of the PUCCH transmission within a slot - if the UE is configured to perform frequency hopping for repetitions of a PUCCH transmission across slots and the UE is provided pucch-DMRS-Bundling = 'enabled' - the UE performs frequency hopping per interval of NPUCCHinterval consecutive slots, that start from a slot indicated to the UE and where the UE would transmit a first repetition of the PUCCH, where NPUCCHinterval is the value of pucch-FrequencyHoppingInterval, if provided; otherwise, NPUCCHinterval is the value of pucch-TimeDomainWindowLength - the UE transmits the PUCCH over intervals until the UE transmits the PUCCH in NPUCCHrepeat slots, where the first interval has number 0 and each subsequent interval is counted regardless of whether or not the UE transmits the PUCCH in a slot - the UE transmits the PUCCH starting from a first PRB, provided by startingPRB, in intervals with even number and starting from a second PRB, provided by secondHopPRB, in intervals of frequency hopping intervals with odd number - the UE does not expect to be configured to perform frequency hopping for a repetition of the PUCCH transmission within a slot - if the UE is not configured to perform frequency hopping for repetitions of a PUCCH transmission across slots and the UE is configured to perform frequency hopping for a repetition of the PUCCH transmission within a slot, the frequency hopping pattern between the first PRB and the second PRB is same within each slot -------------------------------------------Unchanged parts are omitted------------------------------------------- When a PUCCH resource used for repetitions of a PUCCH transmission by a UE includes - first and second spatial settings from PUCCH-SpatialRelationInfo, or first and second sets of power control parameters, as described in [11, TS 38.321] and in clauses 7 and 7.2.1, or - first and second spatial settings provided by two indicated TCI-States or TCI-UL-States, and apply-IndicatedTCIState = 'both' without providing multipanelSfnScheme - if the UE is provided multipanelSfnScheme and apply-IndicatedTCIState = 'both', a repetition of the PUCCH transmission uses first and second spatial domain filters corresponding to first and second TCI-State or TCI-UL-State the UE - uses the first and second spatial settings or the first and second indicated TCI-State or TCI-UL-State, or the first and second sets of power control parameters, for first and second repetitions of the PUCCH transmission, respectively, when NPUCCHrepeat=2, - alternates between the first and second spatial settings or between the first and second indicated TCI-State or TCI-UL-State, or between the first and second sets of power control parameters, respectively, per NPUCCHswitch repetitions of the PUCCH transmission, where NPUCCHswitch=1 if mappingPattern = 'cyclicMapping'; else, NPUCCHswitch=2. When a PUCCH resource used for repetitions of a PUCCH transmission by a UE includes - first and second spatial settings provided by two indicated TCI-States or TCI-UL-States, and multipanelSfnScheme and apply-IndicatedTCIState = 'both' the UE uses first and second spatial domain filters corresponding to first and second TCI-State or TCI-UL-State for each repetition of the PUCCH transmission. -------------------------------------------Unchanged parts are omitted-------------------------------------------

Mod:  For clarify the condition of mTRP TDD PUCCH repetition, vivo’s TP and Samsung’s TP propose almost the same change. However, for description on PUCCH with SFN, vivo and ZTE proposed different version of TPs. However, the change proposed by ZTE seems simpler. Therefore, I would propose one TP based on ZTE’s TP and the condition of mTRP TDM PUCCH repetition proposed by vivo and Samsung.

Proposal 1: Adopt the following text proposal for TS 38.213

Reason for change: the current specification does not describe the behavior of PUCCH with SFN scheme and PUCCH repetition clearly. Furthermore, per to the current version of TS 38.213 v18.1.0, the beam application rule for mTRP TDM PUCCH repetition is applied to mTRP SFN PUCCH scheme. This application procedure is incorrect, which shall be corrected.

Summary of change:  Capture the endorsed agreement on Rel-18 STxMP SFN PUCCH and legacy repetition number parameter in a same PUCCH resource at the same time and revise the text to describe the case when 1st and 2nd TCI states are applied to PUCCH with mTRP TDM repetition scheme.

Consequence if not approved:  Specification does not accurately capture the previous agreement of the UE enabled with Rel-18 STxMP SFN PUCCH and legacy repetition number parameter pucch-RepetitionNrofSlots in a same PUCCH resource and the specification does not describe the case when two TCI states are indicated for mTRP PUCCH TDM repetition scheme.

### 9.2.6 PUCCH repetition procedure <Unchanged parts are omitted> For NPUCCHrepeat>1, - the UE repeats the PUCCH transmission with the UCI over NPUCCHrepeat slots - if the UE is provided multipanelSfnScheme and apply-IndicatedTCIState = 'both', and when the PUCCH resource of the PUCCH transmission is applied with two TCI-State or TCI-UL-State, a repetition of the PUCCH transmission simultaneously uses first and second spatial domain filters corresponding to first and second TCI-State or TCI-UL-State - a repetition of the PUCCH transmission in each of the NPUCCHrepeat slots has a same number of consecutive symbols, as provided by nrofSymbols - a repetition of the PUCCH transmission in each of the NPUCCHrepeat slots has a same first symbol, as provided by startingSymbolIndex if subslotLengthForPUCCH is not provided; otherwise mod(startingSymbolIndex, subslotLengthForPUCCH) <Unchanged parts are omitted> When a PUCCH resource used for repetitions of a PUCCH transmission by a UE includes - first and second spatial settings, or first and second sets of power control parameters, as described in [11, TS 38.321] and in clauses 7 and 7.2.1, or - first and second TCI-State or TCI-UL-State, and apply-IndicatedTCIState = 'both' without providing multipanelSfnScheme - if the UE is provided multipanelSfnScheme and apply-IndicatedTCIState = ‘both’, a repetition of the PUCCH transmission uses first and second spatial domain filters corresponding to first and second TCI-State or TCI-UL-State the UE - uses the first and second spatial settings or the first and second indicated TCI-State or TCI-UL-State, or the first and second sets of power control parameters, for first and second repetitions of the PUCCH transmission, respectively, when NPUCCHrepeat=2, <Unchanged parts are omitted>

| Company | Comments |
| --- | --- |
| Mod00 | Please share your views on the issue/proposal |
| QC | Ok with the TP in general, but for the first change, there seems to be some redundancy (although it is not wrong). “apply-IndicatedTCIState = 'both'” and “when the PUCCH resource of the PUCCH transmission is applied with two TCI-State or TCI-UL-State” seem to have the same meaning. |
| ZTE | Support the TP provided by FL. Besides, we can be agreeable with QC to remove the duplication “and when the PUCCH resource of the PUCCH transmission is applied with two TCI-State or TCI-UL-State, ”. |
| Lenovo | General fine but the first change seems unnecessary. |
| Samsung | Support with QC’s elaboration. Agree with ZTE that we can delete the first modification which seems duplicated. |
| CATT | Need more discussion. The details for STxMP SFN and TDD repetition need to be specified. |
| Google | We have one quick question, what is the possible scenario when NW configures apply-IndicatedTCIState = ‘both’, but PUCCH only applies <2 TCI states? |
| LG | Fine in principle but we also have the same view with QC and Lenovo, regarding redundancy of the first change. |
| Xiaomi | ok in principle. |
| Huawei, HiSilicon | The redundant text (“and when the PUCCH resource of the PUCCH transmission is applied with two TCI-State or TCI-UL-State”) needs to be removed as it can be easily misinterpreted that it points to an additional condition for SFN based PUCCH Tx besides UE being provided multipanelSfnScheme and apply-IndicatedTCIState = 'both'. |
| New H3C | Open to discuss |
| vivo | It is clearer to integrate the beam-related procedure for PUCCH repetition, hence prefer our TP. |
| OPPO | Same view with companies that first change is redundancy. |
| Docomo | Same view with QC. |
| Sharp | Same view with QC |
| Mod | Update the TP according to the inputs: basically, removing the first change ### 9.2.6 PUCCH repetition procedure <Unchanged parts are omitted> For NPUCCHrepeat>1, - the UE repeats the PUCCH transmission with the UCI over NPUCCHrepeat slots - if the UE is provided multipanelSfnScheme and apply-IndicatedTCIState = 'both', a repetition of the PUCCH transmission simultaneously uses first and second spatial domain filters corresponding to first and second TCI-State or TCI-UL-State - a repetition of the PUCCH transmission in each of the NPUCCHrepeat slots has a same number of consecutive symbols, as provided by nrofSymbols - a repetition of the PUCCH transmission in each of the NPUCCHrepeat slots has a same first symbol, as provided by startingSymbolIndex if subslotLengthForPUCCH is not provided; otherwise mod(startingSymbolIndex, subslotLengthForPUCCH) <Unchanged parts are omitted> When a PUCCH resource used for repetitions of a PUCCH transmission by a UE includes - first and second spatial settings, or first and second sets of power control parameters, as described in [11, TS 38.321] and in clauses 7 and 7.2.1, or - first and second TCI-State or TCI-UL-State, and apply-IndicatedTCIState = 'both' without providing multipanelSfnScheme - if the UE is provided multipanelSfnScheme and apply-IndicatedTCIState = ‘both’, a repetition of the PUCCH transmission uses first and second spatial domain filters corresponding to first and second TCI-State or TCI-UL-State the UE - uses the first and second spatial settings or the first and second indicated TCI-State or TCI-UL-State, or the first and second sets of power control parameters, for first and second repetitions of the PUCCH transmission, respectively, when NPUCCHrepeat=2, <Unchanged parts are omitted>. |

## TP on precoding for SDM/SFN scheme in 38.211

ZTE proposed the following TP for 38.211 to clarify the mapping between precoders and PUSCH layers in STxMP SDM/SFN scheme.

Reason for change: As specified in TS 38.211, precoding matrix W depends on the number of antenna ports used for PUSCH transmission. Meanwhile, as specified in TS 38.214, for single DCI based STxMP PUSCH in SDM/SFN scheme, precoders indicated by the first and second TPMIs/SRIs are mapped to different PUSCH antenna ports. In this sense, it will lead to ambiguity with respect to the determination of the precoding matrix W.

For example, if both of two precoders indicated by the first and second TPMI are 4x1 matrices (each is given by Tables for the case of 4 antenna ports in TS 38.211), a total of 8 PUSCH antenna ports are used for the PUSCH transmission based on the description in TS 38.214. However, according to the current description in TS 38.211, precoding matrix W is wrongly being that used for 8-Tx PUSCH transmission (which is given by Tables for the case of 8 antenna ports in TS 38.211).

To avoid this ambiguity, it needs to capture in TS 38.211 that: for STxMP PUSCH in SDM scheme, precoders indicated by the first and second TPMIs/SRIs are applied to different layers, i.e., the block of vectors y0i…yv1-1iT and yv1i…yv1+v2-1iT according to the corresponding part in TS 38.214. For STxMP PUSCH in SFN scheme, precoders indicated by the first and second TPMIs/SRIs are applied to same layers, i.e., the block of vector y0i…yυ-1iT, according to the corresponding part in TS 38.214.

Summary of change: Clarifying that the mapping between indicated precoders and PUSCH layers when STxMP PUSCH in SDM/SFN scheme in TS 38.211.

Consequence if not approved: Specification 38.211 has ambiguity on how to apply the precoding of single DCI based STxMP CB/NCB PUSCH transmission in SDM/SFN scheme.

#### 6.3.1.5 Precoding The block of vectors y0i…yυ-1iT shall be precoded according to z(p0)i⋮z(pρ-1)i=Wy(0)i⋮yυ-1i where i=0,1,…,Msymbap-1, Msymbap=Msymblayer. The set of antenna ports p0,…,pρ-1 shall be determined according to the procedure in [6, TS 38.214]. - When the higher layer parameter multipanelScheme is set to 'SDMScheme', and if codepoint "10" of of SRS Resource Set indicator is indicated, the block of vectors y0i…yv1-1iT shall be precoded by the precoder indicated by the first TPMI and the block of vectorsyv1i…yv1+v2-1iT shall be precoded by the precoder indicated by the second TPMI according to the procedure in [6, TS 38.214]; - When the higher layer parameter multipanelScheme is set to 'SFNScheme', and if codepoint "10" of of SRS Resource Set indicator is indicated, the block of vector y0i…yυ-1iT shall be precoded by the precoder indicated by the first TPMI and the precoder indicated by the second TPMI separately according to the procedure in [6, TS 38.214]. For non-codebook-based transmission, the precoding matrix W equals the identity matrix. For codebook-based transmission, the precoding matrix W depends on the number of antenna ports used for the transmission: - for single-layer transmission on a single antenna port, W=1; - for transmissions using 2, or 4 antenna ports, W is given by Tables 6.3.1.5-1 to 6.3.1.5-7; - for transmissions using 8 antenna ports, W is given by Wf(i)=W'i where - the subscripts i and f(i) denote the row of the respective matrix; - f(i) is given by Table 6.3.1.5-8; - the intermediate precoding matrix W' is given by Tables 6.3.1.5-9 to 6.3.1.5-24, 6.3.1.5-29 to 6.3.1.5-36, and 6.3.1.5-39 to 6.3.1.5-47 with 0m×n representing the all-zero matrix with m rows and n columns; - the submatrices Wm,n are given by Tables 6.3.1.5-25 to 6.3.1.5-28 and 6.3.1.5-37 to 6.3.1.5-38. The TPMI index used in the tables above is obtained from the DCI scheduling the uplink transmission or the higher layer parameters according to the procedure in [6, TS 38.214]. When the higher-layer parameter txConfig is not configured, the precoding matrix W=1. <Unchanged parts are omitted>

Proposal 2: Adopt the above text proposal for TS 38.212

| Company | Comments |
| --- | --- |
| Mod00 | Please share your views/inputs on this TP |
| QC | Do not support the TP. The issue has been discussed for multiple meetings already. The descriptions in 38.214 are enough, and we do not see a need for further clarifications. Furthermore, the agreement achieved in RAN1 #115 clearly intended to close this issue by the following note: “Note: No PUSCH precoder modification and/or PUSCH/SRS port re-indexing in the specification is expected. ” |
| ZTE | Support this TP to reach the consistency between TS 38.211 and TS 38.214. Otherwise, it will lead to misunderstand on precoding of STxMP SDM/DFN scheme as elaborated above. Regarding QC’s comment, we do not agree this deviates from the note. First, this TP does not modify anything in terms of PUSCH precoder of STxMP SDM/SFN scheme, which is indeed to keep alignment with the description in TS 38.214. Second, this TP does not touch anything of PUSCH/SRS port re-indexing, due to it is to correctly specify the mapping between indicated precoders and PUSCH layers. @QC, could you please kindly explain how to understand/derive the precoding matrix W of SDM/SFN scheme based on the current description in TS 38.211? For example, when both of two precoders indicated by the first and second TPMI are 4x1 matrices, and a total of 8 PUSCH antenna ports are used for the PUSCH transmission based on the description in TS 38.214. |
| Lenovo | Agree with QC that the description in TS38.214 is enough. |
| Samsung | Support the TP. Without this, based on current equation in TS38.211, it is unclear how to apply two indicated TPMIs for STxMP, which is same view as mentioned by ZTE. |
| CATT | Fine to support. |
| Google | It seems this issue is closed based on previous discussion. |
| LG | Do not support |
| Huawei, HiSilicon | Do not support. This issue has been discussed in multiple meeting and was closed in RAN1 115 as QC pointed out. |
| New H3C | Similar view with HW |
| vivo | We share similar views as QC. Layer mapping behavior has been clearly depicted in TS 38.214 without any ambiguity. |
| OPPO | The description in TS38.214 is enough. |
| Apple | Do not support. There could be no other interpretation of existing 38.214. |
| Docomo | Similar view as HW. |
| Mod | QC, Lenovo, LG, Huawei/HiSilicon, New H3C, vivo, OPPO, Apple and Docomo do not support the TP and they think it is not needed. |

## TP on maxRankSdmDCI-0-2 in 38.213

Huawei/HiSilicon proposed the following TP on CB PUSCH of SDM scheme for TS 38.214:

Reason for change: maxRankSdmDCI-0-2 in TS 38.214-i10 clause 6.1.1.1 is missing for determining the number of layers for the first TPMI.

Summary of change: Add maxRankSdmDCI-0-2 in TS 38.214-i10 clause 6.1.1.1 for determining the number of layers for the first TPMI.

Consequence if not approved: Missing maxRankSdmDCI-0-2 in TS 38.214-i10 clause 6.1.1.1 may cause misunderstanding.

< Start of the text proposal > #### 6.1.1.1 Codebook based UL transmission < Unchanged parts are omitted > When the higher layer parameter multipanelScheme is set to 'SDMScheme' and two SRS resource sets are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to 'codebook', two SRI(s), and two TPMI(s) are given by the DCI fields of two SRS resource indicator and two Precoding information and number of layers in clause 7.3.1.1.2 and 7.3.1.1.3 of [5, TS 38.212] for DCI format 0_1 and 0_2: - When codepoint "10" of SRS Resource Set indicator is indicated, the first TPMI is used to indicate the precoder to be applied over layers {0…v1-1}, where v1 is the number of layers indicated by the first TPMI, that corresponds to the SRS resource selected by the corresponding SRI when multiple SRS resources are configured for the applicable SRS resource set or if single SRS resource is configured for the applicable SRS resource set, and the second TPMI is used to indicate the precoder to be applied over layers {v1…. v2+v1-1}, where v2 is the number of layers indicated by the second TPMI, that corresponds to the SRS resource selected by the corresponding SRI when multiple SRS resources are configured for the applicable SRS resource set or if single SRS resource is configured for the applicable SRS resource set, v1 ≤ maxRankSdm or maxRankSdmDCI-0-2 and v2 ≤ maxRankSdm or maxRankSdmDCI-0-2 and maxRankSdm or maxRankSdmDCI-0-2 are defining the maximum number of layers applied over the first and the second SRS resource sets, separately. < End of the text proposal >

Proposal 3: Adopt the above text proposal for TS 38.214

| Company | Comments |
| --- | --- |
| Mod00 | Please share your views/inputs on this TP |
| QC | Support the TP. |
| ZTE | Support. |
| Lenovo | Support. |
| Samsung | Support. |
| CATT | Ok to support. |
| Google | Support |
| LG | Support. |
| Xiaomi | support |
| Huawei, HiSilicon | Support |
| New H3C | Support |
| Panasonic | Support |
| vivo | Support the TP. |
| OPPO | Support |
| Docomo | Support |
| Sharp | Support |
| Mod | All companies support the TP. No change |

## TP on SRS resource set indicator codepoint 11 in 38.214

Lenovo proposed the following TP for 38.214 to clarify the case of codepoint 11 for SDM scheme:

Reason for change: The interpretation of SRS resource set indicator codepoint “11” for non-codebook based PUSCH transmission with SDM schemes is not captured.

Summary for change: Add the description on the interpretation of SRS resource set indicator codepoint “11” for non-codebook based PUSCH transmission with SDM schemes.

Consequences if not approved: The interpretation of SRS resource set indicator codepoints are not completed.

TS38.214V18.1.0 6.1.1.2 Non-Codebook based UL transmission <Unchanged text is omitted> When the higher layer parameter multipanelScheme is set to 'SDMScheme' and two SRS resource sets are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to 'nonCodebook', SRIs are given by the DCI fields of two SRS resource indicators in clause 7.3.1.1.2 and 7.3.1.1.3 of [5, TS 38.212] for DCI format 0_1 and 0_2. - When codepoint "10" of SRS Resource Set indicator is indicated, the first SRI is used to indicate resource(s) to be associated with layer(s) {0…v1-1}}, where v1 being the number of layers indicated by the first SRI, and the second SRI is used to indicate resource(s) to be associated with layer(s) {v1…. v2+v1-1}, v1 ≤ Lmax and v2 ≤ Lmax where Lmax is defined is defined in clauses 7.3.1.1.2 and 7.3.1.1.3 of [5, TS 38.212]. The UE shall expect that SRS resource(s) indicated by the first SRI and SRS resource(s) indicated by the second SRI are corresponding to different PUSCH antenna ports. - When codepoint "00" or "01" of SRS Resource Set indicator is indicated, the second SRI is reserved, the first SRI is used to indicate resource(s) to be associated with layers {0…v-1}, v ≤ Lmax. - Codepoint "11" of SRS Resource Set indicator is reserved. <Unchanged text is omitted>

Proposal 4: Adopt the above text proposal for TS 38.214

| Company | Comments |
| --- | --- |
| Mod00 | Please share your views on the issue/proposal |
| QC | Support the TP. |
| ZTE | Support. |
| Lenovo | Support |
| Samsung | Support. |
| CATT | Support. |
| Google | OK |
| LG | Support. |
| Xiaomi | support |
| Huawei, HiSilicon | Support |
| New H3C | Support |
| Panasonic | Support |
| vivo | Support the TP. |
| OPPO | Support |
| Docomo | Support |
| Sharp | Support |
| Mod | No change, all companies support the TP. |

## TP on not configuring maxRank of SDM/SFN in 38.212

Google proposed TP for 38.212 to clarify the case when maxRankSfn is not configured. Google proposed that when SFN/SDM is configured but maxRankSfn/maxRankSdm is not provided, the corresponding maximal number of layers per panel reported in UE capability is used:

Reason for change: In current spec, the payload size for TPMI indication is based on the maximum number of layers for sTRP operation and maximum number of layers per panel for SDM/SFN scheme as follows. However, the NW can decide not to configure the maximum number of layers for SDM/SFN. Then, when the maximum number of layers for SDM/SFN is not configured, how to determine the number of bits for the TPMI for the SDM/SFN scheme is unclear.

Summary of change: Clarify that when the maximum number of layers for SDM/SFN is not configured, the number of bits for the TPMI is based on the corresponding UE capability for the serving cell.

Consequences if not approved: The number of bits for TPMI indication is unclear when the NW does not configure the maximum number of layers for SDM/SFN scheme.

##### 7.3.1.1.2 Format 0_1 <omitted text> - Precoding information and number of layers - number of bits determined by the following: - 0 bits if the higher layer parameter txConfig = nonCodeBook; - 0 bits for 1 antenna port and if the higher layer parameter txConfig = codebook; - 4, 5, or 6 bits according to Table 7.3.1.1.2-2 for 4 antenna ports, if txConfig = codebook, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, transform precoder is disabled, and according to the values of higher layer parameters maxRank if multipanelScheme is not configured or max{maxRank, maxRankSfn} if multipanelScheme = sfnScheme or max{maxRank, maxRankSdm} if multipanelScheme = sdmScheme, and codebookSubset; - 4 or 5 bits according to Table 7.3.1.1.2-2A for 4 antenna ports, if txConfig = codebook, ul-FullPowerTransmission = fullpowerMode1, maxRank=2 if multipanelScheme is not configured or max{maxRank, maxRankSfn} = 2 if multipanelScheme = sfnScheme or max{maxRank, maxRankSdm} = 2 if multipanelScheme = sdmScheme, transform precoder is disabled, and according to the values of higher layer parameter codebookSubset; - 4 or 6 bits according to Table 7.3.1.1.2-2B for 4 antenna ports, if txConfig = codebook, ul-FullPowerTransmission = fullpowerMode1, maxRank=3 or 4, transform precoder is disabled, and according to the values of higher layer parameter codebookSubset; - 2, 4, or 5 bits according to Table 7.3.1.1.2-3 for 4 antenna ports, if txConfig = codebook, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, and according to whether transform precoder is enabled or disabled, and maxRank=1 if multipanelScheme is not configured or max{maxRank, maxRankSfn} = 1 if multipanelScheme = sfnScheme or max{maxRank, maxRankSdm} = 1 if multipanelScheme = sdmScheme, and codebookSubset; - 3 or 4 bits according to Table 7.3.1.1.2-3A for 4 antenna ports, if txConfig = codebook, ul-FullPowerTransmission = fullpowerMode1, maxRank=1 if multipanelScheme is not configured or max{maxRank, maxRankSfn} = 1 if multipanelScheme = sfnScheme or max{maxRank, maxRankSdm} = 1 if multipanelScheme = sdmScheme, and according to whether transform precoder is enabled or disabled, and the values of higher layer parameter codebookSubset; - 2 or 4 bits according to Table7.3.1.1.2-4 for 2 antenna ports, if txConfig = codebook, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, transform precoder is disabled, and according to the values of higher layer parameters maxRank if multipanelScheme is not configured or max{maxRank, maxRankSfn} if multipanelScheme = sfnScheme or max{maxRank, maxRankSdm} if multipanelScheme = sdmScheme, and codebookSubset; - 2 bits according to Table 7.3.1.1.2-4A for 2 antenna ports, if txConfig = codebook, ul-FullPowerTransmission = fullpowerMode1, transform precoder is disabled, maxRank=2 if multipanelScheme is not configured or max{maxRank, maxRankSfn} = 2 if multipanelScheme = sfnScheme or max{maxRank, maxRankSdm} = 2 if multipanelScheme = sdmScheme, and codebookSubset=nonCoherent; - 1 or 3 bits according to Table7.3.1.1.2-5 for 2 antenna ports, if txConfig = codebook, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, and according to whether transform precoder is enabled or disabled, and maxRank=1 if multipanelScheme is not configured or max{maxRank, maxRankSfn}=1 if multipanelScheme = sfnScheme or max{maxRank, maxRankSdm}=1 if multipanelScheme = sdmScheme, and codebookSubset; - 2 bits according to Table 7.3.1.1.2-5A for 2 antenna ports, if txConfig = codebook, ul-FullPowerTransmission = fullpowerMode1, maxRank=1 if multipanelScheme is not configured or max{maxRank, maxRankSfn} = 1 if multipanelScheme = sfnScheme or max{maxRank, maxRankSdm} = 1 if multipanelScheme = sdmScheme, and according to whether transform precoder is enabled or disabled, and the values of higher layer parameter codebookSubset; <omitted text> For the higher layer parameter txConfig = codebook, when the Transform precoder indicator field is present, if the bit width of the Precoding information and number of layers field for the case with transform precoder enabled is not equal to that for the case with transform precoder disabled, a number of most significant bits with value set to '0' are inserted to the Precoding information and number of layers field for the case with smaller bit width until the bit width of the Precoding information and number of layers field for the two cases are the same. If multipanelScheme = sfnScheme and maxRankSfn is not configured, the maxRankSfn above is replaced by maxNumberLayerPerSet reported by the UE capability. If multipanelScheme = sdmScheme and maxRankSdm is not configured, the maxRankSdm above is replaced by maxNumberLayerPerPanel reported by the UE capability.

Proposal 5: Adopt the above text proposal for TS 38.212

| Company | Comments |
| --- | --- |
| Mod00 | Please share your views/inputs on this TP |
| QC | Do not support. There is no reason why gNB does not configure this RRC parameter. In addition to the TP being non-essential, it can cause some problems too. As a general principle, a functionality / procedure (e.g., number of bits for a DCI field) should not be a function of UE capability, especially when an RRC parameter can configure this. If a procedure is a function of UE capability, it may result in ambiguities especially for FGs that are per FS or per FSPC. In the past, RAN1/RAN2 had to add RRC configurations specifically to address similar issues, but thankfully, RRC configuration already exists for this case. |
| ZTE | Do not support, we share the same view to QC. Besides, to our understanding, it is error case of “SFN/SDM is configured but maxRankSfn/maxRankSdm is not provided” from the perspective of NW scheduling. In other words, this can be totally avoided/guaranteed by NW. |
| Lenovo | Do not support. This case should be avoided be NW implementation. |
| Samsung | Not support the TP. This is a case like mis-configuration from gNB, which can be avoided. |
| CATT | Not support. No such agreement is achieved. |
| Google | What QC and ZTE mentioned is also one possible solution. Then we should ask RAN2 to modify the RRC code for the two parameters from “need R” into “condM”. |
| LG | Not support. |
| Xiaomi | Not support |
| Huawei, HiSilicon | Not support. We don’t see why gNB configures multipanelScheme = sfn(sdm)Scheme but not maxRankSfn(Sdm). |
| New H3C | Not support |
| vivo | This issue is not essential, plus gNB implementation can preclude the ambiguity. So, the TP is not needed. |
| OPPO | Not support |
| Apple | Do not support (similar view as QC) |
| Sharp | Not support |
| Mod | Majority of the companies do not support the TP |

## TP on SRI indication for SDM/SFN in 38.212

Google proposed the following TP to clarify that, when the RRC parameter of max rank for SDM/SFN is not configured, the number of bits of SRI should be based on UE capability.

Reason for change: In current spec, the payload size for SRI indication is based on the maximum number of layers for sTRP operation and maximum number of layers per panel for SDM/SFN scheme as follows. However, the NW can decide not to configure the maximum number of layers for SDM/SFN. Then, when the maximum number of layers for SDM/SFN is not configured, how to determine the number of bits for the TPMI for the SDM/SFN scheme is unclear.

Summary of change: Clarify that when the maximum number of layers for SDM/SFN is not configured, the number of bits for the SRI is based on the corresponding UE capability for the serving cell.

Consequences if not approved: The number of bits for SRI indication is unclear when the NW does not configure the maximum number of layers for SDM/SFN scheme.

##### 7.3.1.1.2 Format 0_1 <omitted text> - SRS resource indicator -number of bits determined by the following: <omitted text> - if UE supports operation with maxMIMO-Layers and the higher layer parameter maxMIMO-Layers of PUSCH-ServingCellConfig of the serving cell is configured, - Lmax is given by max{maxMIMO-Layers, maxMIMO-LayersforSdm} if multipanelScheme = sdmScheme - Lmax is given by max{maxMIMO-Layers, maxMIMO-LayersforSfn} if multipanelScheme = sfnScheme - Lmax is given by maxMIMO-Layers otherwise - If multipanelScheme = sfnScheme and maxMIMO-LayersforSfn is not configured, the maxMIMO-LayersforSfn above is replaced by maxNumberLayerPerSet reported by the UE capability. If multipanelScheme = sdmScheme and maxMIMO-LayersforSdm is not configured, the maxMIMO-LayersforSdm above is replaced by maxNumberLayerPerPanel reported by the UE capability. - otherwise, Lmax is given by the maximum number of layers for PUSCH supported by the UE for the serving cell for non-codebook based operation.

Proposal 6: Adopt the above text proposal for TS 38.212

| Company | Comments |
| --- | --- |
| Mod00 | Please share your views/inputs on this TP |
| QC | Do not support. See the explanations in Section 2.5. In fact, the reason for introducing the legacy param “maxMIMO-Layers” was exactly due to the issue mentioned above (so that network can configure Lmax, instead of relying only on UE capability to define # of bits in DCI). And the only reason that “otherwise” part in the paragraph still exists is because that was a late change (for backward compatibility reasons). |
| ZTE | Do not support, same reason as elaborated in section 2.5. |
| Lenovo | Do not support. |
| Samsung | Not support the TP. This is a case like mis-configuration from gNB, which can be avoided. |
| CATT | Not support. Similar reason as in section 2.5. |
| Google | What QC and ZTE mentioned is also one possible solution. Then we should ask RAN2 to modify the RRC code for the two parameters from “need R” into “condM”. |
| LG | Do not support. |
| Xiaomi | Not support. |
| Huawei, HiSilicon | Not support |
| New H3C | Do not support |
| vivo | Same views as in section 2.5. |
| OPPO | Not support |
| Apple | Do not support (same view as QC) |
| Sharp | Not support |
| Mod | Majority of the companies do not support the TP |

## TP on data rate computation in 38.214

Google proposed TP to clarify that in 38.214, the data rate per CC should be calculated based on the overlapped PUSCHs in the overlapped symbols when multi-DCI based STxMP PUSCH+PUSCH is configured:

Reason for change: Currently the NW can schedule two PUSCHs overlapped in time domain within a CC. Therefore the maximum data rate per CC should be calculated based on both PUSCHs. Otherwise, the UE would be unnecessarily required to transmit the uplink signal with a higher throughput.

Summary of change: Clarify that data rate per CC should be calculated based on the overlapped PUSCHs in the overlapped symbols.

Consequences if not approved: The data rate per CC for STxMP PUSCH is unclear.

### 6.1.4 Modulation order, redundancy version and transport block size determination <omitted text> For a j-th serving cell, if higher layer parameter processingType2Enabled of PUSCH-ServingCellConfig is configured for the serving cell and set to 'enable', or if at least one IMCS > W for a PUSCH, where W = 28 for MCS tables 5.1.3.1-1 and 5.1.3.1-3, and W = 27 for MCS tables 5.1.3.1-2, 6.1.4.1-1, and 6.1.4.1-2, or if it is an actual repetition for PUSCH repetition Type B, the UE is not required to handle PUSCH transmissions, if the following condition is not satisfied: m=0M-1Vj,mL×Tsμ≤DataRateCC where - L is the number of symbols assigned to the PUSCH(s), which are based on the first symbol of the first PUSCH and the last symbol for the last PUSCH - M is the total number of TB in the PUSCH(s) - Tsμ=10-32μ∙Nsymbslot where is the numerology of the PUSCH - for the m-th TB, Vj,m=C'∙AC - A is the number of bits in the transport block as defined in Clause 6.2.1 [5, TS 38.212] - C is the total number of code blocks for the transport block defined in Clause 5.2.2 [5, TS 38.212] - C' is the number of scheduled code blocks for the transport block as defined in Clause 5.4.2.1 [5, TS 38.212] - DataRateCC [Mbps] is computed as the maximum data rate for a carrier in the frequency band of the serving cell for any signaled band combination and feature set consistent with the serving cell, where the data rate value is given by the formula in Clause 4.1.2 in [13, TS 38.306], including the scaling factor f(i) - each actual repetition for PUSCH repetition type B is treated as one PUSCH.

Proposal 7: Adopt the above text proposal for TS 38.14

| Company | Comments |
| --- | --- |
| Mod00 | Please share your views/inputs on this TP |
| QC | We have some questions about this TP: - Q1: Why this change is needed for PUSCH+PUSCH, but was not needed for Rel-16 PDSCH+PDSCH? - Q2: Is the assumption for this change that multi-DCI can operate with processingType2Enabled? If so, we do not agree with that. In Rel-16 FG for multi-DCI (16-2a), we already have a note: “Note: Processing capability 2 is not supported in any CC if at least one CC is configured with two values of CORESETPoolIndex”. The same restriction is also needed for PUSCH+PUSCH. In that case, do we still need this TP? - Q3: What is the assumption on “DataRateCC” (right-hand side of inequality)? Is it calculated based on max number of layers for sTRP, or based on max number of layers per PUSCH (for PUSCH+PUSCH)? |
| ZTE | We tend to share the same questions from QC. |
| Samsung | We fail to see a necessity of this TP. |
| CATT | Not support. It is our view that this is an enhancement which should not be included at current stage. |
| Google | Q1: This question seems to be out of scope. But we are also open to consider a Rel-16 CR Q2: No. This is not the intention. Q3: DataRateCC should be based on the total maximum number of layers for both PUSCHs. We can also send an LS to RAN2 to modify it. Otherwise, for STxMP, current dataRateCC is actually defined as data rate per panel instead of data rate per CC. |
| LG | Not support |
| Huawei, HiSilicon | Open to further discuss this issue. The assumptions under which DataRateCC is calculated should be aligned with the possible modifications made on the definition of the parameters on the LHS of the inequality. We prefer to resolve the issue without having to send an LS to RAN2. |
| New H3C | Open to discuss |
| vivo | Don’t support this TP. As QC mentioned, processing capability 2 has been precluded in mDCI based mTRP framework. Plus, separate process units were the possible assumption for PUSCH+PUSCH simultaneous transmission in Rel-18. Hence, the processing capability should be counted per PUSCH not PUSCH+PUSCH. |
| OPPO | Open to further discuss the issue. |
| Apple | Open to discuss |
| Mod | Looks like companies have different understanding on whether we should count it on PUSCH+PUSCH or not. My understanding is: if we do count in that way, such a TP is needed. More discussion is needed. |

## TP on multi-DCI based STxMP in 38.214

OPPO suggested that current description on multi-DCI based STxMP PUSCH+PUSCH in 38.214 is not accurate and proposed TP to clarify that.

Reason for change: If the UE is not provided coresetPoolIndex or is provided coresetPoolIndex with value 0 for the first CORESETs, then the coresetPoolIndex should be considered to have value 0. ‘coresetPoolIndex configured with value 0’ in current specification can only refer to the case that coresetPoolIndex is provided with value 0.

Summary of change: Change the description to cover the case where the UE is not provided coresetPoolIndex.

Consequences if not approved: If the UE is not provided coresetPoolIndex for the first CORESETs, the UE behavior may be confusion.

## 6.1 UE procedure for transmitting the physical uplink shared channel #### <omitted part> When a UE is configured with higher layer parameter enableSTx2PofmDCI and PDCCH-Config contains two different values of coresetPoolIndex in ControlResourceSet for the active BWP of a serving cell, - the UE is expected to be configured with two SRS resource sets with usage 'codebook' or 'nonCodeBook' in srs-ResourceSetToAddModList - if the UE is configured to monitor DCI format 0_2 and there is only one SRS resource sets configured by srs-ResourceSetToAddModListDCI-0-2 and associated with usage 'codebook' or 'nonCodeBook', the UE monitors only CORESETs coresetPoolIndex configuredassociated with coresetPoolIndex value 0. #### <omitted part>

Proposal 8: Adopt the above text proposal for TS 38.214

| Company | Comments |
| --- | --- |
| Mod00 | Please share your views/inputs on this TP |
| QC | Ok with the TP. |
| ZTE | Ok. |
| Lenovo | OK |
| Samsung | We are fine with TP. |
| CATT | Ok to support. |
| Google | OK |
| Xiaomi | OK |
| Huawei, HiSilicon | Support |
| New H3C | Support |
| vivo | Support the TP. |
| Sharp | Support |

## TP on NCB PUSCH of SDM/SFN in 38.214

NTT DCOMO proposed the following TP to correct one description for NCB PUSCH in 38.214:

Reason for change: Precoding information and number of layers field is not indicted in non-codebook UL transmission.

Summary of change: Remove “and two Precoding information and number of layers”

Consequences if not approved: it may be misinterpreted that precoding information and number of layers field is indicated for non-codebook based UL transmission.

## 6.1.1.2 Non-Codebook based UL transmission ## *** Unchanged parts are omitted *** When the higher layer parameter multipanelScheme is set to ‘SFNscheme’ and two SRS resource sets are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to ‘nonCodebook’, two SRI(s) are given by the DCI fields of two SRS resource indicator and two Precoding information and number of layers in clause 7.3.1.1.2 and 7.3.1.1.3 of [5, TS 38.212] for DCI format 0_1 and 0_2. ## *** Unchanged parts are omitted ***

Proposal 9: Adopt the above text proposal for TS 38.214

| Company | Comments |
| --- | --- |
| Mod00 | Please share your views/inputs on this TP |
| QC | TP seems to be correct, but “indicator” should also change to “indicators” (right before the crossed red line), which also makes the description aligned with that of SDM in the previous paragraph. |
| ZTE | Support this TP, and QC’s suggestion is also valid. In addition, the following changes (highlighted in this) for NCB PUSCH of SFN scheme can be captured to complete this part. ## 6.1.1.2 Non-Codebook based UL transmission ## *** Unchanged parts are omitted *** When the higher layer parameter multipanelScheme is set to ‘SFNscheme’ and two SRS resource sets are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to ‘nonCodebook’, two SRI(s) are given by the DCI fields of two SRS resource indicator and two Precoding information and number of layers in clause 7.3.1.1.2 and 7.3.1.1.3 of [5, TS 38.212] for DCI format 0_1 and 0_2. - When codepoint “10” of SRS Resource Set indicator is indicated, the first SRI is used to indicate resource(s) to be associated with layer(s) {0…v-1} and the second SRI is used to indicate resource(s) to be associated with layer(s) {0…v-1}, where v ≤ Lmax and where Lmax is defined in clauses 7.3.1.1.2 and 7.3.1.1.3 of [5, TS 38.212]. The UE shall expect that SRS resource(s) indicated by the first SRI and SRS resource(s) indicated by the second SRI are corresponding to different PUSCH antenna ports. - When codepoint “00” or “01” of SRS Resource Set indicator is indicated, the second SRI is reserved, the first SRI is used to indicate resources(s) to be associated with layers {0…v-1}, where v ≤ Lmax and where Lmax is defined in clauses 7.3.1.1.2 and 7.3.1.1.3 of [5, TS 38.212]. When two SRIs are indicated, the UE shall expect that the number of SRS antenna ports associated with two indicated SRIs to be the same. - Codepoint “11” of SRS Resource Set indicator is reserved. When the UE is configured with the higher layer parameter txConfig set to ‘Noncodebook’the higher layer parameter multipanelScheme is set to ‘SDMScheme’ or ‘SFNscheme’ and two SRS resource sets are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to ‘nonCodebook’, the UE is configured with at least one SRS resource. Each of the indicated one or two SRI(s) in slot n is associated with the most recent transmission of SRS resource of associated SRS resource set identified by the SRI, where the SRS resource is prior to the PDCCH carrying the SRI. When two SRS resource sets are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to ‘Noncodebook’, the UE is not expected to be configured with different number of SRS resources in the two SRS resource sets. *** Unchanged parts are omitted *** |
| Lenovo | Fine with the TP with QC’s suggestion. |
| Samsung | Support the TP and 1st part of ZTE’s further proposal above, but for 2nd part of ZTE’s proposal, we don’t see the necessity of such revision since that part is even from Rel-15 spec. |
| CATT | Agree with QC’s modification. |
| Google | OK |
| LG | Support |
| Xiaomi | ok |
| Huawei, HiSilicon | Support |
| New H3C | Support |
| vivo | OK with QC’s revision. |
| OPPO | Support |
| Docomo | Support and support QC’s suggestion. |
| Sharp | Support |
| Mod | Update the TP based on the inputs (1) add “s” to the indicator (2) include the first change propsed by ZTE (3) regarding the 2nd change proposed by ZTE, I think Samsung’s comments is correct, that paragraph is description for all the cases, including sTRP. ## 6.1.1.2 Non-Codebook based UL transmission ## *** Unchanged parts are omitted *** When the higher layer parameter multipanelScheme is set to ‘SFNscheme’ and two SRS resource sets are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to ‘nonCodebook’, two SRI(s) are given by the DCI fields of two SRS resource indicators and two Precoding information and number of layers in clause 7.3.1.1.2 and 7.3.1.1.3 of [5, TS 38.212] for DCI format 0_1 and 0_2. - When codepoint “10” of SRS Resource Set indicator is indicated, the first SRI is used to indicate resource(s) to be associated with layer(s) {0…v-1} and the second SRI is used to indicate resource(s) to be associated with layer(s) {0…v-1}, where v ≤ Lmax and where Lmax is defined in clauses 7.3.1.1.2 and 7.3.1.1.3 of [5, TS 38.212]. The UE shall expect that SRS resource(s) indicated by the first SRI and SRS resource(s) indicated by the second SRI are corresponding to different PUSCH antenna ports. - When codepoint “00” or “01” of SRS Resource Set indicator is indicated, the second SRI is reserved, the first SRI is used to indicate resources(s) to be associated with layers {0…v-1}, where v ≤ Lmax and where Lmax is defined in clauses 7.3.1.1.2 and 7.3.1.1.3 of [5, TS 38.212]. When two SRIs are indicated, the UE shall expect that the number of SRS antenna ports associated with two indicated SRIs to be the same. - Codepoint “11” of SRS Resource Set indicator is reserved. ## *** Unchanged parts are omitted ***. |

## TP on CG-PUSCH of SDM/SFN scheme in 38.214

NTT DOCOMO suggested that the following agreement on CG-PUSCH made in RAN1#114 was not fully captured in specification:

Agreement: Support single-DCI based SDM and SFN scheme in CG-PUSCH within one CG configuration - For Type-1 CG-PUSCH, configure two SRI fields and two TPMI fields in CG configuration. - For Type-1 CG-PUSCH single-DCI based SFN, the UE may assume the association between UL PT-RS port(s) and DM-RS port(s) defined by value 0 in Table 7.3.1.1.2-25 or value "00" in Table 7.3.1.1.1.2-26 described in Clause 7.3.1 of [5, TS38.212]. - Note: it is the same behavior as Type1 CG-PUSCH for sTRP transmission. - For Type-1 CG-PUSCH single-DCI based SDM, the UE may assume the association between UL PT-RS port(s) and DM-RS port(s) defined by value 0 in Table 7.3.1.1.2-25 or value "00" in Table 7.3.1.1.1.2-25a described in Clause 7.3.1 of [5, TS38.212]. - For Type-2 CG-PUSCH, the SRS resource set indicator/SRI fields/TPMI fields in the activation DCI of the SDM/SFN are applied to the activated CG PUSCH.

Thus, they proposed TP to correct that:

Reason for change: RAN1#114’s agreement on supporting STxMP SDM/SFN in CG-PUSCH is not captured in spec.

Summary of change: Capture the agreement on supporting STxMP SDM/SFN for CG-PUSCH in 38.214.

Consequences if not approved: the agreement on CG-PUSCH of SDM/SFN scheme is not captured.

## 6.1.1.1 Codebook based UL transmission ## *** Unchanged parts are omitted *** When the higher layer parameter multipanelScheme is set to 'SDMScheme' and two SRS resource sets are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to 'codebook', two SRI(s), and two TPMI(s) are given by the DCI fields of two SRS resource indicator and two Precoding information and number of layers in clause 7.3.1.1.2 and 7.3.1.1.3 of [5, TS 38.212] for DCI format 0_1 and 0_2 or given by srs-ResourceIndicator, srs-ResourceIndicator2, precodingAndNumberOfLayers, and precodingAndNumberOfLayers2 in configuredGrantConfig : - When codepoint "10" of SRS Resource Set indicator is indicated, or when srs-ResourceIndicator2 and precodingAndNumberOfLayers2 are provided, the first TPMI is used to indicate the precoder to be applied over layers {0…v1-1}, where v1 is the number of layers indicated by the first TPMI, that corresponds to the SRS resource selected by the corresponding SRI when multiple SRS resources are configured for the applicable SRS resource set or if single SRS resource is configured for the applicable SRS resource set, and the second TPMI is used to indicate the precoder to be applied over layers {v1…. v2+v1-1}, where v2 is the number of layers indicated by the second TPMI, that corresponds to the SRS resource selected by the corresponding SRI when multiple SRS resources are configured for the applicable SRS resource set or if single SRS resource is configured for the applicable SRS resource set, v1 ≤ maxRankSdm and v2 ≤ maxRankSdm or maxRankSdmDCI-0-2 and maxRankSdm or maxRankSdmDCI-0-2 are defining the maximum number of layers applied over the first and the second SRS resource sets, separately. - When codepoint "00" or "01" of SRS Resource Set indicator is indicated, the second SRI and second TPMI are reserved, the first TPMI is used to indicate the precoder to be applied over layers {0…v-1}, where v ≤ maxRank, where maxRank is defining the maximum number of layers. - Codepoint "11" of SRS Resource Set indicator is reserved. - For one or two TPMI(s), the transmission precoder is selected from the uplink codebook that has a number of antenna ports equal to the higher layer parameter nrofSRS-Ports in SRS-Config for the indicated SRI(s), as defined in Clause 6.3.1.5 of [4, TS 38.211]. When two TPMIs are indicated, the UE shall expect that the precoder indicated by the first TPMI and the precoder indicated by the second TPMI are mapped to different PUSCH antenna ports. - When two SRIs are indicated, the UE shall expect that the number of SRS antenna ports associated with two indicated SRIs would be the same. When the UE is configured with the higher layer parameter txConfig set to 'codebook', the UE is configured with at least one SRS resource. Each of the indicated one or two SRI(s) in slot n is associated with the most recent transmission of SRS resource of associated SRS resource set identified by the SRI, where the SRS resource is prior to the PDCCH carrying the SRI. When two SRS resource sets are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to 'codebook', the UE is not expected to be configured with different number of SRS resources in the two SRS resource sets. When higher layer parameter multipanelScheme set to 'SFNscheme' and two SRS resource sets are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to 'codebook', two SRI(s), and two TPMI(s) are given by the DCI fields of two SRS resource indicator and two Precoding information and number of layers in clause 7.3.1.1.2 and 7.3.1.1.3 of [5, TS 38.212] for DCI format 0_1 and 0_2 or given by srs-ResourceIndicator, srs-ResourceIndicator2, precodingAndNumberOfLayers, and precodingAndNumberOfLayers2 in configuredGrantConfig. - When codepoint "10" of SRS Resource Set indicator is indicated, or when srs-ResourceIndicator2 and precodingAndNumberOfLayers2 are provided, the first TPMI is used to indicate precoder to be applied over layers {0…v-1} and the second TPMI is used to indicate the precoder to be applied over layers {0…v-1}, where v ≤ maxRankSfn or maxRankSfnDCI-0-2 and maxRankSfn or maxRankSfnDCI-0-2 defining the maximum number of layers applied over the first SRS resource set and over the second SRS resource set separately. - When codepoint "00" or "01" of SRS Resource Set indicator is indicated, the second SRI and second TPMI are reserved, the first TPMI is used to indicate precoder to be applied over layers {0…v-1}, where v ≤ maxRank and where maxRank is defining the maximum number of layers applied over the first SRS resource set or the seoncd SRS resource. - Codepoint "11" of SRS Resource Set indicator is reserved. - For one or two TPMI(s), the transmission precoder is selected from the uplink codebook that has a number of antenna ports equal to nrofSRS-Ports in SRS-Config for the indicated SRI(s), as defined in Clause 6.3.1.5 of [4, TS 38.211]. When two TPMIs are indicated, the UE shall expect that the precoder indicated by the first TPMI and the precoder indicated by the second TPMI are mapped to different PUSCH antenna ports. - When two TPMIs are indicated, the UE shall expect that the number of SRS antenna ports associated with two indicated SRIs to be the same. When the UE is configured with the higher layer parameter txConfig set to 'codebook', the UE is configured with at least one SRS resource. Each of the indicated one or two SRI(s) in slot n is associated with the most recent transmission of SRS resource of associated SRS resource set identified by the SRI, where the SRS resource is prior to the PDCCH carrying the SRI. When two SRS resource sets are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to 'codebook', the UE is not expected to be configured with different number of SRS resources in the two SRS resource sets. *** Unchanged parts are omitted *** ## 6.1.1.2 Non-Codebook based UL transmission ## *** Unchanged parts are omitted *** When the higher layer parameter multipanelScheme is set to 'SDMScheme' and two SRS resource sets are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to 'nonCodebook', SRIs are given by the DCI fields of two SRS resource indicators in clause 7.3.1.1.2 and 7.3.1.1.3 of [5, TS 38.212] for DCI format 0_1 and 0_2 or given by srs-ResourceIndicator, srs-ResourceIndicator2 in configuredGrantConfig. - When codepoint "10" of SRS Resource Set indicator is indicated, or when srs-ResourceIndicator2 is provided, the first SRI is used to indicate resource(s) to be associated with layer(s) {0…v1-1}}, where v1 being the number of layers indicated by the first SRI, and the second SRI is used to indicate resource(s) to be associated with layer(s) {v1…. v2+v1-1}, v1 ≤ Lmax and v2 ≤ Lmax where Lmax is defined is defined in clauses 7.3.1.1.2 and 7.3.1.1.3 of [5, TS 38.212]. The UE shall expect that SRS resource(s) indicated by the first SRI and SRS resource(s) indicated by the second SRI are corresponding to different PUSCH antenna ports. - When codepoint "00" or "01" of SRS Resource Set indicator is indicated, the second SRI is reserved, the first SRI is used to indicate resource(s) to be associated with layers {0…v-1}, v ≤ Lmax. When the higher layer parameter multipanelScheme is set to 'SFNscheme' and two SRS resource sets are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to 'nonCodebook', two SRI(s) are given by the DCI fields of two SRS resource indicator and two Precoding information and number of layers in clause 7.3.1.1.2 and 7.3.1.1.3 of [5, TS 38.212] for DCI format 0_1 and 0_2 or given by srs-ResourceIndicator and srs-ResourceIndicator2 in configuredGrantConfig. - When codepoint "10" of SRS Resource Set indicator is indicated, or when srs-ResourceIndicator2 is provided, the first SRI is used to indicate resource(s) to be associated with layer(s) {0…v-1} and the second SRI is used to indicate resource(s) to be associated with layer(s) {0…v-1}, where v ≤ Lmax and where Lmax is defined in clauses 7.3.1.1.2 and 7.3.1.1.3 of [5, TS 38.212]. The UE shall expect that SRS resource(s) indicated by the first SRI and SRS resource(s) indicated by the second SRI are corresponding to different PUSCH antenna ports. - When codepoint "00" or "01" of SRS Resource Set indicator is indicated, the second SRI is reserved, the first SRI is used to indicate resources(s) to be associated with layers {0…v-1}, where v ≤ Lmax. When two SRIs are indicated, the UE shall expect that the number of SRS antenna ports associated with two indicated SRIs to be the same. ## - Codepoint "11" of SRS Resource Set indicator is reserved. ## *** Unchanged parts are omitted ***

Proposal 10: Adopt the above text proposal for TS 38.214

| Company | Comments |
| --- | --- |
| Mod00 | Please share your views/inputs on this TP |
| QC | The TP generally looks fine to us. |
| ZTE | Support. |
| Lenovo | Fine |
| Samsung | OK. |
| CATT | Fine to support. |
| Google | OK |
| LG | Support |
| Xiaomi | fine |
| Huawei, HiSilicon | OK. But when there are only two parameters, it is better to separate them with “and” not “comma”. Example: When codepoint "10" of SRS Resource Set indicator is indicated, or when srs-ResourceIndicator2, and precodingAndNumberOfLayers2 are provided |
| New H3C | Support |
| Panasonic | Support |
| vivo | OK with the TP. |
| OPPO | Support |
| Docomo | Support |
| Sharp | Support |
| Mod | Update the TP by replacing “,” with ‘and’ per the input from Huawei/HiSilicon. |

## TP on the number of layers of NCB PUSCH in 38.214

CATT suggested that in 38.214, the description of layers corresponding to the 2nd panel for NCB PUSCH is not included. CATT proposed the following TP to correct that.

Reason for change: For association of SRS resources and PUSCH transmission layers for non-codebook based SDM transmission, the interpretation of index v2 is missing.

Summary of change:  Clarify that v2 being the number of layers indicated by the second SRI for non-codebook based SDM UL transmission in section 6.1.1.2 of TS38.214.

Consequences if not approved: The clarification of index v2 by the second SRI for non-codebook based SDM transmission is missing.

6.1.1.2 Non-Codebook based UL transmission <Unrelated part omitted> When the higher layer parameter multipanelScheme is set to ‘SDMScheme’ and two SRS resource sets are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to ‘nonCodebook’, SRIs are given by the DCI fields of two SRS resource indicators in clause 7.3.1.1.2 and 7.3.1.1.3 of [5, TS 38.212] for DCI format 0_1 and 0_2. - When codepoint “10” of SRS Resource Set indicator is indicated, the first SRI is used to indicate resource(s) to be associated with layer(s) {0…v1-1}}, where v1 being the number of layers indicated by the first SRI, and the second SRI is used to indicate resource(s) to be associated with layer(s) {v1…. V2+v1-1}, where v2 being the number of layers indicated by the second SRI, v1 ≤ Lmax and v2 ≤ Lmax where Lmax is defined is defined in clauses 7.3.1.1.2 and 7.3.1.1.3 of [5, TS 38.212]. The UE shall expect that SRS resource(s) indicated by the first SRI and SRS resource(s) indicated by the second SRI are corresponding to different PUSCH antenna ports. - When codepoint “00” or “01” of SRS Resource Set indicator is indicated, the second SRI is reserved, the first SRI is used to indicate resource(s) to be associated with layers {0…v-1}, v ≤ Lmax. <Unrelated part omitted>

Proposal 11: Adopt the above text proposal for TS 38.214

| Company | Comments |
| --- | --- |
| Mod00 | Please share your views/inputs on this TP |
| QC | The TP looks fine to us. |
| ZTE | Support this TP, and QC’s suggestion is also valid. In addition, the following changes (highlighted in this) for NCB PUSCH of SFN scheme can be captured to complete this part. 6.1.1.2 Non-Codebook based UL transmission <Unrelated part omitted> When the higher layer parameter multipanelScheme is set to ‘SDMScheme’ and two SRS resource sets are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to ‘nonCodebook’, SRIs are given by the DCI fields of two SRS resource indicators in clause 7.3.1.1.2 and 7.3.1.1.3 of [5, TS 38.212] for DCI format 0_1 and 0_2. - When codepoint “10” of SRS Resource Set indicator is indicated, the first SRI is used to indicate resource(s) to be associated with layer(s) {0…v1-1}}, where v1 being the number of layers indicated by the first SRI, and the second SRI is used to indicate resource(s) to be associated with layer(s) {v1…. V2+v1-1}, where v2 being the number of layers indicated by the second SRI, v1 ≤ Lmax and v2 ≤ Lmax where Lmax is defined is defined in clauses 7.3.1.1.2 and 7.3.1.1.3 of [5, TS 38.212]. The UE shall expect that SRS resource(s) indicated by the first SRI and SRS resource(s) indicated by the second SRI are corresponding to different PUSCH antenna ports. - When codepoint “00” or “01” of SRS Resource Set indicator is indicated, the second SRI is reserved, the first SRI is used to indicate resource(s) to be associated with layers {0…v-1}, v ≤ Lmax and where Lmax is defined in clauses 7.3.1.1.2 and 7.3.1.1.3 of [5, TS 38.212]. <Unrelated part omitted> |
| Lenovo | Fine with ZTE’s version. |
| Samsung | Support ZTE’s version. |
| CATT | Support the TP and we are fine with ZTE’s version. |
| Google | OK |
| LG | Support |
| Xiaomi | Fine |
| Huawei, HiSilicon | OK with ZTE’s suggestion except the last added part (and where Lmax is defined in clauses 7.3.1.1.2 and 7.3.1.1.3 of [5, TS 38.212].) since it is redundant: The same sentence is already brought in the previous paragraph and there is no need to repeat where L -max is defined. |
| New H3C | Support |
| vivo | Support the TP. |
| OPPO | Support |
| Docomo | Support |
| Sharp | Support |
| Mod | Update the TP according to the inputs. The last added part by ZTE is not included per Huawei/HiSilicon’s comment that it is redundant. 6.1.1.2 Non-Codebook based UL transmission <Unrelated part omitted> When the higher layer parameter multipanelScheme is set to ‘SDMScheme’ and two SRS resource sets are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to ‘nonCodebook’, SRIs are given by the DCI fields of two SRS resource indicators in clause 7.3.1.1.2 and 7.3.1.1.3 of [5, TS 38.212] for DCI format 0_1 and 0_2. - When codepoint “10” of SRS Resource Set indicator is indicated, the first SRI is used to indicate resource(s) to be associated with layer(s) {0…v1-1}}, where v1 being the number of layers indicated by the first SRI, and the second SRI is used to indicate resource(s) to be associated with layer(s) {v1…. V2+v1-1}, where v2 being the number of layers indicated by the second SRI, v1 ≤ Lmax and v2 ≤ Lmax where Lmax is defined is defined in clauses 7.3.1.1.2 and 7.3.1.1.3 of [5, TS 38.212]. The UE shall expect that SRS resource(s) indicated by the first SRI and SRS resource(s) indicated by the second SRI are corresponding to different PUSCH antenna ports. - When codepoint “00” or “01” of SRS Resource Set indicator is indicated, the second SRI is reserved, the first SRI is used to indicate resource(s) to be associated with layers {0…v-1}, v ≤ Lmax. <Unrelated part omitted>. |

# Proposals for Online Discussion

Proposal 3: Adopt the following text proposal for TS 38.214

Reason for change: maxRankSdmDCI-0-2 in TS 38.214-i10 clause 6.1.1.1 is missing for determining the number of layers for the first TPMI.

Summary of change: Add maxRankSdmDCI-0-2 in TS 38.214-i10 clause 6.1.1.1 for determining the number of layers for the first TPMI.

Consequence if not approved: Missing maxRankSdmDCI-0-2 in TS 38.214-i10 clause 6.1.1.1 may cause misunderstanding.

< Start of the text proposal > #### 6.1.1.1 Codebook based UL transmission < Unchanged parts are omitted > When the higher layer parameter multipanelScheme is set to 'SDMScheme' and two SRS resource sets are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to 'codebook', two SRI(s), and two TPMI(s) are given by the DCI fields of two SRS resource indicator and two Precoding information and number of layers in clause 7.3.1.1.2 and 7.3.1.1.3 of [5, TS 38.212] for DCI format 0_1 and 0_2: - When codepoint "10" of SRS Resource Set indicator is indicated, the first TPMI is used to indicate the precoder to be applied over layers {0…v1-1}, where v1 is the number of layers indicated by the first TPMI, that corresponds to the SRS resource selected by the corresponding SRI when multiple SRS resources are configured for the applicable SRS resource set or if single SRS resource is configured for the applicable SRS resource set, and the second TPMI is used to indicate the precoder to be applied over layers {v1…. v2+v1-1}, where v2 is the number of layers indicated by the second TPMI, that corresponds to the SRS resource selected by the corresponding SRI when multiple SRS resources are configured for the applicable SRS resource set or if single SRS resource is configured for the applicable SRS resource set, v1 ≤ maxRankSdm or maxRankSdmDCI-0-2 and v2 ≤ maxRankSdm or maxRankSdmDCI-0-2 and maxRankSdm or maxRankSdmDCI-0-2 are defining the maximum number of layers applied over the first and the second SRS resource sets, separately. < End of the text proposal >

Proposal 4: Adopt the following text proposal for TS 38.214

Reason for change: The interpretation of SRS resource set indicator codepoint “11” for non-codebook based PUSCH transmission with SDM schemes is not captured.

Summary for change: Add the description on the interpretation of SRS resource set indicator codepoint “11” for non-codebook based PUSCH transmission with SDM schemes.

Consequences if not approved: The interpretation of SRS resource set indicator codepoints are not completed.

TS38.214V18.1.0 6.1.1.2 Non-Codebook based UL transmission <Unchanged text is omitted> When the higher layer parameter multipanelScheme is set to 'SDMScheme' and two SRS resource sets are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to 'nonCodebook', SRIs are given by the DCI fields of two SRS resource indicators in clause 7.3.1.1.2 and 7.3.1.1.3 of [5, TS 38.212] for DCI format 0_1 and 0_2. - When codepoint "10" of SRS Resource Set indicator is indicated, the first SRI is used to indicate resource(s) to be associated with layer(s) {0…v1-1}}, where v1 being the number of layers indicated by the first SRI, and the second SRI is used to indicate resource(s) to be associated with layer(s) {v1…. v2+v1-1}, v1 ≤ Lmax and v2 ≤ Lmax where Lmax is defined is defined in clauses 7.3.1.1.2 and 7.3.1.1.3 of [5, TS 38.212]. The UE shall expect that SRS resource(s) indicated by the first SRI and SRS resource(s) indicated by the second SRI are corresponding to different PUSCH antenna ports. - When codepoint "00" or "01" of SRS Resource Set indicator is indicated, the second SRI is reserved, the first SRI is used to indicate resource(s) to be associated with layers {0…v-1}, v ≤ Lmax. - Codepoint "11" of SRS Resource Set indicator is reserved. <Unchanged text is omitted>

Proposal 8: Adopt the following text proposal for TS 38.214

Reason for change: If the UE is not provided coresetPoolIndex or is provided coresetPoolIndex with value 0 for the first CORESETs, then the coresetPoolIndex should be considered to have value 0. ‘coresetPoolIndex configured with value 0’ in current specification can only refer to the case that coresetPoolIndex is provided with value 0.

Summary of change: Change the description to cover the case where the UE is not provided coresetPoolIndex.

Consequences if not approved: If the UE is not provided coresetPoolIndex for the first CORESETs, the UE behavior may be confusion.

## 6.1 UE procedure for transmitting the physical uplink shared channel #### <omitted part> When a UE is configured with higher layer parameter enableSTx2PofmDCI and PDCCH-Config contains two different values of coresetPoolIndex in ControlResourceSet for the active BWP of a serving cell, - the UE is expected to be configured with two SRS resource sets with usage 'codebook' or 'nonCodeBook' in srs-ResourceSetToAddModList - if the UE is configured to monitor DCI format 0_2 and there is only one SRS resource sets configured by srs-ResourceSetToAddModListDCI-0-2 and associated with usage 'codebook' or 'nonCodeBook', the UE monitors only CORESETs coresetPoolIndex configuredassociated with coresetPoolIndex value 0. #### <omitted part>

Proposal 9: Adopt the following text proposal for TS 38.214

Reason for change: Precoding information and number of layers field is not indicted in non-codebook UL transmission. And parameter Lmax is not clearly defined.

Summary of change: Remove “and two Precoding information and number of layers” and add the reference for Lmax definition.

Consequences if not approved: it may be misinterpreted that precoding information and number of layers field is indicated for non-codebook based UL transmission and interpretation on Lmax is not clear.

## 6.1.1.2 Non-Codebook based UL transmission ## *** Unchanged parts are omitted *** When the higher layer parameter multipanelScheme is set to ‘SFNscheme’ and two SRS resource sets are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to ‘nonCodebook’, two SRI(s) are given by the DCI fields of two SRS resource indicators and two Precoding information and number of layers in clause 7.3.1.1.2 and 7.3.1.1.3 of [5, TS 38.212] for DCI format 0_1 and 0_2. - When codepoint “10” of SRS Resource Set indicator is indicated, the first SRI is used to indicate resource(s) to be associated with layer(s) {0…v-1} and the second SRI is used to indicate resource(s) to be associated with layer(s) {0…v-1}, where v ≤ Lmax and where Lmax is defined in clauses 7.3.1.1.2 and 7.3.1.1.3 of [5, TS 38.212]. The UE shall expect that SRS resource(s) indicated by the first SRI and SRS resource(s) indicated by the second SRI are corresponding to different PUSCH antenna ports. - When codepoint “00” or “01” of SRS Resource Set indicator is indicated, the second SRI is reserved, the first SRI is used to indicate resources(s) to be associated with layers {0…v-1}, where v ≤ Lmax and where Lmax is defined in clauses 7.3.1.1.2 and 7.3.1.1.3 of [5, TS 38.212]. When two SRIs are indicated, the UE shall expect that the number of SRS antenna ports associated with two indicated SRIs to be the same. - Codepoint “11” of SRS Resource Set indicator is reserved. ## *** Unchanged parts are omitted ***

Proposal 10: Adopt the following text proposal for TS 38.214

Reason for change: RAN1#114’s agreement on supporting STxMP SDM/SFN in CG-PUSCH is not captured in spec.

Summary of change: Capture the agreement on supporting STxMP SDM/SFN for CG-PUSCH in 38.214.

Consequences if not approved: the agreement on CG-PUSCH of SDM/SFN scheme is not captured.

## 6.1.1.1 Codebook based UL transmission ## *** Unchanged parts are omitted *** When the higher layer parameter multipanelScheme is set to 'SDMScheme' and two SRS resource sets are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to 'codebook', two SRI(s), and two TPMI(s) are given by the DCI fields of two SRS resource indicator and two Precoding information and number of layers in clause 7.3.1.1.2 and 7.3.1.1.3 of [5, TS 38.212] for DCI format 0_1 and 0_2 or given by srs-ResourceIndicator, srs-ResourceIndicator2, precodingAndNumberOfLayers, and precodingAndNumberOfLayers2 in configuredGrantConfig : - When codepoint "10" of SRS Resource Set indicator is indicated, or when srs-ResourceIndicator2 and precodingAndNumberOfLayers2 are provided, the first TPMI is used to indicate the precoder to be applied over layers {0…v1-1}, where v1 is the number of layers indicated by the first TPMI, that corresponds to the SRS resource selected by the corresponding SRI when multiple SRS resources are configured for the applicable SRS resource set or if single SRS resource is configured for the applicable SRS resource set, and the second TPMI is used to indicate the precoder to be applied over layers {v1…. v2+v1-1}, where v2 is the number of layers indicated by the second TPMI, that corresponds to the SRS resource selected by the corresponding SRI when multiple SRS resources are configured for the applicable SRS resource set or if single SRS resource is configured for the applicable SRS resource set, v1 ≤ maxRankSdm and v2 ≤ maxRankSdm or maxRankSdmDCI-0-2 and maxRankSdm or maxRankSdmDCI-0-2 are defining the maximum number of layers applied over the first and the second SRS resource sets, separately. - When codepoint "00" or "01" of SRS Resource Set indicator is indicated, the second SRI and second TPMI are reserved, the first TPMI is used to indicate the precoder to be applied over layers {0…v-1}, where v ≤ maxRank, where maxRank is defining the maximum number of layers. - Codepoint "11" of SRS Resource Set indicator is reserved. - For one or two TPMI(s), the transmission precoder is selected from the uplink codebook that has a number of antenna ports equal to the higher layer parameter nrofSRS-Ports in SRS-Config for the indicated SRI(s), as defined in Clause 6.3.1.5 of [4, TS 38.211]. When two TPMIs are indicated, the UE shall expect that the precoder indicated by the first TPMI and the precoder indicated by the second TPMI are mapped to different PUSCH antenna ports. - When two SRIs are indicated, the UE shall expect that the number of SRS antenna ports associated with two indicated SRIs would be the same. When the UE is configured with the higher layer parameter txConfig set to 'codebook', the UE is configured with at least one SRS resource. Each of the indicated one or two SRI(s) in slot n is associated with the most recent transmission of SRS resource of associated SRS resource set identified by the SRI, where the SRS resource is prior to the PDCCH carrying the SRI. When two SRS resource sets are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to 'codebook', the UE is not expected to be configured with different number of SRS resources in the two SRS resource sets. When higher layer parameter multipanelScheme set to 'SFNscheme' and two SRS resource sets are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to 'codebook', two SRI(s), and two TPMI(s) are given by the DCI fields of two SRS resource indicator and two Precoding information and number of layers in clause 7.3.1.1.2 and 7.3.1.1.3 of [5, TS 38.212] for DCI format 0_1 and 0_2 or given by srs-ResourceIndicator, srs-ResourceIndicator2, precodingAndNumberOfLayers, and precodingAndNumberOfLayers2 in configuredGrantConfig. - When codepoint "10" of SRS Resource Set indicator is indicated, or when srs-ResourceIndicator2 and precodingAndNumberOfLayers2 are provided, the first TPMI is used to indicate precoder to be applied over layers {0…v-1} and the second TPMI is used to indicate the precoder to be applied over layers {0…v-1}, where v ≤ maxRankSfn or maxRankSfnDCI-0-2 and maxRankSfn or maxRankSfnDCI-0-2 defining the maximum number of layers applied over the first SRS resource set and over the second SRS resource set separately. - When codepoint "00" or "01" of SRS Resource Set indicator is indicated, the second SRI and second TPMI are reserved, the first TPMI is used to indicate precoder to be applied over layers {0…v-1}, where v ≤ maxRank and where maxRank is defining the maximum number of layers applied over the first SRS resource set or the seoncd SRS resource. - Codepoint "11" of SRS Resource Set indicator is reserved. - For one or two TPMI(s), the transmission precoder is selected from the uplink codebook that has a number of antenna ports equal to nrofSRS-Ports in SRS-Config for the indicated SRI(s), as defined in Clause 6.3.1.5 of [4, TS 38.211]. When two TPMIs are indicated, the UE shall expect that the precoder indicated by the first TPMI and the precoder indicated by the second TPMI are mapped to different PUSCH antenna ports. - When two TPMIs are indicated, the UE shall expect that the number of SRS antenna ports associated with two indicated SRIs to be the same. When the UE is configured with the higher layer parameter txConfig set to 'codebook', the UE is configured with at least one SRS resource. Each of the indicated one or two SRI(s) in slot n is associated with the most recent transmission of SRS resource of associated SRS resource set identified by the SRI, where the SRS resource is prior to the PDCCH carrying the SRI. When two SRS resource sets are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to 'codebook', the UE is not expected to be configured with different number of SRS resources in the two SRS resource sets. *** Unchanged parts are omitted *** ## 6.1.1.2 Non-Codebook based UL transmission ## *** Unchanged parts are omitted *** When the higher layer parameter multipanelScheme is set to 'SDMScheme' and two SRS resource sets are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to 'nonCodebook', SRIs are given by the DCI fields of two SRS resource indicators in clause 7.3.1.1.2 and 7.3.1.1.3 of [5, TS 38.212] for DCI format 0_1 and 0_2 or given by srs-ResourceIndicator, srs-ResourceIndicator2 in configuredGrantConfig. - When codepoint "10" of SRS Resource Set indicator is indicated, or when srs-ResourceIndicator2 is provided, the first SRI is used to indicate resource(s) to be associated with layer(s) {0…v1-1}}, where v1 being the number of layers indicated by the first SRI, and the second SRI is used to indicate resource(s) to be associated with layer(s) {v1…. v2+v1-1}, v1 ≤ Lmax and v2 ≤ Lmax where Lmax is defined is defined in clauses 7.3.1.1.2 and 7.3.1.1.3 of [5, TS 38.212]. The UE shall expect that SRS resource(s) indicated by the first SRI and SRS resource(s) indicated by the second SRI are corresponding to different PUSCH antenna ports. - When codepoint "00" or "01" of SRS Resource Set indicator is indicated, the second SRI is reserved, the first SRI is used to indicate resource(s) to be associated with layers {0…v-1}, v ≤ Lmax. When the higher layer parameter multipanelScheme is set to 'SFNscheme' and two SRS resource sets are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to 'nonCodebook', two SRI(s) are given by the DCI fields of two SRS resource indicator and two Precoding information and number of layers in clause 7.3.1.1.2 and 7.3.1.1.3 of [5, TS 38.212] for DCI format 0_1 and 0_2 or given by srs-ResourceIndicator and srs-ResourceIndicator2 in configuredGrantConfig. - When codepoint "10" of SRS Resource Set indicator is indicated, or when srs-ResourceIndicator2 is provided, the first SRI is used to indicate resource(s) to be associated with layer(s) {0…v-1} and the second SRI is used to indicate resource(s) to be associated with layer(s) {0…v-1}, where v ≤ Lmax and where Lmax is defined in clauses 7.3.1.1.2 and 7.3.1.1.3 of [5, TS 38.212]. The UE shall expect that SRS resource(s) indicated by the first SRI and SRS resource(s) indicated by the second SRI are corresponding to different PUSCH antenna ports. - When codepoint "00" or "01" of SRS Resource Set indicator is indicated, the second SRI is reserved, the first SRI is used to indicate resources(s) to be associated with layers {0…v-1}, where v ≤ Lmax. When two SRIs are indicated, the UE shall expect that the number of SRS antenna ports associated with two indicated SRIs to be the same. ## - Codepoint "11" of SRS Resource Set indicator is reserved. ## *** Unchanged parts are omitted ***

Proposal 11: Adopt the following text proposal for TS 38.214

Reason for change: For association of SRS resources and PUSCH transmission layers for non-codebook based SDM transmission, the interpretation of index v2 is missing.

Summary of change:  Clarify that v2 being the number of layers indicated by the second SRI for non-codebook based SDM UL transmission in section 6.1.1.2 of TS38.214.

Consequences if not approved: The clarification of index v2 by the second SRI for non-codebook based SDM transmission is missing.

6.1.1.2 Non-Codebook based UL transmission <Unrelated part omitted> When the higher layer parameter multipanelScheme is set to ‘SDMScheme’ and two SRS resource sets are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to ‘nonCodebook’, SRIs are given by the DCI fields of two SRS resource indicators in clause 7.3.1.1.2 and 7.3.1.1.3 of [5, TS 38.212] for DCI format 0_1 and 0_2. - When codepoint “10” of SRS Resource Set indicator is indicated, the first SRI is used to indicate resource(s) to be associated with layer(s) {0…v1-1}}, where v1 being the number of layers indicated by the first SRI, and the second SRI is used to indicate resource(s) to be associated with layer(s) {v1…. V2+v1-1}, where v2 being the number of layers indicated by the second SRI, v1 ≤ Lmax and v2 ≤ Lmax where Lmax is defined is defined in clauses 7.3.1.1.2 and 7.3.1.1.3 of [5, TS 38.212]. The UE shall expect that SRS resource(s) indicated by the first SRI and SRS resource(s) indicated by the second SRI are corresponding to different PUSCH antenna ports. - When codepoint “00” or “01” of SRS Resource Set indicator is indicated, the second SRI is reserved, the first SRI is used to indicate resource(s) to be associated with layers {0…v-1}, v ≤ Lmax. <Unrelated part omitted>

Proposal 1: Adopt the following text proposal for TS 38.213

Reason for change: the current specification does not describe the behavior of PUCCH with SFN scheme and PUCCH repetition clearly. Furthermore, per to the current version of TS 38.213 v18.1.0, the beam application rule for mTRP TDM PUCCH repetition is applied to mTRP SFN PUCCH scheme. This application procedure is incorrect, which shall be corrected.

Summary of change:  Capture the endorsed agreement on Rel-18 STxMP SFN PUCCH and legacy repetition number parameter in a same PUCCH resource at the same time and revise the text to describe the case when 1st and 2nd TCI states are applied to PUCCH with mTRP TDM repetition scheme.

Consequence if not approved:  Specification does not accurately capture the previous agreement of the UE enabled with Rel-18 STxMP SFN PUCCH and legacy repetition number parameter pucch-RepetitionNrofSlots in a same PUCCH resource and the specification does not describe the case when two TCI states are indicated for mTRP PUCCH TDM repetition scheme.

### 9.2.6 PUCCH repetition procedure <Unchanged parts are omitted> For NPUCCHrepeat>1, - the UE repeats the PUCCH transmission with the UCI over NPUCCHrepeat slots - if the UE is provided multipanelSfnScheme and apply-IndicatedTCIState = 'both', a repetition of the PUCCH transmission simultaneously uses first and second spatial domain filters corresponding to first and second TCI-State or TCI-UL-State - a repetition of the PUCCH transmission in each of the NPUCCHrepeat slots has a same number of consecutive symbols, as provided by nrofSymbols - a repetition of the PUCCH transmission in each of the NPUCCHrepeat slots has a same first symbol, as provided by startingSymbolIndex if subslotLengthForPUCCH is not provided; otherwise mod(startingSymbolIndex, subslotLengthForPUCCH) <Unchanged parts are omitted> When a PUCCH resource used for repetitions of a PUCCH transmission by a UE includes - first and second spatial settings, or first and second sets of power control parameters, as described in [11, TS 38.321] and in clauses 7 and 7.2.1, or - first and second TCI-State or TCI-UL-State, and apply-IndicatedTCIState = 'both' without providing multipanelSfnScheme - if the UE is provided multipanelSfnScheme and apply-IndicatedTCIState = ‘both’, a repetition of the PUCCH transmission uses first and second spatial domain filters corresponding to first and second TCI-State or TCI-UL-State the UE - uses the first and second spatial settings or the first and second indicated TCI-State or TCI-UL-State, or the first and second sets of power control parameters, for first and second repetitions of the PUCCH transmission, respectively, when NPUCCHrepeat=2, <Unchanged parts are omitted>

Proposal 7: Adopt the following text proposal for TS 38.14

Reason for change: Currently the NW can schedule two PUSCHs overlapped in time domain within a CC. Therefore the maximum data rate per CC should be calculated based on both PUSCHs. Otherwise, the UE would be unnecessarily required to transmit the uplink signal with a higher throughput.

Summary of change: Clarify that data rate per CC should be calculated based on the overlapped PUSCHs in the overlapped symbols.

Consequences if not approved: The data rate per CC for STxMP PUSCH is unclear.

### 6.1.4 Modulation order, redundancy version and transport block size determination <omitted text> For a j-th serving cell, if higher layer parameter processingType2Enabled of PUSCH-ServingCellConfig is configured for the serving cell and set to 'enable', or if at least one IMCS > W for a PUSCH, where W = 28 for MCS tables 5.1.3.1-1 and 5.1.3.1-3, and W = 27 for MCS tables 5.1.3.1-2, 6.1.4.1-1, and 6.1.4.1-2, or if it is an actual repetition for PUSCH repetition Type B, the UE is not required to handle PUSCH transmissions, if the following condition is not satisfied: m=0M-1Vj,mL×Tsμ≤DataRateCC where - L is the number of symbols assigned to the PUSCH(s), which are based on the first symbol of the first PUSCH and the last symbol for the last PUSCH - M is the total number of TB in the PUSCH(s) - Tsμ=10-32μ∙Nsymbslot where is the numerology of the PUSCH - for the m-th TB, Vj,m=C'∙AC - A is the number of bits in the transport block as defined in Clause 6.2.1 [5, TS 38.212] - C is the total number of code blocks for the transport block defined in Clause 5.2.2 [5, TS 38.212] - C' is the number of scheduled code blocks for the transport block as defined in Clause 5.4.2.1 [5, TS 38.212] - DataRateCC [Mbps] is computed as the maximum data rate for a carrier in the frequency band of the serving cell for any signaled band combination and feature set consistent with the serving cell, where the data rate value is given by the formula in Clause 4.1.2 in [13, TS 38.306], including the scaling factor f(i) - each actual repetition for PUSCH repetition type B is treated as one PUSCH.

# Contributions in RAN1#116

- R1-2400036	Remaining issues on NR MIMO Evolution for Downlink and Uplink	Spreadtrum Communications

- R1-2400102	Maintenance of Rel-18 MIMO	Huawei, HiSilicon

- R1-2400160	Maintenance on Rel-18 CSI enhancements	New H3C Technologies Co., Ltd.

- R1-2400192	Maintenance on NR MIMO Evolution for Downlink and Uplink	Lenovo

- R1-2400217	Maintenance on Rel-18 NR MIMO Evolution	vivo

- R1-2400275	Maintenance on NR MIMO Evolution for Downlink and Uplink	ZTE

- R1-2400309	Maintenance on NR MIMO evolution for downlink and uplink	CMCC

- R1-2400390	Maintenance on NR MIMO Evolution	Google

- R1-2400406	Maintenance on Rel-18 NR MIMO Evolution	CATT

- R1-2400519	Maintenance on NR MIMO evolution	Ericsson

- R1-2400531	Maintenance on NR MIMO Evolution for Downlink and Uplink	xiaomi

- R1-2400578	Text proposals on NR MIMO Evolution for Downlink and Uplink	OPPO

- R1-2400679	Maintenance on NR MIMO Evolution for Downlink and Uplink	Langbo

- R1-2400704	Maintenance issues for Rel-18 NR MIMO	Samsung

- R1-2400754	Maintenance on NR MIMO Evolution for Downlink and Uplink	MediaTek Inc.

- R1-2400762	Discussion on maintenance issues of Rel-18 MIMO	Fujitsu

- R1-2400913	Maintenance on Rel-18 MIMO	LG Electronics

- R1-2400937	Maintenance on NR MIMO Evolution for Downlink and Uplink	Nokia, Nokia Shanghai Bell

- R1-2400995	On Remaining Issue of Uplink Coverage Enhancement	Apple

- R1-2401048	Maintenance on NR MIMO Evolution for Downlin and Uplink	Sharp

- R1-2401091	Maintenance on NR MIMO Evolution for Downlink and Uplink	NTT DOCOMO, INC.

- R1-2401416	Maintenance on NR MIMO Evolution for Downlink and Uplink	Qualcomm Incorporated