3GPP TSG RAN WG1 #116                               		                                       R1-2401489

Athens, Greece, February 26th – March 1st, 2024

Source:	Moderator (OPPO)

Title:	Summary #1 on Rel-19 asymmetric DL sTRP/UL mTRP

Agenda Item:	9.2.4

Document for:	Discussion and Decision

# Introduction

This document summarizes remaining issues proposed in company contributions of AI 9.2.4 for the following objective in Rel-19 WI of NR MIMO Phase 5:

- Specify enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios, assuming intra-band intra-DU non-co-located mTRP scenarios, without changing existing cell definition or defining a new cell (e.g. UL-only cell), assuming the Rel-17/18 unified TCI framework and fully reusing the legacy QCL/UL spatial relation rules, targeting FR1 and FR2 - Two closed-loop PC adjustment states for SRS, both separate from PUSCH; and pathloss offset configurations for pathloss calculation to UL TRP(s), when the pathloss RS is from DL sTRP.

# Issues for Discussions

## #1: Pathloss offset configuration

Table 1-1 summary of pathloss offset configuration

| # | Issue |
| --- | --- |
| 1.1 | UL transmission to UL TRP and Pathloss offset configuration Per the WID of Rel-19 MIMO, pathloss offset configurations for pathloss calculation to UL TRP shall be supported. Companies proposed to study the number of PL offset configurations, quantization method, value range, etc. Samsung proposed to support more than one PL offset configurations to facilitate the deployment of multiple UL TRPs in the asymmetric DL sTRP/UL mTRP system. NTT DOCOMO suggested that the number of PL offset configuration in one CC should be >= 4. Almost all the companies support PUSCH/PUCCH/SRS towards UL TRP and NTT DOCOMO proposed to support PDCCH order RACH to UL TRP for obtain proper TA, vivo proposed that it is necessary to configure PL offset for PRACH transmission, nokia/NSB proposed to discuss whether PRACH towards UL TRP is in scope. Mod: according to the proposals in the contribution, I would suggest to support PUSCH/PUCCH/SRS and PDCCH order RACH to the UL TRP. PDCCH order RACH does have the use case of TA determination for UL TRP. Generally my understanding is we can provide a few PL offset configurations and each PL offset configuration can correspond to one UL TRP. For a UL transmission that is indicated to use one PL offset configuration, the UE shall use the UL PL, which is DL PL – PL offset, in UL power calculation. The UE only need to replace the DL PL (i.e., the PL estimated from DL PL RS) with this “UL PL” but no need to change the power control method specified in 213. Proposal 1.1: In asymmetric DL sTRP/UL mTRP deployment scenarios, - Support PUSCH/PUCCH/SRS and PDCCH order PRACH transmission towards UL TRP. - Support to configure one or more PL offset configurations in one CC for the PUSCH/PUCCH/SRS/PDCCH-order PRACH transmission towards UL TRP. - When the UE is indicated to apply one PL offset configuration on one PUSCH/PUCCH/SRS/PRACH transmission, the UE calculate the UL Tx power as follows: - Reuse the legacy power control method with replacing the PL estimated from DL PL RS with “PL estimated from the DL PL RS – the indicated PL offset value” - FFS: how many PL offset configurations can be configured in one CC; - FFS: the range of PL offset value and the quantization levels of PL offset value |
| 1.2 | How to configure/indicate pathloss offset to PUSCH/PUCCH/SRS One issue is how to configure/indicate PL offset to PUSCH/PUCCH/SRS. Many companies provided views on this issue and most of the companies proposed to associate PL offset in TCI states: - Interdigital: associate one PL offset in TCI state. - Lenovo: configure one PL offset in joint/UL TCI state - vivo: configure PL offset in UL TCI state. - ZTE: configure PL offset in UL TCI state. - Intel: configure a PL offset in UL TCI state. - CATT: indication on association of TCI state and PL offset - NEC: PL offset can be indicated together with the PL RS or UL TCI state for UL only node - Ericsson: one PL offset is associated with UL TCI state and the initial value is 0. - Xiaomi: (1) associate one PL offset with a UL TCI state (2) MAC update/activate PL offset for each activated TCI states. - OPPO: configure PL offset in joint/UL TCI state - Samsung: configure PL offset in TCI state for UL scheduling. - Apple: an IE for an offset value can be introduced under the IE Uplink-powerControl-r17, or, an IE for an offset value can be introduced under P0AlphaSet-r17. - NTT DOCOMO: PL offset is configured in UL TCI state. - Google: RRC indicate initial PL offset and MAC CE can update/indicate the value. - QC: PL offset is associated with UL TCI state. Mod: It looks like most of companies are ok to associate one PL offset configuration with TCI state. Through the TCI state, the PL offset can be indicate to the PUSCH/PUCCH/SRS. For one PUSCH/PUCCH/SRS, if the indicated TCI state is associated with a PL offset, that means the transmission is towards a UL TRP and the UE shall include this PL offset in PC, on the other hand, if the indicated TCI state is not associated with PL offset, that means the transmission is towards a DL/UL TRP. All companies are ok to associate UL TCI state with PL offset. But for joint TCI state, some companies have concerns on supporting joint TCI state for this deployment scenario. And my understanding is we might have difficulty to use joint TCI state for UL TRP even though applying a joint TCI state to DL/UL TRP seems ok. So, we FFS on the association with joint TCI state for the current moment. Proposal 1.2: When the UE is provided with one or more PL offset configurations in one CC, UL TCI state can be associated with one PL offset configuration: - When a UL TCI state associated with a PL offset configuration is applied to PUSCH/PUCCH/SRS, the UE should calculate the Tx power of the PUSCH/PUCCH/SRS transmission with the PL offset configuration associated with this TCI state. - When a UL TCI state not associated with PL offset configuration is applied to PUSCH/PUCCH/SRS, the UE should calculate the Tx power of the PUSCH/PUCCH/SRS transmission with the legacy power control method. FFS: whether to support associating joint TCI state with a PL offset configuration. |
| 1.3 | How to indicate PL offset to PRACH vivo propose to study how to configure and indicate PL offset for PRACH. NTT DOCOMO proposed to support: PDCCH order RACH to UL TRP, and PL offset on PRACH transmission is configured in PRACH-Config, PDCCH can indicate one PL offset. Mod: using the DCI to indicate a PL offset for PDCCH order RACH seems to be reasonable design. Proposal 1.3: When the UE is provided with one or more PL offset configuration in one CC, the PDCCH order DCI format 1_0 can indicate one PL offset configuration to the triggered PRACH transmission and the UE should apply the indicated PL offset value in the calculation of uplink Tx power of the RACH transmission. - FFS enhancement of DCI format 1_0 to indicate one PL offset configuration for the PDCCH order RACH. |
| 1.4 | How to update pathloss offset value Companies proposed to support updating the value of PL offset: - Vivo: MAC CE to update the pathloss offset value. - ZTE, China Telecom: FFS whether/how to dynamically update PL offset values. - NEC: MAC CE to update the PL offset value - Ericsson: PL offset value is indicated in MAC CE. - OPPO: MAC CE or MAC CE+DCI based method to dynamically update the value of PL offset. - Samsung: support use MAC CE to update PL offset - LG proposed to update the PL offset value via MAC CE - QC proposed (1) PL offset can be updated by MAC CE (2) PL offset is not updated but use MAC CE based TPC command to adjust the UL Tx power. Proposal 1.4: Support updating the value of PL offset through MAC CE. - The gNB can send a MAC CE to update the PL offset values of one or more PL offset configurations |
| 1.6 | The configuration of TCI for asymmetric DL sTRP/UL mTRP deployment scenarios We have specified rel-17 unified TCI framework and then rel-18 unified TCI framework for mTRP systems. We supports two TCI state modes: joint TCI state and separate DL/UL TCI states. Regarding what types of TCI state framework can be configured for the asymmetric DL sTRP/UL mTRP deployment scenarios, companies provided various views: - Some companies (Spreadtrum, ZTE, China Telecom, NEC, Ericsson, xiaomi, Samsung, Nokia/NSB) proposed that only separate TCI states can be configured - Lenovo proposed that both joint TCI state mode and separate TCI state mode can be supported. - Intel proposed to support a mixture of joint TCI state and separate DL/UL TCI state. For example, gNB can activation MAC CE to map a pair of joint TCI state and UL TCI state to a DCI codepoint, where the joint TCI state can be used for the DL and UL transmission of DL/UL TRP and the UL TCI state is only applied to the UL TRP. Samsung also proposed to study the case of using joint TCI state and separate TCI state together. - MTK proposed that (1) If Rel-17 unified TCI framework is configured, we only support separate TCI mode (2) If Rel-18 unified TCI framework is configured, we support both joint and separate TCI mode. - Google also proposed that only separate TCI mode is supported if Rel-17 unified TCI framework is configured. - TCL suggested that for the asymmetric DL sTRP/UL mTRP deployment scenarios, neither joint TCI state nor separate DL/UL TCI state are suitable, thus they propose to enhance the TCI state type. Mod: First of all, separate TCI state can work for this scenario and no company object that. For example, when separate TCI mode is configured, the gNB can indicate one pair of DL TCI state and UL TCI state or the gNB can indicate one DL TCI state and two UL TCI states. However, about joint TCI state, companies do have different views. I understand that for the DL/UL TRP, joint TCI state can work but for UL TRP, it is difficult to use a joint TCI state or in other words, does it make sense to indicate a joint TCI state for UL transmission towards UL TRP? Therefore, my suggestion is (1) we can agree supporting separate TCI mode (2) FFS whether and how to include joint TCI state. Proposal 1.6: For the asymmetric DL sTRP/UL mTRP deployment scenarios, separate TCI state mode of unified TCI framework can be configured: - The gNB can indicate one pair of DL TCI state and UL TCI state to the UE. - The gNB can indicate one DL TCI state, two UL TCI states to the UE. FFS whether to support the following TCI state configuration case: - Configure joint TCI state mode - The gNB can indicate one pair of joint TCI state and UL TCI state to the UE. |
| 1.7 | Mechanism to determine the pathloss offset Companies discussed the mechanism the NW to obtain the PL offset: - NEC: PL offset can be calculated at NW by configuring SRS transmissions to different TRPs with the same Tx power or UE reporting information of transmit power - Ericsson: derivation of PL offset value is up to network implementation. - OPPO: NW use SRS to measure the UL pathloss of different TRPs and need to FFS on how to determine the Tx power of such SRS. - Samsung: introduce association between PL RS and PL offset to assist the gNB to obtain the PL offset. - Panasonic proposed to study the mechanism for estimating the PL offset. - QC suggested that (1) existing SRS for BM can be used to determine UL PL for calculating PL offset (2) study mechanism for UL PL determination by using the existing SRS for BM. Mod: It might be worthwhile to study the mechanism for obtaining PL offset and check whether some specification change is needed for that. Proposal 1.7: Study the mechanism for gNB obtaining the value of PL offset for one UL TRP - FFS how to use SRS to obtain the UL pathloss of multiple TRPs. - FFS the configuration and transmission of SRS for gNB to obtain UL pathloss - FFS the mechanism for estimating PL offset. |
| 1.8 | For inter-cell deployment case: CMCC proposed to support asymmetric DL and UL transmission scheme in inter-cell deployment. To facilitate that, CMCC proposed to support SSB of additional PCI and CSI-RS QCLed with SSB of additional PCI as PL RS and support indicating TCI state associating with additional PCI. Proposal 1.8: To facilitate the asymmetric DL sTRP/UL mTRP in inter-cell deployment case, support the followings: - Configuring SSBs of additional PCI and CSI-RS that is QCLed to SSB of additional PCI as PL RS. - Configuring and indicating TCI states associated with additional PCI. |
| 1.9 | Supporting which mTRP system for asymmetric DL sTRP/UL mTRP deployment scenario: Companies discussed which mTRP system can be configured for this deployment scenarios: - NEC proposed to clarify that only single-DCI based system is supported for asymmetric DL sTRP/UL mTRP deployment scenario, - MTK: Enhancement for asymmetric DL STRP/UL MTRP deployment scenarios can be supported under S-DCI based MTRP operation - Samsung: prioritize single-DCI based mTRP for asymmetric DL sTRP/UL mTRP deployment scenarios. - Nokia/NSB proposed to only support single-DCI based separate DL/UL TCI state framework. - Google proposed (1) support UL mDCI and UL sDCI mTRP system and (2) discuss whether to support configuring more than one coresetPoolIndex value - ZTE, China Telecom: at least includes Rel-17 unified TCI for sTRP/ICBM and Rel-18 unified TCI for sDCI based mTRP. Mod: It looks like we can conclude that single-DCI based mTRP can be configured for this deployment scenario but whether we can configure two coresetPoolIndex values needs FFS Proposal 1.9: Single-DCI based system is supported for asymmetric DL sTRP/UL mTRP deployment scenario - FFS: whether support configuring two different coresetPoolIndex values for this deployment scenario |

Table 1-2: Company input for Issues 1.x

| Company | Comments |
| --- | --- |
| Mod00 | Please share your views/inputs on the issues 1.x |
| Samsung | Proposal 1.1: We are generally fine with the proposal. Proposal 1.2: We are fine with the proposal. Proposal 1.3: We are fine with the proposal. Proposal 1.4: We are fine with the proposal. Proposal 1.6: We are generally fine with the proposal. We would like to ask the intention of two sub-bullets: Indication method in the 1st bullet is for Rel-17 Unified TCI framework, and Indication method in the 2nd bullet is for Rel-18 Unified TCI framework extension to multi-TRP? Proposal 1.7: We are fine with the proposal. Proposal 1.8: We are fine with the proposal. Proposal 1.9: We are fine with the proposal |
| CATT | Proposal 1.1: Generally fine with the proposal and is ok with Samsung’s suggestion. Proposal 1.2: Ok to support. Proposal 1.3: Support PRACH needs further discussions. Proposal 1.4: We are open to discuss the signalling of updating the pathloss offsets. Proposal 1.6: Ok to support. Proposal 1.7: Ok to support. Proposal 1.8: Ok to support while inter-cell scenarios need more discussion. Proposal 1.9: Ok to support. |
| Docomo | Proposal 1.1: Support. We think general expression of “UL TRP” is fine to discuss. Proposal 1.2: Support. We support at least 4 PL-offset value to be configured in a CC to support 4 UL TRPs. Proposal 1.3: Support. Proposal 1.4: We are fine, but we think it can be discussed later, because it is beneficial for a scenario of UE’s movement and some companies think it is not beneficial. Proposal 1.6: Support. Some companies discuss joint TCI because the beam correspondence is typically used for DL TRP in the tdocs. However, even for separate TCI state, we can configure SSB/CSI-RS as QCL source RS of a TCI state. Hence, separate TCI works for DL TRP. One problem of joint TCI state is that SRS cannot be configured as QCL source RS, which is not suitable for asymmetric HetNet scenario. Proposal 1.7: Support to study. Proposal 1.8: We are open to discuss. However, we feel the proposal is not aligned with WID. We think it can be discussed later. Proposal 1.9: Support. |
| OPPO | Proposal 1.1: Support in principle. Regarding PL offset configuration, we understand each configuration corresponds to one TRP. To clarify, within each configuration, can more than 1 PL offset values be configured? If not, then we have to consider how to associate PL offset value to UL TCI state. Finally, given other proposals, we think it is necessary to support configuring more than one PL offset configurations for flexibility. Proposal 1.2: Support. Proposal 1.4: Support. Proposal 1.6: We understand the intention is to support deployments of both asymmetric MTRP and single TRP based on separate TCI states. But it seems Rel-18 uTCI framework for MTRP extension has supported such combined TCI state indication. Proposal 1.7: Support to study, otherwise how could NW know which PL offset value(s) to configure and applied. Proposal 1.9: It makes sense to support S-DCI MTRP, since UL-only TRP cannot sends DCI to UE. |
| Fujitsu | Proposal 1.1: The enhancement related with PRACH is out of scope. But we could be open for discussion if majority companies are interested. Proposal 1.2: Generally fine with the proposal. Proposal 1.3: Discussion on this proposal could be postponed. Whether the pathloss offset factor could be applied for PRACH depends on the outcome of Proposal 1.1. Proposal 1.4: Generally fine with the proposal. Proposal 1.5: We think at least the joint TCI state could be applied to the macro site which is capable of both DL and UL operation. Proposal 1.6: Generally fine with the proposal. Proposal 1.7: Ok with the proposal. Proposal 1.8: We think this item should firstly focus on intra-cell case. Proposal 1.9: We don’t see the necessity on the case of configuring two different coresetPoonIndex values. |
| CMCC | Proposal 1.6: Based on Rel-17 unified TCI state, separate TCI state can be used to indicate one pair of DL TCI state and UL TCI state, where the DL TCI state can be used for DL reception from TRP-1, and the UL TCI state can be used for UL transmission to TRP-2. To consider that there may be also UL transmission to TRP-1, such as SRS for antenna switching, this can be realized by indicate the SRS not to follow unified TCI state and indicate TCI state specific for that SRS. Proposal 1.8: HetNet deployment in our network: One solution to improve UL performance is to deploy UL-only TRP, such UL-only TRP would not be capable of any DL transmission that can reduce the hardware cost. However, the cost of a site is dominated by site rental and deployment cost, other than hardware cost. The site rental and deployment cost would be the same for an UL-only TRP as for a normal TRP. In the end, the lower hardware cost may not lead to a significant reduction in total site cost. So, in our network, heterogeneous network has been deployed to improve UL throughput. In HetNet deployment, the Tx power of macro site is larger than pico site, for example, the Tx power of macro site is 53dBm, and the Tx power of indoor pico site is 23dBm-26dBm. It can be seen larger than 20dB DL-Tx-power gaps is existed in HetNet deployment, and this causes severe interference from outdoor macro site towards indoor pico site, especially for the UEs by the window. Up to 33% data rate loss due to interference from outdoor macro site to indoor pico site: The data rate loss of indoor UEs has been observed in current network deployment. When the DL received power of indoor pico site minus outdoor macro site is equal to 0dB, and UE is served by indoor pico site, the data rate loss is about 33%. When the DL received power of indoor pico site minus outdoor macro site is equal to 0dB-5dB, the data rate loss is about 21.4%. If asymmetric DL and UL transmission scheme is introduced, since outdoor macro site has severe interference to indoor pico site, then outdoor macro site can be used for DL transmission, while indoor pico site has better coverage for UL, then indoor pico site can be used for UL reception. The asymmetric DL and UL transmission scheme eliminates the interference from outdoor macro site to indoor pico site. Inter-cell asymmetric DL and UL: In HetNet deployment, indoor pico site and outdoor macro site are deployed at same frequency with different cell ID (Inter-cell). Both indoor pico site and outdoor macro site transmit SSBs with different cell ID. In this case, when asymmetric DL and UL transmission scheme is used, SSBs from indoor pico site can be used as pathloss RS and reference RS for QCL when UE transmits PUCCH/PUSCH/SRS to indoor pico site. In the WID, intra-DU non-co-located mTRP scenarios includes both inter-cell and intra-cell scenario. How to realize the asymmetric DL and UL transmission under inter-cell scenario can be further discussed. Based on our understanding, the simplest way is to configure SSBs of additional PCI and CSI-RS that is QCLed to SSB of additional PCI as PL RS and configure TCI states associated with additional PCI, which have been supported in Rel-17 unified TCI state for inter-cell beam management. Any other solutions are fine for us, as long as the severe data rate loss observed in our network could be solved. Proposal 1.9: In our network, M-TRP has not been deployed, and UEs support of M-TRP has not been widely commercialized, so we prefer to use Rel-17 unified TCI state as a baseline to support asymmetric DL and UL. |
| TCL | Proposal 1.1: Support. Proposal 1.2: Support. Proposal 1.3: Support. Proposal 1.4: Support. Proposal 1.6: We think both joint and separate TCI state are not suitable. When a joint TCI state type is configured, the network configures a DL TCI state list for the UE but does not configure an UL TCI state list. However, as the UL-only TRP cannot transmit DL signals and can only receive UL signals, the UE needs to use an UL TCI state when communicating with the UL-only TRP. So the joint TCI state type is not suitable for this deployment scenario. When a separate TCI state type is configured, the network configures DL TCI state and UL TCI state lists for the UE. However, the separate TCI state type means that UL and DL channels are not reciprocal, even for the DL and UL TRP. Even if UL and DL channels are reciprocal between UE and DL and UL TRP, UE should also perform UL beam management. This results in unnecessary resource and energy consumption. We propose a new unified TCI state type which is a part joint TCI state type. When this TCI state type is configured, the gNB can indicate a DL TCI state and a UL TCI state to the UE. UE can receive DL signals from and transmit UL signals to DL & UL TRP by using the indicated DL TCI state. UE can transmit UL signals to UL-only TRP by using the UL TCI state. Proposal 1.7: Support. Proposal 1.9: Support. |
| NEC | Proposal 1.1: need more discussion on supporting PRACH to node with no DL. Proposal 1.2: generally OK. May be one clarification as follows. - When a UL TCI state associated with a PL offset configuration is applied to PUSCH/PUCCH/SRS, the UE should calculate the Tx power of the PUSCH/PUCCH/SRS transmission with the PL configuration and PL offset configuration associated with this TCI state. Proposal 1.3: need more discussion on supporting PRACH to node with no DL. Proposal 1.4: support. Proposal 1.6: prefer not to have FFS. In addition, we would like to clarify if with two UL TCI states, does it mean that all previous UL MTRP schemes, like TDM, STxMP, are both supported? Or in the topic, we only focus on selecting one UL TRP to transmit. Proposal 1.7: ok. Proposal 1.8: open to it. Proposal 1.9: prefer not to have FFS |
| LG | Proposal 1.1: it is still not clear the difference between PL offset configuration and PL offset value. Proposal 1.2: we are ok with NEC’s clarification. Proposal 1.3: we can further discuss this issue depending on Proposal 1.1 Proposal 1.4: support. Proposal 1.6: For FFS, configuring joint TCI alone cannot be used for asymmetric DL/UL case but joint TCI + UL TCI can be configurable. Proposal 1.7: ok. Proposal 1.9: Support |
| China Telecom | Proposal 1.1: We are fine with the proposal. Proposal 1.2: We are fine with the proposal. The necessity of introducing the joint TCI state with PL offset configuration should be further discussed. Proposal 1.3: The proposal should be further discussed. Proposal 1.4: We are generally fine with the proposal; the details should be further discussed. Proposal 1.6: We are fine with the proposal. And we don’t see the motivation for enhancement on joint TCI state Proposal 1.7: We are generally fine with the proposal, but it can be up to implementation. Proposal 1.8: We think the inter-cell scenario is out of scope of WID. The discussion on inter-cell should be deprioritised. Proposal 1.9: We support the proposal. Whether to support two different coresetPoolIndex can be further discussed, which can also be beneficial for TA enhancement if supported. |
| Xiaomi | Proposal1.1: Proposal1.2: fine in principle. Proposal1.3: fine to discuss. Proposal1.4: fine in principle Proposal1.6: we prefer to discuss the scenarios supported first. We suggest to study with the following FFS added, Proposal 1.6: For the asymmetric DL sTRP/UL mTRP deployment scenarios, separate TCI state mode of unified TCI framework can be configured: - The gNB can indicate one pair of DL TCI state and UL TCI state to the UE. - The gNB can indicate one DL TCI state, two UL TCI states to the UE. - FFS: Whether these TCI states are associated with 2 TRPs or 3 TRPs. FFS whether to support the following TCI state configuration case: - Configure joint TCI state mode - The gNB can indicate one pair of joint TCI state and UL TCI state to the UE. Proposal1.7: support to study. Proposal1.8: open to discuss Proposal1.9:fine. |
| Ericsson | Proposal 1.1 Bullet 1: PRACH transmission triggered by PDCCH order should be FFS, since there is a need to discuss how to determine PRACH direction towards UL-only TRP without DL RS. Bullet 2: This text should not imply that each UL signal has its own pathloss configuration. A note can be added: this does not imply that each UL signal has its own pathloss offset configuration. Bullet 3: Whether it should be “+” or “-” should be discussed in RAN1. At this stage, it could be sufficient to state: PL estimated from an DL PL RS and a NW indicated pathloss offset Proposal 1.2 First, We prefer that pathloss offset configuration is associated to an UL TCI state; second, we think “PL offset configuration” is not clear and need more explanation. Finally, clarification on “the legacy power control method” is needed. We propose the following modification: When the UE is provided with one or more PL offset configurations, in one CC, each PL offset configuration is associated to one UL TCI state: - When a PL offset configuration is associated to an UL TCI state, the UE should calculate the TX power for PUSCH/PUCCH/SRS transmission including the PL offset, if the UL TCI state is indicated - When an UL TCI state does not include a PL offset configuration, the UE should calculate the TX power for PUSCH/PUCCH/SRS transmission assuming the pathloss of the UL/DL TRP, if the UL TCI state is indicated Proposal 1.3 We think the companies should provide their view on if UE can transmit PRACH to the UL only TRP. Hence we support this proposal only if the main bullet is FFS Proposal 1.4 We support the main bullet, but want to add a sub-bullet: FFS: further study the need of dynamic indication via DCI for FR2. Proposal 1.5 seems missing? Proposal 1.6 support Proposal 1.7 We do not support. Derivation of pathloss offset can use SRS, but is not limited to SRS-based methods. How to estimate pathloss offset is up to NW implementation. Proposal 1.8 We do not support the current formulation. We can be open if the following clarifications: - How is this proposal related to pathloss offset estimation? - What is the scope of inter-cell with respect to intra-band and intra-DU Proposal 1.9 support |
| vivo | Proposal 1.1: support Proposal 1.2: support in principle. Regarding the FFS, the TCI of UL-only TRP cannot be joint TCI state because of the QCL type D source restriction of UL RS. So, there is no need to discuss PL offset configuration for joint TCI state. Proposal 1.3: except for the indication of PL offset, the configuration of PL offset of PRACH transmission should also be discussed. Proposal 1.4: if PL offset is configured per UL TCI, it can be updated according to the TCI state switch. So whether the TCI state activation MAC CE can update the PL offset configurations can be FFS. So we propose to modify the proposal: Proposal 1.4: Support updating the value of PL offset through MAC CE. - The gNB can send a MAC CE to update the PL offset values of one or more PL offset configurations - FFS: whether the TCI state activation MAC CE can realize the update of PL configuration through update the TCI state Proposal 1.6: support the second sub-bullet of the main bullet. The first bullet seems to be the TCI state indication for single TRP transmission which is not the target scenario. Proposal 1.7: OK to discuss. Proposal 1.8: We are wondering why we need this proposal. The first sub-bullet has already supported by configuring PCI for PL-RS in current specification. Regarding the second sub-bullet, even in inter-cell deployment scenario, the UL-only TRP should be deployed in serving cell so no need to configure PCI for the TCI state. Since we have already restricted no UL-cell， which means no UL RS for UL measurement will be configured. Proposal 1.9: Support in principle. |
| Lenovo | Proposal 1.1: We are fine with the proposal except for PDCCH order PRACH. For the latter we need discuss whether separate TA is within the scope of the WID. Proposal 1.2: Support. Proposal 1.3: We need to discuss whether separate TA is within the scope of the WID before discussing PDCCH ordered PRACH. Proposal 1.4: Support. Proposal 1.6: Support. In addition to separate DL and UL TCI states, joint TCI can work for FR1 and shall not be excluded. Proposal 1.7: We think it is up to gNB implementation and there is no need to study. Proposal 1.8: Do not support. Since UL-only TRP is not visible to the UE, it can always be configured as an intra-cell TRP. Proposal 1.9: Support this proposal. Only sDCI shall be supported. |
| MediaTek | Proposal 1.1: We prefer to further study PRACH transmission. It’s still unclear for us how to determine DL reference timing and UL Tx beam with SSB from UL TRP. Proposal 1.2: Okay Proposal 1.3: We prefer to further study PRACH transmission. It’s still unclear for us how to determine DL reference timing and UL Tx beam with SSB from UL TRP. Proposal 1.4: Prefer to discuss later after RAN1 has more conclusions on how to configure/indicate PL offset configuration. Proposal 1.6: First, we think any combination of TCI state indication under separate TCI mode can be supported. Thus, the sub-bullets are not needed. Regarding the 2nd FFS sub-bullet, it should be removed since Rel-17/18 unified TCI framework cannot support such indication. We think enhancement to unified TCI framework would be out-of-scope. Proposal 1.6: For the asymmetric DL sTRP/UL mTRP deployment scenarios, separate TCI state mode of unified TCI framework can be configured: - The gNB can indicate one pair of DL TCI state and UL TCI state to the UE. - The gNB can indicate one DL TCI state, two UL TCI states to the UE. FFS whether to support the following TCI state configuration case: - Configure joint TCI state mode - The gNB can indicate one pair of joint TCI state and UL TCI state to the UE. Proposal 1.7: It should be up to NW implementation. Proposal 1.8: Okay Proposal 1.9: Okay |
| Sony | Proposal 1.1 : support. Proposal 1.2 : support. Proposal 1.4 : support. Proposal 1.6 : support. Proposal 1.7 : support. Proposal 1.8 : Justification for inter-cell use case should be identified before the discussion. Proposal 1.9 : support. |
| Intel | Proposal 1.1: Generally fine, slightly prefer to have PRACH case as FFS. Proposal 1.2: Support. Proposal 1.3: Prefer as FFS. Proposal 1.4: Prefer as FFS. Proposal 1.6: Support. Proposal 1.7:Not fully necessary Proposal 1.8: Prefer as FFS Proposal 1.9: Support. |

## #2: Closed-loop PC for SRS

Table 2-1 summary of closed-loop PC for SRS

| # | Issue |
| --- | --- |
| 2.1 | Configure two closed-loop PC (CLPC) adjustment states for SRS: Per the WID of Rel-19 MIMO, two closed-loop PC adjustment states for SRS, both separate from that of the PUSCH, shall be specified. And in the contributions, almost all the companies proposed to support configuring two closed-loop PC adjustment states for SRS. Proposal 2.1: To facilitate the asymmetric DL sTRP/UL mTRP deployment scenarios, support configuring two closed-loop PC adjustment states for SRS in one CC, both of which are separate from that of the PUSCH. |
| 2.2 | How to indicate one CLPC adjustment states to SRS: The second question we need resolve is how to associate/indicate one CLPC adjustment state to the SRS. The views provided in contribution are: - Huawei/HiSilicon: consider two options: per SRS usage or per SRS set. - Lenovo proposed: add one parameter in P0AlphaSet-r17 in TCI state to indicate one of these two SRS CLPC adjustment states. - vivo: when srs-PowerControlAdjustmentStates is set to 'separateClosedLoop', closedLoopIndex-r17 indicates one of the SRS CLPC adjustment states. - ZTE, China Telecom: in the configuration of SRS resource set, introduce the indication of 2nd SRS CLPC adjustment state in the higher layer parameter srs-PowerControlAdjustmentStates. - CATT: For SRS with TCI state, follow the CL index associated with the TCI state. For SRS not with TCI state, configure CL index in SRS or SRS set. - MTK: If srs-PowerControlAdjustmentStates is set to 'twoSeparateClosedLoop' in an SRS resource set, closedLoopIndex-r17 in TCI state indicate one of the SRS CLPC adjustment states. - OPPO: consider two options (1) directly associate CLI for SRS, (2) associate CLI with TCI state. - Apple: introduce one new value to srs-PowerControlAdjustmentStates to indicate the 2nd SRS CLPC adjustment state. - QC: srs-PowerControlAdjustmentStates in SRS set to indicate the 1st or 2nd SRS CLPC adjustment state. - Interdigital: associate one SRS CLPC adjustment state through TCI state. To summarize, the options provided by the companies can fall into the following Alts: - Alt1: Add one value to srs-PowerControlAdjustmentStates in SRS resource set to indicate the 2nd SRS CLPC adjustment state - Alt2: When srs-PowerControlAdjustmentStates is set to 'separateClosedLoop', closedLoopIndex-r17 in the TCI state indicates one of the SRS CLPC adjustment states: value i0 indicates the 1st SRS CLPC adjustment state and value i1 indicates the 2nd SRS CLPC adjustment state; - Alt3: Add parameter in P0AlphaSet-r17 of TCI state to indicate one of those two SRS CLPC adjustment states - Alt4: the 2nd SRS CLPC adjustment state is configured to one SRS usage and the 1st SRS CLPC adjustment state is indicated to one SRS resource set per the legacy specification. Mod: We need to downselect one method for indicating one of the SRS CLPC adjustment state to the SRS: Proposal 2.2: When one UE is configured with two SRS CLPC adjustment states, regarding how to indicate one SRS CLPC adjustment state to SRS, down-select one from the following Alts: - Alt1: Introduce one new value to the parameter srs-PowerControlAdjustmentStates in SRS resource set and this value indicate the 2nd SRS CLPC adjustment state - Alt2: When the parameter srs-PowerControlAdjustmentStates is set to 'separateClosedLoop', closedLoopIndex-r17 in the TCI state indicates one of the SRS CLPC adjustment states: - value i0 indicates the 1st SRS CLPC adjustment state and - value i1 indicates the 2nd SRS CLPC adjustment state; - Alt3: Add one extra parameter in P0AlphaSet-r17 of TCI state to indicate one of those two SRS CLPC adjustment states - Alt4: the 2nd SRS CLPC adjustment state is configured to one SRS usage and all the SRS set of that SRS usage is associated with the 2nd SRS CLPC adjustment state, and the 1st SRS CLPC adjustment state is indicated to one SRS resource set per the legacy specification. |
| 2.3 | Use DCI format 2_3 to indicate TPC commands for these two SRS closed-loop PC adjustment states: In legacy specification, DCI 2_3 is used to indicate TPC command for the SRS CLPC adjustment state. In Rel-19, we introduce one additional CLPC adjustment state. The question is how to indicate TPC commands for both SRS CLPC adjustment states. Companies proposed to enhance DCI 2_3 to include the TPC command for additional 2nd SRS CLPC adjustment state. There are a few different options on how to enhance DCI format 2_3: - Alt1: Add one additional TPC command for one CC in DCI 2_3, the first TCP command is associated with the first SRS CLPC adjustment state and the second TPC command is associated with the second SRS CLPC adjustment state. - Spreadtrum, Lenovo, vivo, CATT, Ericsson, Xiaomi, NTT DOCOMO, [QC] - Alt2: Introduce one 1-bit closed loop indicator field for each TPC command in DCI format 2_3 - Lenovo, vivo, ZTE, China Telecom, CMCC, CATT, MTK, Samsung, Apple, NTT DOCOMO, OPPO - Alt3: use different TPC-SRS-RNTIs: DCI 2_3 with CRC scrambled with the first TPC-SRS-RNTI and the second TPC-SRS-RNTI indicates the TPC command for the first and second SRS CLPC adjustment state, respectively. - Lenovo, - Alt4: Implicit method: DCI 2_3 in even frame is for the first SRS CLPC adjustment state and DCI 2_3 in odd frame is for the second SRS CLPC adjustment state - China Telecom, ZTE, NTT DOCOMO, DCI format 2_3 has two different formats: srs-TPC-PDCCH-Group = typeA and typeB. Regarding which format of DCI format 2_3 shall be used to indicate TPC command for two SRS CLPC adjustment states, here are the views from companies - Support DCI 2_3 of both typeA and typeB: - Spreadtrum, , Xiaomi, OPPO, Apple, NTT DOCOMO, - Only typeB, FFS on typeA: - QC Mod: All the companies are ok to enhance DCI format 2_3 for the function of two SRS CLPC adjustment states and we need to down-select from those Alts. Regarding typeA vs typeB, more companies proposed to support both. I would suggest to do the enhancement for both typeA and typeB. Proposal 2.3: If a UE is configured two SRS CLPC adjustment states, enhance DCI format 2_3 with higher layer parameter srs-TPC-PDCCH-Group = typeA or typeB to indicate the TPC command for these two SRS CLPC adjustment states and down-select from one from the following Alts for the enhancement of DCI format 2_3: - Alt1: In DCI format 2_3, add one additional TPC command for each CC configured with two SRS CLPC adjustment states, - the first TCP command is associated with the first SRS CLPC adjustment state and the second TPC command is associated with the second SRS CLPC adjustment state. - Alt2: Introduce one 1-bit closed-loop-indicator field for each TPC command in DCI format 2_3 - This 1-bit closed-loop-indicator indicates the first SRS CLPC adjustment state or the second SRS CLPC adjustment state. - Alt3: use two different TPC-SRS-RNTIs for DCI format 2_3: - DCI format 2_3 with CRC scrambled with the first TPC-SRS-RNTI and the second TPC-SRS-RNTI indicates the TPC command for the first and second SRS CLPC adjustment state, respectively. - Alt4: Implicit method: - DCI format 2_3 sent in even frame indicates the TPC command for the first SRS CLPC adjustment state and DCI format 2_3 sent in odd frame indicates the TPC command for the second SRS CLPC adjustment state |
| 2.4 | Using DCI 2_3 vs configuring SRS-CarrierSwitching Legacy specification seems to suggest that the DCI 2_3 can be used to indicate TPC command for SRS CLPC adjustment state only if the UE is configured with high layer parameter SRS-CarrierSwitching. Companies suggested to remove this association when the UE is configured with two SRS CLPC adjustment states: - ZTE, China Telecom, NTT DOCOMO, MTK Mod: My understanding is the current spec does tie the DCI format 2_3 with SRS-CarrierSwitching. For the asymmetric DL sTRP/UL mTRP, the feature of two SRS CLPC adjustment states should not be related with SRS carrier switching. In 331, the RRC parameters are configured as follows: - the IE SRS-CarrierSwitching contains the parameter srs-TPC-PDCCH-Group which can take the value of typeA or TypeB, which determines field definition of each block in DCI format 2_3. - The IE SRS-CarrierSwitching also contains the higher layer parameter cc-IndexInOneCC-set, which is needed for DCI format 2_3 with “typeA”. Thus, to untie SRS carrier switching with DCI format 2_3 used to indicate TPC commands for two SRS CLPC adjustment states, my thinking about what we can do is: - The DCI format 2_3 with ‘typeA’ still needs the configuration of SRS-CarrierSwitching because the configuration of cc-IndexInOneCC-set is still needed for ‘typeA’. - But the DCI format 2_3 with ‘typeB’ can be used without configuration of SRS-CarrierSwitching (of course we need to add some new clariftion). Based on this understanding, here is my proposal: Proposal 2.4: When the UE is provided with two SRS CLPC adjustment states, the DCI format 2_3 can be used to indicate TPC command for the SRS CLPC adjustment state, regardless of whether the higher layer parameter SRS-CarrierSwitching is configured or not. - When SRS-CarrierSwitching is not configured, the UE can assume each block in DCI format 2_3 follows the field definition corresponding to layer parameter srs-TPC-PDCCH-Group = typeB. |
| 2.5 | Add extra delta value for SRS closed-loop power control Here is the allowed delta value for TPC in current spec: TPC Command Field; Accumulated δPUSCH,b,f,c or δSRS,b,f,c [dB]; Absolute δPUSCH,b,f,c or δSRS,b,f,c [dB]. 0; -1; -4. 1; 0; -1. 2; 1; 1. 3; 3; 4. Companies (Sony, OPPO) proposed to extend the TPC command step size values for the asymmetric DL sTRP/UL mTRP deployment scenarios: - Sony proposed to extend the range of TPC command. - OPPO suggested that to facilitate the asymmetric DL sTRP/UL mTRP deployment scenarios, we need to introduce new delta values (bigger values) for TPC so that the system can adjust the UL Tx power faster. Mod: The Tx power to UL TRP is calculated based on the DL RS from the DL/UL TRP. We might rely more on the closed loop to refine the Tx power. Thus, introducing new step size (maybe larger) would assist the system to correct the UL PC more efficiently. Proposal 2.5: To facilitate the asymmetric DL sTRP/UL mTRP deployment scenarios, support to introduce new step sizes for TPC command. - FFS the candidate value for new step sizes. |

Table 2-2: Company input for Issues 2.x

| Company | Comments |
| --- | --- |
| Mod00 | Please share your views/inputs on the issues 2.x |
| Samsung | Proposal 2.1: We are fine with the proposal, which is clearly stated in the Rel-19 WID. Proposal 2.2: We are generally fine with the proposal. Among listed alternatives, we support Alt1 for SRS resource set which is not following indicated TCI state, and Alt2 for SRS resource set which is following indicated TCI state. Proposal 2.3: We are generally fine with the proposal. Among listed alternatives, we support Alt2, which is aligned with the design of DCI format 2_2. Proposal 2.4: We are fine with separating SRS carrier switching and two closed loop adjustment states for DCI format 2_3. Regarding sub-bullet, since it seems second-level detail, our view is that we can further discuss that aspect after agreeing main-bullet first. Proposal 2.5: So far, we don’t have such high resolution TPC command, although we had a closed loop indicator separate from PUSCH. It would be better to have more elaboration from proponents why this proposal is needed. |
| CATT | Proposal 2.1: Support. Proposal 2.2: Fine with the alternatives. Proposal 2.3: Fine with the alternatives. We prefer to alt1 and alt2. Proposal 2.4: We are fine with the proposal and are open for further discussion. Proposal 2.5: We believe it is a valid proposal as for the mTRP cases, the pathloss (pathloss offsets) may be large. Actual step sizes need further discussion. |
| Docomo | Proposal 2.1: Support. Proposal 2.2: Support. We think we also need to discuss whether the legacy “CL-PC adjustment state separate from PUSCH” can be jointly configured in a CC. For example, Alt.1 of Proposal 2.2, if the following is configured in a CC, and if both R19 feature and legacy feature can be configured in a CC, it is not clear whether SRS resource set#1 is “1st CL-adjustment state of the R19 feature” or “legacy feature”. - SRS set#1: srs-PowerControlAdjustmentStates is set to 'separateClosedLoop' - SRS set#2: srs-PowerControlAdjustmentStates is set to ' separateClosedLoopSecond(New)' Proposal 2.3: Support. Regarding to typeA/B, we think it depends on whether the legacy “CL-PC adjustment state separate from PUSCH” can be jointly configured in a CC. If either “R19 feature” or “legacy feature” can be configured in a CC, we believe both typeA/B needs be supported. Ericsson discuss UE dedicated DCI (1_0/1_1/1_2/0_0/0_1/0_2) to indicate TPC command. To make this feature more useful, we are open to discuss the additional support of the UE dedicated DCI in later. Proposal 2.4: Support. Proposal 2.5: Fine. QC proposes similar (MAC CE to indicate TPC command). We are open to discuss with the two directions (i.e. extend TPC commands to 3-bit or introduce MAC CE to indicate TPC command). |
| OPPO | Proposal 2.1: Support. Proposal 2.2: Fine with the alternatives which could be further discussed and refined if necessary. Proposal 2.3: Fine with the alternatives with preference on Alt2. Proposal 2.4: Support. Proposal 2.5: Support. For UE with fast moving speed, large steps of TPC for SRS (e.g. more than 3dB or -1dB) could adjust Tx power much quicker. So, less DCI with format 2_3 can be sent to UE. This also applies to PUCCH/PUSCH, but in this agenda, we should focus on SRS (according to the WID). |
| Fujitsu | Proposal 2.1: Regarding the text “support configuring two closed-loop PC adjustment states for SRS in one CC”, does it mean one SRS resource set will be configured with two closed loop states? This should be clarified. We think the legacy principle that the SRS resource set is configured with only one closed loop PC state should be followed. Proposal 2.2: Fine to list the alternatives. And we prefer with Alt 1. Proposal 2.3: Generally fine with the proposal. Proposal 2.4: Agree with Samsung regarding the sub-bullet. Proposal 2.5: We don’t see the strong need on this. |
| CMCC | Proposal 2.1: Support. Proposal 2.2: Fine with the proposal with preference on Alt2 and Alt 3 to configure the CLPC adjustment states in TCI state, this is benefit for the dynamic switching of UL transmission between TRPs. When UL transmission is switched from TRP1 to TRP2, CLPC adjustment states can be dynamic updated based on TCI state indication. Proposal 2.3: Fine with the proposal with preference on Alt2. Proposal 2.4: Support. |
| TCL | Proposal 2.1: Support. Proposal 2.2: Support Alt 1. Proposal 2.3: Support Alt 1, Alt 2 and Alt 3. Proposal 2.4: Support. Proposal 2.5: Fine. |
| LG | Proposal 2.1: Support. Proposal 2.2: Support Proposal 2.3: Support Proposal 2.5: Study is fine to us but we are not ok with supporting in this meeting. |
| China Telecom | Proposal 2.1: We support the proposal. Proposal 2.2: We are fine with the proposal, and we prefer Alt1, and we are also open to discuss other alternatives. Proposal 2.3: We are fine with the proposal. And we prefer Alt2, of which we think the spec impact is least, though we are also open to discuss other alternatives. Proposal 2.4: We support the proposal. Proposal 2.5: We are fine to further study the enhance on step sizes for TPC command, but it is better to deprioritized at current stage. |
| Xiaomi | Proposal 2.1: Support. Proposal 2.2: Fine with the alternatives. Proposal 2.3: Fine with the alternatives. We prefer to alt1 or alt2. Proposal 2.4: open for further discussion. Proposal 2.5: we also think TPC via UE specific DCI indication can also be supported with larger size and can also applied to PUSCH. |
| Ericsson | Proposal 2.1 support Proposal 2.2 In our view this proposal is about higher layer configuration on how to configure UE with two SRS CLPC adjustment states, not the indication of the two adjustment states. We propose the following modification of the main bullet: When UE is configured with two SRS CLPC adjustment states, regarding how to configure the two SRS CLPC adjustment states, down-select one from the following alternatives: Proposal 2.3 Do not support the main bullet in the current formulation. We think RAN1 should first achieve the agreement on if the enhancement of DCI format 2_3 is about - Legacy Type B is enhanced - Legacy Type A is enhanced - Both legacy Type A and type B are enhanced - A new type X is specified. Secondly, we think the enhancement of Type A and/or Type B can be different and should be discussed separately. We can support Proposal 2.3 if tis i specific for Type B Proposal 2.4 We think RAN1 should first agree to that there will be no new Type X for Rel-19 MIMO Proposal 2.5 We do not support. The motivation is not strong. We are open to an FFS regarding is there is a need to specify a new table of the same size (i.e., the number of TPC command fields) but with other values than the legacy table. We do not support extension of the number of TPC command fields. Proposal |
| vivo | Proposal 2.1: support Proposal 2.2: support Alt 2 and Alt 3. Proposal 2.3: support Alt 1. Proposal 2.4: support Proposal 2.5: we fail to see the necessity to this proposal. |
| Lenovo | Proposal 2.1: Support Proposal 2.2: Support Proposal 2.3: Support Proposal 2.4: Support Proposal 2.5: Do not support. Faster power adaption can be achieved by more frequent power control command (DCI format 2_3) without changing the spec. |
| MediaTek | Proposal 2.1: Support. Proposal 2.2: Fine with the alternatives. Proposal 2.3: Fine with the alternatives. Proposal 2.4: We are fine with the proposal and are open for further discussion. Proposal 2.5: Prefer to further study |
| Sony | Proposal 2.1 : Support. Proposal 2.2 : Support with the alternatives. Proposal 2.3 : Support with Alt 2, since closed loop indication field is already introduced in DCI 2-2, Alt 2 seems to have least impact to spec. for both typeA and typeB. Proposal 2.4 : Support. Proposal 2.5 : Support. As Moderator commented above, if The Tx power to UL TRP based on the pathloss offset indication from DL sTRP has a gap from the desire TX power or pathloss offset indication for Tx power to UL TRP is not conducted dynamically, Closed loop part has a big role to adjust the Tx power, which indicates that more TX power updating by TPC command can be happened. Considering above situation, extension of TPC command field(e.g. introduce new step size for TPC command) can be a solution to quickly converge the adjustment loop by TPC command. FFS the way of extension for TPC command field. |
| Intel | Proposal 2.1: Support. Proposal 2.2: Support Proposal 2.3: Support Proposal 2.4: Support. Proposal 2.5: Support to further study |

## Other Issues

Table 3-1 summary of Other Issues

| # | Issue |
| --- | --- |
| 3.1 | Enhancement of 2TA for asymmetric DL sTRP/UL mTRP Extension of 2 TA to single-DCI based mTRP system was discussed during WID discussion stage and in the tdocs, many companies (ZTE, China Telecom, Interdigital, Intel, Ericsson, Samsung, Sony, Nokia/NSB, NTT DOCOMO) propose to support/study the extension of 2 TA to single-DCI based system to facilitate the asymmetric DL sTRP/UL mTRP scenarios: - ZTE, China Telecom (1) provided SLS results to show that in asymmetric DL sTRP/UL mTRP scenario, 70% UE cannot meet the timing error limit if only one TAG is used and (2) suggested that the normative workload is quite low. - Interdigital proposed to support 2 TA for DL sTRP/UL mTRP. - Intel proposed to consider 2 UL timing for DL sTRP/UL mTRP - Ericsson proposed to consider to support enhancement to enable two-TAs for asymmetric DL sTRP UL mTRP scenario in the Rel-19 scope. Also proposed to consider the practical synchronization error between anchor TRP and UL TRP for UL and DL timing control in this deployment scenior. - Samsung suggested to further investigate on two TA support with single-DCI based multi-TRP framework, at least for asymmetric DL sTRP/UL mTRP scenarios - Sony proposed to discuss whether supporting UL-only cell for timing advance and initial access are within the scope of Rel-19 - Nokia/NSB proposed to explore the possibility of including the 2TA enhancements for single-DCI mode in the WID scope without extending the TU allocation, in order to enable transmissions towards UL-only nodes/TRPs - NTT DOCOMO proposed to support two TA for single DCI based multi-TRP/panel or single TRP Proposal 3.1: To facilitate the asymmetric DL sTRP/UL mTRP deployment scenarios, support extending 2TA to single-DCI based mTRP and sTRP: - The function of Rel-18 two TA for multi-DCI based mTRP is reused with removing the restriction that coresetpoolIndex needs to be configured. |

Table 3-2: Company input for Issues 3.x

| Company | Comments |
| --- | --- |
| Mod00 | Please share your views on the issue/proposal |
| Samsung | We are fine with Proposal 3.1 with the mindset that this is one of necessary functionalities for this agenda item. |
| Docomo | Support. For asymmetric DL sTRP/UL mTRP scenarios, PL difference between UE to DL TRP and UE to UL TRP is large. For such scenario, large propagation delay between UE to DL TRP and UE to UL TRP are observed, and TA offset should be different between UE to DL TRP and UE to UL TRP. |
| OPPO | Support 2TA extension from M-DCI MTRP to S-DCI MTRP. |
| Fujitsu | 2TA is out of scope. But we could be open for discussion if majority companies are interested. |
| NEC | We can follow majority choice on whether supporting 2 TA or not. |
| China Telecom | We support the Proposal. The enhancement of 2TA extension for asymmetric DL sTRP/UL mTRP scenarios should be further introduced to the WID. |
| Xiaomi | Fine to support, but this proposal needs revision of the WID. |
| Ericsson | We support the main bullet, but the sub-bullet should be an FFS. We would like to add a second sub-bullet: FFS: further study the timing synchronization error |
| vivo | 2 TA enhancement is out-of-scope. |
| Lenovo | Do not support. We think 2TA is out of scope for the WID. |
| MediaTek | Open to it. However, if 2 TAs are supported, it is not reasonable to support them under M-DCI for asymmetric DL sTRP/UL mTRP. |
| Sony | Support. |
| Intel | Support main bullet and suggest sub-bullet as FFS |

# Proposals for Online Discussion

Proposal 1.1: For the asymmetric DL sTRP/UL mTRP deployment scenarios, support to configure one PL offset configuration for the PUSCH, PUCCH and SRS towards one UL TRP.

- When the UE is informed to apply one PL offset configuration on one PUSCH, PUCCH or SRS, the UE calculates the UL Tx power as follows:

  - reuse the legacy power control methods with a UL PL estimated from an DL PL RS and the NW indicated PL offset.

  - FFS whether to additionally support updating UL PL as follows:

    - The initial value of UL PL is estimated from the DL PL RS and the NW indicated PL offset

    - The UE updates UL PL with UL PL = UL PL + an update delta indicated by the NW.

- Apply the PL offset on PDCCH-order PRACH transmission too.

  - FFS: how to determine PRACH direction towards UL TRP without DL RS

  - Note: this does not imply to support 2 TA for single-DCI based system.

- FFS: how many PL offset configurations can be configured in one CC to support multiple UL TRPs;

- FFS: the range of PL offset and the quantization levels of PL offset

Proposal 2.1: To facilitate the asymmetric DL sTRP/UL mTRP deployment scenarios, support configuring two closed-loop PC adjustment states for SRS in one CC, both of which are separate from that of the PUSCH.

- Note: one SRS resource set can be indicated with one closed-loop PC adjustment state.

Proposal 1.2: When the UE is provided with one or more PL offset configurations in one CC, UL TCI state can be associated with one PL offset configuration:

- When a UL TCI state is associated with a PL offset configuration, the UE should calculate the Tx power of the PUSCH/PUCCH/SRS transmission including the PL RS configuration and PL offset, if the UL TCI state is indicated.

- When a UL TCI state does not include PL offset, the UE should calculate the Tx power of the PUSCH/PUCCH/SRS transmission assuming the PL of the UL/DL TRP, if the UL TCI state is indicated.

FFS: whether to support associating joint TCI state with a PL offset configuration.

Proposal 2.2: When one UE is configured with two SRS CLPC adjustment states, regarding how to configure one SRS CLPC adjustment state to one SRS resource set, down-select one from the following Alts:

- Alt1: Introduce one new value to the parameter srs-PowerControlAdjustmentStates in SRS resource set and this value indicate the 2nd SRS CLPC adjustment state

- Alt2: When the parameter srs-PowerControlAdjustmentStates is set to 'separateClosedLoop', closedLoopIndex-r17 in the TCI state indicates one of the SRS CLPC adjustment states:

  - value i0 indicates the 1st SRS CLPC adjustment state and

  - value i1 indicates the 2nd SRS CLPC adjustment state;

- Alt3: Add one extra parameter in P0AlphaSet-r17 of TCI state to indicate one of those two SRS CLPC adjustment states

- Alt4: the 2nd SRS CLPC adjustment state is configured to one SRS usage and all the SRS set of that SRS usage is associated with the 2nd SRS CLPC adjustment state, and the 1st SRS CLPC adjustment state is indicated to one SRS resource set per the legacy specification.

Proposal 2.3: Study how to indicate TPC command for those two SRS CLPC adjustment states through DCI when the UE is configured two SRS CLPC adjustment states, down-select from the following options:

- Option 1: enhance the legacy DCI format 2_3 of higher layer parameter srs-TPC-PDCCH-Group = typeA;

- Option 2: enhance the legacy DCI format 2_3 of higher layer parameter srs-TPC-PDCCH-Group = typeB;

- Option 3: enhance the legacy DCI format 2_3 of higher layer parameter srs-TPC-PDCCH-Group = typeA and typeB;

- Option 4: introduce a new DCI format 2_X

For the Options1, 2 and 3, consider and down-select from one from the following Alts:

- Alt1: In DCI format 2_3, add one additional TPC command for each CC configured with two SRS CLPC adjustment states,

  - the first TCP command is associated with the first SRS CLPC adjustment state and the second TPC command is associated with the second SRS CLPC adjustment state.

- Alt2: Introduce one 1-bit closed-loop-indicator field for each TPC command in DCI format 2_3

  - This 1-bit closed-loop-indicator indicates the first SRS CLPC adjustment state or the second SRS CLPC adjustment state.

- Alt3: use two different TPC-SRS-RNTIs for DCI format 2_3:

  - DCI format 2_3 with CRC scrambled with the first TPC-SRS-RNTI and the second TPC-SRS-RNTI indicates the TPC command for the first and second SRS CLPC adjustment state, respectively.

- Alt4: Implicit method:

Proposal 1.4: Support updating the value of PL offset through MAC CE.

- The gNB can send a MAC CE to update the PL offset values of one or more PL offset configurations

  - FFS: whether the TCI state activation MAC CE can realize the update of PL configuration through update the TCI state

- FFS the need of dynamic indication via DCI for FR2.

Proposal 1.6: For the asymmetric DL sTRP/UL mTRP deployment scenarios, separate TCI state mode of unified TCI framework can be configured:

- The gNB can indicate one pair of DL TCI state and UL TCI state to the UE (for Rel-17 unified TCI framework).

- The gNB can indicate one DL TCI state, two UL TCI states to the UE (for Rel-18 unified TCI framework).

- FFS: whether these TCI states are associated with 2 TRPs or 3 TRPs.

FFS whether to support the following TCI state configuration case:

- Configure joint TCI state mode

- The gNB can indicate one pair of joint TCI state and UL TCI state to the UE.

# Contributions in RAN1#116

- R1-2400052	Enhancements for asymmetric DL sTRP/UL mTRP scenarios	Spreadtrum Communications

- R1-2400107	On power control enhancements for DL sTRP and UL mTRP deployment	Huawei, HiSilicon

- R1-2400164	On Asymmetric mTRP Deployment	InterDigital, Inc.

- R1-2400197	Enhancement for asymmetric DL sTRP/UL mTRP scenarios	Lenovo

- R1-2400240	Discussion on asymmetric DL sTRP/UL mTRP scenarios	vivo

- R1-2400267	Discussion on asymmetric DL sTRP/UL mTRP scenarios	TCL

- R1-2400281	Discussion on enhancements for asymmetric DL sTRP/UL mTRP scenarios	ZTE, China Telecom

- R1-2400324	Discussion on enhancement for asymmetric DL sTRP/UL mTRP scenarios	CMCC

- R1-2400384	Enhancements for asymmetric DL/UL scenarios	Intel Corporation

- R1-2400431	Discussion on enhancements for asymmetric DL sTRP/UL mTRP scenarios	CATT

- R1-2400468	Discussion on enhancements for asymmetric DL sTRP and UL mTRP scenarios	NEC

- R1-2400510	Enhancement for asymmetric DL sTRP/UL mTRP scenarios	MediaTek Inc.

- R1-2400521	Enhancement for asymmetric DL sTRP UL mTRP scenarios	Ericsson

- R1-2400556	Discussion on enhancement for asymmetric DL sTRP/UL mTRP scenarios	xiaomi

- R1-2400626	Enhancements on asymmetric DL sTRP/UL mTRP scenarios	OPPO

- R1-2400659	Discussion on enhancements for asymmetric DL sTRP/UL mTRP scenarios	China Telecom

- R1-2400730	Views on Rel-19 asymmetric DL sTRP/UL mTRP scenarios	Samsung

- R1-2400774	Discussion on UL-only mTRP operation	Fujitsu

- R1-2400851	Considerations on enhancement for asymmetric DL sTRP/UL mTRP scenarios	Sony

- R1-2400920	Discussions on asymmetric DL sTRP/UL mTRP scenarios	LG Electronics

- R1-2400941	Enhancement for asymmetric DL sTRP/UL mTRP scenarios	Nokia, Nokia Shanghai Bell

- R1-2400958	Enhancement for Asymmetric DL sTRP/UL mTRP Scenarios	Panasonic

- R1-2401010	Views on R19 enhancement for asymmetric DL sTRP/UL mTRP	Apple

- R1-2401047	Enhancement for asymmetric DL sTRP/UL mTRP scenarios	Sharp

- R1-2401050	Discussion on enhancement for asymmetric DL sTRP/UL mTRP scenarios	Hyundai Motor Company

- R1-2401052	Discussion on enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios	NTPU

- R1-2401472	Discussion on enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios	NTPU

Revision of R1-2401052

- R1-2401115	Discussion on enhancement for asymmetric DL sTRP/UL mTRP scenarios	NTT DOCOMO, INC.

- R1-2401257	Discussion on enhancement for asymmetric DL sTRP and UL mTRP scenarios	Google

- R1-2401439	Enhancement for asymmetric DL sTRP and UL mTRP deployment scenarios	Qualcomm Incorporated