3GPP TSG-RAN WG1 Meeting #116	Tdoc R1-2401546

Athens, Greece, February 26th – March 1st, 2024

Agenda Item:	9.1.2

Source:	Moderator (Ericsson)

Title:	Summary #2 of specification support for positioning accuracy enhancement

Document for:	Discussion, Decision

# Introduction

This document summarizes the discussions during RAN1#116 for the agenda item 9.1.2, Specification support for positioning accuracy enhancement.

This discussion corresponds to the objectives related to the positioning use case described in RP-234039 (WID) below.

RP-234039 (WID), Objective: Provide specification support for the following aspects: - AI/ML general framework for one-sided AI/ML models within the realm of what has been studied in the FS_NR_AIML_Air project [RAN2]: - Signalling and protocol aspects of Life Cycle Management (LCM) enabling functionality and model (if justified) selection, activation, deactivation, switching, fallback - Identification related signalling is part of the above objective - Necessary signalling/mechanism(s) for LCM to facilitate model training, inference, performance monitoring, data collection (except for the purpose of CN/OAM/OTT collection of UE-sided model training data) for both UE-sided and NW-sided models - Signalling mechanism of applicable functionalities/models - Beam management - DL Tx beam prediction for both UE-sided model and NW-sided model, encompassing [RAN1/RAN2]: - Spatial-domain DL Tx beam prediction for Set A of beams based on measurement results of Set B of beams (“BM-Case1”) - Temporal DL Tx beam prediction for Set A of beams based on the historic measurement results of Set B of beams (“BM-Case2”) - Specify necessary signalling/mechanism(s) to facilitate LCM operations specific to the Beam Management use cases, if any - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE NOTE: Strive for common framework design to support both BM-Case1 and BM-Case2 - Positioning accuracy enhancements, encompassing [RAN1/RAN2/RAN3]: - Direct AI/ML positioning: - (1st priority) Case 1: UE-based positioning with UE-side model, direct AI/ML positioning - (2nd priority) Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning - (1st priority) Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning - AI/ML assisted positioning - (2nd priority) Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning - (1st priority) Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning - Specify necessary measurements, signalling/mechanism(s) to facilitate LCM operations specific to the Positioning accuracy enhancements use cases, if any - Investigate and specify the necessary signalling of necessary measurement enhancements (if any) - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE for relevant positioning sub use cases - Core requirements for the above two use cases for AI/ML LCM procedures and UE features [RAN4]: - Specify necessary RAN4 core requirements for the above two use cases. - Specify necessary RAN4 core requirements for LCM procedures including performance monitoring. Study objectives with corresponding checkpoints in RAN#105 (Sept ’24): - CSI feedback enhancement [RAN1]: - For CSI compression (two-sided model), further study ways to: - Improve trade-off between performance and complexity/overhead - e.g., considering extending the spatial/frequency compression to spatial/temporal/frequency compression, cell/site specific models, CSI compression plus prediction (compared to Rel-18 non-AI/ML based approach), etc. - Alleviate/resolve issues related to inter-vendor training collaboration. while addressing other aspects requiring further study/conclusion as captured in the conclusions section of the TR 38.843. - For CSI prediction (one-sided model), further study performance gain over Rel-18 non-AI/ML based approach and associated complexity, while addressing other aspects requiring further study/conclusion as captured in the conclusions section of the TR 38.843 (e.g., cell/site specific model could be considered to improve performance gain). - Necessity and details of model Identification concept and procedure in the context of LCM [RAN2/RAN1] - CN/OAM/OTT collection of UE-sided model training data [RAN2/RAN1]: - For the FS_NR_AIML_Air study use cases, identify the corresponding contents of UE data collection - Analyse the UE data collection mechanisms identified during the FS_NR_AIML_Air (TR 38.843 section 7.2.1.3.2) study along with the implications and limitations of each of the methods - Model transfer/delivery [RAN2/RAN1]: - Determine whether there is a need to consider standardised solutions for transferring/delivering AI/ML model(s) considering at least the solutions identified during the FS_NR_AIML_Air study - Testability and interoperability [RAN4]: - Finalize the testing framework and procedure for one-sided models and further analyse the various testing options for two-sided models, in collaboration with RAN1, and including at least: - Relation to legacy requirements - Performance monitoring and LCM aspects considering use-case specifics - Generalization aspects - Static/non-static scenarios/conditions and propagation conditions for testing (e.g., CDL, field data, etc.) - UE processing capability and limitations - Post-deployment validation due to model change/drift - RAN5 aspects related to testability and interoperability to be addressed on a request basis NOTE: offline training is assumed for the purpose of this project. NOTE: the outcome of the study objectives should be captured in TR 38.843 for future reference. NOTE: Coordination with SA/SA WGs of the ongoing study/work as it may relate to their required work.

In the following, the delegates from participating companies are listed. Please check and add the contact information. We can use this list for future offline discussion, if such a need arises.

| Company/organization name | Contact person | Email of contact person |
| --- | --- | --- |
| Ericsson | Yufei Blankenship | yufei.blankenship@ericsson.com |
| Ericsson | Florent Munier | florent.munier@ericsson.com |
| Fujitsu | Xin WANG Yujia SHAN | wangxin@fujitsu.com shanyujia@fujitsu.com |
| MTK | harrison | harrison.chuang@mediatek.com |
| ZTE | Cong WANG | wang.cong8@zte.com.cn |
| CMCC | Yi Zheng Yongchang Liu | zhengyi@chinamobile.com liuyongchang@chinamobile.com |
| InterDigital | Fumihiro Hasegawa | fumihiro.hasegawa@interdigital.com |
| vivo | Huaming Wu | huaming.wu@vivo.com |
| vivo | Yuanyuan Wang | yuanyuan.wang.txyj@vivo.com |
| CATT | Yongqiang Fei | feiyongqiang@catt.cn |
| NEC | Chen Wei Miao Zhaobang | chen_wei@nec.cn miao_zhaobang@nec.cn |
| Xiaomi | Qin MU | muqin@xiaomi.com |
| OPPO | Zhihua Shi | szh@oppo.com |
| Pengcheng laboratory | Pingye XIANG | xiangpy@pcl.ac.cn |
| NVIDIA | Xingqin Lin | xingqinl@nvidia.com |
| TCL | Tianqi Wu | tianqi1.wu@tcl.com |
| LG Electronics | Jaehoon Chung | jhoon.chung@lge.com |
| Qualcomm | Mohammed Hirzallah (Ali) | mhirzall@qti.qualcomm.com |
| ETRI | Seungjae Bahng | sjbahng@etri.re.kr |
| Baicells | Xiang YUN Xiaonan WANG | yunxiang@baicells.com wangxiaonan@baicells.com |
| Apple | Kome Oteri | ooteri@apple.com |
| Fraunhofer | Mohammad Alawieh Georgios Kontes | mohammad.alawieh@iis.fraunhofer.de georgios.kontes@iis.fraunhofer.de |
| Sony | Basuki Priyanto Yujie Zhang | Basuki.priyanto@sony.com Yujie.zhang@sony.com |
| Samsung | Qi XIONG | Q1005.xiong@samsung.com |
| NTT DOCOMO | Luhua You Kosuke Shima | youlh@docomolabs-beijing.com.cn kousuke.shima.nr@nttdocomo.com |

# (Closed) Positioning sub-cases prioritization in work item

## Companies’ view from contribution

Regarding prioritization of issues for AI/ML based positioning, selected companies’ views are shown below, based on the submitted contributions.

- InterDigital (R1-2401042) Proposal 1: Postpone any discussion related to 2nd priority cases until framework to support the 1st priority items is stabilized

- Huawei, HiSilicon (R1-2400145) Proposal 1: RAN1 follows the WID on the prioritization of use cases, i.e. Case 1/3a/3b are prioritized.

- ZTE (R1-2400169) Proposal 1: Specification impact for use cases with 1st priority can be considered first. - Enhancements specific to the 2nd priority use cases should be considered as lower priority.

- Ericsson (R1-2400101) Proposal 1 RAN1 work on 1st priority cases first. Work on 2nd priority cases can start once discussions on 1st priority cases have matured.

- CMCC (R1-2400317) - Proposal 1: For AI/ML enabled positioning accuracy enhancement, 1st priority cases should be focused. The enhancements, if applicable, can be extended to 2nd priority case.

- Qualcomm (R1-2401432) Observation 1: For specifying AI/ML positioning at UE side (Case1/2a), common specifications can be scoped and jointly discussed for Case1 and Case2a without waiting for full completion of Case1.

- vivo (R1-2400233) Work on all sub-use cases from the beginning.

- MediaTek (R1-2401137) Observation 2d-1: The use case prioritization is unfriendly to DL+UL positioning method Proposal 2d-1: Support in principle the DL+UL positioning with AI/ML method Proposal 2d-2: Model at LMF could be considered for DL+UL positioning with AI/ML method

- NEC (R1-2400469) - Proposal 1: To address the varying priorities among the five use cases, it is recommended to allocate the resources in each meeting proportionately, with the majority assigned to the first priority and the remainder to the second priority. Concurrently address both priorities in each meeting.

- Samsung (R1-2400721) Proposal 1: suggest to change case 3b to case 2b in the 1st priority cases.

- Lenovo (R1-2400923) Proposal 1: RAN1 may initiate specification work for both Cases 2b and 3b for Direct AI/ML and Cases 2a and 3a for AI/ML assisted positioning.

- NTPU (R1-2401476) Observation 4: Among the currently prioritized sub-use cases, Case 1 and Case 3a may have more RAN1 impacts than Case 3b. Proposal 5: RAN1 is suggested to further prioritize Case 1 and Case 3a.

## 1st round discussion

Based on companies input above, the proposal is provided below to clarify the understanding in RAN1 discussion.

Proposal 2.2-1

RAN1 follows the WID and focuses on the positioning accuracy enhancement for 1st priority use cases. The enhancements, if applicable, can be extended to 2nd priority case without delay.

- Enhancements specific to the 2nd priority use cases are considered as lower priority.

|  | Company |
| --- | --- |
| Support | InterDigital (The 2nd sentence is not necessary since the enhancements will be applied to all use cases naturally (e.g., via definition of a new measurement)). ZTE, CMCC, TCL, Xiaomi, HwHiSi, Pengcheng laboratory |
| Not support | Vivo, Nokia/NSB |

| Company | Comments |
| --- | --- |
| Fujitsu | We think parallel study of five use cases and clarify/conclude the potential spec scope per use case at the first stage of Rel-19 (e.g. before Sept.) can be considered. With the first stage conclusion, if the workload for STD impacts of the 2nd priority cases is not affordable, we are fine with this proposal. |
| Mtk | 1, we are generally okay for FL proposal. But we consider that, the enhancement specific to 2nd priority use cases, if identified, could consider the normative work after 1st priority use cases |
| ZTE | Support. InterDigital’s suggestion is also fine for us. |
| CMCC | Generally fine. Companies may also try to figure out what enhancements specific to 2nd cases need to be specified. We can firstly discuss enhancements for 1st priority cases and whether these enhancements can also apply to 2nd priority cases. After that enhancements specific to 2nd cases should be discussed. |
| TCL | Since the five cases has some common features, the enhancements can be extended to 2nd priority case without delay. |
| Vivo | It’s not clear to us what’s the implication of this proposal if agreed. Does that mean issues for the 2nd priority cases cannot be discussed before those for the other cases are finished? If that’s the intention, we disagree and don’t support this proposal. We have no intention to repeat the RAN plenary discussion on case prioritization. However, we want to make assure case 2a and 2b are also addressed/specified as they are tasked by the WID to be specified in Rel-19 timeframe as well. It’s not only case 1, 3a and 3b to be specified in Rel-19. As studied during the SI by us and also mentioned by several other companies in this meeting, lack of specification support for case 2a and 2b may prevent the usage of some positioning methods to be properly enhanced by AI/ML. |
| CATT | Fine in general, as long as we will not drop any of the sub cases. |
| NEC | As the partial of the case is coupled, we tend to study the five use cases parallelly t. For example, when we are in the process the discussion of some issue like data collection, LCM, …, we can also consider whether the propose can be extend to the 2rd priority case. However, if more efforts/discussions are needed, the discussion of the extend can be suspended. |
| Nokia/NSB | We do not agree with the second sentence. Every use case should be discussed based on WID already agreed in the last Plenary. After the completion of Case 1, Case 3b/3a naturally many related agreements on enhancements on signalization/measurements may be raised if we discuss individually the remaining cases. Proposal 2.2-1 (updated) RAN1 follows the WID and focuses on the positioning accuracy enhancement for 1st priority use cases. The enhancements, if applicable, can be extended to 2nd priority case without delay. |
| OPPO | We are generally fine with the proposal. MTK’s suggest for the sub-bullet is also ok for us |
| Qualcomm | We think some common aspects can be discussed together. For example, many aspects related to Case1 & 2a data collection and monitoring are common and can be discussed together. |
| Ericsson | Support. In our understanding, the proposal captures the order of priorities as per the WID. The proposal does not preclude to work on case 2a and 2b during the release time frame. Rather, it clarifies that the specification work of what is missing to support case 2a and 2b will start once other cases progressed enough. |
| InterDigital | We have a question for Feature Lead. What is the intention of creating Section 2.3 and how do you plan to use it? We should not capture the vice chair’s comment in 2.3 in the FL summary. If the vice chair has a comment or note, the vice chair will write the note or comment in the Chairman’s note. |

## Guidance from RAN1 Vice Chairman at Tuesday online session

On how to handle 1st and 2nd priority sub-cases, Xiaodong gave the following guidance verbally.

The principle is that progress of 1st priority cases is not hindered by 2nd priority cases. RAN1 does not have any artificial blocker to 2nd priority sub-cases.

# Model input

## (Closed) DL PRS and UL SRS

### Companies’ view from contribution

In the following, selected inputs from companies’ contributions are provided.

- Ericsson (R1-2400101) Proposal 2 Reuse DL PRS and UL positioning SRS as in existing specification, including their transmission in the already specified RRC states.

- CMCC (R1-2400317) - Proposal 7: For AI/ML based positioning, deprioritize RS enhancements until any aspect is found.

- Samsung (R1-2400721) Proposal 10: using current PRS/SRS configuration framework as starting point and identify the necessary change for LCM operation.

- ZTE (R1-2400169) Proposal 2: The current LPP, NRPPa, and RRC signalling can be reused for the reference signal configuration for AI/ML positioning.

- Lenovo (R1-2400923) Proposal 8: RAN1 to support additional DL-based channel fingerprinting measurements based on other existing RS such as CSI-RS.

- TCL (R1-2400348) - Proposal 3: AI-specific or AI model-specific reference signal configurations, including PRS and SRS for positioning, could be introduced and indicated to the UE, enabling it to distinguish the usage or transmitting method of the configured reference signal.

- Google (R1-2400393) Proposal 1: Support coverage enhancement for PRS to reduce the measurement error. Proposal 4: Support to configure multiple pathloss reference signals for power control for SRS for positioning - UE calculates the transmission power for the SRS based on the maximum pathloss measured from the pathloss reference signals

- Xaomi (R1-2400544) Proposal 6: Consider enhanced on-demand PRS request and configuration mechanism: - UE can request multiple set of PRS configurations in one time of request signalling transmission, in which each set of configurations may correspond to separate StartTime-and-Duration, or TRP pattern, etc.

- NEC (R1-2400469) Proposal 3: Specific distinct RS configuration patterns for the different stage of the LCM, e.g., employing different time domain period for data collection between model training and model monitoring.

### 1st round discussion

As shown above, several companies discussed whether/how to enhance the existing reference signals (DL PRS, UL SRS, CSI-RS) for AI/ML positioning work, while most companies took for granted to reuse the existing reference signal. Let's align the understanding on the reference signals since it's the foundation for channel measurements.

Proposal 3.1.2-1

For Rel-19 AI/ML based positioning, the measurements for model input are based on the existing DL PRS and UL positioning SRS as a starting point.

- Further investigation is not precluded if necessary enhancements are identified.

|  | Company |
| --- | --- |
| Support | MTK, ZTE, CMCC, TCL, CATT, NEC, HW/HiSi, Fraunhofer, Pengcheng laboratory, Ericsson |
| Not support | Nokia/NSB |

| Company | Comments |
| --- | --- |
| Fujitsu | Support, any enhancements for DL PRS and/or UL SRS-pos should be evaluated and studied for legacy positioning first. |
| CMCC | Support. During the SI the enhancements for RS are not clearly identified. |
| InterDigital | We have a suggestion for changing texts. The measurements are made using existing RSs. The sub-bullet should cover “as a starting point” in the FL’s proposal. For Rel-19 AI/ML based positioning, the measurements for model input are made based on using the existing DL PRS and UL positioning SRS as a starting point. Further investigation is not precluded if necessary enhancements are identified. |
| Nokia/NSB | We support partially this proposal. We do not agree with the bullet point, further investigation means potentially new studies that main add extra delay on the standardization. We propose the following rewording: Proposal 3.1.2-1 (Updated) For Rel-19 AI/ML based positioning, the measurements for model input are based on the existing DL PRS and UL positioning SRS. As a starting point. - Further investigation is not precluded if necessary enhancements are identified. |
| OPPO | Support. We are also fine with Nokia’s version |
| HW/HiSi | Support and also fine with Nokia update. |
| Fraunhofer | MIMO-SRS support as in Rel-16 positioning without any additional specification impact can be considered. |
| Ericsson | Support. IDG’s rewording is also fine. |

### 2nd round / offline session

Majority companies suggest to update Proposal 3.1.2-1 and make it more crisp. Nokia's version is presented below for further checking.

Proposal 3.1.3-1

For Rel-19 AI/ML based positioning, the measurements for model input are based on the existing DL PRS and UL positioning SRS.

|  | Company |
| --- | --- |
| Support | InterDigital, Nokia/NSB,Fujitsu,ZTE,Baicells, Apple, Fraunhofer |
| Not support |  |

| Company | Comments |
| --- | --- |
| NTT DOCOMO | The non-positioning specific RS, e.g., CSI-RS/TRS can be leveraged for Rel-19 AI/ML based positioning besides positioning specific RS, and so that the resource overhead and power consumption can be reduced. We propose the following update. For Rel-19 AI/ML based positioning, the measurements for model input are based on the existing RS DL PRS and UL positioning SRS. |
| TCL | based on the signal types or configurations? If it is based on the signal type, we support. Otherwise, the enhancements of configurations are not precluded. |
| Qualcomm | We are fine with Docomo version. |

Offline discussion:

Proposal 3.1.3-1

For Rel-19 AI/ML based positioning, the measurements for model input are based on the existing DL PRS and UL SRS.

- Note: The use of SRS for MIMO resource is transparent to UE.

Supported by: 7 companies

Proposal 3.1.3-1a

For Rel-19 AI/ML based positioning, RAN1 intends to reuse existing RS for positioning.

Supported by: 0 companies

Proposal 3.1.3-1b

For Rel-19 AI/ML based positioning, the measurements for model input are based on the existing DL PRS and UL SRS as a starting point.

- Further investigation is not precluded if necessary enhancements are identified.

Supported by: 1 company

## Sample-based vs path-based measurements for model input

### Companies’ view from contribution

In the following, selected inputs from companies’ contributions are provided.

- vivo (R1-2400233) Proposal 4: Aspects that have been confirmed to be beneficial and necessary for positioning should be specified, including: - Potential new measurement reporting for Case 2b and 3b, e.g., CIR/PDP/DP - Dynamic TRP pattern at least for Case 2b [also beneficial to case 1]. - Methods of reducing reporting overhead at least for Case 2b [also beneficial to case 3b]. - Reporting enhancement to avoid potential inconsistency of model input between training and inference for Case 2b and 3b. Proposal 5: Specify sample-wise channel measurement reporting for Case 2b and 3b of AI/ML based positioning due to the constraints of path-wise reporting.

- MediaTek (R1-2401137) Proposal 2a-1: The time-domain sample based measurement as the model input is preferred Proposal 2a-2: The further extension on the reporting of more additional paths could also be considered when the model is deployed at the LMF Proposal 2a-3: UE may be requested to report based on either the samples or the paths to LMF for LMF-side model Proposal 2a-4: Define the measurement of the samples in TS 38.215 Proposal 2a-5: Consider “DL reference signal channel response” as the measurement of the samples to be capture in TS 38.215 Proposal 2a-6: The measurement type “DL reference signal channel response” could be defined as the channel response obtained from the resource elements that carry DL PRS configured for the measurement Proposal 2a-8: The sample selection mechanism could be left to RAN4 Proposal 2a-9: If RAN1 decides to define the sample selection mechanism for overhead reduction, consider the simple rule. The limitation on the maximum number of non-zero samples within a channel response measurement could be configured Proposal 4-1: Support the sample based channel response measurement as the model input for LMF side model Proposal 4-2: Strive to avoid the simultaneous reporting for the sample based channel response measurement and the legacy timing measurement to LMF, which may disclose the implementation Proposal 4-3: The sample based channel response measurement at the TRP is based on SRS for positioning or SRS for MIMO transmitted by UE Proposal 4-4: To avoid simultaneous reporting on the sample based and the legacy path-timing based measurements, consider to report the sample based channel response measurements only. The path timings are determined by LMF to accomplish the legacy UL-TDOA method Proposal 4-5: To avoid simultaneous reporting on the sample based and the legacy path-timing based measurements, configure the two procedures in TDM manner. This may not have specification impact Proposal 5-1: For case 3a, the number of additional paths could be kept the same as that for the legacy measurement, which is 8. Proposal 5-2: For case 3a, support sample based channel response measurement as model input for gNB side model

- CATT (R1-2400419) Proposal 22: For case 2b, need to ensure that the UE reported measurement and the LMF-side model input are generated with the same rule. The following one or two directions can be considered: • Direction 1: enhance ‘per path’ CIR/PDP/DP report; • Direction 2: support ‘per sample’ CIR/PDP/DP report. Proposal 13: For case 3b, need to ensure that the gNB reported measurement and the LMF-side model input are generated with the same rule. The following one or two directions can be considered: • Direction 1: enhance ‘per path’ CIR/PDP/DP report; • Direction 2: support ‘per sample’ CIR/PDP/DP report.

- Ericsson (R1-2400101) Observation 3. Existing PRS-RSRPP measurement is also a sub-sample of the TD PDP samples except that the sampling time is not restricted to a regular sampling grid (D_p is not necessarily an integer). Proposal 3 For the model input types for Case 3b (1st priority) and Case 2b (2nd priority), consider input based on DP or PDP samples containing sample powers summed over all receive antenna ports. Observation 7 While both PDP sub-samples from regular sampling grid points and RSRPP reports are both sub-samples of the TD PDP samples, there are statistical differences between them that may impact AI/ML model performance. Most notably, the former is obtained via straightforward computation, but the latter depend heavily on the UE’s channel estimation and interpolation capabilities. Observation 8 There has been no systematic study on the use of (off-grid) RSRPP reports as AI/ML model inputs in 3GPP. Without correct modeling of UE channel estimation and interpolation capabilities, the performance of (off-grid) RSRPP reports as AI/ML model inputs can be exaggerated. Observation 9 If the UE receiver implementation issues are not studied, there will be model input distributional mismatch between training and inference phases, when different Ues with different receiver implementation are used in training vs inference. Proposal 4 RAN1 to study the feasibility of (off-grid) RSRPP reports as AI/ML model inputs based on at least realistic modeling of UE’s channel estimation capabilities. The reported per-path delays should be from an actual path delay estimator operating at the correct SNR. Proposal 5 RAN1 discuss and decide which measurement to use as model input: (a) DP/PDP of sub-samples from regular sampling grid points; or (b) (off-grid) RSRPP reports together with per-path delays where the per-path delays may use finer granularity than the sampling period.

- Fraunhofer (R1-2401198) Proposal 13: Consider a sample based approach for the new measurement report: - Vector(s) of samples including the CIR or PDP - Time stamp of the first sample of the vector (or offset of the first sample of the relative to the reported ToA). - Sampling frequency used for the vector - Opt 1: Sampling frequencies inline with the numerology are supported only. - Opt 2: The measurement unit may select the sampling frequency and reports the selected sampling frequency Proposal 14: Support truncated CIR reporting, wherein the report may include - (Short) vector of complex samples - Time stamp of the first sample of the vector - Applied sampling frequency - Estimated delay of the first path or N-paths relative to the time stamp of the first sample (this may allow to convert the report directly into a RSTD report). Proposal 15: Support segmented CIR reporting, wherein for each segment the following data are reported - Offset of the segment relative to the first segment - Number of samples used for the segment - Vector including I/Q values (or magnitude and phase) of the CIR within the segment Proposal 16: Consider multiple reporting configurations that emphasize CIR/PDP information depending on the varied applications, channel conditions or requirements.

- Qualcomm (R1-2401432) Proposal 10: For Case2b/3b, based on observations from Rel-18 Study Item, the increase of reporting size is not justified and no need to enhance reporting size (i.e., number of paths/samples N’t) beyond what is supported in existing specifications.

### 1st round discussion

As shown above, several companies investigated the issue of per-sample or per-path measurements as model input. Ericsson presented analysis of existing requirement in RAN4 specification, and explained that the difference between ‘per-path’ and ‘per-sample’ is whether the time domain samples are on the sampling grid or off-grid. Three companies prefer sample-based channel measurements: vivo, MediaTek, Fraunhofer. CATT proposed that UE reported measurement and the LMF-side model input are generated with the same rule.

Note that in Rel-18 study item, most companies used the sampling period = 1/(Nf ×∆f), See extracts from TR 38.843.

TR 38.843 For evaluations, companies used the following values for sampling period: - 16 Sources used the following sampling period: - Sampling period = 1/(Nf ×∆f). For FR1, sampling period = 1/(4096×30)=8.14 (ns), where Nf =4096 according to 38.211, and ∆f =30 kHz is the subcarrier spacing. - 1 Source used: sampling period = 4.069 ns

In moderator’s understanding, this issue needs further investigation in RAN1. We can clarify the two alternatives at this meeting to facilitate the investigation.

Proposal 3.2.2-1

In Rel-19 AI/ML based positioning, regarding the time domain channel measurements for model input, RAN1 study and select one alternative from the following:

- Sample-based measurements, where the timing information is an integer multiple of sampling periods.

  - FFS: details including but not limited to: definition of starting point for the sample list, sampling period, number of samples

- Path-based measurements, where the timing information is according to the detected path timing and may not be an integer multiple of sampling periods.

  - FFS: granularity of path timing, number of paths (including potential extension from the existing 8 additional paths).

|  | Company |
| --- | --- |
| Support | ZTE, TCL, ericsson |
| Not support | Nokia/NSB, Fraunhofer |

| Company | Comments |
| --- | --- |
| Fujitsu | Slightly prefer to put the focus on sample-based solutions, since it is widely evaluated. While path-based-method rely on the path detection details, and different path detection/path decision method may have different AI/ML performance. |
| MTK | We prefer to make decision in this meeting on which to use |
| ZTE | Support FL’s view, in the first WI meeting we can list potential alternatives, and discuss in the following meetings. |
| CMCC | Support. Both sample-based and path-based measurement have their pros and cons. During the SI, most of the evaluation performs based on the sample-based measurement, and significant gain has been shown. Many companies point out that which additional path UE reported is based on UE implementation. Some rules and enhancements for path-based measurements may be necessary to facilitate AI pos. From our point of view, we slightly prefer sample-based reporting, and sample-based measurements may be supported for at least case 3b, cause it does not impact the air interface payload. |
| TCL | This issue could be further discussed. Whether the path-based measurements deteriorate when bandwidth is limited should be considered. |
| Vivo | We already presented comparison results between sample- and path-based measurements for AI/ML positioning. We already showed the problems of existing path-based measurement report. What else needs to be studied? If RAN1 needs to further study then select, we’d like to know what are the criteria for the down-selection. |
| CATT | Fine to study both. But it seems a too early to say RAN1 will select one alternative. Can we change to main bullet to: …RAN1 study and select one alternative from the following, with potential down-selection: |
| Xiaomi | Support, we slightly prefer sample-based solution since it is already evaluated in the SI phase. |
| Nokia/NSB | Some clarifications are needed on this topic. Thus, we suggest delaying the discussion for our next offline session. There are many aspects to be considered before to make any decision. |
| OPPO | Not sure whether this issue should be disucssed in RAN1. Would some companies like to clarify what’s spec impact in RAN1? In our understanding, the timing accuracy requirement is up to other WG (e.g., RAN4). According to the requirement, UE may decides what sample rate is used to meet the requirement. |
| Qualcomm | We propose to postpone this discussion. It is not clear why we need to agree on this in the first meeting. Whether we need to align on sample-based vs. Path-based is something we can discuss further once the final listing of measurements and their sizes are finalized. We should first focus on understanding tradeoff between accuracy and reporting overhead for different measurements studied in Rel-18 before rushing in enhancement discussions. @Moderator: Please include a Section to discuss tradeoff of accuracy and signaling/reporting overhead for measurement size (i.e., N’t samples/paths). [Moderator] The intention of the proposal is to clearly describe the two approaches, and invite companies to study issues related to this, e.g., reporting overhead of N't samples vs paths. The intention is not to converge on one thing at first meeting. |
| HW/HiSi | Even though we prefer path based measurements, don’t think a down-selection is needed at this stage. Suggestion for an updated proposal: In Rel-19 AI/ML based positioning, regarding the time domain channel measurements for model input, RAN1 discuss further study and select one alternative from the following: - Sample-based measurements, where the timing information is an integer multiple of sampling periods. - FFS: details including but not limited to: definition of starting point for the sample list, sampling period, number of samples - Path-based measurements, where the timing information is according to the detected path timing and may not be an integer multiple of sampling periods. FFS: granularity of path timing, number of paths (including potential extension from the existing 8 additional paths). [Moderator] 'select one' is not expected for this meeting. The selection will be done in a future meeting, after companies have provided their investigation results. |
| Fraunhofer | Depending on the specific cases, both sample-based measurements (option a) and path-based measurements (option b) should be considered. |
| Ericsson | Support. We can further discuss technical merit on both issues in the next meeting based on evaluations. |

### 2nd round / offline session

In the 1st round discussion, there was some confusion about the intention. The intention of the proposal is to clearly describe the two approaches, so that companies can study and bring investigation results to next meeting. Down-selection is not expected at this meeting.

The updated proposal is presented below, taking into account suggestions from companies including CATT and Huawei.

Proposal 3.2.3-1

In Rel-19 AI/ML based positioning, regarding the time domain channel measurements for model input, RAN1 study the following alternatives and make a decision on which alternative to adopt.

- Sample-based measurements, where the timing information is an integer multiple of sampling periods.

  - FFS: details including but not limited to: definition of starting point for the sample list, sampling period, number of samples

- Path-based measurements, where the timing information is according to the detected path timing and may not be an integer multiple of sampling periods.

  - FFS: granularity of path timing, number of paths (including potential extension from the existing 8 additional paths).

|  | Company |
| --- | --- |
| Support |  |
| Not support |  |

| Company | Comments |
| --- | --- |
| InterDigital | By which meeting do we want to make the decision by? If we think the alternatives increase or change in the future meetings, we should consider potential alternatives for now. |
| TCL | Can both the alternatives be adopted? We prefer to remove ‘and make a decision on which alternative to adopt ‘. |
| Nokia/NSB | Listing the options is ok. However, we are not yet in the time to decide between option a or b. |
| LG | Similar view with Nokia |
| ETRI | Support. We can continue to study and make a selection at the next meeting based on evaluations. |
| Apple | Also prefer to remove the text in red |
| Qualcomm | Agree to Nokia and InterDigital perspectives. The current version also puts too much stuff together. Let’s consider a simpler listing. Modified version: In Rel-19 AI/ML based positioning Case2b and Case3b, regarding the time domain channel measurements for model input, RAN1 study the following potential alternatives and make a decision on which alternative to adopt for expressing timing info while considering timing information of existing specifications as a starting point: - Sample-based measurements whose timing information is an integer multiple of sampling periods. - FFS: details including but not limited to: definition of starting point for the sample list, sampling period, number of samples - measurements whose timing information is not an integer multiple of sampling periods. - FFS: granularity of path timing, number of paths (including potential extension from the existing 8 additional paths). Note: Different potetnail alternatives can be applicable for Case2b and Case3b |
| Fujitsu | We are open to the study, if the workload is acceptable for normative work. Otherwise, early down-selection would be a better way for progress. |
| ZTE | We have the same question with TCL. Maybe we can say In Rel-19 AI/ML based positioning, regarding the time domain channel measurements for model input, RAN1 study the following alternatives and select one or both of the following alternatives. It’s our first meeting, we don’t have to block any possibilities. |
| Fraunhofer | We do not need a unified reporting scheme over all direct and assisted cases. For example, assisted cases refining existing measurements based on option b is fine. While case 3b direct positioning sample-based measurements (option a) can be the better option. |

Offline discussion:

Proposal 3.2.3-1a

In Rel-19 AI/ML based positioning, regarding the time domain channel measurements for model input, RAN1 investigate the following alternatives and make a decision on which alternative to adopt.

- Sample-based measurements, where the timing information is an integer multiple of sampling periods.

  - FFS: details including but not limited to: reference time for sample timing, sampling period, number of samples

- Path-based measurements, where the timing information is according to the detected path timing and may not be an integer multiple of sampling periods.

  - FFS: granularity of path timing, reference time for reporting path timing, number of additional paths (including potential extension from the existing 8 additional paths).

The issues to be studied include, but not limited to, the following:

      - Tradeoff of positioning accuracy and signaling overhead

      - Potential implementation issues to obtain the measurement values

      - Whether the same alternative applies to all sub-cases, or different alternative can be applied to different sub-case

Note: different sub-cases may have different issues.

Note: depending on the measurement choice for model input, additional information is also reported together with the timing information (i.e., power, phase).

### 3rd round discussion

Proposal 3.2.3-1a

In Rel-19 AI/ML based positioning Cases 3a and 3b, regarding the time domain channel measurements for model input, RAN1 investigate the following alternatives:

Alternative (a).  Sample-based measurements, where the timing information is an integer multiple of sampling periods.

  - FFS: details including but not limited to: reference time, sampling period, number of samples

Alternative (b).  Path-based measurements, where the timing information is according to the detected path timing and may not be an integer multiple of sampling periods.

  - FFS: details including but not limited to: reference time, granularity of path timing, number of paths

The issues to be studied include, but not limited to, the following:

      - Positioning accuracy comparison when having the same signaling overhead

      - Impact of gNB implementation to obtain the measurement values

      - Whether the same alternative applies to both Cases 3a and 3b or not

Note: different sub-cases may have different issues.

Note: In addition to timing information, the components for the channel measurement for model input may also include power and potentially phase (subject to further discussion). Companies to provide the details of the channel measurement in their investigation.

|  | Company |
| --- | --- |
| Support |  |
| Not support |  |

| Company | Comments |
| --- | --- |
| InterDigital | Case 3a does not apply to this proposal since it is AIML-assisted positioning. Why is Case 1 ruled out here? |
| Sony | Generally support. If it is applicable to other use-cases why not we state it here? Or at least add a bullet point: FFS the applicability to other use-cases. |
| ZTE | Generally fine for the proposal. For the last sentence, does that mean companies are suggested to do simulation and provide the measurement format, i.e., sample/path-based, number/granularity of path/sample, etc.? |
| Xiaomi | - If the intension of the proposal is for evaluation, why gNB-side model is prioritized? We suggest just focus on the evaluation itself and don’t mention the related sub-use case. The applicablility for each sub use case can be discussed later based on the investigation results. - In addtion, we think the first note is not necessary. |
| Nokia/NSB | As this proposal has the intention to investigate, in our understanding, there is no reason to prioritize specific use cases ( usually NW-side model cases). As RAN1, we must focus the study on cases with high impact for RAN1 (air interface between UE and NW). Our suggestion is to open the scope oft he proposal to all cases. |
| Fraunhofer | There may be scenarios where the LMF could benefit from a combination of traditional approaches alongside the AI/ML approach by utilizing aligned path-based measurements. Additionally, the LMF could choose to target sample-based measurements specifically for increased accuracy reporting. We suggest for now to consider these as options rather than strict alternatives. |
| samsung | - „time domain channel measurements for determining model input“, We see some benefits to keep 3a, the measurement is still to be defined, as in legacy pos design, the UL based and LMF based pos, the gNB gather the measurement to report to LMF, the measurement is still defined. The RAN4 will be also clearer to see the measurement to be test and define corresponding requirement. |

### Wednesday offline discussion

Proposal 3.2.5-1

In Rel-19 AI/ML based positioning Cases 2b, 3a and 3b, regarding the time domain channel measurements for model input, RAN1 investigate the following alternatives:

Alternative (a).  Sample-based measurements, where the timing information is an integer multiple of sampling periods.

  - FFS: details including but not limited to: reference time, sampling period, number of samples

Alternative (b).  Path-based measurements, where the timing information is according to the detected path timing and may not be an integer multiple of sampling periods.

  - FFS: details including but not limited to: reference time, granularity of path timing, number of paths

The issues to be studied include, but not limited to, the following:

      - Tradeoff of positioning accuracy and signaling overhead

      - Impact of gNB/UE implementation to obtain the channel measurement values

      - Whether the same alternative applies to all cases or not

Note: different sub-cases may have different issues.

Note: In addition to timing information, the components for the channel measurement for model input may also include power and potentially phase (subject to further discussion). Companies to provide the details of the channel measurement in their investigation. [Nokia: It is recommended to use [PDP] as a representative model input.]

FFS: Case 1/2a

| Company | Comments |
| --- | --- |
| InterDigital | As we commented above, this proposal is focusing on AIML based positioning. Therefore only case2b and case3b are applicable here. Therefore, case 2a should be removed from the main bullet and FFS should focus on case 1. |
| HW/HiSi | Not support. Case 3a should not be studied in our view, since it is by implementation. A question for clarification: What is meant with „The impact on gNB/UE implementation“? Does this bullet imply for the path based reporting method, to also study how to select the paths from the measured channel (and e.g. whether a unified manner across measurement entities could be needed)? If yes, it is our assessment that such an alignment across TRPs is not needed, whereas between Ues it could be needed. Therefore, Case 3b would be much simpler in this regard, and we would not agree to study for Case 2b, since it is not simply a mirrored version of 3b and would require significant extra efforts. Also, the channel measurements do not necessarilty need to be pnöy for model input, this is up to LMF implementation how to use them. They can also be an enhanced measurement report for a legacy method. Therefore, we should only mention channel measurements, not „model input“ This proposal and its intention needs further clarification. A generic comparisom without adressing the specific cases can be done as a first step for further clarification. We would suggest the following update as starting point for further discussion. Updated Proposal 3.2.5-1 In Rel-19 AI/ML based positioning Cases 2b, 3a and 3b, regarding the report of the time domain channel measurements for model input, RAN1 investigate the following alternatives: Alternative (a). Sample-based measurements, where the timing information is an integer multiple of sampling periods. - FFS: details including but not limited to: reference time, sampling period, number of samples Alternative (b). Path-based measurements, where the timing information is according to the detected path timing and may not be an integer multiple of sampling periods. - FFS: details including but not limited to: reference time, granularity of path timing, number of paths The issues to be studied include, but not limited to, the following: - Tradeoff of positioning accuracy and signaling overhead - Impact of gNB/UE implementation to obtain the channel measurement values - Whether the same alternative applies to all cases or not Note: different sub-cases may have different issues (if any). Note: In addition to timing information, the components for the channel measurement report for model input may also include power and potentially phase (subject to further discussion). Companies to provide the details of the channel measurement in their investigation. [Nokia: It is recommended to use [PDP] as a representative model input.] |
| Nokia/NSB | In general, we do not support the current version. As the the target is „investigate“ or „study“, we believe that there is no necessity to indicate specific use cases. Indicating specific use cases may delay the discussion because 2 reasons: - 1st priority cases vs 2nd priority cases - Indicating specific cases now means that in the next meeting, we may make decisions only in the specific mentioned use cases. |
| NTT DOCOMO | Generally fine with this proposal. Potential new/enhanced measurements for model input could be considered for all direct and assisted cases and spec. Imapct for each case can be further discussed. |

Proposal 3.2.5-1

In Rel-19 AI/ML based positioning Cases 2b, 3a and 3b, regarding the time domain channel measurements corresponding to model input, RAN1 investigate the following alternatives:

Alternative (a).  Sample-based measurements, where the timing information is an integer multiple of sampling periods.

  - FFS: details including but not limited to: reference time, sampling period, number of samples

Alternative (b).  Path-based measurements, where the timing information is according to the detected path timing and may not be an integer multiple of sampling periods.

  - FFS: details including but not limited to: reference time, granularity of path timing, number of paths

The issues to be studied include, but not limited to, the following:

      - Tradeoff of positioning accuracy and signaling overhead

      - Impact of gNB/UE implementation to obtain the channel measurement values. Companies describe their implementation details

      - Whether the same alternative applies to all cases or not

      - Applicability and necessity of specifying the Alternative(s) to different cases

Note: different sub-cases may have different issues.

Note: In addition to timing information, the components for the channel measurement for model input may also include power and potentially phase (subject to further discussion). Companies to provide the details of the channel measurement in their investigation.

Note: There is no intention to specify the purpose of channel measurements in the specification.

[Nokia: It is recommended to use [PDP] as a representative model input.]

FFS: Case 1/2a

## (Closed) Measurement types for model input (DP, PDP, CIR)

### Companies’ view from contribution

In the following, selected inputs from companies’ contributions are provided.

- InterDigital (R1-2401042) Proposal 3: If a CIR is to be specified, study how uncertainty related to CIR can be specified Proposal 4: For direct AI/ML based positioning, adopt RSRP and RSTD measurement as input for an AIML model for direct AIML positioning

- Huawei, HiSilicon (R1-2400145) Proposal 9 For the data collection for training and inference of Case 3b, consider the report of delay/power from gNB to LMF as the starting point. • Further discuss enhancements on the number and indication of paths to be reported and the quantization for reported delay and power. Observation 7: The model inference for Case 2b can be done in a similar manner as for Case 3b with measurements based on PRS and reports from the UE to the LMF.

- Xiaomi (R1-2401544) Proposal 1: If there is need to specify the model input type - Prioritize the PDP and DP - Further discuss if there is need to down-select between PDP and DP Proposal 2: If there is need for the model input specification, down select the following options for the time domain samples selection by comparing the positioning accuracy performance and the signaling overhead - Option 1: Select Nt consecutive time domain samples used as model input - Option 2: Select N't time domain samples with the strongest power as model input

- ZTE (R1-2400169) Proposal 5: At least for data collection with LMF-side model, support UE/TRP to report more detailed channel measurements, e.g., CIR.

- OPPO (R1-2400619) Proposal 13: For the model inference for NG-RAN node assisted positioning with LMF-side model (Case 3b) - gNB/TRP can do the measurement and report the results via existing NRPPa signaling - The existing measurement types are reused - FFS: whether or not to introduce new values (N) for the maximal number of additional paths (e.g., N > 8).

- CMCC (R1-2400317) - Proposal 3: For AI/ML based positioning, the potential spec impact of CIR report can be studied, and the impact of dimension of CIR on positioning accuracy can be evaluated in AI 9.2.4.1. - Proposal 4: For AI/ML based positioning, further study current positioning reporting measurement with some additional configuration and / or limitation.

- Google (R1-2400393) Proposal 3: Support to report CIR and L1-SINR for a set of configured PRSs to facilitate the NW-side model based positioning.

- CATT (R1-2400419) Proposal 3: For case 1, at least when UE-side model is trained by UE-side, UE generates the measurement for AI/ML model inference based on the PRS transmission. The input type or format is up to UE implementation. Proposal 7: For case 3a, at least when gNB-side model is trained by gNB/NW-side, gNB/TRP generates the measurement for AI/ML model inference based on the SRS-pos transmission. The input type or format is up to gNB/TRP implementation. Proposal 12: For case 3b, the measurement such as CIR/PDP/DP is generated and reported by gNB/TRP for determining the LMF-side AI/ML model input. Proposal 16: For case 2a, at least when UE-side model is trained by UE-side, UE generates the measurement for AI/ML model inference based on the PRS transmission. The input type or format is up to UE implementation. Proposal 21: For case 2b, at least PDP/DP is supported and reported to LMF side for determining the LMF-side AI/ML model input.

- CICTCI (R1-2400757) Proposal 6: For Case 3b, the measurement report should contain at least timing and power information of the channel response.

- Fujitsu (R1-2400767) Proposal 5 The following new measurements or enhanced measurements can be considered as the candidates of the model input of Case 3b during inference: - New measurements: at least for CIR/PDP/DP. - Enhanced measurements: at least for RSRPP.

- Lenovo (R1-2400923) Proposal 6: RAN1 to support the following one or more of the following new DL-based Direct AI/ML positioning measurements to form a fingerprint based on DL-PRS: - Support channel observation measurements, e.g., DL CIR measurements - Support DL channel profile measurements such as PDP. FFS the inclusion of additional channel profiles such as DL-based power-angle profile and/or angle-delay domain. - NOTE: Above measurements may be considered in conjunction with overhead reduction techniques Proposal 7: RAN1 to support the following one or more of the following new UL-based Direct AI/ML positioning measurements to form a fingerprint based on SRS for positioning: - Support channel observation measurements, e.g., UL CIR measurements - Support UL channel profile measurements such as PDP. FFS the inclusion of additional channel profiles such as UL-based power-angle profile and/or angle-delay domain. - NOTE: Above measurements may be considered in conjunction with overhead reduction techniques Proposal 9: Support the use of legacy positioning measurements for the following Direct AI.ML positioning cases to derive a fingerprint: - Case 1, 2b (DL-based): Support DL-RSTD, UE Rx-Tx time difference, DL-AoD, DL-PRS RSRP/RSRPP measurements - Case 3b (UL-based): Support UL-RTOA, gNB Rx-Tx time difference, UL-AOA and SRS RSRP/RSRPP measurements

- NTT DOCOMO (R1-2401108) Proposal 7: For direct AI/ML positioning at LMF side (case 2b and 3b), measurement report which contains timing/ power/ phase information of the channel response can be configured as measurement input type for model inference. - Further study the report format of UE/gNB reporting CIR/PDP/DP to LMF considering the signaling overhead reduction.

- OPPO (R1-2400619) Observation 1: The new measurement of CIR provides limited performance improvement compared to that of existing RSTD measurement. - The overhead of CIR is much higher than RSTD Observation 2: The existing reporting of RSRPP for additional paths can achieve the purpose and the performance same as or similar to PDP measurement. Proposal 2: For UE-assisted/LMF based positioning with LMF-side model (Case 2b) and NG-RAN node assisted positioning with LMF-side model (Case 3b), reuse the existing measurements of NR positioning as the baseline - FFS: whether or not to introduce new values (N) for the maximal number of additional paths (e.g., N > 8).

- Sharp (R1-2401172) Proposal 1: Support CIR/PDP as AI/ML model input for positioning. Proposal 2: For direct AI/ML positioning with LMF-side mode (case 2b and 3b), - introduce new IEs in LPP or NRPPa signaling to request new types of UE measurement reports (CIR/PDP). - introduce new IEs in LPP or NRPPa signaling to report the new types of UE measurement (CIR/PDP). - Consider the report format of truncated CIR/PDP including truncated length and position.

- Fraunhofer (R1-2401198) Proposal 2: Define new measurement, including sample-based time domain CIR to provide model input for cases 2b and 3b.

- Qualcomm (R1-2401432) Observation 13: For Case2b/3b, based on observations from Rel-18 Study Item, the enhancement in positioning accuracy due to increase of reporting size (i.e., number of paths/samples N’t) is small and does not justify the magnificent increase in reporting overhead. Proposal 10: For Case2b/3b, based on observations from Rel-18 Study Item, the increase of reporting size is not justified and no need to enhance reporting size (i.e., number of paths/samples N’t) beyond what is supported in existing specifications.

- Apple (R1-2401003) Proposal 5: To specify feedback of measurement data for both PRS and UL SRS for positioning, the following parameters may need to be configured: - The measurement type: delay Profile (timing only), Power Delay Profile (timing, power), Channel Impulse Response (timing, power and phase) - The number of measurements (taps): 8, 9, 16, 32, 64, [128], [256] (+1) - Feedback Format - First path parameters: first path magnitude, first path phase first path timing, timing granularity factor. - Additional path parameters - Relative time difference for additional paths - Time reporting granularity factor - Time stamp - Path quality - Additional path magnitude and phase - Option 1: additional path magnitude, additional path phase - Option 2: additional path relative magnitude, additional path phase Proposal 13: Enhance current measurement input reports to LMF to support feedback of the CIR, PDP, and DP.

- Nokia (R1-2400794) Proposal 3: For Case 1 direct AI/ML positioning, RAN1 to consider in the normative work that the input used for inference may not consider new measurements. Proposal 4: For Case 1 direct AI/ML positioning, the reference signal used for measurements corresponding to model input is only DL PRS. Observation 2: Legacy RSRPP measurement in the specifications already corresponds to PDP measurement with a limited number of taps. Proposal 5: In Case 1 direct AI/ML positioning, measurements corresponding to model inference input must consist of DL RSRPP as per its existing definition in the specifications (TS 38.214). FFS to include additional number of paths (e.g., to achieve desired positioning performance). Proposal 60: In Case 3a, reference signal used for measurements for inference input at the gNB side is only UL SRS for positioning. Proposal 61: In Case 3a, measurements corresponding to model inference input must consists of UL RSRPP as per its existing definition in the specifications. FFS to include reporting of additional number of paths (e.g., to achieve desired positioning performance). Proposal 69: In Case 3b, for inference input, RAN1 should not introduce new measurements, but use legacy UL RSRPP measurements. FFS necessary enhancements to UL RSRPP (e.g., number of additional paths).

- Ericsson (R1-2400101) Proposal 6 For Case 3b (1st priority) and Case 2b (2nd priority), RAN1 to study the tradeoff between feedback sizes and achievable positioning accuracy for prioritizing the support of different model input types (CIR, PDP). Observation 12 Multi-port complex-valued CIR samples for both Case 3b (1st priority) and Case 2b (2nd priority) require very large signaling sizes, which can cause significantly negative impacts on the radio and core networks. Observation 13 The phase information contained in the measured CIR samples may depend on not just the radio channel environment but also the random state of the node’s frequency generation circuits. In order not to degrade fingerprinting ML models using the CIR sample inputs, further investigation is needed on how to separate the two types of sources affecting the observed CIR phase. Proposal 7 Before CIR can be adopted as model input, RAN1 need to investigate whether fingerprinting ML models can handle CIR phase measurements, which vary not only with the radio channel environment but also with the transmitter/receiver circuits. Proposal 8 RAN1 to prioritize the signaling approach(es) and/or measurement definitions to support PDP and DP model input types for Case 3b (1st priority) and Case 2b (2nd priority). Proposal 9 RAN1 proceed with specification work for PDP and DP as model input, while CIR is pending further investigation.

### 1st round discussion

From 38.215, DL PRS-RSRPP and UL SRS-RSRPP are defined as follows.

5.1.35	DL PRS reference signal received path power (DL PRS-RSRPP)

| Definition | DL PRS reference signal received path power (DL PRS-RSRPP), is defined as the power of the linear average of the channel response at the i-th path delay of the resource elements that carry DL PRS signal configured for the measurement, where DL PRS-RSRPP for the 1st path delay is the power contribution corresponding to the first detected path in time. For frequency range 1, the reference point for the DL PRS-RSRPP shall be the antenna connector of the UE. For frequency range 2, DL PRS-RSRPP shall be measured based on the combined signal from antenna elements corresponding to a given receiver branch. For frequency range 1 and 2, if receiver diversity is in use by the UE for DL PRS-RSRPP measurements, the reported DL PRS-RSRPP value included in the higher layer parameter NR-DL-AoD-MeasElement for the first and additional measurements shall be provided for the same receiver branch(es) as applied for DL PRS-RSRP measurements. |
| --- | --- |
| Applicable for | RRC_CONNECTED, RRC_INACTIVE, RRC_IDLE |

5.2.6	UL SRS reference signal received path power (UL SRS-RSRPP)

| Definition | UL SRS reference signal received path power (UL SRS-RSRPP) is defined as the power of the linear average of the channel response at the i-th path delay of the resource elements that carry the received UL SRS signal configured for the measurement, where UL SRS-RSRPP for 1st path delay is the power contribution corresponding to the first detected path in time The reference point for UL SRS-RSRPP shall be: - for type 1-C base station TS 38.104 [9]: the Rx antenna connector, - for type 1-O or 2-O base station TS 38.104 [9]: based on the combined signal from antenna elements corresponding to a given receiver branch - for type 1-H base station TS 38.104 [9]: the Rx Transceiver Array Boundary connector. For frequency range 1 and 2, if receiver diversity is in use by the gNB for UL SRS-RSRPP measurements: - The reported UL SRS-RSRPP value for the first and additional paths shall be provided for the same receiver branch(es) as applied for UL SRS-RSRP measurements, or - The reported UL SRS-RSRPP value for the first path shall not be lower than the corresponding UL SRS-RSRPP for the first path of any of the individual receiver branches and the reported UL SRS-RSRPP for the additional paths shall be provided for the same receiver branch(es) as applied UL SRS-RSRPP for the first path. |
| --- | --- |

These definitions do not have limitation on how many additional paths can be reported. Thus the power measurements of Rel-18 evaluated PDP can be the same as the RSRPP, but the time domain measurement can be different, i.e., sample-based vs path-based (see discussion in section 3.2).

Nevertheless, it is common understanding that measurements of time and power (PDP/DP or RSRPP) should be supported for AI/ML based positioning. In contrast, the phase measurement required by CIR needs further investigation, according to companies input. Thus the proposal is provided below for LMF-side model (i.e., Case 2b, 3b).

- Moderator note: Case 1/2a/[3a] are to be discussed within the scope of training data collection.

- (a) corresponds to DP; (b) corresponds to PDP, RSRPP.

- Phase information reporting refers to CIR.

- Note: the exact definition of power information is subject to further discussion, see proposal in 3.5.

Proposal 3.3.2-1

For Case 2b and 3b, at least the following types of measurement report for time domain channel measurements are supported as input to LMF-side AI/ML model:

- A list of timing information;

- A list of paired timing information and power information.

FFS: whether phase information reporting is supported for case 2b and 3b.

|  | Company |
| --- | --- |
| Support | MTK (consider revision), TCL, Hw/HiSi, Pengcheng laboratory |
| Not support | vivo, Nokia/NSB, Fraunhofer |

| Company | Comments |
| --- | --- |
| Fujitsu | It is general fine for us. |
| MTK | 1, A general understanding for “timing” may indicate there is path. Since in earlier FL proposal, it is to further study sample-based and path based, we suggest to add a note that Note: the timings are the samples on the grid for sample based measurement, and are the paths off the grid for path based measurement. 2, We are not picky, but case 2b belongs to second priority. If FL proposal 2.2-1 is agreed then there is no problem for this |
| InterDigital | What does the list consist of? |
| ZTE | We prefer list all possible alternatives in the sub-bullet, wherein CIR has most comprehensive channel information. We can further discuss the potential model input format in following meetings. Suggested proposal is as follows: For Case 2b and 3b, at least the following types of measurement report for time domain channel measurements are supported as input to LMF-side AI/ML model: - A list of timing information; - A list of paired timing information and power information,; - A list of paired timing information with power and phase information. FFS: whether phase information reporting is supported for case 2b and 3b. [Moderator] (c) is pending the investigation on reporting the phase information though. |
| CMCC | Fine. We can then discuss how to report this list based on sample-based and/or path-based measurement. |
| vivo | 1. Question to moderator: “Moderator note: Case 1/2a/3a are discussed within the scope of training data collection.” We didn’t find the related proposal on measurement report type for Case 1/2a/3a in section 5. Could moderator point to us which proposal you are referring to? [Moderator] There is no specific proposal yet. Updated the sentence to "to be discussed...". The intention is to say, for Case 1/2a/[3a], training data collection need to deal with measurements, even though measurments do not need to be specified for inference. For Case 2b/3b, meausurement need to be specified on the interfaces for model inference. 2. During the SI, concern of OTA signaling overhead for CIR reporting is only for case 2b. Why now CIR requires further study for case 3b when OTA signaling for model input is not a concern? [Moderator] For Case 3b, all issues need to be checked also. For reporting overhead, size is not as critical as OTA. However, it's still a cost to carry larger payload. Since the NRPPa interface needs to carry data for all TRPs, the payload size can be quite large. A smaller size is preferred unless the cost is justified. 3. Rel-18 already support report phase information (DL RSCP and UL RSCP), why should we prevent the usage of such report for AI/ML positioning? |
| CATT | We are supportive to DP/PDP. At the same time, we think phase info could be beneficial according to Rel-18 SI results. Off course how much gain can be obtained heavily depends on the implementation. |
| NEC | We prefer to identify DP/PDP as mandatory input and identify CIR as an optional model input for the case that high accuracy positioning is required. |
| Xiaomi | We prefer PDP and DP, whether down-slection is needed can be further discussed |
| Nokia/NSB | As this is topic that is cross related to all use cases studied in the SI, we propose the following rewording: Proposal 3.3.2-1 (Fully Update) The following types of measurement report for time domain channel measurements are supported as input to AI/ML model in all use cases: (a) A list of timing information; (b) A list of paired timing information and power information. |
| Qualcomm | Let’s start with Case3b until more progress is done for aligning companies on cases prioritization and handling in Rel-19 (i.e., Proposal 2.2-1). Modified proposal: For Case 2b and 3b, at least the following types of measurement report for time domain channel measurements are supported as input to LMF-side AI/ML model: - A list of timing information; - A list of paired timing information and power information. FFS: Measurements size for (a) and (b) (e.g., number of measurements N’t) while considering tradeoff positioning accuracy requirement and signalling overhead based on Rel-18 SI observations. |
| Fraunhofer | We do not think using Rel-18 measurements as a starting point for discussion is the best approach. The conventional methods did not utilize multipath information as AI/ML techniques. |
| Pengcheng laboratory | Support. ZTE’s suggestions are fine for us. Support the phase information report since the phase information can be useful for carrier-based positioning and near-field localization. |
| Ericsson | Support. To ZTE: we prefer to exclude phase from the proposal for now (it is not precluded, since the proposal says „at least“), since we are discussing CIR in a section 3.6. To vivo: the overhead of CIR reporting vs benefit should at least be discussed. Note that multipath reporting for phase is not supported. |

### 2nd round / offline session

Proposal 3.3.2-1 is updated to take into companies input including Nokia/NSB, Qualcomm, MediaTek.

Proposal 3.3.3-1

For AI/ML based positioning, the following types of time domain channel measurements are supported as AI/ML model input:

- A list of timing information;

- A list of paired timing information and power information.

Note: for sample-based measurement, the timing information is an integer multiple of sampling periods; for path-based measurements, the timing information is according to the detected paths and may not be an integer multiple of sampling periods.

FFS: Measurement size for (a) and (b) (e.g., number of measurements N’t) while considering the tradeoff between positioning accuracy requirement and signalling overhead.

|  | Company |
| --- | --- |
| Support | TCL |
| Not support |  |

| Company | Comments |
| --- | --- |
| NTT DOCOMO | Genrally fine with the proposal, not sure why “at least“ is removed. |
| InterDigital | With the newly added notes, we don’t need “a list of” in both types. |
| Nokia/NSB | Question to FL: (considering examples for each optoin a or b)Why do we need DP if it is a subset of PDP ?. |
| Qualcomm | Positive with Docomo and Nokia points. To avoid unecessary controversarries and ease discussion, we suggest the following modified version: Modified proposal For AI/ML based positioning Case2b and Case3b, the following types of time domain channel measurements are supported as AI/ML model input: - A list of timing information; - A list of paired timing information and power information. Note: for sample-based measurement, the timing information is an integer multiple of sampling periods; for path-based measurements, the timing information is according to the detected paths and may not be an integer multiple of sampling periods. FFS: Measurement size for (a) and (b) (e.g., number of measurements N’t) while considering the tradeoff between positioning accuracy requirement and signalling overhead, and considering number of measurements N’t in existing specifications (i.e., up to 9 measurements) as a starting point. |
| Fujitsu | It is fine for us in general. |
| LG | Generally fine but the description of the content is enough, i.e., (a) timing information (b) paired timing/power information |
| ETRI | We prefer to include phase information in the listed cases to reflect CIR as one of the model input types. (c) A list of paired timing, power and phase information. |
| ZTE | Thanks for FL’s reply. But in this updated proposal, ‘at least’ is not included. We are wondering whether the note is necessary, this proposal is discussing the type of model input instead of measurements. |
| Apple | With FFS for CIR removed, then leave ‘at least‘ |

Offline discussion:

Proposal 3.3.3-1

At least for AI/ML based positioning Case 2b and 3b, at least the following types of time domain channel measurements are supported for reporting:

- timing information;

- paired timing information and power information.

FFS: the applicability to Cases 1/2a/3a

Note: for sample-based measurement, the timing information is an integer multiple of sampling periods; for path-based measurements, the timing information is according to the detected paths and may not be an integer multiple of sampling periods.

FFS: Measurement size for (a) and (b) (e.g., number of measurements N’t) while considering the tradeoff between positioning accuracy requirement and signalling overhead.

## Timing measurement for model input

### Companies’ view from contribution

In the following, selected inputs from companies’ contributions are provided.

- Intel (R1-2400377) Proposal 8: - For representation of the timing information for CIR/PDP/DP to serve as model input, solutions to convey the information on propagation delay should be pursued. - FFS: The propagation delay information is captured as part of the encoded timing information itself related to CIR/PDP/DP or is conveyed separately in addition to the timing information related to CIR/PDP/DP. Proposal 9: - Support relative differential time instance representation for representation of timing information of the channel response for model input in AI/ML based positioning using CIR/PDP/DP. Proposal 10: - The existing representation/quantization framework for timing-based metric reporting (e.g., RSTD, RTOA), with the smallest value of ReportingGranularityfactor (‘k) of -6 as introduced in Rel-18, can be reused for representation of timing information of the channel response for model input in AI/ML based positioning

- MediaTek (R1-2401137) Proposal 2b-1: RAN1 to agree that the problem of the reception boundary misalignment between the measurements for training data and the measurements for inference needs to be resolved Proposal 2b-2: We consider the solution with “channel response offset”. In that, the channel responses from all the TRPs are shifted in time with the same amount. This amount reindexes the first path delay to be at t=0 within the channel response derived from the reference TRP signal. This method may need to determine the first path delay within the channel response derived from the reference TRP signal Proposal 2b-3: To solve the reception boundary misalignment problem, and if “channel response offset” is not defined, UE could report the channel responses to data collection entity with identifier to indicate whether the measured channel responses are based on same reception boundary

- Ericsson (R1-2400101) Proposal 10 Introduce DL RTOA reference time as an absolute time to facilitate training data collection and model inference for DL. Proposal 11 For model input with embedded timing information (e.g., DP, PDP), for training data collection and model inference, the timing measurements are represented with relative timing values, which are relative to an absolute time. Proposal 12 For both direct AI/ML positioning and AI/ML assisted positioning, for uplink measurements as model input, the timing information are relative to UL RTOA reference time. Proposal 13 For both direct AI/ML positioning and AI/ML assisted positioning, for downlink measurements as model input, the timing information are relative to DL RTOA reference time.

### 1st round discussion

Based on the input from companies, the reference time or time boundary problem needs to be resolved. The issue exists for training data collection and model inference. Information should be provided so that measurement for training data and the measurement for inference can be aligned.

For UL measurement, UL RTOA reference time is already defined in existing specification. Hence this can be re-used to solve the problem for AI/ML positioning Case 3a/3b.

For DL, Intel and Ericsson proposed to use relative time.  Further study from more companies is needed.

TS 38.215, 5.2.2	UL Relative Time of Arrival (TUL-RTOA)

| Definition | The UL Relative Time of Arrival (TUL-RTOA) is the beginning of subframe i containing SRS received in Reception Point (RP) [18] j, relative to the RTOA Reference Time [16]. The UL RTOA reference time is defined as T0+tSRS, where - T0 is the nominal beginning time of SFN 0 provided by SFN Initialization Time [15, TS 38.455] - tSRS=10nf+nsf×10-3, where nf and nsf are the system frame number and the subframe number of the SRS, respectively. … |
| --- | --- |

Proposal 3.4.2-1

For channel measurement as model input for AI/ML positioning, the representation of timing information needs to support alignment between measurement for training data and measurement for model inference.

|  | Company |
| --- | --- |
| Support | Fujitsu, ZTE, TCL, CATT, [HwHiSi], Pengcheng laboratory,Ericsson |
| Not support | Nokia/NSB |

| Company | Comments |
| --- | --- |
| mtk | 1, if we agree on adding a note in 3.3.2, then the “timing” here doesn't need to be further addressed, which is the samples on the grid and paths off the grid 2, the DL and UL are discussed separately 2, if this is agreed, maybe we can further list some options for study |
| InterDigital | Does this need to be discussed now? This relates to alignment condition between training data and inference. Impact of this change on NR positioning needs to be studied also. |
| ZTE | We agree that training data and inference input should be aligned, and as pointed by FL, the model input of UL and DL should be further studied. If this is going to be discussed, it should be clarified that this is different from the NW-side or UE-side additional conditions, just to make sure the format of training data and inference input should be aligned. |
| vivo | Question for clarification, what’s the intention of this proposal? .ame time information measurement report format for training and inference? It’s not clear what’s the specification impact if this proposal is agreed. [Moderator] The intention of the proposal is that the timing info representation has to comply with this requirement. This affects how the timing info is reported from gNB to LMF for Case 3b, for example. |
| Nokia/NSB | We propose to delay the discussion. Maybe after our first offline discission, we will get a better shape to make decisions on it. Question to FL, what is the intention of the proposal ?. [Moderator] The intention of the proposal is that the timing info representation has to comply with this requirement. |
| OPPO | We agree that this alignment is needed. However, we need to study whether any additional mechanism/specification is needed when we define the timing infromaiton for reporting. Maybe there is no additional specification impact. Thus, we perfer to defer this discussion. |
| Qualcomm | The issue on reference time or time boundary problem for model input is not clear. Would you please provide more clarifications? |
| Hw/HiSi | It seems fine but would like to have clarified for which cases this might be an issue: From our understanding, if this is an issue it might occur for PRS measurement reports, since the reference for the timing information can be selected by the UE as the timing from one reference TRP. For SRS measurements this issue would not occur. Is this is the understanding of the FL/other companies? [Moderator] The issue need to be addressed for both UL and DL, since existing spec does not have channel measurement report as model input. Agree that for UL the issue is easy to resolve, since uplink already defined the "UL RTOA reference time", which can be used as reference time for model input as well. |
| Fraunhofer | Agree with interdigital, this issue can be discussed once 3.2/3 is resolved |
| Ericsson | Support. In our understanding, the purpose of this proposal is to frame the discussion on what is needed to represent the timing so that training data can be properly matched to the model input. More concretely, we think it is important that the basis for time of arrival measurements in model input and training data can be aligned. |

Proposal 3.4.2-2

For UL channel measurement as model input in Case 3a and 3b, the timing information is represented relative to the UL RTOA reference time T0+tSRS as defined in 38.215.

|  | Company |
| --- | --- |
| Support |  |
| Not support | ZTE, Nokia/NSB |

| Company | Comments |
| --- | --- |
| mtk | 1, generally okay, but the reference time is not just for “RTOA”, since it has determined the path. If the model input for LMF model is sample based? It is like, two different measurement (sample based and path based) are aligned to use the same reference time, currently named as RTOA reference time. We are okay to use “RTOA reference time” for both measurements |
| ZTE | This should be discussed after proposal 3.4.2.1 is agreed. And further more, only 3 companies provided views for this problem, we can wait for more companies’ investigation. We are open to further study. |
| CATT | In general we think this is straightfoward once we agree to adopt UL RTOA. |
| Nokia/NSB | We do not agree to arise this topic in the first online session. Some aspects need to be discussed offline |
| Qualcomm | It is not clear why there is a need to align model input for Case3a. Let’s first finalize data collection entity in Section 5.2 |
| HW/HiSi | Tentatively positive, but this seems onl to apply for case 3b? |

### 2nd round / offline session

For Proposal 3.4.2-1, it's clarified that the alignment is about reference time alignment (i.e., boundary alignment).

Proposal 3.4.2-1

For channel measurement as model input for AI/ML positioning, the representation of timing information needs to support reference time alignment between measurement for training data and measurement for model inference.

- The reference time alignment needs to be provided for UL and DL, respectively.

|  | Company |
| --- | --- |
| Support | TCL |
| Not support |  |

| Company | Comments |
| --- | --- |
| InterDigital | Probably better to discuss this proposal later once we have clear ideas about channel measurements as model input for AI/ML positioning. |
| Nokia/NSB | Clarification to FL. Is the intention to set a requirement that for AIML inference, the timing information should be based on Absolute timing information?. |
| Qualcomm | We also have equivalent confusion as Nokia indicated. We need first to understand the problem with existing timing references Modified version For channel measurement as model input for AI/ML positioning Case2b and Case3b, study whether the representation of timing information needs to support reference time alignment between measurement for training data and measurement for model inference, while considering existing time referencing for UL and DL as starting point. - The reference time alignment needs to be provided for UL and DL, respectively. |
| ZTE | Prefer to postpone this discussion and wait for further investigation. We believe there will be timing timing measurement discussion for both DL and UL. |
| Lenovo | This deals with a 2nd level detail between training and inference for only one case, i.e., Cases3b. First we would need to define what constitutes a channel observation measurement in the context of Case 1/2b/3b and then further discuss such details |

### 3rd round discussion

38.215 clause 5.2.2:

Proposal 3.4.2-2

For AI/ML based positioning Case 3a and 3b, for channel measurements used to determine model input, the timing information is represented using the UL RTOA reference time T0+tSRS as defined in 38.215.

|  | Company |
| --- | --- |
| Support |  |
| Not support | InterDigital |

| Company | Comments |
| --- | --- |
| InterDigital | We have not made an agreement for potential model input for case 3a, so case 3a should not be mentioned here. Is UL RTOA the only measurement applicable here? What about gNB Rx-Tx time? We should list potential candidates for both (a) and (b) in the agreement below. Agreement - For AI/ML based positioning case 3b, at least the following types of time domain channel measurements are supported for reporting: - timing information; - paired timing information and power information. |
| ZTE | In current TS 38.455, uplink RTOA measurement report is path-based: IE/Group Name; Presence; Range; IE Type and Reference; Semantics Description; Criticality; Assigned Criticality. CHOICE UL RTOA Measurement; M; -. >k0; INTEGER (0.. 1970049); TS 38.133 [16]. >k1; INTEGER (0.. 985025); TS 38.133 [16]. >k2; INTEGER (0.. 492513); TS 38.133 [16]. >k3; INTEGER (0.. 246257); TS 38.133 [16]. >k4; INTEGER (0.. 123129); TS 38.133 [16]. >k5; INTEGER (0.. 61565); TS 38.133 [16]. Additional Path List; O; 9.2.41; This IE is ignored if the Extended Additional Path List IE is included; -. Extended Additional Path List; O; 9.2.74; YES; ignore. TRP Rx TEG Information; O; 9.2.85; YES; ignore. If path-based measurement is agreed, the reference time can be the same as current definition. Otherwise, we need to further discussion. |

### Wednesday offline discussion

Proposal 3.4.5-1

For AI/ML based positioning Case 3b, for gNB channel measurements reported to LMF to determine model input, the timing information is represented relative to a reference time. The reference time supports alignment of channel measurement data between training and inference.

- FFS: details of the reference time

| Company | Comments |
| --- | --- |
| Intel | We would suggest to remove the sentence in red font. It is not entirely accurate since the reference time is necessary even for just training or inference to align the measurements across different gNBs just as in non-AI/ML positioning. |
| InterDigital | Same comment as Intel. We would like to make clarification on the red sentence. If the reference time can be used for alignment purpose, it may be better to add “FFS : Whether the reference time can be used to support alignment between training and infererence” to the proposal. |
| HW/HiSi | We see no need for this proposal. The legacy timing reference can be re-used for the same TRP, and there is no need to further align reference time across different TRPs (if this is the intention of this proposal?). As long as the same reference time for the same TRP between training and inference is used there should not be an issue for Case 3b. Furthermore, the purpose of the report „to determine the model input“ is transparent to the gNB and should be removed. Our first preference is to not have this proposal at all. As a compromise, we would suggest: Update: Observation For AI/ML based positioning Case 3b, for gNB channel measurements reported to LMF to determine model input, the timing information reported from one TRP is represented relative to a reference time for this reporting TRP. The reference time supports alignment of channel measurement data between training and inference for this reporting TRP. FFS: details of the reference time |
| Nokia/NSB | We tend to agree with HW. We need further discussion on this proposal. |
| NTT DOCOMO | Generally fine with the direction. The red sentence can be replaced by FFS suggested by ID. |

Proposal 3.4.5-1

For AI/ML based positioning Case 3b, for gNB channel measurements reported to LMF to determine model input, the timing information is represented relative to a reference time. The reference time supports alignment of channel measurement data between training and inference.

- FFS: details of the reference time

## Power measurement for model input

### Companies’ view from contribution

In the following, selected inputs from companies’ contributions are provided.

- Intel (R1-2400377) Proposal 11: - As a starting point, consider the existing definitions of DL PRS-RSRPP and UL SRS-RSRPP to represent power information for CIR/PDP/DP as AI/ML model input. - FFS: Further adaptations to the existing definitions.

- Ericsson (R1-2400101) Note: The requirements in this clause are derived based on the difference between the estimated PRS-RSRPP-r19 compared to the ideal PRS-RSRPP defined as RSRPPp∝akHa[k]expj2πDpkNIFFT2 Where: Ha[k] is the effective channel frequency response (over Res occupied by PRS) measured without receiver noise on antenna port a. Dp is the exact delay of the p-th path in the channel model.. Proposal 3 For the model input types for Case 3b (1st priority) and Case 2b (2nd priority), consider input based on DP or PDP samples containing sample powers summed over all receive antenna ports. Observation 16 For Case 2b/3b, the need for increased RSRP/RSRPP resolution for UE/gNB reporting should be evaluated.

### 1st round discussion

Regarding power information, Intel suggested to reuse the power information defined for DL PRS-RSRPP and UL SRS-RSRPP as a starting point. Ericsson suggested to use total power (i.e., summation over the RX antenna port). Regardless, companies have used to power information defined in the existing specification as a starting point for investigation. Thus the following is proposed.

Proposal 3.5.2-1

For Case 2b and 3b, RAN1 investigate the power information in measurement report, starting with the power information defined for DL PRS-RSRPP and UL SRS-RSRPP.

|  | Company |
| --- | --- |
| Support | Fujitsu, ZTE, CMCC, TCL, CATT, NEC, Pengcheng laboratory, Ericsson |
| Not support | Nokia/NSB |

| Company | Comments |
| --- | --- |
| MTK | For RSRPP, path has been determined. We have not agreed using samples or paths. Then for the study, we suggest to keep the wording short by removing the last sentence: Proposal: For Case 2b and 3b, RAN1 to investigate the power information in measurement report |
| Nokia/NSB | We agree with the direction of the proposal. However, we have some questions to FL, is the intention to promote a new study on this regard ?. [Moderator] Yes. Since some companies are studying potential ndication on power, this proposal says that DL PRS-RSRPP and UL SRS-RSRPP should be used as starting point. For example, most of the definition of DL PRS-RSRPP and UL SRS-RSRPP can be re-used; only consider enhancements where necessary. |
| OPPO | Support |
| Qualcomm | We think the power information in existing specifications can be reused as a starting point and is sufficient. It is not clear what companies have to investigate. |
| HW/HiSi | Agree with QC. |
| Fraunhofer | Same as for timing, this issue can be discussed once 3.2/3 is resolved |
| Ericsson | Ok. |
| NTT DOCOMO | The association between power information and corresponding timing information need to be considered. Besides, whether the power measurements of PDP can be equivalent to RSRPP should be further clarified. |
| Lenovo | Ok with MTK’s revised update to keep it general, Is the intention of the proposal to define a common power measurement for CIR/PDP? |

## Phase measurement for model input

### Companies’ view from contribution

In the following, selected inputs from companies’ contributions are provided.

- Intel (R1-2400377) Proposal 12: - As a starting point, consider extending the existing definitions of DL RSCP and UL RSCP to represent phase information for CIR as AI/ML model input to include phase information for the additional detected paths in the CIR as model input (if supported). - FFS: Further adaptations to the existing definitions.

- MediaTek (R1-2401137) Proposal 2a-7: Deprioritize to report the phase of the selected samples in the channel response measurement Proposal 2c-1: Further study the impact of geography independent term in the phase part of CIR, for example the initial phase mismatch between TX and RX oscillator

- Qualcomm (R1-2401432) Observation 6: For Case2b/3b, based on observations from Rel-18 Study Item, no clear benefits observed for reporting additional phase information (e.g., CIR) when considering trade-off between accuracy and reporting overhead. Proposal 9: For Case2b/3b, no support for reporting additional phase information (e.g., CIR) for model input running at LMF side.

- Ericsson (R1-2400101) Observation 13 The phase information contained in the measured CIR samples may depend on not just the radio channel environment but also the random state of the node’s frequency generation circuits. In order not to degrade fingerprinting ML models using the CIR sample inputs, further investigation is needed on how to separate the two types of sources affecting the observed CIR phase. Proposal 7 Before CIR can be adopted as model input, RAN1 need to investigate whether fingerprinting ML models can handle CIR phase measurements, which vary not only with the radio channel environment but also with the transmitter/receiver circuits.

### 1st round discussion

Based on the proposals and the evaluation results submitted, the following is proposed to capture the issues listed for phase measurement. Companies are encouraged to study CIR further, taking these issues into consideration.

Proposal 3.6.2-1

For channel measurement as model input for Case 2b/3b, RAN1 investigate the necessity of reporting phase information, considering the following issues:

- tradeoff of positioning accuracy and signaling overhead

- whether fingerprinting ML models can handle CIR phase measurements, which vary not only with the radio channel environment but also with the transmitter/receiver circuits

- specification impact

|  | Company |
| --- | --- |
| Support | Fujitsu, mtk, ZTE, CMCC, TCL, Pengcheng laboratory, Ericsson |
| Not support | Vivo, Nokia/NSB, Fraunhofer |

| Company | Comments |
| --- | --- |
| InterDigital | Is the intention to report more phase information? RSCP and RSCPD are already defined in TS 38.215. Carrier phase measurement reporting only for the first path is supported in R18. |
| ZTE | We prefer adding one more subbullet: For channel measurement as model input for Case 2b/3b, RAN1 investigate the necessity of reporting phase information, considering the following issues: - tradeoff of positioning accuracy and signaling overhead - whether fingerprinting ML models can handle CIR phase measurements, which vary not only with the radio channel environment but also with the transmitter/receiver circuits - specification impact - Other aspects are not precluded |
| vivo | 1. During the SI, concern of OTA signaling overhead for CIR reporting is only for case 2b. Why now CIR requires further study for case 3b when OTA signaling for model input is not a concern? 2. It’s not clear what to be investigated for “whether fingerprinting ML models can handle CIR phase measurements” when Rel-18 already support report phase information (DL RSCP and UL RSCP) for the first path. [Moderator] Rel-18 phase information is different from the phase of CIR. Also Rel-18 DL RSCP and UL RSCP require PRU to calibrate, but CIR does not |
| CATT | OK to consider, but some of them seems not important/valid. For example, for case 3b, signaling overhead is not an issue since it is not in air interface. |
| NEC | The evaluation in Rel-18 has shown that identify CIR as model input can achieve positioning accuracy enhancement comparing with input PDP/DP. So it may be significant to identify CIR as an optional model input at least for the high accuracy scenario. Moreover, can we merge this issue with issue 3.4? |
| Nokia/NSB | As the target related to this proposal is only Case 3b, we agree with vivo that OTA ndicatio for model input is not a concern for Case 3b. We suggest not to arise this proposal in the first online session and lets discuss in the upcoming offline sessions . |
| OPPO | Support |
| Qualcomm | We are fine with the general direction of the proposal. Please consider an equivalent proposal for measurement size (e.g., number of measurements N’t) at least for tradeoff of positioning accuracy and reporting overhead. |
| HW/HiSI | If this is not a problem for Case 3b (see vivo comment), maybe the ndication here can be started again after the use case prioritization has been agreed? |
| Fraunhofer | the second bullet TX/Rx impairments is a general consideration and relevant for timing as well ( that’s why Timing Error Groups (TEG) are introduced in Rel-17). |
| Ericsson | Support. To vivo: we think it needs to be clarified whether the CIR can be matched to training data when the channel and antenna response effect on the phase are considered. Does the training data need to capture all phase variations at tx and rx? |

### 2nd round / offline session

Proposal 3.6.3-1

For channel measurement as model input for AI/ML positioning, RAN1 investigate the necessity of reporting phase information so that CIR can be adopted as model input, considering the following issues:

- tradeoff of positioning accuracy and signaling overhead

- whether fingerprinting ML models can handle CIR phase measurements, which vary not only with the radio channel environment but also with the transmitter/receiver circuits

- specification impact

- Other aspects are not precluded

|  | Company |
| --- | --- |
| Support | TCL, Fujitsu |
| Not support | Fraunhofer |

| Company | Comments |
| --- | --- |
| Qualcomm | Generally ok with direction of the proposal. Please add Case2b and Case3b. |
| Nokia/NSB | If the intention is to study, we are ok with the direction. We suggest to avoid to mention any specific case in the proposal. |
| Apple | Was under the impression that the trade-off in accuracy and overhead was addressed in the study item and may depend on the complexity I AI/ML model. |
| Lenovo | Support, Also agree with the intention to focus the proposal AI/ML Direct Pos. cases. |
| Fraunhofer | From the moderator’s response to vivo’s comment, it seems there is some confusion. In Rel-18 phase measurements, a PRU is required to resolve ambiquities for the first path. However, for AI/ML which involves the CIR model input, a PRU is not needed. We suggest to handle the CIR in 3.3, which is a list of paired timing, power and phase information. |

### 3rd round discussion

Proposal 3.6.3-1

For AI/ML positioning Case 3a and 3b, RAN1 investigate the necessity of reporting phase information for supporting CIR as channel measurement for model input, considering the following issues:

- tradeoff of positioning accuracy and signaling overhead

- whether fingerprinting ML models can handle CIR phase measurements, which vary not only with the radio channel environment but also with the transmitter/receiver circuits

- specification impact

- Other aspects are not precluded

|  | Company |
| --- | --- |
| Support |  |
| Not support |  |

| Company | Comments |
| --- | --- |
| InterDigital | Why are we restricting to case 3a and case 3b only? CIR should be applicable to all cases. Even for case 1, we should discuss whether it is feasible for a UE to derive CIR based on current measurements. |
| Sony | Generally support but extend to other cases whenever it is possible. We have similar view as IDG that CIR can be generic and applicable to any other use-cases. |
| ZTE | We also think CIR should be supported for all cases. OK to investigate the benefit of CIR report. |
| Xiaomi | Similar view with IDC |
| Nokia/NSB | Ok to focus on the investigation (without doing any decision yet). However, we do not agree to focus only on specific use cases. At least the priority cases (e.g., Case 1) must be considered. |
| Fraunhofer | The statement regarding the variation of CIR with both the radio channel environment and the transmitter/receiver circuits appears unclear. It can be replaced by evaluating the conditions under which CIR can be effectively applied in AI/ML so companies can indicate these specific conditions where CIR proves to be effective or ineffective for AI/ML purposes. |
| Pengcheng laboratory | Generally fine with the proposal, but we also think CIR should be supported for all cases since it can achieve a better positioning accuracy according to TR 38.843. Of course it’s necessary to consider the trade-off between the positioning accuracy and ndicatio overhead. |
| Samsung | - For determing model input - „tradeoff of positioning accuracy and signaling overhead including means the reduce signaling overhead“, the reason is, if there is ways to reduce the overhead, to ease the concern on the overhead, it should be considered. |

### Thursday offline discussion

Proposal 3.6.5-1

For AI/ML based positioning Case 3a and 3b, RAN1 investigate the necessity of reporting phase information to supporting CIR as channel measurement for determining model input, considering the following issues:

- tradeoff of positioning accuracy and signaling overhead

- whether fingerprinting ML models can handle CIR phase measurements, which vary not only with the radio channel environment but also with the transmitter/receiver circuits

- specification impact

- Other aspects are not precluded

Proposal 3.6.5-1a

For AI/ML based positioning Case 3a and 3b, RAN1 investigate the necessity of reporting phase information to supporting CIR as channel measurement for determining model input, considering the following issues:

- tradeoff of positioning accuracy and signaling overhead, the impact of transmitter and receiver implementation

- whether fingerprinting ML models can handle CIR phase measurements, which vary not only with the radio channel environment but also with the transmitter/receiver circuits

- specification impact

- Other aspects are not precluded

Note: the phase information may be used in different ways, e.g., a phase value for the first path; triplet of {timing information, power information, phase information} for CIR

## Assistance information

### Companies’ views

In the following, selected inputs from companies’ contributions are provided.

- MediaTek (R1-2401137) Proposal 3-2: Support UE to request TRP coordinate under UE based positioning with direct AI/ML method if NW is not provided. In this way, the UE may perform the model monitoring, and calibrate the results between the legacy and the direct AI methods Proposal 3-3: Support local coordinate, which are “Local 2D point with uncertainty ellipse” and “Local 3D point with uncertainty ellipsoid”, for location information reporting within UE based positioning with AI/ML method Proposal 3-4: The location source is to indicate using which positioning method for location estimate by UE to the LMF. The direct AI/ML method leverages the finger-printing concept and therefore we propose to add “dl-aiml-fp-r18”. Proposal 3-5: Support UE to request the NW to provide the training data with a requested range of the induced delays to deal with the generalization problem due to sync error Proposal 3-6: Provide RTD-info to UE when UE is under UE based mode with AI/ML method

- Xiaomi (R1-2401544) Proposal 11: If multiple data formats are specified, LMF configure the detailed data format for collection in Case 3b Proposal 12: Leave RAN3 to handle the collected data report to LMF for Case 3b Proposal 13: For Case 3b, RAN1 is responsible to identify whether there is any need for assistance information from UE and what kind of assistance information is needed Proposal 21: In case 1, there is no specification impact on the delivery of input data for inference or the inference output data Proposal 22: For inference in Case 3a, leave RAN3 to assess whether is any specification impact on deliver the input from TRP to the associated gNB and deliver the input from one gNB to another gNB in different AI model deployment options. Proposal 24: For inference in Case 3b - Input data format for inference need to be specified and common input data format can be used for both data collection and inference - If multiple input data formats are defined, LMF configure the input data format for inference - The ndicatio exchange between gNB and LMF is left to RAN3

- NEC (R1-2400469) Proposal 9: Specific the period of validity of measurement corresponding to the model input to overcome the overhead of measurement in Rel-19.

# Model output for assisted AI/ML positioning

This section discusses model output for assisted AI/ML positioning and potential measurement enhancements.

## Type of information for model output

Regarding the type of information to generate for model output, the following types are proposed by companies:

- LOS/NLOS indicator;

- Timing information (e.g., TOA, RSTD)

- Angle information

- RSRPP

It is also proposed to attach the following information to the measurement report for model output of AI/ML assisted positioning.

- “made with AI” indicator

- Time stamp

### Companies’ views

In the following, selected inputs from companies’ contributions are provided.

- Nokia (R1-2400794) Proposal 63: For Case 3a, the model inference output may consist of at least ToA estimation and LOS/NLOS indication. Proposal 64: In Case 3a, gNB reports its inference output to LMF, using NRPPa signaling.

- Sony (R1-2400845) Proposal 9: Support LOS/NLOS indication as one of the outputs of AI/ML assisted positioning.

- Lenovo (R1-2400923) Proposal 11: For Cases 2a and 3a, at least support an enhanced LOS/NLOS indicator for model inference output.

- Huawei, HiSilicon (R1-2400145) Proposal 8 For the model inference of Case 3a, the legacy reporting for LOS/NLOS indication and/or timing information can be re-used to carry the AI/ML model output(s). • Discuss if there is a need to make the LMF aware of that the report has been generated with AI/ML and specify if necessary. • Other potential enhancements for the measurement reporting would need further justification. Observation 6: The model inference for Case 2a can be done in a similar manner as for Case 3a with measurements based on PRS and reports from the UE to the LMF.

- ZTE (R1-2400169) Proposal 7: For case 1, case 2b and case 3b (direct AI/ML positioning), no more extra types of model output is required in addition to the output listed in TR 38.843. Proposal 8: For case 3a, one or more types of model output can be supported for AI/ML assisted positioning, e.g., LOS/NLOS identification, timing and/or angle of measurement Proposal 9: For case 2a, the model output based on RSTD is prioritized over TOA for timing estimation. - Other intermediate features in addition to the output listed in TR 38.843 are not precluded. Proposal 10: For case 2a, support PRS-RSRPP value(s) at least for the first path as model output.

- CMCC (R1-2400317) - Proposal 6: For AI/ML based positioning, whether the reported measurement is AI based could have a indication.

- CATT (R1-2400419) Proposal 17: For case 2a, reuse legacy positioning measurement (e.g. RSTD) for reporting UE model output to LMF.

- Xiaomi (R1-2400544) Proposal 23: For the inference output type in Case 3a - Prioritize the specification discussion on TOA and LOS/NLOS indicator - Further discuss whether existing ndicatio or new ndicatio can be utilized to deliver the output to LMF

- Ericsson (R1-2400101) Proposal 17 For AI/ML assisted positioning in Rel-19, focus on positioning accuracy enhancements of time-based methods up to Rel-17. Proposal 18 Support LOS/NLOS indicator as model output for AI/ML assisted positioning in Rel-19. Proposal 19 Support timing information as model output for AI/ML assisted positioning in Rel-19.

- OPPO (R1-2400619) Proposal 2: For UE-assisted/LMF based positioning with LMF-side model (Case 2b) and NG-RAN node assisted positioning with LMF-side model (Case 3b), reuse the existing measurements of NR positioning as the baseline - FFS: whether or not to introduce new values (N) for the maximal number of additional paths (e.g., N > 8). Proposal 3: For UE-assisted/LMF based positioning with UE-side model (Case 2a) and NG-RAN node assisted positioning with gNB-side model (Case 3a), reuse the existing measurements of NR positioning as the baseline - Introduce new information in the reporting to indicate the reported results are generated by legacy method or AI-based method - FFS: whether to reuse the existing quality (e.g., NR-TimingQuality) or new field/IE to reflect the quality/confidence/probability of reported results based on AI model output Proposal 12: For the model inference for NG-RAN node assisted positioning with gNB-side model (Case 3a), - No need to specify the format/contents of AI model inputs since they are up to gNB/TRP implementation and transparent from the perspective of air interface - gNB can report the measurement results that are based on AI model output via existing NRPPa signaling - The existing measurement types are reused - Introduce new information to indicate the reported results are generated by legacy method or AI-based method FFS: whether to reuse the existing quality (e.g., Measurement Quality) or introduce a new field/IE to reflect the quality/confidence/probability of reported measurement results that are based on AI model output Proposal 11: For the model inference for UE-assisted/LMF based positioning with LMF-side model (Case 2b), - UE can do the measurement and report the results via existing LPP signaling - The existing measurement types are reused - FFS: whether or not to introduce new values (N) for the maximal number of additional paths (e.g., N > 8). Proposal 10: For the model inference for UE-assisted/LMF based positioning with UE-side model (Case 2a), - Some indication (e.g., in form of configuration ID or model ID) is signaled from network to ensure the consistency of AI model training and AI model inference - No need to specify the format/contents of AI model inputs since they are up to UE implementation and transparent from the perspective of air interface - UE can report the measurement results via existing LPP signaling - The existing measurement types are reused - Introduce new information to indicate the reported results are generated by legacy method or AI-based method - FFS: whether to reuse the existing quality (e.g., NR-TimingQuality) or introduce a new field/IE to reflect the quality/confidence/probability of reported measurement results that are based on AI model output Proposal 9: For the model inference for UE-based positioning with UE-side model (Case 1), - Some indication (e.g., in form of configuration ID or model ID) is signaled from network to ensure the consistency of AI model training and AI model inference - No need to specify the format/contents of AI model input since they are up to UE implementation and transparent from the perspective of air interface - UE can report the estimated location information via existing LPP signaling - Introduce new information to indicate the reported results are generated by legacy method or AI-based method - FFS: whether the current field about the uncertainty can be reused or some new field/IE should be introduced to report the associated quality/probability/confidence of AI model estimated location

- Samsung (R1-2400721) Proposal 11: in attached to the model output, the time stamp and quality information is supported. Proposal 12: RAN1 supports TRP to signal the model output to LMF in case 3a.

- CICTCI (R1-2400757) Proposal 7: For model inference of case 3a, at least legacy timing estimation information such as RtoA and LOS/NLOS indicator can be reused.

- Sharp (R1-2401172) Proposal 3: For AI/ML assisted positioning (case 2a and 3a), introduce new Ies in LPP or NRPPa signaling to report the new measurement such as ToA.

### 1st round discussion

Based on companies’ contribution, majority companies suggest to support LOS/NLOS indicator and timing information as model output of assisted positioning, which were extensively evaluated in Rel-18 study item phase. In terms of timing information, the exact format needs more discussion, since companies have different views on how to report, e.g., TOA, RSTD, differential timing. Angle information and power information lack evaluations from Rel-18 study item and can be studied further.

Proposal 4.1.2-1

For AI/ML assisted positioning (i.e., Case 2a and 3a), at least LOS/NLOS indicator and timing information are supported as potential model output.

- FFS: The exact format of timing information for DL and UL measurements, respectively.

- FFS: whether angle information and power information are also supported as potential model output.

|  | Company |
| --- | --- |
| Support | Fujitsu, mtk, ZTE, CMCC, TCL, CATT, NEC,Xiaomi, Nokia/NSB (Partially), [HwHiSi],Ericsson |
| Not support |  |

| Company | Comments |
| --- | --- |
| InterDigital | Partial support. Ok with LOS/NLOS indicator. We don’t support timing information part in the proposal. Without having a list of potential candidates for timing information for AIML assisted positioning, the proposal is not clear. |
| ZTE | We can following majority companies’ view to start with LOS/NLOS indicator and timing information. |
| NEC | For LOS/NLOS indicator, does it defaulted that reuse all legacy two ways? i.e., both hard value and soft value? |
| Nokia/NSB | We agree partially with the proposal. However, we propose the following rewording: Proposal 4.1.2-1(Updated) For AI/ML assisted positioning (i.e., Case 2a and 3a), at least LOS/NLOS indicator and timing information are supported as potential model output. - FFS: The exact format of timing information for DL and UL measurements, respectively. - FFS: whether angle information and power information are also supported as potential model output. |
| OPPO | We think the existing measurement type (e.g., RSTD) should be supported. The ndicational part is whether some new type of timing information is supported (e.g., TOA) |
| Qualcomm | - First, would you please include our proposals on model output in the summary? - We suggest better clarifying the scope of formats. Please add wording from TR. E.g., FFS: The exact format of timing information for DL and UL measurements, including hard- or soft- value, respectively. TR 38.843: - Model output of AI/ML assisted positioning. For AI/ML assisted positioning, evaluations are carried out where the model output includes timing information and/or LOS/NLOS indicator, in the format of hard- or soft- value. |
| HW/HiSi | Partial support. For angle info and power as ouput, there is not enough evidence from the SI: Updated Proposal 4.1.2-1 For AI/ML assisted positioning (i.e., Case 2a and 3a), at least LOS/NLOS indicator and timing information are supported as potential model output. - FFS: The exact format of timing information for DL and UL measurements, respectively. - FFS: whether angle information and power information are also supported as potential model output. |

Apart from the measurement report from model output, several companies pointed out the need to provide an indication to LMF that the intermediate measurements are generated by an AI/ML model, not an conventional non-AI method.

Proposal 4.1.2-2

For AI/ML assisted positioning (i.e., Case 2a and 3a), an indication is included in the measurement report to notify LMF that the intermediate measurement is generated by an AI/ML model.

|  | Company |
| --- | --- |
| Support | InterDigital, CMCC, Hw/HiSi |
| Not support | Nokia/NSB |

| Company | Comments |
| --- | --- |
| ZTE | We believe it’s a reasonable enhancement, but from our perspective, this can be discussed if AI/ML use exactly the same intermediate feature and the same IE. (e.g., after proposal 4.2.2-1 is agreed.) |
| TCL | The indication could also be included in the reference signal configurations to avoid the duplicated indication in the measurement report. |
| Vivo | In principle, we support this intention. However, we think this is necessary only if the exact same report format is reused as in existing specification. |
| CATT | Seems too early to consider this. It is not so clear how to use such indication at LMF side. Also, if different reporting RRC/LPP ndicatio is used between AI-based or non-AI-based positioning, this ndication may not be needed. |
| NEC | Does it means that the LMF implements the different algorithm for the measurements sourcing from different ways(AI/ML model or RS) when calculate the UE location? Otherwise, what is the intention to indicate to the LFM that the measurement source either from AI/ML model or the legacy one? |
| Nokia/NSB | To FL, could you clarify of the meaning of “indication to be included ” ?. If LMF requests measurements/estimation from UE by activating the corresponding AI/ML functionality (thus at least indicating that AIML will be used), then does UE needs to again indicate that it used AIML for its estimations? |
| OPPO | Support |
| HW/HiSi | Support |
| NTT DOCOMO | Similar view with CATT. Necessity of such indication should be futher clarified. More detailed information (e.g., which additional condition is applicable?) may be benefitial. |

### 2nd round / offline session

Proposal 4.1.3-1

For AI/ML assisted positioning (i.e., Case 2a and 3a), at least LOS/NLOS indicator and timing information are supported as potential model output.

- FFS: The exact format of timing information for DL and UL measurements, respectively.

- FFS: whether angle information and power information are also supported as potential model output.

|  | Company |
| --- | --- |
| Support | NTT DOCOMO, Fujitsu, LG, ZTE |
| Not support | InterDigital (Support if timing information is removed, since we still don’t know what timing information is. Without resolving the FFS, we cannot agree to support timing information as an output of the AIML model) |

| Company | Comments |
| --- | --- |
| TCL | This proposal mentions the format of timing information, so we propose to merge proposals 4.1.3-1, 4.2.3-1 and 4.2.3-2 to mention the format of LOS/NLOS indicator. |
| Qualcomm | As discussed in TR summary, please incorporate that this can apply to both hard and soft model output: Modified version For AI/ML assisted positioning (i.e., Case 2a and 3a), at least LOS/NLOS indicator and timing information are supported as potential model output, including their soft- and hard-value representation. - FFS: The exact format of timing information for DL and UL measurements, respectively. |
| Nokia/NSB | Ok with the direction of the proposal. |
| Apple | OK |

### 3rd round discussion

As suggested by TCL, the proposals on LOS/NLOS indicator and timing info are combined into one proposal below.

As illustrated in Ericsson (R1-2400101), the timing information at the model output have a different meaning if AI/ML model is used to generate it. Thus at least an indicator is needed to notify LMF about the AI/ML generated timing information. The indicator can be implicit or explicit, and exactly details FFS.

Proposal 4.1.3-1

For AI/ML assisted positioning Case 3a, at least LOS/NLOS indicator and timing information are supported as potential model output.

- The timing information is represented using the UL RTOA reference time T0+tSRS as defined in 38.215.

- If LOS/NLOS indicator is generated as model output and reported, the indicator can be reported as soft indicator or hard indicator, with the same value range as in existing IE LoS-NloSInformation in 38.455, clause 9.2.77.

- An indication (implicit or explicit) is included in the measurement report to notify LMF that the timing information is generated by an AI/ML model. FFS: details of the indication

|  | Company |
| --- | --- |
| Support |  |
| Not support | InterDigital |

| Company | Comments |
| --- | --- |
| InterDigital | As we are still discussing details of timing information, the proposal should focus on LOS/NLOS indicator. We support the details related to the LOS/NLOS indicator. |
| Sony | Generally support. We can leave out the timing information if it creates issue. |
| ZTE | Prefer discuss these two issues separately. |
| Fujitsu | Similar view as that of ZTE. AI/ML running belongs to LCM. |
| Nokia/NSB | This proposal has a dependency of Proposal 3.4.2-2. Our suggestion is to separate the discussion of both aspects LOS/NLOS indicator and timing information separately. |

### Wednesday offline discussion

Proposal 4.1.5-1

For AI/ML assisted positioning Case 3a, at least LOS/NLOS indicator and timing information are supported as candidate report.

- If LOS/NLOS indicator is reported, the indicator can be reported as soft indicator or hard indicator, with the same value range and resolution as in existing IE LoS-NloSInformation in 38.455, clause 9.2.77.

- If timing information is reported, the timing information may be reported in the format according to the associated measurement type as defined in 38.215.

| Company | Comments |
| --- | --- |
| Intel | For the second sub-bullet, why not list the existing measurements, viz., UL RTOA and gnB Rx-Tx time difference? - If timing information is reported, the timing information may be reported in the format according to the associated measurement type the timing information can be reported via UL RTOA and gNB Rx-Tx time difference as defined in 38.215. |
| InterDigital | Support. Proposal 4.1.5-2 discusses more details about timing information |
| HW/HiSi | Ok |
| NTT DOCOMO | Support. |

Proposal 4.1.5-1a

For AI/ML assisted positioning Case 3a, at least LOS/NLOS indicator and/or timing information are supported for reporting.

- If LOS/NLOS indicator is reported, the indicator can be reported as soft indicator or hard indicator as defined in 38.214.

- If timing information is reported, the timing information may be reported in the format according to the associated measurement type at least can be reported via UL RTOA or gNB Rx-Tx time difference as defined in 38.215.

- Note: details of the report are pending further discussion.

Proposal 4.1.5-1b

For AI/ML assisted positioning Case 2a, at least LOS/NLOS indicator and timing information are supported for reporting.

- If LOS/NLOS indicator is reported, the indicator can be reported as soft indicator or hard indicator as defined in 38.214.

- If timing information is reported, the timing information may be reported in the format according to the associated measurement type at least can be reported via DL RSTD or UE Rx-Tx time difference as defined in 38.215.

- Note: details of the report are pending further discussion.

Proposal 4.1.5-2

For AI/ML assisted positioning Case 3a with timing information reported, at least support reporting UL-RTOA.

- FFS: measurements for multi-RTT.

| Company | Comments |
| --- | --- |

Proposal 4.1.5-3

For AI/ML assisted positioning Case 2a with timing information reported, at least support reporting RSTD.

- FFS: measurements for Multi-RTT.

| Company | Comments |
| --- | --- |

Proposal 4.1.5-4

For AI/ML assisted positioning Case 3a with timing information reported, an indication (implicit or explicit) is provided to notify LMF that the reported information is generated by an AI/ML model.

- FFS: details of the indication

| Company | Comments |
| --- | --- |
| HW/HiSi | Why is the indicator limited to model timing information? Also other model output (LOS/NLOS ) should be possible to be indicated as generated by AI/ML |
| NTT DOCOMO | Similiar view with HW. And if necessary, it’s also applicable to case 2a. |

Proposal 4.1.5-5

For AI/ML assisted positioning Case 3a, the reported timing information at model output is represented using the "UL RTOA reference time" T0+tSRS as defined in 38.215.

| Company | Comments |
| --- | --- |
| HW/HiSi | It does not seem to matter what timing information is at the model output. We should focus on the timing information in the report. This proposal includes both „model output“ and „reported timing“ info. They do not need to be the same. Suggest to update: For AI/ML assisted positioning Case 3a, the reported timing information at model output is represented using the „UL RTOA reference time“ T0+tSRS as defined in 38.215. |

## LOS/NLOS indicator for model output

### Companies’ views

In the following, selected inputs from companies’ contributions are provided on the details of LOS/NLOS indicator as model output of AI/ML assisted positioning.

- Ericsson (R1-2400101) Proposal 20 Provide a mechanism to indicate that the timing information provided by AI/ML assisted model is to be treated like that of LoS link (i.e., virtual LoS path) regardless of the LOS/NLOS indication.

- Huawei, HiSilicon (R1-2400145) Proposal 8 For the model inference of Case 3a, the legacy reporting for LOS/NLOS indication and/or timing information can be re-used to carry the AI/ML model output(s). • Discuss if there is a need to make the LMF aware of that the report has been generated with AI/ML and specify if necessary. • Other potential enhancements for the measurement reporting would need further justification.

### 1st round discussion

The LOS/NLOS indicator is defined as follows in 38.355 and 37.355. It is an optional field in the measurement report.

#### 38.455: 9.2.77 LoS/NloS Information This IE contains the LoS/NloS information for UL measurement. IE/Group Name; Presence; Range; IE type and reference; Semantics description. CHOICE LoS/NloS Indicator; M. >Soft Indicator. >>LoS/NloS Indicator Soft; M; INTEGER (0..10); Values provide the likelihood of a LOS propagation path in the range between 0 and 1 with 0.1 steps resolution. Value ‘0’ indicates NLOS and value ‘1’ indicates LOS.. >Hard Indicator. >>LoS/NloS Indicator Hard; M; ENUMERATED (NloS, LoS).

#### 37.355 #### – LOS-NLOS-Indicator The IE LOS-NLOS-Indicator provides information on the likelihood of a Line-of-Sight (LOS) propagation path from the source to the receiver. -- ASN1START LOS-NLOS-Indicator-r17 ::= SEQUENCE { indicator-r17 CHOICE { soft-r17 INTEGER (0..10), hard-r17 BOOLEAN }, ... } -- ASN1STOP LOS-NLOS-Indicator field descriptions. indicator This field provides information on the likelihood of a Line-of-Sight propagation path from the source to the receiver with a value of 1 corresponding to LoS and a value of 0 corresponding to NloS. - soft: Integer value ‘0’ indicates likelihood 0, integer value ‘10’ indicates likelihood 1. Scale factor 0.1; range 0 to 1. - hard: FALSE indicates likelihood ‘0’, TRUE indicates likelihood ‘1’..

The LOS/NLOS indicator are consistent between LPP and NRPPa reports. Also, it seems to be the common understanding that the existing IE can be re-used when the report is generated by an AI/ML model.

Proposal 4.2.2-1

For AI/ML assisted positioning (i.e., Case 2a and 3a), when LOS/NLOS indicator is reported, the existing IE in 37.355 (LPP for Case 2a) and 38.455 (NRPPa for Case 3a) can be re-used from RAN1 perspective.

|  | Company |
| --- | --- |
| Support | Fujitsu, mtk, ZTE, CMCC, TCL, CATT, NEC, Nokia, HW/HiSi, Ericsson |
| Not support | InterDigital (we should discuss the definition of LOS/NLOS indicator generated by an AIML model first) |

| Company | Comments |
| --- | --- |
| vivo | 1. We’re okay with the direction and prefer to use “as starting point” since we still have time to decide if any enhancement is needed on LOS/NLOS indicator. 2. Why there’s no proposal on timing information report in section 4.3 (for the 1st round)? |
| Nokia/NSB | We agree partially with the proposal, but we suggest the following rewording: Proposal 4.2.2-1 (Updated) For AI/ML assisted positioning (i.e. Case 3a), when LOS/NLOS indicator is reported, the existing IE in 37.355 (LPP for Case 2a) and 38.455 (NRPPa for Case 3a) can be re-used from RAN1 perspective. |
| OPPO | Support in general |
| HW/HiSi | Support |

### 2nd round / offline session

Proposal 4.2.3-1

For AI/ML assisted positioning Case 3b, when LOS/NLOS indicator is generated as model output and reported, the indicator can be reported as soft indicator or hard indicator, with the same value range as in existing IE “LoS/NloS Indicator” in 38.455.

|  | Company |
| --- | --- |
| Support | NTT DOCOMO, LG, ZTE |
| Not support |  |

| Company | Comments |
| --- | --- |
| InterDigital | Is it supposed to be “Case 3a” instead of “Case 3b”? If so do we need an agreement to support LOS indicator as an output of AIML assisted positioning for case 3a? Is this proposal to be followed by the proposal in 4.1.3? |
| TCL | 1.Why is the proposal 4.4.2-1 split into proposal 4.2.3-1 and 4.2.3-2? 2.Consider our comments in section 4.1.3. |
| Qualcomm | I guess there is a typo, this is meant for Case3a? |
| Fujitsu | It would be fine if it is for Case 3a as pointed by QC and InterDigital. |
| LG | Fine with the updated wording except for case splitting 3a/3b |
| Apple | Should be 3a ? |
| Nokia/NSB | To discuss on this topic, we request a clarification, why is the relation of the proposal with Case 3b? |

Proposal 4.2.3-2

For AI/ML assisted positioning Case 3a, when LOS/NLOS indicator is generated as model output and reported, the indicator can be reported as soft value or hard value, with the same value range as in existing IE “LOS-NLOS-Indicator” in 37.355.

|  | Company |
| --- | --- |
| Support | NTT DOCOMO |
| Not support |  |

| Company | Comments |
| --- | --- |
| TCL | Same as the comments for proposal 4.2.3-1. |
| Qualcomm | I guess you mean Case2a? Agree to TCL, why not to merge 4.2.3-1 and 4.2.3-2 |
| Nokia/NSB | We request a clarification to FL, what is the relation of this proposal with Case 3a ? |

## Timing information for model output

### Companies’ views

In the following, selected inputs from companies’ contributions are provided.

- vivo (R1-2400233) Proposal 6: Specify TOA or distance ranging between UE and TRP as model output for AI/ML assisted positioning. Proposal 7: The legacy measurement RSTD and LOS/NLOS indicator can be reused as intermediate measurements of AI/ML assisted positioning without specification impacts. An additional indication is needed to indicate that the report measurement comes from AI/ML models.

- Qualcomm (R1-2401432) Proposal 8: For Case3a/2a, specify new enhancements to reporting of earliest path LOS and timing info measurements in which UE/gNB reports multiple-hypotheses soft info of LOS indicator and timing information. Existing additional path reporting (up to 8 paths) could be repurposed to support the multiple-hypothesis LOS reporting.

- Ericsson (R1-2400101) Proposal 21 Reuse the existing measurement reporting in LPP and NRPPa to report timing information for the model output of AI/ML assisted positioning. Proposal 22 Adopt downlink relative time of arrival (TDL-RTOA) as UE-side model output of assisted AI/ML positioning. Based on the discussion above, the following are proposed. Proposal 23 For AI/ML assisted positioning at gNB (Case 3a), the model output is uplink relative time of arrival (TUL-RTOA). Proposal 24 For AI/ML assisted positioning at gNB (Case 3a), postprocessing is applied to generate gNB RxTxTimeDiff for measurement reporting of multi-RTT method. Proposal 25 For AI/ML assisted positioning at UE (Case 2a), the model output is downlink relative time of arrival (TDL-RTOA), which is defined relative to DL RTOA Reference Time. Proposal 26 For AI/ML assisted positioning at UE (Case 2a), postprocessing is applied to generate DL RSTD (for DL-TDOA method) and UE RxTxTimeDiff (for multi-RTT method) for measurement reporting.

# Training data collection

This section discusses issues for the LCM stage of model training.

## Training data content and collection procedure

### Companies’ view from contribution

In the following, selected inputs from companies’ contributions are provided.

- InterDigital (R1-2401042) Observation 2: Measurement forwarding by the LMF specified in Release 18 for carrier phase positioning can be used to deliver training data to the UE by the network Proposal 8: Support both hard (1 or 0) and soft indicator (0, 0.1, 0.2, …, 1.0) for a ground truth label quality indicator Proposal 9: Assistance information from the LMF which contains PRU measurements should also include the ground truth quality indicator associated with the PRU location information Proposal 10: Support LMF to forward location information of non-PRUs (e.g., UE), associated measurements and ground truth label quality indicator to a UE Proposal 11: For UE-based positioning using AIML based positioning, support the UE to request the LMF for a ground truth label quality indicator based on measurement and UE location estimate made by the UE Proposal 12: Support reporting of an AIML model training outcome, RMSE (Root Mean Square Error) result, to the network

- Huawei, HiSilicon (R1-2400145) Proposal 5: For Case 1 label generation, prioritize the PRU, while the benefits and necessity to additionally consider Ues remain to be confirmed.

- MediaTek (R1-2401137) Proposal 2e-1: For training data collection for UE side model before enabling AI/ML positioning, consider the joint UE and NW method Proposal 2e-2: For training data collection for UE side model before enabling AI/ML positioning, consider UE reporting enhancement as the model input candidates, and LMF to determine the label Proposal 2e-3: If the channel response measurement is agreed to support for reporting, the relative power among samples could be considered by setting power to 1 for the maximum sample. A scaling value for absolute power could be optionally reported

- ZTE (R1-2400169) Proposal 4 For AI/ML positioning, additional assistance information of DL PRS configuration is not necessary to be defined for UE side data collection. Proposal 3:: For AI/ML positioning, different data collection requirements can be configured by the data collection node. -The detailed data collection requirements can be further studied, for example, data collection requirements may include data size, type, quality, report periodicity, etc. Proposal 6: For AI/ML positioning, additional assistance information of UL-SRS configuration is not necessary to be defined for gNB side data collection.

- CMCC (R1-2400317) - Proposal 2: For AI/ML based positioning, whether it is feasible to obtain the ground-truth labels via PRUs is related to the training dataset size. - Proposal 5: For UE generates ground truth label based on non-NR and/or NR RAT-dependent positioning methods, the reliability or the positioning accuracy should also be reported.

- Intel (R1-2400377) Proposal 3: - For UE-based AI/ML positioning or UE-assisted AI/ML positioning, the LMF could provide a UE with one or more configurations of measurement time windows during which the UE is expected to perform measurements towards data collection and/or report ground-truth labels. - Reuse/adaptation of the Rel-18 framework for supporting simultaneous DL positioning measurements across Ues could be considered. - Details FFS. Proposal 4: - For UE-based AI/ML positioning or UE-assisted AI/ML positioning, the LMF could provide an NG-RAN node with one or more configurations of time windows to configure and transmit DL PRS with certain requested configuration(s) for data collection for model training. - Reuse/adaptation of the Rel-18 framework for supporting simultaneous DL positioning measurements across Ues could be considered. - Details FFS. Proposal 6: - For UE-based AI/ML positioning or UE-assisted AI/ML positioning, the LMF could provide an NG-RAN node with one or more configurations of time windows during which the NR-RAN node is expected to schedule SRS transmissions from one or more of its served Ues and perform associated measurements on the SRS resources for data collection for model training. - For the measurements on UL SRS from Ues in other cells, LMF could provide NG-RAN nodes with information for SRS scheduling in neighboring cells similar to the current UL positioning framework. - Reuse/adaptation of the Rel-18 framework for supporting simultaneous UL positioning measurements across Ues could be considered. - Details FFS. Proposal 7: - At least for Cases 2b and 3b where the data collection/generation entity is different from the entity performing model training/inference/LCM, RAN1 to investigate potential solutions to enable efficient compression of data collection.

- CATT (R1-2400419) Proposal 1: For case 1, the following options for training data collection containing measurement and UE’s location can be considered. - Option 1: Both measurement and UE’s location are generated by UE; - Option 2: Measurement is generated by UE and UE’s location is provided by LMF; - Option 3: Both measurement and location are provided by PRU. Proposal 2: For case 1, when UE sends a data collection request to LMF, the data collection request contains some assistance information related to the TRP set expected by UE. Proposal 5: For case 3a, the following options for training data collection containing measurement and ground truth label (in form of positioning related information/parameters) can be considered. • Option 1: Both measurement and ground truth label are generated by gNB/TRP; • Option 2: Measurement is generated by gNB/TRP and ground truth label is provided by LMF. Proposal 6: For case 3a, if LMF provides ground truth label (in form of positioning related information/parameters) to gNB/TRP and the following methods for ground truth labels collection are considered: • Method 1: UE/PRU provides the location related information to LMF for determining the ground truth label; • Method 2: Multiple gNBs/TRPs provide the SRS-pos measurements to LMF for estimating UE’s location coordinate and the UE’s location coordinate is used to determine the ground truth label. Proposal 10: For case 3b, the following options for the training data collection containing measurement and location can be considered. • Option 1: Measurement is provided by gNB/TRP and location is generated by LMF based on the received measurement. • Option 2: Measurement is provided by gNB/TRP and location is provided by UE/PRU. Proposal 11: When LMF side collects training data, LMF side can use a quality indicator to indicate the required quality of the collected data. Proposal 15: For case 2a, the following options for the training data collection containing measurement and ground truth label (in form of positioning related information/parameters) can be considered. • Option 1: Both measurement and ground truth label are generated by UE; • Option 2: Measurement is generated by UE and ground truth label is provided by LMF; • Option 3: Both measurement and ground truth label are provided by PRU. Proposal 19: For case 2b, the following options for the training data collection containing measurement and location can be considered. • Option 1: Measurement is provided by UE/PRU and location is generated by LMF based on the received measurement. • Option 2: Measurement and location are provided by UE/PRU. Proposal 20: When LMF side collects training data, LMF side can use a quality indicator to indicate the required quality of the collected data to UE.

- NEC (R1-2400469) - Proposal 2: Specific the gNB-initiate RS configuration request to facilitate model training/retraining/fine-tuning or ground truth based model monitoring at gNB side in Rel-19, at least for the case where data transfer is not supported. Proposal 4: Endorse the integration of mixed datasets from diverse drop/clutter parameters, network synchronization errors, and scenarios to train a model with robust generalization capabilities, or for fine-tuning the model to achieve higher accuracy in the target scenario in Rel-19. Proposal 5: Support the mechanism to configuring, reporting, and determining the quality indicator of ground truth label in Rel-19. Proposal 6: A quality indicator should be defined for a data, and it’s determined based on the quality indicator(s), if available, of associated measurement and ground truth label. Proposal 7: Data generation entity can initially report the data fulfilling the quality indicator threshold and then reports supplemental data in case that previously reported quality data is not adequate.

- Xaomi (R1-2400544) Proposal 4: For data collection in Case 1 - When AI models are developed by the UE side, three is no specification impact on the collection of measurements corresponding to input and the ground truth - When AI models are developed by the network side (if model transfer/delivery is supported), there is need to specify the data format definition and configuration of the data collection - For both cases, RAN1 need to identify whether any assistance information is needed from network. Proposal 5: Leave other WGs e.g., RAN2 to discuss the report of collected data Proposal 7: For Case 3a, leave RAN3 to assess whether there is specification impact on the collected data format definition, information exchange between TRP and gNB and information exchange between different gNBs considering different training location Proposal 8: For Case 3a, RAN1 is responsible to identify whether there is any need for assistance information from UE and what kind of assistance information is needed Proposal 10: There is need to specify the model input format for data collection in Case 3b Proposal 11: If multiple data formats are specified, LMF configure the detailed data format for collection in Case 3b Proposal 12: Leave RAN3 to handle the collected data report to LMF for Case 3b Proposal 13: For Case 3b, RAN1 is responsible to identify whether there is any need for assistance information from UE and what kind of assistance information is needed

- OPPO (R1-2400619) Proposal 4: In order to facilitate the training data collection at UE side for Case 1 and/or Case 2a, introduce a mechanism to enable that UE can send request for preferred or supported configuration(s) to LMF - FFS: Whether this request is sent via UE capability signaling or other signaling/procedure (e.g., functionality identification). Proposal 5: For training data collection at NW side for UE-assisted/LMF-based positioning with LMF-side model (Case 2b), support the following mechanisms - PRU: PRU reports measurement results (corresponding to model input) and the associated timestamps via LPP protocol - The associated ground-truth label(s) can be included optionally - LMF maybe know the label(s) in advance. - UE: UE reports measurement results (corresponding to model input) and the associated timestamps without associated labels via LPP - LPP signaling from LMF to indicate - Configuration of Positioning RS - The type of measurement results - How to report the collected data (e.g., periodic reporting, event-triggered reporting) Proposal 6: Regarding the training data collection for NG-RAN node assisted positioning with gNB-side model (Case 3a) - gNB do measurement based on SRS - The existing procedure/mechanism can be reused for this purpose - gNB can decide itself when/how to start the data collection procedure. - The contents of collected data are up to implementation and no specification is needed in 3GPP - FFS: whether LMF delivers ground-truth labels to gNB Proposal 7: For training data collection at NW side for NG-RAN node assisted positioning with LMF-side model (Case 3b), support the following mechanisms - gNB reports measurement results (corresponding to model input) and the associated timestamps via NRPPa protocol - The corresponding label(s) can be reported optionally or LMF generates the associated labels based on the know location of the corresponding PRU - NRPPa signaling from LMF to indicate gNB - The type of measurement results - How to report the collected data (e.g., periodic reporting, event-triggered reporting)

- Samsung (R1-2400721) Proposal 8: RAN1 supports the collected data signalled from PRU(other UE) to UE deployed with AI model.

- CICTCI (R1-2400757) Proposal 4: For data collection at NW-side, NW-side initiates the data collection procedure by sending request information to the measurement entities and ground-truth label generating entities, and the contents of request information need to be further discussed. Proposal 5: The relationship between Model-ID and dataset related information need further study.

- Fujitsu (R1-2400767) Proposal 13 Regarding data collection for Case 1 model training, study LMF configurations on UE/PRU data storage and reporting. Observation 6 No need to introduce training specific measurement and reporting for data collection of both model input and labels to support AI/ML positioning model training. Observation 7 The data transmission from LMF to UE can be in 3GPP transparent manner since UE-side model will be trained at UE-side OTT server. Proposal 12 To categorize data for model training, the assistance information for data collection is suggested to be studied with the considerations of the following aspects at least: - RS Configurations. - Time stamp. - Quality indicator - Data area information. Proposal 14 Regarding data collection for Case 3a model training, study the specification impacts of the ground truth label provided by LMF to gNB. Proposal 15 Regarding data collection for Case 3b model training, additional specification support is not needed.

- Lenovo (R1-2400923) Proposal 12: Consider the specification of data request and data collection for the enhanced positioning accuracy use case by considering outcomes in the ongoing RAN2 study, and taking into account the following options: - Option 1 – LMF-side Model Training: Positioning-dataset transfer using existing 3GPP-signaling, e.g., LPP/NRPPa - Option 2 – UE-side Model Training: Proprietary signaling: The Positioning-dataset is transferred without specification impact using non-3GPP technologies. Proposal 13: Existing LPP/NRPPa signalling may be used to provide labelled/unlabelled data indication to different Ues/network entities. Proposal 14: Label quality indication may also be associated with ground truth labels, e.g., location. FFS label quality metrics, e.g., location accuracy.

- Baicells (R1-2401082) Proposal 3. For Case 3b data collection for model training, the LMF should indicate its desired SRS configuration in data collection request, and the serving gNB should provide the configuration result to LMF in the response message. Proposal 4. For Case 3b data collection for model training, a measurement request indicating the desired measurements for data collection from LMF, and a measurement response indicating the supported measurements by gNBs can be beneficial to better support data collection.

- CEWiT (R1-2401268) Proposal 1: Specify the following signallings for Data Collection for Case 1, - Reference signal configuration request from UE/PRU to LMF and/or as assistance from LMF via assistance ignaling to UE/PRU indicating RS configuration to derive label and/or other training data. - Report from UE/PRU related to measurement/label generation (if needed). - Report related to Label Quality Indicator (QI) from UE/PRU or in response to request from a different entity. Proposal 3: Specify the following signallings for Data Collection for Case 3b, - Report from LMF related to label generation. - Report from TRP related to measurement generation - Report related to Label Quality Indicator (QI) from LMF. - Report related to time stamp from data generation entity together with collected data to LMF as an assistance ignaling.

- NTT DOCOMO (R1-2401108) Proposal 5: For data collection of each case, the measurement report and related assistance information for AI/ML based positioning is determined by LMF, - For case 1/2a and case 3a, LMF sends indications of corresponding measurements to UE and gNB respectively. - For case 2b and case 3b, LMF initiates corresponding measurement reporting at UE and gNB respectively. Proposal 6: Regarding RS configurations of data collection for each case, - For case 1/2a, UE may send requests to NW for corresponding RS configurations, or NW may send configurations to UE. - For case 2b, NW sends configurations to UE for measurements. - For case3a/3b, NW sends configurations to UE for SRS transmissions.

- Qualcomm (R1-2401432) Proposal 5: For data collection in Case1/2a, specify information that LMF provides to UE side to help obtain labels and develop & train AI/ML positioning model - For specifying information, as a starting point, consider assistance data of UE-based DL-TdoA and DL-AoD methods Proposal 7: For data collection in Case1/2a, specify dedicated PRS configuration and activation that can be initiated/requested by UE/PRU. As a starting point, consider the following options for specifying dedicated data collection PRS configuration and activation: - Dedicated data collection PRS configuration/activation as part of positioning session with an existing positioning method (e.g., DL-TdoA, DL-AoD, multi-RTT) - Dedicated data collection PRS configuration/activation as part positioning session with new AI/ML positioning methods - Dedicated data collection PRS configuration/activation as part of new data collection procedure (e.g., on-demand data collection for positioning procedure)

- Sony (R1-2400845) Proposal 1: Support radio channel characteristics reporting in a form of channel impulse response (CIR) for AI/ML positioning. Proposal 2: Support configurable CIR measurement report (e.g., report size, measurement window size) in an effort to reduce the signalling overhead. Proposal 3: Support labelling procedure as part of the data collection to facilitate supervised learning AI/ML. Proposal 4: Support the association of GT label with the collected data (e.g., CIR measurement report)

- Sharp (R1-2401172) Proposal 4: Regarding the data collection for model training for positioning, support CIR/PDP as AI/ML model input. Proposal 5: For UE-side AI/ML model, specifications support that UE sends a request for obtaining ground truth labels.

- Xaomi (R1-2400544) Proposal 9: UE can notify the NW side about which type of SRS-Pos can be supported for data collection - The types may include aperiodic and/or semi-persistent and/or periodic, and the corresponding parameters for SRS-Pos, e.g., the preferred periodicity value, SRS-Pos bandwidth can also be indicated.

- Nokia (R1-2400794) Proposal 30: For Case 1 direct AI/ML positioning, RAN1 must limit the discussion on content data for training data collection purposes in the normative work.

- NVIDIA (R1-2400693) Proposal 1: For AI/ML based positioning, introduce specification support for indicating quality of ground truth labels.

- vivo (R1-2400233) Proposal 2: Specify the following two types of training data to support supervised learning and semi-supervised learning - Type 1 ：labelled data - Type 2：unlabelled data Proposal 3: The detailed data content with specification impact includes - For type 1 labeled data, at least the following information should be specified o measurement, e.g., CIR, PDP, DP o Ground truth label, e.g., location coordinate, ToA, RSTD, LOS/NLOS indicator o Quality indicator, e.g., SNR/SINR for measurement, labelling quality indicator o time stamp - For type 2 unlabeled data, at least the following information should be specified o measurement, e.g., CIR, PDP, DP o Quality indicator for measurement, e.g., SNR/SINR o time stamp

- Ericsson (R1-2400101) Proposal 28 For training data collection phase, a training dataset validity area can be defined as a list of TRPs where data is collected. Proposal 29 The training dataset validity area is recorded as a part of metadata of the training dataset. Proposal 30 Training data provided to UE/gNB is organized with the same hierarchy as for legacy assistance data, i.e. per TRP/frequency layers for the UE, and per TRP for the gNB assistance data. Proposal 31 RAN1 to study what additional metadata to attach to training dataset, including: a. Range of SNR/SINR b. Range of Channel estimation error c. Range of NW synchronization error d. Range of UE/gNB RX and TX timing error

### 1st round discussion

Companies have provided much detailed discussion on training data collection. In the following, the proposals are provided. The intention is that we start the discussion with the basics.

Proposal 5.1.2-1

For Rel-19 AI/ML based positioning, training data collection content include:

- A list of training data samples,

- Meta data, which provides context information of the collected training dataset.

|  | Company |
| --- | --- |
| Support | Fujitsu, ZTE, TCL, CATT, NEC, Ericsson, NTT DOCOMO |
| Not support | HW/HiSi |

| Company | Comments |
| --- | --- |
| mtk | 1, Using “samples” here may not be appropriate, since we have not determined sample based or timing based 2, maybe some wording change could avoid this, for example, - A list of training data model input |
| InterDigital | What constitutes training data samples needs clarification. |
| ZTE | We are general OK with this proposal. One thing should clarify that whether the meta data is associated with a single dataset or a group of dataset. |
| CATT | Agree in general. |
| Xiaomi | We are OK generally |
| OPPO | We are fine with the main bullet and the 1st bullet. Since the meaning of meta data is not clear, we prefer to add FFS for the 2nd bullet |
| Qualcomm | As starting point, we think this proposal can be too early as specifying data content may not be required, especially, for Case1/2a/3a. Let’s first focus on RAN1 aspects for data collection, including data collection entity, assistance info, and resource signaling based on outcomes in TR 38.843. Let’s also start with Section 5.2 to discuss the data collection entity as this helps the discussion on what type of assistance is needed and whether data content needs to be specified. |
| HW/HiSi | We think, we firstly need to discuss whether the training data collection needs to be specified. Which will be different from case to case. Regarding the meta data, we think more discussion will be needed. What would be the meta data for different use cases, and how would it be used? Would there be any spec impact? We need to have a discussion on the potential spec impact (if any) for cases 1-3b. |
| Fraunhofer | Point (A) in the proposal is the training dataset by definition: For Rel-19 AI/ML based positioning, training data collection content include: - A list of training data samples, The training dataset - Meta data, which provides context information of the collected training dataset. |
| Nokia/NSB | We agree partially with the proposal. In general, RAN1 must focus only to discuss the data content for training. For the case of monitoring/inference, we may include assistance info, RS configuration, etc. Meta data is a term not clear yet between companies, we may avoid it by now or consider as FFS. For this specific proposal we have the following suggestion: Proposal 5.1.2-1 (Updated) For Rel-19 AI/ML based positioning, training data collection content include: - A list of training data samples, consisting of measurements corresponding to model input and ground truth labels corresponding to model output (if available/applicable). FFS: Meta data, which provides context information of the collected training dataset. |
| Baicells | General OK. We may firstly make analysis on the data content on a case-by-case basis (at least the 1st priority cases), then decide what training data collection content includes. |

Proposal 5.1.2-2

For Rel-19 AI/ML based positioning, training data samples can be either type:

- Type 1: labelled data sample

- Type 2: unlabelled data sample

|  | Company |
| --- | --- |
| Support | TCL, |
| Not support |  |

| Company | Comments |
| --- | --- |
| mtk | 1, Similar concern for using “sample” at this moment. Maybe change “sample” as “model input” |
| InterDigital | Do we have a definition for ignaling data? |
| ZTE | We can further study and make final decision until comprehensive investigation is achieved. From our view, the draft proposal can be: In Rel-19 AI/ML based positioning, regarding the training data samples, RAN1 study and select one or both alternative(s) from the following: - Type 1: labelled data sample - Type 2: unlabelled data sample |
| CMCC | Is it possible that one dataset include both labelled and ignaling data samples, e.g., for semi-supervised learning? |
| CATT | Agree in general. |
| NEC | Since we have no additional discussion on semi-supervise training, we prefer to list Type 1 as base line and Type 2 as FFS. |
| OPPO | ZTE’s proposal is fine to us |
| Qualcomm | Same to our comment in Proposal 5.1.2-1 |
| HW/HiSi | In the SI we discussed labels and measurements, we do not need to introduces a new terminology here. Is the intention that „unlabeled data“ is the same as measurements“? |
| Fraunhofer | Same point as above. Suggest changing to: For Rel-19 AI/ML based positioning, training dataset can include two types of data data samples can be either type: - Type 1: labelled data sample - Type 2: unlabelled data sample |
| Nokia/NSB | Question to FL, what is the intention of this proposal ?. Is it targeting any specific use case?. |

Proposal 5.1.2-3

For Rel-19 AI/ML based positioning, an labeled training data sample include at least the following components:

- Channel measurement (corresponding to model input), e.g., CIR, PDP, DP

- Quality indicator of channel measurement, e.g., SNR/SINR

- Time stamp

|  | Company |
| --- | --- |
| Support |  |
| Not support |  |

| Company | Comments |
| --- | --- |
| ZTE | Wait progress on proposal 5.1.2-2. |
| CMCC | For training data sample, we can discuss whether time stamp is mandatory or optional. |
| CATT | How to understand Time stamp and what is the granularity here? Is it always needed? |
| NEC | This issue should be suspended until the consensus of supporting semi-supervise training in Rel-19. |
| OPPO | In general, we are fine with proposal and suggest to the examples from the 1st and 2nd bullets |
| Qualcomm | Same to our comment in Proposal 5.1.2-1 |
| HW/HiSi | In our understanding the time stamp is already supported in legacy. Is the intention here to have a different time-stamp? If yes, can this be explained further? Also, what is the purpose of having a time stamp in the context of training data collection? Could this be clarified? |
| Fraunhofer | Suggest the following change: For Rel-19 AI/ML based positioning, an ignaling training data sample each instance of an ignaling training dataset includes at least the following components: - Channel measurement (corresponding to model input), e.g., CIR, PDP, DP - Quality indicator of channel measurement, e.g., SNR/SINR - Time stamp |
| Nokia/NSB | Lets wait for the progress of Proposal 5.1.2-2 |
| Baicells | We are confused about “ignaling training data sample” and have a clarification question about “sample”. In some scenarios, gNB sends Measurement X without label to LMF, then LMF binds label Y that LMF already has and the Measurement X to become a sample. Under this situation, Measurement X is only part of the sample, which cannot be defined as a sample. In this logic, the Measurement X (e.g., CIR/PDP) which we need to discuss in the WI may be out of the scope of this proposal. Therfore, we may firstly define sample. |

Proposal 5.1.2-4

For Rel-19 AI/ML based positioning, a labelled training data sample include the following components, as well as all the components of the corresponding ignaling training data sample:

- Ground truth label (corresponding to model output), e.g., location coordinate, timing information, LOS/NLOS indicator

- Quality indicator of the label

|  | Company |
| --- | --- |
| Support | Fujitsu, TCL, NEC |
| Not support | HW/HiSi |

| Company | Comments |
| --- | --- |
| InterDigital | We should have “FFS details of quality indicator of the label” in the proposal. |
| CMCC | Same comment as the above . |
| CATT | Still one question: is the quality indicator still needed, if a quality threshold is configured to filter out the low quality data? |
| OPPO | We need to clarify what the quality is and how to decide the quality before we can agree on it |
| Qualcomm | Same to our comment in Proposal 5.1.2-1 |
| HW/HiSi | If the training data is not specified, there is no need to indicate the ground truth quality. It can be ensured by implementation. Therefore, it can be good to discuss examples for data collection for the different cases firstly and then to decide what might need to be specified. Legacy already supports that PRU location can send its location and uncertainty to LMF. Therefore, a specification of location as label could already be supported by legacy and it is not clear to us what more would be needed here. For the timing info or LOS/NLOS, at least for Case 3a we do not think that a specification is needed since the label can be ignalin that the same enity where the ignaling is carried out. It seems that labeled data is supposed to include both measurements and labels, then there seems no need for the time stamp. |
| Fraunhofer | If the ground truth label is generated at a different entity than the one that performs the channel measurements, it might be beneficial to include a separate time stamp for the ground truth label. Propose the following change: For Rel-19 AI/ML based positioning, an instance of a labelled training dataset sample includes the following components, as well as all the components of the corresponding instance of the ignaling training dataset training data sample: - Ground truth label (corresponding to model output), e.g., location coordinate, timing information, LOS/NLOS indicator - Quality indicator of the label Time stamp of the ground truth label |
| Nokia/NSB | Lets wait for the progress of Proposal 5.1.2-2 |

For training data collection, CATT suggested to consider both options where measurements and ground truth labels are both from PRU/UE, or the ground truth label is from LMF. Huawei suggested to prioritize PRU, while the benefits and necessity to additionally consider Ues remain to be confirmed. Thus the following is proposed.

Proposal 5.1.2-5

For training data collection of Case 1, consider both options:

- Both channel measurement and ground truth label are generated by a same PRU;

- Channel measurement is generated by a PRU, ground truth label is provided by LMF;

Note: for Case 1, the ground truth label is the location coordinates of the PRU.

FFS: the benefits and necessity to additionally consider Ues other than PRU.

|  | Company |
| --- | --- |
| Support |  |
| Not support |  |

| Company | Comments |
| --- | --- |
| Fujitsu | Both options are assumed PRU-based data collection only. If it is the case, we think keep option-1 is enough. On the other hand, we think study UE-based data collection for Case 1 is important, since PRU may not be deployed anywhere. PRU’s density has big impact to the quality of data collection. We suggest adding the options for UE-based data collection and delete the FFS part. For training data collection of Case 1, consider the following options: - Both channel measurement and ground truth label are generated by a same PRU; - Both channel measurement and ground truth label are generated by a same UE - Channel measurement is generated by a UE, ground truth label is provided by LMF; |
| mtk | 1, PRU density would be a problem if the proposal is to only rely on PRU. We consider to add option 3 and support the 3 options, not just putting a FFS here Option 3: Channel measurement is generated by a UE, and ground truth label is provided by LMF |
| InterDigital | Are we considering downselection of options in the future? |
| ZTE | Share similar view with Fujitsu and mtk. And we prefer to add another option: Option 4. Both channel measurement and ground truth label are generated by a UE. UE may have some positioning information of itself. We admit that the positioning accuracy should be considered, this option can be a backup alternative if the training data is not adequate. |
| TCL | The number of PRUs should be considered since the training operation needs mass data. |
| Vivo | It’s not clear what’s the implication of this proposal. „Consider“ for what? Study in RAN1? Decision in RAN1? In our opinion, both options are valid and the related signaling/procedure is not in RAN1’s scope. We don’t see the need to discuss this proposal. |
| CATT | Fine in general. But UE can at least provide channel measurement for sure. Suggest to make it more clear: For training data collection of Case 1, consider both options: - Both channel measurement and ground truth label are generated by a same PRU; - Channel measurement is generated by a PRU/UE, ground truth label is provided by LMF; Note: for Case 1, the ground truth label is the location coordinates of the PRU. FFS: the benefits and necessity to additionally consider Ues other than PRU for providing ground truth label. |
| NEC | We support both option 1 and option 2 basically. Since the large number of data is required especially for model training, we are doubtful whether the data from only PRU is enough. We recommend to identify the UE as one of the entity to collect the training data. |
| OPPO | UE collecting data should be supported. Thus, suggest to add one option Option X: Channel measurement is generated by a UE |
| Qualcomm | We have already made an agreement in Rel-18 SI that UE generates label. This proposal can also be extended for Case2a. TR 38.843: - The following options of entity and mechanisms to generate ground-truth label are identified: - UE with estimated/known location generates ground-truth label and corresponding label quality indicator - Based on non-NR and/or NR RAT-dependent and/or NR RAT-independent positioning methods - At least for UE-based positioning with UE-side model (Case 1) and UE-assisted positioning with UE-side model (Case 2a) Modified proposal: For training data collection of Case 1 and Case2a, consider both options: - Both channel measurement and ground truth label are generated by a same PRU or UE; - Channel measurement is generated by a PRU or UE, ground truth label is provided by LMF; Note: for Case 1, the ground truth label is the location coordinates of the PRU or UE. Note: for Case 2a, the ground truth label corresponds to the intermediate measurement at model output. FFS: the benefits and necessity to additionally consider Ues other than PRU. |
| HW/HiSi | We would support Opt1 , there is no need to involve the LMF since labels are known by PRU and the collection could be handled in a spec transparent manner between PRU and training entity. The benfits of Option 2 are not clear and I be explained by proponents. |
| Nokia/NSB | Question to FL. What is the intention of the proposal?. We are wonder that this agreement will derive on further proposals/agreements on data collection signalization, which is not the target of RAN1 for training data collection. Why to consider only PRU ?, we may also consider any regular UE. |
| Baicells | Support in general. |

Proposal 5.1.2-6

For training data collection of Case 3a and 3b, consider both options:

- Channel measurement is generated by a gNB, ground truth label is provided by LMF;

- Channel measurement is generated by a gNB, ground truth label is provided by PRU;

Note: for Case 3a, the ground truth label corresponds to the intermediate measurement at model output; for Case 3b, the ground truth label is the location coordinates of the PRU.

FFS: the benefits and necessity to additionally consider Ues other than PRU.

|  | Company |
| --- | --- |
| Support | InterDigital, NEC |
| Not support | HW/HiSi |

| Company | Comments |
| --- | --- |
| Fujitsu | Both are OK. |
| ZTE | Similar comment with Proposal 5.1.2-5. |
| TCL | Similar comment with Proposal 5.1.2-5. |
| Vivo | It’s not clear what’s the implication of this proposal. „Consider“ for what? Study in RAN1? Decision in RAN1? In our opinion, both options are valid and the related signaling/procedure is not in RAN1’s scope. We don’t see the need to discuss this proposal. |
| CATT | One comment on the FFS: is ‘Ues’ a typo (should be replaced by gNB)? Or it is indeed ‘Ues’ but the motivation is to provide label? - Understanding 1) FFS: the benefits and necessity to additionally consider gNBs other than PRU. - Understanding 2) FFS: the benefits and necessity to additionally consider Ues other than PRU for providing ground truth label. |
| NEC | Support the two options. |
| OPPO | Support |
| Qualcomm | We think Option2 has some privacy concerns with Case3a as obtaining PRU label by entities (other than LMF) should first be assessed against privacy. Let’s consider Option1 as it can be inclusive for Option2 (e.g., LMF can provide label to other entities after resolving privacy concerns). |
| HW/HiSi | This is different for Case 3a and 3b. For Case 3a: Fine with that channel measurement is obtained at gNB but label can also be obtained at gNB (thus neither Op1 or Opt2). In Section 7.2.4 of the TR 38.843, the training data generation termination is described for the gNB-side model: - For gNB-side model, training data can be generated by the gNB, while the termination point for training data may include the gNB, or OAM. - Note: RAN2 identified the case in which LMF may be used for gNB-side model training. However, no conclusion was reached, as this depends on the RAN1 progress.. For terminating the model training, the labels are needed. Therefore, the labels must be made available at the OAM if it is the training entity. These labels can then be kept at the OAM or can be signaled by implementation to the gNB. It should therefore be noted that both the gNB and OAM can obtain ground truth for LOS states and also for TOA by implementation. Regardless of the model output type, another entity to obtain the label may not be needed. As a typical example, the label between TRP and PRU can be manually configured to the vendor domain OAM. For Case 3b: According to agreement from SI, our understanding is that Option 1 is already agreed and there is no need for Option 2. |
| Baicells | Support. Seem to be an continuation from R18 SI. TR 38.843 - At least LMF with known PRU location is identified to generate ground-truth label for UE-assisted/LMF-based positioning with LMF-side model (Case 2b) and NG-RAN node assisted positioning with LMF-side model (Case 3b) - At least network entity with known PRU location is identified to generate ground-truth label for NG-RAN node assisted positioning with gNB-side model (Case 3a) |

Regarding PRS configuration for training data collection, some companies (e.g., OPPO, CEWIT) think that the UE can send a request for preferred or supported configurations to LMF; other companies (e.g., NTT DOCOMO) think the measurement report and related assistance information are determined by LMF; while ZTE suggests that additional assistance information of DL PRS configuration is not necessary to be defined for UE side data collection. All the alternatives are listed below for further investigation.

Proposal 5.1.2-7

For training data collection of Case 1, in terms of DL PRS configuration, RAN1 studies the following alternatives on assistance information associated with the training data:

- (PRU initiated) PRU makes a request to LMF on the preferred DL PRS configuration for training data collection;

- (LMF initiated) LMF determines the DL PRS configuration for training data collection and provides the assistance data to the PRU.

- No need to define assistance information on DL PRS configuration for training data collection.

|  | Company |
| --- | --- |
| Support |  |
| Not support | InterDigital (Why is UE initiated missing from alternatives? Are we down-selecting among the alternatives?) ZTE |

| Company | Comments |
| --- | --- |
| Fujitsu | According to TS37.355 assistance data framework, it is natural to have UE initially requests to LMF. |
| Inter |  |
| ZTE | For alt 1, we on-demand PRS configuration can be used for preferred DL PRS configuration. |
| CATT | We feel that we may remove the ‘XX initiated’ for now. And it seems possible to have some common design between Alt.1 and Alt.2, i.e. UE provides multiple configurations, and LMF configured one of them; or, LMF provide multiple configurations, and UE choose one of them. |
| NEC | Alternative 1. We suggest to also add UE initiated in Alt.1. |
| OPPO | Suggest to add one more alternative: Alternative X: (UE initiated) UE makes a request to LMF on the preferred DL PRS configuration for training data collection; |
| Qualcomm | This needs to include UE and Case2a. Please consider the following updated proposal: Modified proposal: For training data collection of Case 1 and Case2a, in terms of DL PRS configuration, RAN1 studies following alternatives on assistance information associated with the training data: - (PRU/UE initiated) PRU/UE makes a request to LMF on the preferred DL PRS configuration for training data collection; - (LMF initiated) LMF determines the DL PRS configuration for training data collection and provides the assistance data to the PRU/UE. - No need to define assistance information on DL PRS configuration for training data collection. @Moderator: Please add a section to discuss assistance data/info from LMF to UE/PRU for enabling data collection and model development. |
| HW/HiSi | We would suggest to use alt 3. It is not clear why Alt1 or Alt2 would be needed, this should be explained by proponents. |
| Baicells | Support to add UE initiated in Alt.1. |

### Thursday offline discussion

Proposal 5.1.3-1

In Rel-19 AI/ML based positioning, labelled data samples are collected for building a training dataset. A labelled training data sample include at least the following components:

- From measurement data generation entity:

  - Channel measurement (corresponding to model input), e.g., CIR, PDP, DP

  - Quality indicator of channel measurement, e.g., SNR/SINR

  - Time stamp of channel measurement

- From label data generation entity:

  - Ground truth label (corresponding to model output), e.g., location coordinate, timing information

  - Quality indicator of the label

  - Time stamp of the label, if label data generation entity is different from the measurement data generation entity

Note: the measurement data generation entity may or may not be the same as the label data generation entity.

Proposal 5.1.3-2

Rel-19 AI/ML based positioning, support the following for providing measurement data:

- For Case 1, 2a and 2b: PRU

- For Case 3a and 3b: TRP

Proposal 5.1.3-3

Rel-19 AI/ML based positioning, support the following for providing label data:

- For Case 1 and 2a: PRU

- For Case 2b, 3a, 3b: LMF with known PRU location

Proposal 5.1.3-4

For Rel-19 AI/ML based positioning, RAN1 study and decide whether ignaling data samples are collected for building a training dataset in addition to labelled data samples. A labelled training data sample include at least the following components:

- From measurement data generation entity:

  - Channel measurement (corresponding to model input), e.g., CIR, PDP, DP

  - Quality indicator of channel measurement, e.g., SNR/SINR

  - Time stamp of channel measurement

## Data collection entity

### Companies’ view from contribution

In the following, selected inputs from companies’ contributions are provided.

- Huawei, HiSilicon (R1-2400145) Proposal 6: For Case 1 data collection for UE-sided model, prioritize the data collection from UE to the UE-side training entity (e.g. OTT server) via spec transparent manner. • Note: Data collection with network assisted manner bring unnecessary spec effort, and data delivery from 3GPP entity (e.g. LMF) to UE-side training entity is outside the scope of RAN1 discussion. Observation 3: Data collection for model training in Case 2a can be done via implementation similarly to the data collection for model training in Case 1. Proposal 7:: For training data collection for Case 3b, further discuss the approach to indicate or report the quality indicator between gNB and LMF. Observation 4: Data collection for model training in Case 2b can be done similarly to the data collection for model training in Case 3b. Observation 1: For Case 3a data collection for gNB-sided model, the network entity for the label generation can be gNB/OAM, and the label can be collected by implementation. Observation 2: For Case 3b data collection for LMF-sided model, the network entity for the label generation for model training could be LMF.

- TCL (R1-2400348) - Proposal 1: For UE-side model, deploying the model training entity on the network side is recommended. - Proposal 2: For gNB-side model, deploying the management entity on the gNB side is recommended.

- Intel (R1-2400377) Proposal 1: RAN1 to focus on scenarios where the model training and inference occur at the same entity. Scenarios involving model transfer/delivery are deferred to a later phase during the Rel-19 WI when further progress is made on model transfer/delivery as part of the ongoing study aspects

- Oppo (R1-2400619 Proposal 8: For AI-based positioning (including Case 1, Case 2a, Case 2b, Case 3a, Case 3b), Rel-19 is NOT to specify any mechanism to deliver the collected data from the entity that obtains the training data to the training entity when the training entity is not the same entity to obtain label and/or other training data.

- CICTCI (R1-2400757) Proposal 1: For data collection of case 1, if data collection is performed at NW-side, the signaling and mechanism of NW-side data collection need further discussion. Proposal 2: For data collection of case 3a, it is preferred to perform data collection at LMF-side. Proposal 3: For data collection of case 3b, it is preferred to perform data collection at LMF-side.

# Ensure consistency between training and inference

## Companies’ view from contribution

In the following, selected inputs from companies’ contributions are provided.

- Intel (R1-2400377) Proposal 2: - RAN1 to focus on scenarios where the model update may be performed (as part of model LCM) by the entity where the model resides. Considerations of specification impact for model update are deferred to a later phase during the Rel-19 WI when further progress is made on model transfer/delivery and model identification as part of the ongoing study aspects. Proposal 5: - For UE-based AI/ML positioning or UE-assisted AI/ML positioning, the LMF could provide a UE with configuration of a measurement validity area within which the UE may perform the measurements and/or report ground-truth labels for data collection for model training. - The concept of an AreaID-CellList, as described in TS 37.335, to list the NR Cell-IDs of the TRPs belonging to a particular network area within which the UE is to perform measurements and/or report ground-truth labels, could be reused. - Details FFS. Proposal 15: - Consider the alignment between configured measurement validity area and inference validity area as a NW-side additional condition to ensure consistency between training and inference.

- vivo (R1-2400233) Proposal 11: For UE-side model, RAN 1 should focus on NW-side additional condition, and there may be no specification impact for UE-side additional condition for AI/ML based positioning. Proposal 12: Support adopting the following approaches to ensure the consistency between training and inference. - NW side provides some information or indication to assist UE in maintaining the consistency between training and inference, such as dataset categorization information or explicit beam pattern information. - UE can report supported PRS bandwidth in UE capability signaling; - UE can indicate applicability of AI/ML positioning functionality in applicability reporting based on NW provided information/indicated; - UE can indicate supported dataset categorization or beam pattern in other procedures;

- MediaTek (R1-2401137) Proposal 2f-1: The model for positioning is geography dependent. Then each model may need to be associated to ID(s) that could uniquely represent a region

- CATT (R1-2400419) Proposal 24: For case 1 and case 2a, if NW-side additional condition is identified as necessary for positioning, at least performance/model monitoring can be considered to ensure the consistency between training and inference regarding NW-side additional conditions for inference at UE-side model.

- ZTE (R1-2400169) Proposal 11: There is no need to specify any NW-side additional conditions for AI/ML positioning.

- Google (R1-2400393) Proposal 2: Support the NW to provide side information for the UE to select the corresponding model, where the side information could include antenna type (omni-directional or directional) for each TRP, antenna radiation pattern, beam for PRS, location of each TRP and so on.

- NEC (R1-2400469) Proposal 8: Support to collect UE coordination information in a zone-based format where the zone size is predefined.

- Xiaomi (R1-2400544) Proposal 25: Support additional condition alignment is beneficial for both model-based LCM and functionality-based LCM in AI-based positioning use case Proposal 26 : RAN1 further investigate the necessity and feasibility for the following potential network additional condition - Scenario/site information - Measurement configuration - Network synchronization error - Antenna information Proposal 27: Support indication of the network additional condition and unified indication mechanism should be strived for in all cases Proposal 28: For the indication of additional condition, virtual category and/virtual ID can be defined in specification while how to associate with the virtual category and/or virtual ID is up to implementation Proposal 29: Consider solutions to keep consistency regarding UE additional condition for network side model

- OPPO (R1-2400619) Proposal 1: For AI/ML based positioning of Case 1 and Case 2a, some indication (e.g., in form of configuration ID) is signaled from network to ensure the consistency of AI model training and AI model inference - e.g., the ID can be a special ID for positioning configurations, and can be indicated to differentiate the associated training data - The proprietary information of network should not be disclosed.

- CICTCI (R1-2400757) Proposal 8: For model inference at UE-side in positioning use case, to ensure consistency between model training and inference of NW-side additional conditions (if identified), information and/or indication of NW-side additional conditions should be provided to UE.

- Baicells (R1-2401082) Proposal 12. To explore methods that ensure consistency between training and inference, RAN1 may need to firstly identify the NW-side additional conditions regarding positioning. Proposal 13. For Case 1, support the transfer of UE-side model information to LMF via LPP.

- IIT (R1-2401154) Proposal 8: Alignment of NW-side additional conditions could be through an ID assigned by the NW during training data collection. In the absence of an ID, alignment can be achieved by model monitoring.

- Qualcomm (R1-2401432) Proposal 11: For Case1/2a/2b/3b, changes in the following NW conditions and factors affect consistency between training and inference for UE- and LMF- side models: - Changes to network synchronization (i.e., TRP relative time differences), - Changes to gNB RX and TX timing error, - Changes to received power due to changes in PRS TX powers - Changes to TRP/ARP locations and orientation, - Changes to TRP/PRS beam angles, - Changes to mapping between PRS Resource and Set IDs to physical anchor/TRP locations - Changes to mapping between PRS resources and physical beam angles. Proposal 12: For ensuring consistence between training and inference, specify signaling and information to help ensure consistency between training and inference for UE side and LMF side models (i.e., Case1/2a and Case2b/3b) - As a starting point consider existing assistance information and data in NRPPa and LPP.

- Nokia (R1-2400794) Proposal 45: For Case 1 direct AI/ML positioning, RAN1 to consider areas with specific channel characteristics, datasets mapped to specific areas as NW-side additional conditions. Proposal 47: RAN1 to consider rare events as NW-side additional conditions to improve consistency between training and inference. Proposal 48: For Case 1 direct AI/ML positioning, RAN1 to consider the set of channel characteristics (e.g., LoS/NloS rate) as NW-side additional condition. Proposal 51: For Case 1 direct AI/ML positioning, to ensure consistency between training and inference regarding NW-side additional conditions based on areas with specific channel characteristics, UE reports to the LMF set of TRPs identified during inference using the legacy framework (e.g. LPP). Proposal 52: RAN1 to consider on using NW-side additional conditions to improve the usability of fixed-TRP-pattern complemented with performance monitoring.

- CMCC (R1-2400317) - 1) Model identification to achieve alignment on the NW-side additional condition between NW-side and UE-side - 2) Model training at NW and transfer to UE, where the model has been trained under the additional condition - 3) Information and/or indication on NW-side additional conditions is provided to UE - 4) Consistency assisted by monitoring (by UE and/or NW, depends on the performance of UE-side candidate models/functionalities to select a model/functionality). Proposal 10: For AI/ML based positioning, further discuss option 3 and 4. Deprioritize option 1 and 2.

- Ericsson (R1-2400101) Proposal 32 The model training entity records the model inference validity area as a part of metadata for the trained model. Proposal 33 When preparing for model inference, the model inference validity area is checked for compatibility with the deployment area. Proposal 34 When preparing for model inference, configurations of reference signal transmission and measurement are checked for compatibility between the trained model and the deployment environment. Proposal 32 The model training entity records the model inference validity area as a part of metadata for the trained model. Proposal 33 When preparing for model inference, the model inference validity area is checked for compatibility with the deployment area. Proposal 34 When preparing for model inference, configurations of reference signal transmission and measurement are checked for compatibility between the trained model and the deployment environment.

## 1st round discussion

Several companies (e.g., Intel, MediaTek, NEC, Xiaomi, Nokia, Ericsson) mentioned area/site as a condition to ensure consistency between model training and model inference. This reflects the generalization studies carried out in Rel-18 study item. Another issue mentioned by several companies is reference signal configuration and measurement. Proposals are provided below for these two issues to kick off this discussion.

Proposal 6.2-1

For Rel-19 AI/ML positioning, to ensure consistency between model training and model inference, validity area need to be consistent between training and inference.

- FFS: how to indicate the validity area information.

|  | Company |
| --- | --- |
| Support | Fujitsu, InterDigital, TCL, NEC, Fraunhofer (revision), Ericsson |
| Not support | ZTE, Nokia/NSB |

| Company | Comments |
| --- | --- |
| ZTE | Form our view, this is related to the generalization performance, which can be mitigated by training with mixed dataset. We prefer postpone this discussion after the type of data collection and/or model inputs are stable. |
| CMCC | It seems that ‘validity area need to be consistent between training and inference’ may put too much limitation from our point of view. If we marked every single area(like a building) as a validity area, it may need too much effort to manage and store these dataset. |
| CATT | OK to further consider, but we think PLMN-unique cell ID can serve the purpose. |
| OPPO | Validity area is only one ignal when we considering consistency. In the first meeting, if we only hightlight some examples (validity area, RS), it may give a misleading impression that only these examples are important aspects and deprioritize other aspects. Thus, we prefer to have a list including all the possible aspects (e.g., beam pattern use for PRS transmission, TRP deployment, ...) On the other hand, some clarification on validity area is needed. For some reasons, the NW may turn off or turn on some TRPs, the same area is still valid? |
| Qualcomm | We should first have alignment on what factors can cause inconsistency between training and inference. Let’s not sub select some of them and leave others. We suggest having a proposal for listing companies’ views on different factors before agreeing on this one. |
| Fraunhofer | Validity could be not only limited to area, for example channel conditions. FFS: other validity conditions |
| Nokia/NSB | We partially agree with the proposal. The scope of additional conditions to ensure the consistency between model training and inference is not limited to validity area. Thus, in this regard, we propose the following rewording: Proposal 6.2-1 (Updated) For Rel-19 AI/ML positioning, to ensure consistency between model training and model inference, spatial characteristics (e.g. channel features, validity area) need to be consistent between training and inference. - FFS: how to indicate the spatial chanracteristics validity area information. |
| Ericsson | At least the inference must be done on TRPs that have been trained. Thus the area used during inference should be a subset of the trained TRPs. |
| NTT DOCOMO | Validity area is part of additional conditions. Recognition of the ignaling should be firstly discussed. |

Proposal 6.2-2

For Rel-19 AI/ML positioning, to ensure consistency between model training and model inference, reference signal configuration and measurement need to be aligned between training and inference.

- For DL measurement, the reference signal refers to DL PRS

- For UL measurement, the reference signal refers to UL positioning SRS

- FFS: the details of the reference signal configuration.

|  | Company |
| --- | --- |
| Support |  |
| Not support | InterDigital (we should discuss which parts of RS configs need to be aligned as (for example) PRS config is organized in a hierarchical manner), ZTE |

| Company | Comments |
| --- | --- |
| ZTE | Same comments with proposal 6.2-1. |
| CATT | We think that not everything of the RS need to be aligned between training and inference. For example, different periodicity of PRS between training and inference does not have significant impact on model performance. |
| NEC | Other existing reference signal can also be identified to collect measurement, e.g., use the CSI-RS to generate the measurement. |
| OPPO | Same comment as before |
| Qualcomm | Similar to our comment in Proposal 6.2-1 |
| Nokia/NSB | We tend to be aligned with the intention of the proposal, but we believe that it may be merged with the previous proposal. We suggest delaying the discussion of this topic to the offline session. |

# Model monitoring

This section discusses issues for the LCM stage of model performance monitoring.

## Model monitoring methods and procedure

### Companies’ view from contribution

In the following, selected inputs from companies’ contributions are provided.

- Intel (R1-2400377) Proposal 13: - Support both input-based and output-based functionality/model monitoring for AI/ML based positioning.

- vivo (R1-2400233) Proposal 8: Support both label-based and label-free monitoring for AI/ML based positioning. Proposal 9: For label-based monitoring, statistics of the difference between model output and provided ground-truth label (or its approximation) can be adopted as a monitoring metric. Proposal 10: For label-free monitoring, the following assistance date needs to be introduced to calculate the monitoring metric - Statistics distribution of dominant features (e.g., SNR/SINR) of training dataset - Other implicit statistics distribution of training dataset

- Huawei, HiSilicon (R1-2400145) Proposal 10: For model monitoring in Case 1, the monitoring decision is performed in the same UE-side entity that derives the monitoring metric. • The monitoring is done in implementation manner and a monitoring metric does not need to be specified. Proposal 11: For model monitoring in Case 3a, further discuss at least the following method: • The gNB calculates the metric and makes the monitoring decision. Applicable cases may include e.g., the label-free and label-based model monitoring case for Case 3a. Proposal 13: For model monitoring in Case 3b, the entity making the monitoring decision is the LMF, i.e. the model monitoring is performed at the LMF. Observation 9: Model monitoring for Case 2a can be done in a similar manner as Case 3a with signaling from the UE to the LMF. Observation 10: Model monitoring for Case 2b can be done in a similar manner as Case 3b with signaling from the UE to the LMF.

- ZTE (R1-2400169) Proposal 13: For AI/ML assisted model monitoring, the following options can be considered: Option 1: Model monitoring is performed based on AI/ML inference output, i.e., intermediate features. Option 2: Model monitoring is performed based on estimated UE’s location according to AI/ML inference output.

- Nokia (R1-2400794) Proposal 13: For Case 1 direct AI/ML positioning, necessary data for performance monitoring with ground truth label (or its approximation) might be synthetic data, e.g., DL RSRPP measurements associated with virtual UE locations generated by simulation at the network-side. In this case, LMF may inform UE that the provided data is synthetic. Proposal 14: In Case 1 direct AI/ML positioning, performance metric without ground truth may consist of confidence level associated with model inference output, i.e., UE location estimate. Proposal 15: In Case 1 direct AI/ML positioning, performance metric without ground truth consists of statistics of measurement(s) compared to the statistics of measurements in the training dataset. Proposal 18: For Case 1 direct AI/ML positioning, necessary data for monitoring without ground truth consists of i) statistics of the measurements corresponding to inference input, i.e., RSRPP measurements; ii) historical/buffered inference output (UE location); and iii) characteristics (e.g., statistics) of training data. Proposal 20: In all cases (e.g., Cases 1, 3a, 3b) LMF is the only entity to determine functionality decision (e.g., functionality switching, selection, (de)activation, fallback) based on monitoring outcome.

- CMCC (R1-2400317) Proposal 8: For performance monitoring is based on the ground-truth labels, further study method to obtain ground-truth label. Observation 2: For performance monitoring without ground-truth label, specify detail method for monitoring metric may be difficult. Proposal 9: For AI/ML based positioning, the relationship between model monitoring and positioning integrity can be considered.

- CATT (R1-2400419) Proposal 4: For case 1, the following options for the performance/model monitoring can be considered. • Option 1:UE reports LCM decision/suggestion to LMF, which is determined based on monitoring result; • Option 2:UE reports monitoring metric or triggered event to LMF, to facilitate LMF making LCM decision; • Option 3: UE reports the information that assist LMF to derive monitoring metric, e.g. model input or model output, to facilitate LMF making LCM decision. Proposal 9: For case 3a, the following options for the performance/model monitoring can be considered. • Option 1: gNB/TRP reports LCM decision/suggestion to LMF, which is determined based on monitoring result; • Option 2: gNB/TRP reports monitoring metric or triggered event to LMF, to facilitate LMF making LCM decision; • Option 3: gNB/TRP reports the information that assist LMF to derive monitoring metric, e.g. model input or model output, to facilitate LMF making LCM decision. Proposal 14: For case 3b, gNB/TRP reports the information that assist LMF to derive monitoring metric, e.g. measurement for input distribution-based monitoring, to facilitate LMF making LCM decision. Proposal 18: For case 2a, the following options for the performance/model monitoring can be considered. • Option 1: UE reports LCM decision/suggestion to LMF, which is determined based on monitoring result; • Option 2: UE reports monitoring metric or triggered event to LMF, to facilitate LMF making LCM decision; • Option 3: UE reports the information that assist LMF to derive monitoring metric, e.g. model input or model output, to facilitate LMF making LCM decision. Proposal 23: For case 2b, UE reports the information that assist LMF to derive monitoring metric, e.g. measurement for input distribution-based monitoring, to facilitate LMF making LCM decision.

- Xiaomi (R1-2400341) Proposal 30: For performance monitoring in Case 1 - The performance metric calculation by be performed on UE side - The LCM decision making can be performed by UE side Proposal 31: Further study the following two options for the case that UE make the LCM decision in Case 1 - Option 1: UE make the LCM decision based on UE’s implementation and then report the LCM decision to LMF - Option 2: LMF provide a performance monitoring metric threshold, UE could make the LCM decision by comparing the calculated performance metric and the configured threshold. - Standardization effort is needed for the performance metric definition and report Proposal 32: - For input-based performance monitoring - gNB calculate the performance metric and make the LCM decision for the assisted positioning with multi-TRP construction - gNB calculate the performance metric and LMF make the LCM decision for assisted positioning with single TRP construction - Standardization effort is needed for the performance metric definition and report - For ground truth-based performance monitoring - LMF calculate the performance metric and make the LCM decision Proposal 33: LMF derive the performance monitoring metric and make the LCM decision. Proposal 34: Confirm the necessity of assessment/monitoring of inactive models / functionalities, with the following assumptions as the starting point: - One way to monitor inactive models/functionalities is by activating them and reusing mechanisms defined for monitoring of active models/functionalities. - The following aspects may be considered for further study or in WI to assess the applicability and expected performance of an inactive model/functionality: - Configuring an AI/ML model for monitoring without activation (e.g., monitoring-only mode without reporting predicted beams in BM Case 1 and 2) - Dataset delivery from the network to the UE for assessment/monitoring of the applicability and expected performance of the model/functionality. - NW may provide performance criteria/preference for UE’s model selection. - Other aspects are not precluded for further study or specification. Proposal 35: UE group-based LCM operation (e.g., model activation/deactivation/switch) can be considered

- Samsung (R1-2400721) Proposal 13: RAN1 considers the model output based, model input based and other (measurement) based monitoring metrics. Proposal 14: RAN1 considers the monitoring entity as UE/TRP/LMF for UE side model, TRP, LMF for TRP side model, LMF for LMF side model. Proposal 15: RAN1 considers the monitoring metric related threshold to be configured, dependent on the monitoring metric content. Proposal 16: RAN1 supports the signalling for the monitoring refence signal configuration, monitoring measurement and its report. Proposal 17: monitoring outcome determination could be based on one time metric or multiple time metrics.

- CEWIT (R1-2401268) - Proposal 2 : Specify the following for Model Inference and Monitoring for Case 1, - Assistance ignaling and procedure (e.g, RS configuration(s)) to facilitate model inference of UE-side model. - If monitoring does not require ground-truth label, - Signalling from LMF to monitoring entity (UE) to derive the monitoring metric (if needed). - Report of monitoring metric/decision (if needed). - If monitoring require ground-truth label, - Assistance ignaling and procedure from LMF to UE to generate ground-truth label and/or measurement. - Signalling from monitoring entity (UE) to request ground-truth label from label generation entity (eg. PRU) (if needed). - Report of monitoring metric/decision (if needed). - Proposal 4: Specify the following for Model Inference and Monitoring for Case 3b, - Assistance ignaling and procedure (e.g, RS configuration(s)) to LMF to facilitate model inference of LMF-side model. - Signalling to transfer the inference data from gNB to LMF. - If monitoring does not require ground-truth label, - Assistance ignaling from gnB to monitoring entity(LMF) related to RS configurations and the measurement statistics of the inference data. - Report of monitoring metric/decision (if needed). - If monitoring require ground-truth label, - Signalling from gNB to monitoring entity (LMF) to derive the monitoring metric (if needed). - Report of monitoring metric/decision (if needed).

- Fujitsu (R1-2400767) Proposal 7 Label-based model monitoring for Case 1 can be realized via specification-transparent way. Proposal 8 No strong need to study specification enhancement to support label-free model monitoring for Case 1. Proposal 9 Study the candidate schemes for Case 3a label-based model monitoring including: - Alternative-1: gNB derives monitoring metrics if labels are available in gNB. - Alternative-2: LMF derives monitoring metrics and provides to gNB. Proposal 10 Label-free monitoring for Case 3a may not have RAN1 specification impact. Proposal 11 For Case 3b model monitoring, there is no specification enhancement needed for both label-based and label-free model monitoring metrics deriving.

- Lenovo (R1-2400923) Proposal 15: Support Model Performance Monitoring Type 1 (using Ground truth labels) and Type 2 (without ground truth labels) for all use cases. FFS any further specification impacts.

- OPPO (R1-2400619) Proposal 14: For functionality/model performance monitoring, Rel-19 is NOT to specify dedicated specification enhancement for the mechanism without ground-truth labels - Functionality/model performance monitoring without ground-truth labels can be done by implementation without any specification enhancement - Note: we also categorize some mechanisms based on “the approximate ground-truth label” in to that without ground-truth labels. Proposal 15: For UE-based positioning with UE-side model (Case 1), Rel-19 is NOT to specify dedicated specification enhancement for functionality/model performance monitoring - Functionality/model performance monitoring can be done by UE implementation without impact on air interface Proposal 16: For UE-assisted/LMF based positioning with UE-side model (Case 2a), Rel-19 is NOT to specify dedicated specification enhancement for functionality/model performance monitoring - Functionality/model performance monitoring can be done by UE implementation without impact on air interface Proposal 17: For UE-assisted/LMF based positioning with LMF-side model (Case 2b), Rel-19 is NOT to specify dedicated specification enhancement for functionality/model performance monitoring at LMF - Functionality/model performance monitoring can be done by LMF implementation without impact on air interface Proposal 18: For NG-RAN node assisted positioning with gNB-side model (Case 3a), in order to facilitate ground-truth-label-based functionality/model performance monitoring at gNB, support the enhancement on NRPPa signaling to enable the delivery of the ground-truth label or the information that can derive the ground-truth label from LMF to gNB - Functionality/model performance monitoring can be done by gNB implementation Proposal 19: For NG-RAN node assisted positioning with LMF-side model (Case 3b), Rel-19 is NOT to specify dedicated specification enhancement for functionality/model performance monitoring at LMF - Functionality/model performance monitoring can be done by LMF implementation without impact on NRPPa signaling Proposal 20: For UE-based positioning with UE-side model (Case 1) and UE-assisted/LMF based positioning with UE-side model (Case 2a) - The functionality/model can be activated or disactivated by LPP signaling, which is the same as the legacy positioning procedure - UE can autonomously deactivate the AI operations and fall back to the legacy operations - UE reporting includes some field/IE to indicate whether the results are based on legacy operation or AI model output. Proposal 21: For UE-based positioning with UE-side model (Case 1) and UE-assisted/LMF based positioning with UE-side model (Case 2a), specify the UE capability signaling to report the supported configuration(s) associated with given functionality(ies) - E.g., DL-PRS resources capability, DL-PRS Processing capability, measurement capability Proposal 22: For UE-based positioning with UE-side model (Case 1) and UE-assisted/LMF based positioning with UE-side model (Case 2a), UE can report the applicable functionalities by sending a ProvideCapabilities message to the LMF (namely, triggering the capability indication procedure of TS 37.355) - FFS: whether some enhancement is needed to reduce the signaling overhead

- Fraunhofer (R1-2401198) Proposal 1: Support UE-sided models to inform the network on the monitoring event that triggered a request to switch functionality or an update of the supported functionalities. Proposal 4: Consider a sequential monitoring process: upon identifying of a potential issue through early metrics, activate a broader set of monitoring metrics to confirm the presence of the issue. Proposal 5: For functionality/model monitoring, support diverse monitoring configurations encompassing: - Offering options for event-based or scheduled monitoring in different time intervals, with the flexibility to adjust the monitoring frequency. - Providing customization for reporting detail, notifying monitoring entity on the level of detail in monitoring reports. - Supporting different methods of generating ground truth labels for monitoring purposes, with varying quality of labels.

- Ericsson (R1-2400101) Observation 22 Effective ground truth label can be obtained from line-of-sight links which can be observed opportunistically in parallel to AI/ML model inference. Observation 23 Existing IE for line-of-sight (LoS) links in legacy assistance data can be reused to support calculation of model output accuracy of the LoS links, thus enabling label-based model monitoring for AI/ML based positioning. Proposal 27 For both direct and assisted AI/ML positioning, study and support label-based model monitoring using the information of LoS links. Proposal 35 For both direct and assisted AI/ML positioning, the model inference entity is the core entity that performs continuous model monitoring itself, including calculation of model monitoring metrics. Proposal 36 For model inference at UE (Case 1, 2a) or gNB (Case 3a), the LMF can provide assistance information to support the calculation of model monitoring metrics. Proposal 37 LMF interacts with gNB and UE to make decisions on whether to activate or deactivate AI/ML positioning, and which AI/ML positioning method to use.

### 1st round discussion

For model monitoring in Case 1, companies (e.g., Huawei, CATT, xiaomi) suggested several options for model monitoring. The options are summarized below for further investigation.

Proposal 7.1.2-1

For model monitoring in Case 1, consider the following options in terms of model monitoring entity and LCM decision making entity:

- UE makes the LCM decision of UE-side model. UE is also responsible for calculating the performance monitoring metric and making model monitoring decision. The following options can be further considered:

  - Model monitoring is up to UE implementation. There is no need to specify a performance monitoring metric, or a threshold for making performance monitoring decision.

  - LMF provide a performance monitoring metric threshold to the UE. UE makes the LCM decision by comparing the calculated performance metric and the threshold. Standardization effort is needed for the performance metric definition, and signaling for the LMF to send the performance monitoring metric threshold to UE.

- LMF makes the LCM decision of UE-side model. The following options can be further considered:

  - UE reports to LMF the information needed for calculating the monitoring metric. LMF is responsible for calculating the performance monitoring metric and making the LCM decision.

  - UE is responsible for calculating the performance monitoring metric. UE sends the performance monitoring metric to LMF to facilitate LMF making the LCM decision.

  - UE is responsible for calculating the performance monitoring metric and making model monitoring decision. UE reports the model monitoring decision to LMF to facilitate LMF making the LCM decision.

|  | Company |
| --- | --- |
| Support | ZTE, TCL, CATT, Xiaomi |
| Not support |  |

| Company | Comments |
| --- | --- |
| Fujitsu | Support both options of Alt.1. |
| InterDigital | Are we downselecting or is another option to support both alternatives? From our point of view, both UE-led LCM and LMF-led LCM is possible. If downselecting, what is the rationale behind downselection? |
| ZTE | Prefer alternative 2. We open to further study all the options. In the first meeting we may don’t have to make downselection. |
| Vivo | It’s not clear what’s the implication of this proposal. In our opinion, both alternatives are valid and the related signaling/procedure if ignaling is not in RAN1’s scope. RAN1 should focus on what’s the content of monitoring metric (if support) ignal than to discuss whih LCM decision entity. |
| NEC | Support Alternative 2. Does the ‘model monitoring entity’ in the main bullet can be changed to ‘monitoring metric calculation entity’? |
| OPPO | Option a of alternative 1 is our 1st preference and Option c of alternative 2 is the 2nd ignaling. |
| Qualcomm | It is good to have alignment on monitoring framework. This also applies to Case2a. It is also good to clarify that for all options there can be assistance data and signaling still needed from LMF to UE. Please consider the following modifications: Modified proposal: For model monitoring in Case 1 and Case2a, consider, for further investigation, the following options in terms of model monitoring entity and LCM decision making entity, including their necessity: - UE makes the LCM decision of UE-side model. UE is also responsible for calculating the performance monitoring metric and making model monitoring decision. The following options can be further considered: - Model monitoring is up to UE implementation. There is no need to specify a performance monitoring metric, or a threshold for making performance monitoring decision. There can still be assistance from LMF to UE to help UE in monitoring. - LMF provide a performance monitoring metric threshold to the UE. UE makes the LCM decision by comparing the calculated performance metric and the threshold. Standardization effort is needed for the performance metric definition, and signaling for the LMF to send the performance monitoring metric threshold to UE. - LMF makes the LCM decision of UE-side model. The following options can be further considered: - UE reports to LMF the information needed for calculating the monitoring metric. LMF is responsible for calculating the performance monitoring metric and making the LCM decision. - UE is responsible for calculating the performance monitoring metric. UE sends the performance monitoring metric to LMF to facilitate LMF making the LCM decision. Option I UE is responsible for calculating the performance monitoring metric and making model monitoring decision (with potential assistance from LMF to UE). UE reports the model monitoring decision to LMF to facilitate LMF making the LCM decision. |
| HW/HiSi | This proposal comes a bit too early, and its implications are not foresseable at this stage. Simiar to our comments about, we need to discuss whether data collection needs to be specified. In our view for Case 1, it has been defined in the TR that the training will be terminated at the UE-side OTT server, which implies that labels and measurements are available at the OTT server. The monitoring can therefore be carried out by implementation at the OTT server, or labels and measurements could be transferred by implementation to the UE to calculate the monitoring metric. If the data generation entity, training entity, and inference entity are all on the UE-side there would be no clear motivation to have the monitoring decision entity to be on the network side. |
| Fraunhofer | Question to FL: We assume that LCM decision is related to model selection/(de-)activation/switching, etc. What is a monitoring decision? |
| Nokia/NSB | Support both options of Alt.2. the details may be discussed later. |
| NTT DOCOMO | Support Alternative 2. The definition of LCM decision should be clarified. Does it include (de)activation of functionality? If it means UE decides functionality automatically, we do not support especially alt.1 opt.(a). |

Proposal 7.1.2-2

For model monitoring in Case 1, consider both label-based and label-free model monitoring methods.

- FFS: if label-based method is used, whether assistance information needs to be provided by LMF to UE.

|  | Company |
| --- | --- |
| Support | TCL, CATT, NEC, Nokia/NSB |
| Not support |  |

| Company | Comments |
| --- | --- |
| InterDigital | For label-based method is used, assistance information should be provided by LMF to UE. |
| ZTE | Lower priority, this can be discussed after proposal 7.1.2-1, the potential ignaling can be further studied only after model monitoring entity is determined. |
| OPPO | Agree with ZTE. |
| Qualcomm | This also applies to Case2a. For label-free model monitoring, the specification impact is not clear. We can consider the following modified proposal as a starting point: Modified proposal: For model monitoring in Case 1 and Case2a, consider at least both label-based and label-free model monitoring methods. - FFS: if label-based method is used, whether assistance information needs to be provided by LMF to UE. - FFS: label-free monitoring methods |
| HW/HiSi | Agree with ZTE |
| Nokia/NSB | We believe that this proposal is important to start the discussion on performance monitoring, not only for Case 1, but for Case 3b/a. |

Proposal 7.1.2-3

For model monitoring in Case 2b and 3b (i.e., LMF-side model), LMF is responsible for calculating the monitoring metric and making model LCM decision.

- Model monitoring is up to LMF implementation. There is no need to specify a performance monitoring metric, or a threshold for making performance monitoring decision.

- FFS: assistance information from UE or gNB to assist with the model monitoring in LMF.

|  | Company |
| --- | --- |
| Support | Fujitsu, InterDigital, ZTE, CMCC, TCL, CATT, NEC, Xiaomi |
| Not support |  |

| Company | Comments |
| --- | --- |
| ZTE | We are ok with the main bullet, while the subbullets require further consideration. |
| OPPO | Support |
| Qualcomm | We are generally fine with the direction of the proposal. Please consider the following modification. Modified proposal: For model monitoring in Case 2b and 3b (i.e., LMF-side model), LMF is responsible for calculating the monitoring metric and making model LCM decision. - Model monitoring is up to LMF implementation. There is no need to specify a performance monitoring metric, or a threshold for making performance monitoring decision. - FFS: whether assistance information from UE or gNB would be needed to assist with the model monitoring in LMF. |
| HW/HiSi | Main bullet and first sub-bullet is fine. Regarding the FFS, it could be both directions: FFS: assistance information between from UE and or gNB to assist with the model monitoring in LMF. |
| Nokia/NSB | We are ok with the direction of the proposal. However, we suggest the following rewording: Proposal 7.1.2-3(Updated) For model monitoring in Case 2b and 3b (i.e., LMF-side model), LMF Network (i.e. LMF) is responsible for calculating the monitoring metric and making model LCM decision. - Model monitoring is up to Network LMF implementation. There is no need to specify a performance monitoring metric, or a threshold for making performance monitoring decision. - FFS: assistance information from UE or gNB to assist with the model monitoring in LMF. |

### 2nd round / offline session

Proposal 7.1.3-1

For model monitoring of LMF-side model, LMF is responsible for calculating the monitoring metric and making model LCM decision.

- Model monitoring is up to LMF implementation. There is no need to specify a performance monitoring metric, or a threshold for making performance monitoring decision.

- FFS: whether assistance information from UE and/or gNB would be needed to assist with the model monitoring in LMF.

|  | Company |
| --- | --- |
| Support | NTT DOCOMO, InterDigital, Fujitsu, LG, Apple |
| Not support |  |

| Company | Comments |
| --- | --- |
| TCL | Does UE includes PRU? If not, we propose the following rewording: FFS: whether assistance information from UE/PRU and/or gNB would be needed to assist with the model monitoring in LMF. [Moderator] Since it’s model monitoring of a deployed model, it should be just normal target UE. If PRU is required to monitor the model, the deployment cost is too high in my understanding. |
| Nokia/NSB | We propose to generalize LMF with NW, specially if the implementation of the all monitoring scheme is up to implementation. Another important comment, we suggest the term „model monitoring“ by „performance monitoring“ to avoid the issues that we had in the TR wording. |
| ZTE | We prefer to remove the second sentence in the first bullet just as Nokia commented in the 1st round discussion. Or we can put this as FFS, i.e., FFS: whether there is a need to specify a performance monitoring metric |

### 3rd round discussion

Proposal 7.1.4-1

For model monitoring of LMF-side model, NW (e.g., LMF) is responsible for calculating the monitoring metric and making model LCM decision.

- Model monitoring is up to NW (e.g., LMF) implementation. There is no need to specify a performance monitoring metric, or a threshold for making performance monitoring decision.

- FFS: whether assistance information from UE and/or gNB would be needed to assist with the model monitoring in LMF.

|  | Company |
| --- | --- |
| Support |  |
| Not support |  |

| Company | Comments |
| --- | --- |
| InterDigital | Is there a reason why “NW” is used here? Doesn’t the model holder, LMF in this case, monitor models? |
| Sony | If it is generalized to “NW” node and we support the assistance information. How the UE / gNB signal the assistance information? |
| Xiaomi | For the first sub-bullet, if the monitoring metric is calculated by NW entity other than LMF, whether there is spec impact is out of RAN1 scope. RAN 1 can’t say“model monitoring is up to NW implementation“. If some companies want to go with a general proposal, we suggest the following update Proposal 7.1.4-1 For model monitoring of LMF-side model, NW (e.g., LMF) at least LMF is responsible for calculating the monitoring metric and making model LCM decision. - If LMF calculates the monitoring metric, Model monitoring is up to LMF NW (e.g., LMF) implementation. There is no need to specify a performance monitoring metric, or a threshold for making performance monitoring decision. - FFS: whether assistance information from UE and/or gNB would be needed to assist with the model monitoring in LMF. |
| Nokia/NSB | Answering the previous comments, We suggested to generalize the term NW, which includes LMF, because as it is stated in the body of the proposal, the performance monitoring is up to implementation in NW-side. In addition, we suggest to avoid using the term „model monitoring“ and switch to „Performance monitoring“ to avoid the ambiguity carried in the SI Rel. 18 in the Evaluation agenda item. |
| Fraunhofer | We propose we align with the terminology and the contents of TR38843 as much as possible, to avoid confusion. There, the definition of monitoring does not include any model management (e.g., selection/(de-)activation/switching decisions): - Model monitoring: A procedure that monitors the inference performance of the AI/ML model. Based on this, we are not sure what “a threshold for making performance monitoring decision” means. If a single threshold set in a single model performance monitoring metric is violated, this triggers a moded management operation or LCM decision (i.e., anything from fallback to model switching or model to re-training)? In addition, PRU could also provide assistance information for LMF-side model monitoring. Finally, is the proposal intended to cover the case where the model LCM decision dictates to switch to a model that implements/supports a different functionality? We suggest the following changes in the proposal (with blue): Proposal 7.1.4-1 For model performance monitoring and management of LMF-side model, NW (e.g., LMF) is responsible for calculating the monitoring metric and making model LCM decision. - Model performance monitoring is up to NW (e.g., LMF) implementation. There is no need to specify a performance monitoring metric, or a threshold for making performance monitoring decision. - FFS: whether assistance information from UE, PRU and/or gNB would be needed to assist with the model performance monitoring and management in LMF. |

### Wednesday offline discussion

Proposal 7.1.5-1

From RAN1 perspective, for performance monitoring of LMF-side model, LMF is responsible for calculating the monitoring metric and making model management decision.

- Model monitoring is up to NW (e.g., LMF) implementation. From RAN1 perspective, there is no need to specify a performance monitoring metric, or a threshold for making performance monitoring decision.

- Note: it’s outside RAN1 scope to discuss the details of NW entities

- If assistance information is specified to support performance monitoring of LMF-side model, the assistance information is reported to LMF from RAN1 perspective.

- FFS: whether/what assistance information from UE and/or gNB need to be sent to LMF to assist with the performance monitoring.

| Company | Comments |
| --- | --- |
| InterDigital | The intention of the new (3rd) bullet is not clear. We suggest to remove the new (3rd) bullet. The UE/gNB reports measurements to the LMF in conventional positioning methods. The gNB transfers, in addition to measurements, assistance information (e.g., cell ID) to the LMF for UL positioning methods. What is inside “assistance information” in the new bullet? We need more clarity on the new bullet and it is part of the FFS. |
| HW/HiSi | Not agree (why is that proposal marked green as it has changed and was not green before either)? The proposal was relatively stable after the official offline but now has changed. The original intention was to say that no metric is specified in RAN1, but which has been deleted after offline. Also a new bullet has been included. The new bullet is partially covered by the FFS. Propose the following rewording: From RAN1 perspective, for performance monitoring of LMF-side model, LMF is responsible for calculating the monitoring metric and making model management decision. - Model monitoring is up to NW (e.g., LMF) implementation. From RAN1 perspective, there is no need to specify a performance monitoring metric, or a threshold for making performance monitoring decision. - RAN1 is not specifying a performance metric or a threshold for making a performance monitoring decision. - Note: it's outside RAN1 scope to discuss the details of NW entities - If assistance information is specified to support performance monitoring of LMF-side model, the assistance information is reported to LMF from RAN1 perspective. - FFS: whether/what assistance information from UE and/or gNB need to be sent to LMF to assist with the performance monitoring. If assistance information is specified, it is reported to LMF |
| Nokia/NSB | We do not support the current version. It was modified compared to our last official offline meeting. We need to revise it again. |
| NTT DOCOMO | Prefer the last version. |

Proposal 7.1.5-1

From RAN1 perspective, for performance monitoring of LMF-side model, LMF is responsible for calculating the monitoring metric and making model management decision.

- Model monitoring is up to NW (e.g., LMF) implementation. From RAN1 perspective, there is no need to specify a performance monitoring metric, or a threshold for making performance monitoring decision.

- Note: it's outside RAN1 scope to discuss the details of NW entities

- If assistance information is specified to support performance monitoring of LMF-side model, the assistance information is reported to LMF from RAN1 perspective.

- FFS: whether/what assistance information from UE and/or gNB need to be sent to LMF to assist with the performance monitoring.

## Assistance data for model monitoring

### Companies’ view from contribution

In the following, selected inputs from companies’ contributions are provided.

- Qualcomm (R1-2401432) Proposal 13: For AI/ML positioning monitoring of Case1/2a, specify signaling for LMF to provide labelling assistance to UE for monitoring UE sided models: - LMF provides monitoring label to UE - Case1, e.g., LMF generates approximate ground truth UE location coordinates and provides it to UE - Case2a, e.g., LMF generates approximate ground truth LOS indicator or timing info and provides it to UE - LMF provides assistance data to help UE conduct monitoring - Case1, e.g., UE generates approximate ground truth UE location coordinates with assistance data provided by LMF - Case2a, e.g., UE generates approximate ground truth LOS indicator or timing info with assistance data provided by LMF. Proposal 14: For AI/ML positioning monitoring, specify signaling for UE to request monitoring occasions from LMF, including RS resources and provisioning of monitoring labelling assistance.

- NVIDIA (R1-2400693) Proposal 3: For AI/ML based positioning, introduce specification support for assistance signalling and procedure for model performance monitoring and model update/tuning, including monitored metrics, triggers for model update, dedicated reference signals, measurements, and feedback report.

- Huawei, HiSilicon (R1-2400145) Observation 8: For model monitoring in Case 3a, if needed, the labels can be transferred to the monitoring entity by implementation. Proposal 12: The LMF can indicate assistance information, e.g., threshold criterion, to the gNB for the calculation of the monitoring metric or to facilitate the calculation of the monitoring metric and/or monitoring decision at the gNB.

- ZTE (R1-2400169) Proposal 12: For direct AI/ML model monitoring, Option 1 should be supported considering model monitoring criteria and UE’s capability. - Option 1: Model monitoring is performed at LMF (for LMF side and/or UE side model). - For LMF side model, UE is required to report legacy positioning results for model monitoring - For UE side model, UE is required to report legacy and AI/ML positioning results for model monitoring - Option 2: Model monitoring is performed at UE (for UE side model). - LMF needs to send location information to UE for model monitoring

- Sony (R1-2400845) Proposal 5: In order to support AI/ML model update, the AI/ML server/management (e.g., LMF) sends the criteria (e.g., parameters for performance evaluation, thresholds) to the radio node performing AI/ML model inference. Proposal 6: UE or gNB to provide an indication of AI/ML model update to the AI/ML server/management (e.g., LMF).

# Model identification and model functionality

## Model identification, model ID

### Companies’ view from contribution

In the following, selected inputs from companies’ contributions are provided.

- Huawei, HiSilicon (R1-2400145) Proposal 2: Proposal 2: Defer discussion on model-ID based LCM in 9.1.2 until end of Q3/2024, since its necessity still is under discussion in 9.1.3.3 and in RAN2.

- Samsung (R1-2400721) Proposal 2: RAN1 considers the trigger conditions for enabling AI/ML based method for positioning, including potential model applicable conditions or the environment(channel) applicable conditions; Proposal 3: in addition to the entity where the model is deployed, the network can decide or assist to decide the model used at UE side, the LMF can decide and LMF/UE can assist to decide the model used at TRP side, the TRP/UE can assist to decide the model used at LMF side. Proposal 4: RAN1 supports the indication of determined model from one entity to another entity Proposal 5: RAN1 supports the indication of measurement related threshold/report/feedback and model applicable condition to assist the model determination from one entity to another entity Proposal 6: for data collection in training/finetuning, the data generation entity can be PRU/UE for case 1, TRP for case 3a, and TRP(input)/LMF(label) for case 3b; Proposal 7: for data collection in inference, the data generation entity is UE for case 1 and TRP for case 3a/3b;

- NVIDIA (R1-2400693) Proposal 6: For AI/ML based positioning, introduce specification support for additional conditions to be included in model description information during model identification.

- Fujitsu (R1-2400767) Observation 9 Applicable-area-information exchange between NW and UE is needed for model validation checking before activation. Proposal 17 Study Applicable-area-information identification and/or indication between NW and UE for UE-side model, including at least the following: - Identification via UE capability report. - Identification via the procedures other than UE capability report.

- Apple (R1-2401003) Observation 1: Model-ID-based LCM: Applicable to the two-sided model and should not be considered for AI/ML based positioning.

## Model Functionality, UE capability reporting

### Companies’ view from contribution

In the following, selected inputs from companies’ contributions are provided.

- Huawei, HiSilicon (R1-2400145) Proposal 3: Support functionality-based LCM for UE-side model of Case 1/[2a]. Proposal 4: For the functionality identification LCM for UE-side model of [Case 2a], consider indicating the functionality by the format of the measurement report.

- Xiaomi (R1-2400544) Proposal 14: Support functionality identification for Case 1 and Case 2a Proposal 15: Support model identification for Case 1 and Case 2a Proposal 16: Whether there is functionality identification / model identification-like procedure between gNB and LMF is up to RAN3 Proposal 17: Categorize the AI/ML feature from the following 3 aspects for AI-based positioning - Output type of the AI models - The location where AI models are deployed - The positioning RS type i.e., SRS-based or PRS-based Proposal 18: At least measurement related aspects e.g., PRS bandwidth, TRP configuration can be considered as conditions to categorize the AI/ML functionality Proposal 19: Support UE directly report the applicable functionality/model via e.g., an identifier. Proposal 20: Applicable functionality/model report can be supported in both the case that associated AI/ML feature is enabled and the case that associated AI/ML feature is not enabled

- OPPO (R1-2400619) Proposal 21: For UE-based positioning with UE-side model (Case 1) and UE-assisted/LMF based positioning with UE-side model (Case 2a), specify the UE capability signaling to report the supported configuration(s) associated with given functionality(ies) - E.g., DL-PRS resources capability, DL-PRS Processing capability, measurement capability

- NVIDIA (R1-2400693) Proposal 5: For AI/ML based positioning, introduce specification support for conditions of a Feature/FG available for functionality.

- NTT DOCOMO (R1-2401108) Proposal 2: Regarding functionality identification, the UE capability reporting signaling can be used to indicate the capable functionality. Proposal 3: Regarding conditions for functionality-based LCM for AI/ML based positioning, following information can be considered, - Functionality information: - Inference output of the AI/ML model, e.g., UE location, LOS/NLOS indication, ToA - Essential information to facilitate the model inference, e.g., necessary measurements information for inference input, number of consecutive time domain samples, number of ports, etc. - Applicable configuration，e.g., required RS configuration associated with functionalities Proposal 4: The functionality activation/deactivation/switching of a UE side or gNB side functionality (i.e., case1, 2a or 3a) can be configured/ indicated by LMF via LPP or NRPPa, respectively.

- Fraunhofer (R1-2401198) Proposal 6: For all positioning use cases, functionality management is provided by the NW (LMF). Proposal 8: In functionality-based LCM, UE/gNB provides an expected performance vs cost/overhead information to the LMF that is associated with a functionality or model. The cost/overhead information includes: - information on (logical or physical) model properties, such as size, computational complexity or power consumption - availability of the physical model on the device - latency for activating functionality or model. Proposal 9: The NW configures several functionalities at the UE/gNB and provides a configuration for functionality switching, depending on specific parameters or thresholds. The configuration is updated on demand, based on dynamic radio conditions and UE capability.

- Qualcomm (R1-2401432) Proposal 1: For specifying AI/ML positioning, identify functionalities as follows: - Direct AI/ML positioning functionality - AI/ML assisted positioning functionality - AI/ML assisted positioning functionality with timing info model output - AI/ML assisted positioning functionality with LOS indicator model output - AI/ML assisted positioning functionality with joint timing info and LOS indicator model output Proposal 16: For Case1/2a, introduce new capabilities for conveying conditional support by UE for AI/ML positioning: - Area/spatial validity condition of supported AI/ML positioning (Case1/2a) - Timing validity condition of supported AI/ML positioning (Case1/2a) - PRS processing and resource conditions of supported AI/ML positioning (Case1/2a) - PRS measurement reporting conditions of supported AI/ML positioning (Case2a) Proposal 17: For Case1/2a, specify signaling for UE to dynamically convey supported AI/ML positioning capabilities and associated conditions. As a starting point consider the unsolicited capability indication procedure of LPP.

- Apple (R1-2401003) Proposal 1: Regarding LCM of AI/ML based positioning accuracy enhancement, - Functionality based LCM: Applicable to a one-sided model without model transfer and should be used AI/ML based positioning. - For the UE sided model (Case 1 and case 2a) specification shall cover UE capability reporting using UE features and feature groups and help identify the capability of the AI/ML models including scenarios, positioning types (direct or AI-assisted AI-ML positioning), Measurement and Reporting capability and site/scenario/dataset specific capabilities. - For the network side models (case 2b, Case 3a and Case 3b), the specification shall cover the capability of UEs/PRUs on data collection.

- Nokia (R1-2400794) Proposal 10: In case 1 direct AI/ML positioning, for inference, UE can assist LMF by reporting conditions associated to UE capabilities and requirements (e.g., expected QoS, monitoring requirements, ML-related resource requirements, etc.), as well as any preference (order) among different functionalities in order to enable LMF to properly make an LCM functionality decision. Observation 14: A positioning feature corresponds to a positioning method defined in legacy NR positioning framework. Hence, legacy framework with LPP protocol for e.g., capability reporting, measurement reporting can be exploited for functionality-based LCM purposes.

# Proposals for online sessions

## Proposals for Monday online session

Proposal 2.2-1

RAN1 follows the WID and focuses on the positioning accuracy enhancement for 1st priority cases. The enhancements, if applicable as is, can cover 2nd priority case(s) without delay.

- Enhancements specific to the 2nd priority cases, are considered as lower priority if identified, could be considered for normative work after 1st priority use cases.

Proposal 3.1.2-1

For Rel-19 AI/ML based positioning, the measurements for model input are made using based on the existing DL PRS and UL positioning SRS as a starting point.

- Further investigation is not precluded if necessary enhancements are identified.

Proposal 4.1.2-1

For AI/ML assisted positioning (i.e., Case 2a and 3a), at least LOS/NLOS indicator and timing information are supported as potential model output.

- FFS: The exact format of timing information for DL and UL measurements, respectively.

- FFS: whether angle information and power information are also supported as potential model output.

Proposal 4.2.2-1

For AI/ML assisted positioning (i.e., Case 2a and 3a), when LOS/NLOS indicator is reported, the existing IE in 37.355 (LPP for Case 2a) and 38.455 (NRPPa for Case 3a) can be re-used from RAN1 perspective.

## Proposals for Tuesday online session

Proposal 3.1.3-1

For Rel-19 AI/ML based positioning, the measurements for model input are based on the existing DL PRS and UL SRS.

- Note: The use of SRS for MIMO resource is transparent to UE.

Proposal 3.2.3-1a

In Rel-19 AI/ML based positioning, regarding the time domain channel measurements for model input, RAN1 investigate the following alternatives.

- Sample-based measurements, where the timing information is an integer multiple of sampling periods.

  - FFS: details including but not limited to: reference time for sample timing, sampling period, number of samples

- Path-based measurements, where the timing information is according to the detected path timing and may not be an integer multiple of sampling periods.

  - FFS: granularity of path timing, reference time for reporting path timing, number of additional paths (including potential extension from the existing 8 additional paths).

The issues to be studied include, but not limited to, the following:

      - Tradeoff of positioning accuracy and signaling overhead

      - Potential implementation issues to obtain the measurement values

      - Whether the same alternative applies to all sub-cases, or different alternative can be applied to different sub-case

Note: different sub-cases may have different issues.

Note: depending on the measurement choice for model input, additional information is also reported together with the timing information (i.e., power, phase).

Proposal 3.3.3-1

At least for AI/ML based positioning Case 2b and 3b, at least the following types of time domain channel measurements are supported for reporting:

- timing information;

- paired timing information and power information.

FFS: the applicability to Cases 1/2a/3a

Proposal 4.1.3-1

For AI/ML assisted positioning, at least LOS/NLOS indicator and timing information are supported as potential model output.

- FFS: The exact format of timing information for DL and UL measurements, respectively.

Proposal 4.2.3-1

For AI/ML assisted positioning Case 3a, when LOS/NLOS indicator is generated as model output and reported, the indicator can be reported as soft indicator or hard indicator, with the same value range as in existing IE "LoS/NLoS Indicator" in 38.455.

Proposal 4.2.3-2

For AI/ML assisted positioning Case 2a, when LOS/NLOS indicator is generated as model output and reported, the indicator can be reported as soft value or hard value, with the same value range as in existing IE "LOS-NLOS-Indicator" in 37.355.

Proposal 7.1.3-1

For model monitoring of LMF-side model, NW (e.g., LMF) is responsible for calculating the monitoring metric and making model LCM decision.

- Model monitoring is up to NW (e.g., LMF) implementation. There is no need to specify a performance monitoring metric, or a threshold for making performance monitoring decision.

- FFS: whether assistance information from UE and/or gNB would be needed to assist with the model monitoring in LMF.

Proposal 3.4.2-1

For channel measurement as model input for AI/ML positioning, the representation of timing information needs to support reference time alignment between measurement for training data and measurement for model inference.

- The reference time alignment needs to be provided for UL and DL, respectively.

Proposal 3.6.3-1

For channel measurement as model input for AI/ML positioning, RAN1 investigate the necessity of reporting phase information so that CIR can be adopted as model input, considering the following issues:

- tradeoff of positioning accuracy and signaling overhead

- whether fingerprinting ML models can handle CIR phase measurements, which vary not only with the radio channel environment but also with the transmitter/receiver circuits

- specification impact

- Other aspects are not precluded

## Proposals for Thursday online session

Proposal 3.2.5-1

In Rel-19 AI/ML based positioning, regarding the time domain channel measurements, RAN1 investigate the following alternatives:

Alternative (a).  Sample-based measurements, where the timing information is an integer multiple of sampling periods.

Alternative (b).  Path-based measurements, where the timing information is according to the detected path timing and may not be an integer multiple of sampling periods.

The issues to be studied include, but not limited to, the following:

      - Tradeoff of positioning accuracy and signaling overhead

      - Impact of gNB/UE implementation to obtain the channel measurement values. Companies describe their implementation details

      - Whether the same alternative applies to all cases or not

      - Applicability and necessity of specifying the Alternative(s) to different cases

Note: different sub-cases may have different issues.

Note: In addition to timing information, the components for the channel measurement for model input may also include power and potentially phase (subject to further discussion). Companies to provide the details of the channel measurement in their investigation.

Proposal 4.1.5-1a

For AI/ML assisted positioning Case 3a, at least LOS/NLOS indicator and/or timing information are supported for reporting.

- If LOS/NLOS indicator is reported, the indicator can be reported as soft indicator or hard indicator as defined in 38.214.

- If timing information is reported, the timing information at least can be reported via UL RTOA or gNB Rx-Tx time difference as defined in 38.215.

- Note: details of the report are pending further discussion.

Proposal 4.1.5-1b

For AI/ML assisted positioning Case 2a, at least LOS/NLOS indicator and timing information are supported for reporting.

- If LOS/NLOS indicator is reported, the indicator can be reported as soft indicator or hard indicator as defined in 38.214.

- If timing information is reported, the timing information at least can be reported via DL RSTD or UE Rx-Tx time difference as defined in 38.215.

- Note: details of the report are pending further discussion.

Proposal 7.1.5-1

From RAN1 perspective, for performance monitoring of LMF-side model, LMF is responsible for calculating the monitoring metric and making model management decisions.

- FFS: whether/what assistance information from UE and/or gNB need to be sent to LMF to assist with the performance monitoring.

Proposal 3.4.5-1

For AI/ML based positioning Case 3b, for gNB channel measurements reported to LMF to determine model input, the timing information is represented relative to a reference time.

- FFS: details of the reference time

Proposal 3.6.5-1

For AI/ML based positioning, RAN1 investigate the necessity of reporting phase information for determining model input, considering the following issues:

- Tradeoff of positioning accuracy and signaling overhead, the impact of transmitter and receiver implementation

- Specification impact

- Other aspects are not precluded

Note: the phase information may be used in different ways, e.g., a phase value for the first path only; triplet of {timing information, power information, phase information} for CIR

# Agreements made at RAN1#116

The following agreements were made at RAN1#116.

Agreement

For Rel-19 AI/ML based positioning, the measurements for determining model input are based on the DL PRS and UL SRS defined in TS38.211.

- Note: The use of SRS for MIMO resource is transparent to UE.

Agreement

- For AI/ML based positioning case 3b, at least the following types of time domain channel measurements are supported for reporting:

- timing information;

- paired timing information and power information.

Agreement

- For AI/ML based positioning case 2b, at least the following types of time domain channel measurements are supported for UE reporting to LMF:

- timing information;

- paired timing information and power information.

# References

RP-221348, “Revised SID: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface,” 3GPP RAN#96, June 2022.

R1-2400101	AI/ML for Positioning Accuracy Enhancement	Ericsson

R1-2400145	Discussion on positioning accuracy enhancement for AI/ML	Huawei, HiSilicon

R1-2400169	Discussion on specification support for AI/ML positioning accuracy enhancement	ZTE, Pengcheng laboratory

R1-2400233	Specification support for positioning accuracy enhancement	vivo

R1-2400317	Discussion on specification support for positioning accuracy enhancement	CMCC

R1-2400348	Discussion on specification support for positioning accuracy enhancement	TCL

R1-2400377	Specification support for AI/ML for positioning accuracy enhancement	Intel Corporation

R1-2400393	AI/ML based Positioning	Google

R1-2400419	Discussion on AI/ML-based positioning accuracy enhancement	CATT

R1-2400469	Discussion on specification support for AI/ML based positioning accuracy enhancement	NEC

R1-2400544	Discussion on AI/ML-based positioning accuracy enhancement	xiaomi

R1-2400619	On specification for AI/ML-based positioning accuracy enhancements	OPPO

R1-2400693	Specification support for AI-enabled positioning	NVIDIA

R1-2400721	Discussion for supporting AI/ML based positioning accuracy enhancement	Samsung

R1-2400757	Discussion on AI/ML-based positioning accuracy enhancement	CICTCI

R1-2400767	Discussions on specification support for AI/ML positioning accuracy enhancement	Fujitsu

R1-2400794	AI/ML for Positioning Accuracy Enhancement	Nokia, Nokia Shanghai Bell

R1-2400845	Discussions on AI/ML for positioning accuracy enhancement	Sony

R1-2400923	Specification impacts for Enhanced Positioning	Lenovo

R1-2401003	Discussion on Specification support for positioning accuracy enhancement	Apple

R1-2401042	Discussion on support for AIML positioning	InterDigital, Inc.

R1-2401056	Discussion on Support for Positioning Accuracy Enhancement	NTPU

R1-2401476	Discussion on Support for Positioning Accuracy Enhancement	NTPU. Revision of R1-2401056

R1-2401082	Discussion on specification support for AI-ML based positioning accuracy enhancement	Baicells

R1-2401108	Discussion on specification support for positioning accuracy enhancement	NTT DOCOMO, INC.

R1-2401137	Design for AI/ML based positioning	MediaTek Korea Inc.

R1-2401154	Discussion on Specification Support of AI/ML for Positioning Accuracy Enhancement	Indian Institute of Tech (M), IIT Kanpur

R1-2401172	Discussions on specification support for positioning accuracy enhancements	Sharp

R1-2401198	AI/ML positioning accuracy enhancement	Fraunhofer IIS, Fraunhofer HHI

R1-2401268	Discussion on specification support for AI/ML Positioning Accuracy enhancement	CEWiT

R1-2401432	Specification support for AI-ML-based positioning accuracy enhancement	Qualcomm Incorporated

- Ericsson (R1-2400101)

- Huawei (R1-2400145)

- ZTE (R1-2400169)

- vivo (R1-2400233)

- CMCC (R1-2400317)

- TCL (R1-2400348)

- Intel (R1-2400377)

- Google (R1-2400393)

- CATT (R1-2400419)

- NEC (R1-2400469)

- xiaomi (R1-2400544)

- OPPO (R1-2400619)

- NVIDIA (R1-2400693)

- Samsung (R1-2400721)

- CICTCI (R1-2400757)

- Fujitsu (R1-2400767)

- Nokia (R1-2400794)

- Sony (R1-2400845)

- Lenovo (R1-2400923)

- Apple (R1-2401003)

- InterDigital (R1-2401042)

- NTPU (R1-2401476)

- Baicells (R1-2401082)

- NTT DOCOMO (R1-2401108)

- MediaTek (R1-2401137)

- IIT (R1-2401154)

- Sharp (R1-2401172)

- Fraunhofer (R1-2401198)

- CEWiT (R1-2401268)

- Qualcomm (R1-2401432)

# Appendix A. Selected Agreements from XXX