3GPP TSG RAN WG1 #116                                                                                                      R1-2401563

Athens, Greece, Feb 26th – Mar 1st, 2024

Agenda item:	9.1.3.2

Source: 	Moderator (Qualcomm)

Title: 		Final summary for Additional study on AI/ML for NR air interface: CSI compression

Document for:	Discussion and Decision

# Introduction

In RAN#102 plenary meeting, a new WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface was approved ‎[2]. The WID includes study objectives related to AI/ML for CSI compression using a two-sided model:

…… Study objectives with corresponding checkpoints in RAN#105 (Sept ’24): - CSI feedback enhancement [RAN1]: - For CSI compression (two-sided model), further study ways to: - Improve trade-off between performance and complexity/overhead - e.g., considering extending the spatial/frequency compression to spatial/temporal/frequency compression, cell/site specific models, CSI compression plus prediction (compared to Rel-18 non-AI/ML based approach), etc. - Alleviate/resolve issues related to inter-vendor training collaboration. while addressing other aspects requiring further study/conclusion as captured in the conclusions section of the TR 38.843. - …… ……

This document summarizes the issues regarding agenda item 9.1.3.2 (Additional study on AI/ML for NR air interface: CSI compression) in RAN#116.

## Contact Information

Please provide / update your contact information.

| Company | Name | E-mail |
| --- | --- | --- |
| Moderator (Qualcomm) | Jay Kumar Sundararajan | jsundara@qti.qualcomm.com |
| Lenovo | Vahid Pourahmadi | vpourahmadi@lenovo.com wangjf20@lenovo.com |
| New H3C | Lei Zhou | Zhou.leih@h3c.com |
| Samsung | Ameha Tsegaye Abebe | amehat.abebe@samsung.com |
| Indian Institute of Technology Madras | Anil Kumar Yerrapragada | anilkumar@5gtbiitm.in |
| Fujitsu | WANG Guotong (David) | wangguotong@fujitsu.com |
| vivo | Peng SUN | sunpeng@vivo.com |
| SK telecom | Yunesung Kim | yunesung.kim@sktelecom.com |
| Mavenir | Ali Fatih Demir | ali.demir@mavenir.com |
| Panasonic | Tetsuya Yamamoto | yamamoto.tetsuya001@jp.panasonic.com |
| OPPO | Wendong Liu | liuwendong1@oppo.com |
| NTT DOCOMO | Xin Wang Haruhi Echigo | wangx@docomolabs-beijing.com.cn haruhi.echigo.fw@nttdocomo.com |
| ETRI | Anseok Lee | alee@etri.re.kr |
| Ericsson | Jingya Li Siva Muruganathan Jianwei Zhang | Jingya.li@ericsson.com Siva.muruganathan@ericsson.com Jianwei.zhang@ericsson.com |
| ZTE | Lun Li Xingguang Wei | li.lun1@zte.com.cn wei.xingguang@zte.com.cn |
| IIT Kanpur | Shyam Vijay Gadhai Abhishek Kumar Singh | svgadhai@iitk.ac.in abhishekks@iitk.ac.in |
| Spreadtrum | Mimi Chen | Mimi.chen@unisoc.com |
| LG | Jaehoon Chung | jhoon.chung@lge.com |
| CATT | Qianrui Li | liqianrui@catt.cn |
| Intel | Victor Sergeev | Victor.sergeev@intel.com |
| Xiaomi | Liuzhengxuan Liumin | liuzhengxuan@xiaomi.com liumin10@xiaomi.com |
| AT&T | Isfar Tariq Salam Akoum | Isfar.tariq@att.com Salam.akoum@att.com |
| CMCC | Yuhua Cao Yi Zheng Dan Song | caoyuhua@chinamobile.com zhengyi@chinamobile.com songdan@chinamobile.com |
| NVIDIA | Xingqin Lin | xingqinl@nvidia.com |
| MediaTek | Pedram Kheirkhah Sangdeh | Pedram.kheirkhah@mediatek.com |
| Futurewei | Baoling S Sheen | bsheen@futurewei.com |
| TCL | Yunsheng Kuang | yunsheng.kuang@tcl.com |

# Temporal domain aspects of CSI compression

The Release 19 work item description ‎[2] has listed improving the trade-off between performance and complexity/overhead as one of the study objectives and has provided several example approaches.  This section discusses the aspects of “extending the spatial/frequency compression to spatial/temporal/frequency compression” and “CSI compression plus prediction”. In this document, the term “temporal domain aspects of CSI compression” is used as a general term to refer to both these aspects.

## Summary of company proposals

From the submitted contributions, proposals related to temporal domain aspects of CSI compression are summarized below.

FutureWei

Proposal 1: For Rel-19 further study on CSI compression using two-sided model use case, do not consider the option of joint CSI compression and CSI prediction, considering the progress checkpoint schedule.

Proposal 3: In CSI compression using two-sided model sub use case, consider leveraging the temporal-domain attributes as additional input to the AI/ML model to further improve the AI/ML-based CSI reconstruction accuracy.

Huawei, HiSilicon

Proposal 1: To distinguish from the CSI compression plus CSI prediction which uses the future/predicted CSI(s) as input, temporal-spatial-frequency (TSF) domain CSI compression refers to the CSI compression using past CSI as input.

Proposal 2: For the EVM of TSF domain CSI compression, consider the following assumptions for the CSI generation part and CSI reconstruction part, respectively:

- CSI generation part (taking time instance t=2 for example):

  - Model input: original CSI of the current slot (V2) and accumulated CSI information from the last time instance (W1).

  - Model output: CSI feedback of the current slot (VQ2) and accumulated CSI information for the next time instance (W2).

- CSI reconstruction part (taking time instance t=2 for example):

  - Model input: CSI feedback of the current slot (VQ2) and accumulated CSI information from the last time instance (W1').

  - Model output: recovery CSI of the current slot (V2) and accumulated CSI information for the next time instance (W2').

- Note: after inference, the accumulated CSI information at the CSI generation part and CSI reconstruction part are updated from W1 to W2 and from W1' to W2', respectively. The update of the accumulated CSI information does not impact the weights of the models.

Proposal 3: For the EVM of TSF domain CSI compression, consider the following additional EVM aspects on top of the EVM of SF domain CSI compression agreed in Rel-18:

- UCI missing, which may be modelled with a missing rate (e.g., 10%) for each individual CSI report occasion.

- Benchmark: besides Rel-16 eType II CB, Rel-18 SF CSI compression is also considered as benchmark.

- UE speeds for outdoor UEs: besides 30km/h, may optionally consider 10km/h, 60km/h, 90km/h and 120km/h.

- Other EVM aspects, if needed, may follow the principle of Rel-18 SF domain CSI compression, e.g., quantization methods, ground-truth CSI format for training, generalization/scalability cases, training collaboration types, monitoring methods, etc.

Proposal 4: For the additional potential spec impact of TSF domain CSI compression on top of Rel-18 SF domain CSI compression, consider methods to handle the misalignment of the accumulated CSI between NW part model and UE part model due to UCI missing.

Proposal 8: For the EVM of CSI compression plus CSI prediction, consider the following additional EVM aspects on top of the EVM of SF domain CSI compression agreed in Rel-18:

- Non-AI/ML benchmark:

  - Opt1: Rel-16 eType II CB (AI/ML solution predicts one future instance)

  - Opt2: Rel-18 Doppler CB (AI/ML solution predicts multiple future instances)

- AI/ML benchmark: Rel-18 SF domain CSI compression, Rel-18 AI/ML based CSI prediction

- For UE distribution, UE speed, and spatial consistency modelling, it can adopt the EVM for Rel-18 AI/ML based CSI prediction.

Proposal 9: For the particular potential spec impact to support CSI compression plus CSI prediction it may need to be discussed whether these two features are regarded as one model or two separate models from the LCM perspective.

- The impacted LCM procedures include, e.g., data collection, monitoring, inference, model control (activation/deactivation/switching/fallback), etc.

vivo

Proposal 2: Consider AR T-S-F compression with information buffer (without time window) as the starting point for evaluation.

Proposal 3: Consider both AI and non-AI CSI prediction algorithm in the system level evaluation of CSI compression plus prediction, of which the accuracy should be guaranteed to be good enough to distinguish different CSI compression methods.

ZTE

Proposal 1: RAN1 to discuss and clarify the potential candidate alternatives of spatial-frequency-temporal domain CSI compression.

- Alt 1: To leverage past CSI instances to assist the current CSI compression

- Alt 2: To compress future CSI instances

Proposal 2: For CSI compression plus prediction sub-use case, further discuss at least legacy CSI prediction plus AI CSI compression and AI CSI prediction plus AI CSI compression sub-use cases in CSI compression agenda item.

Proposal 3: For CSI compression plus prediction sub-use case, legacy CSI prediction plus AI CSI compression should be prioritized to study and evaluate the performance.

Proposal 4: For legacy CSI prediction plus AI CSI compression sub-use case, further study and evaluate at least the following potential cases:

- Case 1: Model input: predicted precoding matrices of multiple instances

- Case 2: Model input: predicted precoding matrix of each one instance

CMCC

Proposal 1: The use case of spatial/temporal/frequency CSI compression could be evaluated and studied.

Proposal 3: If CSI compression plus CSI prediction is decided to be evaluated, then Rel-18 Doppler codebook and one kind of AI/ML based benchmark could be the benchmarks.

Intel

Proposal 6: At least the following aspects from CSI prediction evaluation methodology are reused for temporal CSI compression simulations:

- Outdoor UEs only with 10/20/30/60/120 km/h.

- Spatial Consistency A for UE movement is assumed.

- Non-AI/ML CSI prediction could be used as an optional benchmark.

- Generalization of the model across UE speeds should be studied.

CATT

Proposal 1: Temporal-spatial-frequency-domain CSI compression is studied in Rel-19 to improve the performance-complexity/overhead trade-off for CSI compression.

Proposal 2: For temporal-spatial-frequency-domain CSI compression, the following two schemes are considered:

- Scheme 1: Extracting the time domain feature from previous time instants for better compression for CSI at a given instant

- Scheme 2: CSI compression of CSI at multiple time instants

NEC

Proposal 1: In order to improve the performance gain of CSI compression, further study to incorporate temporal CSI information into S-F CSI compression, i.e., T-S-F CSI compression.

Proposal 2: In order to reduce the complexity, further study model compression.

Xiaomi

Proposal 1: Companies are encouraged to provide intermediate results and UPT results of SFT compression

Proposal 2: Companies are encouraged to provide intermediate results of generalization / scalability performance for SFT compression.

Proposal 3: Companies are encouraged to provide intermediate results of different training types of SFT compression.

Proposal 4: If AI/ML based CSI prediction performance gain over Rel-18 non-AI/ML based approach is observed, further study the performance of CSI prediction plus CSI compression.

Proposal 12: If there is no output of historic CSI at the previous instance, how to design the current input of historic CSI for two-sided AI/ML model needs to study.

Beijing Jiaotong University

Proposal 1: Study the benefits of using AI/ML for CSI compression in spatial-temporal-frequency domain compression. Further evaluate the performance gains, model size, and FLOPs changes brought by adding the time-domain in the CSI compression domains compared with spatial-frequency compression. The proponent needs to explain whether spatial-frequency compression or spatial-temporal-frequency compression was used when providing results and analysis.

Proposal 2: For the evaluation of spatial-temporal-frequency domain CSI compression, companies are encouraged to report the assumptions of the observation window, input/output CSI type, and model description, with the continuation of the discussion on two-sided training types, model pairing, and model delivery frameworks.

Proposal 3: For the evaluation of CSI compression plus prediction, further evaluate the performance, model size, and FLOPs of UE-side prediction and gNB-side prediction.

Samsung

Proposal 1: Consider the following alternatives for AI/ML-based CSI prediction plus compression in relation to TSF-domain compression

- Non-predictive TSF-domain compression: TSF-domain compression considering multiple CSI measurement occasions (time instants)

- UE-side prediction followed by SF-domain compression: UE predicts for a single future prediction instance and compression in SF-domain

- UE-side prediction followed by TSF-domain compression: UE predicts for a multiple future prediction instances and compression in TSF-domain

- SF-domain compression followed by Network-side prediction: UE compresses CSI from a single measurement occasion in the SF-domain and network predicts for future prediction instance(s) considering multiple past CSI feedbacks

- TSF-domain compression followed by Network-side prediction: UE compresses CSI from multiple measurement occasions in the TSF-domain and network predicts for future prediction instance(s) considering one or more CSI feedback

Fujitsu

Proposal 1: Regarding performance evaluation, RAN1 should focus on the new use cases for two-sided model for the study in Rel-19.

Proposal 2: In Rel-19, RAN1 to study and evaluate the use case of joint CSI prediction and compression for CSI feedback enhancement.

Proposal 3: In Rel-19, RAN1 to study and evaluate the use case of CSI compression over spatial/temporal/frequency domain, i.e., CSI compression assisted by historical CSI information.

Proposal 4: In Rel-19 further study of two-sided model, for performance comparison, the same benchmark should be applied for the new use cases.

Proposal 5: In Rel-19, RAN1 prioritize the study on the use case of joint CSI prediction and compression, and the use case of CSI compression over spatial/temporal/frequency domain, i.e., CSI compression assisted by historical CSI information.

Nokia

Proposal 1: SFT CSI compression schemes should be evaluated in comparison with separate SF CSI compression and prediction as a baseline, in two scenarios: accuracy improvement and overhead reduction.

Proposal 2: In comparing SFT compression with SF CSI compression and prediction for overhead reduction, optimization of the CSI-RS periodicity with respect to the temporal coherence should be taken into account.

Proposal 3: In comparing SFT compression with SF CSI compression and prediction for accuracy improvement, the CSI-RS periodicity should be fixed, and temporal coherence should be at or below the CSI-RS period.

Proposal 4: If recurrent networks are considered for SFT CSI compression, methods for ensuring stability, controlling error propagation, and achieving interoperability should be addressed.

InterDigital

Proposal 1: The performance-complexity trade-off of TSF should be considered for study.

LG Electronics

Proposal 1: If temporal/spatial/frequency (TSF)-CSI compression is supported for an alternative solution for SF-CSI compression,

- consider two-step performance monitoring to check that the performance degradation of the AI/ML model is originated from whether the historical CSI has a problem or the AI/ML model is not suitable for the deployed environment

Proposal 2: If temporal/spatial/frequency (TSF)-CSI compression is supported for an alternative solution for SF-CSI compression, discuss how to generate ground truth CSI for at least for model monitoring.

Proposal 3: Further study on the motivation of CSI compression plus prediction. Also, the performance gain needs to be quantified from a perspective of overall payload and each of the corresponding achievable performance (i.e. CSI compression aspect and CSI prediction aspect).

Proposal 4: On the evaluation of study on CSI compression, the followings are considered:

- Temporal/spatial/frequency domain CSI compression:

- Benchmark scheme: Rel-16 eType-II CSI codebook (considering spatial/frequency domain)

- How to utilize historical CSI for encoding/decoding the current target CSI (e.g. historical channel measurement or extracting the channel properties)

- How to quantify the computational complexity related to the historical CSI on encoder/decoder

- CSI compression plus prediction

- Benchmark scheme

  - Rel-16 eType-II CSI codebook for CSI compression

  - Rel-18 Doppler domain CSI codebook for CSI prediction

- For a comparison purpose, regarding a method via switching the AI/ML model between CSI compression and for CSI prediction, which criterion is adopted to switch between two CSI compression/prediction is notified

Apple

Proposal 1: For time-spatial-frequency domain CSI compression, additional evaluation assumption should be aligned including:

- CSI-RS periodicity of 5ms

- Feedback periodicity of 5ms.

- Time sequence length of 20 samples (100ms) for training/testing

- Detailed methods to generate time sequences for training dataset and testing dataset.

Proposal 3: For joint CSI compression and prediction, focus on compression gain evaluation comparing to R18 codebook design with Doppler basis.

- Assuming perfect channel is known at the encoder side without actual prediction.

Proposal 4: For time-frequency-spatial domain CSI compression, the following potential specification impact are proposed:

- Enable semi-persistent CSI reporting for time-freq-spatial domain AI based CSI compression.

- Enable DCI based reset memory.

- Considering UCI retransmission in case of large amount of UCI drop or loss, to avoid the state at UE and gNB out of sync.

NTT DOCOMO

Proposal 2:

- Study the TSF-CSI compression and joint CSI compression plus prediction as one sub-use case.

- Evaluate its performance gain over R18 eTypeII for predicted PMI and further evaluate the performance of generalization and scalability.

- Study the necessary specification impacts.

Proposal 3: For generalization and scalability, RAN1 focuses on the generalization over multiple aspects, where new aspects introduced by the TSF-CSI or CSI C+P can be included.

- E.g., generalization over UE speeds and carrier frequencies, or different CSI measurement/report period and non-uniform CSI measurement/report period.

- Note: Other aspects are not precluded.

Proposal 7: For R19 performance evaluations, consider at least one group of aligned simulation assumptions for both CSI prediction studied under agenda 9.1.3.1 and CSI compression under agenda 9.1.3.2.

- For the UE distribution and velocity, consider the following two options,

  - Option 1 (mandatory to align the assumptions): 80% indoor and 20% outdoor, where the velocity of outdoor UEs can be [30, 60, 120] kmph.

  - Option 2: 100% outdoor with different velocities [30,  60, 120] kmph to better assess the potential gain of prediction.

- For the baseline performance, consider Rel. 18 doppler CSI as an aligned baseline for both CSI prediction and compression.

  - Revisit the definition of the payload size range for the updated baseline scheme.

- For other parameters, reuse the Rel. 18 simulation assumptions of AI/ML CSI study.

- The complete simulation assumptions are captured in Table 2.

Table  1. Simulation assumptions for both CSI prediction and CSI compression

| Parameter | Value |
| --- | --- |
| Duplex, Waveform | FDD/TDD, OFDM |
| Multiple access | OFDMA |
| Scenario | Dense Urban (Macro only) |
| Frequency Range | 4GHz |
| Inter-BS distance | 200m |
| Channel model | According to TR 38.901 |
| Antenna setup and port layouts at gNB | 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ |
| Antenna setup and port layouts at UE | 2RX: (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ |
| BS Tx power | 44dBm for 20MHz |
| BS antenna height | 25m |
| UE antenna height & gain | According to TR36.873 |
| UE receiver noise figure | 9dB |
| Modulation | Up to 256QAM |
| Coding on PDSCH | LDPC Max code-block size = 8448 bits |
| Numerology | 30KHz, 14 OFDM symbol slot |
| Simulation bandwidth/granularity | 20MHz (48RB)/12 subbands (4 RBs per subband) |
| Frame structure | Slot format 0 for all slots |
| MIMO scheme | SU/MU with rank adaptation Maximum 8 MU layers |
| CSI feedback | CSI feedback periodicity: 5 ms, Scheduling delay: 4 ms |
| Overhead | 2-symbol |
| Traffic model | FTP model 1 with packet size 0.5 Mbytes |
| RU | 20%/50%/70% |
| UE distribution | Option 1: 80% indoor (3km/h), 20% outdoor (30, 60, 120 kmph) Option 2: 100% outdoor (30, 60, 120 kmph) |
| UE receiver | MMSE-IRC |
| Feedback assumption | Realistic |
| Channel estimation | Ideal channel estimation for dataset construction Ideal channel estimation for CSI acquisition Realistic channel estimation for demodulation |
| KPI | 5% UPT, Average UPT |
| Baseline | Rel. 18 enhanced TypeII codebook for predicted PMI |
| (De-)quantization method | 2-bit uniform before/after decoder/encoder |
| Input for AI/ML model | Precoding matrix(eigenvector) |
| Output for AI/ML model | Precoding matrix(eigenvector) |

Proposal 8: Consider the following evaluation items for the Rel. 19 CSI compression study,

- Benchmark performance of eventual KPI for all sub-use cases proposed for Rel. 19 CSI compression, i.e., TSF-CSI, CSI C+P, and site-/cell-specific models.

- Generalization and scalability with intermediate KPI for more practical cases, including the combinations of multiple generalization/scalability factors and the newly introduced factors regarding time-domain processing (UE velocity, CSI measurement/report interval, etc).

CAICT

Proposal 1: Spatial/temporal/frequency CSI feedback compression should be studied to fully develop the potential of AI-based solution.

Proposal 2: The simulation framework for spatial/temporal/frequency CSI feedback compression should follow the basic assumptions for spatial/frequency CSI feedback.

Proposal 3: Spatial/temporal/frequency CSI feedback compression should consider different CSI feedback period.

Proposal 4: Spatial/temporal/frequency CSI feedback compression could reuse the specification effect conclusions of spatial/frequency CSI feedback compression.

IIT Kanpur, IIT Madras

Proposal 1: Consider the following parameters as part of evaluation assumptions for spatial-frequency-temporal domain compression:

- different UE movement profile modelling

- temporal window (in time unit)

- number of time-domain CSI instances, for training and inference

- range of UE speeds

- spatial consistency

Proposal 2: Consider CSI-RS resource configurations, CSI report configurations, and subsequently the feedback overhead size of Rel-18 codebook for high/medium UE velocities for comparison with spatial-frequency-temporal domain CSI compression.

Proposal 3: Study the specification impact considering potential enhancements on CSI-RS resource configurations of Rel-18 codebook for high/medium UE velocities.

Proposal 4: Study specification impact considering Rel-18 codebook reporting configuration and format of output-CSI-UE and input-CSI-NW for including temporal domain CSI.

Sharp

Proposal 1: Select spatial/temporal/frequency domain CSI compression as a sub-use case for CSI feedback enhancement in Rel-19 AI/ML for NR air interface.

Proposal 2: Select joint CSI prediction and CSI compression as a sub-use case for CSI feedback enhancement in Rel-19 AI/ML for NR air interface.

ETRI

Proposal 1: Consider the Spatial-frequency-time (SFT) domain CSI compression sub-use case for further study.

Proposal 2: Consider the Cell/site/scenario specific model for both the Spatial-frequency (SF) domain and the Spatial-frequency-time (SFT) domain CSI compression sub-use cases for further study, and study the performance of the Cell/site/scenario specific model compared to the global models.

CEWiT

Proposal : Consider temporal domain along with spatial-frequency domain for CSI compression. Evaluate the results for realistic channel asumptions and infer the possible gains over other methods.

Proposal : Consider the effect of UE speed while considering the batches of CSI input to the model for AI/ML based SFT-CSI compression.

Proposal :  Study the effect of pre-processing techniques like 2-dimensional Fourier transform to exploit sparsity in the evaluation of CSI prediction.

ITL

Proposal 1: The complexity-performance trade-off to implement Temporal-Spatial-Frequency (TSF) compression should be evaluated for study.

Qualcomm

Proposal 7:	Consider the following sub-use cases related to temporal domain CSI compression:

- Case 1: Compress present CSI and report.

  - Case 1A: Report is a standalone report.

  - Case 1B: Report is a differential report that depends on a prior report.

- Case 2: Predict one future instance or multiple present/future instances of CSI, compress them and report.

  - Case 2A: Report is a standalone report.

  - Case 2B: Report is a differential report that depends on a prior report.

Proposal 8:	For studying different compression algorithms, the evaluation scenarios should at least include cases where the end-to-end performance is either compression-limited or prediction-compression-limited.

Proposal 9:	Study the benefits of ML compression over time/frequency/spatial domains in

- Indoor scenarios having mostly NLOS and low-speed UEs (e.g., Dense Urban scenario with 80% indoor UEs)

- Outdoor scenarios having mostly LOS UEs (e.g., Dense Urban with 100% outdoor UEs) and different UE speeds.

Proposal 10:	For outdoor scenarios with UE speeds of 30 km/h or higher, study the performance assuming an aperiodic CSI-RS burst with K resources which is scheduled periodically.

Proposal 11:	For compressing schemes with different reporting periodicities and/or payload size, for fair comparison consider the reporting bitrate as the overhead metric, e.g., number of bits per CSI-RS period.

Proposal 12:	For capturing the evaluations results on temporal domain compression, Rel-18 results Table-1 can be used as a starting point with the following modifications:

- Add the time domain assumptions (e.g., UE speed, CSI feedback periodicity, observation window, prediction window, …, etc.).

- Add the descriptions of the prediction algorithms for the baseline and the AI/ML algorithms (e.g., input type, output type, and prediction method.).

- If CSI feedback periodicity is different from the CSI-RS periodicity, the overhead is normalized and expressed in terms of reporting bitrate (e.g., bits per CSI-RS period).

Proposal 13:	For the evaluation of temporal domain compression, include the ideal prediction scenario as one evaluation scenario for reference, i.e., a comparison of different temporal domain CSI compression schemes where the CSI prediction is ideal.

NTPU

Proposal 1: RAN1 to down-select one scheme among S-T-F CSI compression and CSI compression plus CSI prediction

## 1st round discussion

### Categorization

FL note:

Several contributions have raised the issue of clarifying the categories of the temporal domain aspects of CSI compression. There are different proposals with some common themes related to whether the UE and/or gNB use past information as input and whether the CSI report is for the present time or includes the future. Defining the different categories of temporal compression would be useful to align the evaluation methodology among companies.

Taking this into account, the following categorization is proposed. In this discussion, “target CSI slot(s)” refers to the slot(s) to which the CSI feedback in the report corresponds.

| Case | Target CSI slot(s) | Whether CSI generation part uses past information | Whether CSI reconstruction part uses past information |
| --- | --- | --- | --- |
| 1 | Present slot | Companies to report | No |
| 2 | Present slot | Yes | Yes |
| 3 | Future slot(s) | Yes | No |
| 4 | Future slot(s) | Yes | Yes |

Case 1 refers to the spatial-frequency compression that was already studied in Release 18. Whether the CSI generation part uses past information as an additional input is up to implementation and left for companies to report.

Case 2 refers to the case where the CSI report is for the present slot, but both the UE-side and gNB-side models use past information. This allows the CSI feedback to be compressed better since it could, for example, be conveyed in a differential manner relative to a previous report. The following figures from the company contributions could be examples of Case 2:

Figure from ‎[5]:

Figure from ‎[8]:

Figure from ‎[28]:

Case 3 covers the case where the CSI report is predictive, i.e., the final CSI retrieved by the gNB corresponds to one or more future slots. In this case, while the UE uses past information, the gNB does not. There are different implementations that can be categorized under Case 3. For example, the UE-side processing may perform prediction as a separate step or jointly with compression as part of the CSI generation model. Similarly, the gNB may perform prediction as a separate step or jointly with reconstruction as part of the CSI reconstruction model.

The following figures from the company contributions could be examples of Case 3:

Figure from ‎[8] (example of separate prediction at UE-side):

Figure from ‎[40] (example where prediction is not a separate step):

Figure from ‎[18] (example of separate prediction at gNB-side):

Figure from ‎[21]:

Case 4 is also a case where the CSI report is predictive, but unlike case 3, the gNB uses past information in the CSI reconstruction part. The following figures from the company contributions could be examples of Case 4:

Figures from ‎[23] (example where prediction is not a separate step):

Based on this discussion, the following proposal is raised. Companies are requested to provide comments on the categorization, and also on which cases should be prioritized.

#### Proposal 2.2.1

For the evaluation of temporal domain aspects of AI/ML-based CSI compression using two-sided model in Release 19, adopt the following categorization for study:

| Case | Target CSI slot(s) | Whether CSI generation part uses past information | Whether CSI reconstruction part uses past information |
| --- | --- | --- | --- |
| 1 | Present slot | Companies to report | No |
| 2 | Present slot | Yes | Yes |
| 3 | Future slot(s) | Yes | No |
| 4 | Future slot(s) | Yes | Yes |

Note 1: For the CSI generation part, the past information may include past model inputs and/or any information derived from them. For the CSI reconstruction part, the past information may include past CSI feedback instances and/or any information derived from them.

Note 2: For case 3 and case 4, the CSI generation model at the UE may perform prediction as a separate step or jointly with compression. Similarly, the CSI reconstruction model at the gNB may perform prediction as a separate step or jointly with reconstruction.

Note 3: “Target CSI slot(s)” refers to the slot(s) to which the CSI feedback in the report corresponds.

| Support / Can accept | Lenovo，New H3C, Futurewei, NTT DOCOMO, NEC, Apple, Spreadtrum, LG |
| --- | --- |
| Object / Have a concern |  |

| Company | Comments |
| --- | --- |
| Samsung | Thank you FL for your efforts. We are in general fine with the above categorization. However, we may need to clarify where the ‘present slot’ is located. Is that the one that carries the CSI report or the slot containing latest measured CSI-RS resource? In legacy non-predictive CSI (and some cases of predictive CSI), the reference slot is still in the past of the slot that carries the CSI report. |
| Huawei/HiSi | In general OK with the categorization, but still some comments in below: 1) For Case 1, as far as we know, R18 simulation did not consider past CSI as input in general, since “Spatial-frequency domain CSI compression using two-sided AI model is selected as one representative sub use case”. So, it is changed to “No as baseline”. 2) For Case 3 and Case 4, there are some variants which should be noted from the EVM perspective. One aspect is whether the predicted future CSI is one instance or multiple instances – for the former case, AI based compression has the same effect as it has to the R16 CB; for the latter case, AI based compression may have additional temporal domain effect over R18 Doppler CB. The other aspect is whether the predicted CSI is subject to non-AI or AI – for the former case, we only observe the gain of AI CSI compression, while for the latter case, we also observe the gain of AI CSI prediction. These assumptions should be reported by companies. 3) As mentioned in the FL note, Case 2 is based on differential CSI report based on accumulated CSI info. for both sides, it is then provided as an example of information derived from past input/past CSI feedback under Note 1. Changes: Based on 1), changes made for Case 1. Based on 2), changes made by adding Note 4 and Note 5. Based on 3), changes made for Note 1. In addition, “past information” is changed to “past CSI information” for clarity. Case; Target CSI slot(s); Whether CSI generation part uses past CSI information; Whether CSI reconstruction part uses past CSI information. 1; Present slot; No as baseline. Companies to report if past CSI is additionally considered; No. 2; Present slot; Yes; Yes. 3; Future slot(s); Yes; No. 4; Future slot(s); Yes; Yes. Note 1: For the CSI generation part, the past information may include past model inputs and/or any information derived from them, e.g., accumulated CSI at CSI generation side. For the CSI reconstruction part, the past information may include past CSI feedback instances and/or any information derived from them, e.g., accumulated CSI at CSI reconstruction side. Note 2: For case 3 and case 4, the CSI generation model at the UE may perform prediction as a separate step or jointly with compression. Similarly, the CSI reconstruction model at the gNB may perform prediction as a separate step or jointly with reconstruction. Note 3: “Target CSI slot(s)” refers to the slot(s) to which the CSI feedback in the report corresponds. Note 4: For Case 3/4, company to report whether the future slot(s) include a single predicted instance (3a/4a), or multiple predicted instances (3b/4b). Note 5: For Case 3/4, company to report whether the predicted CSI is AI based (3a-1/3b-1/4a-1/4b-1), or non-AI based (3a-2/3b-2/4a-2/4b-2) . |
| Fujitsu | Thanks FL for the efforts. Generally fine with the direction to have some categorization on the use cases in Rel-19. Regarding the cases, we suggest to add one more case, which is the target CSI slot is present slot and the past CSI information is applied only at the CSI reconstruction part. Please see below for the change. Case; Target CSI slot(s); Whether CSI generation part uses past information; Whether CSI reconstruction part uses past information. 1; Present slot; Companies to report; No. 2; Present slot; Yes; Yes. 3; Future slot(s); Yes; No. 4; Future slot(s); Yes; Yes. 5; Present slot; No; Yes. |
| vivo | In general fine. For Note 2, we prefer to further limit the case to separate UE CSI prediction. Otherwise, the discussion would be mixed with the performance of CSI prediction itself, which should be studied under another agenda item. By separating CSI prediction, we could fix the prediction algorithm, e.g., to legacy CSI prediction algorithm. Then the performance would be comparable between companies for TSF compression. |
| ETRI | Thank you FL for the great efforts. We are generally fine with the categorization. Our question is whether Case 1 is the same with the Spatial-Frequency domain CSI compression (as studied in Rel-18 study). If yes, 'Whether CSI generation part uses past information' should be changed to No. And we can add new case (e.g., 1a), if we need the case that CSI generation part additionally uses past information, to avoid further confusion. |
| ZTE | Thanks for FL’s categorization. For Case 3/4, there may be still some different understandings among companies. For example, for the separate CSI prediction and CSI compression parts, legacy non-AI method or AI-based method can be used to predict future CSI instances. We suggest to further categorize the potential alternatives for Case 3/4. Case 3/4: - Alt 1: Legacy CSI prediction plus AI CSI compression - Alt 2: AI CSI prediction plus AI CSI compression (separate AI models) - Alt 3: AI CSI prediction plus AI CSI compression (joint AI models) For Alt 1, we think it can be prioritized to study due to the limited time in Rel-19 study phase. For one reason, the primary intention of this agenda item is to improve the performance of Rel-18 AI CSI compression, and Alt 1 can match the motivation well that AI/ML model is just applied for CSI compression part. Compared with Alt 1, Alt 2 and Alt 3 introduce the performance of gain of AI/ML-based CSI prediction in addition to CSI compression, which is difficult to simply identify the performance gain of AI CSI compression. For another reason, Rel-18 Doppler CSI can be directly adopted as the baseline for Alt 1, since Rel-18 Doppler CSI codebook is also a CSI compression method to compress multiple time-domain CSI instances by introducing Q=2 doppler basis. In addition, we think Case 2 and Case 3 can be combined together, for example, there are present slot and future slots in the CSI report. So, it seems a special case in Case 3 as Case 3 also includes the present slot CSI. We propose to add a note as below: Note 4: Different case combination is not precluded, e.g., the target CSI slot can be both present slot and future slot. |
| LG | Generally fine with the proposal. Regarding Case 3/4, the clarification for CSI compression + prediction seems to be needed since the computational complexity relying on two-sided model including CSI generation and reconstruction part is much higher than that of AI/ML for CSI prediction only. And the handling on the related model collaboration/pairing issue is also necessary. |
| CATT | We share similar concern as HW. Case 3 and case 4 have too many possible variants. Especially, when considering separate CSI prediction and CSI compression, further clarification on whether CSI prediction is before (UE side prediction) or after CSI compression (gNB side prediction) is needed. As is mentioned in FL notes, for UE side prediction cascading with CSI compression, the UE uses past information, the gNB does not for both generation and reconstruction part models. Therefore, we propose to add the following categorization: 5; Future slot(s); No; No. where UE side utilizes CSI prediction cascading an AI-based CSI encoder, the gNB side has an AI-based CSI decoder. |
| Intel | For case 1, given that target slot is current slot, prediction is not done for this case, CSI generation part will not do any averaging as well in this case. So, what is the purpose to use past information for the CSI generation part? Can we simply put ‘No’ for Case 1 for ‘Whether CSI generation part uses past information’? |
| Xiaomi | We are fine with proposal, and suggest that Case 2 is studied with higher priority. |
| AT&T | Is this table for clarification purposes or are we asking the companies to also provide for case 3 and 4. Case 1 and 2 corresponds to CSI compression however case 3 and 4 corresponds to CSI compression and prediction which was agreed not to be studied as a representative use case for CSI enhancement in Rel-18. We do not support studying joint CSI compression and prediction. |
| TCL | Thanks for FL’s efforts to summarize the above categorization. We are fine in general but have some doubts and comments in below: 1) For Case 1, we feel confused about the meaning of “companies to report” in the column of “whether CSI generation part uses past information”. If it means UE reports the past CSIs to gNB as the output of the CSI generation part, it seems eccentric that gNB does not utilize the reported past CSIs in the CSI reconstruction part. On the other hand, if it means UE only utilizes the past CSI information reported by companies in CSI generation part, we are wondering how gNB can recover the compressed CSI without using past information. So please make it clear. 2) For Case 3 and 4, the input of CSI generation part is different when the CSI prediction module is AI-based or non AI-based. And the AI/ML-based CSI prediction is another agenda item for study. To facilitate the performance evaluation of CSI compression plus CSI prediction, it is better to limit the CSI prediction algorithm as non AI-based, e.g., Rel-18 Doppler domain compression codebook. |

### Evaluation scenario

FL note: Release 18 study for CSI prediction considered 100% outdoor dense urban scenario, while the CSI compression study considered the 80% indoor, 20% outdoor scenario. For the study on temporal domain aspects of CSI compression, since some cases may also involve prediction, it would be beneficial to study both cases.

#### Proposal 2.2.2

For the evaluation of temporal domain aspects of AI/ML-based CSI compression using two-sided model in Release 19, adopt the following as baseline options for UE distribution:

- Option 1: 80% indoor, 20% outdoor

- Option 2: 100% outdoor

Note: Indoor speed is 3 km/h, outdoor speed is chosen from the following options: 10 km/h, 20 km/h, 30 km/h, 60 km/h, 120 km/h.

| Support / Can accept | Lenovo, New H3C, Samsung, Futurewei, Fujitsu, NTT DOCOMO, NEC, ETRI, ZTE, Spreadtrum, LG, CATT, Intel, CMCC |
| --- | --- |
| Object / Have a concern | Huawei/HiSi |

| Company | Comments |
| --- | --- |
| Huawei/HiSi | The two options should be separately considered for Case 2 and Case 3/4 (or their variants). For Case 2, its applicable case is the same as Case 1, i.e., Option 1. For Case 3/4, as it leverages the CSI prediction, UE distribution should be aligned with CSI prediction only, i.e., Option 2. In addition, besides UE speed, other assumptions of channel modeling for Option 2 should follow R18 AI based CSI prediction. For the evaluation of temporal domain aspects of AI/ML-based CSI compression using two-sided model in Release 19, adopt the following as baseline options for UE distribution: - Option 1 (for Case 2): 80% indoor, 20% outdoor - Option 2 (for Case 3/4 or their variants): 100% outdoor Note: Indoor speed is 3 km/h, outdoor speed is chosen from the following options: 10 km/h, 20 km/h, 30 km/h, 60 km/h, 120 km/h. Assumption on types of model input, O2I car penetration loss and spatial consistency follow the R18 AI based CSI prediction. |
| Fujitsu | Generally fine with the proposal. We don’t agree with the change from HW. For case 2/3/4, all of them utilize the temporal domain information. So, Option 1 and Option 2 could be applied. We don’t see the need to differentiate the cases for Option 2 and Option 2. |
| vivo | Rel-18 experience shows that 10km/h, 20km/h seems to be speed companies typically do not evaluate. |
| ZTE | We are open to discuss the two options. |
| Xiaomi | OK. We suggest that Option 1 is applied to Case 1 or Case 2, while Option 2 is applied to Case 3 and Case 4. |

### Benchmark scheme for performance comparison

FL note: The R18 study item on CSI compression used R16/17 Type II codebook as the benchmark, while R18 CSI prediction used nearest historical CSI, non-AI/ML prediction and level x AI/ML based CSI prediction as the benchmark. For the study on temporal domain aspects of CSI compression, since some cases may include prediction, the benchmark scheme needs to be discussed. The following proposal is to reuse the benchmark schemes assumed in Release 18 for the appropriate cases.

#### Proposal 2.2.3

For the evaluation of temporal domain aspects of AI/ML-based CSI compression using two-sided model in Release 19, adopt the following benchmark scheme for performance comparison:

- For cases without prediction of future CSI, use the same benchmark scheme assumed in R18 AI/ML-based CSI compression study.

- For cases with prediction of future CSI, use the same benchmark scheme assumed in R18 AI/ML-based CSI prediction study, with R18 MIMO eType II codebook for compressing the feedback.

| Support / Can accept | Lenovo, New H3C, Futurewei (with comments below), vivo, NTT DOCOMO, NEC, ETRI, Spreadtrum, LG, CATT, CMCC |
| --- | --- |
| Object / Have a concern | Fujitsu, ZTE |

| Company | Comments |
| --- | --- |
| Futurewei | We are ok for the case without prediction of future CSI. For the 2nd case, we would like to clarify as the AI/ML-based CSI prediction in Rel-18 only studied UE-side model, some companies may not have studied this sub use case. For Rel-19 benchmark, if companies choose to support this case but didn’t perform the UE-side CSI prediction study in Rel-18 (thus, the Excel result file doesn’t contain their results), it would be better to perform CSI prediction study using assumptions agreed in Rel-18 first then use the results as the benchmark for comparison purpose. |
| Huawei/HiSi | 1) Except non-AI based benchmarks, AI based benchmarks evaluated in R18 should also be considered to justify the gain over R18 AI. In particular, for Case 3/4, both AI based SF domain CSI compression and AI based CSI prediction should be considered. 2) For prediction of future CSI, the benchmark of nearest historic CSI is not applicable to R18 Doppler CB. So, benchmark of nearest historic CSI can still adopt R16 eType II CB, while benchmark with predicted CSI (non-AI or level x AI) can adopt R18 Doppler CB. - For cases without prediction of future CSI, use the same benchmark scheme assumed in R18 AI/ML-based CSI compression study. - Also consider R18 AI/ML-based SF domain CSI compression as benchmark - For cases with prediction of future CSI, use the same benchmark scheme assumed in R18 AI/ML-based CSI prediction study, with R18 MIMO eType II codebook for compressing the feedback if the benchmark adopts prediction of future CSI. - Also consider R18 AI/ML-based SF domain CSI compression and R18 level y/z AI/ML-based CSI prediction as benchmarks |
| Fujitsu | We think the same benchmark should be applied for new sub use cases in Release 19. If some cases are compared with Rel-18 eTypeII codebook, while some cases are compared with Rel-16 codebook (the benchmark used in R18 study), then this kind of comparison is not fair. Since Rel-18 codebook for predicted PMI is already specified in Rel-18 spec, when new features are introduced, the performance gain over Rel-18 scheme should be demonstrated. If RAN1 introduces some feature which doesn’t perform as well as prior Release, then it’s just waste of time and efforts and makes no sense. In addition, for the case without prediction of future CSI, since the correlation over the temporal domain are utilized for compression (e.g, CSI compression assisted by historical CSI information), it is not suitable to compare with Rel-16 codebook. |
| ZTE | We agree with Fujitsu. The same benchmark should be adopted for new sub use cases for fair comparison. If Case 2 adopts Rel-16 Type II as benchmark and Case 3 adopts Rel-18 Type II as benchmark to compare the performance gain together, maybe Case 2 observes more performance gain than Case 3 since different benchmarks are compared, which not verifies which case has better performance. Therefore, it is not a fair comparison. We propose that at least Rel-18 MIMO eType II codebook should be adopted as the same benchmark for compressing the feedback overhead among new sub use cases in Rel-19 and detailed performance evaluation between cases can be further discussed. |
| LG | Not only for AI/ML based baseline, non-AI/ML baseline scheme can be taken into account simultaneously. Especially for future CSI, Rel-18 Doppler domain compression CSI can be considered as a baseline performance |
| Intel | To understand the full picture, it is better to have data corresponding to non-AI/ML CSI with non-AI/ML prediction and without prediction given that it will have unique performance and complexity tradeoff. For the evaluation of temporal domain aspects of AI/ML-based CSI compression using two-sided model in Release 19, adopt the following benchmark schemes for performance comparison: - For cases without prediction of future CSI, use the same benchmark scheme assumed in R18 AI/ML-based CSI compression study. - For cases with prediction of future CSI, use the same benchmark scheme assumed in R18 AI/ML-based CSI prediction study, with R18 MIMO eType II codebook for compressing the feedback. |
| Xiaomi | Generally OK with the proposal. But, for cases with prediction of future CSI, R18 MIMO eType II codebook should be R18 MIMO eType II codebook for predicted PMI for clarification. |

### Evaluation metrics

FL note: Release 18 study assumed CSI feedback overhead as one of the baseline metrics, and this was computed based on the CSI payload size of a CSI report. With temporal compression, the scheme may enable a single report to convey CSI for multiple slots. In such a case, the reporting periodicity may be reduced. In contrast, a scheme that does not use temporal compression may operate with more frequent reporting. For a fair comparison of such cases, the CSI feedback overhead would need to be normalized appropriately to account for the difference in the reporting frequency.

#### Proposal 2.2.4

For the evaluation of temporal domain aspects of AI/ML-based CSI compression using two-sided model in Release 19, adopt the CSI feedback overhead rate as a baseline metric, where the CSI feedback overhead rate is the CSI feedback overhead per unit time.

| Support / Can accept | Fujitsu, CATT |
| --- | --- |
| Object / Have a concern |  |

| Company | Comments |
| --- | --- |
| Huawei/HiSi | 1) We have a question: for “a single report to convey CSI for multiple slots”, is it only applicable to Case 3/4? For Case 2, as it only address/reports the CSI of the present slot, it seems no need to include multiple CSIs in the report. 2) If CSI of multiple slots are addressed, the CSI report periodicity can be aligned between schemes. E.g., for Case 3, if future slot(s) include N4>1 predicted CSIs and d as the prediction time unit, i.e., the CSI report periodicity is d*N4 for Case 3, then the CSI report periodicity for benchmark should also be d*N4 for fair comparison. For the evaluation of temporal domain aspects of AI/ML-based CSI compression using two-sided model in Release 19, for Case 3/4, the CSI feedback periodicity over schemes should be aligned. adopt the CSI feedback overhead rate as a baseline metric, where the CSI feedback overhead rate is the CSI feedback overhead per unit time. |
| vivo | May need to define the term “overhead rate” first? Not sure which specific case (case1, 2, 3, 4 defined above) the proposal trying to state. Since for cases where multiple slots of CSI are reported, the benchmark may also corresponds to the case with multiple slots of CSI. Thus the comparison is still fair. |
| NTT DOCOMO | From the NW perspective, the number of UCI transmissions per time unit is also important. Given the feedback overhead rate, it is also beneficial if the UCI transmissions can be reduced by the AI/ML model. Therefore, we propose to also consider UCI transmission rate as a metric along with the overhead rate. |
| ZTE | We are not clear about the definition of unit time (whether it is N4?), which needs further clarification. |
| LG | Rel-16 eType-II CSI and Rel-18 Doppler domain CSI are considered at the same time for each performance gain with sum of each payload depending on each Case for CSI compression (and/or prediction) |
| Intel | It is not clear how overhead rate is calculated if CSI overhead is dynamically changing. In our view maximum CSI overhead in a time instance divided by periodicity shall be used as the overhead rate since gNB shall allocate enough resources without prior knowledge. Thus, we’d prefer to spell it out in the Proposal. |
| Xiaomi | OK |
| TCL | According to the definition of CSI feedback overhead rate, we think this kind of evaluation metric only refers to the case of periodic or semi-persistent CSI report configuration. But for aperiodic CSI report case, how to calculate the “rate”? |
| Intel | As we mentioned in our previous comment, we prefer maximum CSI overhead for overhead rate calculation. Also, if the average UCI rate and maximum CSI overhead per report is disclosed, we are fine to accept the proposal. Also, baseline metrics representing overhead should be only one so that it can be plotted on 2D figure in X-axis (performance is Y-axis). Thus, we propose the following modifications for proposal 2.2.4. #### Proposal For the evaluation of temporal domain aspects of AI/ML-based CSI compression using two-sided model in Release 19, adopt the CSI feedback overhead rate as a baseline metric, where the CSI feedback overhead rate is the average bit-rate of CSI feedback overhead per unit across time. Also adopt Companies to disclose the rate of UCI transmission occasions for CSI report as a baseline metric, where rate of UCI transmission occasions for CSI report is the average number of UCIs for CSI report per time unit. If CSI payload size is dynamically changing across CSI reports in time, companies to disclose maximum possible CSI payload size for the utilized CSI configuration. |

### Other evaluation assumptions

FL note: The study of temporal domain aspects requires alignment among companies on some other evaluation assumptions such as CSI reporting periodicity, CSI-RS configuration, etc. As temporal compression allows a single report to compress CSI for multiple time-steps, it would be useful to consider different CSI reporting periodicities. For other parameters such as the observation window and prediction window, the proposal is for companies to report the assumed values, similar to the CSI prediction study in R18.

#### Proposal 2.2.5

For the evaluation of temporal domain aspects of AI/ML-based CSI compression using two-sided model in Release 19, adopt the following evaluation assumptions:

- CSI-RS configuration

  - Periodic: 5 ms periodicity

  - Aperiodic: CSI-RS burst with K resources and time interval m milliseconds (based on R18 MIMO eType-II)

    - FFS: Value of K and m.

- CSI reporting periodicity: {5, 10, 20} ms

- For cases with the use of past information, companies to report observation window, including number/time distance of historic CSI/channel measurements.

- For cases with prediction, companies to report prediction window, including number/time distance of predicted CSI/channel.

| Support / Can accept | Lenovo, New H3C, Futurewei, NTT DOCOMO(w/ comments), NEC, Apple, Spreadtrum, LG, CATT |
| --- | --- |
| Object / Have a concern | SK telecom |

| Company | Comments |
| --- | --- |
| Huawei/HiSi | 1) For CSI-RS configuration, the K>1 RS resources in one resource Set is applicable to only R18 Doppler CB, which is used for CSI prediction. Therefore, K>1 CSI-RS burst is applicable to Case 3/4. 2) For CSI reporting periodicity, as Case 2 only considers the present CSI, the 5ms periodicity adopted for R16 eType II CB and R18 AI SF domain CSI compression should be kept as baseline for fair comparison. 3) For the 3rd bullet, it mentions the observation window. Under Case 2, the widely adopted method is to feedback differentiated CSI on top of accumulate CSI, it is then suggested companies report whether/how the accumulated CSI is generated from the observation window. For the evaluation of temporal domain aspects of AI/ML-based CSI compression using two-sided model in Release 19, adopt the following evaluation assumptions: - CSI-RS configuration - Periodic: 5 ms periodicity - Aperiodic (for Case 3/4): CSI-RS burst with K resources and time interval m milliseconds (based on R18 MIMO eType-II) - FFS: Value of K and m. - CSI reporting periodicity: 5ms for Case 2 as baseline, {5, 10, 20} ms for Case 3/4 - For cases with the use of past information, companies to report observation window, including number/time distance of historic CSI/channel measurements, and whether/how the accumulated CSI is generated from the observation window. - For cases with prediction, companies to report prediction window, including number/time distance of predicted CSI/channel. |
| Fujitsu | We think the CSI reporting periodicity should be reported by companies in the evaluation. The CSI reporting periodicity should be determined by CSI application time, including the CSI processing delay and scheduling delay. Since the CSI application time is closely related with scheduling scheme, the CSI reporting periodicity should be reported by companies instead of setting fixed values. |
| vivo | We prefer also to have {5, 10, 20}ms of CSI-RS configuration since they are matched with CSI reporting periodicity. |
| SK telecom | We believe that 20ms of CSI-RS configuration is also considered from the following two reasons. i) for periodic CSI-RS and reporting, 5ms periodicity is very rare setting in commercial 5G network (It was confirmed that all gNB manufacturers in our 5G network do not support 5ms CSI report, at least now), ii) for aperiodic CSI-RS and reporting, we have strong concern on DL interference issues for stationary users from aperiodic CSI-RS burst of 5ms periodicity. In UL, aperiodic SRS approach is reasonable since it can be multiplexed with other user’s SRS without interference issues using different comb and cyclic shift. However, CSI-RS do not have that kind of multiplexing mechanism. CSI-RS burst for some high-speed UE is strong interference source for other users in a cell or neighbour cell. |
| NTT DOCOMO | We would like to clarify that if only CSI-RS burst is considered for aperiodic CSI-RS. |
| LG | Various UE speeds also can be considered since it affects the temporal coherence on historical CSI. |
| Intel | If no CSI prediction is done or if prediction window corresponds to a single precoding matrix reported via PMI, it is not necessary to have CSI-RS periodicity smaller than CSI reporting periodicity. So, in our view CSI-RS periodicity should be equal to the CSI report periodicity as baseline assumption. Actual CSI-RS periodicity used for simulations shall be disclosed by companies. |
| Xiaomi | For cases with the use of past information, the definition of observation window is not clear. E.g., how to define the starting point of the observation window? For cases with prediction, the evaluation assumptions of R18 eType II codebook for predicted PMI could be used as a starting point. |
| CMCC | Not sure the difference between AP CSI-RS and P CSI-RS from the perspective of evaluation, except for the value of K is actually the length of observation window, while for P-CSI, the observation window is reported by companies. We suggest to focus on P CSI-RS and the periodicity of it could have multiple candidate values. |

### Ideal prediction

FL note: To better understand the performance results and for calibration among companies, it would be useful to consider a special case where the CSI prediction is assumed to be ideal. This could help to isolate the compression related performance aspect and could also help identify whether the scenario is prediction-limited or compression-limited.

#### Proposal 2.2.6

For the evaluation of temporal domain aspects of AI/ML-based CSI compression using two-sided model in Release 19, for cases with prediction of future CSI, companies may optionally evaluate a scheme with ideal prediction as an additional evaluation case for reference.

| Support / Can accept | Lenovo, New H3C, Samsung, Futurewei, Huawei/HiSi, Fujitsu, vivo, NTT DOCOMO, NEC, ETRI, Apple, ZTE, Spreadtrum, LG, OK, CMCC |
| --- | --- |
| Object / Have a concern | AT&T |

| Company | Comments |
| --- | --- |
| CATT | For cases with prediction of future CSI, ideal prediction should be the baseline. Since we try to improve CSI compression performance in this agenda item, CSI prediction should not be the bottleneck when both CSI compression and prediction exist. Therefore, ideal prediction can serve as a reasonable baseline. |
| TCL | What’s the meaning of “ideal prediction”? Does it mean the predictive CSI refers to the ground truth obtained from the measurement of CSI-RS received in the predictive time slot? |

### Other EVM considerations

#### Question 2.2.7

Are there any other considerations regarding the evaluation methodology for temporal domain aspects of CSI compression that need to be discussed?

| Company | Comments |
| --- | --- |
| Huawei/HiSi | Maybe not urgent, but benchmarks (at least non-AI/ML benchmarks) should be calibrated over companies. In the R18 evaluations, some super high gain is due to a low benchmark of R16 CB or non-AI based CSI prediction, which provides overoptimistic gain of AI. |
| vivo | Spatial consistency following 38.901 methodology can also be considered by companies since CSI prediction in Rel-18 study also included such cases. |
| NTT DOCOMO | We propose to further study the generalizations during Rel. 19. RAN1 have studied the generalization over only single aspect during the Rel. 18. However, the NW and UE may face multiple generalization factors simultaneously in real-life deployments. RAN1 has no conclusion on the generalization over these practice aspects.The study on such complex cases is necessary. Moreover, the conclusion on this study is also important for drawing conclusions about introducing cell-/site-specific models. For the temporal domain aspects, the generalization over time-domain aspects should also be investigated during Rel. 19. |
| CATT | The modelling of realistic channel estimation procedure will have an impact on performance evaluation. To better analyse the evaluation results, companies are encouraged to report their modelling of channel estimation error for the realistic channel estimation procedure. |

## 2nd round discussion

### Categorization (Closed)

FL note:

@Huawei, @Intel, @ETRI, @TCL: The first row has now been clarified by splitting into two rows.

@Samsung, @ZTE: Added a clarification in Note 3 regarding “present slot” and “future slot(s)”.

@Huawei, CATT: The goal is to define the categories at a high level for now. Further sub-categorization can be discussed going forward if needed. For each case, which implementation choice companies should use need not be restricted at this stage and can be left to companies to select and report. A sentence has been added in Note 2 to clarify this.

@AT&T: CSI compression plus prediction is mentioned in the work item description as one of the examples.

#### Proposal 2.2.1 v2 (Closed)

For the evaluation of temporal domain aspects of AI/ML-based CSI compression using two-sided model in Release 19, adopt the following categorization for study:

| Case | Target CSI slot(s) | Whether CSI generation part uses past CSI information | Whether CSI reconstruction part uses past CSI information |
| --- | --- | --- | --- |
| 0 | Present slot | No | No |
| 1 | Present slot | Yes | No |
| 2 | Present slot | Yes | Yes |
| 3 | Future slot(s) | Yes | No |
| 4 | Future slot(s) | Yes | Yes |

Note 1: For the CSI generation part, the past CSI information may include past model inputs and/or any information derived from them. For the CSI reconstruction part, the past CSI information may include past CSI feedback instances and/or any information derived from them.

Note 2: For case 3 and case 4, the CSI generation model at the UE may perform prediction as a separate step or jointly with compression. Similarly, the CSI reconstruction model at the gNB may perform prediction as a separate step or jointly with reconstruction. Companies to report which option is selected, and whether the prediction is AI/ML-based or not.

Note 3: “Target CSI slot(s)” refers to the slot(s) to which the CSI feedback in the report corresponds. “Present slot” refers to the slot of the most recent CSI-RS measurement used to generate the CSI report. “Future slot(s)” includes at least one slot after the present slot and may include the present slot as well.

Note 4: Down-selection is not precluded.

| Support / Can accept | LG, ETRI, Futurewei |
| --- | --- |
| Object / Have a concern |  |

| Company | Comments |
| --- | --- |
| Samsung | We would like to include the following option. In our contribution, we have considered the case where the CSI generation part uses the latest measured CSI and the reconstruction part uses past CSI reports. x; Present slot; No; Yes. |
| OPPO | We are general fine with this categorization. From our understanding, case0, case 1 and case 2 indicate the sub use case “SFT-domain CSI compression”, case 3 and case 4 indicate the sub use case “CSI compression plus prediction”. Here we prefer to discuss CSI compression and CSI prediction separately, so that we can know whether the performance gain is from CSI compression or CSI prediction using AI/ML. Due to the limited time of Rel-19 SI, we suggest to treat case 0, 1 and 2 in higher priority, case 3 and case 4 in second priority. |
| TCL | From our understanding, case 0 refers to the evaluation scheme fully discussed in Rel-18 without utilizing temporal domain information for CSI compression, and case 2 indicates the “TSF-domain CSI compression” which may achieve greater performance gain for CSI compression efficiency and system throughput due to temporal correlation for consecutive CSI instances in time domain. However, we don’t find the necessity and feasibility for evaluating case 1, so we suggest to deprioritize case 1 and treat case 2 in higher priority to make consensus for performance evaluation on TSF CSI compression in Rel-19 SI. |

### Evaluation scenario (Closed)

FL note:

@Huawei: Added a sentence based on your comment, except the part about model input as it would not belong in this proposal.

@Huawei, @Xiaomi: Cases 3 and 4 will also involve compressing the predicted CSI using AI/ML-based compression versus a benchmark compression scheme. It is expected that 100% outdoor cases are not compression limited, and at high speeds they may be prediction limited. It would be important to also study cases that are compression limited to get a full view of the performance.

#### Proposal 2.2.2 v2 (Closed)

For the evaluation of temporal domain aspects of AI/ML-based CSI compression using two-sided model in Release 19, adopt the following as baseline options for UE distribution:

- Option 1: 80% indoor, 20% outdoor

- Option 2: 100% outdoor

Note: Indoor speed is 3 km/h, outdoor speed is chosen from the following options: 10 km/h, 20 km/h, 30 km/h, 60 km/h, 120 km/h. Assumption on O2I car penetration loss and spatial consistency follow the R18 AI based CSI prediction.

| Support / Can accept | LG, Samsung, OPPO, ETRI, Futurewei, NTT DOCOMO |
| --- | --- |
| Object / Have a concern |  |

| Company | Comments |
| --- | --- |
| OPPO | General ok. For the outdoor UE speed, we prefer 30km/h as the baseline, other speeds can be optional. |

### Benchmark scheme for performance comparison (Closed)

FL note:

@Fujitsu, @Xiaomi: For cases without prediction, using R16 codebook would be more aligned with the R18 study on CSI compression. The TR already captures the benchmark scheme for these cases. Regarding a common benchmark scheme for all the cases. FL’s view is that there is not much difference between the two views, since for cases without prediction, R18 codebook with the N4 parameter set to 1 and negative delta value can be viewed as a special case of R18 MIMO CSI that closely resembles R16 eType II codebook.

@Huawei: Added the last item based on your comments.

#### Proposal 2.2.3 v2 (Closed)

For the evaluation of temporal domain aspects of AI/ML-based CSI compression using two-sided model in Release 19, adopt the following benchmark scheme for performance comparison:

- For cases without prediction of future CSI, use the same benchmark scheme assumed in R18 AI/ML-based CSI compression study.

- For cases with prediction of future CSI, use the same benchmark scheme assumed in R18 AI/ML-based CSI prediction study, with R18 MIMO eType II codebook for compressing the feedback.

- Also adopt R18 AI/ML-based CSI enhancement as an additional benchmark scheme.

| Support / Can accept | LG, ETRI, NTT DOCOMO |
| --- | --- |
| Object / Have a concern |  |

| Company | Comments |
| --- | --- |
| Samsung | Support the proposal. We believe, the second bullet applies to the case where there is prediction at the UE side. For the evaluation of temporal domain aspects of AI/ML-based CSI compression using two-sided model in Release 19, adopt the following benchmark scheme for performance comparison: - For cases without UE-side prediction of future CSI, use the same benchmark scheme assumed in R18 AI/ML-based CSI compression study. - For cases with UE-side prediction of future CSI, use the same benchmark scheme assumed in R18 AI/ML-based CSI prediction study, with R18 MIMO eType II codebook for compressing the feedback. - Also adopt R18 AI/ML-based CSI enhancement as an additional benchmark scheme. |
| OPPO | As we have commented in proposal 2.3.1, we prefer to treat cases with prediction of future CSI in second priority. |

### Evaluation metrics

FL note:

@Huawei: The metric should allow comparison of schemes that have different reporting periodicity. It is not clear why the benchmark and AI/ML scheme must use the same reporting periodicity. Since we are investigating temporal domain techniques, reducing the frequency of reporting by relying on prediction should also be considered.

@ZTE: “per unit time” simply means the average bit-rate of the CSI feedback overhead. For example, CSI payload of size P every T milliseconds will result in an overhead rate of P / T bits per millisecond. The proposal wording has been clarified.

@Intel: How to calculate the CSI feedback overhead at a given time instance was already discussed and agreed in Rel-18. The current proposal only introduces an averaging across time.

#### Proposal 2.2.4 v2 (Closed)

For the evaluation of temporal domain aspects of AI/ML-based CSI compression using two-sided model in Release 19, adopt the CSI feedback overhead rate as a baseline metric, where the CSI feedback overhead rate is the average bit-rate of CSI feedback overhead per unit across time.

| Support / Can accept |  |
| --- | --- |
| Object / Have a concern | NTT DOCOMO |

| Company | Comments |
| --- | --- |
| Samsung | Support. |
| OPPO | General ok. But not clear whether ‘across time’ means, what is the time unit here? |
| NTT DOCOMO | As discussed in Round 1, given the average bit-rate of CSI feedback, the reduction on UCI transmission times is also beneficial. Therefore, we propose the following, For the evaluation of temporal domain aspects of AI/ML-based CSI compression using two-sided model in Release 19, adopt both the CSI feedback overhead rate and the rate of UCI transmission occasions for CSI report as a baseline metric, where the CSI feedback overhead rate is the average bit-rate of CSI feedback overhead per unit across time, and the rate of UCI transmission occasions for CSI report is the average number of UCIs for CSI report per time unit. |
| Intel | After further discussion with FL, our understanding now is that CSI overhead in one time instance corresponds to definition agreed in Rel-18 SI. The word ‘average’ is related only to averaging over time. Also, baseline metrics representing overhead should be only one so that it can be plotted on 2D figure in X-axis (performance is Y-axis). Thus, we propose the following changes (in purple). #### Proposal For the evaluation of temporal domain aspects of AI/ML-based CSI compression using two-sided model in Release 19, adopt the CSI feedback overhead rate as a baseline metric, where the CSI feedback overhead rate is the average bit-rate of CSI feedback overhead per unit across time. Also adopt Companies to disclose the rate of UCI transmission occasions for CSI report as a baseline metric, where rate of UCI transmission occasions for CSI report is the average number of UCIs for CSI report per time unit. Note: CSI feedback overhead is calculated as the weighted average of max allowed number of CSI bits per rank and the distribution of ranks reported by the UE (as agreed in Rel-18 AI/ML SI at RAN1#112). |

#### Proposal 2.2.4 v3

For the evaluation of temporal domain aspects of AI/ML-based CSI compression using two-sided model in Release 19, adopt the CSI feedback overhead rate as a baseline metric, where the CSI feedback overhead rate is the average bit-rate of CSI feedback overhead per unit across time.

Also adopt the rate of UCI transmission occasions for CSI report as a baseline metric, where the rate of UCI transmission occasions for CSI report is the average number of UCIs for CSI report per time unit.

| Support / Can accept | NTT DOCOMO |
| --- | --- |
| Object / Have a concern |  |

| Company | Comments |
| --- | --- |
| LG | Generally fine with adopting the CSI feedback overhead rate. As mentioned at online session yesterday to compare the performance of schemes that have different reporting periodicity. But, to my understanding, the overhead rate is not the average bit-rate but the (average) amount of bits for CSI overhead across time. |
| CMCC | OK to use the metric of CSI feedback overhead rate, but for fair comparison, the UPT or throughput of AI based approach and benchmark should achieve similar performance when comparing this metric. In addition, not very clear with the meaning of rate of UCI transmission occasion, what is the relationship with the period of CSI report? It is some kind of 1/CSI report periodicity? |
| NTT DOCOMO | Thanks moderator for your considerations. We support this version. @CMCC It is somehow a kind of CSI report periodicity. But considering different CSI report configurations especially when CSI prediction is involved, it is an averaged CSI periodicity or averaged CSI reporting times given time unit. To simplify the proposals, we accept any description as long as it is equivalent in principle. |

### Other evaluation assumptions (Closed)

FL note:

Changes have been made to address companies’ comments.

#### Proposal 2.2.5 v2 (Closed)

For the evaluation of temporal domain aspects of AI/ML-based CSI compression using two-sided model in Release 19, adopt the following evaluation assumptions:

- CSI-RS configuration

  - Periodic: 5 ms periodicity (baseline), 20 ms periodicity (optional)

  - Aperiodic (for cases with prediction): CSI-RS burst with K resources and time interval m milliseconds (based on R18 MIMO eType-II)

    - FFS: Value of K and m.

- CSI reporting periodicity: {5, 10, 20} ms

- For cases with the use of past CSI information, companies to report observation window, including number/time distance of historic CSI/channel measurements.

- For cases with prediction, companies to report prediction window, including number/time distance of predicted CSI/channel.

| Support / Can accept | NTT DOCOMO |
| --- | --- |
| Object / Have a concern |  |

| Company | Comments |
| --- | --- |
| LG | Generally fine with the proposal. As mentioned in the previous round, various UE speeds also can be considered since it affects the temporal coherence on historical CSI. |
| Samsung | It is important to include 20ms CSI-RS periodicity as additional baseline. In Rel-18, we have evaluated predicted CSI for 5, 10, 15ms after the latest measured CSI-RS. If the CSI-RS resource periodicity is restricted to 5ms, then these prediction instances have no practical benefits. |
| SK telecom | We strongly believe that additional evaluation using 20ms CSI-RS periodicity is needed in order to know whether AI/ML based CSI compression feature is useful for improving real 5G network performance or not. Also, we are not proposing to exclude 5ms CSI-RS periodicity in R19 AI/ML based CSI compression evaluation. We want to encourage additional evaluation using 20ms CSI-RS periodicity in R19 study phase. |
| Xiaomi | The starting point of observation window or prediction window has significantly impact on system performance. Hence, the starting point of observation window and prediction window should be reported for aligning evaluation results. Hence, we suggest the last two bullets are revised as following: - For cases with the use of past CSI information, companies to report observation window, including starting point of observation window /number/time distance of historic CSI/channel measurements. - For cases with the use of past CSI information, companies to report observation window, including starting point of observation window /number/time distance of historic CSI/channel measurements. - For cases with prediction, companies to report prediction window, including starting point of prediction window /number/time distance of predicted CSI/channel. |
| Fujitsu | We agree with Xiaomi that the starting point of observation window or prediction window should be reported for alignments among companies. |

### Ideal prediction

FL note:

@TCL: To your question, ideal prediction refers to the case where the CSI for the future target slots are available to the UE-side CSI generation model without any prediction error. Note that channel estimation error should still be modelled. A note is added to clarify this aspect.

#### Proposal 2.2.6 v2

For the evaluation of temporal domain aspects of AI/ML-based CSI compression using two-sided model in Release 19, for cases with prediction of future CSI, companies may optionally evaluate a scheme with ideal prediction as an additional evaluation case for reference.

Note: The ideal prediction scheme should still model realistic channel estimation.

| Support / Can accept | Samsung, ETRI, Futurewei, NTT DOCOMO, CMCC, NEC |
| --- | --- |
| Object / Have a concern | AT&T |

| Company | Comments |
| --- | --- |
| TCL | General fine with this proposal. |
| Fujitsu | We are fine with this proposal. |
| AT&T | Is this only for the case where the CSI prediction and CSI compression models are separate? How to have a similar sort of baseline for joint CSI prediction and compression model to check how much of gain is due to CSI prediction and how much from CSI compression. |

## 3rd round discussion

### Impact of UCI loss

FL note: For the cases where the network uses past CSI information, when the UCI is lost, past CSI information may not be available, resulting in a performance impact. The proposal below is to study this aspect.

#### Proposal 2.4.1

For the evaluation of temporal domain aspects of AI/ML-based CSI compression using two-sided model in Release 19, for Case 2 and Case 4, study the performance impact resulting from UCI loss.

| Support / Can accept | NTT DOCOMO(w/ comments) |
| --- | --- |
| Object / Have a concern |  |

| Company | Comments |
| --- | --- |
| Samsung | Thank you FL. It is important to study this aspect for the cases wherein the network use past CSI. We think the formulation should be generalized to the impact of ‘error propagation’ which could be a result of multiple factors, e.g., UCI loss, Part2 CSI drop and CSI generation error due to CE error from interference burst, etc. For the evaluation of temporal domain aspects of AI/ML-based CSI compression using two-sided model in Release 19, for Case 2 and Case 4, study the performance impact of error propagation, e.g., resulting from UCI loss, Part2 CSI dropping, CSI generation error. |
| LG | To our understanding, the UCI loss highly affects the performance on TSF-CSI especially for Case 2 and Case 4. But, from a perspective of evaluation, how to evaluate the impact on UCI loss may have the following issues: - Modelling the method for UCI missing (e.g. based on probability or a certain occasion(s), etc.) - Modelling the method related to the loss information via UCI missing (e.g. whether the missed CSI is highly correlated with the (accumulated) past CSI or not) |
| ZTE | We agree with Samsung, that some factors can result in the performance degradation in Case 2 and Case 4. So, we have to further study the performance impact resulting from at least UCI loss. |
| Panasonic | Case 5 also uses the past CSI information at the NW (but not use at the UE). We would like to clarity whether it is necessary to study the impact of UCI missing also for Case 5. |
| Xiaomi | UCI loss includes the following two cases: - Case 1: the total UCI is lost - Case 2: the UCI of partial layers is lost for rank > 1. In order to clarify this, we suggest the proposal is reworded as follows: For the evaluation of temporal domain aspects of AI/ML-based CSI compression using two-sided model in Release 19, for Case 2 and Case 4, study the performance impact resulting from UCI loss total UCI loss or UCI loss of partial layers for rank>1. |
| TCL | Fine with this proposal. UCI loss highly affects the feasibility and performance of TSF-CSI compression using past CSI instance (like Case 2 and 4). So it is essential to study the performance impact of UCI loss and further clarify whether and how to address it. |
| NTT DOCOMO | We general support the evaluation study on this issue. But more details on the EVM should be discussed in future, e.g., simply evaluate the performance loss of a model trained by assuming the ideal UCI receptions, or consider some generalization/scalability-like techniques and evaluation methods. |
| CMCC | Agree with Samsung, the intention of this proposal is to evaluate the imperfections of past CSI, such as UCI missing, CSI dropping, CSI without updating, although in some cases, it can be solved by implementation. We could use more generic wording. |
| CATT | Agree with Samsung, to study the performance impact of imperfect part CSI. Also the study is not only relevant to case 2 and 4, all cases that leverage past CSI information should analyse the impact of imperfect past CSI. |
| Fujitsu | We propose to study the performance impact from UCI loss for all cases. This could make us more clear how does each scheme perform for UCI loss. For the cases that the past CSI information is used at both sides, UCI missing may lead to some “misalignment” for past CSI information at UE side and gNB side which may impact the performance. It seems the cases that past CSI information is used at one side (e.g., Case 1/3/5) is more robust to UCI loss. |

# Localized models

## Summary of company proposals

From the submitted contributions, proposals related to the study of localized models, i.e., models specific to a cell, site, location, or region, are summarized below.

Huawei, HiSilicon

Proposal 5: For the EVM of cell/site specific models, start the discussion with the following candidates for channel modeling to reflect the correlation of the scenario between the training phase and the inference phase for per cell/site.

- Candidate 1: Ray-tracing model.

- Candidate 2: Spatial consistency, where UEs between training and inference are subject to the same drop.

- Candidate 3: Different deployment scenarios/configurations for different sites.

Proposal 6: For the EVM of cell/site specific models, it needs to discuss how to reflect the variation of the channel between the training phase and the inference phase to avoid overfitting model and over-optimistic performance.

Proposal 7: The particular potential spec impact to support cell/site specific models can refer to model transfer/delivery, which is better to be discussed in 9.1.3.3.

vivo

Proposal 1: Consider to use spatial consistency in 38.901 channel model and simulation assumptions for CSI compression in TR 38.843 to evaluate the performance of cell/site specific models.

ZTE

Proposal 5: For cell/site specific model, prioritize the alignment on the understandings and EVMs for the cell/site specific model among companies first during Rel-19 study phase.

- Further study and evaluate at least the method of a smaller dataset in the specific cell/site collected for AI/ML model training as a starting point.

Proposal 6: For EVM calibration on cell/site specific model, prioritize to construct the dataset for cell/site specific model with the same number of data samples per cell/site compared with generalized model.

CMCC

Proposal 2: Overfitting issue should be avoided when considering training data generation, if cell/site specific model is decided to be evaluated.

Intel

Proposal 4: To model the radio environment in a single cell/site a single UE drop in combination with the channel consistency model described in TR 38.901 should be considered.

Proposal 5: RAN1 to study the delivery/update of a cell/site-specific model to a UE.

CATT

Proposal 3: If cell/site-specific model is considered as a candidate solution, evaluation methodology of cell/site-specific channel model should be aligned firstly. To avoid duplicated work, only discuss the feasibility and detailed evaluation methodology of cell/site-specific channel model in either CSI compression or CSI prediction agenda item.

OPPO

Proposal 1: suggest to study AI/ML based CSI compression with cell-specific model in Rel-19, and discuss the EVM including the following aspects:

- Impact of spatial consistency

- Different scenarios, e.g., indoor/outdoor UE distributions, LoS/NLoS ratios.

Proposal 2: regarding the data collection for CSI compression, cell/site/scenario related “condition information” and “addition condition information” should be considered during the data collection stage

- Condition information including CSI-related information such as the CSI type, e.g. raw channel or precoding matrix, and the CSI configurations, e.g. number of antenna ports, number of sub-bands, ranks.

- Additional condition information including cell/site/scenario related information such as cell/site/scenario ID, indoor/outdoor indication, LoS/NLoS flag and UE ID.

Proposal 3: regarding the cell/site/scenario specific model training, two ways can be considered, including

- Direct training based on large cell/site/scenario-specific datasets

- Finetuning based on cell-common model with small cell/site/scenario-specific datasets

The trade-off between potential performance gain and complexity/overhead should be further studied.

NVIDIA

Proposal 1: Site-specific AI/ML models for CSI compression should be considered to improve performance gain.

Proposal 2: Define a common reference scenario with site specificity as a basis for further study of AI/ML based CSI compression.

Proposal 3: Select one the following options to define a common reference scenario with site specificity as a basis for further study of AI/ML based CSI compression:

- Option 1: Real-scenario map that is a virtual representation of a real area on earth.

- Option 2: Synthetic-scenario map that is artificially constructed to mimic a certain environment such as urban macro, rural macro, indoor office, or indoor factory.

Proposal 4: Consider the Madrid grid developed by the METIS project for urban scenarios for further study of site-specific AI/ML based CSI compression.

Proposal 5: With a common reference scenario with site specificity, ray tracing is used to generate channel data for the development and evaluation of site-specific AI/ML models for CSI compression.

Samsung

Proposal 2: Study the performance of site/cell/location-specific models.

Proposal 3: Study the following alternatives to evaluate site/cell/location-specific models

- Alt1: A few/single UE drop(s) on a few/single sector(s) with spatial consistency turned on.

- Alt2: Ray-tracing channel model

- Alt3: Field data (channel samples collected from real deployment)

Nokia

Proposal 5: Selecting a few representative specific scenarios/configurations to investigate the advantages of employing cell/site-specific models. Preferably, focus should be on evaluating SLS results, while not excluding intermediate KPIs like SGCS.

Proposal 6: To further investigate and evaluate cell/site-specific models, the following alternatives could be considered:

- Option 1:	Ray-Tracing Analysis

- Option 2:	Het-Net Environment

- Option 3:	Mixed Scenario Training vs. Single Scenario Training

Proposal 7: In addition to cell/site specific models, the use of specific model/feedback overhead within the same cell/site should also be studied.

Apple

Proposal 2: For cell specific model, generalization performance over different time samples should be evaluated.

NTT DOCOMO

Proposal 4: Study the cell-/site-specific model as a solution to the generalization and scalability of AI/ML for CSI compression and conclude with the outcome of the generalization and scalability study of other approaches.

Sharp

Proposal 3: Regarding cell/site specific models, training collaboration Type 3 with NW side training first is preferred.

ETRI

Proposal 2: Consider the Cell/site/scenario specific model for both the Spatial-frequency (SF) domain and the Spatial-frequency-time (SFT) domain CSI compression sub-use cases for further study, and study the performance of the Cell/site/scenario specific model compared to the global models.

ITL

Proposal 2: The site-specific AI/ML models for CSI compression schemes can be considered/evaluated by using a common reference scenario with site specificity.

Qualcomm

Proposal 4:	Study the performance improvement associated with localized models (models developed specifically for use within a local region) compared to global models.

Proposal 5:	For the evaluation of localized models, the local region could be defined as UEs dropped within a specific boundary (e.g., 25 m. by 25 m. square), with the same serving cell, and with the same indoor/outdoor state.

Proposal 6:	Study techniques and potential specification impact to enable the use of localized models to achieve the associated improvement in the performance-complexity tradeoff.

## 1st round discussion

### Evaluation aspects

FL note: In the work item description, the study of cell/site-specific models has been mentioned under the study objective of improving the trade-off between performance and complexity/overhead of CSI compression. The channel samples from UEs in a local region may be correlated, and developing a model specific to such a scenario may provide a benefit in terms of performance and model complexity. The following proposal aims to identify the key aspects for the evaluations related to this topic.

#### Proposal 3.2.1

For the evaluation of AI/ML-based CSI compression using localized models in Release 19, study the following aspects of the performance/complexity trade-off when comparing the localized model with a benchmark model that is not localized:

- Performance of the localized model that has similar complexity as the benchmark model.

- Model complexity of the localized model that achieves similar performance as the benchmark model.

| Support / Can accept | Lenovo, New H3C, Samsung, Futurewei, vivo, OPPO, NTT DOCOMO, NEC, ETRI, LG, CATT, AT&T |
| --- | --- |
| Object / Have a concern |  |

| Company | Comments |
| --- | --- |
| Lenovo |  |
| Samsung | In our understanding, the above is about model inference complexity and doesn’t account to other complexities, e.g., LCM complexity for frequent model switching, storing, transfer, etc. |
| Futurewei | We are ok to evaluate the performance of localized model, but we think LCM complexity needs to be considered as well. |
| Huawei/HiSi | We can understand the intention of this proposal, but it may be difficult from simulation perspective to artificially align the complexity of two models, or align the performance of the two models – that may bring in tremendous efforts of developing various models and select one for comparison. If the localized model has lower complexity than generalized model, it can also justify the performance, and vice versa for higher performance. - Performance of the localized model that has similar or lower complexity as the benchmark model. - Model complexity of the localized model that achieves similar or higher performance as the benchmark model. |
| Fujitsu | Agree with other companies that LCM related complexity should be considered. |
| OPPO | We are general fine with this proposal. We also share the same view to Huawei/Hisi that ‘or lower’ can be added to the proposal. Moreover, we prefer to evaluate the first aspect in higher priority, since the performance gain over benchmark model seems more important. And we also suggest to use intermediate KPI instead of SLS throughput for initial comparison in first stage. |
| ETRI | We agree and also fine with the HW's modification. |
| Ericsson | Comparing to the generalized model case, localized models can pose even more challenges when it comes to multi-vendor model training collaboration and RAN4 conformance testing of two-sided models. The interoperability issue for localized model shall be addressed first, before checking the gain vs. complexity. In addition, we agree with Samsung and Futurewei that the additional LCM complexity shall be considered. |
| ZTE | We are open to discuss the two aspects. For evaluation convenience, we can adopt the first bullet as a starting point since model may be not re-designed and just reducing the dataset is enough. It is more convenient for companies to do simulations based on Rel-18 and evaluate the performance with benchmark model. |
| LG | Generally fine with the proposal. Regarding LCM aspects, while we also think that the LCM related complexity should be considered on the performance as other companies mentioned, the potential LCM solution for model transfer/delivery can be discussed in 9.1.3.3., hence, the related complexity in detail can be adopted depending on the progress in 9.1.3.3. |
| Xiaomi | We prefer to firstly discuss the evaluation assumption for localized models. Different companies have different views for localized models. If the evaluation assumption is different, it is hard to evaluate the performance of model. |

### Dataset to model local region

FL note: The issue of aligning the evaluation methodology for studying localized models has been raised by several contributions. The goal of the study is to explore the potential improvement in performance/complexity from using a localized model, i.e., a model specific to a cell, site, location, or region, by utilizing the consistency of the channel characteristics within such a local region. Hence, it would be important to carefully model this aspect when deriving the training and inference datasets.

#### Proposal 3.2.2

For the evaluation of AI/ML-based CSI compression using localized models in Release 19, consider the following options to model the spatial consistency/correlation in the dataset for a local region:

- Option 1: The dataset is derived from UEs dropped within the local region, with spatial consistency modelling as per TR 38.901.

    - E.g., Dropped in a specific cell or within a specific boundary.

- Option 2: By using a scenario/configuration specific to the local region.

    - E.g., Indoor-outdoor ratio, LOS-NLOS ratio, TXRU mapping, etc.

- Option 3: The dataset is derived using ray-tracing model.

    - E.g., Based on a scenario map.

- Option 4: The dataset is derived from field data collection within a local region.

| Support / Can accept | Lenovo (comment), New H3C, Samsung, Futurewei, vivo, OPPO, NEC, ETRI, LG, CATT, AT&T, CMCC, NVIDIA |
| --- | --- |
| Object / Have a concern |  |

| Company | Comments |
| --- | --- |
| Lenovo | Investigate the possibility of sharing a field dataset as a common dataset for evaluation |
| Futurewei | We are open to using field data as one of the options if some companies can share field datasets. |
| Huawei/HiSi | 1) For Option 1/3, we need to discuss how to avoid overfitting due to static channel between training and inference. E.g., if the spatial consistency is modelled so that training and inference are subject to one drop, then the number of path, path delay, AoA/AoD are fixed between training UE and inference UE, which is biased from the real network – that will cause overfitting performance. 2) For Option 1, need to FFS calibration of modelling of spatial consistency method? 3) For Option 2, Indoor outdoor ration and LOS/NLOS ration is likely to be different for different cells as it depends on the surrounding environment, but TxRU mapping depends on the type of RF module, which is likely to be fixed within a certain area. So this factor is removed. - Option 1: The dataset is derived from UEs dropped within the local region, with spatial consistency modelling as per TR 38.901. - E.g., Dropped in a specific cell or within a specific boundary. - FFS on calibrating the method of spatial consistency modelling - Option 2: By using a scenario/configuration specific to the local region. - E.g., Indoor-outdoor ratio, LOS-NLOS ratio, TXRU mapping, etc. - Option 3: The dataset is derived using ray-tracing model. - E.g., Based on a scenario map. - Option 4: The dataset is derived from field data collection within a local region. - Note: For Option 1/3, need to address how to avoid overfitting due to static channel between the training phase and the inference phase |
| Fujitsu | We prefer with Option 1 and Option 2. Regarding Option 3, ray-tracing method may need further discussion to achieve alignment among companies. Regarding Option 4, how to obtain the field data and who should provide the field data require a lot of discussion. |
| OPPO | We prefer Option 1 and Option 2. For Option 1, companies should firstly calibrate the method of spatial consistency modelling. For Option 2, different indoor-outdoor ratios and LOS-NLOS ratios can be evaluated. Some typical parameters (e.g. 1:0, 0.4:0.6, 0.2:0.8, 0:1 LOS-NLOS ratios) can be evaluated. |
| Apple | Need to discuss testing dataset generation. Need to test localized model performance at different time, where testing dataset and training dataset are separated long time. |
| ZTE | We prefer Option 1 as a starting point to evaluate performance. Based on the 38.901 model with spatial consistency, it is convenient for companies to obtain the training dataset and evaluate the performance. For Option 2, it seems as a generalization case evaluated in Rel-18 and some generalization results are already captured in TR. So, maybe no duplicated evaluation is needed in Rel-19. In addition, considering the data generation complexity, Option 3 and Option 4 should be deprioritized since the data generation may not be calibrated in the potential limited study phase in Rel-19. |
| CATT | Option 1 is preferred. Option 2 cannot reflect the spatial consistency/correlation in the local region, which might under estimate the performance of cell/site specific model. Option 3 proposes to adopt a new framework of channel modelling which might lead to further problem on model validation and calibration. Given the limited time budget before the checkpoint, it is not preferred. |
| Intel | For option 2,3,4 it may be challenging to align results between companies due to different UE sets/maps/measurements. |
| Xiaomi | OK, we prefer to study Option 1 with higher priority. |

### Other EVM considerations

#### Question 3.2.3

Are there any other considerations regarding the evaluation methodology for AI/ML-based CSI compression using localized models that need to be discussed?

| Company | Comments |
| --- | --- |
| NTT DOCOMO | The price of the performance gain from cell-/site-specific models are the generalizations. Therefore, the evaluation results of cell-/site-specific should be compared with the results of general models with generalization aspects, based on which we can draw conclusions on the cell-/site-specific model. |
| LG | To our understanding, the cell/site specific models achieve more performance gain in the specific area while the generalization performance can be degraded. In this case, over-fitting issue would be more crucial on the certain cell/site only. Hence, for comparison, the similar scenarios adopting appropriate Simulation assumptions for CSI compression seem to be considered. |

## 2nd round discussion

### Evaluation aspects (Closed)

FL note: Small update based on company comments.

#### Proposal 3.2.1 v2 (Closed)

For the evaluation of AI/ML-based CSI compression using localized models in Release 19, study the following aspects of the performance/complexity trade-off when comparing the localized model with a benchmark model that is not localized:

- Performance of the localized model that has similar or lower complexity as the benchmark model.

- Model complexity of the localized model that achieves similar or better performance as the benchmark model.

| Support / Can accept | Samsung, OPPO, ETRI, Futurewei, NTT DOCOMO, CMCC, NEC, Panasonic |
| --- | --- |
| Object / Have a concern |  |

| Company | Comments |
| --- | --- |
| OPPO | General ok, it is better to add a note that sub-bullet 1 should be firstly evaluated. |
| Lenovo | We also prefer to keep Option 4 and see if any company can share their dataset with others so we can have more results based on the field data. This could be optional simulation of course. |
| Fujitsu | Generally fine with this proposal. |

### Dataset to model local region

FL note: Based on the responses in round 1, several companies have expressed a preference for Option 1 or 2.

#### Proposal 3.2.2 v2

For the evaluation of AI/ML-based CSI compression using localized models in Release 19, consider the following options as a starting point to model the spatial consistency/correlation in the dataset for a local region:

- Option 1: The dataset is derived from UEs dropped within the local region, with spatial consistency modelling as per TR 38.901.

    - E.g., Dropped in a specific cell or within a specific boundary.

- Option 2: By using a scenario/configuration specific to the local region.

    - E.g., Indoor-outdoor ratio, LOS-NLOS ratio, TXRU mapping, etc.

- Option 3: The dataset is derived using ray-tracing model.

    - E.g., Based on a scenario map.

- Option 4: The dataset is derived from field data collection within a local region.

| Support / Can accept | OPPO, ETRI, Futurewei, NTT DOCOMO(Option 1), CMCC, NEC, Panasonic, Fujitsu |
| --- | --- |
| Object / Have a concern |  |

| Company | Comments |
| --- | --- |
| Samsung | Ok with the update. |
| Lenovo | We also prefer to keep Option 4 and see if any company can share their dataset with others so we can have more results based on the field data. This could be optional simulation of course. |
| Apple | We would like to further emphasize the dataset for training and testing should be separated in time long enough. We observe overfitting issue for the localized models. We suggest to add a note: Note: company to report the methods to generate training dataset and testing dataset when SGCS is used as evaluation metric. |

## 3rd round discussion

### Dataset to model local region

FL note: A note has been added to Option 1 regarding the concern that temporal variations in the channel may cause lack of correlation and that needs to be modelled correctly to avoid over-fitting.

#### Proposal 3.2.2 v3

For the evaluation of AI/ML-based CSI compression using localized models in Release 19, consider the following options as a starting point to model the spatial consistency/correlation in the dataset for a local region:

- Option 1: The dataset is derived from UEs dropped within the local region, with spatial consistency modelling as per TR 38.901.

    - E.g., Dropped in a specific cell or within a specific boundary.

    - Note: While modelling the spatial correlation, companies should ensure that the dataset distribution also correctly captures the decorrelation due to temporal variations in the channel.

- Option 2: By using a scenario/configuration specific to the local region.

    - E.g., Indoor-outdoor ratio, LOS-NLOS ratio, TXRU mapping, etc.

- Option 3: The dataset is derived using ray-tracing model.

    - E.g., Based on a scenario map.

- Option 4: The dataset is derived from field data collection within a local region.

| Support / Can accept | Samsung, LG, Panasonic, OPPO |
| --- | --- |
| Object / Have a concern |  |

| Company | Comments |
| --- | --- |
| Samsung | Thanks FL for your efforts. Rather than leaving it up to companies, it is better for RAN1 to align the simulation modelling so that model overfitting can be avoided. - Option 1: The dataset is derived from UEs dropped within the local region, with spatial consistency modelling as per TR 38.901. - E.g., Dropped in a specific cell or within a specific boundary. - Note: While modelling the spatial correlation, companies should ensure that the dataset distribution also correctly captures the decorrelation due to temporal variations in the channel. - FFS: Simulation modelling to avoid model overfitting |
| ZTE | For Option 2, to our understanding, it seems as a generalization case evaluated in Rel-18 and some generalization results are already captured in TR. So, maybe no duplicated evaluation is needed in Rel-19. If my understanding is not correct, FL can make a clarification, thanks. |
| Xiaomi | Fine |
| ETRI | We agree with preference of Option 1. |
| OPPO | Thanks for FL’s effort. We also prefer to have such an agreement so that RAN1 can evaluate localized models based on the same EVM. As for Option 1, the overfitting concerns raised by some companies, we suggest companies to clearly report how the training and testing datasets are generated and separated. From our understanding, two kinds of methods may be considered: Method 1: N UEs with T samples in temporal domain are generated. The first K<N UEs are used for training, the remained N-K UEs are used for testing. - Totally K*T samples in training dataset and (N-K)*T UEs for testing. Method 1 is to evaluate the performance of localized model over different UEs Method 2: N UEs with T samples in temporal domain are generated. The first T1<T samples are used for training, the remained T-T1 samples are used for testing. - Totally N*T1 samples in training dataset and N*(T-T1) UEs for testing. Method 2 is to evaluate the performance of localized model over different time. As for the note, we suggest the following wording : Note: how to generate and separate training and testing datasets should be reported. To ZTE’s comments on Option 2, we think that it is different to generalization case in Rel-18. For generalization, we aim to train a common model used for all scenarios/configurations. As for Option 2, we want to evaluate in which scenario/configuration, the localized model can achieve higher performance gain than common model, and find how indoor-outdoor ratio and LOS-NLOS ratio affect the performance. |
| NTT DOCOMO | We would like to change the Note in Option 1 to FFS. RAN1 should have at least a reference model on how to model the decorrelation due to the temporal variations in the channel. If it is up to the companies, it will cause the divergence of the results. |
| CMCC | Our preference is Option 1 and the method to avoid overfitting issue should be aligned so that companies’ results can be trustable. As for Option 2, it seems the spirit is to narrow the data generation entity to several cells/sites in a region, and some configurations/scenarios is same in this region. But for the listed examples in Option 2, it seems the Indoor-outdoor ratio, LOS-NLOS ratio, TXRU mapping is already same over multiple cells/sites in Rel-18 EVM. If my understanding is not correct, companies can make a clarification, thanks. |
| CATT | The note under option 1 is not clear. |

# Inter-vendor training collaboration

## Summary of company proposals

From the submitted contributions, proposals related to inter-vendor training collaboration are summarized below.

Huawei, HiSilicon

Proposal 10: Deprioritize training collaboration Type 2 over the 3GPP signaling in Rel-19.

Proposal 11: For the content of dataset delivery under training collaboration Type 3, consider the following typical behaviors (which have been widely considered in the Rel-18 evaluation) as the baseline for studying potential spec impact:

- For NW first training, NW side shares UE side with a dataset (generated after joint training of the NW side CSI generation part and the NW side CSI reconstruction part) that is used by the UE side to be able to train the UE side CSI generation part.

  - The dataset includes the input and output of the NW side CSI generation part.

  - FFS the quantization behavior, e.g., whether the shared output of the NW side CSI generation part is before or after quantization.

- For UE first training, UE side shares NW side with a dataset (generated after joint training of the UE side CSI generation part and the UE side CSI reconstruction part) that is used by the NW side to be able to train the NW side CSI reconstruction part.

  - The dataset includes the input and label of the UE side CSI reconstruction part.

  - FFS the quantization behavior, e.g., whether the shared input of the UE side CSI reconstruction part is before or after quantization.

Ericsson

Proposal 1: The continued Rel.19 study in RAN1 and RAN4 on the CSI compression use case shall prioritize to solve the fundamental issue first; how to achieve multi-vendor inter-operability and testing for two-sided model use cases.

Proposal 2: Study at least these solutions for the CSI compression use case, to resolve the inter-vendor collaboration issue and to maintain 3GPP based interoperability:

- Fully standardize the inference encoder (architecture and parameters)

- Standardize the input and output relation of the inference encoder

- Standardize the parameters/conditions that shall be considered for inference encoder training (similar to RAN4 option 4)

vivo

Observation 12: The following principles need to be considered for inter-vendor training collaborations:

- The collaboration should have no offline coordination between NW side entities and UE sided entities;

- The collaboration should involve as few intra-vendor entities as possible;

- The collaboration should ensure performance benefits.

Observation 13: Training collaboration type1 with known model structure does not require offline coordination either between NW side entities and UE sided entities or intra-vendor entities if model structure can be specified.

Observation 14: Training collaboration type1 with known model structure facilitates cell/site specific model parameter update.

Observation 15: Specification support of reference/testing model (structure) is also needed for the testability of two-sided models.

ZTE

Proposal 9: 3GPP RAN1 should further consider how to resolve the inter-vendor training collaboration related issues.

CMCC

Proposal 4: When discussing the pros and cons of various collaboration types, deprioritize Type 2 and focus on Type 1 and Type 3.

Intel

Proposal 10: Training collaboration types are further discussed based on progress achieved in the Rel-18 SI.

- Re-discuss entries in Table 5.1-1 and Table 5.1-2 of TR 38.843 where consensus has not been reached.

Proposal 11: For CSI compression using two-sided model use case, the following changes (in red) are endorsed for the table with the pros/cons of training collaboration type 1.

| Training type Characteristics | Type1: NW side |  | Type 1: UE side |  |
| --- | --- | --- | --- | --- |
|  | Unknown model structure at UE | Known model structure at UE | Unknown model structure at NW | Known model structure at NW |
| Extendibility: to train new UE-side model compatible with NW-side model in use; | Yes | Yes | Yes | Yes |
| Extendibility: To train new NW-side model compatible with UE-side model in use | Yes | Yes | Yes | Yes |

Proposal 12: For CSI compression using two-sided model use case, the following changes (in red) are endorsed for the table with the pros/cons of training collaboration types 2 and 3.

| Training types Characteristics | Type 2 |  | Type 3 |  |
| --- | --- | --- | --- | --- |
|  | Simultaneous | Sequential NW first (note 1) | NW first | UE first |
| Feasibility of allowing UE side and NW side to develop/update models separately | Infeasible | Feasible | Feasible | Feasible |
| Extendibility: to train new UE-side model compatible with NW-side model in use; | Not support | Support | Support | Support |
| Extendibility: To train new NW-side model compatible with UE-side model in use | Not support | Not Support | Support | Support |

CATT

Proposal 4: In CSI compression using two-sided model use case, consider to complete the Rel-18 pros and cons analysis on items with no consensus for training collaboration types.

Proposal 5: In CSI compression using two-sided model use case, down select training collaboration types in SI phase.

NEC

Proposal 3: Prioritize training collaboration Type-1: NW-side based model training (i.e., Joint training of the two-sided model at Network side) for CSI compression in Rel-19.

Xiaomi

Proposal 5: In CSI compression using two-sided model use case, the following Table 5 captures the pros/cons of extendibility for training collaboration Type 1:

Table 5: Pros and Cons of training collaboration Type 1

| Characteristics \ Training Types | Type 1: NW side |  | Type 1: UE side |  |
| --- | --- | --- | --- | --- |
|  | Unknown model structure at UE | Known model structure at UE | Unknown model structure at UE | Known model structure at UE |
| Whether model can be kept proprietary | No | No | No | No |
| Whether require privacy-sensitive dataset sharing | No (Note 1) | No (Note 1) | No (Note 1) | No (Note 1) |
| Flexibility to support cell/site/scenario/configuration specific model | Flexible except for UE defined scenarios. Not flexible for UE defined scenarios unless UE assistance information is supported and available. (Note 6) | Flexible except for UE defined scenarios. Not flexible for UE defined scenarios unless UE assistance information is supported and available. (Note 6) | Flexible except for NW defined scenarios. Not flexible for NW defined scenarios unless NW assistance information is supported and available. (Note 6) | Flexible except for NW defined scenarios. Not flexible for NW defined scenarios unless NW assistance information is supported and available. (Note 6) |
| Whether gNB/device specific optimization is allowed | gNB: Yes UE: No | gNB: Yes UE: less flexible compared to UE side | gNB: No UE: Yes | gNB: less flexible compared to NW side UE: Yes |
| Model update flexibility after deployment | Flexible only if UE supports the new structure | Flexible for parameter update | Flexible, less flexible than Type 1 NW side | Flexible for parameter update, less flexible than Type 1 NW side |
| Feasibility of allowing UE side and NW side to develop/update models separately | gNB: Feasible UE: Not feasible due to Type 1 definition. | gNB: Feasible with restriction for CSI reconstruction model. UE: Not feasible due to Type 1 definition. | gNB: Not feasible due to Type 1 definition. UE: Feasible. | gNB: Not feasible due to Type 1 definition. UE: Feasible with restriction for CSI generation model. |
| Whether gNB can maintain/store a single/unified CSI reconstruction model over different UEs (Note 2) | Yes | Yes. Performance refers to observations in “1 NW part model to M>1 UE part models” of clause 6.2.2.4 (Note 4) | No | No |
| Whether UE device can maintain/store a single/unified CSI generation model over different NW vendors (Note 3) | No | No | Yes | Yes. Performance refers to observations in “1 UE part model to N>1 NW part models” of clause 6.2.2.4 (Note 4) |
| Extendibility: to train new UE-side model compatible with NW-side model in use; (Note 5) | Yes | Yes | No consensus Yes (Note x1) | No consensus Yes (Note x1) |
| Extendibility: To train new NW-side model compatible with UE-side model in use; (Note 5) | No consensus Yes (Note x2) | No consensus Yes (Note x2) | Yes | Yes |
| Whether training data distribution can match the inference device | Limited | Limited | Yes | Yes |
| Software/hardware compatibility (Whether device capability can be considered for model development) | No for UE | Yes | No for NW | Yes |
| Model performance | Performance refers to clause 6.2.2 | Performance refers to clause 6.2.2 | Performance refers to clause 6.2.2 | Performance refers to clause 6.2.2 |

Note x1: The user which is to train new UE-side model has the NW-side model in use.

Note x2: The network which is to train new NW-side model has the UE-side model in use.

Proposal 6: In CSI compression using two-sided model use case, the following Table 6 captures the pros/cons of training collaboration Type 2 and Type 3:

Table 6: Pros and Cons of training collaboration Type 2 and Type 3

| Characteristics \ Training Types | Type 2 |  | Type 3 |  |
| --- | --- | --- | --- | --- |
|  | Simultaneous | Sequential | NW first | UE first |
| Whether model can be kept proprietary | Yes (Note 1) | Yes (Note 1) | Yes (Note 1) | Yes (Note 1) |
| Whether requires privacy-sensitive dataset sharing | No (Note 2) | No (Note 2) | No (Note 2) | No (Note 2) |
| Flexibility to support cell/site/scenario/configuration specific model | No consensus Not flexible | No consensus Not flexible | [Semi] flexible except for UE defined scenarios. (Note 3) [Semi] flexible for UE defined scenarios if UE assistance information is supported and available. | [Semi] flexible except for NW defined scenarios (Note 3). [Semi] flexible for NW defined scenarios if NW assistance information is supported and available. |
| Whether gNB/device specific optimization is allowed | Yes | Yes | Yes | Yes |
| Model update flexibility after deployment (Note 4) | Not flexible | No consensus Semi-flexible | Semi-flexible | Semi-flexible |
| Feasibility of allowing UE side and NW side to develop/update models separately | Infeasible | No consensus Infeasible | Feasible | Feasible |
| Whether gNB can maintain/store a single/unified model over different UE vendors (Note 5) | Yes. Performance refers to observations in “1 NW part model to M>1 UE part models” and “1 UE part model to N>1 NW part models” of clause 6.2.2.4. | Yes. Performance refers to observations in “1 NW part model to M>1 UE part models” of clause 6.2.2.4. | Yes. Performance refers to observations in “NW first training, 1 NW part model to 1 UE part model, same backbone” and “NW first training, 1 NW part model to 1 UE part model, different backbones” of clause 6.2.2.5. | Yes. Performance refers to observations in “UE first training, M>1 UE part models to 1 NW part model” of clause 6.2.2.5. |
| Whether UE device can maintain/store a single/unified CSI generation model over different NW vendors (Note 6) | Yes. Performance refers to observations in “1 NW part model to M>1 UE part models” and “1 UE part model to N>1 NW part models” of clause 6.2.2.4. | Yes. Performance refers to observations in “1 NW part model to M>1 UE part models” of clause 6.2.2.4. | Yes. Performance refers to observations in “NW first training, 1 UE part model to N>1 NW part models” of clause 6.2.2.5. | Yes. Performance refers to observations in “UE first training, 1 NW part model to 1 UE part model, same backbone”. And “UE first training, 1 NW part model to 1 UE part model, different backbones” of clause 6.2.2.5. |
| Extendibility: to train new UE-side model compatible with NW-side model in use; | Not support | Support | Support | No consensus Support (Note x1) |
| Extendibility: To train new NW-side model compatible with UE-side model in use | Not support | Not support | No consensus Support (Note x2) | Support |
| Whether training data distribution can match the inference device | No consensus Limited | Yes for UE-part model, limited for NW-part model | Limited | Yes |
| Software/hardware compatibility (Whether device capability can be considered for model development) | Compatible | Compatible | Compatible | Compatible |
| Model performance | Performance refers to clause 6.2.2 | Performance refers to clause 6.2.2 | Performance refers to clause 6.2.2 | Performance refers to clause 6.2.2 |

Note x1: The user which is to train new UE-side model has the NW-side model in use.

Note x2: The network which is to train new NW-side model has the UE-side model in use.

Proposal 7: Both training Type 1 at NW side and Type 3 with NW-first are suggested to study with higher priority.

NVIDIA

Proposal 6: Adopt training type 3 (i.e., separate training at network side and UE side, where the UE-side CSI generation part and the network-side CSI reconstruction part are trained by UE side and network side, respectively) for inter-vendor training collaboration.

Samsung

Observation 10: The bi-lateral model development framework for Type 2 and Type 3 training types incurs prohibitively large model development and engineering efforts. The vendors’ effort to develop a single model compatible to multiple models from multiple vendors may converge to a common CSI reconstruction scheme. Thus, RAN1 should strive to adopt a common reference CSI reconstruction scheme.

Proposal 5: In CSI compression using two-sided model use case, for pairing the UE-part and network-part of two-sided models trained/updated/fine-tuned by Type 3 training starting at the network, indication on network-side setting (network-side additional condition) information consider a solution that provides

- Abstraction of network’s proprietary implementation information similar to the TCI indication

- Consistency between corresponding data collection for model training and inference.

Fujitsu

Proposal 6: Regarding the extendibility of training collaboration Type 3, UE first training is not applicable to train new UE-side model compatible with NW-side model in use, and NW first training is not applicable to train new NW-side model compatible with UE-side model in use, as shown in below revised table.

Pros/cons of training collaboration types 2 and type 3

| Training types Characteristics | Type 2 |  | Type 3 |  |
| --- | --- | --- | --- | --- |
|  | Simultaneous | Sequential NW first | NW first | UE first |
| Feasibility of allowing UE side and NW side to develop/update models separately | Infeasible | No consensus | Feasible. | Feasible |
| Extendibility: to train new UE-side model compatible with NW-side model in use; | Not support | Support | Support | No consensus N/A |
| Extendibility: To train new NW-side model compatible with UE-side model in use | Not support | Not Support | No consensus N/A | Support |

Nokia

Proposal 8: For support of interoperability in CSI compression using a two-sided model, RAN1 shall consider partial standardization of the decoder models at the gNB side. As regards model training collaboration types, RAN1 shall focus on NW-first approaches, i.e., Type 2 Sequential and Type 3 NW-first. Type 3 UE-first can be also considered, provide that its potential scalability issue of training a new UE-side model with a NW-side model in use can be resolved.

Proposal 9: In CSI compression using a two-sided model, RAN1 shall evaluate which side, i.e., encoder or decoder, is more critical to E2E model inference performance as a preparation step for partial standardization of the decoder model.

Proposal 10: In CSI compression using a two-sided model, RAN1 shall investigate which aspects can be standardized and which aspects can be left out when it comes to partial standardization of the decoder.

Lenovo

Proposal 4: Add the following steps to better explain the Type-3 of training collaboration approach. Training collaboration Type-3 can be explained as the following:  Part1: UE side trains the UE side CSI generation part using a dataset it has received.  Part2: NW side trains the NW side CSI reconstruction part using a dataset it has received.  The dataset that each side receives and how the UE or the NW side train their respective part can be changed as needed.

Proposal 5: Add the Pros and cons of the general training collaboration Type-3 to the table of Technical Report where we compare the Pros and Cons of different training collaboration Types.

Proposal 6:	We proposed the addition of the following column for Type-3 Training to Table 5.1-2 of the technical report TR 38.843. The highlighted part are the ones that need change.

Table 5.1-2: Pros and Cons of training collaboration Type 3

| Characteristics \ Training Types | Type 3 |  |  |
| --- | --- | --- | --- |
|  | NW first | UE first | General form |
| Whether model can be kept proprietary | Yes (Note 1) | Yes (Note 1) | Yes (Note 1) |
| Whether requires privacy-sensitive dataset sharing | No (Note 2) | No (Note 2) | No (Note 2) |
| Flexibility to support cell/site/scenario/configuration specific model | [Semi] flexible except for UE defined scenarios. (Note 3) [Semi] flexible for UE defined scenarios if UE assistance information is supported and available. | [Semi] flexible except for NW defined scenarios (Note 3). [Semi] flexible for NW defined scenarios if NW assistance information is supported and available. | [Semi] flexible if no assistance information is needed. If assistance information is needed, [Semi] flexible if assistance information is supported and available |
| Whether gNB/device specific optimization is allowed | Yes | Yes | Yes |
| Model update flexibility after deployment (Note 4) | Semi-flexible | Semi-flexible | Semi-flexible |
| Feasibility of allowing UE side and NW side to develop/update models separately | Feasible | Feasible | Feasible |
| Whether gNB can maintain/store a single/unified model over different UE vendors (Note 5) | Yes. Performance refers to observations in "NW first training, 1 NW part model to 1 UE part model, same backbone" and "NW first training, 1 NW part model to 1 UE part model, different backbones" of clause 6.2.2.5. | Yes. Performance refers to observations in "UE first training, M>1 UE part models to 1 NW part model" of clause 6.2.2.5. |  |
| Whether UE device can maintain/store a single/unified CSI generation model over different NW vendors (Note 6) | Yes. Performance refers to observations in "NW first training, 1 UE part model to N>1 NW part models" of clause 6.2.2.5. | Yes. Performance refers to observations in "UE first training, 1 NW part model to 1 UE part model, same backbone". And "UE first training, 1 NW part model to 1 UE part model, different backbones" of clause 6.2.2.5. |  |
| Extendibility: to train new UE-side model compatible with NW-side model in use; | Support | No consensus | Support |
| Extendibility: To train new NW-side model compatible with UE-side model in use | No consensus | Support | Support |
| Whether training data distribution can match the inference device | Limited | Yes | Yes |
| Software/hardware compatibility (Whether device capability can be considered for model development) | Compatible | Compatible | Compatible |
| Model performance | Performance refers to clause 6.2.2 | Performance refers to clause 6.2.2 |  |

Proposal 7: Study the performance gains, and the extra training costs incurred by, “iterative separate training” as a potential method to improve the performance of single step Type-3 training.

Sony

Proposal 1: RAN1 should consider introducing cell specific AI/ML models for two-sided model use cases.

Proposal 2: Cell specific AI/ML model information should be broadcast (e.g. as system information) to all UEs which have AI/ML capability.

Proposal 3: If cell specific AI/ML model is provided from gNB to UE, UE can fine-tune the provided AI/ML model at UE side.

Proposal 4: gNB can configure a permission whether UE can or not fine-tune the cell specific AI/ML model which is provided from gNB

Proposal 5: If gNB allows fine-tuning of cell-specific AI/ML model, gNB needs to provide training data which is used for training of the fine-turned cell-specific AI/ML model to UE.

Proposal 6: If gNB allows fine-tuning of cell-specific AI/ML model, then during inference, gNB needs to monitor the performance of the fine-tuned AI/ML model and if the performance is not optimal, gNB can indicate to the UE to fallback to the original cell-specific model.

Panasonic

Observation 15: Type 1 training involves the exchange of AI/ML model and then, requires some common AI/ML inference algorithm and common reference/structure for model inference.

Observation 16: For Type 2 with offline training, if the consideration on the air interface specification impact on FP/BP interaction is not needed, there might be no Type 2 specific specification impact.

Observation 17: For Type 3 training collaboration with network-first training, at least the option that network generates training dataset to enable UE side supervised learning should be studied.

Observation 18: For Type 3, 3GPP may need to define some kind of requirement of CSI encoding by input and output relation, performance test or something else. The input for the training can be 3GPP specified channel model or field raw data. The output for the training can be something 3gpp defined output or network vendor specific information. The UE model performance can be checked by 3gpp specification or inter-operability test (IOT).

Observation 19: Type 3 with network-first separate training might be feasible options at least Re.19 timeline from standardization effort perspective. Type 1 with network sided training can be potential in the long-term.

NTT DOCOMO

Proposal 5: The Rel. 19 study on inter-vendor training collaboration aims to identify feasible approaches with extendibility based on existing or new proposed training types.

CAICT

Proposal 5: Training type 1 could be based on Z3(known model) and model transfer should be based on parameter exchange.

Proposal 6: Training type 3 should consider the case of dataset transfer with air interface impacts.

Fraunhofer

Proposal 1: In CSI compression using two-sided model use case, use Table 1 to capture the pros/cons of training collaboration Type 1, Type 2 and Type 3.

Table 1: Comparison of different types of offline training

| Type of training | Type 1- UE side | Type 1 – NW side | Type 1 – 3rd party | Type 2 | Type 3 – NW first | Type 3 – UE first |
| --- | --- | --- | --- | --- | --- | --- |
| UE-processing compatibility | No issues | UE vendor assistance required for UE compatibility | UE and gNB vendors assistance required | No issues (UE-vendor trained) | UE and gNB vendors assistance required | No issues (UE-vendor trained) |
| Encoder-decoder compatibility | Trained at one side | Trained at one side | Trained at one side | Jointly trained | Separately trained | Separately trained |
| Logistics of AI model design | Design by a single vendor, high complexity at UE-side | Easy (training at single entity) | Easy (training at single entity) | Harder (vendor pairings needed for AI model development) | Easy (independent training regardless of encoder-decoder pairing) | Easy (independent training regardless of encoder-decoder pairing) |
| Logistics of AI model training | No issues | Training data exchange is necessary (UE vendors need to exchange training data with | Training data exchange is necessary | Harder (vendor pairs need to exchange training data) | Harder (vendor pairs need to exchange training data) | Harder (vendor pairs need to exchange training data) |
| Revealing proprietary information | NW vendor may need to reveal some implementation info to UE vendor for decoder design | Some proprietary information may be revealed by UE side vendor to NW vendor for designing UE-compatible encoder | Some proprietary information may be revealed by UE | Some proprietary information may be revealed between pairing vendors | No revealing of proprietary information is required | No revealing of proprietary information is required |
| Performance guarantees | Possible | Possible | Possible | Possible | Hard to guarantee due to higher chances of encoder-decoder mismatch | Hard to guarantee due to higher chances of encoder-decoder mismatch |
| Whether training data distribution can match the inference device | Conditional and restricted, with assisted information from UE for device (group) specific model. | Yes | Yes | Possible | Conditional, with assisted information from UE | Yes |
| Extendibility: to train new UE-side model compatible with NW-side model in use | Not support | Not Support | Support | Not Support | Support | Support using procedures other than the example UE-first procedure |
| Extendibility: To train new NW-side model compatible with UE-side model in use | Not Support | Not Support | Not Support | Not Support | Support using procedures other than the example NW-first procedure | Support |

CEWiT

Proposal : Study the remaining aspect of inter-vendor training with respect to different training collaborations. It shall narrow down identifying the appropriate information for avoiding any discrepancy.

Proposal : Consider using model ID based identification for ensuring proper training between UE sided model and NW sided model

Proposal : In case of improving inter-vendor collaboration, store the additional information of an NW-sided model like vector-quantisation codebook name or its properties(size,feature length).

Proposal:  In case of Type-III UE first training, train the CSI reconstruction model with the knowledge of UE specific codebook.

ITL

Proposal 4: It is proposed that the training collaboration type 3 is baseline inter-vendor training collaboration for AI/ML based CSI compression.

Qualcomm

Proposal 1: Study potential specification support to enable vendors to achieve a common understanding of the PMI mapping (i.e., the mapping between the precoding matrix indicator (PMI) in the CSI feedback payload and the precoding matrix it represents), both for training and inference.

Proposal 2: To address concerns related to inter-vendor training collaboration complexity, study the approach of 2-sided model training based on registering the mapping between the PMI payload and the preferred precoding matrix that it represents.

Proposal 3: The procedure to enable a training entity to register and retrieve the PMI-precoder mapping information from the central registry can be standardized.

## 1st round discussion

### Scalable approaches for training collaboration

FL note: Resolving the issues related to the complexity of inter-vendor collaboration has been identified as one objective of the study. Different approaches have been proposed by companies in their contributions. A common theme is to provide standards-based support in some form to facilitate interoperability in a scalable way. What should be standardized and to what extent needs further analysis. Based on this discussion, the following proposal aims to identify the directions to pursue towards resolving the collaboration complexity issue.

#### Proposal 4.2.1

To resolve the issues related to inter-vendor training collaboration complexity, study the following options:

- Option 1: Standardized procedure for registration / retrieval of information about the CSI representation required for training compatible models.

  - E.g., Registering a dataset, reference model, or gradient exchange interface, for sequential training.

- Option 2: Standardized reference model structure

  - Option 2a: CSI generation part

  - Option 2b: CSI reconstruction part

- Option 3: Standardized reference model structure and parameters

  - Option 3a: CSI generation part

  - Option 3b: CSI reconstruction part

Note 1: The above options may not be mutually exclusive and may be used together.

Note 2: Other options are not precluded.

| Support / Can accept | AT&T |
| --- | --- |
| Object / Have a concern | Lenovo (comment) |

| Company | Comments |
| --- | --- |
| Lenovo | In option2, although the model structure is the same but still different UE and NW vendors needs to train their models, which may lead to different weights. So, the inter-vendor training collaboration issues (e.g., storage of different model, model identification, model switching) will still exist. Option3 is actually having a fixed encoder/decoder (or both) which is okay solution if all companies agree, however, this solution might be not acceptable as we may need to limit the complexity of the model to what low-end UE/NW equipment can handle. It also put limit on innovation from different companies in designing higher performance NN modules. Option 1 is also a way to facilitate the execution of the previous training collaboration types, but still not solving the inter-vendor training collaboration issues (e.g., storage of different model, model identification, model switching). We also wanted to point out another inter-vendor training collaboration issue which is beneficial to be further studied and discussed. Consider a case in which during training phase a specific UE Chipset vendor, e.g., vendor-C1, trains model1 in collaboration with the first NW vendor, e.g., vendor-N1, and model2 in collaboration with another NW vendor, e.g., vendor-N2. For now we can assume that we have resolved the issue related to model training and thus we have already the trained model1 and model2. Having these models, during the inference time, we expect a UE with chipset from vendor-C1 to use model1 and model2 if the UE is attached to a cell with gNB from vendor-N1 or vencodr-N2, respectively. The main issue is that, during the inference time, the gNB vendor is transparent to the UE. So, the question is how the UE should determine the correct model among model1 and model2. - Proposal: Further study different approaches of training collaboration with respect to possibility of model selection/activation during inference phase, when the manufacturer of the UE-chipset and NW-gNB are not known to the other side. |
| Huawei/HiSi | 1) Option 1 and Option 2/3 are addressing different aspects: Option 1 addresses the spec impact on the procedure, while Option 2/3 address the spec impact on the model itself. Therefore, we move Option 1 to a separate bullet and apply it to all the options above. In addition, as gradient exchange over the air is deprioritized, we do not need to discuss it. 2) Except for standardizing the model itself (structure and parameter if any), standardizing the dataset delivery (format/content/procedure) for Type 3 can also alleviate the offline cross-vendor collaboration. So, it is added as an option. - Option 1: Standardized procedure for registration / retrieval of information about the CSI representation required for training compatible models. - E.g., Registering a dataset, reference model, or gradient exchange interface, for sequential training. - Option 1: Standardized dataset format/content - Option 1a: Dataset for training CSI generation part - Option 2a: Dataset for training CSI reconstruction part - Option 2: Standardized reference model structure - Option 2a: CSI generation part - Option 2b: CSI reconstruction part - Option 3: Standardized reference model structure and parameters - Option 3a: CSI generation part - Option 3b: CSI reconstruction part - For Option 1~3, standardized procedure for alleviating the cross-vendor offline interaction about the CSI representation required for training compatible models - E.g., specified signalling to deliver a dataset, deliver a reference model, or gradient exchange interface, for sequential training. Note 1: The above options may not be mutually exclusive and may be used together. Note 2: Other options are not precluded. |
| Fujitsu | We think the dataset delivery could be included in Option 1. Please see below for some suggested change. - Option 1: Standardized procedure for registration / retrieval /delivery of information about the CSI representation required for training compatible models. |
| vivo | Support to study these options. To Lenovo, these are options that are to be further studied by companies. It seems that you have in your mindset that UE needs to train the model with vendors in vendor specific way, these are exact issues that we need to avoid. Moreover, the performance of the model cannot be automatically guaranteed by simply knowing which inference vendor they are collaborating. |
| NTT DOCOMO | In our opinions, Option 3 can solve the collaboration issues, but it may introduce some performance loss. The further study on the performance of Option 3 is necessary to identify its performance, based on which the down selection between Option 3a and 3b can be considered. We also would like to clarify that the Option 3 does not mean only one CSI generation or CSI reconstruction part will be specified. |
| NEC | For Option 1, to our understanding, model or dataset delivery/transfer should be included in Option 1. Furthermore, we tend to remove gradient exchange in Option 1 as Type 2 training collaboration type has been deprioritized. So we suggest the following update: - Option 1: Standardized procedure for registration / retrieval / delivery (or transfer) of information about the CSI representation required for training compatible models. - E.g., Registering/retrieving/delivering a dataset, or reference model, or gradient exchange interface, for sequential training. |
| Ericsson | As a high-level principle, the candidate solutions discussed in RAN1 shall be aligned with the RAN4 progress on conformance testing of two-sided models. We are ok with studying option 2 and option 3. In our understanding, the reference model mentioned in options 2 and 3 in proposal 4.2.1 are test encoder/decoder discussed in RAN4 for verifying the performance of a two-sided model. Option 1 in proposal 4.2.1 requires further clarification. It is unclear how option 1 can be used to solve the interoperability issue for model training and RAN4 conformance testing in a multi-NW-vendor and multi-UE-vendor eco-system. Like RAN4 option 4, we suggest revising the option 1 as the following: Option 1: Standardize the reference dataset (e.g., RAN4 test channel models /conditions) including the dataset content/format, and reference parameters (e.g., RAN4 test performance metric) that shall be considered for training inference model(s) - Option 1a: Training CSI generation part - Option 2a: Training CSI reconstruction part |
| Apple | We would like to further understand what RAN1 can discuss for option 1. This seems to be one method to implement training collaboration types. Whether offline approach is easier or specified approach is easier, seems out of RAN1 expertise. For approach 2 and 3, we are open to further discuss. |
| ZTE | For Option 1, we think it can not be determined in RAN1 since it is a high-layer discussion. For Option 2 and 3, it seems under heated discussion in RAN4, so we suggest further discussing it until concrete outcomes are determined from RAN4 perspective. So, we think the above options need further clarification. |
| LG | In addition to the options above, standardizing the dataset delivery (format/content/procedure) related to Type 3 also be considered for the potential solution for inter-vendor collaboration as similar to Huawei’s understanding. |
| CATT | We think that training collaboration Type 1 (model transfer) and Type 3 (dataset delivery) previously discussed in Rel-18 should also be added as candidate Options. We also propose to add a Note 3: discussions on training collaboration Type 1 and Type 3 will be treated in agenda item 9.1.3.3. |
| Xiaomi | Two-sided model is trained via offline. In our view, these options used to resolve the issues related to inter-vendor training collaboration complexity can be implemented through offline predefinition as well. At least, there are no specification impact on RAN1. If so, a note should be given to clarify. |
| TCL | In our opinion, Option 3 may be removed in this proposal. The difference between Option 3 and Option 2 it that the model parameters also need to be standardized. However, it is known that the AI model includes two types of parameters, that are hyper parameters which are configured by developers in advance before model training, and learnable parameters which are obtained from the model training process and determined by the model structure and collected training samples. Both of them are based on implementation and do not have the necessity and feasibility of standardization. To resolve the issues related to inter-vendor training collaboration complexity, study the following options: - Option 1: Standardized procedure for registration / retrieval of information about the CSI representation required for training compatible models. - E.g., Registering a dataset, reference model, or gradient exchange interface, for sequential training. - Option 2: Standardized reference model structure - Option 2a: CSI generation part - Option 2b: CSI reconstruction part - Option 3: Standardized reference model structure and parameters - Option 3a: CSI generation part - Option 3b: CSI reconstruction part Note 1: The above options may not be mutually exclusive and may be used together. Note 2: Other options are not precluded. |

## 2nd round discussion

### Scalable approaches for training collaboration

FL note: Companies’ comments have identified a few other approaches that were not listed in the 1st  round version of the proposal. There were also comments regarding the organization of the proposal, e.g., to separate out the procedure aspects discussion. These comments have been incorporated in the next version of the proposal. In this version, the options are organized based on what aspects are standardized, i.e., model structure, parameters, data format, dataset, model format, etc. Within each case, different options are listed for the procedures to transfer the relevant information.

The goal of the proposal is to list the different approaches towards addressing the inter-vendor collaboration issue in order to facilitate further study and analysis of the options.

#### Proposal 4.2.1 v2

To resolve the issues related to inter-vendor training collaboration complexity, study the following options:

- Option 1: Standardized reference model structure + parameters

  - Option 1a: CSI generation part

  - Option 1b: CSI reconstruction part

- Option 2: Standardized dataset

- Option 3: Standardized reference model structure

  - Option 3a: Parameters are registered by NW-side/UE-side at a central registry and retrieved by UE-side/NW-side from the registry using standardized procedure.

    - Option 3a-1: CSI generation part registered by NW-side, retrieved by UE-side.

    - Option 3a-2: CSI reconstruction part registered by UE-side, retrieved by NW-side.

  - Option 3b: Parameters are transferred between NW-side/UE-side servers using standardized signaling.

    - Option 3b-1: CSI generation part parameters from NW-side server to UE-side server.

    - Option 3b-2: CSI reconstruction part parameters from UE-side server to NW-side server.

  - Option 3c: Parameters are transferred over-the-air using standardized air-interface signaling.

    - Option 3c-1: Model transfer/delivery of CSI generation part from NW to UE.

    - Option 3c-2: Model transfer/delivery of CSI reconstruction part from UE to NW.

- Option 4: Standardized data format

  - Option 4a: Dataset registered by NW-side/UE-side at a central registry and retrieved by UE-side/NW-side from the registry using standardized procedure.

  - Option 4b: Dataset delivery between NW-side/UE-side servers using standardized signaling.

  - Option 4c: Dataset delivery over-the-air using standardized air-interface signaling.

- Option 5: Standardized model format

  - Option 5a: Reference model registered by NW-side/UE-side at a central registry and retrieved by UE-side/NW-side from the registry using standardized procedure.

    - Option 5a-1: CSI generation part or CSI reconstruction part registered by NW-side, retrieved by UE-side.

    - Option 5a-2: CSI generation part or CSI reconstruction part registered by UE-side, retrieved by NW-side.

  - Option 5b: Reference model transferred between NW-side/UE-side servers using standardized signaling.

    - Option 5b-1: CSI generation part or CSI reconstruction part from NW-side server to UE-side server.

    - Option 5b-2: CSI generation part or CSI reconstruction part from UE-side server to NW-side server.

Note 1: The above options may not be mutually exclusive and may be used together.

Note 2: Other options are not precluded.

Note 3: Registration / retrieval happens through a central registry and does not require direct collaboration between vendors.

| Support / Can accept |  |
| --- | --- |
| Object / Have a concern |  |

| Company | Comments |
| --- | --- |
| Lenovo | The difference between Option 2 and Option 4 is not clear to us. Option 3/4/5: In our understanding, these methods “assume” that “the NW/UE is able to create an encoder/decoder pair that is generalizable, and it does not need to be trained again for other vendors (a new UE-Type/NW-Type)”. This needs to be verified, and for simulation we should first agree on how to simulate different UE-Types/NW-Types. We note that, in our understanding, collecting different set of samples for the same EVM, is not a correct representation of different UE-Types/NW-Types (as they will still have the same statistics). |
| Intel | In general, we are fine with the proposal. However, some effort is needed to make it clearer. - It is not clear if word ‘standardized’ means that 3GPP will be responsible for it or not. We prefer to add a note to clarify it. - Seems that ‘standardized signalling’ excludes over air interface signalling. So, we think option 5c shall be added as over the air signalling is considered for other options (e.g., 3c, 4c) Thus, we propose the following changes (in yellow). #### Proposal To resolve the issues related to inter-vendor training collaboration complexity, study the following options: - Option 1: Standardized reference model structure + parameters - Option 1a: CSI generation part - Option 1b: CSI reconstruction part - Option 2: Standardized dataset - Option 3: Standardized reference model structure - Option 3a: Parameters are registered by NW-side/UE-side at a central registry and retrieved by UE-side/NW-side from the registry using standardized procedure. - Option 3a-1: CSI generation part registered by NW-side, retrieved by UE-side. - Option 3a-2: CSI reconstruction part registered by UE-side, retrieved by NW-side. - Option 3b: Parameters are transferred between NW-side/UE-side servers using standardized signaling. - Option 3b-1: CSI generation part parameters from NW-side server to UE-side server. - Option 3b-2: CSI reconstruction part parameters from UE-side server to NW-side server. - Option 3c: Parameters are transferred over-the-air using standardized air-interface signaling. - Option 3c-1: Model transfer/delivery of CSI generation part from NW to UE. - Option 3c-2: Model transfer/delivery of CSI reconstruction part from UE to NW. - Option 4: Standardized data format - Option 4a: Dataset registered by NW-side/UE-side at a central registry and retrieved by UE-side/NW-side from the registry using standardized procedure. - Option 4b: Dataset delivery between NW-side/UE-side servers using standardized signaling. - Option 4c: Dataset delivery over-the-air using standardized air-interface signaling. - Option 5: Standardized model format - Option 5a: Reference model registered by NW-side/UE-side at a central registry and retrieved by UE-side/NW-side from the registry using standardized procedure. - Option 5a-1: CSI generation part or CSI reconstruction part registered by NW-side, retrieved by UE-side. - Option 5a-2: CSI generation part or CSI reconstruction part registered by UE-side, retrieved by NW-side. - Option 5b: Reference model transferred between NW-side/UE-side servers using standardized signaling. - Option 5b-1: CSI generation part or CSI reconstruction part from NW-side server to UE-side server. - Option 5b-2: CSI generation part or CSI reconstruction part from UE-side server to NW-side server. - Option 5c: Reference model transferred between NW-side/UE-side servers using standardized air-interface signaling. - Option 5b-1: CSI generation part or CSI reconstruction part from NW-side server to UE-side server. - Option 5b-2: CSI generation part or CSI reconstruction part from UE-side server to NW-side server. Note 1: The above options may not be mutually exclusive and may be used together. Note 2: Other options are not precluded. Note 3: Registration / retrieval happens through a central registry and does not require direct collaboration between vendors. Note 4: Word ‘standardized’ mainly referred to standardization in 3GPP, but it is not limited to 3GPP only and may include other organizations |
| Apple | It is not clear the difference between option 2 and option 4. Also not clear the difference between option 3 and option 5. In the main sentence, we suggest to add the feasibility needs to be studied. In addition, the central registry needs to be clarified. Is this an network entity and run by operator? Or 3rd party server? Can model be kept proprietary? How to ensure model performance in the field? Does any cell specific/site specific model allow with these approaches? |
| Fujitsu | We support Option 4 in this version. A question we have is how to specify an AI/ML model in RAN 1. |

## 3rd round discussion

### Scalable approaches for training collaboration

FL note:

Thanks for the comments and discussion. Based on the discussion, the next version of the proposal is raised below. The sub-bullets have been removed and abstracted into a note.

#### Proposal 4.2.1 v3

To resolve the issues related to inter-vendor training collaboration complexity of AI/ML-based CSI compression using two-sided model, study the following options:

- Option 1: Fully standardized reference model (structure + parameters)

- Option 2: Standardized dataset

- Option 3: Standardized reference model structure + Parameter exchange between NW-side and UE-side

- Option 4: Standardized data format + Dataset exchange between NW-side and UE-side

- Option 5: Standardized model format + Reference model exchange between NW-side and UE-side

Note 1: The above options may not be mutually exclusive and may be used together.

Note 2: Other options are not precluded.

Note 3: The study should consider how different delivery methods of the parameters / dataset / reference model would affect the feasibility and collaboration complexity of options 3 / 4 / 5 respectively, e.g., over the air-interface, within the core-network, via a central repository, etc.

| Support / Can accept | Samsung (comment), LG, Panasonic |
| --- | --- |
| Object / Have a concern |  |

| Company | Comments |
| --- | --- |
| Samsung | We fully support the direction. Regarding Note 3. It is not clear how within the core network and a central repository are different. We suggest the following modifications. Note 3: The study should consider how different delivery methods of the parameters / dataset / reference model would affect the feasibility and collaboration complexity of options 3 / 4 / 5 respectively, e.g., with or without over the air-interface, within the core-network, via a central repository, etc. |
| ZTE | For the main text, considering in the WID, we didn’t limit to study the issues of inter-vendor training collaboration complexity, we need to also consider and study the feasibility of the listed options. So, we suggest rewording it as below: To resolve Regarding the issues related to inter-vendor training collaboration complexity of AI/ML-based CSI compression using two-sided model, study the following options: For Note 3, the example listed here may instruct us to study the related method which may involve other working group other than RAN1. In RAN1, we cannot decide the different delivery method within the high-layer, e.g., core-network. So, we suggest removing the e.g. in the Note 3 as below. Note 3: The study should consider how different delivery methods of the parameters / dataset / reference model would affect the feasibility and collaboration complexity of options 3 / 4 / 5 respectively, e.g., over the air-interface, within the core-network, via a central repository, etc. |
| Xiaomi | From RAN1’s perspective, we should focus on delivery methods via air-interface. We are fine with the wording for Note 3 from Samsung. |
| ETRI | We agree on the direction, however, we are little unclear on the terminologies 'standardized' and 'reference'. It would be better if it is further clarified if its possible. |
| OPPO | Support this proposal. We prefer to modify Option 1 as: - Option 1: Fully/partially standardized reference model (structure + parameters) From our view, one missing case is that only part of model is specified, and the remaining part or additional blocks/layers can be added on the specified model by implementation. Moreover, some clarifications are also required, e.g. ‘standardized’, ‘reference’, ‘format’. |
| TCL | We generally agree with this proposal. But we feel the difference between Option 1 and Option 3 is ambiguous. These two options may be reworded to make it clear which part is considered to be standardized. |
| Ericsson | We support the proposal in general. Agree with Samsung’s suggestions on Note 3 modification. Clarification on dataset for option 2 is needed. |
| CMCC | Agree with the version form Samsung. BTW, we are not clear with the meaning of ‘reference’ in Option 5, and for the model format, is it something like ONNX? |
| NEC | We are fine with all options. But for Note3, we think RAN1 should ideally not be determining the feasibility of these solutions in core network or via central repository. RAN1 work should probably stop at air interface-based issues. For other issues RAN2 or SA groups can be consulted. Therefore, we suggest the following updates: Note 3: The study should consider how different delivery methods of the parameters / dataset / reference model would affect the feasibility and collaboration complexity of options 3 / 4 / 5 respectively, e.g., over the air-interface, within the core-network, via a central repository, etc. |
| CATT | Genrally fine with the proposal |

### Aspects for comparison

FL note: After the different options have been identified for how to address the inter-vendor collaboration issues, the next step would be to list the different aspects to consider when comparing the options. Accordingly, the following proposal is raised.

#### Proposal 4.4.2

For the study of inter-vendor collaboration issues for AI/ML-based CSI compression using a two-sided model, consider the following aspects when comparing different options:

- Inter-vendor collaboration complexity, i.e., whether bilateral collaboration is required between vendors.

- Performance gain over benchmark scheme.

- Implications for minimum performance guarantee, i.e., RAN4 / testing related aspects.

- Device feasibility aspects.

| Support / Can accept | Samsung, LG, Panasonic (with comment), NTT DOCOMO |
| --- | --- |
| Object / Have a concern |  |

| Company | Comments |
| --- | --- |
| Samsung | Fully support the proposal with the below added aspect. For the study of inter-vendor collaboration issues for AI/ML-based CSI compression using a two-sided model, consider the following aspects when comparing different options: - Inter-vendor collaboration complexity, i.e., whether bilateral collaboration is required between vendors. - Performance gain over benchmark scheme. - Implications for minimum performance guarantee, i.e., RAN4 / testing related aspects. - Device feasibility aspects. - Scalability to support large number of models, e.g., site/cell-specific models. |
| LG | Generally fine with the proposal and add ‘at least’ in the main-bullet. |
| ZTE | Considering there are many options which may alleviate the issues of inter-vendor training collaboration. So, dataset/model/parameters delivery may be involved, so the related aspects such as delivery overhead, latency should also be considered when comparing the different options. So we suggest adding a bullet as - Dataset/model/parameters delivery related aspects, e.g., overhead, latency, etc. |
| Panasonic | We agree to LG’s comment. It would be better to add “at least” in the main bullet as follows. For the study of inter-vendor collaboration issues for AI/ML-based CSI compression using a two-sided model, consider at least the following aspects when comparing different options: |
| Xiaomi | What is the benchmark scheme? We have not discussed it. |
| ETRI | We agree on the proposal and modified versions of Samsung and ZTE also. |
| Ericsson | Support the proposal in general. Suggest revising the third bullet as the following: - Implications for minimum performance guarantee Feasibility, complexity, and interoperability at a minimum required performance level, i.e., RAN4 / testing related aspects. |
| CMCC | OK with this direction. But what is the benchmark scheme? It is ideal inter-vendor collaboration, offline collaboration or something else? |
| NEC | We agree with Samsung and ZTE. |
| CATT | Agree with LG’s proposal |
| Fujitsu | Please clarify what is the benchmark scheme. |

# Data collection

## Summary of company proposals

From the submitted contributions, proposals related to data collection are summarized below.

Huawei, HiSilicon

Proposal 12: For the NW side data collection, confirm the necessity and feasibility of UE report of the ground-truth CSI.

- For the data sample type, prioritize precoding matrix over channel matrix.

- For the data sample format, prioritize Rel-16 eType II CB based quantization with new parameters, and take the following new parameters (captured in the Rel-18 observation) as candidates for discussion.

  - L= 8, 10, 12; pv = 0.8, 0.9, 0.95; reference amplitude = 6 bits, 8 bits; differential amplitude = 4bits; phase = 5 bits, 6 bits.

- For the number/index(es) of layers for the collected ground truth CSI, it can be indicated by NW.

Proposal 13: In CSI compression with training collaboration Type 3, the following aspects could be further studied for over the air dataset delivery from RAN1 perspective, including:

- Dataset ID, which is used to differentiate the models to be trained at the opposite side.

- Dataset size, e.g., the number of data samples contained in the delivered dataset.

Proposal 14: For the dataset delivery of CSI compression over air-interface, NW can split the overall dataset into many subsets each with a limited number of data samples (e.g., with an overhead comparable to the RRC signaling). The subsets can be separately sent to different UEs, and all subsets are associated with a common dataset ID for the UE side re-combination.

ZTE

Proposal 10: For network side data collection, support to further study

- Enhanced Rel-16 eTypeII codebook design to achieve high-resolution CSI for model training and performance monitoring

Proposal 11: To enable high-quality data collection from UE to network, at least support

- UE reports data quality related information to NW, e.g., SINR, CQI, positioning information

- NW configures a threshold of data quality to UE and UE only reports the qualified data to NW

CMCC

Proposal 6: In CSI compression using two-sided model use case, regarding the ground truth CSI format for NW side data collection for performance monitoring and model training, R16 eType II codebook and Rel-18 Doppler codebook can be used as a starting point.

Proposal 7: In CSI compression using two-sided model use case, regarding the ground truth CSI format for NW side data collection, the basic codebook structure could be reused, along with the basic concept of spatial domain, frequency domain and Doppler domain basis.

Proposal 8: In CSI compression using two-sided model use case, regarding the ground truth CSI format for NW side data collection, the exact supported values of codebook parameters can be studied to make sure high resolution data report.

Intel

Proposal 7: Whether/how to support new CSI report format for ground truth CSI quantization should be further studied considering the corresponding CSI overhead.

Google

Proposal 7: Support to configure the number of layers for the report for NW side data collection for performance monitoring.

Proposal 8: Support to report singular values for the ground-truth CSI.

Proposal 9: Support to report CQI/RI in addition to the ground-truth CSI.

Proposal 10: Reuse the existing CPU framework to handle the UE complexity for the measurement and report for NW side data collection.

Proposal 11: Support to maintain the same understanding between the NW and UE on when to perform the measurement for UE side data collection based on the following options:

- Option 1: The measurement for UE side data collection is configured by the NW

- Option 2: UE request CSI-RS for data collection

Xiaomi

Proposal 8: At least for training Type 1 at NW side and Type 3 with NW-first, it is necessary and feasibility for NW side data collection considering the following aspects:

- Significant feedback overhead reduction by using codebook-based quantization.

- Signalling overhead reduction by using cell-specific CSI-RS resource configuration.

- No much strict latency requirement for data collection for model training or performance monitoring.

OPPO

Proposal 2: regarding the data collection for CSI compression, cell/site/scenario related “condition information” and “addition condition information” should be considered during the data collection stage

- Condition information including CSI-related information such as the CSI type, e.g. raw channel or precoding matrix, and the CSI configurations, e.g. number of antenna ports, number of sub-bands, ranks.

- Additional condition information including cell/site/scenario related information such as cell/site/scenario ID, indoor/outdoor indication, LoS/NLoS flag and UE ID.

Beijing Jiaotong University

Proposal 4: For AI/ML model training for CSI feedback enhancement, study the benefits of using data-augmented training compared to non-augmented training, and potential specification impact related to training data type and training data source determination for training data collection.

China Telecom

Proposal 1: Support to enable high-quality data collection from UE to network, at least including:

- UE reports data quality related information to NW, e.g., SINR, CQI, positioning information

- NW configures a threshold of data quality to UE and UE only reports the qualified data to NW

Fujitsu

Proposal 7: For CSI compression using two-sided models, RAN 1 to further discuss using codebook-like approach to report ground-truth CSI for AI/ML model training, e.g. Rel-16 e-type II-like codebook with enhanced parameter values.

Lenovo

Proposal 1: Support procedures/signaling enabling UE/NW to associate the data/sample with the conditions/additional conditions under which the data/sample has been collected.

Proposal 2: Support procedures/signaling enabling UE/NW for transmission of subset of samples among the set of measured/collected samples from the environment.

Proposal 3: For transmission of ground-truth CSI samples consider the performance transmitting more samples instead of fewer samples with higher accuracy per sample (e.g., Rel-16 Type II with new parameters), especially for cases that the number of samples are not large, e.g., data transfer for model monitoring or model update.

Panasonic

Observation 4: Data collection for model training and non-real time (slow) monitoring is not required to be real-time and then latency requirement can be relaxed.

Observation 5: Ground-truth CSI reporting could be realized through U-plane at least for data collection for model training and non-real time (slow) monitoring.

Observation 6: Assuming fast monitoring is 100s of ms order, U-plane, RRC or MAC-CE can be sufficient.

Observation 7: On data sample type / format for ground-truth CSI reporting, high resolution codebook-based format e.g., legacy codebook (e.g., eType II codebook) with potential enhancements such as extend more configurations in some parameters, should be studied.

Observation 8: For NW-side data collection, at least time stamps / situation of measurement, cell ID and UE location should be considered as the UE-side additional condition.

Observation 9: For NW-side data collection, the necessity and feasibility of UE reporting Rx filter assumption to network should be studied.

Observation 10: For UE-side data collection for UE side training, in order to identify the scenario / configuration, how to share the NW-side additional condition should be studied. Instead of informing actual configuration, some kind of configuration ID and /or change timing of NW-side additional condition is necessary.

Fraunhofer

Proposal 2: If the NW is collecting the data for training, the UE should provide the NW node with essential information about the configuration of data e.g., type of the CSI included in the data, quantization parameters, how often the data should be collected, etc.

Proposal 3: The signaling/configuration for data collection should include a quality requirement/threshold for the ground truth labels. If such requirement is not guaranteed to be met, then data collection should not be initiated.

CEWiT

Proposal : For data collection study the impact of semi-persistent and aperiodic based CSI  prediction.

# Monitoring

## Summary of company proposals

From the submitted contributions, proposals related to monitoring are summarized below.

Huawei, HiSilicon

Proposal 15: For monitoring metrics, consider intermediate KPI and eventual KPI as the starting point in Rel-19.

- Further discuss the reporting mode, e.g., per sample reporting and statistic reporting over a number of monitored samples.

- Legacy CSI based monitoring and input distribution-based or output distribution-based monitoring can be deprioritized in Rel-19.

Proposal 16: There is no strong motivation for specifying the UE side proxy model for monitoring.

Proposal 17: For the intermediate KPI based monitoring, consider the signaling of ground-truth CSI/recovery CSI between NW and UE to assist the calculation of the intermediate KPI.

- NW side monitoring based on the ground-truth CSI (target CSI with realistic channel estimation) reported by the UE.

- UE side monitoring based on the recovery CSI (output of the CSI reconstruction model) indicated by NW.

ZTE

Proposal 13: Prioritize to study the specification impacts on at least the following case for model performance monitoring,

- NW-side monitoring based on the target CSI with realistic channel estimation associated to the CSI report, reported by the UE.

Proposal 14: In CSI compression using two-sided model use case, deprioritize the study on UE-side monitoring in Rel-19 study phase.

CMCC

Proposal 5: For performance monitoring, the following two options could be prioritized:

- NW-side monitoring based on the ground-truth CSI report.

- UE-side monitoring based on the recovery CSI indication.

Intel

Proposal 8: NW-side model performance monitoring based on an intermediate KPI calculated using channel measured via SRS can be supported without additional specification impact.

- Target CSI: channel/precoding matrix derived via SRS.

- Output CSI: output of the two-sided model with channel/precoding matrix derived via SRS at the input.

Proposal 9: For SRS-based model performance monitoring, reuse methodology for UL channel generation for FDD systems agreed for FDD CSI enhancement in Rel-17 at RAN1#102-e

Google

Proposal 4: Do not support to use SGCS as the metric for ML performance monitoring.

Proposal 5: Support the hypothetical BLER as the metric for ML performance monitoring.

Proposal 6: Support the baseline for model performance monitoring based on the non-ML based CSI, i.e. the CSI based on existing codebook that the UE supports.

- A model performance failure is identified if the hypothetical BLER measured based the ML based CSI and the CQI from the non-ML based CSI is above a threshold

- ML based CSI compression should not mandate the UE to support eType2 codebook

NEC

Proposal 4: Further study the feasibility of the CSI reconstruction model at UE side.

Proposal 5: If the CSI reconstruction model at UE side is proven to be feasible, at least support UE-side monitoring based on the output of the CSI reconstruction model at UE side.

Xiaomi

Proposal 9: It is necessary and feasible that performance monitoring by using intermediated KPIs or an existing CSI feedback scheme as the reference considering the following aspects:

- Significant overhead reduction for quantization of the target CSI or output of CSI reconstruction via enhanced eType II codebook parameters

- Affordable complexity for quantization of the target CSI or output of CSI reconstruction, which is similar to that of legacy eType II codebook.

- Ensuring the robust of monitoring performance by using an existing CSI feedback scheme as the reference

China Telecom

Proposal 3: In CSI compression using two-sided model use case, deprioritize the study on UE-side monitoring based on the output-CSI transmitted from NW to UE.

Proposal 4: Prioritize to study the specification impacts on at least the following case for model performance monitoring

- NW-side monitoring based on the target CSI with realistic channel estimation associated to the CSI reported by the UE.

Fujitsu

Proposal 14: For CSI compression using two-sided AI/ML models, the feasibility, reliability, and generalization capability of the UE-side AI/ML model performance monitoring using proxy model(s) should be evaluated and concluded before any further discussion on the related specification impacts.

Proposal 15: For the NW-side AI/ML model performance monitoring for CSI compression, RAN1 to prioritize the study of using the codebook-based quantization method to obtain the ground-truth CSI. Besides, adding new parameter values to legacy codebook for higher resolution ground-truth CSI should be studied.

Proposal 16: For CSI compression using two-sided AI/ML models, RAN1 to study the signaling and configuration for NW-side AI/ML model performance monitoring.

Proposal 17: For CSI compression using two-sided AI/ML models, regarding the NW-side AI/ML model performance monitoring using an existing CSI feedback scheme as a reference, RAN1 to study the potential specification impacts for the following three options:

- Option-1: UE selects and reports PMI to the NW.

- Option-2: UE computes and reports the intermediate KPI for the reference scheme, e.g., the SGCS of the recovered CSI from PMI and the ground-truth CSI.

- Option-3: NW selects the PMI based on the ground-truth CSI reported by a UE.

Proposal 18: For CSI compression using two-sided AI/ML models, RAN1 to study the procedures and signaling needed for the follow-up actions after the AI/ML model performance monitoring, including falling back to legacy codebook-based CSI reporting from AI/ML-based methods.

Proposal 19: For the performance monitoring of CSI compression using two-sided AI/ML models, RAN1 to study the potential specification impacts on monitoring the performance of an inactive AI/ML model, taking at least the following cases into consideration:

- Initial activation of an AI/ML model.

- Re-activation of an AI/ML model.

InterDigital

Proposal 2: For UE-side monitoring, study both time- and event-based triggers for reporting the monitoring metrics.

Proposal 3: For UE-side monitoring, study appropriate monitoring metrics to avoid unnecessary model updating or switching.

Proposal 4: Study the benefits of using out-of-distribution metrics for UE-side monitoring.

Proposal 5: For UE-side monitoring, study the UE-side monitoring metrics (including report size, metrics quantization, report frequency) to avoid increasing the feedback overhead.

Proposal 6: In case of NW-side monitoring, study monitoring approaches with low signaling overhead.

LG Electronics

Proposal 2: If temporal/spatial/frequency (TSF)-CSI compression is supported for an alternative solution for SF-CSI compression, discuss how to generate ground truth CSI for at least for model monitoring.

Panasonic

Observation 11: There are at least two purposes for performance monitoring. One is to check new untested model / parameter behavior. The other is to check current model / parameters are suitable to current environment.

Observation 12: If AI/ML models are trained by UE side and AI/ML model update cycle is non-real time, UE vendor specific monitoring in offline sufficiently work.

Observation 13: If AI/ML models are trained by network side and AI/ML model update cycle is non-real time, A/B test can be used, and data collected for non-real time performance monitoring can also be used for model training.

Observation 14: Further study Direction 1 and Direction 3 with proxy model framework.

- Direction 1: Network-side monitoring based on the target CSI with realistic channel estimation associated to the CSI report, reported by the UE or obtained from the UE side.

- Direction 3: UE-side monitoring based on the output of the CSI reconstruction model at the UE side.

The CSI reconstruction part for performance monitoring at the UE is a proxy model, which is different from the actual CSI reconstruction part at the network.

Fraunhofer

Proposal 4: For the two-sided model for the CSI feedback or CSI prediction use cases, the gNB monitors the performance of the AI model and detects possible faults based on the CSI report from the UE.

MediaTek

Proposal 1: Study and evaluate the possibility of using uplink CSI collected from SRS for training and monitoring.

Proposal 4: Study AI/ML model monitoring techniques other than NW-side intermediate KPI-based monitoring

# Inference aspects (pairing / CQI / quantization)

## Summary of company proposals

From the submitted contributions, proposals related to inference aspects (pairing, CQI, quantization, etc.) are summarized below.

Huawei, HiSilicon

Proposal 18: For quantization methods of the CSI report, further study potential specification impact on quantization alignment using standardized quantization scheme.

- For vector quantization,

  - Configuration/reporting/updating of the quantization dictionary.

  - Segmentation of the CSI generation model output to map with short VQ vector.

- For scalar quantization,

  - The configuration of the quantization granularity/range.

Proposal 19: The down selection of the model pairing options can be discussed in Rel-19 after other aspects are clearer, e.g., model identification, training collaboration types.

Proposal 20: For the study of CQI determination in inference, consider Option 1 (CQI is NOT calculated based on the output of CSI reconstruction part from the realistic channel estimation) as a starting point.

Proposal 21: For CSI report in inference, on top of the legacy CSI reporting principles, the following AI/ML specific aspects may be additionally studied:

  - The CSI priority rules, e.g., priority rules by considering the AI/ML specific reporting type, priority rules within the bit sequence of per AI/ML specific inference CSI report.

  - The CSI processing unit (CPU), e.g., the required CPU value may consider difference of UE part model complexity.

  - The CSI mapping, e.g., factors representing the part 2 size in CSI part 1, mapping of the CSI generation part output in CSI part 2, etc.

ZTE

Proposal 12: For CQI determination, at least prioritize the specification impact discussions on Option 1a, Option 1b.

Google

Proposal 1: Support the following types of CSI report for CSI compression:

- Type 1 (Compression of channel): UE reports subband L1-SINR and compressed channel

- Type 2 (Compression of channel eigenvector): UE reports compressed channel eigenvector for a configured rank

- Type 3 (Compression of W2): UE reports W1 and compressed W2 for a configured rank

Proposal 2: The priority for non-ML based CSI report should be higher than the priority of ML based CSI report.

Proposal 3: Support the CPU occupancy rule for ML based CSI based on two types processing unit

- Type1 CPU: a measurement processing unit (MPU) used for channel estimation and pre-processing

- Type2 CPU: an inference processing unit (IPU) used for inference for ML based CSI

Proposal 12: Support hybrid AI/ML based and non-AI/ML based CSI measurement and report

- UE reports the CSI based on AI/ML if it reports a small RI and the UE can report the CSI based on Type1 codebook if it reports a large RI

NEC

Proposal 6: If the CSI reconstruction part at UE side is proven to be feasible, at least support Option 2a for CQI determination in CSI report. If not, support Option 1a/1b.

Proposal 7: For defining the pairing information used to enable the UE to select a CSI generation model(s) that is compatible with the CSI reconstruction model(s) used by the gNB, down select from the following options:

- Option 1: The pairing information is in the forms of the CSI reconstruction model ID that NW will use.

- Option 2: The pairing information is in the forms of the CSI generation model ID that the UE will use.

- Option 3: The pairing information is in the forms of the paired CSI generation model and CSI reconstruction model ID.

Comba

Proposal 1:Develop standard for quantization methods to mitigate the effect of quantization noise on performance fading.

Proposal 2:Start to develop dequantization network to address the noise issue introduced by quantization.

TCL

Proposal 1: In the case of CSI compression using a two-sided model, the design of an AI/ML-specific CSI-RS resource and CSI reporting configuration that may be compatible with the traditional CSI reporting scenario should be considered in the following aspects:

- AI/ML-specific CSI-RS resource configuration for CSI compression

- AI/ML-specific fields in CSI-ReportConfig IE

- Dedicated report quantities and report configurations for AI

Proposal 2: The definition of AI/ML-specific priority for CSI reporting in relation to CSI compression should be considered in comparison with the traditional CSI priority rules.

Proposal 3: When the UE supports both AI/ML and non-AI/ML CSI reporting, it is necessary to redefine the priority rule considering different types of CSI reporting.

Proposal 4: The study of how to describe the capabilities of a UE to implement AI/ML models for inference on CSI compression and calculations should be undertaken.

Xiaomi

Proposal 10: The legacy priority rule can be reused to define the priority the AI/ML based CSI reporting, and a priority value PriiCSIy,k,c,sPriiCSIy,k,c,sPriiCSIy,k,c,s PriiCSIy,k,c,sPriiCSIy,k,c,swith new parameter value or introducing new parameter  PriiCSIy',y,k,c,s is used to indicate the priority of AI/ML based CSI reporting.

Proposal 11: The compressed CSI part 2 should be divided into 1<N groups for CSI omission. How to divide compressed CSI part 2 into N groups needs to further study.

China Telecom

Proposal 2: For CSI compression sub use case, the pairing information can be included in the process of functionality/model identification.

Fujitsu

Proposal 8: For CSI compression using two-sided AI/ML models, support both the following alternatives of precoding matrix for output-CSI-UE and input-CSI-NW:

- Alt 1: The precoding matrix in spatial-frequency domain

- Alt 2: The precoding matrix represented using angular-delay domain projection.

Proposal 9: For CSI compression using two-sided AI/ML models, support the following approaches for AI/ML model alignment:

- UE initiated: UE reports the pairing information for NW confirmation.

- NW initiated: NW indicates the pairing information supported for UE confirmation.

Pairing information could be in the form of model ID.

Proposal 10: For CSI compression using two-sided AI/ML models, RAN1 to further study using local model IDs in AI/ML model operations and CSI configuration/reporting after model alignment between UE and NW, which reduces the overhead compared to global model IDs.

Proposal 11: For CSI compression using two-sided AI/ML models, global model ID is sufficient for model alignment, and there is no need to introduce pairing IDs.

Proposal 12: For CSI compression using two-sided AI/ML models, RAN1 to further study the configurations and CSI reporting formats required for various AI/ML model settings. To reduce the normative workload, the following could be down selected:

- AI/ML-model-setting-specific CSI configurations and CSI reporting formats.

- A configuration and CSI reporting format adapting to various possibilities, including at least

  - layer specific and rank common.

  - layer specific and rank specific.

  - layer common and rank common.

  - layer common and rank specific.

Proposal 13: For CSI compression using two-sided AI/ML models, deprioritize Option 2 proposed in RAN1 #112 for CQI determination.

- Option 2: CQI is calculated based on the output of CSI reconstruction part from the realistic channel estimation.

Nokia

Proposal 11: RAN1 to focus on the evaluation of Options 1a and 2a-1 for CQI calculation, also considering proposals for Options 1b and 2a-2.

Proposal 12: RAN1 to study the feedback of CQI for different rank hypotheses.

Proposal 13: RAN1 to study the specification effect of layer common, layer specific, rank common, and rank specific architectures to determine how specifications affect which architectures are supported.

Lenovo

Proposal 8: Support definition of pairing information based on the conditions/additional conditions assigned to the samples of the datasets used for training of the model.

Proposal 9: Further study model identification/selection procedures during inference time when different models have been developed for different UE-NW vendor pairs.

Proposal 10: Support procedures/signalling enabling CSI-compression models having both Scaler and vector Quantizers for generation of the CSI-feedback bits.

Panasonic

Observation 3: For CQI determination in CSI report, further study following options.

- Option 1: CQI is NOT calculated based on the output of CSI reconstruction part from the realistic channel estimation, including

  - Option 1a: CQI is calculated based on target CSI with realistic channel measurement

  - Option 1b: CQI is calculated based on target CSI with realistic channel measurement and potential adjustment

- Option 2: CQI is calculated based on the output of CSI reconstruction part from the realistic channel estimation, including

  - Option 2a: CQI is calculated based on CSI reconstruction output, if CSI reconstruction model is available at the UE and UE can perform construction model inference with potential adjustment

    - The CSI reconstruction part for CQI determination at the UE is a proxy model, which is different from the actual CSI reconstruction part at the network.

CEWiT

Proposal:  Model pairing procedure to be performed before inference operation, with the assistance of UE capability report information to ensure NW sided model can avoid any model mismatch.

# Other topics (work plan, new use cases)

## Summary of company proposals

From the submitted contributions, proposals related to other aspects not covered in other sections (work plan, new use cases, etc.) are summarized below.

FutureWei

Proposal 2: For Rel-19 further study on CSI compression using two-sided model use case, consider CSI LUT-based approach as one option/mechanism for balancing the tradeoff between performance (reconstruction accuracy and CSI feedback overhead reduction) and potential complexity involved in leveraging AI/ML based approach.

Ericsson

Proposal 3: Consider assuming antenna arrays where the NR codebook design assumption of uniform planar 2D antenna array of equally spaced identical subarrays doesn’t hold for the CSI compression use case.

vivo

Proposal 4: Give higher priority to the study on improving trade-off between performance and complexity and discussion of inter-vendor training collaborations in R19 before checkpoints in RAN#105.

ZTE

Proposal 7: For CSI compression use case in Rel-19 study phase, further down-select at most one new sub-use case for the potential Rel-19 normative work.

Proposal 8: The same baseline is suggested to be adopted for evaluation on the new sub use-cases, at least using Rel-18 enhanced Type II codebook for predicted PMI and Rel-18 AI/ML-based spatial-frequency domain CSI compression for performance calibration.

Intel

Proposal 1: PMI search and PMI reconstruction complexity assumed for performance evaluation of eType II PMI codebook should be disclosed by companies.

- Alignment of PMI search and PMI reconstruction complexity assumption should be further discussed in RAN1.

Proposal 2: Performance and complexity tradeoffs for AI/ML CSI compression should be analyzed for a certain CSI overhead point.

Proposal 3: The following aspects are also considered for the additional study on AI/ML CSI compression:

- Complexity of RI and CQI determination.

- UE and gNB memory size requirements.

CATT

Proposal 6: In CSI compression using two-sided model use case, for the study in SI phase in Rel-19, the following prioritizes are considered:

- (High priority) Improving the performance of AI/ML based CSI compression sub use case

- (High priority) Down selecting the training collaboration types

- (High/medium priority) Issues related to data collection:

  - Down selecting data collection method

  - Contents of data sample

    - Data sample type

    - Data sample format

  - Assistance information, e.g., information for categorizing the data for the purpose of differentiating characteristics of data due to specific configuration, scenarios, site etc.

    - Study the necessity and potential solutions(if the necessity has been identified)

  - Enhancement on CSI-RS configuration

    - Study the necessity and potential solutions(if the necessity has been identified)

- (High/medium priority) Down selecting  performance monitoring methods

- (High/medium priority) Conclusion on whether standardized quantization scheme is adopted

- (Medium/low priority) Issues related to CSI configuration and report

- (low priority) Dataset delivery

Proposal 7: In CSI compression using two-sided model use case, for the study in SI phase in Rel-19, the following prioritization on the options in corresponding aspects are considered:

- Training collaboration Type 1 and Type 3 is prioritized, and Type 2 is deprioritized

- Data collection: Data collection at NW-side for training is prioritized

- Performance monitoring at NW-side is prioritized, proxy model based performance monitoring is deprioritized

- Standardized quantization scheme is prioritized

NEC

Proposal 2: In order to reduce the complexity, further study model compression.

Comba

Proposal 3: Develop deep learning models for complex channels.

Proposal 4: Investigate fine-tuning methods for models applied to complex channels.

Xiaomi

Proposal 13: At least model-ID-based LCM for two-sided AI/ML model should be supported.

Proposal 14: Type B1 or Type B2 could be considered to identify two-sided AI/ML model between UE and network. The mechanisms or procedure for model identification of two-sided AI/ML model by using Type B1 or Type B2 needs to study.

Samsung

Proposal 4: Study the impact of input pre-processing (dimensionality reduction) on performance and model.

Proposal 6: Consider to study joint source-coding, channel-coding and modulation (JSCCM)-based CSI feedback for CSI compression.

Fujitsu

Proposal 1: Regarding performance evaluation, RAN1 should focus on the new use cases for two-sided model for the study in Rel-19.

Proposal 5: In Rel-19, RAN1 prioritize the study on the use case of joint CSI prediction and compression, and the use case of CSI compression over spatial/temporal/frequency domain, i.e., CSI compression assisted by historical CSI information.

Panasonic

Observation 1: For complexity comparison to study the scalability of rank > 1 solutions, the total complexity with multiple models should be taken into account.

Observation 2: Further generalization / scalability performance evaluation taking into account multiple scenario / configurations aspects is necessary.

NTT DOCOMO

Proposal 1: Study the following approaches to improve the performance of AI/ML CSI compression and feedback during Rel. 19

- Temporal-spatial-frequency domain CSI compression and feedback

- Joint CSI compression and prediction

- Cell/site-specific models

- Other approaches are not precluded.

Proposal 6: Reuse Rel. 18 intermediate KPIs and eventual KPIs for Rel. 19 CSI feedback study.

- Use the eventual KPIs to assess the overall system performance of AI/ML-based CSI compression.

- Use the intermediate KPIs to assess the potential significant performance loss over specific aspects.

- No need to introduce additional intermediate KPIs.

MediaTek

Proposal 2: Evaluate effectiveness of complexity reduction techniques in reducing both computational and storage complexities of AI/ML models for CSI compression

Proposal 3: Consider and evaluate eType II algorithm’s complexity as the baseline of computational complexity

ITL

Proposal 3: RAN1 should carefully consider CSI compression scheme for the balancing the trade-offs between performance and complexity/overhead reduction as the complexity of AI/ML models can impact the feasibility of system implementation and operational costs.

NTPU

Proposal 1: RAN1 to down-select one scheme among S-T-F CSI compression and CSI compression plus CSI prediction

# Proposals for offline/online sessions

## Proposals for Tuesday February 27

#### Proposal 2.2.1 v2

For the evaluation of temporal domain aspects of AI/ML-based CSI compression using two-sided model in Release 19, adopt the following categorization for study:

| Case | Target CSI slot(s) | Whether CSI generation part uses past CSI information | Whether CSI reconstruction part uses past CSI information |
| --- | --- | --- | --- |
| 0 | Present slot | No | No |
| 1 | Present slot | Yes | No |
| 2 | Present slot | Yes | Yes |
| 3 | Future slot(s) | Yes | No |
| 4 | Future slot(s) | Yes | Yes |

Note 1: For the CSI generation part, the past CSI information may include past model inputs and/or any information derived from them. For the CSI reconstruction part, the past CSI information may include past CSI feedback instances and/or any information derived from them.

Note 2: For case 3 and case 4, the CSI generation model at the UE may perform prediction as a separate step or jointly with compression. Similarly, the CSI reconstruction model at the gNB may perform prediction as a separate step or jointly with reconstruction. Companies to report which option is selected, and whether the prediction is AI/ML-based or not.

Note 3: “Target CSI slot(s)” refers to the slot(s) to which the CSI feedback in the report corresponds. “Present slot” refers to the slot of the most recent CSI-RS measurement used to generate the CSI report. “Future slot(s)” includes at least one slot after the present slot and may include the present slot as well.

Note 4: Down-selection is not precluded.

#### Proposal 2.2.2 v2

For the evaluation of temporal domain aspects of AI/ML-based CSI compression using two-sided model in Release 19, adopt the following as baseline options for UE distribution:

- Option 1: 80% indoor, 20% outdoor

- Option 2: 100% outdoor

Note: Indoor speed is 3 km/h, outdoor speed is chosen from the following options: 10 km/h, 20 km/h, 30 km/h, 60 km/h, 120 km/h. Assumption on O2I car penetration loss and spatial consistency follow the R18 AI based CSI prediction.

#### Proposal 2.2.3 v2

For the evaluation of temporal domain aspects of AI/ML-based CSI compression using two-sided model in Release 19, adopt the following benchmark scheme for performance comparison:

- For cases without prediction of future CSI, use the same benchmark scheme assumed in R18 AI/ML-based CSI compression study.

- For cases with prediction of future CSI, use the same benchmark scheme assumed in R18 AI/ML-based CSI prediction study, with R18 MIMO eType II codebook for compressing the feedback.

- Also adopt R18 AI/ML-based CSI enhancement as an additional benchmark scheme.

#### Proposal 2.2.4 v2

For the evaluation of temporal domain aspects of AI/ML-based CSI compression using two-sided model in Release 19, adopt the CSI feedback overhead rate as a baseline metric, where the CSI feedback overhead rate is the average bit-rate of CSI feedback overhead per unit across time.

#### Proposal 2.2.5 v2

For the evaluation of temporal domain aspects of AI/ML-based CSI compression using two-sided model in Release 19, adopt the following evaluation assumptions:

- CSI-RS configuration

  - Periodic: 5 ms periodicity (baseline), 20 ms periodicity (optional)

  - Aperiodic (for cases with prediction): CSI-RS burst with K resources and time interval m milliseconds (based on R18 MIMO eType-II)

    - FFS: Value of K and m.

- CSI reporting periodicity: {5, 10, 20} ms

- For cases with the use of past CSI information, companies to report observation window, including number/time distance of historic CSI/channel measurements.

- For cases with prediction, companies to report prediction window, including number/time distance of predicted CSI/channel.

#### Proposal 2.2.6 v2

For the evaluation of temporal domain aspects of AI/ML-based CSI compression using two-sided model in Release 19, for cases with prediction of future CSI, companies may optionally evaluate a scheme with ideal prediction as an additional evaluation case for reference.

Note: The ideal prediction scheme should still model realistic channel estimation.

#### Proposal 3.2.1 v2

For the evaluation of AI/ML-based CSI compression using localized models in Release 19, study the following aspects of the performance/complexity trade-off when comparing the localized model with a benchmark model that is not localized:

- Performance of the localized model that has similar or lower complexity as the benchmark model.

- Model complexity of the localized model that achieves similar or better performance as the benchmark model.

#### Proposal 3.2.2 v2

For the evaluation of AI/ML-based CSI compression using localized models in Release 19, consider the following options as a starting point to model the spatial consistency/correlation in the dataset for a local region:

- Option 1: The dataset is derived from UEs dropped within the local region, with spatial consistency modelling as per TR 38.901.

    - E.g., Dropped in a specific cell or within a specific boundary.

- Option 2: By using a scenario/configuration specific to the local region.

    - E.g., Indoor-outdoor ratio, LOS-NLOS ratio, TXRU mapping, etc.

- Option 3: The dataset is derived using ray-tracing model.

    - E.g., Based on a scenario map.

- Option 4: The dataset is derived from field data collection within a local region.

## Proposals for Wednesday February 28

#### Proposal 3.2.2 v2

For the evaluation of AI/ML-based CSI compression using localized models in Release 19, consider the following options as a starting point to model the spatial consistency/correlation in the dataset for a local region:

- Option 1: The dataset is derived from UEs dropped within the local region, with spatial consistency modelling as per TR 38.901.

    - E.g., Dropped in a specific cell or within a specific boundary.

- Option 2: By using a scenario/configuration specific to the local region.

    - E.g., Indoor-outdoor ratio, LOS-NLOS ratio, TXRU mapping, etc.

- Option 3: The dataset is derived using ray-tracing model.

    - E.g., Based on a scenario map.

- Option 4: The dataset is derived from field data collection within a local region.

#### Proposal 3.2.1 v2

For the evaluation of AI/ML-based CSI compression using localized models in Release 19, study the following aspects of the performance/complexity trade-off when comparing the localized model with a benchmark model that is not localized:

- Performance of the localized model that has similar or lower complexity as the benchmark model.

- Model complexity of the localized model that achieves similar or better performance as the benchmark model.

#### Proposal 2.2.5 v2

For the evaluation of temporal domain aspects of AI/ML-based CSI compression using two-sided model in Release 19, adopt the following evaluation assumptions:

- CSI-RS configuration

  - Periodic: 5 ms periodicity (baseline)

    - FFS: 20 ms periodicity

  - Aperiodic (for cases with prediction): CSI-RS burst with K resources and time interval m milliseconds (based on R18 MIMO eType-II)  (optional)

    - FFS: Value of K and m.

- CSI reporting periodicity: {5, 10, 20} ms; other values are not precluded

- For cases with the use of past CSI information, companies to report observation window, including number/time distance of historic CSI/channel measurements.

- For cases with prediction, companies to report prediction window, including number/time distance of predicted CSI/channel.

#### Proposal 4.2.1 v2 (updated version)

To resolve the issues related to inter-vendor training collaboration complexity, study the following options:

- Option 1: Fully standardized reference model (structure + parameters)

- Option 2: Standardized dataset

- Option 3: Standardized reference model structure

  - Option 3a: Parameters are registered by NW-side/UE-side at a central registry and retrieved by UE-side/NW-side from the registry using standardized procedure.

  - Option 3b: Parameters are transferred between NW-side/UE-side servers using standardized signaling.

  - Option 3c: Parameters are transferred over-the-air using standardized air-interface signaling.

- Option 4: Standardized data format

  - Option 4a: Dataset registered by NW-side/UE-side at a central registry and retrieved by UE-side/NW-side from the registry using standardized procedure.

  - Option 4b: Dataset delivery between NW-side/UE-side servers using standardized signaling.

  - Option 4c: Dataset delivery over-the-air using standardized air-interface signaling.

- Option 5: Standardized model format

  - Option 5a: Reference model registered by NW-side/UE-side at a central registry and retrieved by UE-side/NW-side from the registry using standardized procedure.

  - Option 5b: Reference model transferred between NW-side/UE-side servers using standardized signaling.

Note 1: The above options may not be mutually exclusive and may be used together.

Note 2: Other options are not precluded.

Note 3: Registration / retrieval happens through a central registry and does not require direct collaboration between vendors.

#### Proposal 4.2.1 v2 (previous version)

To resolve the issues related to inter-vendor training collaboration complexity, study the following options:

- Option 1: Fully standardized reference model (structure + parameters)

  - Option 1a: CSI generation part

  - Option 1b: CSI reconstruction part

  - Option 1c: Both

- Option 2: Standardized dataset

- Option 3: Standardized reference model structure

  - Option 3a: Parameters are registered by NW-side/UE-side at a central registry and retrieved by UE-side/NW-side from the registry using standardized procedure.

    - Option 3a-1: CSI generation part registered by NW-side, retrieved by UE-side.

    - Option 3a-2: CSI reconstruction part registered by UE-side, retrieved by NW-side.

  - Option 3b: Parameters are transferred between NW-side/UE-side servers using standardized signaling.

    - Option 3b-1: CSI generation part parameters from NW-side server to UE-side server.

    - Option 3b-2: CSI reconstruction part parameters from UE-side server to NW-side server.

  - Option 3c: Parameters are transferred over-the-air using standardized air-interface signaling.

    - Option 3c-1: Model transfer/delivery of CSI generation part from NW to UE.

    - Option 3c-2: Model transfer/delivery of CSI reconstruction part from UE to NW.

- Option 4: Standardized data format

  - Option 4a: Dataset registered by NW-side/UE-side at a central registry and retrieved by UE-side/NW-side from the registry using standardized procedure.

  - Option 4b: Dataset delivery between NW-side/UE-side servers using standardized signaling.

  - Option 4c: Dataset delivery over-the-air using standardized air-interface signaling.

- Option 5: Standardized model format

  - Option 5a: Reference model registered by NW-side/UE-side at a central registry and retrieved by UE-side/NW-side from the registry using standardized procedure.

    - Option 5a-1: CSI generation part or CSI reconstruction part registered by NW-side, retrieved by UE-side.

    - Option 5a-2: CSI generation part or CSI reconstruction part registered by UE-side, retrieved by NW-side.

  - Option 5b: Reference model transferred between NW-side/UE-side servers using standardized signaling.

    - Option 5b-1: CSI generation part or CSI reconstruction part from NW-side server to UE-side server.

    - Option 5b-2: CSI generation part or CSI reconstruction part from UE-side server to NW-side server.

Note 1: The above options may not be mutually exclusive and may be used together.

Note 2: Other options are not precluded.

Note 3: Registration / retrieval happens through a central registry and does not require direct collaboration between vendors.

#### Proposal 2.2.4 v3

For the evaluation of temporal domain aspects of AI/ML-based CSI compression using two-sided model in Release 19, adopt the CSI feedback overhead rate as a baseline metric, where the CSI feedback overhead rate is the average bit-rate of CSI feedback overhead per unit across time.

Also adopt the rate of UCI transmission occasions for CSI report as a baseline metric, where rate of UCI transmission occasions for CSI report is the average number of UCIs for CSI report per time unit.

Note: The CSI feedback overhead of a single report is calculated as in R18 CSI compression study.

#### Proposal 2.2.6 v2

For the evaluation of temporal domain aspects of AI/ML-based CSI compression using two-sided model in Release 19, for cases with prediction of future CSI, companies may optionally evaluate a scheme with ideal prediction as an additional evaluation case for reference.

Note: The ideal prediction scheme should still model realistic channel estimation.

## Proposals for Thursday February 29

#### Proposal 4.2.1 v3

To alleviate / resolve the issues related to inter-vendor training collaboration complexity of AI/ML-based CSI compression using two-sided model, study the following options:

- Option 1: Fully standardized reference model (structure + parameters)

- Option 2: Standardized dataset

- Option 3: Standardized reference model structure + Parameter exchange between NW-side and UE-side

- Option 4: Standardized data / dataset format + Dataset exchange between NW-side and UE-side

- Option 5: Standardized model format + Reference model exchange between NW-side and UE-side

Note 1: The above options may not be mutually exclusive and may be used together.

Note 2: Other options are not precluded.

Note 3: The study should consider how different delivery methods of exchanging the parameters / dataset / reference model would affect the feasibility and collaboration complexity of options 3 / 4 / 5 respectively, e.g., over the air-interface, within the core-network, via a central repository, offline delivery, etc.

Note 4: “Dataset” refers to a set of data samples of CSI feedback and associated target CSI.

#### Proposal 3.2.2 v3

For the evaluation of AI/ML-based CSI compression using localized models in Release 19, consider the following options as a starting point to model the spatial consistency/correlation in the dataset for a local region:

- Option 1: The dataset is derived from UEs dropped within the local region, with spatial consistency modelling as per TR 38.901.

    - E.g., Dropped in a specific cell or within a specific boundary.

    - Note: While modelling the spatial correlation, companies should ensure that the dataset distribution also correctly captures the decorrelation due to temporal variations in the channel.

- Option 2: By using a scenario/configuration specific to the local region.

    - E.g., Indoor-outdoor ratio, LOS-NLOS ratio, TXRU mapping, etc.

- Option 3: The dataset is derived using ray-tracing model.

    - E.g., Based on a scenario map.

- Option 4: The dataset is derived from field data collection within a local region.

Note: While modelling the spatial correlation, how to ensure that the dataset distribution also correctly captures the decorrelation due to temporal variations in the channel is up to companies.

#### Proposal 2.2.4 v3

For the evaluation of temporal domain aspects of AI/ML-based CSI compression using two-sided model in Release 19, adopt the CSI feedback overhead rate as a baseline metric, where the CSI feedback overhead rate is the average bit-rate of CSI feedback overhead per unit across time.

Also adopt the rate of UCI transmission occasions for CSI report as a baseline metric, where rate of UCI transmission occasions for CSI report is the average number of UCIs for CSI report per time unit.

Note: The CSI feedback overhead of a single report is calculated as in R18 CSI compression study.

#### Proposal 2.2.6 v2

For the evaluation of temporal domain aspects of AI/ML-based CSI compression using two-sided model in Release 19, for cases with prediction of future CSI, companies may optionally evaluate a scheme with ideal prediction as an additional evaluation case for reference.

Note: The ideal prediction scheme should still model realistic channel estimation.

#### Proposal 2.4.1

For the evaluation of temporal domain aspects of AI/ML-based CSI compression using two-sided model in Release 19, for Case 2, Case 4 and Case 5, study the performance impact resulting from UCI loss.

#### Proposal 4.4.2

For the study of inter-vendor collaboration issues for AI/ML-based CSI compression using a two-sided model, consider at least the following aspects when comparing different options:

- Inter-vendor collaboration complexity, i.e., whether bilateral collaboration is required between vendors.

- Performance gain over benchmark scheme.

- Implications for minimum performance guarantee, i.e., RAN4 / testing related aspects.

- Device feasibility aspects.

# FL summary

In the RAN1#116 meeting, progress has been made on multiple topics:

- Temporal domain aspects of CSI compression

  - The use cases have been categorized into 5 cases defined based on whether the UE and/or the network use past CSI information, and whether the target CSI is for a present slot or future slot(s).

  - Evaluation scenarios, the benchmark scheme for performance comparison, evaluation metrics, and other evaluation assumptions have been identified for the study of this topic in Release 19.

  - Other aspects such as evaluation of an ideal prediction scheme for reference, and the impact of non-ideal UCI feedback on performance were also discussed and agreed.

- Localized models

  - Different options and aspects were discussed related to the question of how to model the spatial correlation within the dataset for a local region. Based on the discussion, options have been identified to be used as a starting point for the evaluation study.

  - Performance and model complexity were identified as aspects to study when evaluating the performance benefits of localized models.

- Inter-vendor collaboration

  - To alleviate the concerns related to inter-vendor collaboration, several options have now been identified for companies to study. The options cover a wide range of possibilities in terms of what aspects are standardized and how much flexibility is left for implementation. For example, what is standardized could be a reference model, a dataset, a reference model structure, a data / dataset format, or a model format. (The term reference model means a model that serves as a reference implementation of a functional relationship. The actual model may use a different implementation to implement the functional relationship.)

  - Some of the identified options require the exchange of parameters, dataset, or reference model between the NW-side and UE-side. Different examples of the method for such an exchange were discussed, including exchange over the air-interface, exchange between entities within the core network, and exchange via a centralized registry or repository.

  - Based on the discussion, it was noted that the study of the options should consider the method of exchanging the parameters / dataset / reference model, e.g, over the air-interface, offline delivery, etc. and how the choice of the method affects the feasibility and collaboration complexity of the overall training collaboration process.

  - Some aspects to consider when comparing the different collaboration options were identified: collaboration complexity, performance, interoperability / testing, and feasibility.

For the next meeting, discussion on these topics is expected to continue.

Regarding the temporal domain aspects of CSI compression, companies are encouraged to evaluate the different agreed cases to better understand the potential benefit for an AI/ML based approach in each case. Companies are also requested to provide inputs and views on the evaluation results template to capture the results.

For the topic of localized models, companies are encouraged to evaluate the performance and model complexity benefits for the localized model approach relative to an approach that does not use a localized model, using the identified options for modelling spatial correlation.

Regarding inter-vendor collaboration aspects, based on the categorization of options to study, companies are encouraged to analyse the differences between the options in terms of the identified aspects (complexity, performance, interoperability/testing and feasibility), considering the method of exchanging the parameters / dataset / reference model, if applicable. Please also consider the question of whether there is a need to identify sub-categories within each option. If different flavours within the same option have very different pros and cons, then it may be better to discuss them separately than to mix them into one case.

# References

- TR 38.843 v18.0.0, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface” (Release 18), December 2023.

- RP-234039, “New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, Qualcomm (Moderator), 3GPP TSG RAN #102, December 2023.

- R1-2400047	Disscussion on AIML for CSI compression	Spreadtrum Communications, BUPT

- R1-2400095	Discussion on potential performance enhancements/overhead reduction with AI/ML-based CSI feedback compression	FUTUREWEI

- R1-2400150	Discussion on CSI compression for AI/ML	Huawei, HiSilicon

- R1-2400166	AI/ML for CSI compression	Ericsson

- R1-2400235	Discussion on CSI compression	vivo

- R1-2400265	Discussion on study for AI/ML CSI compression	ZTE

- R1-2400319	Discussion on AI/ML for CSI compression	CMCC

- R1-2400379	AI/ML for CSI prediction	Intel Corporation

- R1-2400395	AI/ML based CSI Compression	Google

- R1-2400421	Study on AI/ML-based CSI compression	CATT

- R1-2400464	Discussion on CSI compression	NEC

- R1-2400501	Discussion on AI/ML for CSI compression	Comba

- R1-2400511	Discussions on the remaining issues for other aspects of AI/ML for CSI compression	TCL

- R1-2400546	Discussion on two-sided AI/ML model based CSI compression	xiaomi

- R1-2400621	Additional study on AI/ML-based CSI compression	OPPO

- R1-2400653	Discussion on CSI compression for AI/ML	BJTU

- R1-2400657	Discussion on AI/ML-based CSI compression	China Telecom

- R1-2400695	Addtional study on AI-enabled CSI compression	 NVIDIA

- R1-2400723	Discussion for further study on AI/ML-based CSI compression	Samsung

- R1-2400769	Discussion on CSI compression with AI/ML	Fujitsu

- R1-2400796	AI/ML for CSI Compression	Nokia, Nokia Shanghai Bell

- R1-2400833	On AI/ML for CSI compression	Lenovo

- R1-2400847	Discussions on CSI compression	Sony

- R1-2400909	Discussion on AI/ML-based CSI compression	InterDigital, Inc.

- R1-2400916	Study on CSI compression	LG Electronics

- R1-2401005	Discussion on AI based CSI compression	Apple

- R1-2401037	Discussion on AI/ML for CSI compression	Panasonic

- R1-2401110	Discussion on the AI/ML for CSI compression	NTT DOCOMO, INC.

- R1-2401135	Discussions on AI/ML for CSI feedback	CAICT

- R1-2401152	Discussion on study of AI/ML for CSI compression	IIT Kanpur, Indian Institute of Tech (M)

- R1-2401155	Discussion on Additional Study of AI/ML for CSI Compression	Indian Institute of Tech (M), IIT Kanpur

- R1-2401174	Discussions on CSI compression	Sharp

- R1-2401224	Discussion on AI/ML for CSI compression	ETRI

- R1-2401242	Discussion on AI/ML for CSI Compression	Fraunhofer IIS, Fraunhofer HHI

- R1-2401270	Discussion on  AI/ML for CSI Compression	CEWiT

- R1-2401304	CSI Compression	MediaTek Inc.

- R1-2401339	Discussion on AI/ML based CSI compression	ITL

- R1-2401434	Additional study on CSI compression	Qualcomm Incorporated

- R1-2401478	Discussion on AI/ML-based CSI compression	NTPU