3GPP TSG RAN WG1 #116			R1-2401596

Athens, Greece, February 26th – March 1st, 2024

Agenda item:	9.1.1

Source:	Samsung (Moderator)

Title:	FL summary #0 for AI/ML in management

Document for:	Discussion and Decision

- Introduction

In RAN#102, Rel-19 work item on “New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface” is endorsed. The objective of the work item is as follows.

Provide specification support for the following aspects: - Beam management - DL Tx beam prediction for both UE-sided model and NW-sided model, encompassing [RAN1/RAN2]: - Spatial-domain DL Tx beam prediction for Set A of beams based on measurement results of Set B of beams (“BM-Case1”) - Temporal DL Tx beam prediction for Set A of beams based on the historic measurement results of Set B of beams (“BM-Case2”) - Specify necessary signalling/mechanism(s) to facilitate LCM operations specific to the Beam Management use cases, if any - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE NOTE: Strive for common framework design to support both BM-Case1 and BM-Case2

In this contribution, summarized the contributions in RAN 1 #116 on AI/ML for beam management.

Question 0

- Please consider entering contact info below for the points of contact for this email discussion.

| Company | Contact | Email address |
| --- | --- | --- |
| New H3C | Lei Zhou | Zhou.leih@h3c.com |
| Indian Institute of Technology Madras | Sai Prasad Pirati | venkatasiva@5gtbiitm.in |
| OPPO | Jeffrey (Jianfei) Cao | caojianfei@oppo.com |
| Ericsson | Henrik Ryden | Henrik.a.ryden@ericsson.com |
| Fujitsu | WANG Guotong (David) | wangguotong@fujitsu.com |
| MediaTek | Yu-Jen Ku | yu-jen.ku@mediatek.com |
| TCL | Pu Yuan | pu.yuan@tcl.com |
| CMCC | Jiazhen Zhang | zhangjiazhen@chinamobile.com |
| NTT DOCOMO | Haruhi Echigo | haruhi.echigo.fw@nttdocomo.com |
| InterDigital | Youngwoo Kwak | youngwoo.kwak@interdigital.com |
| vivo | Hao Wu | hao.wu@vivo.com |
| NEC | Peng Guan Pravjyot Deogun | Guan_peng@nec.cn pravjyot.deogun@EMEA.NEC.COM |
| ETRI | Yongjin Kwon | yjkwon@etri.re.kr |
| Xiaomi | Mingju | limingju@xiaomi.com |
| ZTE | Wenfeng LIU Xingguang WEI | liu.wenfeng@zte.com.cn wei.xingguang@zte.com.cn |
| Qualcomm | Hamed Pezeshki | hamedp@qti.qualcomm.com |
| Spreadtrum | Shijia Shao | Shijia.shao@unisoc.com |
| Panasonic | Henry Tran | xuantuong.tran@sg.panasonic.com |
| CATT | Min Zhu | zhumin@catt.cn |  |
| CEWiT | Ebin Chacko Shiv Shankar | echacko@cewit.org.in shiv@cewit.org.in |  |
| Google | Yushu Zhang | yushuzhang@google.com |  |
| Sharp | Liqing Liu | liu.liqing@sharp.co.jp |  |
| Futurewei | Zhigang Rong | zrong@futurewei.com |  |
| LG | Seongwon Go | sw.go@lge.com |  |
| Lenovo | Bingchao Liu | Liubc2@lenovo.com |  |
| Fraunhofer HHI | Baris Göktepe | Baris.goektepe@hhi.fraunhofer.de |  |
| KDDI | Taishi Watanabe | ta-watanabe@kddi.com |  |
| NVIDIA | Xingqin Lin | xingqinl@nvidia.com |  |

- Use case

| Company | Proposals |
| --- | --- |
| Spreadtrum [1] | Proposal 5：For DL Tx beam prediction, no additional spec enhancements are required to ensure the common understanding between NW and UE on Rx beam assumption. |
| Ericsson [3] | Proposal 15 For UE-sided models, regarding set A/B, standardize a fixed set B/A over training and inference. Hence, down prioritize a variable set B. |
| TCL[6] | Proposal 1: Measurement patterns for AI/ML beam management should be designed for spatial domain beam prediction, and should consider both the fixed regular and random patterns. |
| Xiaomi [12] | Proposal 4-1: Support Tx Beam ID(s) and/or the predicted L1-RSRP of the N predicted DL Tx beams as the AI/ML model output with high priority. |
| OPPO [13] | Proposal 1: For BM-Case1, configure Set B as a subset of Set A during training and inference phase. Proposal 2: For BM-Case2, configure the same Set B and Set A during training and inference phase. Proposal 3: For UE-side model, support NW to configure more than one Set Bs/Set As and then select/activate one pair of associated Set B/Set A. Proposal 4: For BM-Case2, support the configuration of Set A and/or Set B on a per instance basis. Proposal 16: For temporal domain beam prediction, suggest to study and evaluate the beam dwelling time prediction. |
| InterDigital [14] | Proposal 8: Support reporting of UE selected Set B based on a rule (e.g., subset of best measured beams). |
| NVIDIA [15] | Proposal 1: For BM-Case 1, at least introduce specification support for using L1-RSRP measurement based on Set B of beams as AI/ML model input. Proposal 2: For BM-Case 2, at least introduce specification support for using historical optimal beam index and/or L1-RSPR measurement based on Set B of beams as AI/ML model input. |
| Fujitsu [17] | Proposal 30: - The case that Set B is subset of Set A should be prioritized in Rel-19. RAN1 to further discuss whether/how to indicate the beam pattern for Set A and Set B. |
| Nokia [19] | Observation 8: The NW sided AI/ML based DL Tx beam prediction can be integrated to partially replace and/or optimize the legacy P-1 and P-2 procedures for Tx beam management. RAN1 shall identify different options for BM-Case1 and BM-Case2. |
| Rakuten [22] | - Tx and/or Rx Beam angle(s) of Set A beams are defined and predicted with more granularity to enable beamforming of precise and accurate data beams. |
| Hyundai [26] | Proposal #1: - Clarify the meaning of Set A of beams, Set B of beams and Set C of beams. Proposal #2 - Prioritize the cases to be discussed or choose a part of cases to focus on early in Rel-19. |
| FGI, NTPU [27] | - Proposal 1: For BM Case 1 model, whether Set A and Set B can be in different Frequency Ranges needs to be clarified. |
| DoCoMo [28] | Observation 1: In Rel-18, generalization evaluations over a signal factor (e.g., only carrier frequency is variable) were performed, while generalization evaluations over multiple factors (e.g., both carrier frequency and gNB setting are variable), which is closer to actual deployment environments, were not done. Proposal 3: RAN1 should conduct the generalization evaluations over multiple factors (e.g., both carrier frequency and gNB setting are variable) to verify the generalization capability in practical scenarios. |
| CAICT [29] | Proposal 1: The default setting of Set A could be all Tx beam for BM-Case1 and BM-Case2. Proposal 2: Fixed pattern could be chosen as baseline for Set B indication. |
| KT [31] | Proposal 1. Deprioritize discussion on DL Tx beam prediction for NW-sided model. Proposal 2. Support L1-RSRP measurement based on Set B and the corresponding DL Tx beam ID as a baseline of AI/ML input for BM use case. Proposal 3. Assume AI/ML offline training without model transfer for beam management in Rel-19. |
| Sharp [33] | Proposal 1: For BM Case 1, support both Alt.1:Set A and Set B are different and Alt.2: Set B is a subset of Set A. Proposal 2: For BM Case 2, support Alt.2: Set B is a subset of Set A and Alt.3: Set A and Set B are the same. |
| MTK[37] | Proposal 1: Do not consider beam pair prediction in this WI, focus on DL Tx beam prediction for further study. |

Companies clarified some aspects for use cases, e.g., fixed or variable of Set B, Set B is subset of Set A and/or Set B equal to set A, model inputs, e.g. L1-RSRP as inputs. From FL point of view, I don’t see strong need to have agreements for the above aspects for now. Therefore, I suggest to directly discuss on LCM aspects and clarify those assumption during those discussion.

- Data collection for training

This session only discusses data collection for training. Measurement report for inference or performance monitoring will be discussed in section 4 and section 5 respectively.

3.1 NW-sided model

| Company | Proposals |
| --- | --- |
| Spreadtrum [1] | Proposal 2：For data collection at NW-side, option 1, option 2 and option 3 all can be considered. Proposal 3: For data collection for a network-side AI/ML model, the legacy beam sweeping and CSI report mechanism can be reused. Proposal 4: For BM-Case1 and BM-Case2 with a network-side AI/ML model, existing quantitative criteria should be reused at least for model inference. |
| Huawei/HiSi [2] | Proposal 1: For potential enhancements to support AI/ML based beam management during Rel-19, - For NW-side model, the enhancements would be mainly focus on improving the reporting of measured RSRP and/or CRI, while the purpose of how to use the reporting by NW can be transparent to the UE (e.g., training, inference, monitoring, or non-AI/ML). Proposal 3: Regarding data collection for NW-side AI/ML model, it is no urgent to down select Opt.1, Opt.2 and Opt.3 for the contents of collected data. - The decision on the values of M1, M2 and M3 can be deferred until further progress is made on the size of beam set(s) for CSI measurement. Proposal 4: For reporting overhead reduction of NW-side AI/ML model, regarding omission/selection of collected data, discuss the following options: - Opt1: fixed number of beams. - Opt2: adaptive number of beams based on threshold. Proposal 5: For reporting overhead reduction of NW-side AI/ML model, regarding compression of collected data, this option requires further justification before being confirmed. - Why for BM-Case 1 (RSRPs to be reported are from the same time instance), legacy differential RSRP does not seem sufficient? - For BM-Case 2, why RSRPs to be obtained across time instances should be reported in one instance (with potential compression over temporal domain), considering the larger latency it would inflict. |
| Ericsson [3] | Observation 2 For BM-case2, setA is equal to setB in some of the evaluated scenarios, hence the setB reporting should be able to support setA number of beams in the UE report for NW-sided model inference Proposal 7 For NW-sided training data collection, standardize Options 1,2,3 Proposal 8 For NW-sided training data collection via higher-layer signalling, RAN1 to consider UE logged measurement report using RRC (details upto RAN2) Proposal 9 For NW-sided data collection, standardize “omission/selection of collected data” by the following aspects as a starting point, • UE to remove “duplicated” samples, • UE can remove samples based on certain events, for example that the UE experienced large channel variation during set A measurements, |
| Vivo [4] | Proposal 6: Regarding NW-side data collection to trade off training performance and RS overhead, support P3+P2 resource configuration, e.g., • Perform P3 UE Rx beam search based on multiple Tx beams, which can be configured within a time interval limitation for P2 resource set receiving for set B and Set A measurement result acquisition. Proposal 7: Regarding NW-side data collection for useless beam omission, support below methods on beam report enhancement for overhead reduction: • Support to reduce unnecessary beam indices and/or corresponding L1-RSRPs in a report where the omitted L1-RSRP of beams may be smaller than an extra threshold. • Support to report partial measured L1-RSRPs and beam indices with beam index type indicator. Proposal 8: Regarding NW-side data collection on quantization aspects for overhead reduction, • Support to enlarge quantization steps for differential beams. • Multi-resolution L1-RSRP quantization, e.g. high-resolution quantization for a group of best RSRPs and low-resolution quantization for others |
| ZTE [5] | - Proposal 3: For the contents of collected data from UE to NW, all three options can be supported to serve different LCM operations at the NW side. Opt.1: M1 L1-RSRPs (corresponding to M1 beams) with the indication of beams based on the measurement corresponding to a beam set, where M1 can be larger than 4, if applicable Opt.2: M2 L1-RSRPs (corresponding to M2 beams) based on the measurement corresponding to a beam set, where M2 can be larger than 4, if applicable Opt.3: M3 beam indices based on the measurement corresponding to a beam set, where M3 can be larger than 4, if applicable - Proposal 4: Regarding measurement results report, If measurement results of all beams in a measured beam set are to be reported, support enhanced reporting methods for the purpose of reporting overhead reduction, e.g., beam ID can be obtained implicitly from the reporting order of all measured RSRPs. If measurement results of partial beams in a measured beam set are to be reported, support enhanced method (e.g., bitmap) for the indication of beam ID in UE reporting. - Proposal 5: One or two RS resource sets for beam measurement can be configured to the UE for data collection of NW-side AI/ML model, which are dependent on the beam set construction of Set A and Set B. - Proposal 6: For the collection of both model input and model label data at the NW side, one RS resource set can be configured to the UE with indication of necessary assistance information, such as mapping of Set A and Set B. - Proposal 7: Regarding data collection for NW-side AI/ML model of BM-Case1 and BM-Case2, support specification enhancements for overhead reduction from the following aspects: Data omission among samples (e.g., according to data quality) - • Data selection per sample (e.g., based on threshold criteria) |
| CMCC [7] | Proposal 1: Regarding the data collection for NW-side model of BM-Case1 and BM-Case2, one shot measurement of beams in each sample is preferred compared with periodic measurement, especially for BM-Case2. Proposal 2: Regarding data collection for NW-side model of BM-Case1 and BM-Case2, UE capability on maximum number of RS for RSRP measurement and CSI reporting framework need enhancement to support unified beam sweeping within a short period. Proposal 3: Regarding signaling of data collection for training, L1 signaling or higher-layer signaling is considered. Regarding signaling of data collection for inference and monitoring, L1 signaling is supported. Proposal 4: Regarding data collection for NW-side model of BM-Case1 and BM-Case2, support the enhancement of L1 reporting of more than 4 beams. Proposal 5: Regarding data collection for NW-side model of BM-Case1 and BM-Case2, consider the specification impact of L1 reporting overhead reduction from the following aspects: · the omission of collected data · option 1: fixed beam number per sample · option 2: fixed RSRP threshold per sample · the compression of data content · new quantization method · temporal compression among multiple past time instances |
| Intel [8] | Proposal 11: For data collection for a network-side model, further discuss the benefits of the UE reporting Rx beam assumption related information for L1 measurement reporting corresponding to beams in set A. Proposal 12: RAN1 should further discuss if and how the same UE Rx beam assumption can be maintained for measurement of set A for data collection for model training and for measurement of set B for model input of the corresponding model. |
| Google [9] | Proposal 2: For beam report for NW model training for SD beam prediction, UE reports the following information: - SSBRI/CRI with the best L1-RSRP measured - L1-RSRP for a subset of SSBs/CSI-RSs configured by the NW - Hypothetical measurement error for the subset of SSBs/CSI-RSs with L1-RSRP reported Proposal 3: For beam report for NW model inference for SD beam prediction, UE reports the following information: - L1-RSRP for a set of SSBs/CSI-RSs configured by the NW - Hypothetical measurement error for the subset of SSBs/CSI-RSs with L1-RSRP reported Proposal 11: For beam report for NW model training for temporal beam prediction, UE reports the following information: - SSBRI/CRI with the best L1-RSRP measured for each of the measured slots configured by the NW - L1-RSRP for a subset of SSBs/CSI-RSs configured by the NW for each of the measured slots configured by the NW - Hypothetical measurement error for the subset of SSBs/CSI-RSs with L1-RSRP reported |
| CATT [10] | Proposal 1: Regarding NW-side data collection, consider the following options for reporting overhead reduction: - Alt.1: Reporting Top-N L1-RSRP values or L1-RSRP values larger than a threshold - Alt.2: Omission the index of RS, e.g., reporting the L1-RSRP in order - Alt.3: Larger quantization step for reporting differential L1-RSRP - Alt.4: Reporting L1-RSRP of multiple time instances in one reporting |
| NEC [11] | Proposal 11: Study overhead reduction mechanisms for beam reports supporting both BM-Case1 and BM-Case2. |
| Xiaomi [12] | Proposal 3-1: Report collected data for NW-side model training by RRC signaling. Proposal 3-2: For data collection of NW-side AI/ML model training, support to define a time window or a data size for each report with more than one data sample. Proposal 3-3: For data collection of NW-side AI/ML model training, support UE to indicate the RS IDs whose L1-RSRP are not reported because of lower than threshold to reduce overhead. Proposal 3-4: Exchange the UE-side additional condition such as Rx beam assumption and UE speed during the procedure of data collection for NW-side AI/ML model training. |
| OPPO [13] | Proposal 21: For NW-side model training, UE reports M2 L1-RSRPs measurements of fixed Set B (via Opt.2) to NW. Proposal 22: For NW-side model training, UE reports Top-K L1-RSRP(s) and its associated Tx beam index(es) to NW (via Opt.1). Proposal 23: For BM-Case2, the temporal domain information of collected data could be reported to NW in an implicit manner (no time stamps) by enhancing reporting format. |
| Samsung [16] | Proposal 1. For NW-side AI/ML model data collection for training, support at least the following as data collection content: - L1-RSRP(s) for all beam(s) of Set B - L1-RSRP(s) for all beam(s) of Set A - Top-K Beam ID(s) for Set A - Related timestamp - The information to facilitate model training (FFS details) Proposal 2. For NW-side AI/ML model data collection for training, at least support the enhancement to use high layer signaling to convey data collection content. |
| Fujitsu [17] | Proposal 1: - For training data collection, L3 signaling is preferred for the reporting. Proposal 2: - For training data collection, at least the following information should be included: - Reference signal ID - Beam quality, e.g., L1-RSRP Proposal 3: - For training data collection, the model input data and the ground truth data should be included. Whether and how to map/associate the model input data with the ground truth data could be further discussed per sub-use case for beam management. Proposal 4: - For training data collection, RAN1 to further discuss the quantization for the ground truth data. Proposal 5: - For training data collection, the reference signals same as Set B should be configured to obtain the model input data. And the reference signals same as Set A should be configured to obtain the ground truth data. Proposal 6: - RAN1 to further discuss whether the same or separate reference signal configuration between training data collection and model inference. Proposal 7: - Regarding training data collection, RAN1 to further discuss the reference signal configuration for different sub-use cases. Proposal 8: - If multiple beam patterns for beam management with AI/ML are considered in Rel-19, then the model input data and the ground truth data should also be associated with beam pattern. |
| Nokia [19] | Observation 12: NW-sided data collection should be made taking into account the existing beam measurement and reporting frameworks. Enhancements discussed for reporting beam measurements should be considered when discussing the data collection aspects. |
| Lenovo [20] | Proposal 2: Study beam report with larger number of beams in L1 as well as beam report in higher layers. Proposal 3: Support procedures that enable a UE to transmit a subset of the set of measured/collected samples from the environment (e.g., more informative samples among all samples). |
| LGE [23] | Proposal #7: For NW-sided AI/ML in temporal DL Tx beam prediction, support the following UE reporting enhancements: - Past/present best N beam(s) per time stamp - Tendency/variance of best N beam(s) |
| Apple [24] | Proposal 3-2: MDT framework can be leveraged for training data collection for NW-side model. Observation 4-1: weak beams’ RSRPs can be omitted in the beam reporting for overhead reduction. |
| DoCoMo [28] | Proposal 2: Discuss how to handle measurement sensitivity issue in the measurements of Set B/C. Proposal 12: Support L1 signaling data collection for inference purpose in NW side beam prediction. Proposal 13: Consider enhancements on data collection for inference in NW side temporal beam prediction, such as the reduction on the number of signaling and payload overhead reduction. |
| CAICT [29] | Proposal 5: gNB initiated data collection for gNB-sided model training should be specified for BM-Case1 and BM-Case2. |
| Indian Institute of Technology Madras (IITM), IIT Kanpur [31] | Proposal 4: For NW-side model training, NW transmits a measurement configuration to the UE to which the UE responds. NW trains the model using UE measurements. |
| CEWiT [36] | Proposal 5: For data collection for a NW sided AI/ML model, to reduce the overhead caused by reporting more than 4 L1-RSRP values, consider approaches including removing unnecessary fields in measurement report and not including L1-RSRP values very close to each other or close to the noise floor. Proposal 6: For data collection for a NW sided AI/ML model, consider approaches to ensure common understanding between NW and UE on the Rx beam assumptions. |
| MTK [37] | Proposal 5: For Opt.1 of the content of collected data for NW-side model, consider the following cases between the reported RSRP and RI of beams - The beams of the reported M1 L1-RSRPs are the same as the beams of the reported RIs, where RIs can be discard in the report. - The beams of the reported M1 L1-RSRPs are not the same as the beams of the RI, where RIs cannot be discard in the report. - M1 and the number of the indication of beams can be different Proposal 6: For the content of collected data for model inference of NW-side model, consider NW configures only Set B of beams for measurement and UE doesn’t report the RI for all the Set B of beams. Proposal 7: For the content of collected data for model training of NW-side model, consider UE reports only the L1-RSRP measurement of Set B of beams in the CSI report L1-RSRP field while reporting the label (i.e., the RIs within Set A of beams which have the best L1-RSRP measurements) in the CSI report RI field, without reporting the RI of all the measured Set B and/or Set A of beams. Proposal 8: For the content of collected data for NW-side model, further study defining the value of M1, M2, and M3 based on the following factors, - Set A and Set B sizes - data collection purposes - model output design - overhead reduction strategies Proposal 9: For UE to report the measurement results of more than 4 beams, consider using the Rel-18 report sub-configuration framework Proposal 10: For the data collection for NW-side model inference/training/monitoring for BM-Case2, consider the following two options of UE reporting as a starting point, - Option1: NW configures separate reports for each time instance in the observation window - Option2: NW configures one report for all the time instances in the observation window Proposal 11: For data collection for NW-side AI/ML model, consider reporting overhead reduction with quantizing L1-RSRP and/or normalized L1-RSRP measurement with lower number of bits than the current spec. |

3.2 UE-sided model

No explicit proposal for data collection for training specific in this meeting, considering the on-going discussion as SI phase in RAN 1 and RAN 2.. Some proposals may also apply for inference, e.g., request RS to obtain measurement of Set A and/or Set B of beams.

| Company | Proposals |
| --- | --- |
| Spreadtrum [1] | Proposal 1：For UE-side model, support UE to request the data collection and report training-related information, such as expected measurement resources, etc. |
| Huawei/HiSi[2] | Proposal 1: For potential enhancements to support AI/ML based beam management during Rel-19, - For UE-side model, it may be needed to let the UE know the purpose of reports, since the UE behavior or the content of the UE report may be different depending on if the measurement is for training, inference or monitoring. Proposal 6: For the data collection for the UE-side model under BM-Case 1/BM-Case 2, for a specific Set B which the UE measures, the UE can be indicated with the associated Set A of the local cell, e.g. - The associated Set A ID can be indicated in the CSI-reportConfig of Set B Proposal 9: For measurements configured to facilitate the AI/ML operations of a UE-side model, the purpose or implied UE behavior of the measurement configuration (e.g. training, inference, monitoring, non-AI/ML) needs to be indicated to the UE. E.g.: - Purpose training, implied behavior - no report needed. - Purpose inference, implied behaviors - report the predicted beams/RSRPs. - Purpose monitoring, implied behaviors - report the label, predicted output or a metric. - Purpose non-AI/ML, implied behaviors - report the measured beams/RSRPs. |
| Vivo[4] | Proposal 9: Regarding UE-side data collection, support UE to report preferred configurations of DL RS transmission, e.g., • Minimum number of beam resource in Set B • Minimum number of resource repetitions for quasi-Rx beam acquisition Proposal 10: Regarding UE-side data collection to trade off training performance and RS overhead, support P3+P2 resource configuration, e.g., • Perform P3 UE Rx beam search based on multiple Tx beams, which can be configured within a time interval limitation for P2 resource set receiving for set B and Set A measurement result acquisition. Proposal 11: Regarding UE-side data collection, support to configure dataset ID or virtual beam information from NW to UE as NW-side assistance information to address NW-side proprietary/privacy information disclosing issue. |
| CMCC[7] | Proposal 6: Regarding data collection for UE-side model of BM-Case1 and BM-Case2, UE capability on maximum number of RS for RSRP measurement and CSI reporting framework need enhancement to support unified beam sweeping within a short period. Proposal 7: Regarding data collection for UE-side model, down-select the indication of association/mapping of beams within Set A and beams within Set B from the following options: · Opt1: QCL information between beams within Set A and beams within Set B · Opt2: Identifier of info representing the association/mapping of Set A and Set B · Opt3: The bitmap or pre-defined rule that beams of Set B is subset of beams of Set A |
| Intel [8] | Proposal 13: For data collection for a UE-side AI/ML model, consider UE triggering for data collection from the network based on a configured set A of beams. |
| CATT[10] | Proposal 2：For UE side data collection, consider the following options for RS overhead reduction in network: - Option 1: NW sends common RS configuration to different UEs for UE-side data collection - Option 2: UE requests the preferred RS for data collection from NW pre-configured RS |
| NEC [11] | Proposal 7: For triggering/initiating data collection at UE side for UE-side AI/ML model, - Option 1: data collection initiated/triggered by configuration from NW is preferred, and - Option 2: request from UE for data collection can serve as a supplementary. Moreover, Option 2 may need additional clarification on the steps that follow after request initiated by the UE. Proposal 8: For avoiding the proprietary/privacy issue, study to provide the assistance information (e.g., angle related information, channel estimation based information) implicitly from one side to the other side. |
| OPPO [13] | Observation 9: For BM-Case1 and BM-Case2 with UE-side model training, data set including model inputs and labels can be collected by UE via legacy approach. Proposal 20: For UE-side model training, it is up to UE to request the data collection followed by configuration of Set A and/or Set B from NW. |
| Futurewei [18] | Proposal 2: For Rel-19 AI/ML-based BM, for data collection for UE-side AI/ML model of BM-Case1 and BM-Case2, support that NW provides/signals multiple possible configurations of DL RS transmission to the UE and the UE reports one out of the multiple configurations as its supported/preferred one. |
| Nokia [19] | Observation 4: Enhancements related to UE data collection should be made taking into account the legacy beam measurement and reporting frameworks. Considerations on signalling aspects related to assistance information can be discussed under the section related to information and/or indication on NW-side additional conditions. |
| Sony [21] | Proposal 1 : Support the NW in assisting UE model training based on UE requests. Proposal 2 : Support collecting training data from legacy measurement schemes based on model input & output settings. |
| LGE [23] | Proposal #3: Support reporting of UE assistance information for determining Set A, e.g., UE to report preferred Set A among candidate beams of Set A. Proposal #4: Consider extending sub-configuration based Rel-18 NES mechanism for Set B beam measurement and reporting. |
| Apple [24] | Proposal 3-4: data collection can be initiated/triggered by configuration from NW; or is requested from UE and then may be configured by NW at NW’s discretion. |
| CAICT [29] | Proposal 4: UE initiated data collection for UE-sided model should be considered for BM-Case1 and BM-Case2. |
| Indian Institute of Technology Madras (IITM), IIT Kanpur [31] | Proposal 2: Signaling for training data collection should be based on whether the AI/ML model is at the UE-side or NW-side. Proposal 3: For UE-side model training, UE sends a request to the NW for data collection and the NW responds with the measurement configuration. UE makes the measurements and trains the model. During configuration, the NW could assign an ID to identify the current instance of data being collected. This ID could be referenced, if necessary, to achieve alignment of additional conditions at a later stage. |
| MTK [37] | Proposal 4: For UE side data collection, consider both initiated/triggered by NW and/or requested from UE at this stage. - For UE side data collection initiated/triggered by NW, consider starting from the existing CSI measurement and report configurations. New spec support may be required for the case when Set B is different from Set A. - For UE side data collection requested from UE, starting from considering the current CSI beam report and/or Scheduling Request (SR) in the UCI framework. |

3.3 Association/mapping/configuration of set A/B beams

The issue has been covered by proposal 2.1.1 as FFS. It is pre-mature to have detail proposal for this meeting. Some offline during the meeting is encouraged.

| Company | Proposals |
| --- | --- |
| Huawei/HiSi[2] | Proposal 2: RAN1 should consider potential solutions to enable the UE to perform CSI measurements on larger sizes of beam set(s) including Set A, Set B, and/or possibly non-AI/ML, than what would be possible with the current mechanism. |
| Ericsson [3] | Observation 6 The association/mapping of beams within Set A and beams within Set B provides UE with information of which beams that are part SetA/B Proposal 13 For UE-sided models, conclude that NW should determine which beams that are part of Set A and set B Observation 7 For the scenario when the NW configures the UE with set B, the UE can still reduce its measurement further by selecting a subset of set B that it needs to reach adequate prediction performance. Observation 8 For the association/mapping for the UE-sided inference scenario, the NW can provide the association/mapping of set B and set A, where set A might not be transmitted by the NW. Proposal 14 For UE-sided models, study how to extend the existing CSI framework with minimal specification impact to enable NW indication of set A and set B |
| Vivo [4] | Proposal 1: Due to significant performance degradation for mismatch pattern, support pre-configured patterns in Set B with corresponding information provided for inference for both UE sided and NW sided model. Proposal 2: Support Set B pattern indication between gNB and UE to ensure consistency of Set B across training, inference and monitoring for NW-side model. Proposal 5: Support to perform P3 UE Rx beam search based on multiple Tx beams, which can be configured within a time interval limitation for P2 resource set receiving for set B measurement result acquisition. |
| ZTE [5] | Proposal 13: Support flexibly indication/activation/deactivation of arbitrary beams or beam subsets among all beams in Set A to reduce the signaling overhead for Set B configuration. |
| CMCC [7] | Proposal 7: Regarding data collection for UE-side model, down-select the indication of association/mapping of beams within Set A and beams within Set B from the following options: · Opt1: QCL information between beams within Set A and beams within Set B · Opt2: Identifier of info representing the association/mapping of Set A and Set B · Opt3: The bitmap or pre-defined rule that beams of Set B is subset of beams of Set A |
| Intel [8] | Proposal 1: For a network-side AI/ML model, for BM-Case 1/2, RAN1 should consider both explicit and implicit configuration of set B for AI/ML model input. The need for configuration of set A should also be further discussed. |
| CATT [10] | Proposal 4: For indication of the associated Set A from network to UE, both explicit and implicit method can be considered. |
| OPPO [13] | Proposal 1: For BM-Case1, configure Set B as a subset of Set A during training and inference phase. Proposal 2: For BM-Case2, configure the same Set B and Set A during training and inference phase. Proposal 3: For UE-side model, support NW to configure more than one Set Bs/Set As and then select/activate one pair of associated Set B/Set A. Proposal 4: For BM-Case2, support the configuration of Set A and/or Set B on a per instance basis. |
| Nokia [19] | Proposal 3: Consider the following when configuring Set A and Set B beams in a beam prediction related CSI report, - For BM-Case1 and BM-Case2, considering the case of Set B is a subset of Set A, reuse the legacy RS resource set (resourcesForChannelMeasurement) as the RS set applicable for Set B beams and configure/Indicate another RS resource set associated with the CSI report configuration to consider as Set A. - For BM-Case1 and BM-Case2, considering the case of Set A and Set B are different, reuse the legacy RS resource set (resourcesForChannelMeasurement) as the RS set applicable for Set B beams. Discuss the following variants for Set A. - Option 1: Configure/Indicate Set A associated with the CSI report configuration. - Option 2: UE determines Set A based on QCL relations between Set B and configured CSI-RS resources. - For BM-Case2, considering Set A and Set B are the same, reuse the legacy RS resource set (resourcesForChannelMeasurement) applicable to Set B and Set A. |
| Samsung [16] | Proposal 8. For UE-side AI/ML model inference, support the configuration of mapping or association information of Set A and Set B, where the information includes at least the following: - The size of Set A - Spatial domain association of Set A and Set B - FFS: Whether Set A beams correspond to RS resources |
| Fujitsu [17] | Proposal 5: - For training data collection, the reference signals same as Set B should be configured to obtain the model input data. And the reference signals same as Set A should be configured to obtain the ground truth data. Proposal 6: - RAN1 to further discuss whether the same or separate reference signal configuration between training data collection and model inference. |
| Futurewei [18] | Proposal 3: For Rel-19 AI/ML-based BM, for data collection for AI/ML model of BM-Case1 and BM-Case2, support using RS ID as implicit indication of beam ID and reusing L1-RSRP reporting as much as possible. |
| Rakuten [22] | Proposal 2: RAN1 agrees to define Set A to Set B association differently for transmitted and un-transmitted beams before prediction. |
| FGI, NTPU [27] | - Proposal 1: For BM Case 1 model, whether Set A and Set B can be in different Frequency Ranges needs to be clarified. - Proposal 2: For BM Case 1 model, when Set A and Set B are different, how to provide the configuration of the output beam information needs further discussion. - Proposal 3: For BM Case 1 model, when Set B is a subset of Set A, specify Set A and Set B in the same configuration or different configurations. - Proposal 4: For BM Case 1 model, when Set B is a subset of Set A, whether to maintain the beam information of Set B to infer Set A needs further discussion. - Proposal 5: For BM Case 1 model, when Set B is a subset of Set A, how to inform UE of the beam information if the beam information for Set A and Set B is different needs further discussion. |
| KT [32] | Proposal 8. Discuss a method for mapping/association between Set A and Set B for UE-side model. |
| Sharp [33] | Proposal 3: Regarding configurations for Set A and Set B, reuse legacy CSI-RS resource configuration (e.g., CSI-ResourceConfig) as baseline to configure beam resources for Set A and Set B. - Separate resource configurations for set A and set B can be configured for BM case 1 and BM case 2. - Separate resource configurations for set A and set B can be configured for different LCM functions for a BM case. |
| MTK[37] | Proposal 2: Study how different types of Set A and Set B resources can be configured in one or multiple report configurations for UE to measure for data collection starting from the following options: - Option1: When Set A and Set B resources are configured in one report configuration, study how UE can report the model input and label at the same time. - Option2: when Set A and Set B resources are configured in multiple report configurations, study how UE can identify the corresponding report configurations for the current data collection instance. Proposal 3: Study specification support on how NW and UE align with each other to reuse and combine the measurement and report results of the existing report configurations to obtain data samples for AI/ML-based beam management model training. |

3.4 Other aspects related to measurements

The measurement accuracy had been studied in SI phase. However, there is no clear evidence to show that the measurement accuracy has to be improved, assuming non-AI based beam management can work based on existing L1-RSRP. Therefore, it is encouraged companies to have some further study on whether/how to increase the accuracy.

| Company | Proposals |
| --- | --- |
| Ericsson [3] | Proposal 2 Study how to mitigate the impact from UE RSRP measurement errors for example by UE indicates its RSRP measurement accuracy. Proposal 10 For NW-sided training data collection, to improve the quality of the collected data, send an LS to RAN4 on improving UE measurement accuracy for the purpose of AI/ML data collection Observation 6 For NW-sided AI/ML model, RAN4 could study the possibility for tightening requirements on the L1-RSRP measurement accuracies for beam prediction use cases, |
| Intel [8] | Proposal 10: RAN1 should further discuss if one-shot L1 measurements are used for set B beams or if averaging of L1 measurements over time is needed. |
| Google [9] | Proposal 1: Support SSB/CSI-RS repetition to improve the measurement accuracy for SD beam prediction Proposal 10: Support SSB/CSI-RS repetition to improve the measurement accuracy for temporal beam prediction. |
| CAICT [29] | Proposal 3: The enhancement on RSRP accuracy could be realized by configuring appropriate RS pattern for special scenario. |
| ETRI [36] | Proposal 1: Support the methods to reduce RS overhead and signalling overhead for AI/ML beam management. Proposal 4: Support the method that the NW configures the output of the AI/ML model located on the UE-side. |

3.5 First round discussion

Proposal 3.1.1a (contents)

For NW-sided model, consider the following options for the contents of data collection for training:

- Opt 1: All L1-RSRP from the resource for Set A of beams

  - Where Set A of beams are for UE measurement and report

  - FFS on whether the information on all or a part of Set A of beams ID needs to be reported or not.

- Opt 2: A subset of L1-RSRP from the resource for Set A of beams, with information on corresponding beam/RS ID

  - Where Set A of beams are for UE measurement

  - FFS on how to determinate the subset, at least including data selection, data omission

    - E.g. L1-RSRP of Top K Set A of beams

- Opt 3: All L1-RSRP from the resource for Set B of beams, and information for Top 1 (FFS: Top K) DL beam/RS ID among Set A of beams

  - Where both Set A of beams and Set B of beams are for UE measurement and report respectively

  - FFS on whether the information on all or a part of Set B of beams/RS ID needs to be reported or not.

- Opt 4: A subset of L1-RSRP from the resource for Set B of beams with information on corresponding beam/RS ID, and information for Top 1 (FFS: Top K) DL beam/RS ID among Set A of beams

  - Where both Set A of beams and Set B of beams are for UE measurement and report respectively

  - FFS on how to determinate the subset, at least including data selection, data omission

    - E.g. L1-RSRP of Top N Set B of beams

- Opt 5: A subset of L1-RSRP from the resource for Set B of beams with information on corresponding beam/RS ID, and a subset of L1-RSRP from the resource for Set A of beams, with information on corresponding beam/RS ID

  - Where both Set A of beams and Set B of beams are for UE measurement and report respectively

  - FFS on how to determinate the subset, at least including data selection, data omission

    - E.g. L1-RSRP of Top N Set B of beams

    - E.g. L1-RSRP of Top K Set A of beams

- Other options are not precluded.

- FFS on whether to report the contents of each option in one or multiple measurement reports

- FFS on whether existing quantization step and range for L1-RSRP need to be optimized

- FFS on whether/how to report the information associated with time stamp for BM-Case2, including whether to report measurements from multiple time stamp in one report instance

- FFS on whether to configure the purpose of the report by the NW

- FFS on whether to report the contents of each option in one or multiple measurement reports

| Company | Y/N | Comments |
| --- | --- | --- |
| New H3C | Y | FFS items are a little bit much and we need categorize them |
| OPPO |  | Given the content of four options listed in Proposal 3.1.1, we are in general fine to train AI/ML model(s) in different approaches. And we also would like to share some comments for specific options. Firstly, Proposal 2 mentions a subset of L1-RSRP of Set A of beams to be reported, UE needs to measure Set A of beams, but may not to need to report full Set A. Secondly, on how to determining the subset, we believe selecting Top-K beams with highest L1-RSRP(s) among Set A should be one method. If we don’t intend to list it clearly at the early stage of WI, we do hope this selection method is of course included. - Opt 2: A subset of L1-RSRP of Set A of beams, with information on corresponding beam/RS ID - Where Set A of beams are for UE measurement and report - FFS on how to determinate the subset, at least including data selection, data omission On Option 3, we guess this option applies to the AIML with multi-classification model. To be more specific, we could say that Set A and Set B are for measurement and report, respectively. In addition, for BM-Case2, we don’t see a strong motivation to restrict that Set A and Set B should be different. - Opt 3: All L1-RSRP of Set B of beams, and information for Top 1 (FFS: Top K) DL beam/RS ID among Set A of beams - Where both Set A of beams and Set B of beams are for UE measurement and report, respectively. and Set A and Set B are different. - FFS on whether the information on all or a part of Set B of beams/RS ID needs to be reported or not. Finally, to facilitate model implementation, we would like to confirm that there could be more than Options to be finally supported, rather than only selecting one option to be specified. |
| Ericsson |  | In general fine with the direction of the proposal. In addition to Oppos comment, the FFS are a bit too detailed. Our view is to keep them more general for now, which is captured below: - Other options are not precluded. - FFS on reporting content for each option and BM-case (1 and 2). - FFS on whether the NW need to indicate purpose of report (including set A/B categorization) - FFS on whether existing L1 measurement assumptions are sufficient - FFS on quantization step and range for L1-RSRP including differential L1-RSRP reporting - FFS on whether/how to report the information associated with time stamp, including whether to report measurements from multiple time stamp in one report instance - FFS on whether the purpose of the report needs to be specified or not, including implicit and explicit - FFS on measurements are one-shot L1 measurements and/or averaged of L1 measurements over time, or up to UE implementation - FFS on whether to report the contents of each option in one or multiple measurement reports |
| HW/HiSi | N | We agree with H3C that the FFS are too much. It seems that the proposal is merging two aspects, one is the reporting of Set A and Set B and the other is overhead reduction. We are concerned that it this could become very complicated to overview and suggest to discuss the reporting and potential overhead reduction in separate proposals. In the study item, we spent a long discussion on Set A and Set B reporting, and eventually it was agreed that the measured sets from which would be reported could be any type of set that is indicated by the gNB. Therefore, no Set B and Set A mentioned in the agreement from RAN1#112b. If desired we could make an update to the agreement from RAN1#112b to clarify that the Set A and Set B are included, but otherwise, we do not see what the reporting part here brings new on top what already was agreed in the SI? Could this please be clarified? Would one way forward be to e.g. discuss the updated proposal below for the reporting and then to have a separate high-level proposal on the reporting overhead reduction? Updated Agreement from(#112b-e) Regarding data collection for NW-side AI/ML model, study the following options (including the combination of options) for the contents of collected data, - Opt.1: M1 L1-RSRPs (corresponding to M1 beams) with the indication of beams (beam pairs) based on the measurement corresponding to a beam set, where M1 can be larger than 4, if applicable - Where the measured beam set can be Set A, Set B, or a subset of either of them. - FFS: the range of M1 - Opt.2: M2 L1-RSRPs (corresponding to M2 beams) based on the measurement corresponding to a beam set, where M2 can be larger than 4, if applicable - Where the measured beam set can be Set A, Set B, or a subset of either of them. - FFS: the range of M2 - Opt.3: M3 beam (beam pair) indices based on the measurement corresponding to a beam set, where M3 can be larger than 4, if applicable - Where the measured beam set can be Set A, Set B, or a subset of either of them. - FFS: the range of M3 - FFS: How to select the M1/M2/M3 beam(s) or beam pair(s) Note: Overhead, UE complexity and power consumption should be considered for the above options |
| Fujitsu |  | Thanks FL for the great efforts. In Proposal 3.1.1, it looks Opt 3 and Opt 4 is for the case that Set A and Set B are different. Then the question for clarification is for Opt 1 and Opt 2, is it for the case that Set B is subset of Set A? If yes, it’s better to explicitly capture it in Opt 1and Opt 2. Regarding the following FFS bullet: - FFS on quantization step and range for L1-RSRP including differential L1-RSRP reporting We think the non-differential L1-RSRP reporting could also be applied for the training data collection. Therefore, the following update is suggested. - FFS on quantization step and range for L1-RSRP including differential L1-RSRP reporting and non-differential L1-RSRP reporting Another question for clarification is for the FFS bullet “FFS on whether the purpose of the report needs to be specified or not, including implicit and explicit”, does “the purpose of the report” means that whether the data is for model input or for ground truth data? If yes, it’ better to clearly capture it in the bullet. |
| MediaTek |  | We agree with OPPO that Option3 can be applied to both cases when Set B is a subset of Set A and when Set B is different from Set A. |
| TCL |  | In general, we support the proposals, but the FFS may be postponed. The data collection for training should consider the accuracy in the first place, therefore the overhead reduction should be with low priority. Whether these FFS options are to be discussed relies on the down selection of the four options. |
| CMCC |  | Option 3 and 4 are also applicable to the case that set B is subset of set A. When set B is subset of set A, regarding to classification model, the label of AI model can be RS ID of Top 1 beam. |
| NTT DOCOMO |  | In Opt3/4, "information for Top 1 (FFS: Top K) DL beam/RS ID among Set A of beams” is not clear. If it is meant to include DL beam/RS ID and/or L1-RSRP, it should be clarified in the same manner as Opt1/2. |
| InterDigital |  | We are generally fine with proposals in main bullets and FFSs for corresponding sub-bullets. However, FFSs in main bullets are too early and further discussions are needed. |
| vivo |  | Thanks FL for the great efforts. First of all, the description may be too detailed and most of the options and FFS is irrelevant to data content. From our perspective, data content is used to describe the types of data required in data collection procedure. How to report data belongs to the following report enhancement. Secondly, we tend to agree with HW's suggestion that it is not necessary to differentiate between Set A and Set B in above proposal. We can combine set A and set B in a unique set of options. Further, considering data collection procedure in situation of multiple measurement occasions reported in one report instance, time resource information shall be explicit or implicit reported with L1-RSRP and/or corresponding beam resource indication for time stamp acquisition. For NW-sided model, further study the following options for the contents of data collection for training: - Opt 1: All L1-RSRP of Set A of beams a beam set - Where Set A of beams the beam set are is for UE measurement and report - FFS on whether the information on all or a part of Set A of beams ID needs to be reported or not. - Opt 2: A subset of L1-RSRP of Set A of beams a beam set, with information on corresponding beam/RS ID - Where Set A of beams the beam set are is for UE measurement and report - FFS on how to determinate the subset, at least including data selection, data omission - Opt 3: All L1-RSRP of Set B of beams, and information for Top 1 (FFS: Top K) DL beam/RS ID among Set A of beams - Where both Set A of beams and Set B of beams are for UE measurement and report, and Set A and Set B are different. - FFS on whether the information on all or a part of Set B of beams/RS ID needs to be reported or not. - Opt 4: A subset of L1-RSRP of Set B of beams with information on corresponding beam/RS ID, and information for Top 1 (FFS: Top K) DL beam/RS ID among Set A of beams - Where both Set A of beams and Set B of beams are for UE measurement and report, and Set A and Set B are different. - FFS on how to determinate the subset, at least including data selection, data omission - For BM Case-2, support the report of multiple measurement occasions in one report instance. Consider explicit or implicit time resource indicator for time stamp acquisition. - Other options are not precluded. - FFS on quantization step and range for L1-RSRP including differential L1-RSRP reporting - FFS on whether/how to report the information associated with time stamp, including whether to report measurements from multiple time stamp in one report instance - FFS on whether the purpose of the report needs to be specified or not, including implicit and explicit - FFS on measurements are one-shot L1 measurements and/or averaged of L1 measurements over time, or up to UE implementation |
| NEC |  | For Opt.3 and Opt.4, we suggest adding the following relationship between Set A and Set B: Where both Set A of beams and Set B of beams are for UE measurement and report, and Set A and Set B are different, or Set B is a subset of Set A. For second FFS, [whether/how to report the information associated with time stamp] and [whether to report measurements from multiple time instances in one report instance] seem to be two independent FFS? |
| Xiaomi |  | We support the principle of the proposal. And we have some comments: - We suggest to replace “further study” by “consider”. And it will be better to clarify whether down-selection will be done in next meetings or not - Does it mean each opt will be used standalone? No combination of more than one opt? if yes, it is better to clarify in the proposal. - Does it mean Opt 1 and Opt 2 are used for set B is a subset (or same) of set A, Opt 3 and Opt 4 are used for set B is different from set A? we share same view that Opt 3 and Opt 4 can also be used for set B is a subset (or same) of set A. so we suggest to delete “Set A and Set B are different” in Opt 3 and Opt 4. - According to TR 38. 843, Set B is a set of beams whose measurements are taken as inputs of the AI/ML model. If only a subset of L1-RSRP of set B of beams is reported, how to decide the value of L1-RSRP of beams whose L1-RSRP is not reported? It depends on the rule for subset determination or it depends on NW implementation? But we prefer to strive for a common solution for NW side model and UE side model. - Suggest to delete the last second FFS since it is for training, one shot is reasonable. and average may be needed for performance monitoring. |
| ZTE |  | It seems that Opt.1 and Opt.2 are used for the case that Set B is a subset of Set A, while Opt.3 and Opt.4 are used for the case that Set B and Set A are different. - Then, for Opt.2, the UE may report L1-RSRP of Set C or Set B for model input data and Top-1/K beam ID of Set A for model label data. Thus, we can modify Opt.2 as 'A subset of L1-RSRP of Set A of beams, with information on corresponding necessary beam/RS ID'. - For Opt 3 and Opt 4, all L1-RSRPs of Set B and Set A accompanied by necessary beam ID information may be reported if regression model is trained at the NW side. For the 3rd FFS from the end, our understanding is that the content of data collection is configured by the NW without any explicitly notifying data collection purpose. For the 2nd FFS from the end, it can be configured by the high layer parameter timeRestrictionForChannelMeasurements in the CSI report setting. |
| QC |  | For training we would need measurements from both input Set A and Set B, so not sure about the motivation for Opt. 1 and Opt.2. We believe this can be abstracted in a simpler manner, as mentioned below. As we are talking about the contents of the collected data in this proposal, any aspect with regards to the report should be removed. With that said, we suggest the following updated proposal: Updated Proposal 3.1.1: For NW-sided model, further study the following options for the contents of data collection for training: L1-RSRP (along with beam IDs) of top-M beams from Set B and L1-RSRPs along with beam IDs of top-N beams from Set A. - FFS on M, N |
| IIT Madras |  | We recommend Opt3 since this supports collection of data required for training. |
| Spreadtrum |  | Little confusion for option1 and option2, why these 2 options only include set A information. It means these 2 options only used for ‘set B is a subset of set A’? |
| Panasonic |  | We have a few comments for clarifications as follows: - For Options 3 and 4, like comment of DOCOMO, it is not clear about “information for Top 1 (FFS: Top K) DL beam/RS ID among Set A of beams”. Can FL explain the motivation for it? - Regarding Option 1, it seems “FFS on whether the information on all or a part of Set A of beams ID needs to be reported or not.” can cover Option 2. Can FL clarify it? |
| CATT |  | For Opt 3 and Opt 4, why they can only be used when Set A and Set B are different? We think Opt 3 and Opt 4 can be used for the case of Set B is subset of Set A. Agree with HW and vivo, there is no need to differentiate Set A and Set B case. For Opt 1 and Opt 2, do they only use for regression AI model and the case of Set B subset of Set A? If yes, we think it’s better to clarify the applicable use. For regression AI model, we also have other options for the case different Set A and Set B: - All L1-RSRP of Set A and Set B beams A subset of L1-RSRP of Set A of beams, with information on corresponding beam/RS ID, and a subset of L1-RSRP of Set B of beams, with information on corresponding beam/RS ID |
| CEWiT |  | Agree with proposal in principle. Our reading is that Opt1 and Opt2 are only for the case when “Set-B is a subset of Set-A”. |
| Google |  | We think the proposal requires more discussion. From spec impact point of view, the options should be re-formulated. UE reports L1-RSRP for SSB/CSI-RS instead of a beam. So we need to figure out what should be configured as CMR and what should be reported based on the configured CMR. In addition, we think it is necessary to report a quality indicator to reflect the potential measurement error. |
| Futurewei |  | Agreed with other companies that the FFS are too many/detailed and is mixing the content aspect and how to reduce the overhead of the content together, making it difficult to make progress. We thus suggest the following modifications for a high level/simplified proposal based on Vivo’s version: For NW-sided model, further study the following options for the contents of data collection for training: - Opt 1: All L1-RSRP of Set A of beams a beam set - Where Set A of beams the beam set are is for UE measurement and report - FFS on whether the information on all or a part of Set A of beams ID needs to be reported or not. - Opt 2: A subset of L1-RSRP of Set A of beams a beam set, with information on corresponding beam/RS ID - Where Set A of beams the beam set are is for UE measurement and/or report - FFS on how to determinate the subset, at least including data selection, data omission - Opt 3: All L1-RSRP of Set B of beams, and information for Top 1 (FFS: Top K) DL beam/RS ID among Set A of beams - Where both Set A of beams and Set B of beams are for UE measurement and report, and Set A and Set B are different. - FFS on whether the information on all or a part of Set B of beams/RS ID needs to be reported or not. - Opt 4: A subset of L1-RSRP of Set B of beams with information on corresponding beam/RS ID, and information for Top 1 (FFS: Top K) DL beam/RS ID among Set A of beams - Where both Set A of beams and Set B of beams are for UE measurement and report, and Set A and Set B are different. - FFS on how to determinate the subset, at least including data selection, data omission - Other options are not precluded. - FFS on quantization step and range for L1-RSRP including differential L1-RSRP reporting - FFS on whether/how to report the information associated with time stamp, including whether to report measurements from multiple time stamp in one report instance - FFS on whether the purpose of the report needs to be specified or not, including implicit and explicit - FFS on measurements are one-shot L1 measurements and/or averaged of L1 measurements over time, or up to UE implementation |
| Lenovo |  | Firstly, it should be clarified this proposal applies to both use cases in the main bullet. Secondly, we tend to agree with lots of companies that the FFS are a bit too detailed and lots of them can be discussed in the next stage. Thirdly, we suggest to remove the description “Set A and Set B are different” since Set A can be same as Set B for BM-Case2. Therefore, we suggest the following update: For NW-sided model of both BM cases, further study the following options for the contents of data collection for training: - Opt 1: All L1-RSRP of Set A of beams - Where Set A of beams are for UE measurement and report - FFS on whether the information on all or a part of Set A of beams ID needs to be reported or not. - Opt 2: A subset of L1-RSRP of Set A of beams, with information on corresponding beam/RS ID - Where Set A of beams are for UE measurement and report - FFS on how to determinate the subset, at least including data selection, data omission - Opt 3: All L1-RSRP of Set B of beams, and information for Top 1 (FFS: Top K) DL beam/RS ID among Set A of beams - Where both Set A of beams and Set B of beams are for UE measurement and report, and Set A and Set B are different. - FFS on whether the information on all or a part of Set B of beams/RS ID needs to be reported or not. - Opt 4: A subset of L1-RSRP of Set B of beams with information on corresponding beam/RS ID, and information for Top 1 (FFS: Top K) DL beam/RS ID among Set A of beams - Where both Set A of beams and Set B of beams are for UE measurement and report, and Set A and Set B are different. - FFS on how to determinate the subset, at least including data selection, data omission - Other options are not precluded. - FFS on quantization step and range for L1-RSRP including differential L1-RSRP reporting - FFS on whether/how to report the information associated with time stamp, including whether to report measurements from multiple time stamp in one report instance - FFS on whether the purpose of the report needs to be specified or not, including implicit and explicit - FFS on measurements are one-shot L1 measurements and/or averaged of L1 measurements over time, or up to UE implementation - FFS on whether to report the contents of each option in one or multiple measurement reports |
| Fraunhofer |  | We agree with the previous companies that the FFSs are too specific for the current discussion. Hence, we propose to remove all FFS. |

Proposal 3.1.2 (Report)

For NW-sided model, for L1-RSRP measurement report for the data collection for training, FFS:

- L1 signaling

- High layer signaling

| Company | Y/N | Comments |
| --- | --- | --- |
| New H3C | Y |  |
| OPPO | Y | Fine with the FL proposal. |
| Ericsson | Y | Support |
| HW/HiSi |  | L1 signaling will already need to be used for inference and probably also for monitoring. It would be good to have a unified solution for all stages. Therefore we think that we can say that at least L1 signaling should be supported. We suggest an FFS for higher layer signaling, if it additionally be used. For NW-sided model, for L1-RSRP measurement report for the data collection for training, FFS: - L1 signaling FFS: High layer signaling |
| Fujitsu | Y | Fine with the proposal. Higher layer signaling is preferred for training data report. |
| MediaTek | Y | We are OK with this proposal |
| TCL | Y | Support the FL proposal. |
| CMCC | Y |  |
| InterDigital |  | We support Huawei’s version. |
| vivo | Y | OK |
| NEC | Y |  |
| ETRI | Y | We support |
| Xiaomi |  | FFS means down selection or consider at least one? |
| ZTE | Y | Since the NW can trigger L1-RSRP measurement report from many UEs at the same time, both L1 signaling and high layer signaling can be possible for the data collection with different timing and overhead requirements. |
| Apple | Yes |  |
| QC | N | As opposed to inference for which L1 signaling is required for Set B measurements, there is no such requirement for data collection for training with regards to Set B/Set A measurements. With that said, this is outside the scope of RAN1. RAN1 should focus on the content of the collected data, which is the focus of Proposal 3.1.1. |
| IIT Madraas | Yes | We support the FL’s proposal |
| Spreadtrum | Y |  |
| Panasonic | Y |  |
| CATT | Y | OK |
| CEWiT | Y |  |
| Google |  | I think we can make it more specific, e.g., UCI, MAC CE or RRC. |
| Sharp | Yes |  |
| Futurewei |  | The existing CSI framework should be reused as much as possible to reduce the required specification effort. Therefore, L1 signaling should be at least supported. Higher layer signaling can be FFS. |
| LG | Y |  |
| Lenovo | Y | Support FL proposal |
| Fraunhofer | Y |  |
| KDDI | Y | Support |

Proposal 3.1.3 (Configuration of Set A and Set B)

At least for training collection for NW-sided, study on how to configure the RS resources for Set A of beams and RS resources for Set B of beams (if applicable)

- FFS on whether to extend the max number of RS resources for Set A of beams (including up to [256]) and corresponding Set B of beams, potentially with UE feature

- FFS on corresponding TCI states indication

| Company | Y/N | Comments |
| --- | --- | --- |
| New H3C | Y |  |
| OPPO |  | As we all know, the TCI framework starting from Rel-15 to Rel-18 serves for beam indication. With indicated TCI state, UE may adopt one particular Tx beam for reception. But as for beam measurement and reporting, the legacy applies the configurations of RS resource(s). It could be up to NW to determine a Rx beam. We failed to see the benefit of configuring TCI state for measurements of Set B and/or Set A, which seems quite diverge from legacy approach. From this sense, we suggest to remove the contents in bracket as below. Study on how to configure the RS resources [and the corresponding TCI states] for Set A of beams and RS resources [and the corresponding TCI states] for Set B of beams (if applicable) |
| Ericsson |  | Fine in general, is this for NW or UE-sided model? Or both? |
| HW/HiSi |  | Agree with Oppo. How to configure RS resources for Set A and Set B should definitely be discussed. This is a fundamental part of the AI-BM. Our suggestion is to put the focus on this part and to find concrete solutions for when Set B is a subset of Set A and when Set B is different from Set A. The TCI states are a separate discussion and companies had different views on the necessity of this discussion. Suggest to handle these aspects decoupled from each other and to focus here on the first one. Minor comment, maybe “discuss” is a more suitable word to use in a WI than “study”? Suggested update. Discuss Study on how to configure the RS resources [and the corresponding TCI states] for Set A of beams and RS resources [and the corresponding TCI states] for Set B of beams (if applicable) |
| Fujitsu |  | Since this section is for training data collection, we suggest to add the following text to the main bullet. For training data collection, study on how to configure the RS resources [and the corresponding TCI states] for Set A of beams and RS resources [and the corresponding TCI states] for Set B of beams (if applicable) Also, we have the same question as Ericsson regarding whether it is NW or UE side model. |
| MediaTek |  | First, we agree with OPPO, configuring TCI states is just one of the possible approaches to configure Set A and Set B for UE measurements. There are other approaches which do not need to change the configuration of the TCI state. Suggest removing the corresponding brackets. Second, we do not support the FFS, the max number of RS resources for Set A of beams should be the same as the current MIMO framework. If there is any needs in enhancing the capacity of configurable RS resources, it should be proposed and discussed in the main MIMO session. Suggest removing FFS. |
| TCL | Y | We support the proposals. Resue or extend the TCI framework for AI beam prediction will be least standard impact. |
| CMCC |  | Agree to focus on configuration of set A and set B and delete TCI state part. |
| NTT DOCOMO | Y | Support |
| InterDigital |  | As use of TCI state is already agreed at least for BM-Case 1, we don’t see any reason to use brackets for TCI states. In addition, it’s too early to discuss max number of RS resources. Therefore, we propose the following modification. Study on how to configure the RS resources [and the corresponding TCI states] for Set A of beams and RS resources [and the corresponding TCI states] for Set B of beams (if applicable) - FFS on the max number of RS resources for Set A of beams (including up to [256]) and corresponding Set B of beams |
| vivo |  | Agree with OPPO. Maybe We can focus on the configuration enhancement of Set A and Set B in this proposal. Relative TCI indication should be discussed in the association between Set A and Set B or in beam indication part. |
| NEC |  | OK in general. As mentioned by companies, we would like to clarify the purpose of [and the corresponding TCI states]. Furthermore, to our understanding, multiple Set B or Set A may be configured, so we suggest adding the following FFS: FFS on configure multiple Set Bs or Set As. |
| ETRI |  | We think that configuring RS resources for set B is not necessary part for data collection for training. Because, set B can be a subset of set A. If FL consider the case where set B is different from set A, then configuring RS resource for set B might be needed. |
| Xiaomi |  | - We also suggest to replace “Study on” to “Consider” - Is it for both NW side model and UE-side model? Please clarify in the proposal. - The usage of resource configuration is data collection for model training, so we are wondering what is the meaning of “TCI state” in this stage? So we share same view as OPPO that we suggest to delete “[and the corresponding TCI states]”. |
| ZTE |  | Support in general. For the FFS part, shouldn’t the max number of RS resources for Set A be 32 or 64 based on Rel-18 evaluation? |
| QC |  | Do not agree with “including up to [256]”, and the value can be discussed and agreed upon later after careful considerations at least with regards to UE capability. |
| Spreadtrum | Y |  |
| CATT |  | We support the intention of the proposal. But for FSS, we suggest to discuss it in the UE feature part. |
| Google |  | Is it about spatial domain configuration, e.g., location in a beam grid for a DL RS? |
| Sharp | Yes |  |
| Futurewei |  | Agree with OPPO. The contents related to TCI states should be removed. |
| LG | Y |  |
| Lenovo |  | Agree with OPPO that TCI state configuration should be removed. At least at this stage, we fail to see the motivation to increase the max number of beams in Set A. Further, maybe the FFS part should be discussed in UE feature session. |
| Fraunhofer | Y |  |

Proposal 3.1.4(Association)

For UE sided model, further study the association of Set B and Set A, including

- Alt 1: Method to define/represent the association within the current CSI framework as the baseline, including:

  - Based on TCI state

  - Based on resource ID (e.g., SSB ID, CSI-RS resource ID)

  - Based on resourceSet ID,

  - Based on reportConfig ID,

  - Based on resourceConfig ID,

  - Other options within the CSI frameworkare not precluded.

- Alt 2: Method to define/represent the association via introducing beam related IDs

  - FFS how to introduce such beam IDs or beam pattern IDs are introduced

- Alt 3: Method to define/represent the association via introducing dataset related IDs

  - FFS how to introduce data collection configuration ID or dataset ID

- FFS on other aspects

| Company | Y/N | Comments |
| --- | --- | --- |
| New H3C | Y |  |
| OPPO |  | As for BM-Case2, as we evaluated in Rel-18 SI phase, Set B could be the same as Set A. Could we suggest to keep such relationship with slight modification as below. - Association relationship: - Set B of beams are from a subset/full subset of Set A of beams - Set B of beams are different from Set A of beams FFS on QCL relationship |
| Ericsson | Y | Is this for NW or UE-sided model (our understanding is UE-sided)? We think it is important to highlight that there is no such definition on “beam ID” in existing CSI framework. Also set A/B can also be indicated on the level of resourceSets, reportConfigs, resourceConfigs in addition to resources in the existing CSI framework. Hence, we think it would be clearer with the following rewording: For UE-sided model, Further study the association of Set B and Set A, including - Method to define/represent the association within the current CSI framework, including: - Based on TCI state - Based on resource ID (e.g., SSB ID, CSI-RS resource ID) - Based on resourceSet ID, - Based on reportConfig ID, - Based on resourceConfig ID, - Based on beam ID - Other options within the CSI framework are not precluded - Method to define/represent the association via introducing beam IDs. - FFS where such beam IDs are introduced - Association relationship: - Set B of beams are from a subset of Set A of beams - Set B of beams are different from Set A of beams - FFS on QCL relationship FFS on other aspects |
| HW/HiSi |  | It seems one straight forward approach is missing here, that one resource set is defined for Set B another resource set for Set A and they are linked about resource set ID? For the FFS on QCL relationship, why does it only apply on when beams are different, wouldn't’t it also be possible for to define a QCL when both are CSI-RS and then they would apply also to the case when Set B is a subset of Set A? The proposal is on our view only needed for the UE side model. A suggestion on update: For the UE side model Further discuss further study the association of Set B and Set A, including - Method to define/represent the association, including: - Based on TCI state - Based on resource ID (e.g., SSB ID, CSI-RS resource ID) - Based on resource set ID - Based on beam ID - Other options are not precluded. - Association relationship: - Set B of beams are from a subset of Set A of beams - Set B of beams are different from Set A of beams - FFS on QCL relationship - FFS on QCL relationship FFS on other aspects |
| Fujitsu |  | In the main bullet, it should be clarified that this is for training data collection. In addition, we also think Set A and Set B association could be via resource set level, resource config level or report config level. The modification to the first bullet and corresponding sub-bullets from Ericsson is fine for us. |
| MediaTek |  | We agree with Ericsson, beam ID should be removed and association might be indicated through resourceSets ID, reportConfigs ID, resourceConfigs ID too. |
| TCL | Y | Support the FL proposal. |
| CMCC |  | Agree to remove beam ID. No matter when set B is different from set A or set B is subset of set A, resource ID implicitly corresponds to beam ID. |
| NTT DOCOMO |  | Association can be determined based on data collection configuration ID. To cover that, the following update should be made. - Based on beam ID - Based on data collection configuration ID - Based on dataset ID Other options are not precluded. |
| InterDigital |  | We prefer to keep beam IDs. As expressed in our contribution and Set B beams may not be transmitted, different approaches on beam IDs should be applicable and different from RS resource IDs. For association relationship, FFS on QCL relationship should be applied to both bullets as Set B beams may not be transmitted. Therefore, we propose the following modification. For the UE side model, Ffurther study the association of Set B and Set A, including - Method to define/represent the association, including: - Based on TCI state - Based on resource ID (e.g., SSB ID, CSI-RS resource ID) - Based on beam ID - Other options are not precluded. - Association relationship: - Set B of beams are from a subset of Set A of beams - Set B of beams are different from Set A of beams - FFS on QCL relationship - FFS on other aspects |
| vivo | Y |  |
| NEC |  | How to assign resource IDs to beams is up to NW implementation, therefore, even with the same resource IDs, the mapping between set A/B beams could be different. We would like to add - Method to define/represent the association, including: - Based on TCI state - Based on resource ID (e.g., SSB ID, CSI-RS resource ID) - Based on beam ID - Based on beam pattern ID - Other options are not precluded. |
| Xiaomi |  | - Replace “further study” to “consider” - We share same view that it is better to clarify it is for UE side model - We agree to delete “beam ID” and to add “resource set ID” |
| ZTE |  | Agree with Ericsson. The association of Set B and Set A may also be indicated by the CSI reporting setting or resource setting, and thus we have the following modification: - Method to define/represent the association, including: - Based on TCI state - Based on resource/resource set ID (e.g., SSB ID, CSI-RS resource ID) - Based on beam ID - Based on CSI reporting setting - Other options are not precluded. |
| QC |  | Agree with the direction of the proposal, but still a few aspects are not clear. Is the first bullet only applicable to “Set B subset of Set A”? Maybe some examples would be useful here to see how definitions/representations of the association are useful. As mentioned in our contribution, a very important aspect in this direction is ensuring consistency of Set A beams, Set B beams across training and inference. Now, this consistency should be in indexing/ordering of Set B/Set A beams, and also in beam shape associated with Set B/Set A beams. Main question to address is how such association information could help with the above-mentioned consistency. |
| Panasonic |  | We are fine with the updated proposal of InterDigital. |
| CATT |  | We suggest the following update: - Based on resource set ID (e.g., SSB resource set ID, CSI-RS resource set ID) |
| CEWiT |  | We agree with MediaTek and feel that association should be indicated through “resourceConfigID” etc. |
| Google |  | It seems this proposal can be deprioritized. We can figure out whether additional configuration for the association is needed or not after we identify a solution for proposal 3.1.3. |
| Sharp | Yes |  |
| Futurewei |  | Agree with other companies that there is no definition on “beam ID” in existing CSI framework, therefore “Based on beam ID” should be removed. |
| LG |  | Similar view as Ericsson that CSI-reportConfig/CSI-resourceConfig can represent Set A and/or Set B. |
| Lenovo |  | First, it should be clarified that this proposal is applied to UE-side model in the main bullet. Secondly, the beam ID is not clear to us. It should be the same as the resource ID based on the current spec. And the association can be based on resource set ID as HW pointed. Therefore, we suggest the follow proposal: For UE-side model, Further study the association of Set B and Set A, including - Method to define/represent the association, including: - Based on TCI state - Based on resource ID (e.g., SSB ID, CSI-RS resource ID) - Based on resource Set ID - Based on beam ID - Other options are not precluded. - Association relationship: - Set B of beams are from a subset of Set A of beams - Set B of beams are different from Set A of beams - FFS on QCL relationship - FFS on other aspects |
| Fraunhofer | Y |  |

Proposal 3.1.5a(Rx beam assumption)

For NW-sided model, for measurement report, further study whether/how to align the Rx information of the measurements.

Proposal 3.1.5b(Rx beam assumption)

For NW-sided model, for measurement report, “best” or “Quasi-optimal” Rx beam is assumed as current Rx beam assumption for a measurement report.

| Company | Y/N | Comments |
| --- | --- | --- |
| New H3C | Y |  |
| OPPO | Y | Support the FL proposal. |
| Ericsson | Y | Support |
| HW/HiSI |  | The Rx information does not need to be aligned between NW and UE. The UE does not need to provide Rx info the gNB and the gNB can with legacy methods ensure Rx beam consistency. |
| Fujitsu |  | Open for discussion. |
| MediaTek |  | We are generally OK with the reason of this proposal. Since it is for NW-side data collection, there is no mechanism guarantee that UE will do its best effort to find the best Rx beam. However, the meaning of “Rx information” is not very vague to us. To our understanding, NW and UE just need to align with the following cases: “Random Rx”, “Best Rx”, “Quasi-Rx”. Therefore, we suggest the following changes and adding the following notes: For NW-sided model, for measurement report at least for training, further study whether/how to align Rx operating assumption as follows information for of the measurements. - Assumption 1: UE uses “best” Rx beam obtained through exhaustive beam sweeping for each model input sample - Assumption 2: UE uses random Rx beam for each model input sample - Assumption 3: UE uses quasi-optimal Rx beams obtained based on the previous measurements for each model input sample - FFS: mechanism for UE to obtain the quasi-optimal Rx beams - Note: UE-side proprietary/privacy information should be preserved |
| TCL | Y | Support the FL proposal. |
| CMCC | Y |  |
| NTT DOCOMO |  | Support |
| InterDigital | Y | Support |
| vivo |  | This is not just for training. Inference should have the same issue. Hence we suggest the following revision. For NW-sided model, for measurement report at least for training, further study whether/how to align the Rx information of the measurements. |
| NEC |  | Support |
| ETRI | Y | We support |
| Xiaomi | Y | Replace “further study” to “consider” |
| ZTE | Y | Open to discuss. During the study in Rel-18, it is observed that ‘evaluation results from 12 sources show 20%~50% degradation with random Rx beam(s) comparing with the “best” Rx beam in terms of Top-1 prediction accuracy’. Thus, further clarification is needed regarding how to obtain the best or quasi-best Rx beam for measurement reporting. |
| Apple | N | What Rx information should be retained or reported? Consider the training data can be collected by a UE with vendor-1, and essentially the NW-side model is used for a UE with vendor-2? Is some kind of alignment assumed between vendor-1 and vendor-2? |
| QC | N | Regarding this aspect, we have the following text from TR 38.843, Section 6.3.2.5: For NW side model, … - for DL Tx beam prediction, generalization performance with various unseen UE parameters is acceptable at least with the measurement from the best or fixed Rx beam. … It is important to highlight that various assumptions with regards to UE Rx beam selection (best/fixed/random/quasi-optimal) were fundamentally for evaluation purposes, because it was difficult to model Rx UE beam selection during evaluations. Some companies argue that the (generalization) performance of random Rx beam selection is not good, hence the motivation for such proposals, but in practice there is no such thing as “random” Rx beam selection. |
| Spreadtrum |  | Share the same view with ZTE, performance degradation with random Rx beam was shown in R18 study. So we should further discuss how to make sure the best Rx beam is selected and used. |
| Panasonic |  | Regarding “Rx information”, is it related to Rx filter information? Can FL clarify it? |
| CATT | Y | Support |
| CEWiT | Y | We support the proposal. |
| Google | N | We failed to see the necessity for the proposal. The definition of Rx information is also unclear. |
| Sharp | Yes |  |
| Futurewei |  | It is unclear to us the need of Rx information of the measurements. |
| LG |  | The proposal is not clear on its specification impact |
| Lenovo | Y | Support |
| KDDI | Y | Support |

- Model inference

  - NW-sided model

| Company | Proposals |
| --- | --- |
| Spreadtrum [1] | Proposal 6: For BM-Case 1 and BM-Case 2, support UE to report the measurement results of up to 16 beams in one reporting instance. Proposal 7: Reporting multiple past time instances in one reporting instance for BM-Case2 is not needed. Proposal 8: For BM-Case2, implicit indication or report of time information should be considered. |
| Huawei/HiSi[2] | Proposal 10: For NW-side model inference, regarding measurement and reporting (including potential overhead reduction approach), the same applicable enhancements as for training data collection could be supported. Observation 4: For BM-Case1 and BM-Case2 with a NW-side model, the legacy mechanism is sufficient for handling beam indication of beams in Set A not in Set B. |
| Ericsson [3] | Proposal 3 For NW-sided model inference, extend the IE nrofReportedRS in 38.331 to support more than 4 beams, use 256 beams as a starting point (to support BMcase2 with setA equal to setB) Proposal 4 For NW-sided model inference, support NW configuration for Ues to pre-process set B beams to reduce reporting overhead, via: • Support configuring reporting of only beams within X dB of the strongest beam • Support configuring reporting of at most N strongest set B beams Proposal 5 For NW-sided model inference, support methods for Ues to compress the set B temporal domain measurement results to reduce the reporting overhead |
| Vivo [4] | Proposal 12: Support time domain compression of beam resource indication to further reduce report overhead with a report including results of multiple occasions. E.g. considering TRI (time resource indicator) to be reported for each reported beam. Proposal 13: Regarding NW-side model inference to trade off training performance and RS overhead, support P3+P2 resource configuration, e.g., • Perform P3 UE Rx beam search based on multiple Tx beams, which can be configured within a time interval limitation for P2 resource set receiving for set B measurement result acquisition. Proposal 14: Regarding NW-side model inference for improving prediction performance, support to configure multiple pre-configured patterns in Set B. Proposal 15: Regarding NW-side model inference for report overhead reduction, support to report Set B pattern indication if a unique set of beam measurement resources along with multiple supported Set B patterns are transmitted to UE. Proposal 16: Regarding NW-side model inference for useless beam omission, support below methods on beam report enhancement for overhead reduction: • Support to reduce unnecessary beam indices and/or corresponding L1-RSRPs in a report where the omitted L1-RSRP of beams may be smaller than an extra threshold. • Support to report partial measured L1-RSRPs and beam indices with beam index type indicator. Proposal 17: Regarding NW-side model inference on quantization aspects for overhead reduction, • Support to enlarge quantization steps for differential beams. • Multi-resolution L1-RSRP quantization, e.g. high-resolution quantization for a group of best RSRPs and low-resolution quantization for others |
| ZTE[5] | Proposal 8: Support enhancements to report information about measurements of multiple past time instances in one reporting instance for BM-Case2, at least from the following aspects. • Indication of the timestamp information • Indication of the reference beam • Indication of the common beam information Proposal 9: Support a common framework design for the measurement reporting of multiple past time instances for NW-side model and prediction reporting of multiple future time instances for UE-side model. |
| CMCC[7] | Proposal 8: Top K beam sweeping procedure can be introduced and is configurable by gNB. |
| Intel [8] | Proposal 1: For a network-side AI/ML model, for BM-Case 1/2, RAN1 should consider both explicit and implicit configuration of set B for AI/ML model input. The need for configuration of set A should also be further discussed. Proposal 2: For a network-side AI/ML model, for BM-Case 1/2, the network may configure the size of the L1 report based on the measurements from set B. Discuss further if configuration is expected to be explicit or is related to size of set B when or if set B is explicitly configured to the UE. Proposal 3: For a network-side AI/ML model, for BM-Case 2, the network may configure the UE with an observation window and the number of measurements on reference resources related to set B to be reported. The UE should provide L1 reports with measurements associated with related timestamps of measurements. Proposal 4: For a network-side AI/ML model, for BM-Case 2, the UE may not need to be configured with a prediction window. |
| Google [9] | Proposal 8: For BFR based on NW side model for SD beam prediction, after UE declares beam failure, if the UE cannot identify a candidate beam, it can trigger a UE initiated beam report - UE reports L1-RSRP for a set of SSBs/CSI-RSs configured by the NW in the UE initiated beam report - NW performs beam prediction based on the received L1-RSRP Proposal 12: For beam report for NW model inference for temporal beam prediction, UE reports the following information: - L1-RSRP for a set of SSBs/CSI-RSs configured by the NW for each of the measured slots configured by the NW - Hypothetical measurement error for the subset of SSBs/CSI-RSs with L1-RSRP reported Proposal 19: For BFR based on NW side model for temporal beam prediction, after UE declares beam failure, if the UE cannot identify a candidate beam, it can trigger a UE initiated beam report - UE reports L1-RSRP for a set of SSBs/CSI-RSs configured by the NW for multiple measured slots in the UE initiated beam report - NW performs beam prediction based on the received L1-RSRP |
| CATT [10] | Proposal 3: For model inference with NW-side model, the following potential enhancements can be considered. - For BM-Case1, the number of reported L1-RSRP values in one reporting instance can be larger than 4, e.g., 8 - For BM-Case2, the measurements of multiple past time instances can be reported in one reporting instance with enhanced L1-RSRP quantization |
| NEC [11] | Proposal 11: Study overhead reduction mechanisms for beam reports supporting both BM-Case1 and BM-Case2. Proposal 13: For BM-Case2, consider periodic beam report with discontinuous reception. |
| OPPO [13] | Proposal 11: For BM-Case1 with NW-side model, support to reduce the reporting overhead for both fixed or variable Set B, e.g. by dropping the part of SSBRIs/CRIs. Proposal 12: For BM-Case2 with NW-side model, further study how UE reports multiple instances of Set B measurements into one beam reporting instance. |
| InterDigital [14] | Proposal 7: Indicating configuration information associated with Set B to UE should be supported. Proposal 8: Support reporting of UE selected Set B based on a rule (e.g., subset of best measured beams). |
| Samsung [16] | Proposal 3. For NW-side AI/ML model inference, support CSI-ReportConfig with the measurements for more than 4 beams in one reporting instance. - FFS: Overhead reduction - FFS: Whether to support new quantization method Proposal 4. For NW-side AI/ML model inference, support CSI-ReportConfig with measurements for each of multiple past time instances in one reporting instance. - FFS: How to identified the multiple past time instances - FFS: The support of P/SP/AP reporting |
| Fujitsu [17] | Proposal 17: - For BM Case-1 with NW-side model, RAN1 to further discuss whether reporting enhancement is needed for Set B measurement results if the number of beams in Set B is larger than 4. Proposal 18: - For BM Case-1 with NW-side model, there is no need to discuss the beam indication enhancement. Proposal 26: - For BM Case-2 with NW-side model, RAN1 to further discuss whether reporting enhancement is needed for Set B measurement results if the number of beams in Set B is larger than 4. Proposal 27: - For BM Case-2 with NW-side model, RAN1 to discuss that the UE can report the preferred pattern for measurement including the number of measurement instances and the measurement interval. Proposal 28: - For BM Case-2 with NW-side model, similar beam indication enhancement as BM Case-2 with UE side model could be considered. |
| Futurewei [18] | Proposal 5: For Rel-19 AI/ML-based BM, for model inference with a NW-side AI/ML model of BM-Case1 and BM-Case2, support a simple and straightforward extension of the existing L1-RSRP report to support more than 4 beams in one reporting instance. |
| Nokia [19] | Proposal 19: Consider the following for a CSI report that enables beam prediction at the NW, - For BM-Case1 and BM-Case2, consider enhancements for reporting N>4 beams for each reporting instance. - FFS: Values of N limiting additional UCI reporting overhead. - For BM-Case1 and BM-Case2, consider enhancements for Set B L1-RSRP quantization, increasing the differential L1-RSRPs in the report to X dB quantization step. - FFS: Value(s) of X (larger than legacy X>2dB) to reduce UCI reporting overhead. - For BM-Case2, consider enhancements to report multiple past time instances in one reporting instance - FFS: Number of consecutive measurements of beams/RSs to be made between reporting instances. - FFS: whether specific beams/RSs can be reported for each reporting instance. - FFS: whether for BM-Case2 time stamp information can be derived from report based on report configuration. Observation 9: For NW-sided inference, the UE may need to measure Set B and report the measured beams to the NW, e.g., considering enhancements for reporting beam measurements. The NW can use a CSI report to get such beam measurements and reuse legacy RS resource set (resourcesForChannelMeasurement) as the RS set applicable for Set B beams. |
| Lenovo [20] | Proposal 13: Rel-18 CSI reporting framework can be reused for NW-side beam prediction by increasing the number of beams in a beam report. Proposal 14: To Support NW-side AI/ML inference, the gNB can configure one or more CSI reports for the UE to report the L1-RSRPs of all the beams configured in the CMR associated with the CSI report. Proposal 15: Study schemes on differential RSRP report for UCI overhead reduction for larger number of beam report in a beam report. Proposal 16: Study mechanism to adapt/change the Set B configuration or the beam selection for AI/ML model input for RS and UCI report overhead reduction. |
| Apple [24] | Observation 4-2: - Reporting selected beams out of all set B beams rather than reporting all set B beams is beneficial in reducing feedback overhead, which can be supported by bitmap(s) or combinatorial index/indices. - Using a common reference beam across multiple occasions helps reduce feedback overhead. - Consider temporal correlation, the ehaviour of selected un-omitted beams can be shared among occasions, which can be supported by a common bitmap or a common combinatorial index across occasions. Observation 4-3: It is beneficial to support two-part beam reporting to facilitate feedback overhead control for AI/ML-BM. Proposal 4-1: to control feedback overhead, beam reporting for BM Case-1 consists of - Indication of the strongest beam index - Indication of the number of un-omitted beams - The strongest beam’s RSRP - Bitmap to indicate un-omitted beams - Differential RSRPs for uno-omitted beams except the strongest beam Proposal 4-2: to control feedback overhead, beam reporting for BM Case-2 consists of - Indication of the strongest beam index among all occasions - Bitmap to indicate un-omitted/omitted beams - Alt. 1: bitmap size equals to the number of set B beams across occasions - Alt. 2: bitmap size equals to the number of set B beams at a single occasion - Indication of the number of un-omitted beams - The strongest beam’s RSRP - Differential RSRPs for un-omitted beams except the strongest beam |
| CAICT [29] | Proposal 10: More than 4 beams in one reporting instance should be specified for gNB-sided model inference. |
| KDDI [30] | Proposal 1: Existing L1-RSRP reporting signalling should be used as a starting point, at least with respect to data collection for inference. Proposal 2: The number of L1-RSRPs that can be included in one reporting instance needs to be expanded. Proposal 3: The number of representation bits and quantization step size of differential RSRP should be reconsidered. Proposal 4: In considering signalling for inference in the NW-side model, ways to reduce reporting overhead should be considered. Proposal 5: For RSRP quantization, methods to reduce the number of reported bits should be considered within the prescribed beam prediction accuracy. Proposal 6: For the number of beam reporting, ways in which the number of reporting beams can be reduced should be considered within a range that meets the prescribed beam prediction accuracy. Proposal 7: Methods to allow adaptive changes by the UE in the number of reporting beams for overhead reduction should be considered. |
| Indian Institute of Technology Madras (IITM), IIT Kanpur [31] | Proposal 10: For inference data collection for a NW-sided model, the NW configures measurements and corresponding report based on the NW-side model input characteristics. The UE reports the measurements to the NW followed by inference at the NW. |
| Sharp [33] | Proposal 4: Regarding the data collection for various purposes in LCM, the legacy CSI reporting framework can be reused as base line to trigger the UE report for the collected data. - Different purposes in LCM can be associated with respective CSI report configuration. Proposal 5: Support the number of beam measurement results including L1-RSRP and/or beam ID to be larger than 4. The number of beam measurement results is indicated by NW in its associated CSI report configuration. Proposal 6: Regarding the data collection for NW-side AI/ML model training, support both the option 1 and option 2 as below. Indication of which option is used is provided to UE. - Opt.1: M1 L1-RSRPs (corresponding to M1 beams) with the indication of beams (beam pairs) based on the measurement corresponding to a beam set, where M1 can be larger than 4, if applicable - Opt.2: M2 L1-RSRPs (corresponding to M2 beams) based on the measurement corresponding to a beam set, where M2 can be larger than 4, if applicable |
| ITL [34] | Proposal 1: It is proposed for AI/ML model inference to primarily focus on the P2 stage for narrow beam refinement. The initial selection (P1) and Rx beam selection (P3) stages can be also considered relying on the relative benefits in terms of overhead reduction and performance gain. |
| MTK [37] | Proposal 13: To facilitate AI model inference for a NW-side AI/ML model, discuss potential reporting overhead reduction starting from the following direction: - quantizing L1-RSRP with lower number of bits than the current spec. - report the normalized L1-RSRP measurement instead of actual L1-RSRP values. Proposal 14: For NW-side model inference, consider re-using the Rel-18 report sub-configuration framework for UE to report the measurement results of more than 4 beams. |

4.2 UE-sided model

| Company | Proposals |
| --- | --- |
| Huawei/HiSi[2] | Proposal 11: For the CSI report for the inference of a UE-side AI/ML model, the predicted RSRP(s) and the measured RSRP need to be differentiated, since the two types of CSI reports are corresponding to different meanings (e.g., CRI for predicted RSRP may not be subject to the measured beam set). - The two types of CSI reports can be differentiated with an indicator included in CSI-reportConfig. Proposal 12: For AI/ML model inference at the UE-side, the motivation to introduce a report of confidence/probability of the AI/ML output is not clear and can be considered with lower priority unless justified. Proposal 13: For BM-case 2 with a UE-side model, the model output for N future time instances can be sent in one report. - Overhead reduction techniques can be considered, e.g. model output compression over the temporal domain. The time stamp of the reports can be derived implicitly from the order of the prediction instances. Proposal 14: For AI/ML model inference at the UE-side under BM-Case 1 and BM-Case 2, support to report the predicted beam IDs/RSRPs of more than 4 beams in one reporting instance, because • It improves the beam prediction accuracy. • It improves the generalization performance. • It makes the functionality symmetric with the capabilities of a NW-side model. • It enables efficient reporting for BM-Case 2. Proposal 15: For AI/ML model inference at the UE-side under BM-Case 1 and BM-Case 2, to reduce the reporting overhead, study to report an adaptive number of beam IDs/RSRPs determined by the UE, i.e. adaptive values for the Top-K reported beams. |
| Ericsson [3] | Observation 9 Uncertainty can capture a possible data drift in a UE-sided model, and if reported, it can enable NW to configure different number of K beams, or fallback to legacy method. Proposal 17 For UE-sided model inference, specify the following reporting contents - Predicted strongest beam index - Time index prediction is valid, including validity of such prediction (e.g. 10 ms or 20ms) - Uncertainty of prediction - Predicted RSRP Observation 10 Prioritization rules on UCI likely needs to be defined for the UE-sided beam predictions Proposal 16 For UE-sided model inference, enable NW to specify set A beam subset restriction similar to codebook subset restriction (CBSR) that is specified for CSI feedback Proposal 18 For UE-sided model inference, RAN1 to first agree on the content of the beam prediction prior to addressing how such predictions can be part of the existing CSI framework. |
| Vivo [4] | Proposal 12: Support time domain compression of beam resource indication to further reduce report overhead with a report including results of multiple occasions. E.g. considering TRI (time resource indicator) to be reported for each reported beam. Proposal 18: Regarding UE-side model inference, support to configure dataset ID or virtual Tx beam information to ensure consistency between model training and model inference. Proposal 19: Regarding UE-side model inference, support UE to report preferred configurations of DL RS transmission, e.g., • Preferred beam pattern suggestion from UE to NW • Minimum number of beam resource in Set B • Minimum number of resource repetitions for quasi-Rx beam acquisition Proposal 20: Regarding UE-side model inference to trade off training performance and RS overhead, support P3+P2 resource configuration, e.g., • Perform P3 UE Rx beam search based on multiple Tx beams, which can be configured within a time interval limitation for P2 resource set receiving for set B measurement result acquisition. Proposal 21: Regarding UE-side model inference for improving prediction performance, support to configure different pre-configured patterns for different occasions for BM-Case2. Proposal 22: Regarding UE-side model inference on quantization aspects for overhead reduction, • Support to enlarge quantization steps for differential beams. • Multi-resolution L1-RSRP quantization, e.g. high-resolution quantization for a group of best RSRPs and low-resolution quantization for others |
| ZTE[5] | Proposal 11: For BM-Case1 and BM-Case2 with a UE-side AI/ML model, at least support the reporting of predicted RSRP for beam selection at the NW side. Proposal 12: For BM-Case1 and BM-Case2 with a UE-side AI/ML model, if both the predicted RSRP and measured RSRP to the same beam are available at the UE side, the measured RSRP should be reported due to its higher reliability. |
| CMCC[7] | Proposal 9: Regarding inference with UE-side model for BM-Case1 and BM-Case2, support L1 reporting of more than 4 beams and the associated L1-RSRP (if applicable) for at least one of N time instance(s) in one reporting instance. - FFS: values of N (e.g., fixed or variable) Proposal 10: For BM-Case1 and BM-Case2 with UE-side model, whether the predicted L1-RSRP is reported can be configured by gNB, whether/how to differentiate measured L1-RSRP and predicted L1-RSRP needs further discussion. |
| Intel [8] | Proposal 5: For a UE-side AI/ML model, for BM-Case 1/2, explicit configuration of set A and set B should be considered and the configuration may be based on UE capability related to supported model and input/output types. Proposal 6: For a UE-side AI/ML model, for BM-Case 1/2, discuss further if model input/output types should be informed by the UE to the gNB for activated or all supported AI/ML models at the UE-side. Proposal 7: For a UE-side AI/ML model, for BM-Case 1/2, the number of best predicted beams may be configured to the UE by the network based on UE capability and the L1 report from UE to gNB after inference may be defined based on the configured value. Proposal 8: For a UE-side AI/ML model, for BM-Case 2, UE may be configured with an observation window by the network. The prediction window configuration may be based on UE capability on the length of the window that the UE-side model can support. Proposal 9: For a UE-side AI/ML model, for BM-Case 2, the UE should report one or multiple predicted beams per time instance for the configured prediction window. Proposal 14: For model inference of UE-side AI/ML model, consider including information in UL beam report on whether the beam is measured by the UE or the L1 measurement is predicted by the AI/ML model. |
| Google [9] | Proposal 5: For beam report based on UE model inference for SD beam prediction, UE reports the following information: - N beam index(es) based on a configured beam codebook and measurement of a set of SSBs/CSI-RSs configured as CMR Proposal 9: For BFR based on UE side model for SD beam prediction, after UE declares beam failure, if the UE cannot identify a candidate beam, it can report N predicted beam index(es) based on a beam codebook by the BFRQ. Proposal 20: For BFR based on UE side model for SD beam prediction, after UE declares beam failure, if the UE cannot identify a candidate beam, it can report predicted beam index(es) in one or multiple future slots by the BFRQ. |
| CATT [10] | Proposal 4: For indication of the associated Set A from network to UE, both explicit and implicit method can be considered. Proposal 5: For BM-Case1 and BM-Case2 with a UE-side model, consider reporting the predicted L1-RSRP corresponding to the predicted Top-K beams if regression AI/ML model is used. Proposal 6: For BM-Case2 with a UE-side model, consider reporting the predicted beams of multiple future time instances in one reporting instance. - To reduce the reporting overhead, the temporal correlation of the predicted beams of multiple time instances can be considered Proposal 7: For BM-Case2 with a UE-side model, consider RS configuration and L1 reporting enhancement at least for Set B=Set A. |
| NEC [11] | Proposal 13: For BM-Case2, consider periodic beam report with discontinuous reception. |
| Xiaomi [12] | Proposal 4-2: Support to report the predicted L1-RSRP with indication whether it is predicted L1-RSRP or measured L1-RSRP in the beam report. Observation 4-1: The definition of confidence/probability information related to the output of AI/ML model inference should be clarified first. Observation 4-2: For the beam in Set A not in Set B, legacy procedure for Rx beam sweeping can be used to find the best Rx beam first and no new spec impact. |
| OPPO [13] | Proposal 5: For Tx beam prediction (Alt.1) with UE-side model, legacy beam reporting and indication mechanism could serve as starting point. Proposal 6: Predicted L1-RSRP(s) by UE-side model should be reported to NW along with predicted Top-K Tx beam(s). Proposal 7: Confidence/probability of UE-side model output could be quantized and reported to NW. Proposal 8: For BM-Case2 with UE-side model, UE reports the predicted Tx beam for multiple future time instances by single reporting instance. Proposal 9: For BM-Case2 with UE-side model, the timestamp of N future time instance(s) could be implicitly reported to NW. Proposal 10: For BM-Case2, support to indicate multiple beam indications for future N time instances with one-shot indication. |
| Samsung [16] | Proposal 6. For UE-side AI/ML model inference, support UE to send a request for DL RS configuration and/or for DL RS transmission. - FFS: the procedure/contents associated with the request. Proposal 7. For UE-side AI/ML model inference, existing mechanism/framework for the configuration or triggering CSI-RS/SSB resources should be the baseline. - Note: This is for the configuration/triggering of Set B - FFS: configuration to facilitate measurements for multiple past time instance. Proposal 9. For UE-side AI/ML model inference, support CSI-ReportConfig with the following options: - Option 1. Report K predicted beams in one reporting instance. - Option 2. Report K predicted beams and the corresponding predicted L1-RSRP in one reporting instance. - Option 3. Report K predicted beams and the corresponding confidence information in one reporting instance. - FFS: K Proposal 10. For UE-side AI/ML model inference, support CSI-ReportConfig to report predicted beams of N future time instances in one reporting instance - FFS: N Proposal 11. For UE-side AI/ML model inference, support differential L1-RSRP reporting for predicted beams. |
| Fujitsu [17] | Proposal 9: - For BM Case-1 with UE side model, RAN1 to discuss the reference signal configuration for Set B and Set A and possible reporting enhancement. The legacy CSI framework could be a starting point. Proposal 10: - For BM Case-1 with UE side model, the legacy beam indication can work and there is no need to discuss the beam indication enhancement. Proposal 21: - For BM Case-2 with UE side model, regarding configuration of Set A and Set B, similar design framework as BM Case-1 with UE side model could be considered. Proposal 22: - For BM Case-2 with UE side model, RAN1 to discuss that the UE can report the preferred pattern for measurement and prediction, including the number of measurement instances, the number of prediction instances, the measurement interval, and the prediction interval. Proposal 23: - For BM Case-2 with UE side model, RAN1 to discuss beam reporting enhancements considering that the beams for multiple future time instances could be reported. Proposal 24: For BM Case-2 with UE side model, RAN1 to discuss beam indication enhancement, for example, TCI states of multiple time instances could be indicated via one DCI. |
| Futurewei [18] | Proposal 4: For Rel-19 AI/ML-based BM, for model inference with a UE-side AI/ML model of BM-Case1 and BM-Case2, support that NW provides indication of the associated Set A to UE at least for the case in which Set B is a subset of Set A, e.g., association/mapping of beams within Set A and beams within Set B, with a simple ehaviour mechanism. |
| Nokia[19] | Proposal 2: Consider the following for a CSI report that enables beam prediction at the UE, - For BM-Case1, support reporting of “Top-K Predicted-CRIs” or “Top-K Predicted-CRIs, predicted L1-RSRPs” corresponding to a Set A, where K is configurable to the UE. - FFS: Exact values of K - For BM-Case2, support reporting of “Top-K Predicted-CRIs for N time periods” or “Top-K Predicted-CRIs, predicted L1-RSRPs for N time periods” corresponding to a Set A, where K and N are configurable to the UE. - FFS: Exact values of K and N - FFS: whether measured beam related quantities of Set B can be considered within the same beam report - FFS: whether confidence information for predicted beams can be included in the beam report. Proposal 4: For BM-Case1 with the UE-sided model, reuse Rel-17 TCI state activation and indication ehaviour methods. |
| Lenovo [20] | Proposal 4: An indication is needed for the UE to select proper AI/ML model for AI/ML inference for a CSI-ReportConfig used for AI based beam report. Proposal 6: Specify a common beam report configuration to support both BM-Case1 and BM-Case2 for UE-side inference. Proposal 7: For a CSI report configuration for beam report with AI inference, the measurement beam Set B can be configured as the CMR for the UE to obtain the AI/ML model input. How to determine the prediction beam Set A can be further studied. Proposal 8: Study the mechanism for the UE to determine the measurement window and a prediction window for BM-Case2. Proposal 9: RS configuration should be enhanced to support discontinuous RS transmission and adaptation. Proposal 10: Consider to introduce AI process units for AI based operation. Proposal 11: Study the mechanism for beam report associated with AI/ML inference when there is no available AI/ML model inference processing resource. Proposal 12: For a beam report associated with AI inference, the UE indicates that the reported beams are predicted beams or measured beams in the beam report. |
| Sony [21] | Proposal 6 : For the output of AI/ML, the criterion associated with the predicted beam ID in BM-case1 and BM-case2 can be the sum of probabilities of being the best beams being higher than a threshold. Proposal 7 : At the inference stage, the UE should report the quality of input data to the gNB when the measurement results fail to meet the model’s input requirements. |
| LGE [23] | Proposal #5: For the UE AI/ML model training and inference, assistance information on relation/association between Set A beams and Set B beams should be provided to UE. To represent beams in Set A and/or Set B while preserving sensitive proprietary information, consider following exemplary methods: - Set A beams are represented by linear combining coefficients of Set B beams - Tx beam directions are represented as ordered numbers on a 2D or 3D coordinate Proposal #6: For UE-sided model inference, consider reporting of performance monitoring related information such as actual Set A measurement or performance monitoring output/result together with predicted beam information on Set A, where the performance monitoring related information may be reported with longer periodicity. Proposal #8: Support predicted L1-RSRP report together with CRI/SSBRI. For temporal DL Tx beam prediction, information on time-variation of L1-RSRP can also be included in the report for helping intra-/extra-polation at NW side. Proposal #9: For predicted L1-RSRP report, confidence/probability information may be helpful for NW to decide whether/how to use the reported L1-RSRP. Further study whether the information is per model/functionality, per report, or per report parameter. Proposal #10: For temporal DL Tx beam prediction with UE-sided models, following beam reporting enhancements can be considered: - Report of beam(s) for each future time instance or beam(s) for a time duration, i.e., from the first time instance to the last time instance - Report of beam(s) for current time instance for fallback operation - Report of timestamps by UE or NW to indicate timestamps |
| DoCoMo [28] | Proposal 5: Consider predicted L1-RSRP of top-1/K Set A beam(s) and the associated RS indicator(s) as outputs of UE side model in beam prediction. Proposal 6: Consider using CSI reporting framework for data collection for model inference and reporting of inference results. Proposal 7: Reuse multiple CSI-RS resource occasion association for Rel-18 type II ehavio CSI reporting, if applicable. Proposal 8: Enhancements of CSI processing units should be considered for beam prediction. Proposal 9: Support reporting of predicted results at multiple time instances in one UCI reporting to reduce the number of uplink transmissions. Proposal 10: Consider the payload overhead reduction for UE reporting of predicted results at multiple time instances. |
| Indian Institute of Technology Madras (IITM), IIT Kanpur [31] | Proposal 9: For inference data collection for a UE-sided model, the UE reports selected model. The NW then configures measurements at the UE based on the UE-side model input characteristics. Lastly the UE performs inference and reports the model-specific output characterises to the NW |
| Sharp [33] | Proposal 7: For triggering data collection at UE-side AI/ML model, support that UE sends a request for data collection. - FFS: contents and container of the request. |
| ITL [34] | Proposal 1: It is proposed for AI/ML model inference to primarily focus on the P2 stage for narrow beam refinement. The initial selection (P1) and Rx beam selection (P3) stages can be also considered relying on the relative benefits in terms of overhead reduction and performance gain. |
| ETRI [35] | Proposal 6: Support the CSI report format for inference and performance monitoring when an AI/ML model is located on the UE-side. |
| CWEiT [36] | Proposal 7: For model inference of UE sided model, consider signaling to differentiate predicted beam IDs/L1-RSRP report. |
| MTK[37] | Proposal 15: For UE-sided model, consider how to adapt current beamReportTiming framework/definition to include the AI/ML’s model inference delay. Proposal 16: For BM-Case1 and BM-Case2 with a UE-side AI/ML model, study spec impact to facilitate UE to report various number of Top-K beams in one beam report (K ≤ nrofReportedRS) as AI/ML model output. |
| Qualcomm [38] | Proposal 3 For beam prediction of UE-side AI/ML models, identify necessary enhancements to existing CSI report framework (up to Rel-18) to enable the following: • Reporting predicted top-K beam IDs • Report of predicted L1-RSRPs associated with the predicted top-K beams • Report of quantities indicating confidence level of predictions for Top-K beams (e.g., std of predicted L1-RSRPs or statistics of past RSRP measurements as a proxy for confidence level of the predictions) Proposal 4 For UE-side beam prediction, associate a first group of SSB/CSI-RS resources as measurement resources (i.e., Set-B beams) and a 2nd group of SSB/CSI-RS resources as prediction targets (i.e., Set-A beams) with a CSI report wherein the associations are signaled based on signaling associated with the CSI report. Proposal 5 For UE-side temporal beam prediction, define signaling with respect to a CSI report that can indicate which specific multiple future time domain occasions and how many CMRs should be addressed for the respective temporal beam prediction occasions, for the corresponding CSI report. • FFS: overhead reduction methods for such CSI report |

4.3 Beam/TCI indication

| Company | Proposals |
| --- | --- |
| Google [9] | Proposal 6: Support the NW indicates the inactive SSB/CSI-RS for TCI activation/indication - The SSB/CSI-RS configured as QCL source in the TCI state for TCI activation/indication for SD beam prediction are automatically activated after the action time of the TCI activation/indication. Proposal 7: Since the activated/indicated TCI based on SD beam prediction is usually an unknown TCI state, to reduce the latency for TCI activation/indication based on SD beam prediction, support the NW to trigger aperiodic CSI-RS resources QCLed with the SSB/CSI-RS configured as the QCL source in the TCI state. - UE measures time/frequency offset and Rx beam based on the aperiodic CSI-RS resources - UE can also measure the pathloss based on the aperiodic CSI-RS resources Proposal 15: To differentiate the TCI state for legacy beam indication and TCI state for beam prediction, support to configure separate TCI state pools for legacy beam indication and TCI state for beam prediction. Proposal 16: Support to configure the action delay for the TCI state for beam prediction. Proposal 17: For temporal beam prediction, the beam quality for current beam from an indicated TCI can be used for performance validation, and if none of the predicted beam(s) can provide better beam quality than current beam, the predicted beam(s) are assumed to fall to pass the performance validation. Proposal 18: Support UE feedback before the beam action time for performance validation for predicted beam in addition to the ACK/NACK for the TCI update signaling for temporal beam prediction. |
| CATT [10] | Proposal 8: For beam indication of BM-Case2, consider the motivation, necessity, and TCI indication flexibility for indicating TCI states of multiple future time instances using single indication ehaviour. |
| NEC [11] | Proposal 26: Legacy TCI framework needs to be extended to indicate the predicted beam(s). Proposal 27: Support to configure RS in associated Set A as the QCL reference signal in the TCI state. Observation 12: Additional measurements of Top-N UE predicted beams can be performed to allow UE to determine the suitable Rx beam for the predicted beams. However, this operation results in higher signaling overhead and diminishing the AI/ML benefits. Observation 13: If UE has the capability to predict Tx-Rx beam pair, then additional procedure of measuring Top-1 UE predicted beams can be avoided. Proposal 28: Discuss how to reduce the signaling overhead of UE performing measurements of predicted beams to determine the suitable Rx beam. Proposal 29: For BM-Case 2, support to configure the future beam(s) in the TCI state. Observation 14: Either spatial-domain beam prediction or time-domain beam prediction can improve BFD and BFR. Proposal 30: At least for BM-Case 2, support beam failure detection based on time-domain prediction. Proposal 31: Support predicted beam as the newly identified beam q_new. |
| Xiaomi [12] | Proposal 4-5: Support following two TCI state indication mechanism for TCI state indication of more than one predicted time instance. - Option 1: reuse legacy TCI state indication with multiple MAC CE or DCI and each MAC CE or DCI indicates TCI state of one time instance. - Option 2: enhance TCI state indication to indicate TCI state of more than one time instance and the application time gap between two adjacent TCI states can be configured semi-statically. |
| OPPO [13] | Observation 4: For Tx beam prediction with NW-side model, it seems not necessary to enhance the ehaviour, i.e. by combining or associating Tx beam indication and DL Rx beam sweeping. Proposal 13: For BM-Case1 and BM-Case2 with NW-side model, it seems not necessary to associate beam indication with Rx beam sweeping. Proposal 14: For BM-Case2 with NW-side model, support the beam indication for multiple future time instances in a single beam indication. |
| InterDigital [14] | Proposal 1: To indicate Set A beams not in Set B (i.e., unmeasured beams), support indication of Set A beams not in Set B based on a TCI state using a logical beam ID as a QCL Type-D reference RS. Proposal 2: Support a procedure for the UE to obtain QCL-parameters for an unmeasured Set A beam by using neighboring beams of the unmeasured Set A beam. Observation 4: Enhanced beam indication mechanism is needed to enable future beam indication based on prediction of AI/ML model in BM-Case 2. Proposal 3: Support a beam indication mechanism with a beam pattern and corresponding TCI states required for the indicated beam pattern. |
| Samsung [16] | Proposal 5. Support single beam indication for multiple future time instances using unified TCI framework. Proposal 12. For UE-side AI/ML model, support beam indication for Set A beams. |
| Nokia [19] | Proposal 4: For BM-Case1 with the UE-sided model, reuse Rel-17 TCI state activation and indication ehaviour methods. Proposal 5: For BM-Case1 with the UE-sided model, consider enhancements/limitations/changes related to the applicability of the beam indication. - FFS: Details on enhancements/changes when the indicated TCI state is associated with a predicted RS resource (e.g., Applicable channels/signals for the indicated TCI state, Changes to the timelines/definitions on known or unknown TCI states (to be checked with RAN4), etc..). Proposal 6: For BM-Case2 with the UE-sided model, extend the Rel-17 TCI state activation/indication ehaviour methods to activate/indicate N TCI states which are corresponding to future time N instances. - FFS: maximum number for N - FFS: Time periods that each indicated TCI state is applicable. Proposal 7: For BM-Case2 with the UE-sided model, consider enhancements/limitations/changes related to applicability of the beam indication. - FFS: Details on enhancements/changes when the indicated TCI state is associated with a predicted RS resource (e.g., Applicable channels/signals for the indicated TCI state, Changes to the timelines/definitions on known or unknown TCI states (to be checked with RAN4), etc..). Proposal 20: For BM-Case1 with the NW-sided model, reuse Rel-17 TCI state activation and indication ehaviour methods. Proposal 21: For BM-Case2 with the NW-sided model, consider extending the Rel-17 TCI state activation/indication ehaviour methods to activate/indicate N TCI states corresponding to future time N instances. |
| Lenovo [20] | Proposal 1: Support UE initiated beam management procedure for data collection, at least for UE-side model training. |
| Rakuten [22] | Proposal 3: RAN1 discusses and agrees a solution to associate and identify un-transmitted beams predicted by UE in a UE-sided model. Proposal 1: RAN1 agrees the use case where, in a UE-sided model, beams of Set A cannot be always assumed to be transmitted before the prediction. |
| LGE [23] | Proposal #1: For beam indication, TCI/QCL RS should be based on Set B beams of which UE can measure and maintain its Rx beam. Proposal #2: In order to indicate one beam in Set A not in Set B, support indicating multiple neighboring beams from Set B for helping UE to find its Rx beam for the Set A beam. |
| FGI, NTPU [27] | - Proposal 2: For BM Case 1 model, when Set A and Set B are different, how to provide the configuration of the output beam information needs further discussion. - Proposal 3: For BM Case 1 model, when Set B is a subset of Set A, specify Set A and Set B in the same configuration or different configurations. - Proposal 4: For BM Case 1 model, when Set B is a subset of Set A, whether to maintain the beam information of Set B to infer Set A needs further discussion. - Proposal 5: For BM Case 1 model, when Set B is a subset of Set A, how to inform UE of the beam information if the beam information for Set A and Set B is different needs further discussion. - Proposal 6: For BM Case 2 model, when Set A and Set B are the same, beam ID pool and beam related configuration for Set A and Set B should be the same. Proposal 7: For beam reporting information of BM Case 2 model, the range of past time instances or a time window should be defined. |

4.4 Other aspects including common parts for NW-/UE-sided model

| Company | Proposals |
| --- | --- |
| ZTE[5] | Proposal 9: Support a common framework design for the measurement reporting of multiple past time instances for NW-side model and prediction reporting of multiple future time instances for UE-side model. |
| Xiaomi [12] | Proposal 4-3: Report the predicted beam information with the measured beam information of the last measurement time instance together or standalone. Proposal 4-4: Consider one absolute L1-RSRP for each time instance or one absolute L1-RSRP for all time instances in one beam report which including beam reports of more than one time instance for BM-case 2. |
| Sony [21] | Proposal 5 : For use case 2, model input and output should include the dwelling time of predicted candidate beams. Proposal 8 : For use case 2, gNB can indicate to the UE the duration of the time window for collecting input data based on the UE’s movement speed. |
| ITL [34] | Proposal 2: It is proposed to take into account the following key design factors for specifying BM-Case 2: - gNB/UE’s operation: This includes synchronization of measurement windows and inference activities to ensure that the prediction is based on the most recent and relevant data. - Configuration and Signaling - Measurement Window Settings with DL RS measurement configurations - Inference Window and Instance Configurations: The inference window should be set up to allow for adequate time to predict future beam(s), considering the UE’s mobility and channel conditions - Processing Time Minimization with processing time requirement - Feedback Mechanisms |
| ETRI [36] | Proposal 2: Support the method for representing the relationship between multiple beams and the predicted beam from AI/ML beam management. |
| MTK [37] | Proposal 12: For BM-Case2, consider the following potential report configuration ehaviour as a start point: - NW configures periodic beam report with a “duty cycle” parameter - NW configures separate reports for each BM-Case2 model inference instance. |

4.5 First round discussion

Proposal 4.1.1 (contents for measurement report)

For NW-sided model inference, consider the following options for measurement report for inference:

- Opt 1: All L1-RSRP from the resources for Set B of beams, where Set B of beams are for UE measurement and report

  - the max number of Set B of beams can be larger than 4

  - FFS on whether the information on all or a part of Set B of beams/RS ID needs to be reported or not.

- Opt 2: A subset of L1-RSRP from the resources for  Set B of beams, with information on corresponding beam/RS ID

  - where Set B of beams are for UE measurement and report

  - the max number of Set B of beams can be larger than 4

  - FFS on how to determinate the subset, at least including data selection, data omission

- Other options are not precluded.

- [FFS on quantization step and range for L1-RSRP including differential L1-RSRP reporting

- FFS on whether/how to report the information associated with time stamp, including whether to report measurements from multiple time instances in one report instance

- FFS on whether the purpose of the report needs to be specified or not, including implicit and explicit

- FFS on how to configure the RS resources [and the corresponding TCI states] for Set B of beams

  - FFS on fixed or variable Set B pattern

- FFS on measurements are one-shot L1 measurements and/or averaged of L1 measurements over time, or up to UE implementation

- FFS on whether to report the contents of each option in one or multiple measurement reports]

| Company | Y/N | Comments |
| --- | --- | --- |
| New H3C | Y | FFS items are a little bit much and we need categorize them |
| OPPO |  | Support the FL proposal in principle. Regarding one FFS, as we said in previous comment, TCI state is for beam indication, not for beam measurement and report. - FFS on how to configure the RS resources [and the corresponding TCI states] for Set B of beams - FFS on fixed or variable Set B pattern |
| Ericsson | Y | Support the proposal in general. The FFs are a bit to detailed at this stage. We can simplify the FFSs to the following: - Other options are not precluded. - FFS on reporting content for each option and sub-use case (BM case 1 and 2) - FFS on whether the NW need to indicate purpose of report (including set A/B categorization) - FFS on whether existing L1 measurement assumptions are sufficient - FFS on quantization step and range for L1-RSRP including differential L1-RSRP reporting - FFS on whether/how to report the information associated with time stamp, including whether to report measurements from multiple time instances in one report instance - FFS on whether the purpose of the report needs to be specified or not, including implicit and explicit - FFS on how to configure the RS resources [and the corresponding TCI states] for Set B of beams - FFS on fixed or variable Set B pattern - FFS on measurements are one-shot L1 measurements and/or averaged of L1 measurements over time, or up to UE implementation - FFS on whether to report the contents of each option in one or multiple measurement reports |
| HwHiSi |  | For the UE it is not important to know whether a report is from a measured Set B or any other set. The NW just configures a measurement report and determines whether the measurement report for all beams or for a subset should be reported. The FFS are a lot and definitely more study is needed. Suggest to remove the FFSs for now, since otherwise more discussion on other FFS will be probably come. Suggest to write instead “as starting point” into the proposal Proposal 4.1.1 (contents for measurement report) For NW-sided model inference, as starting point further study the following options for measurement report for inference: - Opt 1: All L1-RSRP of a configured Set B of beams, where the configured Set B of beams is are for UE measurement and report - the max number of Set B of beams in the set can be larger than 4 - FFS on whether the information on all or a part of Set B of beams/RS ID needs to be reported or not. - Opt 2: A subset of L1-RSRP of a configured Set B of beams for report, with information on corresponding beam/RS ID - where the configured Set B of beams is are for UE measurement and report - the max number of reported Set B of beams can be larger than 4 - FFS on how to determinate the subset for reporting, at least including data selection, data omission - Other options are not precluded. - FFS on quantization step and range for L1-RSRP including differential L1-RSRP reporting - FFS on whether/how to report the information associated with time stamp, including whether to report measurements from multiple time instances in one report instance - FFS on whether the purpose of the report needs to be specified or not, including implicit and explicit - FFS on how to configure the RS resources [and the corresponding TCI states] for Set B of beams - FFS on fixed or variable Set B pattern FFS on measurements are one-shot L1 measurements and/or averaged of L1 measurements over time, or up to UE implementation |
| Fujitsu |  | Regarding the last bullet, what’s the use case to report the content of Opt 1 or Opt 2 over multiple measurement reports? |
| TCL |  | In general, we support the proposals, but the FFS may be postponed. For the selection of report quantities, the overhead should be considered especially for BM-case2. This can be either achieved by simply reducing the number of reported RSRPs, i.e., Opt 2, or by reporting all RSRPs with compressed/quantized values, i.e., Opt 1. Whether these FFS options are to be discussed relies on the down selection of these two options. |
| CMCC |  | Generally fine with the proposal. |
| InterDigital |  | I think we can remove “the max number of Set B of beams can be larger than 4” as we already agreed this during the SI multiple times. In addition, as commented for data collection for training, FFFs in main bullets are too early and further discussion is needed. |
| vivo |  | - Similar as proposal 4.1.2 for report, we should also discuss the supported time domain behaviors of RS, i.e., whether all three types (P/SP/AP) are supported for measurement. - For BM Case 2, NW side model requires the report of multiple time instances in one report to deliver the measurement results in multiple time instances. Is there any other way to do this? Why is this an FFS point? |
| NEC |  | Support in general and similar comment as to FL proposal 3.1.1. - For second FFS, [whether/how to report the information associated with time stamp] and [whether to report measurements from multiple time instances in one report instance] seem to be two independent FFS? |
| Xiaomi |  | - We suggest to replace “further study” by “consider”. And it will be better to clarify whether down-selection will be done in next meetings or not - According to TR 38. 843, Set B is a set of beams whose measurements are taken as inputs of the AI/ML model. If only a subset of L1-RSRP of set B of beams is reported, how to decide the value of L1-RSRP of beams whose L1-RSRP is not reported? It depends on the rule for subset determination or it depends on NW implementation? But we prefer to strive for a common solution for NW side model and UE side model. - Delete the last two “FFS” since it is for inference. - As for the last 3rd FFS, delete “[and the corresponding TCI states]” |
| ZTE |  | For the 5th sub-bullet, the time stamp information only needs to be indicated explicitly or implicitly if the measurements from multiple time instances are reported in one report instance. Thus, we suggest to modify it as - FFS on whether/how to report measurements from multiple time instances in one report instance, including whether/how to report the information associated with time stamp |
| QC |  | Similar comments as Proposal 3.1.1: As we are talking about the contents of the collected data for inference, aspects related to the reporting should be removed. Additionally, the proposal can be simplified as below: For NW-sided model inference, further study the following options for contents of measurement report for inference: - L1-RSRP (along with beam IDs) of top-M beams from Set B - FFS on M - where Set B of beams are for UE measurement |
| IIT Madras |  | We support Opt1 |
| Spreadtrum | Y | We prefer option1. Because indexes of option1 can be omitted to reduce overhead, and provide more information for the model. For option2, although some measurement value can be reduced, indexes have to be indicated. Thus the overhead reduction is limited. |
| Panasonic |  | Regarding Option 1, it seems “FFS on whether the information on all or a part of Set B of beams ID needs to be reported or not.” can cover Option 2. Can FL clarify it? |
| CATT |  | The second FFS and the last FFS seems duplicate, either one can be removed. |
| CEWiT |  | Support the FL proposal in principle. |
| Google |  | Similar to proposal 3.1.1, we think the options in the proposal needs to be re-formulated From spec impact point of view, the options should be re-formulated. UE reports L1-RSRP for SSB/CSI-RS instead of a beam. So we need to figure out what should be configured as CMR and what should be reported based on the configured CMR. In addition, we think we also needs to consider the impact due to BFR. |
| Futurewei |  | Similar to our comments on Proposal 3.1.1, the FFS are too many/detailed and is mixing the content aspect and how to reduce the overhead of the content together, making it difficult to make progress. Modifications similar to those suggested for Proposal 3.1.1 should also apply here. |
| LG |  | Suggest to discuss on L1-RSRP report enhancement compared to current beam report feature more specifically. The only difference between Opt1 and Opt2 seems whether to report CRI/SSBRI together with L1-RSRP. |
| Lenovo |  | We are general fine with the proposal, but we suggest moving all the FFS points in the next stage. |
| Fraunhofer |  | We propose removing the FFS from the proposal. Furthermore, in Opt.2 it is mentioned that the max number of Set B beams can be larger than 4. However, we think that the subset of Set B beams can also be larger than 4. Hence, we propose the following: - the max number of subset of Set B of beams can be larger than 4 This also implies that the Set B can be larger than 4. |
| KDDI |  | In general, we support the proposal. |

Proposal 4.1.2 (Report)

For NW-sided model, L1 signaling is supported at least for the measurement report associated with Set B for inference.

- FFS: the support of P/SP/AP report

- FFS on MAC

| Company | Y/N | Comments |
| --- | --- | --- |
| New H3C | Y |  |
| OPPO |  | Support the main content. But as for beam reporting for NW-side model inference, we think it’s so time-critical that upper layer signaling i.e. MAC or RRC may bring undesired latency for collecting data for inference. Suggest to remove the last bullet. - FFS: the support of P/SP/AP report - FFS on MAC and/or RRC |
| HW/HiSi |  | Agree with Oppo. |
| Fujitsu |  | For inference operation, we don’t see the need to use MAC/RRC for the reporting and the second sub-bullet could be removed. |
| TCL | Y | We support the proposal. Since the prediction results are guaranteed to be valid in a short period of time in the future, it should not be sensitive to signaling delays. |
| CMCC |  | Agree with Oppo. |
| NTT DOCOMO |  | Support the proposal. |
| InterDigital |  | We generally agree with OPPO, but suggest the following update. - FFS: whether and how to the support of P/SP/AP report - FFS on MAC and/or RRC |
| vivo |  | Agree with OPPO. |
| NEC | Y | Support. |
| Xiaomi |  | - It is better to update it to “at least support L1 signaling for the measurement report for inference.” - We think it also can be used for UE-side model |
| ZTE |  | The second FFS seems contradict with the following Rel-18 agreement in clause 7.1.3 of TR 38.843. Regarding data collection for NW-side AI/ML model of BM-Case1 and BM-Case2, the following reporting signalling for beam-specific aspects maybe applicable: - L1 signalling to report the collected data - Higher-layer signalling to report the collected data - At least not applicable to AI/ML model inference - Note1: higher layer signalling design is up to RAN2 - Note2: Whether each signalling applicable to each LCM purpose is a separate discussion - Note3: The legacy signalling principle (e.g. RSRP reporting for L1) can be re-used. |
| QC |  | Despite the fact that Proposal 4.1.1 talks about the contents, it is important to highlight that the measurement report is associated with Set B. The proposal is about L1 signaling for NW-side model inference. The justification for second bullet is not clear. Updated Proposal 4.1.2: For NW-sided model, L1 signaling is supported at least for the measurement report associated with Set B for inference. - FFS: the support of P/SP/AP report - FFS on MAC and/or RRC |
| IIT Madras |  | We support the FL’s proposal |
| Spreadtrum | Y | Agree with OPPO. L1 signaling should be enough. |
| Panasonic |  | We are fine with the proposal. |
| CATT |  | Agree with OPPO |
| Google |  | We think MAC CE should be used for BFRQ and P/SP/AP report can be used for beam report, similar to legacy. |
| Sharp | Y | Support |
| Futurewei |  | Agree with OPPO. |
| Lenovo |  | We are general fine with to support L1 based report at the early stage, but the door of higher layer based report should not be closed at this stage. We suggest the following update: For NW-sided model, L1 signaling is at least supported at least for the measurement report for inference. - FFS: the support of P/SP/AP report - FFS on MAC and/or RRC |
| Fraunhofer | Y |  |
| KDDI | Y | Support |

Proposal 4.1.3 (contents for prediction results)

For UE-sided model inference, further study the following options for report of inference results:

- Opt 1: Information on Top K predicted beam among Set A of beams

  - FFS on K

  - where Set A of beams are the beams for UE prediction

- Opt 2: Information on Top K predicted beam among Set A of beams, and each corresponding L1-RSRP of of Top K beam(s)

  - FFS on K

  - FFS on whether and how to indicate the L1-RSRP is predicted and/or measured

  - FFS on how to determinate the subset of Top K beam(s) when applicable

  - where Set A of beams are the beams for UE for prediction

- Opt 3: Information on Top K predicted beam among Set A of beams, and each corresponding confidence/probability information of  Top K beam(s)

  - FFS on K

  - FFS on the definition and quantization method of confidence/probability information

  - FFS on how to determinate the subset of Top K beam(s) when applicable

- Other options are not precluded.

- [Note: The top K predicted beam are derived based on the measurement of Set B of beams.

- FFS on quantization step and range for L1-RSRP including differential L1-RSRP reporting

- FFS on the report of prediction results of one or multiple time instances in one report instance

- FFS on whether the purpose (e.g., measurement results or prediction results) of the report needs to be specified or not, including implicit and explicit

- FFS on how to define or configure beam/RS ID information for Set A and/or Set B]

| Company | Y/N | Comments |
| --- | --- | --- |
| OPPO |  | In general, we are supportive to the inference results. As for Opt2/Opt 3, for Top K beam(s), we don’t see a reason why a subset of corresponding L1-RSRP/confidence/probability is predicted. To be aligned with legacy beam reporting, at least for Opt2, each of corresponding L1-RSRP should be predicted. Hence, we suggest to modify accordingly as below. - Opt 2: Information on Top K predicted beam/RS ID among Set A of beams, and each corresponding L1-RSRP of [each or a subset] of Top K beam(s) - Opt 3: Information on Top K predicted beam/RS ID among Set A of beams, and each corresponding confidence/probability information of [each or a subset] of Top K beam(s) |
| Ericsson |  | Support in general this proposal. The FFS can be simplified, for example if the report will be implicit or not will depend how we introduce the report in the CSI framework. Also, we don’t need to discuss the exact content at this stage. Hence we propose. - FFS on reporting content for each option, and for each use case (BM case 1 and2) - FFS on how the report will be introduced in the CSI framework, - FFS on how beam set A/B will be introduced in the CSI framework, - FFS on quantization step and range for L1-RSRP including differential L1-RSRP reporting - FFS on the report of prediction results of one or multiple time instances in one report instance - FFS on whether the purpose (e.g., measurement results or prediction results) of the report needs to be specified or not, including implicit and explicit FFS on how to define or configure beam/RS ID information for Set A and/or Set B |
| HwHiSi |  | Same comment as before on the number of FFS. Support Opt.1 and Opt.2. Do not agree with Option 3. Regarding the confidence/probability information related to the AI/ML model output, it is our understanding that the report of the predicted beam ID would already provide the needed information to the gNB to obtain the best beam information irrespective of the output type (predicted probability or predicted RSRP). After receiving the predicted beam ID, the NW can simply take a post inference beam sweeping to obtain the measured RSRP of the predicted beam. Furthermore, the confidence of the predicted RSRP does not contribute meaningful information to the gNB unless it would be defined in a RAN4 requirement (which is not likely); otherwise, different UEs may report different confidence values to the same beam (e.g., for the same group of measured Set B as model input, different models may derive different confidence values for the same beam) and the gNB would have no idea how to use the divergent information. Therefore, we think Opt3 needs more evidence it can be captured under “other options are not precluded” Proposal 4.1.3 (contents for prediction results) For UE-sided model inference, as starting point further study the following options for report of inference results: - Opt 1: Information on Top K predicted beam/RS ID among Set A of beams - FFS on K - where Set A of beams are the beams for UE prediction - Opt 2: Information on Top K predicted beam/RS ID among Set A of beams, and corresponding L1-RSRP of [each or a subset] of Top K beam(s) - FFS on K - FFS on whether and how to indicate the L1-RSRP is predicted and/or measured - FFS on how to determinate the subset of Top K beam(s) when applicable - where Set A of beams are the beams for UE for prediction - Opt 3: Information on Top K predicted beam/RS ID among Set A of beams, and corresponding confidence/probability information of [each or a subset] of Top K beam(s) - FFS on K - FFS on the definition and quantization method of confidence/probability information - FFS on how to determinate the subset of Top K beam(s) when - Other options are not precluded. - Note: The top K predicted beam/RS ID are derived based on the measurement of Set B of beams. - FFS on quantization step and range for L1-RSRP including differential L1-RSRP reporting - FFS on the report of prediction results of one or multiple time instances in one report instance - FFS on whether the purpose (e.g., measurement results or prediction results) of the report needs to be specified or not, including implicit and explicit - FFS on how to define or configure beam/RS ID information for Set A and/or Set B |
| Fujitsu |  | Generally fine with the formulation. And we prefer with Opt 1 and Opt 2. |
| TCL |  | In general, we support the proposal. We suggest during the early stage, the reporting quantities concluded in this proposal could be a starting point, and other options are not precluded. |
| CMCC |  | According to “FFS on the report of prediction results of one or multiple time instances in one report instance”, Proposal 4.1.3 seems including BM case2, but reporting content now does not include time domain information of prediction results. Does it imply that time domain information is implicitly included in the content? Agree to postpone option3 after justify the benefit of option3. |
| NTT DOCOMO |  | The following FFS can be removed as it is discussed in Proposal 3.1.3. - FFS on how to define or configure beam/RS ID information for Set A and/or Set B |
| InterDigital |  | Too early for FFSs in main bullets. |
| vivo |  | Support in General. We should focus on what should be reported rather than how to report in data content discussion. Further, maybe confidence or probability can be predicted as AI model output for UE classification model. It can be implicit indicated by report Top-k beams. Thus, the benefit of confidence/probability report is not clear. For BM case 2, prediction result report needs to convey beam report for multiple time instances. It shouldn’t be an FFS point, right? Therefore, we suggest the following change on this bullet. - Opt 3: Information on Top K predicted beam/RS ID among Set A of beams, and corresponding confidence/probability information of [each or a subset] of Top K beam(s) - FFS on K - FFS on the definition and quantization method of confidence/probability information - FFS on how to determinate the subset of Top K beam(s) when - For BM Case-2, support the report of multiple predicted occasions in one report instance. Consider explicit or implicit time resource indicator for time stamp acquisition. |
| NEC | Y | The following note is not needed. Note: The top K predicted beam/RS ID are derived based on the measurement of Set B of beams. |
| Xiaomi |  | - We suggest to replace “further study” by “consider”. And it will be better to clarify whether down-selection will be done in next meetings or not - As for Opt 2, both Top K predicted beam/RS ID and corresponding L1-RSRP of [each or a subset] of Top K beam(s) are the contents of the model output of a single model? - For Opt 3, since the definition of confidence/probability is not clear, and in our opinion, it is possible to report the common confidence/probability for all Top K beam(s). so we suggest to update to “and corresponding confidence/probability information of [each or a subset or all] of Top K beam(s)”. and a typo in the last FFS, should be “when applicable” - What is the motivation of the note? - The last second FFS has overlap with the first FFS of Opt 2, suggest to delete the last second one. |
| ZTE |  | Agree with OPPO to delete ‘[each or a subset]’. For the 2nd FFS in opt 2, the reporting of predicted RSRP or measured RSRP depends on whether the associated beam is from Set B. For example, if a beam to be reported is from Set B, the measured RSRP should be reported due to its higher reliability. Otherwise, the predicted RSRP should be reported. Additionally, due to potential changes in UE behavior and communication environment, the model may not always be able to accurately predict the optimal beam information, and in this case, the UE reporting of beam ID or beam quality information can be based on measurement results of Set B instead of prediction results of Set A. Therefore, apart from the three listed options, we suggest to add another FFS for potential UE reporting: - FFS on the report of measurement results of Set B |
| QC | Y |  |
| Panasonic |  | For Option 3, it is not clear a motivation of reporting “confidence/probability”. Can proponents explain the motivation for it? |
| CATT |  | We support Opt 1 and Opt 2. For Opt3, we don’t think the confidence/probability information can facilitate network obtain more information. Because if a beam is with a low confidence/probability, it doesn’t mean the beam can’t be the best beam. |
| Google |  | We think “predicted beam/RS ID” should be changed into “predicted beam ID”. It is hard for UE to predict an RS ID. |
| Futurewei |  | Support Opt 1 and Opt 2. Suggest removing Opt 3 as it is difficult to define and test the confidence/probability information. FFS should be removed as commented before. |
| LG |  | Suggest to discuss whether the predicted Set A beams and measured Set A beams can be reported in one report instance or not. I think it is related with Opt 2. |
| Lenovo |  | For opt 2, the motivation of predict L1-RSRP of a subset of top-K beams is not clear to us, suggest to remove it. Based on the discussion in SI, we understand that report of prediction results of multiple time instances in one report instance should be supported for BM-Case2 other than a FFS. |
| Fraunhofer | Y |  |

Proposal 4.1.4 (Beam indication)

Further study on predicted beam indication, including the following aspects:

- The indication of predicted beam in Set A

  - FFS on whether the predicted beam is associated with RS resource for Set B of beams

  - FFS on whether/how to extend the timeline methodologies for unknown TCI state for the predicted beam indication

- Beam indication of multiple future time instances

- Whether it is based on unified TCI state framework

| Company | Y/N | Comments |
| --- | --- | --- |
| New H3C | Y |  |
| OPPO | Y | Support the FL proposal. |
| Ericsson |  | This is for both UE and NW-sided models? We prefer to separate the UE-sided and NW sided bullets before discussing the bullets above. |
| HW/HiSi |  | For clarification, what is meant with a unified TCI state framework? Is it meant unified for Case-1 and Case-2 or unified for UE-side and NW-side model? For a NW-side model, we think an indication of multiple future time instances is restrictive. It is more flexible to indicate the beam in DCI prior to PDSCH transmission. That means for NW-side model, we do not need to indicate multiple future times instances. Just one time instance is enough, this could be the unified TCI state approach for the NW-side model, which is the same as in legacy. For a UE side model, all future time instances can be send in one report after inference. Thus for UE-side we would support reporting multiple future time instances. But we are not sure whether this affects the TCI state, the TCI state is a separate discussion. Further discuss study on predicted beam indication, including the following aspects: - The indication of predicted beam in Set A - FFS on whether the predicted beam is associated with RS resource - FFS on whether/how to extend the timeline for unknown TCI state for the predicted beam indication - For UE-side model Beam indication of multiple future time instances Whether it is based on unified TCI state framework |
| Fujitsu |  | Regarding “the indication of predicted beam in Set A”, there was the following observation from RAN1 #114: Observation At least for BM-Case1 with a UE-side AI/ML model, for AI model inference, the legacy TCI state mechanism can be used to perform beam indication of beams So what is the use case for the first bullet in Proposal 4.1.4? |
| MediaTek | N | We do not support this proposal. We believe the current framework is sufficient to perform beam indication. UE requires additional measurement if there is any unknown TCI state. Also, this issue only happens when model output is Top-1 beam prediction (for Top-K prediction, addition measurements are inevitable to find the best in Top-K). Since additional measurement for just 1 beam will not induce too much overhead, we failed to see the needs for this proposal. |
| TCL | Y | We support the proposal. The TCI framework could be reused smoothly if predicted beam is associated with the RS resource. |
| CMCC |  | Existing timeline for unknown TCI state switching includes Rx beam refinement to determine Rx beam and acquire QCL information, the extension of timeline seems not needed. The motivation/benefit of beam indication of multiple future time instances needs justification before we support it. |
| vivo | Y | Support |
| NEC | Y |  |
| Xiaomi |  | - We suggest to replace “further study on” by “consider”. - Is it for both NW-side model and UE-side model? If yes, please clarify in the proposal. - For the first FFS of the first sub-bullet, does it mean “FFS on whether the predicted beam is associated with RS resource in set B” - As for “the indication of predicted beam in set A”, we think legacy procedure can be reused, i.e., by performing Rx beam sweeping before beam indication. Thus we suggest to add “if necessary” in the main bullet. |
| ZTE |  | Fine in general. For the first FFS, our understanding is that the predicted beam is always associated with a RS resource in Set A. |
| QC |  | We think what is meant by the second sub-bullet is “extend the timeline methodologies” rather than “extend the timeline”, right? Hence the update below: FFS on whether/how to extend the timeline methodologies for unknown TCI state for the predicted beam indication |
| Spreadtrum | Y | We think the second FFS is not needed. |
| Panasonic |  | We would like to clarify whether this proposal is to target to UE-sided model or NW-sided model or both of them? |
| CATT |  | We support first and second bullet. But for third bullet, the motivation and use case should be further clarify. From the network perspective, it may decide to change the Tx beam for transmissions. To preserve the TCI indication flexibility of the network, the indicated TCI state should be able to be updated before it is applied. |
| Google |  | We think it is important to reduce the unknown TCI activation latency. For AI/ML based beam prediction, the indicated TCI is usually unknown TCI. The latency could be so big that the whole feature could become useless. In addition, we think it is necessary to study BFR related aspect, especially CBD. |
| Sharp | Y | We support the proposal. Both network and UE side models should be considered. Multiple future time instances should be reported. Discuss how the unified TCI framework maybe used to indicate and activate the TCI states associated with beam indication of future time instances. |
| Futurewei |  | Existing CSI framework including unified TCI framework should be reused as much as possible. |
| Lenovo |  | Does the following bullet need to be discussed in RAN4? - FFS on whether/how to extend the timeline for unknown TCI state for the predicted beam indication |
| Fraunhofer | Y |  |

Proposal 4.1.5 (AI process units)

For UE-sided model, further study whether to define AI process time including re-use or modified the existing CSI computation time.

| Company | Y/N | Comments |
| --- | --- | --- |
| New H3C | Y |  |
| OPPO |  | Open to discuss. |
| Ericsson |  | Ok to discuss, but with lower priority. |
| HW/HiSi |  | Interesting question that we need to look into, but probably a little later. |
| Fujitsu |  | Open for discussion. |
| TCL |  | Open for discussion. |
| CMCC |  | Open for discussion. |
| NTT DOCOMO |  | Support. |
| vivo |  | In our understanding, not just the process time, but also the number of simultaneous processes need to be considered. Hence, we suggest the following revision. For UE-sided model, further study whether to define AI process capability including re-use or modified the existing CSI computation time and CSI processing units. |
| NEC | Y |  |
| ETRI |  | Open to discuss AI process time |
| Xiaomi | Y |  |
| ZTE |  | Support |
| QC | Y |  |
| Panasonic |  | Open for the discussion. |
| CATT |  | It seems that the AI process time is also applicable to other use cases. Whether it is proper to discuss it in AI 9.1.1? |
| Google |  | Support vivo’s change. |
| Futurewei |  | Open to discuss but with lower priority. |
| LG | Y | It seems that this proposal is related to processing ‘unit’ not ‘time’. Thus, update is needed, i.e. - AI process time AI/ML processing unit (APU) - CSI computation time CSI processing unit (CPU) |
| Lenovo |  | Agree with vivo. |
| Fraunhofer | Y | We support the proposal. |

- Performance monitoring

| Company | Proposals |
| --- | --- |
| Spreadtrum [1] | Proposal 9: For BM-Case1 and BM-Case2, Alt.1 and Alt.4 can be used as the performance metric. Proposal 10: For BM-Case1 and BM-Case2, the configuration of the set (Alt.1: The best beam(s) obtained by measuring beams of a set indicated by gNB) should be further considered. - The measurement overhead should be reduced on the premise that the best beam is included. |
| Huawei/HiSi[2] | Proposal 16: For the performance metrics of monitoring, the discussion on spec impact for input or output data based monitoring (Alt.3) and RSRP difference evaluated by comparing measured and predicted RSRP (Alt.4) can be deprioritized. Proposal 17: For benchmarks of AI/ML model monitoring, spec impact (if any) should only be discussed for a UE-side model. The benchmarks for the NW-side model are up to implementation. Proposal 18: For benchmarks of UE-side AI/ML model monitoring, the spec impact (if any) should be studied for Alt.1, - It should be possible for the gNB to configure one or multiple sets for one or multiple benchmarks. Potential reporting of the multiple benchmarks could be carried out in one instance. - For Alt.4, the benefits need further justification – if the predicted Top-1/K beam ID(s) differ a lot from the genie-aided best beam, the RSRP accuracy would be obtained from irrelevant beams and does not provide useful information. |

5.1 NW-sided model

| Company | Proposals |
| --- | --- |
| Huawei/HiSi[2] | Proposal 19: For monitoring the NW-side model, no additional specification impact on top of the data collection for training and inference is needed. |
| Ericsson [3] | Proposal 11 For NW-sided monitoring data collection, reuse mechanisms from training or inference data collection, no need to discuss performance monitoring of NW-sided models. |
| Vivo [4] | Observation 5: Significant UCI overhead reduction can be achieved by UE-assisted model monitoring compared to direct measurement results report for NW-side AI/ML model. Proposal 24: Support UE-assisted model monitoring for NW-side AI/ML model to reduce UCI reporting overhead, i.e., UE reports monitoring KPIs for NW side model monitoring. Proposal 25: Regarding NW-side model monitoring to trade off training performance and RS overhead, support P3+P2 resource configuration, e.g., • Perform P3 UE Rx beam search based on multiple Tx beams, which can be configured within a time interval limitation for P2 resource set receiving for set B and Set A measurement result acquisition. Proposal 26: Regarding NW-side model monitoring, support to report performance metrics instead of measured results report for further UCI overhead reduction. Proposal 27: Regarding NW-side model monitoring, support to define new default beam procedure for model switching/model deactivation/model activation |
| CMCC[7] | Proposal 11: NW-side monitoring of NW-side AI/ML model is supported. |
| Intel [8] | Proposal 15: For network-side AI/ML models, consider UE event triggered model monitoring with periodic or aperiodic measurement on set B of beams. |
| Google [9] | Proposal 4: For beam report for NW model monitoring for SD beam prediction, UE reports the following information: - SSBRI/CRI and L1-RSRP for the best SSB/CSI-RS from the SSBs/CSI-RSs configured as CMR - L1-RSRP for a subset of SSBs/CSI-RSs configured by the NW Proposal 13: For beam report for NW model monitoring for temporal beam prediction, UE reports the following information: - SSBRI/CRI and L1-RSRP for the best SSB/CSI-RS from the SSBs/CSI-RSs configured as CMR for one or multiple slots configured by the NW - L1-RSRP for a subset of SSBs/CSI-RSs configured by the NW |
| CATT [10] | Proposal 11: For performance monitoring of NW-side model, the reporting contents are relevant to the performance metric and can be configured by the network. Proposal 12: For performance monitoring of NW-side model, UE can report multiple measurement results of benchmarks/reference in one reporting instance. |
| NEC [11] | Observation 4: For model testing and performance monitoring for NW-side model, if there isn't a substantial need for vast amounts of field data, leveraging the existing CSI report framework to report measured L1-RSRP may be advantageous. Proposal 22: At least for BM-Case2 performance monitoring, study the method to configure the associated measurement and report resources for obtaining the ground truth in future time instances, and the method to configure the associated measurement and report resources for obtaining the historical measurement results as model input. Observation 8: Monitoring based on data distribution can accurately identify which (part of) beams suffer from inaccurate predictions, which is crucial for determining whether to disable the model/functionality and for reasonable model update. Proposal 23: Monitoring based on data distribution should be supported. |
| Xiaomi [12] | Proposal 5-7: For performance monitoring of network-side AI/ML model, support to report measurement results of set B and set A separately. Set B can be reported based on beam report, and set A can be reported by MAC CE or RRC with multiple samples. Proposal 5-8: For performance monitoring for network-side AI/ML model, support an event-triggered report if the indicated TCI state is different from the best beams obtained by measurements. |
| OPPO [13] | Observation 5: For BM-Case1 and BM-Case2 with a network-side AI/ML model and monitoring, there seems no additional specification impact on LCM. |
| InterDigital [14] | Observation 5: The UE’s frequent reporting of AIML input measurements for inference at gNB greatly increases UE reporting overhead. Observation 6: Beam prediction accuracy with sparse reporting (e.g., reporting odd/even beams alternatively or best 50% of beams) is as good as baseline case (i.e., reporting all beams in Set B). Proposal 4: Support a sparse reporting mechanism for AIML inputs (e.g., based on channel conditions). Proposal 5: Information about the time stamp for measurement instances should be supported. |
| Fujitsu [17] | Proposal 19: - Regarding NW-side monitoring for BM Case-1 with NW-side model, RAN1 to further discuss the performance metric, and the following alternative is preferred. - Alt.4: The L1-RSRP difference evaluated by comparing measured RSRP and predicted RSRP. Proposal 20: - Regarding NW-side monitoring for BM Case-1 with NW-side model, RAN1 to further discuss the reference signal configuration and possible reporting enhancement, e.g., quantization of L1-RSRP. Proposal 29: - Regarding the performance monitoring for BM Case-2 with NW-side model, similar scheme as BM Case-1 with NW-side model could be considered. |
| Nokia [19] | Observation 10: For NW-sided performance monitoring for beam prediction at the NW, the NW may configure a single CSI report for both inference and monitoring, where an additional RS set may be required to be considered as the monitoring RS resource set. This may make the performance monitoring visible to the UE. Monitoring may not be visible if the NW configures a different CSI report as in the legacy beam measurement and reporting frameworks. Observation 11: For NW-sided performance monitoring for beam prediction at the NW, the NW may require a different CSI report configuration for monitoring. Monitoring may not be visible if the NW configures a different CSI report. Proposal 22: For BM-Case1 and BM-Case2 with the NW-sided model, to enable the NW-sided performance monitoring, further discuss following variants, - Case1: No enhancement is needed to support NW-sided performance monitoring. - E.g., the NW can use a different CSI report to get beam measurements/reporting for a monitoring RS resource set (as NW prefer) within the legacy CSI reporting framework. - Case 2: NW is using the same CSI reporting configuration for monitoring and inference. - Monitoring RS resource set is configured/indicated separately from Set B. For the monitoring RS resource set, the NW may configure separate reporting timelines and reporting quantities. |
| Lenovo [20] | Proposal 17: For NW-side AI/ML model performance monitoring, support Tx beam repetition for the UE to report the best L1-RSRP of a Tx beam among all its Rx beams. |
| Apple [24] | Proposal 3-3: L1 beam reporting for performance monitoring for NW-side model is supported. |
| Indian Institute of Technology Madras (IITM), IIT Kanpur [31] | Proposal 12: For monitoring data collection for a NW-side model, the NW configures measurements for model monitoring. The UE then collects the monitoring data (which could include ground truth). NW computes the model output, computes the KPIs and makes a decision regarding model activation/deactivation/switching/fallback |
| MTK [37] | Proposal 24: For BM-Case1 and BM-Case2 with a NW-side AI/ML model, consider BM-specific conditions/additional conditions of UE for functionality(ies) and/or model(s) at least from the following aspects: - data collection (e.g., conditions on data omitting/filtering methods, conditions on the reporting contents) - assistance information |

5.2 UE-sided model

| Company | Proposals |
| --- | --- |
| Huawei/HiSi[2] | Proposal 20: For UE-side model monitoring, consider all 3 options further and assess their potential spec impact: - For Type 1 Option 1, UE reports the label and inference output. - For Type 1 Option 2, UE reports calculated metric. The relevant spec impact includes: - The metric calculation approach, e.g., per sample report, or report of the statistical value. - The type of metric, e.g., prediction accuracy, or RSRP gap to the genie-aided best beam. - For Type 2, UE does not report CSI, but reports the monitoring decision (e.g., activation/deactivation/fallback) to NW. - For Type 2, gNB may configure a threshold criterion to facilitate the UE to perform model monitoring. Proposal 21: Functionality based LCM is appropriate for UE-side model of BM-Case 1/2. - Note: Whether model based LCM is applicable to UE-side model can be further clarified and is discussed in other agenda items. Discussion in 9.1.1 should be deferred at least until end of Q3/2024. Proposal 22: For UE capability report of the condition for UE-side model, discuss the report of supported/needed configurations, including at least: - The number of the needed data samples for training/monitoring. - The supported configurations of RS/CSI report for Set A and/or Set B for model training/monitoring/inference. - The supported values of Top-K for inference. Proposal 23: Additional conditions could be discussed with lower priority after its content, necessity and feasibility are clarified. |
| Ericsson [3] | Proposal 20 Regarding type-1 NW performance monitoring of UE-sided models, specify both Option 1 and Option 2 Proposal 22 Regarding the latency requirement for UE-sided performance metric report, further study the need for NW real-time monitoring for the beam prediction use case |
| Vivo[4] | Proposal 28: Regarding UE-side model monitoring, support below enhancement on monitoring request signaling, • Minimum number of requested beams • Minimum number of requested repetitions Proposal 29: Regarding UE-side model monitoring, support to configure dataset ID or virtual Tx beam information to address NW-side proprietary/privacy information disclosing issue. Proposal 30: Regarding UE-side model monitoring to trade off training performance and RS overhead, support P3+P2 resource configuration, e.g., • Perform P3 UE Rx beam search based on multiple Tx beams, which can be configured within a time interval limitation for P2 resource set receiving for set B and Set A measurement result acquisition. Proposal 31: Regarding UE-side model monitoring, support to report performance metrics instead of measured and predicted results report for further UCI overhead reduction. |
| ZTE[5] | Proposal 15: Support Type 1-2 performance monitoring (i.e., hybrid performance monitoring) as it achieves the best trade-off between reporting overhead and system performance assurance. Proposal 16: Type 2 performance monitoring (i.e., UE-side performance monitoring) can only be supported if the UE is authorized by the NW for functionality or model operations. Proposal 17: Model/functionality failure detection should be based on monitoring results of several consecutive times within a predefined monitoring window. Proposal 18: UE reporting based on measurement of Set B can serve as an always-on fallback method to guarantee continuous services quality. |
| CMCC [7] | Proposal 12: For UE-side AI/ML model, UE-side monitoring, NW-side monitoring and hybrid monitoring mechanisms can be considered. Proposal 13: Regarding UE-side model monitoring of UE-side AI/ML model, NW may configure a threshold criterion to facilitate UE to perform model monitoring. The specification impact of decision and configuration reporting, and decision acknowledgement mechanism are considered. Proposal 14: Regarding hybrid model monitoring of UE-side AI/ML model, NW may configure a threshold criterion to facilitate UE to perform model monitoring. The specification impact of performance metric or event reporting, and configuration reporting are considered. |
| Intel [8] | Proposal 16: For UE-side AI/ML models, consider UE-event-triggered model monitoring. Proposal 17: For model monitoring for BM-Case-2, further consider if the time window for model monitoring should be identical to observation window for model inferencing. |
| Google [9] | Proposal 14: For beam report based on UE model inference for temporal beam prediction, UE reports the following information: - N beam index(es) for one or multiple configured predicted slot(s) based on a configured beam codebook and measurement of a set of SSBs/CSI-RSs configured as CMR |
| CATT [10] | Proposal 13: For Type 1 performance monitoring of UE-side model, taking Option 2 with higher priority for discussion. Proposal 14: For Type 2 performance monitoring for UE-side model, the request signaling for performance monitoring should indicate the required RS for performance monitoring. Proposal 15: For Type 2 performance monitoring for UE-side model monitoring, UE can recommend LCM decision and final decision should be made by NW to avoid misunderstanding between UE and NW. |
| NEC [11] | Proposal 18: Support NW triggered performance monitoring. At least for UE-side model, support UE initiated performance monitoring. Observation 7: AI/ML performance monitoring is only needed when the legacy procedures like BFD, RLM detected a physical layer problem. Proposal 19: Support UE to initiate performance monitoring of current AI/ML model if beam failure instance indication or out-of-sync indication is received. Proposal 20: Support UE to activate candidate models and to initiate performance monitoring of candidate models before model switching or model selection. Proposal 21: Study simultaneous performance monitoring for multiple candidate models. Observation 8: Monitoring based on data distribution can accurately identify which (part of) beams suffer from inaccurate predictions, which is crucial for determining whether to disable the model/functionality and for reasonable model update. Proposal 23: Monitoring based on data distribution should be supported. |
| Xiaomi [12] | Proposal 5-3: For UE-side AI/ML model, Option 2 (UE-assisted performance monitoring) of Type 1 performance minoring should be supported at least. Option 1 (NW-side performance monitoring) of Type 1 performance minoring can be considered with enhancement on signaling overhead reduction. Proposal 5-4: For UE-side AI/ML model, NW-side initiated performance monitoring based on measurement/report configuration via RRC can be considered for Type 1 performance monitoring. Proposal 5-5: For UE-side AI/ML model, UE-side initiated performance monitoring based on RA or SR can be considered for Type 2 performance monitoring. Proposal 5-6: For UE-side AI/ML model with Type 2 performance monitoring, it is better to indicate UE’s decision to NW for consistency of the NW-side additional condition for the new applied UE-side model. |
| OPPO [13] | Proposal 17: For BM-Case1 and BM-Case2 with UE-side model, at least support the (Alt1) UE-side model monitoring. |
| InterDigital [14] | Proposal 6: Reporting prediction results of multiple future time instances in one report should be supported. |
| Samsung [16] | Proposal 14. For UE-side AI/ML model, support both Type 1 performance monitoring and Type 2 performance monitoring. |
| Fujitsu [17] | Proposal 11: - Regarding performance monitoring for BM Case-1 with UE side model, either the UE-side monitoring (Type-2 performance monitoring) or the hybrid monitoring (Option 2 of Type-1 performance monitoring) could be considered. Proposal 12: - Regarding UE-side monitoring (Type-2 performance monitoring) for BM Case-1 with UE side model, Rel-16 SCell BFR framework could be used as starting point. RAN1 to further discuss: - the timer/counter/threshold configuration for performance monitoring - how to report the information that the current functionality doesn’t work well and the newly activated functionality. Proposal 13: - Regarding UE-side monitoring (Type-2 performance monitoring) for BM Case-1 with UE side model, RAN1 to further discuss the corresponding reference signal configuration. Proposal 14: - Regarding UE-side monitoring (Type-2 performance monitoring) for BM Case-1 with UE side model, RAN1 to further discuss the performance metric, and the following alternatives are preferred. - Alt.1: Beam prediction accuracy related KPIs, e.g., Top-K/1 beam prediction accuracy - Alt.4: The L1-RSRP difference evaluated by comparing measured RSRP and predicted RSRP. Proposal 15: - Regarding hybrid monitoring (Option 2 of Type-1 performance monitoring) for BM Case-1 with UE side model, RAN1 to further discuss the performance metric, and the following alternatives are preferred. - Alt.1: Beam prediction accuracy related KPIs, e.g., Top-K/1 beam prediction accuracy - Alt.4: The L1-RSRP difference evaluated by comparing measured RSRP and predicted RSRP Proposal 16: - Regarding hybrid monitoring (Option 2 of Type-1 performance monitoring) for BM Case-1 with UE side model, RAN1 to further discuss the reference signal configuration and reporting enhancement. Proposal 25: - Regarding performance monitoring for BM Case-2 with UE side model, RAN1 to consider similar design scheme as BM Case-1 with UE side model. |
| Nokia [19] | Observation 1: For NW-sided performance monitoring of a beam prediction related CSI report, the UE may need to measure Set A or subset of Set A and report the measured beams (e.g., Top-1/4 strongest beams) to the NW. The NW can use a different CSI report to get such beam measurements within the legacy CSI reporting framework. Proposal 8: For BM-Case1 and BM-Case2, support NW-sided performance monitoring for beam prediction related CSI reporting, and further discuss the following variants, - Case1: No enhancement is needed to support NW-sided performance monitoring. - E.g., the NW can use a different CSI report to get beam measurements/reporting for a monitoring RS resource set (as preferred by the NW) within the legacy CSI reporting framework. - Case 2: Enhancements when using the same CSI reporting configuration for monitoring and inference. - Option 1: Consider monitoring RS resource set = Set A (same RS resource set for inference and monitoring). - Option 2: Monitoring RS resource set is configured/indicated separately from Set A. - Option 3: Monitoring RS resource set is determined by the UE based on active TCI states or inference outcome(s). - For Options 1-3, the NW configures (associated to the CSI report) the reporting timelines and reporting quantities for the monitoring RS resource set. Proposal 9: For BM-Case1 and BM-Case2, considering UE-assisted performance monitoring for a beam prediction related CSI reporting, discuss whether the following options can be used for the reporting of performance monitoring related KPIs, - Option 1: support reporting of “Top-K beam prediction accuracy” corresponding to predicted Top-K beam IDs, where K is configurable to the UE. - Option 2: support reporting of “L1-RSRP difference” corresponding to Top-1 predicted beam. - Option 3: support reporting of “L1-RSRP difference predicted” corresponding to predicted L1-RSRP of Top-1 predicted beam, if predicted L1-RSRP is supported by AI/ML model output. - Note: The UE shall refer to the KPIs definition contained in TR 38.843, more realistic assumptions for ideal L1-RSRP and genie-aided beam shall be described if any of the above metrics are defined in the normative specifications. Observation 2: For UE-assisted performance monitoring of a beam prediction related CSI report, the UE may need to measure Set A or subset of Set A, and report the performance monitoring related KPIs to the NW. Proposal 10: For BM-Case1 and BM-Case2, considering UE-assisted performance monitoring for a beam prediction related CSI reporting, study whether the same CSI report (that enables the beam prediction) or a different CSI report should be used. - For the case where the same CSI report is used for monitoring and inference. Discuss following variants, - Option 1: Consider monitoring RS resource set = Set A (same RS resource set for inference and monitoring). - Option 2: Monitoring RS resource set is configured/indicated separately from Set A. - Option 3: Monitoring RS resource set is determined by the UE based on active TCI states or inference outcome(s). - For Options 1-3, the NW configures (associated to the CSI report) the reporting timelines and monitoring KPIs for the monitoring RS resource set. - For the case where different CSI reports are used for monitoring and inference, NW can configure/indicate the monitoring RS resource set (resourcesForChannelMeasurement) within the legacy CSI reporting framework. Proposal 12: RAN1 to prioritize work on specifying NW-side performance monitoring and discuss/study different options for UE-assisted performance monitoring. Deprioritize UE-side performance monitoring. |
| Lenovo [20] | Proposal 18: For UE-side AI/ML inference, support aperiodic beam measurement for performance monitoring and dynamic beam updating within the beam set associated with the aperiodic trigger state for beam measurement. |
| LGE [23] | Proposal #11: Support event-triggered UE reporting for UE-sided AI/ML performance monitoring. |
| DoCoMo[28] | Proposal 11: Support at least type 1 performance monitoring for functionality activation/deactivation decisions. |
| CAICT [29] | Proposal 6: Both type 1 and type 2 UE-sided model monitoring should be supported. Proposal 7: Even-based feedback from UE is proposed for UE-sided model monitoring. Proposal 8: gNB-sided monitoring should be supported by UE configuration. Whether the detail design of UE configuration is L1 or RRC signaling based needs discussed further. |
| Indian Institute of Technology Madras (IITM), IIT Kanpur [31] | Proposal 11: For monitoring data collection for a UE-side model, the NW configures measurements. The UE collects the monitoring data (which could include ground truth). Decision regarding model activation/deactivation/switching/fallback can either be taken at the UE or NW based either on KPIs or direct measurements of model output. |
| KT [32] | Proposal 9. Support that UE calculates performance metric(s) for UE-sided model. Proposal 10. Support L1-RSRP difference evaluated by comparing measured RSRP and predicted RSRP as a performance metric. Proposal 11. Discuss how to configure the predicted best beam(s) as a benchmark/reference for the performance comparison. |
| Sharp [33] | Proposal 8: Regarding the performance metrics of model monitoring, Alt.1 (Beam prediction accuracy related KPIs) is preferred. |
| ITL [35] | Proposal 3: It is proposed to consider the following signaling mechanisms and procedures for BM: - Identification of AI/ML Model Recovery Need - Triggering AI/ML Model Recovery - Execution of AI/ML Model Recovery - Signaling for Sparse Reporting Mechanisms - Support for Reporting Prediction Results - Dynamic Switching of AI/ML Inference Location |
| MTK[37] | Proposal 17: For BM-Case1 and BM-Case2 with a UE-side AI/ML model, when functionality-based LCM is applicable, study a performance monitoring method with UE initiating LCM operation request and NW indicating/granting the corresponding activation/deactivation/fallback/switching LCM operation of AI/ML functionality to UE. Proposal 18: For BM-Case1 and BM-Case2 with a UE-side AI/ML model, when functionality-based LCM is applicable, identify for each AI/ML functionality whether it is feasible for UE to initiate LCM operation requests. Proposal 19: For BM-Case1 and BM-Case2 with a UE-side AI/ML model, regarding the event that UE reports for performance monitoring, defining the event at least from the following aspects, - The performance metrics monitored for the event - The threshold of the performance metrics for determining the occurrence of the event - The number of samples of the occurrence instances required for determining the occurrence of the event, where the occurrence instances are the monitoring samples that the monitored metrics falls below a threshold - The number of monitoring samples required for determining the occurrence of the event - The frequency of each monitoring samples Proposal 20: For BM-Case1 and BM-Case2 with a UE-side AI/ML model, model, to facilitate UE to detect a monitoring event for performance monitoring, considering NW signaling to UE the following aspects, - The performance metrics monitored for the event - The threshold of the performance metrics for determining the occurrence of the event - The number of samples of the occurrence instances required for determining the occurrence of the event, where the occurrence instances are the monitoring samples that the monitored metrics falls below a threshold - The number of monitoring samples required for determining the occurrence of the event - The frequency of each monitoring samples Proposal 21: Consider a new report container for UE to report for performance monitoring for BM-Case1 and BM-Case2 with a UE-side AI/ML model, which can include at least the following contents, - Performance monitoring metrics - Beam measurements - Events for monitoring Proposal 22: For BM-Case1 and BM-Case2 with a UE-side AI/ML model, for the BM-specific conditions regarding “information regarding model inference”, consider at least the following sub-conditions, - conditions on the number of predicted best beams (e.g., value of K for Top-K predicted beams) - conditions on the model output (e.g., predicted beam ID/confidence score of each beam/beam RSRP) Proposal 23: For BM-Case1 and BM-Case2 with a UE-side AI/ML model, for the BM-specific conditions regarding “performance monitoring”, consider at least for the following sub-conditions, - conditions on performance metrics - conditions on the detectable events |
| Qualcomm [38] | Proposal 6 For beam prediction for UE-side AI/ML models, specify signalling details associated with transmission of reference signals for performance monitoring (that span entire Set A or subset of beams from Set A), helping UE to assess the performance of UE-side AI/ML models. • FFS: Type of RS for performance monitoring purpose (periodic/semi-persistent/aperiodic) • FFS: details of the case in which RS for performance monitoring is a subset of Set A (e.g., how to determine the subset, its variability over time, and the signalling details for indicating the corresponding subsets) • Note: measurement of RS for performance monitoring (from Set A) is expected to be much less frequent than Set B measurements used for inference. Proposal 7 For UE-side beam prediction, identify metrics for inference error, for performance monitoring instances. • FFS: details of metrics based on availability of RS for performance monitoring (e.g., occasional measurements of entire Set A or subset(s) of Set A) • FFS: details of performance monitoring reports (contents, frequency of report, carrier). |

5.3 Metric

There is no sufficient discussion in this meeting on the metric. For example, how to counting/estimate the beam prediction accuracy for Alt1? how to set the threshold (benchmark). For Alt-4, predicted L1-RSRP difference, whether/how can be used for BM-Case 1 without time information? For Alt-3, evidence to show the feasibility is needed. Further study until next meeting or some further discussion offline is encouraged.

| Company | Proposals |
| --- | --- |
| Ericsson[3] | Proposal 21 For UE model performance monitoring at NW-side, first agree on the reporting content of AI/ML model inference prior to discussing performance metrics 1-4. Note that: • metric 4 is only needed if RAN1 standardize a UE reported predicted L1-RSRP • metric 3 is needed if UE does not provide any confidence/uncertainty of its predictions. Observation 11 The number of samples and statistical metrics of the performance metrics needs to be addressed. |
| ZTE[5] | Proposal 14: Support beam prediction accuracy related KPIs (i.e., Alt.1) as the primary performance metric for AI/ML performance monitoring. |
| CMCC[7] | Proposal 16: The best beam(s) obtained by measuring beams of a set indicated by gNB is considered as the benchmark/reference for monitoring performance comparison: o gNB configures one or multiple sets for one or multiple benchmarks/references Proposal 17: For UE-side monitoring and hybrid model monitoring of UE-side AI/ML model, monitoring mechanism of multiple benchmarks needs discussion to facilitate UE to perform model monitoring. |
| CATT [10] | Proposal 9: For performance monitoring of BM-Case1 and BM-Case2, the following performance metrics can be supported. - Alt.1: Beam prediction accuracy related KPIs - Alt.4: The L1-RSRP difference evaluated by comparing measured RSRP and predicted RSRP Proposal 10: For performance monitoring of BM-Case1, L1-RSRP difference of beams in Set B can be considered as the performance metric. |
| Xiaomi [12] | Proposal 5-1: Support following two performance metrics with high priority for performance monitoring. - Alt.1: Beam prediction accuracy related KPIs, e.g., Top-K/1 beam prediction accuracy - Alt.4: The L1-RSRP difference evaluated by comparing measured RSRP and predicted RSRP Corresponding to the supported performance metric, both benchmark/reference should be considered for the performance comparison. Proposal 5-2: Both of the following two Benchmark/reference for performance comparison should be supported. - Alt.1: The best beam(s) obtained by measuring beams of a set indicated by gNB (e.g., Beams from Set A) - Alt.4: Measurements of the predicted best beam(s) corresponding to model output (e.g., Comparison between actual L1-RSRP and predicted RSRP of predicted Top-1/K Beams) |
| OPPO [13] | Proposal 19: On performance metric of AI/ML model monitoring, at least support the beam prediction accuracy related KPIs (Alt.1). |
| Samsung [16] | Proposal 13. For performance monitoring of AI/ML model, Alt-1, Alt-3 and Alt-4 can be considered as the performance metric(s). |
| Futurewei [18] | Proposal 1: For Rel-19 AI/ML-based BM, support UE reporting L1-RSRP for performance monitoring of BM-Case1 and BM-Case2. |
| Nokia [19] | Observation 3: For UE-assisted performance monitoring of a beam prediction related CSI report, it is desirable that performance monitoring based on reporting monitoring related KPIs and performance monitoring based on events can be enabled separately. Proposal 11: For BM-Case1 and BM-Case2, considering UE-assisted performance monitoring for beam prediction related CSI reporting, discuss the following for event-based reporting, - Details of monitoring RS resource(s) (in other words, failure detection RS resources) - Details of failure events, including the associated configurations to determine the failure instances for a beam prediction related CSI report. - Details of reporting for failure event, including reporting content - Strive to use similar mechanisms as in BFR procedures. |
| Lenovo [20] | Proposal 19: Select Alt 1 and Alt 4 as the performance metric(s) of AI/ML model monitoring.  Alt.1: Beam prediction accuracy related KPIs, e.g., Top-K/1 beam prediction accuracy.  Alt.4: The L1-RSRP difference evaluated by comparing measured RSRP and predicted RSRP. Proposal 20: For a monitoring sample, the association of the beam measurement for Set B and the beam measurement for benchmark/reference should be ensured. Proposal 21: Support BFR-like AI/ML model failure report for hybrid performance monitoring for UE-side AI/ML model |
| Sony [21] | Proposal 9 : Considering the trade-off between measurement overhead and monitoring accuracy, the L1-RSRP difference between the predicted and actual value should serve as the performance metric. |
| Sharp [33] | Proposal 9: For AI/ML model performance monitoring for BM-Case1 and BM-Case2, support the following Alt.1 as the benchmark/reference for performance comparison: - Alt.1: The best beam(s) obtained by measuring beams of a set indicated by gNB (e.g., Beams from Set A). |

5.4 First round discussion

Proposal 5.1.1 (for NW-sided model)

For NW-sided model, study whether the mechanism for data collection for training or measurement report for inference can be re-used for performance monitoring

| Company | Y/N | Comments |
| --- | --- | --- |
| New H3C | Y |  |
| OPPO |  | Suggest to slightly postpone this discussion. When the data collection for training or measurement report for inference gets stable, we could further discuss to reuse that for model monitoring, since monitoring depends on types of data collection. |
| Ericsson |  | Ok to postpone the discussion after the training/inference data collection are concluded. In general, the performance monitoring mechanisms can be discussed for UE/NW sided after we had made progress on inference/training data collection. |
| Hw/HiSI |  | We think it is clear that the same mechanism can be re-used. The UE just needs to report RSRPs for indicated set(s), and the NW side can perform the monitoring based in implementation. For NW-sided model monitoring, study whether at least the mechanism for data collection for training or measurement report for inference can be re-used for performance monitoring |
| Fujitsu |  | Question for clarification. Does the proposal mean whether the measurement and report mechanism for training data collection or inference could be reused for monitoring? If yes, we suggest the following update to make it clearer. For NW-sided model, study whether the measurement and report mechanism for data collection for training or measurement report for inference can be re-used for performance monitoring |
| TCL |  | We suggest to postpone the discussion. The measurement and report behaviours of model monitoring are slightly different from those of training and inference, whether the mechanisms can be reused depends on further progresses of training/inference. |
| CMCC |  | Ok to postpone the discussion after the training/inference data collection are stable. |
| vivo |  | We think it is important to consider overhead for NW side model monitoring report considering monitoring report should be more frequent than training and deliver more information than one-shot inference. Further, it is not needed to link monitoring report and training/inference report as they serve different purposes. For monitoring report, we think reporting KPI from UE is a typical way for both UE side and NW side models. In SI, the following is agreed as captured in TR. For BM-Case1 and BM-Case2 with a NW-side AI/ML model - Beam measurement and report for model monitoring - UE reporting of beam measurement(s) based on a set of beams indicated by gNB. We can further discuss starting by listing detailed options. Hence we suggest the follow proposal. For NW-sided model, study the following options on reporting content from UE for performance monitoring based on measurements of a set of beams indicated by gNB. - Option 1: performance monitoring KPI - Option 2: beam/resource indices - Option 3: L1-RSRPs - … |
| ETRI |  | We support to postpone the discussion after mechanism for data collection for training/inference are made. |
| Xiaomi | Y |  |
| ZTE |  | For NW-side model, the mechanism for data collection for training or inference can be re-used for performance monitoring and thus no additional spec enhancement is needed. Nevertheless, fine to postpone the discussion. |
| Apple | N | Whether there is a convergence or not, we can wait until respective designs are more clear. This does not need to be a set goal from the beginning. |
| QC | N | As pointed out in the RAN2 LS reply, inference, monitoring, and data collection have separate requirements and implications at least with regards to content of collected data, and latency. So the respective mechanisms should be designed and optimized accordingly, based on the requirements for each phase. |
| Spreadtrum | Y |  |
| Panasonic |  | We think it is reasonable to postpone the discussion for now. We can discuss it after discussions on Sections 3 and 4. |
| CATT | Y | The data collection for performance monitoring is related to the performance metric, we agree to first discuss the metric. |
| Google |  | We do not think training and inference can be reused for monitoring. The latency for monitoring is smaller than training. NW cannot keep sending the set A beam related RSs for monitoring. The monitoring could require the measurement results for set A beam, which is different from inference. |
| Futurewei |  | Ok to postpone the discussion after the training/inference data collection are stable. |
| LG | Y |  |
| Lenovo |  | We prefer to use a same mechanism for data collection, inference and performance monitoring. |
| Fraunhofer | Y |  |
| KDDI |  | We support to postpone this discussion until data collection for training and inference is clarified. |

Proposal 5.1.2 (for UE-sided model)

TBD

- Consistency between training and inference

6.1 NW-sided model

UE Rx assumption has been covered in other proposals during data collection, as one FFS point. No separate discussion here.

| Company | Proposals |
| --- | --- |
| Ericsson [3] | Observation 1 A UE-indicated RX-beamIndex could be used by the NW to reduce ambiguity in data collection, and ensure that a UE is not changing beams randomly. Proposal 1 For a NW-sided model, to ensure consistency from training to inference, specify the following mechanisms • UE indicate RxBeamIndex during set A/B data collection. • NW indication that UE should use fixed RX-beam during set A/B data collection. |
| ZTE[5] | Proposal 19: There is no necessity to specify any NW-side additional conditions for AI/ML based beam management. |
| Intel [8] | Proposal 19: For network-side AI/ML model, UE Rx beam assumptions for measuring sets A/B during training data generation may be considered part of additional conditions. |
| Xiaomi [12] | Proposal 3-4: Exchange the UE-side additional condition such as Rx beam assumption and UE speed during the procedure of data collection for NW-side AI/ML model training. |
| OPPO [13] | Proposal 24: To maintain consistency of QCL assumption across training and inference phases, UE may assume the same rule for QCL assumption, e.g. apply the best/fixed/same Rx beam(s). |

6.2 UE-sided model

| Company | Proposals |
| --- | --- |
| Huawei/HiSi[2] | Proposal 7: For the data collection for the UE-side model under BM-Case 1/BM-Case 2, the mapping between Set B and Set A can be supported for the case when Set B is a subset of Set A, e.g. with a bitmap or a list of CRIs of Set B. Observation 2: The motivation to introduce assistance information for the purpose of UE/NW side data categorization is not clear considering the following points: - UE/NW can train a generalized model that is applicable to multiple scenarios/antenna layouts/beam shapes. - UE/gNB can autonomously identify the scenario without the need for gNB/UE notification. - The categorization principle and granularity of the scenarios identified by NW/UE may not match the categorization principle of their counterpart. - To achieve an aligned categorization principle, offline interpretation to the physical meaning of the scenarios/antenna layouts/beam shapes the between NW side and UE side may be inevitable, which may probably disclose the proprietary. Proposal 8: For the data collection for the UE-side model, the impact factors for consistency between training and inference should be clarified before discussing specific solutions. Observation 3: For the data collection for the UE-side model, for the following impact factors, they may not significantly impact the consistency between training and inference if appropriate implementation manner is applied: - The UE is provided with information about Set A/Set B sizes and their association/mapping in a local cell so that it is aware of the difference of size/association/mapping by CSI measurement configuration. - For a particular Set A/Set B size/association/mapping, - Beam flipping may not impact the consistency between training and inference. - Dataset mixing can be applied to train generalized model over codebook designs. |
| Ericsson [3] | - Observation 4 Different sites/cells may have different antenna/beam configurations. - Observation 5 One additional condition for UE-sided BM comprises that different cells might have different antenna/beam configurations, and that its configurations can change over time within the cell - Proposal 12 For UE-sided models, to ensure consistency from training to inference (e.g. NW might change the beam pattern/configuration), consider the following methods - - UE autonomously assess the applicability of its prediction functionality, - - Monitoring based solution to assess the prediction applicability, - - Extend the CSI-RS framework with “consistency” identifiers. - Proposal 19 For UE-sided models, deprioritize the NW assistance information discussion until the method for ensuring consistency from training to inference is concluded. - Proposal 23 RAN1 to outline the candidate conditions that can be part of, or outside the UE capability framework (RAN2 to decide the exact details) |
| Vivo [4] | - Proposal 2: Support Set B pattern indication between gNB and UE to ensure consistency of Set B across training, inference and monitoring for NW-side model. - Proposal 3: Network-side additional conditions at least include Set B pattern, Tx beam boresight direction (azimuth and elevation), 3dB beam-width for DL Tx beam prediction. - Proposal 4: For UE-side model, support that dataset ID or virtual beam information representing NW-side additional conditions is provided to UE to ensure consistency between training and inference for model inference, as well as to address NW-side proprietary/privacy information disclosing issue. |
| ZTE [5] | - Proposal 10: For BM-Case1 and BM-Case2, support to indicate the association or mapping of Set A and Set B from NW to UE to enable UE-side model inference. |
| CATT [10] | Proposal 16: To ensure the consistency between training and inference, gNB can provide the beam codebook information with preserving privacy information to UE, e.g. the number of vertical beams and the number of horizontal beams of the Tx beam codebook. Proposal 17: To ensure the consistency between training and inference, consider triggering the monitoring when NW-side additional condition (e.g., Tx beam codebook size) is consistent across training and inference to avoid unnecessary monitoring. Proposal 18: To ensure the consistency of patterns/association of Set B and Set A, the QCL relation of RS with Set B beams and Set A beams can be considered. Proposal 19: To ensure the consistency of model input/output order, the mapping rule between RS and Tx beams can be pre-defined. |
| NEC [11] | Observation 3: The timing information of Set B measurements and Set A predictions is crucial for UE to determine the model to be used for inference. Proposal 2: In addition to information of beams in Set A and Set B, the timing information of Set B measurements and Set A prediction occurrences should be specified as conditions for BM-Case1 and BM-Case2. Proposal 3: Specify pattern ID as a condition to indicate the site/cell specific variables used for model training like antenna configuration. Proposal 4: Support a reference beam pattern to ensure the consistency. Proposal 5: Support a calibration procedure to ensure the consistency. Proposal 6: Support to configure the association between CSI-RS in AI/ML model training phase, CSI-RS in AI/ML model inference phase, as well as CSI-RS in AI/ML performance monitoring phase. |
| Xiaomi [12] | Proposal 2-8: Support Additional condition alignment is beneficial for both model-based LCM and functionality-based LCM in AI-based beam management use case. Proposal 2-9: Support indication of the network additional condition and unified indication mechanism should be strived for in all cases. Proposal 2-10: The following NW side additional conditions need to be indicated to UE to ensure consistency between training and inference for UE side model. - Set B/ set A configuration - The number of beams in set B - The number of beams in set A - Pattern of set B - Contiguous beams or non-contiguous beams in set B - Fixed or random or preconfigured patterns - Time window configuration (BM case 2 only) - The number of history measurement time instance - The number of predicted future time instance - Deployment - Scenario - Uma, Umi, indoor hotspot - ISD Proposal 2-11: For some NW side additional conditions which are sensitive, relative information or virtual ID can be indicated to UE to ensure consistency between training and inference for UE side model. |
| NVIDIA [15] | Proposal 7: For AI/ML based beam prediction in spatial/time domain, introduce specification support for additional conditions to include them into model description information during model identification. |
| Samsung [16] | Proposal 15. For UE-side AI/ML model, the discussion on the consistency between training and inference should be deferred until there is more progress on model identification. Proposal 6. For UE-side AI/ML model inference, support UE to send a request for DL RS configuration and/or for DL RS transmission. - FFS: the procedure/contents associated with the request. Proposal 7. For UE-side AI/ML model inference, existing mechanism/framework for the configuration or triggering CSI-RS/SSB resources should be the baseline. - Note: This is for the configuration/triggering of Set B - FFS: configuration to facilitate measurements for multiple past time instance. |
| Fujitsu [17] | Proposal 32: - RAN1 to consider performance monitoring to ensure the consistency between training and inference. RAN1 to further discuss the details on additional conditions. |
| Nokia [19] | Proposal 13: For beam prediction use-cases, RAN1 shall support a solution to ensure consistency between training and inference regarding NW-side additional. Proposal 14: RAN1 to deprioritize model identification until study details captured in Rel-19 agenda item 9.1.3.3 (other aspects of AI/ML model and data) are finalized. Proposal 15: RAN1 to deprioritize training model at NW and model transfer to the UE until study details captured in Rel-19 AI 9.1.3.3 are finalized. Observation 5: Methods on NW-side additional conditions determination applicable to consistency between inference and training should follow the constraints concerning proprietary information disclosure. Proposal 16. For beam prediction use cases, to ensure consistency between training and inference regarding NW-side additional conditions, consider enhancements related to information/indications on NW-side additional conditions, - FFS: Details on indication that facilitate alignment between the measurements used for inference and the measurements used during training Proposal 17: For beam prediction use cases, the performance monitoring/assessment framework shall ensure consistency between training and inference regarding NW-side additional conditions, further discuss the following options, - Option 1: UE-sided model assessment in a NW-transparent manner (e.g., UE is doing performance assessment to select suitable UE models when supporting beam prediction under different NW assumptions). No spec impacts. - Option 2: UE-sided functionality assessment and reporting the functionality assessment (e.g., as applicable functionality reporting) - Consider enhancements to enable monitoring of multiple beam prediction related CSI reporting configurations and reporting of applicable CSI report configuration IDs. - Option 3: NW-sided functionality assessment (e.g., NW implementation option where NW selects suitable functionalities based on its own assessments). This option can either be UE-transparent (with no spec impact) or UE-assisted (with some spec impact on RS measurements). - For UE-assisted operations, consider the changes required on RS measurement and reporting framework. - Option 4: Joint model and functionality assessment by UE and NW. This can be considered as a combination of options 1-3. - FFS: further discuss details of signalling support. |
| LGE [23] | Proposal #12: For data collection at UE-sided AI/ML model, it should be studied on how to ensure the same Tx beam for different transmission instances of same or different CSI-RS resources. |
| Apple [24] | Proposal 5-1: The dataset ID in assisted information needs to be PLMN unique, and core network or O&M is involved in assigning/managing the dataset ID. Proposal 5-2: The assistance information/dataset ID/model ID, if assigned by higher layer is embedded as part of reference signal configuration. |
| DoCoMo [28] | Observation 3: Consistency guarantee via performance monitoring incurs measurement/signalling overhead and detection latency. Also, additional mechanism is necessary to help preparing UE sided models specific to NW side additional conditions. Observation 4: Consistency guarantee via information/indication on NW side additional condition provided to UE may disclose proprietary information. |
| Indian Institute of Technology Madras (IITM), IIT Kanpur [31] | Proposal 8: Alignment of NW-side additional conditions could be through an ID assigned by the NW during training data collection (proposal 3). In the absence of an ID, alignment can be achieved by model monitoring. |
| KT [32] | Proposal 7. Consider performance monitoring of UE-side candidate models/functionalities as a mechanism to ensure consistency between training and inference for UE-side models. |
| Qualcomm [38] | Proposal 1 For beam prediction for UE-side AI/ML models, consider the following aspects to ensure consistency between training and inference regarding NW-side additional conditions (with regards to Set A, Set B consistency) for inference at UE • Order/indexing consistency: consistency in ordering of resources (e.g., resource index consistency) for Set B beams and Set A beams, across training and inference. • Beam shape consistency: relative pointing direction and beamwidth difference between physical beams with respect to Set A and Set B resources across training and inference should be under predefined tolerances. Proposal 2 For beam prediction for UE-side AI/ML models, support mechanisms to ensure consistency between training and inference regarding NW-side additional conditions (with regards to Set A, Set B consistency) for inference at UE. For this purpose, investigate the following two approaches: • Introduction of a new QCL Type: Identify air interface enhancements that would improve upon the existing QCL framework, e.g., by introducing a new QCL type. • Ensuring consistency via referring to previous data collection configuration: For a given set of NW-side additional conditions, a dataset-ID is identified during data collection for model training procedure, and during inference, consistency in NW-side additional conditions is materialized by referring to the corresponding dataset-ID. Figure 3 Ensuring consistency via referring to previous data collection (dataset-ID) Figure 4 Ensuring consistency of NW-side additional conditions, through dataset-ID |

6.3 First round discussion

Proposal 6.1.1 (Consistency, Request)

For UE-sided model, further study whether/how to support the following:

- A request/report from UE to NW for information for all and/or subset of Set A of beams

  - FFS on information, including resource/ TCI/beam ID

  - FFS on the purpose of the request, e.g., for measurement

  - FFS on the content of request/report, including:

    - Total number of beams in Set A

    - Other information

- A request/report from UE to NW for the RS resource configuration for all Set B of beams for measurement

  - FFS on the content of request/report including:

    - Total number of beams in Set B

    - Associated time domain information of Set B (e.g., periodicity/offset)

    - TCI states/beam IDs/pattern of Set B

    - Other information

| Company | Y/N | Comments |
| --- | --- | --- |
| New H3C | Y |  |
| OPPO |  | One generic comment would be that beside the UE request/reported based solution for consistency, it also matters for NW to configure proper Set B/Set A across training and inference. |
| Ericsson | N | We don’t understand why this proposal is relevant for the consistency discussion, we should discuss how the consistency from training to inference should be ensured. This proposal does not cover the proposals listed in the table under 6.2. Only one company seems to propose that UE should perform such request. In our view, based on the listed proposals, the following would be a better starting point: Updated Proposal 6.1.1 (Consistency, Request) For UE-sided model, further study the feasibility on the following methods to ensure consistency: - UE to autonomously assess the applicability of its functionality, - Consistency assisted by monitoring, - Beam use-case specific Information and/or indication on NW-side additional conditions is provided to UE, including: - New identifiers that can ensure consistency on csi-Reosource/ csi-ResourceSet/ csi-reportConfig /csi-resourceConfig - New QCL types - Number of beams in set A and B - Pattern ID, dataset ID, beam ID…. - Other information is not precluded - Via UE request/report - Other methods are not precluded |
| HWHiSi |  | It seems that the intention of this proposal is acquire information from the NW which also would be needed for data collection, monitoring and inference. To avoid overlapping discussions, we suggest to firstly define what is needed for UE side training, inference and monitoring and then come back to this proposal. |
| Fujitsu | N | Don’t support. For consistency, if it is based on UE request/report, there would be a lot of conditions to be specified, which will bring a lot of difficulties to the normative work. In comparison, we prefer to use performance monitoring to ensure the consistency. |
| TCL |  | We suggest to postpone the discussion. For UE-sided model, the consistency could be guaranteed if the model is trained and transferred to the UE, therefore we may wait for the decision whether there is a need for model training at the UE side. |
| CMCC |  | This proposal is relevant to the consistency of set A and set B configuration from training to inference, there can be more options other than UE request, so we can postpone the discussion. |
| NTT DOCOMO |  | Do not support the proposal. To achieve consistency between training and inference, it is not always necessary for UE to request/report Set A/B info. For example, NW can indicate assistance information of Set A/B (e.g., ID) to UE. According to that, UE can autonomously confirm consistency without any request/report. Given that, the proposal should focus on indication from NW to UE instead of signaling from UE to NE too. |
| InterDigital |  | Do not support. Further discussion is needed. |
| vivo | Y | Support. |
| NEC | N | Similar views as companies and the discussion should be based on the methods captured in TR. |
| Xiaomi |  | - We suggest to replace “further study” by “discuss”. - As for the first FFS of the firs sub-bullet and the 3rd sub-bullet of the FFS of the 2nd sub-bullet, i.e., “TCI states/beam IDs/pattern of Set B”, if my understanding is correct, the motivation is to ensure the consistency on the association between set B of beams and set A of beams. Thus we suggest to update to “TCI states/beam IDs/resource ID/pattern of Set B” - As for the second FFS of the first sub-bullet, the example “for measurement” can only be reasonable for model training. It is better to delete the example, Or list all examples. For model inference, the purpose of the request is for consistency. |
| ZTE |  | Different UEs may have different preferences on Set A and Set B patterns, resulting in potentially large RS overhead at the NW side. Thus, the content of UE request/report should be restricted based on NW side indication on Set A and Set B patterns, instead of being arbitrarily determined by the UE. Thus, we prefer to at least delete the examples for the content of request/report. |
| QC |  | We believe a two-phased approach would be useful here. When we talk about consistency, we first need to agree on which NW-side additional conditions need to be consistent (based on Rel-18 study outcome), hence what needs to be consistent, and then discuss how to ensure such consistency. From the above proposal, the consistency aspect is not really clear, and the proposal is talking about UE request (?). There are quite a few companies who have talked about some form of ID-based approaches to ensure such consistency, e.g., dataset ID, etc. Also some companies have talked about so-called consistency IDs in CSI report configurations, etc. None of those methods have been abstracted in a proposal. We suggest listing proposed solutions by companies to trigger discussions on this topic, as it is critical for UE-side model operation, and its importance is highlighted in the WID: Provide specification support for the following aspects: - Beam management - DL Tx beam prediction for both UE-sided model and NW-sided model, encompassing [RAN1/RAN2]: - Spatial-domain DL Tx beam prediction for Set A of beams based on measurement results of Set B of beams (“BM-Case1”) - Temporal DL Tx beam prediction for Set A of beams based on the historic measurement results of Set B of beams (“BM-Case2”) - Specify necessary signalling/mechanism(s) to facilitate LCM operations specific to the Beam Management use cases, if any Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE |
| CATT |  | The motivation of the proposal is not clear to us. To align the understanding among companies, we think we can first discuss/identify the consistency aspects that will impact the performance. |
| CEWiT |  | We share the same view as CATT and we think that we should first discuss the consistency aspects first. |
| Google |  | One question, is the “report” a UE capability report? |
| Futurewei |  | Agree with other companies to postpone the discussion. |
| Lenovo |  | Don’t support. We don’t think the consistency between training and inference should be based on UE request. |
| Fraunhofer |  | Agree. However, for consistency with the first main bullet, the second main bullet should say “information about all Set B of beams for measurement”. |

- Other LCM aspects

UE capability, LCM operation (activation/deactivation/switching/fallback) were proposed by companies. UE capability report can be discussed later, and LCM operation can be discussed together with performance monitoring, after better understanding how to monitor the performance.

| Company | Proposals |
| --- | --- |
| Ericsson [3] | - Proposal 23 RAN1 to outline the candidate conditions that can be part of, or outside the UE capability framework (RAN2 to decide the exact details) |
| ZTE[5] | Functionality-based LCM without specifying any model ID based signaling should be utilized for AI/ML based beam management. |
| CMCC[7] | Proposal 2: Regarding data collection for NW-side model of BM-Case1 and BM-Case2, UE capability on maximum number of RS for RSRP measurement and CSI reporting framework need enhancement to support unified beam sweeping within a short period. |
| Intel [8] | - Proposal 18: For UE-side AI/ML model selection/switching, consider network configuration, subject to UE capability, of periodic performance monitoring of non-active models and comparison to the KPI of current models such that an alternate model with potentially better performance can be selected or switched. |
| CATT [10] | Proposal 20: For functionality identification of BM-Case1 and BM-Case2, the following conditions can be considered. - Information regarding model inference (functionality usage, Set A and Set B relationship, model output type) - Set A / Set B configuration (Size of Set B and Set A, QCL relation of Set B and Set A, the periodicity and time instances for measurement and prediction ) - Performance monitoring (monitoring Type, performance metric) |
| NEC [11] | Proposal 1: Support to clarify the granularity of functionality (or an AI/ML-enabled Feature/FG) before discussing functionality based LCM for beam management use case. Proposal 9: Support sub configuration of CSI report for AI/ML, each sub configuration may correspond to a different model or a different AI/ML LCM stage requiring data collection. Observation 9: Multiple beam prediction models may run simultaneously at UE corresponding to different RF bands in CA. Observation 10: Some of the beam prediction configuration parameters may be BWP specific when UE is configured with multiple BWPs Proposal 24: RAN1 to discuss the impact of CA and BWP operation on the beam prediction framework. Observation 11: If legacy BM is configured simultaneously with the AI/ML based BM, the fallback can be determined by the legacy BM. Proposal 25: Support to provide associated legacy BM configuration together with the configuration of AI/ML based BM. |
| Xiaomi [12] | Proposal 2-1: BM Case 1 and BM Case 2 can be considered as different conditions for different functionalities. Proposal 2-2: For BM case 1, different association/mapping between beams within set B and beams within set A can be considered as different conditions for different functionalities. - Condition 1: Set A and Set B are different (Set B is NOT a subset of Set A) - Condition 2: Set B is a subset of Set A (Set A and Set B are not the same) Proposal 2-3: For BM Case 2, different repeat window can be considered as different conditions for different functionality. Proposal 2-4: For each repeat window in BM case 2, different association/mapping between beams within set B and beams within set A can be considered as different conditions for different functionalities. - Condition 1: Set A and Set B are different (Set B is NOT a subset of Set A) - Condition 2: Set B is a subset of Set A (Set A and Set B are not the same) - Condition 3: Set A and Set B are the same Proposal 2-5: Define different ranges of number of beams in set B and/ or set A as different conditions for different functionalities. Proposal 2-6: Different content of model output can be considered as different conditions for different functionalities. Proposal 2-7: Different performance metric and performance monitoring type can be considered as different conditions for different functionalities. Proposal 5-9: Confirm the necessity of assessment/monitoring of inactive models / functionalities, with the following assumptions as the starting point: - One way to monitor inactive models/functionalities is by activating them and reusing mechanisms defined for monitoring of active models/functionalities. - The following aspects may be considered for further study or in WI to assess the applicability and expected performance of an inactive model/functionality: - Configuring an AI/ML model for monitoring without activation (e.g., monitoring-only mode without reporting predicted beams in BM Case 1 and 2) - Dataset delivery from the network to the UE for assessment/monitoring of the applicability and expected performance of the model/functionality. - NW may provide performance criteria/preference for UE’s model selection. - Other aspects are not precluded for further study or specification. |
| OPPO [13] | Proposal 18: For BM-Case1 and BM-Case2 with UE-side model, study and specify the LCM-related event(s) which would trigger LCM procedure. Proposal 25: For BM-Case1 and BM-Case2, consider the UE capability on AI/ML beam management at later stage. |
| InterDigital [14] | Observation 12: AI/ML inference at UE side can be limited due to limited computational power and battery consumption at UE implementation, however, UE can easily utilize more information that the UE acquired by measuring SSB/CSI-RS without consuming any reporting overhead. Observation 13: AI/ML inference at NW side could be a good implementation option as UE implementation is generally limited due to computational power and battery consumption than gNB implementation. However, AI/ML inference generally requires more detailed explicit information which leads significant reporting overhead. Moreover, although gNB has greater computational capacity than UE, the capacity can still prove to be insufficient in dense UE scenarios. Proposal 9: Consider a procedure to dynamically switch AIML inference location (e.g., based on NW workload and/or UE’s prediction performance). Proposal 10: For AIML LCM in BM, consider procedures for identification of need of AI/ML model recovery, UE request/gNB trigger and AI/ML model recovery. |
| NVIDIA [15] | Proposal 3: For AI/ML based beam prediction in spatial/time domain, introduce specification support for assistance signalling and procedure for model configuration, model activation/deactivation, model recovery/termination, and model selection. Proposal 4: For AI/ML based beam prediction in spatial/time domain, introduce specification support for assistance signalling and procedure for model performance monitoring and model update/tuning. Proposal 5: For AI/ML based beam prediction in spatial/time domain, introduce specification support for UE capability signaling for AI/ML based beam prediction including model training, model inference and model monitoring. Proposal 6: For AI/ML based beam prediction in spatial/time domain, introduce specification support for conditions of a Feature/FG available for functionality. |
| Fujitsu [17] | Proposal 31: - RAN1 to discuss LCM operation which is specific to beam management use case. |
| Nokia [19] | Observation 6: UE capability reporting framework in TS 38.306 is also relevant as the baseline when enabling CSI configurations applicable to both BM-Case1 and BM-Case2 beam prediction. Observation 7. Selecting/switching/activating/de-activating of beam prediction CSI reports can already be supported by legacy signalling. Proposal 18: RAN1 to consider the reuse of legacy CSI measurement and reporting frameworks for beam prediction related CSI reports. There is no additional requirement for defining any new signalling for functionality LCM for beam prediction use case. |
| Lenovo [20] | Proposal 5: Dynamic switching between AI/ML based beam prediction and non-AI/ML based beam report schemes as well as dynamic switching between different AI/ML models/functionalities for a CSI report configuration with AI inference should be supported. |
| Sony [21] | Proposal 3 : For the UE-side models, UE can report the difference between the predicted results and the traditional beam measurement-based results to the NW, when requesting assistance in model updates. Proposal 4 : Model activation period should be defined and known by both gNB and UE. Proposal 10 : Model failure detection mechanism can be defined according to the process of the beam failure mechanism. Proposal 11 : For known models within the NW, model selection can rely on channel-related measurements like RSRP, SINR, or other relevant information. Proposal 12 : For unknown models within the NW, select the target model based on the performance of the candidate models obtained through detection. |
| Apple [24] | Proposal 3-1: NW-side model UE capability feature group includes capabilities: - data collection for training - data collection for inference/monitoring UE-side model UE capability feature group includes: - data collection for training - data collection for inference/monitoring |
| DoCoMo [28] | Proposal 1: Focus on functionality base LCM without model transfer in 9.1.1, until the necessity of model identification and model transfer is confirmed in 9.1.3.3. Observation 2: UE capability can be considered as functionality identification. Proposal 4: Discuss the granularity of functionality for beam prediction. |
| CAICT [29] | Proposal 9: A confirmation process could be considered before AI model inference operation at UE. Proposal 11: UE should inform functionality/model selection, activation, deactivation, switching, and fallback operation to gNB. Proposal 12: It should be discussed further on whether gNB could inform UE to make functionality/model selection, activation, deactivation, switching, and fallback operation. |
| Indian Institute of Technology Madras (IITM), IIT Kanpur [31] | Proposal 1: Specification support for AI/ML-based beam management must encompass signalling Training Data collection, UE capability reporting, model identification and selection, alignment of NW-side additional conditions, inference and monitoring data collection. Proposal 5: In general, for an AI/ML functionality, the UE capability report should group of AI/ML models into buckets corresponding to non-intersecting sets of supporting conditions. Proposal 6: As a bare minimum, the UE capability must contain details on the model input (type, size), model output, training and testing characteristics and model complexity. Proposal 7: The UE capability report should contain a field (boolean flag) indicating whether alignment of additional conditions is required or not. |
| KT [32] | Proposal 3. Assume AI/ML offline training without model transfer for beam management in Rel-19. Proposal 4. Support model switching as a baseline for achieving good model performance. Proposal 5. Discuss a method to support the functionality-based LCM procedure at least for UE-sided models of BM use case. Proposal 6. Define UE capabilities/conditions for supported functionalities of UE-sided models in a given sub-use cases. |
| ETRI [36] | Proposal 3: As part of the functionality identification process, consider a separate step for transmitting additional conditions between the NW and the UE. |
| CEWiTc [37] | Proposal 1: For BM-Case1 and BM-Case2, consider a common set of conditions/additional conditions for functionality identification. Proposal 2: For beam management, consider the following conditions as a starting point for functionality identification of beam management FG. - Type of beam prediction type, BM-Case1 and/or BM-Case2 - Conditions on AI/ML model input, such as input size, type. - Conditions on AI/ML model output, such as output size, and type - Conditions on Set-B, such as Set-B size, pattern type, time domain periodicity. - Conditions on Set-A and its relationship to Set-B, such as Set-A size, different from Set-B. - Conditions on performance monitoring, such as NW or UE monitoring, measurement periodicity - Conditions on generalization capabilities Proposal 3: Along with legacy UE capability report to signal the condition/additional conditions for functionality identification, consider more dynamic procedure for UE capability reporting. |
| Qualcomm [38] | Proposal 8 For beam prediction of UE-side AI/ML models, support mechanisms to identify an AI/ML functionality by associating it to a CSI report setting (including reference signal configurations), wherein activation/deactivation of an AI/ML functionality is indicated by activation/deactivation of the associated CSI report setting option. |

References

| 1 | R1-2400045 | Disscussion on AIML for beam management | Spreadtrum Communications |
| --- | --- | --- | --- |
| 2 | R1-2400144 | Discussion on beam management for AI/ML | Huawei, HiSilicon |
| 3 | R1-2400171 | AI/ML for beam management | Ericsson |
| 4 | R1-2400232 | Specification support for beam management | vivo |
| 5 | R1-2400263 | Discussion on specification support for AI/ML beam management | ZTE |
| 6 | R1-2400274 | Specification Support for AI/ML beam management | TCL |
| 7 | R1-2400316 | Discussion on specification support for beam management | CMCC |
| 8 | R1-2400376 | Specification support for AI/ML for beam management | Intel Corporation |
| 9 | R1-2400392 | AI/ML based Beam Management | Google |
| 10 | R1-2400418 | Discussion on AI/ML-based beam management | CATT |
| 11 | R1-2400465 | Discussion on specification support for beam management | NEC |
| 12 | R1-2400543 | Specification support for beam management | xiaomi |
| 13 | R1-2400618 | On specification for AI/ML-based beam management | OPPO |
| 14 | R1-2400683 | Discussion on AI/ML for beam management | InterDigital, Inc. |
| 15 | R1-2400692 | Specification support for AI-enabled beam management | NVIDIA |
| 16 | R1-2400720 | Discussion for supporting AI/ML based beam management | Samsung |
| 17 | R1-2400766 | Discussion on specification support on AI/ML for beam management | Fujitsu |
| 18 | R1-2400781 | Discussion on specification support for AI/ML-based beam management | FUTUREWEI |
| 19 | R1-2400793 | AI/ML for Beam Management | Nokia, Nokia Shanghai Bell |
| 20 | R1-2400831 | AI/ML specification support for beam management | Lenovo |
| 21 | R1-2400844 | Discussions on AI/ML for beam management | Sony |
| 22 | R1-2400895 | Prediction of untransmitted beams in a UE-side AI-ML model | Rakuten Mobile, Inc |
| 23 | R1-2400907 | Specification support for beam management | Fraunhofer HHI, Fraunhofer IIS |
| 24 | R1-2400914 | Discussions on AI/ML for beam management | LG Electronics |
| 25 | R1-2401002 | Discussion on AI/ML beam management | Apple |
| 26 | R1-2401043 | Discussion on AI/ML based beam management | Hyundai Motor Company |
| 27 | R1-2401055 | Discussion on specification support for beam management | NTPU |
| 28 | R1-2401107 | Discussion on AI/ML for beam management | NTT DOCOMO, INC. |
| 29 | R1-2401134 | Discussions on AI/ML for beam management | CAICT |
| 30 | R1-2401136 | Specification support for beam management | KDDI Corporation |
| 31 | R1-2401153 | Discussion on Specification Support of AI/ML for Beam Management | Indian Institute of Tech (M), IIT Kanpur |
| 32 | R1-2401161 | Considerations on AI/ML based beam management | KT Corp. |
| 33 | R1-2401171 | Discussions on specification support for beam management | Sharp |
| 34 | R1-2401179 | Discussion on support for AI/ML beam management | ITL |
| 35 | R1-2401223 | Discussion on specification support for beam management | ETRI |
| 36 | R1-2401267 | Discussion on Specification Support for Beam Management | CEWiT |
| 37 | R1-2401297 | AI/ML - Specification support for beam management | MediaTek Inc. |
| 38 | R1-2401431 | Specification support for AI-ML-based beam management | Qualcomm Incorporated |