3GPP TSG RAN WG1 #116			R1-2401609

Athens, Greece, February 26th – March 1st, 2024

Agenda Item: 	8.10

Source: 	Moderator (Apple)

Title:		   Moderator Summary of Reply LS to R1-2400009 (LS on RAN2 agreements for satellite switch with resync)

Document for:	Discussion and Decision

# Introduction

RAN2 sent an LS [1] on satellite switch with resync. In the LS, a question is raised on the feasibility for a UE to start synchronizing to a target satellite while still being connected to a source satellite in soft satellite switch with resync.

- For soft satellite switch, RAN2 would like to get feedback on the feasibility that a UE supporting soft satellite switch can start synchronizing to the DL of the SpCell served by the target satellite while still being connected to the source satellite (without any simultaneous communication with the source and the target satellites). To RAN1 ACTION: For the feasibility issue, RAN2 respectfully asks RAN1 to provide feedback if there are any concern.

In this contribution, we discuss the feasibility for a UE to start synchronizing to a target satellite while still being connected to a source satellite in soft satellite switch with resync.

# Discussions

## Contribution summary

There are contributions from 8 companies, discussing the topic of feasibility for a UE to start synchronizing to a target satellite while still being connected to a source satellite in soft satellite switch with resync [2] - [10]. Companies’ views are summarized in the following table.

|  | Feasible | Proposals |
| --- | --- | --- |
| Spreadtrum [2] | Yes | Conclusion: A UE supporting soft satellite switch can detect the DL sync of the SpCell served by the target satellite while still being connected to the source satellite. |
| Vivo [3] | Condition: No UL Tx | Proposal 1. NW should guarantee that no UL transmission would occur in the duration [T-start, T-service] of soft satellite switch procedure. No RAN1 spec change is expected for supporting soft satellite switching. |
| CMCC [4] | Yes | Observation 1: It is feasible that a UE supporting soft satellite switch can start synchronizing to the DL of the SpCell served by the target satellite while still being connected to the source satellite (without any simultaneous communication with the source and the target satellites). Proposal 1: No technical issue is observed for the feasibility from RAN1 perspective. |
| OPPO [5] | No need | Proposal: RAN1 replies to RAN2 that the feasibility depends on whether the UE is required to keep two synchronization at the same time. Given that the UE does not perform communications with two satellite at the same time, there is no need to keep two synchronization at the same time. |
| Nokia [6] | Condition: NGSO due to non-overlapped SSB reception | Observation 1: For NGSO, it is possible to guarantee that a proper offset can be set in any scenario to avoid overlap between SSBs. Observation 2: For GSO, it is not always possible to guarantee non overlapping SSB at UE reception in every scenario simply based on offset. Observation 3: Depending on the difference in power between source and target satellite, the destructive interference caused by the DL payload from either source or target satellite might make the SSB data from the other satellite not decodable at the UE side. Proposal 1: RAN1 should make an observation that soft satellite switch is only feasible for NGSO scenarios. |
| Apple [7], [8] | Condition: non-overlapped SSB reception | Proposal 1: If UE’s received time difference of SSBs from source satellite and target satellite is larger than a threshold, it is feasible that a UE supporting soft satellite switch can start synchronizing to the DL of the SpCell served by the target satellite while still being connected to the source satellite. Otherwise, it is infeasible. |
| Ericsson [9] | Yes | - Observation 1: Synchronizing to the DL of the old and new satellite simultaneously is feasible and does not contradict the previous RAN1 assumption that “UE is not required to connect to two satellites simultaneously during soft satellite switching”. - Proposal 1: RAN1 to conclude that there is no concern with synchronizing to the DL of the old and new satellite simultaneously. |
| Huawei [10] | Condition: UE capability | Proposal 1: Adopt the following draft LS reply to RAN2 on agreements for satellite switch with resync： “It is feasible for UE to perform the downlink synchronization with the target satellite and keep the communication with the source satellite of the same serving cell simultaneously in soft satellite switch, which depends on UE’s capability. The corresponding UE capability can be introduced as a component in the UE feature group introduced by RAN2 to support soft satellite switch.” |

## Round 1 discussion

In the moderator’s understanding, companies’ views can be categorized in 4 different types:

- Simply feasible (Spreadtrum, CMCC, Ericsson)

- Conditional feasible

  - Condition on non-overlapping SSB reception at UE side (Nokia, Apple)

    - UE is unable to distinguish whether a received SSB is from source satellite or from destination satellite

  - Condition on UE capability (Huawei)

    - Like in L3 mobility where it is up to UE capability to perform intra-frequency L3 measurement without gap.

- Infeasible (vivo)

  - UE’s UL transmissions to source satellite may have time overlap with UE’s SSB reception from destination satellite.

- Unnecessary (OPPO)

  - A UE does not need to keep two synchronization simultaneously.

Regarding OPPO’s question on the necessity/motivation of keeping two synchronization, in moderator’s view, it mainly aims to reduce the latency of UE’s switching from source satellite to destination satellite (at the period of switching). A UE is not expected to perform communication with two satellites simultaneously. Additionally, the RAN2 LS asks RAN1’s opinion on the feasibility (rather than motivation/necessity) of keeping two synchronization.

Regarding vivo’s proposal, it is unclear to the moderator what is the issue if UE’s UL transmissions to source satellite overlap with UE’s SSB reception from destination satellite, considering it is an FDD system.

Regarding Huawei’s proposal about UE capability, the UE capability of “performing intra-frequency L3 measurement without gap” is defined as parallelMeasurementWithoutRestriction, according to the moderator’s understanding. It is probably RAN4’s responsibility to check whether a new UE capability is introduced or an existing UE capability is reused.

Hence, we have the following proposal.

Proposal 1-a: Regarding RAN2’s LS on satellite switch with resync, RAN1’s reply is:

- It is feasible that a UE supporting soft satellite switch can start synchronizing to the DL of the SpCell served by the target satellite while still being connected to the source satellite under the following conditions:

  - UE’s reception time of SSB from source satellite and reception time of SSB from destination satellite are not overlapped.

  - It depends on UE’s capability.

- Otherwise, it is infeasible.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Nokia, Nokia Shanghai Bell | Maybe | From our side we still have some concerns related to how the gNB is expected to be informed about the UE performing the actual switch. What would be the expected behavior if a gNB schedules a UE within the window for the soft satellite switch? Or is it expected that the gNB would “halt” scheduling during the switching window? Basically, the gNB would be left in the dark and not be able to perform scheduling during the switching window. Also, as we pointed out in our contribution, there may be scenarios for the GSO deployment scenario, where it may not be possible to guarantee that the SSBs of current serving cell and the target serving cell are non-overlapping. Hence we would suggest to update the proposal 1a to include that the feasibility is only applicable for non-GSO (or even better – for LEO deployment scenarios). We would also like to propose that under proposal 1-a, there is a further condition that the UE should have a valid SSB-timeOffset. Regarding the UE’s capability, it may not necessarily be needed, but will of course help the gNB scheduler implementation if it is known whether the UE supports the soft switching of the cell. Therefore, we would suggest that the LS response includes a statement that UE feedback related the actual switching point is communicated back to the network upon the switch. |
| Ericsson | Yes, with modification | We are not sure what the meaning is of “It depends on UE capability” as a condition for feasibility. Does it mean that it is a separate (independent) UE capability from the UE capability of “soft satellite switch”, or is it a pre-requisite or component of “soft satellite switch” (as proposed by Huawei in [10])? Our view is that it should be a pre-requisite or component of “soft satellite switch”, since without this capability, the benefit of “soft satellite switch” versus “hard satellite switch” is small. Therefore, it does not have to be mentioned as condition for feasibility, and we propose to remove this sub-bullet. |
| CMCC |  | In general, we are fine with the proposal. We share similar views that if the SSBs from the source satellite and the target satellite are overlapped at the UE side, UE cannot distinguish the two SSBs from each other. We have no problem for the 1st sub-bullet. We think it is feasible that based on the propagation delay of UE to the two satellites, gNB can configure a proper SSB time offset to separate the two SSBs at the UE side to avoid the overlapping. Then it is feasible depending on gNB’s configurations. For the 2nd sub-bullet of the UE capability, it should be further clarify that which kind of UE capability is needed. Our understanding is that the capability of ‘parallelMeasurementWithoutRestriction’ in Rel-17 supports that the UE can take the measurements and the normal operation over the exactly same symbols, without any scheduling restrictions. But the basic feature is that, UE can take the measurement (including the synchronization) of the target satellite, during which procedure the UE can still maintain the synchronization with the source satellite. And after the measurement procedure, the UE can still connect to and communicate with the source satellite without losing the DL synchronization. It is the same case for UE in TN to take the intra-frequency measurement of neighbour cells but without losing the DL synchronization with the serving cell. Although UE during the measurements of the neighbour cell will not expect to receive any other DL transmission from serving satellite over the exact symbols where UE take the measurements. It is the same UE behavior for TN without any additional UE capabilities. UE does not need to deal with the measurement of the target satellite and the transmission from the serving cell over the same symbol. It can depend on gNB’s configuration of the SSB time offset and the scheduling to avoid the overlapping, which is a basic feature without the need of supporting the capability of ‘parallelMeasurementWithoutRestriction’ in Rel-17. Then our thinking is that the capability is to maintain synchronization of two satellites. When UE with the synchronization with the serving cell take measurements over the target cell SSB’s, the UE should have the capability without losing the synchronization with the source satellites, which is exactly the same behavior as UE in TN for neighboring cells. The 2nd sub-bullet in the 1st bullet is proposed to update as below, - It depends on UE’s capability of maintaining synchronization with two satellites. And if companies do not think it is a problem for UE to maintain the synchronization with two satellites, we have no problem to remove this bullet |
| CATT | Yes, with modification | First point for non-overlapped SSB, basically it depends on realistic implementation. For UE side, SSBs for source cell and target cell shouldn’t be overlapped. But this condition is not very strong pre-condition. For the second bullet, UE capability doesn’t need to be defined, because it likes the neighboring cell or neighboring beam measurement, not one new capability. |

## Offline discussion

After offline discussions, the following proposal seems stable.

Proposal 1-b: Regarding RAN2’s LS on satellite switch with resync, RAN1’s reply is:

- From RAN1 perspective, it is feasible that a UE supporting soft satellite switch can start synchronizing to the DL of the SpCell served by the target satellite while still being connected to the source satellite under the following conditions:

  - UE’s reception time of SSB from source satellite and reception time of SSB from destination satellite are not overlapped. (This may not be satisfied when the satellite beam is very large.)  (e.g., would occur in GSO case and for cases with SSB time offset configured with a too small value.)

  - [If UE’s capability supports maintaining DL synchronization with two satellites.]

- Otherwise, it is infeasible.

The moderator would like to make the above proposal to the online discussions.

In case you have any further comments, please provide it as soon as possible.

| Company | Yes/No | Comments |
| --- | --- | --- |

After some further discussions with interesting companies, the proposal is updated as follows:

Proposal 1-c: Regarding RAN2’s LS on satellite switch with resync, RAN1’s reply is:

- From RAN1 perspective, it is feasible that a UE supporting soft satellite switch can start synchronizing to the DL of the SpCell served by the target satellite while still being connected to the source satellite under the following conditions:

  - UE’s reception time of SSB from source satellite and reception time of SSB from destination satellite are not overlapped. (A proper SSB time offset configuration may be required to avoid overlap due to large differential delay.) This may not be satisfied when the satellite beam is very large.)

  - If UE’s capability supports maintaining DL synchronization with two satellites.

- Otherwise, it is infeasible.

# Conclusion

TBD

# References

- R1-2400009 LS on RAN2 agreements for satellite switch with resync	RAN2, Apple

- R1-2400170 Discussion on LS on RAN2 agreements for satellite switch with resync	Spreadtrum Communications

- R1-2400215	Discussions on RAN2 LS for satellite switch with resync	vivo

- R1-2400308	Discussion on RAN2 LS on the agreements for satellite switch with re-sync	CMCC

- R1-2400589	Discussion on LS for satellite switch with resync	OPPO

- R1-2400969	Considerations for satellite switch with resync for NR over NTN	Nokia, Nokia Shanghai Bell

- R1-2400982	Discussion on RAN2 LS on Satellite Switch with Resync	Apple

- R1-2400983	Draft Reply LS on Satellite Switch with Resync		Apple

- R1-2401262	Discussion on LS on RAN2 agreements for satellite switch with resync	Ericsson

- R1-2401392	Discussion on the reply LS on RAN2 agreements for satellite switch with resync	Huawei, HiSilicon