3GPP TSG-RAN WG1 #116	R1- 2401637

Athens, Greece, February 26th – March 1st, 2024

Agenda Item:	8.3

Source:	Moderator (Ericsson)

Title:	Feature Lead summary #2 for Positioning for RedCap UEs

Document for:	Discussion, Decision

# Introduction

This document summarizes the proposals received as part of Agenda Item 8.3  for RAN1#116 for the maintenance of Rel-18 work item on expanded and improved NR positioning, focusing on on the issues for positioning for RedCap UEs. [1]. The objectives relevant for this agenda item are as follow:

- Specify support of positioning for UEs with Reduced Capabilities (RedCap UEs) - Specify support of Frequency Hopping (FH) beyond maximum RedCap UE bandwidth for reception of DL PRS and transmission of UL SRS for positioning [RAN1, RAN2]. - NOTE: The complexity of the corresponding capabilities for RedCap UEs should be addressed for the introduction of appropriate capabilities for RedCap UEs. - Specify RRM requirements for positioning including RRM measurements and procedures for RedCap UEs for both with and without frequency hopping [RAN4].

# Text Proposals for corrections

## Support of non-RedCap UEs [2]

### Text Proposal

| TP 2.1-1 |  |
| --- | --- |
| reason for change: | The collision for SRS resources overlapping is not defined in the specification |
| summary of change: | Proposed that the UE is not expected to be configured with overlapping SRS resources with TX hopping |
| Consequences if not approved: | UE behaviour definition is incomplete. |
| ---------------------------- Start of Text Proposal for TS 38.214 ---------------------------- < Unchanged parts are omitted > ##### 5.1.6.5.1 PRS receiver frequency hopping The UE may be configured to measure and report, subject to UE capability, via [nr-Requested-DL-PRS-measurementBasedOnMultihopRx] the DL RSTD, DL PRS-RSRP, DL PRS-RSRPP, or UE Rx-Tx time difference using receiver frequency hopping for a DL PRS resource, with a requested bandwidth of all hops that may be greater than the maximum UE bandwidth. The UE performing receiver frequency hopping may report via [higher layer parameter] one measurement associated with one received frequency hop or one measurement based on multiple hops of the DL PRS. The UE may report whether the measurement is associated with one received frequency hop or multiple frequency hops of the DL PRS. In RRC_CONNECTED mode, the UE is expected to use a single instance of a configured measurement gap to receive all hops of the DL PRS using receiver frequency hopping. < Unchanged parts are omitted > ##### 6.2.1.4.1 SRS frequency hopping for positioning The UE may be configured via [higher layer parameter], subject to UE capability, to perform transmit frequency hopping separate from the UL BWP configuration and outside of the UL BWP, where the UE may be configured with subcarrier spacing, CP and bandwidth that are different from the UL active BWP. The UE transmit frequency hopping is configured within one SRS resource for positioning, that may be configured with a bandwidth larger than the maximum bandwidth of the UE, in RRC_CONNECTED or RRC_INACTIVE mode. The UE transmit frequency hopping, may be configured with overlapping or non-overlapping frequency hops in the frequency domain. When the UE is configured to perform transmit frequency hopping: - it expects to be configured with the following parameters: - starting PRB of the first hop in time domain in [higher layer parameter] - starting slot offset and starting symbol for each hop in [higher layer parameter] - number of symbols in each hop in [higher layer parameter] - hop bandwidth in [higher layer parameter] - number of overlapping resource block(s) between hops, if present, in [higher layer parameter] - number of hops in [higher layer parameter]. - it does not expect to be configured withthe sum of [StartingSymbol] and [Length] for a hop that exceeds a slot duration. - it expects to be configured with the same periodicity of each hop of an SRS resource with the transmit frequency hopping. The UE may be configured, via [uplinkTimeWindow-Config], subject to UE capability, with an UL time window where the UE is not expected to transmit other signals/channels and is only expected to transmit the SRS for positioning using frequency hopping. The UE is not expected to be configured with one [cycle] of the transmit frequency hopping, including the switching time from/to active BWP required ahead of the first hop and after the last hop, that is partially overlapped with the time window. The UE is expected to switch back to the active BWP if the time between two consecutive hops exceeds twice the switching time from/to the active BWP. For a transmission of a hop for an SRS resource for positioning with frequency hopping starting in symbol Nc1 and a colliding PUSCH or PUCCH transmission starting in symbol NS , the UE shall apply the dropping rules taking into account: - DCI(s) for which the time interval between the last symbol of PDCCH and the SRS symbol Nc1is at least N2 symbols and additional time duration TSRSh, where TSRSh is the switching time to/from the active BWP. - DCI(s) for which the time interval between the last symbol of PDCCH and the colliding PUSCH/PUCCH symbol NS is at least N2 symbols, where calculation of N2 is based on the smallest SCS between the SCS configured for positioning SRS with the frequency hopping, the SCS of the PUSCH, and the SCS of the PDCCH. If the SRS symbol(s), including the switching time to and from the active bandwidth part, of the transmit frequency hopping collides with PUSCH or PUCCH, and if the UE determines the SRS to be dropped, the colliding SRS symbol(s) are dropped. For operation in the same carrier, the UE is not expected to be configured on overlapping symbols with an SRS resource of the transmit frequency hopping configured by the higher layer parameter [to_be_defined] including the switching time to or from the active bandwidth part and an SRS resource with resourceType of both SRS resources as 'periodic'. For operation in the same carrier, the UE is not expected to be activated or triggered to transmit SRS on overlapping symbols with a SRS resource of the transmit frequency hopping configured by the higher layer parameter [XX] including the switching time to or from the active bandwidth part and a SRS resource with resourceType of both SRS resources as 'semi-persistent' or 'aperiodic'. ---------------------------- End of Text Proposal for TS 38.214 ---------------------------- |  |

### First round

Companies are encouraged to comment on the TP 2.1-1 in the table below:

TP 2.1-1:

| Company | Comment |
| --- | --- |
| Huawei, HiSilicon | We support the TP. The cover page items can be revised as reason for change:; The frequency hopping feature was not applicable to non-RedCap UE (eMBB UE) artificially. summary of change:; Remove “reduced capability” from 5.1.6.5.1 and 6.2.1.4.1.. Consequences if not approved:; eMBB UE cannot use the frequency hopping feature.. |
| vivo | Based on the following WID, we are not sure about the above correction is needed - Specify support of positioning for UEs with Reduced Capabilities (RedCap UEs) - Specify support of Frequency Hopping (FH) beyond maximum RedCap UE bandwidth for reception of DL PRS and transmission of UL SRS for positioning [RAN1, RAN2]. |
| ZTE | We prefer the original spec. |
| CATT | OK. |
| Qualcomm | We sympathize with vivo’s understanding, but if there is majority view to support the change, we can also accept the change. |
| Nokia/NSB | It looks like the proposed change is not aligned with the reason of change. Does this mean the typical NR UE can also support frequency hopping functionality ? |
| NTT DOCOMO | We support the TP. Frequency hopping feature can be applicable to other UE types (eMBB and/or eRedCap UEs). |
| Ericsson | Support. |
| LGE | We share similar viw with vivo. Moreover, since we already have similar functionality that FH is not required, BW aggregation, we are not sure such extention is really required. |

## (closed) Correction to hopping pattern equation [2]

### Text Proposal

| TP 2.2-1 |  |
| --- | --- |
| reason for change: | In the current TS 38.211, there is a redundant NscRB in the current hopping pattern, which should be removed. - ninithop=nshiftNscRBmSRS,0-moverlaphop is the initial hop index. In addition, there was also discussion on how the hop transmission counter is mapped to actual transmission. We noticed that in TS 38.331, the slot offsets for the second and the remaining hops are provided in the sequence of ASN.1, and thus the hop index counts from 2 following the order, which is not aligned with the explanation for nSRSTxHopping in TS 38.211. |
| summary of change: | Correction to the hopping equation in 38.211 Addition of the higher layer parameter name for slot offset of remaining hops. |
| Consequences if not approved: | Incorrect hopping equation |
| ---------------------------- Start of Text Proposal for TS 38.211 ---------------------------- < Unchanged parts are omitted > ##### 6.4.1.4.3 Mapping to physical resources < Unchanged parts are omitted > and - kF∈0,1,…,PF-1 is given by the higher-layer parameter StartRBIndex if configured, otherwise kF=0; - khop is given by Table 6.4.1.4.3-3 with khop=nSRSb'=bhopBSRSNb' mod PF Nbhop=1 if the higher-layer parameter EnableStartRBHopping is configured, otherwise khop=0. - moverlaphop∈0,1,2,4 is given by the higher-layer parameter YYY. - nSRSTxHopping=0,1,…,NhopsSRS-1is the hop transmission counter in the time domain, which corresponds to the order of the higher-layer parameter SlotOffsetForRemainingHops. - ninithop=nshiftNscRBmSRS,0-moverlaphop is the initial hop index. < Unchanged parts are omitted > ---------------------------- End of Text Proposal for TS 38.211 ---------------------------- |  |

### First round

Companies are encouraged to comment on the TP 2.2-1 in the table below:

TP 2.2-1:

| Company | Comment |
| --- | --- |
| ZTE | OK |
| Nokia | Ok |
| Ericsson | Ok |
| LGE | Ok |
| FL | TP was agreed online: Agreement TP 2.2-1 for 38.211 in section 2.2.1 of R1- 2401636 is endorsed. |

## (closed) Correction to PRS measurement with Rx hopping[3]

### Text Proposal

| TP 2.3-1 |  |
| --- | --- |
| reason for change: | When a reduced capabiltiy UE reports a RSTD measurement with the DL PRS frequency hopping, it also reports whetehr the measurement is made from a single or multiple frequency hops. In the case of RSTD measurement, the UE uses two different propagation time measurements from the received two different PRSs. The UE may be able to use different number of fruency hops of received PRSs. If the UE uses a single hop from a DL PRS resource and multiple hops from another DL PRS resource, the UE reporting beahvior is uncelar. In addition, the LMF needs to know which measurements are made from wideband or narrowband to select measurements to be used to run a location estimation algorithm. |
| summary of change: | Clarify the UE behavior for RSTD measurement reporting with DL PRS frequency hopping. The proposal is that the redcap UE uses two different ToA mesaurements made from the same number of DL PRS frequency hope in case it reports a RSTD composed of the ToA measurements. |
| Consequences if not approved: | In case the UE uses a single frequency hop for a ToA measurement and multiple frequency hops for anotehr ToA measurement, the UE reporting behavior is unclear. Also, network cannot obtain the proper information on the number of used frequency hops. |
| < Unchanged parts are omitted > ##### 5.1.6.5.1 PRS receiver frequency hopping The reduced capability UE may be configured to measure and report, subject to UE capability, via [nr-Requested-DL-PRS-measurementBasedOnMultihopRx] the DL RSTD, DL PRS-RSRP, DL PRS-RSRPP, or UE Rx-Tx time difference using receiver frequency hopping for a DL PRS resource, with a requested bandwidth of all hops that may be greater than the maximum reduced capability UE bandwidth. The reduced capability UE performing receiver frequency hopping may report via [higher layer parameter] one measurement associated with one received frequency hop or one measurement based on multiple hops of the DL PRS. The reduced capability UE may report whether the measurement is associated with one received frequency hop or multiple frequency hops of the DL PRS. For a RSTD measurement, the reduced capability UE is expected to measure two different DL PRS resources with the same number of frequency hops. In RRC_CONNECTED mode, the reduced capability UE is expected to use a single instance of a configured measurement gap to receive all hops of the DL PRS using receiver frequency hopping. < Unchanged parts are omitted > |  |

### First round

Companies are encouraged to comment on the TP 2.3-1 in the table below:

TP 2.3-1:

| Company | Comment |
| --- | --- |
| Huawei, HiSilicon | We do not think the restriction is needed from RAN1 specification. It could somehow be reflected in RAN4 if they consider it as the condition for the requirement. |
| InterDigital | Support. We are not sure how FH based RSTD measurements can be compared if two different PRS resources have the different number of frequency hops. |
| vivo | We would like to confirm whether the motivation is about target resource measurement A and reference resource measurement B (of RSTD=A-B,) needs to be with the same number of frequency hops. If is, we prefer not to capture it since the legacy RSTD doesn’t have this restriction |
| ZTE | We do not support this TP. It may up to UE implementation and in positioning bandwidth aggregation agenda, we also do not introduce such an restriction from RAN1’s perspective. |
| CATT | Not needed. At least RAN1 don’t reach such agreement. |
| Qualcomm | We dont support this change. It can be left up to RAN4 in their requirements. |
| Nokia/NSB | Support. We think it is necessary to clarify the UE behavior. The UE reports a RSTD measurement and provides if the measurement is from multi-hop or a single-hop. If the UE measure multiple hops from a reference TRP and a single hop from a target TRP, it is ambiguous which information should be reported by the UE. |
| Ericsson | Similar view as Huawei, we don’t think that restriction is needed in ran1 specs. |
| LGE | Since different number of frequency hops only impact on measurement accuracy, there is no ambiguity of UE behaviour. So we don’t think this change is needed in RAN1 perspective. |
| FL | Considering the lack of support we can close the dicussion on this TP |

## (closed) Editorial correction to 38.211 [4]

### Text Proposal

| TP 2.4-1 |  |
| --- | --- |
| reason for change: | 1. l' is the number of OFDM symbol number within the hop if SRShoppingNrofHops for SRS-PosResource is provided other than the OFDM symbol number within the SRS resource. 2. Align the typeface and description of the two sentences in the paragraph. |
| summary of change: | Summary of change: Modify the l'is the number of OFDM symbol number within the hop if SRShoppingNrofHops for SRS-PosResource is provided, and modify the typeface of the second sentence as Times New Roman, and modify the “consecutive OFDM symbol” as “consecutive OFDM symbols”. |
| Consequences if not approved: | there are some typos and error issues in the specification |
| < Unchanged parts are omitted > ### TS 38.211 6.4.1.4.1 SRS resource - NsymbSRS∈1,2,4,8,10,12,14 consecutive OFDM symbols given by the field nrofSymbols contained in the higher layer parameter resourceMapping. If Nhop>1, NsymbSRS is the number of consecutive OFDM symbols per hop. < Unchanged parts are omitted > ### 6.4.1.4.2 Sequence generation < Unchanged parts are omitted > where Msc,bSRS is given by clause 6.4.1.4.3, ru,vα,δ(n) is given by clause 5.2.2 with δ=log2KTC and the transmission comb number KTC∈2,4,8 is contained in the higher-layer parameter transmissionComb. The quantity l'∈0,1,…,NsymbSRS-1 is the OFDM symbol number within the SRS resource or is the number of OFDM symbol number within the hop if SRShoppingNrofHops for SRS-PosResource is provided. < Unchanged parts are omitted > |  |

### First round

Companies are encouraged to comment on the TP 2.4-1 in the table below:

TP 2.4-1:

| Company | Comment |
| --- | --- |
| Huawei, HiSilicon | First change is OK. Second change is not needed. The following is already in 6.4.1.4.2. The sounding reference signal sequence for an SRS resource, or if SRShoppingNrofHops for SRS-PosResource is provided, for a given hop within an SRS resource, shall be generated according to. |
| vivo | We support it, for the second change, our motivation is to make the specification clearer since NsymbSRS can be the symbol number of SRS resource or the symbol number of a hop. But we are okay to follow the majority view. NsymbSRS∈1,2,4,8,10,12,14 consecutive OFDM symbols given by the field nrofSymbols contained in the higher layer parameter resourceMapping. If Nhop>1, NsymbSRS is the number of consecutive OFDM symbol per hop.. |
| ZTE | For the first change: we are ok For the second change: may be an more simple change can be made, the OFDM symbol number is indicated in higher layer parameter nrofsymbols regardless of whether frequency hopping is configured. The quantity l'∈0,1,…,NsymbSRS-1 is the OFDM symbol number as indicated in nrofsymbols within the SRS resource |
| CATT | It seems that only the first change is needed. |
| Nokia/NSB | For the second part, “the number of OFDM symbol number” is unclear to us. Would it be intended for “the number of OFDM symbols” within the hop ? |
| Ericsson | OK with first change |
| LGE | For the first change: OK For the first change: Same view with Huawei. |

### Text Proposal for endorsement

| TP 2.4-2 |  |
| --- | --- |
| reason for change: | 1. l' is the number of OFDM symbol number within the hop if SRShoppingNrofHops for SRS-PosResource is provided other than the OFDM symbol number within the SRS resource. 2. Align the typeface and description of the two sentences in the paragraph. |
| summary of change: | Summary of change: Modify the l'is the number of OFDM symbol number within the hop if SRShoppingNrofHops for SRS-PosResource is provided, and modify the typeface of the second sentence as Times New Roman, and modify the “consecutive OFDM symbol” as “consecutive OFDM symbols”. |
| Consequences if not approved: | there are some typos and error issues in the specification |
| < Unchanged parts are omitted > ### TS 38.211 6.4.1.4.1 SRS resource - NsymbSRS∈1,2,4,8,10,12,14 consecutive OFDM symbols given by the field nrofSymbols contained in the higher layer parameter resourceMapping. If Nhop>1, NsymbSRS is the number of consecutive OFDM symbols per hop. < Unchanged parts are omitted > ### 6.4.1.4.2 Sequence generation < Unchanged parts are omitted > where Msc,bSRS is given by clause 6.4.1.4.3, ru,vα,δ(n) is given by clause 5.2.2 with δ=log2KTC and the transmission comb number KTC∈2,4,8 is contained in the higher-layer parameter transmissionComb. The quantity l'∈0,1,…,NsymbSRS-1 is the OFDM symbol number within the SRS resource. < Unchanged parts are omitted > |  |

| FL | The Final TP agreed online is as follow: Agreement The TP below for 38.211 is endorsed. TP 2.4-2. reason for change:; 1. is the number of OFDM symbol number within the hop if SRShoppingNrofHops for SRS-PosResource is provided other than the OFDM symbol number within the SRS resource. 2. Align the typeface and description of the two sentences in the paragraph.. summary of change:; Summary of change: Modify the is the number of OFDM symbol number within the hop if SRShoppingNrofHops for SRS-PosResource is provided, and modify the typeface of the second sentence as Times New Roman, and modify the “consecutive OFDM symbol” as “consecutive OFDM symbols”.. Consequences if not approved:; there are some typos and error issues in the specification. < Unchanged parts are omitted > ### 6.4.1.4.1 SRS resource - consecutive OFDM symbols given by the field nrofSymbols contained in the higher layer parameter resourceMapping. If , is the number of consecutive OFDM symbols per hop. < Unchanged parts are omitted >. |
| --- | --- |

## Closed Timeline for scheduling aperiodic positioning SRS with frequency hopping [5]

### Text Proposal

| TP 2.5-1 |  |
| --- | --- |
| reason for change: | The following agreement regarding the minimal time interval necessary for the scheduling of aperiodic positioning SRS with frequency hopping was not captured in the specification. Agreement For aperiodic positioning SRS with frequency hopping, switching time to/from active UL BWP is added in the minimal time interval between the last symbol of PDCCH triggering A-SRS and the first symbol of the triggered SRS in the first hop. |
| summary of change: | To capture the above agreement in the specification. |
| Consequences if not approved: | Minimal time interval on the scheduling of aperiodic positioning SRS with frequency hopping is missing. |
| ------------------------------ TP#3: TS 38.214 ----------------------------------- ##### 6.2.1.4.1 SRS frequency hopping for positioning The reduced capability UE may be configured, via [uplinkTimeWindow-Config], subject to UE capability, with an UL time window where the UE is not expected to transmit other signals/channels and is only expected to transmit the SRS for positioning using frequency hopping. The UE is not expected to be configured with one [cycle] of the transmit frequency hopping, including the switching time from/to active BWP required ahead of the first hop and after the last hop, that is partially overlapped with the time window. For aperiodic positioning SRS with Tx frequency hopping, the minimal time interval between the last symbol of the PDCCH triggering the aperiodic SRS transmission and the first symbol of SRS resource is N2 symbols and an additional time duration corresponding to the switching time from the active uplink BWP if the first hop of the positioning SRS is outside the active UL BWP. < Unchanged text omitted > |  |

### First round

Companies are encouraged to comment on the TP 2.5-1 in the table below:

TP 2.5-1:

| Company | Comment |
| --- | --- |
| Huawei, HiSilicon | Do not support. We do not think it makes sense to define different UE timelines between the first hop being inside and outside the active UL BWP. |
| vivo | Same view as Huawei, “if the first hop of the positioning SRS is outside the active UL BWP.” may arise the different UE timelines |
| ZTE | OK to capture the agreement. |
| CATT | This TP is not needed. |
| Qualcomm | We would like to discuss this further to understand how people interpret the above agreement. |
| Nokia/NSB | Generally okay. We assume that overlapping between the first hop and within/outside active BWP should also be such case (i.e., outside BWP) |
| Ericsson | We see a potential issue with scheduling multiple UEs if the timeline varies due to leaving the BWP in the first hop. |
| LGE | The proposal seems reasonable and we are open to discuss further on this issue. |
| FL | Several companies commented that this TP would introduce different timelines between UEs. Considering the lack of time for discussion, I propose to bring back the TP next meeting. |

## (closed) Collision handling of positioning SRS with frequency hopping[5]

### Text Proposal

| TP 2.6-1 |  |
| --- | --- |
| reason for change: | For the timeline for determination of collision between PUSCH or PUCCH and positioning SRS with frequency hopping, it was agreed to use the smallest SCS between the SCS configured for the SRS with Tx hopping, the SCS of the PUSCH, and the SCS of the PDCCH for the calculation of N2. However, the SCS of the PUCCH should be also included for the calculation of N2. |
| summary of change: | Include the SCS of PUCCH for the calculation of N2 for determination of collision between PUSCH or PUCCH and positioning SRS with frequency hopping for RedCap UEs. |
| Consequences if not approved: | Timeline for determination of collision between PUCCH and positioning SRS with frequency hopping for RedCap UEs is not correct. |
| ------------------------------ TP#4: TS 38.214 ----------------------------------- ##### 6.2.1.4.1 SRS frequency hopping for positioning ##### < Unchanged text omitted > The reduced capability UE is expected to switch back to the active BWP if the time between two consecutive hops exceeds twice the switching time from/to the active BWP. For a transmission of a hop for an SRS resource for positioning with frequency hopping starting in symbol Nc1 and a colliding PUSCH or PUCCH transmission starting in symbol NS , the UE shall apply the dropping rules taking into account: - DCI(s) for which the time interval between the last symbol of PDCCH and the SRS symbol Nc1is at least N2 symbols and additional time duration TSRSh, where TSRSh is the switching time to/from the active BWP. - DCI(s) for which the time interval between the last symbol of PDCCH and the colliding PUSCH/PUCCH symbol NS is at least N2 symbols, where calculation of N2 is based on the smallest SCS between the SCS configured for positioning SRS with the frequency hopping, the SCS of the PUSCH/PUCCH, and the SCS of the PDCCH. < Unchanged text omitted > |  |

### First round

Companies are encouraged to comment on the TP 2.6-1 in the table below:

TP 2.6-1:

| Company | Comment |
| --- | --- |
| Huawei, HiSilicon | No need. Why would the SCS of PUSCH and PUCCH be different? |
| ZTE | OK |
| CATT | Not needed. |
| Qualcomm | I think the “PUCCH” is added to mean that if SRS collides with PUCCH, then the SCS of the PUCCH is taken into account. If that is the understanding, it looks OK to us. |
| Nokia/NSB | Ok |
| Ericsson | Ok |
| LGE | Ok |
| FL | Agreement TP 2.6-1 for 38.214 in section 2.6.1 of R1- 2401636 is endorsed |

## (closed) Collision rules between SRS and other signals/channels [8]

### Text Proposal

| TP 2.7-1 |  |
| --- | --- |
| reason for change: | The collision rules between SRS for Tx hopping and other signals/channels are not specified. Current collision rules can be reused with minor changes: considering the switching time from/to active BWP required. |
| summary of change: | when it comes to the collision rule between SRS for hopping and other signal/channel, considering the switching time to or from the active bandwidth part if the SRS transmission is configured for positioning frequency hopping |
| Consequences if not approved: | The collision rule between SRS for hopping and other signals/channels is not specified in TS 38.214. |
| TP#6-1 38.214 6.2.1 UE sounding procedure <Omitted> If a PUSCH transmission with a priority index 1 or a PUCCH transmission with a priority index 1 would overlap in time with an SRS transmission, including the switching time to or from the active bandwidth part if the SRS transmission is configured for positioning frequency hopping on a serving cell, the UE does not transmit the SRS in the overlapping symbol(s). <Omitted> For PUCCH and SRS on the same carrier, a UE shall not transmit SRS when semi-persistent or periodic SRS is configured in the same symbol(s) with PUCCH carrying only CSI report(s), or only L1-RSRP report(s), or only L1-SINR report(s), including the switching time to or from the active bandwidth part if the SRS is configured for positioning frequency hopping. A UE shall not transmit SRS when semi-persistent or periodic SRS is configured or aperiodic SRS is triggered to be transmitted in the same symbol(s) with PUCCH carrying HARQ-ACK, link recovery request (as defined in clause 9.2.4 of [6, 38.213]) and/or SR, including the switching time to or from the active bandwidth part if the SRS is configured for positioning frequency hopping. In the case that SRS is not transmitted due to overlap with PUCCH, only the SRS symbol(s) that overlap with PUCCH symbol(s) are dropped. PUCCH shall not be transmitted when aperiodic SRS, including the switching time to or from the active bandwidth part if the SRS is configured for positioning frequency hopping, is triggered to be transmitted to overlap in the same symbol with PUCCH carrying semi-persistent/periodic CSI report(s) or semi-persistent/periodic L1-RSRP report(s) only, or only L1-SINR report(s). 6.2.1.4 UE sounding procedure for positioning purposes <Omitted> For operation on the same carrier, if an SRS configured by the higher parameter SRS-PosResource, including the switching time to or from the active bandwidth part if the SRS is configured for positioning frequency hopping collides with a scheduled PUSCH, the SRS is dropped in the symbols where the collision occurs. |  |

### First round

Companies are encouraged to comment on the TP 7.1-1 in the table below:

TP 2.1-1:

| Company | Comment |
| --- | --- |
| Huawei, HiSilicon | We prefer not to spill over this frequency hopping feature beyond 6.2.1.4.1. |
| ZTE | Support, we are also ok to capture the collision rule for positioning SRS frequency hopping in section 6.2.1.4.1 of TS 38.214. |
| Qualcomm | Is there a way to just include this in the 6.2.1.4.1 section with a general statement instead of changing it in multiple places? |
| Nokia/NSB | We understand the issue, but frequency hopping for positioning is only supported for RedCap UE. The current spec clearly mentioned reduced capability UE, but the proposed text seems to include the normal NR UE. further discussion would be necessary. |
| Ericsson | We think the text in 6.2.1.4.1 is already capuring this when defining what is a colliding resource. |
| LGE | Similar as HW, QC. We prefer to find the way to include this in the 6.2.1.4.1 for preventing the confusion. |
| FL | The main concern is the location of the new text in the spec. My suggestion is to discuss a new version of the TP next meeting for section 6.2.1.4.1 |

## (closed) Collision rules between different types of SRS[8]

| TP 2.1-1 |  |
| --- | --- |
| Reason for change | : The collision rules between different types of SRS for Tx hopping should consider the switching time from/to active BWP. |
| summary of change: | when it comes to the collision rule between different types of SRS for Tx hopping, considering the switching time to or from the active bandwidth part if the SRS transmission is configured for positioning frequency hopping |
| Consequences if not approved: | : The collision rule between different types of SRS for frequency hopping is not captured in the spec. |
| 6.2.1 UE sounding procedure <Omitted> In case a SRS resource with resourceType set as 'aperiodic' is triggered on the OFDM symbol(s) configured with periodic/semi-persistent SRS transmission, the UE shall transmit the aperiodic SRS resource and only the periodic/semi-persistent SRS symbol(s) overlapping within the symbol(s), including the switching time to or from the active bandwidth part if the SRS is configured for positioning frequency hopping are dropped, while the periodic/semi-persistent SRS symbol(s) that are not overlapped with the aperiodic SRS resource are transmitted. In case a SRS resource with resourceType set as 'semi-persistent' is triggered on the OFDM symbol(s) configured with periodic SRS transmission, the UE shall transmit the semi-persistent SRS resource and only the periodic SRS symbol(s) overlapping within the symbol(s), including the switching time to or from the active bandwidth part if the SRS is configured for positioning frequency hopping are dropped, while the periodic SRS symbol(s) that are not overlapped with the semi-persistent SRS resource are transmitted. |  |

### First round

Companies are encouraged to comment on the TP 2.8-1 in the table below:

TP 2.8-1:

| Company | Comment |
| --- | --- |
| ZTE | Support, the priority/collision rule between different types of SRS transmission for Tx hopping was not defined yet. - the priority of aperiodic SRS transmission has higher priority than periodic or semi-persistent SRS transmission; - (2) the priority of semi-persistent SRS transmission has higher priority than periodic SRS transmission. |
| Nokia/NSB | We have the same comment. The frequency hopping for positioning is only supported for RedCap UE. The current spec clearly mentioned reduced capability UE, but the proposed text seems to include the normal NR UE. further discussion would be necessary. |
| FL | Similar to TP.2.7 The main concern is the location of the new text in the spec. My suggestion is to discuss a new version of the TP next meeting for section 6.2.1.4.1 |

## Timeline for semi-persistent SRS of frequency hopping

| TP 2.9-1 |  |
| --- | --- |
| reason for change: | For semi-persistent case, timeline for determination of collision between SRS for frequency hopping and other signals or channels was not defined. A UE may not be able to drop an SRS symbol without enough time for decoding MAC CE. |
| summary of change: | Specify the timeline for determination of collision between SRS for positioning frequency hopping and other signals or channels for semi-persistent case. |
| Consequences if not approved: | UE behavior on the condition of performing dropping SRS for Tx hooping for semi-persistent case is not clear |
| 6.2.1.4.1 SRS frequency hopping for positioning For a transmission of a hop for an SRS resource for positioning with frequency hopping starting in symbol Nc1 and a colliding PUSCH or PUCCH transmission starting in symbol NS , the UE shall apply the dropping rules taking into account: - DCI(s) for which the time interval between the last symbol of PDCCH and the SRS symbol Nc1is at least N2 symbols and additional time duration TSRSh, where TSRSh is the switching time to/from the active BWP. - DCI(s) for which the time interval between the last symbol of PDCCH and the colliding PUSCH/PUCCH symbol NS is at least N2 symbols, where calculation of N2 is based on the smallest SCS between the SCS configured for positioning SRS with the frequency hopping, the SCS of the PUSCH, and the SCS of the PDCCH. - PUSCH/PUCCH or SRS considered active at least N2 symbols and an additional time duration TSRSh before Nc1, and considered active at least N2 symbols before NS. |  |

### First round

Companies are encouraged to comment on the TP 2.9-1 in the table below:

TP 2.9-1:

| Company | Comment |
| --- | --- |
| Huawei, HiSilicon | It is not clear to us what this TP is mainly addressing. Is it about MAC CE activating hopping-SRS, and the to-be-activated hopping-SRS collides with other signals? |
| vivo | Even for MAC CE activating hopping-SRS, we are not sure the modification is correct or needed. |
| ZTE | To Huawei’s question: Yes. We tried to apply similar rule for the to-be-activated hopping-SRS collides with other signals as “UE sounding procedure between component carriers” in TS 38.214 as shown below: For an SRS transmission starting in symbol Nc1 of carrier c1 and a conflicting transmission in any carrier within the set Sc2 starting in symbol NS, the UE shall apply the prioritization / dropping rules in the remainder of this clause taking into account: - DCI(s) for which the time interval between the last symbol of PDCCH and Nc1 is at least N2 symbols and an additional time duration TSRSCS, and the time interval between the last symbol of PDCCH and NS is at least N2 symbols; and - semi-persistent CSI reports or SRS considered active at least N2 symbols and an additional time duration TSRSCS before Nc1, and considered active at least N2 symbols before NS.. |
| CATT | The motivation of this TP should be clarified first. |
| Nokia/NSB | We have a question to clearly understand the text. Shouldn’t we have to say “is considered active ?” |
| FL | To be discussed online |

# Measurement reporting

No issues proposed in contributions this meeting.

# DL-PRS Frequency Hopping

## PRS location request procedure

### Summary of contributions

In [8]  it is proposed to clarify the interpretation of the location request regarding the total bandwidth. The issue pointed is that the requested total bandwidth to be reported on may not be aligned with the PRS bandwidth in assistance data. From the FL perspective, the LMF is in charge of transferring the assistance data hence proper implementation can avoid the issue.

| Company | Proposal |
| --- | --- |
| [8] | Proposal 20: For a RedCap UE receiving nr-DL-PRS-RxHoppingTotalBandwidth in location information request, clarify with one of the following interpretations: - Interpretation 1: For each DL-PRS resource, the RedCap UE hops to a bandwidth of min {the requested bandwidth in request location information, the configured DL-PRS bandwidth in provide assistance data}. - Interpretation 2: A RedCap UE is requested to provide measurement result/location information for DL PRS Rx hopping with the requested total hop bandwidth, wherein the requested total bandwidth is measured by the UE within a DL PRS resource. Note: if the configured bandwidth of a DL-PRS PFL is smaller than the requested bandwidth, the UE is not requested to report measurements for the DL PRS PFL. |

### First round

Proposal 4-1.-1 For a RedCap UE receiving nr-DL-PRS-RxHoppingTotalBandwidth in location information request, clarify with one of the following interpretations:

- Interpretation 1: For each DL-PRS resource, the RedCap UE hops to a bandwidth of min {the requested bandwidth in request location information, the configured DL-PRS bandwidth in provide assistance data}.

- Interpretation 2: A RedCap UE is requested to provide measurement result/location information for DL PRS Rx hopping with the requested total hop bandwidth, wherein the requested total bandwidth is measured by the UE within a DL PRS resource.

- Note: if the configured bandwidth of a DL-PRS PFL is smaller than the requested bandwidth, the UE is not requested to report measurements for the DL PRS PFL.

Companies are encouraged to comment on the proposal and TP preference  in the table below:

Proposal 4-1.1:

| Company | Comment |
| --- | --- |
| Huawei, HiSilicon | We think it is RAN4 related work. We could recommend interpretation 1 to RAN4 and ask for feasibility. |
| vivo | We prefer the measurement bandwidth larger or equal than the nr-DL-PRS-RxHoppingTotalBandwidth in location information request |
| ZTE | RAN1 agreed that total hopping bandwidth can be requested but the corresponding UE’s behavior is not clear. We prefer to clarify it in RAN1. |
| CATT | We prefer interpretation 1. |
| Qualcomm | We don’t really see the ambiguity. It should be interpretation 1. |
| Nokia/NSB | We have a similar view with FL. It is not reasonable if the LMF requests frequency hopping bandwidth larger than the configured PRS BW. We don’t think this would be a problematic. |
| Ericsson | We cannot think of a case where this cannot be avoided by implementation. |
| LGE | We have same view with FL that the problematic situation can be avoided by LMF. |
| FL | Let’s see if the agreement on interpretation 1 is possible online. |

# UL SRS Tx Hopping

## UL SRS Tx hopping pattern

### Background

in [2],  the issue of overlapping frequency pattern is discussed. The contribution proposes to exclude the case of overlapping SRS resources, even if these resources are TDD from the hop perspective, from the supported cases. A similar proposal is discussed in [10].

| Company | Proposal |
| --- | --- |
| [2] | Proposal 12: UE does not expect to be configured with more than one SRS frequency hopping resources overlap. ● Endorse the following TP to clause 6.2.1.4.1 of TS 38.214. |
| [10] | Proposal 4 For operation in the same carrier, the reduced capability UE is not expected to be configured with an SRS resource of the transmit frequency hopping including the switching time to or from the active bandwidth part and an SRS resource with resourceType of both SRS resources as 'periodic', such that the first symbol of one SRS resource starts before the end of the other SRS resource, including all hops and retuning time. |

Associated TPs:

From [2]

Offline consensus:

| TP 5-1.1a |  |
| --- | --- |
| reason for change: | The collision for SRS resources overlapping is not defined in the specification |
| summary of change: | Proposed that the UE is not expected to be configured with overlapping SRS resources with TX hopping |
| Consequences if not approved: | UE behaviour definition is incomplete. |
| ---------------------------- Start of Text Proposal for TS 38.214 ---------------------------- < Unchanged parts are omitted > #### 6.2.1.4 UE sounding procedure for positioning purposes < Unchanged parts are omitted > ##### 6.2.1.4.1 SRS frequency hopping for positioning The reduced capability UE may be configured via [higher layer parameter], subject to UE capability, to perform transmit frequency hopping separate from the UL BWP configuration and outside of the UL BWP, where the UE may be configured with subcarrier spacing, CP and bandwidth that are different from the UL active BWP. The reduced capability UE transmit frequency hopping is configured within one SRS resource for positioning, that may be configured with a bandwidth larger than the maximum bandwidth of the reduced capability UE, in RRC_CONNECTED or RRC_INACTIVE mode. The reduced capability UE transmit frequency hopping, may be configured with overlapping or non-overlapping frequency hops in the frequency domain. When the reduced capability UE is configured to perform transmit frequency hopping: - it expects to be configured with the following parameters: - starting PRB of the first hop in time domain in [higher layer parameter] - starting slot offset and starting symbol for each hop in [higher layer parameter] - number of symbols in each hop in [higher layer parameter] - hop bandwidth in [higher layer parameter] - number of overlapping resource block(s) between hops, if present, in [higher layer parameter] - number of hops in [higher layer parameter]. - it does not expect to be configured withthe sum of [StartingSymbol] and [Length] for a hop that exceeds a slot duration. - it expects to be configured with the same periodicity of each hop of an SRS resource with the transmit frequency hopping. The reduced capability UE may be configured, via [uplinkTimeWindow-Config], subject to UE capability, with an UL time window where the UE is not expected to transmit other signals/channels and is only expected to transmit the SRS for positioning using frequency hopping. The UE does not expect to be configured with one [cycle] of the transmit frequency hopping, including the switching time from/to active BWP required ahead of the first hop and after the last hop, that is partially overlapped with the time window. The UE does not expect to be configured with more than one SRS resources for positioning with frequency hopping within any one [cycle] of the SRS transmission frequency hopping. < Unchanged parts are omitted > ---------------------------- End of Text Proposal for TS 38.214 ---------------------------- |  |

From [10]

| TP5.1-1b |  |
| --- | --- |
| Reason for Change: | The current specification text in 38.214 allow for overlapping SRS resources with FH as long these resources symbol do not overlap. |
| Summary of Change: | Clarify that two periodic SRS resource cannot overlap at all, that is to say one resource transmission cannot start before the previous one has ended. |
| Consequence if not approved: | The specification text supports a scheduling scheme that UE may not support for periodic UL SRS for positioning. |
| Text Proposal: | In: 38.214 6.2.1.4.1 SRS frequency hopping for positioning -----Text omitted ------ For operation in the same carrier, the reduced capability UE is not expected to be configured with an SRS resource of the transmit frequency hopping configured by the higher layer parameter [to_be_defined] including the switching time to or from the active bandwidth part and an SRS resource with resourceType of both SRS resources as 'periodic', so that the starting symbol of one SRS resource is located before or at the same position of the last symbol of the other SRS resource, including all configured hops and retuning time to or from the active bandwidth path. |

### (closed) First round

The issue was not yet discussed but is indeed important to fully define the collision rules.  We can discuss which TP is preferred from the ones submitted

Proposal 5-1.-1 for SRS for positioning with Tx hopping, the UE does not expect to be configured with overlapping SRS resources with Tx hopping

- The overlap is defined from the beginning of the first hop to the end of the last hop, including retuning time and time between hops.

Companies are encouraged to comment on the proposal in the table below:

Proposal 5-1.1:

| Company | Comment |
| --- | --- |
| vivo | We are okay for the proposal, and the sub-bullet can be removed or put as note |
| ZTE | OK |
| Qualcomm | The subbulet seems to be needed. The word “overlapping” should clarify that it means “time-domain overlapping” |
| Nokia/NSB | OK. We have a minor suggestion on the second bullet: The overlap is defined from the beginning starting symbol of the first hop to the last symbol end of the last hop, including retuning time and time between hops. |
| FL | Offline consensus to endorse the TP in [2]. However the TP was not agreeable online. Considering the lack of time for discusssion, let’s try to close the issue next meeting. |

## (closed) Support of SRS outside the BWP with a single hop

### Summary of contributions

In [2], it is propose to clarify that the UE may hop outside the active BWP but only perform a single hop. From the FL perspective, it seems this is already supported in RAN1 specifications and RRC, since the number of hop can be 1. Hence a conclusion is proposed.

Company views are summarized in the table below:

| Company | Proposal |
| --- | --- |
| [2] | Proposal 14: RAN1 to conclude that network may configure positioning SRS outside the active UL BWP without frequency hopping (number of hops equal to 1). |

### First round

Proposal 5.2-1: (for conclusion) The network may configure positioning SRS outside the active UL BWP without frequency hopping (number of hops equal to 1

Offline consensus:

Proposal 5.2-1: (for conclusion) The network may configure positioning SRS outside the active UL BWP with tx hopping configured with the number of hops equal to 1.

Companies are encouraged to comment on the proposal in the table below:

Proposal 5.2-1:

| Company | Comment |
| --- | --- |
| vivo | More clarification is needed for the motivation considering the single hop bandwidth will be smaller than 20M for FR1. So, why we need to do one hop outside the active UL BWP? Conclusion For the positioning of redcap UEs, for the DL PRS reception and UL SRS transmission, the maximum hopping bandwidth for a single hop is 20MHz for FR1 and 100MHz with FR2.. |
| ZTE | OK with the conclusion, the hop number can be configured to {1, ..., 6} which includes 1. |
| Qualcomm | We prefer NOT to call it “without frequency hopping”, but rather with “frequency hopping with number of hops equals to 1” |
| Nokia/NSB | Ok |
| FL | We have an offline consensus from Monday’s offline: Offline consensus: Proposal 5.2-1: (for conclusion) The network may configure positioning SRS outside the active UL BWP with tx hopping configured with the number of hops equal to 1. Online conclusion: Conclusion The network may configure positioning SRS outside the active UL BWP with Tx hopping configured with the number of hops equal to 1. |

## (closed) Use of SRS UTW

### Summary of contributions

In[6] it is proposed to clarify that UTW and measurement gaps should not be configured at the same time for the UE. the proposal is to clarify that the UE should interpret such event as a misconfiguration.

Company views are summarized in the table below:

| Company | Proposal |
| --- | --- |
| [6] | Proposal 3: A UE does not expect both to be configured with an UTW and a measurement gap simultaneously. |

### First round

Proposal 5.3-1: A UE does not expect both to be configured with an UTW and a measurement gap simultaneously.

Offline discussion (closed):

Proposal 5.3-1: A UE does not expect both to be configured with an UTW and a measurement gapoverlapping in time domain..

Companies are encouraged to comment on the proposal in the table below:

Proposal 5.3-1:

| Company | Comment |
| --- | --- |
| Huawei, HiSilicon | We support it. Otherwise, a new capability is needed. |
| vivo | OK |
| ZTE | The wording is not clear enough, a UE can be configured with both UTW and measurement gap as long as they do not overlap in time domain. We prefer the following change: A UE does not expect both to be configured with an UTW and a measurement gap overlapping in time domain. |
| CATT | OK |
| Qualcomm | UTW is about UE transmission. Measurement gap is about UE reception. Does the above proposal means that the UE is not expected to transmit SRS within a measurement gap? |
| Nokia/NSB | We don’t think the proposal is necessary. The configuration can be provided at the same time, but these would not be used at the same time. |
| FL | Offline discussion (closed): Proposal 5.3-1: A UE does not expect both to be configured with an UTW and a measurement gapoverlapping in time domain.. |

## (closed) Availability of UTW to UL transmission when no UL SRS is transmitted

### Summary of contributions

In [6] and [7] it is proposed to support UL transmissions in UTW when no SRS is transmitted.

Company views are summarized in the table below:

| Company | Proposal |
| --- | --- |
| [6] | Proposal 4: UL signals or channels can be transmitted during the UL time window if SRS for positioning is not scheduled to be transmitted during the UL time window. |
| [7] | Proposal 1: Adopt the following TP in TS 38.214 to support the UE to transmit PUCCH or PUSCH if SRS for positioning is not scheduled during a UTW. The reason for the change is to enable transmission of UL channels in the UTW. The summary of change is introduction of a condition for transmission of UL channels in the UTW. The consequence if not approved is that the UE may not be able to transmit UL channels, interrupting communication frequently if UTW is configured. |

### First round

We can first discuss the proposal and further discuss to the TP in case we have an agreement.

Proposal 5.4-1: UL signals or channels can be transmitted during the UL time window if SRS for positioning is not scheduled to be transmitted during the UL time window.

Companies are encouraged to comment on the proposal in the table below:

Proposal 5.4-1:

| Company | Comment |
| --- | --- |
| InterDigital | Support |
| vivo | It is more like enhancement, we prefer not to pursuing enhancement in the maintenance |
| ZTE | This proposal is not aligned with our previous agreement. |
| Qualcomm | Not really needed; we think it is an enhancement. |
| Nokia/NSB | Our first preference is to not change the current spec. According to the proposal, the UE can transmit UL signals/channels, and it looks like it is up to the UE. The network needs a clear expectation. We may be okay with a revision. “The UE should transmit UL signals or channels within the UL time window if SRS frequency hopping for positioning is not configured to be transmitted within the UL time window.” |
| Ericsson | Do not support |
| FL | Considering the lack of support, let’s close the issue. |
| LGE | There is no need to prohibit the other UL transmission if if SRS for positioning is not scheduled to be transmitted within the UTW. Hence we support this proposal. |

## (closed) LMF requests for the SRS with hopping

### Summary of contributions

In [6] it is proposed to details what the LMF may request in terms of Tx hoppoing parameters:

Company views are summarized in the table below:

| Company | Proposal |
| --- | --- |
| [6] | Proposal 6: As part of the LMF request for SRS for positioning configuration, support the following to be included in the request: - An indicator to request SRS with Tx hopping - The amount of overlap between hops - Total bandwidth across hops - The existing field for bandwidth in the SRS for positioning LMF request can be re-used |

### First round

Proposal 5.4-1: As part of the LMF request for SRS for positioning configuration, support the following to be included in the request:

-	An indicator to request SRS with Tx hopping

-	The amount of overlap between hops

-	Total bandwidth across hops

	The existing field for bandwidth in the SRS for positioning LMF request can be re-used

Companies are encouraged to comment on the proposal in the table below:

Proposal 5.4-1:

| Company | Comment |
| --- | --- |
| Huawei, HiSilicon | We believe there is no such change needed. Radio resource allocation details is up to RAN, and LMF could use the overall measurement BW to assist RAN allocate the resource based on conditions core network does not have access to. |
| vivo | We prefer only support the last sub-bullet. |
| ZTE | Not sure about the benefit and necessity on introducing amount of overlap between hops in the request. |
| Nokia | Support. |
| Ericsson | This is an optimization in our view. Ok with the last bullet, but it is not needed, as said above from Huawei’s comment. |
| LGE | Seems not necessary. |
| FL | Considering the lack of support and many comments on the lack of need, let’s close the issue. |

## (closed)CPP with redcap UEs

### Summary of contributions

In [6] it is proposed to clarify that redcap positioning does not support carrier phase positioning.

Company views are summarized in the table below:

| Company | Proposal |
| --- | --- |
| [6] | Proposal 5: CPP should not be combined with RedCap positioning. |

### First round

Proposal 5.4-1: Proposal 5: CPP should not be combined with RedCap positioning.

Companies are encouraged to comment on the proposals in the table below:

Proposal 5.4-1:

| Company | Comment |
| --- | --- |
| Huawei, HiSilicon | RedCap UE positioning could have non-frequency hopping feature, and it is not clear why it cannot be supported. |
| CATT | We don’t think such limitation is needed. CPP can be combined with RedCap UE positioning. |
| Nokia/NSB | We understand the intention as the CPP and RedCap positioning have been discussed separately. However, we are not sure if the CPP should be ruled out. |
| LGE | Based on the evaluation results during SI phase, our understanding is that liminted BW size without FH support is not enough for acheiving higher accuracy due to the lack of integer ambiguity resolution. Meanwhile, supporting RedCap CPP with FH may requires additional specification works which may not acceptable at this stage. From this perspective, we think it is pre-mature to support CPP combined with RedCap positioning. |

## TA command with different SCS

### Summary of contributions

In [9] the issue of potential misalignment of the expected time for using the received TA command when Tx hopping is operating.  Two options are proposed and associated TPs are provided. One option would be to specify which of the SCS configured for TA or for Tx hopping should be used to count the slots before using the new TA. Alternatively, the TA timeline should be instead postponed to the end of the Tx hopping resource.

Company views are summarized in the table below:

| Company | Proposal |
| --- | --- |
| [9] | Proposal 2: For TA adjustment operation of UE configured with SRS for positioning frequency hopping, RAN1 supports: - Alt 1: The UE configured with SRS for positioning with frequency hopping is expected to determine the SCS for TA adjustment with including the SCS of SRS for positioning with frequency hopping. - Alt 2: If UE is indicated to apply an adjustment for the uplink transmission timing at a slot where SRS for positioning with frequency hopping is configured/indicated, the UE is expected to apply a corresponding adjustment for the uplink transmission timing at the slot after the slot in which last hop is configured. Proposal 3: Support either Alt 1 or Alt 2 to resolve the ambiguity of the UE indicated TA adjustment when SRS for positioning with frequency hopping is configured: |

### First round

Let’s first discuss the issue before proceeding to TP:

Proposal 5.7-1: For TA adjustment operation of UE configured with SRS for positioning frequency hopping, RAN1 supports:

- Alt 1:  The UE configured with SRS for positioning with frequency hopping is expected to determine the SCS for TA adjustment with including the SCS of SRS for positioning with frequency hopping.

- Alt 2: If UE is indicated to apply an adjustment for the uplink transmission timing at a slot where SRS for positioning with frequency hopping is configured/indicated, the UE is expected to apply a corresponding adjustment for the uplink transmission timing at the slot after the slot in which last hop is configured.

Companies are encouraged to comment on the proposals in the table below:

Proposal 5.7-1:

| Company | Comment |
| --- | --- |
| Huawei, HiSilicon | Do not support. No change to the TA mechanism at this stage. |
| vivo | We think the SCS of virtual BWP also belongs the “the SCSs of all configured UL BWPs for all uplink carriers in the TAG” in current specification |
| ZTE | Do not support. |
| CATT | We prefer not to pursue the TA enhancement in the maintenance phase. |
| Qualcomm | Similar view ith vivo actually, so we dont think a change is needed. |
| LGE | After offline discussion, I realize that companies have different understanding of the current specification on how to determine TA timing/values. Therefore, we believe it is necessary to at least clarify the behavior of the UE on this issue. As you can see 38.213, the minimum SCS for UL BWP is used for for TA adjustment timing determination. Our understanding is that the SCS configured for the SRS positioning with FH should be included for this rule. However, some company thinks that SCS for SRS positioning with FH should be excluded. Therefore, it is recommended to discuss and determine the appropriate UE behavior between the two interpretations and change the specification accordingly. Our preference is to take account the SCS of SRS positioning with FH as well as othe UL BWP; Alt 1 in the proposal. Meanwhile, Alt 2 can be another method to solve this ambuiguity problem. Additionally, it would be useful to prevent measurement performance degradation of the neighbor gNB. If TA is adjusted in a middle of SRS positioning with FH cycle, timing measurements of neighbor gNBs will be currupted since gNBs do not have prior information on TA adjustment of the serving cell. This issue can be solved by not applying TA adjustment in the middle of SRS positioning with FH cycle. Note that similar mechanism is already supported for the DMRS bundling for coverage enhacement. From LG’s perspective, our preference is Alt 2 that is useful not only for solving ambiguity of TA adjustment but also for guranteeing measurement accuracy. But we also ok with Alt 1 if Alt 2 is not acceptable. For a timing advance command received on uplink slot n and for a transmission other than a PUSCH scheduled by a RAR UL grant or a fallbackRAR UL grant as described in clause 8.2A or 8.3, or a PUCCH with HARQ-ACK information in response to a successRAR as described in clause 8.2A, the corresponding adjustment of the uplink transmission timing applies from the beginning of uplink slot n+k+1+2μ∙Koffset where k=Nslotsubframe, μ∙NT,1+NT,2+NTA,max+0.5Tsf, NT,1 is a time duration in msec of N1 symbols corresponding to a PDSCH processing time for UE processing capability 1 when additional PDSCH DM-RS is configured, NT,2 is a time duration in msec of N2 symbols corresponding to a PUSCH preparation time for UE processing capability 1 [6, TS 38.214], NTA,max is the maximum timing advance value in msec that can be provided by a TA command field of 12 bits, Nslotsubframe, μ is the number of slots per subframe, Tsf is the subframe duration of 1 msec, and Koffset=Kcell,offset-KUE,offset, where Kcell,offset is provided by cellSpecificKoffset and KUE,offset is provided by a Differential Koffset MAC CE command [11, TS 38.321]; otherwise, if not respectively provided, Kcell,offset=0 or KUE,offset=0. N1 and N2 are determined with respect to the minimum SCS among the SCSs of all configured UL BWPs for all uplink carriers in the TAG and of all configured DL BWPs for the corresponding downlink carriers. For μ=0, the UE assumes N1,0=14 [6, TS 38.214]. Slot n and Nslotsubframe, μ are determined with respect to the minimum SCS among the SCSs of all configured UL BWPs for all uplink carriers in the TAG. NTA,max is determined with respect to the minimum SCS among the SCSs of all configured UL BWPs for all uplink carriers in the TAG and for all configured initial UL BWPs provided by initialUplinkBWP. The uplink slot n is the last slot among uplink slot(s) overlapping with the slot(s) of PDSCH reception assuming TTA=0, where the PDSCH provides the timing advance command and TTA is defined in [4, TS 38.211].. |

## (closed) Transmit power for SRS with Tx hopping

### Summary of contributions

In [10], the issue of the definition of the transmit occasion for the purpose of computing the transmit power is raised:

Company views are summarized in the table below:

| Company | Proposal |
| --- | --- |
| [6] | The SRS transmit occasion for SRS with tx hopping includes all hops, and may span multiple slots The power of the transmitted SRS with TX hopping is constant over all hops in a SRS transmission spanning multiple slots. - For the purpose of power control, the UE determine the SRS transmission power for all the hops in the transmission using the first slot of the SRS transmission as the slot for the SRS transmission occasion. |

### First round

Proposal 5.8-1: For the definition of an SRS resource with Tx hopping transmission occasion and transmit power:

- The SRS transmit occasion for SRS with tx hopping includes all hops, and may span multiple slots

- The power of the transmitted SRS with TX hopping is constant over all hops in a SRS transmission spanning multiple slots

- For the purpose of power control, the UE determine the SRS transmission power for all the hops in the transmission using the first slot of the SRS transmission as the slot for the SRS transmission occasion.

Companies are encouraged to comment on the proposals in the table below:

Proposal 5.8-1:

| Company | Comment |
| --- | --- |
| Huawei, HiSilicon | We understand the intention of this proposal, but we believe in most cases, the transmission power change between SRS hops is not likely. |
| vivo | For the larger gap case, the transmission power may be changed. But redefined the transmission occasion may be still unclear, considering the transmission power is calculated based on configured parameters (such as alpha, p0) andPLb,f,c(qd) , and the configured parameters are the same, so that we propose only to add sentence as follows: PLb,f,c(qd) is a downlink pathloss estimate in dB across all the hops of one SRS resource are the same. Agreement The UE is expected to switch back to the active BWP when the time between two consecutive hops exceeds twice the switching time to/from the BWP. - Note: this is applicable when UTW is configured or not configured.. |
| CATT | We prefer the transmission power for different SRS hops should be the same. Only the last bullet in the proposal is needed. |
| Qualcomm | Similar view with Huawei. We dont really think it is a likely case, and we prefer to not optimize further. |
| Nokia/NSB | An SRS resource for frequency hopping includes overall frequency hops. The transmission power should be the same across all hops as the power as all hops are within a single SRS resource. We are not sure if this clarification is necessary. |
| FL | From the received comments at the offline session, a majority of companies see no issue. Let’s close this topic. |

# Online sessions

## Tuesday session

TPs for endorsements:

Proposal:

TP 2.2-1 for 38.211 in section 2.2.1 of R1- 2401636 is endorsed

Proposal:

TP 2.4-2 for 38.211 in section 2.4.3 of R1- 2401636 is endorsed

Proposal:

TP 2.6-1 for 38.214 in section 2.6.1 of R1- 2401636 is endorsed

Proposal:

TP 5.1-1a for 38.214 in section 5.2.1 of R1- 2401636 is endorsed

Offline consensus:

Proposal 5.2-1: (for conclusion) The network may configure positioning SRS outside the active UL BWP with tx hopping configured with the number of hops equal to 1.

Offline discussion:

Proposal 5.3-1: A UE does not expect both to be configured with an UTW and a measurement gapoverlapping in time domain..

## Thursday session

Proposal 4-1.-1 For a RedCap UE receiving nr-DL-PRS-RxHoppingTotalBandwidth in location information request, clarify with one of the following interpretations:

- Interpretation 1: For each DL-PRS resource, the RedCap UE hops to a bandwidth of min {the requested bandwidth in request location information, the configured DL-PRS bandwidth in provide assistance data}.

- Send an LS to RAN4 with this agreement

Proposal 5.7-1: For TA adjustment operation of UE configured with SRS for positioning frequency hopping, RAN1 supports:

- Alt 1:  The UE configured with SRS for positioning with frequency hopping is expected to determine the SCS for TA adjustment with including the SCS of SRS for positioning with frequency hopping.

- Alt 2: If UE is indicated to apply an adjustment for the uplink transmission timing at a slot where SRS for positioning with frequency hopping is configured/indicated, the UE is expected to apply a corresponding adjustment for the uplink transmission timing at the slot after the slot in which last hop is configured.

TPs for discussion:

Proposal:

TP 2.1-1 for 38.211 in section 2.1.1 of R1- 2401637 is endorsed

Proposal:

TP 2.9-1 for 38.214 in section 2.4.3 of R1- 2401637 is endorsed

# Conclusion

TBD

# References

RP-223549, “New WID on expanded and improved NR positioning”, RAN#98-e, December 12-16, 2022.

R1-2400138	Maintenance of expanded and improved NR positioning	Huawei, HiSilicon

R1-2400191	Remaining issues on NR Positioning	Nokia, Nokia Shanghai Bell

R1-2400219	Maintenance on Rel-18 Positioning	vivo

R1-2400374	Maintenance issues on Rel-18 Positioning	Intel Corporation

R1-2400989	Remaining Issues On Expanded and Improved Positioning	Apple

R1-2401039	Discussion on remaining issues for R18 NR positioning	InterDigital, Inc.

R1-2401073	Maintenance on expanded and improved NR positioning	ZTE

R1-2401247	Maintenance of expanded and improved NR positioning	LG Electronics

R1-2401351	Remaining issues on expanded and improved NR positioning	Ericsson  Incorporated