3GPP TSG RAN WG1 #116			R1-2401645

Athens, Greece, February 26th – March 1st, 2024

Agenda item:	8.12.5

Source: 	Moderator (NTT DOCOMO, INC.)

Title: 	Summary#2 of discussion on UE features for dedicated spectrum less than 5MHz

Document for:	Discussion and Decision

- Introduction

This document summarizes contributions submitted to AI 8.12.5 regarding UE features for dedicated spectrum less than 5MHz.

According to the updated UE features list agreed in RAN1#115 [1], there are following feature groups for dedicated spectrum less than 5MHz.

- FGs for NR support for dedicated spectrum less than 5MHz for FR1

  - 51-1	Support for 3 MHz channel bandwidth

  - 51-2	Support 12 PRB CORESET0

  - 51-3	Support 5 MHz channel bandwidth with 20 PRB CORESET0

- FGs for NR support for dedicated spectrum less than 5MHz for FR1

In [1], FGs for NR support for dedicated spectrum less than 5MHz for FR1 are captured as below.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 51. NR_FR1_lessthan_5MHz_BW | 51-1 | Support for 3 MHz channel bandwidth | 1) Reception of 12 PRB PBCH based on RB-level puncturing 2) Short RACH preamble formats with 15kHz SCS, and long PRACH formats with 1.25kHz SCS 3) Reception of 15 PRB CORESET0 |  | Yes | N/A | UE is not able to support 3 MHz channel bandwidth with 15 PRB CORESET0 | Per Band | FDD only | FR1 only | N/A | This FG is supported for 15 kHz SCS only This FG is applicable only when an associated SS/PBCH block is located according to Table 5.4.3.3-2 in TS 38.101-1 in Rel-18 Note: The UE supporting this FG supports configuration of 15 PRB BWP operation | Optional with capability signalling |
| 51. NR_FR1_lessthan_5MHz_BW | 51-2 | Support 12 PRB CORESET0 | 1) Reception of 12 PRB CORESET0 | 51-1 | Yes | N/A | UE is not able to support 3 MHz channel bandwidth with 12 PRB CORESET0 | Per Band | FDD only | FR1 only | N/A | This FG is supported for 15 kHz SCS only Note: The UE supporting this FG supports configuration of 12 PRB BWP operation | Optional with capability signalling |
| 51. NR_FR1_lessthan_5MHz_BW | 51-3 | Support 5 MHz channel bandwidth with 20 PRB CORESET0 | 1) Short RACH preamble formats with 15kHz SCS, and long PRACH formats with 1.25kHz SCS 2) Reception of 20 PRB CORESET0 |  | Yes | N/A | UE is not able to support 5 MHz channel bandwidth with 20 PRB CORESET0 | Per UE | FDD only | FR1 only | N/A | This FG is supported for 15 kHz SCS only This FG is only applicable when an associated SS/PBCH block is located in band n100 at GSCN 41638 of Table 5.4.3.1-3 in TS 38.101-1 in Rel-18. Note: The UE supporting this FG supports configuration of 20 PRB BWP operation | Optional with capability signalling |

Following inputs are provided in contributions for the RAN1#116 meeting.

| [2] | Nokia, NSB | Applicability of 51-2 – 12 PRB CORESET#0 As discussed in RAN1#114bis and RAN1#115 already, we think the usage of the 12 PRB CORESET#0 should be limited to the specific sync raster point having a 12 PRB TX bandwidth configuration (i.e. GSCN 41638 of Table 5.4.3.1-3 in TS 38.101-1). For the 5MHz channel bandwidth, this is already captured that the punctured 20 PRB CORESET#0 is only applicable to the specific sync raster points of 20 PRB TX bandwidth configuration. But a related note for the 12 PRB CORESET#0 is still missing. We think that a related note should also be added to 51-2, based on the following reasoning: - During RAN1#113, it was extensively discussed if the 12 PRB CORESET#0 is to be used for all 3MHz channel bandwidth sync raster points and the conclusion was to support the 15 PRB CORESET#0 for the normal 3MHz channel bandwidth (with 3MHz / 15 PRB transmission bandwidth) to achieve better performance compared to using the 12 PRB CORESET#0 design. Therefore, it would be now slightly strange to require a UE to support the 12 PRB CORESET#0 for normal 3MHz channel operation for other than the specific sync raster point requiring the 12 PRB transmission BW. - We think the operation with the 3MHz and 5MHz channel bandwidths should be logically aligned: - For 3MHz and 5MHz channel bandwidths with the transmission bandwidths equal to the channel bandwidth (i.e. 15 & 25 PRBs), the maximum CORESET#0 bandwidth of 15 PRB and 24 PRB should be applied. - For 3MHz and 5MHz channel bandwidths with maximum transmission bandwidth configurations of 12 PRB and 20 PRBs with the specific sync raster points of Table 5.4.3.1-3, CORESET#0 bandwidth should be equal to the maximum transmission bandwidth configurations of 12 and 20 PRBs, respectively. - A reasonable gNB implementation for a 3 MHz carrier would anyhow indicate the 15PRB CORESET#0 for a 3MHz channel bandwidth carrier (with 15 PRB TX bandwidth), as the gNB would not be aware if a UE accessing the cell would support the 12 PRB CORESET#0 in addition to the 15 PRB CORESET#0. And even if some UEs would be supporting it, as the indication of the CORESET#0 is per MIB, the gNB would need to react on the least capable UE for a carrier and that would be a UE supporting the 15 PRB CORESET#0 only. So why would the UE supporting the 12 PRB CORESET#0 for the specific sync raster point and indicating the support in band n100 for the specific sync raster point, would need to support the same also for the sync raster points of 3MHz channel bandwidth with 15 PRB. - And most importantly, the 12 PRB CORESET#0 would only allow for 12 PRB BWP operation also for 3MHz channel bandwidths with 15 PRB TX bandwidth as alignment with the common RB grid is not possible, based on the following: - During the WI phase, we discussed the required RB offsets and k_SSB offsets in order to enable 12 PRB and 15 PRB BWP operation for the 12 PRB & 15 PRB TX bandwidth which lead to the following RAN1 agreement: Agreement Revise the previous agreement as below, Agreement For 3MHz channel BW, for kssb and PRB offset for the determining the CORESET#0 position in frequency domain, - kssb follows legacy configuration. - Note: based on existing specifications, UE does not expect other values than kssb = 0 for 12PRBs CORESET#0, [kssb = 8 [or 20] for 15PRBs CORESET#0 with offset 0 PRBs, kssb = 4 [or 16] for 15RPBs CORESET#0 with offset 2 PRBs] - PRB offset - For 12 PRBs CORESET#0 transmission BW, PRB offset = 0 - For 15 PRBs CORESET#0 transmission BW, at least PRB offset = 0, [2] 2 are supported - FFS if offset = 1, 3 are supported. Note: the above assumes that PRB offset to be the frequency gap between the first PRB of the actually transmitted 12/15 PRBs CORESET#0 and the first CRB overlapping with the first PRB of the actually transmitted 12 PRBs SS/PBCH block, how to capture it in the specification is left to editors. Note: the range of the PRB offsets is only applicable to the introduced new sync. raster points for 3MHz channel BW and 100kHz channel raster.. - For the new 3MHz sync raster points for 3MHz channel bandwidth with 15 PRB TX bandwidth according to Table 5.4.3.3-2 in TS 38.101-1, a k_SSB offset of 4 or 8 would need to be indicated to the UE to align the CORESET#0 with the common RB grid (based on the blue parts) in order to fit a 15 PRB BWP in the 3MHz channel bandwidth with 15 PRB TX bandwidth. But based on the related agreements in the yellow parts, the UE would not expect any other value than k_SSB=0. - The fact, that when using 12 PRB CORESET#0 for 15 PRB TX bandwidth being non-aligned with the common RB grid leads to the fact that 15 PRB BWP configuration could then not be supported and the gNB could only configure 12 PRB BWP also for a normal, 15 PRB 3MHz channel as 15 not fitting and the gNB being unaware of the UE supporting any other PRB size than 12 & 15 PRBs. - Therefore, this would be leading to a performance reduction as only 12/15=80% PRB utilization is achieved compared to using the 15 PRB CORESET#0. Observation 4.1: In addition to the earlier discussed arguments, it should be noted that applying the 12 PRB CORESET#0 to 3MHz channels with 15 PRB TX bandwidths (of Table 5.4.3.3-2 in TS 38.101-1) leads to a maximum PRB utilization of 80 % (12 out of 15 PRBs) and consequently reduced radio performance & throughput, as the PRBs would not be aligned with the common RB grid based on the supported k_SSB=0 for 12 PRB CORESET#0. Therefore, we suggest restricting the support of 12 PR CORESET#0 to the single special sync raster point with 12 PRB X bandwidth by adding a similar note as for 51-3 also to 51-2 as: This FG is only applicable when an associated SS/PBCH block is located in band n100 at GSCN 41637 of Table 5.4.3.1-3 in TS 38.101-1 in Rel-18. Based on the discussion the following is proposed: Proposal 4.1: FG51-2 (12 PRB CORESET#0) is only applicable to the single sync raster point with 12 PRB TX bandwidth by applying the following modifications to FG51-2: - Add the following note: This FG is only applicable when an associated SS/PBCH block is located in band n100 at GSCN 41637 of Table 5.4.3.1-3 in TS 38.101-1 in Rel-18. - Change the reporting type from ‘Per Band’ to ‘Per UE’ - i.e. adopt the following changes in red: Index; Feature group; Components; Type; Note. 51-2; Support 12 PRB CORESET0; 1) Reception of 12 PRB CORESET0; Per UE Band; This FG is supported for 15 kHz SCS only This FG is only applicable when an associated SS/PBCH block is located in band n100 at GSCN 41637 of Table 5.4.3.1-3 in TS 38.101-1 in Rel-18. Note: The UE supporting this FG supports configuration of 12 PRB BWP operation. |
| --- | --- | --- |
| [3] | Ericsson | For the 3 MHz channel bandwidth (CBW), and for FG 51-1 when transmissions span up to 15-PRBs “with 15 PRB CORESET0,” the endorsement post-RAN1# 115 is as follows [2]: 51. NR_FR1_lessthan_5MHz_BW; 51-1; Support for 3 MHz channel bandwidth; 1) Reception of 12 PRB PBCH based on RB-level puncturing 2) Short RACH preamble formats with 15kHz SCS, and long PRACH formats with 1.25kHz SCS 3) Reception of 15 PRB CORESET0; Yes; N/A; UE is not able to support 3 MHz channel bandwidth with 15 PRB CORESET0; Per Band; FDD only; FR1 only; N/A; This FG is supported for 15 kHz SCS only This FG is applicable only when an associated SS/PBCH block is located according to Table 5.4.3.3-2 in TS 38.101-1 in Rel-18 Note: The UE supporting this FG supports configuration of 15 PRB BWP operation; Optional with capability signalling. For FG 51-1, we have the following observation: FG 51-1 seems to be stable. Perhaps the only amendment to be performed is replacing “RB-level puncturing” by “PRB-level puncturing” as to be consistent with similar editorial updates recently performed. For the 3 MHz channel bandwidth (CBW), and for FG 51-2 when transmissions span up to 12-PRBs “with 12 PRB CORESET0,” the endorsement post-RAN1# 115 is as follows [2]: 51. NR_FR1_lessthan_5MHz_BW; 51-2; Support 12 PRB CORESET0; 1) Reception of 12 PRB CORESET0; 51-1; Yes; N/A; UE is not able to support 3 MHz channel bandwidth with 12 PRB CORESET0; Per Band; FDD only; FR1 only; N/A; This FG is supported for 15 kHz SCS only Note: The UE supporting this FG supports configuration of 12 PRB BWP operation; Optional with capability signalling. For FG 51-1, we have the following observations and proposal: For FG 51-2, the following note should be added: “This FG is only applicable when an associated SS/PBCH block is located in band n100 at GSCN 41637 of Table 5.4.3.1-3 in TS 38.101-1 in Rel-18”, like the note added in RAN1# 114bis for FG 51-3. In relation with the previous observation, the absence of the note implies 12-PRB CORESET0 can be used at sync-raster points other than “GSCN 41637,” which results in the following remarks and technical issues: - A 12-PRB CORESET0 could in any case only be used after initial access. - There is an unclear motivation about downgrading the performance when passing from using a 15-PRB CORESET0 (which shall be used during initial access) to a 12-PRB CORESET0 after initial access. - There is an unclear motivation about underutilizing available spectrum since using a 12-PRB CORESET0 leads to 3-PRB unused. - It is unclear under which basis/criteria the gNodeB should switch from a 15-PRB CORESET0 to a 12-PRB CORESET0. - More importantly, if the gNodeB switches the cell’s CORESET0 to a 12-PRB CORESET0, then there will be a major issue for UEs entering the cell that only support 15-PRB CORESET0. For FG 51-2, given that it is applicable to a single sync-raster point in only one frequency band (n100), the Type can be updated to be “Per UE” just as it was done for FG 51-3. For FG 51-2, add the following note: “This FG is only applicable when an associated SS/PBCH block is located in band n100 at GSCN 41637 of Table 5.4.3.1-3 in TS 38.101-1 in Rel-18”, similarly to the note added in RAN1# 114bis for FG 51-3. For FG 51-2 which is applicable to a single sync-raster point in only one frequency band (n100), update the Type to be “Per UE” just as it was done for FG 51-3. For the 5 MHz channel bandwidth (CBW), and for FG 51-3 when transmissions span up to 20-PRBs “with 20 PRB CORESET0,” the endorsement post-RAN1# 115 is as follows [2]: 51. NR_FR1_lessthan_5MHz_BW; 51-3; Support 5 MHz channel bandwidth with 20 PRB CORESET0; 1) Short RACH preamble formats with 15kHz SCS, and long PRACH formats with 1.25kHz SCS 2) Reception of 20 PRB CORESET0; Yes; N/A; UE is not able to support 5 MHz channel bandwidth with 20 PRB CORESET0; Per UE; FDD only; FR1 only; N/A; This FG is supported for 15 kHz SCS only This FG is only applicable when an associated SS/PBCH block is located in band n100 at GSCN 41638 of Table 5.4.3.1-3 in TS 38.101-1 in Rel-18. Note: The UE supporting this FG supports configuration of 20 PRB BWP operation; Optional with capability signalling. For FG 51-3, we have the following observation: FG 51-3 seems to be stable; no further updates seem to be needed. |
| [4] | MediaTek | In the RAN#99 reply LS [3], the following was agreed: RAN Plenary has discussed question 2 on the feature list to be considered, and concluded that the less-than-5MHz WI in Rel-18 should consider single-carrier operation, excluding RedCap. In addition, UE speeds up to 500km/h should be targeted for Band n100 without impact to RAN1. Both the updated WID in [3] and the WI summary in have reflected the RAN agreement. However, it has not been implemented into specifications. We have the following proposal. Proposal 2: Add the following notes to FG 51-1, 51-2, and 51-3. - This FG is only applicable to single-carrier operation. - This FG is not applicable to UEs indicating supportOfRedCap-r17 (i.e., FG 28-1) or supportOfERedCap-r18 (i.e., FG 48-1). |
| [5] | Qualcomm | As defined in TS 38.101-1, there are two different sizes of transmission bandwidth for 3MHz channel bandwidth, corresponding to 12PRBs at GSCN41637 and 15PRBs at other GSCNs in Table 5.4.3.3-2. According to the following RAN1 agreement, only kssb =0 will be used for 12PRB CORSET0 in FG 51-2. However, if 15PRB transmission bandwidth is configured, kssb for SSB cannot be 0, which means 12PRB CORESET0 in FG 51-2 cannot be configured for 15PRB transmission bandwidth. Agreement For 3MHz channel BW, for kssb and PRB offset for the determining the CORESET#0 position in frequency domain, - kssb follows legacy configuration. - Note: based on existing specifications, UE does not expect other values than kssb = 0 for 12PRBs CORESET#0, [kssb = 8 [or 20] for 15PRBs CORESET#0 with offset 0 PRBs, kssb = 4 [or 16] for 15RPBs CORESET#0 with offset 2 PRBs] - PRB offset - For 12 PRBs CORESET#0 transmission BW, PRB offset = 0 - For 15 PRBs CORESET#0 transmission BW, at least PRB offset = 0, [2] 2 are supported - FFS if offset = 1, 3 are supported. Note: the above assumes that PRB offset to be the frequency gap between the first PRB of the actually transmitted 12/15 PRBs CORESET#0 and the first CRB overlapping with the first PRB of the actually transmitted 12 PRBs SS/PBCH block, how to capture it in the specification is left to editors. Note: the range of the PRB offsets is only applicable to the introduced new sync. raster points for 3MHz channel BW and 100kHz channel raster. Therefore, 12PRB CORESET0 in FG51-2 can only be configured for the SSB located in GSCN41637 with 12PRB transmission bandwith, but not be able to configured for that of other GSCNs with 15PRB transmission bandwidth as defined in TS38.101-1. Table 5.4.3.1-2: GSCN parameters for the global frequency for 3 MHz channel bandwidth Range of frequencies (MHz); SS block frequency position SSREF; GSCN; Range of GSCN. 0 – 1000; N * 600 kHz + M * 50 kHz + 300 kHz, N = 1:1665, M ϵ {1,3,5} (Note 1); 26638+3N + (M-3)/2; 26640 – 31634. NOTE 1: Only applicable for 15 PRB DCH transmission within 3 MHz channel bandwidth with punctured PBCH defined in TS 38.211 [6] clause 7.4.3.1.. Table 5.4.3.1-3: Additional GSCN parameters for band n100 SS Block frequency position SSREF (MHz); GSCN; Note. 920.73; 41637; Only applicable for 12 PRB transmission bandwidth configuration within 3 MHz channel with punctured PBCH defined in TS 38.211 [6] clause 7.4.3.1.. 921.45; 41638; Only applicable for 20 PRB transmission bandwidth configuration within 5 MHz channel with unpunctured PBCH defined in TS 38.211 [6] clause 7.4.3.1.. Therefore, we suggest adding a note that FG51-2 is applicable when SSB is located in band n100 at GSCN41637 only and the reporting type is ‘per UE’ instead of ‘per Band’. Proposal 2: Introduce the following modification of the FG51-2: Features; Index; Feature group; Components; Prerequisite feature groups; Need for the gNB to know if the feature is supported; Applicable to the capability signalling exchange between UEs (Sidelink WI only)”.; Consequence if the feature is not supported by the UE; Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC); Need of FDD/TDD differentiation; Need of FR1/FR2 differentiation; Capability interpretation for mixture of FDD/TDD and/or FR1/FR2; Note; Mandatory/Optional. 51. NR_FR1_lessthan_5MHz_BW; 51-1; Support for 3 MHz channel bandwidth; 1) Reception of 12 PRB PBCH based on RB-level puncturing 2) Short RACH preamble formats with 15kHz SCS, and long PRACH formats with 1.25kHz SCS 3) Reception of 15 PRB CORESET0; Yes; N/A; UE is not able to support 3 MHz channel bandwidth with 15 PRB CORESET0; Per Band; FDD only; FR1 only; N/A; This FG is supported for 15 kHz SCS only This FG is applicable only when an associated SS/PBCH block is located according to Table 5.4.3.3-2 in TS 38.101-1 in Rel-18 Note: The UE supporting this FG supports configuration of 15 PRB BWP operation; Optional with capability signalling. 51. NR_FR1_lessthan_5MHz_BW; 51-2; Support 12 PRB CORESET0; 1) Reception of 12 PRB CORESET0; 51-1; Yes; N/A; UE is not able to support 3 MHz channel bandwidth with 12 PRB CORESET0; Per UE; FDD only; FR1 only; N/A; This FG is supported for 15 kHz SCS only This FG is only applicable when an associated SS/PBCH block is located in band n100 at GSCN 41637 of Table 5.4.3.1-3 in TS 38.101-1 in Rel-18. Note: The UE supporting this FG supports configuration of 12 PRB BWP operation; Optional with capability signalling. 51. NR_FR1_lessthan_5MHz_BW; 51-3; Support 5 MHz channel bandwidth with 20 PRB CORESET0; 1) Short RACH preamble formats with 15kHz SCS, and long PRACH formats with 1.25kHz SCS 2) Reception of 20 PRB CORESET0; Yes; N/A; UE is not able to support 5 MHz channel bandwidth with 20 PRB CORESET0; Per UE; FDD only; FR1 only; N/A; This FG is supported for 15 kHz SCS only This FG is only applicable when an associated SS/PBCH block is located in band n100 at GSCN 41638 of Table 5.4.3.1-3 in TS 38.101-1 in Rel-18. Note: The UE supporting this FG supports configuration of 20 PRB BWP operation; Optional with capability signalling. |

Discussion

Proposal 2-1:

- Add a note “This FG is only applicable when an associated SS/PBCH block is located in band n100 at GSCN 41637 of Table 5.4.3.1-3 in TS 38.101-1 in Rel-18” in FG 51-2

- Reporting type of FG 51-2 is revised to per UE

| Company | Comment |
| --- | --- |
| Moderator | Summary of companies’ views: - Add following note and revise the reporting type from per band to per UE in FG 51-2: Nokia/NSB, E///, QC - This FG is only applicable when an associated SS/PBCH block is located in band n100 at GSCN 41637 of Table 5.4.3.1-3 in TS 38.101-1 in Rel-18. |
| Ericsson | Ok with Proposal 2-1. It is worth noting that beyond the inefficient usage of resources and downgraded performance, at least two major technical issues have been identified if 12-PRB CORESET#0 were used on a 15-PRB transmission bandwidth (Major issues: 1) Unsuitable kSSB for the valid locations of SSB on 15-PRB transmission bandwidth, 2) Usage of 12-PRB CORESET#0 only possible after initial access, if cell’s CORESET#0 were switched then there will be an issue for UEs entering the cell supporting only 15-PRB CORESET#0). The technical issues arise because the design and corresponding agreements were made not accounting for a hybrid usage of a 12-PRB CORESET#0 and a 15-PRB transmission bandwidth, therefore major issues arise. |
| Nokia/NSB | Support As we discuss in Sec. 2.4 of our contribution in R1-2400085, using 12 PRB CORESET#0 for regular 3MHz channel would also only allow to use 12PRB UL/DL BWPs out of 15 available PRBs for PDSCH/PUSCH – which would reduce the performance as only 80% of the PRBs could be used. |
| Moderator | Proposal 2-1: - Add a note “This FG is only applicable when an associated SS/PBCH block is located in band n100 at GSCN 41637 of Table 5.4.3.1-3 in TS 38.101-1 in Rel-18” in FG 51-2 - Reporting type of FG 51-2 is revised to per UE |
| Huawei, HiSilicon | Sorry that it is not agreeable. It has been discussed for multiple meetings. It reverts the previous agreement of “Per-band” granularity without clear justification on any incapability of UE implementation. |
| Moderator | Alternative proposal 2-1: - Add a new FG 51-2a which is copied from 51-2 with following changes - Add a note “This FG is only applicable when an associated SS/PBCH block is located in band n100 at GSCN 41637 of Table 5.4.3.1-3 in TS 38.101-1 in Rel-18” - Reporting type of FG 51-2a is per UE Continue discussion towards Thursday morning session |
| Nokia2 | Our preference would still be the original proposal 2-1, but if that is not agreeable (as it seems based on the feedback by Huawei on Tuesday) – we strongly support alternative proposal 2-1, to make sure the UE that supports the sync raster point with 12 PRB TX bandwidth would not be required to support the 12 PRB BWP size for normal 3 MHz carriers in band n100. |
| Ericsson v008 | We are not willing to create a new FG given that the former one will artificially remain there with lots of technical issues. For example, a k_ssb different than 0 is needed to operate a 12-PRB CORESET#0 on a 15-PRB Transmission Bandwidth, however a 12-PRB CORESET#0 is tied to a k_ssb = 0. Moreover, there are different understandings among companies on how the transmissions will be after having switched to a 12-PRB CORESET#0, some companies think that after having switched to 12-PRB CORESET#0 all subsequent transmissions will be confined to up 12-PRBs, whereas some other companies think there is no such a restriction and subsequent transmissions can span up to 15-PRBs. All those issues happen because the 12-PRB CORESET#0 was designed to operate when the transmission bandwidth is 12-PRBs, and not designed for when the transmission bandwidth is 15-PRBs, that is why the note is needed as per the original Proposal 2-1. |

Question 2-2:

- Companies are invited to provide view on whether to add following notes in FG 51-1/2/3

  - This FG is only applicable to single-carrier operation.

  - This FG is not applicable to UEs indicating supportOfRedCap-r17 (i.e., FG 28-1) or supportOfERedCap-r18 (i.e., FG 48-1).

| Company | Comment |
| --- | --- |
| Moderator | This is proposed by MediaTek. -- In the RAN#99 reply LS, the following was agreed: RAN Plenary has discussed question 2 on the feature list to be considered, and concluded that the less-than-5MHz WI in Rel-18 should consider single-carrier operation, excluding RedCap. In addition, UE speeds up to 500km/h should be targeted for Band n100 without impact to RAN1. Both the updated WID and the WI summary have reflected the RAN agreement. However, it has not been implemented into specifications. |
| Ericsson | From previous discussions when this was brought up, we recall that it was mentioned that it would be sufficient referring to the conclusion from RAN#99 without further impacting the UE Feature list. |
| Nokia/NSB | We share the views from Ericsson, i.e. this is clear already from RAN LS already |
| Moderator | It seems proposed notes are not necessary. |
| Moderator | Continue discussion towards Thursday morning session |
| MediaTek | We think agreements including RAN’s should be implemented to specifications. Otherwise, are we expecting our implementation teams to read not only specifications but also LSs, reply LSs, Chairman’s notes, etc.? If I follow the same logic, can I say we don’t need write specifications? People can just read Chairman’s notes, right? |

- Conclusions

To be updated

References

[1]	R1-2312705	Corrected RAN1 UE features list for Rel-18 NR after RAN1#115	Moderators (AT&T, NTT DOCOMO, INC.)

[2]	R1-2400085	Remaining issues of UE Features for Other Topics A (eRedCap, XR, CovEnh, sub-5MHz, eDSS, TEI)	Nokia, Nokia Shanghai Bell

[3]	R1-2400676	Rel-18 UE features topics set A	Ericsson

[4]	R1-2401305	UE features for other Rel-18 work items (Topics A)	MediaTek Inc.

[5]	R1-2401428	UE features for other Rel-18 work items (Topics A)	Qualcomm Incorporated