3GPP TSG RAN WG1 #116			R1-2401649

Athens, Greece, February 26th – March 1st, 2024

Agenda item:	9.2.2

Source: 	Moderator (Samsung)

Title: 	Moderator Summary for Tuesday offline session on Rel-19 CSI enhancements

Document for:	Discussion and Decision

## Introduction

This contribution provides the summary of the Tuesday offline session for Rel-19 MIMO Phase 5 CSI enhancements.

## Summary

Agreement (Tue online session)

For the Rel-19 aperiodic standalone CJT calibration reporting, the following use cases are assumed:

- For per-TRP delay offset reporting:

  - Use case 1.1: TRP selection

  - Use case 1.2: delay offset compensation for at least one TRP to ensure the CJT-composite delay spread doesn’t exceed a pre-defined dynamic range/threshold

- For per-TRP frequency offset (FO) reporting:

  - Use case 2.1: TRP selection

  - Use case 2.2: per-TRP FO compensation at NW side

Agreement (Tue online session)

For the Rel-19 aperiodic standalone CJT calibration reporting, support the following:

- The UE is configured with NTRP NZP CSI-RS resources/resource sets via higher-layer (RRC) signalling where NTRP∈{1, 2, 3, 4}

  - FFS (by RAN1#116bis): Whether further restriction(s) on applicable NZP CSI-RS resources/resource sets need to be introduced (e.g. number of ports, only TRS with multiple resource sets, TD/FD locations, QCL assumptions)

- For the purpose of CJT calibration reporting, decide, by RAN1#116bis, from the following

  - Opt1:  The UE reports for all the configured NTRP NZP CSI-RS resources/resource sets

  - Opt2: The UE reports for N out of NTRP NZP CSI-RS resources/resource sets where the selection of N resources/resource sets is dynamically signalled by the NW to the UE

  - Opt3: The UE reports for N out of NTRP NZP CSI-RS resources/resource sets where the selection of N resources/resource sets is performed by the UE and included in the CSI report

- Interference measurement is not supported, hence neither CSI-IM nor NZP CSI-RS resource for interference measurement can be configured (analogous to Rel-18 TDCP)

- FFS: One-part or two-part UCI on PUSCH (analogous to Rel-18 TDCP)

- The priority of the CSI report(s) is the same as CSI report(s) not carrying L1-RSRP or L1-SINR (analogous to Rel-18 TDCP)

Proposal 3.C: For the Rel-19 aperiodic standalone CJT calibration reporting, given the NTRP configured NZP CSI-RS resources/resource sets and the selected N resources/resource sets, support reporting, in one CSI reporting instance, {(Dn,offset, dn), n=0, 1, …, N – 1} where

- Dn,offset is a B-bit indicator representing the delay offset associated with the n-th CSI-RS resource/resource set

  - For the reference CSI-RS resource/resource set nref, the value of Dnref,offset is assumed 0 and not reported

    - FFS (by RAN1#116bis): Whether nref is fixed, NW-configured, or is included in the report (selected by the UE)

  - The value of Dn,offset indicates the interval [δi,δi+1) which the delay offset falls into

    - Down-select, by RAN1#116bis, from the following

      - Alt1: δ0,δ1,…,δM-2 is uniformly spaced between 0 and AD, i.e. δi=i×ADM-2,   i=0,1,…, M-2, with M=2B

      - Alt2: δ0,δ1,…,δM-2 is uniformly spaced between -AD and AD, i.e. δi=-AD+i×2ADM-2,   i=0,1,…, M-2, with M=2B

    - Each interval [δi,δi+1)  corresponds to a codepoint, and (-∞,δ0) and/or [δM-2,∞) represent ‘out-of-range’

    - FFS (by RAN1#116bis): supported quantization alphabet(s) (including AD, M)

- dn is a 1-bit indicator associated with the n-th CSI-RS resource/resource set, indicating whether the measured delay offset, plus delay spread, is inside or outside a pre-defined range/interval

  - FFS (RAN1#116bis): The pre-defined range(s), e.g. CP length or its multiple

- FFS: Detailed UCI design on codepoint encoding details

- FFS: The need for a new QCL assumption

FL assessment: Merge scheme 1 and 2 into a single scheme with clearer wording, centered around scheme 1 (delay offset), and scheme 2 (spread) as a 1-bit add-on to indicate whether offset + spread is out of range or not.

Support/fine: Ericsson, Nokia/NSB, vivo, Samsung, Qualcomm, ZTE, CMCC, Intel, NEC, Xiaomi, OPPO, Fujitsu, NTT DOCOMO, AT&T, Spreadtrum, KDDI, Sharp, Huawei/HiSi, IDC, Apple, Google, Panasonic, Lenovo/MotM, [Sony], [Sharp], [CATT],

Not support:

Proposal 3.A.2: For the Rel-19 aperiodic standalone CJT calibration reporting, the following use cases are assumed:

- For per-TRP DL/UL Rx-Tx phase/timing misalignment reporting:

  - Use case 3.1: TRP selection

  - Use case 3.2: per-TRP DL/UL Rx-Tx phase compensation at NW side for reciprocity (e.g. using both CSI-RS and SRS for measurement)

  - Use case 3.3: per-TRP DL/UL Rx-Tx timing compensation at NW side for reciprocity (e.g. using both CSI-RS and SRS for measurement)

- Use case 4: Per-TRP phase compensation at NW side (e.g. assisting precoding change until a new PMI updated)

Not yet discussed:

Proposal 1.B: For the Rel-19 Type-I codebook refinement for up to 128 CSI-RS ports, at least for RI=1-4, study and decide, by RAN1#116bis, from the following:

- Scheme1 (baseline): Adding new (N1, N2) values for the Rel-15 Type-I single-panel codebook where 2N1N2 (>32) is the total number of CSI-RS ports across aggregated NZP CSI-RS resources

  - FFS: Whether to further down-select between mode-1 (L=1) and mode-2 (L=4)

  - FFS: For rank-3/4, follow legacy mechanisms for <16 ports, or for >=16 ports

- Scheme2: Adding new (N1, N2) values where 2N1N2 (>32) is the total number of CSI-RS ports across aggregated NZP CSI-RS resources, and

  - W1 structure:

    - For each layer, reuse legacy Rel-16 eType-II with L=1 to determine the DFT-based SD basis candidates

      - FFS: Whether the indication of SD basis indices follows Rel-16 eType II or Rel-15 Type I

    - For RI>1, L=1 SD basis vector is independently selected for different layers

  - W2 structure: Layer-specific inter-polarization M-PSK co-phasing where M is further down-selected from {2, 4, 8, 16}

  - FFS: Common SD vector selection for a pair of layers (reduced total number of bits for SD basis vector selection); layer multiplexing via orthogonal polarization co-phasing for the layer pairs with common SD vector (reduced number of bits for co-phasing indication for the layer pairs with common SD vector).

  - FFS: Additional support for L>1

- Scheme2B: Adding new (N1, N2) values where 2N1N2 (>32) is the total number of CSI-RS ports across aggregated NZP CSI-RS resources, and

  - W1 structure:

    - For each layer, determine L=1 DFT-based SD basis candidate

      - FFS: Whether the indication of SD basis indices follows Rel-16 eType II or Rel-15 Type I

    - For RI>1, L=1 SD basis vector is independently selected across layers

    - FFS: Common SD vector selection for a pair of layers (reduced total number of bits for SD basis vector selection)

  - W2 structure:

    - Option 1: Layer-specific inter-polarization amplitude and phase scaling (single scaling coefficient per polarization)

      - FFS: WB/SB amplitude and phase reporting.

    - Option 2: Layer-specific intra-polarization (two scaling coefficients per polarization) amplitude and phase scaling.

      - FFS: WB/SB amplitude and phase reporting.

    - FFS: Rel. 15 3-bit WB amplitude and M-PSK co-phasing and M is further down-selected from {2, 4, 8, 16}.

- Scheme3: Adding new (N1, N2) values where 2N1N2 (>32) is the total number of CSI-RS ports across aggregated NZP CSI-RS resources, and

  - W1 structure:

    - Reuse legacy Rel-16 eType-II DFT-based SD basis with L>1

    - For RI>1, L>1 SD basis vectors are commonly selected across layers

  - W2 structure:

    - Option 1: Layer-specific sub-band SD basis selection (1 out of L) and inter-polarization M-PSK co-phasing where M is further down-selected from {2, 4, 8, 16}

    - Option 2: Layer-specific wideband SD basis linear combination and inter-polarization scaling coefficient (e.g., amplitude scaling + M-PSK co-phasing) where M is further down-selected from {2, 4, 8, 16}

- Scheme4: Using legacy Rel-15 Type-I codebook including legacy (N1, N2) values per NZP CSI-RS resource (or port group) where the PMI (associated with W1 and W2) is calculated according to

  - W1 structure: Reuse legacy Rel-15 Type-I SD basis with L=1 or L=4 for either each or some of the NZP CSI-RS resources (or port groups)

  - W2 structure: Layer common inter-NZP CSI-RS resource (or port group) co-phasing along with reusing legacy Rel-15 Type-I inter-polarization co-phasing per NZP CSI-RS resource (or port group)

    - inter-CSI-RS resource (or port group) co-phasing is used to combine the different PMIs to come up with a single precoder with >32 ports

- Scheme5: Adding new (N1, N2) values where 2N1N2 (>32) is the total number of CSI-RS ports across aggregated NZP CSI-RS resources, and extending the set of orthogonal beams for the selection of the second beam based on the Rel-15 Type-I single-panel codebook

  - (i1,1, i1,2) is used to refer to the 1st beam as in legacy Rel-15 Type-I

  - The 2nd beam is selected from the extended set of orthogonal beams of size: N1-1N2O2+N2-1N1O1-N1-1N2-1

  - FFS: whether to apply any restrictions to the extended orthogonal set of beams

- Scheme6: Adding new (N1, N2) values where 2N1N2 (>32) is the total number of CSI-RS ports across aggregated NZP CSI-RS resources, and

  - Beam(s) is(are) selected for each antenna group or NZP CSI-RS resource.

  - Inter-group (or CSI-RS resource) co-phasing along with inter-polarization co-phasing per group (or CSI-RS resource) are used to combine different beam(s), FFS using scalar quantization or vector quantization for the co-phasings

FFS (by RAN1#116bis): Down-select (O1, O2) value between (2,2) and (4,4), whether (O1, O2) and/or (q1, q2) is layer-common or layer-specific

FFS (by RAN1#116bis): Whether extension of Rel-15 Type-I MP codebook for Rel-19 Type-I is also supported

FFS (by RAN1#116bis): Whether to introduce larger L values (e.g. 6, 8, 10)

FFS: Whether to refine CBSR design to reduce RRC overhead

Proposal 3.D: For the Rel-19 aperiodic standalone CJT calibration reporting, given the NTRP configured NZP CSI-RS resources/resource sets and the selected N resources/resource sets, support reporting, in one CSI reporting instance, {FOn , n=0, 1, …, N – 1, n≠nref}, where FOn denotes the measured frequency offset associated with the n-th CSI-RS resource/resource set relative to the reference CSI-RS resource/resource set nref

- For the reference CSI-RS resource/resource set nref, the value of FOnref is assumed 0 and not reported

  - FFS (by RAN1#116bis): Whether nref=0 (fixed), NW configured, or is included in the report (selected by the UE)

  - FFS (by RAN1#116bis): whether the UE assumes that the measured and reported per-TRP residual frequency offsets can include Doppler shift (if existent) associated with the reference CSI-RS resource/resource set nref

  - FFS: Measurement resource/resource set for FO reporting

- FOn is uniformly quantized between –AFO and AFO

  - If N<NTRP, the rest (NTRP–N) resources/resource sets are indicated with a state “out of range”

  - FFS (by RAN1#116bis): supported quantization alphabet(s) (including AFO and resolution) for FOn

- FFS: Detailed UCI design