3GPP TSG RAN WG1 #116		                                                      R1-2401652

Athens, Greece, February 26th – March 1st, 2024

Source:	Moderator (CATT)

Title:	Summary #1 of 	SBFD TX/RX/measurement procedures

Agenda Item:	9.3.1

Document for:	Discussion and Decision

- Introduction

NR duplex evolution was studied in Rel-18 with outcome captured in TR 38.858 [1].

Rel-19 WI on evolution of NR duplex operation: Sub-band full duplex (SBFD) was approved in RP-234035 [2] with the following objectives.

This document summarizes the inputs and the discussions in RAN1#116 on Rel-19 SBFD TX/RX/measurement procedures corresponding to objectives highlighted in cyan below.

- For subband non-overlapping full duplex (SBFD) operation at gNB side within a TDD carrier: - Specify semi-static indication of time location of SBFD subbands to UEs in RRC_CONNECTED mode [RAN1, RAN2] - Indication of time location of SBFD subbands in SIB is not precluded - Specify semi-static indication of frequency domain location of SBFD subbands to UEs in RRC_CONNECTED mode [RAN1, RAN2] - Indication of frequency domain location of SBFD subbands in SIB is not precluded - Specify SBFD operation to support random access in SBFD symbols by UEs in RRC CONNECTED mode [RAN1, RAN2] - Study and specify, if justified, SBFD operation to UE in RRC_IDLE/INACTIVE mode for random access [RAN1, RAN2] - RAN#104 to check whether to proceed normative work - Specify UE transmission, reception and measurement behavior and procedures in SBFD symbols and/or non-SBFD symbols for SBFD aware UE [RAN1, RAN2] - Transmission and reception behaviours on SBFD subbands configured in DL and/or flexible symbol indicated by TDD-UL-DL-ConfigCommon - UL transmissions within UL subband only - DL receptions within DL subband(s) only, except for CLI measurement by the UE outside of the DL subbands Note: When flexible symbols are used, it is not expected that any legacy Uplink symbol is converted to Downlink/SBFD symbols - Enhancement on resource allocation in frequency domain in SBFD symbols, including - resource allocation in frequency domain for PDSCH/CSI-RS across two DL subbands in SBFD symbols - handling of unaligned boundaries between SBFD subband(s) and RBG, CSI reporting subband, CSI-RS resource, PRG - Enhancements on physical channels/signals and procedure across SBFD symbols and non-SBFD symbols in different slots, where each transmission/reception within a slot has either all SBFD or all non-SBFD symbols, including - resource allocation in frequency domain for transmission or reception in SBFD symbols and non-SBFD symbols with different available frequency resource in different slots - CSI report of which associated CSI-RS instances occur in both SBFD symbols and non-SBFD symbols in different slots - Configurations for SRS, PUCCH and PUSCH on SBFD symbols and non-SBFD symbols, e.g., resources, frequency hopping parameters, UL power control parameters and/or beam/spatial relation - Collision handling between DL reception in DL subband(s) and UL transmission in UL subband in a SBFD symbol - Followings are assumed based on TR 38.858 - SBFD at the gNB side - Half duplex operation at the UE side - FR1 and FR2-1 - SBFD operation Option 4, i.e., both time and frequency locations of subbands for SBFD operation are known to SBFD aware UEs - Coexistence between non-SBFD aware UEs (including legacy UEs) and SBFD aware UEs in the cell operating SBFD at gNB side - SBFD scheme within a single configured DL and UL BWP pair with aligned center frequencies - One UL subband for SBFD operation in an SBFD symbol (excluding legacy UL symbol/slot) within a TDD carrier - Mechanisms for SBFD operation shall also consider the adjacent channel coexistence between two operators - Specify enhancements for CLI handling [RAN1, RAN2, RAN3]: - Support gNB-to-gNB co-channel CLI handling scheme(s) (the detailed schemes are to be down-selected from those in TR38.858 by RAN1#117) - Support UE-to-UE co-channel CLI handling scheme(s) (the detailed schemes are to be down-selected from those in TR38.858 by RAN1#117) - Note: Without dedicated optimization for dynamic/flexible TDD. - Specify BS RF requirements for SBFD operation at gNB [RAN4] - Specify applicable RRM core requirements for co-channel CLI handling mechanisms [RAN4] - Specify other RRM core requirements for SBFD operation, if identified [RAN4]

- Proposals for online sessions

  - Feb 27th (Tue)

Proposal 1-1a

Proposed Agreement:

At least cell-specific configuration on time location of SBFD subbands is supported within a TDD carrier.

          - FFS cell-specific configuration on frequency location of SBFD subbands

          - FFS UE-specific configuration on time and/or frequency locations of SBFD subbands

Proposal 1-3a

Proposed Agreement:

SBFD subband time locations are configured within a period.

          - At least when only one TDD-UL-DL pattern is configured, the period is down-selected from one of the following options.

            - Option 1: The period is the same as TDD-UL-DL pattern period configured by dl-UL-TransmissionPeriodicity in TDD-UL-DL-ConfigCommon.

            - Option 2: The period is integer multiple of TDD-UL-DL pattern period configured by dl-UL-TransmissionPeriodicity in TDD-UL-DL-ConfigCommon.

  - FFS details

  - FFS when two TDD-UL-DL patterns are configured

Proposal 1-4

Proposed Agreement:

A slot can consist of SBFD symbols and non-SBFD symbols.

Symbol-level granularity is supported for semi-static indication of SBFD subband time location.

- referenceSubcarrierSpacing in TDD-UL-DL-ConfigCommon is used as reference SCS.

Proposal 1-6

Proposed Agreement:

The maximum number of UL subbands for SBFD operation in an SBFD symbol within a TDD carrier is one.

The UL subband can be located at one side of the carrier or can be located at the middle part of the carrier.

For semi-static indication of SBFD subband frequency location, down-select from the following options.

- Option 1: Frequency locations of UL subband and DL subband(s) are explicitly configured. Guardband(s) if any are implicitly derived as the RBs which are not within UL subband or DL subband(s).

- Option 2: Frequency location of UL subband and the number of RBs for guardband(s), if any, are explicitly configured. DL subband(s) are implicitly derived as RBs which are not within UL subband or guardband(s).

Proposal 1-5a

Proposed Agreement:

The subband frequency-domain resources are same across different SBFD symbols.

Frequency location of UL/DL subband, if explicitly indicated, is indicated with reference to CRB grid.

- RB-level granularity is supported for semi-static indication of SBFD subband frequency location.

  - Subject to RAN4 guidance on the size of subband/guardband, if any

  - FFS starting RB and reference SCS

Proposal 2-6

Proposed Agreement:

For SBFD-aware UEs, collisions between DL reception in DL subband(s) and UL transmission in UL subband in a SBFD symbol may be addressed or alleviated with proper scheduling. The following cases of potential collisions can be further studied to see if any change to the current specs is necessary:

          - Case 1: Dynamically scheduled DL reception vs. semi-statically configured UL transmission

            - e.g., dynamic PDSCH or CSI-RS collides with configured SRS, PUCCH, or CG PUSCH

          - Case 2: Semi-statically configured DL reception vs. dynamically scheduled UL transmission

            - e.g., PDCCH or SPS PDSCH collides with dynamic PUSCH or PUCCH

          - Case 3: Semi-statically configured DL reception vs. semi-statically configured UL transmission

          - Case 4: Dynamically scheduled DL reception vs. dynamic scheduled UL transmission

          - Case 5: Configured SSB vs. dynamically scheduled or configured UL transmission

            - e.g., PUSCH, PUCCH, PRACH, SRS

          - Case 6: Dynamic or semi-static DL vs. valid RO

          - Case 7: Collision due to direction switching

Proposal 2-1a

Proposed Agreement:

For SBFD-aware UE transmission and reception in the SBFD symbols configured in DL and/or flexible in TDD-UL-DL-ConfigCommon,

- UL transmissions within UL subband within active UL BWP are allowed

  - Whether or not UL transmission in SSB symbols is allowed is separately discussed

- DL receptions within DL subband(s) within active DL BWP are allowed

- UL transmissions outside UL subband are not allowed

- DL receptions outside DL subband(s) are not allowed

CLI measurement behaviours for SBFD-aware UE are discussed in agenda item 9.3.3.

Proposal 2-4

Proposed Agreement:

Support enhancement on frequency domain RA type 1 for PDSCH across two DL subbands in SBFD symbols for both interleaved and non-interleaved VRB-to-PRB mapping.

- FFS details

Proposal 2-9

Proposed Agreement:

For an RBG that overlaps the subband boundary for SBFD-aware UEs,

- The part of the DL RBG inside the DL subband can be used for DL reception. The part of the DL RBG outside the DL subband cannot be used for DL reception.

- The part of the UL RBG inside the UL subband can be used for UL transmission. The part of the UL RBG outside the UL subband cannot be used for UL transmission.

- FFS whether partial RBG as existing partial RBG at BWP edges is defined for this case

- FFS whether TBS is calculated with or without the part of the RBG that cannot be used

Proposal 2-11

Proposed Agreement:

For a CSI-RS resource which overlaps with SBFD subband boundaries, only CSI-RS resources within DL subband(s) are valid for SBFD-aware UEs.

For a CSI reporting subband which overlaps with SBFD subband boundaries, CSI report is derived based on CSI-RS resources excluding CSI-RS resources outside DL subband(s) for SBFD-aware UEs.

- Summary of input contributions

The inputs from companies’ contributions are summarized below according to moderator’s understanding. Moderator would like to apologize in advance if your views are not correctly captured or are missed, in which case please feel free to correct/update the summary with revision marks.

  - SBFD subband indication

For semi-static indication of time and frequency locations of SBFD subbands, the following agreements were made in Rel-18 SI.

Agreement For semi-static configuration of subband time locations for SBFD operation, it is agreed that explicit configuration of SBFD subband time locations within a period is the baseline. Agreement For semi-static configuration of subband location, consider same subband frequency resources across different SBFD symbols as baseline. Agreement For semi-static configuration of subband frequency locations for SBFD operation, at least explicit indication of frequency location of UL subband is required. - FFS: Whether frequency location of other subbands types is explicitly indicated or implicitly determined. Agreement At least for semi-static SBFD, the following two options are viable solutions for frequency location configuration of DL subband(s) and guardband(s) if any. - Option 1: Frequency locations of DL subband(s) are explicitly configured. Guardband(s) if any are implicitly derived as the RBs which are not within UL subband or DL subband(s). - Option 2: The number of RBs for guardband(s), if any, is explicitly configured. DL subband(s) are implicitly derived as RBs which are not within UL subband or guardband(s). Agreement For the purpose of RAN1 study, the understanding is that for semi-static configuration of subband frequency locations for SBFD operation, frequency location of UL/DL subband is with reference to CRB grid.

    - General

Several companies discussed whether SBFD subband configuration is cell-specifc, UE-specific or BWP-specific. Views are summarized below.

Cell-specific vs. UE-specific

- Cell-specific

  - Support: Spreadtrum, Huawei, Transsion, vivo (baseline), ZTE, CMCC, CATT, xiaomi, OPPO, Fujitsu, Google, Nokia, Samsung, QC, CEWiT, MTK, LGE

- UE-specific

  - Support: Sony (time domain)

- BWP-specific

  - Support:

Majority companies think SBFD subband configuration is cell-specific. With cell-specific SBFD subband configurations, UE determines UL/DL subband(s) within active BWPs, if any.

Figure 1: Examples of UL/DL subband within active DL/UL BWP [37]

Signalling design

Semi-static indication of SBFD subbands time and frequency locations can be carried in SIB and/or UE dedicated signalling. Companies’s views are summarized as follows. Note that in current design cell-specific parameters may also be indicated to UEs via UE dedicated signalling, e.g. when configuring a UE with a Cells or with an SCG or for SpCells upon reconfiguration with sync.

- SIB

  - Support: Spreadtrum, Huawei, Tejas, vivo, TCL, CMCC, CATT, CT, NEC, Samsung, Nokia (baseline), DCM, Sony, MTK

- UE dedicated signalling

  - Support: CT, Sony, Samsung, Nokia, MTK

    - For RRC connected UE for Scell Addtion case [Samsung]

    - For EN-DC, NR-DC or CA [Nokia]

    - In ServingCellConfigCommon [MTK]

  - FFS: vivo, CATT

- Postpone the decision after the RAN#104 decision on supporting random access in RRC_IDLE mode or not

  - Support: Ericsson, DCM

    - Time indication

SBFD subband periodicity

Companies support the SI agreement that SBFD subband time locations are configured within a period. Different options are proposed by companies on how to determine the SBFD subband periodicity as summarized below and illustrated in Figure 3 for one TDD pattern case.

- SBFD subband time locations are configured within a period

  - Support: Spreadtrum, Huawei, Transsion, vivo, TCL, CMCC, CATT, xiaomi, OPPO, CT, Fujitsu, Apple, Panasonic, Nokia, Lenovo, KT, ITRI, Ericsson, Samsung, CEWiT, QC

  - SBFD subband periodicity

    - Option 1: The period is TDD-UL-DL pattern period configured by dl-UL-TransmissionPeriodicity in TDD-UL-DL-ConfigCommon

      - Support: Spreadtrum, Huawei, Transsion, vivo, TCL, ZTE, CMCC, CATT, xiaomi, OPPO, CT, Fujitsu, Apple, Panasonic, Nokia, Lenovo, KT, ITRI, Ericsson, Samsung, CEWiT, QC

      - If two TDD-UL-DL patterns are configured,

        - Option A: SBFD subband time locations are separately configured within each TDD-UL-DL pattern period, as illustrated in Figure 2(a).

          - Support: ZTE, CATT, Lenovo, KT, Samsung, QC

        - Option B: SBFD subband period is the sum of the two TDD-UL-DL pattern periods. One example is shown in Figure 2(b), but it is not clear whether it is the intention from all the proponents.

          - Support: OPPO, Ericsson, Samsung

Figure 2: SBFD configuration for two TDD patterns [18]

        - Option C: A single SBFD pattern is associated with two TDD UL/DL patterns. The proponent also noted that considering that the two TDD UL/DL patterns normally have different configurations, how to link a single SBFD pattern to two TDD UL/DL patterns need to be studied.

          - Support: Lenovo

    - Option 2: The period is integer multiple of slot configuration period configured by dl-UL-TransmissionPeriodicity in TDD-UL-DL-ConfigCommon

      - Support: Samsung

    - Option 3: The period is determined by a separately configured periodicity

      - Support: vivo, ZTE, Samsung (not integer multiple of TDD-UL-DL pattern period to support a traffic with a specific periodicity)

Figure 3: SBFD subband periodicity for one TDD pattern [18]

SBFD symbol locations

Regarding whether SBFD symbols are consecutive or not within a period, companies’ views are summarized as follows.

- Consecutive SBFD symbols within a period

  - Support: New H3C, TCL, ZTE, CATT, xiaomi, CMCC, OPPO, LGE, Lenovo, ITRI, QC

- Non-consecutive SBFD symbols within a period

  - Support: xiaomi, MTK

SBFD symbols can be configured in DL and flexible symbols indicated by TDD-UL-DL-ConfigCommon.

- Support: Spreadtrum, CMCC, Sony, LGE, QC

Time domain granularity

Companies support allowing that a slot can consist of both SBFD and non-SBFD symbols. Accordingly, companies propose that the transition point can be aligned with slot boundary of within a slot, and symbol level granularity is supported for semi-static indication of time location of SBFD subbands.

- A slot can consist of SBFD symbols and non-SBFD symbols

  - Support: Huawei, vivo, TCL, ZTE, CATT, xiaomi, Fujitsu, IDC, Samsung, Nokia, DCM, Ericsson, MTK, Lenovo, QC

    - The transition point between SBFD and non-SBFD symbols can be aligned with slot boundary or within a slot

      - Support: Huawei, vivo, CATT, Ericsson

    - Symbol-level granularity is supported for semi-static indication of time location of SBFD subbands

      - Support: ZTE, Samsung, Nokia, DCM, QC

      - Not support: MTK (slot level configuration)

Some companies discussed the reference SCS for SBFD subband time location indication and propose to reuse the same configuration for TDD pattern configurations.

- Reference SCS for SBFD subband time location indication

  - referenceSubcarrierSpacing in TDD-UL-DL-ConfigCommon

    - Support: Huawei, OPPO, CT, Fujitsu, Samsung

Limit of number of transition/switching points

Some companies propose to limit the number of transition points.

- Maximum number of transition points

  - Maximum of two transition points including one from non-SBFD symbols to SBFD symbols and one from SBFD symbols to non-SBFD symbols within TDD-UL-DL pattner period

    - Support: Huawei, vivo (starting point), CATT, Sony (FFS configuration using dedicated signalling), Ericsson (at least for configuration in cell-specific signalling), QC

  - Two or more transition points between SBFD and non-SBFD symbols within a TDD pattern period

    - Support: Nokia

      - Guard periods may or may not be needed depending on gNB architecture and antenna configuration. The number of transition points should not be limited at least from spec point of view.

  - Maximum two transition point within a slot consisting of both SBFD symbols and non-SBFD symbols

    - Support: TCL

  - Maximum one transition point within a slot consisting of both SBFD symbols and non-SBFD symbols

    - Support: xiaomi

- Maximum number of DL/UL switching points for SBFD-aware UE within SBFD time pattern

  - RAN1 should discuss some limitation

    - Support: QC

Guard periods

Some companies discussed whether to explicitly configure guard periods between SBFD and non-SBFD symbols.

- Guard periods between SBFD and non-SBFD symbols

  - Explicitly configured

    - Support: CMCC, Apple, Google

  - Up to RAN4

    - Support: Samsung (can be added right before SBFD symbol or right after SBFD symbol if agreed)

  - FFS

    - Support: Spreadtrum, CATT, LGE

    - Frequency indication

For the following two bullets mentioned by some companies, moderator assumes that it is common understanding among companies even if some companies did not mention in their contributions.

- The maximum number of UL subbands for SBFD operation in an SBFD symbol within a TDD carrier is one

  - Support: Huawei, ZTE, Sony, LGE, Samsung, QC

- The UL subband can be located at one side of the carrier or can be located at the middle part of the carrier

  - Support: Huawei, Sony, LGE, Samsung, QC

Companies agree with the following SI agreement as summarized below.

- The subband frequency-domain resources are same across different SBFD symbols

  - Support: Huawei, vivo, ZTE, CATT, Nokia, LGE, Ericsson, Samsung

- Frequency location of UL/DL subband is with reference to CRB grid

  - Support: Spreadtrum, Huawei, vivo, ZTE, CMCC, CATT, xiaomi, OPPO, CT, Fujitsu, Sony, Nokia, Ericsson, Samsung, CEWiT, QC

  - Starting location

    - Point A

      - Support: Spreadtrum, CMCC, OPPO, Fujitsu

    - Starting RB of the carrier configured by offsetToCarrier

      - Support: CMCC, Fujitsu

    - Offset to SSB or CORESET0

      - Support: Nokia

Figure 4: SBFD subband frequency location indication [37]

- Reference SCS

  - The subband frequency-domain location indication are configured for each subcarrier spacing in a serving cell since CRB grids are configured for each subcarrier spacing by SCS-SpecificCarrier

    - Support: Spreadtrum, Huawei, CMCC, ZTE, OPPO, Sharp (support different GB configuration for different SCSs), Fujitsu

Figure 5: Frequency location indication of SBFD subbands for two carriers with different SCSs

(gNB can configure SBFD subbands in any one or the carriers or both)

  - a single reference SCS

    - Support: Fujitsu, Samsung (largest SCS)

- Granularity

  - RB-level

    - Support: Sharp, Samsung, CEWiT

  - RB group

    - Support: CEWiT

For indication of UL subband, DL subband(s) and guardband(s), majority companies shared their preference between two options studied during SI. In addition, some other options are also proposed by companies.

- Option 1: Frequency locations of UL subband and DL subband(s) are explicitly configured. Guardband(s) if any are implicitly derived as the RBs which are not within UL subband or DL subband(s).

  - Support: Spreadtrum, Huawei, vivo, ZTE (no need to define guardbands), Nokia, Ericsson, LGE, ITRI, WILUS, QC

- Option 2: Frequency location of UL subband and the number of RBs for guardband(s), if any, are explicitly configured. DL subband(s) are implicitly derived as RBs which are not within UL subband or guardband(s).

  - Support: vivo, CMCC, CATT, xiaomi, OPPO, CT, IDC, Google, Fujitsu, Lenovo, ETRI, CEWiT

- Option 3: Numbers of RBs in lower and upper DL subbands and the number of RBs for guardband(s), if any, are explicitly configured. UL subband is implicitly derived as RBs which are not within DL subband(s) or guardband(s).

  - Support: Samsung

- Option 4: Both the number of RBs for guardband(s) and frequency locations of UL subband and DL subband(s) are explicitly configured.

  - Support: Samsung

Apple proposed that UE indicates as part of UE capability signaling, the minimum required guard band between UL and DL sub-bands, in the unit of frequency, or number of PRBs for each tone spacing.

Xiaomi proposed to use a default BWP to determine the UL subband frequency location if gNB does not provide any configuration for UL subband frequency location.

Apple proposed that for unpaired spectrum, UE may be configured with UL and DL BWPs (cell-specific or UE specific) that have different center frequencies as an indication of UL and DL sub-bands within SBFD symbols/slots since some UE implementations may prefer to be scheduled in only one of the DL sub-bands in SBFD slots/symbols.

Figure 6: Concept of SBFD sub-band from UE and gNB perspectives [24]

  - TX/RX/measurement procedures

    - TX/RX/measurement behaviors

The WID includes the following objective.

    - Transmission and reception behaviours on SBFD subbands configured in DL and/or flexible symbol indicated by TDD-UL-DL-ConfigCommon

      - UL transmissions within UL subband only

      - DL receptions within DL subband(s) only, except for CLI measurement by the UE outside of the DL subbands

Note: When flexible symbols are used, it is not expected that any legacy Uplink symbol is converted to Downlink/SBFD symbols

CLI measurement within guardband(s)

Regarding whether UE can perform CLI measurement within guardband(s) within DL BWP, companies’ views are as follows.

            - Support:

              - Support: IDC, Nokia, QC, MTK, LGE

            - Not support:

              - Support: ZTE

                - Unclear intention

Link direction from UE perspective

For link direction in SBFD symbols, i.e. whether to transmit in UL subband or receive in DL subband(s), there are two options based on companies’ inputs.

- Option 1: based on configured and scheduled transmissions/receptions

  - Support:QC

- Option 2: explicit indication

  - Support: CMCC, Fujitsu (RRC signalling), KT

    - legacy signalling such as TDD-UL-DL-ConfigDedicated and SFI

      - support: CMCC (for flexible symbol)

    - New signalling

      - support: CMCC (for DL symbol)

Companies discussed interation with existing signalling for slot configuration based on the agreement that dynamic SBFD is excluded from Rel-19.

Interaction with TDD-UL-DL-ConfigDedicated and dyamic SFI

In current specification, a symbol configured as flexible in TDD-UL-DL-ConfigCommon can be further overridden as DL/UL by TDD-UL-DL-ConfigDedicated.

For a flexible symbol which is not configured with SBFD subbands,

            - TDD-UL-DL-ConfigDedicated can override the flexible symbol into an UL symbol or a DL symbol

              - Support: Huawei

For a flexible symbol which is configured with SBFD subbands,

            - TDD-UL-DL-ConfigDedicated

  - Option 1: the symbol cannot be converted to an UL symbol

    - Support: Spreadtrum, ZTE

    - Not support: OPPO (the last few consecutive semi-static SBFD symbols can be converted into UL symbols)

  - Option 2: the symbol cannot be converted to a non-SBFD symbol

    - Support: Huawei, MTK, Google (UE is not expect to receive plural indications)

    - Not support: CEWiT

  - Option 3: TDD-UL-DL-ConfigDedicated indicates link direction for the UE

    - Support: CMCC

  - Option 4: Not support to configure SBFD subbands in a symbol indicated as flexible by TDD-UL-DL-ConfigCommon and indicated as UL by TDD-UL-DL-Dedicated

    - Support: DCM

            - SFI in DCI format 2_0

  - Option 1: SFI cannot override the symbol into non-SBFD symbol

    - Support: Spreadtrum, Google (UE is not expect to receive plural indications)

    - Not support: OPPO (the last few consecutive semi-static SBFD symbols can be converted into UL symbols)

  - Option 2: Ignore SFI in DCI format 2_0

    - Support: Ericsson

  - Option 3: SFI indicates link direction for the UE

    - Support: CMCC

  - Option 4: Not support to enable SBFD operation and dynamic TDD operation in a cell simultaneously

    - Support: DCM

  - Option 5: The symbol is converted into UL/DL

    - Support: CEWiT

Tx/Rx occasion mapped to SBFD and non-SBFD symbols within a slot

For a physical channel/signal occasion mapped to SBFD and non-SBFD symbols within a slot if any,

            - Option 1: UE does not transmit or receive the physical channel/signal within the slot.

              - Support: OPPO (avoid by gNB for DG), DCM, Ericsson (avoid by gNB for DG), QC (avoid by gNB for DG)

            - Option 2: UE can transmit or receive the physical channel/signal within the slot only under certain conditions.

              - Support: vivo, TCL, Nokia, Sony, Apple (same FDRA)

Nokia proposed to support gNB to configure different SBFD UE operational modes to reduce unnecessary UE operations, e.g. following three operational modes

| UE operation modes | Description | Useful Scenario |
| --- | --- | --- |
| Mode 1 | UE is aware of SBFD subband configuration and signalling, but it doesn’t support SBFD UL transmission. | Legacy UE upgraded by SW. Operate in SBFD configured gNB. |
| Mode 2 | UE supports PUCCH transmission in SBFD UL, no SBFD UL PUSCH transmission. UE monitor PDCCH and transmit PUCCH in the same slot. | Low latency DL application |
| Mode 3 | UE supports both PDCCH and PUSCH in SBFD UL. | Advanced UE, both coverage and UL/DL low latency application. |

    - FDRA enhancements in SBFD symbols

Accroding to WID, enhancement on resource allocation in frequency domain in SBFD symbols include:

      - resource allocation in frequency domain for PDSCH/CSI-RS across two DL subbands in SBFD symbols

      - handling of unaligned boundaries between SBFD subband(s) and RBG, CSI reporting subband, CSI-RS resource, PRG

      - FDRA across two DL subbands

For FDRA across two DL subbands in SBFD symbols, the following agreements were made in Rel-18 SI.

Agreement If PRG is determined as wideband, study the following two options: - Option 1: non-contiguous frequency resources across two DL subbands but contiguous frequency resource within each DL subband can be allocated - FFS: Precoding assumption within and across the two DL subbands - Option 2: non-contiguous frequency resources across two DL subbands cannot be allocated The study should include the impact on UE complexity Agreement The following conclusion is to be captured in the TR If PRG is determined as wideband, better scheduling flexibility and higher DL data rate can be achieved if non-contiguous frequency resources across two DL subbands but contiguous frequency resource within each DL subband can be allocated. Compared to the case that PRG is determined as wideband and only contiguous frequency resources can be allocated, non-contiguous frequency resources across two DL subbands requires UE to handle two non- contiguous segments of contiguous RBs that may increase UE complexity for channel estimation. Agreement: Study the frequency resource allocation for CSI-RS across downlink subbands for SBFD-aware UEs considering the following options: - Option 1: Two contiguous CSI-RS resources that are linked - Option 2: One CSI-RS resource - Option 2-1: Non-contiguous CSI-RS resource allocation - Option 2-2: One contiguous CSI-RS resource allocation with non-contiguous CSI-RS resource derived by excluding frequency resources outside DL subband (s) Conclusion For the options agreed to study in RAN1#112 for frequency resource allocation for CSI-RS across downlink subbands for SBFD-aware UEs, the following observations are agreed. - For all the options, there is no impact on CSI-RS sequence generation. - Option 1 requires additional signalling to link two CSI-RS resources in two DL subbands. - Option 2-1 requires new RRC structure to configure non-contiguous RBs for one CSI-RS resource, which may require additional signalling overhead. - Option 2-2 can reuse the existing signalling design for CSI-RS resource configuration. Option 2-2 can be used to resolve the potential unaligned boundaries between CSI-RS resource configuration and SBFD subbands - Further discussion is required on the UE complexity due to: - UE capability of maximum number of configured CSI-RS resources - Processing non-contiguous CSI-RS

Wideband PRG

If PRG is determined as wideband,

- Option 1: non-contiguous frequency resources across two DL subbands but contiguous frequency resource within each DL subband can be allocated

  - Support: Spreadtrum, Huawei, vivo, CMCC, CATT, OPPO, CT, NEC, Samsung (no separate PDSCH processing timeline), Panasonic, Ericsson, DCM (prefer)

  - Precoding assumptions can be different in two DL subbands

    - Support: Spreadtrum, vivo

    - Not support: NEC

      - the difference in the channel conditions in two DL subbands may be small [NEC]

- Option 2: non-contiguous frequency resources across two DL subbands cannot be allocated

  - Support: ZTE, QC, DCM (acceptable if strong concern on UE complexity increase)

For dynamic bundling indication when gNB configures two sets of bundles and first set is configured with two values ‘n2-wideband’ or ‘n4-wideband’, the condition for determining wideband or narrowband depends on number scheduled RBs with respect to half of the size of the BWP NBWP2. Companies propose to further study the conditions in SBFD symbols.

FFS conditions for wideband precoder determination when precoding bundling is determined dynamically.

- Support: OPPO, QC

PDSCH RA type 1 FDRA across two DL subbands

Type-1 resource allocation in SBFD symbol with two downlink subband may be challenging especially when interleaving is enabled. There is a restriction in current specification where on the rate matching is only applied to the PDSCH data symbol, not the DMRS symbols. Several companies proposed to relax the restriction for flexible resource allocation.

Enhancements for PDSCH RA type 1 FDRA across two DL subbands

- Support: Spreadtrum, vivo, CMCC, CATT, xiaomi, Sharp (except PDSCH scheduled by DCI format 1_0 in CSS), CT, Panasonic, Sony, Nokia, DCM, LGE, Ericsson, Lenovo, CEWiT, QC, MTK

- Option 1: Rate matching is performed for PDSCH, including its DM-RS, around the UL subband and guardband(s) if any.

  - Support: Spreadtrum, vivo, CMCC, CATT, xiaomi, Sharp, DCM (exclude RBs outside DL subband(s)), LGE(RM or puncturing), Lenovo (non-interleaved), CEWiT (discard RBs outside DL subband(s)), QC

- Option 2: New RB indexing within RBs in DL subbands only, and legacy VRB-to-PRB mapping is used

  - Support: CT, Nokia, MTK, Lenovo (interleaved)

- Option 3: Legacy RB indexing, and modified interleaver for VRB-to-PRB mapping

  - Support: Nokia

- Option 4: Legacy RB indexing, and modified VRB-to-PRB mapping rule such that one VRB bundle is mapped to two PRB bundles to allow “mirror image” FDRA

  - Support: Nokia, Sony

- Option 5: warpped-around index to ensure allocated PRB within DL subbands

  - Support: Nokia

- Option 6: Enhancing the frequency resource indication method

  - Support: LGE

CSI-RS across two DL subbands

For the frequency resource allocation for CSI-RS across downlink subbands for SBFD-aware UEs,

- Option 1: Two contiguous CSI-RS resources that are linked

  - Support:

- Option 2: One CSI-RS resource

  - Support: Sharp, Lenovo

  - Option 2-1: Non-contiguous CSI-RS resource allocation

    - Support: Huawei, Ericsson, Samsung

      - Better flexibility [Huawei, Ericsson]

  - Option 2-2: One contiguous CSI-RS resource allocation with non-contiguous CSI-RS resource derived by excluding frequency resources outside DL subband(s)

    - Support: Spreadtrum, vivo, ZTE, CMCC, CATT, IDC, Samsung, xiaomi, Fujitsu, NEC, Nokia, DCM, WILUS, CEWiT, QC

      - Backward compatibility [vivo]

      - Less spec impact [vivo, ZTE, DCM]

      - Can enable single CSI-RS resource configuration across SBFD and non-SBFD symbols [CMCC, QC]

      - Similar as handling of unaligned boundaries [DCM]

  - Option 2-3: One contiguous CSI-RS resource allocation with contiguous CSI-RS resource derived by wrapping-around the frequency resources over the two DL subbands

    - Support: Lenovo

| Option 1 | Option 2-1 | Option 2-2 |
| --- | --- | --- |

Figure 7: CSI-RS resource allocation across DL subbands [4]

      - Handling of unaligned boundaries

For handling of unaligned boundaries between SBFD subband(s) and RBG, CSI reporting subband, CSI-RS resource, PRG, the following agreements were made in Rel-18 SI.

Agreement For SBFD-aware UEs, Option 1 with update is agreed for resource allocation in frequency-domain in case of unaligned boundaries between RBG and SBFD subbands for better resource utilization. For an RBG that overlaps the subband boundary, - Option 1 (with update): - The Part of the DL RBG inside the DL subband can be used - The Part of the UL RBG inside the UL subband can be used Agreement The following is to be captured in the TR: The part of the RBG outside the DL subband cannot be used for DL reception and the part of the RBG outside the UL subband cannot be for UL transmission at least for semi-static SBFD. Agreement - For semi-static SBFD, for a CSI-RS resource which overlaps with SBFD subband boundaries, only CSI-RS resources within DL subband(s) are valid for SBFD-aware UE. - For semi-static SBFD, for a CSI reporting subband which overlaps with SBFD subband boundaries, CSI report is derived based on CSI-RS resources excluding CSI-RS resources outside DL subband(s). Conclusion For a PRG that overlaps with subband boundary, if the part of DL PRG inside the DL subband can be used, better scheduling flexibility and resource utilization can be achieved, however degraded channel estimation quality in the partial PRG is expected compared to a PRG due to limited RBs in the partial PRG. - Note: UE complexity could increase if this feature is supported

Partial RBG

Figure 8: Partial RBG at boundary of the DL subband [37]

For an RBG that overlaps the subband boundary,

- The part of the DL RBG inside the DL subband can be used for DL reception. The part of the DL RBG outside the DL subband cannot be used for DL reception.

- The part of the UL RBG inside the UL subband can be used for UL transmission. The part of the UL RBG outside the UL subband cannot be used for UL transmission.

- Support: Spreadtrum, Huawei, ZTE, CMCC, CATT, xiaomi, Sharp, Sony, Lenovo, Samsung, Nokia, WILUS, QC

- Not support: LGE (restrict the position of the SBFD subband boundary to prevent misalignment)

- Mechanisms

  - Partial RBG similar as partial RBG at the BWP edges: CMCC

  - Rate matching: Lenovo, Samsung, WILUS

Partial PRG

For a PRG that overlaps the subband boundary,

- The part of the DL PRG inside the DL subband can be used

  - Support: Spreadtrum, Huawei, vivo, ZTE, CMCC, CATT, xiaomi, OPPO, DCM, Sony, Ericsson, Lenovo

- Avoid by gNB implementation (UE doesn’t expect partial PRG(s) other than the first and last PRG within the UE DL BWP)

  - Support: vivo, LGE, ASUSTeK, QC

    - Expected gain does not motivate UE complexity increase [QC]

Partial 4RBs CSI-RS resource group

Figure 9: CSI-RS frequency resource granularity in SBFD symbols [37]

In case a CSI-RS resource and DL subband are misaligned relative to 4RB grid of the CSI-RS as illustrated in the above figure, majority companies agree with the SI agreement that only CSI-RS resources within DL subband(s) are valid.

For a CSI-RS resource which overlaps with SBFD subband boundaries,

- Only CSI-RS resources within DL subband(s) are valid for SBFD-aware UE.

  - Support: Huawei, ZTE, CATT, xiaomi, Sony, LGE, QC

- Drop the whole 4RBs CSI-RS resource group

  - Support: CMCC

    - To maintain the same CSI-RS frequency resource allocation granularity as legacy CSI-RS

Partial CSI reporting subband

Figure 10: Partial CSI subband [37]

Majority companies agree with the SI agreement below. Although CMCC proposed to drop the whole 4 RBs CSI resource group, CMCC thinks that CSI can be reported even the CSI-RS is not fully allocated in all PRBs in the CSI reporting subband.

- For a CSI reporting subband which overlaps with SBFD subband boundaries, CSI report is derived based on CSI-RS resources excluding CSI-RS resources outside DL subband(s).

- Support: Huawei, ZTE, CATT, Sharp, Sony, LGE, QC

    - Physical channels/signals across SBFD and non-SBFD symbols

Accroding to WID, enhancements on physical channels/signals and procedure across SBFD symbols and non-SBFD symbols in different slots, where each transmission/reception within a slot has either all SBFD or all non-SBFD symbols include:

      - resource allocation in frequency domain for transmission or reception in SBFD symbols and non-SBFD symbols with different available frequency resource in different slots

      - CSI report of which associated CSI-RS instances occur in both SBFD symbols and non-SBFD symbols in different slots

For FDRA in SBFD and non-SBFD symbols in different slots, the following agreements were made in Rel-18 SI.

Agreement For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols), if the transmissions/receptions can be in SBFD symbols and non-SBFD symbols with different available resources, study at least the following frequency resource allocation options for PDSCH, CSI-RS, PUSCH, PUCCH, SRS for SBFD-aware UE: - Option 1: Separate FDRA determination for SBFD slots and non-SBFD slots. - Option 1-1: Separate FDRA configurations/indications for SBFD slots and non-SBFD slots - Option 1-2: Separate frequency resources determined for SBFD slots and non-SBFD slots based on single FDRA configuration/indication - Option 1-3: single FDRA configuration/indication and RB offset(s) - Option 2: Perform rate matching or puncturing on the RBs outside DL/UL subbands for DL/UL channels/signals. - Option 3: A DL/UL channel/signal overlapping with RBs outside DL/UL subbands in a SBFD slot is dropped or postponed. Note: Different options can be studied for different signals/channels. Agreement The following conclusion is to be captured in the TR gNB can configure a CORESET and a search space in a way such that the MOs of the search space occur in either SBFD or non-SBFD symbols, or the MOs of the search space occur in both SBFD and non-SBFD symbols but the associated CORESET does not overlap the boundary of a DL subband in SBFD symbols. If it is agreed to be beneficial that a CORESET and a search space are configured that the MOs of the search space occur in both SBFD and non-SBFD symbols and the associated CORESET overlaps the boundary of a DL subband in SBFD symbols, at least the following options can be considered for SBFD-aware UE: - Option 1: Separate valid resources for the CORESET in SBFD symbols and in non-SBFD symbols. - Option 2: Rate matching or puncturing on the REG(s) of a PDCCH outside DL subband(s). - Option 3: UE does not monitor a PDCCH candidate if it is mapped to one or more REs that overlap with REs outside DL subband(s). - Option 4: Drop search space(s) when the associated CORESET overlaps with RBs outside DL subband(s) - Option 5: Separate search spaces associated with a CORESET in SBFD and non-SBFD symbols Note: These options are applicable to at least USS

For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols),

- Option 1: The transmissions/receptions are restricted to SBFD symbols only or non-SBFD symbols only

  - Support: ZTE (subject to UE capability), Samsung (channel-specific, for CSI-RS and SRS), Ericsson, QC (for CG-PUSCH, SPS PDSCH)

- Option 2: The transmissions/receptions can be in SBFD symbols and non-SBFD symbols

  - Support: vivo, ZTE (subject to UE capability), Samsung (unified design; or channel-specific for PDSCH/PUSCH/PUCCH), KT, Ericsson, QC (multi-slot UL transmission)

For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols), if the transmissions/receptions can be in SBFD symbols and non-SBFD symbols with different available resources, study at least the following frequency resource allocation options for PDSCH, CSI-RS, PUSCH, PUCCH, SRS for SBFD-aware UE:

- Option 1: Separate FDRA determination for SBFD slots and non-SBFD slots.

  - Support: vivo (PDSCH, CSI-RS, PUSCH, PUCCH, SRS), MTK (for P/SP resource allocation), KT

  - Option 1-1: Separate FDRA configurations/indications for SBFD slots and non-SBFD slots

    - Support: vivo, CMCC (semi-static scheduling), ZTE

    - Not support: DCM (for PDSCH/PUSCH)

  - Option 1-2: Separate frequency resources determined for SBFD slots and non-SBFD slots based on single FDRA configuration/indication

    - Support: Spreadtrum (for PUSCH, PDSCH, CSI-RS), CMCC (dynamic scheduling), CT (PDSCH&PUSCH w/ repetition), Ericsson (PUSCH)

  - Option 1-3: single FDRA configuration/indication and RB offset(s)

    - Support: vivo, ZTE, OPPO, CT (PDSCH&PUSCH w/ repetition), IDC, QC (multi-slot PUSCH, PUSCH Type A repetition, TBoMS), Ericsson (PUSCH)

    - Not support: DCM (for PDSCH/PUSCH, CSI-RS/SRS)

- Option 2: Perform rate matching or puncturing on the RBs outside DL/UL subbands for DL/UL channels/signals.

  - Support: vivo (for PDSCH and CSI-RS), ZTE (as complement), QC (PDSCH repetition), WILUS (PDSCH)

- Option 3: A DL/UL channel/signal overlapping with RBs outside DL/UL subbands in a SBFD slot is dropped or postponed.

  - Support: Spreadtrum (for PUCCH, SRS), vivo (PUSCH, PUCCH, SRS), ZTE (as complement), CATT (repetition, TBoMS, multi-slot PDSCH/PUSCH), MTK (for repetition), DCM, NEC (multi-slot PUSCH/PDSCH), WILUS (PUSCH/PUCCH repetition)

  - Not support: Sony

MediaTek proposed to support frequency hopping only on non-SBFD symbols for inter-slot frequency hopping considering the diversity gain obtained from frequency hopping is reduced on SBFD symbols because of the smaller uplink frequency resources. DOCOMO also propsed to study disabling FH in SBFD symbols.

PDSCH & PUSCH

For PDSCH and PUSCH across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols),

- Option 1: Separate FDRA determination for SBFD slots and non-SBFD slots.

  - Support: Huawei (consider with higher priority)

  - Option 1-1: Separate FDRA configurations/indications for SBFD slots and non-SBFD slots

    - Support: Huawei (consider with higher priority)

  - Option 1-2: Separate frequency resources determined for SBFD slots and non-SBFD slots based on single FDRA configuration/indication

    - Support: Huawei (consider with higher priority), Nokia (PUSCH repetition and a TBoMS, with a scaling factor)

  - Option 1-3: single FDRA configuration/indication and RB offset(s)

    - Support: Huawei (consider with higher priority)

- Option 2: Perform rate matching or puncturing on the RBs outside DL/UL subbands for DL/UL channels/signals.

  - Support:

- Option 3: A DL/UL channel/signal overlapping with RBs outside DL/UL subbands in a SBFD slot is dropped or postponed.

  - Support: Huawei (depending on gNB configuration), Samsung

PUCCH

For PUCCH across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols),

- Option 1: Separate FDRA determination for SBFD slots and non-SBFD slots.

  - Support:

  - Option 1-1: Separate FDRA configurations/indications for SBFD slots and non-SBFD slots

    - Support: Huawei (separate PUCCH resource sets configuration), QC

  - Option 1-2: Separate frequency resources determined for SBFD slots and non-SBFD slots based on single FDRA configuration/indication

    - Support:

  - Option 1-3: single FDRA configuration/indication and RB offset(s)

    - Support: QC

- Option 2: Perform rate matching or puncturing on the RBs outside DL/UL subbands for DL/UL channels/signals.

  - Support:

- Option 3: A DL/UL channel/signal overlapping with RBs outside DL/UL subbands in a SBFD slot is dropped or postponed.

  - Support:

SRS

For SRS across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols),

- Option 1: Separate FDRA determination for SBFD slots and non-SBFD slots.

  - Support:

  - Option 1-1: Separate FDRA configurations/indications for SBFD slots and non-SBFD slots

    - Support: Huawei (separate SRS resource sets configuration)

  - Option 1-2: Separate frequency resources determined for SBFD slots and non-SBFD slots based on single FDRA configuration/indication

    - Support:

  - Option 1-3: single FDRA configuration/indication and RB offset(s)

    - Support:

- Option 2: Perform rate matching or puncturing on the RBs outside DL/UL subbands for DL/UL channels/signals.

  - Support:

- Option 3: A DL/UL channel/signal overlapping with RBs outside DL/UL subbands in a SBFD slot is dropped or postponed.

  - Support:

PDCCH

Enhancements on PDCCH for the case that a CORESET and a search space are configured that the MOs of the search space occur in both SBFD and non-SBFD symbols and the associated CORESET overlaps the boundary of a DL subband in SBFD symbols

- Support: Nokia, NEC, Sony, Lenovo, New H3C, ETRI

- Not support: Spreadtrum, CATT, xiaomi, OPPO, Sharp, Ericsson

gNB can configure a CORESET and a search space in a way such that the MOs of the search space occur in either SBFD or non-SBFD symbols, or the MOs of the search space occur in both SBFD and non-SBFD symbols but the associated CORESET does not overlap the boundary of a DL subband in SBFD symbols.

If it is agreed to be beneficial that a CORESET and a search space are configured that the MOs of the search space occur in both SBFD and non-SBFD symbols and the associated CORESET overlaps the boundary of a DL subband in SBFD symbols, at least the following options can be considered for SBFD-aware UE:

- Option 1: Separate valid resources for the CORESET in SBFD symbols and in non-SBFD symbols.

  - Support: NEC, Lenovo, ETRI

- Option 2: Rate matching or puncturing on the REG(s) of a PDCCH outside DL subband(s).

  - Support: ETRI

- Option 3: UE does not monitor a PDCCH candidate if it is mapped to one or more REs that overlap with REs outside DL subband(s).

  - Support: Nokia, ETRI

- Option 4: Drop search space(s) when the associated CORESET overlaps with RBs outside DL subband(s)

- Option 5: Separate search spaces associated with a CORESET in SBFD and non-SBFD symbols

- Option 6: The CCEs of a CORESET that collides with REs outside of DL subband are dropped and the Aggregation Levels (AL) of PDCCH candidates with these colliding CCEs are reduced to the nearest valid AL

  - Support: Sony

CSI measurements and reporting

For CSI report of which associated CSI-RS instances occur in both SBFD symbols and non-SBFD symbols in different slots, the following agreements were made in Rel-18 SI.

Agreement: For SBFD-aware UEs, study the following options for CSI report associated with periodic/semi-persistent CSI-RS, at least, across SBFD symbols and non-SBFD symbols in different slots (each CSI-RS resource within a slot has either all SBFD or all non-SBFD symbols): - Option 1: separate CSI reporting for SBFD symbols and non-SBFD symbols - Option 2: same CSI reporting for SBFD symbols and non-SBFD symbols Agreement For SBFD-aware UEs, study the following options for CSI report associated with periodic/semi-persistent CSI-RS in case the periodicity is such that CSI-RS instances occur in both SBFD and non-SBFD symbols: - Option 1: two CSI-ReportConfigs, where one is associated with SBFD symbols and the other is associated with non-SBFD symbols - Option 1-1: One CSI-ReportConfig is associated with a CSI-RS restricted to SBFD symbols only and the second CSI-ReportConfig is associated with a second CSI-RS restricted to non-SBFD symbols only; - Option 1-2: Both CSI-ReportConfigs are associated with the same CSI-RS. The CSI report associated with one CSI-ReportConfig is derived based on CSI-RS instances in SBFD symbols only. The CSI report associated with the second CSI-ReportConfig is derived based on CSI-RS instances in non-SBFD symbols only. - Option 2: one CSI-ReportConfig associated with both SBFD symbols and non-SBFD symbols - Option 2-1: One CSI-ReportConfig is associated with two CSI-RSs which are restricted to SBFD symbols and non-SBFD symbols respectively. Separate CSI measurements are derived based on the first and second CSI-RSs respectively. - Option 2-2: One CSI-ReportConfig is associated with one CSI-RS. The CSI report is derived based on CSI-RS which can be in SBFD symbols or non-SBFD symbols in different time instances. - FFS impact on UE CSI processing and reporting timeline Note: Whether the CSI-RS resource can be used for SBFD and non-SBFD symbols may depend on, e.g., gNB implementation of same/different antenna configuration in both symbols. Option 1-1 can be supported according to existing specification by gNB configuration of appropriate periodicities to ensure that the CSI-RS associated with each CSI-ReportConfig is confined to either SBFD symbols or non-SBFD symbols only. But it may restrict the gNB configuration flexibility and enhancements can be considered by additional indication or rules to determine the CSI-RS is valid within one symbol type and is invalid in the other symbol type. Option 2-2 can be supported according to existing specification to configure measurement restriction so that UE would not average CSI measurements across SBFD and non-SBFD symbols.

Figure 11: P/SP CSI-RS associated with same or separate CSI Report configurations [30]

For SBFD-aware UEs, for CSI report associated with periodic/semi-persistent CSI-RS in case the periodicity is such that CSI-RS instances occur in both SBFD and non-SBFD symbols:

- Option 1: two CSI-ReportConfigs, where one is associated with SBFD symbols and the other is associated with non-SBFD symbols

  - Option 1-1: One CSI-ReportConfig is associated with a CSI-RS restricted to SBFD symbols only and the second CSI-ReportConfig is associated with a second CSI-RS restricted to non-SBFD symbols only;

    - Support: Huawei, vivo (prioritized), xiaomi, OPPO, Sony, Ericsson, MTK (baseline), Lenovo, QC

  - Option 1-2: Both CSI-ReportConfigs are associated with the same CSI-RS. The CSI report associated with one CSI-ReportConfig is derived based on CSI-RS instances in SBFD symbols only. The CSI report associated with the second CSI-ReportConfig is derived based on CSI-RS instances in non-SBFD symbols only.

    - Support: ZTE, IDC

- Option 2: one CSI-ReportConfig associated with both SBFD symbols and non-SBFD symbols

  - Support: Panasonic, CEWiT

  - Option 2-1: One CSI-ReportConfig is associated with two CSI-RSs which are restricted to SBFD symbols and non-SBFD symbols respectively. Separate CSI measurements are derived based on the first and second CSI-RSs respectively.

    - Support: Fujitsu, Sony, KT, CATT (based on NES framework)

  - Option 2-2: One CSI-ReportConfig is associated with one CSI-RS. The CSI report is derived based on CSI-RS which can be in SBFD symbols or non-SBFD symbols in different time instances.

    - Support: Spreadtrum, Huawei, vivo (prioritized), CMCC (Option 2-2’), New H3C, xiaomi, OPPO, IDC, Sharp, Fujitsu, Ericsson, WILUS

      - Option 2-2’: One CSI-ReportConfig is associated with one CSI-RS which contains two CSI sub-configurations. The CSI report is derived based on CSI-RS which can be in SBFD symbols or non-SBFD symbols in different time instances associated with different sub-configurations. [CMCC]

    - Not support: DCM

      - gNB can’t differentiate whether the CSI report instance is based on CSI-RS instance in SBFD symbols or in non-SBFD symbols [DCM]

In addition, MediaTek proposed to allow MAC CE to activate at least 2 CSI reporting configurations, one for each slot type for semi-persistent CSI reporting PUCCH.

    - Configurations in SBFD and non-SBFD symbols

WID includes the following objective.

- Configurations for SRS, PUCCH and PUSCH on SBFD symbols and non-SBFD symbols, e.g., resources, frequency hopping parameters, UL power control parameters and/or beam/spatial relation

The following related agreements were made in Rel-18 SI.

Agreement: Study at least the followings for SRS, PUCCH and PUSCH on SBFD symbols and non-SBFD symbols in different slots: - Whether/how to have separate resources - Whether/how to have separate FH parameters - Whether/how to have separate UL power control parameters - Whether/how to have separate beam/spatial relation Agreement The following conclusion is to be captured in the TR For SRS, PUCCH and PUSCH on SBFD symbols and non-SBFD symbols in different slots, it may be beneficial to have separate resources, FH parameters, UL power control parameters and/or beam/spatial relation.

      - General

Companies’ views on whether to support separate resources, FH parameters, UL PC parameters and beam/spatial relation are summarized below.

- Separate resources

  - Support: ZTE, CATT (not for PUSCH), Fujitsu, NEC (CSI-RS, PUCCH, SRS, CG-PUSCH, SPS PDSCH), Panasonic (configured UL) , LGE (PUSCH, PUCCH), Ericsson (SRS), QC (CG-PUSCH, SPS-PDSCH, SRS)

  - Not support: Spreadtrum (for time domain)

- Separate FH parameters

  - Support: Spreadtrum, ZTE, CATT, Fujitsu, Panasonic (configured UL) , LGE (PUSCH, PUCCH), Samsung (PUSCH, PUCCH)

  - Not support:

- Separate UL power control parameters

  - Support: ZTE, CATT, Fujitsu, Samsung, QC, Panasonic (configured UL) , LGE (PUSCH, PUCCH), Lenovo, WILUS (PUSCH, PUCCH), KT, Apple (CG type 1)

  - Not support: Spreadtrum

- Separate beam/spatial relation

  - Support: ZTE, CATT, Fujitsu, Panasonic (configured UL), LGE (PUSCH, PUCCH), Samsung, KT (separate BM procedures), Apple (CG type 1)

  - Not support: Spreadtrum

Sony proposed to support different TCI state indications for transmissions in SBFD and non-SBFD OFDM symbols.

PUSCH

- Separate resource sets for PUSCH on SBFD and non-SBFD symbols

  - Support: Nokia (CG PUSCH)

- Separate FH parameters for PUSCH on SBFD and non-SBFD symbols

  - Support: Spreadtrum, Huawei, CMCC, MTK

- Separate UL power control parameters for PUSCH on SBFD and non-SBFD symbols

  - Support: Huawei (reusing R17 m-TRP based PUSCH repetition mechanism with necessary modifications), CMCC (open loop and close loop PC parameters)

- Separate beam/spatial relation for PUSCH on SBFD and non-SBFD symbols

  - Support: Huawei (reusing R17 m-TRP based PUSCH repetition mechanism with necessary modifications), CMCC (SRI associated with SRS resource with same symbol type)

For PUSCH/PUCCH FH, Spreadtrum, Huawei, CMCC, MTK, WILUS propose to update the FH pattern/formula to ensure that the PUSCH FH is always within UL subband in SBFD symbols.

PUCCH

- Separate resource sets for PUCCH on SBFD and non-SBFD symbols

  - Support: Huawei, CMCC, OPPO, QC

- Separate FH parameters for PUCCH on SBFD and non-SBFD symbols

  - Support: Huawei, CMCC, OPPO, MTK

- Separate UL power control parameters for PUCCH on SBFD and non-SBFD symbols

  - Support: Huawei, CMCC, OPPO

- Separate beam/spatial relation for PUCCH on SBFD and non-SBFD symbols

  - Support: Huawei, CMCC, OPPO

In addition, Huawei proposed to support enhancements for UCI multiplexing on PUSCH in SBFD symbols, e.g., separate UCI multiplexing parameters for SBFD slots and non-SBFD slots, considering that the gNB may suffer more severe interferences in SBFD symbols.

SRS

- Separate resource sets for SRS on SBFD and non-SBFD symbols

  - Support: Huawei, CMCC, CATT, New H3C, Ericsson, QC

- Separate FH parameters for PUCCH on SBFD and non-SBFD symbols

  - Support: MTK, CATT

- Separate UL power control parameters for SRS on SBFD and non-SBFD symbols

  - Support: Huawei (can be supported naturally if separate SRS resource sets for SBFD slots and non-SBFD slots are supported since power control parameters are configured per SRS resource set), CMCC, CATT

- Separate beam/spatial relation for SRS on SBFD and non-SBFD symbols

  - Support: CMCC, CATT

For SRS FH, MTK proposed to update the FH procedure to ensure that the SRS FH is always within UL subband in SBFD symbols.

Qualcomm proposed to further discuss the available slot counting for A-SRS transmission with SBFD operation.

    - Collision handling

WID includes the following objective.

- Collision handling between DL reception in DL subband(s) and UL transmission in UL subband in a SBFD symbol

There are following agreements made in Rel-18 SI.

Agreement Identify if there are any cases of time domain conflict of UE’s UL and DL operation in the same SBFD symbol for SBFD aware UE - If there are, whether/how to avoid/handle such collision cases (as second step) Agreement Study the following options for SBFD operation in SSB symbols. - Option 1: UL subband cannot be configured in an SSB symbol - FFS handling of misaligned periodicities between SSB and semi-static SBFD subband time location configuration - Option 2: An UL subband can be configured in an SSB symbol - FFS whether/when and/or under which conditions an SBFD-aware UE transmits in the UL subband or may receive SSB in the symbol. Agreement An UL subband can be configured in an SSB symbol. - Note: It is SSB from serving cell perspective, which can be CD-SSB or NCD-SSB. - Whether actual UL transmission can be done is for further discussion Agreement The following conclusion is to be captured in the TR If SBFD-aware UEs are not allowed to transmit in the SSB symbol but is allowed to receive within the DL BWP in the SSB symbol, negative impact on SSB detection and measurement can be avoided but UL performance may be degraded due to fewer UL opportunities. If SBFD-aware UE is allowed to transmit in the SSB symbol, the UE may only transmit UL in an UL subband depending on gNB scheduling, configuration, UE measurement or priority rule. There may be negative impact on SSB detection and measurement if the SBFD-aware UE is requested to transmit in the SSB symbol.

Collision handling for HD-FDD operation for RedCap UEs was discussed in Rel-17 and the following cases were identified.

- Case 1: Dynamically scheduled DL reception vs. semi-statically configured UL transmission

  - e.g., dynamic PDSCH or CSI-RS collides with configured SRS, PUCCH, or CG PUSCH

- Case 2: Semi-statically configured DL reception vs. dynamically scheduled UL transmission

  - e.g., PDCCH or SPS PDSCH collides with dynamic PUSCH or PUCCH

- Case 3: Semi-statically configured DL reception vs. semi-statically configured UL transmission

- Case 4: Dynamically scheduled DL reception vs. dynamic scheduled UL transmission

- Case 5: Configured SSB vs. dynamically scheduled or configured UL transmission

  - e.g., PUSCH, PUCCH, PRACH, SRS

- Case 8: Dynamic or semi-static DL vs. valid RO

- Case 9: Collision due to direction switching

Assuming the same list of collision cases for SBFD, companies’ views are summaried below.

- Case 1: dynamic DL vs. semi-static UL

  - Support: Spreadtrum, Huawei, vivo, ZTE, CMCC, xiaomi, OPPO, IDC, Panasonic, Sony, Apple, Nokia, DCM, Ericsson, WILUS, ITRI, QC

  - Collision handling rule

    - Scheduled DL reception(s) prioritized: Spreadtrum, Huawei, vivo, ZTE, OPPO, IDC, Panasonic, DCM, Sony, Apple, Ericsson, QC (within cancellation timeline)

    - Higher PHY priority transmission/reception is prioritized: vivo

    - UL CG override DG DL: CMCC (HP CG vs. LP DG)

    - Up to UE decision: CMCC

- Case 2: semi-static DL vs. dynamic UL

  - Support: Spreadtrum, Huawei, vivo, ZTE, CMCC, xiaomi, OPPO, IDC, Sony, Apple, Nokia, DCM, Panasonic, Ericsson, WILUS, ITRI, QC

  - Collision handling rule

    - Scheduled UL transmission(s) prioritized: Spreadtrum, Huawei, vivo, ZTE, CMCC, OPPO, IDC, Apple, DCM, Panasonic, Sony, Ericsson, QC

    - Higher PHY priority transmission/reception is prioritized: vivo

- Case 3: semi-static DL vs. semi-static UL

  - Support: vivo, ZTE, CMCC, New H3C, xiaomi, Nokia, DCM, WILUS, QC (FFS)

  - Not support (error case): Spreadtrum, Huawei, OPPO, Panasonic, Apple, Ericsson

  - Collision handling rule

    - Higher PHY priority is prioritized: vivo, ZTE, CMCC, New H3C

    - Indication by gNB: vivo (in TDD-UL-DL-ConfigDedicated and/or dynamic SFI), CMCC

    - Predefined rules: ZTE, CMCC

    - UL transmission within UL subband is prioritized: ZTE( for same PHY priority), Apple

- Case 4: dynamic DL vs. dynamic UL

  - Support: vivo, ZTE, CMCC, xiaomi, ITRI (w/ different priorities)

  - Not support: Spreadtrum, Huawei, OPPO, IDC, Panasonic, Ericsson, QC, DCM (at least for non-repetition)

  - Collision handling rule

    - Transmission(s)/reception(s) scheduled later is prioritized: vivo, CMCC

    - Higher PHY priority transmission/reception is prioritized: vivo, ZTE, CMCC

    - Dynamic-grant PDSCH can have higher priority than dynamic-grant PUSCH without UCI, whereas dynamic-grant PDSCH can have lower priority than dynamic-grant PUSCH with UCI: IDC

- Case 5: SSB vs. UL

  - An UL subband can be configured in an SSB symbol

    - Support: Huawei, vivo, ZTE, New H3C, CMCC, CATT, xiaomi, Sharp, CT, IDC, NEC, Panasonic, Sony, Apple, Ericsson, LGE, Spreadtrum, QC, MTK, DCM, WILUS, ITRI

  - UL transmission in SSB symbols

  - UL transmission is allowed within UL subband in SSB symbols

    - Support: Huawei, vivo, New H3C, CMCC, CATT, Sharp, CT, IDC, NEC, Panasonic, Sony, Apple, Ericsson, LGE, WILUS

    - Subject to certain conditions

      - Support: Huawei, vivo, New H3C, CMCC, CATT, Sharp, CT, NEC, Panasonic, Sony, Apple, Ericsson, LGE

      - Conditions

        - allowed on some SSB symbols: Huawei (non-protected SSB), CMCC (based on gNB configuration), Panasonic (UE receives SSB periodically once per the maximum SSB periodicity), New H3C (UL subband protection window), Apple (bitmask indication)

        - Tx power below a predefined threshold: vivo

        - Base on gNB configuration: Ericsson, Sharp (Enable/Disable indication), NEC, LGE

        - Distance between SSB and UL transmission: CT

        - Certain type of UL transmission is allowed: Sony (at least CG-PUSCH and HP UL), LGE, WILUS

      - UL subband used for DL if conditions are not met

        - UL subband cannot be used for DL reception

          - Support: CMCC

        - UL subband can be used for DL reception

  - UL transmission is not allowed within UL subband in SSB symbols

    - Support: Spreadtrum, QC, MTK, DCM, ITRI

    - SSB symbols fallback to full DL symbols

      - Support: Spreadtrum, MTK

- Case 8: Dynamic or semi-static DL vs. valid RO

  - WILUS proposed that different UE behaviors can be further investigated for the cases whether PDSCH is semi-statically scheduled via RRC configuration or PDSCH is dynamically indicated via DCI.

- Case 9: Collision due to direction switching

  - QC proposed to consider collision in SBFD symbols occurs due to the overlapping between UL transmission and DL reception at least one SBFD symbol or due to the lack of sufficient timeline for UE to switch from DL reception to UL transmission.

Figure 12: Time domain collision in SBFD symbols [37]

Others

CATT proposed to study the order of collision handling of the following collision types.

- Type 1: Collision between transmissions/receptions and SBFD subbands;

- Type 2: Collision between transmissions with same transmission direction;

- Type 3: Collision between transmissions and receptions.

ZTE discussed UL inter-UE collision handling in SBFD subbands with the following observations and proposal.

Observation 2: UL transmission in UL subband within a DL symbol or SS/PBCH block symbols cannot be cancelled by the DCI format 2_4.

Observation 3: The existing UL power control mechanism can hardly meet the more diversified power control requirements caused by CLI.

Proposal 20: How to apply the UL inter-UE multiplexing mechanisms, including UL cancelation and UL power control enhancement, in the UL subband needs to be further discussed.

DOCOMO also proposed to study SBFD impact on UL cancellation DCI format 2_4.

WILUS proposed that an UL subband in symbols configured as DL by TDD-UL-DL-ConfigCommon and symbols configured for SSB reception should be included as reference time-frequency resource region for UL CI in addition to UL symbols and flexible symbols not configured for SSB reception.

Sony proposed to reduce inter subband CLI to SSB in SBFD OFDM symbols by disabling some of the RBs in the UL subband in OFDM symbols that overlap with the SSB. In addition, Sony observed that an SSB burst set containing SSBs that are in SBFD and non-SBFD OFDM symbols, and/or the same SSB that in different periods is in SBFD and non-SBFD OFDM symbols, may lead to inconsistent SSB measurements since some SSBs suffer from inter subband CLI whilst others do not suffer from such CLI, and proposed to apply an offset SBFD to measurements for SSBs in SBFD OFDM symbols for operations where SSBs can reside in SBFD and non-SBFD OFDM symbols.

Nokia proposed to study options for SBFD PUSCH scheduling restriction to avoid collision of PDCCH monitoring and SBFD PUSCH scheduling.

- Option 1: UE only supports configured grant PUSCH in SBFD UL

  - In this option, UE is already configured with a PUSCH resource, and UE will not monitor PDCCH in the scheduled SBFD UL symbols.

  - UE is not expected to be scheduled with DG-PUSCH in SBFD symbols.

- Option 2: UE only supports multi-PUSCH scheduling for SBFD UL.

  - In this option, UE is configured to be scheduled with multi-PUSCH for PUSCH. The UL DCI can be sent in a specific time slot. Because UE has full information of PUSCH scheduling in a TDD period, UE can easily perform DL and UL operation in a TDD frame.

- Option 3: UE is pre-configured with a DL/UL pattern for SBFD slots/symbols.

  - In this option, UE is configured with its own TDD frame pattern in SBFD slots. This pattern can be different per UEs. The prior knowledge may limit the flexible use of DL/UL for SBFD slots, but it may reduce the complexity in UE.

  - Miscellaneous

CMCC proposed to discuss RBG size for RA type 0 in SBFD symbols by considering the following options.

- Option 1: RBG size is determined based on the size of DL/UL BWP;

- Option 2: RBG size is determined based on the size of available DL/UL resources.

CMCC and Samsung proposed to discuss CSI reporting subband size in SBFD symbols by considering the following options.

- Option 1: CSI reporting subband size is determined based on the size of DL BWP;

- Option 2: CSI reporting subband size is determined based on the size of available DL frequency resource.

OPPO proposed that the RBG size is determined based on the actual UL/DL subband size, while the FDRA field size is still determined based on active BWP size. In addition, for TBS determination, OPPO proposed that the amount of RBs used to determine the PDSCH TB size does not count the RBs overlapping with the actual UL subband and the guard bands (if any).

Samsung proposed no TBS determination rule update for partial RBG handling.

DOCOMO proposed that for PUSCH repetition type B, nominal repetition can be segmented into actual repetitions around boundary of SBFD symbols and non-SBFD symbols. In addition, DOCOMO proposed to study SBFD impact on SPS HARQ-ACK deferring.

InterDigital proposed to consider the methods to handle the UL/DL timing misalignment issues at the UE with non-zero TA scheduled for UL transmission in UL subband in an SBFD slot, right after the legacy ‘D’ slot back-to-back, when the number of symbols N for TA exceeds the UL grant starting symbol N’, which directly impacts on legacy operations.

Ericsson proposed that DL and UL subbands in SBFD symbols are time aligned with the DL and UL frame timing, respectively.

WILUS proposed that zero value of NTA,offset,SBFD applied for SBFD symbols and non-zero value of NTA,offset applied for UL symbols can be considered to mitigate the interference due to time misalignment at a gNB between DL transmission and UL reception.

Xiaomi proposed that guard period between DL region and UL subband is needed and at least the following mechanism can be considered.

- Option 1: The guard period is configured by gNB for SBFD aware UE.

- Option 2: UE does not expect to transmit in the uplink on the first G symbols within UL subband.

- Option 3: The first OFDM symbol of UL subband is always adjacent UL symbol or flexible symbol.

NEC proposed to specify DMRS bundling enhancement when the TDW overlaps with SBFD symbols. LGE proposed to discuss how to support joint channel estimation when PUSCH/PUCCH repetition is transmitted across SBFD symbols and non-SBFD symbols in different slots. Qualcomm proposed when DMRS bundling is enabled, the phase coherency is not maintained across SBFD and non-SBFD symbols. The actual TDW is terminated at the boundary and new actual TDW is started. ETRI proposed to study enhancements of nominal/actual time domain window for DMRS bundling of PUCCH/PUSCH for SBFD-aware UE.

WILUS proposed to consider availability in frequency domain to determine HPN of multi-slot PUSCH scheduled by a single DCI.

ETRI proposed to clarify CPU occupation for SBFD CSI generation.

- [Open] 1st round discussion

Proposal 1-1

Proposed Agreement:

Semi-static indication of time and frequency locations of SBFD subbands is cell-specific, i.e. not UE-specific or BWP-specific.

|  | Company |
| --- | --- |
| Support | Sony (support also UE specific), New H3C, Huawei, HiSilicon, OPPO, TCL, CATT, CEWiT, Spreadtrum, Nokia, NSB, NEC, Transsion, Lenovo (also UE specific), LG, Ericsson, Fujitsu, DOCOMO, QC(time-only), Tejas Networks |
| Not support | ETRI, Sharp |

| Company | Comments |
| --- | --- |
| Sony | Since legacy signaling allows for cell specific and UE specific, we do not see a reason to stop UE specific signaling. The UE specific signaling can be used to configure flexible symbols to SBFD. |
| ETRI | We propose the following revision: Cell-specific configuration on time and frequency locations of SBFD subbands is supported. |
| Huawei, HiSilicon | Our view is that the most practical SBFD deployment scenario involves configuring identical SBFD subbands for all SBFD-aware UEs within the network. While different UEs with different UL coverage might benefit from individual subband configurations over time, this introduces significant gNB-to-gNB co-channel intra-subband CLI, especially in macro deployments. Implementing UE-specific time-frequency locations for SBFD subbands may be practically infeasible, similar to the limited adoption of UE-specific TDD configurations despite being supported since Rel-15. |
| TCL | We share similar view with Huawei regarding the identicial SBFD subbands for all SBFD aware UEs. In order to achieve this it may not be possible to use UE specific or BWP specific signaling. |
| CEWiT | A cell specific configuration would ensure that a common pattern can be maintained across the cells that will be useful in managing CLI. Further, we do not see the need of configuring subbands in a UE specific manner. Instead, UE specific behaviors can be defined if flexibility is required for Tx/Rx. |
| CT | Support also UE specific similar to current UE-specific TDD configurations, agree ETRI’s version. |
| Spreadtrum | Subband time domain location can be same for all the SBFD-aware UEs. No need to introduce different subband time domain location for different UEs. If different UEs are configured with different time location, it would lead to deversed gNB-gNB CLI condition, which make CLI mitigation more difficult. In addition, if SBFD time region is configured by UE specific signalling, there would be more transmition points due to the slot format. So we prefer cell-common SBFD time location indication. |
| Sharp | At least frequency domain is SCS-specific. Time domain can be UE-specific. |
| NEC | We agree that SBFD subband configuration should be cell specific. Also, although we agreed to support the case of single UL subband per carrier, we need to discuss the case of CA as well. We need to consider that for the case of CA, how many UL subbands can be simultaneously configured to the UE for different cells and the associated UE capability for such an operation. |
| LG | We support the proposal, but the revision from ETRI seems better. |
| Samsung | We support cell-specific for frequency-location. The SBFD UL subband location can be expected fixed as a function of the gNB SIC. Time domain requires more discussion. In DXXXU, even if we have 3 system-wide SBFD slots available, only cell edge UEs requiring UL coverage would be configured with all 3 system-wide SBFD slots. UEs in medium-good SINR coverage benefit from reduced latency with SBFD. 1 out of 3 SBFD slots is sufficient. If we force these UEs to monitor all 3 system-wide SBFD slots for DL/UL assignments, we penalize the UE power consumption of Rel-19 UEs similar to what we observe for Rel-15 DDDDU vs. DFFFU type of frame allocations. |
| QC | We believe this this proposal should be discussed after the details of time/frequency SBFD locations are finalized. In our views, the ‘time’ locations of the SBFD subband should be cell specific (cell-common) as all the SBFD-aware should have the same indication of the SBFD symbols. However, further discussion is required for the semi-static indicaton of ‘frequnecy’ locations of SBFD subbands. We believe that per-BWP UE-dedicated UL SBFD frequency configurations are needed for RRC-connected UE. Some suggestion: - Clarify this indication is for RRC-connected UE. - Focus the discussion on ‘time’ locations at this point. Semi-static indication of time and frequency locations of SBFD subbands is cell-specific for SBFD-aware UE in RRC-connected state. |
| ZTE | We agree that cell-specific configuration should be supported. But it seems no need to preclude UE-specific signalling at this stage. In this sense, the revision from ETRI is safer. |
| Tejas | We support all SBFD UE in a cell be configured with a similar cell specific configuration for time and frequency location of SBFD bands. |

Proposal 1-2

Proposed Agreement:

Semi-static indication of time and frequency locations of SBFD subbands in SIB is supported.

          - FFS interaction with existing TDD slot configuration indications via TDD-UL-DL-ConfigDedicated and SFI in DCI format 2_0

|  | Company |
| --- | --- |
| Support | IDC, Sony,New H3C, Huawei, HiSilicon, OPPO, TCL, CATT , CEWiT, CT, Spreadtrum, NEC, Transsion, Lenovo, LG, Nokia, NSB, Fujitsu, Panasonic, DOCOMO, Tejas Networks |
| Not support | ETRI, Sharp, Samsung, Ericsson |

| Company | Comments |
| --- | --- |
| ETRI | We propose the following revision: Study interaction between time and frequency locations of SBFD subbands indication and existing TDD slot configuration. |
| Huawei, HiSilicon | Using SIB is preferable if the configuration is common across all UEs within a cell. This minimizes the signaling overhead and simplifies gNB management. In addition, if SBFD operation extends to UEs in RRC_IDLE/INACTIVE mode for random access, SIB is the only viable choice. |
| Spreadtrum | The interpretation between SBFD confiugation and existing TDD slot configuration should be discussed. |
| Sharp | Physical layer parameters should be included in SIB1 if those are used in RRC IDLE mode. Since RRC IDLE mode operation is under study, using SIB1 is not justified. Furthermore, SIB1 indication will not reduce signaling overhead because SIB1 is repeated (e.g., in 20 ms) using the lowest possible MCS while RRC reconfiguration message can be sent once per RRC connection establishment using appropriate MCS for each UE. For example, if the number of SBFD aware UEs are less than the legacy UE in the cell, RRC reconfiguration with the SBFD configuration is necessary only for the smaller number of UEs. In this case, broadcasting the SBFD indication using SIB1 with low MCS will increase the overhead. We think this topic is one of the fundamental aspects of SBFD. We don’t prefer delaying this agreement. Rather, we should firstly agree on the semi-static indication of time and frequency locations using UE-specific configuration. |
| Samsung | We are fine to support SIB-based indication if SBFD operation in RRC_IDLE is supported. If SIB-based indication is supported, SIB1 can be used For RRC_CONNECTED mode, a cell common configuration certainly makes sense in combination with some UE dedicated configuration component. The cell common configuration, i.e., ServingCellConfigCommon, is not SIB based even if the IE reuses the same IE that we also use in SI1/SIB1. Even for Rel-15 UEs, SIB-based indication of the TDD UL-DL config is not baseline, so it appears counterintuitive that we would impose such a solution in Rel-19 for SBFD-aware UEs. For FFS bullet, we do not think this bullet is necessary. - According to proposal 2-1, if DL/UL subbands are configured on flexible symbol by tdd common, the flexible symbol is regarded as DL symbol (i.e., same UE behaviors on both DL symbol with subband and flexble symbol with subband). So, tdd dedicated signaling is not need to revert the flexible symbol with subbands to DL symbol/flexible symbol/UL symbol - For dynamic SFI, we fails to see the motivation to support dynamic TDD operation for SBFD-aware UE. |
| Ericsson | It is too early to make this agreement. SIB only needed for random access if RRC_IDLE is supported, if not SIB should be avoided. |
| QC | Similar views as previous comments in regards of the ‘frequency’ indication. For some scenarios, e.g. SBFD SCell addition, the UE may receive the cell-common configuration via RRC signalling. We could add an FFS point for ‘cell-common’ RRC indication. Semi-static indication of time and frequency locations of SBFD subbands in SIB is supported. - FFS cell-specific RRC signalling. - FFS interaction with existing TDD slot configuration indications via TDD-UL-DL-ConfigDedicated and SFI in DCI format 2_0 |
| ZTE | Ok with the main bullet. For FFS, further clarification is needed what should we further study. If it is to let the dedicated configuration or SFI to override the SBFD symbols, we don’t think such behavior should be allowed. In addition, we suggest adding one FFS for interaction with DL/UL active BWP. |
| Tejas Networks | A new SIB-x can help to ensure cell specific configuration and also also ensure legacy UE is not affected. |

Proposal 1-3

Proposed Agreement:

SBFD subband time locations are configured within a period.

          - The period is down-selected from one of the following options.

            - Option 1: The period is TDD-UL-DL pattern period configured by dl-UL-TransmissionPeriodicity in TDD-UL-DL-ConfigCommon.

                - SBFD subbands can only be configured in DL and flexible symbols configured in TDD-UL-DL-ConfigCommon.

                - FFS when two TDD-UL-DL patterns are configured

            - Option 2: The period is integer multiple of TDD-UL-DL pattern period configured by dl-UL-TransmissionPeriodicity in TDD-UL-DL-ConfigCommon.

                - SBFD subbands can only be configured in DL and flexible symbols configured in TDD-UL-DL-ConfigCommon.

                - FFS when two TDD-UL-DL patterns are configured

            - Option 3: The period is configured by a new parameter.

|  | Company |
| --- | --- |
| Support | IDC, Sony, New H3C, ETRI, Huawei, HiSilicon, OPPO, TCL (option 1, and 3), CATT, CEWiT, CT, Spreadtrum, Sharp, Nokia, NSB, NEC (excluding Option-3), Transsion, LG, Samsung, Ericsson, Fujitsu, Panasonic, DOCOMO, QC, ZTE, Tejas Networks |
| Not support |  |

| Company | Comments |
| --- | --- |
| Sony | We are fine with either Option 1 or Option 2. However, we understand the intention is to least the options for this meeting and downselect later. |
| Huawei, HiSilicon | We are fine with either Option 1 or Option 2. From signaling point of view, Option 2 offers a better flexibility and it also covers Option 1. The question is whether there is a necessity, i.e., from operation point of view what Option 2 can achieve while Option 1 cannot? For Option 3, the intention is no clear to us. |
| TCL | We are ok with option 1 and option 3. For option 2, if the SBFD period in the integer multiple of TDD-UL-DL pattern period, there may be a misalignment between the SBFD period and TDD period. For instance, in current specification the lowest TDD period given is 0.5 msec if we multiply it with integers the period will become {1msec, 1.5msec, 2 msec, etc.}, which is not aligned with the current TDD pattern period. Therefore we do not support option 2. |
| CATT | We are open to further discuss Option 2. For Option 3, we are not clear about the intention. |
| Spreadtrum | We prefer Option 1. It is simple and straightforward. |
| NEC | We support to study Option-1 and Option-2 but the motivation for Option-3 is not clear. It should be clear that SBFD period should be an integer multiple of TDD-UL-DL pattern period irrespective of option selected, if that is the case then Option-2 seems to have the same functionality as Option-3 (i.e. defining a new parameter for SBFD period). Hence, we prefer to remove Option-3 from here. |
| LG | We have similar view with Huawei. The motivation of Option 3 is not clear. |
| Samsung | For Option 2, the 1st subbullet should be removed for now, details can be further discussed. |
| Ericsson | We prefer Option 1. |
| QC | We believe option 1 is sufficient. Also, we need to clarify that for both options 1-2, when multiple TDD pattern are configured then the period is sum of P1 and P2. Suggest following changes for further clarity. - Option 1: The period is same as the dl-UL-TransmissionPeriodicity of the TDD-UL-DL-Pattern when one pattern is configured per TDD-UL-DL-ConfigCommon - Option 2: The period is integer multiple of the dl-UL-TransmissionPeriodicity of the TDD-UL-DL-Pattern when one pattern is configured by in TDD-UL-DL-ConfigCommon. |

Proposal 1-4

Proposed Agreement:

A slot can consist of SBFD symbols and non-SBFD symbols.

Symbol-level granularity is supported for semi-static indication of SBFD subband time location.

- referenceSubcarrierSpacing in TDD-UL-DL-ConfigCommon is used as reference SCS.

|  | Company |
| --- | --- |
| Support | IDC, Sony,New H3C, ETRI (support main bullets only), Huawei, HiSilicon, OPPO, TCL, CATT, CEWiT, CT, Spreadtrum, Sharp, Nokia, NSB, NEC, Transsion, LG, Samsung, Ericsson, Fujitsu, Panasonic, DOCOMO, QC, ZTE, Tejas Networks |
| Not support |  |

| Company | Comments |
| --- | --- |

Proposal 1-5

Proposed Agreement:

The subband frequency-domain resources are same across different SBFD symbols.

Frequency location of UL/DL subband is indicated with reference to CRB grid.

- RB-level granularity is supported for semi-static indication of SBFD subband frequency location.

  - FFS starting RB and reference SCS

|  | Company |
| --- | --- |
| Support | Sony,New H3C, Huawei, HiSilicon, OPPO, TCL, CATT, Spreadtrum, Sharp, Nokia, NSB (with update), NEC, Transsion, Samsung, Ericsson, Fujitsu (with update), Panasonic, DOCOMO, ZTE, Tejas Networks |
| Not support | ETRI, CEWiT |

| Company | Comments |
| --- | --- |
| ETRI | We prefer to configure guradbands rather than to configure DL subbands. |
| CEWiT | We prefer to configure guardbands. So, the proposal can be updated only for UL subband. Further, the granularity can be in terms on “N” RBs where “N” can be defined as 1 or more. |
| Nokia, NSB | We agree that CRB grid can be a general reference for frequency location but it is also the reference location that need the largest overhead for signaling the frequency location of the SBFD subband. When reading the SBFD configuration, UE already knows multiple frequency location, e.g. SSB, CORSEST0, etc. As the frequency location SBFD subband is configured as relative distance to the reference, we can see that to save overhead, e.g. frequency location of CORESET0 is not with reference as CRB but as with the reference to the SSB. RAN1 can consider to select a better reference so that to reduce the overhead for the configuration. Thus, we propose to update the proposed agreement as following: Updated Proposed Agreement: The subband frequency-domain resources are same across different SBFD symbols. Frequency location of UL/DL subband is indicated with reference to e.g. CRB grid, SSB, CORESET0. - RB-level granularity is supported for semi-static indication of SBFD subband frequency location. - FFS starting RB and reference SCS |
| NEC | We support this as this follows from the discussion in SI. |
| LG | We are fine with the first and second sentences. However, the second sentence of this proposal seems to conflict with Proposals 1-6. For granularity issue, we need to discuss the granularity of the RB group, so we would like to leave that part as FFS. |
| Samsung | At least signaling perspective, the starting RB and length of subband are not constrained, i.e., a subband can be started from any RB and ended at any RB. But, RAN4 may or may not define the starting RB and length. |
| Ericsson | Additionally, we think that the FD config should be carrier and SCS specific considering multiple logical carriers can be configured in FrequencyInfoDL-SIB. |
| Fujitsu | Considering the 2nd bullet may imply explicit indication of DL subband, which should not be the intention, we suggest the following change. Frequency location of UL/DL subband is indicated with reference to CRB grid. |
| QC | Similar to the discussion in Rel-18 study item, the sizes of the UL/DL subbands and guardband may have some guidance or restriction by RAN1/4, especially the minimum size of the subband and whether a subband size need to match an existing CC BW. - RB-level granularity is supported for semi-static indication of SBFD subband frequency location. - FFS RAN1/4 guidance on the size of the subband/guardband, if any. - FFS starting RB and reference SCS |
| FL | Based on the comments, the proposal is updated as below. Proposal 1-5a Proposed Agreement: The subband frequency-domain resources are same across different SBFD symbols. Frequency location of UL/DL subband, if explicitly indicated, is indicated with reference to CRB grid. - RB-level granularity is supported for semi-static indication of SBFD subband frequency location. - Subject to RAN4 guidance on the size of subband/guardband, if any - FFS starting RB and reference SCS |

Proposal 1-6

Proposed Agreement:

The maximum number of UL subbands for SBFD operation in an SBFD symbol within a TDD carrier is one.

The UL subband can be located at one side of the carrier or can be located at the middle part of the carrier.

For semi-static indication of SBFD subband frequency location, down-select from the following options.

- Option 1: Frequency locations of UL subband and DL subband(s) are explicitly configured. Guardband(s) if any are implicitly derived as the RBs which are not within UL subband or DL subband(s).

- Option 2: Frequency location of UL subband and the number of RBs for guardband(s), if any, are explicitly configured. DL subband(s) are implicitly derived as RBs which are not within UL subband or guardband(s).

|  | Company |
| --- | --- |
| Support | IDC, Sony,New H3C, ETRI, Huawei, HiSilicon, OPPO, TCL, CATT, CEWiT, CT, , Spreadtrum, Sharp, Nokia, NSB, NEC, Transsion, LG, Samsung, Ericsson, Fujitsu, Panasonic, DOCOMO, ZTE, Tejas Networks |
| Not support |  |

| Company | Comments |
| --- | --- |
| Sony | Preference for Option 1 albeit we believe the intention is to list the options for now. |
| New H3C | Support Option 1 |
| Huawei, HiSilicon | We ar fine with this proposal and prefer Option 1. Option 1 is more future proof. If same frequency full duplex is introduced in the future, it can be supported by option 1 by allowing the overlap of UL subband and DL subband. |
| OPPO | We support option 2 to save signaling overhead. |
| TCL | We perfere option 1, but also ok with option 2. |
| CEWiT | We prefer option 2. |
| Spreadtrum | We support Option 1. |
| Sharp | We prefer clarifying what is TDD carrier. We think it represents SCS-specific carrier. |
| NEC | We are okay to study this, but as mentioned in Proposal-1-1 we also need to discuss the case of CA and whether UE can be configured with UL subbands in different carriers. |
| LG | We support Option 1. |
| QC | Further clarify that whether the carrier is serving cell carrier or the UE-dedicated carrier. |
| ZTE | We support Option 1. We don’t see a need to define guardband. |
| Tejas | We support option 1. |

Proposal 2-1

Proposed Agreement:

For SBFD-aware UE transmission, reception and CLI measurement in the SBFD symbols configured in DL and/or flexible in TDD-UL-DL-ConfigCommon,

- UL transmissions within UL subband within active UL BWP are allowed

- DL receptions within DL subband(s) within active DL BWP are allowed

- CLI measurements in DL subband(s), UL subband and/or guardband(s) within active DL BWP are allowed

- UL transmissions outside UL subband are not allowed

- DL receptions outside DL subband(s) are not allowed

|  | Company |
| --- | --- |
| Support | Sony,New H3C, ETRI, TCL, CATT, CT, Spreadtrum, Nokia, NSB, LG, Samsung (except CLI bullet), Ericsson (with conditions), Fujitsu (except CLI bullet), DOCOMO (modification on the 3rd bullet), QC |
| Not support | Huawei, HiSilicon, OPPO, CEWiT, Sharp, NEC (not support the CLI measurements in guard band), Transsion, Tejas Networks |

| Company | Comments |
| --- | --- |
| Huawei, HiSilicon | For CLI measurements, it is not quite clear what it refers to, e.g. gNB-to-gNB CLI measurement or UE-to-UE CLI measurement. It is also not clear what kind of measurement it refers to, e.g. RSRP measurement ot RSSI measurement. Our view is that CLI measurement can be discussed under 9.3.3. |
| OPPO | We support these sub-bullets for SBFD symbols configured in DL, however, for SBFD symbols configured in flexible symbol, there is some relation between proposal 2-1 and the FFS of proposal 1-2 (interaction with existing TDD slot configuration indications via TDD-UL-DL-ConfigDedicated and SFI in DCI format 2_0). So we prefer to separate the discussion of SBFD operation for DL symbol and flexible symbol. |
| CEWiT | Same view as Huawei. CLI measurements can be discussed under 9.3.3. Further, the last 2 bullets are not very clear. A flexible symbol can be configured to be DL/UL. E.g., an F symbol is configured as UL, in that case, will UL be not allowed outside the UL subband? |
| Sharp | The above has an issue in a case of the configured SBFD symbols overlaps with SSB. Considering that in an option, uplink transmission can be dropped if it collides with the SSB, the whole UL subband in the configured SBFD symbols is no longer available for SBFD aware UEs. Therefore, to keep the discussion open, we suggest the following change. For SBFD-aware UE transmission, reception and CLI measurement in the SBFD symbols configured in DL and/or flexible in TDD-UL-DL-ConfigCommon, |
| Nokia, NSB | Our understanding is that details on the CLI measurements scenario, and whether it’s applicable to gNB-to-gNB or UE-to-UE CLI measurements, will be discussed under AI 9.3.3. For CLI measurements in guardband(s), the main advantage would be that the RSSI measured in the guardband would provide to the UE a better estimate of the UE-to-UE CLI than measurements in the DL subband, which contains the gNB transmissions unless the gNB mutes itself during the resources configured for CLI-RSSI (and thus impact the performance). |
| NEC | We don’t understand the motivation why CLI measurements need to be performed on guardband. For CLI measurements we already had good baseline agreement in SI and we should discuss how to downselect between one of the options below: -Method#1: victim UE measures RSSI within DL subband -Method#2: victim UE measures RSRP of aggressor UE within UL subband -Method#3: victim UE measures RSSI within UL subband |
| Lenovo | One question for the first bullet, if the UL subband is within the configured active UL BWP, then anyway the UL transmission in the UL subband is allowed? |
| Samsung | CLI measurement should be discussed separately. It is too premature to decide CLI measurements is allowed in guardband(s) and UL subband. |
| Ericsson | For a UE to be able to measure UE-UE CLI, it is necessary to allow CLI measurement at least in the DL subband. However, it is not clear to us how to handle CLI measurement result from UL subband and guard-bands. For UE-UE CLI measurement, the CLI (UE power leakage) in the DL subband and guard-bands is much smaller compared to the UE tx power in the UL subband (according to the inband emission (IBE) model). Considering it is the CLI in the DL subbands that is of interest forassessment of UE-UE interference level, we don't think the UE should report CLI based on the UL subbands and guard-band (the measurement result will be significantly higer if it is the average across DL subbands, UL subband and guardbands). We would like to have a clarification whether the SBFD-aware UE can receive in the guardband for the CLI measurements if the UE is configiured with DL and UL subbands, and guardbands. We believe that UE-to-UE CLI measurement. We would like to have the following modification for the proposal. Proposed Agreement: For SBFD-aware UE transmission, reception and CLI measurement in the SBFD symbols configured in DL and/or flexible in TDD-UL-DL-ConfigCommon, - UL transmissions within UL subband within active UL BWP are allowed - DL receptions within DL subband(s) within active DL BWP are allowed - CLI measurements in DL subband(s) , UL subband and/or guardband(s) within active DL BWP are allowed - UL transmissions outside UL subband are not allowed - DL receptions outside DL subband(s) are not allowed |
| Fujitsu | UE behaviours of CLI measurement in SBFD symbol depends on which CLI handling scheme is selected. If out-of-subband CLI measurement is not required, CLI measurements in UL subband and guardband(s) may be precluded. |
| DOCOMO | For the third bullet, prefer to put “CLI measurement in UL suband and/or guardband(s) within DL BWP” as FFS. The benefit of CLI measurement in guardband(s) is not very clear. For CLI measurement in UL subband, there are more impact than CLI measurement in DL subband, e.g. when and how to perform CLI measurement in UL subband, possible impact on UE DL reception and UL transmission behavior, etc. Fine with other bullets. |
| ZTE | For CLI measurement, we don’t see clear motivation to do CLI measurement in guardband. We are also ok to wait for the discussion in AI 9.3.3 first. |
| FL | Based on the comments, the proposal is updated as below. Proposal 2-1a Proposed Agreement: For SBFD-aware UE transmission, and reception and CLI measurement in the SBFD symbols configured in DL and/or flexible in TDD-UL-DL-ConfigCommon, - UL transmissions within UL subband within active UL BWP are allowed - Whether or not UL transmission in SSB symbols is allowed is separately discussed - DL receptions within DL subband(s) within active DL BWP are allowed - CLI measurements in DL subband(s), UL subband and/or guardband(s) within active DL BWP are allowed - UL transmissions outside UL subband are not allowed - DL receptions outside DL subband(s) are not allowed CLI measurement behaviours for SBFD-aware UE are discussed in agenda item 9.3.3. |
| Tejas | This need to be clarified with diagram as what the FL expects by this proposed agreement is not clear. |

Proposal 2-2

Proposed Agreement:

For a physical channel/signal with transmission/reception occasion mapped to SBFD and non-SBFD symbols within a slot, SBFD aware UE does not transmit or receive the physical channel/signal within the slot.

|  | Company |
| --- | --- |
| Support | CATT, CEWiT, Spreadtrum, Sharp, NEC, Ericsson, Fujitsu, Panasonic, DOCOMO, QC |
| Not support | Sony, ETRI, Huawei, HiSilicon, TCL , CT, Nokia, NSB, Transsion, Tejas Networks |

| Company | Comments |
| --- | --- |
| Sony | This is too restrictive. There is no reason why this cannot be done if there is no change in Tx parameters across SBFD and non-SBFD symbols. |
| ETRI | We think it is worth to study partial transmission and reception in such cases. |
| Huawei, HiSilicon | We propose to discuss this issue further. For example, for PUSCH, if the transmission occasion is one of the occasions of PUSCH repetition type B, we propose to split the PUSCH transmission into two PUSCH transmission which are within SBFD symbols and non-SBFD symbols respectively. |
| OPPO | We have one clarification question for this proposal, is it intended to cover both dynamically scheduled channel/signal and semi-static configured channel/singnal? We are fine with the proposal for semi-static configured channel/singnal, however, for dynamically scheduled channel/signal, we suppose this can be avoided by gNB appropriate scheduling. |
| CT | We think it can be further studied. |
| Spreadtrum | The FL proposal means SBFD aware UE does not transmit or receive the physical channel/signal if a physical channel/signal with transmission/reception occasion mapped to SBFD and non-SBFD symbols within a slot. The physical channels/signals across SBFD symbols and non-SBFD symbols in different slots is discussed in Proposal 2-3. |
| Nokia, NSB | We prefer to further discuss and find a better solution. Dropping the entire transmission is sub-optimal. |
| NEC | We support this proposal as it might be too much complicated to design a solution where a Tx/Rx occasion is allowed which spans both SBFD and non-SBFD considering the issues like phase continuity, UL power adjustment, etc |
| LG | Basically, we support that a physical channel/signal is not transmitted/received across SBFD and non-SBFD symbols within a slot. However, we think it's too restrictive that a UE does not transmit/receive a physical channel/signal within the slot, if the physical/channel is mapped to both SBFD and non-SBFD symbols within a slot In our view, for PUSCH with repetition type B, if a nominal repetition is mapped to SBFD symbols and non-SBFD symbols, it can be considered that the UE divides the nominal repetition into actual repetitions based on the boundary between SBFD symbols and non-SBFD symbols. Considering that PUSCH repetition type B is used for latency purpose, it is desirable to use as many symbols as possible for PUSCH transmission. |
| Samsung | We understand the intention of this proposal. It is not clear whether the channel/signal is with or without a DCI format, for the latter it should be avoided by gNB implementation for unexpected UE behaviour. For SRS, the transmission is determined per symbol, it should be separately discussed. For Type-B PUSCH repetition, it should be separately discussed. |
| QC | Tx/Rx across SBFD and non-SBFD symbols is corner case optimization that require many conditions. We should save RAN1 time to discuss other important issues. |
| ZTE | We can deprioritize this issue at this stage and focus on the case on different slots first. |
| Tejas | This needs further study. |

Proposal 2-3

Proposed Agreement:

For physical channels/signals across SBFD symbols and non-SBFD symbols in different slots, where each transmission/reception within a slot has either all SBFD or all non-SBFD symbols, the transmissions/receptions can be in SBFD symbols and non-SBFD symbols.

- FFS physical channels/signals across SBFD symbols and non-SBFD symbols in a same slot, where each transmission/reception occasion has either all SBFD or all non-SBFD symbols

|  | Company |
| --- | --- |
| Support | IDC, Sony (clarification needed), New H3C, CATT, CEWiT, Spreadtrum, Nokia, NSB, Transsion, Samsung, Ericsson (with condition) , Fujitsu, DOCOMO, ZTE,Tejas Networks |
| Not support | Sharp, LG |

| Company | Comments |
| --- | --- |
| Sony | FFS is not clear, - The 1st part said physical channel/signals cross SBFD & non-SBFD symbols in a same slot and - The 2nd part then said the transmission/reception occasion is all SBFD and non-SBFD symbols. 1st part seems to contradict 2nd part. |
| Huawei, HiSilicon | Some clarification is needed. It seems that the main bullet covers quite a lot of cases, e.g. signals/channels without repetition, signals/channels with repetition. It is not clear what we are trying to agree here. |
| CT | The FFS sub-bullet needs clarification same as sony’s view. |
| Spreadtrum | The main bullet covers PDSCH/PUSCH/PUCCH repetitions, TBoMS and other signals/channels without repetition, such as SPS PDSCH/CG PUSCH, etc. The FFS part means each transmission/reception occasion has either all SBFD or all non-SBFD symbols in case of same slot. It is same as main bullet and we support it. The FFS part also can be removed. |
| Sharp | We prefer discussing channel by channel. |
| NEC | Rather than making a general agreement it might be preferable to discuss this for each type of channel/signal being considered. It might be possible that we may identify that for a dynamic transmission (PDSCH/PUSCH) we might want to restrict its transmission only to SBFD or non-SBFD slots depending on its resource allocation (e.g. when bandwidth of PDSCH/PUSCH is greater than UL subband size). |
| LG | Rather than applying this proposal for all cases, we think we should discuss the possibility of applying the following two options depending on cases. For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols) - Option 1: The transmissions/receptions are restricted to SBFD symbols only or non-SBFD symbols only - Option 2: The transmissions/receptions can be in SBFD symbols and non-SBFD symbols Depending on the signal/channel, different options may be applied. For example, Option 2 is applied for PUSCH and PUCH with repetitions, but Option 1 is applied for CSI-RS, and SRS. In addition, it is also possible to discuss the possibility of applying Option 1 or Option 2 depending on the capabilities and circumstances of the UE/gNB. |
| Ericsson | In our view the FFS in the proposed agreement is dependent on Proposal 2-2. The FFS should be reformulated as Note mentioned below if Proposal 2-2 is agreed as following: Note: physical channels/signals across SBFD symbols and non-SBFD symbols in a same slot, where each transmission/reception occasion has either all SBFD or all non-SBFD symbols is not supported. |
| QC | Proposal is not clear. |

Proposal 2-4

Proposed Agreement:

Support enhancement on frequency domain RA type 1 for PDSCH across two DL subbands in SBFD symbols for both interleaved and non-interleaved VRB-to-PRB mapping.

- FFS details

|  | Company |
| --- | --- |
| Support | IDC, Sony,New H3C, Huawei, HiSilicon, OPPO, TCL, CATT, CEWiT, CT, Sharp, Nokia, NSB, NEC, Transsion, Ericsson, Fujitsu, Panasonic, DOCOMO, ZTE, Tejas Networks |
| Not support | Spreadtrum |

| Company | Comments |
| --- | --- |
| IDC | Simple and common solution for other cases (e.g., repetition case) is preferred, which can be based on a rate matching behavior around not valid resource outside the DL subbands. With this, the legacy RA type 1 can be reused as is, and the UE applies the rate matching behavior on the not valid resource outside the DL subbands, which should be sufficient. |
| Spreadtrum | Basically, we support enhancement on frequency domain RA type 1 for PDSCH across two DL subbands in SBFD symbols. However, the interleaved VRB-to-PRB mapping case has not been discussed. We would like to check whether it has new problems or not. So we think interleaved VRB-to-PRB mapping should be FFS. |
| Samsung | There is the similar problem when one DL subband is configured. For example, consider DXXXU (X={D,U}) and DL BWP fully include a DL subband. Since VRB-to-PRB interleved mapping is performed based on DL BWP, the PRBs may overlap with UL subband. |
| QC | It is better to first consider/study the different options for FDRA Type-1 RA and then decide or down-select similar to the summary listed in section 3.2.2.1 For frequency domain RA type 1 enhancement for PDSCH across two DL subbands in SBFD symbols for both interleaved and non-interleaved VRB-to-PRB mapping, RAN1 to study/consider the following options: - Option 1: Rate matching is performed for PDSCH, including its DM-RS, around the UL subband and guardband(s) if any. - Support: Spreadtrum, vivo, CMCC, CATT, xiaomi, Sharp, DCM (exclude RBs outside DL subband(s)), LGE(RM or puncturing), Lenovo (non-interleaved), CEWiT (discard RBs outside DL subband(s)), QC - Option 2: New RB indexing within RBs in DL subbands only, and legacy VRB-to-PRB mapping is used - Support: CT, Nokia, MTK, Lenovo (interleaved) - Option 3: Legacy RB indexing, and modified interleaver for VRB-to-PRB mapping - Support: Nokia - Option 4: Legacy RB indexing, and modified VRB-to-PRB mapping rule such that one VRB bundle is mapped to two PRB bundles to allow “mirror image” FDRA - Support: Nokia, Sony - Option 5: warpped-around index to ensure allocated PRB within DL subbands - Support: Nokia - Option 6: Enhancing the frequency resource indication method - Support: LGE |

Proposal 2-5

Proposed Conclusion:

SBFD-aware UEs does not expect a CORESET and a search space are configured that the MOs of the search space occur in both SBFD and non-SBFD symbols and the associated CORESET overlaps the boundary of a DL subband in SBFD symbols.

          - No enhancements on PDCCH

|  | Company |
| --- | --- |
| Support | Huawei, HiSilicon, OPPO, CATT, Spreadtrum, Sharp, Transsion, LG, Ericsson, Fujitsu |
| Not support | Sony, ETRI, TCL, CT, Nokia, NSB, NEC, Lenovo, Samsung, DOCMO |

| Company | Comments |
| --- | --- |
| Sony | UE can be configured with 3 CORESETs per BWP, UE may exceed this limit if multiple configurations are required for the same CORESET. |
| TCL | It may restric the gNB flexibility. |
| Spreadtrum | From our perspective, CORESET bitmap configuration has already sufficient flexibility. When PDCCH monitoring occasions crossing SBFD and non-SBFD symbols in different slots, it can be guaranteed by PDCCH monitoring occasion and CORESET configuration to never overlap with UL subband in SBFD symbols. The FL proposal means no enhancements on PDCCH, instead of increase the CORESET number per BWP. |
| Nokia, NSB | We think that leaving the issue to NW configuration is sub-optimal in the sense that we can leverage the full flexibility in non-SBFD symbols. Additionally that will also make a significant restriction of the resource allocation for CORESET and search space, resulting significant impact to performance of SBFD. |
| NEC | The wording in the proposal “No enhancements on PDCCH” is too early. Based on this proposal the CORESET may only be constrained to one DL subband bandwidth which reduces the overall PDCCH capacity especially when UL subband bandwidth is comparable to the DL subband bandwidth. Further, it is not clear that how does CCE-to-REG mapping is expected to work for the case when interleaving is performed. Note that interleaving is essential for SBFD to ensure good diversity in presence of CLI from UL subbands. Therefore, we think PDCCH enhancement is needed as pointed in our tdoc, and we do not support this proposal. Further more, we also need to consider the case when the subband/guardband boundary is not aligned with REG (6 RB) boundary, where some of the RBs may not be usable for CORESET in one DL subband. |
| Lenovo | We are not fine with the bullet. PDCCH repetition across SBFD and non-SBFD slots should be considered, which is aligned with the objective “Enhancements on physical channels/signals and procedure across SBFD symbols and non-SBFD symbols in different slots, where each transmission/reception within a slot has either all SBFD or all non-SBFD symbols”. |
| Samsung | No need to add such a RRC configuration restriction. This restriction may result in two CORESET configurations (one for DL-only symbol another for SBFD symbol) for SBFD-aware UE. We strive to make a common/unified UE behaivor for a channel configured by higher layers (including SPS/CG, and repetitions). This is just to drop if an occasion of the channel overlaps with invalid resource. This is also applicable to PDCCH, i.e., If a PDCCH candicate overlap with UL subband/guardband(s), UE is not required to monitor the PDCCH candidate. |
| Panasonic | If CORESET is not allowed to overlap the boundary of a DL subband in SBFD symbols, as Sony mentioned, the flexibility of CORESET configuration could be limited because only 3 CORESETs can be configured. |
| QC | A simpler version of the proposal is: - No PDCCH enhancements for SBFD operation in Rel-19 |
| ZTE | It is better to explicitly clarify the proposal is only for RRC connected mode. Depending on the detailed design on subband locations, we are not sure whether it is feasible to avoid the overlap between CORESET 0 and subband boundary. For instance, if the initial BWP equals the size of CORESET 0, it seems not able to avoid the overlap. |

Proposal 2-6

Proposed Agreement:

For SBFD-aware UEs, collisions between DL reception in DL subband(s) and UL transmission in UL subband in a SBFD symbol may be addressed or alleviated with proper scheduling. The following cases of potential collisions can be further studied to see if any change to the current specs is necessary:

          - Case 1: Dynamically scheduled DL reception vs. semi-statically configured UL transmission

            - e.g., dynamic PDSCH or CSI-RS collides with configured SRS, PUCCH, or CG PUSCH

          - Case 2: Semi-statically configured DL reception vs. dynamically scheduled UL transmission

            - e.g., PDCCH or SPS PDSCH collides with dynamic PUSCH or PUCCH

          - Case 3: Semi-statically configured DL reception vs. semi-statically configured UL transmission

          - Case 4: Dynamically scheduled DL reception vs. dynamic scheduled UL transmission

          - Case 5: Configured SSB vs. dynamically scheduled or configured UL transmission

            - e.g., PUSCH, PUCCH, PRACH, SRS

          - Case 6: Dynamic or semi-static DL vs. valid RO

          - Case 7: Collision due to direction switching

|  | Company |
| --- | --- |
| Support | Sony,New H3C, ETRI, Huawei, HiSilicon, OPPO, TCL, CATT, CT, Spreadtrum, Nokia, NSB (with update), NEC, Transsion, Samsung (except Case 7), Ericsson, Fujitsu (need clarification on Case 7), Panasonic, DOCOMO (comment on case 6) |
| Not support | Sharp |

| Company | Comments |
| --- | --- |
| Moderator | I drafted the proposal following the previous agreements for Rel-17 RedCap HD-FDD below. Agreements: (RAN1#104-e) - For HD-FDD operation for RedCap UEs, collisions may be addressed or alleviated with proper scheduling. The following cases of potential collisions can be further studied to see if any change to the current specs is necessary: - Case 1: Dynamically scheduled DL reception vs. semi-statically configured UL transmission - e.g., dynamic PDSCH or CSI-RS collides with configured SRS, PUCCH, or CG PUSCH - Case 2: Semi-statically configured DL reception vs. dynamically scheduled UL transmission - e.g., PDCCH or SPS PDSCH collides with dynamic PUSCH or PUCCH - Case 3: Semi-statically configured DL reception vs. semi-statically configured UL transmission - Case 4: Dynamically scheduled DL reception vs. dynamic scheduled UL transmission - Case 5: Configured SSB vs. dynamically scheduled or configured UL transmission - e.g., PUSCH, PUCCH, PRACH, SRS - Case 8: Dynamic or semi-static DL vs. valid RO - Case 9: Collision due to direction switching. |
| Sharp | We think all the above cases other than Case 5 are specified in the existing specification. Case 5 is also in the legacy specification. However, under the WI assumption that UL subband can be used for UL only, if SSB overlaps with the UL subband, the UL subband cannot be used for any uplink transmission for SBFD aware UEs. To avoid this resource inefficiency, invalidating the UL subband should be discussed. |
| Nokia, NSB | Generally we think it should be fine. While the way to mitigate/avoid collision by scheudilng may impact on the flexibility and performance of scheduling. Thus we think may be we can remove the “For SBFD-aware UEs, collisions between DL reception in DL subband(s) and UL transmission in UL subband in a SBFD symbol may be addressed or alleviated with proper scheduling.” Or we may need to update as “For SBFD-aware UEs, collisions between DL reception in DL subband(s) and UL transmission in UL subband in a SBFD symbol may be addressed or alleviated with proper scheduling, with some impact on flexibility/performance of scheduling in SBFD.” |
| Samsung | It is unclear to me that the motivation to include direction switching. Is it clear from the text of TS38.211 (see below) since the UE is still half-duplex? A UE not capable of full-duplex communication is not expected to transmit in the uplink earlier than NRx-TxTc after the end of the last received downlink symbol in the same cell where NRx-Tx is given by Table 4.3.2-3. A UE not capable of full-duplex communication is not expected to receive in the downlink earlier than NTx-RxTc after the end of the last transmitted uplink symbol in the same cell where NTx-Rx is given by Table 4.3.2-3. |
| DOCOMO | Not very sure about case 6. For PRACH, shall we discuss it in AI 9.3.1 or AI 9.3.2? |
| QC | It is okay to list the different collision scenarios at this point and then decide when case should be specified, if needed. |
| ZTE | It seems the proposal is only for intra-UE collision. We suggest inter-UE collision handling should also be discussed. For instance, whether the UL transmission in SBFD symbols can be cancelled by UL CI. |

Proposal 2-7

Proposed Agreement:

If PRG is determined as wideband, non-contiguous frequency resources across two DL subbands but contiguous frequency resource within each DL subband can be allocated.

- SBFD-aware UE makes no assumption of same precoding across two DL subbands

- FFS conditions for wideband precoder determination when precoding bundling is determined dynamically

|  | Company |
| --- | --- |
| Support | Sony,New H3C, ETRI, Huawei, HiSilicon, OPPO, TCL, CATT, CT, Spreadtrum, Sharp, NEC, Transsion, Ericsson, Fujitsu, Panasonic, DOCOMO, Tejas Networks |
| Not support | QC |

| Company | Comments |
| --- | --- |
| ETRI | Support option 2-2, which is aligned with proposal 2-7. |
| LG | We are fine for the main bullet, but further discussion seems necessary for the first sub-bullet. |
| QC | We do have strong concerns on this proposal. Unfortunately, our views didn’t change since Rel-18 study and we are not ready to relax current restrictions of wideband precoding to have only contiguous PRBs. |
| ZTE | It’s better to first discuss how to address the concerns on UE complexity. If no simple solutions, we prefer not allow such allocation. |

Proposal 2-8

Proposed Agreement:

For the frequency resource allocation for CSI-RS across downlink subbands for SBFD-aware UEs, down-select from the following options.

- Option 2-1: One CSI-RS resource with non-contiguous resource allocation

- Option 2-2: One CSI-RS resource with contiguous resource allocation. Non-contiguous CSI-RS resource is derived by excluding frequency resources outside DL subband(s).

|  | Company |
| --- | --- |
| Support | IDC, Sony,New H3C, Huawei, HiSilicon, OPPO, TCL, CATT, CEWiT, CT, Spreadtrum, Sharp, Nokia, NSB, NEC, Transsion, Lenovo (after including Option 2-3 below), LG, Ericsson, Fujitsu, Panasonic, DOCOMO, ZTE, Tejas Networks |
| Not support |  |

| Company | Comments |
| --- | --- |
| Moderator | I intentionally used the same numbering for the options as in SI with some rewording without intention to change the meaning. |
| Huawei, HiSilicon | Option 2-1 is prefered. It offers better flexibility than Option 2-2 as the CSI-RS resources can be positioned anywhere within the subbands. |
| Lenovo | @Moderator. As you have captured in section 3.2.2.1 above, we have proposed another option for discussion, i.e., Option 2-3, but it is not listed in the Proposal 2-8. - Option 2-3: One contiguous CSI-RS resource allocation with contiguous CSI-RS resource derived by wrapping-around the frequency resources over the two DL subbands. An illustrative example of Option 2-3 is given by the following figure. Like Option 2-2, Option 2-3 can reuse the existing signalling design for CSI-RS resource configuration. However, unlike Option 2-2, Option 2-3 can limit or even avoid completely excluding part of CSI-RS frequency resources. Figure 5: Modified Option 2-2 for frequency resource allocation for a CSI-RS resource across two DL subbands for SBFD-aware UEs: One contiguous CSI-RS resource allocation with contiguous CSI-RS resource derived by wrapping-around the frequency resources over the two DL subbands [34]. |
| QC | For either option, there is extra UE complexity on processing non-contigous CSI-RS resources. RAN1 should further investigate UE complexity and timeline consideration. Impact on UE complexity and CSI processing timeline |

Proposal 2-9

Proposed Agreement:

For an RBG that overlaps the subband boundary for SBFD-aware UEs,

- The part of the DL RBG inside the DL subband can be used for DL reception. The part of the DL RBG outside the DL subband cannot be used for DL reception.

- The part of the UL RBG inside the UL subband can be used for UL transmission. The part of the UL RBG outside the UL subband cannot be used for UL transmission.

- FFS whether partial RBG as existing partial RBG at BWP edges is defined for this case

- FFS whether TBS is calculated with or without the part of the RBG that cannot be used

|  | Company |
| --- | --- |
| Support | IDC, Sony,New H3C, Huawei, HiSilicon, OPPO, TCL, CATT , CEWiT, CT, Spreadtrum, Sharp, Nokia, NSB, NEC, Transsion, Ericsson, Fujitsu, Panasonic, DOCOMO, ZTE, Tejas Networks |
| Not support |  |

| Company | Comments |
| --- | --- |
| Huawei, HiSilicon | We are fine with this proposal. On the last FFS bullet, our understanding is that in current specification, TBS is calculated without the part of the RBG that cannot be used. |
| QC | Further clarity on the objectives of the two FFS points are needed. Is the intention the determination of the #PRBs? |

Proposal 2-10

Proposed Agreement:

For a PRG that overlaps the subband boundary, the part of the DL PRG inside the DL subband can be used for SBFD-aware UEs.

|  | Company |
| --- | --- |
| Support | IDC, Sony,New H3C, ETRI, Huawei, HiSilicon, OPPO, TCL, CATT, CT, Spreadtrum, Sharp, Nokia, NSB, NEC, Transsion, Ericsson, Fujitsu, DOCOMO, ZTE, Tejas Networks |
| Not support | QC |

| Company | Comments |
| --- | --- |
| QC | We do have strong concerns on this proposal. Unfortunately, our views didn’t change since Rel-18 study and we are not ready to add additional PRGs on top of the two partial PRGs at the BWP edges. |

Proposal 2-11

Proposed Agreement:

For a CSI-RS resource which overlaps with SBFD subband boundaries, only CSI-RS resources within DL subband(s) are valid for SBFD-aware UEs.

For a CSI reporting subband which overlaps with SBFD subband boundaries, CSI report is derived based on CSI-RS resources excluding CSI-RS resources outside DL subband(s) for SBFD-aware UEs.

|  | Company |
| --- | --- |
| Support | Sony,New H3C, ETRI, Huawei, HiSilicon, TCL, CATT, CEWiT, CT, Spreadtrum, Sharp, Nokia, NSB, NEC, Transsion, LG, Ericsson, Fujitsu, DOCOMO, QC, ZTE, Tejas Networks |
| Not support |  |

| Company | Comments |
| --- | --- |
| Sony | I think we may need to also consider CSI subband (i.e. CQI subband) that overlaps non DL subband. |
| ETRI | Similar agreeements on CSI reference resource are required. |
| NEC | We agree with this proposal but we also need to advance the discussion on CSI reporting framework (options for CSI report associated with periodic/semi-persistent CSI-RS in case the periodicity is such that CSI-RS instances occur in both SBFD and non-SBFD symbols) which is somehow missing from this summary. |

- Contact person

Please provide/update the information of the contact person in the following table to facilitate the discussions.

| Company | Name | Email address |
| --- | --- | --- |
| CATT | Yanping Xing | xingyanping@catt.cn |
| InterDigital | Jonghyun Park | jonghyun.park@interdigital.com |
| New H3C | Lei Zhou | Zhou.leih@h3c.com |
| ETRI | Hoondong Noh | hoondong.noh@etri.re.kr |
| ETRI | Cheolsoon Kim | cs.kim@etri.re.kr |
| ETRI | Junhyeong Kim | jhkim41jf@etri.re.kr |
| Huawei | Xinghua Song Zhiheng Guo | songxinghua@huawei.com guozhiheng@huawei.com |
| TCL | Shahid Jan | shahid.jan@tcl.com |
| CEWiT | Priyanka Dey | priyanka@cewit.org.in |
| CT | Nanxi Li | linanxi@chinatelecom.cn |
| Spreadtrum | Zhongdan Zhang | Zhongdan.Zhang@unisoc.com |
| Spreadtrum | Huan Zhou | Huan.Zhou@unisoc.com |
| Sharp | Tomoki Yoshimura | yoshimurat@sharplabs.com |
| Nokia, NSB | Jingyuan Sun | jingyuan.sun@nokia-sbell.com |
| Nokia, NSB | Quang Nhan | nhat-quang.nhan@nokia.com |
| Transsion | Xingya Shen | xingya.shen@transsion.com |
| LG | Hyunsoo Ko | hyunsoo.ko@lge.com |
| LG | Hyangsun You | sssun.you@lge.com |
| Panasonic | Tomoya Nunome | nunome.tomoya@jp.panasonic.com |
| Panasonic | Hidetoshi Suzuki | suzuki.hidetoshi@jp.panasonic.com |
| DOCOMO | Qiping Pi Hiroki Harada | piqp@docomolabs-beijing.com.cn hiroki.harada.sv@nttdocomo.com |
| Qualcomm | Muhammad Abdelghaffar | mabdelgh@qti.qualcomm.com |
| ZTE | Xianghui Han | han.xianghui@zte.com.cn |

- Reference

- TR 38.858	Study on Evolution of NR Duplex Operation		Rapporteur (CMCC)	V18.0.0

- RP-234035	New WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)	CMCC (Moderator, RAN1 VC)

- R1-2400053		Discussion on SBFD TX/RX/measurement procedures	Spreadtrum Communications

- R1-2400108		On subband full duplex design	Huawei, HiSilicon

- R1-2400154		Discussion for SBFD TX/RX/measurement procedures	New H3C Technologies Co., Ltd.

- R1-2400177		Discussion on SBFD TX/RX/measurement procedures	Tejas Network Limited

- R1-2400207		Discussion on SBFD operation	Transsion Holdings

- R1-2400241		Discussion on Rel-19 SBFD operation	vivo

- R1-2400269		Discussion on SBFD TX/RX/measurement procedures	TCL

- R1-2400300		Discussion on transmission, reception and measurement procedures for SBFD operation	ZTE

- R1-2400325		Discussion on SBFD TX/RX/measurement procedures	CMCC

- R1-2400432		Discussion on SBFD TX/RX/measurement procedures	CATT

- R1-2400557		Discussion on reception and transmission procedure for SBFD operation	xiaomi

- R1-2400606		Discussion on SBFD Tx/Rx/measurement procedures		OPPO

- R1-2400643		SBFD procedures in RRC_CONNECTED mode	Sharp

- R1-2400660		Discussion on SBFD TX/RX/measurement procedures	China Telecom

- R1-2400687		Discussion on SBFD TX RX and measurement procedures	InterDigital, Inc.

- R1-2400731		SBFD operation and procedures	Samsung

- R1-2400775		Discussion on Tx/Rx/Measurement procedures for SBFD operation	Fujitsu

- R1-2400802		Discussion on subband non-overlapping full duplex Tx-Rx and measurement operations	NEC

- R1-2400826		Discussion on SBFD TX/RX/measurement procedures	Panasonic

- R1-2400838		On SBFD TX/RX/measurement procedures	Nokia, Nokia Shanghai Bell

- R1-2400852		SBFD operations	Sony

- R1-2401011		Views on SBFD TX/RX/measurement procedures	Apple

- R1-2401081		On semi-static indication of SBFD resources		Google Inc.

- R1-2401116		Discussion on SBFD TX/RX/measurement procedures	NTT DOCOMO, INC.

- R1-2401159		Considerations on SBFD Tx/Rx/measurement procedure	KT Corp.

- R1-2401188		Discussion on SBFD TX/RX/measurement procedures	ITRI

- R1-2401191		Tx Rx procedure for SBFD	ASUSTeK

- R1-2401214		Discussion on SBFD TX/RX/measurement procedures	WILUS

- R1-2401217		SBFD TX/RX/measurement procedures	Ericsson

- R1-2401227		Discussion on SBFD TX/RX/measurement procedures	ETRI

- R1-2401249		Discussion on SBFD TX/RX/measurement procedures	LG Electronics

- R1-2401263		Views on sub-band full duplex operations	Lenovo

- R1-2401273		Discussion on SBFD TX/RX/ measurement procedures	CEWiT

- R1-2401294		Discussion on SBFD TX/RX/measurement procedures	MediaTek Inc.

- R1-2401440		SBFD Transmission, Reception and Measurement Procedures	Qualcomm Incorporated

Appendix A: Rel-18 SI agreements

RAN1#109-e

Agreement

Study whether/how to inform the UE of the time and/or frequency location of subbands that gNB would use for SBFD operation.

Agreement

Study the impact/potential enhancements of resource allocation in symbols with subbands that gNB would use for SBFD operation.

Agreement

At least study SBFD operation within a TDD carrier

Conclusion

For discussion purpose only, SBFD symbols is defined as symbols with subbands that gNB would use for SBFD operation.

Conclusion

For discussion purpose, for SBFD operation within a TDD carrier, a SBFD subband consists of 1 RB or a set of consecutive RBs for the same transmission direction.

Agreement

The time and frequency location of subbands within a TDD carrier are not fixed in the specification.

- Subject to any RAN4 guidance on minimum or maximum subband and guardband size and subband location within TDD carrier.

- Note that whether the time and/or frequency location of subbands are informed to UE is separately discussed.

Guideline for future meetings

- Note: AI 9.3.3 handles the potential inter-gNB and inter-UE CLI handling schemes that are specific for dynamic TDD and schemes that are common for both SBFD and dynamic/flexible TDD.

- Note: AI 9.3.2 handles the potential inter-gNB and inter-UE CLI handling schemes that are specific for SBFD.

RAN1#110

Agreement

Study the following alternatives with Alt 4 prioritized, for SBFD operation at least for RRC_CONNECTED state.

- SBFD operation Alt 1:

  - Time and frequency locations of subbands for SBFD operation are not known to UEs.

  - UE behaviors follow existing specifications without introducing new UE behaviors for SBFD operation at gNB side.

- SBFD operation Alt 2:

  - Time and frequency locations of subbands for SBFD operation are not known to UEs.

  - UE behaviors for non-SBFD aware UEs follow existing specifications.

  - From RAN1 perspective, new UE behaviors can be introduced for SBFD aware UEs

- SBFD operation Alt 3:

  - Only time location of subbands for SBFD operation is known to SBFD aware UEs.

  - UE behaviors for non-SBFD aware UEs follow existing specifications.

  - From RAN1 perspective, new UE behaviors can be introduced for SBFD aware UEs based on the time location of subbands for SBFD operation

- SBFD operation Alt 4:

  - Both time and frequency locations of subbands for SBFD operation are known to SBFD aware UEs.

  - UE behaviors for non-SBFD aware UEs follow existing specifications.

  - From RAN1 perspective, new UE behaviors can be introduced for SBFD aware UEs based on the time and frequency locations of subbands for SBFD operation.

UE capability discussion is held in work item phase.

Agreement

For indication of subband locations for SBFD operation, study semi-static configuration of subband time and frequency location as baseline.

Agreement

For semi-static configuration of subband location, consider same subband frequency resources across different SBFD symbols as baseline.

Working Assumption

For SBFD operation within a TDD carrier, study SBFD scheme within a single configured DL and UL BWP pair with aligned center frequencies as baseline.

- FFS feasibility and potential benefit of SBFD scheme within a single configured DL and UL BWP pair with unaligned center frequencies

- FFS feasibility and potential benefit of SBFD scheme with more than one configured DL and UL BWP pair with aligned/unaligned center frequencies for a DL and UL BWP pair

Agreement

For SBFD operation Alt 4, for an SBFD aware UE configured with an UL subband in an SBFD symbol, study the following options:

- Option 1: The SBFD aware UE does not expect to be scheduled with UL transmission outside the UL subband or to be scheduled with DL reception within the UL subband in the SBFD symbol

- Option 2: The SBFD aware UE does not expect to be scheduled with UL transmission outside the UL subband and may be scheduled with DL reception within the UL subband in the SBFD symbol

- Option 3: The SBFD aware UE does not expect to be scheduled with DL reception within the UL subband and may be scheduled with UL transmission outside the UL subband in the SBFD symbol

- Option 4: The SBFD aware UE may be scheduled with UL transmission outside the UL subband or DL reception within the UL subband in the SBFD symbol

Agreement

Study the feasibility and potential benefit of UE-to-UE co-channel CLI measurement and reporting, which can be specific for SBFD, at least includes:

- Measurement resource/reporting configuration

- Measurement/reporting details (including UE processing delay)

- Relevant information exchange (between gNBs) if needed

- Usage of measurement at gNB

Note: other enhancement(s) for gNB-to-gNB and UE-to-UE CLI handling specific for SBFD are not precluded.

RAN1#110bis-e

Agreement

For SBFD operation at least for RRC_CONNECTED state, it is agreed that SBFD operation Alt 4 is the baseline.

- SBFD operation Alt 4:

  - Both time and frequency locations of subbands for SBFD operation are known to SBFD aware UEs.

  - UE behaviors for non-SBFD aware UEs follow existing specifications.

  - From RAN1 perspective, new UE behaviors can be introduced for SBFD aware UEs based on the time and frequency locations of subbands for SBFD operation.

Agreement

For semi-static configuration of subband frequency locations for SBFD operation, at least explicit indication of frequency location of UL subband is required.

- FFS: Whether frequency location of other subbands types is explicitly indicated or implicitly determined.

Agreement

Study impact and potential enhancements of CSI-RS resource set frequency domain resource allocation and CSI reporting configuration across non-contiguous DL subbands.

Agreement

Identify if there are any cases of time domain conflict of UE’s UL and DL operation in the same SBFD symbol for SBFD aware UE

- If there are, whether/how to avoid/handle such collision cases (as second step)

Agreement

Study impact/potential enhancements for UE-to-UE CLI-RSSI measurement/report considering non-contiguous measurement resource in frequency.

Agreement

Study whether SBFD operation in SSB symbols is supported or not.

Agreement

For SBFD operation within a TDD carrier, it is agreed that SBFD scheme within a single configured DL and UL BWP pair with aligned center frequencies is the baseline.

Agreement

The maximum number of UL subbands for SBFD operation in an SBFD symbol (excluding legacy UL symbol) within a TDD carrier is one for the study in RAN1.

- The UL subband can be located at one side of the carrier.

- The UL subband can be located at the middle part of the carrier

Note: RAN1 considers the above two possibilities unless RAN4 concludes that any one is infeasible.

Note: Two UL subbands for SBFD operation in an SBFD symbol within a TDD carrier due to SBFD operation in legacy UL symbols is subject to further RAN1 discussions which is 2nd priority as per RAN guidance.

Send an LS to RAN4 to inform the above agreement. If RAN4 has response, it will be taken into account but in the meanwhile, RAN1 work will continue based on the above.

LS on maximum number of UL subbands for duplex evolution to RAN4 is endorsed. Final LS in R1-2210671.

Agreement

For semi-static configuration of subband time locations for SBFD operation, it is agreed that explicit configuration of SBFD subband time locations within a period is the baseline.

RAN1#111

Agreement

For a SBFD aware UE semi-statically configured with UL subband in a SBFD symbol configured as DL in TDD-UL-DL-ConfigCommon, the following is agreed as baseline in the RAN1 study:

- UL transmissions within UL subband are allowed in the symbol

- UL transmissions outside UL subband are not allowed in the symbol

- Frequency locations of DL subband(s) are known to the SBFD aware UE

  - The frequency location of DL subband(s) can be explicitly indicated or implicitly derived

- DL receptions within DL subband(s) are allowed in the symbol

- Note: UL transmissions are within active UL BWP and DL receptions are within active DL BWP in the symbol

Agreement

For the purpose of RAN1 study, the understanding is that for semi-static configuration of subband frequency locations for SBFD operation, frequency location of UL/DL subband is with reference to CRB grid.

Agreement

Study impact and potential enhancements for UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols, including at least the following:

- PDCCH, scheduled/configured PUCCH/PUSCH/PDSCH, without repetition in SBFD symbols and non-SBFD symbols

- Scheduled/configured SRS/CSI-RS in SBFD symbols and non-SBFD symbols

- Scheduled/configured TBoMS across SBFD symbols and non-SBFD symbols with or without repetition

- Multi-PUSCH/PDSCH scheduled by a single DCI in SBFD symbols and non-SBFD symbols

- Scheduled/configured PDSCH/PUSCH/PUCCH with repetitions across SBFD symbols and non-SBFD symbols

Note: Inter-slot/intra-slot/inter-repetition/inter-group frequency hopping with DMRS bundling of PUSCH/PUCCH, if applicable, is considered.

Examples of potential enhancements include:

- Resource allocation in frequency domain including frequency hopping

- Resource allocation in time domain

- Power domain

- Spatial domain

FFS: If the PUCCH/PUSCH/PDSCH/PDCCH can be mapped to SBFD and non-SBFD in the same slot if configured.

Agreement

For SBFD operation in a symbol configured as flexible in TDD-UL-DL-ConfigCommon, study the following options for SBFD aware UEs,

Option 1:

- UL transmissions within UL subband are allowed in the symbol

- UL transmissions outside UL subband are not allowed in the symbol

- Frequency locations of DL subband(s) are known to the SBFD aware UE

- DL receptions within DL subband(s) are allowed in the symbol

- FFS: Whether DL receptions outside DL subband(s) are allowed or not in the symbol

Option 2:

- UL transmissions within UL subband are allowed in the symbol

- The RBs outside the UL subband can be used as either UL, or DL excluding guardband(s) if used, in the symbol from gNB’s perspective, and the transmission direction for all those RBs is the same

  - FFS: SBFD aware UE behaviours

  - FFS: Whether or not signalling of guardband(s) is needed

- FFS: Whether or not the symbol can be converted to a DL-only symbol

- Frequency locations of DL subband(s) are known to the SBFD aware UE

- DL receptions within DL subband(s) are allowed in the symbol

Note: UL transmissions are within active UL BWP and DL receptions are within active DL BWP in the symbol for both options. For all RBs outside the UL subband, UE cannot use separate RBs for DL and UL simultaneously.

Agreement

Study the impact and benefits of potential enhancements to resource allocation in frequency-domain for SBFD operation, considering unaligned boundaries between resource block group(s)/reporting subband(s) and SBFD subbands, including at least the following:

- RBG for PDSCH RA type 0

- CSI reporting configuration

- CSI-RS resource configuration

- PRG of PDSCH

RAN1#112

Agreement

For dynamic SBFD,

  - For SBFD-aware UEs, further study whether DL receptions outside semi-statically configured DL subband(s) are allowed or not in a symbol configured as DL in TDD-UL-DL-ConfigCommon based on the following options:

    - Option 1 (semi-static): DL receptions outside semi-statically configured DL subband(s) are not allowed

    - Option 2: DL receptions outside semi-statically configured DL subband(s) are allowed

  - For SBFD-aware UEs, further study whether DL receptions outside semi-statically configured DL subband(s) and UL transmissions outside semi-statically configured UL subband are allowed or not in the symbol configured as flexible in TDD-UL-DL-ConfigCommon based on the following options:

    - Option 1 (semi-static): DL receptions outside semi-statically configured DL subband(s) are not allowed and UL transmissions outside semi-statically configured UL subband are not allowed

    - Option 2: DL receptions outside semi-statically configured DL subband(s) are allowed

      - UL transmissions outside the semi-statically configured UL subbands are not allowed

    - Option 3: DL receptions outside semi-statically configured DL subband(s) are allowed

      - UL transmissions outside the semi-statically configured UL subbands are allowed

Dynamic SBFD should be compared with dynamic TDD and/or semi-static SBFD in terms of performance, implementation complexity, switching latency.

For each option, additional conditions may apply to determine whether the option is applicable.

Agreement

Study whether or not a slot can consist of both SBFD and non-SBFD symbols including

  - Benefits

  - Use cases

  - Scheduling flexibility

  - Implementation complexity

  - Compatibility with legacy TDD DL/UL configuration

Agreement

For inter-UE inter-subband CLI measurement, study at least the following methods:

  - Method#1: victim UE measures RSSI within DL subband

    - FFS: Whether SINR can be measured

  - Method#2: victim UE measures RSRP of aggressor UE within UL subband

  - Method#3: victim UE measures RSSI within UL subband

  - Note: the restriction in Rel-16 that CLI is only measured within DL BWP does not forbid UE to measure CLI in UL subband when UL subband is confined within DL BWP.

Agreement

For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols)

  - Study the following options for SBFD-aware UEs:

    - Option 1: The transmissions/receptions are restricted to SBFD symbols only or non-SBFD symbols only

    - Option 2: The transmissions/receptions can be in SBFD symbols and non-SBFD symbols

  - UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols include the following:

    - PDSCH/PUSCH/PUCCH repetitions

    - SPS PDSCH/CG PUSCH

    - TBoMS

    - Multi-PUSCH/PDSCH scheduled by a single DCI

    - Periodic/semi-persistent SRS/CSI-RS/PUCCH

    - PDCCH

Agreement

For SBFD-aware UEs, study the at least following options for resource allocation in frequency-domain in case of unaligned boundaries between RBG and SBFD subbands. For an RBG that overlaps the subband boundary,

  - Option 1:

    - Part of the DL RBG inside the DL subband can be used

    - Part of the UL RBG inside the UL subband can be used

  - Option 2:

    - Part of the DL RBG inside the DL subband cannot be used

    - Part of the UL RBG inside the UL subband cannot be used

FFS: The part of the RBG outside.

Agreement

For SBFD-aware UEs, study at least the following issues for PDSCH:

  - PRG(s) with size of 2 and 4 that overlaps with subband boundary

  - Wideband precoder in case of non-contiguous DL subbands

Agreement:

Study the frequency resource allocation for CSI-RS across downlink subbands for SBFD-aware UEs considering the following options:

  - Option 1: Two contiguous CSI-RS resources that are linked

  - Option 2: One CSI-RS resource

    - Option 2-1: Non-contiguous CSI-RS resource allocation

    - Option 2-2: One contiguous CSI-RS resource allocation with non-contiguous CSI-RS resource derived by excluding frequency resources outside DL subband (s)

Agreement:

For SBFD-aware UEs, study the following options for CSI report associated with periodic/semi-persistent CSI-RS, at least, across SBFD symbols and non-SBFD symbols in different slots (each CSI-RS resource within a slot has either all SBFD or all non-SBFD symbols):

  - Option 1: separate CSI reporting for SBFD symbols and non-SBFD symbols

  - Option 2: same CSI reporting for SBFD symbols and non-SBFD symbols

Agreement:

Study at least the followings for SRS, PUCCH and PUSCH on SBFD symbols and non-SBFD symbols in different slots:

  - Whether/how to have separate resources

  - Whether/how to have separate FH parameters

  - Whether/how to have separate UL power control parameters

  - Whether/how to have separate beam/spatial relation

RAN1#112bis-e

Conclusion

The following RAN1 observation is made:

One motivation for allowing that a slot can consist of both SBFD and non-SBFD symbols is for compatibility with symbol-level TDD UL/DL configuration.

Frequent switching between SBFD and non-SBFD symbols may increase the implementation complexity and interruptions of transmissions/receptions during transition.

- Further study whether limitation(s) on the maximum number of switching points between SBFD and non-SBFD symbols within a slot, a TDD UL/DL pattern period, and/or semi-static SBFD configuration period (if different from TDD UL/DL pattern period) are needed

- Further study scenarios a guard period between SBFD and non-SBFD symbols is required/not required and the length of the guard period if required

Note: Whether or not a physical channel/signal occasion is mapped to both SBFD and non-SBFD symbols within a slot is a separate discussion.

Agreement

At least for semi-static SBFD, the following two options are viable solutions for frequency location configuration of DL subband(s) and guardband(s) if any.

- Option 1: Frequency locations of DL subband(s) are explicitly configured. Guardband(s) if any are implicitly derived as the RBs which are not within UL subband or DL subband(s).

- Option 2: The number of RBs for guardband(s), if any, is explicitly configured. DL subband(s) are implicitly derived as RBs which are not within UL subband or guardband(s).

Agreement

If PRG is determined as wideband, study the following two options:

- Option 1: non-contiguous frequency resources across two DL subbands but contiguous frequency resource within each DL subband can be allocated

    - FFS: Precoding assumption within and across the two DL subbands

- Option 2: non-contiguous frequency resources across two DL subbands cannot be allocated

The study should include the impact on UE complexity

Agreement

For UE-to-UE CLI-RSSI measurement/report across downlink subbands, study the following methods:

- Method#1: separate CLI-RSSI measurement resources/reports in each DL subband

    - Note: supported in existing specifications

- Method#2: CLI-RSSI measure/report in one DL subband only

    - Note: supported in existing specifications

- Method#3: CLI-RSSI measurement/report based on non-contiguous CLI-RSSI resource across downlink subbands

    - FFS: report single or separate CLI-RSSI report(s)

    - FFS: details on determination of non-contiguous CLI-RSSI resource allocation

Agreement

Endorse the text proposal in R1-2303639 for the TR with the following update.

6.1.1.3 SBFD operation in symbols configured as flexible in TDD-UL-DL-ConfigCommon For SBFD operation in a symbol configured as flexible in TDD-UL-DL-ConfigCommon, the following optionsalternatives are studied for SBFD aware UEs, OptionAlt 1: - UL transmissions within UL subband are allowed in the symbol - UL transmissions outside UL subband are not allowed in the symbol - Frequency locations of DL subband(s) are known to the SBFD aware UE - DL receptions within DL subband(s) are allowed in the symbol - FFS: Whether DL receptions outside DL subband(s) are allowed or not in the symbol OptionAlt 2: - UL transmissions within UL subband are allowed in the symbol - The RBs outside the UL subband can be used as either UL, or DL excluding guardband(s) if used, in the symbol from gNB’s perspective, and the transmission direction for all those RBs is the same - FFS: SBFD aware UE behaviours - FFS: Whether or not signalling of guardband(s) is needed - FFS: Whether or not the symbol can be converted to a DL-only symbol - Frequency locations of DL subband(s) are known to the SBFD aware UE - DL receptions within DL subband(s) are allowed in the symbol

Agreement

For SBFD-aware UEs, Option 1 with update is agreed for resource allocation in frequency-domain in case of unaligned boundaries between RBG and SBFD subbands for better resource utilization.

For an RBG that overlaps the subband boundary,

  - Option 1 (with update):

    - The Part of the DL RBG inside the DL subband can be used

    - The Part of the UL RBG inside the UL subband can be used

Agreement

For semi-static SBFD, a SBFD aware UE does not transmit UL channels/signals or receive DL channels/signals on the guardband(s) that the UE is aware of.

  - FFS: Measurement in guardband for the purpose of CLI measurement

Agreement

  - For semi-static SBFD, for a CSI-RS resource which overlaps with SBFD subband boundaries, only CSI-RS resources within DL subband(s) are valid for SBFD-aware UE.

  - For semi-static SBFD, for a CSI reporting subband which overlaps with SBFD subband boundaries, CSI report is derived based on CSI-RS resources excluding CSI-RS resources outside DL subband(s).

Conclusion

For the two options agreed in RAN1#112 for UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols), the following observations are agreed.

- Option 1 can be achieved by gNB configuration or scheduling to ensure that all transmission/reception occasions are confined to either SBFD symbols or non-SBFD symbols. Alternatively, Option 1 can be achieved by additional indication or rules to determine the transmission/reception occasions are valid within one symbol type and are invalid within the other symbol type.

- The frequency resources, power control and beam/spatial relation for all the transmission/reception occasions can be the same for Option 1 but may be different for Option 2. If different, it may require additional specification efforts.

- Option 1 may or may not increase the transmission/reception latency if the transmission/reception in the other symbol type is postponed and may degrade the performance if the transmission/reception in the other symbol type is dropped. Option 2 may or may not reduce the transmission/reception latency and improve coverage.

Agreement

For inter-UE inter-subband CLI measurement, study Method#2 and Method#3 considering:

- Necessity/benefit compared with measurement within DL subband

- Whether/how to estimate CLI from RSRP/RSSI measurements within UL subband / guardband

- Whether UE is required to measure RSRP/RSSI within UL subband and receive DL in DL subband(s) simultaneously

- Whether existing CLI measurement and report framework can be reused to support RSRP/RSSI measurements within UL subband

  - If not, identify the potential impact

Conclusion

Time misalignment at gNB between UL receptions and DL transmissions due to configuration of non-zero NTA,offset at UE can lead to increased interference assuming no gNB transmit chain side impairments and no filtering of DL subband(s) in the gNB Rx chain.

- FFS the case with gNB transmit chain impairments and/or filtering of DL subband(s) in the gNB Rx chain

- FFS whether/how to mitigate the interference increase, including impact to legacy UEs

Agreement

Study the following options for SBFD operation in SSB symbols.

- Option 1: UL subband cannot be configured in an SSB symbol

  - FFS handling of misaligned periodicities between SSB and semi-static SBFD subband time location configuration

- Option 2: An UL subband can be configured in an SSB symbol

  - FFS whether/when and/or under which conditions an SBFD-aware UE transmits in the UL subband or may receive SSB in the symbol.

Agreement

Study whether the transmission/reception occasion of a physical channel/signal can be mapped to SBFD and non-SBFD symbols within a slot for a UE, and whether a UE can transmit/receive in the occasion mapped to SBFD symbols and non-SBFD symbols including:

- Use-case(s) including the locations and number of switching points of the SBFD and non-SBFD symbols in the slot.

- Potential benefits if any

- Phase continuity

- Potential interruption of transmissions/receptions during transition

- Required guard time if any

- Potential impact on performance

- Impact on link adaptation, channel estimation, and other procedures

- UL transmission timing if any

- Implementation complexity

- Applicability for SBFD aware UE and non-SBFD aware UEs

- NOTE: There are more than one scenario where a transmission overlaps SBFD and non-SBFD symbols and some may or may not face the aspects listed above

- NOTE: This study doesn’t mean RAN1 agreement on a slot consisting of SBFD and non-SBFD symbols.

Conclusion

For the options agreed to study in RAN1#112 for frequency resource allocation for CSI-RS across downlink subbands for SBFD-aware UEs, the following observations are agreed.

- For all the options, there is no impact on CSI-RS sequence generation.

- Option 1 requires additional signalling to link two CSI-RS resources in two DL subbands.

- Option 2-1 requires new RRC structure to configure non-contiguous RBs for one CSI-RS resource, which may require additional signalling overhead.

- Option 2-2 can reuse the existing signalling design for CSI-RS resource configuration. Option 2-2 can be used to resolve the potential unaligned boundaries between CSI-RS resource configuration and SBFD subbands

- Further discussion is required on the UE complexity due to:

  - UE capability of maximum number of configured CSI-RS resources

  - Processing non-contiguous CSI-RS

Agreement

For SBFD-aware UEs, study the following options for CSI report associated with periodic/semi-persistent CSI-RS in case the periodicity is such that CSI-RS instances occur in both SBFD and non-SBFD symbols:

- Option 1: two CSI-ReportConfigs, where one is associated with SBFD symbols and the other is associated with non-SBFD symbols

  - Option 1-1: One CSI-ReportConfig is associated with a CSI-RS restricted to SBFD symbols only and the second CSI-ReportConfig is associated with a second CSI-RS restricted to non-SBFD symbols only;

  - Option 1-2: Both CSI-ReportConfigs are associated with the same CSI-RS. The CSI report associated with one CSI-ReportConfig is derived based on CSI-RS instances in SBFD symbols only. The CSI report associated with the second CSI-ReportConfig is derived based on CSI-RS instances in non-SBFD symbols only.

- Option 2: one CSI-ReportConfig associated with both SBFD symbols and non-SBFD symbols

  - Option 2-1: One CSI-ReportConfig is associated with two CSI-RSs which are restricted to SBFD symbols and non-SBFD symbols respectively. Separate CSI measurements are derived based on the first and second CSI-RSs respectively.

  - Option 2-2: One CSI-ReportConfig is associated with one CSI-RS. The CSI report is derived based on CSI-RS which can be in SBFD symbols or non-SBFD symbols in different time instances.

- FFS impact on UE CSI processing and reporting timeline

Note: Whether the CSI-RS resource can be used for SBFD and non-SBFD symbols may depend on, e.g., gNB implementation of same/different antenna configuration in both symbols.

Option 1-1 can be supported according to existing specification by gNB configuration of appropriate periodicities to ensure that the CSI-RS associated with each CSI-ReportConfig is confined to either SBFD symbols or non-SBFD symbols only. But it may restrict the gNB configuration flexibility and enhancements can be considered by additional indication or rules to determine the CSI-RS is valid within one symbol type and is invalid in the other symbol type.

Option 2-2 can be supported according to existing specification to configure measurement restriction so that UE would not average CSI measurements across SBFD and non-SBFD symbols.

Agreement

For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols), if the transmissions/receptions can be in SBFD symbols and non-SBFD symbols with different available resources, study at least the following frequency resource allocation options for PDSCH, CSI-RS, PUSCH, PUCCH, SRS for SBFD-aware UE:

- Option 1: Separate FDRA determination for SBFD slots and non-SBFD slots.

  - Option 1-1: Separate FDRA configurations/indications for SBFD slots and non-SBFD slots

  - Option 1-2: Separate frequency resources determined for SBFD slots and non-SBFD slots based on single FDRA configuration/indication

  - Option 1-3: single FDRA configuration/indication and RB offset(s)

- Option 2: Perform rate matching or puncturing on the RBs outside DL/UL subbands for DL/UL channels/signals.

- Option 3: A DL/UL channel/signal overlapping with RBs outside DL/UL subbands in a SBFD slot is dropped or postponed.

Note: Different options can be studied for different signals/channels.

Agreement

For the case that:

- The monitoring periodicity of a search space is such that different monitoring occasions in different slots occur in SBFD and non-SBFD symbols, respectively, and,

- The associated CORESET overlaps the boundary of a DL subband in SBFD symbols

Consider whether/how the above could be supported considering both existing tools in specifications on CORESET and search space configuration as well as at least the following options for potential enhancement for SBFD-aware UE:

- Option 1: Separate valid resources for the CORESET in SBFD symbols and in non-SBFD symbols.

- Option 2: Rate matching or puncturing on the REG(s) of a PDCCH outside DL subband(s).

- Option 3: UE does not monitor a PDCCH candidate if it is mapped to one or more REs that overlap with REs outside DL subband(s).

- Option 4: Drop search space(s) when the associated CORESET overlaps with RBs outside DL subband(s)

- Option 5: Separate search spaces associated with a CORESET in SBFD and non-SBFD symbols

Note: Whether these enhancements are applicable to only USS or also CSS

RAN1#113

Conclusion

At least for semi-static SBFD, in order to avoid frequent switching between SBFD and non-SBFD symbols, potential limitation on the maximum number of transition points between SBFD and non-SBFD symbols can be considered from SBFD subband configuration perspective. Maximum of two transition points including one transition point from non-SBFD symbols to SBFD symbols and one transition point from SBFD symbols to non-SBFD symbols within a TDD UL/DL pattern period can be considered as a starting point where the transition point can be aligned with slot boundary or within a slot.

- Agreement: The usage of ‘switching point’ in previous conclusions/agreements are revised to ‘transition point’

A guard period between SBFD and non-SBFD symbols may or may not be required at gNB and/or UE side depending on gNB/UE implementation and/or SBFD operation.

Agreement

For the three methods agreed to be studied for UE-to-UE CLI-RSSI measurement/report across downlink subbands, the following observations are agreed.

  - Method #1 allows flexible configuration of measurement reporting in one DL subband or two DL subbands but it consumes multiple CLI-RSSI measurement resources from the UE capability budget.

  - Method #2 restricts gNB configuration flexibility and does not account for whether or not the CLI is asymmetric across two DL subbands. This method does not consume multiple CLI-RSSI measurement resources from UE capability point of view.

  - Method #3 requires additional specification efforts to support non-contiguous CLI-RSSI resource allocation across downlink subbands. This method is similar to non-contiguous CSI-RS resource allocation. A single CLI-RSSI report based on non-contiguous CLI-RSSI resource may be sufficient. This method does not consume multiple CLI-RSSI measurement resources from UE capability point of view.

Note: Above does not imply whether L1 or L2 based measurement is supported.

Conclusion

For a PRG that overlaps with subband boundary, if the part of DL PRG inside the DL subband can be used, better scheduling flexibility and resource utilization can be achieved, however degraded channel estimation quality in the partial PRG is expected compared to a PRG due to limited RBs in the partial PRG.

  - Note: UE complexity could increase if this feature is supported

Agreement

An UL subband can be configured in an SSB symbol.

  - Note: It is SSB from serving cell perspective, which can be CD-SSB or NCD-SSB.

  - Whether actual UL transmission can be done is for further discussion

Agreement

The following conclusion is to be captured in the TR

If PRG is determined as wideband, better scheduling flexibility and higher DL data rate can be achieved if non-contiguous frequency resources across two DL subbands but contiguous frequency resource within each DL subband can be allocated.

Compared to the case that PRG is determined as wideband and only contiguous frequency resources can be allocated, non-contiguous frequency resources across two DL subbands requires UE to handle two non- contiguous segments of contiguous RBs that may increase UE complexity for channel estimation.

Agreement

The following conclusion is to be captured in the TR

gNB can configure a CORESET and a search space in a way such that the MOs of the search space occur in either SBFD or non-SBFD symbols, or the MOs of the search space occur in both SBFD and non-SBFD symbols but the associated CORESET does not overlap the boundary of a DL subband in SBFD symbols.

If it is agreed to be beneficial that a CORESET and a search space are configured that the MOs of the search space occur in both SBFD and non-SBFD symbols and the associated CORESET overlaps the boundary of a DL subband in SBFD symbols, at least the following options can be considered for SBFD-aware UE:

- Option 1: Separate valid resources for the CORESET in SBFD symbols and in non-SBFD symbols.

- Option 2: Rate matching or puncturing on the REG(s) of a PDCCH outside DL subband(s).

- Option 3: UE does not monitor a PDCCH candidate if it is mapped to one or more REs that overlap with REs outside DL subband(s).

- Option 4: Drop search space(s) when the associated CORESET overlaps with RBs outside DL subband(s)

- Option 5: Separate search spaces associated with a CORESET in SBFD and non-SBFD symbols

Note: These options are applicable to at least USS

Agreement

The following conclusion is to be captured in the TR

- For the methods agreed to be studied for inter-UE inter-subband CLI measurement, Method #2 and Method #3 can be used for identifying the aggressor UE(s) if orthogonal resources are allocated for different aggressor UE(s); and Method #2 and #3 can at least provide higher interference signal strength than inter-subband interference leakage based measurements in Method #1. Furthermore, such measurement is not subject to inter-cell DL interference.

- It is feasible for UE to measure RSRP/RSSI within UL subband if within active DL BWP and receive DL in DL subband(s) simultaneously similar as simultaneous RSRP/RSSI measurement and DL reception in Rel-16.

- The existing CLI measurement and report framework can be reused to support RSRP/RSSI measurements within UL subband when UL subband is confined within active DL BWP.

Agreement

The following conclusion is to be captured in the TR

If SBFD-aware UEs are not allowed to transmit in the SSB symbol but is allowed to receive within the DL BWP in the SSB symbol, negative impact on SSB detection and measurement can be avoided but UL performance may be degraded due to fewer UL opportunities.

If SBFD-aware UE is allowed to transmit in the SSB symbol, the UE may only transmit UL in an UL subband depending on gNB scheduling, configuration, UE measurement or priority rule. There may be negative impact on SSB detection and measurement if the SBFD-aware UE is requested to transmit in the SSB symbol.

Agreement

The following conclusion is to be captured in the TR

For a physical channel/signal occasion mapped to SBFD and non-SBFD symbols within a slot if any, the following options for UE transmission/reception can be considered in the normative stage

- Option 1: UE does not transmit or receive the physical channel/signal within the slot.

- Option 2: UE can transmit or receive the physical channel/signal within the slot only under certain conditions.

  - The conditions may depend on at least the following: whether or not phase continuity can be maintained across SBFD and non-SBFD symbols, whether or not there are same or different transmission/reception parameters e.g. power control, spatial/QCL, UL timing etc. applied in SBFD and non-SBFD symbols, and whether or not there is a guard period between the SBFD and non-SBFD symbols, etc.

- Other options are not precluded

Agreement

The following conclusion is to be captured in the TR

For SRS, PUCCH and PUSCH on SBFD symbols and non-SBFD symbols in different slots, it may be beneficial to have separate resources, FH parameters, UL power control parameters and/or beam/spatial relation.

RAN1#114

Agreement

Endorse the text proposal in R1-2308258 for TR 38.858 section 6.

Agreement

Endorse the text proposals in R1-2307333 and R1-2307334 for TR 38.858 in principle with possibility for revision if necessary.

Agreement

The following conclusion is to be captured in the TR 38.858 section 13:

SBFD operation at gNB for UEs was studied under the following assumptions,

  - SBFD operation within a TDD carrier,

  - SBFD scheme within a single configured DL and UL BWP pair with aligned center frequencies, and

  - Up to one UL subband for SBFD operation in an SBFD symbol (excluding legacy UL symbol) within a TDD carrier.

RAN1 concluded SBFD operation Alt 4 is feasible for RRC_CONNECTED state from the RAN1 specification perspective, where SBFD operation Alt 4 assumes

  - Both time and frequency locations of subbands for SBFD operation are known to SBFD aware UEs.

  - UE behaviors for non-SBFD aware UEs follow existing specifications.

  - From RAN1 perspective, new UE behaviors can be introduced for SBFD aware UEs based on the time and frequency locations of subbands for SBFD operation.

Non-SBFD aware UEs, including legacy UEs, and SBFD aware UEs can coexist in cells with SBFD operation at gNB side from RAN1 specification point of view.

To support SBFD operation Alt 4 for RRC_CONNECTED state, RAN1 identified the following potential specification impact for SBFD-aware UE:

  - Indication of time and frequency domain locations of SBFD subbands to UEs

  - UE transmission, reception and measurement behavior and procedures in SBFD symbols and/or non-SBFD symbols

Agreement

The following is to be captured in the TR:

Simulation results from one source [ref] show that the increase of self interference on the UL subband due to misaligned timing between UL reception and DL transmission at the gNB can be quite small (~1dB) when impairments in the gNB transmit chains and filtering of DL subbands in the gNB Rx chains are considered. Filtering that suppresses self interference from DL subbands in the gNB Rx chains could incur some switching time/delay to bypass the filter in UL symbols and could introduce some insertion loss.

Agreement

The following is to be captured in the TR:

The part of the RBG outside the DL subband cannot be used for DL reception and the part of the RBG outside the UL subband cannot be for UL transmission at least for semi-static SBFD.

Agreement

The following is to be captured in the TR section 6.

  - If random access is allowed in SBFD symbols for SBFD-aware UEs, it may potentially reduce the random access latency, reduce the PRACH collision probability and/or improve the coverage of PRACH and Msg3. These aspects were not fully evaluated in RAN1.

  - PRACH and Msg3 transmissions in UL subband in SBFD symbols may cause UE-to-UE CLI. The system performance impact is not evaluated in RAN1.

  - Specification impact is expected to allow random access in SBFD symbols at least for PRACH and Msg3 transmissions in symbols configured as DL in TDD-UL-DL-ConfigCommon.

Agreement

The following is to be captured in the TR in Section 6

  - Compared to semi-static SBFD, dynamic SBFD can better adapt to the UL/DL resource requirements based on UL/DL traffic loads.

  - Dynamic SBFD may increase gNB implementation complexity due to dynamic antenna/panels switching and filters/RF tuning, may incur loss of resources due to transition time, may increase inter-gNB CLI, may increase scheduling complexity, and can result in additional specification impact on top of semi-static SBFD

  - UE implementation complexity may be increased if the UE supports dynamic SBFD and dynamic SBFD may result in increased UE-to-UE CLI

Agreement

The following is to be captured in the TR in Section 6

If dynamic SBFD is supported, the following options can be considered.

    - Option 1: Dynamic SBFD is achieved by scheduling DCI which is used to schedule DL receptions outside semi-statically configured SBFD DL subband and/or UL transmission outside semi-statically configured SBFD UL subband.

    - Option 2: Dynamic SBFD is achieved by non-scheduling DCI which indicates whether a symbol is SBFD symbol or not.

    - Option 3: Dynamic SBFD is achieved by MAC-CE which indicates whether a symbol is SBFD symbol or not.

Note 1: Whether or not dynamic SBFD is beneficial from a performance and complexity perspective is a separate discussion.

Note 2: The possibility of introducing flexible subband type for Option 1 to achieve DL receptions outside semi-statically configured SBFD DL subband and/or UL transmission outside semi-statically configured SBFD UL subband is not precluded.

Note 3: None of the above options imply that there is a dynamic change in the DL/UL subband sizes.

Agreement

- The TP in R1-2308464 is agreed.

- The TP in R1-2308457 is agreed.

- The TP in R1-2308385 is agreed.