# 3GPP TSG RAN WG1 #116																	R1-2401659

# Athens, Greece, February 26th – March 1st, 2024

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.212 | CR | 0175 | rev | - | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on typeA SRS TPC commands for SUL |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core, TEI17 |  |  |  |  |  | Date: |  |  | 2024-02-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In TS 38.212, the following is specified about typeA SRS TPC commands transmitted over DCI format 2_3: The following information is transmitted by means of the DCI format 2_3 with CRC scrambled by TPC-SRS-RNTI: - block number 1, block number 2, …, block number where the starting position of a block is determined by the parameter startingBitOfFormat2-3 or startingBitOfFormat2-3SUL-v1530 provided by higher layers for the UE configured with the block. If the UE is configured with higher layer parameter srs-TPC-PDCCH-Group = typeA for an UL without PUCCH and PUSCH or an UL on which the SRS power control is not tied with PUSCH power control, one block is configured for the UE by higher layers, with the following fields defined for the block: - SRS request – 0 or 2 bits. The presence of this field is according to the definition in Clause 11.4 of [5, TS38.213]. If present, this field is interpreted as defined by Table 7.3.1.1.2-24. - TPC command number 1, TPC command number 2, ..., TPC command number N, where each TPC command applies to a respective UL carrier provided by higher layer parameter cc-IndexInOneCC-Set Additionally, TS 38.331 includes the following field description for the fields indicating the starting bit: startingBitOfFormat2-3 The starting bit position of a block within the group DCI with SRS request fields (optional) and TPC commands. The value 1 of the field corresponds to the first/left most bit of format2-3. The value 2 of the field corresponds to the second bit format2-3, and so on (see TS 38.212 [17], clause 7.3.1 and TS 38.213 [13], clause 11.3).. startingBitOfFormat2-3SUL The starting bit position of a block within the group DCI with SRS request fields (optional) and TPC commands for SUL carrier (see TS 38.212 [17], clause 7.3.1 and TS 38.213 [13], clause 11.3).. The highlighted sentences in TS 38.212 can be seen to be contradictory for the following reasons: - The first sentence states that the starting position of a block is determined by startingBitOfFormat2-3 or startingBitOfFormat2-3SUL-v1530. This, together with the field description in TS 38.331, indicates that two starting positions (and therefore two blocks) are provided, one for the SUL and one for the non-SUL carriers. - The second sentence states that “one block is configured for the UE by higher layers”, which contradicts the previous bullet. |
| Summary of change: |  | Clarify that for typeA SRS TPC command up to two blocks can be indicated by higher layers, one for non-SUL carriers and one for SUL carriers. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification is contradictory. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Impact analysis: This CR only applies to cases where both SUL and non-SUL carriers are power controlled by DCI format 2_3 (e.g. this CR does not affect the case where SUL is not configured) |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

##### 7.3.1.3.4	Format 2_3

DCI format 2_3 is used for the transmission of a group of TPC commands for SRS transmissions by one or more UEs. Along with a TPC command, a SRS request may also be transmitted.

The following information is transmitted by means of the DCI format 2_3 with CRC scrambled by TPC-SRS-RNTI:

-	block number 1, block number 2, …, block number

where the starting position of a block is determined by the parameter startingBitOfFormat2-3 or startingBitOfFormat2-3SUL-v1530 provided by higher layers for the UE configured with the block.

If the UE is configured with higher layer parameter srs-TPC-PDCCH-Group = typeA for an UL without PUCCH and PUSCH or an UL on which the SRS power control is not tied with PUSCH power control, one or two blocks are configured for the UE by higher layers where one block applies to non-SUL carriers and another block applies to SUL carriers, with the following fields defined for each block:

-	SRS request – 0 or 2 bits. The presence of this field is according to the definition in Clause 11.4 of [5, TS38.213]. If present, this field is interpreted as defined by Table 7.3.1.1.2-24.

-	TPC command number 1, TPC command number 2, ..., TPC command number N, where each TPC command applies to a respective UL carrier provided by higher layer parameter cc-IndexInOneCC-Set

< Unchanged parts are omitted >