3GPP TSG RAN WG1 #116			R1-2401697

Athens, Greece, February 26th – March 1st, 2024

Agenda item:	7.1

Source: 	Moderator (Qualcomm)

Title: 	FL summary on follow up discussion on CSI feedback with SUL

Document for:	Discussion/Decision

# Introduction

This document summarizes the discussion regarding Clarification on SUL under Agenda 7.1. Relevant contributions are following.

Follow up discussion on CSI feedback with SUL from RAN1#114bis:

| R1-2400682 | Continuation discussion on clarifications on SUL | Nokia, Nokia Shanghai Bell |
| --- | --- | --- |
| R1-2401285 | On common PUCCH/PUSCH configuration and SUL | MediaTek Inc. |
| R1-2401286 | [R16] Draft 38.212 CR on common PUCCH/PUSCH configuration and SUL | MediaTek Inc. |
| R1-2401287 | [R17] Draft 38.212 CR on common PUCCH/PUSCH configuration and SUL | MediaTek Inc. |
| R1-2401288 | [R18] Draft 38.212 CR on common PUCCH/PUSCH configuration and SUL | MediaTek Inc. |
| R1-2401388 | Discussion on uplink carrier determination for SUL without UE dedicated configuration | Huawei, HiSilicon |
| R1-2401399 | Draft CR on PUCCH carrier determination for SUL | Huawei, HiSilicon |
| R1-2401400 | Draft CR on PUSCH carrier determination for SUL | Huawei, HiSilicon |
| R1-2401408 | Clarification on SUL | Qualcomm Incorporated |

# Companies’ proposals

Following table summarizes the proposals by companies.

| Contribution | Proposals |
| --- | --- |
| 0682 (Nokia) | Observation 1: For PUSCH operation with SUL, there is no need for a specification change for the case that “the UL/SUL indicator is present in DCI format 0_0 and the higher layer parameter pusch-Config is not configured on both UL and SUL” and this case includes the sub-cases of pusch-Config being configured on (i) either UL or SUL and (ii) neither UL nor SUL. Proposed (potential) RAN1 conclusion: If a UE is configured with supplementaryUplink in ServingCellConfig in the cell and the higher layer parameter pusch-Config is not configured on both UL and SUL, TS 38.212 is specified such that the higher layer parameter pucch-Config is provided on either UL or SUL AND either - the higher layer parameter pusch-Config is provided on the carrier (UL or SUL) with the pucch-Config, or - the higher layer parameter pusch-ConfigCommon is provided at least for the carrier (UL or SUL) with the pucch-Config. Observation 2: If a clarification on the UL carrier for PUCCH transmission is seen as needed, Nokia would be supporting Option 2 of Proposal 3-rev1, i.e. - If a UE is configured with supplementaryUplink in ServingCellConfig in the cell, RAN1 spec is specified such that higher layer parameter pucch-Config is provided on either UL or SUL |
| 1285, 1286, 1287, 1288 (MediaTek) | Observation 1: In RAN1 #114-bis, the following way forward is drawn for the discussion about [1]: For RAN1#115: Continue discussion of the following case in RAN1#115: - UL/SUL indicator is not presented in DCI 0_0/0_1 or it is presented but ignored by a UE, pucch-Config is not configured, PUSCH-Config is configured on either UL or SUL but not on both, and PUCCH-ConfigCommon is configured. - Sub-case 1: PUCCH-ConfigCommon is only configured on one carrier, either on SUL or UL. - Sub-case 2: PUCCH-ConfigCommon is configured on both SUL and UL. Observation 2: For this issue, there was no consensus achieved in RAN1 #115 and the discussion may continue in RAN1 #116. Observation 3: In 38.212 [2], for the related spec on the relation between SUL indicator and PUSCH configuration of DCI 1_1/1_2, the text “configured for PUSCH transmission” should include the configuration of pusch-Config or pusch-ConfigCommon. Observation 4: For the discussion in [1], one main point is about the lack of definition for SUL indicator in 0_0/0_1/0_2 when pucch-ConfigCommon is provided or not. Observation 5: Following the logic of Observation 2, we think the spec text “pucch-Config is configured” in 38.212 for DCI 0_0 actually means “pucch-Config or pucch-ConfigCommon is configured”, and the spec text “pusch-Config is configured” in 38.212 for DCI 0_0 actually means “pusch-Config or pusch-ConfigCommon is configured”. Proposal 1: RAN1 to draw the following conclusion: - For the 38.212 spec on the relation between SUL indicator and PUSCH configuration of DCI 1_1/1_2, the text “configured for PUSCH transmission” include the configuration of pusch-Config or pusch-ConfigCommon. Proposal 2: Adopt the following R16 CR (and its R17/R18 mirror CR) to 38.212 [1] 7.3.1.1.1 about SUL indicator and PUSCH/PUCCH configuration for DCI 0_0, to reflect the inference in Observation 5: 7.3.1.1 DCI formats for scheduling of PUSCH 7.3.1.1.1 Format 0_0 … - UL/SUL indicator – 1 bit for UEs configured with supplementaryUplink in ServingCellConfig in the cell as defined in Table 7.3.1.1.1-1 and the number of bits for DCI format 1_0 before padding is larger than the number of bits for DCI format 0_0 before padding; 0 bit otherwise. The UL/SUL indicator, if present, locates in the last bit position of DCI format 0_0, after the padding bit(s). - If the UL/SUL indicator is present in DCI format 0_0 and both the higher layer parameters pusch-Config and pusch-ConfigCommon isare not configured on both UL and SUL the UE ignores the UL/SUL indicator field in DCI format 0_0, and the corresponding PUSCH scheduled by the DCI format 0_0 is for the UL or SUL for which high layer parameter pucch-Config or pucch-ConfigCommon is configured; - If the UL/SUL indicator is not present in DCI format 0_0 and pucch-Config or pucch-ConfigCommon is configured, the corresponding PUSCH scheduled by the DCI format 0_0 is for the UL or SUL for which high layer parameter pucch-Config or pucch-ConfigCommon is configured. - If the UL/SUL indicator is not present in DCI format 0_0 and both pucch-Config and pucch-ConfigCommon isare not configured, the corresponding PUSCH scheduled by the DCI format 0_0 is for the uplink on which the latest PRACH is transmitted. … Observation 6: In RAN1 #112bis-e, a related issue was discussed with the following RAN1 conclusion achieved: Conclusion If a UE is configured with NUL and SUL carrier in a serving cell, and pusch-config is configured, then the UL carrier configured with pucch-config, if any, should also be configured with pusch-config. Proposal 3: Adopt the below update to the RAN1 #112bis-e conclusion following the logic of Observation 2: Conclusion If a UE is configured with NUL and SUL carrier in a serving cell, and pusch-config or pusch-ConfigCommon is configured, then the UL carrier configured with pucch-config or pucch-ConfigCommon, if any, should also be configured with pusch-config or pusch-ConfigCommon. Observation 7: With the update to the RAN1 #112bis-e conclusion as in Proposal 3, combined with Proposal 2, the issue mentioned in [1] is resolved except for the gap period (ambiguous period) illustrated in [3]. Observation 8: The gap period (ambiguous period) illustrated in [3] can happen during, for example, initial access, that gNB does not configure dedicated pucch-Config right after RACH and rely on PUCCH-ConfigCommon for PUCCH feedback, as shown in Figure 1. During this period, it could occur that only PUCCH-ConfigCommon is configured on both UL and SUL, and current specification does not define UE behaviour on PUSCH/PUCCH transmission with SUL. Proposal 4: Further revise Proposal 2 by adopting below R16 CR (and its R17/R18 mirror CR) to 38.212 [1] 7.3.1.1.1, to address the problem of PUSCH/PUCCH transmission with SUL in Observation 7: 7.3.1.1 DCI formats for scheduling of PUSCH 7.3.1.1.1 Format 0_0 … - UL/SUL indicator – 1 bit for UEs configured with supplementaryUplink in ServingCellConfig in the cell as defined in Table 7.3.1.1.1-1 and the number of bits for DCI format 1_0 before padding is larger than the number of bits for DCI format 0_0 before padding; 0 bit otherwise. The UL/SUL indicator, if present, locates in the last bit position of DCI format 0_0, after the padding bit(s). - If the UL/SUL indicator is present in DCI format 0_0 and both the higher layer parameters pusch-Config and pusch-ConfigCommon isare not configured on both UL and SUL the UE ignores the UL/SUL indicator field in DCI format 0_0, and the corresponding PUSCH scheduled by the DCI format 0_0 is for the UL or SUL for which high layer parameter pucch-Config or pucch-ConfigCommon is configured; - If the UL/SUL indicator is not present in DCI format 0_0 and pucch-Config or pucch-ConfigCommon is configured, the corresponding PUSCH scheduled by the DCI format 0_0 is for the UL or SUL for which high layer parameter pucch-Config or pucch-ConfigCommon is configured. - If the UL/SUL indicator is not present in DCI format 0_0 and both pucch-Config and pucch-ConfigCommon isare not configured, the corresponding PUSCH scheduled by the DCI format 0_0 is for the uplink on which the latest PRACH is transmitted. - If the UL/SUL indicator is not present in DCI format 0_0 and pucch-Config or pucch-ConfigCommon is configured on both UL and SUL, the corresponding PUSCH scheduled by the DCI format 0_0 is for the uplink on which the latest PRACH is transmitted. - If the UL/SUL indicator is not present in DCI format 0_0 and pucch-Config or pucch-ConfigCommon is configured on both UL and SUL, the PUCCH for this cell is transmitted on the uplink where the latest PRACH is transmitted. … |
| 1388, 1389, 1390 (Huawei) | Observation 1: For all features including SUL, a UE state#2 is inevitable and presents in real network where the UE state#2 refers to a state after random access procedure but before UE dedicated parameter is configured. - The UE state#2 cannot be avoided by a gNB at least for the RRCReestablishment procedure. - The UE state#2 also occurs when UE dedicated parameters are not included in the Msg4 PDSCH at the end of random access procedure. - The UE state#2 also occurs when a UE completes a CBRA procedure in connected mode that is triggered by RAN2 events, e.g. TA expiration or SR fallback event. In this case, Msg4 message is a Msg4 PDCCH rather than a Msg4 PDSCH. Observation 2: For SUL, before random access, a UE has received cell-specific parameter PUCCH-ConfigCommon for both SUL and NUL via SIB1 because HARQ feedback for Msg4 PDSCH requires PUCCH resources. - A gNB has to broadcast PUCCH-ConfigCommon for both SUL and NUL because which uplink is used for random access procedure is selected by the UE instead of the gNB Observation 3: For SUL, after UE dedicated parameter PUCCH-Config is configured on only one of two uplinks, it is clear that PUCCH can be transmitted only on the uplink configured with PUCCH-Config, although the UE still has cell-specific parameters PUCCH-ConfigCommon for both SUL and NUL via SIB1. Observation 4: In the UE state#2, considering both SUL and NUL are configured with cell-specific parameters pucch-ConfigCommon, whether SUL or NUL is used for PUCCH transmission is unclear. Observation 5: To exit from the UE state#2, a PDSCH carrying the UE dedicated pucch-Config has to be scheduled to the UE but it is unclear in case of SUL which uplink should be used to send HARQ ACK/NCK feedback for the PDSCH. Observation 6: Because the RRC configurations in both the UE state#1 and UE state#2 are the same, a simple solution is to reuse the UE behavior specified in UE state#1, i.e. the uplink used for PRACH transmission during the random access procedure is used for PUCCH transmission in UE state#2. Proposal 1: For a cell configured with SUL, if a UE is not configured with pucch-Config yet after its completion of random access procedure on the cell, then the uplink used for PRACH transmission during the random access procedure is used for PUCCH transmission on the cell if any, which reuses the existing UE behavior of PUCCH carrier determination of random access procedure for SUL. - Note: the uplink carrier is expected to be already configured with cell specific pucch-ConfigCommon. Proposal 2: For PUCCH carrier determination in case of SUL, the CR in [2] for TS 38.213 is adopted. Proposal 3: For a cell configured with SUL, if a UE is not configured with pusch-Config yet after its completion of random access procedure on the cell, then the uplink used for PRACH transmission during the random access procedure is used for PUSCH transmission on the cell if any, which reuses the existing UE behavior of PUSCH carrier determination of random access procedure for SUL. - Note: the uplink carrier is expected to be already configured with cell specific pusch-ConfigCommon. Proposal 4: For PUSCH carrier determination in case of SUL, the CR in [3] for TS 38.212 is adopted. |
| 1408 (Qualcomm) | Proposal 1: Clarify following: If the higher layer parameter pucch-Config is provided on neither NUL nor SUL of the serving cell, and if the higher layer parameter pucch-ConfigCommon is provided on both NUL and SUL of the serving cell, the PUCCH transmission is on NUL or SUL of the serving cell where the latest PRACH is transmitted If the clarification is not achieved in RAN1#116, companies may assume the following consequence: If the higher layer parameter pucch-Config is provided on neither NUL nor SUL of the serving cell, and if the higher layer parameter pucch-ConfigCommon is provided on both NUL and SUL of the serving cell, the PUCCH transmission on NUL or SUL of the serving cell is up to UE implementation Proposal 2: Clarify following: If the UL/SUL indicator is present in DCI format 0_0 and the higher layer parameter pusch-Config is configured on neither UL nor SUL, the UE ignores the UL/SUL indicator field in DCI format 0_0, and the corresponding PUSCH scheduled by the DCI format 0_0 is for the UL or SUL for which high layer parameter pucch-Config is configured If the higher layer parameter pucch-Config is provided on neither NUL nor SUL of the serving cell, and if the higher layer parameter pucch-ConfigCommon is provided on both NUL and SUL of the serving cell or the higher layer parameter pucch-ConfigCommon is provided on neither NUL nor SUL of the serving cell, the PUSCH transmission is on NUL or SUL of the serving cell where the latest PRACH is transmitted If the higher layer parameter pucch-Config is provided on neither NUL nor SUL of the serving cell, and if the higher layer parameter pucch-ConfigCommon is provided on either NUL or SUL of the serving cell, the PUSCH transmission on NUL or SUL of the serving cell where the pucch-ConfigCommon is provided If the clarification is not achieved in RAN1#116, companies may assume the following consequence: If the UL/SUL indicator is present in DCI format 0_0 and the higher layer parameter pusch-Config is configured on neither UL nor SUL, the UE ignores the UL/SUL indicator field in DCI format 0_0, and the corresponding PUSCH scheduled by the DCI format 0_0 is for the UL or SUL for which high layer parameter pucch-Config is configured If the higher layer parameter pucch-Config is provided on neither NUL nor SUL of the serving cell, the PUSCH transmission on NUL or SUL of the serving cell is up to UE implementation Proposal 3: Clarify following: UL/SUL indicator field is present in the DCI format regardless of whether NUL and/or SUL of the serving cell has multiple BWP configurations. BWP indicator field is present in the DCI format if UL/SUL indicator field indicates a UL carrier that has multiple BWP configurations, and is not present in the DCI format, otherwise. |

# 1st round

## PUCCH transmission when pucch-Config is provided on neither NUL nor SUL

Q3-1-1: Are you OK to discuss in RAN1#116 whether/how to clarify PUCCH transmission when pucch-Config is provided on neither NUL nor SUL? If no, please provide your comment on what the current spec specifies.

| Company | View |
| --- | --- |
| Spreadtrum | Yes. The current spec also includes this case: If a UE does not have dedicated PUCCH resource configuration, provided by PUCCH-ResourceSet in PUCCH-Config, a PUCCH resource set is provided by pucch-ResourceCommon through an index to a row of Table 9.2.1-1 for transmission of HARQ-ACK information on PUCCH in an initial UL BWP of NBWPsize PRBs.. |
| ZTE | No, please see our detailed comments in the follow-up question. |
| vivo | Open to discuss, but we think clarifications or spec changes are not necessary. |
| Samsung | No, we believe there is no need to discuss this issue further. |
| New H3C | Yes, current spec already mentioned this. |
| Nokia/NSB | We don’t see a need for any specs change or clarifications (aligned with vivo). Therefore, we think we should not continue the discussion (as ZTE & Samsung). |
| NEC | OK to discuss. At least to collect companies views to see if there are some different understanding of current TS. |
| MTK | Yes. |

Q3-1-2: The observation 2 of R1-2400682 would be straightforward. Do you agree that, if a UE is configured with supplementaryUplink in ServingCellConfig in the cell, RAN1 spec is specified such that higher layer parameter pucch-Config is provided on either UL or SUL? If no, please provide comment on how the spec is written.

| Company | View |
| --- | --- |
| Spreadtrum | Yes |
| ZTE | We agree with the observation: “if a UE is configured with supplementaryUplink in ServingCellConfig in the cell, RAN1 spec is specified such that higher layer parameter pucch-Config is provided on either UL or SUL”. |
| Vivo | Yes. For RRC connected mode, NW should provide pucch-config to specify the UL carrier for UL transmissions. |
| Samsung | Yes. If a UE is configured with supplementaryUplink in ServingCellConfig in the cell, RAN1 spec is specified such that higher layer parameter parameter pucch-Config is provided on either UL or SUL. Pucch-Config PUCCH configuration for one BWP of the normal UL or SUL of a serving cell. If the UE is configured with SUL, the network configures PUCCH only on the BWPs of one of the uplinks (normal UL or SUL). The network configures PUCCH-Config at least on non-initial BWP(s) for SpCell and on all BWP(s) for PUCCH Scell. |
| New H3C | Yes |
| Nokia/NSB | Yes / agree with observation |
| NEC | At least for Pcell, yes. |
| MTK | Yes |

Q3-1-3: R1-2401388 points out that there are cases where a UE has to transmit PUCCH before dedicated RRC configuration such as supplementaryUplink in ServingCellConfig with pucch-Config is provided after the random access procedure completion, and for these cases, where to transmit PUCCH is unclear. Do you agree with the concern? If no, please provide why you think there is no concern on this.

| Company | View |
| --- | --- |
| Spreadtrum | Open to discuss |
| ZTE | We don’t think this is an issue. The similar ambiguity exists since Rel-15 even for the case without SUL configuration. For example, if UE receives pucch-configCommon in SIB1 and receives pucch-config in RRC ReConfiguration, UE needs to transmit PUCCH carrying HARQ-ACK for the PDSCH carrying the RRC ReConfiguration, UE is not clear whether to transmit PUCCH according to pucch-configCommon or pucch-config. Besides, there some other implementation methods to address this issue, e.g., 1) carrying PUCCH-config in msg4; 2) all these signalling transmitted between msg4 and RRC Reconfiguration, there will be high layer follow up confirmation signalling, e.g., RRC Reconfiguration Complete message. |
| Vivo | No. For this phase(e.g., reestablishment phase), the UE can determine which carrier to use for UL transmission. Usually, it opts for the carrier used for PRACH transmission, while the network could conduct blind detection on both UL carriers. Additionally, reestablishment occurrences are rare. |
| Samsung | We share the similar view to ZTE. We still find it challenging to identify the motivation to define the UE operation of UE to resolve the proposed ambiguity. |
| New H3C | Share similar view with ZTE |
| NEC | From pure TS’s perspective, we think it is not very clear on using which uplink to transmit PUCCH. However, from implementation’s perspective, there seems to be no issue. We are open on whether to clarify it. |
| MTK | Yes. Better to clarify. |

Q3-1-4: When a UE is NOT configured with supplementaryUplink in ServingCellConfig with pucch-Config in the cell and has to transmit PUCCH on either NUL or SUL after random access procedure, there are some proposals including potential solutions, CR, clarification, conclusion, in R1-2401285, R1-2401388, and R1-2401408. The common ground among them is that the UE transmits PUCCH on either NUL or SUL whichever the latest PRACH is transmitted. Do you agree with this understanding? If no, please provide your alternative.

| Company | View |
| --- | --- |
| Spreadtrum | Support “UE transmits PUCCH on either NUL or SUL whichever the latest PRACH is transmitted” |
| ZTE | We assume the question is target to the case: - UE is configured with supplementaryUplink in ServingCellConfig, and - UE is not configured with pucch-config If my understanding regarding the question is correct, our answer is the same as Q3-1-2, i.e., “if a UE is configured with supplementaryUplink in ServingCellConfig in the cell, RAN1 spec is specified such that higher layer parameter pucch-Config is provided on either UL or SUL” |
| Samsung | Please refer to our above input. |
| NEC | Agree this understanding. |
| MTK | Agree |

## PUSCH transmission when pucch-Config is provided on neither NUL nor SUL

Q3-2-1: Some companies pointed out that it is not clear where to transmit PUSCH when it is scheduled by DCI format 0_0 with UL/SUL indicator and pusch-Config/pucch-Config are configured on neither NUL nor SUL. However, if the observation 2 of R1-2400682, “if a UE is configured with supplementaryUplink in ServingCellConfig in the cell, RAN1 spec is specified such that higher layer parameter pucch-Config is provided on either UL or SUL” is the case, the issue does not exist. Do you agree with this observation? If no, please provide your comment on what the current spec specifies.

| Company | View |
| --- | --- |
| Spreadtrum | Agree |
| ZTE | We agree that “if a UE is configured with supplementaryUplink in ServingCellConfig in the cell, RAN1 spec is specified such that higher layer parameter pucch-Config is provided on either UL or SUL”. |
| Vivo | Yes |
| Samsung | Agree. There is no ambiguity for PUSCH transmission. |
| New H3C | Yes |
| Nokia/NSB | Yes |
| NEC | At least for Pcell, yes. For Scell, there is neither PUCCH configuration nor latest PRACH transmission. For Scell’s SUL configuration when UL/SUL indicator is not present, it’s not clear which uplink is used for PUSCH transmission. Since we don’t have Scell’s SUL deployment today, there is no interoperation issue yet. |
| MTK | Yes |

## Relation b/w UL/SUL indicator and BWP indicator in the DCI format 0_1/0_2

Q3-3-1: R1-2401408 pointed out clarification is necessary between UL/SUL indicator and BWP indicator in the DCI format 0_1/0_2. At RAN1#115, there are following different understandings:

Opt.1: UL/SUL indicator is per UL BWP. The field exists when the UL BWP is active on both UL and SUL.

Opt.2: UL/SUL indicator is per cell. The field exists regardless of which UL BWP is active on UL and SUL. When a UL BWP is active only on one of UL and SUL, the UL/SUL indicator indicates the UL or SUL where the UL BWP is active.

Opt.3: BWP indicator is present only when UL/SUL indicator indicates a UL carrier that has multiple BWP configurations.

According to the discussion, Opt.3 would make sense due to the following reasons: (1) the condition on whether UL/SUL indicator field is present is whether supplementaryUplink in ServingCellConfig is configured, AND PUSCH transmission is configured on both UL and SUL; (2) for any other field (except for “identifier for DCI formats” and “carrier indicator”) in the DCI format, DCI fields pursing (including whether the field exists and how many bit the field has) is based on whether the UL/SUL indicator indicates UL or SUL. Given this, it would make sense to clarify that the intention is Opt.3 as following:

- Clarify following:

  - UL/SUL indicator field is present in the DCI format 0_1/0_2 if the UE is configured with supplementaryUplink in ServingCellConfig in the cell and if both carriers in the cell are configured for PUSCH transmission, regardless of whether NUL and/or SUL of the serving cell has multiple UL BWP configurations. BWP indicator field is present in the DCI format 0_1/0_2 if the UL/SUL indicator field indicates an UL carrier that has multiple UL BWP configurations.

  - No specification change is necessary.

With that, are you OK to clarify the above? If no, please explain the reason.

| Company | View |
| --- | --- |
| Spreadtrum | It has been clear in DCI alignment in 38212. Padding is done at end of DCI, which means it is per SUL/UL DCI size alignment. So if DCI for SUL and UL are different size, including have different number of BWP, zeros are appended at the end. BWP indication field can be different according to the following description. 7.3.1.0 DCI size alignment Step 2: - Determine DCI format 0_1 monitored in a UE-specific search space according to clause 7.3.1.1.2. - Determine DCI format 1_1 monitored in a UE-specific search space according to clause 7.3.1.2.2. - For a UE configured with supplementaryUplink in ServingCellConfig in a cell, if PUSCH is configured to be transmitted on both the SUL and the non-SUL of the cell and if the number of information bits in format 0_1 for the SUL is not equal to the number of information bits in format 0_1 for the non-SUL, zeros shall be appended to smaller format 0_1 until the payload size equals that of the larger format 0_1. - If the size of DCI format 0_1 monitored in a UE-specific search space equals that of a DCI format 0_0/1_0 monitored in another UE-specific search space, one bit of zero padding shall be appended to DCI format 0_1. - If the size of DCI format 1_1 monitored in a UE-specific search space equals that of a DCI format 0_0/1_0 monitored in another UE-specific search space, one bit of zero padding shall be appended to DCI format 1_1. Step 2A: - Determine DCI format 0_2 monitored in a UE-specific search space according to clause 7.3.1.1.3. - Determine DCI format 1_2 monitored in a UE-specific search space according to clause 7.3.1.2.3. - For a UE configured with supplementaryUplink in ServingCellConfig in a cell, if PUSCH is configured to be transmitted on both the SUL and the non-SUL of the cell and if the number of information bits in format 0_2 for the SUL is not equal to the number of information bits in format 0_2 for the non-SUL, zeros shall be appended to smaller format 0_2 until the payload size equals that of the larger format 0_2.. |
| ZTE | We agree with the above understanding and no need to clarify based on current spec in clause 7.3.1.0 of TS 38.212 |
| vivo | We think UL/SUL indicator field is a per cell indicator |
| NEC | OK to clarify. |
| MTK | OK to clarify the above |

# Conclusions

Based on the companies input and further offline discussion with each company, following would be convergences on the issue discussed in 3.1 and the issue discussed in 3.3.

Proposal 1 (for conclusion):

When a UE is NOT configured with supplementaryUplink in ServingCellConfig and is NOT configured with pucch-Config in the cell, and is to transmit a PUCCH after random access procedure, the UE transmits the PUCCH on either NUL or SUL whichever the latest PRACH is transmitted.

Proposal 2 (for conclusion):

When UL/SUL indicator is present in a DCI format 0_1, BWP indicator is present when the UL/SUL indicator indicates UL or SUL that has multiple UL BWP configurations.