3GPP TSG RAN WG1#116 			R1-2401705

Athens, Greece, February 26th – March 1st, 2023

Agenda Item:	9.7.1

Source:	Moderator (AT&T)

Title:		FL summary #2 on ISAC deployment scenarios

Document for:	Discussion and Decision

# Introduction

This document provides a summary of the discussion of the ISAC deployment scenarios, which is within the objectives in the Rel-19 newly approved SID on the “Study on channel modelling for Integrated Sensing And Communication (ISAC) for NR” [1] as per the objectives shown below:

The focus of the study is to define channel modelling aspects to support object detection and/or tracking (as per the SA1 meaning in TS 22.137). The study should aim at a common modelling framework capable of detecting and/or tracking the following example objects and to enable them to be distinguished from unintended objects: - UAVs - Humans indoors and outdoors - Automotive vehicles (at least outdoors) - Automated guided vehicles (e.g. in indoor factories) - Objects creating hazards on roads/railways, with a minimum size dependent on frequency All six sensing modes should be considered (i.e. TRP-TRP bistatic, TRP monostatic, TRP-UE bistatic, UE-TRP bistatic, UE-UE bistatic, UE monostatic). Frequencies from 0.5 to 52.6 GHz are the primary focus, with the assumption that the modelling approach should scale to 100 GHz. (If significant problems are identified with scaling above 52.6 GHz, the range above 52.6 GHz can be deprioritized.) For the above use cases, sensing modes and frequencies: - Identify details of the deployment scenarios corresponding to the above use cases. - Define channel modelling details for sensing using 38.901 as a starting point, and taking into account relevant measurements, including: - modelling of sensing targets and background environment, including, for example (if needed by the above use cases), radar cross-section (RCS), mobility and clutter/scattering patterns; - spatial consistency. It will be discussed at RAN#105 whether to include additional study beyond channel modelling for ISAC.

The summary is prepared based on the contributions submitted to the current meeting ([2-45]).

The discussion will focus on the following aspects:

- Priority deployment scenarios

- Sensing modes and terminology

- Evaluation parameters

- Others and next steps

Please note that in this FL summary, a FL proposal may be designated as (H)(M)(L) to indicate its high, medium or low priority for online or offline discussions in this meeting. Nevertheless, we encourage all interested companies to provide feedback on all FL proposals. The FL may revise the priority of the proposals based on inputs from interested companies during the meeting, if it is deemed necessary.

# Contact Information:

See sub-folder in the inbox for AI 9.7 for list of contact for ISAC topics.

# Topic #1: Priority Deployment scenarios

The related proposals are copied below.

| [4] Spreadtrum Communications | Proposal 1: The sensing targets can be modeled as additional objects distributed within at least one of the scenarios defined in TR 38.901. |
| --- | --- |
| [5] Huawei | Proposal 1: Take Error: Reference source not found as a starting point for discussing scenarios for different use cases and sensing modes. - The table is dimensioned to cover both mono-static and bi-static sensing modes in general. - E.g., for TRP mono-static, the UT related parameters within the table are not applied. |
| [6] New H3C | Proposal 1: The scenario of intruder detection in smart home is the focus of Rel-19 ISAC with high priority. Proposal 2: The scenario of UAV flight trajectory tracing is the focus of Rel-19 ISAC with high priority. |
| [7] [8] Tiami Networks | In this proposal, we have proposed a few deployment scenarios for ISAC including two indoor and two outdoor deployment configurations where we have provided a list of objects and parameters along with their corresponding numerical values. |
| [9] NIST | The 80 configurations for the human gesture use case (see detail in [2]) , and the 27 configurations for the human walking use case are applicable to “Human indoors and outdoors”[1]. Furthermore, the 24 vehicle configurations are applicable to “Automotive vehicles (at least outdoors)” [1]. Sensing mode for human gesture is monostatic. Sensing mode for human walking and car sensing is bistatic. We propose to approve this categorization of measurement use cases with the scenarios in the SI [1] and, that the measurement data for the used cases identified above be used for ISAC channel modelling for FR2 (28.5 GHz). |
| [11] vivo | Error: Reference source not foundError: Reference source not found Error: Reference source not foundError: Reference source not found Error: Reference source not foundError: Reference source not found Error: Reference source not foundError: Reference source not found Error: Reference source not foundError: Reference source not found Error: Reference source not foundError: Reference source not found |
| [12] CMCC | Proposal 1: Prioritization of example objects should be considered in the study item, and the following example objects are preferred: - UAVs; - Automotive vehicles and humans for V2X; - Objects creating hazards on roads. Proposal 2: For detecting or tracking UAVs, the deployment scenario of UMa should be prioritized. - Use UMa-AV deployment scenario in TR 36.777 as a starting point. Further study on parameters including the ISD, carrier frequency, etc. Proposal 4: For detecting or tracking of automotive vehicles, humans, and objects creating hazards on roads, the deployment scenario of Urban grid and highway should be prioritized. - Use deployment scenario in TR 37.885 as a starting point. Further study on parameters including the ISD, carrier frequency, etc. |
| [13] CATT | Proposal 1: Reuse scenarios defined in TR 38.901 for ISAC as much as possible. New scenarios/ configurations/ parameters shall only be introduced if indeed necessary. Proposal 2: For UAV case, the scenarios of UMi-street canyon, UMa and RMa in TR 38.901 can be considered. Proposal 4: For humans indoors and outdoors case, the scenarios of Indoor-office, InF, UMi-street canyon and UMa can be considered. Proposal 6: For automotive vehicles case, the scenarios of UMi-street canyon, UMa and RMa can be considered. Proposal 10: For the use case of objects creating hazards on roads/railways, - ‘High Speed Train’ defined in TR 38.802 is considered as the scenario for railway-related objects. - ‘Highway for eV2X’ defined in TR 37.885 is considered as the scenario for road/highway-related objects. Proposal 11: The following objects are considered for railway and road/highway. - Railway: Pedestrian, vehicle, animal, bicycle/motorcycle, and any other obstacle that has comparable size and/or speed to the previous objects. - Road/highway: All the objects that considered for railway, and also vehicles with abnormal parameters (e.g. size, speed, direction). Proposal 12: Consider the prioritized scenarios, sensing modes and frequency range for each use case in Table 1. |
| [14] Intel | Proposal 1: Use existing 3GPP deployment scenarios of Error: Reference source not found as starting point for ISAC deployment scenarios. Proposal 2: Prioritize ISAC deployment scenarios according to Error: Reference source not found. |
| [15] Ericsson | Proposal 5 A sensing scenario models only one type of sensing target, either indoor or outdoor. Proposal 6 Support the following scenarios: · All of the sensing transmitters, sensing receivers, and targets are outdoor. · All of the sensing transmitters, sensing receivers, and targets are indoor. · If outdoor gNBs are the sensing transmitters/receivers, the targets are indoor. Proposal 7 Both indoor and outdoor UAVs as sensing target are in the scope of Rel-19 ISAC channel modelling SI. Proposal 8 The pairing between sensing scenario and existing deployment scenarios in TR 38.901, is defined according to Table 1. |
| [16] Xiaomi | Proposal 1: A limited set of typical deployment scenarios and corresponding sensing modes shall be selected to study. Proposal 2: Indoor office, highway and UMa-AV/UMi-AV shall be considered as high priority deployment scenario for ISAC channel model. |
| [17] OPPO | Observation 1: A fairly complete study over five example use cases in SID leads to a need of considering all the indoor scenarios (indoor office, INF) and outdoor scenarios (UMi,UMa,RMa). |
| [18] Sharp | Proposal 1: UAVs, Automotive vehicles, and Objects creating hazards on roads/railways should focus on UMi, UMa and RMa deployments. Proposal 2: Humans, Automotive guided vehicles should focus on InH and InF deployments. Proposal 9: On further down selection – Automotive vehicles (UMi, UMa and RMa), Automotive guided vehicles (InF) and Human indoors (InH) are of highest priority. |
| [19] Nokia | Proposal 8: Adopt at least the models proposed in Error: Reference source not found for study in 3GPP ISAC study. |
| [20] NTPU | Proposal 1: Elevating UAV Detection as a Priority |
| [21] China Telecom | Proposal 1: The study on detection and tracking of UAV is with the highest priority. Three types of deployment scenarios including UMi-AV, UMa-AV and RMa-AV defined in TR 36.777 can be referred as the starting point. Proposal 3: At least the combination of UAV deployment scenario, TRP monostatic mode should be prioritized in ISAC channel modelling. |
| [23] Samsung | Proposal 1 Considering all 5 scenarios in TR38.901 as starting point for ISAC channel model study. Proposal 2 TR36.777 and TR37.885 can be considered for the use cases involving UAVs and automotive vehicles, respectively, with taking into account the factors that require adjustment. Proposal 3 Study whether sensing-specific deployment scenario such as new BS deployment or multiple sub-scenarios is needed |
| [24] CICTCI | Observation 1: For the use cases corresponding to the example objects listed in the SID, the sensing transmitter or receiver can be BS or UE. Proposal 1: To minimize to complexity of deployment scenario designs, the following use cases can be prioritized in Rel-19 ISAC. - Sensing for UAV intrusion detection, with BS as the sensing transmitter or receiver - Intruder detection in surrounding of smart home, with BS or UE-type RSU as the sensing transmitter or receiver - Sensing assisted automotive manoeuvring and navigation, with BS or UE-type RSU as the sensing transmitter or receiver - Vehicle sensing for ADAS, with BS or UE as the sensing transmitter or receiver - AGV detection and tracking in factories, with BS (or UE, optionally) as the sensing transmitter or receiver - Pedestrian/animal intrusion detection on a highway, with BS or UE-type RSU as the sensing transmitter or receiver |
| [27] Sony | Proposal 1: Use the existing NR deployment scenario as the starting point and identify the deployment scenario to evaluate different type of use-cases. Proposal 2: Consider the following mapping of object target and existing deployment scenario as the starting point: – Dense urban, urban macro, and/or rural scenarios with added UAV for UAV deployment scenario. – Urban grid scenario for connected car scenario for Automotive vehicles deployment scenario. – Indoor factory and/or indoor hotspot scenarios for AGV deployment scenario. Proposal 3: Introduce sensing target(s) parameters, such as RCS, velocity, and its distribution in a selected deployment scenario. Other parameters may be required and for further study. Proposal 4: Reuse the UE and gNB hardware assumptions as in the existing NR deployment scenario. Proposal 5: UAV (as the sensing target) parameters such as RCS, speed/velocity, and UAV distribution should be considered in the deployment scenario. Proposal 6: Vehicles (motor/non-motor, and as the target object) parameters such as RCS, speed/velocity, and the vehicles distribution should be considered in the deployment scenario. Proposal 7: AGV (as the sensing target) parameters such as speed/velocity, RCS, and AGV distribution should be considered in the deployment scenario. |
| [28] EURECOM | Proposal 2: The applications with ISAC operation are deployed in both indoor and outdoor areas. |
| [31] Ruijie Network Co. Ltd | Proposal 1: ISAC deployment scenarios for UAVs should include path management and intrusion detection. This may need more than one frequency band (i.e., multi-band collaborative system) to be able to provide complete information for objects of various sizes (from small to large sized) on the surrounding environment. Furthermore, it may require contiguous coverage for a specific area under different kinds of weathers. Proposal 2: ISAC deployment scenarios for humans indoors should include smart home as well as medical and health related scenarios. Proposal 3: ISAC deployment scenarios for humans outdoors should include social service-related scenarios. Proposal 4: ISAC deployment scenarios for automotive vehicles should include C-V2X. The deployment scenarios are mainly outdoors where networking environment with contiguous coverage is needed in sensing service area. Proposal 5: ISAC deployment scenarios for automated guided vehicles should include indoor factories and restricted outdoors, e.g., wharfs. The deployment scenarios include sensing for cargo transportation, sorting, handling, stacking and management, where requirements for latency and reliability are key factors to be considered. |
| [33] Panasonic | Proposal 1 For each of the targets of interest, consider the following deployment scenarios in to be studied: |
| [34] Apple | P1: RAN1 to identify specific use cases for each of the target types to assist defining the deployment scenarios and corresponding deployment parameters. P2: If scenarios are to be prioritized, ensure that indoor/outdoor scenarios that can be found in TR 38.901 are included. |
| [35] Interdigital | Observation 1: The sensing use cases can be aligned with the defined scenarios from TR 38.901 and TR 37.885 with modifications specific to sensing. This allows (e.g., partial) reuse of well-studied and validated deployment parameters for ISAC channel model. Proposal 1: Consider the sensing targets mentioned in the SID (namely UAVs, humans, automotive vehicles, automated guided vehicles and the hazard objects on roads/railways) in the use cases for defining the deployment scenarios for the ISAC channel model. Proposal 2: Prioritize the outdoor urban scenarios (e.g., UMi and UMa) for UAVs, indoor home scenario (e.g., InH) for humans and outdoor urban scenarios (e.g., urban grid) for automotive use cases. |
| [36] ZTE |  |
| [37] ITL | Proposal 1: The following scenarios for ISAC deployment in the 5G NR network are mainly focused on Rel-19 ISAC from a RAN1 perspective: - Intruder detection in smart home - UAV tracking/detection - Sensing Assisted Automotive Maneuvering and Navigation |
| [38] Lenovo | Proposal 1: For the UAV use cases, the deployment scenarios for an aerial UE defined in TR 36.777 can be selected as a starting point, e.g., UMa-AV, RMa-AV, where the related parameters of such aerial UE can be considered for a UAV as a sensing target. Proposal 2: For the human indoor use cases, the deployment scenario for an Indoor Office evaluation with parameters defined in Table 6.1.1-3 of TR 38.855 based on the channel modeling details of TR 38.901 can be assumed as a starting point. Proposal 3: For the human outdoor use cases, the deployment scenario for UMi street canyon evaluation with parameters defined in Table 6.1.1-4 of TR 38.855 based on the channel modeling details of TR 38.901 can be assumed as a starting point. Proposal 4: For the automotive vehicles use cases, the deployment scenarios for the evaluation on V2X defined in TR 37.885 can be selected as a starting point. Proposal 5: For the AGV use cases, the deployment scenarios for an indoor factory evaluation with the parameters defined in Table 6.1-1 of TR 38.857, based on the channel modeling details of TR 38.901 can be selected as a starting point. Proposal 6: For the use cases of objects creating hazards on roads/railways, the deployment scenarios of highway for the evaluation on V2X defined in TR 37.885 can be selected as a starting point. Proposal 7: Select the deployment scenarios that have been used for the evaluations with RAT-dependent techniques as the starting point, which can be enhanced with the potential new sensing-related parameters as summarized in the table below. |
| [39] IIT Kanpur | Proposal 1: Prioritize use of existing deployment scenarios as mentioned for 5G NR in TR 38.913 |
| [40] Continental Automotive | Proposal 2: For the different sensing targets, consider the prioritization in terms of deployment types and sensing modes shown in Table 1. |
| [41] AT&T, FirstNet | Proposal 1: RAN1 considers outdoor deployment scenarios use cases as a high priority for object detection and tracking |
| [43] MediaTek | Observation 1: The four deployment scenarios have been defined in TR 38.901. Proposal 1: The TR 38.901 deployment scenario can be referred as a starting point for ISAC use case deployment scenario. |

Moderator Discussion:

There are divergent views among the various contributions on deployment scenarios and prioritization of deployment scenarios. However, most companies do prefer some level of prioritization in particular per use case scenario. Additionally, most companies have captured that we should leverage the existing scenarios defined in, e.g., 38.901, 37.855, TR 36.777 as a starting point. Note that a wide range of use cases are discussed in the contributions reflecting the use cases prioritized in the ISAC SID and that should be reflected in RAN1 agreements as well.

It should also be noted that without any prioritization, there will be challenges in having any additional ISAC study, evaluation and/or validation of the ISAC channel modelling in the Rel-19 timeframe, in particular for the RAN plenary checkpoint (determining additional SID scope) in Sept 2024.

## Moderator Tentative Plan

Based on the wide-ranging and divergent contributions at this very early stage for this SID, RAN1 should take into account the following tentative plan relative to the deployment scenarios:

- RAN1 should come to a common understanding for high level principles for deployment scenarios in the 1st meeting, including, but not limited to,

  - (de)prioritization of the following: use cases, deployment scenarios, sensing modes, and other parameters such as frequency bands of interest

    - Also if no prioritization is to occur

  - agree/confirm definitions/terminology for e.g., sensing modes, targets, etc.

- Following step one, agree additional evaluation parameters and principles, as well as the modifications necessary to model target(s). - FFS

- Include any additional (other) agreements as time permits, e.g., target related parameters, KPIs – FFS

- Goal is for completion of detailed of deployment scenarios at RAN1#117.

## Proposal 3-1 (FL1)

Proposal 3-1: For progressing ISAC deployment scenarios/channel modelling, all of the following use cases/targets will be addressed as part of the SI without any explicit prioritization:

- UAVs

- Humans indoors and outdoors

- Automotive vehicles (at least outdoors)

- Automated guided vehicles (e.g. in indoor factories)

- Objects creating hazards on roads/railways (excluding target(s) already covered above, e.g., pedestrians, vehicles, etc.)

NOTE: (De)prioritization of certain deployment scenarios addressing a subset of the above use cases is not precluded in the future dependent upon RAN1 progress.

Companies can provide comments/inputs in the following table:

| Company | Comment |
| --- | --- |
| CMCC | Among the 5 use cases in proposal 3-1, the following example objects are preferred from our perspective: • UAVs; • Automotive vehicles and humans for V2X; • Objects creating hazards on roads. Based on our initial investigations from industries, there are some urgent requirements on enabling radio sensing functionality in UAV and V2X scenarios. Firstly, effective and seamless surveillance and regulation of UAV operation based on 5G cellular networks have become emerging needs from civil aviation authority and local governments. Secondly, network facilitated NR based sensing for automotive vehicles detection and tracking is needed to help expand the sensing range and processing capability, solve the sensing problems of the on-board sensors in harsh environments and in blocking blind areas. Thirdly, Sensing of human in V2X scenario can be prioritised, especially for the case that pedestrian suddenly rushing to the road from the invisible place. Lastly, the network facilitated NR based sensing can be used to sense the pedestrian/animal intrusion detection in highway with wide area and seamless coverage. |
| ZTE | Generally OK with one thing to clarify: For case 5 “objects creating hazards on roads/railways”, the sensing objects should be clearly specified, for example, the objects can be a human (e.g., a child) rushing into the highway. If human on roads/railways (i.e., human outdoor) is already covered by case 5, then case 2 should focus on human indoor only. |
| LGE | Support. Those are already discussed and determined at RAN plenary meeting. We don’t need to iterate the same discussion in RAN1 meeting. |
| vivo | First of all, we believe that the deployment scenario should be independent of use cases and sensing modes. The channel modelling should be unified for any use case and sensing mode. The use cases selected are only for the validation of channel modelling, other than the channel modelling. In consideration of the factors of limited TUs and potential experiment capability and workload, thus, RAN1 prioritizes use cases/targets on 2), 1), and 4). |
| CICTCI | We generally agree the direction proposed by FL that making down selection on the 5 sensing targets will be difficult at current stage because of the different priorities of different companies. But it should be firstly discussed that whether all or part of the use cases related to each sensing target should be evaluated, which will cause some effect on deployment scenarios designs. From our perspective, one or two typical use cases can be selected per sensing target to limit the workload. |
| InterDigital | Support |
| Panasonic | Support |
| Sharp | Support. However, for use case 5 we need to classify the “objects”. Some of the “objects” might already be a part of use case 1-4 and thus there is no addition need to support use case 5. Explicitly defining “objects” for use case 5 is important. Use case 5 can be de-prioritized as it might have overlap with other use cases. |
| China Telecom | Share the similar view as vivo, we think some prioritization is needed considering the limited TUs. From our perspective, we think at least “UAVs” case is with high priority, considering the accelerating of “low altitude economy” recent years. |
| CATT | We are supportive in general. The discussion on deployment scenarios can take all use cases (sensing targets) into consideration at first. (De)prioritization is possible in the future depending on the progress. |
| MTK | We are generally ok with the proposal since these exampled use cases have been explicitly captured in approved SID. For the use case 1, it is better to clarify the UAV is outdoor scenario case since the other cases have reflected indoor/outdoor clearly. |
| Ericsson | Agree with the proposal. No prioritization is mentioned in the SI. Our concern on prioritization is that a channel model agreed for some priority use case may not be workable for other use cases. Either a rework on the previous channel model is needed, or different channel models are developed for different use cases. Both should be avoided. |
| Xiaomi | We can accept the proposal if this is the majority view. We also think the 5th target “Objects creating hazards on roads/railways” needs to be clarified, and if possible, it can be combined with other targets/use cases, e.g. humans. |
| Toyota ITC | Support |
| Spreadtrum | Support since the use cases have been discussed and written in SID. For use case 5, we also suggest to have a clear definition on the potential sensing target. |
| AT&T | We agree with the direction of the proposal, but even if we consider all use cases as proposed, we still need to agree on the deployment scenarios for the chosen use cases and the corresponding frequencies of interest. This will be important to develop a common framework of the channel modelling among all considered sensing targets and sensing scenarios |
| Huawei, HiSilicon | In our view, we’d like to prioritize UAV and vehicle cases. However, I do understand companies may have different views. If the discussion ends up to no prioritization, we’d like to drop this proposal for saving time. |
| SONY | Support. However, we can potentially define the prioritization at latter stage. |
| Qualcomm | We support to not attempt to do an explicit prioritization (support FL proposal). |
| OPPO | Support |
| Nokia | Support the proposal, but agree with ZTE that case 5 may require some further clarification on scope. |
| CEWiT | Support the proposal. We do agree that “Objects creating hazards on roads/railways” is ambiguous and what we consider it is as “undesired object on the roads/railways”. Modified wording is fine with us. |
| Apple | Support the FL’s proposal |
| Samsung | For case 5, it needs to be clarified which target is considered |
| New H3C | Support in general and similar view on case 5 with other company |
| Continental Automotive | Suppporth |

## Proposal 3-2 (FL1)

Proposal 3-2: As a starting point for the UAV use cases, reuse and prioritize the deployment scenarios for an aerial UE defined in TR 36.777, and TR 38.901.

- FFS modifications as needed for UAV sensing target characteristics, and modelling the path between sensing transmitter and sensing receiver

Companies can provide comments/inputs in the following table:

| Company | Comment |
| --- | --- |
| CMCC | For detecting or tracking UAVs, the deployment scenario of UMa should be prioritized. Additionally, for detecting or tracking UAVs, using sub-6 GHz network can provide much larger sensing coverage due to the low pathloss when compared to higher frequency bands. A larger ISD can be considered for the deployment scenario of UAV. |
| ZTE | Support to use deployment scenarios defined in TR 36.777 as a starting point, on top of that, we think RMa scenarios should be the first priority deployment scenario considering the needs of long-distance sensing. |
| LGE | We prefer to start with all the existing evaluation scenarios for AV. That is, UMi-AV, UMa-AV, RMa-AV. |
| vivo | Agree. |
| CICTCI | Agree |
| InterDigital | Support |
| Panasonic | We support UMa, RMa. But prefer to reuse deployment scenarios in 38.901. The objective is not to communicate with a UAV, but to detect a UAV in a certain environment. |
| Sharp | UMa to be prioritized using FR1 bands. TR 36.777 can be used as starting point. |
| China Telecom | Generally Support. However, from our understanding, three types of deployment scenarios including UMi-AV, UMa-AV and RMa-AV defined in TR 36.777. Is there some consideration to not involved the UMi-AV? Although we know it is just an e.g. |
| CATT | OK. BTW, we think the difference between UMa, UMi and RMa in TR 38.901 and UMa-AV, UMi-AV, RMa-AV in TR 36.777 is small. Perhaps one notable difference is that TR 36.777 may not consider FR2 related frequency band. |
| Ericsson | In general, we think we think deployment scenarios in TR 38.901 can be used as a starting point. Scenarios in TR 36.777 may be too restrictive for UAV sensing use case. For example, we think indoor UAVs are a use case as important as outdoor UAVs, which are commonly used warehouses and factories and map the deployment scenario of Indoor Factory in TR 38.901. Our preference is as follows. Use cases; Compatible deployment scenarios from Section 7.2 in TR38.901. Outdoor UAV; UMa, RMa, UMi. Indoor UAV; Indoor Factory. Regarding the sub-bullet, how to model the direct path between sensing transmitter and sensing receiver TRP-TRP bistatic, TRP monostatic, UE monostatic, and UE-UE bistatic sensing modes is to be studied, which is not available in TR 38.901. |
| Xiaomi | To reduce the number of deployment scenarios for parameters determination, validation and calibration, selecting one from the three scenarios, e.g., UMa-AV or UMi-AV or RMa-AV, may be enough at first stage. UMa-AV may be a good compromise among the three deployment scenarios. |
| Spreadtrum | Support. We prefer UAV in UMa, UMi, and also RMa scenarios. |
| AT&T | Ok with this proposal |
| Huawei, HiSilicon | ‘UMA-AV, RMa-AV’ as example is made in the proposal, so we wonder what is ‘deprioritized’ by this proposal. In our views, the difference among UMi, Uma, Rma lies the ISD, BS height, such difference is tolerable for simulation. It can be up to company to evaluate the interested scenario. In addition, what’s ‘characteristics’ exactly meaning in the discussion of ‘scenarios’. Moreover, we think what’s additionally needed include the minimum distance between BS and UAV |
| SONY | Support |
| Qualcomm | We are generally supportive with one comment: In the examples, we want to keep Umi-AV (or remove all the examples) |
| OPPO | It seems to say that, for UAV case, the TRP-based mono-static/bi-static modes are prioritized, since the reused deployment from 36.777 only contains gNB (mapping to TRP) and aerial UE (mapping to target). This also echoes Proposal 4-1. In our view, prioritizing TRP-based modes over other modes (like TRP-UE bi-static) would make UAV sensing very limited, especially when TRP are fix-located and sensing turns out to be sensitive to geometry with TRP locations, and/or not every gNB/TRP can work in duplex mode to receive the sensing signal when the neighouring gNB/TRP in a synchronized network transmits the sensing signal. So our preference to move forward with this proposal is to remove “and prioritize” in the main sentence. This question is applicable to all Proposal 3-2 to |
| Apple | OK with the proposal |
| Samsung | The reusing or prioritization for the deployment scenario is fine but the further checking for parameters and modelling is needed. |
| New H3C | OK |
| Continental Automotive | Support, agreed with Huawei, at least minimum distance between UAV and BS is needed. |

## Proposal 3-3 (FL1)

Proposal 3-3: As a starting point for the human indoor use cases, reuse and prioritize the deployment scenario for an Indoor Office evaluation on the channel modelling details of TR 38.901. As a starting point for the human outdoor use cases, reuse and prioritize the deployment scenario for UMi street canyon based on the channel modelling details of TR 38.901.

- FFS modifications as needed for human sensing target characteristics, and sensing background/environment

Companies can provide comments/inputs in the following table:

| Company | Comment |
| --- | --- |
| ZTE | For human indoor: Basically, we prefer to support limited deployment scenarios for all sensing targets, indoor factory scenario can be used for both human indoor use case and AGV use case. For human outdoor: As we commented in FL Proposal 3-1, if case 5 already include human outdoor, then maybe simplification here can be made. |
| LGE | We prefer to start with all the existing evaluation scenarios. For the indoor use cases, we can reuse indoor office and indoor factory scenario defined in TR38.901. For the outdoor use cases, we can reuse UMa, UMi, RMa defined in TR38.901, and urban grid and highway for V2X defined in TR37.885. |
| vivo | Basically, we are fine with the proposal with the comment as follows. Considering the use case of smart home, the room size should not be too big. As discussed in TR 38.808, the small room defined as the scenario of indoor-B can be constructed with an area of 20m×20m, where TRP is equipped at ceiling, UEs are deployed in the room and the sensing target of human is deployed in the room, moving around. Therefore, for the deployment scenario for indoor, RAN1 should take into account use case with both large room size (120m×50m) and small room sizes (20m×20m defined in TR 38.808). Regarding the deployment scenario in the proposal, there is no necessity to refer to TR 38.855. It is the same as in 38.901, except that there are additional configurations related to the antennas of FR1 and FR2. The antenna configuration has nothing to do with channel modelling, and thus, the deployment scenario and channel modelling of indoor office and UMi can directly use 38.901 as a reference. |
| CICTCI | Regarding human indoor and human outdoor, it is preferred to select one of them to reduce the workload and human outdoor is our preference. Compared with UMi scenario, we think urban grid in TR 37.885 is a more suitable scenario where the pedestrians are dropped along the sidewalk, such as for the use case of intruder detection in surrounding of smart home, the blocks defined in TR 37.885 can be treated as the smart home and the pedestrian (may need to configure a lower density) can be treated as the “intruder”. |
| InterDigital | Support |
| Panasonic | can reuse scenarios defined in TR38.901 |
| Sharp | Support. InH for Human indoor is priority and UMi for Human outdoor is priority. |
| CATT | OK in general. Indoor: OK to go with indoor-office, but this scenario is already defined in TR 38.901 too. Do we really need to consult TR 38.855? Outdoor: OK to go with UMi. Fine to consider either UMa or UMi. |
| MTK; Prefer directly using the TR 38.901 indoor/outdoor deployment scenario as a starting point.. |  |
| Ericsson | We think UMa scenario is an important scenario for both the two use cases, human indoors and human outdoors. O2I sensing link is an important part for the use cases of indoor sensing targets, which can be based on O2I communication model defined in TR 38.901. Moreover, TR 38.855 also includes UMa scenario with 50% indoor UEs and 50% outdoor UEs. In general, we think deployment scenarios in TR 38.901 can be used as a starting point. Our preference is as follows. Use cases; Compatible deployment scenarios from Section 7.2 in TR38.901. Outdoor humans; UMa, UMi, RMa. Indoor humans; UMa, UMi, Indoor Office, Indoor Factory. Same comment to the sub-bullet as mentioned in the previous proposal. |
| Xiaomi | From our perspective, human indoor and outdoor use cases are more related to smart home use cases, and thus human indoor shall be prioritized. Therefore, we support to consider indoor office scenarios defined in TR 38.901 as the starting point for human indoor use cases. We are open to consider UMi as the deployment scenario for human outdoor use cases. |
| Spreadtrum | We think the scenarios defined in TR 38.901 can be reused, and the UE distribution can be considered as the distribution of humans. |
| AT&T | Ok with this proposal. Agree to reuse TR38.901 without the need to go to TR 38.855 |
| Qualcomm | For human indoor, we would like to keep both Indoor Office and Indoor factory. This is a starting point discussion, and depending on progress further changes may be done, but for now, we think both cases will be very useful for human indoor sensing. |
| OPPO | If the SI output is still TR 38.901, which seems the case at present, we wonder whether the proposal 3-3 would bring scenario parameters from 38.855 into 38.901 if there is anything about scenario to be different between the two TRs . Because 38.901 focuses on channel modeling, any scenario difference that does not lead to difference in channel modeling should be avoided to be stated in 38.901. Further, if 38.901 ends up with an indoor office scenario (as in current 38.901) for communication evaluation and another different indoor office scenario (as what Proposal 3-3 brings into) for sensing, it is not clear how to mix them in a “Joint” evaluation, given the SI is for Joint-SAC as it is called. So we think it is sufficient to take indoor office scenario in 38.901 as the starting point for indoor human detection and to leave 38.855 aside. |
| Nokia | Similar view InF can be included and are open to inclusion of other outdoor scenarios for as well |
| Apple | Okay with the proposal |
| Samsung | Support of revised proposal. It is difficult to find difference between technical reports from the deployment scenario perspective, at least. We think that TR 38.901 is sufficient. |
| New H3C | OK |
| Continental Automotive | Support. |

## Proposal 3-4 (FL1)

Proposal 3-4: As a starting point for the automotive vehicles use cases, reuse and prioritize the deployment scenarios for the evaluation on V2X defined in TR 37.885.

- FFS modifications as needed for automotive vehicle sensing target characteristics, and sensing background/environment

Companies can provide comments/inputs in the following table:

| Company | Comment |
| --- | --- |
| CMCC | For detecting or tracking of automotive vehicles and humans in V2X scenario, the deployment scenario of urban grid and highway should be prioritized. |
| ZTE | V2X scenario in TR 37.885 include highway and urban grid. In ZTE’s view, Highway scenario should be at least included considering the strong need of public safety, high probability of LOS channel conditions and the feasibility of wireless sensing. |
| LGE | Support |
| vivo | In consideration of the factors of limited TUs and potential experiment capability and workload, this use case should be deprioritized. |
| CICTCI | Support. And highway scenario is preferred. |
| InterDigital | Support |
| Panasonic | Support |
| Sharp | Support |
| CATT | OK. Specifically, in TR 37.855, we think ‘Urban grid’ can be focused in this use case, while ‘Highway’ is focused in use case of Objects creating hazards on roads/railways |
| MTK | Ok |
| Ericsson | Channel models defined in TR 37.885 are mainly for sidelink. E.g., in urban grid scenario, vehicle speed is in the range from 15kmh to 60kmh. We are not sure if such fast moving UEs are suitable as sensing transmitters/receivers. In general, we think deployment scenarios in TR 38.901 can be used as a starting point. Our preference is as follows. Use cases; Compatible deployment scenarios from Section 7.2 in TR38.901. Automotive vehicles; UMa, UMi, RMa. |
| Xiaomi | To reduce the number of deployment scenarios for parameters determination, validation and calibration, selecting a single deployment scenario between urban grid and highway may be sufficient. Considering that highway scenario has less complexity and higher vehicle velocity, highway scenario can be prioritized. |
| Toyota ITC | Support. Both urban grid and highway in TR 37.885 should be considered. |
| Spreadtrum | OK |
| Huawei, HiSilicon | For vehicle use case, the cell layout from TR37.885 can be referenced. The METIS section 4.2.1 can also be as a reference for geometry map. In addition, the environment target related parameters can also be added into the scenario discussion. |
| SONY | We have a similar view as Ericsson. In principle, we can consider both TRs and select the relevant assumptions / parameters for ISAC. |
| Qualcomm | Support |
| OPPO | Support |
| Nokia | Support, but also believe that scenarios covering UMa, and RMa could be relevant to modelling UE tracking in suburban environments. |
| CEWiT | Support. |
| Apple | Support |
| Samsung | The reusing or prioritization for the deployment scenario is fine but the further checking for parameters and modelling is needed. |
| New H3C | OK |
| Continental Automotive | Support. TR 37.885 as starting point. At least, urban scenarios (intersections) and highways. |

## Proposal 3-5 (FL1)

Proposal 3-5: As a starting point for the Automated Guided Vehicles (AGV) use cases, reuse and prioritize the deployment scenarios for an indoor factory evaluation with the parameters defined in TR 38.857, based on the channel modelling details of TR 38.901.

- FFS modifications as needed for AGV sensing target characteristics, and sensing background/environment

Companies can provide comments/inputs in the following table:

| Company | Comment |
| --- | --- |
| ZTE | OK |
| LGE | We support FL proposal for the indoor use cases for AGV. We think that we can also study the use case where AGV is operating in outdoor such as the port area. In this case, we can start with UMi defined in TR38.901, and develop a new evaluation scenario for V2X. |
| vivo | Support. |
| CICTCI | Support. |
| InterDigital | Support |
| Sharp | Support. AGV for InF is high priority for Industry 4.0. |
| CATT | Support. One minor comment similar to before: indoor factory is already defined in TR 38.901 too. Do we really need to consult TR 38.857? |
| MTK | Prefer directly using the TR 38.901 indoor factory deployment scenario as a starting point. |
| Ericsson | Channel models defined in TR 38.857 aim to support advanced positioning techniques for higher accuracy (horizontal and vertical) than those studied in TR 38.855. These requirements are different from sensing performance requirements defined in TS 22.137. Therefore, we think deployment scenarios in TR 38.901 can be used as a starting point. Our preference is as follows. Use cases; Compatible deployment scenarios from Section 7.2 in TR38.901. AGVs; Indoor Factory. |
| Spreadtrum | We think the indoor factory scenario defined in TR 38.901 can be reused. |
| Sony | Support. In principle, we can use both TR 38.857 and TR 38.901. TR 38.857 is the most recent one. |
| Qualcomm | Support |
| OPPO | Same comment as for Proposal 3-3. We would like to see what impacts this proposal would bring to 38.901. |
| Nokia | Support. Ok to include UMi as well. |
| CEWiT | Support |
| Apple | Support |
| Samsung | Same comments with CATT. InF scenarios are already defined in TR 38.901. |
| New H3C | OK |

## Proposal 3-6 (FL1)

Proposal 3-6: As a starting point for the use cases of objects creating hazards on roads/railways, reuse and prioritize the deployment scenarios of urban grid and highway for the evaluation on V2X defined in TR 37.885, and reuse UMa, UMi, RMa defined in TR38.901.

- FFS modifications as needed for hazards on roads/railways and associated sensing target characteristics, including distinguishing from background/environment

Companies can provide comments/inputs in the following table:

| Company | Comment |
| --- | --- |
| CMCC | Based on the initial tests of V2X scenario at sub-6 GHz and above 6 GHz, we found that the interference caused by clutter and jamming from strong reflective surfaces and from inter-target cases is serious in urban city. Sensing in urban city has several challenges on satisfying the sub-meter accuracy of positioning, 5% missed detection and 5% false alarm requirements. However, the interference of clutter and jamming at highway is weak, the scenario of V2X at highway can be prioritized at the first stage. Additionally, we have received demands from the highway management department that the major accidents caused by pedestrians or animals crossing highways occur frequently. Currently, the highway supervision systems are mainly based on traditional sensors (e.g. cameras) equipped in the roadside infrastructure, but it only has partial coverage along the roadside. The network facilitated NR based sensing can be used to sense the pedestrian/animal intrusion detection with wide area and seamless coverage. In TR 22.837, the assumption of size and typical velocity of traffic participant is described in the Table below, which can be used as a starting point. Size (Length x Width x Height); Typical velocity. Pedestrian (Adult); 0.5m x 0.5m x 1.75m; 5km/h. Animal (Sheep/deer); 1.5m x 0.5m x 1 m; 5km/h. Vehicle; 4m x 1.75m x 1.5m; 60km/h - 120km/h. |
| ZTE | Support |
| LGE | We prefer to start with all the existing evaluation scenarios. We can reuse UMa, UMi, RMa defined in TR38.901, and urban grid and highway for V2X defined in TR37.885. |
| vivo | In consideration of the factors of limited TUs and potential experiment capability and workload, this use case should be deprioritized. |
| CICTCI | Support. |
| Sharp | Support. The definition of “objects” is crucial to characterize the exact deployment scenario. This use case can be deprioritized as it can have overlap with other use cases. Defining “Objects” is like adding new use cases. |
| China Telecom | Share the similar view that the “objects” needs further clarification. |
| CATT | Support using highway of TR 37.855 for this use case. But if we only consider highway, the ‘railway’ part in this use case seems dropped. We suggest to reuse ‘high speed train’ in TR 38.802/TR 38.913 for ‘railway’ part. |
| Ericsson | It needs some study on the types of hazardous objects as sensing targets. They may not be vehicles. In this sentence, we are not sure if channel models defined in V2X TR can be reused for the sensing use case of objects creating hazards on roads/railways. In general, we think deployment scenarios in TR 38.901 can be used as a starting point. Our preference is as follows. Use cases; Compatible deployment scenarios from Section 7.2 in TR38.901. Objects creating hazards on roads; UMa, RMa. Objects creating hazards on railways; UMa, RMa. |
| Xiaomi | Fine to accept the proposal. |
| Toyota ITC | Both urban grid and highway in TR 37.885 should be considered. |
| Spreadtrum | As commented for proposal 3-1, we suggest to have a clear definition on the potential sensing target. |
| Qualcomm | With regards to roads, the above proposal seems a good starting point. |
| OPPO | Ok with the proposal 3-6. |
| Nokia | In our view, clarification on target types and use cases may need to be clarified before the scenario is clear. Prefer not to down-select at this time. |
| New H3C | OK |
| Continental automotive | Support. |

# Topic #2: Sensing Modes/terminology

The related proposals are copied below.

| [4] Spreadtrum Communications | Proposal 2: For channel modeling for ISAC, each deployment scenario can be defined supporting all six sensing modes. |
| --- | --- |
| [5] Huawei | Proposal 1: Take Error: Reference source not found as a starting point for discussing scenarios for different use cases and sensing modes. - The table is dimensioned to cover both mono-static and bi-static sensing modes in general. - E.g., for TRP mono-static, the UT related parameters within the table are not applied. |
| [9] NIST | The 80 configurations for the human gesture use case (see detail in [2]) , and the 27 configurations for the human walking use case are applicable to “Human indoors and outdoors”[1]. Furthermore, the 24 vehicle configurations are applicable to “Automotive vehicles (at least outdoors)” [1]. Sensing mode for human gesture is monostatic. Sensing mode for human walking and car sensing is bistatic. We propose to approve this categorization of measurement use cases with the scenarios in the SI [1] and, that the measurement data for the used cases identified above be used for ISAC channel modelling for FR2 (28.5 GHz). |
| [11] vivo | Error: Reference source not foundError: Reference source not found Error: Reference source not foundError: Reference source not found |
| [12] CMCC | Proposal 7: Both mono-static and bi-static modes can be considered for ISAC channel modeling. |
| [13] CATT | Proposal 12: Consider the prioritized scenarios, sensing modes and frequency range for each use case in Table 1. |
| [15] Ericsson | Proposal 3 For TRP-UE bistatic and UE-TRP bistatic sensing modes, the direct propagation path between sensing transmitter and sensing receiver can be referred to the existing gNB-UE communication channel model. Proposal 4 For TRP-TRP bistatic, TRP monostatic, UE monostatic, and UE-UE bistatic sensing modes, the direct propagation path between sensing transmitter and sensing receiver is not defined in TR 38.901 and needs to be studied. |
| [16] Xiaomi | Proposal 1: A limited set of typical deployment scenarios and corresponding sensing modes shall be selected to study. |
| [17] OPPO | Proposal 1: Besides mono-static sensing and bi-static sensing, multi-static sensing mode is also supported in Rel-19 ISAC channel modeling. - Support of multi-static mode is mostly implemented in the support of bi-static mode. |
| [18] Sharp | Proposal 4: Narrowing down on the type of sensing mode and frequency bands of interest can expedite the process for target characterization, modeling, and verification of ISAC channel model. Proposal 5: We may further prioritize and narrow down the gNB-gNB and gNB-UE based sensing modes for ISAC channel modeling and verification and deprioritize UE based bi-static based sensing modes. Proposal 6: Monostatic sensing (gNB based and UE based) can be deprioritized. |
| [19] Nokia | Proposal 1: Define the term sensing signal as a reference signal or synchronization signal (e.g., UE or gNB) which is transmitted or received as part of an ISAC sensing operation. Proposal 2: Define an ISAC sounder as a RAN entity (e.g., UE or gNB) which transmits a sensing signal to be measured for an ISAC sensing operation. Proposal 3: Define an ISAC sensor as a RAN entity (e.g., UE or TRP) which receives a sensing signal to be measured for an ISAC sensing operation. Proposal 4: Define a monostatic sensing operation as an ISAC sensing operation in which a single RAN entity (e.g., UE or TRP) acts as the sole ISAC sounder and sole ISAC sensor. Proposal 5: Define a bistatic sensing operation as an ISAC sensing operation in which a single RAN entity (e.g., UE or TRP) acts as the sole ISAC sounder while a different RAN entity acts as the sole ISAC sensor. Proposal 6: Define a target as an entity, RAN or non-RAN, which is illuminated by an ISAC sounder with a sensing signal for the purpose of being measured by an ISAC sensor during an ISAC sensing operation. Proposal 7: Define a multi-static sensing operation as an ISAC sensing operation in which multiple RAN entities (e.g., UE or TRP) act as either ISAC sounder or sensor. Proposal 9: Define new monostatic channels for sensing modes a. and c. and cross-link channels between proximate TRPs and UEs for sensing modes b. and d. and define calibration scenarios for all new sensing modes. |
| [20] NTPU | Proposal 2: Primacy of TRP Monostatic Channel Modeling |
| [21] China Telecom | Proposal 2: Study TRP-based sensing mode with high priority, including both TRP monostatic and TRP-TRP bistatic sensing mode. Proposal 3: At least the combination of UAV deployment scenario, TRP monostatic mode should be prioritized in ISAC channel modelling. |
| [23] Samsung | Proposal 4 RAN1 reuses/refers to the sensing related terminology as defined in TS22.137 or TR22.837 for the study. - Sensing transmitter (Tx): the entity that sends out the sensing signal which the sensing service will use in its operation. A sensing transmitter can be either a BS or a UE. A sensing transmitter can be located in the same or different entity as the sensing receiver - Sensing receiver (Rx): the entity that receives the sensing signal which the sensing service will use in its operation. A sensing receiver can be either a BS or UE. A sensing transmitter can be located in the same or different entity as the sensing transmitter - Sensing target: target that need to be sensed by deriving characteristics of the objects within the environment with the certain sensing service quality from the impacted wireless signal (i.e., target in interest) - Environment background/target: background and/or target that no need to be sensed within the environment with the certain sensing service quality from the impacted wireless signal (i.e., target that not in interest) - Mono-static sensing: sensing where the sensing transmitter and sensing receiver are co-located in the same entity - Bi-static sensing: sensing transmitter and sensing receiver are located in different entities - Sensing signal: signals being transmitted on the 3GPP radio interface that can be used for sensing purpose - Forward signal: signals being transmitted on the sensing transmitter to sensing target (arrival : sensing transmitter and departure : sensing target) - Echo signal: signals being reflected and/or scattered on the sensing target to sensing receiver (arrival : sensing target and departure : sensing receiver) Proposal 6 Discuss how to make working scope of channel modeling for 6 sensing modes and whether RAN1 need to focus on specific some of sensing mode |
| [29] CAICT | Observation1: For each sensing scenario, it usually doesn’t need to use all of six sensing modes. Proposal1: It is suggested to down select the sensing modes with low priority for each scenario, and thus to simplify the work of validation and calibration of ISAC channel modelling. |
| [30] Toyota | Proposal 3: RAN1 to define ISAC channel modelling that is flexible enough to support all the six sensing modes from the SID (i.e., TRP-TRP bistatic, TRP monostatic, TRP-UE bistatic, UE-TRP bistatic, UE-UE bistatic, UE monostatic). |
| [33] Panasonic | Proposal 2 Consider the following prioritization in sensing modes for each of the target objects of interest: |
| [36] ZTE |  |
| [40] Continental Automotive | Proposal 2: For the different sensing targets, consider the prioritization in terms of deployment types and sensing modes shown in Table 1. |
| [41] AT&T, FirstNet | Proposal 3: RAN1 considers UE-based sensing, both monostatic and bi (multi-) static, with lower priority Proposal 4: For ISAC channel modelling development, consider both monostatic and bistatic (multi-static) sensing |

Discussion: Based on the contributions for this meeting, there are a wide variety of inputs on the sensing modes, how they should be treated, relative to the use cases/deployment scenarios, and priority. It should be noted that the SID for ISAC channel modelling states the following:

All six sensing modes should be considered (i.e. TRP-TRP bistatic, TRP monostatic, TRP-UE bistatic, UE-TRP bistatic, UE-UE bistatic, UE monostatic).

Additionally, there were quite a few contributions that mention “multi”-static vs. bi-static. There seems to be some need to define these sensing modes/targets and perhaps also define/include multi-static as several companies also expressed support for defining this additional mode.

## Proposal 4-1 (FL1)

Proposal 4-1: For ISAC use cases and sensing targets, RAN1 should consider all six sensing modes

NOTE: Any (de)prioritization based on sensing mode is FFS.

- .

Companies can provide comments/inputs in the following table:

| Company | Comment |
| --- | --- |
| CMCC | Support |
| ZTE | Support. For UAV case, more antennas are needed for high angle accuracy estimation and high transmission power for large coverage, therefore, TRP only scenario is more suitable. |
| LGE | We support the main sentence. We don’t support the sub-bullets since all 6 sensing modes are already discussed and determined at RAN plenary meeting. We don’t need to iterate the same discussion in RAN1 meeting. |
| vivo | As we pointed out in the beginning, the deployment scenario should be independent of use cases and sensing modes. The channel modelling should be unified for any use case and sensing mode. Considering the reuses of TR38.901, RAN1 should focus on the sensing mode(s) based on Uu link, as a starting point. In addition, the 2nd sub-bullet is not that clear to us in terms of its intention. We believe that the down-scope is too large. |
| CICTCI | - For UAV case, we agree that it is not reasonable to take the UE (UAV) as the sensing transmitter or receiver considering that the high speed will affect the sensing accuracy. - For human case, the RSU (Road side unit) as a UE-type equipment which also has a fixed location similar as gNB can be considered as sensing transmitter or sensing receiver. Thus, the following update is proposed for Proposal 4-1. Proposal 4-1: For ISAC use cases and sensing targets, RAN1 should consider all six sensing modes with the following exceptions: - UAV sensing target should deprioritise sensing modes where the UE is either a sensing transmitter or receiver. - For Human sensing target RAN1 should consider deprioritising sensing modes where there is no TRP/gNB and RSU involvement. |
| InterDigital | Regarding the first bullet about UAV, we are not sure why involvement of UE is deprioritized. For example, the use case in 5.10 in TR 22.837 states that UEs can be involved in sensing. The use case in 5.12 in TR 22.837 also infers involvement of UE in sensing. |
| Sharp | Supporting all six sensing modes can be very demanding for channel modelling validation. We should ensure that we have sufficient time for channel model validation if we want to support all 6-sensing mode without any prioritization. However, if no company is interested in prioritization we support the above proposal. |
| CATT | Support. For UAV, it is more typical that only TRP can provide sensing service. The distance between UAV and ground UE is too large so as the pathloss. For human, it is about life. We agree that the involvement of TRP should be guaranteed for robust and continuous sensing. |
| MTK | Share the similar view with InterDigital. It seems that the proposal is not needed, which sensing mode can be used for each use case from the deployment scenario, it can be up to company’s interest or real implementation. |
| Ericsson | For UAV sensing target, we think gNB-UE bistatic mode needs to be considered. The number of UEs is orders of magnitude more than that of gNB. So even though the LOS probability to a UAV is typically lower for a specific UE than a specific gNB, it is very likely that there will be at least as many or possibly more UEs than gNBs with LOS to the UAV. Lastly this sensing mode can work well for indoor UAVs in indoor factory scenario. |
| Xiaomi | Support. |
| Spreadtrum | Support the main bullet at least. We assume there will not be channel modelling discussion at the beginning of 6G system design. So all six sensing modes should be supported. |
| Sony | We have similar view as InterDigital. We would prefer not to have any exceptions at this stage. UE can still be involved in sensing procedure, including in UAV case. For example, PRU (i.e., A UE at a known location) or PRU-like can be utilized. |
| Qualcomm | We agree that we should consider all 6 sensing modes in principle and if we can reach consensus on what to exclude, we could discuss this aspect, but if no consensus is reached, then all are included. On the 2 cases that are proposed to be excluded above: - For UAV sensing, another UE (e.g. UAV) could be receiving the reflected signals, or transmitting to sense other UEs. - For human sensing, we don’t agree deprioritizing the modes without TRP/gNB involvement. We would be excluding main cases in which a UE is picking up reflections (e.g. UE-UE monostatic, UE-UE bi-static, UE being an RSU, etc) which can be very useful for the ecosystem to be studied. Based on the above, we don’t think that we currently agree on deprioritizing the above. |
| OPPO | We disagree with the two sub-bullets. From channel model point of view, there is no difference between TRP and UE. The model only cares about Tx and Rx. The two bullets do not simplify the modeling methodology but put great limitation on the applicability of the model. |
| Nokia | Support main proposal but think that UE as a sensor could be relevant for UAV tracking in both indoor and outdoor scenarios. |
| CEWiT | Support new modification. |
| Apple | Support updated proposal. All modes should be supported. |
| Samsung | Support new modification |
| New H3C | OK |
| Continental Automotive | Support. Each sensing mode has advantages for at least certain type of targets. |

## Proposal 4-2 (FL1)

Proposal 4-2: For ISAC channel modelling, RAN1 reuses/refers/confirms the sensing related terminology as defined in TS22.137 or TR22.837 for the study as follows:

- Sensing transmitter (Tx): the entity that sends out the sensing signal which the sensing service will use in its operation. A sensing transmitter can be either a TRP or a UE. A sensing transmitter can be located in the same or different entity as the sensing receiver

- Sensing receiver (Rx): the entity that receives the sensing signal which the sensing service will use in its operation. A sensing receiver can be either a TRP or UE. A sensing receiver can be located in the same or different entity as the sensing transmitter

- Sensing target: target that need to be sensed by deriving characteristics of the objects within the environment with the certain sensing service quality from the impacted wireless signal (i.e., target in interest)

- Environment background: background that does not need to be sensed within the environment with the certain sensing service quality from the impacted wireless signal

- Mono-static sensing: sensing where the sensing transmitter and sensing receiver are co-located in the same entity

- Multi/Bi-static sensing: sensing transmitter(s) and sensing receiver(s) are located in different entities

- Sensing signal: signals being transmitted on the 3GPP radio interface that can be used for sensing purpose

Companies can provide comments/inputs in the following table:

| Company | Comment |
| --- | --- |
| ZTE | Generally OK. |
| LGE | We’re fine with the proposal |
| vivo | We are basically fine with the proposal, with the following modifications. For 2), it should be modified as Sensing receiver (Rx): the entity that receives the sensing signal which the sensing service will use in its operation. A sensing receiver can be either a BS or UE. A sensing transmitter receiver can be located in the same or different entity as the sensing transmitter. For 4), if the target is not used for sensing, it should be not defined. Thus, the definition should be modified as Environment background/target: background and/or target that does not need to be sensed within the environment with the certain sensing service quality from the impacted wireless signal (i.e., target that is not of interest). In addition, in 1) and 2), BS needs to be changed to TRP in order to have the terminology consistent. |
| CICTCI | We may need to firstly discuss whether/how to support multi-static sensing mode which is not listed in the SID before terminology definition. |
| InterDigital | Support |
| Sharp | Support. Typo in bullet 2. - A sensing transmitter receiver can be in the same or different entity as the sensing transmitter. In bullet 4 refrain from using the word target. It should just be environment background. |
| CATT | Mostly support. For 4), we prefer the original terminology in SID, i.e. ‘background environment’. |
| Ericsson | In ISAC Channel Modelling SID: The study should aim at a common modelling framework capable of detecting and/or tracking the following example objects and to enable them to be distinguished from unintended objects: We think the unintended objects belong to 4. environment background/target. They need to be sensed, so that sensing receivers can distinguish them from the sensing targets. |
| Xiaomi | Not sure what “entity” means here. For mono-static sensing, does it mean the transmitter and receiver are the same entity, or transmitter and receiver can be different entity with the same location? |
| Toyota ITC | Support the proposal with a correction of the bullet 2 as commented by vivo and Sharp. |
| Spreadtrum | Support |
| AT&T | Ok with this proposal. Agree with the modifications suggested by Sharp and CATT |
| Huawei, HiSilicon | We think it should be better to split the environment into two parts: Environment target: the scatter in the Tx-Rx propagation channel, which contribute main power in the channel and can interact with the sensing target (e.g. the wall and ground). Background clutters: the scatter in the Tx-Rx propagation channel, which contribute small power in the channel and do not interact with the sensing target ( e,.g. the tree). |
| SONY | Support |
| OPPO | Support. In addition, from scenario point of view, both sensing Tx and sensing Rx can be unified as sensing entity, because it is not precluded one sensing Tx cannot be the sensing Rx at the same time. |
| Nokia | Ok with proposal. |
| CEWiT | WE are fine with the proposal |
| Apple | OK |
| Samsung | Ok with proposal |
| New H3C | OK |
| Continental Automotive | Support |

# Topic #3: Initial evaluation parameters/principles

The related proposals are copied below.

| [4] Spreadtrum Communications | Proposal 3: Adopt Table 2.1 on evaluation parameters for UAVs in UMi, UMa and RMa scenarios. Proposal 4: Adopt Table 2.2 on evaluation parameters for humans in UMi, UMa, Indoor Office, Indoor Factory and RMa scenarios. Proposal 5: Adopt Table 2.3 on evaluation parameters for automotive vehicles in UMi, UMa and RMa scenarios. Proposal 6: Adopt Table 2.4 on evaluation parameters for automated guided vehicles in Indoor Factory scenario. Proposal 7: For the use case of objects creating hazards on roads/railways, it is suggested to discuss the exact definition first. |
| --- | --- |
| [5] Huawei | Proposal 1: Take Error: Reference source not found as a starting point for discussing scenarios for different use cases and sensing modes. - The table is dimensioned to cover both mono-static and bi-static sensing modes in general. - E.g., for TRP mono-static, the UT related parameters within the table are not applied. |
| [9] NIST | The 80 configurations for the human gesture use case (see detail in [2]) , and the 27 configurations for the human walking use case are applicable to “Human indoors and outdoors”[1]. Furthermore, the 24 vehicle configurations are applicable to “Automotive vehicles (at least outdoors)” [1]. Sensing mode for human gesture is monostatic. Sensing mode for human walking and car sensing is bistatic. We propose to approve this categorization of measurement use cases with the scenarios in the SI [1] and, that the measurement data for the used cases identified above be used for ISAC channel modelling for FR2 (28.5 GHz). |
| [12] CMCC | Proposal 2: For detecting or tracking UAVs, the deployment scenario of UMa should be prioritized. - Use UMa-AV deployment scenario in TR 36.777 as a starting point. Further study on parameters including the ISD, carrier frequency, etc. Proposal 3: Consider the requirements on detection or tracking of UAVs in Table 3 as baseline design targets in future evaluations. Proposal 5: Consider the requirements on detecting or tracking of automotive vehicles, humans, and objects creating hazards on roads in Table 5 as baseline design targets in future evaluations. Proposal 6: Study the ISAC channel modeling for both FR1 and FR2. |
| [13] CATT | Proposal 1: Reuse scenarios defined in TR 38.901 for ISAC as much as possible. New scenarios/ configurations/ parameters shall only be introduced if indeed necessary. Proposal 2: For UAV case, the scenarios of UMi-street canyon, UMa and RMa in TR 38.901 can be considered. - BS antenna down-tile may be adjusted or up to companies, when evaluation/calibration is considered. Proposal 3: The following aspects are considered for characterizing UAV. - Characteristics and parameters of UAV in TR 36.777; - Maximum flight height up to 600m; - 3~4 reference sizes (Length x Width x Height) vary from 0.15m x 0.15m x 0.1m to 3m x 3m x 1m. Proposal 4: For humans indoors and outdoors case, the scenarios of Indoor-office, InF, UMi-street canyon and UMa can be considered. Proposal 5: The following aspects are considered for characterizing humans indoors and outdoors. - Human velocity (e.g. 0-20 km/h); - 2-3 reference human sizes (e.g. children and adult) vary from 0.2m x 0.3m x 1m to 0.3 x 0.4m x 1.7m. Proposal 6: For automotive vehicles case, the scenarios of UMi-street canyon, UMa and RMa can be considered. Proposal 7: The characteristics captured in TR 37.885 (e.g. vehicle types/sizes, dropping, mobility) can be considered for characterizing Automotive vehicles. Proposal 8: For AGV case, the scenario of InF (including 5 types sub-cases) defined in TR 38.901 can be considered. Proposal 9: The following aspects are considered for characterizing AGV. - AGV speed: <30km/h. - Trajectory: regular, e.g. linear motion, or turn with a fixed degree (e.g. 90 degree). - AGV sizes: 3 reference sizes (small, medium, large). Proposal 10: For the use case of objects creating hazards on roads/railways, - ‘High Speed Train’ defined in TR 38.802 is considered as the scenario for railway-related objects. - ‘Highway for eV2X’ defined in TR 37.885 is considered as the scenario for road/highway-related objects. Proposal 11: The following objects are considered for railway and road/highway. - Railway: Pedestrian, vehicle, animal, bicycle/motorcycle, and any other obstacle that has comparable size and/or speed to the previous objects. - Road/highway: All the objects that considered for railway, and also vehicles with abnormal parameters (e.g. size, speed, direction). Proposal 12: Consider the prioritized scenarios, sensing modes and frequency range for each use case in Table 1. |
| [16] Xiaomi | Proposal 3: The detail parameters of BS and UT for ISAC channel model may reuse the current scenario descriptions in exiting TR. More specifically, - Indoor office scenario can refer to 38.901 - Highway scenario can refer to 37.885 - UMa-AV scenario can refer to 36.777 Proposal 5: For indoor office scenario, the details are listed Proposal 6: For highway deployment scenario, the details are listed |
| [17] OPPO | Proposal 2: RAN1 takes Table 2-1 and Table 3-1 as starting point for discussion of ISAC deployment scenarios. - Use “sensing entity” and “sensing target”, instead of UE/TRP/Tx/Rx, in description of sensing deployment scenarios. |
| [18] Sharp | Proposal 3: The descriptions in Section 6.2 of TR 38.901 [1] can be reused for ISAC, but ISD may be chosen carefully based on the desired/maximum sensing range (which may depend on sensing mode, frequency, target). Proposal 7: For UMi, UMa, RMa deployments consider FR1, FR3, FR2 (less preferred) bands as they provide reasonable coverage and accuracy [5,6]. If the sensing requirements (accuracy, resolution, etc.) for outdoor sensing applications cannot be met by the FR1 or FR3 bands, the FR2 band may be a viable alternative. Proposal 8: For InH and InF deployments consider FR2, FR3 bands as they provide smaller coverage but greater sensing accuracy [5]. |
| [19] NTPU | Proposal 3: Strategic Focus on S and X Frequency Bands |
| [23] Samsung | Proposal 5 Study and specify signal directionality Proposal 7 Study and model the ISAC channel considering validation of efficacy for frequency bands Proposal 8 Study the channel parameters for ISAC with possible frequency dependency Proposal 9 study whether/how the mobility of sensing target and/or background environment targets are to be modelled including motion state, velocity and mobility Proposal 10 Study minimum distance between sensing transmitter, receiver and target |
| [24] CICTCI | Observation 1: For the use cases corresponding to the example objects listed in the SID, the sensing transmitter or receiver can be BS or UE. Proposal 1: To minimize to complexity of deployment scenario designs, the following use cases can be prioritized in Rel-19 ISAC. - Sensing for UAV intrusion detection, with BS as the sensing transmitter or receiver - Intruder detection in surrounding of smart home, with BS or UE-type RSU as the sensing transmitter or receiver - Sensing assisted automotive manoeuvring and navigation, with BS or UE-type RSU as the sensing transmitter or receiver - Vehicle sensing for ADAS, with BS or UE as the sensing transmitter or receiver - AGV detection and tracking in factories, with BS (or UE, optionally) as the sensing transmitter or receiver - Pedestrian/animal intrusion detection on a highway, with BS or UE-type RSU as the sensing transmitter or receiver Proposal 2: The deployment scenario including UMi, UMa and RMa for aerial vehicles in TR 36.777 can be treated as a starting point for UAV scenario in Rel-19 ISAC. - It is preferred that one target UAV is modelled per sector, i.e., the ratio of UAV is 7.1%. Proposal 3: Urban scenario in TR 37.885 can be treated as a starting point for human modelling in Rel-19 ISAC. - It is preferred to update the density of pedestrian/human to a lower level. Proposal 4: General principles on highway scenario modeling defined in TR 37.885 can be inherited for scenario deployment related to automotive vehicles in Rel-19 ISAC with the following updates. - It is preferred to introduce N target vehicles to indicate the vehicles that need to be detected or tracked. - The dropping model of RSU in Rel-18 NR sidelink positioning should be reused where RSUs are uniformly dropped on both sides of road and the distance between two adjacent RSUs are 200m Proposal 5: Indoor-factory defined in TR 38.901 can be reused to evaluate automated guided vehicles (AGV) in Rel-19 ISAC. - One or two AGVs are modelled per factory and the speed is up to 30km/h - It is optional whether human is modelled in such scenario. Proposal 6: Highway scenario in TR 37.885 can be treated as a starting point for scenario modelling for objects creating hazards on roads with the introduction of sensing targets. - Two sensing targets per side and total four sensing targets - The location of sensing targets is fixed - The distance between two sensing targets at the same side is 1000m, the distance between sensing target and highway is 5m |
| [25] LG | Proposal 1: For evaluation scenario of TRP-related monostatic and bistatic sensing, UMa, UMi, RMa, indoor office and indoor factory scenario defined in TR38.901 are reused for outdoor and indoor use case. Proposal 2: For evaluation scenario of UE-only-related monostatic and bistatic sensing, urban grid and highway for V2X defined in TR37.885 are reused. Proposal 3: For UAV detection use case, UMi-AV, UMa-AV, RMa-AV defined in TR36.777 are the starting point of the evaluation scenarios. |
| [27] Sony | Proposal 1: Use the existing NR deployment scenario as the starting point and identify the deployment scenario to evaluate different type of use-cases. Proposal 2: Consider the following mapping of object target and existing deployment scenario as the starting point: – Dense urban, urban macro, and/or rural scenarios with added UAV for UAV deployment scenario. – Urban grid scenario for connected car scenario for Automotive vehicles deployment scenario. – Indoor factory and/or indoor hotspot scenarios for AGV deployment scenario. Proposal 3: Introduce sensing target(s) parameters, such as RCS, velocity, and its distribution in a selected deployment scenario. Other parameters may be required and for further study. Proposal 4: Reuse the UE and gNB hardware assumptions as in the existing NR deployment scenario. Proposal 5: UAV (as the sensing target) parameters such as RCS, speed/velocity, and UAV distribution should be considered in the deployment scenario. Proposal 6: Vehicles (motor/non-motor, and as the target object) parameters such as RCS, speed/velocity, and the vehicles distribution should be considered in the deployment scenario. Proposal 7: AGV (as the sensing target) parameters such as speed/velocity, RCS, and AGV distribution should be considered in the deployment scenario. |
| [29] CAICT | Obervation2: Scenarios assumptions including UMa-AV/UMi-AV/RMa-AV in TR 36.777, UMi-street canyon/UMa/RMa/InH/Indorr-office/Indoor Factory in TR 38.901, urban grid/highway for eV2X in TR 37.885, and high speed train TR38.802 can be used as baselines for sensing assumptions. Proposal2: UMa-AV/UMi-AV/RMa-AV for UAVs sensing, urban gird/highway for humans/cars/hazardous objects sensing, and InH/Indoor-office for humans sensing can be considered preferentially for channel model calibration. Proposals3: In the sensing assumptions, it should include parameters related to the sensing target (e.g., target mobility, number of targets, RCS, target distribution, and target location) and min BS/UT-target distance and set frequency and bandwidth for sensing purpose. |
| [41] AT&T, FirstNet | Proposal 2: RAN1 consider high FR1 bands and mm Wave (FR2-1) bands as priority bands for ISAC channel modelling. |

## Proposal 5-1 (FL1) - Placeholder

Proposal 5-1: For ISAC evaluation parameters, agree on the following considerations as a starting point:

- To be determined following consensus/agreements on Topics #1 and #2.

Companies can provide comments/inputs in the following table:

| Company | Comment |
| --- | --- |

# Topic #3: Other Proposals (placeholder)

The related proposals are copied below.

| [15] Ericsson | Proposal 1 To model sensing objects, study parameters of their locations, trajectories, radar cross-sections, and clutter types. Proposal 2 RAN1 to define new sensing scenarios and ensure that the sensing-related model additions to TR 38.901, when enabled, do not have an undue effect on perceived communication quality and potential conclusions thereof. |
| --- | --- |
| [16] Xiaomi | Proposal 4: Target related parameters should be introduced for each deployment scenario, including at least: - type of target, target location (Outdoor/indoor, LOS/NLOS, height of target), target distribution(2D/3D), target velocity, min BS-to-target/target-to-UT/target-to-target distance(2D/3D), sensing modes |
| [17] OPPO | Proposal 3: RAN1 prioritizes LOS condition over NLOS for the link between sensing entity (Tx/Rx) and sensing target. |
| [19] Nokia | Proposal 1: Define the term sensing signal as a reference signal or synchronization signal (e.g., UE or gNB) which is transmitted or received as part of an ISAC sensing operation. Proposal 2: Define an ISAC sounder as a RAN entity (e.g., UE or gNB) which transmits a sensing signal to be measured for an ISAC sensing operation. Proposal 3: Define an ISAC sensor as a RAN entity (e.g., UE or TRP) which receives a sensing signal to be measured for an ISAC sensing operation. Proposal 4: Define a monostatic sensing operation as an ISAC sensing operation in which a single RAN entity (e.g., UE or TRP) acts as the sole ISAC sounder and sole ISAC sensor. Proposal 5: Define a bistatic sensing operation as an ISAC sensing operation in which a single RAN entity (e.g., UE or TRP) acts as the sole ISAC sounder while a different RAN entity acts as the sole ISAC sensor. Proposal 6: Define a target as an entity, RAN or non-RAN, which is illuminated by an ISAC sounder with a sensing signal for the purpose of being measured by an ISAC sensor during an ISAC sensing operation. Proposal 7: Define a multi-static sensing operation as an ISAC sensing operation in which multiple RAN entities (e.g., UE or TRP) act as either ISAC sounder or sensor. Proposal 8: Adopt at least the models proposed in Error: Reference source not found for study in 3GPP ISAC study. Proposal 9: Define new monostatic channels for sensing modes a. and c. and cross-link channels between proximate TRPs and UEs for sensing modes b. and d. and define calibration scenarios for all new sensing modes. |
| [22] NVIDIA | Proposal 1: Define a common reference scenario for ray tracing to be used in ISAC evaluation. Proposal 2: Select one the following option to define a common reference scenario for ray tracing to be used in ISAC evaluation: - Option 1: Real-scenario map that is a virtual representation of a real area on earth. - Option 2: Synthetic-scenario map that is artificially constructed to mimic a certain environment such as urban macro, rural macro, indoor office, or indoor factory. Proposal 3: Describe the scene geometry and the characteristics of the materials involved in the common reference scenario for ray tracing to be used in ISAC evaluation. |
| [28] EURECOM | Proposal 1: Latency requirement of ISAC is strict for some applications. Mechanism to reduce latency of ISAC operation should be studied. Proposal 3: All of three coverage scenarios: in-coverage scenario, out-of-coverage scenario and partial-coverage scenario are supported. Proposal 4: Both Uu interface and PC5 interface are supported. Proposal 5: Licensed and unlicensed spectrum are considered for ISAC operation. |
| [30] Toyota | Proposal 1: For object detection and/or tracking of automotive vehicles, RAN1 to consider outdoor scenarios (e.g., highway, urban intersections, general roads) and indoor scenarios (e.g., indoor/underground parking, tunnels). Proposal 2: For objects creating hazards on roads/railways, RAN1 to consider motorcyclists, vulnerable road users (e.g., pedestrian, road workers, cyclists), animal (e.g., deer), debris from vehicles (e.g., tires, wheels, cargo), natural debris (e.g., rocks, fallen trees). |
| [39] IIT Kanpur | Proposal 2: Prioritize the use of 3GPP 38.901 channel modelling as a starting point, with modifications as needed, to include sensing for different deployment scenarios at least with • Sensing target characteristics, • Sensing background/noise/clutter characteristics, • Channel characteristics as per mode of operation, • Channel characteristics for different frequencies of operation, • Different sensing KPIs • Other options are not precluded |
| [40] Continental Automotive | Proposal 1: Decisions and agreements made by RAN1 in this SI (FS_ISAC_NR) should be based on and consistent with the assumption that the ISAC SI targets “sensing as a service” as priority. Proposal 3: For the work to be done in the ISAC SI (FS_ISAC_NR), consider the work segmentation/division illustrated in Figure 1. Proposal 4: Automotive-relevant ISAC services include, at least, the scenarios indicated in Table 2. These involve both outdoor and indoor situations, in- and out-of-coverage, and potentially supported by Uu- and/or PC5-based sensing. |
| [41] AT&T, FirstNet | Proposal 5: RAN1 discuss and agree that defining and evaluating sensing KPIs should be in scope of the SI, Proposal 6: Sensing KPIs to be evaluated at least include range and velocity resolution, probability of missed detection and false alarm, unambiguous range, and unambiguous velocity. |
| [42] Qualcomm | Proposal 1: Consider cases where only a single scatter-point is dropped into the scenario. The drop may be random or may be at a specific location in the scenario layout, relative to other nodes (e.g., relative to gNB(s) and/or UE(s)). Proposal 2: Consider the simplified models of specific objects with gain-functions as proposed in this contribution, as building blocks to model actual sensing targets (such as humans, AGVs, UAVs, vehicles, hazard objects, etc) and clutter. |

## Proposal 6-1 (FL1) - Placeholder

Proposal 6-1: TBD

Companies can provide comments/inputs in the following table:

| Company | Comment |
| --- | --- |

# Round 2 Proposals (For Thursday online session)

## Proposal 3-0.1 (FL2)

Based on the official offline discussion, to get a common understanding of the definition of deployment scenarios for ISAC, the following proposal is made:

[HIGH] Proposal 3-0.1: As a starting point for defining ISAC deployment scenarios, the following parameters may be considered for each of the sensing targets defined in [1]. Additional parameters are not precluded.

Parameters

Carrier frequency

Cell layout (e.g., ISD, grid, no. of sites/sectors, etc.)

Sensing transmitter location (e.g., Outdoor/indoor, Height, etc.)

Sensing receiver location (e.g., Outdoor/indoor, LOS/NLOS, height, mobility)

Min. sensing transmitter – sensing target distance

Background environment/clutter characteristics

Moderator comments. Based on offline discussion, the following proposal replaces the proposals in 7.0, 7.2-7.7.

## Proposal 3-0.2 (FL3) – For Online Discussion

[HIGH] Proposal 3-0.2: For progressing ISAC deployment scenarios/channel modelling, the following use cases/sensing targets and corresponding deployment scenarios will be considered as a starting point:

| Sensing Targets | Deployment scenarios |
| --- | --- |
| UAVs | RMa-AV, UMa-AV, UMi-AV (TR 36.777) |
| Humans indoors | INF, Indoor Office, Indoor Room (TR 38.808) |
| Humans outdoors | UMi, UMa |
| Automotive vehicles (at least outdoors) | Highway and Urban grid |
| Automated guided vehicles (e.g. in indoor factories) | INF |
| Objects creating hazards on roads/railways (examples defined in TR 22.837) | Highway and Urban grid, HST |

Companies can provide comments/inputs in the following table:

| Company | Comment |
| --- | --- |
| Ericsson | We are supportive but would like to ensure that we reuse deployment parameters in e.g. 38.901 as much as possible. This could be done by defining sensing scenarios, e.g. “UAV” or “human indoor”, that could be paired with any of the compatible existing scenarios such as e.g. UMa or RMa. This way, the existing scenarios would specify the network geometry while the sensing scenario specifies the targets and clutter geometries. The parameters in the new sensing scenario do not need to include parameters already specified in the existing scenario but can instead be e.g.: Target and clutter types Positions and orientations Trajectories and velocity Radar cross sections Some examples of such pairing could be “UAV” + UMa, “UAV” + RMa, or “human indoor” + InF., Note: Additional channel model parameters for the sensing add-on may need to be specified per sensing scenario, for example the LOS probability between the Tx and target and between the Rx and target. |
| Moderator | Moderator comments. Based on offline discussion, the proposal in 7.1 Proposal 3-0.2 replaces the proposals in 7.0, 7.2-7.7. |
| EURECOM | UMa, UMi, Rma can be considered for automotive vehicles. |
| Ericsson | The deployment scenarios from TR38.901 should be considered for the geometry of the active UEs and BS, while the use of the other TRs is only for reference. For example, deployment scenarios from other TRs can be used to define the geometry of the passive objects of the sensing targets, for example UAVs in TR 36.777. Note that, in the other TRs, the relevant objects are active devices, namely UAVs are UEs. They should be considered as passive sensing targets when defining the sensing targets. Hence these deployment scenarios cannot be used to define the geometry for the active UEs and BS, which may be sensing transmitters and/or receivers. We suggest to update the table of the proposal as follows: Sensing Targets (target and clutter deployment); Deployment scenarios (active UEs and BS deployment). UAVs; RMa, UMa, Umi, InF. Humans indoors; InF, Indoor Office, UMa, UMi. Humans outdoors; UMi, Uma, RMa. Automotive vehicles (at least outdoors); UMa, UMi, RMa. Automated guided vehicles (e.g. in indoor factories); InF. Objects creating hazards on roads/railways (examples defined in TR 22.837); , UMa, RMa. It would also be expedient to progress, if a separate proposal could define the sensing parameters. For example: - Target and clutter types - Positions and orientations - Trajectories and velocity - Radar cross sections |
| Panasonic | We share the same view as Ericsson. Perhaps 36.777 would be suited for TRP-monostatic or TRP-bistatic sensing, but 38.901 is better suited for TRP-UE bistatic or UE-TRP bistatic, but here this also requires a discussion to define the UE in this case. In principle, we are trying to detect a UAV in a certain environment, with the UAV possibly not involved in any communication. We prefer to use what Ericsson suggested. |
| SONY | Is this a general understanding that indoor room is only for the case of above 52 GHz case? If so, we need to mention it. In general, we prefer the updated input made by Ericsson |
| Huawei, HiSilicon | In general support this proposal from FL and prefer not to take the suggestion from Ericsson. If it is bi-static sensing modes with TRP-UE as transmitter/receiver, the scenario defined in TR38.901 would be naturally reused as much as possible by default. What we need to define is how to drop the sensing target. For example, UAV, referring to TR36.777 is exactly taking sensing target as aerial UE for the scenario deployment, basically defining the height, speed, and minim distance between transmitter/receiver and sensing target (UAV). |
| Toyota ITC | Support. At least highway and urban grid should be considered for automotive vehicles and objects creating hazards on roads/railways. |

## Proposal 3-1.2 (FL2)

[HIGH] Proposal 3-1.2: For progressing ISAC deployment scenarios/channel modelling, all of the following use cases/targets will be addressed as part of the SI:

- UAVs

- Humans indoors and outdoors

- Automotive vehicles (at least outdoors)

- Automated guided vehicles (e.g. in indoor factories)

- Objects creating hazards on roads/railways (examples defined in TR 22.837)

NOTE: (De)prioritization of certain deployment scenarios for the above sensing targets is not precluded, dependent upon RAN1 progress.

Companies can provide comments/inputs in the following table:

| Company | Comment |
| --- | --- |
| Ericsson | Support |
| Moderator | Moderator comments. Based on offline discussion, the proposal in 7.1 Proposal 3-0.2 replaces the proposals in 7.0, 7.2-7.7. |
| EURECOM | Support |
| SONY | Support. |

## Proposal 3-2.2 (FL2)

[HIGH] Proposal 3-2.2: As a starting point for the UAV use cases, reuse the deployment scenarios for an aerial UE defined in TR 36.777 as a sensing target.

FFS modifications as needed for UAV detection and tracking

Companies can provide comments/inputs in the following table:

| Company | Comment |
| --- | --- |
| Ericsson | Please see our comments to proposal 3-0.1 To reiterate, the existing deployment scenarios in TR 38.901 should be used to specify the network geometry, while the sensing scenario should specify the target and clutter geometry. Parts of this sensing geometry can be taken from 36.777, but we should keep in mind that in 36.777 the UAVs are actively communicating 3GPP UEs while the sensing targets in some of the considered use cases may be uncooperative UAV that actively try to avoid detection. |
| Moderator | Moderator comments. Based on offline discussion, the proposal in 7.1 Proposal 3-0.2 replaces the proposals in 7.0, 7.2-7.7. |
| EURECOM | The parameters from TR 38.901 can be reused with some parameters from TR 36.777. |
| SONY | At this stage, we could just write both TRs. (36.777 and 38.901). |

## Proposal 3-3.2 (FL2)

[HIGH] Proposal 3-3.2: As a starting point for the human indoor use cases, reuse the deployment scenario for an Indoor Office evaluation based on the channel modelling details of TR 38.901. As a starting point for the human outdoor use cases, reuse the deployment scenario for UMi street canyon based on the channel modelling details of TR 38.901.

- FFS modifications as needed for human detection and tracking

Companies can provide comments/inputs in the following table:

| Company | Comment |
| --- | --- |
| Ericsson | Please see our comments to proposal 3-0.1 To reiterate, the existing deployment scenarios in TR 38.901 should be used to specify the network geometry, while the sensing scenario should specify the target and clutter geometry. The human indoor sensing scenario can be used together with different existing deployment scenarios such as InH, InF, UMa, UMi, RMa. Similarly, the human outdoor sensing scenario can be paired with existing UMa, UMi, RMa. |
| Moderator | Moderator comments. Based on offline discussion, the proposal in 7.1 Proposal 3-0.2 replaces the proposals in 7.0, 7.2-7.7. |
| EURECOM | Support |

## Proposal 3-4.2 (FL2)

[HIGH] Proposal 3-4.2: As a starting point for the automotive vehicles use cases, reuse the deployment scenarios for the evaluation on V2X defined in TR 37.885, and compatible deployment scenarios from Section 7.2 in TR38.901.

- FFS modifications as needed for automotive vehicle detection and tracking

Companies can provide comments/inputs in the following table:

| Company | Comment |
| --- | --- |
| Ericsson | Please see our comments to proposal 3-0.1 To reiterate, the existing deployment scenarios in TR 38.901 should be used to specify the network geometry, while the sensing scenario should specify the target and clutter geometry. Parts of this sensing geometry can be taken from 37.885, but we should keep in mind that in 37.885 the vehicles have actively communicating 3GPP UEs while the sensing targets can include a broader category of vehicles. |
| Moderator | Moderator comments. Based on offline discussion, the proposal in 7.1 Proposal 3-0.2 replaces the proposals in 7.0, 7.2-7.7. |
| EURECOM | Support |

## Proposal 3-5.2 (FL2)

[HIGH] Proposal 3-5.2: As a starting point for the Automated Guided Vehicles (AGV) use cases, reuse the deployment scenarios for an indoor factory evaluation with the parameters defined in TR 38.857, based on the channel modelling details of TR 38.901.

- FFS modifications as needed for AGV detection and tracking

Companies can provide comments/inputs in the following table:

| Company | Comment |
| --- | --- |
| Ericsson | Please see our comments to proposal 3-0.1 To reiterate, the existing deployment scenarios in TR 38.901 should be used to specify the network geometry, while the sensing scenario should specify the target and clutter geometry. Parts of this sensing geometry can be taken from 38.857. |
| Moderator | Moderator comments. Based on offline discussion, the proposal in 7.1 Proposal 3-0.2 replaces the proposals in 7.0, 7.2-7.7. |
| EURECOM | Support |

## Proposal 3-6.2 (FL2)

[HIGH] Proposal 3-6.2: As a starting point for the use cases of objects creating hazards on roads/railways, reuse the deployment scenarios of urban grid and highway for the evaluation on V2X defined in TR 37.885, and reuse UMa, UMi, RMa defined in TR38.901.

- FFS modifications as needed for detection of hazards on roads/railways

Companies can provide comments/inputs in the following table:

| Company | Comment |
| --- | --- |
| Ericsson | Please see our comments to proposal 3-0.1 To reiterate, the existing deployment scenarios in TR 38.901 should be used to specify the network geometry, while the sensing scenario should specify the target and clutter geometry. Parts of this sensing geometry can be taken from 37.885, but we should keep in mind that in 37.885 does not consider additional hazardous objects so these need to be additionally specified in the sensing scenario. |
| Moderator | Moderator comments. Based on offline discussion, the proposal in 7.1 Proposal 3-0.2 replaces the proposals in 7.0, 7.2-7.7. |
| EURECOM | Support |

## Proposal 3-7.2 (FL2) – For Online Discussion

[HIGH] Proposal 3-7.2: For ISAC use cases and sensing targets, RAN1 should consider all six sensing modes with the following exceptions:

NOTE: Future (de)prioritization based on sensing mode is not precluded.

Companies can provide comments/inputs in the following table:

| Company | Comment |
| --- | --- |
| Ericsson | We see no need for this proposal at this stage. Depending on companies’ contributions and progress of the SI it may turn out that some sensing modes are more difficult to model. That would lead to a natural de-prioritization through lack of progress or consensus. |
| EURECOM | This proposal was discussed in RAN planery. We can just follow the agreement from RAN planery meeting. |
| SONY | These are more or less the same wording in the study item (except the note). We think we do not need this proposal. |
| Huawei, HiSilicon | Like the discussion of use case prioritization, we don’t see the need. |
| Toyota ITC | We share the views of other companies. At this moment, we don’t see the need of this proposal. |

## Proposal 3-8.2 (FL2) – For Online discussion

[HIGH] Proposal 3-8.2: For ISAC channel modelling, RAN1 uses the sensing related terminology as defined in TS22.137 or TR22.837 as a starting point for discussion purposes with the following definitions:

- Sensing transmitter (Tx): the entity that sends out the sensing signal which the sensing service will use in its operation. A sensing transmitter can be either a TRP or a UE. A sensing transmitter can be located in the same or different entity as the sensing receiver.

- Sensing receiver (Rx): the entity that receives the sensing signal which the sensing service will use in its operation. A sensing receiver can be either a TRP or UE. A sensing receiver can be located in the same or different entity as the sensing transmitter.

- Sensing target: target that need to be sensed by deriving characteristics of the objects within the environment from the impacted wireless signal (i.e., target of interest).

- Background environment: background (clutter and/or environmental objects) that are not the sensing target(s).

- Mono-static sensing: sensing where the sensing transmitter and sensing receiver are co-located in the same entity

- Bi-static sensing: sensing where the sensing transmitter and sensing receiver are in different entities.

- Multi-static sensing: sensing where there are multiple sensing transmitters and/or multiple sensing receivers..

- Sensing signal: Transmissions on the 3GPP radio interface that can be used for sensing purposes.[

Companies can provide comments/inputs in the following table:

| Company | Comment |
| --- | --- |
| Ericsson | Supportive. We note that a SLS channel model that supports bi-static sensing will by definition also support multi-static sensing. |
| Moderator | Based on the offline discussion, proposal is updated to align with RAN1 common understanding and SA TR/TSs definitions. |
| EURECOM | Support |
| Panasonic | In 38.901, “background environment” term is not used, but only “environment”. We prefer to stick to “Environment” for consistency with the communication channel terminology. The word “background” is not adding much information. |
| Sony | Support |
| Toyota ITC | Support |

# Round 3 Final Proposals for Thursday Online Session

Moderator comments. Based on the feedback in the offline session(s) and on the email discussion, it seems that the majority view is to remove the proposal for prioritization of the sensing modes. It is also the view to consolidate the deployment scenario proposals into the following table, as a starting point. Defining some general sensing definitions as a starting point for discussion purposes is also essential to have a common understanding going forward to defining ISAC deployment scenarios.

## Proposal 3-0.3 (FL3) – For Online Discussion

[HIGH] Proposal 3-0.3: For progressing ISAC deployment scenarios/channel modelling, the following use cases/sensing targets and corresponding deployment scenarios will be considered as a starting point:

| Sensing Targets | Deployment scenarios |
| --- | --- |
| UAVs | RMa-AV, UMa-AV, UMi-AV (TR 36.777) |
| Humans indoors | InF, Indoor Office, [Indoor Room (TR 38.808)] |
| Humans outdoors | UMi, UMa |
| Automotive vehicles (at least outdoors) | Highway and Urban grid, UMa, UMi, RMa |
| Automated guided vehicles (e.g. in indoor factories) | InF |
| Objects creating hazards on roads/railways (examples defined in TR 22.837) | Highway and Urban grid, HST |

## Proposal 3-8.3 (FL3) – For Online Discussion

[HIGH] Proposal 3-8.3: For ISAC channel modelling, RAN1 uses the sensing related terminology as defined in TS22.137 or TR22.837 as a starting point for discussion purposes with the following definitions:

- Sensing transmitter (Tx): the entity that sends out the sensing signal which the sensing service will use in its operation. A sensing transmitter can be either a TRP or a UE. A sensing transmitter can be located in the same or different entity as the sensing receiver.

- Sensing receiver (Rx): the entity that receives the sensing signal which the sensing service will use in its operation. A sensing receiver can be either a TRP or UE. A sensing receiver can be located in the same or different entity as the sensing transmitter.

- Sensing target: target that need to be sensed by deriving characteristics of the objects within the environment from the impacted wireless signal (i.e., target of interest).

- Background environment: background (clutter and/or environmental objects) that are not the sensing target(s).

- Mono-static sensing: sensing where the sensing transmitter and sensing receiver are co-located in the same entity.

- Bi-static sensing: sensing where the sensing transmitter and sensing receiver are in different entities.

- Multi-static sensing: sensing where there are multiple sensing transmitters and/or multiple sensing receivers.

- Sensing signal: Transmissions on the 3GPP radio interface that can be used for sensing purposes.

# References

- RP-234069	New SID: Study on channel modelling for Integrated Sensing And Communication (ISAC) for NR

- R1-2400574	Proposed work plan on channel modelling for ISAC	xiaomi, AT&T

- R1-2400575	Discussion on CR to introduce channel model for ISAC	xiaomi, AT&T

- R1-2400068	Discussion on ISAC deployment scenarios	Spreadtrum Communications

- R1-2400126	Deployment scenarios for ISAC channel model	Huawei, HiSilicon

- R1-2400155	Discussion for ISAC deployment scenarios	New H3C Technologies Co., Ltd.

- R1-2400167	Overview of ISAC Deployment Scenarios	Tiami Networks

- R1-2401468	Overview of ISAC Deployment Scenarios	Tiami Networks (Revision of R1-2400167)

- R1-2400174	Measurement data for ISAC deployment scenarios	NIST

- R1-2400175	Power Calibration for Radar Cross Section (RCS) Measurement	NIST

- R1-2400256	Views on Rel-19 ISAC deployment scenarios	vivo

- R1-2400341	Discussion on ISAC deployment scenarios	CMCC

- R1-2400447	Discussion on ISAC deployment scenarios	CATT

- R1-2400504	Deployment scenarios for ISAC channel modeling	Intel Corporation

- R1-2400529	Discussion on ISAC Deployment Scenarios	Ericsson

- R1-2400572	Deployment scenarios and evaluation assumptions for ISAC channel model	xiaomi

- R1-2400616	Discussion on ISAC deployment scenarios	OPPO

- R1-2400644	ISAC Deployment Scenarios	Sharp

- R1-2400648	Discussion on ISAC deployment scenarios	Nokia, Nokia Shanghai Bell

- R1-2400652	Proposals for Integrated Sensing And Communication (ISAC) Deployment Scenarios	NTPU

- R1-2400673	Discussion on ISAC deployment scenarios	China Telecom

- R1-2400690	Deployment scenarios for integrated sensing and communication with NR	NVIDIA

- R1-2400746	Discussion on  ISAC deployment scenarios	Samsung

- R1-2400759	Discussion on deployment scenarios for Integrated Sensing and Communication	CICTCI

- R1-2400791	Discussion on ISAC deployment scenarios	LG Electronics

- R1-2400801	Discussion on ISAC Deployment Scenarios	Lekha Wireless Solutions

- R1-2400866	Considerations on ISAC Deployment Scenario	Sony

- R1-2400881	Discussion on ISAC deployment scenarios and requirements	EURECOM

- R1-2400898	Considerations on ISCA deployment scenarios	CAICT

- R1-2400931	Discussion on ISAC deployment scenarios	TOYOTA InfoTechnology Center

- R1-2400935	Discussions on ISAC deployment scenarios	Ruijie Network Co. Ltd

- R1-2400953	FL summary on ISAC deployment scenarios	Moderator (AT&T)

- R1-2400960	ISAC Deployment Scenarios	Panasonic

- R1-2401026	Discussion on ISAC deployment scenarios	Apple

- R1-2401040	Discussion on ISAC deployment scenarios	InterDigital, Inc.

- R1-2401063	Discussion on ISAC deployment scenarios	ZTE Corporation

- R1-2401178	Discussion on ISAC deployment scenarios	ITL

- R1-2401220	Discussion on ISAC deployment scenarios	Lenovo

- R1-2401340	Discussions on ISAC deployment scenarios	IIT Kanpur

- R1-2401346	Deployment scenarios for ISAC	Continental Automotive

- R1-2401355	Views on Deployment Scenarios for ISAC for NR	AT&T, FirstNet

- R1-2401455	Discussion on ISAC deployment scenarios	Qualcomm Incorporated

- R1-2401466	Discussion on ISAC deployment scenario	MediaTek