3GPP TSG RAN WG1 #116								     R1-2401792

Athens, Greece, 26 February - 1 March, 2024

Agenda item:	8.3

Sourcce: 	Moderator (Qualcomm)

Title: 	Moderator Summary #2 on resource allocation for SL PRS

Document for:	Discussion and Decision

# Introduction

At RAN#98, a new work item “Expanded and improved NR positioning” (NR_pos_enh2) was approved [1]. The relevant WID objective is copied here for convenience:

- Specify solutions for support of sidelink positioning (including ranging) in NR systems, including the following [RAN1, RAN2, RAN3, RAN4]: - Specify SL PRS for support of sidelink positioning such that the SL PRS uses a comb-based (full RE mapping pattern is not precluded) frequency domain structure and a pseudorandom-based sequence where the existing sequence of DL-PRS is used as a starting point [RAN1]. - Specify support for SL PRS bandwidths of up to 100 MHz in FR1 spectrum. - NOTE: SL PRS transmission in FR2 is not precluded but no FR2 specific aspects will be specified. - Specify measurements to support RTT-type solutions using SL, SL-AoA, and SL-TDOA [RAN1, RAN2]. - Specify support of resource allocation for SL PRS: - Including resource allocation Scheme 1 and Scheme 2, where Scheme 1 corresponds to a network-centric SL PRS resource allocation and Scheme 2 corresponds to UE autonomous SL PRS resource allocation [RAN1]. - For resource allocation mechanism for SL PRS in Scheme 2: - Study and specify support of sensing-based resource allocation, and/or a random resource selection [RAN1]. - Study and specify solutions for congestion control for SL PRS and/or inter-UE coordination for SL-PRS [RAN1]. - Support resource allocation for shared resource pool with Rel-16/17/18 sidelink communication and dedicated resource pool for SL PRS [RAN1]. - NOTE: For SL positioning resource (pre)-configuration in a shared resource pool with Rel-16/17/18 sidelink communication, backward compatibility with legacy Rel-16/17 UEs should be ensured. - Specify procedures for transmit power control for SL PRS transmissions at least based on open loop power control (OLPC) [RAN1]. - Specify signalling and associated UE behavior for support of unicast, groupcast (not including many to one) and broadcast of SL PRS transmissions [RAN1, RAN2]. - Specify reporting signalling and procedures to facilitate support of SL positioning in all coverage scenarios and for PC5-only and joint PC5-Uu scenarios [RAN2, RAN3]: - Specify the protocol and procedures for SL positioning between UEs (Protocol for Sidelink positioning procedures (SLPP)). - Specify the protocol and procedures for SL positioning between UEs and LMF. - Specify signalling to NG-RAN for sidelink positioning and ranging service authorizations as needed. [RAN3, RAN2] - Specify corresponding new core requirements, as well as identifying and specify the impact on the existing RAN4 specification, including RRM measurements and procedures [RAN4].

In this contribution, we present the summary of the documents submitted in this subagenda.

# Simultaneous Tx/Rx of SL PRS - SL CA framework [Huawei][Intel][QC]

| Huawei, HiSilicon | For the dedicated RP, however, we think it is late in Rel-18 to extend the dedicated RP within the SL CA framework due to difficulty in harmonizing the AGC symbols prior to SL-PRS, when it is FDMed with data on another SL communication CC. Proposal 3: RAN1 to conclude that - It is supported to transmit/receive SL-PRS for a shared RP on one SL CC, and data for a communication RP on another SL CC. - It is not supported to transmit/receive SL-PRS for a dedicated RP on one SL CC, and anything else on another SL CC. - Endorse the following TP to clause 6.3 of TS 38.202. |
| --- | --- |
| Intel | With the introduction of SL positioning, the section for SL in TS 38.202 for simultaneous transmission and reception of physical channels and signals has been identified to be impacted. Accordingly, new “Transmission Type” and “Reception Type” (Type E) for SL PRS transmission have been captured in the latest version of the specifications. However, for the characterization of simultaneous “Reception Type” combinations, further qualification would be necessary to clarify the scope (time-frequency region) within which the numbers of simultaneous “Reception Type” for SL PRS reception combinations apply. Accordingly, a text proposal to address this issue is provided in Table 7. Table 7. Text proposal for description of Sidelink "Reception Type" combinations Reason for change; For the characterization of simultaneous “Reception Type” combinations for sidelink, further qualification would be necessary to describe the scope within which the numbers of simultaneous “Reception Type” combinations apply for SL PRS reception. In particular, - For a shared SL PRS resource pool, the number of simultaneous SL PRS receptions should be defined within one sub-channel to align with SL communications (cf. Note 1 applicable for PSSCH and PSCCH). - For a dedicated SL PRS resource pool, the number of simultaneous SL PRS receptions should be defined within a dedicated SL PRS resource pool (analogous to a sub-channel for SL communications).. Summary of change; - For Note 3 in Table 6.3-4, clarify the scope (time-frequency region) within which the numbers of sidelink “Reception Type” combinations are defined for simultaneous reception of SL PRS in shared SL PRS resource pool. - For Note 4 in Table 6.3-4, clarify the scope (time-frequency region) within which the numbers of sidelink “Reception Type” combinations are defined for simultaneous reception of SL PRS in dedicated SL PRS resource pool.. Consequences if not approved; Incomplete/ambiguous specifications: It is unclear as to the time-frequency region within which the maximum numbers of simultaneous receptions of SL PRS for a shared and dedicated SL PRS resource pool is defined.. Text proposal; ------------------------------ TP#7: TS 38.202 ----------------------------------- ## 6.3 Sidelink ##### < Unchanged text omitted > Table 6.3-4: Sidelink "Reception Type" combinations Supported Combinations; Comment. A. B; Note 1, Note 2. C; Note 1, Note 2. E; Note 3. M1× E; Note 4. M× D; Note 2. B+C; Note 1, Note 2. Note 1: Corresponds to simultaneous reception within one sub-channel Note 2: Depending on the UE capability, the UE may be able to perform simultaneous sidelink communication receptions of the same sidelink “Reception Type” combinations across multiple SL carriers. Note 3: Applicable for a shared SL PRS resource pool. Corresponds to simultaneous reception within one sub-channel. Note 4: Applicable for a dedicated SL PRS resource pool with M_1≥1. Corresponds to simultaneous reception within one dedicated SL PRS resource pool.. < Unchanged text omitted >. Proposal 7 - Agree on TP#7 to clarify the scope (time-frequency region) within which the numbers of simultaneous “Reception Type” for SL PRS reception combinations apply. |
| Qualcomm | ## SL-PRS Tx and Rx on multiple carriers In case of SL CA, if SL-PRS can only occur in a single carrier, that specific carrier will be loaded unnecessarily with all the SL-PRS traffic. Note that the SL-PRS configuration may need to be a system-wide configuration. On the other hand, if a UE supports SL CA, the SL PRS can spread evently across multiple carriers as needed by deployment. Furthermore, the current TS 38.202 needs to be further updated to avoid confusion. Specifically, with regards to SL PRS reception, - in a shared resource pool, a UE can only receive a single SL PRS in a given subchannel, but across subchannels, a device can FDM the reception of multiple SL-PRS as it is the case for legacy SL communications - in a dedicated resource pool, a UE can receive a single SL PRS associated with a single comb-offset, but it can receive FDMed SL-PRS resources on the same bandwidth on different comb-offsets. Based on the above, we make the following proposals: Proposal 4: Depending on the UE capability, the UE may be able to perform simultaneous sidelink SL-PRS receptions across multiple SL carriers. Clarify also that a single devive ina shared resource pool can receive one SL-PRS in a given subchannel, and in the dedicated resource pool it can receive a single SL-PRS within one SL-PRS comb offset. - Update the Table 6.3-4 in TS 38.202 as follows: Table 6.3-4: Sidelink "Reception Type" combinations Supported Combinations; Comment. A. B; Note 1, Note 2. C; Note 1, Note 2. E; Note 2, Note 3. M1×E; Note 2, Note 4. M× D; Note 2. B+C; Note 1, Note 2. Note 1: Corresponds to simultaneous reception within one sub-channel Note 2: Depending on the UE capability, the UE may be able to perform simultaneous sidelink communication receptions of the same sidelink “Reception Type” combinations across multiple SL carriers. Note 3: Applicable for a shared SL PRS resource pool within one subchannel. Note 4: Applicable for a dedicated SL PRS resource pool within one SL-PRS comb offset... Proposal 5: Depending on the UE capability, the UE may be able to perform simultaneous SL-PRS transmissions across multiple SL carriers - Update the Table 6.3-2 in TS 38.202 as follows: Table 6.3-2: Sidelink "Transmission Type" combinations Supported Combinations; Comment. A; Note 2. B; Note 2. C; Note 2. E; Note 2, Note 4. E; Note 2, Note 5. N× D; Note 2. B+C; Note 2. Note 1: Depending on the UE capability, the UE may be able to perform simultaneous Uplink and Sidelink transmissions. If the simultaneous transmission of Sidelink and Uplink is beyond the UE capability, the one not prioritized can be dropped according to [TS 38.321]. Note 2: Depending on the UE capability, the UE may be able to perform simultaneous sidelink communication transmissions of the same sidelink “Transmission Type” combinations across multiple SL carriers. Note 3: Simultaneous transmissions over multiple SL carriers with one or more UL carriers is left up to UE implementation. Note 4: Applicable for a shared SL PRS resource pool. Note 5: Applicable for a dedicated SL PRS resource pool... |

[HIGH] Feature Lead Proposal 2-v0

Across 2 or more SL carriers, and depending on UE capability, conclude the support of the following:

- For the case of 2 or more dedicated SL-PRS resource pools:

  - Alt 1.1: Support simultaneous Rx/Tx of SL-PRS resources

  - Alt 1.2: Do not support simultaneous Rx/Tx of SL-PRS resources

- For the case of 2 or more shared SL-PRS resource pools:

  - Alt 2.1: Support simultaneous Rx/Tx of SL-PRS in CC1 and data (but not SL-PRS) in CC2

  - Alt 2.2: Support simultaneous Rx/Tx of SL-PRS in CC1 and any channel (including SL-PRS) in CC2

  - Alt 2.3: Do not support simultaneous Rx/Tx of SL-PRS in CC1 and anything else on another CC2

- For the case of a dedicated SL-PRS resource pool (CC1) and shared SL-PRS resource pool (CC2):

  - Alt 3.1: Support simultaneous Rx/Tx of SL-PRS in CC1 and data (but not SL-PRS) in CC2

  - Alt 3.2: Support simultaneous Rx/Tx of SL-PRS in CC1 and any channel in CC2

  - Alt 3.3: Do not support simultaneous Rx/Tx of SL-PRS in CC1 and anything else on another CC2.

- Notes:

  - Across multiple dedicated SL-PRS resource pools, “simultaneous Tx/Rx of SL-PRS” refers to the Tx/Rx of SL-PRS resources that have fully overlapping symbols across the multiple SL carriers.

  - For a shared SL-PRS resource pool in a CC, simultaneous SL-PRS in the CC with any other channel in another CC refers to Tx/Rx of at least one SL-PRS symbol overlapping with any symbol carrying the other channel.

  - No additional specification changes are expected beyond those in 38.202 Tables 6.3-2 and 6.3-4.

Companies views

| vivo | We only support transmitting SL SRS in one carrier Alt 1.2 Alt 2.3 Alt 3.3 |
| --- | --- |
| Huawei, HiSilicon | Alt 1.1: it should be clear that they may not be coherent. Alt 2.2 Alt 3.3 |
| ZTE | SL CA feature is introduced in Rel-18 SL agenda and for Rel-18 positioning we never spend time on designing SL PRS transmission or reception behavior for both shared SL PRS resource pool and dedicated resource pool. We prefer not to have any new design in this stage. Alt 1.2 Alt 2.3 Alt 3.3 |
| CATT | Alt 1.2 Alt 2.3 Alt 3.3 |
| Qualcomm | We support to enable simultaneous SL-PRS Tx/Rx in Dedicated+dedicated or Shared+shared RP. There is additional impact from UE complexity standpoint, nor from specification support. There is no reason to preclude it in the spec. Therefore we support: Alt. 1.1 Alt. 2.2 Alt. 3.3 |

Offline consensus

- Simultaneous (same symbol(s)) SL-PRS Tx/Rx across multiple SL CCs is not supported

# 3 ACK/NAK for DCI 3_2 [Huawei]

| Huawei, HiSilicon | In mode 1, the gNB controls resource allocation on sidelink. For legacy SL communication, a UE may transmit the HARQ-ACK information on uplink to the gNB based on the PSFCH reception from another UE. The HARQ-ACK information informs the gNB whether the data carried by the allocated resource has been successfully transmitted or not. This should also be applicable to SL PRS transmission in dedicated resource pool under mode 1 resource allocation. It is essential to send the ACK/NACK information to the grant in DCI 3_2 to the gNB so that the gNB could be aware whether the allocated SL PRS resource in one or multiple slots has been effectively used or not. It follows the generic principle of UE sending acknowledgement to the gNB for the DCI received in USS (except SL CG release). Proposal 1: Support to report ACK/NACK to gNB for DCI 3-2. Proposal 2: For reporting ACK/NACK for DCI 3_2, the SL Tx UE generates - ACK if the UE transmits a SL PRS in at least one of the resources scheduled by a DCI format 3_2; or - NACK if the UE does not transmit a SL PRS in any of the resources scheduled by a DCI format 3_2. |
| --- | --- |

[HIGH] Feature Lead Proposal 3-v0

Companies are encouraged to comment on the above proposal

Companies views

| vivo | For us, receiving a ACK/NACK can not provide enough information, ie, it doesn’t mean the received UE can receive the SL PRS or more retransmission is not needed even a ACK is received. In this case, the motivation and benefit are unclear. |
| --- | --- |
| ZTE | We doubt the necessity of introducing ACK/NACK feedback in such a late phase. Even if the Tx UE successfully transmit SL PRS in the scheduled resource, it is possible that Rx UE does not receive the SL PRS. |
| Qualcomm | It looks like an optimization that may not be necessary and we have similar views with vivo and ZTE |
| Nokia, NSB | Agree with Vivo. |

# 4 SL PRS transmission behavior in time domain [ZTE]

Agreement For a dedicated resource pool for SL positioning, SL-PRS cannot be transmitted in a slot without associated PSCCH. Agreement In a shared resource pool, SL-PRS, associated PSCCH and PSSCH scheduled by the PSCCH are included in the same slot: - The PSSCH is used for 2nd SCI and SL-SCH - Note: the UE may not have data available for transmission. Up to RAN2 how to define the specification support for this case.

As we agreed in RAN1, a UE shall transmit the SL PRS in the same slot as the associated PSCCH for either dedicated SL PRS resource pool or shared SL PRS resource pool. This feature was duplicated in TS 38.214 in clause 8.2.4 and clause 8.2.4.1.1.

Proposal 3: Adopt TP#2-3 for TS 38.214 for SL PRS transmission in time domain

Reason for change: A UE shall transmit the SL PRS in the same slot as the associated PSCCH for either dedicated SL PRS resource pool or shared SL PRS resource pool. But this feature was duplicated in TS 38.214 in clause 8.2.4 and clause 8.2.4.1.1.

Summary of change: delete “Each SL PRS transmission is associated with an PSCCH transmission in the same slot.” in clause 8.2.4 TS 38.214.

Consequences if not approved: redundant descriptions for SL PRS transmission in time domain.

Clause affected: 8.2.4 in 38.214

TP#2-3 38.214 8.2.4 SL PRS transmission procedure <Omitted> In the case of dedicated SL PRS resource pool, that PSCCH carries the SCI format 1-B associated with the SL PRS transmission. <Omitted> 8.2.4.1.1 Resource allocation in time domain The UE shall transmit the SL PRS in the same slot as the associated PSCCH. For a dedicated SL PRS resource pool, the minimum resource allocation unit in the time domain is a SL PRS resource in a slot. The UE shall transmit the SL PRS in consecutive symbols within the slot. A UE does not transmit multiple SL PRS resources in the same slot.

[MEDIUM] Feature Lead Proposal 4-v0

Companies are encouraged to comment on the proposal

Companies views

| vivo | We prefer to keep the sentence other than remove it. |
| --- | --- |
| Huawei, HiSilicon | This sentence does not bring any value, and we are fine to remove it. |
| ZTE | We support this TP as the description is duplicated in 8.2.4 and 8.2.4.1 in TS 38.214. 8.2.4: Each SL PRS transmission is associated with an PSCCH transmission in the same slot. 8.2.4.1.1: The UE shall transmit the SL PRS in the same slot as the associated PSCCH. |
| CATT | OK to remove the redundant sentence. |
| Qualcomm | We don’t really see the need to remove it. |
| Nokia, NSB | No need to remove it. |

# 5 RSRP measurement in a dedicated SL PRS resource pool [CATT]

| Reason for change: | Corrections on description associated with RSRP measurement in a dedicated SL PRS resource pool. |
| --- | --- |
| Summary of change: | In clause 8.4.2.1 of TS 38.214, RSRP measurement in a dedicated SL PRS resource pool is captured. |
| Consequences if not approved: | The description associated with RSRP measurement in a dedicated SL PRS resource pool is vague. |

-------------------------------------------- Start of text proposal to TS 38.214 v18.1.0 ---------------------------------------

8.4.2.1	RSRP for resource selection in sidelink resource allocation mode 2

In sidelink resource allocation mode 2, the UE measures RSRP for resource selection as follows:

-	PSSCH-RSRP over the DM-RS resource elements for the PSSCH according to the received SCI format 1-A if higher layer parameter sl-RS-ForSensing is set to ‘pssch’, and

-	PSCCH-RSRP over the DM-RS resource elements for the PSCCH carrying the received SCI format 1-A if higher layer parameter sl-RS-ForSensing is set to ‘pscch’.

In sidelink resource allocation mode 2 for a dedicated SL PRS resource pool, the UE measures the PSCCH-RSRP over the DM-RS resource elements for the PSCCH carrying the received SCI format 1-B.

…

-------------------------------------------- End of text proposal to TS 38.214 v18.1.0 ---------------------------------------

[MEDIUM] Feature Lead Proposal 5-v0

Companies are encouraged to comment on the above proposal

Companies views

| vivo | OK |
| --- | --- |
| Huawei, HiSilicon | OK |
| ZTE | We do not think this TP is needed, as the following was already captured in section 8.2.4.2 in TS38.214 - In condition b of step 6, the RSRP measurement is the PSCCH-RSRP over the DM-RS resource elements of the PSSCH; |
| CATT | For ZTE comments, the cited sentence in the comments is in section 8.2.4.2. Section 8.2 is UE procedure for transmitting sidelink reference signals, section 8.4 is UE procedure for receiving reference signals, we believe both 8.2 and 8.4 need to mention how to measure PSCCH-RSRP for SL PRS. However, in section 8.4, the description in 8.4.2.1 lacks the processing description after receiving 1-B, so we think this TP is still needed. |
| Qualcomm | OK |
| Nokia, NSB | This change is not needed. How to determine RSRP for dedicated SL PRS resource pool is specified in clause 8.2.4.2 and 8.2.4.2 does not refer to clause 8.4.2.1. |

# 6 PDCCH monitoring for DCI format 3_2

| Reason for change: | It is not clear whether UE monitors PDCCH for DCI format 3_2 on more than one cell. |
| --- | --- |
| Summary of change: | Restrict UE to monitor PDCCH for DCI format 3_2 on one cell, as the same as DCI format 3_0 and DCI format 3_1. |
| Consequences if not approved: | Unclear PDCCH monitoring for DCI format 3_2 on more than one cell. |
| Clauses affected: | Section 16 in TS 38.213. |

---------------------------------------< Text Proposal 2 for 38.213> --------------------------------------

16	UE procedures for sidelink

A UE is provided by SL-BWP-Config or SL-BWP-ConfigCommon a BWP for SL transmissions (SL BWP) with numerology and resource grid determined as described in [4, TS 38.211].

For a resource pool within the SL BWP,

-	for operation without shared spectrum channel access, or for operation with shared spectrum channel access and when sl-transmissionStructureForPSCCHandPSSCH = ‘contiguousRB’, the UE is provided by sl-NumSubchannel a number of sub-channels where each sub-channel includes a number of contiguous RBs provided by sl-SubchannelSize. The first RB of the first sub-channel in the SL BWP is indicated by sl-StartRB-Subchannel

-	for operation with shared spectrum channel access and when sl-transmissionStructureForPSCCHandPSSCH = ‘interlaceRB’, the UE is provided by sl-NumSubchannel a number of sub-channels where each sub-channel includes a number of interlaces, provided by sl-numInterlacePerSubchannel, and the interlaces have contiguous interlace indexes

Available slots for a resource pool are provided by sl-TimeResource and occur with a periodicity of 10240 ms. For an available slot without S-SS/PSBCH blocks, SL transmissions can start from a first symbol indicated by sl-StartSymbol or, for operation with shared spectrum channel access, can start from a first symbol indicated by sl-StartingSymbolFirst or from a second symbol indicated by sl-StartingSymbolSecond [6, TS 38.214], and be within a number of consecutive symbols indicated by sl-LengthSymbols. For an available slot with S-SS/PSBCH blocks, the first symbol and the number of consecutive symbols is predetermined.

The UE expects to use a same numerology in the SL BWP and in an active UL BWP in a same carrier of a same cell. If the active UL BWP numerology is different than the SL BWP numerology, the SL BWP is deactivated.

A priority of a PSSCH according to NR radio access or according to E-UTRA radio access is indicated by a priority field in a respective scheduling SCI format. A priority of a PSSS/SSSS/PSBCH according to E-UTRA radio access is provided by sl-SSB-PriorityEUTRA [13, TS 36.213]. A priority of an S-SS/PSBCH block is provided by sl-SSB-PriorityNR. A priority of a PSFCH is determined as described in clause 16.2.4.2.

A UE does not expect to be provided search space sets associated with CORESETs on more than one cell to monitor PDCCH for detection of DCI format 3_0, DCI format 3_2, or DCI format 3_1.

-------------------------------------------< End of Text Proposal 2> ---------------------------------------

[MEDIUM] Feature Lead Proposal 6-v0

Companies are encouraged to comment on the above two proposals

Companies views

| vivo | OK |
| --- | --- |
| Huawei, HiSilicon | OK |
| ZTE | OK |
| CATT | OK |
| Qualcomm | OK |
| Nokia, NSB | OK |

# 7 SL PRS delay budget in shared SL PRS resource pool [Asustek]

| Reason for change: | In shared SL PRS resource pool, UE can transmit both PSSCH and SL PRS in same slot, while SL PRS has its own delay budgets different from PSSCH. However, sensing-based resource selection does not consider SL PRS delay budget, thus SL PRS transmission even with higher priority cannot be guaranteed. |
| --- | --- |
| Summary of change: | When setting T2min, consider SL PRS delay budget, if any. |
| Consequences if not approved: | For SL PRS in shared SL PRS resource pool, its delay budget cannot be guaranteed. |
| Clauses affected: | 8.1.4 in TS 38.214. |

---------------------------------------< Text Proposal 3 for 38.214> --------------------------------------

8.1.4	UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2

In resource allocation mode 2, the higher layer can request the UE to determine a subset of resources from which the higher layer will select resources for PSSCH/PSCCH transmission for a carrier. To trigger this procedure, in slot n for this carrier, the higher layer provides the following parameters for this PSSCH/PSCCH transmission:

-	the resource pool from which the resources are to be reported;

-	L1 priority, prioTX;

-	the remaining packet delay budget and the remaining SL PRS delay budget, if any;

-	…

…

The following steps are used:

1)	If a number of consecutive slots Nslot,MCSt is provided with a value larger than 1, the candidate multi-slot resource definition is applied. Otherwise, the candidate single-slot resource definition is applied.

If the higher layer parameter transmissionStructureForPSCCHandPSSCH is set to 'contiguousRB', a candidate multi-slot resource Rx,y  is defined as a set of LsubCH contiguous sub-channels starting from sub-channel x in Nslot,MCSt consecutive slots starting from slot t'ySL, when the set of Nslot,MCSt slots that are consecutive in physical slots.

If the higher layer parameter transmissionStructureForPSCCHandPSSCH is set to 'interlaceRB', a candidate multi-slot resource Rx,y,z is defined as a set of LsubCH contiguous sub-channels starting from sub-channel x in Nslot,MCSt consecutive slots starting from slot t'ySL in LRBset contiguous RB sets starting from RB set z, when the set of Nslot,MCSt slots that are consecutive in physical slots. A candidate single-slot resource Rx,y,z is defined as a set of LsubCH contiguous sub-channels starting from sub-channel x in slot t'ySL in LRBset contiguous RB sets starting from RB set z.

If the higher layer parameter transmissionStructureForPSCCHandPSSCH is not provided or if the higher layer parameter transmissionStructureForPSCCHandPSSCH is set to ‘contiguousRB’, a candidate single-slot resource for transmission Rx,y is defined as a set of LsubCH contiguous sub-channels with sub-channel x+j in slot t'ySL where j=0,...,LsubCH-1. The UE shall assume that any set of LsubCH contiguous sub-channels or LsubCH contiguous sub-channels in LRBset contiguous RB sets included in the corresponding resource pool within the time interval [n+T1,n+T2] correspond to one candidate single-slot resource or the UE shall assume that any set of LsubCH contiguous sub-channels or LsubCH contiguous sub-channels in LRBset contiguous RB sets in Nslot,MCSt consecutive slots included in the corresponding resource pool within the time interval n+T1,n+T2 correspond to one candidate multi-slot resource for UE performing full sensing. The UE shall assume that any set of LsubCH contiguous sub-channels included in the corresponding resource pool in a set of Y candidate slots within the time interval n+T1,n+T2 correspond to one candidate single-slot resource or one candidate multi-slot resource for UE performing periodic-based partial sensing together with contiguous partial sensing and resource (re)selection triggered by periodic transmission (Prsvp_TX≠0), or in a set of Y' candidate slots within the time interval [n+T1,n+T2] correspond to one candidate single-slot resource or one candidate multi-slot resource for UE performing at least contiguous partial sensing and resource (re)selection triggered by aperiodic transmission (Prsvp_TX=0), where

-	selection of T1 is up to UE implementation under 0 ≤ T1≤ Tproc,1SL  , where Tproc,1SL  is defined in slots in Table 8.1.4-2 where μSL is the SCS configuration of the SL BWP;

-	if  T2min is shorter than the remaining packet delay budget (in slots) then T2 is up to UE implementation subject to  T2min ≤ T2 ≤ remaining packet delay budget (in slots); otherwise T2 is set to the remaining packet delay budget (in slots). In case there is remaining SL PRS delay budget shorter than the remaining packet delay budget, if  T2min is shorter than the remaining SL PRS delay budget (in slots) then T2 is up to UE implementation subject to  T2min ≤ T2 ≤ remaining SL PRS delay budget (in slots); otherwise T2 is set to the remaining SL PRS delay budget (in slots).

-	Y is selected by UE where Y≥Ymin.

-	Y' is selected by UE where Y'≥Ymin'. When the UE performs at least contiguous partial sensing and if Prsvp_TX=0, the UE selects a set of Y' candidate slots with corresponding PBPS and/or CPS results (if available). If the number of candidate slots Y' is smaller than Ymin', it is up to UE implementation to include other candidate slots.

-------------------------------------------< End of Text Proposal 3> ---------------------------------------

[MEDIUM] Feature Lead Proposal 7-v0

Companies are encouraged to comment on the above proposal

Companies views

| vivo | We prefer the physical layer to only be provided one remaining packet delay budget by high layer for PSSCH and SL PRS. Otherwise, the UE behavior is unclear. |
| --- | --- |
| Huawei, HiSilicon | We understand the intention, but it actually defines a new behavior for shared RP. Before we could agree to the new functionality, we suggest to send an LS to RAN2 to check for shared RP, whether two delay budget can be provided by higher layers in the first place. |
| ZTE | We do not support this TP. In Chicago meeting, RAN2 agreed that “When there are both SL-PRS and SL-SCH data pending for transmission at resource selection, the resource selection should be within the smaller one of the SL-PRS delay budget of the pending SL-PRSs and PDB of the logical channels.” We do not think MAC layer will provide two delay budget to physical layer. |
| CATT | It seems that only one delay buget will be provided to physical layer. |
| Nokia, NSB | Agree with vivo. |

# 8 Sensing RS typo [CMCC] [Xiaomi][Asustek]

RAN1 has agreed that PSCCH DMRS is used to derive L1 SL-RSRP for resource exclusion. There is a typo in the corresponding description in TS 38.214 Clause 8.2.4.2:

The UE shall perform this procedure according to clause 8.1.4, with the following modifications: - Partial sensing is not applicable in a dedicated SL PRS resource pool; - A candidate single-slot resource for transmission Rx,y is defined as the SL PRS resource with index x within the Set of SL-PRS resource ID(s) provided by the higher layer and in slot t'ySL - “SCI format 1-A” is replaced by “SCI format 1-B”, - In step 5 [] - In condition b of step 6, the RSRP measurement is the PSCCH-RSRP over the DM-RS resource elements of the PSSCH; - In condition c of step 6 “determines according to clause 8.1.5 the set of resource blocks and slots” is replaced by “determines according to clause 8.2.4.X the set of slots and SL PRS resources”;

Therefore, we propose to change the above typo.

Proposal 1: Adopt the following TP for TS 38.214 Clause 8.2.4.2:

| Reasons for change | Typo in procedure for determining the SL PRS resources |
| --- | --- |
| Summary of change | Correct the typo. |
| Consequences if not approved | The specification is not correct. |
| Text proposal | The UE shall perform this procedure according to clause 8.1.4, with the following modifications: - Partial sensing is not applicable in a dedicated SL PRS resource pool; - A candidate single-slot resource for transmission Rx,y is defined as the SL PRS resource with index x within the Set of SL-PRS resource ID(s) provided by the higher layer and in slot t'ySL - “SCI format 1-A” is replaced by “SCI format 1-B”, - In step 5 [] - In condition b of step 6, the RSRP measurement is the PSCCH-RSRP over the DM-RS resource elements of the PSSCHPSCCH; - In condition c of step 6 “determines according to clause 8.1.5 the set of resource blocks and slots” is replaced by “determines according to clause 8.2.4.X the set of slots and SL PRS resources”; |

[Xiaomi]

For SL positioning, we have identified a typo in current version of TS38.214 [2] where the “PSSCH” should be changed to “PSCCH”, so the following TP was provided to correct the corresponding wording:

Proposal 1: The following TP#1 should be adopted to correct a typo in TS38.214.

- 8.2.4.2 UE procedure for determining the subset of resources to be reported to higher layers in SL PRS resource selection in a dedicated resource pool in sidelink resource allocation mode 2 <Unchanged parts omitted> The UE shall perform this procedure according to clause 8.1.4, with the following modifications: - Partial sensing is not applicable in a dedicated SL PRS resource pool; - A candidate single-slot resource for transmission Rx,y is defined as the SL PRS resource with index x within the Set of SL-PRS resource ID(s) provided by the higher layer and in slot t'ySL - "SCI format 1-A” is replaced by “SCI format 1-B", - In step 5, the second condition is modified as follows: for any periodicity value allowed by the higher layer parameter reservationPeriodAllowed-Dedicated-SL-PRS-RP and any SL PRS resource ID in the set of SL PRS resource ID(s) provided by the higher layer, and a hypothetical SCI format 1-B received in slot t'mSL with 'Resource reservation period' field set to that periodicity value and indicating that SL-PRS resource ID, condition c in step 6 would be met. - In condition b of step 6, the RSRP measurement is the PSCCH-RSRP over the DM-RS resource elements of the PSCCH; - In condition c of step 6 "determines according to clause 8.1.5 the set of resource blocks and slots" is replaced by "determines according to clause 8.2.4.X the set of slots and SL PRS resources".

2.1 Correction on PSCCH-RSRP

| Reason for change: | In dedicated SL PRS resource pool, RSRP measurement is the PSCCH-RSRP over the DM-RS resource elements of the PSCCH. |
| --- | --- |
| Summary of change: | Correct PSCCH-RSRP over the DM-RS resource elements of the PSCCH, not PSSCH. |
| Consequences if not approved: | Incorrect RSRP measurement in dedicated SL PRS resource pool. |
| Clauses affected: | 8.2.4.2 in TS 38.214. |

---------------------------------------< Text Proposal 1 for 38.214> --------------------------------------

8.2.4.2	UE procedure for determining the subset of resources to be reported to higher layers in SL PRS resource selection in a dedicated SL PRS resource pool in sidelink resource allocation mode 2

…

The UE shall perform this procedure according to clause 8.1.4, with the following modifications:

-	"packet delay budget" is replaced by "SL PRS delay budget",

-	partial sensing is not applicable in a dedicated SL PRS resource pool,

-	"candidate single-slot resource" is replaced by "candidate SL PRS resource",

-	a candidate single-slot resource for transmission Rx,y is defined as the SL PRS resource with index x within the Set of SL-PRS resource ID(s) provided by the higher layer and in slot t'ySL,

-	"SCI format 1-A" is replaced by "SCI format 1-B",

-	in step 5, the second condition is modified as follows: for any periodicity value allowed by the higher layer parameter reservationPeriodAllowed-Dedicated-SL-PRS-RP and any SL PRS resource ID in the set of SL PRS resource ID(s) provided by the higher layer, and a hypothetical SCI format 1-B received in slot t'mSL with 'Resource reservation period' field set to that periodicity value and indicating that SL-PRS resource ID, condition c in step 6 would be met,

-	In condition b of step 6, the RSRP measurement is the PSCCH-RSRP over the DM-RS resource elements of the PSCCH;

-	In condition c of step 6 "determines according to clause 8.1.5 the set of resource blocks and slots" is replaced by "determines according to clause 8.2.4.2A the set of SL PRS resources and slots ".

-------------------------------------------< End of Text Proposal 1> ---------------------------------------

[MEDIUM] Feature Lead Proposal 8-v0

Support the following change in Section 8.2.4.2 in TS 38.214:

| Reason for change: | In dedicated SL PRS resource pool, RSRP measurement is the PSCCH-RSRP over the DM-RS resource elements of the PSCCH. |
| --- | --- |
| Summary of change: | Correct PSCCH-RSRP over the DM-RS resource elements of the PSCCH, not PSSCH. |
| Consequences if not approved: | Incorrect RSRP measurement in dedicated SL PRS resource pool. |
| Clauses affected: | 8.2.4.2 in TS 38.214. |

---------------------------------------< Text Proposal 1 for 38.214> --------------------------------------

8.2.4.2	UE procedure for determining the subset of resources to be reported to higher layers in SL PRS resource selection in a dedicated SL PRS resource pool in sidelink resource allocation mode 2

…

The UE shall perform this procedure according to clause 8.1.4, with the following modifications:

-	"packet delay budget" is replaced by "SL PRS delay budget",

-	partial sensing is not applicable in a dedicated SL PRS resource pool,

-	"candidate single-slot resource" is replaced by "candidate SL PRS resource",

-	a candidate single-slot resource for transmission Rx,y is defined as the SL PRS resource with index x within the Set of SL-PRS resource ID(s) provided by the higher layer and in slot t'ySL,

-	"SCI format 1-A" is replaced by "SCI format 1-B",

-	in step 5, the second condition is modified as follows: for any periodicity value allowed by the higher layer parameter reservationPeriodAllowed-Dedicated-SL-PRS-RP and any SL PRS resource ID in the set of SL PRS resource ID(s) provided by the higher layer, and a hypothetical SCI format 1-B received in slot t'mSL with 'Resource reservation period' field set to that periodicity value and indicating that SL-PRS resource ID, condition c in step 6 would be met,

-	In condition b of step 6, the RSRP measurement is the PSCCH-RSRP over the DM-RS resource elements of the PSCCH;

-	In condition c of step 6 "determines according to clause 8.1.5 the set of resource blocks and slots" is replaced by "determines according to clause 8.2.4.2A the set of SL PRS resources and slots ".

-------------------------------------------< End of Text Proposal 1> ---------------------------------------

Companies views

| vivo | OK |
| --- | --- |
| Huawei, HiSilicon | Support. |
| ZTE | Support |
| CATT | OK |
| Qualcomm | OK |
| Nokia, NSB | OK |

# 9 TP for 38.212 [ZTE]

In this section, we provide an alignment TP for improving spec readability:

Proposal 7: Adopt TP#2-4 in TS 38.212 for “Time resource alignment” indicated in SCI 1-B

Reason for change: “Time resource alignment” indicated in SCI 1-B is associated with TS 38.214 Clause 8.2.4.2A instead of Clause 8.2.4.3, TS 38.214 Clause 8.2.4.3 is related to sidelink congestion control in a dedicated SL PRS resource pool.

Summary of change: change clause 8.2.4.3 to clause 8.2.4.2A for field“Time resource alignment” in SCI 1-B

Consequences if not approved: wrong reference clause number in TS 38.212

Clause affected: 8.3.1.2 in 38.212

TP#2-4 38.212 #### 8.3.1.2 SCI format 1-B SCI format 1-B is used for the scheduling of SL PRS for a dedicated SL PRS resource pool. The following information is transmitted by means of the SCI format 1-B: <Omitted> - Time resource assignment – 5 bits when the value of the higher layer parameter sl-MaxNumPerReserve-Dedicated-SL-PRS-RP is configured to 2; otherwise 9 bits when the value of the higher layer parameter sl-MaxNumPerReserve-Dedicated-SL-PRS-RP is configured to 3, as defined in clause 8.2.4.2A of [6, TS 38.214].

[MEDIUM] Feature Lead Proposal 9-v0

Companies are encouraged to comment on the above alignment TP

Companies views

| ZTE | Support |  |  |
| --- | --- | --- | --- |
| Qualcomm | OK |  |  |

# 10 Change the “[delay budget]” to “SL PRS delay budget" in 38.214

[MEDIUM] Feature Lead Proposal 10-v0

Companies are encouraged to comment on the change of “[delay budget]” shown below:

| Reason for change: | There are unresolved brackets in 8.2.4.2 of TS 38.214 |
| --- | --- |
| Summary of change: | In clause 8.2.4.2 of TS 38.214, change the “[delay budget]” to “SL PRS delay budget" |
| Consequences if not approved: | Brackets in 8.2.4.2 of TS 38.214 will result into incomplete specification |

------------------------------------- Start of text proposal to TS 38.214 v18.1.0 -----------------------------------

8.2.4.2	UE procedure for determining the subset of resources to be reported to higher layers in SL PRS resource selection in a dedicated SL PRS resource pool in sidelink resource allocation mode 2

In resource allocation mode 2 in a dedicated SL PRS resource pool, the higher layer can request the UE to determine a subset of resources from which the higher layer will select resources for SL PRS/PSCCH transmission. To trigger this procedure, in slot n, the higher layer provides the following parameters for this SL PRS/PSCCH transmission:

-	the resource pool from which the resources are to be reported;

-	L1 priority, prioTX;

-	the remaining SL PRS delay budget;

-	Set of SL-PRS resource ID(s);

-	optionally, the resource reservation interval, Prsvp_TX, in units of msec.

-	if the higher layer requests the UE to determine a subset of resources from which the higher layer will select resources for SL PRS[/PSCCH] transmission as part of re-evaluation or pre-emption procedure, the higher layer provides a set of resources (r0,r1,r2,…) which may be subject to re-evaluation and a set of resources (r0',r1',r2',…) which may be subject to pre-emption.

-	it is up to UE implementation to determine the subset of resources as requested by higher layers before or after the slot ri'' - T3, where ri'' is the slot with the smallest slot index among (r0,r1,r2,…) and (r0',r1',r2',…) , and T3 is equal to Tproc,1SL, where Tproc,1SL  is defined in slots in Table 8.1.4-2 where μSL is the SCS configuration of the SL BWP.

------------------------------------- End of text proposal to TS 38.214 v18.1.0 -----------------------------------

Companies views

| vivo | OK |
| --- | --- |
| ZTE | OK |
| CATT | Support |
| Qualcomm | OK |

# 11 PSSCH DMRS used for Positioning purposes [Nokia]

| Nokia, NSB | In RAN1#115, the FL proposed the following in SL PRS agenda [2]: ### [Medium] FL1 Proposal 4-1 - Support to indicate in SCI 2-D whether the same antenna port is applied for SL-PRS and PSSCH DMRS for Rx UE joint processing.. RAN1 should consider resource efficiency in a shared resource pool with backward compatibility, and it would be beneficial to re-use PSSCH DMRS symbols to assist SL positioning. The PSSCH DMRS symbols can also be used by the UE to improve, for example, the timing estimation accuracy. To facilitate the utilization of PSSCH DMRS with SL PRS for positioning, as proposed by the FL in [2], the SCI can be used to indicate the association information between SL-PRS antenna port and PSSCH DMRS antenna port to the Rx UE. Proposal 1: Support the following text proposal of Clause 8.4.1.4 of TS 38.212. #### 8.4.1.4 SCI format 2-D SCI format 2-D is used for the decoding of PSSCH and the scheduling of SL PRS for a shared SL PRS resource pool. The following information is transmitted by means of the SCI format 2-D: - SL PRS resource ID - log2NSL-PRS bits, where the value NSL-PRS is the total number of SL PRS resource IDs within a slot in a shared SL PRS resource pool and provided by the higher layer parameter sl-PrsResources-Shared-SL-PRS-RP. - SL PRS request – 1 bit as defined in clause 8.4.4 of [6, TS 38.214] when the higher layer parameter sl-SCI-based-SL-PRS-Tx-Trigger-SCI2-D is provided; 0 bit otherwise. - Embedded SCI format - 2 bits. This field indicates the embedded SCI format as defined in Table 8.4.1.4-1. - Embedded SCI format payload - number of bits determined according to Table 8.4.1.4-1. This field is set to the associated payload of the embedded SCI format indicated by the ‘Embedded SCI format’ field as defined in Table 8.4.1.4-1. - Same antenna port indication – 1 bit. This field indicates whether the same antenna port is applied for SL PRS and PSSCH DMRS <omitted text>. Reason for change:; To allow utilization of PSSCH DMRS for positioning. In case the UE uses the same antenna port to transmit PSCCH DMRS and SL PRS, the receiving UE also can utilize PSSCH DMRS, which is useful for postioning. Thus, RAN1 does not need to preclude this implementation.. Summary of change:; A field is introduced in SCI format 2-D which indicates whether the same antenna port is applied for SL PRS and PSSCH DMRS.. Consequences if not approved:; PSSCH DMRS cannot be utilized for positioning.. |
| --- | --- |

[LOW] Feature Lead Proposal 11-v0

Companies are encouraged to comment on the above proposal

Companies views

| vivo | Not supported |
| --- | --- |
| Huawei, HiSilicon | Prefer to discuss it SL PRS summary. |
| ZTE | Same proposal was discussed in previous meeting but not agreed, we prefer to conclude that PSSCH DMRS used for Positioning purposes will not be pursued. |
| CATT | This issue had been discussed in SL PRS agenda, and this TP seems not needed. |
| Qualcomm | OK to discuss it in SL PRS summary. In either case, we do not support the feature. |

# 12 1-symbol PSCCH [Apple]

In RAN1 #113, the following agreement was made [4]:

Agreement In a dedicated resource pool, with regards to the PSCCH, reuse the PSCCH channel structure of SL communications, at least with regards to the following aspects: - The first PSCCH symbol is mapped to the 2nd symbol available for SL transmissions in a slot - Note: 1st symbol available for SL transmissions in a slot is for PSCCH AGC similar to legacy - PSCCH DM-RS in the slot is being reused from legacy - The number of PSCCH symbol(s) is (pre-)configured to (down-select at RAN1#114): - Alt. 1: 2 or 3 symbols (same as legacy) - Alt. 3: 1, or 2 or 3 symbols - The number of PRBs is (pre-)configured using the legacy values - FFS: reconsider if 1-symbol PSCCH is supported

The last FFS has not be concluded and we propose that we conclude 1-symbol PSSCH is not supported.

Conclusion 1: 1-symbol PSSCH is not supported for Rel-18

[LOW] Feature Lead Proposal 12-v0

Companies are encouraged to comment on the above proposal

Companies views

| vivo | OK |
| --- | --- |
| ZTE | OK |
| CATT | OK |

# 13 TP for aligning parameter names

## 13.1 Alignment TP for TS38.212 [vivo][Asustek]

[vivo]

- Adopt the following modification into TS38.212 for SL PRS.

< Unchanged parts are omitted > ### 8.3.1.2 SCI format 1-B SCI format 1-B is used for the scheduling of SL PRS for a dedicated SL PRS resource pool. The following information is transmitted by means of the SCI format 1-B: - Priority - 3 bits as specified in clause 5.7 of [12, TS 23.586] and clause 5.22 of [8, TS 38.321]. Value '000' of Priority field corresponds to priority value '1', value '001' of Priority field corresponds to priority value '2', and so on. - Source ID – 12 or 24 bits determined by higher layer parameter sl-SRC-ID-Len-Dedicated-SL-PRS-RP, as defined in clause 16.4A of [5, TS 38.213]. - Destination ID – 24 bits as defined in clause 16.4A of [5, TS 38.213]. - Cast type indicator – 2 bits as defined in Table 8.3.1.2-1 and in clause 16.4A of [5, TS 38.213]. - Resource reservation period – log2Nrsv_period bits as defined in clause 16.4A of [5, TS 38.213], where Nrsv_period is the number of entries in the higher layer parameter sl -reservationPeriodAllowedDedicatedSL-PRS-RP, if higher layer parameter sl -reservationPeriodAllowedDedicatedSL-PRS-RP is configured; 0 bit otherwise. - Time resource assignment – 5 bits when the value of the higher layer parameter sl-MaxNumPerReserveDedicatedSL-PRS-RP is configured to 2; otherwise 9 bits when the value of the higher layer parameter sl-MaxNumPerReserveDedicatedSL-PRS-RP is configured to 3, as defined in clause 8.2.4.3 of [6, TS 38.214]. - Resource ID indication – log2NSL-PRS bits when the value of the higher layer parameter sl-MaxNumPerReserveDedicatedSL-PRS-RP is configured to 2; otherwise 2log2NSL-PRS bits when the value of the higher layer parameter sl-MaxNumPerReserveDedicatedSL-PRS-RP is configured to 3. The value NSL-PRS is the total number of SL PRS resources within a slot in a dedicated SL PRS resource pool and provided by the higher layer parameter sl-PRS-ResourcesDedicatedSL-PRS-RP. - SL PRS request – 1 bit as defined in clause 8.4.4 of [6, TS 38.214] when the higher layer parameter sl-SCI-basedSL-PRS-TxTriggerSCI1-B is provided; 0 bit otherwise. - Reserved - Nreserved bits as configured by higher layer parameter sl-NumReservedBitsSCI1B-DedicatedSL-PRS-RP, with value set to zero. < Unchanged parts are omitted > ### 8.4.1.4 SCI format 2-D SCI format 2-D is used for the decoding of PSSCH and the scheduling of SL PRS for a shared SL PRS resource pool. The following information is transmitted by means of the SCI format 2-D: - SL PRS resource ID - log2NSL-PRS bits, where the value NSL-PRS is the total number of SL PRS resource IDs within a slot in a shared SL PRS resource pool and provided by the higher layer parameter sl-PRS-ResourcesSharedSL-PRS-RP. - SL PRS request – 1 bit as defined in clause 8.4.4 of [6, TS 38.214] when the higher layer parameter sl-SCI-based-SL-PRS-Tx-Trigger-SCI2-D is provided; 0 bit otherwise. - Embedded SCI format - 2 bits. This field indicates the embedded SCI format as defined in Table 8.4.1.4-1. - Embedded SCI format payload - number of bits determined according to Table 8.4.1.4-1. This field is set to the associated payload of the embedded SCI format indicated by the ‘Embedded SCI format’ field as defined in Table 8.4.1.4-1.

- Reason for change: To resolve the misalignment issue about higher-layer parameter names between RAN1 and RAN and remaining issue about quote chapter. - Summary of change: Modify some higher-layer parameter name, remove some redundant spaces and address the remaining issue. - Consequences if not approved: The specification may be not clear and aligned with higher layer specification.

[Asustek]

-------------------------------------< Text Proposal 5-3 for 38.212> --------------------------------------

8.3.1.2	SCI format 1-B

SCI format 1-B is used for the scheduling of SL PRS for a dedicated SL PRS resource pool.

The following information is transmitted by means of the SCI format 1-B:

-	Priority - 3 bits as specified in clause x.x of [12, TS 23.586] and clause x.x of [8, TS 38.321]. Value '000' of Priority field corresponds to priority value '1', value '001' of Priority field corresponds to priority value '2', and so on.

-	Source ID – 12 or 24 bits determined by higher layer parameter sl-SRC-ID-LenDedicatedSL-PRS-RP, as defined in clause 16.4A of [5, TS 38.213].

-	Destination ID – 24 bits as defined in clause 16.4A of [5, TS 38.213].

-	Cast type indicator – 2 bits as defined in Table 8.3.1.2-1 and in clause 16.4A of [5, TS 38.213].

-	Resource reservation period – log2Nrsv_period bits as defined in clause 16.4A of [5, TS 38.213], where Nrsv_period is the number of entries in the higher layer parameter sl-PRS-ResourceReservePeriodList, if higher layer parameter sl-PRS-ResourceReservePeriodList is configured; 0 bit otherwise.

-	Time resource assignment – 5 bits when the value of the higher layer parameter sl-MaxNumPerReserveDedicatedSL-PRS-RP is configured to 2; otherwise 9 bits when the value of the higher layer parameter sl-MaxNumPerReserveDedicatedSL-PRS-RP is configured to 3, as defined in clause 8.2.4.3 of [6, TS 38.214].

-	Resource ID indication – log2NSL-PRS bits when the value of the higher layer parameter sl-MaxNumPerReserveDedicatedSL-PRS-RP is configured to 2; otherwise 2log2NSL-PRS bits when the value of the higher layer parameter sl-MaxNumPerReserveDedicatedSL-PRS-RP is configured to 3. The value NSL-PRS is the total number of SL PRS resources within a slot in a dedicated SL PRS resource pool and provided by the higher layer parameter SL-PRS-ResourceDedicatedSL-PRS-RP.

-	SL PRS request – 1 bit as defined in clause 8.4.4 of [6, TS 38.214] when the higher layer parameter sl-SCI-basedSL-PRS-TxTriggerSCI1-B is provided; 0 bit otherwise.

-	Reserved - Nreserved bits as configured by higher layer parameter sl-NumReservedBitsSCI1B-DedicatedSL-PRS-RP, with value set to zero.

…

8.4.1.4	SCI format 2-D

SCI format 2-D is used for the decoding of PSSCH and the scheduling of SL PRS for a shared SL PRS resource pool.

The following information is transmitted by means of the SCI format 2-D:

-	SL PRS resource ID - log2NSL-PRS bits, where the value NSL-PRS is the total number of SL PRS resource IDs within a slot in a shared SL PRS resource pool and provided by the higher layer parameter SL-PRS-ResourceSharedSL-PRS-RP.

-	SL PRS request – 1 bit as defined in clause 8.4.4 of [6, TS 38.214] when the higher layer parameter sl-SCI-based-SL-PRS-Tx-Trigger-SCI2-D is provided; 0 bit otherwise.

-	Embedded SCI format - 2 bits. This field indicates the embedded SCI format as defined in Table 8.4.1.4-1.

-	Embedded SCI format payload - number of bits determined according to Table 8.4.1.4-1. This field is set to the associated payload of the embedded SCI format indicated by the ‘Embedded SCI format’ field as defined in Table 8.4.1.4-1.

-----------------------------------< End of Text Proposal 5-3 for 38.212> -------------------------------

[LOW] Feature Lead Proposal 13.1-v0

Companies are encouraged to comment on the alignment TP below:

-------------------------------------< Text Proposal 5-3 for 38.212> --------------------------------------

8.3.1.2	SCI format 1-B

SCI format 1-B is used for the scheduling of SL PRS for a dedicated SL PRS resource pool.

The following information is transmitted by means of the SCI format 1-B:

-	Priority - 3 bits as specified in clause 5.7 of [12, TS 23.586] and clause 5.22 of [8, TS 38.321]. Value '000' of Priority field corresponds to priority value '1', value '001' of Priority field corresponds to priority value '2', and so on.

-	Source ID – 12 or 24 bits determined by higher layer parameter sl-SRC-ID-LenDedicatedSL-PRS-RP, as defined in clause 16.4A of [5, TS 38.213].

-	Destination ID – 24 bits as defined in clause 16.4A of [5, TS 38.213].

-	Cast type indicator – 2 bits as defined in Table 8.3.1.2-1 and in clause 16.4A of [5, TS 38.213].

-	Resource reservation period – log2Nrsv_period bits as defined in clause 16.4A of [5, TS 38.213], where Nrsv_period is the number of entries in the higher layer parameter sl-PRS-ResourceReservePeriodList, if higher layer parameter sl-PRS-ResourceReservePeriodList is configured; 0 bit otherwise.

-	Time resource assignment – 5 bits when the value of the higher layer parameter sl-MaxNumPerReserveDedicatedSL-PRS-RP is configured to 2; otherwise 9 bits when the value of the higher layer parameter sl-MaxNumPerReserveDedicatedSL-PRS-RP is configured to 3, as defined in clause 8.2.4.3 of [6, TS 38.214].

-	Resource ID indication – log2NSL-PRS bits when the value of the higher layer parameter sl-MaxNumPerReserveDedicatedSL-PRS-RP is configured to 2; otherwise 2log2NSL-PRS bits when the value of the higher layer parameter sl-MaxNumPerReserveDedicatedSL-PRS-RP is configured to 3. The value NSL-PRS is the total number of SL PRS resources within a slot in a dedicated SL PRS resource pool and provided by the higher layer parameter SL-PRS-ResourceDedicatedSL-PRS-RP.

-	SL PRS request – 1 bit as defined in clause 8.4.4 of [6, TS 38.214] when the higher layer parameter sl-SCI-basedSL-PRS-TxTriggerSCI1-B is provided; 0 bit otherwise.

-	Reserved - Nreserved bits as configured by higher layer parameter sl-NumReservedBitsSCI1B-DedicatedSL-PRS-RP, with value set to zero.

…

8.4.1.4	SCI format 2-D

SCI format 2-D is used for the decoding of PSSCH and the scheduling of SL PRS for a shared SL PRS resource pool.

The following information is transmitted by means of the SCI format 2-D:

-	SL PRS resource ID - log2NSL-PRS bits, where the value NSL-PRS is the total number of SL PRS resource IDs within a slot in a shared SL PRS resource pool and provided by the higher layer parameter SL-PRS-ResourceSharedSL-PRS-RP.

-	SL PRS request – 1 bit as defined in clause 8.4.4 of [6, TS 38.214] when the higher layer parameter sl-SCI-based-SL-PRS-Tx-Trigger-SCI2-D is provided; 0 bit otherwise.

-	Embedded SCI format - 2 bits. This field indicates the embedded SCI format as defined in Table 8.4.1.4-1.

-	Embedded SCI format payload - number of bits determined according to Table 8.4.1.4-1. This field is set to the associated payload of the embedded SCI format indicated by the ‘Embedded SCI format’ field as defined in Table 8.4.1.4-1.

-----------------------------------< End of Text Proposal 5-3 for 38.212> -------------------------------

Companies views

| CATT | OK |
| --- | --- |

## 13.2 Alignment TP for TS38.214 [vivo] [ASUSTeK][CATT]

[vivo]

### TS 38.214 ### 8.2.4 SL PRS transmission procedure The following parameters for SL PRS transmission are associated with each SL PRS resource: - sL-PRS-ResourceID indicates an identity of a SL PRS resource. The SL PRS resource is identified by the SL PRS resource ID that is unique within a slot of a dedicated SL PRS resource pool. For a shared SL PRS resource pool, a SL PRS resource is uniquely identified by a combination of the SL PRS resource ID, SL PRS frequency domain allocation within a slot indicated by “frequency resource assignment” field in the associated SCI format 1-A, and a starting symbol within the slot as determined by clause 8.2.4.1.1. - sl-CombSize indicates a comb size of the SL PRS resource and sl-PRS-comb-offset indicates comb offset of the SL PRS resource - sl-PRS-starting-symbol indicates the starting symbol index of the SL PRS resource and sl-NumberOfSymbols indicates the number of symbols of the SL PRS resource within a slot in a dedicated SL PRS resource pool. sl-NumberOfSymbols indicates the number of symbols of the SL PRS resource within a slot in a shared SL PRS resource pool < Unchanged parts are omitted > The UE may report the association information between the already transmitted SL PRSs of SL PRS resources and UE Tx ARP ID. The association information includes ARP ID(s), SL PRS transmission timestamp(s), and optional SL PRS resource ID(s) < Unchanged parts are omitted > For a dedicated SL PRS resource pool, the UE transmits SL PRS subject to the following restrictions: - the UE shall not transmit SL PRS and associated PSCCH in the same symbol; - the number of contiguous symbols and the starting symbol for SL PRS transmission shall correspond to one of the SL PRS resources in parameter sl-PRS-ResourcesDedicatedSL-PRS-RP. < Unchanged parts are omitted > ### 8.2.4.2 UE procedure for determining the subset of resources to be reported to higher layers in SL PRS resource selection in a dedicated SL PRS resource pool in sidelink resource allocation mode 2 In resource allocation mode 2 in a dedicated SL PRS resource pool, the higher layer can request the UE to determine a subset of resources from which the higher layer will select resources for SL PRS/PSCCH transmission. To trigger this procedure, in slot n, the higher layer provides the following parameters for this SL PRS/PSCCH transmission: - the resource pool from which the resources are to be reported; - L1 priority, prioTX; - the remaining [delay budget]; - Set of SL-PRS resource ID(s); - optionally, the resource reservation interval, Prsvp_TX, in units of msec. - if the higher layer requests the UE to determine a subset of resources from which the higher layer will select resources for SL PRS/PSCCH transmission as part of re-evaluation or pre-emption procedure, the higher layer provides a set of resources (r0,r1,r2,…) which may be subject to re-evaluation and a set of resources (r0',r1',r2',…) which may be subject to pre-emption. - it is up to UE implementation to determine the subset of resources as requested by higher layers before or after the slot ri'' - T3, where ri'' is the slot with the smallest slot index among (r0,r1,r2,…) and (r0',r1',r2',…) , and T3 is equal to Tproc,1SL, where Tproc,1SL is defined in slots in Table 8.1.4-2 where μSL is the SCS configuration of the SL BWP. The following higher layer parameters affect this procedure: - [sl-SelectionWindowList: internal parameter T2min is set to the corresponding value from higher layer parameter sl-SelectionWindowList for the given value of prioTX.] - [sl-Thres-RSRP-List]: this higher layer parameter provides an RSRP threshold for each combination pi, pj, where pi is the value of the priority field in a received SCI format 1-B and pj is the priority of the transmission of the UE selecting resources; for a given invocation of this procedure, pj = prioTX. - sl-PRS-ResourceReservePeriodList: the resource reservation interval, Prsvp_TX, is set to the corresponding value from higher layer parameter in units of mse - sl-SensingWindowDedicatedSL-PRS-RP: internal parameter T0 is defined as the number of slots corresponding to sl-SensingWindowDedicatedSL-PRS-RP msec - sl-TxPercentageDedicatedSL-PRS-RP-List: internal parameter X for a given prioTX is defined as sl-TxPercentageDedicatedSL-PRS-RP-List (prioTX) converted from percentage to ratio - [sl-PreemptionEnable]: if sl-PreemptionEnable is provided, and if it is not equal to 'enabled', internal parameter priopre is set to the higher layer provided parameter sl-PreemptionEnable. < Unchanged parts are omitted > ### 8.2.4.2A UE procedure for determining slots and SL PRS resource(s) associated with an SCI format 1-B in a dedicated SL PRS resource pool The set of slots and SL PRS resources for SL PRS transmission is determined by the PSCCH containing the associated SCI format 1-B, and fields 'SL-PRS resource ID (s)', 'Time resource assignment' of the associated SCI format 1-B as described below. The set of slots is determined as in clause 8.1.5, with the following modifications: - "SCI format 1-A" is replaced by "SCI format 1-B", - [potential parameter name changes]. The first SL PRS resource is determined according to the sub-channel used for the PSCCH transmission containing the associated SCI format 1-B, where the index of the sub-channel in the resource pool is identical to the index of the SL PRS resource provided by sl-PRS-ResourceID. The second SL-PRS and third SL PRS resource, if reserved by SCI format 1-B, are determined from " Resource ID indication" which is equal to a PRS Resource ID value (PRIV) where, If sl-MaxNumPerReserveDedicatedSL-PRS-RP is 2 then PRIV=r1 If sl-MaxNumPerReserveDedicatedSL-PRS-RP is 3 then PRIV=r2*NSL-PRS+r1 Where - r1 denotes the SL PRS resource ID for the second resource - r2 denotes the SL PRS resource ID for the third resource - NSL-PRS is the number of SL-PRS resources (pre-)configured in a slot of a resource pool. If TRIV determined according to clause 8.1.5 indicates N < sl-MaxNumPerReserveDedicatedSL-PRS-RP, the SL PRS resource indices corresponding to sl-MaxNumPerReserveDedicatedSL-PRS-RP minus N last resources are not used.

- Reason for change: To resolve the misalignment issue about higher-layer parameter names between RAN1 and RAN2. - Summary of change: Modify some higher-layer parameter name and address the remaining issue. - Consequences if not approved: The specification may be not clear and aligned with higher layer specification.

[Asustek]

| Reason for change: | - Align field and parameter names as in TS 38.331 and TS 38.212. - Clarify some terminologies with bracket. |
| --- | --- |
| Summary of change: | - Align parameter names as TS 38.331. Change “SL-PRS resource ID (s)” to “Resource ID indication”. - Remove bracket of “[Time resource assignment] “. Remove bracket of “SL PRS[/PSCCH]”. Change ”[delay budget]” to ” SL PRS delay budget”. |
| Consequences if not approved: | - Unaligned field and parameter names. - Unclear terminologies. |
| Clauses affected: | 8.2.4.1.1, 8.2.4.2, 8.2.4.2A in TS 38.214. 16.2.3A and 16.4A in TS 38.213. 8.3.1.2 and 8.4.1.4 in TS 38.212. |

---------------------------------------< Text Proposal 5-1 for 38.214> -------------------------------------- 8.2.4 SL PRS transmission procedure The following parameters for SL PRS transmission are associated with each SL PRS resource: - sl-PRS-ResourceID indicates an identity of a SL PRS resource. The SL PRS resource is identified by the SL PRS resource ID that is unique within a slot of a dedicated SL PRS resource pool. For a shared SL PRS resource pool, a SL PRS resource is uniquely identified by a combination of the SL PRS resource ID, SL PRS frequency domain allocation within a slot indicated by “frequency resource assignment” field in the associated SCI format 1-A, and a starting symbol within the slot as determined by clause 8.2.4.1.1. - sl-PRS-comb-offset and sl-CombSize indicates a comb offset and a comb size of the SL PRS resource - sl-PRS-starting-symbol and sl-NumberOfSymbols indicates the starting symbol index and the number of symbols of the SL PRS resource within a slot in a dedicated SL PRS resource pool. [number of SL PRS symbols] indicates the number of symbols of the SL PRS resource within a slot in a shared SL PRS resource pool. … 8.2.4.1.1 Resource allocation in time domain The UE shall transmit the SL PRS in the same slot as the associated PSCCH. For a dedicated SL PRS resource pool, the minimum resource allocation unit in the time domain is a SL PRS resource in a slot. The UE shall transmit the SL PRS in consecutive symbols within the slot. A UE does not transmit multiple SL PRS resources in the same slot. For a shared SL PRS resource pool, the UE transmits the SL PRS in PSSCH symbols according to clause 8.1.2.1, with the following restrictions: - the number of contiguous symbols for SL PRS transmission, LSL-PRS, shall correspond to one of the SL PRS resources in parameter SL-PRS-ResourceSharedSL-PRS-RP. - the UE shall not transmit SL PRS in symbols where associated PSCCH is transmitted. - the UE shall not transmit SL PRS and PSSCH DMRS in the same symbol. - the UE shall not transmit SL PRS and SL CSI-RS in the same symbol. - the UE shall transmit SL PRS on contiguous symbols either in between or after symbols where PSSCH DMRS is transmitted. - the UE shall transmit SL PRS only after the last symbol with second stage SCI. - For a given value of LSL-PRS, SL PRS resource is mapped to the last consecutive LSL-PRS SL symbols in the slot that meet all the other restrictions For a dedicated SL PRS resource pool, the UE transmits SL PRS subject to the following restrictions: - the UE shall not transmit SL PRS and associated PSCCH in the same symbol; - the number of contiguous symbols and the starting symbol for SL PRS transmission shall correspond to one of the SL PRS resources in parameter SL-PRS-ResourceDedicatedSL-PRS-RP. In sidelink resource allocation mode 1 for a shared SL PRS resource pool, the time domain behaviour for sidelink dynamic grants and sidelink configured grants for SL PRS follows the behaviour in clause 8.1.2.1. In sidelink resource allocation mode 1 for a dedicated SL PRS resource pool, the time domain behaviour for sidelink dynamic grants and sidelink configured grants for SL PRS follows the behaviour in clause 8.1.2.1, with the following modifications: - "DCI format 3_0" is replaced by "DCI format 3_2". - "PSSCH" is replaced by "SL PRS". … 8.2.4.2 UE procedure for determining the subset of resources to be reported to higher layers in SL PRS resource selection in a dedicated SL PRS resource pool in sidelink resource allocation mode 2 In resource allocation mode 2 in a dedicated SL PRS resource pool, the higher layer can request the UE to determine a subset of resources from which the higher layer will select resources for SL PRS/PSCCH transmission. To trigger this procedure, in slot n, the higher layer provides the following parameters for this SL PRS/PSCCH transmission: - the resource pool from which the resources are to be reported; - L1 priority, prioTX; - the remaining SL PRS delay budget; - Set of SL-PRS resource ID(s); - optionally, the resource reservation interval, Prsvp_TX, in units of msec. - if the higher layer requests the UE to determine a subset of resources from which the higher layer will select resources for SL PRS/PSCCH transmission as part of re-evaluation or pre-emption procedure, the higher layer provides a set of resources (r0,r1,r2,…) which may be subject to re-evaluation and a set of resources (r0',r1',r2',…) which may be subject to pre-emption. - it is up to UE implementation to determine the subset of resources as requested by higher layers before or after the slot ri'' - T3, where ri'' is the slot with the smallest slot index among (r0,r1,r2,…) and (r0',r1',r2',…) , and T3 is equal to Tproc,1SL, where Tproc,1SL is defined in slots in Table 8.1.4-2 where μSL is the SCS configuration of the SL BWP. The following higher layer parameters affect this procedure: - [sl-SelectionWindowList: internal parameter T2min is set to the corresponding value from higher layer parameter sl-SelectionWindowList for the given value of prioTX.] - [sl-Thres-RSRP-List]: this higher layer parameter provides an RSRP threshold for each combination pi, pj, where pi is the value of the priority field in a received SCI format 1-B and pj is the priority of the transmission of the UE selecting resources; for a given invocation of this procedure, pj = prioTX. - sl-PRS-ResourceReservePeriodList - sl-SensingWindowDedicatedSL-PRS-RP: internal parameter T0 is defined as the number of slots corresponding to sl-SensingWindowDedicatedSL-PRS-RP msec - sl-TxPercentageDedicatedSL-PRS-RP-List: internal parameter X for a given prioTX is defined as sl-TxPercentageDedicatedSL-PRS-RP-List (prioTX) converted from percentage to ratio - [sl-PreemptionEnable]: if sl-PreemptionEnable is provided, and if it is not equal to 'enabled', internal parameter priopre is set to the higher layer provided parameter sl-PreemptionEnable. The UE shall perform this procedure according to clause 8.1.4, with the following modifications: - "packet delay budget" is replaced by "SL PRS delay budget", - partial sensing is not applicable in a dedicated SL PRS resource pool, - "candidate single-slot resource" is replaced by "candidate SL PRS resource", - a candidate single-slot resource for transmission Rx,y is defined as the SL PRS resource with index x within the Set of SL-PRS resource ID(s) provided by the higher layer and in slot t'ySL, - "SCI format 1-A" is replaced by "SCI format 1-B", - in step 5, the second condition is modified as follows: for any periodicity value allowed by the higher layer parameter sl-PRS-ResourceReservePeriodList and any SL PRS resource ID in the set of SL PRS resource ID(s) provided by the higher layer, and a hypothetical SCI format 1-B received in slot t'mSL with 'Resource reservation period' field set to that periodicity value and indicating that SL-PRS resource ID, condition c in step 6 would be met, - In condition b of step 6, the RSRP measurement is the PSCCH-RSRP over the DM-RS resource elements of the PSSCH; - In condition c of step 6 "determines according to clause 8.1.5 the set of resource blocks and slots" is replaced by "determines according to clause 8.2.4.2A the set of SL PRS resources and slots ". 8.2.4.2A UE procedure for determining slots and SL PRS resource(s) associated with an SCI format 1-B in a dedicated SL PRS resource pool The set of slots and SL PRS resources for SL PRS transmission is determined by the PSCCH containing the associated SCI format 1-B, and fields 'Resource ID indication', 'Time resource assignment' of the associated SCI format 1-B as described below. The set of slots is determined as in clause 8.1.5, with the following modifications: - "SCI format 1-A" is replaced by "SCI format 1-B", - [potential parameter name changes]. The first SL PRS resource is determined according to the sub-channel used for the PSCCH transmission containing the associated SCI format 1-B, where the index of the sub-channel in the resource pool is identical to the index of the SL PRS resource provided by [higher layer parameter]. The second SL-PRS and third SL PRS resource, if reserved by SCI format 1-B, are determined from " Resource ID indication" which is equal to a PRS Resource ID value (PRIV) where, If sl-MaxNumPerReserveDedicatedSL-PRS-RP is 2 then PRIV=r1 If sl-MaxNumPerReserveDedicatedSL-PRS-RP is 3 then PRIV=r2*NSL-PRS+r1 Where - r1 denotes the SL PRS resource ID for the second resource - r2 denotes the SL PRS resource ID for the third resource - NSL-PRS is the number of SL-PRS resources (pre-)configured in a slot of a resource pool. If TRIV determined according to clause 8.1.5 indicates N < sl-MaxNumPerReserveDedicatedSL-PRS-RP, the SL PRS resource indices corresponding to sl-MaxNumPerReserveDedicatedSL-PRS-RP minus N last resources are not used. -----------------------------------< End of Text Proposal 5-1 for 38.214> -------------------------------

[CATT]

| Reason for change: | Align the higher layer parameters of current specification associated with resource allocation mode 2 in a dedicated SL PRS resource pool. |
| --- | --- |
| Summary of change: | In clause 8.2.4.2 of TS 38.214, the higher layer parameters are captured. |
| Consequences if not approved: | The names of higher layer parameters between 38.214 and 38.331 are unaligned. |

------------------------------------- Start of text proposal to TS 38.214 v18.1.0 -----------------------------------

8.2.4.2	UE procedure for determining the subset of resources to be reported to higher layers in SL PRS resource selection in a dedicated SL PRS resource pool in sidelink resource allocation mode 2

In resource allocation mode 2 in a dedicated SL PRS resource pool, the higher layer can request the UE to determine a subset of resources from which the higher layer will select resources for SL PRS/PSCCH transmission. To trigger this procedure, in slot n, the higher layer provides the following parameters for this SL PRS/PSCCH transmission:

-	the resource pool from which the resources are to be reported;

-	L1 priority, prioTX;

-	the remaining SL PRS delay budget;

-	Set of SL-PRS resource ID(s);

-	optionally, the resource reservation interval, Prsvp_TX, in units of msec.

-	if the higher layer requests the UE to determine a subset of resources from which the higher layer will select resources for SL PRS[/PSCCH] transmission as part of re-evaluation or pre-emption procedure, the higher layer provides a set of resources (r0,r1,r2,…) which may be subject to re-evaluation and a set of resources (r0',r1',r2',…) which may be subject to pre-emption.

-	it is up to UE implementation to determine the subset of resources as requested by higher layers before or after the slot ri'' - T3, where ri'' is the slot with the smallest slot index among (r0,r1,r2,…) and (r0',r1',r2',…) , and T3 is equal to Tproc,1SL, where Tproc,1SL  is defined in slots in Table 8.1.4-2 where μSL is the SCS configuration of the SL BWP.

The following higher layer parameters affect this procedure:

-	sl-SelectionWindowListDedicatedSL-PRS-RP: internal parameter T2min is set to the corresponding value from higher layer parameter sl-SelectionWindowList for the given value of prioTX.

-	sl-Thres-RSRP-ListDedicatedSL-PRS-RP: this higher layer parameter provides an RSRP threshold for each combination pi, pj, where pi is the value of the priority field in a received SCI format 1-B and pj is the priority of the transmission of the UE selecting resources; for a given invocation of this procedure, pj = prioTX.

-	sl-ReservationPeriodAllowedDedicatedSL-PRS-RP

-	sl-SensingWindowDedicatedSL-PRS-RP: internal parameter T0 is defined as the number of slots corresponding to sl-SensingWindowDedicatedSL-PRS-RP

-	sl-TxPercentageListDedicatedSL-PRS-RP: internal parameter X for a given prioTX is defined as sl-TxPercentageList (prioTX) converted from percentage to ratio

-	sl-PreemptionEnableDedicatedSL-PRS-RP: if sl-PreemptionEnable is provided, and if it is not equal to 'enabled', internal parameter priopre is set to the higher layer provided parameter sl-PreemptionEnable.

The UE shall perform this procedure according to clause 8.1.4, with the following modifications:

-	"packet delay budget" is replaced by "SL PRS delay budget",

-	partial sensing is not applicable in a dedicated SL PRS resource pool,

-	"candidate single-slot resource" is replaced by "candidate SL PRS resource",

-	a candidate single-slot resource for transmission Rx,y is defined as the SL PRS resource with index x within the Set of SL-PRS resource ID(s) provided by the higher layer and in slot t'ySL,

-	"SCI format 1-A" is replaced by "SCI format 1-B",

-	in step 5, the second condition is modified as follows: for any periodicity value allowed by the higher layer parameter sl-PRS-ResourceReservePeriodList and any SL PRS resource ID in the set of SL PRS resource ID(s) provided by the higher layer, and a hypothetical SCI format 1-B received in slot t'mSL with 'Resource reservation period' field set to that periodicity value and indicating that SL-PRS resource ID, condition c in step 6 would be met,

-	In condition b of step 6, the RSRP measurement is the PSCCH-RSRP over the DM-RS resource elements of the PSCCH;

<<< UNCHANGED PARTS OMITTED >>>

8.2.4.2A	UE procedure for determining slots and SL PRS resource(s) associated with an SCI format 1-B in a dedicated SL PRS resource pool

The set of slots and SL PRS resources for SL PRS transmission is determined by the PSCCH containing the associated SCI format 1-B, and fields '[SL-PRS resource ID (s))', '[Time resource assignment]' of the associated SCI format 1-B as described below.

The set of slots is determined as in clause 8.1.5, with the following modifications:

-	"SCI format 1-A" is replaced by "SCI format 1-B",

-   "sl-MaxNumPerReserve" is replaced by "sl-MaxNumPerReserve-Dedicated-SL-PRS-RP"

The first SL PRS resource is determined according to the sub-channel used for the PSCCH transmission containing the associated SCI format 1-B, where the index of the sub-channel in the resource pool is identical to the index of the SL PRS resource provided by SL-PRS-ResourceDedicatedSL-PRS-RP-r18.

The second SL-PRS and third SL PRS resource, if reserved by SCI format 1-B, are determined from " Resource ID indication" which is equal to a PRS Resource ID value (PRIV) where,

If sl-MaxNumPerReserve-Dedicated-SL-PRS-RP is 2 then

PRIV=r1

If sl-MaxNumPerReserve-Dedicated-SL-PRS-RP is 3 then

PRIV=r2*NSL-PRS+r1

Where

-	 r1 denotes the SL PRS resource ID for the second resource

-	 r2 denotes the SL PRS resource ID for the third resource

-	 NSL-PRS is the number of SL-PRS resources (pre-)configured in a slot of a resource pool.

If TRIV determined according to clause 8.1.5 indicates N < sl-MaxNumPerReserve-Dedicated-SL-PRS-RP, the SL PRS resource indices corresponding to sl-MaxNumPerReserve-Dedicated-SL-PRS-RP minus N last resources are not used.

<<< UNCHANGED PARTS OMITTED >>>

8.2.4.3	Sidelink congestion control in a dedicated SL PRS resource pool in sidelink resource allocation mode 2

When transmitting SL-PRS in a dedicated SL PRS resource pool the UE shall perform sidelink congestion control as specified in clause 8.1.6, with the following modification(s):

-	"PSSCH" is replaced by "SL PRS"

-	[potential parameter name changes]

<<< UNCHANGED PARTS OMITTED >>>

8.4.4	SL PRS reception procedure

The UE may be configured, via RequestLocationInformation, to measure and report one or more of the SL RSTD, SL Rx-Tx time difference, SL RTOA, SL PRS-RSRPP, for the first detected path and up to 8 additional detected paths, and SL PRS-RSRP measurements. The UE may be configured, via RequestLocationInformation, to measure and report one or more of the SL AoA, SL PRS-RSRPP for the first path and up to 2 additional detected paths, and SL PRS-RSRP measurement.

The UE may report an ARP ID associated with the reported measurements. The UE may provide the ARP location information via sl-ARP-LocationInfoPerTxUE.

The UE uses the same ARP for both the transmission and reception of sidelink positioning reference signals while performing an SL Rx-Tx time difference measurement.

The UE may include SL PRS resource ID(s) when it reports one or more of the SL RSTD, SL Rx-Tx time difference, SL RTOA, SL AoA, SL PRS-RSRP, and SL PRS-RSRPP measurements.

For the SL RSTD, SL Rx-Tx time difference, SL RTOA, SL AoA, SL PRS-RSRP, and SL PRS-RSRPP measurements, the UE reports an associated SL PRS reception timestamp via higher layer parameter sl-TimeStamp. For SL Rx-Tx time difference, the UE may report an associated SL PRS transmission timestamp via higher layer parameter sl-TimeStamp and the UE may be configured to report a SL PRS transmission timestamp via tx-TimeInfo. The timestamp includes the SFN, slot number, and optionally nr-PhysCellID, nr-ARFCN, nr-CellGlobalID, or the timestamp includes DFN and slot number. The timestamp of DFN and slot number may include synchronization source indication of DFN.

The UE may be configured to report up to N Rx-Tx time difference measurements for the same SL PRS transmission associated with N different SL PRS receptions for the same pair of UE(s). The UE may be configured to report up to N Rx-Tx time difference measurements for the same SL PRS reception associated with N different SL PRS transmissions for the same pair of UE(s).

The UE may report, LoS/NLoS indicator(s) via los-NLOS-Indicator associated with each SL RSTD, SL Rx-Tx time difference, SL RTOA, SL AoA, SL PRS-RSRP, and SL PRS-RSRPP measurements.

The UE may report synchronization source type via syncSourceType and/or relative time difference with the associated quality metric, via rtdBetweenAnchorUEs. For the SL RSTD measurement, the UE may report a reference UE information, via sl-RSTD-ReferenceUE-Info.

For SL RTOA measurement, SFN or DFN initialization time may be provided to the UE by a UE or the network.

The UE may be provided with the location information of other UEs via [higher layer parameter]. The UE may report the location information of the UE to the network.

The UE may be provided with expected SL AoA and uncertainty range of the expected SL AoA via expectedSL-AzimuthAoA-AndUncertainty and expectedSL-ZenithAoA-AndUncertainty.

The UE may report quality metric sl-TimingQuality corresponding to the SL RSTD, SL RTOA or SL Rx-Tx time difference measurements. The UE may report quality metric sl-AngleQuality corresponding to the SL AoA measurement.

If the 'SL PRS request' field in the SCI associated with the received SL PRS is set to 1 then this request for SL PRS transmission is reported to higher layers.

<<< UNCHANGED PARTS OMITTED >>>

--------------------------------------- End of text proposal to TS 38.214 v18.1.0 ---------------------------------------------

[LOW] Feature Lead Proposal 13.2-v0

Companies are encouraged to comment on the alignment TP shown below:

--------------------------------------- Start of text proposal to TS 38.214 v18.1.0 ---------------------------------------------

8.2.4	SL PRS transmission procedure

The following parameters for SL PRS transmission are associated with each SL PRS resource:

-	sL-PRS-ResourceID indicates an identity of a SL PRS resource. The SL PRS resource is identified by the SL PRS resource ID that is unique within a slot of a dedicated SL PRS resource pool. For a shared SL PRS resource pool, a SL PRS resource is uniquely identified by a combination of the SL PRS resource ID, SL PRS frequency domain allocation within a slot indicated by “frequency resource assignment” field in the associated SCI format 1-A, and a starting symbol within the slot as determined by clause 8.2.4.1.1.

-	sl-CombSize indicates a comb size of the SL PRS resource and sl-PRS-comb-offset indicates comb offset of the SL PRS resource

-	sl-PRS-starting-symbol indicates the starting symbol index of the SL PRS resource and sl-NumberOfSymbols indicates the number of symbols of the SL PRS resource within a slot in a dedicated SL PRS resource pool. sl-NumberOfSymbols indicates the number of symbols of the SL PRS resource within a slot in a shared SL PRS resource pool

< Unchanged parts are omitted >

The UE may report the association information between the already transmitted SL PRSs of SL PRS resources and UE Tx ARP ID. The association information includes ARP ID(s), SL PRS transmission timestamp(s), and optional SL PRS resource ID(s)

< Unchanged parts are omitted >

For a dedicated SL PRS resource pool, the UE transmits SL PRS subject to the following restrictions:

-	the UE shall not transmit SL PRS and associated PSCCH in the same symbol;

-	the number of contiguous symbols and the starting symbol for SL PRS transmission shall correspond to one of the SL PRS resources in parameter sl-PRS-ResourcesDedicatedSL-PRS-RP.

< Unchanged parts are omitted >

8.2.4.1.1	Resource allocation in time domain

The UE shall transmit the SL PRS in the same slot as the associated PSCCH.

For a dedicated SL PRS resource pool, the minimum resource allocation unit in the time domain is a SL PRS resource in a slot.

The UE shall transmit the SL PRS in consecutive symbols within the slot.

A UE does not transmit multiple SL PRS resources in the same slot.

For a shared SL PRS resource pool, the UE transmits the SL PRS in PSSCH symbols according to clause 8.1.2.1, with the following restrictions:

-	the number of contiguous symbols for SL PRS transmission, LSL-PRS, shall correspond to one of the SL PRS resources in parameter SL-PRS-ResourceSharedSL-PRS-RP.

-	the UE shall not transmit SL PRS in symbols where associated PSCCH is transmitted.

-	the UE shall not transmit SL PRS and PSSCH DMRS in the same symbol.

-	the UE shall not transmit SL PRS and SL CSI-RS in the same symbol.

-	the UE shall transmit SL PRS on contiguous symbols either in between or after symbols where PSSCH DMRS is transmitted.

-	the UE shall transmit SL PRS only after the last symbol with second stage SCI.

-	For a given value of LSL-PRS, SL PRS resource is mapped to the last consecutive LSL-PRS SL symbols in the slot that meet all the other restrictions

For a dedicated SL PRS resource pool, the UE transmits SL PRS subject to the following restrictions:

-	the UE shall not transmit SL PRS and associated PSCCH in the same symbol;

-	the number of contiguous symbols and the starting symbol for SL PRS transmission shall correspond to one of the SL PRS resources in parameter SL-PRS-ResourceDedicatedSL-PRS-RP.

In sidelink resource allocation mode 1 for a shared SL PRS resource pool, the time domain behaviour for sidelink dynamic grants and sidelink configured grants for SL PRS follows the behaviour in clause 8.1.2.1.

In sidelink resource allocation mode 1 for a dedicated SL PRS resource pool, the time domain behaviour for sidelink dynamic grants and sidelink configured grants for SL PRS follows the behaviour in clause 8.1.2.1, with the following modifications:

-	"DCI format 3_0" is replaced by "DCI format 3_2".

-	"PSSCH" is replaced by "SL PRS".

< Unchanged parts are omitted >

8.2.4.2 UE procedure for determining the subset of resources to be reported to higher layers in SL PRS resource selection in a dedicated SL PRS resource pool in sidelink resource allocation mode 2

In resource allocation mode 2 in a dedicated SL PRS resource pool, the higher layer can request the UE to determine a subset of resources from which the higher layer will select resources for SL PRS/PSCCH transmission. To trigger this procedure, in slot n, the higher layer provides the following parameters for this SL PRS/PSCCH transmission:

-	the resource pool from which the resources are to be reported;

-	L1 priority, prioTX;

-	the remaining [delay budget];

-	Set of SL-PRS resource ID(s);

-	optionally, the resource reservation interval, Prsvp_TX, in units of msec.

-	if the higher layer requests the UE to determine a subset of resources from which the higher layer will select resources for SL PRS/PSCCH transmission as part of re-evaluation or pre-emption procedure, the higher layer provides a set of resources (r0,r1,r2,…) which may be subject to re-evaluation and a set of resources (r0',r1',r2',…) which may be subject to pre-emption.

-	it is up to UE implementation to determine the subset of resources as requested by higher layers before or after the slot ri'' - T3, where ri'' is the slot with the smallest slot index among (r0,r1,r2,…) and (r0',r1',r2',…) , and T3 is equal to Tproc,1SL, where Tproc,1SL  is defined in slots in Table 8.1.4-2 where μSL is the SCS configuration of the SL BWP.

The following higher layer parameters affect this procedure:

-	[sl-SelectionWindowList: internal parameter T2min is set to the corresponding value from higher layer parameter sl-SelectionWindowList for the given value of prioTX.]

-	[sl-Thres-RSRP-List]: this higher layer parameter provides an RSRP threshold for each combination pi, pj, where pi is the value of the priority field in a received SCI format 1-B and pj is the priority of the transmission of the UE selecting resources; for a given invocation of this procedure, pj = prioTX.

-	sl-PRS-ResourceReservePeriodList:  the resource reservation interval, Prsvp_TX, is set to the corresponding value from higher layer parameter in units of mse

-	sl-SensingWindowDedicatedSL-PRS-RP: internal parameter T0 is defined as the number of slots corresponding to sl-SensingWindowDedicatedSL-PRS-RP msec

-	sl-TxPercentageDedicatedSL-PRS-RP-List: internal parameter X for a given prioTX is defined as sl-TxPercentageDedicatedSL-PRS-RP-List (prioTX) converted from percentage to ratio

-	[sl-PreemptionEnable]: if sl-PreemptionEnable is provided, and if it is not equal to 'enabled', internal parameter priopre is set to the higher layer provided parameter sl-PreemptionEnable.

The UE shall perform this procedure according to clause 8.1.4, with the following modifications:

-	"packet delay budget" is replaced by "SL PRS delay budget",

-	partial sensing is not applicable in a dedicated SL PRS resource pool,

-	"candidate single-slot resource" is replaced by "candidate SL PRS resource",

-	a candidate single-slot resource for transmission Rx,y is defined as the SL PRS resource with index x within the Set of SL-PRS resource ID(s) provided by the higher layer and in slot t'ySL,

-	"SCI format 1-A" is replaced by "SCI format 1-B",

-	in step 5, the second condition is modified as follows: for any periodicity value allowed by the higher layer parameter sl-PRS-ResourceReservePeriodList and any SL PRS resource ID in the set of SL PRS resource ID(s) provided by the higher layer, and a hypothetical SCI format 1-B received in slot t'mSL with 'Resource reservation period' field set to that periodicity value and indicating that SL-PRS resource ID, condition c in step 6 would be met,

-	In condition b of step 6, the RSRP measurement is the PSCCH-RSRP over the DM-RS resource elements of the PSSCH;

-	In condition c of step 6 "determines according to clause 8.1.5 the set of resource blocks and slots" is replaced by "determines according to clause 8.2.4.2A the set of SL PRS resources and slots ".

< Unchanged parts are omitted >

8.2.4.2A	UE procedure for determining slots and SL PRS resource(s) associated with an SCI format 1-B in a dedicated SL PRS resource pool

The set of slots and SL PRS resources for SL PRS transmission is determined by the PSCCH containing the associated SCI format 1-B, and fields 'SL-PRS resource ID (s)', 'Time resource assignment' of the associated SCI format 1-B as described below.

The set of slots is determined as in clause 8.1.5, with the following modifications:

-	"SCI format 1-A" is replaced by "SCI format 1-B",

-	[potential parameter name changes].

The first SL PRS resource is determined according to the sub-channel used for the PSCCH transmission containing the associated SCI format 1-B, where the index of the sub-channel in the resource pool is identical to the index of the SL PRS resource provided by sl-PRS-ResourceID.

The second SL-PRS and third SL PRS resource, if reserved by SCI format 1-B, are determined from " Resource ID indication" which is equal to a PRS Resource ID value (PRIV) where,

If sl-MaxNumPerReserveDedicatedSL-PRS-RP is 2 then

PRIV=r1

If sl-MaxNumPerReserveDedicatedSL-PRS-RP is 3 then

PRIV=r2*NSL-PRS+r1

Where

-	 r1 denotes the SL PRS resource ID for the second resource

-	 r2 denotes the SL PRS resource ID for the third resource

-	 NSL-PRS is the number of SL-PRS resources (pre-)configured in a slot of a resource pool.

If TRIV determined according to clause 8.1.5 indicates N < sl-MaxNumPerReserveDedicatedSL-PRS-RP, the SL PRS resource indices corresponding to sl-MaxNumPerReserveDedicatedSL-PRS-RP minus N last resources are not used.

--------------------------------------- End of text proposal to TS 38.214 v18.1.0 ---------------------------------------------

Companies views

| CATT | OK |
| --- | --- |

## 13.3 Alignment TP for TS38.213 [ASUSTeK]

---------------------------------------< Text Proposal 5-2 for 38.213> --------------------------------------

16.2.3A	SL PRS

A UE determines a power PSL-PRSi for a SL PRS transmission on a resource pool in SL PRS transmission occasion i on active SL BWP b of carrier f as:

PSL-PRSi=minPCMAX,PMAX,CBR,minPSL-PRS,Di, PSL-PRS,SLi

where,

-	PCMAX is defined in [8-1, TS 38.101-1]

-	PMAX,CBR is determined by a value of sl-MaxTxPower based on a priority level and a CBR range for a CBR measured in slot i-N, where N is the congestion control processing time [6, TS 38.214] ]; if sl-MaxTxPower is not provided, then PMAX,CBR=PCMAX

-	if the resource pool is a shared SL PRS resource pool, the priority level is same for PSSCH and SL PRS

-	if the resource pool is a dedicated SL PRS resource pool, the priority level is for SL PRS

-	if a value for PO,D is provided

-	PSL-PRS,Di=PO,D+10log102μ∙MRBSL-PRSi+αD∙PLD [dBm]

-	else

-	PSL-PRS,Di=minPCMAX,PMAX,CBR [dBm]

where

-	if the resource pool is a shared SL PRS resource pool, PO,D is a value of dl-P0-PSSCH-PSCCH or dl-P0-PSSCH-PSCCH-r17; else, if the resource pool is a dedicated SL PRS resource pool, PO,D is a value of dl-P0-SL-PRS

-	if the resource pool is a shared SL PRS resource pool, αD is a value of dl-Alpha-PSSCH-PSCCH, if provided, and αD=1 if dl-Alpha-PSSCH-PSCCH is not provided; else, if the resource pool is a dedicated SL PRS resource pool, αD is provided by dl-Alpha-SL-PRS, if provided, and αD=1 if dl-Alpha-SL-PRS is not provided

-	PLD=PLb,f,c(qd) when the active SL BWP is on a serving cell c, as described in clause 7.1.1 except that

-	the RS resource qd is the one the UE uses for determining a power of a PUSCH transmission scheduled by a DCI format 0_0 in serving cell c when the UE is configured to monitor PDCCH for detection of DCI format 0_0 in serving cell c

-	the RS resource qd is the one corresponding to the SS/PBCH block the UE uses to obtain MIB when the UE is not configured to monitor PDCCH for detection of DCI format 0_0 in serving cell c

-	MRBSL-PRS(i) is a number of resource blocks for the SL PRS transmission occasion i and μ is a SCS configuration for the SL PRS transmission

-	if a value for PO,SL is provided

-	PSL-PRS,SLi=PO,SL+10log102μ∙MRBSL-PRSi+αSL∙PLSL [dBm]

-	else

-	PSL-PRS,SLi=minPCMAX,PSL-PRS,Di [dBm]

where

-	if the resource pool is a shared SL PRS resource pool, PO,SL is a value of sl-P0-PSSCH-PSCCH or sl-P0-PSSCH-PSCCH-r17; else, if the resource pool is dedicated for SL PRS transmissions, PO,SL is a value of sl-P0-SL-PRS

-	if the resource pool is a shared SL PRS resource pool, αSL is a value of sl-Alpha-PSSCH-PSCCH, if provided and αSL=1 if sl-Alpha-PSSCH-PSCCH is not provided; else, if the resource pool is a dedicated SL PRS resource pool, αSL is provided by sl-Alpha-SL-PRS if provided, and αSL=1 if sl-Alpha-SL-PRS is not provided

…

16.4A	UE procedure for transmitting PSCCH in dedicated SL PRS resource pool

For a dedicated SL PRS resource pool, a UE can be provided a number of symbols in the resource pool, by timeResourcePSCCH-DedicatedSL-PRS-RP, starting from a second symbol that is available for SL transmissions in a slot, and a number of PRBs in the resource pool, by freqResourcePSCCH-DedicatedSL-PRS-RP, starting from a PRB with lowest index for a sub-channel determined according to an index of an associated SL PRS resource, for a PSCCH transmission with a SCI format 1-B.

A UE that transmits a PSCCH with SCI format 1-B using SL PRS resource allocation mode 2 [6, TS 38.214] sets

-	"Source ID" as indicated by higher layers

-	"Destination ID" as indicated by high layers

-	"Cast type indicator" as indicated by higher layers

-	"Resource reservation period" as an index in sl-PRS-ResourceReservePeriodList corresponding to a reservation period provided by higher layers [11, TS 38.321], if the UE is provided sl-MultiReserveResource

-	the values of the time resource assignment field and of the resource ID indication field as described in [6, TS 38.214] to indicate N resources from a set Ry of resources selected by higher layers as described in [11, TS 38.321] with N smallest slot indices  yi for 0≤i≤N-1 such that y0<y1<…<yN-1≤y0+31, where:

-	N=minNselected, Nmax_reserve, where Nselected is a number of resources in the set Ry with slot indices  yj, 0≤j≤Nselected-1, such that y0<y1<…<yNselected-1≤y0+31, and Nmax_reserve is provided by sl-MaxNumPerReserveDedicatedSL-PRS-RP

-	each resource, from the set of Ry resources, corresponds to a SL PRS resource and the corresponding PSCCH, and a slot in a set of slots {t'ySL}

-	t'0SL,t'1SL,t'2SL,... is a set of slots in a sidelink resource pool [6, TS 38.214]

-	y0 is an index of a slot where the PSCCH with SCI format 1-B is transmitted.

-	"SL PRS request" as indicated by higher layers

A UE that transmits a PSCCH with SCI format 1-B using SL PRS resource allocation mode 1 [6, TS 38.214] sets

-	"Source ID" as indicated by higher layers

-	"Destination ID" as indicated by high layers

-	"Cast type indicator" as indicated by higher layers

-	the values of the resource ID indication field and the time resource assignment field for the SCI format 1-B transmitted in the m-th resource for SL PRS and the corresponding PSCCH transmission provided by a dynamic grant or by a SL configured grant, where m= 1,…,M and M is the total number of resources for SL PRS and the corresponding PSCCH transmission provided by a dynamic grant or the number of resources for SL PRS transmission in a period provided by a SL configured grant type 1 or SL configured grant type 2, as follows:

-	the resource ID indication field and time resource assignment field indicate the m-th to M-th resources as described in [6, TS 38.214].

-	"SL PRS request" as indicated by higher layers

For decoding of a SCI format 1-B, a UE may assume that a number of bits provided by sl-NumReservedBitsSCI1B-DedicatedSL-PRS-RP can have any value as described in [4, TS 38.212].

-----------------------------------< End of Text Proposal 5-2 for 38.213> -------------------------------

[LOW] Feature Lead Proposal 13.3-v0

Companies are encouraged to comment on the above alignment TP

Companies views

| CATT | OK |
| --- | --- |

## 13.4 Alignment TP for TS38.215 [Intel]

Agree on TP#8 to correct the higher layer parameter names sl-ThreshS-PRS-RSSI-CBR and sl-TimeWindowSize-PRS-CR referenced in Section 5.1.49 and Section 5.1.48 of TS 38.215 to accurately reflect the quantity represented and align parameter names, respectively.

| Reason for change | 1. In the definition for SL PRS-CBR, the current parameter name” is not reflective of the quantity represented. A more appropriate choice would be sl-ThreshS-PRS-RSSI-PRS-CBR since “S-RSSI”, corresponds to SL RSSI in the intended name. 2. The current parameter name sl-TimeWindowSize-PRS-CR is not aligned with the parameter name suggested to RAN2, i.e., sl-TimeWindowSizeCR-Dedicated-SL-PRS-RP. |
| --- | --- |
| Summary of change | 1. Section 5.1.49: Update the parameter name sl-ThreshS-PRS-RSSI-CBR to sl-ThreshS-RSSI-PRS-CBR. 2. Section 5.1.48: Update the parameter name sl-TimeWindowSize-PRS-CR to sl-TimeWindowSizeCR-Dedicated-SL-PRS-RP. |
| Consequences if not approved | Incorrect parameter naming. |
| Text proposal | ------------------------------ TP#8: TS 38.215 ----------------------------------- ### 5.1.48 Sidelink PRS channel occupancy ratio (SL PRS-CR) Definition; Sidelink PRS Channel Occupancy Ratio (SL PRS-CR) evaluated at slot n is defined as the total number of SL PRS resource in the dedicated SL PRS resource pool used for its transmissions in slots [n-a, n-1] and granted in slots [n, n+b] divided by the total number of configured SL PRS resources in the transmission pool over [n-a, n+b].. Applicable for; sidelink. NOTE 1: a is a positive integer and b is 0 or a positive integer; a and b are determined by UE implementation with a+b+1 = 1000 or 1000·2µ slots, according to higher layer parameter sl-TimeWindowSizeCR-Dedicated-SL-PRS-RP, b < (a+b+1)/2, and n+b shall not exceed the last transmission opportunity of the grant for the current transmission. NOTE 2: SL PRS-CR is evaluated for each (re)transmission. NOTE 3: In evaluating SL PRS-CR, the UE shall assume the transmission parameter used at slot n is reused according to the existing grant(s) in slot [n+1, n+b] without dropping. NOTE 4: The slot index is based on physical slot index. NOTE 5: SL PRS-CR can be computed per priority level NOTE 6: A resource is considered granted if it is a member of a selected sidelink grant as defined in TS 38.321 [7]. ### 5.1.49 Sidelink PRS channel busy ratio (SL PRS-CBR) Definition; SL PRS Channel Busy Ratio (SL PRS-CBR) measured in slot n is defined as the number of SL PRS resources in the dedicated SL PRS resource pool whose SL PRS RSSI measured by the UE exceed a (pre-)configured threshold provided by the higher layer parameter sl-ThreshS -RSSI-PRS-CBR sensed over a SL PRS-CBR measurement window [n-a, n-1], wherein a is equal to 100 or 100·2µ slots, according to higher layer parameter [sl-TimeWindowSize-PRS-CBR-positioning] divided by the total number of the configured SL PRS resources in the transmission pool over [n-a,n-1]. The calculation of SL PRS-CBR is limited within the slots for which the SL PRS-RSSI is measured. If the number of SL PRS-RSSI measurement slots within the SL PRS-CBR measurement window is below a (pre-)configured threshold, a (pre-)configured SL PRS-CBR value is used.. Applicable for; sidelink. < Unchanged text omitted > |

[LOW] Feature Lead Proposal 13.4-v0

Companies are encouraged to comment on the above alignment TP

Companies views

- TP for 38.202 [Intel][Qualcomm]

[MEDIUM] Feature Lead Proposal 14-v0

Please comment which of the following TPs you are supporting:

| Reason for change | For the characterization of simultaneous “Reception Type” combinations for sidelink, further qualification would be necessary to describe the scope within which the numbers of simultaneous “Reception Type” combinations apply for SL PRS reception. In particular, - For a shared SL PRS resource pool, the number of simultaneous SL PRS receptions should be defined within one sub-channel to align with SL communications (cf. Note 1 applicable for PSSCH and PSCCH). - For a dedicated SL PRS resource pool, the number of simultaneous SL PRS receptions should be defined within a dedicated SL PRS resource pool (analogous to a sub-channel for SL communications). |
| --- | --- |
| Summary of change | Clarify notes Note 3 and 4 in Table 6.3-4 |
| Consequences if not approved | Incomplete/ambiguous specifications: It is unclear as to the time-frequency region within which the maximum numbers of simultaneous receptions of SL PRS for a shared and dedicated SL PRS resource pool is defined. |

- TP #1:

< Unchanged text omitted >

Table 6.3-4: Sidelink "Reception Type" combinations

| Supported Combinations | Comment |
| --- | --- |
| A |  |
| B | Note 1, Note 2 |
| C | Note 1, Note 2 |
| E | Note 3 |
| M1× E | Note 4 |
| M× D | Note 2 |
| B+C | Note 1, Note 2 |
| Note 1: Corresponds to simultaneous reception within one sub-channel Note 2: Depending on the UE capability, the UE may be able to perform simultaneous sidelink communication receptions of the same sidelink “Reception Type” combinations across multiple SL carriers. Note 3: Applicable for a shared SL PRS resource pool. Corresponds to simultaneous reception within one sub-channel. Note 4: Applicable for a dedicated SL PRS resource pool with M_1≥1. Corresponds to simultaneous reception within one dedicated SL PRS resource pool. |  |

< Unchanged text omitted >

- TP #2:

< Unchanged text omitted >

Table 6.3-4: Sidelink "Reception Type" combinations

| Supported Combinations | Comment |
| --- | --- |
| A |  |
| B | Note 1, Note 2 |
| C | Note 1, Note 2 |
| E | Note 3 |
| M1×E | Note 4 |
| M× D | Note 2 |
| B+C | Note 1, Note 2 |
| Note 1: Corresponds to simultaneous reception within one sub-channel Note 2: Depending on the UE capability, the UE may be able to perform simultaneous sidelink communication receptions of the same sidelink “Reception Type” combinations across multiple SL carriers. Note 3: Applicable for a shared SL PRS resource pool within one subchannel. Note 4: Applicable for a dedicated SL PRS resource pool within one SL-PRS comb offset. |  |

< Unchanged text omitted >

Companies views

- RAN2 spreadsheet update [Intel]

Send an LS to RAN2 to inform them of the parameter sl-ThreshS-RSSI-PRS-CBR that needs to be introduced in TS 38.331 and is currently missing from the list of higher layer parameters in R1-2312708:

| Sub-feature group | RAN1 specification | Parameter name in the spec | New or existing? | Description | Value range | Per (UE, cell, TRP, …) | Required for initial access or IDLE/INACTIVE | Specification |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SL PRS configuration in a dedicated resource pool | 38.215 | sl-ThreshS- RSSI-PRS-CBR | New | Indicates the S-RSSI threshold for determining the contribution of a sub-channel to the SL PRS-CBR measurement in a dedicated SL PRS resource pool. Value 0 corresponds to -112 dBm, value 1 to -110 dBm, value n to (-112 + n*2) dBm, and so on. | INTEGER (0..45) | Per dedicated SL PRS resource pool | Yes | 38.331 |

[HIGH] Feature Lead Proposal 14-v0

Companies are encouraged to comment on the above Proposal

Companies views

- Proposal for Online Discussion

##### Feature Lead Proposal 13.2-v1

Support the following alignment TP for TS38.214:

---------------------------------------< Text Proposal 1 for 38.214> --------------------------------------

< Unchanged parts are omitted >

8.2.4.2 UE procedure for determining the subset of resources to be reported to higher layers in SL PRS resource selection in a dedicated SL PRS resource pool in sidelink resource allocation mode 2

In resource allocation mode 2 in a dedicated SL PRS resource pool, the higher layer can request the UE to determine a subset of resources from which the higher layer will select resources for SL PRS/PSCCH transmission. To trigger this procedure, in slot n, the higher layer provides the following parameters for this SL PRS/PSCCH transmission:

-	the resource pool from which the resources are to be reported;

-	L1 priority, prioTX;

-	the remaining [delay budget];

-	Set of SL-PRS resource ID(s);

-	optionally, the resource reservation interval, Prsvp_TX, in units of msec.

-	if the higher layer requests the UE to determine a subset of resources from which the higher layer will select resources for SL PRS/PSCCH transmission as part of re-evaluation or pre-emption procedure, the higher layer provides a set of resources (r0,r1,r2,…) which may be subject to re-evaluation and a set of resources (r0',r1',r2',…) which may be subject to pre-emption.

-	it is up to UE implementation to determine the subset of resources as requested by higher layers before or after the slot ri'' - T3, where ri'' is the slot with the smallest slot index among (r0,r1,r2,…) and (r0',r1',r2',…) , and T3 is equal to Tproc,1SL, where Tproc,1SL  is defined in slots in Table 8.1.4-2 where μSL is the SCS configuration of the SL BWP.

The following higher layer parameters affect this procedure:

-	[sl-SelectionWindowList: internal parameter T2min is set to the corresponding value from higher layer parameter sl-SelectionWindowList for the given value of prioTX.]

-	[sl-Thres-RSRP-List]: this higher layer parameter provides an RSRP threshold for each combination pi, pj, where pi is the value of the priority field in a received SCI format 1-B and pj is the priority of the transmission of the UE selecting resources; for a given invocation of this procedure, pj = prioTX.

-	sl-PRS-ResourceReservePeriodList:  the resource reservation interval, Prsvp_TX, is set to the corresponding value from higher layer parameter in units of mse

-	sl-SensingWindowDedicatedSL-PRS-RP: internal parameter T0 is defined as the number of slots corresponding to sl-SensingWindowDedicatedSL-PRS-RP msec

-	sl-TxPercentageDedicatedSL-PRS-RP-List: internal parameter X for a given prioTX is defined as sl-TxPercentageDedicatedSL-PRS-RP-List (prioTX) converted from percentage to ratio

-	[sl-PreemptionEnable]: if sl-PreemptionEnable is provided, and if it is not equal to 'enabled', internal parameter priopre is set to the higher layer provided parameter sl-PreemptionEnable.

The UE shall perform this procedure according to clause 8.1.4, with the following modifications:

-	"packet delay budget" is replaced by "SL PRS delay budget",

-	partial sensing is not applicable in a dedicated SL PRS resource pool,

-	"candidate single-slot resource" is replaced by "candidate SL PRS resource",

-	a candidate single-slot resource for transmission Rx,y is defined as the SL PRS resource with index x within the Set of SL-PRS resource ID(s) provided by the higher layer and in slot t'ySL,

-	"SCI format 1-A" is replaced by "SCI format 1-B",

-	in step 5, the second condition is modified as follows: for any periodicity value allowed by the higher layer parameter sl-PRS-ResourceReservePeriodList and any SL PRS resource ID in the set of SL PRS resource ID(s) provided by the higher layer, and a hypothetical SCI format 1-B received in slot t'mSL with 'Resource reservation period' field set to that periodicity value and indicating that SL-PRS resource ID, condition c in step 6 would be met,

-	In condition b of step 6, the RSRP measurement is the PSCCH-RSRP over the DM-RS resource elements of the PSSCH;

-	In condition c of step 6 "determines according to clause 8.1.5 the set of resource blocks and slots" is replaced by "determines according to clause 8.2.4.2A the set of SL PRS resources and slots ".

< Unchanged parts are omitted >

8.2.4.2A	UE procedure for determining slots and SL PRS resource(s) associated with an SCI format 1-B in a dedicated SL PRS resource pool

The set of slots and SL PRS resources for SL PRS transmission is determined by the PSCCH containing the associated SCI format 1-B, and fields 'SL-PRS resource ID (s)', 'Time resource assignment' of the associated SCI format 1-B as described below.

The set of slots is determined as in clause 8.1.5, with the following modifications:

-	"SCI format 1-A" is replaced by "SCI format 1-B",

-	[potential parameter name changes].

The first SL PRS resource is determined according to the sub-channel used for the PSCCH transmission containing the associated SCI format 1-B, where the index of the sub-channel in the resource pool is identical to the index of the SL PRS resource provided by sl-PRS-ResourceID.

The second SL-PRS and third SL PRS resource, if reserved by SCI format 1-B, are determined from " Resource ID indication" which is equal to a PRS Resource ID value (PRIV) where,

If sl-MaxNumPerReserveDedicatedSL-PRS-RP is 2 then

PRIV=r1

If sl-MaxNumPerReserveDedicatedSL-PRS-RP is 3 then

PRIV=r2*NSL-PRS+r1

Where

-	 r1 denotes the SL PRS resource ID for the second resource

-	 r2 denotes the SL PRS resource ID for the third resource

-	 NSL-PRS is the number of SL-PRS resources (pre-)configured in a slot of a resource pool.

If TRIV determined according to clause 8.1.5 indicates N < sl-MaxNumPerReserveDedicatedSL-PRS-RP, the SL PRS resource indices corresponding to sl-MaxNumPerReserveDedicatedSL-PRS-RP minus N last resources are not used.

---------------------------------------< END of Text Proposal 1 for 38.214> --------------------------------------

##### Feature Lead Proposal 13.3-v1

Support the following alignment TP for TS38.213:

---------------------------------------< Text Proposal 2 for 38.213> --------------------------------------

16.4A	UE procedure for transmitting PSCCH in dedicated SL PRS resource pool

For a dedicated SL PRS resource pool, a UE can be provided a number of symbols in the resource pool, by timeResourcePSCCH-DedicatedSL-PRS-RP, starting from a second symbol that is available for SL transmissions in a slot, and a number of PRBs in the resource pool, by freqResourcePSCCH-DedicatedSL-PRS-RP, starting from a PRB with lowest index for a sub-channel determined according to an index of an associated SL PRS resource, for a PSCCH transmission with a SCI format 1-B.

A UE that transmits a PSCCH with SCI format 1-B using SL PRS resource allocation mode 2 [6, TS 38.214] sets

-	"Source ID" as indicated by higher layers

-	"Destination ID" as indicated by high layers

-	"Cast type indicator" as indicated by higher layers

-	"Resource reservation period" as an index in sl-PRS-ResourceReservePeriodList corresponding to a reservation period provided by higher layers [11, TS 38.321], if the UE is provided sl-MultiReserveResource

-	the values of the time resource assignment field and of the resource ID indication field as described in [6, TS 38.214] to indicate N resources from a set Ry of resources selected by higher layers as described in [11, TS 38.321] with N smallest slot indices  yi for 0≤i≤N-1 such that y0<y1<…<yN-1≤y0+31, where:

-	N=minNselected, Nmax_reserve, where Nselected is a number of resources in the set Ry with slot indices  yj, 0≤j≤Nselected-1, such that y0<y1<…<yNselected-1≤y0+31, and Nmax_reserve is provided by sl-MaxNumPerReserveDedicatedSL-PRS-RP

-	each resource, from the set of Ry resources, corresponds to a SL PRS resource and the corresponding PSCCH, and a slot in a set of slots {t'ySL}

-	t'0SL,t'1SL,t'2SL,... is a set of slots in a sidelink resource pool [6, TS 38.214]

-	y0 is an index of a slot where the PSCCH with SCI format 1-B is transmitted.

-	"SL PRS request" as indicated by higher layers

A UE that transmits a PSCCH with SCI format 1-B using SL PRS resource allocation mode 1 [6, TS 38.214] sets

-	"Source ID" as indicated by higher layers

-	"Destination ID" as indicated by high layers

-	"Cast type indicator" as indicated by higher layers

-	the values of the resource ID indication field and the time resource assignment field for the SCI format 1-B transmitted in the m-th resource for SL PRS and the corresponding PSCCH transmission provided by a dynamic grant or by a SL configured grant, where m= 1,…,M and M is the total number of resources for SL PRS and the corresponding PSCCH transmission provided by a dynamic grant or the number of resources for SL PRS transmission in a period provided by a SL configured grant type 1 or SL configured grant type 2, as follows:

-	the resource ID indication field and time resource assignment field indicate the m-th to M-th resources as described in [6, TS 38.214].

-	"SL PRS request" as indicated by higher layers

For decoding of a SCI format 1-B, a UE may assume that a number of bits provided by sl-NumReservedBitsSCI1B-DedicatedSL-PRS-RP can have any value as described in [4, TS 38.212].

-----------------------------------< End of Text Proposal 2 for 38.213> -------------------------------

##### Feature Lead Proposal 14-v0

Please comment which of the following TPs you are supporting:

| Reason for change | For the characterization of simultaneous “Reception Type” combinations for sidelink, further qualification would be necessary to describe the scope within which the numbers of simultaneous “Reception Type” combinations apply for SL PRS reception. In particular, - For a shared SL PRS resource pool, the number of simultaneous SL PRS receptions should be defined within one sub-channel to align with SL communications (cf. Note 1 applicable for PSSCH and PSCCH). - For a dedicated SL PRS resource pool, the number of simultaneous SL PRS receptions should be defined within a dedicated SL PRS resource pool (analogous to a sub-channel for SL communications). |
| --- | --- |
| Summary of change | Clarify notes Note 3 and 4 in Table 6.3-4 |
| Consequences if not approved | Incomplete/ambiguous specifications: It is unclear as to the time-frequency region within which the maximum numbers of simultaneous receptions of SL PRS for a shared and dedicated SL PRS resource pool is defined. |

- TP #1:

< Unchanged text omitted >

Table 6.3-4: Sidelink "Reception Type" combinations

| Supported Combinations | Comment |
| --- | --- |
| A |  |
| B | Note 1, Note 2 |
| C | Note 1, Note 2 |
| E | Note 3 |
| M1× E | Note 4 |
| M× D | Note 2 |
| B+C | Note 1, Note 2 |
| Note 1: Corresponds to simultaneous reception within one sub-channel Note 2: Depending on the UE capability, the UE may be able to perform simultaneous sidelink communication receptions of the same sidelink “Reception Type” combinations across multiple SL carriers. Note 3: Applicable for a shared SL PRS resource pool. Corresponds to simultaneous reception within one sub-channel. Note 4: Applicable for a dedicated SL PRS resource pool with M_1≥1. Corresponds to simultaneous reception within one dedicated SL PRS resource pool. |  |

< Unchanged text omitted >

- TP #2:

< Unchanged text omitted >

Table 6.3-4: Sidelink "Reception Type" combinations

| Supported Combinations | Comment |
| --- | --- |
| A |  |
| B | Note 1, Note 2 |
| C | Note 1, Note 2 |
| E | Note 3 |
| M1×E | Note 4 |
| M× D | Note 2 |
| B+C | Note 1, Note 2 |
| Note 1: Corresponds to simultaneous reception within one sub-channel Note 2: Depending on the UE capability, the UE may be able to perform simultaneous sidelink communication receptions of the same sidelink “Reception Type” combinations across multiple SL carriers. Note 3: Applicable for a shared SL PRS resource pool within one subchannel. Note 4: Applicable for a dedicated SL PRS resource pool within one SL-PRS comb offset. |  |

< Unchanged text omitted >

##### Feature Lead Proposal 2-v2

In NR Rel-18, for a UE configured with SL CA,

- Alt. 1: SL PRS transmission/reception in a SL carrier is left up to UE implementation.

- Alt. 2: SL PRS transmission /reception is supported in a shared SL PRS resource pool in a single SL carrier.

- Alt. 3: SL PRS transmission/reception in a SL carrier is not supported in NR Rel.18.

- Alt 4: The following operations across SL carriers are supported:

  - Simulataneous SL PRS transmission

  - Simultanesous SL PRS reception

  - Simultaneous SL PRS and SL data transmission

  - Simultaneous SL PRS and SL data reception

  - Tx power control follows the rule defined for SL CA in NR Rel.18

- Proposal for Offline Discussion

# 18 References

# 19 Previous Agreements for SL Positioning Solutions

## RAN1 #109-e

Agreement

Study power control mechanisms for SL-PRS transmission, including whether it is necessary.

Agreement

With regards to the Positioning methods supported using SL measurements study further the following methods:

  - RTT-type solutions using SL

    - Study both single-sided (also known as one-way) and double-sided (also known as two-way) RTT

  - SL-AoA

    - Include both Azimuth of arrival (AoA) and zenith of arrival (ZoA) in the study

  - SL-TDOA

  - SL-AoD

    - Corresponds to a method where RSRP and/or RSRPP measurements similar to the DL-AoD method in Uu.

    - Include both Azimuth of departure (AoD) and zenith of departure (ZoD) in the study

- Consider in the study at least the following aspects:

  - Definition(s) of the corresponding SL measurements for each method

  - Which method is applicable to absolute or relative positioning or ranging, including whether such categorization is needed to be discussed.

  - For angle-based methods, antenna configuration consideration(s) using practical UE capabilities

  - Per-panel location, if UE uses multiple panels.

  - UE’s mobility, especially for V2X scenarios

  - Impact of synchronization error(s) between UEs

  - Existing SL measurements (e.g. RSSI, RSRP), and UE ID information etc, may be used.

- Note: The above categorization does not necessarily mean that there will be separate SL positioning methods specified, or whether there will be a unified SL Positioning method.

- Note: When the study of carrier phase positioning and the evaluations of sidelink positioning have progressed, it can be reviewed whether carrier phase for sidelink can be considered in further work. Checkpoint at RAN1#110-e-Bis to see if sufficient information is available for this review.

- Note: Companies are encouraged to describe the role of SL nodes and their interaction/coordination participating in each method.

Agreement

With regards to the numerologies of the SL-PRS, limit the study to those supported for NR Sidelink.

- Note 1: NR Sidelink supports {15, 30, 60 kHz} in FR1 and {60, 120 kHz} in FR2

- Note 2: This doesn’t imply that SL-PRS FR2-specific optimization(s) are expected to be studied

Agreement

Study new reference signal for SL positioning/ranging using the existing PRS/SRS design and SL design framework as a starting point.

- The study could at least include: Sequence design, frequency domain pattern, time domain pattern (e.g. number of symbols, repetitions, etc), time domain behavior, configuration/triggering/activation/de-activation of the SL-PRS, AGC time, Tx-Rx Turanround time, supportable bandwidth(s), multiplexing options with other SL channels, randomization/orthogonalization options.

- Note: The study of existing SL reference signal for SL positioning/ranging is not precluded. Companies are encouraged to perform performance evaluation/comparison to investigate whether such reference signals can meet the positioning accuracy requirements.

Agreement

With regards to the configuration/activation/deactivation/triggering of SL-PRS, study the following options:

- Option 1: High-layer-only signaling involvement in the SL-PRS configuration

  - No Lower layer involvement, e.g., SL-MAC-CE or SCI or DCI, for the activation or the triggering of a SL-PRS.

  - Based on the study, this option may correspond to

    - A SL-PRS configuration that is a single-shot or multiple shots

    - A high-layer configuration that may be received from an LMF, a gNB, or a UE

- Option 2: High-layer and lower-layer signaling involvement in the SL-PRS configuration

  - Lower-layer may correspond to SL-MAC-CE, or SCI, or DCI

  - For example, high layer signaling can may be used for SL-PRS configuration and lower layer signaling can may be used for initiating SL positioning and/or configuration/triggering/activating/deactivating/indicating and potential resource indication/reservation transmission of SL-PRS.

- Option 3: Only lower-layer signaling involvement in the SL-PRS configuration

  - Lower-layer may correspond to SL-MAC-CE, or SCI, or DCI

- Note 1: Include aspects in the study related to flexibility, overhead, latency, and reliability as/if needed.

Agreement

With regards to the Sidelink Positioning measurement report,

- Study the contents of the measurement report  (e.g. time stamp(s), quality metric(s), ID(s), angular/timing/power measurements, etc)

- Study the time domain behavior of the measurement report (e.g. one-shot, triggered, aperiodic, semi-persistent, periodic)

- FFS whether the Sidelink Positioning measurement can be a high-layer report and/or a lower layer report.

Agreement

For the purpose of RAN1 discussion during this study item, at least the following terminology is used:

- Target UE: UE to be positioned (in this context, using SL, i.e. PC5 interface).

- Sidelink positioning: Positioning UE using reference signals transmitted over SL, i.e., PC5 interface, to obtain absolute position, relative position, or ranging information.

- Ranging: determination of the distance and/or the direction between a UE and another entity, e.g., anchor UE.

- Sidelink positioning reference signal (SL PRS): reference signal transmitted over SL for positioning purposes.

- SL PRS (pre-)configuration: (pre-)configured parameters of SL PRS such as time-frequency resources (other parameters are not precluded) including its bandwidth and periodicity.

- Continue discussion on additional terminology clarification(s) such as: Initiator UE, Responder UE, Sidelink Positioning group, reference UE, etc, including whether such terminology is needed within RAN1 discussion.

Agreement

For the purpose of RAN1 discussion during this study item, at least the following terminology is used:

- Anchor UE: UE supporting positioning of target UE, e.g., by transmitting and/or receiving reference signals for positioning, providing positioning-related information, etc., over the SL interface.

  - FFS: clarification of the knowledge of the location of the anchor UE

Agreement

With regards to the frequency domain pattern, study further a Comb-N SL-PRS design. Study at least the following aspects:

- N>=1 (where N=1 corresponds to full RE mapping pattern)

- Fully staggered SL-PRS pattern (e.g., M symbols of SL-PRS with comb-N with M=N and, at each symbol a different RE offset is used), Partially staggered SL-PRS pattern (e.g., M symbol(s) of SL-PRS with comb-N, with M<N, at each symbol a different RE offset is used), Unstaggered SL-PRS patterns (e.g., M symbol(s) of SL-PRS with comb- N, at each symbol a same RE offset is used, N > 1)

- The number of symbols of SL-PRS within a slot

  - Any relation to the comb-N option

  - RE offset pattern repetitions within a slot

- FFS: Other frequency domain pattern(s)

Agreement

For a potential new SL PRS, study further the following

- Number of symbol(s) for AGC and/or Rx-Tx turnaround time

- Conditions under which AGC training and/or Rx-Tx turnaround time are needed

Agreement

With regards to the SL Positioning resource allocation, study further the following 2 options for SL Positioning resource (pre-)configuration:

- Option 1: Dedicated resource pool for SL-PRS

  - Include in the study at least the following aspects:

    - which slots can be used, SL frame structure, SL positioning slot structure, multiplexing of SL-PRS with control information (if included in the same slot)

    - positioning measurement report

    - whether a dedicated frequency allocation (e.g., layer/BWP) is needed for SL PRS

    - resource allocation procedure(s) of SL-PRS

    - This option may or may not include control information (i.e., configuration/activation/deactivation/triggering of SL-PRS) for the purpose of SL positioning operation

- Option 2: Shared resource pool with sidelink communication.

  - Include in the study at least the following aspects:

    - co-existence between SL communication and SL positioning, backward compatibility

    - Multiplexing considerations of SL-PRS with other PHY channels (PSCCH, PSSCH, PSFCH) and any modifications in the SL-slot structure

Agreement

With regards to the SL-PRS resource allocation, study the following two schemes:

- Scheme 1: Network-centric operation SL-PRS resource allocation (e.g. similar to a legacy Mode 1 solution)

  - The network (e.g. gNB, LMF, gNB & LMF) allocates resources for SL-PRS

- Scheme 2: UE autonomous SL-PRS resource allocation (e.g. similar to legacy Mode 2 solution)

  - At least one of the UE(s) participating in the sidelink positioning operation allocates resources for SL-PRS

  - Applicable regardless of the network coverage

- FFS: potential mechanisms, if needed, for SL-PRS resource coordination across a number of transmitting UEs (e.g. IUC-like solutions).

- Note: Other Schemes are not precluded to be studied

- FFS how to handle resource allocation of SL-Positioning measurement report

## RAN1 #110-e

Agreement

With regards to the Positioning methods supported using at least SL measurements, potential candidate positioning methods include at least the following:

  - RTT-type solution(s) using SL

  - SL-AoA

  - SL-TDOA

- Note: other methods can still be studied

- Note: The above categorization does not necessarily mean that there will be separate SL positioning methods specified.

Agreement

A new reference signal should be introduced for supporting SL positioning/ranging.

Agreement

Regarding SL-PRS resource allocation, both Scheme 1 and Scheme 2 should be introduced for supporting SL positioning/ranging:

- Scheme 1: Network-centric operation SL-PRS resource allocation (e.g. similar to a legacy Mode 1 solution)

  - The network (e.g. gNB, LMF, gNB & LMF) allocates resources for SL-PRS.

- Scheme 2: UE autonomous SL-PRS resource allocation (e.g. similar to legacy Mode 2 solution)

  - At least one of the UE(s) participating in the sidelink positioning operation allocates resources for SL-PRS

Agreement

With regards to the SL Positioning resource allocation, one of the following alternatives should be introduced for supporting SL positioning/ranging:

- Alt. 1: only dedicated resource pool(s) can be (pre-)configured for SL-PRS

- Alt. 2: either dedicated resource pool(s) and/or a shared resource pool(s) with sidelink communication can be (pre-)configured for SL-PRS

- Note: whether other signals/channels can be present in the dedicated resource pool can be further discussed

Agreement

For the content of the sidelink positioning measurement report, potential elements may include at least the following:

- One or more sidelink positioning measurement(s)

- Timestamp(s) associated with a sidelink positioning measurement

- Quality metric(s) associated with a sidelink positioning measurement

- Identification Information for a sidelink positioning measurement

- FFS any detail for the above

Agreement

For the sequence of the new reference signal for SL positioning/ranging, down select between Alt 1 and Alt 2:

- Alt. 1: pseudorandom-based. Use existing sequence of DL-PRS as a starting point.

- Alt. 2: ZC-based (SRS sequence as a starting point)

Agreement

With regards to the frequency domain pattern, a Comb-N SL-PRS occupying M symbol(s) design should be introduced for the support of NR SL positioning

- Note: there could be multiple values for M, N

Agreement

Regarding Scheme 2 SL-PRS resource allocation, study at least the following aspects:

- Resource selection mechanism for SL-PRS

- Inter-UE coordination

- Aspects for congestion control mechanisms for SL-PRS

Agreement

- With regards to the configuration/activation/deactivation/triggering of SL-PRS, Option 3 from the previous corresponding RAN1 #109 agreement will not be considered further.

- With regards to reservation of SL-PRS, it can be considered based on the Option 1 or Option 2 from the previous corresponding RAN1 #109 agreement.

Agreement

With regards to the frequency domain pattern for multi-symbol SL-PRS, prioritize partially and fully staggered SL-PRS.

- Note: this does not preclude comb N=1

- FFS: single symbol SL-PRS, if supported

## RAN1 #110-bis-e

Agreement

- With regards to the RTT-type solutions using SL, down-select between the following 2 alternatives:

  - Alt. 1: it corresponds to a single-sided RTT method

  - Alt. 2: it may correspond to either a single-sided or double-sided RTT method

    - With regards to the double-sided RTT,

      - companies are encouraged to analyze and evaluate the effect in performance for the single-sided SL RTT due to clock drift

      - Study the order of the SL-PRS transmissions for double-sided RTT

      - Study the impact of UE mobility

    - FFS study whether there is or what is the spec impact of double-sided RTT method

- Note: the above may correspond to RTT with one or multiple devices

Agreement

For the study of SL-AoA positioning method,

- Both SL Azimuth of arrival (AoA) and SL zenith of arrival (ZoA) measurement should be included

  - FFS: Definition of the measurements

- Study further whether other measurements can be applicable

- Study further the frame of reference (LCS or GCS)

Agreement

With regards to SL-TDOA positioning method for SL-only positioning,

- It corresponds to a method where SL-PRS is being transmitted from multiple anchor UEs to a target UE (i.e., DL-TDOA-like operation), and/or from a target UE to multiple anchor UEs (i.e. UL-TDOA-like operation) at least for the purpose of absolute positioning estimation of the target UE

- Study the detailed procedure, necessary signalling for SL-TDOA, method(s) to mitigate impact of synchronization error between multiple anchor UEs including whether such method(s) are needed.

Agreement

- From the potential candidate Positioning methods using at least SL measurements, at least the following should be introduced:

  - RTT-type solution(s) using SL

  - SL-AoA

  - SL-TDOA

- FFS: SL-AoD

Agreement

Regarding Scheme 1 SL-PRS resource allocation, a transmitting UE receives a SL-PRS resource allocation signaling from the network. Consider one or more of the following options:

- Opt. 1: through higher layers from the LMF

- Opt. 2: through Dynamic grant, or through configured grant type 1/type 2 from gNB

  - Up to further discussion which one or more of these shall be applicable

Agreement

For the sequence of the new reference signal for SL positioning/ranging, use

- Alt. 1: pseudorandom-based. Use existing sequence of DL-PRS as a starting point.

Agreement

From RAN1 perspective, the following cast types of SL-PRS transmission can be introduced for SL positioning: Unicast, Groupcast (not including many to one)

- Broadcast (as a working assumption).

- FFS: Applicability of the above cast types

Agreement

With regards to the frequency and time domain pattern of a SL-PRS resource within a slot has the following characteristics:

- With regards to the value N (comb size) and the number M of SL-PRS symbols within a slot excluding the symbol(s) used for AGC training / RxTx Turnaround:

  - At least the following values are considered as potential candidate values: N = {1,2,4,6,8,12}

  - FFS: the values considered as potential candidate values for M

  - FFS1: Whether to consider N>12 as a potential candidate value(s)

- The symbols of a SL-PRS resource within a slot are consecutive symbols

  - FFS: consecutive and/or non-consecutive symbols for shared resource pool (if supported)

- FFS: RE-Offset sequence within a SL-PRS resource, including whether to have in the end of the SL-PRS pattern a symbol with the same RE-offset as the first symbol, for phase-tracking purpose

Agreement

For a dedicated resource pool for SL positioning,

- With regards to which channels can be included in the resource pool in addition to SL-PRS, consider the following options:

  - Opt. 1: No other channel can be included beyond SL-PRS

  - Opt. 2: PSCCH which carries SCI associated with SL-PRS transmission(s) is included

  - Opt. 3: PSCCH which carries SCI associated with SL-PRS transmission(s) and PSSCH associated with SL-PRS transmission(s) are included

    - FFS: Details

    - FFS: definition of PSSCH associated with SL-PRS transmission(s)

- Note: Companies are encouraged to provide their analysis and views on the above

Agreement

With regards to the SL Positioning resource allocation, for SL Positioning resource (pre-)configuration in a shared resource pool with Rel-16/17/18 sidelink communication (if supported), backward compatibility with legacy Rel-16/17 UEs should be ensured.

Agreement

With regards to SL signaling of the reservation/indication of SL-PRS resource(s) for dedicated resource pool and shared resource pool (if supported) for positioning:

- Option A.1: SCI can be used for reserving/indicating one or more SL-PRS resource(s)

  - Note: This does NOT mean that only SCI is being used. There can still be higher layer signaling for the purpose of indicating a part of SL-PRS configuration.

  - FFS: Whether SCI is single stage SCI or two stage SCI

- FFS: SL-MAC-CE or other higher-layer signaling reservation/indication

Agreement

With regards to the Positioning methods supported using SL-PRS measurements

- at least the following measurements are considered:

  - SL Rx-Tx measurement

  - SL RSTD measurement

  - SL RSRP measurement

  - SL RSRPP measurement

  - SL RTOA measurement

  - SL Azimuth of arrival (AoA) and SL zenith of arrival (ZoA) measurement

- Companies are encouraged to study other measurements (e.g., time difference of arrival of 2 SL-PRS transmitted at 2 different times from the same anchor) and provide their analysis into why they are needed in light of the above measurements.

- Companies are encouraged to study potential enhancements, such as SL Rx-Tx measurement not being reported but the transmit time of SL-PRS being adjusted based on the measurement

- FFS any additional measurements

Agreement

At least for a dedicated resource pool for positioning,

- With regards to the bandwidth of SL-PRS transmission, downselect from the following alternatives:

  - Alt. 1: The bandwidth of SL-PRS can be same or smaller than that of the resource pool

  - Alt. 2: The bandwidth of SL-PRS shall be the same as that of the resource pool

- Note: Companies are encouraged to provide their analysis and views on the above alternatives

- FFS: Bandwidth of SL-PRS transmission for shared resource pool (if supported)

Agreement

Study further the granularity of time-domain resource allocation for SL-PRS transmission.

Agreement

With regards to the SL-PRS time domain behavior, at least study the following behaviors from Tx UE perspective:

- Periodic SL-PRS

  - SL-PRS is transmitted periodically with a transmission periodicity

  - FFS: any additional details, including whether or not higher layers can start/stop transmission.

- Semi-persistent SL-PRS

  - SL-PRS is transmitted periodically with a transmission periodicity after activation and until deactivation

  - FFS: any additional details

- Aperiodic SL-PRS

  - SL-PRS is transmitted at least once after [triggering/request]

    - Note: the brackets in the above means that companies are encouraged to study further whether “triggering” and/or “request” should be used and provide their definitions.

  - FFS: any additional details

- FFS: Applicability of the above time behaviors for scheme 1 & scheme 2

- FFS: Rx UE behavior is separately discussed.

- FFS: What mechanism(s) are used for activation/deactivation/triggering is part of the study

## RAN1 #111

Agreement

SL-AoD is included as a potential candidate positioning method, and

- SL-AoD should be deprioritized over the remaining methods that have been recommended to be introduced.

Agreement

With regards to the SL Positioning resource allocation, support

- Alt. 2: either dedicated resource pool(s) and/or a shared resource pool(s) with sidelink communication can be (pre-)configured for SL-PRS.

- Note: this does not imply that the design is the same for both types of resources pools

- Note: shared resources pool(s) should be supported with backward compatibility

Agreement

From RAN1 perspective, at least the following 2 operation scenarios are recommended for normative work:

- Operation Scenario 1: PC5-only-based positioning.

- Operation Scenario 2: Combination of Uu- and PC5-based positioning.

Agreement

For Scheme 2, with regards to Resource allocation mechanism for SL-PRS, pick one or both of the following options:

- Option 1: A sensing based resource allocation should be introduced

- Option 2: A random resource selection should be introduced

- In either option 1 or 2, the legacy designs for UE autonomous resource allocation should be used as a starting point. Study if/what enhancements may be needed.

Agreement

With regards to the RTT-type solutions using SL, both single-sided and double-sided RTT methods should be introduced

- Strive to minimize the changes needed on top of the specification support for single-sided RTT, if any, for the introduction of double-sided RTT.

- Note: a UE should be able to support single-sided RTT without having to support double-sided RTT

Agreement

Capture the following TP into the TR 38.859 as a conclusion:

For the solutions for sidelink positioning,

- The following 2 operation scenarios are recommended for normative work

  - Operation Scenario 1: PC5-only-based positioning.

  - Operation Scenario 2: Combination of Uu- and PC5-based positioning.

- RTT-type solution(s) using SL, SL-AoA and SL-TDOA are recommended for normative work.

  - both single-sided and double-sided RTT methods, striving to minimize the changes needed on top of the specification support for single-sided RTT, if any, for the introduction of double-sided RTT

- A new sidelink reference signal (SL-PRS) is recommended for normative work.

  - Such a reference signal should use a Comb frequency domain structure and a pseudorandom-based sequence where the existing sequence of DL-PRS should be used as a starting point.

  - SCI can be used for reserving/indicating one or more SL-PRS resources

- Both a resource allocation Scheme 1 and Scheme 2 is recommended for normative work, where Scheme 1 corresponds to a network-centric operation SL-PRS resource allocation and Scheme 2 corresponds to UE autonomous SL-PRS resource allocation.

- With regards to the SL-PRS transmission, both dedicated resource pool and shared resource pool with Rel-16/Rel-17/Rel-18 SL communication are recommended for normative work.

  - For SL Positioning resource (pre-)configuration in a shared resource pool with Rel-16/17/18 sidelink communication, backward compatibility with legacy Rel-16/17 UEs should be ensured.

- Unicast, Groupcast (not including many to one) and Broadcast of SL-PRS transmission are recommended for normative work.

Agreement

A dedicated SL-PRS resource pool is (pre-)configured in the only SL BWP of a carrier.

Agreement

With regards to the power control for SL-PRS at least Open Loop PC should be introduced.

Agreement

For SL-TDOA, DL-TDOA-like operation and UL-TDOA-like operation should be introduced.

- A UE is not required to support both DL-TDOA-like operation and UL-TDOA-like operation

Agreement

With regards to the Positioning methods supported using SL-PRS measurements

- at least the following measurements should be introduced:

  - SL-PRS based Rx-Tx measurement

  - SL-PRS based RSTD measurement

  - SL-PRS based RSRP measurement

  - SL-PRS based RSRPP measurement

  - SL-PRS based RTOA measurement

  - SL-PRS based Azimuth of arrival (AoA) and SL zenith of arrival (ZoA) measurement

Agreement

Update the agreed TP into the conclusion section of the TR 38.859 as follows:

For the solutions for sidelink positioning,

- The following 2 operation scenarios are recommended for normative work

  - Operation Scenario 1: PC5-only-based positioning.

  - Operation Scenario 2: Combination of Uu- and PC5-based positioning.

- RTT-type solution(s) using SL, SL-AoA and SL-TDOA are recommended for normative work.

  - both single-sided and double-sided RTT methods, striving to minimize the changes needed on top of the specification support for single-sided RTT, if any, for the introduction of double-sided RTT

  - For SL-TDOA, DL-TDOA-like operation and UL-TDOA-like operation is recommended for normative work.

  - For the support of the above methods the following measurements are recommended for normative work:

    - SL-PRS based Rx-Tx measurement

    - SL-PRS based RSTD measurement

    - SL-PRS based RSRP measurement

    - SL-PRS based RSRPP measurement

    - SL-PRS based RTOA measurement

    - SL-PRS based Azimuth of arrival (AoA) and SL zenith of arrival (ZoA) measurement

- A new sidelink reference signal (SL-PRS) is recommended for normative work.

  - Such a reference signal should use a Comb frequency domain structure and a pseudorandom-based sequence where the existing sequence of DL-PRS should be used as a starting point.

  - SCI can be used for reserving/indicating one or more SL-PRS resources

  - With regards to the power control for SL-PRS at least Open Loop PC is recommended for normative work.

- Both a resource allocation Scheme 1 and Scheme 2 is recommended for normative work, where Scheme 1 corresponds to a network-centric operation SL-PRS resource allocation and Scheme 2 corresponds to UE autonomous SL-PRS resource allocation.

  - For resource allocation mechanism for SL-PRS in Scheme 2, a sensing based resource allocation, or a random resource selection, or both, should be introduced, where the legacy designs for UE autonomous resource allocation are used as a starting point.

- With regards to the SL-PRS transmission, both dedicated resource pool and shared resource pool with Rel-16/Rel-17/Rel-18 SL communication are recommended for normative work.

  - For SL Positioning resource (pre-)configuration in a shared resource pool with Rel-16/17/18 sidelink communication, backward compatibility with legacy Rel-16/17 UEs should be ensured.

- Unicast, Groupcast (not including many to one) and Broadcast of SL-PRS transmission are recommended for normative work.

## RAN1 #112

- Agreement

- A UE can be configured to perform either resource allocation Scheme 1 or Scheme 2, applicable to all resource pools (dedicated or shared resource pools).

- SL PRS unicast/groupcast/broadcast can occur in either a shared or a dedicated resource pool.

Agreement

For a dedicated resource pool for positioning:

- The set of slots that belong to a resource pool is determined in the same way as for legacy SL communication pool (i.e. see section 8 of 38.214).

  - FFS: additional slots that can be used for SL PRS is not precluded

- Study what the dedicated resource pools for positioning (pre-)configuration should include, and consider at least the following: The start PRB position, the number of contiguous PRBs, SL-PRS configuration, synchronization configuration, resource allocation scheme 2 related configuration, power control configuration, sub-channel size and sub-channel count, time-domain bitmap, reporting configuration

- Agreement

For a dedicated resource pool for Positioning,

- With regards to which channels can be included in the resource pool in addition to SL-PRS, option 1 (No other channel can be included beyond SL-PRS) is NOT pursued further.

- Continue discussion between Option 2 and 3, and whether any other channel could also be included (e.g. PSFCH).

Agreement

Regarding Scheme 1 SL-PRS resource allocation, do not further consider a transmitting UE to receive the SL-PRS resource allocation through higher layers from the LMF (i.e. Option 1 is not pursued further).

Agreement

Sensing based or random selection in Scheme 2 is allowed by (pre-)configured per resource pool (similar to Rel-17 NR sidelink communication).

  - Working assumption: Sensing-based and random selection can be allowed in the same resource pool

  - FFS: whether any enhancements are needed for coexistence of random selection and sensing-based resource selection in a resource pool

- FFS: Details on the sensing-based resource selection and random selection, whether it will be similar to NR Rel-16 or NR Rel-17.

Agreement

With regards to random resource selection, reuse existing Rel-17 random selection mechanism from sidelink communications.

  - Study if any changes are needed

Agreement

In Scheme 2, with regards to the triggering of SL-PRS, support one or both of the following options:

- Option 1: Support SL-PRS transmission triggering at the physical layer by the UE’s own higher layers.

  - Note: this also includes higher layer triggering from another UE

- Option 2: Support UE-A to request UE-B to transmit SL-PRS via lower layer signaling sent by UE-A.

  - FFS: Whether lower-layer signaling is SCI or SL MAC-CE

Agreement

For SL-PRS transmission, at least support the following

- SL-PRS transmissions with periodic reservation: SL-PRS transmissions which are being reserved with a similar mechanism as the SL periodic resource reservation for another TB in legacy SL communication

  - FFS: whether/what changes are needed

- SL-PRS transmissions without periodic reservation: SL-PRS transmissions in which the SL-PRS is transmitted at least once without periodic reservation, with a similar mechanism as in legacy SL communication with SL resource without periodic reservation.

  - FFS: Maximum number of reservations and transmissions after triggering

Agreement

For the scheme 2 sensing-based resource allocation,

- Rel-16/17 resource (re)-selection procedure is reused for SL-PRS in the shared resource pool.

  - Study if/what changes are needed

- Rel-16[/17] resource (re)-selection procedure with periodic and without periodic reservations is the starting point for the design of SL-PRS in the dedicated resource pool.

  - Study what changes, if any, are needed at least with regards to the following: sensing window, resource selection window, reservation interval, Resource exclusion mechanism (e.g. definition of resource set for SL-PRS, how RSRP is measured, etc)

- From RAN1 perspective, priority value for SL PRS should be provided by higher layers from Tx UE perspective

## RAN1#112-bis

Agreement

For Scheme 1 SL-PRS resource allocation, a transmitting UE can receive a SL-PRS resource allocation signaling from gNB through a

- Dynamic grant

  - FFS Reuse DCI format 3_0 for signalling SL-PRS resource allocation or Support a new DCI format (3_X) and consider DCI format 3_0 as a starting point

- Configured grant type 1

  - the SL-PRS transmission(s) follows the higher layer configuration

- Configured grant type 2

  - Support activating and releasing the configured grant using a new DCI format 3_X or 3_0 (to be down-selected between the two DCI formats)

- The above mechanisms use NR Rel-16 mode-1 signaling as a starting point

- FFS: whether same/different DCI format(s) are applied for shared pool and dedicated pool.

- FFS: Further details

Agreement

For a dedicated resource pool for positioning:

- No additional slots are needed to be supported

Agreement

For SL-PRS transmission, either dedicated resource pool(s) or shared resource pool(s) or both can be (pre-)configured in the only SL BWP of a carrier.

- A UE can be (pre-)configured with one or more dedicated SL resource pools.

- A UE can be (pre-)configured with one or more shared SL resource pools.

Agreement

Confirm the working assumption: Sensing-based and random selection can be allowed in the same resource pool.

- Note: It is possible to (pre-)configure a resource pool to exclusively use sensing-based resource allocation.

Agreement

For Scheme 2 SL-PRS resource allocation, specify congestion control mechanisms using the existing congestion control mechanisms as a starting point.

- Study at least the following aspects on potential changes over the existing congestion control mechanisms:

  - CBR and CR definition for SL-PRS

  - Which parameters  of a SL-PRS configuration could be impacted by the congestion control mechanism, the mapping between congestion measurements, SL-PRS priority and SL-PRS parameters

  - CR and CBR measurement time window

  - Congestion control processing time

  - Number of CBR ranges

  - Whether any proposed changes could be applicable to shared resource pools in addition to the dedicated resource pool

Agreement

With regards to the SCI signaling in a shared resource pool, in addition to SL PRS transmission, the UE transmits

- Opt. 1: SCI1-A & a 2nd stage SCI format are used for SL-PRS indication

  - FFS: Details including a new or existing 2nd stage SCI

Agreement

In a shared resource pool:

- SL-PRS, associated PSCCH and PSSCH scheduled by the PSCCH are included in the same slot

  - With regards to PSSCH and SL-PRS multiplexing, downselect one of the following alternatives in RAN1#113 meeting:

    - Alt. A.1: Only TDMing is supported

    - Alt. A.2: Only FDMing of PSSCH and SL-PRS is supported

      - FFS: Rate-matched around SL-PRS REs and/or PRB/sub-channel-level FDMing are supported potentially for different cases

      - Note: Rate-matched around SL-PRS REs is not applicable to comb-1 SL-PRS

    - Alt. A.3: Both Alt. A.1 and A.2 are supported in the specification

  - With regards to PSCCH and SL-PRS multiplexing, downselect one of the following alternatives in RAN1#113 meeting:

    - Alt. B.1: Only TDMing is supported

    - Alt. B.2: TDMing or PRB/sub-channel-based FDMing is supported

  - The PSSCH is used for (downselect one of the following alternatives in RAN1#113 meeting):

    - Alt. C.1: 2nd SCI only

    - Alt. C.2: 2nd SCI and SL-SCH

    - Alt. C.3: “2nd SCI only” or “2nd SCI and SL-SCH”

  - FFS: Handling of PT-RS and SL-PRS

Agreement

For a dedicated resource pool for SL positioning, only a single stage SCI is used. PSCCH and associated SL-PRS are TDMed in the same slot.

- FFS: whether SL-PRS can be transmitted in a slot without associated PSCCH

Agreement

For the scheme 2 sensing-based resource allocation:

- Alt. 2: Rel-16 resource (re)-selection procedure with periodic and without periodic reservations is the starting point for the design of SL-PRS in the dedicated resource pool.

  - Note: This means that Rel-17 partial sensing is not considered a starting point for the design

Agreement

For Scheme 2, in a dedicated resource pool, using Rel-16 resource (re)-selection procedure as the starting point, consider at least the following potential modifications:

- Modification 1: For the RS used to derive L1 SL-RSRP for resource exclusion:

  - Option 1: SL-PRS

  - Option 2: PSCCH DMRS

  - Option 3: PSSCH DMRS (if PSSCH is included in the dedicated resource pool)

- Modification 2: For the resource selection window:

  - Option 1: for the derivation of the window, using the legacy approach as a starting point, substitute the Packet Delay Budget (PDB) with a new delay budget

  - Option 2: the selection window is provided by higher layers

- Modification 3: For the SL-PRS priority:

  - Option 1: A single L1 SL-PRS priority is allowed in a resource pool

  - Option 2: Multiple L1 SL-PRS priority are allowed  in a resource pool

- Modification 4: For the definition of a candidate resource within the resource selection window:

  - Options TBD

- Modification 5: For the reservation interval of SL-PRS:

  - Option 1: Provided by UE’s higher layers with values TBD. The set of values is (pre-)configured.

- Modification 6: For the sensing window length (T0):

  - Option 1: Use the legacy (pre-)configuration with values (100 msec, 1100 msec)

  - Option 2: Equal to or larger than the largest reservation interval

  - Option 3: Provided by higher layers with values TBD

- Modification 7: For the initial S-RSRP threshold & stepsize, target resource ratio X(%):

  - Options TBD

- Modification 8: For the pre-emption of the reserved resources:

  - Options TBD

- Note 1: Other potential modifications and/or other options within each modification are not precluded

- Note 2: Multiple options for each potential modification may be supported

Agreement

In Scheme 2, with regards to the triggering of SL-PRS,

- Support SL-PRS transmission triggering at the physical layer by the UE’s own higher layers

- Working assumption: Support UE-A to request UE-B to transmit SL-PRS via lower layer signaling sent by UE-A.

  - Up to UE-B’s own higher layers to transmit SL-PRS in response to the lower layer request from UE-A

  - FFS: Lower layer signaling corresponds to SCI, MAC-CE, or SL-PRS

## RAN1#113

Agreement

For a dedicated resource pool for SL positioning, SL-PRS cannot be transmitted in a slot without associated PSCCH.

Agreement

PSSCH is not included in dedicated resource pool for SL positioning.

Agreement

With regards to the SCI signaling in a shared resource pool,

- Support a new format for 2nd stage SCI.

  - FFS how to indicate the new 2nd stage SCI format

- FFS: If a 2nd stage SCI indicates both SL-PRS and SL-SCH, the cast type, destination ID, source ID are shared.

Agreement

In shared resource pools,

- With regards to PSCCH and SL-PRS multiplexing, support Alt. B.1. from previous agreement (i.e., Only TDMing is supported)

Agreement

In a shared resource pool, SL-PRS, associated PSCCH and PSSCH scheduled by the PSCCH are included in the same slot:

- With regards to PSSCH and SL-PRS multiplexing, only TDMing is supported for the already agreed comb sizes 1, 2, 4

Agreement

In a shared resource pool, SL-PRS, associated PSCCH and PSSCH scheduled by the PSCCH are included in the same slot:

- The PSSCH is used for 2nd SCI and SL-SCH

  - Note: the UE may not have data available for transmission. Up to RAN2 how to define the specification support for this case.

Agreement

For the shared resource pool, reuse the existing IUC signaling of both Scheme 1 and Scheme 2.

- SL-PRS transmissions are treated as any other legacy transmission for SL communication when considering IUC information exchanges.

Conclusion

For Rel-18 sidelink positioning:

- For the dedicated resource pool, IUC signalling is not supported

- Do not support that a UE can reserve a SL-PRS resource for the transmission of another UE

Conclusion

Do not support ACK/NACK feedback for SL-PRS or lower-layer feedback-based retransmissions in Release 18.

Agreement

PSFCH is not included in dedicated resource pool for SL positioning.

Agreement

In the dedicated resource pool,

- with regards to the SL-PRS time-domain resource allocation within the resource pool support a

  - SL-PRS-resource-based allocation

- SCI for SL-PRS should at least indicate the following values:

  - Source ID

  - Destination ID

  - Resource reservation period

  - SL-PRS Priority

  - Cast type

  - With regards to the SL-PRS configuration and/or SL-PRS time assignment information, select one alternative at RAN1#114:

    - Alt. 3.1: support a one-to-one mapping relationship between a PSCCH resource and an associated SL-PRS resource in the same slot.

      - Note: In this case, there is no need of an explicit signaling of which SL PRS resource for the same slot

      - Note: Same number of PSCCH resource(s) and SL-PRS resource(s)

    - Alt. 3.2: explicit signaling of SL PRS resource in the same slot

    - Alt. 3.3: support a mapping relationship between a PSCCH resource and one or more associated SL-PRS resource(s) in the same slot and explicit signaling of SL PRS resource

      - Only a one-to-one mapping is used between a PSCCH resource and an associated SL-PRS resource in the same slot if explicit signalling is not used

        - Note: with a one-to-one mapping, some SL-PRS resources might not be mapped

        - FFS: details, including (pre)configuration

    - FFS: Whether and how to indicate SCI resource(s) or SL-PRS resource (s) for a future slot

  - FFS: Additional information, e.g. SL-PRS request, Positioning Session ID, number of resource reservation periods

Agreement

In Scheme 2, with regards to the triggering of SL-PRS, confirm the related WA for shared and dedicated resource pools.

- With regards to the lower-layer signalling, support SCI associated with SL-PRS transmission

  - FFS: whether this is enabled by (pre)configuration

- FFS: to support also SL-PRS

Agreement

For Scheme 2, in a dedicated resource pool,

- Multiple L1 SL-PRS priority are allowed in a resource pool

- A SL PRS resource within the resource selection window is used as a candidate resource

- with regards the reservation interval of SL-PRS, it is provided by UE’s higher layers with values TBD. The set of values is (pre-)configured.

  - Use the periodicities available for legacy SL communication and the ones defined for DL-PRS as a starting point.

- with regards to the resource (re)-selection procedure

  - support re-evaluation & pre-emption for SL-PRS using the Rel-16 re-evaluation and pre-emption respectively as a starting point.

Agreement

In dynamic grant type resource allocation in scheme 1,

- For shared resource pool, DCI format 3_0 is being used as a starting point, down-select between the two alternatives below:

  - Alt. 1: Indication SL-PRS specific information is explicitly included in DCI

    - FFS: Which SL-PRS specific information

  - Alt. 2: Indication SL-PRS specific information is not explicitly included in DCI

- FFS: Dedicated resource pool

Agreement

In Scheme 2, congestion control can restrict the range of parameters for SL PRS configuration per resource pool by CBR and priority. Consider further the following parameter(s):

- Option 1: SL PRS transmission power

- Option 2: Periodicity of SL PRS

- Option 3: Number of occupied subchannels of SL-PRS (for shared resource pool)

- Option 4: Number of SL PRS resources in a slot

- Option 5: comb-size of a SL PRS resource in a slot

- Option 7: Number of OFDM symbols of a SL PRS resource in a slot

- Option 8: Number of SL PRS (re-)transmissions

- FFS: Other options are not precluded

Agreement

In a dedicated resource pool, with regards to the PSCCH, reuse the PSCCH channel structure of SL communications, at least with regards to the following aspects:

- The first PSCCH symbol is mapped to the 2nd symbol available for SL transmissions in a slot

  - Note: 1st symbol available for SL transmissions in a slot is for PSCCH AGC similar to legacy

- PSCCH DM-RS in the slot is being reused from legacy

- The number of PSCCH symbol(s) is (pre-)configured to (down-select at RAN1#114):

  - Alt. 1: 2 or 3 symbols (same as legacy)

  - Alt. 3: 1, or 2 or 3 symbols

- The number of PRBs is (pre-)configured using the legacy values

  - FFS: reconsider if 1-symbol PSCCH is supported

## RAN1#114

Agreement

For SL-PRS transmissions without periodic reservation, the maximum number of reservations signaled in an SCI is

- (pre-)configurable with a value of 2 or 3, which is similar with Rel-16 sidelink.

This is applicable to both shared and dedicated resource pool and both scheme 1 and scheme 2

Working assumption

In Scheme 2, with regards to the triggering of SL-PRS, for the SCI-based triggering, the SL-PRS request, in either SCI-1B or SCI-2D, is an explicit field

- If (pre-)configured per resource pool, then 1 bit is used, otherwise, it is 0 bits

Agreement

For dedicated resource pool, with regards to the SL-PRS configuration and/or SL-PRS time assignment information, support Alt. 3.1, i.e.

- support a one-to-one mapping relationship between a PSCCH resource and an associated SL-PRS resource in the same slot.

  - Note: In this case, there is no need of an explicit signaling of which SL PRS resource for the same slot

  - Note: Same number of PSCCH resource(s) and SL-PRS resource(s)

Agreement

For PSCCH configuration in a dedicated resource pool,

- (pre-)configure the number of PRBs of a PSCCH in the resource pool:

  - Alt. 1: One parameter for all PSCCHs

Agreement

For PSCCH configuration in a dedicated resource pool,

- The number of PSCCH symbol(s) is (pre-)configured to 2 or 3 symbols (same as legacy)

Agreement

In a shared resource pool, when PSSCH and SL-PRS are multiplexed in the same slot, they share the same source ID, destination ID, cast type fields.

Agreement

In a shared resource pool,

- To indicate the SCI format 2-D, the reserved state of the “2nd-stage SCI format” field (Codepoint “11”) in SCI format 1-A is used

Agreement

- For Scheme 2, in a dedicated resource pool, with regards to the sensing window length:

  - Use the legacy (pre-)configuration with values (100 msec, 1100 msec)

- For Scheme 2, in a dedicated resource pool, for the initial S-RSRP threshold & stepsize, target resource ratio X(%), reuse the legacy values from NR sidelink.

Agreement

For Scheme 2, in a dedicated resource pool, with regards to the resource (re)-selection procedure, the RS used to derive L1 SL-RSRP for resource exclusion is at least PSCCH DMRS.

- FFS: SL-PRS can be (pre)configured to derive L1 SL-RSRP for resource exclusion

Working assumption

In the shared resource pool, if SL PRS is multiplexed in slot, for the determination of a transmission of a TB, the UE shall determine the number of REs (NRE) within the slot as

NRE'=NscRBNsymbsh-NsymbPSFCH-NsymbSL-PRS-NohPRB-NREDMRS

where NsymbSL-PRS represents the number of OFDM symbols used for SL PRS in the slot.

The Tx UE should ensure the determined TB size unchanged across re-transmission(s) of the TB.

Agreement

In a shared resource pool,

- Aspect 4: In addition to the SL-PRS specific parameters, the following information related to PSSCH scheduling to indicate in the new second stage SCI 2-D, support at least the legacy content of SCI format 2-A and 2-B

  - FFS: to support the legacy content of SCI format 2-C

Working assumption

For Scheme 2, in a dedicated resource pool, using Rel-16 resource (re)-selection procedure as the starting point, support the following modification:

- Modification 2: For the resource selection window:

  - Option 1: for the derivation of the window, using the legacy approach as a starting point, substitute the Packet Delay Budget (PDB) with a Delay Budget for SL-PRS

Send an LS to RAN2 asking RAN2 whether they can confirm RAN1’s working assumption, and if not let RAN2 decide an alternative solution.

Agreement

Endorse the draft LS in R1-2308479 with the following modification to the action and adding SA2 in cc:

Change the Action Item as follows:

RAN1 respectfully asks RAN2 whether they can confirm RAN1’s working assumption, and if not, RAN1 requests RAN2 to decide an alternative solution and inform RAN1.

LS in R1-2308651 is endorsed.

Agreement

With regards to PSSCH and SL-PRS TDMed multiplexing, when determining the number of coded modulation symbols generated for 2nd-stage SCI transmission, symbols with SL-PRS are excluded when calculating {MscSCI2l, l=0,1,2⋯,NsymbolPSSCH-1},

- Alt. 1: based on a value (pre-)configured in the resource pool for this purpose (new parameter).

  - FFS: possible values (to be decided when discussing RRC parameters)

Agreement

From RAN1 perspective, for scheme 1 SL-PRS resource allocation for a UE requiring to transmit SL-PRS, the serving gNB may receive a request for specific SL PRS resource characteristic(s)/SL-PRS resource configuration(s).

- Up to other WGs to decide on the appropriate signaling and details on SL PRS characteristic(s) and/or SL-PRS configuration(s) request

Agreement

In a shared resource pool, with regards to the fields in SCI format 2-D, include the following fields:

- SL PRS resource information indication of the current slot – ceiling(log2(#SL-PRS resources (pre-)configured in the resource pool) bits)

- SL PRS request – 0 or 1 bit

- Embedded SCI format – [X] bit(s)

  - If the “Embedded SCI format” field is set to [0], the SCI 2-A fields are included with necessary padding

  - If the “Embedded SCI format” field is set to [1], the SCI 2-B fields are included

Agreement

For the PSCCH configuration in a dedicated resource pool,

- A PSCCH is mapped in a single subchannel similar to shared resource pool and:

  - the resource pool is (pre-)configured with the size of a subchannel in PRBs and the number of subchannels, and follow the legacy PSCCH mapping to resources of NR SL.

    - FFS: whether to add additional values for the subchannel (pre-)configuration

  - the PSCCH in the ith subchannel is associated with the ith SL-PRS resource ID

  - Note: if the number of subchannels is larger than the (pre-)configured number of SL PRS resources, then subchannels with index larger than or equal to the (pre-)configured number of SL PRS resources are not mapped to any resource

Agreement

For Scheme 2 SL-PRS resource allocation, with regards to the congestion control for a dedicated RP, the following modifications are supported:

- Modification 1: For the definition of SL PRS CR and CBR:

  - Alt. 2: redefine CBR/CR by considering the SL-PRS resource allocation/configuration.

Agreement

For Scheme 2 SL-PRS resource allocation, with regards to the congestion control for a dedicated RP, the following modifications are supported:

- Modification 2: For the evaluation of RSSI used in the CBR definition:

  - SL-RSSI is measured on a slot configured for transmission of PSCCH and SL-PRS

  - A single SL-RSSI is measured on symbols with both SL-PRS and PSCCH

Agreement

For Scheme 2 SL-PRS resource allocation, with regards to the congestion control for a dedicated RP, the following modifications are supported:

- For the CR and CBR measurement time window size,

  - it can be separately configured for a dedicated resource pool and could take the legacy values

Agreement

In Scheme 2,

- For a dedicated resource pool for positioning,

  - congestion control can restrict at least the following range of parameters for SL PRS configuration per resource pool by CBR and priority:

    - Maximum SL PRS transmission power

    - Maximum Number of SL PRS (re-)transmissions

    - Discuss further the following four SL PRS transmission parameters:

      - Minimum Periodicity of SL PRS

      - Maximum Number of SL PRS resources in a slot

      - Maximum comb-size of a SL PRS resource in a slot

      - Maximum Number of OFDM symbols of a SL PRS resource in a slot

  - For congestion control similar to legacy, the CR limits are (pre)-configured per priority in a resource pool

    - Note: Similar to SL communication how to achieve the CR limit is left to UE implementation.

- For a shared resource pool for positioning, the SL PRS can share the same restriction of PSSCH without specific enhancement in addition to what is already specified.

Agreement

In the dedicated resource pool for positioning, with regards to the SCI for SL-PRS, information carried in SCI for SL-PRS should at least include:

- Field 1: SL-PRS priority - 3 bits

- Field 2: Source ID – Up to resource pool (pre-)configuration 12 or 24 bits

- Field 3: Destination ID - 24 bits

- Field 4: Cast type – 2 bits

- Field 5: Resource reservation period - Ceil(log2(Number of candidate values in (pre-)configuration))

  - Alt. 5.1: Up to 16 values

- Field 6: Time resource assignment for SL-PRS future reservations

  - 1 or 2 max future slots within 32 slots – 5 bits or 9 bits, based on the maximum number of the (pre-)configured future reservations

- Field 7: SL-PRS resource ID (s) for the future 1 or 2 reservations

  - Number of bits:

    - In case of max number of future reservations is (pre-)configured to 2: [2*Ceil(log2(Number of SL-PRS resources in (pre-)configuration))]

    - In case of max number of future reservations is (pre-)configured to 1: Ceil(log2(Number of SL-PRS resources in (pre-)configuration))

- Field 8: SL-PRS request – 0 or 1 bit

- Field 9: Reserved bits – up to (pre-)configuration

Agreement

In Scheme 2, with regards to the congestion control for SL PRS:

- SL-PRS congestion processing time: based on both SCS and UE capability, similar to legacy

- The maximum number of CBR ranges for SL positioning is 8

- Number of CBR levels is 16

- CBR measurement for SL PRS can be reported to gNB

FFS: Whether it is needed to be reported to LMF or another UE

Agreement

For Scheme 2, in dedicated resource pools, with regards to the procedure for determining the subset of resources to be reported to higher layers, when triggering the resource (re-)selection procedure, the higher layers provide the following parameters for candidate SL-PRS transmission(s):

- resource pool from which to report SL-PRS resources

- Priority

- Delay budget

- Reservation period

- List of resources for pre-emption and re-evaluation

- Set of SL-PRS resource ID (s) which can include all (pre-)configured SL-PRS resource IDs

Agreement

For Scheme 2, in dedicated resource pools, with regards to the pre-emption,

- Pre-emption can be enabled/disabled by resource pool (pre)configuration using the same (pre-)configuration parameters as SL communications

- Reuse the legacy mechanism from SL communications with regards to SL-PRS priority-based comparison

  - The priority of SL-PRS to be transmitted is compared with the priority of SL-PRS reserved by other UEs.

- A resource for pre-emption checking is defined in terms of a sub-channel for PSCCH and the associated SL-PRS resource in a slot

Agreement

In resource allocation in scheme 1, for a dedicated resource pool

- in the DCI, introduce at least the following fields:

  - Resource pool index – number of bits same to SL communications

  - Time gap - 3 bits

  - SCI format 1-B fields:

    - Time resource assignment for SL-PRS future reservation(s)

    - SL-PRS resource ID (s) for the future 1 or 2 reservations

  - SL-PRS resource ID for the first SL-PRS transmission

  - Configuration index – number of bits same to SL communications

  - Padding bits, if required

- For configured grant type 1 resource allocation,

  - RRC is used for indicating at least the following:

    - Info-1: the periodicity,

    - Info-2: the slot offset relative to a logical slot defined by Info-3,

    - Info-3: SFN used for determination of the slot offset,

    - Info-4: Resource pool index

    - Info-5: Time resource assignment for SL-PRS future reservation(s)

    - Info-6: SL-PRS resource ID (s) for the future 1 or 2 reservations

    - Info-7: SL-PRS resource ID for the first SL-PRS transmission

- For configured grant type 2 resource allocation,

  - RRC is used for indicating at least the following:

    - Info 1: the periodicity

  - DCI is used for the activation/release of the configured grant resources

Conclusion

For Scheme 2 SL-PRS resource allocation, with regards to the congestion control for a shared RP, CBR and CR mechanisms from Rel.16 NR SL are reused.

- Add this agreement in the LS related to the priority handling

Agreement

Support the following for SL-PRS multiplexing/collision with the following channels:

- A SL-PRS resource and PSFCH (including the preceding gap symbol) are not mapped on the same symbols

Working assumption

The number of bits in the embedded SCI format field of SCI format 2-D is 2 bits

- If the “Embedded SCI format” field is set to 00, the SCI 2-A fields are included with necessary padding

- If the “Embedded SCI format” field is set to 01, the SCI 2-B fields are included

- If the “Embedded SCI format” field is set to 10, “size of SCI 2-B” number of reserved bits are included

- If the “Embedded SCI format” field is set to 11, “size of SCI 2-B” number of reserved bits are included

- Note: the size of SCI format 2-D is the same regardless of the value of the embedded SCI format field

Conclusion

In a shared resource pool, in the embedded SCI format of SCI format 2-D, there is no consensus to support the legacy content of SCI format 2-C.

Conclusion

For Scheme 2, in a dedicated resource pool, with regards to the resource (re)-selection procedure, there is no consensus to support to (pre-)configured SL-PRS to derive L1 SL-RSRP for resource exclusion.

## RAN1 #114-Bis

Agreement

In scheme 1, with regards to distinguishing between DCI format 3_0 and 3_2:

- New RNTIs, i.e., SL-PRS-RNTI & SL-PRS-CS-RNTI, are introduced.

- Support DCI size alignment between DCI format 3_0, 3_1 and 3_2.

Agreement

Sidelink PRS Received Signal Strength Indicator (SL PRS-RSSI) is defined as the linear average of the total received power (in [W]) observed in:

- the SL-PRS resource and the associated PSCCH in OFDM symbols of slots configured for PSCCH and in OFDM symbols of slots configured for SL-PRS.

  - Introduce larger values for congestion control processing time capability than legacy SL

Agreement

The working assumption is confirmed with the following revision with regards to the number of padding bits:

- the padding bits, if any, are such that the size of the SCI format 2-D is the same as if the larger of SCI format 2-A or 2-B is embedded

Working assumption The number of bits in the embedded SCI format field of SCI format 2-D is 2 bits - If the “Embedded SCI format” field is set to 00, the SCI 2-A fields are included with necessary padding - If the “Embedded SCI format” field is set to 01, the SCI 2-B fields are included - If the “Embedded SCI format” field is set to 10, “size of SCI 2-B” number of reserved bits are included - If the “Embedded SCI format” field is set to 11, “size of SCI 2-B” number of reserved bits are included - Note: the size of SCI format 2-D is the same regardless of the value of the embedded SCI format field

Agreement

In Scheme 2, with regards to the SCI-based triggering of SL-PRS, the following WA is confirmed:

Working assumption In Scheme 2, with regards to the triggering of SL-PRS, for the SCI-based triggering, the SL-PRS request, in either SCI-1B or SCI-2D, is an explicit field - If (pre-)configured per resource pool, then 1 bit is used, otherwise, it is 0 bits

Agreement

- Regarding the following text in brackets in Section 8.4.4 of TS38.214:

[ If the '[SL PRS request]' field in the SCI associated with the received SL PRS is set to 1 then the UE shall report this request for SL PRS transmission to higher layers.]

- Keep the text and remove the brackets with the change shown below:

  - '[SL PRS request]' field in the SCI associated with the received SL PRS is set to 1 then the UE shall report this request for SL PRS transmission is reported to higher layers.

Conclusion

In scheme 1, with regards to an explicit indication of SL-PRS specific information in DCI format 3_0:

- Indication of SL-PRS specific information is not explicitly included in DCI

Agreement

With regards to the bitwidth of the field “Resource ID indication” when the value of the higher layer parameter sl-MaxNumPerReserveSL-PRS is configured to 3:

- Ceil(2*log2(Number of SL-PRS resources in (pre-)configuration)) bits should be used

Further discuss at TP for the above at RAN1#114bis.

Conclusion

In a dedicated resource pool, with regards to the PSCCH, do not introduce additional values for the subchannel (pre-)configuration.

Agreement

The following TP is endorsed for clause 16.4A of TS 38.213:

- Reason for change: to provide information regarding the starting PRB of PSCCH.

- Summary of change: include the information that the PSCCH starts from the lowest PRB of the sub-channel determined according to the index of the associated SL PRS resource

- The consequence if not approved is: the UE will not be able to determine which resource to use for PSCCH transmission

---------------------------- Start of Text Proposal for TS 38.213 ----------------------------- < Unchanged parts are omitted > 16.4A UE procedure for transmitting PSCCH in dedicated resource pool for SL PRS For a resource pool dedicated for SL PRS transmissions, a UE can be provided a number of symbols in the resource pool, by sl-TimeResourcePSCCH, starting from a second symbol that is available for SL transmissions in a slot, and a number of PRBs in the resource pool, by sl-FreqResourcePSCCH, starting from the lowest PRB of the sub-channel determined according to the index of the associated SL PRS resource for a PSCCH transmission with a SCI format 1-B. A UE that transmits a PSCCH with SCI format 1-B using SL PRS resource allocation scheme 2 [6, TS 38.214] sets < Unchanged parts are omitted > ---------------------------- End of Text Proposal for TS 38.213 -----------------------------

Agreement

Confirm the working assumption related to the TB size determination from RAN1 #114, and endorse the following TP:

| Reason for change: | Corrections on TBS in a shared resource pool |
| --- | --- |
| Summary of change: | In clause 8.1.3.2 of TS 38.214, complement the value of NsymbSL-PRS under different conditions. |
| Consequences if not approved: | The TBS procedure in a shared resoruce pool is incomplete. |

----------------------------------------- Start of text proposal to TS 38.214 v18.0.0------------------------------------------- 8.1.3.2 Transport block size determination <<< UNCHANGED PARTS OMITTED >>> The UE shall first determine the number of REs (NRE) within the slot. - A UE first determines the number of REs allocated for PSSCH within a PRB (NRE') by NRE'=NscRBNsymbsh-NsymbPSFCH-NsymbSL-PRS-NohPRB-NREDMRS, where - NscRB=12 is the number of subcarriers in a physical resource block, - Nsymbsh = sl-LengthSymbols -2, where sl-LengthSymbols is the number of sidelink symbols within the slot provided by higher layers, - NsymbPSFCH = 3 if 'PSFCH overhead indication' field of SCI format 1-A indicates "1", and NsymbPSFCH = 0 otherwise, if higher layer parameter sl-PSFCH-Period is 2 or 4. If higher layer parameter sl-PSFCH-Period is 0, NsymbPSFCH=0. If higher layer parameter sl-PSFCH-Period is 1, NsymbPSFCH=3. - NsymbSL-PRS is the number of OFDM symbols used for SL PRS in the slot as indicated by the ‘SL PRS resource ID’ in SCI format 2-D if the 2nd-stage SCI is SCI format 2-D, and NsymbSL PRS = 0 otherwise， - NohPRB is the overhead given by higher layer parameter sl-X-Overhead, - NREDMRS is given by Table 8.1.3.2-1 according to higher layer parameter sl-PSSCH-DMRS-TimePatternList. <<< UNCHANGED PARTS OMITTED >>> ----------------------------------------- End of text proposal to TS 38.214 v18.0.0-------------------------------------------

Conclusion

For a dedicated resource pool, no more discussion on potential restriction by SL PRS-CBR and priority for the following SL PRS transmission parameters:

- Maximum Number of SL PRS resources in a slot

- Maximum comb-size of a SL PRS resource in a slot

- Maximum Number of OFDM symbols of a SL PRS resource in a slot

Agreement

With regards to the dedicated resource pool for positioning, suggest to the editors to align the terminology used as:

- “Dedicated SL PRS resource pool” defined in 38.214 as shown below:

  - A sidelink resource pool which can be used for transmission of SL PRS and cannot be used for transmission of PSSCH will be referred to as dedicated SL PRS resource pool.

Conclusion

From RAN1 perspective, there is no need to introduce an association between a dedicated resource pool for positioning and a shared resource pool, or between a dedicated resource pool for positioning and a sidelink communication resource pool.

Agreement

- With regards to the SL PRS Channel Occupancy Ratio (SL PRS CR):

  - Sidelink PRS Channel Occupancy Ratio (SL PRS CR) evaluated at slot n is defined as the total number of SL PRS resources in the dedicated SL PRS resource pool used for its transmissions in slots [n-a, n-1] and granted in slots [n, n+b] divided by the total number of configured SL PRS resources in the transmission pool over [n-a, n+b].

- With regards to the SL PRS Channel Busy Ratio (SL PRS CBR):

  - SL PRS Channel Busy Ratio (SL PRS CBR) measured in slot n is defined as the number of SL PRS resources in the dedicated SL PRS resource pool whose SL PRS RSSI measured by the UE exceed a (pre-)configured threshold sensed over a SL PRS-CBR measurement window [n-a, n-1], wherein a is equal to 100 or 100·2µ slots, according to [sl-TimeWindowSizeCBR-positioning] divided by the total number of the configured sidelink PRS resources in the transmission pool over [n-a, n-1].

Working assumption

Endorse the following TP for clause 8.2.4.2 of TS 38.214:

---------------------------- Start of Text Proposal for TS 38.214 ----------------------------- < Unchanged parts are omitted > 8.2.4.2 UE procedure for determining slots and SL PRS resource(s) associated with an SCI format 1-B in a dedicated resource pool The set of slots and SL PRS resources for SL PRS transmission is determined by the PSCCH containing the associated SCI format 1-B, and fields '[SL-PRS resource ID (s))', '[Time resource assignment]' of the associated SCI format 1-B as described below. The set of slots is determined as in clause 8.1.5, with the following modifications: - "SCI format 1-A" is replaced by "SCI format 1-B", - [ potential parameter name changes]. The first SL PRS resource is determined according to the sub-channel used for the PSCCH transmission containing the associated SCI format 1-B, where the index of the sub-channel in the resource pool is identical to the index of the SL PRS resource provided by [higher layer parameter]. The second SL-PRS and third SL PRS resource, if reserved by SCI format 1-B, are determined from " Resource ID indication" which is equal to a PRS Resource ID value (PRIV) where, If [sl-MaxNumPerReserve] is 2 then PRIV=r1 If [sl-MaxNumPerReserve] is 3 then PFRIV=r2*NSL-PRS+r1 where - r1 denotes the SL PRS resource ID for the second resource - r2 denotes the SL PRS resource ID for the third resource - NSL-PRS is the number of SL-PRS resources (pre-)configured in a slot of a resource pool. ---------------------------- End of Text Proposal for TS 38.214 ----------------------------- < Unchanged parts are omitted >

Agreement

For activation and deactivation of configured grant type 2 for SL PRS for DCI 3-2, use a dedicated field of size 1 bit.

Agreement

From RAN1 perspective, whether to support or not reporting of CBR measurements to LMF or another UE, is left up to other WGs.

Agreement

With regards to the shared resource pool for positioning, suggest to the editors to align the terminology used as:

- “shared SL PRS resource pool” defined in 38.214 as shown below:

A sidelink resource pool which can be used for transmission of both SL PRS and PSSCH will be referred to as shared SL PRS resource pool.

Agreement

Endorse the TP below for clause 8.5.2.3 of TS 38.214

| Reason for change: | Corrections on description associated with SCI format 2-D in a shared resource pool for the CSI reference resource definition |
| --- | --- |
| Summary of change: | In clause 8.5.2.3 of TS 38.214, SCI format 2-D is captured. |
| Consequences if not approved: | The description associated with SCI format 2-D in CSI reference resource definition is missing |

8.5.2.3 CSI reference resource definition <<< UNCHANGED PARTS OMITTED >>> If configured to report CQI index and RI index, in the CSI reference resource, the UE shall assume the following for the purpose of deriving the CQI index and RI index: - The reference resource uses the CP length and subcarrier spacing configured for the SL BWP. - Redundancy Version 0. - PSCCH occupies 2 OFDM symbols. - The number of PSSCH and DM-RS symbols is equal to sl-LengthSymbols‒2. - Assume no REs allocated for sidelink CSI-RS. - Assume no REs allocated SCI format 2-A, SCI format 2-B, or SCI format 2-C or SCI format 2-D. <<< UNCHANGED PARTS OMITTED >>> ----------------------------------------- End of text proposal to TS 38.214 v18.0.0-------------------------------------------

Agreement

Step 5 for the resource selection procedure in Section 8.2.4.2 of 38.214 is modified as follows:

- In step 5, the second condition is modified as follows: for any periodicity value allowed by the higher layer parameter sl-ResourceReservePeriodList and any SL PRS resource ID in the set of SL PRS resource ID(s) provided by the higher layer, and a hypothetical SCI format 1-B received in slot t'mSL with 'Resource reservation period' field set to that periodicity value and indicating that SL-PRS resource ID, condition c in step 6 would be met.

Agreement

Confirm the following Working Assumption made in RAN1#114bis:

Working assumption Endorse the following TP for clause 8.2.4.2 of TS 38.214: ---------------------------- Start of Text Proposal for TS 38.214 ----------------------------- < Unchanged parts are omitted > 8.2.4.2 UE procedure for determining slots and SL PRS resource(s) associated with an SCI format 1-B in a dedicated resource pool The set of slots and SL PRS resources for SL PRS transmission is determined by the PSCCH containing the associated SCI format 1-B, and fields '[SL-PRS resource ID (s))', '[Time resource assignment]' of the associated SCI format 1-B as described below. The set of slots is determined as in clause 8.1.5, with the following modifications: - "SCI format 1-A" is replaced by "SCI format 1-B", - [ potential parameter name changes]. The first SL PRS resource is determined according to the sub-channel used for the PSCCH transmission containing the associated SCI format 1-B, where the index of the sub-channel in the resource pool is identical to the index of the SL PRS resource provided by [higher layer parameter]. The second SL-PRS and third SL PRS resource, if reserved by SCI format 1-B, are determined from " Resource ID indication" which is equal to a PRS Resource ID value (PRIV) where, If [sl-MaxNumPerReserve] is 2 then PRIV=r1 If [sl-MaxNumPerReserve] is 3 then PFRIV=r2*NSL-PRS+r1 where - r1 denotes the SL PRS resource ID for the second resource - r2 denotes the SL PRS resource ID for the third resource - NSL-PRS is the number of SL-PRS resources (pre-)configured in a slot of a resource pool. ---------------------------- End of Text Proposal for TS 38.214 ----------------------------- < Unchanged parts are omitted >.

## RAN1 #115

Agreement

For DCI format 3-2, the resource pool index “I” should be an index over

- the number of dedicated SL PRS resource pools (pre-)configured to the UE

Conclusion:

For sidelink resource allocation scheme 1 (i.e. mode 1 in the specs), dynamic grant, configured grant type 1, and configured grant type 2 are supported for both dedicated and shared SL PRS resource pools.

Agreement

Support the following TP for 38.214 clause 8.2.4.1:

In sidelink resource allocation mode 1: - For SL PRS transmission, a UE may be configured with dynamic grant, configured grant type 1, and [/]or configured grant type 2 are supported

Agreement

- Use SL PRS delay budget instead of packet delay budget in SL PRS resource selection in a dedicated SL PRS resource pool in sidelink resource allocation mode 2.

  - Agree the below text proposal on Clause 8.2.4.2 of TS 38.214.

Text Proposal 1 for TS 38.214 clause 8.2.4.2 <omitted text> The UE shall perform this procedure according to clause 8.1.4, with the following modifications: - “packet delay budget” is replaced by “SL PRS delay budget” <omitted text>

Conclusion

For a dedicated resource pool, the periodicity of SL PRS cannot be restricted by congestion control.

Agreement

The TP below is endorsed

| Reason for change: | Correction on step 6 of SL-PRS resource allocation |
| --- | --- |
| Summary of change: | In clause 8.2.4.2, add modification on step 6 regarding the SL-PRS resource and slot determination based on 8.2.4.2A. |
| Consequences if not approved: | The determination of resources applied for SL-PRS resource exclusion is not clear. |

*** Unchanged parts are omitted *** #### 8.2.4.2 UE procedure for determining the subset of resources to be reported to higher layers in SL PRS resource selection in a dedicated SL PRS resource pool in sidelink resource allocation mode 2 In resource allocation mode 2 in a dedicated SL PRS resource pool, the higher layer can request the UE to determine a subset of resources from which the higher layer will select resources for SL PRS/PSCCH transmission. To trigger this procedure, in slot n, the higher layer provides the following parameters for this SL PRS/PSCCH transmission: *** Unchanged parts are omitted *** The UE shall perform this procedure according to clause 8.1.4, with the following modifications: - Partial sensing is not applicable in a dedicated SL PRS resource pool; - A candidate single-slot resource for transmission is defined as the SL PRS resource with index within the Set of SL-PRS resource ID(s) provided by the higher layer and in slot - "SCI format 1-A” is replaced by “SCI format 1-B", - In step 5, the second condition is modified as follows: for any periodicity value allowed by the higher layer parameter reservationPeriodAllowed-Dedicated-SL-PRS-RP and any SL PRS resource ID in the set of SL PRS resource ID(s) provided by the higher layer, and a hypothetical SCI format 1-B received in slot with 'Resource reservation period' field set to that periodicity value and indicating that SL-PRS resource ID, condition c in step 6 would be met. - In condition c of step 6 “determines according to clause 8.1.5 the set of resource blocks and slots” is replaced by “determines according to clause 8.2.4.2A the set of SL PRS resources and slots”. *** Unchanged parts are omitted ***

Agreement

With regards to the UE SL PRS preparation procedure time, the TP below is endorsed

- Note to the editor of TS 38.214: it is up to the editor whether to create a new section or add this text to an existing section as appropriate

In sidelink resource allocation mode 1 for a dedicated SL PRS resource pool, the UE shall perform this procedure according to clause 8.6 (excluding the case of PSSCH for retransmission of a transport block), with the following modifications: - "PSSCH for a transport block" is replaced by "SL PRS" - "PSSCH" is replaced by "SL PRS"

Agreement

The TPs below related to the description of SCI format 2-D are endorsed

- In clause 8.1.3/8.2.1/8.3/8.5.1.2/8.5.2.2/8.5.2.3 of TS 38.214, SCI format 2-D is captured as shown below:

-------------------------- Start of text proposal to TS 38.214 v18.0.0 with draft CR R1-2310764------------------------- ### 8.1.3 Modulation order, target code rate, redundancy version and transport block size determination The redundancy version is given by the "Redundancy version" field in SCI format 2-A, 2-B, or 2-C or 2-D. <<< UNCHANGED PARTS OMITTED >>> ### 8.2.1 CSI-RS transmission procedure A UE transmits sidelink CSI-RS within a unicast PSSCH transmission if the following conditions hold: - CSI reporting is enabled by higher layer parameter sl-CSI-Acquisition; and - the 'CSI request' field in the corresponding SCI format 2-A, or 2-C or 2-D is set to 1. <<< UNCHANGED PARTS OMITTED >>> ### 8.3 UE procedure for receiving the physical sidelink shared channel For sidelink resource allocation mode 1, a UE upon detection of SCI format 1-A on PSCCH can decode PSSCH according to the detected SCI formats 2-A, 2-B, and 2-C and 2-D, and associated PSSCH resource configuration configured by higher layers. The UE is not required to decode more than one PSCCH at each PSCCH resource candidate. For sidelink resource allocation mode 2, a UE upon detection of SCI format 1-A on PSCCH can decode PSSCH according to the detected SCI formats 2-A, 2-B, and 2-C and 2-D, and associated PSSCH resource configuration configured by higher layers. The UE is not required to decode more than one PSCCH at each PSCCH resource candidate. A UE is required to decode neither the corresponding SCI formats 2-A, 2-B, and 2-C and 2-D nor the PSSCH associated with an SCI format 1-A if the SCI format 1-A indicates an MCS table that the UE does not support. <<< UNCHANGED PARTS OMITTED >>> ### 8.5.1.2 Triggering of sidelink CSI reports The CSI-triggering UE is not allowed to trigger another aperiodic CSI report for the same UE before the last slot of the expected reception or completion of the ongoing aperiodic CSI report associated with the SCI format 2-A ,or 2-C or 2-D with the 'CSI request' field set to 1, where the last slot of the expected reception of the ongoing aperiodic CSI report is given by [10, TS38.321]. An aperiodic CSI report is triggered by an SCI format 2-A, or 2-C or 2-D with the 'CSI request' field set to 1. <<< UNCHANGED PARTS OMITTED >>> ### 8.5.2.2 Reference signal (CSI-RS) The UE can be configured with one CSI-RS pattern as indicated by the higher layer parameters sl-CSI-RS-FreqAllocation, sl-CSI-RS-FirstSymbol in SL-CSI-RS-Config. Parameters for which the UE shall assume non-zero transmission power for CSI-RS are configured according to clause 8.2.1. A UE is not expected to be configured such that a CSI-RS and the corresponding PSCCH can be mapped to the same resource element. A UE is not expected to receive sidelink CSI-RS and PSSCH DM-RS, nor CSI-RS and 2nd-stage SCI, on the same symbol. Sidelink CSI-RS shall be transmitted according to [4, TS 38.211] in the resource blocks used for the PSSCH associated with the SCI format 2-A, or 2-C or 2-D triggering a report. <<< UNCHANGED PARTS OMITTED >>> -------------------------- End of text proposal to TS 38.214 v18.0.0 with draft CR R1-2310764-------------------------

Agreement

The following TP for TS 38.214 Clause 8.1 is endorsed

| Reasons for change | The description of UE setting ‘Embedded SCI format’ field of SCI format 2-D is not correct. |
| --- | --- |
| Summary of change | Change the description of UE setting ‘Embedded SCI format’ field of SCI format 2-D. |
| Consequences if not approved | The specification is not aligned with the agreement. |
| Text proposal | The UE shall set the contents of the SCI format 2-D as follows: - the UE shall set value of the '[SL PRS resource ID]' field as indicated by higher layers. - the UE shall set value of the '[SL PRS request]' field as indicated by higher layers. - the UE shall set value of the '[Embedded SCI format]' field as indicated by higher layers. - if 'Embedded SCI format' indicates that SCI format 2-A is embedded within this SCI format 2-D then the UE shall include in the '[Embedded SCI format payload]' field the fields of SCI format 2-A, set as specified above. - if 'Embedded SCI format' indicates that SCI format 2-B is embedded within this SCI format 2-D then the UE shall include in the '[Embedded SCI format payload]' field the fields of SCI format 2-B, set as specified above. |

R1-2312472	Moderator Summary #1 on resource allocation for SL PRS	Moderator (Qualcomm)

Conclusion

With regards to the SL PRS (re)transmission(s):

- RAN1 assumes that higher layers may provide to PHY layer more than one SL-PRS resource(s), which are used for the (re-)transmission of multiple SL-PRS(s) on different slots to the same target UE(s)

  - It is up to RAN2 to specify a mechanism for selection of multiple resources for SL-PRS

Conclusion

“Maximum Number of SL PRS (re-)transmissions” parameter is applicable to SL-PRS resource (re)-selection.

Agreement

Modify the description of current specification associated with definition of SL PRS-CBR and adopt TP #4 for TS38.215.

### TP #4 Reason for change:; The current definition on SL PRS-CBR in clause 5.1.49 misses the RRC parameter name for the SL PRS RSSI measurement threshold which is [sl-ThreshS-PRS-RSSI-CBR].. Summary of change:; Add the RRC parameter name for the SL PRS RSSI measurement threshold.. Consequences if not approved:; Unclear which RRC parameter is referred by the current specification.. -------------------------- Start of text proposal to TS 38.215 v18.0.0 with draft CR R1-2310743------------------------- 5.1.49 Sidelink PRS channel busy ratio (SL PRS-CBR) Definition; SL PRS Channel Busy Ratio (SL PRS-CBR) measured in slot n is defined as the number of SL PRS resources in the dedicated SL PRS resource pool whose SL PRS RSSI measured by the UE exceed a (pre-)configured threshold provided by the higher layer parameter [sl-ThreshS-PRS-RSSI-CBR] sensed over a SL PRS-CBR measurement window [n-a, n-1], wherein a is equal to 100 or 100·2µ slots, according to higher layer parameter [sl-TimeWindowSize-PRS-CBR-positioning] divided by the total number of the configured SL PRS resources in the transmission pool over [n-a,n-1]. The calculation of SL PRS-CBR is limited within the slots for which the SL PRS-RSSI is measured. If the number of SL PRS-RSSI measurement slots within the SL PRS-CBR measurement window is below a (pre-)configured threshold, a (pre-)configured SL PRS-CBR value is used.. Applicable for; RRC_IDLE intra-frequency, RRC_IDLE inter-frequency, RRC_INACTIVE intra-frequency, RRC_INACTIVE inter-frequency, RRC_CONNECTED intra-frequency, RRC_CONNECTED inter-frequency. NOTE 1: The slot index is based on physical slot index. --- unchanged text omitted --- -------------------------- End of text proposal to TS 38.215 v18.0.0 with draft CR R1-2310743-------------------------

Conclusion

For a dedicated SL-PRS resource pool, for comparing priority between SL-PRS and UL, support two threshold parameters, similar to legacy, which are used for comparing SL-PRS priority versus the threshold,

- if the priority value of SL-PRS is lower than the threshold, SL-PRS has higher priority;

- otherwise, UL has higher priority.

Note: No RAN1 specification change is expected from the above.

Agreement

The TP below is endorsed for TS38.214

Reason for change:; Corrections on description associated with resource allocation in a dedicated SL PRS resource pool.. Summary of change:; In clause 8.2.4.1.1 and 8.2.4.2 of TS 38.214, minimum resource allocation unit of SL PRS is captured.. Consequences if not approved:; The description associated with resource allocation in a dedicated SL PRS resource pool is inaccurate.. -------------------------- Start of text proposal to TS 38.214 v18.0.0 with draft CR R1-2310764------------------------- 8.2.4.1.1 Resource allocation in time domain The UE shall transmit the SL PRS in the same slot as the associated PSCCH. For a dedicated SL PRS resource pool, the minimum resource allocation unit in the time domain is a SL PRS resource in a slot. The UE shall transmit the SL PRS in consecutive symbols within the slot. A UE does not transmit multiple SL PRS resources in the same slot. <<< UNCHANGED PARTS OMITTED >>> 8.2.4.2 UE procedure for determining the subset of resources to be reported to higher layers in SL PRS resource selection in a dedicated resource pool in sidelink resource allocation mode 2 <<< UNCHANGED PARTS OMITTED >>> The UE shall perform this procedure according to clause 8.1.4, with the following modifications: - Partial sensing is not applicable in a dedicated SL PRS resource pool; - ‘Candidate single-slot resource’ is replaced by ’candidate SL PRS resource’. - A candidate single SL PRS resource for transmission is defined as the SL PRS resource with index within the Set of SL-PRS resource ID(s) provided by the higher layer and in slot - "SCI format 1-A” is replaced by “SCI format 1-B", - In step 5, the second condition is modified as follows: for any periodicity value allowed by the higher layer parameter reservationPeriodAllowed-Dedicated-SL-PRS-RP and any SL PRS resource ID in the set of SL PRS resource ID(s) provided by the higher layer, and a hypothetical SCI format 1-B received in slot with 'Resource reservation period' field set to that periodicity value and indicating that SL-PRS resource ID, condition c in step 6 would be met. - In condition b of step 6, the RSRP measurement is the PSCCH-RSRP over the DM-RS resource elements of the PSSCH; - In condition c of step 6 "determines according to clause 8.1.5 the set of resource blocks and slots" is replaced by "determines according to clause 8.2.4.2A the set of slots and SL PRS resources". <<< UNCHANGED PARTS OMITTED >>> -------------------------- End of text proposal to TS 38.214 v18.0.0 with draft CR R1-2310764-------------------------

Agreement

Send an LS to RAN2 and RAN3 with the following:

- From RAN1 perspective, for scheme 1, it is important for the following request to be specified:

  - a gNB is able to receive a request from either LMF or UE for SL-PRS bandwidth

- Action to RAN2 and RAN3 to consider how to specify support for such request, if not already specified.

R1-2312629	Draft LS on the request for specific SL PRS resource characteristic(s)/SL-PRS resource configuration	Qualcomm Incorporated

R1-2312630	LS on the request for specific SL PRS resource characteristic(s)/SL-PRS resource configuration	RAN1, Qualcomm Incorporated

Agreement

The draft LS in R1-2312629 is endorsed, with clarification that it goes to RAN WG2 and RAN WG3. Final LS is agreed in R1-2312630.

Agreement

The total number of SL configured grants (including both Type1 and Type2) at a UE across all resource pools is not larger than 8.

Agreement

For support of IUC in shared SL PRS resource pool, value 1 of parameter sl-TriggerConditionRequest means the explicit request can be triggered only when UE-B has data or SL PRS to be transmitted to UE-A.

- Including this into the higher parameter list.