sa3GPP TSG RAN WG1 #116		                                                R1-2401809

Athens, Greece, February 26th – March 1st, 2024

Source:	Moderator (Xiaomi)

Title:	Summary of discussion on maximum number of PDCCH candidates determination for CSS

Agenda Item:	7.1

Document for:	Discussion and Decision

- Introduction

This document is made for issues on default beam for cross-carrier scheduling raised by the following contributions.

R1-2401181	Correction on maximum number of PDCCH candidates determination for CSS	Beijing Xiaomi Software Tech

To be moderated by Qin (Xiaomi).

- Contact information

Please consider entering contact info below for the convenience of email contact and F2F discussions.

| Company | Name | Email |
| --- | --- | --- |
| Xiaomi | Qin MU | muqin@xiaomi.com |
| Samsung | Kyungjun Choi | kyungj.choi@samsung.com |
| Nokia | Klaus Hugl | Klaus.hugl@nokia.com |
| vivo | Zichao Ji | jizichao@vivo.com |
| ZTE | Xianghui Han | han.xianghui@zte.com.cn |
| MTK | James (CH) Hsieh | CH.Hsieh@mediatek.com |
| CATT | Min Zhu | zhumin@catt.cn |

- Background: Potential problem in current specification

In the PDCCH assignment equation of  TS38.213, the parameter of Ms,maxL  for CSS equals to Ms(L)  of nCI=0 as describled in the following specification copied from TS 38.213.

## 10.1 UE procedure for determining physical downlink control channel assignment … For a search space set associated with CORESET , the CCE indexes for aggregation level corresponding to PDCCH candidate of the search space set in slot for an active DL BWP of a serving cell corresponding to carrier indicator field value are given by … , where is the number of PDCCH candidates the UE is configured to monitor for aggregation level of a search space set for a serving cell corresponding to ; for any CSS, ;

As for the determination of Ms,0L ,  the following two cases can be considered:

Case 1: For some CSS (e.g., type 0/0A/2),  Ms,0L is given by Table10-1.1 of TS 38.213. The following is the related specification description and related conclusion reached in RAN1#96 meeting. According to the moderator’s view, how to determine Ms,0L is clear for this case.

10.1 UE procedure for determining physical downlink control channel assignment … For a DL BWP, if a UE is not provided searchSpaceSIB1 for Type0-PDCCH CSS set by PDCCH-ConfigCommon, the UE does not monitor PDCCH candidates for a Type0-PDCCH CSS set on the DL BWP. The Type0-PDCCH CSS set is defined by the CCE aggregation levels and the number of PDCCH candidates per CCE aggregation level given in Table 10.1-1. … For a DL BWP, if a UE is not provided searchSpaceOtherSystemInformation for Type0A-PDCCH CSS set, the UE does not monitor PDCCH for Type0A-PDCCH CSS set on the DL BWP. The CCE aggregation levels and the number of PDCCH candidates per CCE aggregation level for Type0A-PDCCH CSS set are given in Table 10.1-1. … If a UE is not provided pagingSearchSpace for Type2-PDCCH CSS set, the UE does not monitor PDCCH for Type2-PDCCH CSS set on the DL BWP. The CCE aggregation levels and the number of PDCCH candidates per CCE aggregation level for Type2-PDCCH CSS set are given in Table 10.1-1. Table 10.1-1: CCE aggregation levels and maximum number of PDCCH candidates per CCE aggregation level for CSS sets configured by searchSpaceSIB1 CCE Aggregation Level; Number of Candidates. 4; 4. 8; 2. 16; 1.

RAN1#96 meeting conclusion Done: According to email conclusion posted on March 11th, common understanding in RAN1 is as follows: - For a Type0/0A/2-CSS set, in R15, regardless of whether the CSS set is the SS#0 or not, the UE determines CCE aggregation levels and max number of candidates per CCE aggregation level as specified in Table 10.1-1 of TS38.213, which are applicable to all RNTIs monitored in the CSS set. - It means if a Type0/0A/2-CSS set is not the SS#0, gNB may configure parameter nrofCandidates in TYPE0/0A/2 SS set to any value allowed by ASN.1, and R15 UE ignores the configured values by parameter nrofCandidates and assumes values specified in Table 10.1-1 of TS38.213 instead for the SS set. - No spec change is necessary for the above.

Case 2: For Type 3 CSS (e.g., CSS for SFI), how to determine Ms,0L  is not clear. And clarification is needed.

According to the specification in TS 38.213, when the search space is configured by SearchSpace , the Ms(L) is configured by aggregationLevel1 …aggregationLevel16,   respectively in  SearchSpace information element

10.1 UE procedure for determining physical downlink control channel assignment For each DL BWP configured to a UE in a serving cell, the UE is provided by higher layers with search space sets where, for each search space set from the search space sets, the UE is provided the following by SearchSpace: - a search space set index , , by searchSpaceId - an association between the search space set and a CORESET by controlResourceSetId - a PDCCH monitoring periodicity of slots and a PDCCH monitoring offset of slots, by monitoringSlotPeriodicityAndOffset - a PDCCH monitoring pattern within a slot, indicating first symbol(s) of the CORESET within a slot for PDCCH monitoring, by monitoringSymbolsWithinSlot - a duration of slots indicating a number of slots that the search space set exists by duration - a number of PDCCH candidates per CCE aggregation level by aggregationLevel1, aggregationLevel2, aggregationLevel4, aggregationLevel8, and aggregationLevel16, for CCE aggregation level 1, CCE aggregation level 2, CCE aggregation level 4, CCE aggregation level 8, and CCE aggregation level 16, respectively

However,  according to the search space configuration message defined in TS 38.331, there is possiblility that multiple DCI formats can be included in the same CSS. For example DCI format 0-0 ,1-0 and DCI format 2-0 can be configured in the same search space. In this case, both the IE of nrofCandidates and the IE of nrofCandidates-SFI contain the parameters of aggregationLevel1, aggregationLevel2 …and aggregationLevel16 as indicated in the following configuration message. In this case, there are more than one sets of aggregationLevel1,  aggregationLevel2, …and aggregationLevel16.  Which set of aggregationLevel1,  aggregationLevel2, …and aggregationLevel16 should be applied for the PDCCH assignment equation in TS 38.213 is not clearly described in current specification.

#### – SearchSpace … SearchSpace information element -- ASN1START -- TAG-SEARCHSPACE-START SearchSpace ::= SEQUENCE { searchSpaceId SearchSpaceId, controlResourceSetId ControlResourceSetId OPTIONAL, -- Cond SetupOnly monitoringSlotPeriodicityAndOffset CHOICE { sl1 NULL, sl2 INTEGER (0..1), sl4 INTEGER (0..3), sl5 INTEGER (0..4), sl8 INTEGER (0..7), sl10 INTEGER (0..9), sl16 INTEGER (0..15), sl20 INTEGER (0..19), sl40 INTEGER (0..39), sl80 INTEGER (0..79), sl160 INTEGER (0..159), sl320 INTEGER (0..319), sl640 INTEGER (0..639), sl1280 INTEGER (0..1279), sl2560 INTEGER (0..2559) } OPTIONAL, -- Cond Setup4 duration INTEGER (2..2559) OPTIONAL, -- Need S monitoringSymbolsWithinSlot BIT STRING (SIZE (14)) OPTIONAL, -- Cond Setup nrofCandidates SEQUENCE { aggregationLevel1 ENUMERATED {n0, n1, n2, n3, n4, n5, n6, n8}, aggregationLevel2 ENUMERATED {n0, n1, n2, n3, n4, n5, n6, n8}, aggregationLevel4 ENUMERATED {n0, n1, n2, n3, n4, n5, n6, n8}, aggregationLevel8 ENUMERATED {n0, n1, n2, n3, n4, n5, n6, n8}, aggregationLevel16 ENUMERATED {n0, n1, n2, n3, n4, n5, n6, n8} } OPTIONAL, -- Cond Setup searchSpaceType CHOICE { common SEQUENCE { dci-Format0-0-AndFormat1-0 SEQUENCE { ... } OPTIONAL, -- Need R dci-Format2-0 SEQUENCE { nrofCandidates-SFI SEQUENCE { aggregationLevel1 ENUMERATED {n1, n2} OPTIONAL, -- Need R aggregationLevel2 ENUMERATED {n1, n2} OPTIONAL, -- Need R aggregationLevel4 ENUMERATED {n1, n2} OPTIONAL, -- Need R aggregationLevel8 ENUMERATED {n1, n2} OPTIONAL, -- Need R aggregationLevel16 ENUMERATED {n1, n2} OPTIONAL -- Need R }, ... nrofCandidates-SFI The number of PDCCH candidates specifically for format 2-0 for the configured aggregation level. If an aggregation level is absent, the UE does not search for any candidates with that aggregation level. The network configures only one aggregationLevel and the corresponding number of candidates (see TS 38.213 [13], clause 11.1.1).. nrofCandidates Number of PDCCH candidates per aggregation level. The number of candidates and aggregation levels configured here applies to all formats unless a particular value is specified or a format-specific value is provided (see inside searchSpaceType). If configured in the SearchSpace of a cross carrier scheduled cell, this field determines the number of candidates and aggregation levels to be used on the linked scheduling cell (see TS 38.213 [13], clause 10)..

- Discussion

  - 1st round discussion

Question 1: If the aggregation level and associated number of PDCCH candidates for a CSS (e.g., type 3 CSS for SFI) are configured by  SearchSpace (i.e., Case 2 mentioned in the background), which set of parameters is used to determine Ms,0L in the PDCCH assignment equation

- Option 1:  Ms,0L is given by Table 10.1-1 in TS 38.213

- Option 2:  Ms,0L  per CCE aggregation level L is given by aggregationLevel1, aggregationLevel2, aggregationLevel4, aggregationLevel8, and aggregationLevel16 in nrofCandidates

- Option 3: Ms,0L  per CCE aggregation level L is given by aggregationLevel1, aggregationLevel2, aggregationLevel4, aggregationLevel8, and aggregationLevel16 in nrofCandidates-SFI

- Option 4: Others. For companies choosing option 4,  please indicate how to determine  Ms,0L in your understanding

| Company | Comments |
| --- | --- |
| Samsung | Option 2. |
| Xiaomi | Option 2 |
| Nokia | Option 2 |
| vivo | If this question is specifically asking for the type-3 CSS, our understanding is option-2. |
| Spreadtrum | Option 2 |
| ZTE | Option 2 for type-3 CSS. |
| MTK | Option 2 |
| CATT | Option 2 |

Question 2: Any action is needed to clarify how to determine Ms,0L for CSS ?

- Option 1: Current specification is clear enough and no action is needed. (For companies choosing this option, please provide related specification part which you think clearly describe how to determine Ms,0L)

- Option 2-1: Yes,   action is needed. A clarification conclusion in chairman notes is sufficient. E.g., It is common understanding that if the aggregation level and associated number of candidates are configured by SearchSpace, Ms,0L is given by aggregationLevel1, aggregationLevel2, aggregationLevel4, aggregationLevel8, and aggregationLevel16 in nrofCandidates

- Option 2-2: Yes, action is needed. CR to update the current specification is necessary.

| Company | Comments |
| --- | --- |
| Samsung | Option 1. No action is needed for Case 2. The text from TS38.213 clearly mentioned that nrofCandidates-SFI indicates the number of PDCCH candidates among nrofCandidates PDCCH candidates configured for a search space set. And the PDCCH hashing function is determined per search space set, not DCI format, so that nrofCandidates is used. Section 10.1 For each DL BWP configured to a UE in a serving cell, the UE is provided by higher layers with search space sets where, for each search space set from the search space sets, the UE is provided the following by SearchSpace: - … - a number of PDCCH candidates per CCE aggregation level by aggregationLevel1, aggregationLevel2, aggregationLevel4, aggregationLevel8, and aggregationLevel16, for CCE aggregation level 1, CCE aggregation level 2, CCE aggregation level 4, CCE aggregation level 8, and CCE aggregation level 16, respectively - an indication that search space set is either a CSS set or a USS set by searchSpaceType - if search space set is a CSS set - an indication by dci-Format0-0-AndFormat1-0 to monitor PDCCH candidates for DCI format 0_0 and DCI format 1_0 - an indication by dci-Format2-0 to monitor one or two PDCCH candidates for DCI format 2_0 and a corresponding CCE aggregation level - … Section 11.1.1 The Mp,sLSFI PDCCH candidates are the first Mp,sLSFI PDCCH candidates for CCE aggregation level LSFI for search space set s in CORESET p. Mod： It seems the highlighted part for DCI format 2-0 only describe how to assign PDCCH candidates for DCI-format2-0. The conclusion that nrofCandidates is used for the Hash function calculation is the deduction based on this part. |
| Xiaomi | We think at least option 2-1 is needed. For the specification cited by Samsung, there is no words clearly saying that the maximum number of PDCCH candidates for CSS in case 2 is given by nrofCandidates. The conclusion that nrofCandidates is used for Hash function for CSS is like a deduction based on the specification description. |
| Nokia | We agree with Samsung that no change is needed (Option 1). On another note: we are focusing only on SFI specific handling, but there is similar separate configuration also for ‘CI’ and ‘IAB’ (in R16) as well. |
| Vivo | Our view is no additional action is needed (i.e., option 1). Please note that the formulation of Hash unction in 38.213 is defined for a Search Space, not for a DCI format. For a search space set associated with CORESET , the CCE indexes for aggregation level corresponding to PDCCH candidate of the search space set in slot for an active DL BWP of a serving cell corresponding to carrier indicator field value are given by And, for each Search Space, the PDCCH candidate for each AL is provided by corresponding RRC parameter below. While for a specific DCI format 2-0, there is additional parameter to configure the number of candidate (one or two), which is only for that DCI format. For each DL BWP configured to a UE in a serving cell, the UE is provided by higher layers with search space sets where, for each search space set from the search space sets, the UE is provided the following by SearchSpace: - a search space set index , , by searchSpaceId - an association between the search space set and a CORESET by controlResourceSetId - a PDCCH monitoring periodicity of slots and a PDCCH monitoring offset of slots, by monitoringSlotPeriodicityAndOffset - a PDCCH monitoring pattern within a slot, indicating first symbol(s) of the CORESET within a slot for PDCCH monitoring, by monitoringSymbolsWithinSlot - a duration of slots indicating a number of slots that the search space set exists by duration - a number of PDCCH candidates per CCE aggregation level by aggregationLevel1, aggregationLevel2, aggregationLevel4, aggregationLevel8, and aggregationLevel16, for CCE aggregation level 1, CCE aggregation level 2, CCE aggregation level 4, CCE aggregation level 8, and CCE aggregation level 16, respectively - an indication that search space set is either a CSS set or a USS set by searchSpaceType - if search space set is a CSS set - an indication by dci-Format0-0-AndFormat1-0 to monitor PDCCH candidates for DCI format 0_0 and DCI format 1_0 - an indication by dci-Format2-0 to monitor one or two PDCCH candidates for DCI format 2_0 and a corresponding CCE aggregation level - an indication by dci-Format2-1 to monitor PDCCH candidates for DCI format 2_1 - an indication by dci-Format2-2 to monitor PDCCH candidates for DCI format 2_2 - an indication by dci-Format2-3 to monitor PDCCH candidates for DCI format 2_3 This is also reflected in the RRC description: nrofCandidates-SFI The number of PDCCH candidates specifically for format 2-0 for the configured aggregation level Thus, the spec is already clear. We don’t see the confusing part. Mod：It seems the highlighted part for DCI format 2-0 only describe how to assign PDCCH candidates for DCI-format2-0. The conclusion that nrofCandidates is used for the Hash function calculation is the deduction based on this part. |
| Spreadtrum | Option 1. Agree with Samsung and vivo. |
| ZTE | Agree with other companies that the spec is clear. |
| MTK | We tend to think current spec is clear, but open to discuss Option 2-1. |
| CATT | We are open to both Option 1 and Option 2-1. |

  - 2nd  round discussion

According to the first round discussion, all companies have the common understanding that for type 3 common search space, the parameter of  Ms,0L used for hash function calculation is given by nrofCandidates.

As for whether further action is needed, some companies kindly provide the related specification description. But there is no words explicitly and clearly demonstrates that nrofCandidates is used for Hash function calculation for CSS. The conclusion that nrofCandidates  is used for the Hash function for CSS is kind of deduction based on the specification description. In addition, some companies also support for further clarification. Considering this situation, the moderate suggest capturing our common understanding in the chairman notes.

Proposed conclusion in chairman notes for clarification:

- For type 3 CSS, it is common understanding that the maximum number of PDCCH candidates for PDCCH assignment equation included in Clause 10.1 of TS 38.213 is given by nrofCandidates

Please provide your view on the proposed conclusion.

| Company | Comments |
| --- | --- |

- Conclusion

According to the comment received, there is no consensus on the need for the CR draft in R1-2401181.