3GPP TSG-RAN WG1 Meeting #116	R1-2401906

# Athens, Greece, February 26th – March 1st, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.212 | CR | 0186 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on NR positioning enhancement in 38.212 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-03-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Capture the agreed TPs achieved in RAN1#116 for NR positioning. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the specification to reflect the following: - replace the placeholders with the correct referred clauses - align the RRC parameters |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification for Rel-18 NR positioning enhancement is incomplete or incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.1.2, 8.4.1.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.213, TS 38.214 |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 8.3.1.2 SCI format 1-B

SCI format 1-B is used for the scheduling of SL PRS for a dedicated SL PRS resource pool.

The following information is transmitted by means of the SCI format 1-B:

-	Priority - 3 bits as specified in clause 5.7 of [12, TS 23.586] and clause 5.22 of [8, TS 38.321]. Value '000' of Priority field corresponds to priority value '1', value '001' of Priority field corresponds to priority value '2', and so on.

-	Source ID – 12 or 24 bits determined by higher layer parameter sl-SRC-ID-LenDedicatedSL-PRS-RP, as defined in clause 16.4A of [5, TS 38.213].

-	Destination ID – 24 bits as defined in clause 16.4A of [5, TS 38.213].

-	Cast type indicator – 2 bits as defined in Table 8.3.1.2-1 and in clause 16.4A of [5, TS 38.213].

-	Resource reservation period – log2Nrsv_period bits as defined in clause 16.4A of [5, TS 38.213], where Nrsv_period is the number of entries in the higher layer parameter sl-PRS-ResourceReservePeriodList, if higher layer parameter sl-PRS-ResourceReservePeriodList is configured; 0 bit otherwise.

-	Time resource assignment – 5 bits when the value of the higher layer parameter sl-MaxNumPerReserveDedicatedSL-PRS-RP is configured to 2; otherwise 9 bits when the value of the higher layer parameter sl-MaxNumPerReserveDedicatedSL-PRS-RP is configured to 3, as defined in clause 8.2.4.2A of [6, TS 38.214].

-	Resource ID indication – log2NSL-PRS bits when the value of the higher layer parameter sl-MaxNumPerReserveDedicatedSL-PRS-RP is configured to 2; otherwise 2log2NSL-PRS bits when the value of the higher layer parameter sl-MaxNumPerReserveDedicatedSL-PRS-RP is configured to 3. The value NSL-PRS is the total number of SL PRS resources within a slot in a dedicated SL PRS resource pool and provided by the higher layer parameter sl-PRS-ResourcesDedicatedSL-PRS-RP.

-	SL PRS request – 1 bit as defined in clause 8.4.4 of [6, TS 38.214] when the higher layer parameter sl-SCI-basedSL-PRS-TxTriggerSCI1-B is provided; 0 bit otherwise.

-	Reserved - Nreserved bits as configured by higher layer parameter sl-NumReservedBitsSCI1B-DedicatedSL-PRS-RP, with value set to zero.

< Unchanged parts are omitted >

#### 8.4.1.4	SCI format 2-D

SCI format 2-D is used for the decoding of PSSCH and the scheduling of SL PRS for a shared SL PRS resource pool.

The following information is transmitted by means of the SCI format 2-D:

-	SL PRS resource ID - log2NSL-PRS bits, where the value NSL-PRS is the total number of SL PRS resource IDs within a slot in a shared SL PRS resource pool and provided by the higher layer parameter sl-PRS-ResourcesSharedSL-PRS-RP.

-	SL PRS request – 1 bit as defined in clause 8.4.4 of [6, TS 38.214] when the higher layer parameter sl-SCI-based-SL-PRS-Tx-Trigger-SCI2-D is provided; 0 bit otherwise.

-	Embedded SCI format - 2 bits. This field indicates the embedded SCI format as defined in Table 8.4.1.4-1.

-	Embedded SCI format payload - number of bits determined according to Table 8.4.1.4-1. This field is set to the associated payload of the embedded SCI format indicated by the ‘Embedded SCI format’ field as defined in Table 8.4.1.4-1.

< Unchanged parts are omitted >