3GPP TSG-WG Meeting #116	R1-2401909

# Athens, Greece, February 25 - March 01, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.215 | CR | 0064 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of Rel-18 positioning enhancements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh2-Core |  |  |  |  |  | Date: |  |  | March 04, 2024 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 1. In the definition for SL PRS-CBR, the current parameter name is not reflective of the quantity represented. A more appropriate choice would be sl-ThreshS-RSSI-PRS-CBR since “S-RSSI”, corresponds to SL RSSI in the intended name. 2. The current parameter name sl-TimeWindowSize-PRS-CR is not aligned with the parameter name suggested to RAN2, i.e., sl-TimeWindowSizeCR-Dedicated-SL-PRS-RP. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Section 5.1.49: Update the parameter name sl-ThreshS-PRS-RSSI-CBR to sl-ThreshS-RSSI-PRS-CBR. 2. Section 5.1.48: Update the parameter name sl-TimeWindowSize-PRS-CR to sl-TimeWindowSizeCR-Dedicated-SL-PRS-RP. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect parameter naming. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.48, 5.1.49 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

--- unchanged text omitted ---

### 5.1.48	Sidelink PRS channel occupancy ratio (SL PRS-CR)

| Definition | Sidelink PRS Channel Occupancy Ratio (SL PRS-CR) evaluated at slot n is defined as the total number of SL PRS resource in the dedicated SL PRS resource pool used for its transmissions in slots [n-a, n-1] and granted in slots [n, n+b] divided by the total number of configured SL PRS resources in the transmission pool over [n-a, n+b]. |
| --- | --- |
| Applicable for | sidelink |

NOTE 1:	a is a positive integer and b is 0 or a positive integer; a and b are determined by UE implementation with a+b+1 = 1000 or 1000·2µ slots, according to higher layer parameter sl-TimeWindowSizeCR-Dedicated-SL-PRS-RP, b < (a+b+1)/2, and n+b shall not exceed the last transmission opportunity of the grant for the current transmission.

NOTE 2:	SL PRS-CR is evaluated for each (re)transmission.

NOTE 3:	In evaluating SL PRS-CR, the UE shall assume the transmission parameter used at slot n is reused according to the existing grant(s) in slot [n+1, n+b] without dropping.

NOTE 4:	The slot index is based on physical slot index.

NOTE 5:	SL PRS-CR can be computed per priority level

NOTE 6:	A resource is considered granted if it is a member of a selected sidelink grant as defined in TS 38.321 [7].

### 5.1.49	Sidelink PRS channel busy ratio (SL PRS-CBR)

| Definition | SL PRS Channel Busy Ratio (SL PRS-CBR) measured in slot n is defined as the number of SL PRS resources in the dedicated SL PRS resource pool whose SL PRS RSSI measured by the UE exceed a (pre-)configured threshold provided by the higher layer parameter sl-ThreshS-RSSI-PRS-CBR sensed over a SL PRS-CBR measurement window [n-a, n-1], wherein a is equal to 100 or 100·2µ slots, according to higher layer parameter [sl-TimeWindowSize-PRS-CBR-positioning] divided by the total number of the configured SL PRS resources in the transmission pool over [n-a,n-1]. The calculation of SL PRS-CBR is limited within the slots for which the SL PRS-RSSI is measured. If the number of SL PRS-RSSI measurement slots within the SL PRS-CBR measurement window is below a (pre-)configured threshold, a (pre-)configured SL PRS-CBR value is used. |
| --- | --- |
| Applicable for | sidelink |

NOTE 1:	The slot index is based on physical slot index.

--- unchanged text omitted ---