3GPP TSG RAN WG1 Meeting #118	R1-2406041

Maastricht, NL, August 19th – 23rd, 2024

Source:	ZTE Corporation, Sanechips

Title:	Discussion on SRS transmission for NCB PUSCH in STxMP SDM/SFN scheme in TS 38.211

Agenda Item:	8.1

Document for:	Discussion and Decision

Introduction

In this contribution, we present our views on the issue of simultaneous transmitted SRS resources in one symbol for NCB PUSCH in STxMP SDM/SFN scheme.

Discussion

According to the following excerpt in TS 38.213, the maximum number of simultaneous transmitted SRS resources in one symbol just capture the case from one single SRS resource set [1].

TS 38.213-i30

#### 6.1.1.2 Non-Codebook based UL transmission For non-codebook based transmission, PUSCH can be scheduled by DCI format 0_0, DCI format 0_1, DCI format 0_2, DCI format 0_3 or semi-statically configured to operate according to Clause 6.1.2.3. If this PUSCH is scheduled by DCI format 0_1, DCI format 0_2, DCI format 0_3 or semi-statically configured to operate according to Clause 6.1.2.3, the UE can determine its PUSCH precoder(s) and transmission rank based on the SRI(s) when multiple SRS resources are configured, where the SRI(s) is given by one or two SRS resource indicator(s) in DCI according to clause 7.3.1.1.2 and 7.3.1.1.3 of [5, 38.212] for DCI format 0_1 and DCI format 0_2, or the SRI is given by one SRS resource indicator in DCI according to clause 7.3.1.1.4 of [5, 38.212] for DCI format 0_3, or the SRI is given by srs-ResourceIndicator according to clause 6.1.2.3, or SRIs given by srs-ResourceIndicator and srs-ResourceIndicator2 according to clause 6.1.2.3.. The SRS-ResourceSet(s) applicable for PUSCH scheduled by DCI format 0_1 and DCI format 0_2 are defined by the entries of the higher layer parameter srs-ResourceSetToAddModList and srs-ResourceSetToAddModListDCI-0-2 in SRS-config, respectively. The UE shall use one or multiple SRS resources for SRS transmission, where, in a SRS resource set, the maximum number of SRS resources which can be configured to the UE for simultaneous transmission in the same symbol and the maximum number of SRS resources are UE capabilities. The SRS resources transmitted simultaneously occupy the same RBs. For a given CC, multiple SRS resources in a set with usage “nonCodebook” are not expected to be partially overlapped in time. Only one SRS port for each SRS resource is configured. Only one or two SRS resource sets can be configured in srs-ResourceSetToAddModList with higher layer parameter usage in SRS-ResourceSet set to 'nonCodebook', and only one or two SRS resource sets can be configured in srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to 'nonCodebook'. When two SRS resource sets are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to 'nonCodebook', SRIs are given by the DCI fields of two SRS resource indicators in clauses 7.3.1.1.2 and 7.3.1.1.3 of [5, TS 38.212] for DCI format 0_1 and 0_2 and the UE applies the indicated SRI(s) to one or more PUSCH repetitions according to the associated SRS resource set of a PUSCH repetition according to clause 6.1.2.1. A UE does not expect two SRS resource sets are configured in srs-ResourceSetToAddModList with higher layer parameter usage in SRS-ResourceSet set to 'nonCodebook' for a serving cell, when the serving cell is included in schedulingCellListDCI-0-3-r18 for a set of serving cells provided by mc-DCI-SetOfCellsToAddModList-r18. The maximum number of SRS resources per SRS resource set that can be configured for non-codebook based uplink transmission is 1, 2, 4 or 8 depending on UE capability. Each of the indicated SRIs in slot n is associated with the most recent transmission of SRS resource(s) of associated SRS resource set identified by the SRI, where the SRS transmission is prior to the PDCCH carrying the SRI. When two SRS resource sets are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to 'nonCodebook', the UE is not expected to be configured with different number of SRS resources in the two SRS resource sets. <------------------------- Irrelevant parts are omitted ------------------------->

In the meanwhile, for non-codebook based PUSCH in STxMP SDM/SFN scheme, it can be noticed that the UE can simultaneously transmit multiple SRS resources from two SRS resource sets in one symbol, which depends on component-9 in FG 40-6-1a and component-9 in FG 40-6-2a for SDM/SFN, respectively, as follows [2].

| 40. NR_MIMO_evo_DL_UL | 40-6-1a | Single-DCI based STx2P SDM scheme for PUSCH—noncodebook | 1. Dynamic switching by DCI 0_1/0_2 between single-DCI STxMP SDM and sTRP for PUSCH—noncodebook 2. 1 PTRS port for single-DCI based STx2P SDM scheme for PUSCH—noncodebook 3. Support of two SRS resource sets with usage set to 'noncodebook' 4. Maximum number of SRS resources in one SRS resource set 5. Maximum number of layers of each panel for Single-DCI STx2P with SDM 8. Maximum number of simultaneous transmitted SRS resources from one SRS resource set at in one symbol 9. Maximum number of simultaneous transmitted SRS resources from two SRS resource sets in one symbol | Component 4 candidate values: {1, 2 ,3, 4} Component 5 candidate values: {1, 2} Component 8 candidate values: {1, 2, 3, 4} Component 9 candidate values: {1, 2, 3, 4, 5, 6, 7, 8} |
| --- | --- | --- | --- | --- |
| 40. NR_MIMO_evo_DL_UL | 40-6-2a | Single-DCI based STx2P SFN scheme for PUSCH—noncodebook | 2.Dynamic switching by DCI 0_1/0_2 between single-DCI STxMP SFN and sTRP 3. 1 PTRS port for single-DCI based STx2P SFN scheme for PUSCH—noncodebook 4. Support of two SRS resource sets with usage set to 'noncodebook' 5. Maximum number of SRS resources in one SRS resource set 6. Maximum number of MIMO layers of each SRS resource set for NCB PUSCH with SFN scheme 8. Maximum number of simultaneous transmitted SRS resources from one SRS resource set at in one symbol 9. Maximum number of simultaneous transmitted SRS resources from two SRS resource sets in one symbol | Component 5 candidate values: {1, 2 ,3, 4} Component 6 candidate values: {1, 2} Component 8 candidate values: {1, 2, 3, 4} Component 9 candidate values: {1, 2, 3, 4, 5, 6, 7, 8} |

As a result, it should be captured in TS 38.213 that, for NCB PUSCH transmission in STxMP SDM/SFN scheme, multiple SRS resources from two SRS resource sets can be simultaneous transmitted in one symbol, which is subject to UE capability.

Proposal 1: Support to specify in TS 38.214 that for NCB PUSCH in STxMP SDM/SFN scheme, multiple SRS resources from two SRS resource sets can be simultaneous transmitted in one symbol, which is subject to UE capability.

Then, the corresponding draft CR can be found in our companion contribution [3].

Conclusion

For the issue of simultaneous transmitted SRS resources in one symbol for NCB PUSCH in STxMP SDM/SFN scheme, we have the following observation and proposal:

Proposal 1: Support to specify in TS 38.214 that for NCB PUSCH in STxMP SDM/SFN scheme, multiple SRS resources from two SRS resource sets can be simultaneous transmitted in one symbol, which is subject to UE capability.

References

3GPP TS 38.214: "NR; Physical layer procedure for data"

R1-2405571, Session Notes of AI 8.2.2, Ad-Hoc Chair (AT&T)

R1-2406042, Draft CR on SRS transmission for NCB PUSCH in STxMP SDM/SFN scheme in TS 38.214,	ZTE Corporation, Sanechips