3GPP TSG RAN WG1 #118			R1-2407504

Maastricht, NL, August 19th – 23rd, 2024

Title:	[Draft] Reply LS on UL Tx switching

Response to:	R1-2400007 (R2-2313959)

Release:	Rel-18

Work Items:	NR_MC_enh-Core

Source:	NTT DOCOMO, INC. [RAN WG1]

To:	RAN WG2

CC:	RAN WG4

Contact Person:

#### Name:	Hiroki Harada

E-mail Address:	hiroki.harada.sv@nttdocomo.com

Attachment:

1. Overall Description:

RAN1 would like to thank again RAN2 for the LS R1-2400007/R2-2313959.

In previous RAN1 reply LS R1-2401776, RAN1 confirmed that the first RAN2 agreement in the LS R1-2400007/R2-2313959 has no issue from RAN1 perspective, except for a case where Rel-18 UL Tx switching is configured with band combination {A, B} to a UE reporting support of UL Tx switching for band combination {A, B, C} /{A, B, C, D} and no UL-MIMO on band A nor band B.

For the case concerned in R1-2401776, RAN1 made following agreements at RAN1#118 meeting.

Agreement

- Agree the CR in R1-2406993.

- Introduce following capability for the support of fallback to 1T-1T UL Tx switching when just 2 bands configured.

  - FG name

    - Support of 2-band configuration of 1T-1T UL Tx switching by using Rel-18 UL Tx switching configurations

  - Component

    - Support of 2-band configuration of 1T-1T UL Tx switching by using Rel-18 UL Tx switching configurations

  - Prerequisite

    - “switchedUL” is reported via uplinkTxSwitchingOptionForBandPair

  - Consequence if the feature is not supported by the UE

    - UE is not expected to be configured with 1T-1T UL Tx switching when just 2 bands configured (band A and B) by using Rel-18 UL Tx switching configurations

  - Type

    - Per band pair per BC

  - Note

    - The new capability is only needed for a band pair where the UE reports no UL-MIMO on both bands.

    - For a band pair for which the UE separately indicated UL CA support without UL Tx switching, the new capability is not expected to be reported as supported.

  - Mandatory or optional

    - Optional with capability signaling

Agreement

- Send LS to inform RAN2 about the above agreement.

2. Actions:

To RAN WG2

ACTION: RAN1 respectfully asks RAN2 to take above into account and to introduce corresponding capability signalling in Rel-18.

3. Date of Next RAN WG1 Meetings:

TSG-RAN WG1 Meeting #118bis			October 14 to 18, 2024		Hefei, China

TSG-RAN WG1 Meeting #119				November 18 to 22, 2024		Orlando, US