3GPP TSG RAN WG1 #119													R1-2409554

Orlando, US, November 18th – 22nd, 2024

Title: 	Discussion on channel and signal for Ambient IoT

Source: 	ZTE Corporation, Sanechips

Agenda item:	9.4.2.3

Document for:	Discussion and decision

- Introduction

In RAN1 #118bis meeting, agreements on downlink/uplink channel/signal for Ambient IoT were reached in [1].

Agreement For the clock-acquisition part of the R2D time acquisition signal, following is captured in the TR 38.769: - Clock-acquisition part is based on OOK without line coding and includes rising/falling edges, including at least two rising or at least two falling edges for the device to determine the OOK chip duration Agreement For the PRDCH design details, following is captured in the TR 38.769: Following design options have been studied for PRDCH: - Option 1: If there is no L1 R2D control information carried by PRDCH, then PRDCH transmission with only R2D data is considered - Note: In this option, R2D control information (if any) maybe carried by higher-layer signaling - Option 2: If any L1 R2D control information is supported & carried by PRDCH, then for the CRC attachment (appended if there is non-zero length CRC), following two options have been studied - Option 2-1: Joint CRC is attached to L1 R2D control information and R2D data - Option 2-2: Separate CRC is attached to L1 R2D control information and R2D data, respectively Agreement For the start-indicator part of the R2D time acquisition signal, for providing the start of the R2D transmission, following is captured in the TR 38.769: - Following options have been studied for the start-indicator part of the R2D time acquisition signal: - Option 1: ON-OFF transmission is considered based on energy/edge detection, and multiple alternatives have been studied including - Alt 1: A single ON-OFF transmission, i.e. one high-voltage transmission followed by one low-voltage transmission, where ON and OFF may have same or different durations - Alt 2: A multi-ON-OFF transmission, where different ON and different OFF may have same or different durations and different parts may have same or different duration - Option 2: ON-OFF sequence-based design is considered which consists of a pre-defined sequence for detection of start-indicator part based on digital correlation - For both the options, it is observed that a fixed duration for the start-indicator part can be considered, regardless of the value of M used for PRDCH transmissions. - Miss-detection ratio (MDR), false-alarm ratio (FAR) and detection complexity have been considered for the design of the R2D start indicator part by following companies - It is observed by 1 source [Huawei] that for an FAR of ~0%, the MDR of less than 1% can be achieved with Alt 2 of option 1 (considering 2 ON-OFF transmissions with different durations) and it is also observed that low-complexity and reduced power consumption can be achieved - 1 source [ZTE] evaluated Alt 1 of option 1 (considering same duration for ON and OFF) and Alt 2 of option 1 (considering multiple ON-OFF transmissions with same duration) and observed that for an FAR of ~0%, the MDR of less than 1% can be achieved and Alt 1 of option 1 performs better than Alt 2 of option 1. - 1 source [CATT] observed with ON-OFF pattern, that for an FAR of ~0%, the MDR of less than 1% can be achieved with a duration of at least 1 OFDM symbol - 1 source [Qualcomm] compares the performance between option 1 and option 2. It shows almost similar coverage range (SNR requirement) for target MDR of 1%. For MDR of 10%, it shows that sequence-based design provides better performance, and it is observed that during the available time, it is feasible for all devices to detect the start-indicator sequence within the power budget. It is further observed that the FAR with sequence-based design can be improved in case of interference scenarios when compared with pattern-based design. - For both the options, it may be beneficial that the start-indicator part is distinguishable at least from other parts of the R2D transmissions Agreement For single PDRCH generation at the device, following options are captured in TR 38.769: - Option 1: With line coding - Note: Location of repetition block depends on the repetition types agreed to be studied under agenda 9.4.2.1 - Note: Presence of the block “small frequency shift with square wave generation” depends on the agreed options for small frequency shift under agenda 9.4.2.1 - Note: the figure below is for the case where FEC and repetition are present - The figure will be updated by updating the text in the dotted box as follows: “Small frequency shift” - Option 2: Without line coding - Note: Location of repetition block depends on the repetition types agreed to be studied under agenda 9.4.2.1 - Note: the figure below is for the case where FEC and repetition are present Note: Further updates to PDRCH generation can be considered, depending on the progress Agreement For the clock-acquisition part of the R2D time acquisition signal for OOK chip duration determination, following options are studied: - Option 1: Duration of the clock-acquisition part is variable for different M values, i.e. the duration becomes shorter with increasing value of M - Option 2: Duration of the clock-acquisition part is constant for different M values based on repetition, i.e. repetition factor is increased with increasing value of M to keep the duration constant - FFS: Whether/what restriction on M values for the clock-acquisition part Note: Other functionalities of clock-acquisition part is a separate discussion Agreement For the D2R preamble, binary signal is considered. Agreement For the topology 2, following is captured in the TR 38.769: For resource allocation and/or controlling of intermediate UE in topology 2 for R2D and D2R transmissions via the Uu interface between the BS and intermediate UE, following two options have been studied: - Option 1: Higher-layer signaling is used - Option 2: L1 and higher-layer signaling are used From RAN1 perspective, detailed aspects on the two options are left up to work item phase. Following observations related to the two options have been made by following companies: - Four sources [Ericsson, Continental, Vivo, Mediatek] observed that option 1 can save overhead compared to option 2. - One source [Huawei] observed that the timing gap between R2D message and D2R message is not extended, and inventory rate is comparable with UHF RFID and furthermore, it is observed that with option 1, the standard impact includes no or less impact on L1 signaling for resource allocation and controlling of intermediate UE - Six sources [Ericsson, Qualcomm, Oppo, Futurewei, IIT-K, Cewit] observed that option 2 with L1 signaling provides better resource utilization compared to option 1 For the purpose of the Rel-19 study, the study on resource allocation and/or controlling of intermediate UE in topology 2 in RAN1 is considered completed based on two options above. Additional observations can be captured at RAN1#119.

In this contribution, Ambient IoT physical layer design about downlink and uplink channel/signal is discussed.

- Downlink(R2D) channel/signal

  - R2D control information

In RAN1#118bis meeting, the following were discussed but not be achieved.

Proposal 2.1.3-1A For D2R scheduling, following options are studied for indication of each information (potentially including time domain resources, frequency domain resources, MCS-like information, chip duration, ID associated with device(s), repetitions, TBS, midamble related information) to the device via corresponding PRDCH: - Option 1: L1-R2D control information - Option 2: Higher-layer signaling Note: Other L1 R2D control information for D2R scheduling can still be discussed

- D2R scheduling - midamble related information

If midamble is supported in D2R transmission, the following must be considered:

- how to determine the position of the midamble,

- how to determine the number of midambles.

The number of midambles and position of the midambles can be determined according to the following options:

Option 1: predefined rule

Option 2: R2D control information indication

For option 1, predefined values for adding none, one, two, or three or more midambles should be established, along with predefined rules for the placement of midambles. Device can transmit the D2R signal with or without midambles according to predefined rules, and the reader can detect the midamble(s) according to the predefined rules. A straightforward method to determine the position and number of the midamble is to insert one after every X bits that have been transmitted. The value of X is a predefined value.

Option 2 enhances the flexibility of midamble usage, allowing the gNB to provide A-IoT devices with midamble related information, including the number and positioning, based on channel conditions and data rate requirements. For instance, if the gNB specifies that Y midambles are to be used, the process of determining their positions involves evenly dividing the data to be transmitted into (Y+1) segments, with a midamble inserted between each adjacent pair of segments. This results in Y midambles being interspersed throughout the data. Additionally, the gNB can specify the interval at which midambles should be inserted. For example, if the gNB indicates an interval of Z, a midamble is inserted after every Z bits of data have been transmitted.

Both options can be considered.

If midamble is supported in D2R transmission, determining the number and position of midamble(s) can be facilitated by indicating midamble related information (e.g., number of midamble, interval of midambles) in R2D control information or by predefined rules.

- D2R scheduling - MCS-like information

For D2R scheduling, it is necessary to clarify the meaning of ‘MCS-like information’. In NR, the following information are included in MCS table: modulation order, target code rate, and spectral efficiency. And one MCS index is associated with these three information.

For A-IoT D2R modulation, OOK/BPSK/BFSK are studied. The supported modulation schemes for A-IoT devices have not yet been conclusively determined. Different modulation methods may be used for different channel conditions or different coverage requirement. Therefore, when an A-IoT device is capable of supporting multiple modulation schemes, the reader can indicate one of them to the A-IoT device via control information. Thus the modulation method should be explicitly specified within the control information.

In addition to the modulation schemes, if FEC is not a mandatory requirement for the D2R transmission, whether or not FEC is employed in the D2R transmission should also be indicated.

Moreover, if line code is not a mandatory requirement for the D2R transmission, whether or not using line code should also be indicated.

In A-IoT system, due to the diverse use cases and coverage requirements, more flexible and adaptable coding rate should be considered. For example, one information bit can be encoded into T bits, which correspond to different code rates. This flexibility allows for the utilization of different code rates for the target use case or coverage requirement. Different coding rates can be adjusted according to changes in channel conditions. For example, when the channel quality is good, a higher coding rate can be chosen to increase data transmission speed; when the channel quality is poor, a lower coding rate can be selected to enhance interference resistance. In order to decode the downlink data or transmit the uplink data correctly, A-IoT devices must be aware of the information. Thus, if multiple coding rates are used in A-IoT system, the applied scheme should be conveyed in control information.

One thing should be noted is that, code rate is associated with repetition numbers, whether FEC is used[2]. Using repetition or FEC can achieve a lower code rate, for higher code rate, puncturing or other rate matching methods can also be considered.

‘MCS-like information’ includes at least the followings:

  - coding rate,

  - modulation methods, if multiple modulation methods are supported by one A-IoT device,

  - FEC indication, if FEC is not a mandatory function in D2R,

  - line code indication, if line code is not a mandatory function in D2R.

For D2R scheduling information, there is no necessity to decode these information as rapidly as possible. The D2R transmission is always triggered by a R2D signaling, device only provides D2R signaling feedback once they have fully received the R2D signaling. If the R2D signaling is incorrectly decoded, the device will be unable to provide D2R signaling feedback. Consequently, there is no compelling reason to transmit D2R scheduling information via L1 signaling.

For D2R scheduling, higher-layer signaling is used for indication of each information to the device via corresponding PRDCH.

- R2D reception- repetitions

For R2D scheduling, similar as D2R scheduling, supporting multiple coding rates is beneficial for data scheduling, and should be considered. Different coding rates can be achieved by varying the number of repetitions. According to the discussion in [2], using block level repetition is easier to guarantee different channel within the time frame of repeating block so that the detection performance is largely improved under the case of larger transmission block.

Therefore, the number of repetitions can be considered.

For R2D transmission, ‘repetitions’ can be considered in control information.

- R2D reception - reader ID

It would be beneficial for an A-IoT device to establish exclusive communication with a single reader. This selection reduce the interference between different readers. The reader ID enables A-IoT device to recognize and connect with the specific reader. Upon receiving multiple inventory signals from different readers, the A-IoT device has the capability to recognize different readers and respond to only one reader, thereby optimizing its response strategy and reduce interference.

Reader ID can be considered at least in R2D signaling.

- R2D reception- ID associated with device(s)

A-IoT devices have low energy storage, reducing unnecessary energy consumption and charging time is beneficial for latency reduction.

In previous RAN1 meeting, ID associated with device(s) is supported in R2D control information. The ID could be unique to a single device or belong to a group of devices. A-IoT device can ascertain that the signal is intended for it by identifying the ID in the R2D/D2R signaling. If the signaling is not for the A-IoT device, the A-IoT device can stop receiving and decoding the signaling. However, different paging messages utilize distinct ID formats. As shown in the RAN2 agreement as below, some paging messages may not contain an ID, while others may include an ID for a single device or a group ID for a group of devices. The bit width allocated to the ID field varies with each paging message and is not predetermined for the cases, complicating the decoding process at device side.

Agreements RAN2#126 1 RAN2 will study the following cases for AIoT paging message: - a message containing an ID of a single A-IoT device. - a message containing a group ID that maps to multiple A-IoT devices. - a message that does not contain an ID, i.e., addressed for all devices that can receive the AIoT message. - a message containing multiple IDs of A-IoT devices. Need to confirm the need for this use case based on SA2 discussion. What device ID and group ID and scenarios is depending on SA2 discussion.

Since different paging message types utilize distinct ID formats, the bit width allocated to the ID field varies with each paging message type and is not predetermined for A-IoT devices, complicating the decoding process.

Due to the variable bit width of the ID field, the ID associated with device(s) should be transmitted via higher layer signaling.

The ID associated with device(s) should be transmitted via higher layer signaling.

- R2D reception- message type indication

To save power, the device should be able to quickly obtain the ID information, necessitating a straightforward decoding process for the ID field. However, according to the above discussion, the ID associated with the device(s) should be transmitted via higher layer signaling, which can not used for early termination. To support the early termination, a message type specified in the control information can be considered.

For example, message type indication indicates whether the R2D carries the signaling that is transmitted before Msg1 (e.g., paging message), or after Msg1 (specific signaling for one or a group of device, e.g., Msg 2 or Msg 4). This helps to prevent devices that have not sent Msg1 from detecting unnecessary R2Ds transmitted for other devices (that have sent Msg1). If there are 1024 devices being inventoried, then the last device being inventoried can avoid receiving more than 1023*x R2Ds, where x represents the number of times the R2D transmitted after Msg1 for other devices (including retransmissions, as well as Msg2, Msg4, etc.).

Since there would be difference paging message, i.e., for one device, device group, or all the devices according to RAN2 agreements. The paging message type indication via R2D is also needed. Additionally, the message type can also denote which devices should receive the R2D signaling, such as indicating the device type.

Message type indication which used to indicate whether the R2D signaling that is transmitted before Msg1 or after Msg1 can be considered.

As discussed above, the R2D control information can include the code rate. It is obvious that, the code rate should be know for the device to decode it correctly. Therefore, the code rate should also be encoded independently and be transmitted in L1.

For R2D reception, the following information can be explicitly indicated to the device via the corresponding PRDCH via L1 R2D control signalling:

  - Message type indication

  - repetitions

- Note: Other L1 R2D control information for R2D reception can still be discussed

For R2D reception, the following information can be explicitly indicated to the device via the corresponding PRDCH via higher-layer signalling:

  - ID associated with devices

  - Reader ID

- Note: Other higher-layer R2D control information for R2D reception can still be discussed

  - R2D signals

    - Preamble (timing acquisition signal)

- Start-indicator part(SIP)

- In RAN1#118bis meeting, the following options were captured.

- Option 1: ON-OFF transmission is considered based on energy/edge detection, and multiple alternatives have been studied including

- Alt 1: A single ON-OFF transmission, i.e. one high-voltage transmission followed by one low-voltage transmission, where ON and OFF may have same or different durations.

- Alt 2: A multi-ON-OFF transmission, where different ON and different OFF may have same or different durations and different parts may have same or different duration.

- Option 2: ON-OFF sequence-based design is considered which consists of a pre-defined sequence for detection of start-indicator part based on digital correlation.

- Furthermore, for both options, several observations have been made to assist in the SIP design process.

- For both the options, it is observed that a fixed duration for the start-indicator part can be considered, regardless of the value of M used for PRDCH transmissions.

- For both the options, it may be beneficial that the start-indicator part is distinguishable at least from other parts of the R2D transmissions

In this section, some other design principles for start-indicator part are discussed.

Alt 1 in option 1 is the simplest approach among all alternatives. However, the durations of the ON and OFF should be discussed. Figure 1 shows the simulated MDR of start-indicator part with different M values and different bit sequences. According to the simulation results, the performance of fixed M = 1 demonstrates superior performance compared to that with M = 2. Sequence [1 0] with M=1 provides a better MDR and proves that Alt 1 in option 1 can be considered.

Figure 1 MDR of start-indicator part

Apart from MDR, FAR is also simulated and the evaluation results are provided in the following table.

Table 1 FAR of start-indicator part

| (M value, bit sequence) | FAR |
| --- | --- |
| (1,[10]) | 0 |
| (1,[1010]) | 0 |
| (2,[1010]) | 0 |
| (2,[10101010]) | 0 |

According to the simulation results, the FAR of fixed sequence with fixed M=1 or M=2 are small. Simulation results shows that an increased M value correlates with a higher required SNR.

Agreement The following table is a starting point for the study of M values and the associated minimum Btx,R2D value - Reader can use any R2D transmission bandwidth >= minimum Btx,R2D - FFS: Impacts, if any, of CP handling solutions, and other impacts - Note: depending on further study, the maximum value of M may be less than 32 M; Minimum Btx,R2D # of PRBs. 1; 1. 2; 1. 4; 1. 6; 1. 8; 2. 12; 2. 16; 2. 24; 2. 32; 3.

Moreover, according to the agreement shown above, a larger M value requires a larger bandwidth, if a large M value is used in SIP, the required PRB number will be large.

Therefore, the M value for SIP should be kept modest, as SIP is utilized by A-IoT devices to recognize the start of R2D transmissions. In RFID, the duration of delimiter is 12.5us. The A-IoT system can take this value into consideration. Consider that 12.5us is a duration slightly greater than the chip length of M=6, the M value of both ON and OFF durations should not greater than 6.

- Considering that an SIP is transmitted with every downlink, the overhead of SIP should not be high. Even when SIP uses chips with M=1, a maximum of two OFDM symbols is sufficient. SIP can be accommodated within one or two OFDM symbols. However, if SIP does not occupy a whole number of OFDM symbols (for instance, occupying just 1/2 of an OFDM symbol), then SIP and CAP will need to share a single OFDM symbol. Given that the M value for CAP should be identical to that of the data, the OFDM symbol will contain two M values, which could complicate the decoding process for the device.

Consider the following SIP design principles:

  - The M value of both ON and OFF in the SIP should not be greater than 6.

  - The durations of the SIP should be 1 or 2 OFDM symbols.

- Another consideration is the necessity for SIP to be distinguishable from other parts of the R2D transmissions. The primary reason for this is to prevent A-IoT devices from erroneously identifying data as SIP and subsequently attempting to receive additional content, which would drain the device's power. However, the actual power savings from such differentiation are not clear. When an A-IoT device detects the SIP, it will proceed to receive the subsequent R2D transmission. Conversely, if the A-IoT device fails to detect the SIP, it will continue to attempt SIP detection. In any case, A-IoT devices must remain active, decoding and consuming power. Additionally, the clock-acquisition part (CAP) can be used in conjunction with SIP to indicate the start of the R2D transmission, which diminishes the urgency to distinctly differentiate SIP from other R2D transmission parts. However, we are open to discuss it.

The advantage of distinctly differentiating SIP from other parts of the R2D transmission is small.

- If it is necessary to distinguish SIP from subsequent transmissions, the SIP should include a pattern that will not occur during data transmission. For example, ‘1 0 0 0’ with M=4 or ‘1 0 1 1 0 0 1 1 1 0 0 0’ of M=6. Using multiple M values can also satisfied this principle. For instance, using '1 0 1' with M=6 followed by ‘1 0' with M=4 ensures that the combination of these distinct chips with various M values will not recur in subsequent R2D transmissions.

- Figure 2 SIP with two M values

Capture the SIP design principles in the following TP

The R-TAS start-indicator part is not included in TD2R_min, and an ON/OFF pattern i.e., high/low voltage transmission, is applied. The following options have been studied for the start-indicator part: Option 1: ON-OFF transmission is considered based on energy/edge detection, and multiple alternatives have been studied including Alt 1: A single ON-OFF transmission, i.e. one high-voltage transmission followed by one low-voltage transmission, where ON and OFF may have same or different durations Alt 2: A multi-ON-OFF transmission, where different ON and different OFF may have same or different durations and different parts may have same or different duration Alt 2-1: A multi-ON-OFF transmission with single M value. Alt 2-2: A multi-ON-OFF transmission with multiple M values. Option 2: ON-OFF sequence-based design is considered which consists of a pre-defined sequence for detection of start-indicator part based on digital correlation For both the options, it is observed that a fixed duration for the start-indicator part can be considered, regardless of the value of M used for PRDCH transmissions. For both the options, it is observed that a larger M value for the start-indicator part, the larger bandwidth is required. For both the options, the M value of both ON and OFF in the start-indicator part should not be greater than 6. For both the options, the durations of the start-indicator part should be 1 or 2 OFDM symbols.

- Clock-acquisition part

The clock-acquisition part provides at least the chip synchronization of the subsequent physical channel transmission. This allows the A-IoT device to determine the chip duration of the subsequent physical channel transmission.

Similar as the design of start-indicator part, the length of the clock-acquisition part and how to handle the CP should also be discussed.

When designing the clock-acquisition part, the following factors should be taken into account:

  - the length of the clock-acquisition part,

  - the CP handling.

In order to distinguish between the start-indicator part and the clock-acquisition part, the voltage of the first chip of the clock-acquisition part must be different from the voltage of the last chip of the start-indicator part, that is, there should be a voltage transition.

There should be a voltage transition between the start-indicator part and clock-acquisition part.

Agreement For the clock-acquisition part of the R2D time acquisition signal for OOK chip duration determination, following options are studied: - Option 1: Duration of the clock-acquisition part is variable for different M values, i.e. the duration becomes shorter with increasing value of M - Option 2: Duration of the clock-acquisition part is constant for different M values based on repetition, i.e. repetition factor is increased with increasing value of M to keep the duration constant - FFS: Whether/what restriction on M values for the clock-acquisition part Note: Other functionalities of clock-acquisition part is a separate discussion

In previous RAN1 meeting, the above agreement was achieved.

Using option 1, the duration of clock-acquisition part is variable for different M values. For example, only one pattern of clock-acquisition part is used for all M values, the duration of CAP will be shorter with increasing value of M. This may enhance the complexity of device detection for the CAP.

With option 2, the duration of the clock-acquisition part remains consistent across all M values. However, this approach may incur a significant overhead. To ensure that the device can determine the OOK chip duration, the CAP must contain at least two rising or two falling edges. Given that the minimum number of chips required is three, this necessitates a minimum of three OFDM symbols for M=1. If the clock-acquisition part's duration is to be uniform for all M values, then the CAP for other M values would also require at least three OFDM symbols, which may be excessive and not optimally efficient in terms of chip usage within the CAP.

With option 2 ‘the duration of the clock-acquisition part remains consistent across all M values’, at least three OFDM symbols are required for clock-acquisition part, and it is excessive and not optimally efficient in terms of chip usage within the clock-acquisition part.

For the pattern of clock-acquisition part, the simplest way is to use multiple alternating high and low voltages, where each voltage occupies one chip(i.e., 101010...).

It is important to recognize that the clock-acquisition part determines the chip duration for the subsequent transmission of physical channels. Consequently, the M value employed in the clock-acquisition part should be consistent with the M value utilized for data and/or control transmissions. This alignment ensures synchronization and optimal performance in the communication system. The duration of the clock-acquisition part may vary with the M values used data and/or control transmissions. By employing multiple alternating high and low voltages(i.e., 101010...), we can ensure that the CP does not introduce additional transition edges, as shown in Figure 3 as below, thereby facilitating better decoding performance.

Figure 3 Multiple alternating high and low voltages(i.e., 101010...) ensure that the CP does not introduce additional transition edges

For example, as shown in Figure 3, the clock-acquisition part includes multiple alternating high and low voltage. In the case of, e.g., M = 2 or M = 4, the boundaries of the OFDM symbols are transition edges, which means that regardless of whether the CP is a high or low voltage, no additional transition edges are introduced by CP.

Using multiple alternating high and low voltages, where each voltage occupies one chip, will not introduce additional transition edges, at least for small M values, thereby facilitating better decoding performance. This approach is applicable across different M values, simplifying the decoding processing.

To ensure not introduce new transition edges in CP, as discussed in our contribution [2], one method for CP handling in data transmission is Alt 1-2 in Method 2, which requires that the data transmission must start from an even-numbered chip within an OFDM symbol. Hence, the length of clock-acquisition part occupies an odd number of chips can avoid introducing new transition edges in CP.

Figure 4 One example of clock-acquisition part

For example, as shown in Figure 4, the clock-acquisition part includes 7 chips. Since the two chips resulting from Manchester encoding are split across two OFDM symbols, due to the characteristics of Manchester encoding, there must be a transition edge between these two chips. Therefore, the OFDM symbol containing the data will not cause any new transition edges.

Given that the initial chip of data is random, the final chip of the CAP might be same as the first chip of data and can not be utilized to ascertain the chip duration. Employing an odd number of CAPs can ensure that the high voltage and low voltage have an equal number of chips, which could be more advantageous for determining chip duration. As discussed above, the chip number of CAP should not less than 3. Thus, an odd number not less than 3 can be considered for the chip number of CAP.

Clock-acquisition part with a pattern of multiple alternating high and low voltages(i.e., 101010... or 01010...), the choice between using 101010... or 010101... is dictated by the voltage of the last chip in start indicator part.

Clock-acquisition part with an odd number of chips , with a minimum of three chips, can be a start point.

Using the pattern discussed above, the number of chips in the clock-acquisition part is one important parameter. Different number of chips may provide different performance. For different M values, different chip number of clock-acquisition part can be considered. To evaluate the performance of different patterns of clock-acquisition part, MDR and False Detection Rate (FDR) are used, where FDR is the probability that the PRDCH M value is erroneously estimated by the device, despite successful detection of preamble.

Table 2: Simulation results for clock-acquisition part

| Preamble MDR, M=4 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| M=4 | SNR=16 | SNR=18 | SNR=20 | SNR=22 | SNR=24 |
| chip number of clock-acquisition part = 3 | 9.93% | 7.06% | 4.67% | 3.00% | 1.73% |
| chip number of clock-acquisition part = 9 | 13.57% | 10.03% | 7.17% | 4.75% | 3.29% |
| Preamble FDR, M=4 |  |  |  |  |  |
| M=4 | SNR=16 | SNR=18 | SNR=20 | SNR=22 | SNR=24 |
| chip number of clock-acquisition part = 3 | 16.31% | 11.95% | 8.68% | 6.00% | 4.07% |
| chip number of clock-acquisition part = 9 | 3.73% | 2.58% | 1.97% | 1.79% | 1.27% |
| Preamble MDR, M=8 |  |  |  |  |  |
| M=8 | SNR=18 | SNR=20 | SNR=22 | SNR=24 | SNR=26 |
| chip number of clock-acquisition part = 3 | 6.98% | 4.48% | 2.98% | 1.71% | 1.19% |
| chip number of clock-acquisition part = 9 | 12.75% | 8.88% | 6.08% | 3.55% | 2.22% |
| Preamble FDR, M=8 |  |  |  |  |  |
| M=8 | SNR=18 | SNR=20 | SNR=22 | SNR=24 | SNR=26 |
| chip number of clock-acquisition part = 3 | 12.37% | 9.61% | 6.97% | 5.01% | 3.05% |
| chip number of clock-acquisition part = 9 | 6.01% | 4.79% | 3.71% | 2.51% | 1.61% |

According to the simulation results, it can be observed that the more the number of chips in clock-acquisition part, the lower the FDR. However, as the number of chips increases, the MDR and overhead will be slightly impacted. Thus, the number of chips in clock-acquisition part should not be to large, for example, the number of chips is not exceeding 9.

With the increase of the chip number of clock-acquisition part, the MDR is slightly increased, while the FDR decreases.

According to the simulation results, different M values may require different number of chips of CAP. A larger M value requires a larger number of chips in CAP. Thus, the following options can be considered:

- Option 1: same CAP pattern for all M values

- Option 2: different CAP patterns for different M value range.

For option 1, employing a consistent CAP pattern across all M values simplifies the decoding process for the device. However, the number of chips must be determined based on the requirements of the largest available M value, which could result in redundancy for smaller M values.

For option 2, using different CAP patterns will increase the processing complexity for the device. However, this method allows for the design of different patterns tailored to the needs of various M values, offering greater flexibility.

Both options can be considered.

For clock-acquisition part design, the following can be considered:

  - Option 1: same CAP pattern for all M values

  - Option 2: different CAP patterns for different M value range.

Using odd number of chips for CAP will not cause additional transition edge in CP at least for small M values. However, it is noted that in the case of larger M value (i.e. M≥16), the CP covers more than one chip which will introduce additional transition edge(s) during the CP duration.

In our views, the following options can be used to resolve this issue.

- Option 1: Using CP handling method for PRDCH

- Option 2: adding protection bits in CAP.

Option 1 is not a CAP design principle, it applicable to both the CAP and the other PRDCH transmissions. Specifically, with large M values, CAP does not necessitate a specialized design. By handling the CP within the CAP structure similarly to how it is managed in other parts of the PRDCH, the adverse effects of CP can be effectively mitigated. All the CP handling method in [2] can be considered.

Option 2 requires multiple CAP patterns. One simple way is to use a special design for the first ON/OFF in CAP. For example, add ‘1 1’ before CAP sequence for small M values.

Figure 5 CAP for M=16

According to the Figure 5, using multiple consecutive '1's at the beginning of CAP helps to distinguish the start of CAP from the CP.

To generate CAP sequences for larger M values(i.e., M>=16), '11' as protection bits can be prepended to the CAP sequences used for smaller M values. For example, CAP sequence is ‘10101’ for M <16, and CAP sequence is ‘1110101’ for M>=16.

One thing should be noticed is that the CAP for large M value should within one OFDM symbol, and only the CP before CAP needs to be handled.

Capture the CAP design principles in the following TP

The clock-acquisition part is based on OOK without line coding, and includes rising/falling edges, including at least two rising or at least two falling edges for the device to determine the OOK chip duration. The following options for design of the clock-acquisition part are studied: Option 1: Duration of the clock-acquisition part is variable for different M values, i.e. the duration becomes shorter with increasing value of M. - - Alt 1: same CAP pattern for all M values. - - Alt 2: different CAP patterns for different M value range. Option 2: Duration of the clock-acquisition part is constant for different M values based on repetition, i.e. repetition factor is increased with increasing value of M to keep the duration constant. It is for further study whether/what restriction on M values for the clock-acquisition part. It is observed that with option 2, at least three OFDM symbols are required for clock-acquisition part which may be excessive. For both options, if the clock-acquisition part spans multiple OFDM symbols, ensure that the boundaries of the OFDM symbols within the clock-acquisition part are transition edges. For both options, an odd number of chips in clock-acquisition part can be start point. To mitigate the impact of the CP in the clock-acquisition part for large M values, the following options can be considered: - Option 1: Using CP handling method for PRDCH. - Option 2: adding protection bits in clock-acquisition part.

    - M values for control information and R2D data

In previous RAN1 meetings, some companies prefer that control information using different M value from the R2D data. One thing should be emphasized is that the last chip of the control information and the first chip of the R2D data may be in one OFDM symbol. In some cases, one OFDM symbol may not be able to fully accommodate chips with two M values as shown in Figure below.

Figure 6 Different M values used in control information and R2D data

To prevent the aforementioned situation, if the M value of the control information differs from that of the R2D data, then the M values should be chosen cautiously. For instance, the M value of the R2D data could be selected as a multiple of the M value of the control information. However, the simplest way is use same M value for both control information and data, if control information is supported.

If the M value of the control information differs from that of the R2D data, there is a case that one chip across the boundary of OFDM symbol.

Using same M value for both control information and R2D data, if control information is supported in PRDCH.

    - Postamble

During the previous RAN1 meetings, there was a discussion regarding the necessity of a postamble, but consensus was not reached.

Agreement To determine or derive the end of PRDCH transmission, study at least following options: - Option 1: R2D postamble immediately follows the PRDCH to indicate the end of the PRDCH. - Option 2: Based on R2D control information.

For Option 2, the more TBS values need to be indicated, the larger payload is required. And for different M values, the candidate TBS may be varied to align with the OFDM symbol boundary. In contrast, employing a postamble enables greater flexibility in accommodating various payload sizes. As discussed in our contribution [3], using postamble is preferred to be used to align the end of R2D transmission with OFDM symbol boundary. Therefore, Option 1 is the preferred choice. It should be noted that, the overhead of postamble should not be too large.

To determine or derive the end of PRDCH transmission, Option 1：R2D postamble immediately follows the PRDCH to indicate the end of the PRDCH is proposed.

For postamble pattern, the following three options can be considered:

Option#1: high voltage

Option#2: low voltage

Option#3: ON/OFF pattern, e.g., using a sequence

All these three options can be considered. However, if the postamble is also used to align the end of R2D transmission and the OFDM symbol boundary, option#3 is not suitable. For option#1 and option#2, to distinguish from the data with Manchester code, their lengths should be greater than two chips.

For R2D postamble pattern, the following two options can be considered:

  - Option#1: high voltage (preferred)

  - Option#2: low voltage

  - Note: The length of R2D postamble should be greater than two chips.

- Uplink(D2R) channel/signal

  - D2R Control information

In previous RAN1 meetings, the following was agreed.

- Agreement For L1 D2R control information (if defined), the following are not considered for further study: - CSI feedback - Autonomous SR - FFS: Whether any other L1 D2R control information is needed or not

Proposal 2.2.2-1C For information regarding the (un)successful reception of the R2D command at the device, following options have been studied: - Option 1: The information can be signaled via higher-layer signaling - From PHY perspective, this information is D2R data - Option 2: The information is signaled via L1 D2R control information

If the A-IoT system allows the A-IoT device to autonomously select certain parameters (such as encoding/modulation scheme), it may need to carry corresponding information in the D2R control information. Otherwise, there is no need to have a L1 D2R control information.

For ACK information, most R2D commands are typically followed by a D2R response. Upon receiving this D2R response, the reader can confirm that the R2D transmission has been successfully decoded. Consequently, for commands that require D2R transmission from the device, an ACK feedback is unnecessary.

Moreover, the procedure of commands are being discussed by RAN2. Whether ACK feedback is needed should be postponed till RAN2 agreements, or decided by RAN2, which has larger impact on RAN2 regarding the potential re-tx procedure.

The necessity of D2R control information in D2R transmission should be clarified firstly.

  - D2R signals

    - Preamble (timing acquisition signal)

The D2R preamble can serve the purposes of facilitating channel estimation, determination of the start of D2R transmission, SFO estimation, etc. For the D2R preamble detection, correlation operation can be implemented by reader. Therefore, it is straightforward to design the D2R preamble with a sequence. Furthermore, it can consist of one or multiple sequences. Leveraging multiple orthogonal sequences enables support for multi-user, thereby, improving the system capacity and minimizing interference between different cells.

Similar as R2D preamble, the crucial aspect is determining the length of the preamble.

To assess the performance of the preamble, metrics such as synchronization accuracy, channel measurement results, and the BLER of the data with the preamble can be considered.

One or multiple sequences can be studied for the D2R preamble. The design/length of D2R preamble should be determined based on the metrics such as synchronization accuracy, channel estimation, BLER performance, etc..

The figure below presents the simulation results for preambles of varying lengths when TBS is set to 16 bits, employing Manchester coding and 1/3 FEC. The simulation result indicates that a longer preamble yields superior performance. However, utilizing a preamble with a large number of bits can significantly increase overhead, particularly for smaller TBS values. Balancing the trade-off between overhead and performance, a preamble with 32 bit can be considered.

Figure 7 preamble with various lengths

A sequence with 32 bits for D2R preamble can be a start point.

    - Postamble

During the previous RAN1 meeting, there was a discussion regarding the necessity of a postamble, but consensus was not reached.

Agreement For the reader to acquire the end of PDRCH transmission, study at least following options: - Option 1: D2R postamble immediately follows the PDRCH - Option 2: Based on control information

Option 2 is using TBS indication within the R2D or D2R control information to specify the payload size. Similar as R2D, TBS indication is limited in its ability to represent a range of TBS values. In contrast, employing a postamble enables greater flexibility in accommodating various payload sizes.

However, there is a concern that the misidentification or failure to detect the postamble could potentially lead to the failure of PDRCH decoding. And TBS indication may be more trustworthy. Thus, the TBS can be used to indicate the end of PDRCH transmission. One thing should be noticed is that it does not means postamble is not needed. In addition to marking the end of PDRCH transmission, the postamble also plays a role in SFO estimation and channel measurement, which are crucial for maintaining the integrity and performance of the communication system.

Knowing the TBS can also help the reader to perform SFO estimation according to the position of the corresponding peak of postamble and the position of the corresponding peak of preamble. As demonstrated in our simulation results in [4], utilizing the postamble offers superior performance and should therefore be taken into consideration. Similar as preamble, one or multiple sequences can be studied for the D2R postamble. Each preamble is associated with a postamble.

D2R postamble should be supported. One or multiple sequences can be studied for the D2R postamble. Each preamble is associated with a postamble.

    - Midamble

In the case of D2R transmissions, the receiving end is either a gNB or an intermediate UE that possesses advanced data processing capabilities, which allows for the possibility of employing coherent reception techniques. The implementation of such coherent reception methods is complicated by the varying conditions of the channel. Given that D2R transmissions can extend over a considerable duration, the channel characteristics might evolve over time. Relying on the preamble and postamble for channel estimation might not provide adequate accuracy. Midamble can also used for channel measureemnt, and significantly improve the efficiency of data reception, especially in the case of large TBS and low data rate.

The difference between midamble and preamble should be discussed. Utilizing the same sequence as preamble for the midamble is a viable option. Other sequence, such as sequences distinct from the preamble could also be considered. More discussion on D2R midamble can be found in our accompanying contribution [4]. Similar as preamble, one or multiple sequences can be studied for the D2R midamble.

Midamble for D2R should be considered, and one or multiple sequences can be studied for the D2R midamble.

- Conclusion

In this contribution, the downlink and uplink signal/channels are discussed for A-IoT. We have the following observations and proposals.

Observation 1: Since different paging message types utilize distinct ID formats, the bit width allocated to the ID field varies with each paging message type and is not predetermined for A-IoT devices, complicating the decoding process.

Observation 2: The advantage of distinctly differentiating SIP from other parts of the R2D transmission is small.

Observation 3: With option 2 ‘the duration of the clock-acquisition part remains consistent across all M values’, at least three OFDM symbols are required for clock-acquisition part, and it is excessive and not optimally efficient in terms of chip usage within the clock-acquisition part.

Observation 4: Using multiple alternating high and low voltages, where each voltage occupies one chip, will not introduce additional transition edges, at least for small M values, thereby facilitating better decoding performance. This approach is applicable across different M values, simplifying the decoding processing.

Observation 5: With the increase of the chip number of clock-acquisition part, the MDR is slightly increased, while the FDR decreases.

Observation 6: If the M value of the control information differs from that of the R2D data, there is a case that one chip across the boundary of OFDM symbol.

Proposal 1: If midamble is supported in D2R transmission, determining the number and position of midamble(s) can be facilitated by indicating midamble related information (e.g., number of midamble, interval of midambles) in R2D control information or by predefined rules.

Proposal 2: ‘MCS-like information’ includes at least the followings:

- coding rate,

- modulation methods, if multiple modulation methods are supported by one A-IoT device,

- FEC indication, if FEC is not a mandatory function in D2R,

- line code indication, if line code is not a mandatory function in D2R.

Proposal 3: For D2R scheduling, higher-layer signaling is used for indication of each information to the device via corresponding PRDCH.

Proposal 4: For R2D transmission, ‘repetitions’ can be considered in control information.

Proposal 5: Reader ID can be considered at least in R2D signaling.

Proposal 6: The ID associated with device(s) should be transmitted via higher layer signaling.

Proposal 7: Message type indication which used to indicate whether the R2D signaling that is transmitted before Msg1 or after Msg1 can be considered.

Proposal 8: For R2D reception, the following information can be explicitly indicated to the device via the corresponding PRDCH via L1 R2D control signalling:

- Message type indication

- repetitions

Note: Other L1 R2D control information for R2D reception can still be discussed

Proposal 9: For R2D reception, the following information can be explicitly indicated to the device via the corresponding PRDCH via higher-layer signalling:

- ID associated with devices

- Reader ID

Note: Other higher-layer R2D control information for R2D reception can still be discussed

Proposal 10: Consider the following SIP design principles:

- The M value of both ON and OFF in the SIP should not be greater than 6.

- The durations of the SIP should be 1 or 2 OFDM symbols.

Proposal 11: Capture the SIP design principles in the following TP

The R-TAS start-indicator part is not included in TD2R_min, and an ON/OFF pattern i.e., high/low voltage transmission, is applied. The following options have been studied for the start-indicator part: Option 1: ON-OFF transmission is considered based on energy/edge detection, and multiple alternatives have been studied including Alt 1: A single ON-OFF transmission, i.e. one high-voltage transmission followed by one low-voltage transmission, where ON and OFF may have same or different durations Alt 2: A multi-ON-OFF transmission, where different ON and different OFF may have same or different durations and different parts may have same or different duration Alt 2-1: A multi-ON-OFF transmission with single M value. Alt 2-2: A multi-ON-OFF transmission with multiple M values. Option 2: ON-OFF sequence-based design is considered which consists of a pre-defined sequence for detection of start-indicator part based on digital correlation For both the options, it is observed that a fixed duration for the start-indicator part can be considered, regardless of the value of M used for PRDCH transmissions. For both the options, it is observed that a larger M value for the start-indicator part, the larger bandwidth is required. For both the options, the M value of both ON and OFF in the start-indicator part should not be greater than 6. For both the options, the durations of the start-indicator part should be 1 or 2 OFDM symbols.

Proposal 12: When designing the clock-acquisition part, the following factors should be taken into account:

- the length of the clock-acquisition part,

- the CP handling.

Proposal 13: There should be a voltage transition between the start-indicator part and clock-acquisition part.

Proposal 14: Clock-acquisition part with a pattern of multiple alternating high and low voltages(i.e., 101010... or 01010...), the choice between using 101010... or 010101... is dictated by the voltage of the last chip in start indicator part.

Proposal 15: Clock-acquisition part with an odd number of chips , with a minimum of three chips, can be a start point.

Proposal 16: For clock-acquisition part design, the following can be considered:

- Option 1: same CAP pattern for all M values

- Option 2: different CAP patterns for different M value range.

Proposal 17: Capture the CAP design principles in the following TP

The clock-acquisition part is based on OOK without line coding, and includes rising/falling edges, including at least two rising or at least two falling edges for the device to determine the OOK chip duration. The following options for design of the clock-acquisition part are studied: Option 1: Duration of the clock-acquisition part is variable for different M values, i.e. the duration becomes shorter with increasing value of M. - - Alt 1: same CAP pattern for all M values. - - Alt 2: different CAP patterns for different M value range. Option 2: Duration of the clock-acquisition part is constant for different M values based on repetition, i.e. repetition factor is increased with increasing value of M to keep the duration constant. It is for further study whether/what restriction on M values for the clock-acquisition part. It is observed that with option 2, at least three OFDM symbols are required for clock-acquisition part which may be excessive. For both options, if the clock-acquisition part spans multiple OFDM symbols, ensure that the boundaries of the OFDM symbols within the clock-acquisition part are transition edges. For both options, an odd number of chips in clock-acquisition part can be start point. To mitigate the impact of the CP in the clock-acquisition part for large M values, the following options can be considered: - Option 1: Using CP handling method for PRDCH. - Option 2: adding protection bits in clock-acquisition part.

Proposal 18: Using same M value for both control information and R2D data, if control information is supported in PRDCH.

Proposal 19: To determine or derive the end of PRDCH transmission, Option 1：R2D postamble immediately follows the PRDCH to indicate the end of the PRDCH is proposed.

Proposal 20: For R2D postamble pattern, the following two options can be considered:

- Option#1: high voltage (preferred)

- Option#2: low voltage

- Note: The length of R2D postamble should be greater than two chips.

Proposal 21: The necessity of D2R control information in D2R transmission should be clarified firstly.

Proposal 22: One or multiple sequences can be studied for the D2R preamble. The design/length of D2R preamble should be determined based on the metrics such as synchronization accuracy, channel estimation, BLER performance, etc..

Proposal 23: A sequence with 32 bits for D2R preamble can be a start point.

Proposal 24: D2R postamble should be supported. One or multiple sequences can be studied for the D2R postamble. Each preamble is associated with a postamble.

Proposal 25: Midamble for D2R should be considered, and one or multiple sequences can be studied for the D2R midamble.

- References

R1-2409304, Final FL summary on downlink and uplink channel/signal aspects, Moderator (Apple), RAN1#118bis

R1-2408067, Discussion on general aspects of physical layer design for Ambient IoT, ZTE, Sanechips, RAN1#118is

R1-2408086, Discussion on frame structure and physical layer procedure for Ambient IoT, ZTE, Sanechips, RAN1#118bis

R1-2409553, Discussion on frame structure and physical layer procedure for Ambient IoT, ZTE, Sanechips, RAN1#119

- Annex

Table A.1: Simulation assumptions

| Parameters | Assumptions |
| --- | --- |
| R2D/D2R common parameters |  |
| Carrier frequency | 900MHz |
| SCS | 15 kHz |
| Channel model | TDL-A |
| Delay spread | 30 ns |
| Device velocity | 3 km/h |
| Sampling frequency | 1.92MHz |
| R2D specific parameters |  |
| ADC bit width | 4-bit |