# 3GPP TSG-RAN WG1 Meeting #119                                            R1-2409577

Orlando, US, November 18th – 22nd, 2024

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | xxxx | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on applying apply-IndicatedTCIState to PDCCH repetition under the Rel-18 unified TCI framework |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MIMO_evo_DL_UL |  |  |  |  |  | Date: |  |  | 2024-11-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | It has been agreed in RAN1#110-e that on unified TCI framework extension for S-DCI based MTRP in Rel-18, the RRC configuration – apply-IndicatedTCIState – used to inform the UE to apply the first, the second, both or none of the indicated joint/DL TCI states can be applied to PDCCH with repetition. Agreement On unified TCI framework extension for S-DCI based MTRP, to inform the association with the joint/DL TCI state(s) indicated by DCI/MAC-CE for PDCCH repetition, PDCCH-SFN, and PDCCH w/o repetition/SFN, support the following: - Use RRC configuration to inform that the UE shall apply the first one, the second one, both, or none of the joint/DL TCI states indicated by DCI/MAC-CE to a CORESET or a group of CORESETs (if CORESET group configuration is supported) Applying the RRC configuration apply-IndicatedTCIState to the PDCCH-SFN (when sfnSchemePdcch is configured) and PDCCH without repetition/SFN has been implemented in the corresponding specifications (the TS 38.213 and TS 38.214). However, applying the RRC configuration apply-IndicatedTCIState (when set to ‘first’ or ‘second’) for PDCCH with repetition is not captured in the corresponding specification (the TS 38.213). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that, the RRC configuration apply-IndicatedTCIState for PDCCH reception, can be set to ‘first’ and ‘second’ for PDCCH repetition. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unified TCI framework extension for S-DCI based MTRP in Rel-18 is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

10.1 UE procedure for determining physical downlink control channel assignement

< Unchanged parts are omitted >

For a CORESET with index 0,

-	if the UE is provided TCI-State and followUnifiedTCI-State for the CORESET, the UE assumes that a DM-RS antenna port for PDCCH receptions in the CORESET and a DM-RS antenna port for PDSCH receptions scheduled by DCI formats provided by PDCCH receptions in the CORESET are quasi co-located with the reference signals provided by the indicated TCI-State [6, TS 38.214]

-	else if the UE is provided dl-OrJointTCI-StateList and is indicated a first TCI-State and a second TCI-State, and apply-IndicatedTCIState for the CORESET

-	if the CORESET is associated with a Type 0/0A/2-PDCCH CSS set that has search space set index 0

-	if apply-IndicatedTCIState = 'first', the UE assumes that a DM-RS antenna port for PDCCH receptions in the CORESET is quasi co-located with the reference signals provided by the first TCI-State,

-	if apply-IndicatedTCIState = 'second', the UE assumes that a DM-RS antenna port for PDCCH receptions in the CORESET is quasi co-located with the reference signals provided by the second TCI-State,

-	if apply-IndicatedTCIState = 'none', the UE assumes that a DM-RS antenna port for PDCCH receptions in the CORESET is quasi co-located with the one or more DL RS configured by a TCI state, where the TCI state is indicated by a MAC CE activation command for the CORESET, if any

-	else

-	if apply-IndicatedTCIState = 'first', the UE assumes that a DM-RS antenna port for PDCCH receptions in the CORESET is quasi co-located with the reference signals provided by the first TCI-State,

-	if apply-IndicatedTCIState = 'second', the UE assumes that a DM-RS antenna port for PDCCH receptions in the CORESET is quasi co-located with the reference signals provided by the second TCI-State,

-	if apply-IndicatedTCIState = 'both', the UE assumes that a DM-RS antenna port for PDCCH receptions in the CORESET is quasi co-located with the reference signals provided by the first and the second TCI-State,

-	if apply-IndicatedTCIState = 'none', the UE assumes that a DM-RS antenna port for PDCCH receptions in the CORESET is quasi co-located with the one or more DL RS configured by a TCI state, where the TCI state is indicated by a MAC CE activation command for the CORESET.

If apply-IndicatedTCIState = ‘first’ or ‘second’ for the CORESET, the PDCCH receptions in the CORESET can be associated with a search space set configured with searchSpaceLinkingId.

-	else, the UE assumes that a DM-RS antenna port for PDCCH receptions in the CORESET is quasi co-located with

-	the one or more DL RS configured by a TCI state, where the TCI state is indicated by a MAC CE activation command for the CORESET, if any, or

-	a SS/PBCH block the UE identified during a most recent random access procedure not initiated by a PDCCH order that triggers a contention-free random access procedure, if no MAC CE activation command indicating a TCI state for the CORESET is received after the most recent random access procedure, or a SS/PBCH block the UE identified during a most recent configured grant PUSCH transmission as described in clause 19.

< Unchanged parts are omitted >

If a UE is provided dl-OrJointTCI-StateList and is indicated a first TCI-State and a second TCI-State, and is provided apply-IndicatedTCIState for a CORESET, other than a CORESET with index 0,

-	if the CORESET is associated only with USS sets and/or Type3-PDCCH CSS sets

-	if apply-IndicatedTCIState = ‘first’, the UE assumes that a DM-RS antenna port for PDCCH receptions in the CORESET is quasi co-located with the reference signals provided by the first TCI-State

-	if apply-IndicatedTCIState = ‘second’, the UE assumes that a DM-RS antenna port for PDCCH receptions in the CORESET is quasi co-located with the reference signals provided by the second TCI-State

-	if apply-IndicatedTCIState = ‘both’, the UE assumes that a DM-RS antenna port for PDCCH receptions in the CORESET is quasi co-located with the reference signals provided by the first TCI-State and the second TCI-State

-	if the CORESET is associated at least with CSS sets other than Type3-PDCCH CSS sets,

-	if apply-IndicatedTCIState = ‘first’, the UE assumes that a DM-RS antenna port for PDCCH receptions in the CORESET is quasi co-located with the reference signals provided by the first TCI-State

-	if apply-IndicatedTCIState = ‘second’, the UE assumes that a DM-RS antenna port for PDCCH receptions in the CORESET is quasi co-located with the reference signals provided by the second TCI-State

-	if apply-IndicatedTCIState = ‘both’, the UE assumes that a DM-RS antenna port for PDCCH receptions in the CORESET is quasi co-located with the reference signals provided by the first TCI-State and the second TCI-State

-	if apply-IndicatedTCIState = ‘none’, the UE assumes that a DM-RS antenna port for PDCCH receptions in the CORESET is quasi co-located with the one or more DL RS configured by a TCI state indicated by a MAC CE activation command for the CORESET

If apply-IndicatedTCIState = ‘first’ or ‘second’ for the CORESET, the PDCCH receptions in the CORESET can be associated with a search space set configured with searchSpaceLinkingId.

< Unchanged parts are omitted >