3GPP TSG RAN WG1 #119		R1-2409683

Orlando, US, November 18th – 22nd, 2024

Source:	vivo

Title:	Discussion on Frame structure, random access, scheduling and timing aspects for Ambient IoT

Agenda Item:	9.4.2.2

Document for:	Discussion and Decision

- Introduction

In this contribution, we discuss the remaining issues for frame structure and timing aspects for A-IoT, including the D2R time/frequency synchronization, frame structure, random access, scheduling and timing relationships.

- A-IoT Synchronization

  - Time synchronization for D2R transmission and reception

Following WA and agreements were made in RAN1#117 meeting [2]:

Working assumption on [0q] (sampling frequency) in link level simulation table Companies to report the Sampling frequency (e.g., 1.92Msps or other feasible values if any) Initial SFO (Sampling Frequency Offset) (Fe): - (M) Randomly select a value from the range of [0.1 ~ 1] *10^4 ppm for device 2, - (M) Randomly select a value from the range of [0.1 ~ 1] * 10^5 ppm for device 1, - (O) Randomly select a value from the range of [0.1 ~ 1] *10^5 ppm for device 2, - FFS: Optionally evaluate a fixed value SFO for device 1 and 2 - Note: For random selection, the value is randomly selected per simulation drop, according to a uniform distribution - Note: Above values are only for sampling purpose. - FFS other values - Note: Above assumptions are only for LLS evaluation purpose only for R2D and [D2R]. The timing drift ΔT over a time T is modelled as ΔT = ±Fe * T. - Note: Accuracy can be improved after clock calibration for at least device 2. FFS applicable for device 1 - Note: SFO after clock calibration can be applied to Fe. - FFS other models CFO for device 2b. - [100ppm/200ppm/1000ppm, 0.1ppm/s]” Note: Above assumptions are for LLS evaluation purpose only Agreement Study whether/how an A-IoT device can count the time with sufficient accuracy (with a certain timing error due to SFO) at least for the purposes related to TDM(A) (if needed), and if so for how long after receiving an R2D transmission.

In RAN1#118 meeting, there were some discussions regarding time synchronization for D2R transmission at device side and reception at the reader side. One key point is whether/how the SFO accuracy can be improved for device 1. The answer to this point is very important considering its impact on many aspects such as the R2D and D2R preamble design, whether and how many D2R midamble and/or postamble are needed for fine timing recovery, and the feasibility/benefits of FDMA/TDMA of multiple D2R transmissions etc, as summarized in [5].

There are two views/options:

- One view, Option 1 is the device transmits the D2R with the [initial] SFO (as agreed for LLS that the value for initial SFO (Sampling Frequency Offset) (Fe) is from the range of [0.1 ~ 1] * 10^5 ppm for device 1)

- The other view, Option 2 is the device transmits the D2R with improved SFO accuracy which is smaller than the ‘initial’ SFO

For option 1, based on discussions in the last meeting, device 1 calibrates the clock by for example updating the number of clock samples for a D2R chip/bit without changing the clock frequency, while the SFO accuracy does not improve much; or taking RFID as a reference, the SFO after clock calibration is in the range from 4% - 22% for different data rates. Considering the low cost/complexity of A-IoT device and high capability for gNB or UE as reader, device 1 is not required to improve the SFO accuracy and the reader should correct the SFO. Since the device is not required to improve the SFO, the R2D preamble design can be simpler and can be with short length to reduce device’s power consumption. While D2R preamble may need to be longer and/or D2R midamble and/or postamble is needed for reader to correct the timing for D2R reception/demodulation.

For option 2, the SFO accuracy is improved after device 1 calibrates the clock. To improve the SFO accuracy, a longer R2D preamble part may be needed and/or device can even use R2D data part to calibration its clock. However, it may increase device complexity and power consumption. From reader side, D2R postamble is not needed, which relaxes reader’s processing and buffer requirement since the reader does not need to store all the I/Q samples until SFO estimation is completed based on D2R preamble + D2R postamble. In addition, reader-side scheduling on FDMA of multiple D2R transmissions may be easier with improved SFO accuracy at the device side.

Table 2-1 outlines the pros and cons of each option. A proposal follows accordingly.

Table 2-1 Summary of pros and cons of each option on SFO accuracy for D2R transmission after device calibrates clock

| Options | Pros | Cons |
| --- | --- | --- |
| Option 1 | - Relaxed timing accuracy requirement for device - R2D preamble can be designed simpler and less overhead - Less complexity and/or power consumption for device to detect R2D preamble | - Strict timing accuracy requirement for reader - May complex D2R Preamble design with more overhead - D2R Midamble/Postamble may need to be introduced for SFO estimation - Increase reader side processing/buffering complexity if postamble is used - May reduce the efficiency for FDMA of D2R transmissions |
| Option 2 | - Loose timing accuracy requirement for reader side - D2R Preamble can be designed simpler and less overhead - D2R Midamble/Postamble may not be necessary for SFO estimation - Decrease reader side processing/buffering complexity without postamble - May enhance the efficiency for FDMA of D2R transmissions | - Strict timing accuracy requirement for device - May complex R2D Preamble design with more overhead - Higher complexity and/or power consumption for device to detect R2D preamble |

Proposal 2.1: For device 1, study following two options of time synchronization for D2R transmission and reception.

- Option 1: device transmits D2R with SFO of 10^5 ppm. Reader estimates the time/frequency error of the received D2R transmission for detection/demodulation/decoding.

  - FFS: how D2R transmission should be designed to facilitate reader’s estimation of the time/frequency error of the received D2R transmission

- Option 2: device transmits D2R with improved SFO smaller than 10^5 ppm. Reader estimates the time/frequency error of the received D2R transmission for detection/demodulation/decoding.

  - FFS how R2D signal should be designed to improve the SFO estimation accuracy and what is the achievable accuracy at the device side.

  - FFS how D2R transmission should be designed to facilitate reader’s estimation of the time/frequency error of the received D2R transmission.

  - Note: the complexity of calibrating the device clock should be minimized.

  - D2R x-amble(s)

Following agreement was made for A-IoT D2R preamble, midamble and postamble:

Agreement@RAN1#116bis For D2R transmission, study the necessity of midamble at least for the purpose of performing timing/frequency tracking or channel estimation or interference estimation, considering at least the following: - Modulation and Coding schemes, e.g., data modulation, line/channel coding - Receiving methods, e.g., coherent or non-coherent - D2R transmission length/packet size - Midamble overhead - Timing/frequency accuracy - Phase accuracy Agreement@RAN1#116bis For the reader to acquire the end of PDRCH transmission, study at least following options: - Option 1: D2R postamble immediately follows the PDRCH - Option 2: Based on control information Agreement@ RAN1#118 For D2R transmission, preamble preceding the PDRCH is studied also for the potential additional functionalities: - SFO estimation - CFO estimation - Channel estimation - Interference estimation Note: this does not preclude studying the above functionalities by using a midamble and/or postamble, if supported FFS: Other functionalities, if any Agreement@ RAN1#118bis - The TR will capture the following options, and companies are encouraged to analyze the tradeoffs among the following D2R amble(s) options: - Option 1: D2R preamble only - Option 2: D2R preamble + X midamble(s), where X 1 - Option 3: D2R preamble + postamble - Option 4: D2R preamble + Y midamble(s) + postamble, where Y1 - For the above options, companies are encouraged to report at least the following: - Purpose(s) of the preamble, midamble and postamble - Whether companies assume multiple options can be supported Agreement For analysing the trade-offs among the D2R amble(s) options, companies can refer to the Table 3.2.4 in section 3.2.4 of R1-2408993 for information.

It was agreed to study D2R preamble and D2R midamble for time/frequency tracking, channel estimation and interference estimation. However, there is no agreement yet to study using D2R postamble for reader to perform time/frequency tracking. During SI phase, it is valuable to study the condition(s) under which the PDRCH with D2R preamble only is sufficient or insufficient. For the insufficient case, additional “x-amble” besides the preamble is needed, then we can further analyze the amount and time-domain location of the required x-ambles. In our view, if the functionalit(ies) for the D2R midamble and D2R postamble are the same, just time domain location is different, it may not necessarily restrict that  one of the additional “x-amble” is always  placed at the end of the PDRCH for the insufficient case. Therefore, the agreement made in RAN1#118bis meeting categorizes the cases/options for a more structured analysis or observations.

Observation 2.2-1: If the D2R midamble and postamble serve identical functions, but are placed at different locations in the time domain, it is not necessary to use different terminologies and always place an additional "x-amble" at the end of the PDRCH.

In the following, we provide our evaluations and observations for the D2R xmable(s). First we provide our method on how D2R preamble, midamble/postamble is used for SFO estimation and channel estimation.

Amble(s) used for SFO estimation

Coarse SFO estimation by preamble: The received preamble part can be transformed to frequency domain through FFT. Since the D2R preamble is frequency shifted through on/off transitions to a target center frequency, the frequency position with the largest frequency response can be regarded as the estimated center frequency of the D2R transmission. The coarse SFO estimation is determined as the difference between the target center frequency and the frequency position with the largest frequency response.

Fine SFO estimation by preamble: To further improve the accuracy of SFO estimation, a set of SFO hypotheses based on coarse SFO estimation is used to perform time domain correlation for preamble part. For each hypothesis, a local preamble sequence is generated with different number of samples per chip according to the assumed SFO, and this local preamble sequence is correlated with the received signal to obtain the timing of the D2R transmission. The branch with the closest SFO hypothesis is expect to have the maximum correlation value among the branches. And the closest SFO is considered as the the fine SFO, which is used to determine the number of samples per chip for the follow up D2R detection.

Timing sync by preamble: Timing is obtained through correlation together with the fine SFO estimation above.

(a)_Coarse SFO estimation on preamble                       (b) Fine SFO estimation on preamble

Figure 2.2-1 Coarse and fine SFO estimation on D2R preamble

In case PDRCH transmission with additional amble(s) such as midamble/postamble, the SFO estimation and timing can be further updated/refined based the additional amble(s) as follows:

Timing sync based on midamble/postamble: Time domain correlation using midamble or postamble, and derive an updated timing of current midamble/postamble.

Fine SFO estimation on midamble/postamble: Time domain correlation using midamble or postamble, and estimate SFO by comparing the time interval between timing obtained from current midamble/postamble and the timing from previous preamble/midamble. Update the SFO assumption of the part after the preamble part (or previous midamble part).

Figure 2.2-2 update SFO and timing estimation on D2R midamble/postamble

Amble(s) used for channel estimation

From channel estimation perspective, it is necessary for coherent receiver to remove channel from received signal to recover the D2R signal. At least for short D2R transmission, it is natural and simple to just use D2R preamble for channel estimation. For long D2R transmission length, D2R midamble may be needed in addition to preamble or alternatively, non-coherent detection can be used to avoid large overhead caused by sufficient number of midamble required for coherent detection. Details on Non-Coherent detection can be found in Annex B.

For Coherent detection, following steps are used:

- Step 1: LS estimation on preamble/midamble/postamble and MMSE filtering within in X-amble duration

- Step 2: Get Channel estimation on data resources through MMSE filtering across X-ambles

The data part is detected based on the estimated channel and SFO discussed above.

To summarize, Table 2.2-1 provides an overview of the detection procedure for D2R with various amble options.

Table 2.2-1 Summary of receiver processing for D2R w/ and w/o midamble/postamble

|  | D2R w/o midamble/postamble | D2R w/ midamble/postamble |
| --- | --- | --- |
| Coarse SFO estimation | Preamble part is transformed to frequency domain to obtain coarse SFO estimation. |  |
| Fine SFO estimation based on preamble | A set of SFO hypothesis based on coarse SFO estimation used to perform time domain correlation for preamble part. Fine SFO estimation is obtained through select the SFO hypothesis with highest correlation value. |  |
| Fine SFO estimation based on midamble and/or postamble | N/A | Timing domain correlation using midamble or postamble, and update SFO by comparing the time interval between timing obtained from current midamble/postamble and previous preamble/midamble. |
| Timing/sync on preamble | Obtained through time domain correlation together with Fine SFO estimation |  |
| Timing/sync on midamble and/or postamble | N/A | Timing domain correlation using midamble or postamble, and update sync by timing obtained from current midamble/postamble |
| Channel estimation(Only for coherent detection) | N/A for non-coherent detection | Step 1: Channel estimation on preamble/midamble/postamble and MMSE filtering within in X-amble Step 2: Get Channel estimation on data samples through MMSE filtering across X-ambles |
| Detection | Non-coherent (preamble is only used to estimate SFO) is more appropriate | Coherent detection (Refer to Annex B) is more appropriate |

The following options are evaluated for different message size with data rate of 5~7 kbps. The basic evaluation assumption can be found in Annex A.

- Option 1: D2R preamble only

- Option 2: D2R preamble + X midamble(s), where X 1

- Option 3: D2R preamble + postamble

- Option 4: D2R preamble + Y midamble(s) + postamble, where Y1

- Ambles used for sync/SFO estimation

Compared to Option 1 of D2R preamble only case (D2R w/o midamble/postamble), other options using additional amble(s) i.e., midamble/postamble besides preamble are expected to improve the SFO estimation accuracy and/or reduce the number of SFO hypotheses for preamble. This is because SFO estimation can be updated based on timing estimation from current midamble/postamble and previous preamble/midamble.

To isolate the effects of channel estimation and assess the benifits obtained from SFO update and timing update based on additional amble(s), non-coherent detection at receiver side is considered in the evaluation such that midamble/postamble is only used for SFO and timing estimation. Channel estimation is not needed for non-coherent detection. The SFO estimation accuracy for different amble options with device side SFO assumptions of [0.1 - 1] * 10^5 ppm and [0.1 - 1] * 10^4 ppm are evaluated for PRDCH with palyload size of 20, 96 and 400 bits. From the evaluation results, similar observations can be made for PDRCH with different payload size. For ease of observation and anaysis, we present the evaluation results for medium payload (96-bit PDRCH) w/ and w/o midamble/postamble with different number of SFO hypotheses in Figure 2.2-3. The evaluation results for small payload size (20-bit PDRCH) and large payload size (400-bit PDRCH) can be found in Annex C.

(a)  Option-1 (preamble only), SFO=10^4 ppm       (b)   Option-1(preamble only), SFO=10^5 ppm

(c) Option-2 (preamble+1 midamble), SFO=10^4 ppm      (d) Option-2 (preamble+1 midamble), SFO=10^5 ppm

(e) Option-3 (preamble+1 postamble), SFO=10^4 ppm     (f) Option-3(preamble+1 postamble), SFO=10^5 ppm

Figure 2.2-3 SFO estimation accuracy for 96-bit PDRCH for different amble options

In our evaluation, the required number of SFO hypotheses for each amble option is determined by increasing the number until performance improvement is less than 1 dB in BLER. Table 2.2-2, 2.2-3 and 2.2-4 summarize the required number of SFO hypothesis and SNR to achieve 1% and 10% BLER for 20-bit, 96-bit and 400-bit PDRCH respectively.

Table 2.2-2: SNR and SFO hypothesis needed to achieve (1%, 10%) BLER for 20-bit PDRCH using non-coherent detection

| Ideal timing, w/o SFO | non-ideal timing SFO = [0.1 – 1] * 10^4 ppm |  |  | non-ideal timing SFO = [0.1 – 1] * 10^5 ppm |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Option 1 | Option 2 | Option 3 | Option 1 | Option 2 | Option 3 |
| 10%: 0 dB 1%: 5.1 dB | 4 hypotheses 10%: 0.2 dB 1%: 5.3 dB | 2 hypotheses 10%: 0.1 dB 1%: 5.2 dB | 2 hypotheses 10%: 0 dB 1%: 5.3 dB | 8 hypotheses 10%: 0.2 dB 1%: 5.7 dB | 4 hypotheses 10%: 0.2 dB 1%: 5.7 dB | 6 hypotheses 10%: 0.2 dB 1%: 5.7 dB |

Table 2.2-3: SNR and SFO hypothesis needed to achieve (1%, 10%) BLER for 96-bit PDRCH using non-coherent detection

| Ideal timing, w/o SFO | non-ideal timing SFO = [0.1 – 1] * 10^4 ppm |  |  | non-ideal timing SFO = [0.1 – 1] * 10^5 ppm |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Option 1 | Option 2 | Option 3 | Option 1 | Option 2 | Option 3 |
| 10%: 0.3 dB 1 % : 6 dB | 6 hypotheses 10%: 0.5 dB 1%: 6.5 dB | 4 hypotheses 10%: 0.4 dB 1%: 6.6 dB | 4 hypotheses 10%: 0.4 dB 1%: 6.2 dB | 10 hypotheses 10%: 0.7 dB 1%: 7.3 dB | 8 hypotheses 10%: 0.7 dB 1%: 7.3 dB | 8 hypotheses 10%: 0.7 dB 1%: 7.3 dB |

Table 2.2-4: SNR and SFO hypothesis needed to achieve (1%, 10%) BLER for 400-bit PDRCH using non-coherent detection

| Ideal timing, w/o SFO | non-ideal timing SFO = [0.1 – 1] * 10^4 ppm |  |  | non-ideal timing SFO = [0.1 – 1] * 10^5 ppm |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Option 1 | Option 2 | Option 4 | Option 1 | Option 2 | Option 4 |
| 10%: 2 dB 1%: 8.9 dB | 8 hypotheses 10%: 2.3 dB 1%: 9.1 dB | 6 hypotheses 10%: 2.3 dB 1%: 9.1 dB | 6 hypotheses 10%: 2.3 dB 1%: 8.9 dB | 12 hypotheses 10%: 2.76 dB 1%: 10.1 dB | 8 hypotheses 10%: 2.76 dB 1%: 9.1 dB | 8 hypotheses 10%: 2.76 dB 1%: 10.1 dB |

It can be observed that under the same device SFO assumption and for a given PDRCH payload, the difference between required number of SFO hypothesis for different options is not remarkable. In detail, for device SFO = [0.1 – 1] * 10^5 ppm and 96-bit PDRCH, Opt.1 requires 10 SFO hypothesis and Opt.2 and Opt.3 require 8 SFO hypothesis to achieve similar BLER performance. However, more overhead and transmission power consumption are introduced by the additional amble(s).

Proposal 2.2-1: Capture the following into the TR.

Following is observed by source [R1-2409683] for non-coherent detection of 20-bit PDRCH with 6-bit CRC and ~5 kbps data rate, using 1/2 manchester encoding and 1/3 convolutional code:

  - With up to 10% device SFO for D2R transmission, the SNR needed to achieve 10% and 1% BLER is around 0.2dB and 5.7 dB for the three options: option 1 of D2R preamble only, option 2 of D2R preamble + 1 midamble and option 3 of D2R preamble + 1 postamble.

    - Option 1 requires reader to test 8 SFO hypotheses, the overhead of D2R amble is 45.07%.

    - Option 2 requires reader to test 4 SFO hypotheses, the overhead of D2R amble is 55.17%.

    - Option 3 requires reader to test 6 SFO hypotheses, the overhead of D2R amble is 55.17%.

  - With up to 1% device SFO for D2R transmission, the SNR needed to achieve 10% and 1% BLER is about 0.1dB and 5.2dB for the three options: option 1 of D2R preamble only, option 2 of D2R preamble + 1 midamble and option 3 of D2R preamble + 1 postamble.

    - Option 1 requires reader to test 4 SFO hypotheses, the overhead of D2R amble is 45.07%.

    - Option 2 requires reader to test 2 SFO hypotheses, the overhead of D2R amble is 55.17%.

    - Option 3 requires reader to test 2 SFO hypotheses, the overhead of D2R amble is 55.17%.

Following is observed by source [R1-2409683] for non-coherent detection of 96-bit PDRCH with 16-bit CRC and 5~6 kbps data rate, using 1/2 manchester encoding and 1/3 convolutional code:

  - With up to 10% device SFO for D2R transmission, the SNR needed to achieve 10% and 1% BLER is around 0.7dB and 7.3 dB for the three options: option 1 of D2R preamble only, option 2 of D2R preamble + 1 midamble and option 3 of D2R preamble + 1postamble.

    - Option 1 requires reader to test 10 SFO hypotheses, the overhead of D2R amble is 16%.

    - Option 2 requires reader to test 8 SFO hypotheses, the overhead of D2R amble is 22.22%.

    - Option 3 requires reader to test 8 SFO hypotheses, the overhead of D2R amble is 22.22%.

  - With up to 1% device SFO for D2R transmission, the SNR needed to achieve 10% and 1% BLER is 0.4dB and 6.5dB for the three options: option 1 of D2R preamble only, option 2 of D2R preamble + 1 midamble and option 3 of D2R preamble + 1postamble.

    - Option 1 requires reader to test 6 SFO hypotheses, the overhead of D2R amble is 16%.

    - Option 2 requires reader to test 4 SFO hypotheses, the overhead of D2R amble is 22.22%.

    - Option 3 requires reader to test 4 SFO hypotheses, the overhead of D2R amble is 22.22%.

Following is observed by source [R1-2409683] for non-coherent detection of 400-bit PDRCH with 16-bit CRC and 5~6 kbps data rate, using 1/2 manchester encoding and 1/3 convolutional code:

  - With up to 10% device SFO for D2R transmission, the SNR needed to achieve 10% and 1% BLER is around 2.7dB and 10dB for the three options: option 1 of D2R preamble only, option 2 of D2R preamble + 5 midambles and option 4 of D2R preamble + 4 midambles+1 postamble.

    - Option 1 requires reader to test 12 SFO hypotheses, the overhead of D2R amble is 4.88%.

    - Option 2 requires reader to test 8 SFO hypotheses, the overhead of D2R amble is 15.22%.

    - Option 4 requires reader to test 8 SFO hypotheses, the overhead of D2R amble is 15.22%.

  - With up to 1% device SFO for D2R transmission, the SNR needed to achieve 10% and 1% BLER is around 2.3dB and 9dB for the three options: option 1 of D2R preamble only, option 2 of D2R preamble + 5 midambles and option 4 of D2R preamble + 4 midambles+1 postamble.

    - Option 1 requires reader to test 8 SFO hypotheses, the overhead of D2R amble is 4.88%.

    - Option 2 requires reader to test 6 SFO hypotheses, the overhead of D2R amble is 15.22%.

    - Option 4 requires reader to test 6 SFO hypotheses, the overhead of D2R amble is 15.22%.

Proposal 2.2-2: For non-coherent detection of PDRCH with payload size of 20, 96 and 400 bits,

- The number of SFO hypotheses and the SNR needed for 10% and 1% BLER cannot significantly be reduced for the option(s) of a D2R with preamble and additional amble(s) compared to the option 1 of a D2R with preamble only. Moreover, the addition of midamble(s) and/or postamble introduces about 10% more overhead.

- For option 2 without a D2R postamble and option 3/option 4 with a D2R postamble, the number of SFO hypotheses and the SNR needed for 10% and 1% BLER remains almost unchanged with the same amble overhead. However, D2R postamble prevents pipelined processing of the reception, as the receiver side processing must wait until after SFO estimation and correction.

Proposal 2.2-3: For PDRCH using non-coherent detection, option 1 of D2R preamble is sufficient to achieve 10% and 1% BLER, with no more than 12 SFO hypotheses tested at the reader side.

- Ambles used for Channel estimation

As described in Annex A, coherent detection operates under the principle that only real part is used to determine Manchester symbol 0 and Manchester symbol 1. This method neglects the imaginary part of the noise, thereby retaining the desired signal and consequently the noise suppression is achieved in coherent detection, if accurate channel estimation can be ensured. Coherent detection relies on accurate channel estimation, which may requires additional amble overhead at transmitter and complexity on channel estimation.

In the following, we present the evaluation results in Figure 2.2-4 for coherent detection for PRDCH with palyload size of 20, 96 and 400 bits for different amble options with device side SFO assumptions of [0.1 - 1] * 10^5 ppm and [0.1 - 1] * 10^4 ppm.

(a) with SFO assumption of [0.1 - 1] * 10^4 ppm          (b) with SFO assumption of [0.1 - 1] * 10^5 ppm

Figure 2.2-4 D2R BLER performance for different amble options using coherent detection

Table 2.2-5, 2.2-6 and 2.2-7 summarize the required number of SFO hypothesis and SNR to achieve 1% and 10% BLER for 20-bit, 96-bit and 400-bit PDRCH respectively.

Table 2.2-5: SNR and SFO hypothesis needed to achieve (1%, 10%) BLER for 20-bit PDRCH using coherent detection

| Ideal timing, w/o SFO | non-ideal timing SFO = [0.1 – 1] * 10^4 ppm |  |  | non-ideal timing SFO = [0.1 – 1] * 10^5 ppm |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Option 1 | Option 2 | Option 3 | Option 1 | Option 2 | Option 3 |
| Option 1 - 10%: -5 dB - 1%: 0.6 dB Option 2 - 10%: -5 dB - 1%: 0.4 dB Option 3 - 10%: -5.1 dB - 1%: 0 dB | 4 hypotheses 10%: -2.5 dB 1%: 3.6 dB | 2 hypotheses 10%: -2.8 dB 1%: 3 dB | 2 hypotheses 10%: -2.8 dB 1%: 3.3 dB | 8 hypotheses 10%: -1.9 dB 1%: 4.2 dB | 6 hypotheses 10%: -2.1 dB 1%: 3.7 dB | 6 hypotheses 10%: -2.1 dB 1%: 3.8 dB |

Table 2.2-6: SNR and SFO hypothesis needed to achieve (1%, 10%) BLER for 96-bit PDRCH using coherent detection

| Ideal timing, w/o SFO | non-ideal timing SFO = [0.1 – 1] * 10^4 ppm |  |  | non-ideal timing SFO = [0.1 – 1] * 10^5 ppm |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Option 1 | Option 2 | Option 3 | Option 1 | Option 2 | Option 3 |
| Option 1 - 10%: -3.8 dB - 1%: 8.6 dB Option 2 - 10%: -4.4 dB - 1%: 2 dB Option 3 - 10%: -4.4 dB - 1%: 2 dB | 6 hypotheses 10%: -1.3 dB 1%: 11 dB | 6 hypotheses 10%: -3.1 dB 1%: 3.6 dB | 4 hypotheses 10%: -2.5 dB 1%: 4.5 dB | 10 hypotheses 10%: 0.7 dB 1%: 10 dB | 10 hypotheses 10%: -1.8 dB 1%: 5.1 dB | 10 hypotheses 10%: -1.7 dB 1%: 5.2 dB |

Table 2.2-7: SNR and SFO hypothesis needed to achieve (1%, 10%) BLER for 400-bit PDRCH using coherent detection

| Ideal timing, w/o SFO | non-ideal timing SFO = [0.1 – 1] * 10^4 ppm |  |  | non-ideal timing SFO = [0.1 – 1] * 10^5 ppm |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Option 1 | Option 2 | Option 4 | Option 1 | Option 2 | Option 4 |
| Option 1 - error floor Option 2 - 10%: -2.3 dB - 1%: 7 dB Option 4 - 10%: -2.1 dB - 1%: 8 dB | error floor | 6 hypotheses 10%: -1.2 dB 1%: 7.8 dB | 6 hypotheses 10%: -1.2 dB 1%: 9.1 dB | error floor | 8 hypotheses 10%: 0.2 dB 1%: 9.2 dB | 8 hypotheses 10%: 0.2 dB 1%: 12.8 dB |

For conherent detection, it is observed that for 20-bit PDRCH, with ideal SFO estimation i.e., without SFO, the SNR difference between Option 1 and Option 2/Option 3 is only 0.1dB and 0.6dB for 10% and 1% BLER. With SFO up to 10^4 ppm or 10^5 ppm, the SNR difference between Option 1, Option 2/Option 3 is around 0.3dB and 0.5dB for 10% and 1% BLER. While only additional 2 SFO hypothesis number is required for Option 1. Therefore, for transmission with short duration, the total length for PDRCH is limited, then the channel coefficients on data chips can be well derived from only preamble, and additional amble(s) e.g., midamble/postamble is not needed.

For 96-bit and 400-bit PDRCH, similar BLER performance and hypothesis number can be achieved when the amble overhead is the same. However, as discussed above, the options with postamble, i.e, Option 3 for 96-bit PDRCH and Option 4 for 400-bit PDRCH, disable pipelined processing of the reception, as the receiver-side processing can only be performed after SFO estimation and correction.

Proposal 2.2-4: Capture following into the TR.

Following is observed by source [R1-2409683] for coherent detection of 20-bit PDRCH with 6-bit CRC and ~5 kbps data rate, using 1/2 manchester encoding and 1/3 convolutional code:

- With up to 10% device SFO for D2R transmission, the SNR needed to achieve 10% and 1% BLER is around -2dB and 4 dB for the three options: option 1 of D2R preamble only, option 2 of D2R preamble + 1 midamble and option 3 of D2R preamble + 1postamble.

  - Option 1 requires reader to test 8 SFO hypotheses, the overhead of D2R amble is 45.07%.

  - Option 2 requires reader to test 6 SFO hypotheses, the overhead of D2R amble is 55.17%.

  - Option 3 requires reader to test 6 SFO hypotheses, the overhead of D2R amble is 55.17%.

- With up to 1% device SFO for D2R transmission, the SNR needed to achieve 10% and 1% BLER is around -2.8dB and 3.3dB for the three options: option 1 of D2R preamble only, option 2 of D2R preamble + 1 midamble and option 3 of D2R preamble + 1postamble.

  - Option 1 requires reader to test 4 SFO hypotheses, the overhead of D2R amble is 45.07%.

  - Option 2 requires reader to test 2 SFO hypotheses, the overhead of D2R amble is 55.17%.

  - Option 3 requires reader to test 2 SFO hypotheses, the overhead of D2R amble is 55.17%.

Following is observed by source [R1-2409683] for coherent detection of 96-bit PDRCH with 16-bit CRC and 5~6 kbps data rate, using 1/2 manchester encoding and 1/3 convolutional code:

- With up to 10% device SFO for D2R transmission, the SNR needed to achieve 10% and 1% BLER is around 0.7dB and 10dB for option 1 of D2R preamble only; and around -1.7dB and 5.2dB for option 2 of D2R preamble + 1 midamble and option 3 of D2R preamble + 1postamble.

  - Option 1 requires reader to test 10 SFO hypotheses, the overhead of D2R amble is 16%.

  - Option 2 requires reader to test 10 SFO hypotheses, the overhead of D2R amble is 22.22%.

  - Option 3 requires reader to test 10 SFO hypotheses, the overhead of D2R amble is 22.22%.

- With up to 1% device SFO for D2R transmission, the SNR needed to achieve 10% BLER is around -1.3dB for option 1 of D2R preamble only, -3.1dB for option 2 of D2R preamble + 1 midamble and -2.5dB for option 3 of D2R preamble + 1 postamble; the SNR needed to achieve 1% BLER is around 11dB for option 1 of D2R preamble only, 3.6dB for option 2 of D2R preamble + 1 midamble and 4.5dB for option 3 of D2R preamble + 1postamble.

  - Option 1 requires reader to test 6 SFO hypotheses, the overhead of D2R amble is 16%.

  - Option 2 requires reader to test 6 SFO hypotheses, the overhead of D2R amble is 22.22%.

  - Option 3 requires reader to test 4 SFO hypotheses, the overhead of D2R amble is 22.22%.

Following is observed by source [R1-2409683] for coherent detection of 400-bit PDRCH with 16-bit CRC and 5~6 kbps data rate, using 1/2 manchester encoding and 1/3 convolutional code:

- 10% BLER cannot be achieved by option 1 of D2R preamble only, and error floor is observed.

- With up to 10% device SFO for D2R transmission, the SNR needed to achieve 10% BLER is around 0.2dB for option 2 of D2R preamble+5 midambles and option 4 of D2R preamble+4 midambles+1 postamble; the SNR needed to achieve 1% BLER is around 9.2dB for option 2 of D2R preamble+5 midambles, 12.8dB for option 4 of D2R D2R preamble + 4 midambles+1 postamble.

  - Option 2 requires reader to test 8 SFO hypotheses, the overhead of D2R amble is 15.22%.

  - Option 4 requires reader to test 8 SFO hypotheses, the overhead of D2R amble is 15.22%.

- With up to 1% device SFO for D2R transmission, the SNR needed to achieve 10% BLER is around -1.2dB for option 2 of D2R preamble+5 midambles and option 4 of D2R preamble+4 midambles+1 postamble; the SNR needed to achieve 1% BLER is around 7.8dB for option 2 of D2R preamble+5 midambles, 9.1dB for option 4 of D2R preamble + 4 midambles+1 postamble.

  - Option 2 requires reader to test 6 SFO hypotheses, the overhead of D2R amble is 15.22%.

  - Option 4 requires reader to test 6 SFO hypotheses, the overhead of D2R amble is 15.22%.

Observation 2.2-2: For coherent detection of PDRCH with small payload and/or short transmission duration such as 20-bit PDRCH with 6-bit CRC and ~5 kbps data rate, using 1/2 manchester encoding and 1/3 convolutional code, option 1 of D2R preamble only is sufficient to achieve 10% and 1% BLER, with no more than 8 SFO hypotheses tested at the reader side.

Observation 2.2-3: For coherent detection of PDRCH with large payload and/or long transmission duration such as 400-bit PDRCH with 16-bit CRC and 5~6kbps data rate, using 1/2 manchester encoding and 1/3 convolutional code, option 1 of D2R preamble only cannot achieve 10% BLER. With additional 5 ambles, 1% BLER can be achieved.

Observation 2.2-4: For option 2 without a D2R postamble and option 3/option 4 with a D2R postamble, with the same amble overhead, the SNR needed for 10% and 1% BLER is slightly lower for option 2 compared to option 3/option 4 for coherent detection. Moreover, D2R postamble prevents pipelined processing of the reception, as the receiver side processing must wait until after SFO estimation and correction.

Based on above, if additional ‘amble’ is needed for the timing correction and channel estimation, it should not be placed at the end of the PDRCH.

Proposal 2.2-5: For timing correction and channel estimation, if the D2R preamble alone is insufficient for certain cases, such as PDRCH with long transmission durations, it is recommended to use one or more D2R midambles within the PDRCH instead of a D2R postamble located at the end of the PDRCH.

  - Carrier and frequency synchronization for device 2b

Following agreement was made in RAN1#118bis meeting

Agreement

For clock purpose #5, adopt following update.

| # | Purpose | Clock speed | Applicable Device types | Power consumption | Initial clock Accuracy (ppm) | Accuracy after clock sync / calibration at device side (ppm) |
| --- | --- | --- | --- | --- | --- | --- |
| #5 | LO for carrier frequency (for up/down conversion) | According to carrier frequency e.g., 900 MHz | 2b | 10s ~ 100s uW | [10^23 ~ 10^4] | Option 1: 10s (Note2) Option 2: 100s (Note2) |
| Note 2: Either internal clock counter for calibration clock counting based on at least R2D signals or clock counting based on transmission from reader Calibration is based on existing or new signal from reader. FFS how to calibrate (9.4.2.2). No drastic temperature change is assumed during calibration. |  |  |  |  |  |  |

- Further study above options and strive to down select considering its implication on performance, system design, necessity, etc.

- Note: study of whether additional signal is necessary or not for calibration is discussed under agenda 9.4.2.2.

In the following, we provide our views on whether additional signal is necessary or not for device 2b to do carrier/frequency calibration.

Injection locking is a technique where an external signal is injected into an oscillator to control its frequency. When the frequency of the injected signal is close to the natural frequency of the oscillator, the oscillator will lock onto the frequency of the injected signal. This injection locking effect can be leveraged to calibrate and stabilize the frequency of the oscillator and widely used for lower power transceivers. In [6], the frequency of the transceiver at ~900MHz can be calibrated to ~100 ppm with 300uw power consumption. Considering the locking range is -/+2.5MHz, the initial frequency error can be assumed as a few thousands ppm. And the required time duration for the injection external signal is 50us to calibrate the frequency drift in [6].

Depending on the length required for calibration, the external signal for frequency calibration for device 2b can be either embedded in exisiting R2D signal, or transmitted as additional signal. For example, if ON chip duration of existing R2D transmission is long enough, the reference frequency can be provided by ON chip of existing R2D transmission, e.g., start indicator part or clock acquisition part in preamble, as shown in Figure 2.3-1. However, if the time duration for reference frequency is too long to be embedded in existing R2D signal, additional sync signal would be needed. Since the D2R transmission must be scheduled by R2D, periodic sync signal for device 2b is not needed. Instead, an aperiodic sync signalcan be transmitted along with exsiting R2D signal, e.g., after the R2D transmission that triggers D2R transmission. As for the frequency resource of the external signal, it can be up to reader to determine the frequency the AIoT device 2b is expected to be locked on, as long as the frequency is in the locking range of the oscillator.

Figure 2.3-1 Example of potential resource(s) for injection signal as part of existing R2D signal for device 2b frequency calibration

Observation 2.3-1: An external signal which provides a reference frequency can be leveraged to calibrate and stabilize the frequency of the oscillator for device 2b. Depending on the time duration required for the external signal for frequency calibration, the signal can be embedded in existing R2D signal or transmitted as additional signal.

- If the ON chip duration of the current R2D transmission is sufficient, the reference frequency can be provided by ON chip(s) of current R2D transmission, such as in the start indicator part or clock acquisition part in preamble;

- Otherwise, the additional sync signal can be transmitted along with the current R2D transmission,  after the current R2D transmission that triggers D2R transmission.

- A-IoT Frame structure

For D2R transmission, let’s first review the uplink transmission in NR system. NR is a synchronized system, each UE maintains the synchronization with the gNB and knows about the frame/subframe/slot/OFDM symbol index. As shown in TS 38.211 Figure 4.3.1-1, in a given frame/subframe/slot/OFDM symbol with index i, when a UE performs uplink transmission, the UE is required to adjust its uplink transmission timing by applying timing advance (TA) with reference to the downlink frame/subframe/slot/OFDM symbol with index i, so that the received uplink transmissions at gNB side is aligned with downlink frame/subframe/slot/OFDM symbol with index i. Therefore, in NR, from the transmitter i.e., UE perspective, after applying the TA, the actual uplink transmission is not aligned with the NR OFDM symbol boundary. But from the receiver, i.e., gNB perspective, the actual reception is aligned to the NR OFDM symbol boundary within the CP range.

In A-IoT system, it is a kind of asyncronized system, different from NR UE, the A-IoT device is not able to know and maintain the NR frame/subframe/slot/OFDM symbol index. In addition, considering the short communication range (~10m - ~50m), the AIoT device does not apply or maintain a timing advance. Therefore, from the transmitter i.e., device and the receiver i.e., Reader perspective, the start of D2R transmission or reception cannot be assumed to be aligned with NR OFDM symbol boundary within the CP range.

Proposal 3-1: A-IoT device is not required to acquire the NR frame/subframe/slot/OFDM symbol index.

Proposal 3-2: For D2R transmission, A-IoT device does not apply and maintains the timing advance (TA).

For both R2D and D2R transmission, an OOK symbol/chip length can be defined as the basic unit of time-domain resource allocation for A-IoT R2D and D2R transmission. It is also desirable to support integer multiple numbers of OOK symbols within a slot for efficient resource usage and enable slot-level TDMed multiplexing between NR and AIoT. The detailed design for an OOK symbol duration also depends on target data rate, waveform/modulation, and tolerance to timing variation (e.g., timing error, or channel variation).

Proposal 3-3: From the A-IoT device side, an OOK symbol/chip length can be considered as the basic resource unit for constructing A-IoT DL/UL channel/signals.

- Energy harvest on device availability

In the last RAN1 meeting, the TP capturing the solutions for the agreed two directions was endorsed in principle [7]. Given this is the last meeting for Rel-19 AIoT, following conclusions are proposed:

Proposal 4-1: Capture following conclusion for Section 8 of TR38.769:

Conclusion: RAN1 has studied the potential impact of energy harvesting on device availability for transmission and reception procedures and identified the following two directions. The solutions for direction 1 and direction 2 are captured in the Table 6.2.1-1 and Table 6.2.2-1.

- Direction 1: Reader does not provide information to a device regarding when the device may become available/unavailable.

- Direction 2: Reader can provide information to a device based on which the device may become available/unavailable.

Proposal 4-2: Conclude that for the purpose of the Rel-19 study, the study on the potential impact of energy harvesting on device availability for transmission and reception procedures in RAN1 is considered completed based on the above conclusion.

- Resource allocation for A-IoT Random access

  - TDMA and/or FDMA of Msg1/Msg3 transmissions

For Msg1 and Ms3 resource allocation, the following agreements was made in RAN1#118 and #118bis meetings [3] – [4]:

RAN1#118 meeting: Agreement Study FDMA of D2R transmissions for Msg.1 from multiple devices in response to a R2D transmission triggering random access, including following - How the frequency domain resources are allocated for the FDMA of D2R transmissions for Msg.1 - How a device determines the frequency domain resource for the D2R transmissions for Msg.1 Note: this does not preclude discussion on TDMA for D2R transmissions for Msg.1 RAN1#118bis meeting: Agreement RAN1 studies following: - A R2D transmission triggering random access determines X time domain resource(s) for D2R transmission(s) for Msg1, where each D2R transmission for Msg1 occurs in one time domain resource of the X time domain resource(s). - The study includes - Study X=1 and X>1 and X>=1, the maximum value of X>1 should be set considering the device implementation complexity, device power consumption, the resource usage efficiency affected at least by SFO, and inventory latency. - Size(s) for resource allocation in the time domain - Determination of the X time domain resource(s) by the device - Addressing timing errors for adjacent time domain resources due to residual SFO of the device Agreement Study FDMA and/or TDMA of D2R transmissions for Msg3 from multiple devices in response to a given set of one or multiple Msg2 transmission(s) during access procedure, including following - How the frequency and time domain resources are allocated for the FDMA and/or TDMA of D2R transmissions for Msg3

In addition, it is notied that RAN2 made following agreements on Random access at RAN2#127bis meeting

Agreements for Random Access - In case of D2R data transmission failure, device follows the reader instruction: - it is supported to re-access in another opportunity controlled/provided by the reader (i.e. retry the random access); and - reader can repeat the R2D “upper layer command” to trigger the device to re-send the same D2R “response” (i.e., device just follows the received R2D to transmit D2R). - Support optional explicit R2D failure/success feedback indication for at least MSG3 for re-access purpose. FFS for following D2R data. - For 2step CBRA, if mgs2 is not received by the device, the device is not expected to autonomously re-access. The re-access is always controlled by reader. - For 2step and 3step CBRA, The A-IoT device performs re-access in another opportunity provided by the reader (i.e. retry the random access) at least in case of contention resolution failure. - R2D message is used to provide access occasion(s) which can be used for re-access purpose. One option to be captured in the TR is that this R2D message is AIoT paging message. FFS additional indications to differentiate if needed in paging message. FFS if other message can be used. - From RAN2 perspective, at least for TDMA the device can randomly select one access occasion for A-IoT Msg1 within the access occasions provided/assigned by the reader, as the baseline for CBRA. FFS if this is applicable to FDMA. Further enhancement option(s) can be considered after more RAN1 progress on TDMA/FDMA.

About how device selects one of multiple TDMed and/or FDMed access occasions should be discussed in RAN2. RAN1 focus on the resource allocations and timing relations for random access.

Frequency domain resource allocation for a D2R transmission in terms of frequency location and transmission bandwidth

For the frequency location,

- For device 2b with internally generated D2R transmission, it is straightforward to indicate the different FDMed frequency resources for different A-IoT devices for D2R transmissions by R2D control information.

- For device 1/2a with D2R backscattered on a CW, different frequency resources for different A-IoT devices can be achieved by allocating different small frequency shifts in R2D control information [8].

For the transmission bandwidth, it is determined by the D2R data rate which can be indicated/determined by the R2D control information.

The transmission bandwidth for multiple FDMed Msg.1 transmissions scheduled by one R2D transmission triggering random access should be the same and indicated explicitly or implicitly. However, Different from Msg1's contention-based transmission, Msg3 is effectively treated as contention-free or scheduling-based because the device has already been identified by the reader during the contention resolution process in Msg2. Therefore, the frequency domain transmission location should be assigned to each device by the parameter related to small frequency shift. Further discussion is needed on whether the transmission bandwidth is the same or can be different for the FDMed resources for Msg3 transmissions in response to a Msg.2 transmission.

Proposal 5.1-1: For FDMA of multiple Msg.1 transmissions in response to a R2D transmission triggering random access,

- The transmission locations in the frequency domain, i.e., small frequency shift, FSx are different for the FDMed resources for Msg1 transmissions.

- The transmission bandwidth, i.e., Btx,D2R is the same for all the FDMed resources for Msg1 transmissions.

Proposal 5.1-2: For FDMA of multiple Msg.3 transmissions in response to a given set of one or multiple Msg2 transmission(s) during access procedure,

- At least the frequency domain transmission locations should be assigned in device specific manner by the Msg2 transmission.

- FFS the transmission bandwidth is allocated either specifically for each device or commonly across devices.

Time domain resource allocation for a D2R transmission in terms of transmission starting time and transmission duration/length

For Msg1/Msg3 transmission duration/length, it is decided by the Msg1/Msg3 TBS and Msg1/Msg3 data rate. The Msg1/Msg3 TBS can be a fixed value like 16 or 96 bits respectively and the Msg1/Msg3 data rate can be further derived by the D2R chip length, coding rate and repetition factor if any which can be indicated by the corresponding R2D transmission. In case of TDMA of multiple Msg1/Msg3 transmissions, some gaps in time domain between adjacent Msg1/Msg3 transmissions should be reserved to account for the SFO and clock drift.

In addition, it is observed that with the increase of the X, the gap between adjacent Msg1/Msg3 transmissions due to SFO impact also increases for the later Msg1/Msg3 transmissions, which impacts the transmission efficiency and also impacts the indication of the starting time/transmission length for each TDMed Msg1/Msg3 transmissions.

For the starting times for Msg1/Msg3 transmissions, there are two options agreed in RAN1#117 meeting for studying D2R response timing. In high level, one option is based on a time window by TR2Dmin and TR2Dmax, similar to RFID; the other option is based on transmission timing indication, similar to NR k2 indication for NR PUSCH. In case of TDMA of multiple e.g., two Msg1 transmissions, if option 1 that the time window [TR2Dmin and TR2Dmax] is predefined/fixed in the specification, it can work for the 1st Msg1 transmission, but it is not so straightforward and efficient to be used to determine the response timing for the remaining subsequent TDMed Msg1 transmissions, as for the 2nd Msg1 transmission, TR2Dmin and TR2Dmax from processing perspective are always met by the device. Option 2 is more straightforward to be used to determine the transmission timing for the TDMed D2R transmissions, one example is given in Figure 5-1.

Figure 5-1: Illustration for Option 2 that multiple Msg1 response timings are indicated by a R2D triggering RA

On the timing error (initial/residual SFO) handling for a D2R transmission, regardless of timing relations option 1 that time window based or option 2 that indication based, it is up to reader to take into account the SFO impacts, that is the reader should start detecting the D2R signal no later than the time TR2D_min*(1-clock error) or TR2D *(1-clock error).

Proposal 5.1-3: For TDMA of X>1 time domain resources for D2R transmission(s) for Msg1/Msg3, where each D2R transmission for Msg1 /Msg3 occurs in one time domain resource of the X time domain resource(s),

- the starting time of each Msg1/Msg3 is determined based on the control information in the correpsonding R2D transmission.

- It is up to reader to reserve the sufficient time gap between the adjancet D2R transmissions due to residual SFO of the device.

In addition, similar as for FDMed Msg1 and FDMed Msg3, other than the transmission starting time, other transmission parameters such as TBS, chip length, coding rate and repetition factor if any should be the same or can be different for the TDMed Msg1 and TDMed Msg3 need further discussion.

Proposal 5.1-4: For TDMA of multiple Msg.1 transmissions in response to a R2D transmission triggering random access,

- The transmission starting time is different for the TDMed resources for Msg1 transmissions.

- The transmission duration/length is the same for all the FDMed resources for Msg1 transmissions.

Proposal 5.1-5: For TDMA of multiple Msg.3 transmissions in response to a given set of one or multiple Msg2 transmission(s) during access procedure,

- At least the transmission starting time should be assigned in device specific manner by the Msg2 transmission.

- FFS the transmission duration/length is allocated either specifically for each device or commonly across devices.

  - Msg2 transmissions and receptions

For Msg2 transmission and reception, the following agreements was made in RAN1#118 and #118bis meetings [3] – [4]:

RAN1#118 meeting: Agreement When a R2D transmission in response to a D2R transmission is expected for A-IoT Msg2 response to A-IoT Msg1 for the A-IoT device, study to define a maximum time TD2R_max between the D2R transmission and the corresponding R2D transmission following it, so that the R2D transmission timing is expected to be within [TD2R_min, TD2R_max]. - FFS: whether there is a necessity to define different maximum times for different A-IoT devices RAN1#118bis meeting: Agreement RAN1 studies the following options for Msg2 transmission in response to multiple Msg1 transmissions, which is initiated by a R2D transmission triggering random access. - Option 1: A PRDCH for Msg2 transmission corresponds to a A-IoT Msg1 received from one device - Option 2: A PRDCH for Msg2 transmission corresponds to multiple A-IoT Msg1 received from different devices Agreement RAN1 studies the starting time and time duration for Msg2 monitoring for Msg2 reception.

For the case that a single Msg1 transmission is triggered by a R2D transmission triggering random access (no FDMA and/or TDMA of multiple Msg1 transmissions), the Msg2 response timing based on above agreement is clear. However, for the case that FDMA and/or TDMA of multiple Msg1 transmissions are scheduled by a R2D transmission triggering random access, how a device receives Msg2 after it transmits Msg1 needs to be discussed. Based on above time window framework, following two options for the Msg2 response time window can be considered.

- Option 1: Common window based that is the starting time for the Msg2 response time window is TD2R_min after the “last” Msg1 time domain resource that is scheduled by a R2D transmission triggering random access.

- Option 2: Independent Time window based that is the starting time for the Msg2 response time window is determined based on the control information in a R2D transmission triggering random access.

For both options, the length/duration for Msg2 response time window can be pre-defined and/or indicated. Figure 5-2 gives an example.

| 5-2a: Same size/length for 1st and 2nd sub-windows | 5-2b: Different size/length for 1st and 2nd sub-windows |
| --- | --- |

Figure 5-2: illustration on options for the Msg2 response time window

Between the two options, it is obvious that option 1 is simper, but option 2 has more flexibility compared to option 1. In case Reader has the flexibility to decide whether one Msg2 contains one or more than one response to echo the received ID(s), in other words, there is no need to restrict that one Msg2 should contain all the responses to echo all the ID(s) the reader received in Msg1 transmissions, then Option 2 may have more benefit to help device power saving, of course at the cost of signaling overhead and/or complexity. Both options can be studied further.

Proposal 5.2: Study following options for a device to determine the Msg.2 reception timing in case multiple TDMed/FDMed Msg1 are scheduled by a R2D transmission triggering random access.

- Opt.1 Common time window based: the starting time for the Msg2 response time window is TD2R_min after the “last” Msg1 time domain resource that is scheduled by the R2D transmission triggering random access.

- Opt.2 Independent Time window: the starting time for the Msg2 response time window is determined based on the control information in the R2D transmission triggering random access.

- For both options, the length/duration for Msg2 response time window can be pre-defined and/or indicated.

- Scheduling and timing relationships

This section provides our views on timing relations and scheduling/control information for R2D and D2R transmissions.

  - Timing relations

In RAN1#106 meeting, following agreements were made for A-IoT scheduling and timing relations:

Agreement For further discussion, the following terminologies are used for A-IoT for studying processing time aspects: - TR2D_min: Minimum Time between a R2D transmission and the corresponding D2R transmission following it. - TD2R_min: Minimum Time between a D2R transmission and the corresponding R2D transmission following it. - TR2D_R2D_min: Minimum Time between two different consecutive R2D transmissions to the same A-IoT device. - TD2R_D2R_min: Minimum Time between two different consecutive D2R transmissions from the same A-IoT device. - The study should consider at least following aspects - Implementation restrictions for the existing BS/UE - [Processing time is common or different for different A-IoT devices] - [Processing time for different traffic types/command types (e.g. DT or DO-DTT) and/or different use case (e.g., Inventory or Command)] - FFS other timing aspects

Note that about the exact time point (e.g. starting and/or ending time) for the above time intervals, it can be discussed after more progress is made for D2R and R2D waveform, basic resource allocation unit e.g. chip or bit symbol in time-domain, whether there is postamble or not etc.

    - Timing for D2R response to R2D and R2D response to D2R

In RAN1#117 meeting, following agreement was made for D2R response timing to a R2D:

Agreement Study the following options for the time interval between a R2D transmission and the corresponding D2R transmission following it: - Option 1: Define a maximum time TR2D_max between a R2D transmission and the corresponding D2R transmission following it, so that the device transmits D2R transmission within [TR2D_min, TR2D_max]. - FFS: maximum time is common or different for different A-IoT devices - FFS: maximum time for different traffic types/command types (e.g. DT or DO-DTT) and/or different use case (e.g., Inventory or Command) - Option 2: The corresponding D2R transmission timing TR2D following a R2D transmission is determined based on the control information in the R2D transmission, where TR2D TR2D_min - FFS the maximum value(s) for TR2D

During the discussions, many companies think both options can be considered for different cases/conditions [5]. The details including down-selection if any can be left to the potential normative phase.

In RAN1#118 meeting, the following agreement was made for R2D response timing, more specifically Msg2 to a Msg1 transmission.

Agreement When a R2D transmission in response to a D2R transmission is expected for A-IoT Msg2 response to A-IoT Msg1 for the A-IoT device, study to define a maximum time TD2R_max between the D2R transmission and the corresponding R2D transmission following it, so that the R2D transmission timing is expected to be within [TD2R_min, TD2R_max]. - FFS: whether there is a necessity to define different maximum times for different A-IoT devices

As noticed in the following RAN2 agreement in RAN2#127bis, the expected R2D response for a A-IoT device is not only for Msg1, it is also expected for Msg3 for re-access purpose.

RAN2 agreement @RAN2#127bis meeting 2. Support optional explicit R2D failure/success feedback indication for at least MSG3 for re-access purpose. FFS for following D2R data.

Therefore, we propose to extend the above agreement to cover more cases where the feedback from R2D is expected for a device.

Proposal 6.1.1: For the case(s) where a R2D transmission in response to a D2R transmission is expected, study to define a maximum time TD2R_max between the D2R transmission and the corresponding R2D transmission following it, so that the R2D transmission timing is expected to be within [TD2R_min, TD2R_max].

    - Time unit for the timing relations/intervels

During the meeting, it was mentioned that the time-domain unit for resource allocation should be discussed in AI 9.4.2.1 based on the following agreement.

Agreement RAN1#116 For R2D, line codes studied are: Manchester encoding and pulse-interval encoding (PIE). - FFS: Mapping(s) from bit(s) to line-code codewords - FFS: Time domain definition of e.g., chips and relation to OFDM symbols, resource allocation unit, etc.

Then for AI 9.4.2.2, the time unit for timing relations/intervals between R2D and D2R or between D2R and R2D can be further discussed.

In NR, the smallest time unit is defined as Tc and Ts corresponding to the sampling time for SCS = 480KHz and 15KHz respectively. For time timing indications such as k0, k1 and k2 for PDSCH, PUCCH and PUSCH, the time unit is based on a NR slot.

In RFID, as observed in Table 6-16 (copy below), the time unit for defining the time window is based on RTcal and Tpri, where RTcal is equal to the length of a data-0 symbol plus the length of a data-1 symbol; and Tpri denotes either the commanded period of an FM0 symbol or the commanded period of a single subcarrier cycle.

Table 6-16: Link timing parameters (in RFID C1G2 Standard)

| Para. | Minimum | Nominal | Maximum | Description |
| --- | --- | --- | --- | --- |
| T1 | MAX(RTcal,10Tpri) × (1 – ¦FrT¦) – 2μs | MAX(RTcal,10Tpri) | MAX(RTcal,10Tpri) × (1 + ¦FrT¦) + 2μs | Immediate reply time from Interrogator transmission to Tag reply. Specifically, the time from the last rising edge of the last bit of the Interrogator transmission to the first rising edge of the Tag reply for an immediate Tag reply, measured at the Tag’s antenna terminals. |
| T2 | 3.0Tpri |  | 20.0Tpri | Interrogator reply time if a Tag is to demodulate the Interrogator signal, measured from the end of the last (dummy) bit of the Tag reply to the first falling edge of the Interrogator transmission |
| T3 | 0.0Tpri |  |  | Time an Interrogator waits, after T1, before it issues another command |
| T4 | 2.0 RTcal |  | 20ms | Minimum time between Interrogator commands |
| T5 | MAX(RTcal,10Tpri) × (1 – ¦FrT¦) – 2μs |  | 20ms | Delayed reply time from Interrogator transmission to Tag reply. Specifically, the time from the last rising edge of the last bit of the Interrogator transmission to the first rising edge of the Tag reply for a delayed Tag reply, measured at the Tag’s antenna terminals. |
| T6 | MAX(RTcal,10Tpri) × (1 – ¦FrT¦) – 2μs |  | 20ms | In-process reply time from Interrogator transmission to the first Tag reply. Specifically, the time from the last rising edge of the last bit of the Interrogator transmission to the first rising edge of the first Tag reply indicating that the Tag is either (a) still working, or (b) is done, measured at the Tag’s antenna terminals |
| T7 | MAX(250μs, T2(max)) |  | 20ms | In-process reply time between Tag replies. Specifically, the time from the end of the last (dummy) bit of the Tag’s prior transmission indicating that the Tag is still working to the first rising edge of the current Tag reply indicating that the Tag is either (a) still working, or (b) is done, measured at the Tag’s antenna terminals |

For A-IoT, to determine the time unit for the timing relations, the following options can be considered.

- Option 1: Based on the chip length used for PRDCH and/or PDRCH

- Option 2: Based on the information bit length used for PRDCH and/or PDRCH

- Option 3: Based on the shortest chip length that the A-IoT system supports, corresponding to the maximum OOK-4 M values

Proposal 6.1.2: For the time unit for A-IoT timing relationship, study the following options:

- Option 1: Based on the chip length used for PRDCH and/or PDRCH

- Option 2: Based on the information bit length used for PRDCH and/or PDRCH

- Option 3: Based on the shortest chip length that the A-IoT system supports, corresponding to the maximum OOK-4 M values

  - Scheduling aspects

    - Reader’s identification of the end of PDRCH transmission

In RAN1#106bis meeting, following agreement was made for A-IoT D2R postamble [1]:

Agreement For the reader to acquire the end of PDRCH transmission, study at least following options: - Option 1: D2R postamble immediately follows the PDRCH - Option 2: Based on control information

From our understanding, the above agreement is mainly for TBS indication. Whether/how D2R postamble is needed for other purposes like D2R timing correction is a separate issue and is discussed in section 2.2 of this contribution.

For PDRCH transmission, using D2R postamble implies that the A-IoT device should decide the PDRCH TBS. Then it is difficult for the Reader to control the resource usage and make scheduling decisions since the Reader does not know when D2R transmission is finished until postambe is detected.

Observation 6.2-1: for indication of the end of PDRCH transmssion,

- D2R Postamble may not be needed for PDRCH with fixed TBS.

- D2R Postamble may result in PDRCH decoding failure or additional power consumption due to false or miss detection.

- D2R Postamble makes the scheduling and resource management difficult for the Reader

For using control information to indicate the end of PDRCH transmission, the control information can be R2D. Using R2D control information is straightforward and provides controllability for the Reader. For D2R control information, there could be different interpretations for the D2R control information. Firstly, physical layer D2R control information is not well justified based on the SID of no HARQ, ARQ. Therefore, if D2R control information if any, it should be higher layer D2R control information with fixed TBS, similar to RFID Error codes. In case the PDRCH transmission includes only the higher layer D2R control information with fixed TBS, a header with the command code which is located in the early part of the PDRCH can be used for the Reader to know the end of PDRCH transmission. In case the PDRCH transmission includes the D2R data transmission with variable TBS, using R2D control information to indicate/derive the end of PRDCH transmission is preferred to give the reader controllability. In our view, the reader is able to know and should control the PDRCH TBS/transmission length for all traffic types (i.e., DT, DO-DTT and DO-A) tailored for the target use cases.

Observation 6.2-2: Physical layer D2R control information is not well justified for A-IoT operation. If any D2R control information is needed, it should be higher layer D2R control information with fixed TBS.

Observation 6.2-3: In case the PDRCH transmission includes only higher layer D2R control information with fixed TBS, the TBS/transmission length can be determined by the higher layer D2R control information; In case the PDRCH transmission includes D2R data information with variable TBS, the TBS/transmission length determined by the R2D control information is preferred.

Proposal 6.2-1: Study the reader acquires the end of PDRCH transmission at least based on R2D control information.

- FFS D2R control information for PDRCH transmission with fixed TBS

    - Device’s identification of the end of PRDCH transmission

In RAN1#106bis meeting, following agreement was made for A-IoT R2D postamble [1]:

Agreement To determine or derive the end of PRDCH transmission, study at least following options: - Option 1: R2D postamble immediately follows the PRDCH to indicate the end of the PRDCH. - Option 2: Based on R2D control information.

In the following, we provide our views for above two options.

For Option 1, the main benefit of using postamble to mark the end of R2D transmission is to achieve the most variable and flexible R2D payload size (TBS), e.g. the payload size can be as fine as 1 bit granularity. However, for the intended A-IoT use cases like indoor inventory and command, it is not clear how much flexibility is needed for the R2D payload size. For some specific R2D transmissions, like the R2D transmission during the contention-based access, the TBS can be fixed without the postamble. In case the higher layer data e.g., MAC PDU is constructed with byte aligned, the benefit of using postamble to realize the variable/flexible R2D payload size at bit-level is reduced. In addition, the use of postamble to indicate the end of PRDCH transmission may result in decoding errors and additional power consumption due to the miss detection and false alarm.

The R2D control information in Option 2 can be physical layer control information scheduling the PRDCH and/or higher layer control information. For example, the device can derive the end of PRDCH or PDRCH transmission by some kind of Control Header which is located in the early part of the PRDCH. The Control Header includes the scheduling information and/or command code without or with length field indication for fixed TBS or variable TBS of PRDCH/PDRCH transmission. The necessity of option 1 should be justified first for the A-IoT inventory and command use cases. Therefore, Option 2 is preferred as a unified solution for the determination of the end of PRDCH and PDRCH transmission.

Observation 6.2-4: Using R2D Postamble to indicate the end of PRDCH transmission can enable any TBS for PRDCH transmission. However, it is not clear how variable TBS is required for PRDCH for indoor inventory and command use cases.

Observation 6.2-5: R2D Postamble may not be needed for PRDCH with fixed TBS.

Observation 6.2-6: R2D Postamble may result in PRDCH decoding failure or additional power consumption due to false or miss detection.

Observation 6.2-7: Using R2D control information to indicate or derive the end of PRDCH transmission can enable limited number of TBS for PRDCH transmission. There is tradeoff between the overhead of R2D control information and the number of TBS for PRDCH transmission.

Observation 6.2-8: Using R2D control information to indicate or derive the end of PRDCH transmission is more robust to the ambiguity of the end of transmission.

Proposal 6.2-2: The end of PRDCH transmission that is determined or derived by the R2D control information should be studied as baseline.

- FFS the R2D control information is/are the physical layer and/or higher layer control information

- Conclusion

In this contribution, we provide our views on frame structure and timing aspects for A-IoT. The observations, conclusions and proposals are summarized as follows.

For D2R x-amble(s)

Observation 2.2-1: If the D2R midamble and postamble serve identical functions, but are placed at different locations in the time domain, it is not necessary to use different terminologies and always place an additional "x-amble" at the end of the PDRCH.

Proposal 2.2-1: Capture the following into the TR.

Following is observed by source [R1-2409683] for non-coherent detection of 20-bit PDRCH with 6-bit CRC and ~5 kbps data rate, using 1/2 manchester encoding and 1/3 convolutional code:

  - With up to 10% device SFO for D2R transmission, the SNR needed to achieve 10% and 1% BLER is around 0.2dB and 5.7 dB for the three options: option 1 of D2R preamble only, option 2 of D2R preamble + 1 midamble and option 3 of D2R preamble + 1 postamble.

    - Option 1 requires reader to test 8 SFO hypotheses, the overhead of D2R amble is 45.07%.

    - Option 2 requires reader to test 4 SFO hypotheses, the overhead of D2R amble is 55.17%.

    - Option 3 requires reader to test 6 SFO hypotheses, the overhead of D2R amble is 55.17%.

  - With up to 1% device SFO for D2R transmission, the SNR needed to achieve 10% and 1% BLER is about 0.1dB and 5.2dB for the three options: option 1 of D2R preamble only, option 2 of D2R preamble + 1 midamble and option 3 of D2R preamble + 1 postamble.

    - Option 1 requires reader to test 4 SFO hypotheses, the overhead of D2R amble is 45.07%.

    - Option 2 requires reader to test 2 SFO hypotheses, the overhead of D2R amble is 55.17%.

    - Option 3 requires reader to test 2 SFO hypotheses, the overhead of D2R amble is 55.17%.

Following is observed by source [R1-2409683] for non-coherent detection of 96-bit PDRCH with 16-bit CRC and 5~6 kbps data rate, using 1/2 manchester encoding and 1/3 convolutional code:

  - With up to 10% device SFO for D2R transmission, the SNR needed to achieve 10% and 1% BLER is around 0.7dB and 7.3 dB for the three options: option 1 of D2R preamble only, option 2 of D2R preamble + 1 midamble and option 3 of D2R preamble + 1postamble.

    - Option 1 requires reader to test 10 SFO hypotheses, the overhead of D2R amble is 16%.

    - Option 2 requires reader to test 8 SFO hypotheses, the overhead of D2R amble is 22.22%.

    - Option 3 requires reader to test 8 SFO hypotheses, the overhead of D2R amble is 22.22%.

  - With up to 1% device SFO for D2R transmission, the SNR needed to achieve 10% and 1% BLER is 0.4dB and 6.5dB for the three options: option 1 of D2R preamble only, option 2 of D2R preamble + 1 midamble and option 3 of D2R preamble + 1postamble.

    - Option 1 requires reader to test 6 SFO hypotheses, the overhead of D2R amble is 16%.

    - Option 2 requires reader to test 4 SFO hypotheses, the overhead of D2R amble is 22.22%.

    - Option 3 requires reader to test 4 SFO hypotheses, the overhead of D2R amble is 22.22%.

Following is observed by source [R1-2409683] for non-coherent detection of 400-bit PDRCH with 16-bit CRC and 5~6 kbps data rate, using 1/2 manchester encoding and 1/3 convolutional code:

  - With up to 10% device SFO for D2R transmission, the SNR needed to achieve 10% and 1% BLER is around 2.7dB and 10dB for the three options: option 1 of D2R preamble only, option 2 of D2R preamble + 5 midambles and option 4 of D2R preamble + 4 midambles+1 postamble.

    - Option 1 requires reader to test 12 SFO hypotheses, the overhead of D2R amble is 4.88%.

    - Option 2 requires reader to test 8 SFO hypotheses, the overhead of D2R amble is 15.22%.

    - Option 4 requires reader to test 8 SFO hypotheses, the overhead of D2R amble is 15.22%.

  - With up to 1% device SFO for D2R transmission, the SNR needed to achieve 10% and 1% BLER is around 2.3dB and 9dB for the three options: option 1 of D2R preamble only, option 2 of D2R preamble + 5 midambles and option 4 of D2R preamble + 4 midambles+1 postamble.

    - Option 1 requires reader to test 8 SFO hypotheses, the overhead of D2R amble is 4.88%.

    - Option 2 requires reader to test 6 SFO hypotheses, the overhead of D2R amble is 15.22%.

    - Option 4 requires reader to test 6 SFO hypotheses, the overhead of D2R amble is 15.22%.

Proposal 2.2-2: For non-coherent detection of PDRCH with payload size of 20, 96 and 400 bits,

- The number of SFO hypotheses and the SNR needed for 10% and 1% BLER cannot significantly be reduced for the option(s) of a D2R with preamble and additional amble(s) compared to the option 1 of a D2R with preamble only. Moreover, the addition of midamble(s) and/or postamble introduces about 10% more overhead.

- For option 2 without a D2R postamble and option 3/option 4 with a D2R postamble, the number of SFO hypotheses and the SNR needed for 10% and 1% BLER remains almost unchanged with the same amble overhead. However, D2R postamble prevents pipelined processing of the reception, as the receiver side processing must wait until after SFO estimation and correction.

Proposal 2.2-3: For PDRCH using non-coherent detection, option 1 of D2R preamble is sufficient to achieve 10% and 1% BLER, with no more than 12 SFO hypotheses tested at the reader side.

Proposal 2.2-4: Capture following into the TR.

Following is observed by source [R1-2409683] for coherent detection of 20-bit PDRCH with 6-bit CRC and ~5 kbps data rate, using 1/2 manchester encoding and 1/3 convolutional code:

- With up to 10% device SFO for D2R transmission, the SNR needed to achieve 10% and 1% BLER is around -2dB and 4 dB for the three options: option 1 of D2R preamble only, option 2 of D2R preamble + 1 midamble and option 3 of D2R preamble + 1postamble.

  - Option 1 requires reader to test 8 SFO hypotheses, the overhead of D2R amble is 45.07%.

  - Option 2 requires reader to test 6 SFO hypotheses, the overhead of D2R amble is 55.17%.

  - Option 3 requires reader to test 6 SFO hypotheses, the overhead of D2R amble is 55.17%.

- With up to 1% device SFO for D2R transmission, the SNR needed to achieve 10% and 1% BLER is around -2.8dB and 3.3dB for the three options: option 1 of D2R preamble only, option 2 of D2R preamble + 1 midamble and option 3 of D2R preamble + 1postamble.

  - Option 1 requires reader to test 4 SFO hypotheses, the overhead of D2R amble is 45.07%.

  - Option 2 requires reader to test 2 SFO hypotheses, the overhead of D2R amble is 55.17%.

  - Option 3 requires reader to test 2 SFO hypotheses, the overhead of D2R amble is 55.17%.

Following is observed by source [R1-2409683] for coherent detection of 96-bit PDRCH with 16-bit CRC and 5~6 kbps data rate, using 1/2 manchester encoding and 1/3 convolutional code:

- With up to 10% device SFO for D2R transmission, the SNR needed to achieve 10% and 1% BLER is around 0.7dB and 10dB for option 1 of D2R preamble only; and around -1.7dB and 5.2dB for option 2 of D2R preamble + 1 midamble and option 3 of D2R preamble + 1postamble.

  - Option 1 requires reader to test 10 SFO hypotheses, the overhead of D2R amble is 16%.

  - Option 2 requires reader to test 10 SFO hypotheses, the overhead of D2R amble is 22.22%.

  - Option 3 requires reader to test 10 SFO hypotheses, the overhead of D2R amble is 22.22%.

- With up to 1% device SFO for D2R transmission, the SNR needed to achieve 10% BLER is around -1.3dB for option 1 of D2R preamble only, -3.1dB for option 2 of D2R preamble + 1 midamble and -2.5dB for option 3 of D2R preamble + 1 postamble; the SNR needed to achieve 1% BLER is around 11dB for option 1 of D2R preamble only, 3.6dB for option 2 of D2R preamble + 1 midamble and 4.5dB for option 3 of D2R preamble + 1postamble.

  - Option 1 requires reader to test 6 SFO hypotheses, the overhead of D2R amble is 16%.

  - Option 2 requires reader to test 6 SFO hypotheses, the overhead of D2R amble is 22.22%.

  - Option 3 requires reader to test 4 SFO hypotheses, the overhead of D2R amble is 22.22%.

Following is observed by source [R1-2409683] for coherent detection of 400-bit PDRCH with 16-bit CRC and 5~6 kbps data rate, using 1/2 manchester encoding and 1/3 convolutional code:

- 10% BLER cannot be achieved by option 1 of D2R preamble only, and error floor is observed.

- With up to 10% device SFO for D2R transmission, the SNR needed to achieve 10% BLER is around 0.2dB for option 2 of D2R preamble+5 midambles and option 4 of D2R preamble+4 midambles+1 postamble; the SNR needed to achieve 1% BLER is around 9.2dB for option 2 of D2R preamble+5 midambles, 12.8dB for option 4 of D2R D2R preamble + 4 midambles+1 postamble.

  - Option 2 requires reader to test 8 SFO hypotheses, the overhead of D2R amble is 15.22%.

  - Option 4 requires reader to test 8 SFO hypotheses, the overhead of D2R amble is 15.22%.

- With up to 1% device SFO for D2R transmission, the SNR needed to achieve 10% BLER is around -1.2dB for option 2 of D2R preamble+5 midambles and option 4 of D2R preamble+4 midambles+1 postamble; the SNR needed to achieve 1% BLER is around 7.8dB for option 2 of D2R preamble+5 midambles, 9.1dB for option 4 of D2R preamble + 4 midambles+1 postamble.

  - Option 2 requires reader to test 6 SFO hypotheses, the overhead of D2R amble is 15.22%.

  - Option 4 requires reader to test 6 SFO hypotheses, the overhead of D2R amble is 15.22%.

Observation 2.2-2: For coherent detection of PDRCH with small payload and/or short transmission duration such as 20-bit PDRCH with 6-bit CRC and ~5 kbps data rate, using 1/2 manchester encoding and 1/3 convolutional code, option 1 of D2R preamble only is sufficient to achieve 10% and 1% BLER, with no more than 8 SFO hypotheses tested at the reader side.

Observation 2.2-3: For coherent detection of PDRCH with large payload and/or long transmission duration such as 400-bit PDRCH with 16-bit CRC and 5~6kbps data rate, using 1/2 manchester encoding and 1/3 convolutional code, option 1 of D2R preamble only cannot achieve 10% BLER. With additional 5 ambles, 1% BLER can be achieved.

Observation 2.2-4: For option 2 without a D2R postamble and option 3/option 4 with a D2R postamble, with the same amble overhead, the SNR needed for 10% and 1% BLER is slightly lower for option 2 compared to option 3/option 4 for coherent detection. Moreover, D2R postamble prevents pipelined processing of the reception, as the receiver side processing must wait until after SFO estimation and correction.

Proposal 2.2-5: For timing correction and channel estimation, if the D2R preamble alone is insufficient for certain cases, such as PDRCH with long transmission durations, it is recommended to use one or more D2R midambles within the PDRCH instead of a D2R postamble located at the end of the PDRCH.

For Carrier and frequency synchronization for device 2b

Observation 2.3-1: An external signal which provides a reference frequency can be leveraged to calibrate and stabilize the frequency of the oscillator for device 2b. Depending on the time duration required for the external signal for frequency calibration, the signal can be embedded in existing R2D signal or transmitted as additional signal.

- If the ON chip duration of the current R2D transmission is sufficient, the reference frequency can be provided by ON chip(s) of current R2D transmission, such as in the start indicator part or clock acquisition part in preamble;

- Otherwise, the additional sync signal can be transmitted along with the current R2D transmission,  after the current R2D transmission that triggers D2R transmission.

For A-IoT Frame structure

Proposal 3-1: A-IoT device is not required to acquire the NR frame/subframe/slot/OFDM symbol index.

Proposal 3-2: For D2R transmission, A-IoT device does not apply and maintains the timing advance (TA).

Proposal 3-3: From the A-IoT device side, an OOK symbol/chip length can be considered as the basic resource unit for constructing A-IoT DL/UL channel/signals.

For Energy harvest

Proposal 4-1: Capture following conclusion for Section 8 of TR38.769:

Conclusion: RAN1 has studied the potential impact of energy harvesting on device availability for transmission and reception procedures and identified the following two directions. The solutions for direction 1 and direction 2 are captured in the Table 6.2.1-1 and Table 6.2.2-1.

- Direction 1: Reader does not provide information to a device regarding when the device may become available/unavailable.

- Direction 2: Reader can provide information to a device based on which the device may become available/unavailable.

Proposal 4-2: Conclude that for the purpose of the Rel-19 study, the study on the potential impact of energy harvesting on device availability for transmission and reception procedures in RAN1 is considered completed based on the above conclusion.

For Random access

Proposal 5.1-1: For FDMA of multiple Msg.1 transmissions in response to a R2D transmission triggering random access,

- The transmission locations in the frequency domain, i.e., small frequency shift, FSx are different for the FDMed resources for Msg1 transmissions.

- The transmission bandwidth, i.e., Btx,D2R is the same for all the FDMed resources for Msg1 transmissions.

Proposal 5.1-2: For FDMA of multiple Msg.3 transmissions in response to a given set of one or multiple Msg2 transmission(s) during access procedure,

- At least the frequency domain transmission locations should be assigned in device specific manner by the Msg2 transmission.

- FFS the transmission bandwidth is allocated either specifically for each device or commonly across devices.

Time domain resource allocation for a D2R transmission in terms of transmission starting time and transmission duration/length

Proposal 5.1-3: For TDMA of X>1 time domain resources for D2R transmission(s) for Msg1/Msg3, where each D2R transmission for Msg1 /Msg3 occurs in one time domain resource of the X time domain resource(s),

- the starting time of each Msg1/Msg3 is determined based on the control information in the correpsonding R2D transmission.

- It is up to reader to reserve the sufficient time gap between the adjancet D2R transmissions due to residual SFO of the device.

Proposal 5.1-4: For TDMA of multiple Msg.1 transmissions in response to a R2D transmission triggering random access,

- The transmission starting time is different for the TDMed resources for Msg1 transmissions.

- The transmission duration/length is the same for all the FDMed resources for Msg1 transmissions.

Proposal 5.1-5: For TDMA of multiple Msg.3 transmissions in response to a given set of one or multiple Msg2 transmission(s) during access procedure,

- At least the transmission starting time should be assigned in device specific manner by the Msg2 transmission.

- FFS the transmission duration/length is allocated either specifically for each device or commonly across devices.

Proposal 5.2: Study following options for a device to determine the Msg.2 reception timing in case multiple TDMed/FDMed Msg1 are scheduled by a R2D transmission triggering random access.

- Opt.1 Common time window based: the starting time for the Msg2 response time window is TD2R_min after the “last” Msg1 time domain resource that is scheduled by the R2D transmission triggering random access.

- Opt.2 Independent Time window: the starting time for the Msg2 response time window is determined based on the control information in the R2D transmission triggering random access.

- For both options, the length/duration for Msg2 response time window can be pre-defined and/or indicated.

For Scheduling and timing relationships

Proposal 6.1.1: For the case(s) where a R2D transmission in response to a D2R transmission is expected, study to define a maximum time TD2R_max between the D2R transmission and the corresponding R2D transmission following it, so that the R2D transmission timing is expected to be within [TD2R_min, TD2R_max].

Proposal 6.1.2: For the time unit for A-IoT timing relationship, study the following options:

- Option 1: Based on the chip length used for PRDCH and/or PDRCH

- Option 2: Based on the information bit length used for PRDCH and/or PDRCH

- Option 3: Based on the shortest chip length that the A-IoT system supports, corresponding to the maximum OOK-4 M values

Observation 6.2-1: for indication of the end of PDRCH transmssion,

- D2R Postamble may not be needed for PDRCH with fixed TBS.

- D2R Postamble may result in PDRCH decoding failure or additional power consumption due to false or miss detection.

- D2R Postamble makes the scheduling and resource management difficult for the Reader

Observation 6.2-2: Physical layer D2R control information is not well justified for A-IoT operation. If any D2R control information is needed, it should be higher layer D2R control information with fixed TBS.

Observation 6.2-3: In case the PDRCH transmission includes only higher layer D2R control information with fixed TBS, the TBS/transmission length can be determined by the higher layer D2R control information; In case the PDRCH transmission includes D2R data information with variable TBS, the TBS/transmission length determined by the R2D control information is preferred.

Proposal 6.2-1: Study the reader acquires the end of PDRCH transmission at least based on R2D control information.

- FFS D2R control information for PDRCH transmission with fixed TBS

Observation 6.2-4: Using R2D Postamble to indicate the end of PRDCH transmission can enable any TBS for PRDCH transmission. However, it is not clear how variable TBS is required for PRDCH for indoor inventory and command use cases.

Observation 6.2-5: R2D Postamble may not be needed for PRDCH with fixed TBS.

Observation 6.2-6: R2D Postamble may result in PRDCH decoding failure or additional power consumption due to false or miss detection.

Observation 6.2-7: Using R2D control information to indicate or derive the end of PRDCH transmission can enable limited number of TBS for PRDCH transmission. There is tradeoff between the overhead of R2D control information and the number of TBS for PRDCH transmission.

Observation 6.2-8: Using R2D control information to indicate or derive the end of PRDCH transmission is more robust to the ambiguity of the end of transmission.

Proposal 6.2-2: The end of PRDCH transmission that is determined or derived by the R2D control information should be studied as baseline.

- FFS the R2D control information is/are the physical layer and/or higher layer control information

# References

- 3GPP RAN1#116bis meeting, Chairman’s note

- 3GPP RAN1#117 meeting, Chairman’s note

- 3GPP RAN1#118 meeting, Chairman’s note

- 3GPP RAN1#118bis meeting, Chairman’s note

- R1-2409244, Final FL summary on frame structure and timing aspects for Rel-19 Ambient IoT, Moderator (vivo)

- J. Bae, L. Yan and H. -J. Yoo, "A Low Energy Injection-Locked FSK Transceiver With Frequency-to-Amplitude Conversion for Body Sensor Applications," in IEEE Journal of Solid-State Circuits, vol. 46, no. 4, pp. 928-937, April 2011, doi: 10.1109/JSSC.2011.2109450

- R1-2409187, TP on section 6.2 Device (un)availability for TR 38.769, Moderator (vivo)

- R1-2409682, Discussion on general aspects of physical layer design, vivo

- R1-2409684, Discussion on Downlink and uplink channel/signal aspects, vivo

# Annex A: Evaluation assumptions for D2R amble(s)

| Parameters |  | Assumptions |
| --- | --- | --- |
| Carrier frequency |  | 900MHz |
| SFO |  | Initial SFO (Fe): - Option 1: [0.1~1]*10^5ppm - Option 2: [0.1~1]*10^4ppm Frequency drift (Fr): 400 ppm/s |
| Chip number for amble(s) |  | Preamble: 128 chips Midamble/Postamble: 64 chips Purpose(s): 1. For SFO estimation; 2. For channel estimation for coherent detection. |
| Message size | 20bits with BLF of 40KHz | Block structure, data rate and amble overhead Total chips for PRDCH is (20-bit payload+6-bit CRC)*2*3=156 chips - Opt.1: Preamble+ PRDCH, 5.63 kbps data rate and 45.07% amble overhead - Opt.2: Preamble+100-chip PRDCH+midamble+56-chip PRDCH, 4.6 kbps data rate and 55.17% amble overhead - Opt.3: Preamble+ PRDCH +Postamble, 4.6 kbps data rate and 55.17% amble overhead |
|  | 96bits with BLF of 24KHz | Block structure, data rate and amble overhead Total chips for PRDCH is (96-bit payload+16-bit CRC)*2*3=672 chips - Opt.1: Preamble+PRDCH, 5.76 kbps data rate and 16% amble overhead - Opt.2: Preamble+480-chip PDRCH + midamble + 192-chip PDRCH, 5.33 kbps data rate and 22.22% amble overhead - Opt.3: Preamble + PDRCH + postamble, 5.33 kbps data rate and 22.22% amble overhead |
|  | 400bits with BLF of 20KHz | Block structure, data rate and amble overhead Total chips for PRDCH is (400-bit payload+16-bit CRC)*2*3= 2496 chips - Opt.1: Preamble+PDRCH, 6.1 kbps and 4.88% amble overhead - Opt.2: Preamble+(480-chip PDRCH +midamble)*5 + 96-chip PDRCH, 5.43 kbps data rate and 15.22% amble overhead - Opt.4: Preamble+ (500-chip PDRCH + midamble)*4 + 496-chip PDRCH + Postamble, 5.43 kbps data rate and 15.22% amble overhead |
| CW waveform |  | Single tone |
| Waveform |  | OOK |
| Line code |  | Manchester |
| FEC |  | TBCC with 1/3 code rate |
| Number of Rx |  | 2 |
| Channel model (CW to Device) |  | TDL-A with DS 30ns |
| Channel model (Device to Reader) |  | TDL-A with DS 30ns |
| Device velocity |  | 3km/h |

# Annex B: Coherent detection and non-coherent detection of D2R transmission with Manchester code

For coherent detection, with channel coefficients h estimated, the detection rule for coherent detection for Manchester symbols becomes

Real (h’*(y1-y0))>0,   bit 1<0,           bit 0

Where y0 and y1 is the received signal on the first half of Manchester code and second half of Manchester code, which can be denoted as

y=k=0L-1y[k]

Where L is the sampling number of  half of Manchester code, and y[k] is the sampled signal value.

For non-coherent detection, the detection rule for Manchester symbols becomes

if y12>y02,  bit 1otherwise,  bit 0

# Annex C: Evaluation results for different hypothesis numbers

- 20-bit PDRCH, Option 1 (Preamble only)

| 10^4 ppm, Non-Coherent detection; （required hypo_num = 4） | 10^4 ppm,Coherent detection （required hypo_num = 4） |
| --- | --- |
| 10^5 ppm, Non-Coherent detection （required hypo_num = 8） | 10^5 ppm,Coherent detection （required hypo_num = 8） |

- 20-bit PDRCH, Option 2 (Preamble+1 Midamble)

| 10^4 ppm, Non-Coherent detection （required hypo_num = 2） | 10^4 ppm, Coherent detection （required hypo_num = 2） |
| --- | --- |
| 10^5 ppm, Non-Coherent detection （required hypo_num = 4） | 10^5 ppm,Coherent detection （required hypo_num = 6） |

- 20-bit PDRCH, Option 3 (Preamble+1 Postamble)

| 10^4 ppm, Non-Coherent detection （required hypo_num = 2） | 10^4 ppm, Coherent detection （required hypo_num = 2） |
| --- | --- |
| 10^5 ppm, Non-Coherent detection （required hypo_num = 6） | 10^5 ppm,Coherent detection （required hypo_num = 6） |

- 96-bit PDRCH, Option 1 (Preamble only)

| 10^4 ppm, Non-Coherent detection （required hypo_num = 6） | 10^4 ppm, Coherent detection （required hypo_num = 6） |
| --- | --- |
| 10^5 ppm, Non-Coherent detection （required hypo_num = 10） | 10^5 ppm,Coherent detection （required hypo_num = 10） |

- 96-bit PDRCH, Option 2 (Preamble+1 Midamble)

| 10^4 ppm, Non-Coherent detection （required hypo_num = 4） | 10^4 ppm, Coherent detection （required hypo_num = 6） |
| --- | --- |
| 10^5 ppm, Non-Coherent detection （required hypo_num = 8） | 10^5 ppm,Coherent detection （required hypo_num = 10） |

- 96-bit PDRCH, Option 1 (Preamble+1 Postamble)

| 10^4 ppm, Non-Coherent detection （required hypo_num = 4） | 10^4 ppm, Coherent detection （required hypo_num = 4） |
| --- | --- |
| 10^5 ppm, Non-Coherent detection （required hypo_num = 8） | 10^5 ppm,Coherent detection （required hypo_num = 10） |

- 400-bit PDRCH, Option 1 (Preamble only)

| 10^4 ppm, Non-Coherent detection （required hypo_num = 8） | 10^5 ppm, Non-Coherent detection （required hypo_num = 12） |
| --- | --- |

- 400-bit PDRCH, Option 2 (Preamble+5 Midambles)

| 10^4 ppm, Non-Coherent detection （required hypo_num = 6） | 10^4 ppm, Coherent detection （required hypo_num = 6） |
| --- | --- |
| 10^5 ppm, Non-Coherent detection （required hypo_num = 8） | 10^5 ppm,Coherent detection （required hypo_num = 8） |

- 400-bit PDRCH, Option 4 (Preamble+4 Midambles+1 Postamble)

| 10^4 ppm, Non-Coherent detection （required hypo_num = 6） | 10^4 ppm, Coherent detection （required hypo_num = 6） |
| --- | --- |
| 10^5 ppm, Non-Coherent detection （required hypo_num = 8） | 10^5 ppm,Coherent detection （required hypo_num = 8） |