3GPP TSG RAN WG1#119                                              R1-2409730

Orlando, US, November 18th – 22nd, 2024

Source:	ZTE Corporation, Sanechips

Title:	Discussion on IoT-NTN TDD mode

Agenda Item:	9.11.5

Document for: Discussion

- Introduction

              - In RAN1#118bis meeting, the basic simulation assumptions and target channels to be evaluated for IoT-NTN TDD mode were discussed and agreed [1]. In this contribution, the potential issues for IoT-NTN TDD mode are elaborated and evaluated based on the analysis on the periodic pattern.

- Discussion on the periodic pattern definition

  - Compatibility with the legacy system

As described in WID, the new NB-IoT NTN TDD mode allows configuring the usage of radio resources in the targeted MSS allocated band with a periodic subset of the UL and DL subframes in N radio frames. The periodic pattern should consist of non-overlapping set of usable contiguous UL subframes and set of usable contiguous DL subframes, and guard periods, which is periodic every N radio frames, with N=9 as baseline.

In RAN1#118bis, the parameters of periodic pattern were discussed and following agreements achieved, which list the potential scenarios to be evaluated.

Agreement For the study phase, RAN1 evaluates the following combination of N (in radio frames) and D (D = number of consecutive downlink subframes): - N = 9: - D = 8 - D = 20 - D = 30 - N = 8: - D = 20 - Companies to report which subframe indices are included in the set of consecutive downlink subframes. - Other combinations are not precluded to be reported by companies NOTE: This does not imply that the above combinations are the only ones to be considered during the normative phase. Agreement For the study phase, RAN1 evaluates the following combination of N (in radio frames) and U (U = number of consecutive uplink subframes): - N = 9: - U = 8 - U = 20 - U = 30 - N = 8: - U = 20 - Companies to report which subframe indices are included in the set of consecutive uplink subframes. - Other combinations are not precluded to be reported by companies - For evaluations including both uplink and downlink (if any), companies are encouraged to prioritize the case of D=U NOTE: This does not imply that the above combinations are the only ones to be considered during the normative phase.

During the discussion, the following two points are highlighted by the proponents:

- At least the case of N=9, D=8, and U=8 should be supported to be compatible to TDD pattern of Iridium’s legacy system deployed in the 1616-1626.5 MHz band as shown in Figure 1.

- D=8 and U=8 were proposed to be minimum duration to accommodate the 8.28ms slot length in legacy system. Larger D and U can be further configured by network.

However, it leads to the different interpretation on the “compatibility” with the legacy system. In general, if compatibility to TDD pattern of legacy system should always be considered as restriction to the design of TDD pattern of IoT-NTN, D and U should not be configured larger than 8 with consideration of 8.28ms slot length in legacy system and gap between slots. That is, only a fixed pattern with N=9, D=8, and U=8 is enough without additional configurability. If the TDD pattern of legacy system is not considered as a hard restriction, it seems not necessary to make the scenario N=9, D=8, and U=8 mandatory for specification.

Then, with above consideration, it should be clarified whether compatibility to Iridium’s legacy system should always be considered. And what’s the impact should also be clarified for further discussion on TDD design, e.g., the gap length between slots.

Observation 1: If compatibility to TDD pattern of Iridium’s legacy system should always be considered, configuring DL and UL duration larger than 8 subframes should not be allowed due to the restriction of slot length and gap in legacy system.

Observation 2: If compatibility to TDD pattern of Iridium’s legacy system is not hard restriction, it is not necessary to consider N=9, D=8, and U=8 as a mandatory pattern to be supported.

Proposal 1: RAN1 should discuss and clarify whether compatibility to TDD pattern of Iridium’s legacy system should always be considered when determining configurable values of N, D, and U.

Proposal 2: RAN1 should discuss and clarify what will be restrictions when considering compatibility to TDD pattern of Iridium’s legacy system.

Figure 1 TDD pattern of legacy system deployed in the 1616-1626.5 MHz band

  - Configurability

In TDD mode, different from HD-FDD, cross link interference can happen when UL and DL transmissions are performed simultaneously even for different UEs. For example, when UE1 is receiving DL transmission while UE2 performing UL transmission, the DL signal received by UE1 will be interfered by the UL signal transmitted by UE2, and UL signal received by satellite from UE2 will be interfered by the DL signal transmitted to UE1. Thus, inter-UE interference may happen if UE-specific periodic pattern is introduced but not properly configured, especially considering large and variant TA in NTN scenario.

Observation 3: Inter-UE interference may happen if UE-specific periodic pattern is introduced but not properly configured.

Besides the UL-DL interference among UEs, the satellite side interference may also happen if periodic pattern is variant among different cells even with frequency reuse factor (e.g., multiple cells cross different beams from same satellite). For example, if two cells served by same satellite and same frequency band have different periodic patterns as shown in Figure 2, their transmission directions may be different at certain time such as radio frame 4 and 5. When the transmission directions of these two cells are different, the DL signal transmitted to UEs in first cell may be interference for UL signal received from UEs in second cell at satellite side, e.g., as shown in Figure 3. Thus, inter-cell interference may happen if cell-specific periodic pattern is introduced but not properly configured.

Observation 4: Inter-cell interference may happen if cell-specific periodic pattern is introduced but not properly configured.

Figure 2 Example of variant periodic patterns for different cells

Figure 3 Illustration of inter-cell interference

With above analysis, it can be observed that the flexibility of periodic pattern is limited when considering interference and collision control. Therefore, a fixed periodic pattern common to all UEs can be enough.

Proposal 3: A fixed periodic pattern can be considered for IoT-NTN TDD mode without additional configuration signaling.

  - Offset in TDD pattern

In order to cover at least the subframes carrying NPSS(subframe 5), NSSS(subframe 9), and MIB-NB(subframe 0), the start of DL duration will not able to align with the radio frame timing when D=8, e.g., as shown in Figure 4. Therefore, the offset between start of DL duration and radio frame timing need to be known by UE to determine DL subframes.

Figure 4 DL duration not align with radio frame timing

If the start of DL duration is not aligned with radio frame timing, the start time of TDD pattern may also need to be considered. Two potential definitions of TDD pattern may be considered:

- The start of periodic pattern is aligned with the start of DL duration

- The start of periodic pattern is aligned with radio frame timing, e.g., the start time of first periodic pattern is the start of SFN0

In our view, the second solution is preferred for easier resource determination of periodic pattern. In this case, the offset that need to be known by UE can be the offset between start time of TDD pattern and start time of DL duration as shown in Figure 5. When D>=10, it is enough to set the start time of DL duration aligned with the radio frame timing, and no need to introduce the offset.

Figure 5 Illustration of offset between start of DL duration and start of TDD pattern

Proposal 4: The TDD pattern is preferred to be aligned with radio frame timing for easier resource determination.

Proposal 5: An offset between start of DL duration and start of TDD pattern is needed to ensure the subframes for common channel and synchronization signals are covered when D<10.

  - SFN wrap around

In NB-IoT, the SFN index will be wrap around after 1024 radio frames. When N=9, a hyper frame will not carry integer number of periodic patterns. For example, assuming the start of first TDD pattern is aligned with SFN0, there will be 7 remaining radio frames before the SFN wrap around. If continuing the consecutive TDD patterns, the last TDD pattern will be across SFN wrap around and ends in SFN1 in next hyper frame. As a result, the start time of first TDD pattern in next hyper frame will not be SFN0, which means the relationship between periodic pattern and SFN is not constant. In this case, the resource determination of TDD pattern will be more complex. Hence, the preferred solution is to align the start time of first TDD pattern with SFN0 in each hyper frame, and skip remaining radio frames before SFN wrap around if they cannot carry a complete TDD pattern.

Proposal 6: The start time of first TDD pattern is preferred to be aligned with SFN0 in each hyper frame for easier resource determination of TDD pattern. The remaining radio frames before SFN warp around that cannot carry a complete TDD pattern can be skipped in TDD pattern determination.

Figure 6 Skip remaining radio frames before SFN wrap around if they cannot carry a TDD pattern

- Simulation for DL synchronization

The link budget was discussed in RAN1#118bis and following agreement was achieved.

Agreement The target operating DL CNR of NB-IoT NTN TDD is obtained following the parameters in TR 36.763 and modifying the carrier frequency to 1.6GHz: - For LEO-600, the operating DL CNR is 4.91dB (i.e. with 0dBi UE antenna gain) - For LEO-1200, the operating DL CNR is 5.51dB (i.e. with 0dBi UE antenna gain) - When reporting link level simulation results, companies are encouraged to report the CNR margin with respect to the above operating DL CNRs. - Companies may report values for other scenarios (e.g. UE antenna gain of -5.5 dBi, LEO-800) with the corresponding assumptions.

It is worth noting that 0 dBi UE antenna gain is assumed in TR 36.763 for IoT-NTN. However, for coverage enhancement in NR-NTN, the UE antenna gain is assumed as -5.5 dBi for FR1. It is questionable to assume the UE antenna gain of NB-IoT device is higher than NR device. Hence, in this contribution, the DL CNR with UE antenna gain of -5.5 dBi is also considered in following evaluation. Moreover, for NPBCH, TS 36.101 has also defined minimum performance requirement as SNR=-2.0dB for single NPBCH TTIs (TS 36.101, Table 8.12.3.1.1-1). With above considerations, the SNR constraints to be considered are listed in Table 1.

Table 1 SNR constraints to be considered

| UE antenna gain | CNR for LEO-600 Set-1 | CNR for LEO-1200 Set-1 | Minimum performance for NPBCH with single TTI in TS 36.101, Table 8.12.3.1.1-1 |
| --- | --- | --- | --- |
| 0 dBi | 4.91 dB | 5.51 dB | -2.0 dB |
| -5.5 dBi | -0.59 dB | 0.01 dB | -2.0 dB |

In RAN1#118bis, the basic simulation assumptions for DL synchronization are agreed as shown in Table 2. Due to limited time in previous meeting, there are still some assumptions kept open and to be reported, e.g., the definition of correct detection of NPSS/NSSS. The additional assumptions in this contribution are listed in Table 3. Note that for NPSS detection, the detection is thought correct when the residual TO is smaller than half of CP, which is at most 80Ts. Moreover, since NPSS detection has been performed before NSSS detection, only residual TO and FO are considered for NSSS simulation in this contribution. The TO for NSSS simulation is assumed as [-80Ts, 80Ts], which aligns with the correct detection rule for NPSS. Regarding the FO for NSSS simulation, 0.1ppm is assumed similar as NPBCH.

Table 2 Agreed simulation assumptions for NPSS/NSSS/NPBCH in RAN1#118bis

| Parameter | Value |
| --- | --- |
| Carrier frequency | 1.6GHz |
| Channel model | NTN TDL-C rural, 30 degrees |
| Frequency error / timing drift (including XO error) | 34 ppm for NPSS / NSSS (24ppm Doppler + 10ppm XO error) 0.1ppm for NPBCH |
| Variation of frequency error / variation of timing drift | 0.27ppm/s for NPSS / NSSS 0 for NPBCH |
| UE velocity | 3km/h |
| Target performance | For NPSS/NSSS: - 99% detection probability with 0.1% false alarm rate - FFS: How to define correct detection, including successful timing / frequency estimation / cell ID detection. For NPBCH: - 1% BLER For the channels / signals above, acquisition delay shall be reported for the corresponding detection probability |
| Other assumptions (e.g. combining length, frequency error hypothesis, etc.) | To be reported and justified by companies |

Table 3 Additional simulation assumptions for NPSS/NSSS/NPBCH

| Parameter | Value |
| --- | --- |
| Definition of correct detection | For NPSS: timing error within [-80Ts, 80Ts] For NSSS: correct detection of cell ID and SFN |
| Combining | For NPSS: 1-NPSS, 2-NPSS with 90ms periodicity For NSSS: 1-NSSS For NPBCH: available subframes (i.e., MIB-NB subframes falling in the configured DL consecutive subframes in TDD pattern) within 80ms/160ms/320ms/640ms |
| Frequency error hypothesis | Multiple hypothesis for NPSS with 10kHz interval |
| TO and FO for NSSS | TO [-80Ts, 80Ts] FO [-0.1ppm, 0.1ppm] |

  - NPSS simulation

The simulation results for NPSS are as illustrated in Figure 7. The margin of NPSS detection performance and link budget are listed in Table 4. It can be observed that the required SNR for single-shot NPSS detection is lower than CNR for LEO-1200 and LEO-600 with 0 dBi UE antenna gain but higher than the CNR with -5.5 dBi UE antenna gain. When 2-NPSS combination with 90ms periodicity is performed, the required SNR can also be lower than CNR with -5.5 dBi UE antenna gain, which implies no need of enhancement for NPSS.

Figure 7 Performance of NPSS detection

Table 4 Margin of required SNR for NPSS

|  | Required SNR | Margin w.r.t CNR for LEO-600 with 0 dBi UE antenna gain | Margin w.r.t CNR for LEO-1200 with 0 dBi UE antenna gain | Margin w.r.t CNR for LEO-600 with -5.5 dBi UE antenna gain | Margin w.r.t CNR for LEO-1200 with -5.5 dBi UE antenna gain |
| --- | --- | --- | --- | --- | --- |
| 1-NPSS | 3.3 dB | 1.61 dB | 2.21 dB | -3.89 dB | -3.29 dB |
| 2-NPSS, 90ms periodicity | -1.2 dB | 6.11 dB | 6.71 dB | 0.61 dB | 1.21 dB |

Observation 5: The required SNR for 2-NPSS combination detection with 90ms periodicity is -1.2 dB, which is lower than CNRs for LEO-600 and LEO-1200 with -5.5 dBi UE antenna gain. The acquisition delay is 2*90=180ms if D=8 and 90ms if D=20 or 30.

Proposal 7: The required SNR for NPSS detection can be satisfied and no need of NPSS enhancement.

  - NSSS simulation

The simulation results for NSSS are as illustrated in Figure 8. The margin of NSSS detection performance and link budget are listed in Table 5. It can be observed that the required SNR for single-shot NSSS detection is lower than CNR for LEO-1200 and LEO-600 even with -5.5 dBi UE antenna gain, which implies no need of enhancement for NSSS.

Figure 8 Performance of NSSS detection

Table 5 Margin of required SNR for NSSS

|  | Required SNR | Margin w.r.t CNR for LEO-600 with 0 dBi UE antenna gain | Margin w.r.t CNR for LEO-1200 with 0 dBi UE antenna gain | Margin w.r.t CNR for LEO-600 with -5.5 dBi UE antenna gain | Margin w.r.t CNR for LEO-1200 with -5.5 dBi UE antenna gain |
| --- | --- | --- | --- | --- | --- |
| 1-NSSS | -2.5 dB | 7.41 dB | 8.01 dB | 1.91 dB | 2.51 dB |

Observation 6: The required SNR for 1-NSSS detection is -2.5 dB, which is lower than CNRs for LEO-600 and LEO-1200 with -5.5 dBi UE antenna gain. The acquisition delay is 2*90=180ms if D=8 and 90ms if D=20 or 30.

Proposal 8: The required SNR for NSSS detection can be satisfied and no need of NSSS enhancement.

Proposal 9: At least 2 consecutive radio frames are preferred to be allocated for DL in the periodic pattern, which ensures at least one NSSS can be received within a single TDD pattern.

  - NPBCH simulation

The simulation results for NPBCH are as illustrated in Figure 9. When D=8, only 1 MIB-NB subframe can be received in each TDD pattern. When D=20, 2 MIB-NB subframes can be received in each TDD pattern. Note that the periodicity of MIB-NB is not aligned with TDD pattern when N=9. In such case, the offset between the start of TDD pattern and MIB-NB periodicity is randomly generated.

The margin of NPBCH detection performance and link budget are listed in Table 6. When D=8 and N=9, it can be observed that the required SNR for NPBCH detection is lower than CNR for LEO-1200 and LEO-600 with -5.5 dBi UE antenna gain but higher than minimum performance requirement in TS 36.101 even for 640ms combination. While for D=20 and N=9, the required SNR for NPBCH can be lower than the minimum performance requirement in TS 36.101 with 320ms combination. With above consideration, D=20 is preferred to ensure the NPBCH detection performance to be good enough.

Figure 9 Performance of NPBCH detection

Table 6 Margin of required SNR for NPBCH

|  | Required SNR | Margin w.r.t CNR for LEO-600 with 0 dBi UE antenna gain | Margin w.r.t CNR for LEO-1200 with 0 dBi UE antenna gain | Margin w.r.t CNR for LEO-600 with -5.5 dBi UE antenna gain | Margin w.r.t CNR for LEO-1200 with -5.5 dBi UE antenna gain | Minimum performance for NPBCH with single TTI in TS 36.101, Table 8.12.3.1.1-1 |
| --- | --- | --- | --- | --- | --- | --- |
| D=8, N=9, 160ms combination | >5dB | / | / | / | / | / |
| D=20, N=9, 160ms combination | 1.7 dB | 3.21 dB | 3.81 dB | -2.29 dB | -1.69 dB | -3.7 dB |
| D=8, N=9, 320ms combination | 1.5 dB | 3.41 dB | 4.01 dB | -2.09 dB | -1.49 dB | -3.5 dB |
| D=20, N=9, 320ms combination | -2.4 dB | 7.31 dB | 7.91 dB | 1.81 dB | 2.41 dB | 0.4 dB |
| D=8, N=9, 640ms combination | -1.8 dB | 6.71 dB | 7.31 dB | 1.21 dB | 1.81 dB | -0.2 dB |
| D=20, N=9, 640ms combination | -5.1 dB | 10.01 dB | 10.61 dB | 4.51 dB | 5.11 dB | 3.1 dB |

Observation 7: When D=8 and N=9, the required SNR for NPBCH detection within 640ms time window is -1.8 dB, which is lower than CNRs for LEO-600 and LEO-1200 with -5.5 dBi UE antenna gain but higher than minimum performance requirement in TS 36.101. The acquisition delay is 640ms.

Observation 8: When D=20 and N=9, the required SNR for NPBCH detection within 320ms time window is -2.4 dB, which is lower than CNRs for LEO-600 and LEO-1200 with -5.5 dBi UE antenna gain and minimum performance requirement in TS 36.101. The acquisition delay is 320ms.

Proposal 10: The required SNR for NPBCH detection can be lower than CNR by only receiving available subframes within downlink duration. However, the minimum performance of NPBCH defined in TS 36.101 may not be satisfied for D=8 and N=9.

Proposal 11: At least 2 consecutive radio frames are preferred to be allocated for DL in the periodic pattern, which ensures the required SNR for NPBCH detection can also be lower than the minimum performance of NPBCH defined in TS 36.101.

- Impacts on other channels

  - SIB1-NB

In NB-IoT, the SIB1-NB is transmitted once every 20ms and repeated in 160ms. As a result, if N=9 and D=8, UE can only receive SIB1-NB once in every two TDD patterns, which requires 180ms. In this case, UE cannot receive repetitions of SIB1-NB, which means a coverage loss of at least 9 dB. Moreover, since the SIB1-NB transmission period 160ms is smaller than 180ms duration of two TDD patterns, the UE may even not able to receive one transmission of a SIB1-NB packet during a SIB1-NB period in certain scenario. With above considerations, D=20 or D=30 is preferred to ensure no SIB1-NB packet is missed and increase the coverage performance.

Observation 9: When D=8 and N=9, UE can only receive SIB1-NB once every 180ms, which is even longer than the SIB1-NB period. That is, the SIB1-NB coverage performance will degrade at least 9dB and UE may miss the reception of some SIB1-NB packet.

Proposal 12: At least 2 consecutive radio frames are preferred to be allocated for DL in the periodic pattern, which ensures UE will not miss SIB1-NB reception.

  - Paging

In current spec, the paging occasion is mainly determined using UE ID. As a result, when using D=8 and N=9, 82/90 paging occasions will be invalid. The paging resources will degrade significantly. Then, the paging delay will increase a lot since UE may only have one valid paging resource for several paging cycles or TDD patterns.

Observation 10: When introducing TDD pattern, the paging resource will degrade significantly and paging delay will increase significantly.

- Conclusions

      - According to the analysis given above, we have the following observations and proposals:

Observation 1: If compatibility to TDD pattern of Iridium’s legacy system should always be considered, configuring DL and UL duration larger than 8 subframes should not be allowed due to the restriction of slot length and gap in legacy system.

Observation 2: If compatibility to TDD pattern of Iridium’s legacy system is not hard restriction, it is not necessary to consider N=9, D=8, and U=8 as a mandatory pattern to be supported.

Proposal 1: RAN1 should discuss and clarify whether compatibility to TDD pattern of Iridium’s legacy system should always be considered when determining configurable values of N, D, and U.

Proposal 2: RAN1 should discuss and clarify what will be restrictions when considering compatibility to TDD pattern of Iridium’s legacy system.

Observation 3: Inter-UE interference may happen if UE-specific periodic pattern is introduced but not properly configured.

Observation 4: Inter-cell interference may happen if cell-specific periodic pattern is introduced but not properly configured.

Proposal 3: A fixed periodic pattern can be considered for IoT-NTN TDD mode without additional configuration signaling.

Proposal 4: The TDD pattern should be aligned with radio frame timing for easier resource determination.

Proposal 5: An offset between start of DL duration and start of TDD pattern is needed to ensure the subframes for common channel and synchronization signals are covered when D<10.

Proposal 6: The start time of first TDD pattern is preferred to be aligned with SFN0 in each hyper frame for easier resource determination of TDD pattern. The remaining radio frames before SFN warp around that cannot carry a complete TDD pattern can be skipped in TDD pattern determination.

Observation 5: The required SNR for 2-NPSS combination detection with 90ms periodicity is -1.2 dB, which is lower than CNRs for LEO-600 and LEO-1200 with -5.5 dBi UE antenna gain. The acquisition delay is 2*90=180ms if D=8 and 90ms if D=20 or 30.

Proposal 7: The required SNR for NPSS detection can be satisfied and no need of NPSS enhancement.

Observation 6: The required SNR for 1-NSSS detection is -2.5 dB, which is lower than CNRs for LEO-600 and LEO-1200 with -5.5 dBi UE antenna gain. The acquisition delay is 2*90=180ms if D=8 and 90ms if D=20 or 30.

Proposal 8: The required SNR for NSSS detection can be satisfied and no need of NSSS enhancement.

Proposal 9: At least 2 consecutive radio frames are preferred to be allocated for DL in the periodic pattern, which ensures at least one NSSS can be received within a single TDD pattern.

Observation 7: When D=8 and N=9, the required SNR for NPBCH detection within 640ms time window is -1.8 dB, which is lower than CNRs for LEO-600 and LEO-1200 with -5.5 dBi UE antenna gain but higher than minimum performance requirement in TS 36.101. The acquisition delay is 640ms.

Observation 8: When D=20 and N=9, the required SNR for NPBCH detection within 320ms time window is -2.4 dB, which is lower than CNRs for LEO-600 and LEO-1200 with -5.5 dBi UE antenna gain and minimum performance requirement in TS 36.101. The acquisition delay is 320ms.

Proposal 10: The required SNR for NPBCH detection can be lower than CNR by only receiving available subframes within downlink duration. However, the minimum performance of NPBCH defined in TS 36.101 may not be satisfied for D=8 and N=9.

Proposal 11: At least 2 consecutive radio frames are preferred to be allocated for DL in the periodic pattern, which ensures the required SNR for NPBCH detection can also be lower than the minimum performance of NPBCH defined in TS 36.101.

Observation 9: When D=8 and N=9, UE can only receive SIB1-NB once every 180ms, which is even longer than the SIB1-NB period. That is, the SIB1-NB coverage performance will degrade at least 9dB and UE may miss the reception of some SIB1-NB packet.

Proposal 12: At least 2 consecutive radio frames are preferred to be allocated for DL in the periodic pattern, which ensures UE will not miss SIB1-NB reception.

Observation 10: When introducing TDD pattern, the paging resource will degrade significantly and paging delay will increase significantly.

- Reference

- Chair’s notes for RAN1 #118bis