3GPP TSG RAN WG1 #119                                              R1-2409923

Orlando, US, November 18th – 22nd, 2024

Source: 	CATT

Title:	Discussion on last DCI determination of DCI format 1_3 for HARQ-ACK codebook

Agenda Item:	8.1

Document for:	Discussion and Decision

Introduction

In this contribution, we discuss the issue on last DCI format determination of DCI format 1_3 for HARQ-ACK codebook in TS 38.213.

Discussion

According to current description in TS 38.213[1], the smallest index among two or more scheduled cells co-scheduled by DCI format 1_3 is used to determine last DCI format for PUCCH determination among DCI formats within a PDCCH MO. The related description in TS 38.213 is highlighted below.

From section 9 of TS 38.213 In the remaining of this clause, a last DCI format is from a set of detected DCI formats for which the UE would provide HARQ-ACK information in a PUCCH in a same slot. Detected DCI formats are first indexed in ascending order across indexes of respective scheduled cells for a same PDCCH monitoring occasion, and are then indexed in ascending order across indexes of PDCCH monitoring occasions. For indexing a detected DCI format associated with two or more scheduled cells, a respective scheduled cell is the one with the smallest index among the two or more scheduled cells.

As specified in TS 38.212[2] in section 7.3.1.2.4, if the higher layer parameter scheduledCellComboListDCI-1-3 for the scheduled cell set is configured, the scheduled cells refer to the cells indicated by scheduled cell indicator or configured by scheduledCellComboListDCI-1-3 when only one entry is configured. It should be noted that the scheduled cell of a DCI format can be the cell with scheduling PDSCH reception, or without scheduling PDSCH reception, such as for the purpose of HARQ-ACK retransmission trigger.

From section 7.3.1.2.4 of TS 38.212 - Scheduled cells indicator - number of bits determined by the following: - 0 bit if the higher layer parameter scheduledCellComboListDCI-1-3 for the scheduled cell set is not configured; - otherwise log2IDL bits indicating the scheduled cells in the scheduled cell set according to Table 7.3.1.2.4-1, where IDL is the number of entries in the higher layer parameter scheduledCellComboListDCI-1-3. If only one entry is configured in the higher layer parameter scheduledCellComboListDCI-1-3, the scheduled cells are the cells configured by higher layer parameter scheduledCellComboListDCI-1-3.

Observation 1: According to current description in TS 38.212, if the higher layer parameter scheduledCellComboListDCI-1-3 is configured, the scheduled cell refer to the cell indicated by scheduled cell indicator or configured by scheduledCellComboListDCI-1-3 when only one entry is configured.

However, in RAN1#112 meeting, for DCI format 1_3 associated with more than one PDSCH, it was agreed that the last DCI format among DCI formats within a PDCCH MO is the PDSCH with the smallest serving cell index among the set of co-scheduled cells. Obviously, the last DCI format determination in current specification doesn’t align with this agreement.

Agreement (RAN1#112) - For a set of cells which is co-scheduled by a DCI format 1_X, the PDSCH with the smallest serving cell index among the set of co-scheduled cells is used to determine last DCI format for PUCCH determination among DCI formats within a same PDCCH MO. - It is up to gNB implementation to resolve the last DCI format issue when both DCI format 1_X and other DCI format 1_0/1_1/1_2/1_X are received in a same PDCCH monitoring occasion on a same scheduling cell for scheduling PDSCHs on same scheduled cell.

Observation 2: Current description on the last DCI for PUCCH determination in TS 38.213 doesn’t align with the achieved agreement on last DCI format 1_3 determination in RAN1#112.

Consequently, companies have different understandings on last DCI format 1_3 determination, and it may lead to different understanding between gNB and UE on last DCI format for PUCCH determination. As shown in Figure 1, DCI-1 is a DCI 1_3 with scheduled cell indicator, and the scheduled cell indicator indicate Cell#2, Cell#4 and Cell#5. The Cell#2 is with invalid FDRA and is used to trigger HARQ-ACK retransmission. The DCI-2 is another DCI 1_3 within same PDCCH MO. It schedules PDSCH reception on Cell #3 and Cell #4 respectively. And the HARQ-ACK information associated with DCI-1 and DCI-2 are in a same HARQ-ACK codebook.

- Understanding-1: If last DCI format determination is according to current description in TS 38.213 and TS 38.212, the Cell #2 associated with DCI-1 is the reference cell.

- Understanding-2: If last DCI format determination is based on the achieved agreement in RAN#112, Cell #4 associated with DCI-1 is the reference cell.

Figure 1: Last DCI format determination between two DCIs within same PDCCH MO

If the understanding-1 is the common understanding, the DCI-2 is the last DCI within the PDCCH MO. While, if the understanding-2 is the common understanding, the DCI-1 is the last DCI within the PDCCH MO.

To correct ambiguity, it’s better to modify the description on last DCI format for PUCCH determination as following.

TP#1 for section 9 of TS 38.213: In the remaining of this clause, a last DCI format is from a set of detected DCI formats for which the UE would provide HARQ-ACK information in a PUCCH in a same slot. Detected DCI formats are first indexed in ascending order across indexes of respective scheduled cells for a same PDCCH monitoring occasion, and are then indexed in ascending order across indexes of PDCCH monitoring occasions. For indexing a detected DCI format associated with two or more scheduled cells with scheduling PDSCH receptions, a respective scheduled cell is the one with the smallest index among the two or more scheduled cells.

On one hand, it can make the last DCI format determination method consistent with the agreement in RAN1#112. On another hand, the wording of “scheduled cells with scheduling PDSCH receptions” is the same as wording of the reference cells determination for DAI counting in TS 38.213. Therefore, a clarification on “scheduled cell” for last DCI format determination in TS 38.213 should be adopted.

Proposal 1: For last DCI format determination procedure of DCI format 1_3 in TS 38.213, clarify the ‘scheduled cell’ refers to the scheduled cell with scheduling PDSCH receptions.

In [3], we provide corresponding draft CR to capture ‘the scheduled cell with scheduling PDSCH receptions’ for DCI format 1_3 in the last DCI format determination procedure.

Conclusion

In this contribution, the following observations and proposal are presented:

Observation 1: According to current description in TS 38.212, if the higher layer parameter scheduledCellComboListDCI-1-3 is configured, the scheduled cell refers to the cell indicated by scheduled cell indicator or configured by scheduledCellComboListDCI-1-3 when only one entry is configured.

Observation 2: Current description on the last DCI for PUCCH determination in TS 38.213 doesn’t align with the achieved agreement on last DCI format 1_3 determination in RAN1#112.

Proposal 1: For last DCI format determination procedure of DCI format 1_3 in TS 38.213, clarify the ‘scheduled cell’ refers to the scheduled cell with scheduling PDSCH receptions.

References

- TS 38.213 V18.4.0, Physical layer procedures for control.

- TS 38.212 V18.4.0, Physical channels and modulation

- R1-2409923, Discussion on last DCI determination of DCI format 1_3 for HARQ-ACK codebook, CATT