3GPP TSG-RAN WG1 Meeting #119	R1-2410907

Orlando, US, November 18th – 22nd, 2024

Agenda Item:	9.6.3

Title:	FL summary #5 on LP-WUS operation in CONNECTED mode

Source:	Moderator (NTT DOCOMO)

Document for:	Discussion, Decision

1	Introduction

The core part of the Rel-19 LP-WUS/WUR WI [1] has the following objective:

The objectives of the work item are the following: - To specify an LP-WUS design commonly applicable to both IDLE/INACTIVE and CONNECTED modes (RAN1, RAN4) - Specify OOK (OOK-1 and/or OOK-4) based LP-WUS with overlaid OFDM sequence(s) over OOK symbol - The LP-WUS design shall ensure that for IDLE/INACTIVE operation, the same information is delivered irrespective of LP-WUR type. The OFDM sequence can carry information. - At least duty-cycled monitoring of LP-WUS is supported - For IDLE/INACTIVE modes - Specify procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring (RAN2, RAN1, RAN3, RAN4) - Specify LP-SS with periodicity with Yms for LP-WUR, for synchronization and/or RRM for serving cell. (RAN1, RAN4) - LP-SS is based on OOK-1 and/or OOK-4 waveform with or without overlaid OFDM sequences. Further down selection between with and without overlaid OFDM sequences is to be done within WI. - Note: For LP-WUR that can receive existing PSS/SSS, existing PSS/SSS can be used for synchronization and RRM instead of LP-SS. - Y will be decided within WI. 320ms is the start point. - Specify further RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR, including the necessary conditions (RAN4, RAN2) - For CONNECTED mode, specify procedures to allow UE MR PDCCH monitoring triggered by LP-WUS including activation and deactivation procedure of LP-WUS monitoring (RAN2, RAN1) - Note: In CONNECTED mode, UE MR ultra-deep sleep is not considered, and UE RRM/RLM/BFD/CSI measurements are performed by MR - Note: The target coverage of LP-WUS and LP-SS shall be the coverage of PUSCH for message3. - Note: The optimization of LP-WUS signal design for idle/inactive mode is prioritized over the optimization for connected mode. - Specify the necessary RAN4 core requirement(s) to support the feature (RAN4). - Specify UE low-power wake-up receiver requirements, at least REFSENS, ACS and ASCS requirements with consideration of possible new methodology to assess the low-power wake-up receiver performance - Define guard RBs for ACS and ASCS cases - Study testability of above requirements - Consider impacts of different architecture and impairments, and set requirements that enable all types of reasonable implementation - Study and if necessary specify or support by declaration, the corresponding BS requirements, e.g., dynamic range for LP-WUS/LP-SS. - Current NR BS requirements is baseline - Specify necessary RRM requirements

This document summarizes contributions [4] – [27] submitted to agenda item 9.6.3 (LP-WUS operation in CONNECTED mode).

2	Proposals for Online Sessions

2.1	Proposals for Monday Online

[Mon-high]Proposal 3.2-1a:

- For RRC CONNECTED mode, the number X of candidate values for the determination of the minimum time gap between LP-WUS reception and MR to start PDCCH monitoring via UE capability reporting is [2, 3, 4].

  - The reported candidate values are selected from {0.25, 0.5, 1, 2, 3, 6, 10, 20}ms as starting point

    - Note: Other values are not precluded

    - FFS whether different values are selected for OOK-based WUR and OFDM-based WUR

  - FFS whether/how to consider the case of CA

[Mon-high]Proposal 3.4-1:

- For LP-WUS monitoring in RRC CONNECTED mode, SSB and/or CSI-RS can be the QCL source of LP-WUS

QCL type(s) is {Type A or Type C} and Type D, when applicable

[Mon-high]Proposal 5-1a:

- LP-WUS in RRC CONNECTED mode does not include Scell dormancy indication

FFS whether/how to support dormant SCell behaviours

If time allows...

[Mon]Proposal 3.1-4a:

- For LP-WUS CONNECTED mode operation, the periodic CSI/L1-RSRP reporting operation is same as Rel-16 DCP, that is the UE can be configured with a parameter to enable/disable periodic CSI/L1-RSRP reporting, respectively:

  - If the parameter is NOT configured:

    - If the UE is not indicated to wake up by LP-WUS, the periodic CSI/L1-RSRP is not reported.

    - If the UE is indicated to wake up by LP-WUS, the periodic CSI/L1-RSRP is reported during the DRX active time as in legacy specification.

  - If the parameter is configured:

    - If the UE is not indicated to wake up by LP-WUS, the periodic CSI/L1-RSRP is reported during the time given by the configured drx-onDurationTimer in DRX-Config, even if the drx-onDurationTimer does not start.

    - If the UE is indicated to wake up by LP-WUS, the periodic CSI/L1-RSRP is reported during the DRX active time as in legacy specification.

  - Note: no RAN1 spec impact is expected.

[Mon]Proposal 3.4-2:

- For LP-WUS monitoring in RRC CONNECTED mode, consider following options on how to define/configure/indicate TCI-state of LP-WUS

Opt.1: Reuse TCI-state framework to indicate TCI-state of LP-WUS

Opt.2: TCI-state association between LP-WUS and CORESET

Opt.3: Default TCI-state

Note: Other options are not precluded

2.2	Proposals for Wednesday Online

[Wed]Proposal 3.1-2a:

Alt1

- For LP-WUS CONNECTED mode operation, it is NOT supported that a UE monitors LP-WUS based on Option 1-1 and Option 1-2 simultaneously for a given cell group

Alt2

- For LP-WUS CONNECTED mode operation, it is NOT supported that the case when both Option 1-1 and Option 1-2 are simultaneously configured for the same UE in a given cell group

[Wed]Proposed conclusion 3.1-3b:

- For Option 1-1 of LP-WUS CONNECTED mode operation, RAN1 assumes existing DRX Command MAC CE and Long DRX Command MAC CE can be used to stop drx-onDurationTimer and drx-InactivityTimer for the DRX group

When UE is not in C-DRX active time, UE monitors LP-WUS

- Send LS to RAN2 to ask whether RAN2 has same understanding or not.

[Wed]Proposal 3.1-3c:

- For Option 1-2 of LP-WUS CONNECTED mode operation, RAN1 considers following two options of explicit L1/L2 signalling to stop the new timer for PDCCH monitoring in Option 1-2 and drx-InactivityTimer for the DRX group

Option 1: existing DRX Command MAC CE and Long DRX Command MAC CE

Option 2: additional explicit L1/L2 signalling

When UE is not in C-DRX active time, UE monitors LP-WUS

- Send LS to RAN2 to ask the feasibility/necessity of these options.

[Wed]Proposal 3.2-2b:

- For LP-WUS in CONNECTED mode, if a UE receives LP-WUS targeting to itself, the UE is not required to start monitoring PDCCH before t0+T1, where:

t0 is the reference time point. Details FFS

T1 is the time duration reported as the one value from X candidate values in UE capability reporting.

If time allows...

[Wed]Proposal 4-1:

- When LP-WUS monitoring is enabled for RRC CONNECTED mode, support UE autonomous fallback to PDCCH monitoring when UE monitors LP-WUS

FFS the condition for the fallback

For LP-WUS operation option 1-1, UE fallbacks to legacy C-DRX behavior

For LP-WUS operation option 1-2,

    - FFS whether UE fallbacks to

      - Alt1: legacy C-DRX behavior

      - Alt2: PDCCH monitoring without legacy C-DRX

      - Alt3: PDCCH monitoring according to LP-WUS cycle

    - FFS whether/how to avoid misalignment between gNB and UE on the UE LP-WUS/PDCCH monitoring

2.3	Proposals for Thursday Online

[Thu]Combined Proposal 3.3-1/2a:

- For LP-WUS MOs in connected mode for Option 1-1, following is considered further

  - Approach 1:

    - LP-WUS MOs, including periodicity and offset, are configured independently from the C-DRX periodicity/offset by new RRC parameter(s).

    - UE selectively monitors LP-WUS in the LP-WUS MOs identified by other parameters such as time offset prior to the start of drx-onDurationTimer.

  - Approach 2:

    - LP-WUS MOs, including periodicity and offset, are identified based on the slot that drx-onDurationTimer would start and a new RRC parameter indicating the time offset until the slot that drx-onDurationTimer would start.

  - FFS: An RRC parameter indicates the duration where the UE monitors LP-WUS [consecutively in time]

    - FFS: the consecutive LP-WUS MOs in time identified by the RRC parameter are associated with a same C-DRX cycle;

    - FFS: LP-WUS MOs in time identified by the RRC parameter provides consistent indication for the same C-DRX cycle of the UE, i.e., UE does not expect to detect more than one LP-WUSs with different indications for the UE in the LP-WUS MOs associated with the same C-DRX cycle.

- For LP-WUS MOs in connected mode for Option 1-2, following is considered further

  - Approach 1:

    - LP-WUS MOs, including periodicity and offset, are configured independently from the C-DRX periodicity/offset by new RRC parameter(s).

    - UE monitors LP-WUS in the LP-WUS MOs and starts a new timer for PDCCH monitoring triggered by LP-WUS, after a configured time offset. FFS whether LP-WUS triggers drx-onDurationTimer

  - Approach 2:

    - Periodicity/offset of the slot that the new timer for PDCCH monitoring triggered by LP-WUS are configured by RRC parameters.

    - LP-WUS MOs, including periodicity and offset, are identified based on the slot that new timer would start and a new RRC parameter indicating the time offset until the slot that new timer would start.

  - FFS: An RRC parameter indicates the duration where the UE monitors LP-WUS [consecutively in time]

    - FFS: the consecutive LP-WUS MOs in time identified by the RRC parameter are associated with the same slot that the new timer would start;

    - FFS: LP-WUS MOs in time identified by the RRC parameter provides consistent indication for the same slot that the new timer would start, i.e., UE does not expect to detect more than one LP-WUSs with different indications for the UE in the LP-WUS MOs associated with the same slot that the new timer would start.

[Thu]Proposal 5-2:

- For the case when UE is configured with NR-DC and/or CA without dual DRX groups in RRC CONNECTED mode,

LP-WUS is configured on a serving cell per cell-group

LP-WUS indication is applicable to all activated serving cells in the cell-group

- For the case when UE is configured with NR-DC and/or CA with dual DRX groups in RRC CONNECTED mode,

Opt.1

    - LP-WUS is configured on a serving cell per cell-group

    - LP-WUS carries an indication for which all activated serving cells of DRX group(s) it triggers PDCCH monitoring

Opt.2

    - LP-WUS is configured on a serving cell per DRX group per cell-group

    - LP-WUS indication is applicable to all activated serving cells in the DRX group per cell-group

Opt.3

    - LP-WUS is configured on a serving cell per cell-group

    - LP-WUS indication is applicable to all activated serving cells of all DRX groups per cell-group.

2.4	Proposals for Friday Online

[Fri]Proposal 3.1-3e:

Update the following agreement in red:

Agreement

- For Option 1-2 of LP-WUS CONNECTED mode operation, for explicit indication to stop the new timer for PDCCH monitoring in Option 1-2 and drx-InactivityTimer for the DRX group, RAN1 identified the following potential solution

  - Use existing DRX Command MAC CE and Long DRX Command MAC CE

    - FFS: Applicability of this feature for new timer for PDCCH monitoring in Option 1-2

  - When UE is not in C-DRX active time, UE monitors LP-WUS

- Send LS to RAN2 to ask the feasibility of the above potential solution. LS in R1-2410XXX.

[Fri]Proposal 3.1-3e’:

Update the following conclusion in red:

Conclusion

- For Option 1-1 of LP-WUS CONNECTED mode operation, RAN1 assumes existing DRX Command MAC CE and Long DRX Command MAC CE can be used to stop drx-onDurationTimer and/or drx-InactivityTimer for the DRX group

  - When UE is not in C-DRX active time, UE monitors LP-WUS

- Send LS to RAN2 to ask whether RAN2 has same understanding or not. LS in R1-2410XXX.

[Fri]Proposal 3.1-3f:

Endorse draft LS to RAN2 in R1-2410908, final LS in R1-2410909

[Fri]Proposal 3.1-4c:

Update following agreement in red:

Agreement

- For Option 1-2, for LP-WUS CONNECTED mode operation, the periodic CSI/L1-RSRP reporting operation is same as Rel-16 DCP, that is the UE can be configured with a parameter to enable/disable periodic CSI/L1-RSRP reporting, respectively:

  - If the parameter is NOT configured:

    - If the UE is not indicated to wake up by LP-WUS, the periodic CSI/L1-RSRP is not reported.

    - If the UE is indicated to wake up by LP-WUS, the periodic CSI/L1-RSRP is reported during the DRX active time.

  - If the parameter is configured:

    - If the UE is not indicated to wake up by LP-WUS, the periodic CSI/L1-RSRP is reported during the time given by the configured drx-onDurationTimer in DRX-Config [and also outside DRX active time], even if the drx-onDurationTimer does not start. for the case when UE is outside C-DRX active time.

    - If the UE is indicated to wake up by LP-WUS, the periodic CSI/L1-RSRP is reported during the DRX active time.

  - FFS: Whether RAN1 spec impact is required.

[Fri]Combined Proposal 3.2-1/3.2-2c:

For future meetings:

Consider the following alternatives for UE capability report on the minimum time gap between LP-WUS reception and MR to start PDCCH monitoring:

- Alt 1: For the X = [2, 3, 4] candidate values for the UE capability report on the minimum time gap, consider {0.25, 0.5, 1, 2, 3, 6, 10, 20}ms as starting point

  - Note: Other values are not precluded

  - FFS whether different values are selected for OOK-based WUR and OFDM-based WUR

  - FFS whether/how to consider the case of CA

- Alt 2: For the UE capability report on the minimum time gap, UE reports a pair of values for main radio ramp up and the number of SSBs/TRSs needed for synchronization (D, NSSB/NTRS). This may require RAN4 coordination

- Alt 3: UE capability report on the minimum time gap includes the main radio ramp up time. The number of SSBs/TRSs needed for synchronization is defined in specs, and different number of SSBs/TRSs can be defined for different UE capability on the main radio ramp up time. This may require RAN4 coordination

- Alt 4: UE capability report on the minimum time gap includes the main radio ramp up time. The time needed for synchronization will be defined by RAN4 requirements later.

- FFS: For the case of UAI

3	LP-WUS Procedures to trigger PDCCH monitoring

3.1	High-level procedures

This issue has been discussed since RAN1#116 and following agreements were made so far.

Agreement Update the following agreement in RAN1#116 in red: Agreement - For RRC CONNECTED mode, from RAN1 perspective, further study following LP-WUS procedures to trigger PDCCH monitoring: - Case 1: PDCCH monitoring is triggered by LP-WUS with C-DRX configuration - Option 1-1: LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer. - This option may replace DCP functionality - Option 1-2: LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring. - PDCCH monitoring possibly irrespective of drx-onDurationTimer - Option 1-2-1: PDCCH monitoring may be additionally triggered based on legacy C-DRX cycle and drx-onDurationTimer when monitoring LP-WUS - If this is adopted, it should be configured together with Option 1-1 to achieve power saving gain compared to legacy C-DRX - Option 1-2-2: PDCCH monitoring is not triggered by legacy C-DRX cycle and drx-onDurationTimer when monitoring LP-WUS - Option 1-3: LP-WUS monitoring inside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring. - Case 2: PDCCH monitoring is triggered by LP-WUS without C-DRX configuration. LP-WUS can be monitored at any time according to the LP-WUS monitoring configuration - FFS duty-cycled and/or continuous LP-WUS monitoring - Combination of options in Case 1 and combination of options in Case 1 and Case 2 are not precluded should be considered. - RAN1 does not discuss C-DRX related timers other than drx-onDurationTimer, this topic is up to RAN2 - Note: Above does not preclude to support fallback mechanism to trigger PDCCH monitoring, if any Agreement - For Option 1-1 of LP-WUS CONNECTED mode operation, the followings are assumed from RAN1 perspective. LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer UE is configured with legacy C-DRX configurations as Rel-18 UE behaviors related to CDRX active time triggered by all legacy DRX timers are not affected unless included in this proposal No impact on RRM/RLM/BFD measurement requirements is assumed For periodic CSI/L1-RSRP reporting, UE can be configured with one of the following (same as Rel-16 DCP) - Periodic CSI/L1-RSRP is not reported during the time given by the configured drx-onDurationTimer if UE is not indicated to wake-up - Periodic CSI/L1-RSRP is periodically reported during the time given by the configured drx-onDurationTimer regardless if UE is indicated to wake-up or not - Note: Periodic CSI/L1-RSRP is reported during CDRX active time as in legacy specification Agreement - For option 1-2 of LP-WUS CONNECTED mode operation, the followings are assumed from RAN1 perspective. LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring. UE is configured with legacy C-DRX configurations as Rel-18 UE is expected to be configured with LP-WUS monitoring configuration (periodicity and offset can be different from those from C-DRX configuration) - FFS potential restriction for LP-WUS configuration in relation with C-DRX configuration LP-WUS triggers the start of a timer during which UE monitors PDCCH - FFS the timer is existing timer and/or new timer UE PDCCH monitoring behaviors related to other legacy DRX timers are not affected - drx-InactivityTimer, drx-RetransmissionTimerDL, drx-RetransmissionTimerUL, drx-HARQ-RTT-TimerDL, drx-HARQ-RTT-TimerUL No impact on RRM/RLM/BFD measurement requirements is assumed For periodic CSI/L1-RSRP reporting, UE can be configured with one of the following (same as Rel-16 DCP and option 1-1) - Periodic CSI/L1-RSRP is not reported if UE is not indicated to wake-up - Periodic CSI/L1-RSRP is periodically reported regardless if UE is indicated to wake-up or not UE PDCCH monitoring is not triggered by legacy C-DRX cycle and drx-onDurationTimer when monitoring LP-WUS Agreement Confirm following working assumption Working Assumption From RAN1 perspective, for RRC CONNECTED mode, PDCCH monitoring is triggered by LP-WUS with C-DRX configuration Support Option 1-1: LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer. Support Option 1-2: LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring. FFS whether/how to support both Option 1-1 and Option 1-2 simultaneously configured for the same UE Note: Above can be revisited considering RAN2 decisions. Agreement - For LP-WUS CONNECTED mode operation, Option 1-2 can be configured separately from Option 1-1 - FFS whether/how to support both Option 1-1 and Option 1-2 simultaneously configured for the same UE

Note that RAN2 have made following agreements

- In RRC_CONNECTED mode, RAN2 to further discuss the impacts of LP-WUS operation methods identified in RAN1. - For Option 1-1 (as described in RAN1 agreement), the LP-WUS monitoring occasion locates at a configured time offset before the start of drx-onDurationTimer. The range of time offset can be determined by RAN1. - For Option 1-1, RAN2 assumes the solutions/ operations introduced for DCP mechanism is taken as baseline. - RAN2 assume that legacy DCP and Option 1-1 is not configured simultaneously for a UE. - The LP-WUS related configuration for RRC CONNECTED state UE is provided via dedicated RRC message. For option 1-2, - After LP-WUS triggers the UE to perform PDCCH monitoring, the UE starts one timer. When the timer is running, the UE monitors PDCCH. FFS on the timer (e.g., newly defined timer or legacy timer.) - The timer is started at a time offset after receiving the LP-WUS indication for PDCCH monitoring. The range of time offset is left for RAN1. For Option 1-2, LP-WUS monitoring is performed at least outside legacy C-DRX Active Time. FFS if the legacy drx-onDurationTimer is started or not if the new timer is configured in Option 1-2. In option 1-2, a new timer triggered by LPWUS is introduced. When this new timer is running, UE is in C-DRX active time. When UE is not in C-DRX active time, UE goes back to LPWUS monitoring. When UE is in C-DRX active time, UE PDCCH monitoring behaviors related to other legacy DRX timers (except for drx-onDurationTimer) are not affected.

Before discussing further details on the LP-WUS procedures, it would be helpful to have common understanding among companies whether/how LR and MR can operate simultaneously. Following proposal was discussed in the last RAN1 meeting while no consensus was achieved.

Proposal For the purpose of future discussions, RAN1 assumes at least the following For RRC connected mode, - LP-WUR is NOT able to receive signal(s) during the time where MR transmits/receives signals/channels. - FFS: additonal consideration of simultaneous operation between LP-WUR and MR

A number of companies provided their view on this aspect in this meeting as below:

- QC

  - At least support following:

    - UE does not monitor/receive LP-WUS in C-DRX active time.

- Vivo

  - For option 1-1 of LP-WUS CONNECTED mode operation, a UE does not monitor LP-WUS in DRX Active Time.

  - For option 1-2 of LP-WUS CONNECTED mode operation, a UE does not monitor LP-WUS in DRX Active Time. An explicit indication can be used to terminate the DRX active time and to activate LP-WUS monitoring.

  - For RRC connected mode, at least support the case where LP-WUR is NOT able to receive signal(s) during the time period when MR is transmitting or receiving signals/channels.

- HW/HiSi

  - In (a), the MR and LP-WUR share RF modules and some BB modules, and two branches are decoupled until BB BPF. In this case, without considering the hardware degradation, all the received energy goes pass the common modules (until BB AMP) without loss. After BB BPFs, the energy to MR/LP-WUR is also not degraded, since the two BB BPFs decouple the energy on different RBs. Such kind of implementation can enable the simultaneous reception of MR and LP-WUR. However, the power consumption of LP-WUR would be very high since many high-power modules of MR are shared to LP-WUR, and this may not be able to achieve the power consumption saving target.

  - In (b), the MR and LP-WUR share RF passive modules, and two branches are decoupled before RF LNA. In this case, the ‘wide-band’ signal is divided into two parts by splitter, and the two parts of signal pass two branches, respectively. Since both parts of signal are ‘wide-band’, there will be several dBs loss, e.g. 3dB loss, for the energy on each RB, which could impact the performance of both MR and LP-WUR.

  - In (c), the MR and LP-WUR only share RF passive modules, and two branches are decoupled before RF LNA. Different from Figure 1 (b), there is a switch before two branches. The received signal can only go pass one of the branches, and thus there is no energy loss. Such kind of implementation CANNOT enable the simultaneous reception of MR and LP-WUR. The power consumption of LP-WUR could be low.

  - For the purpose of future discussion, RAN1 assumes at least the following as baseline.

    - LP-WUR is NOT able to receive signal(s) during the time where MR transmits/receives signals/channels.

    - FFS: additional consideration of simultaneous operation between LP-WUR and MR.

  - Further discuss the UE behaviour with respect to the LP-WUR monitoring when MR is performing measurement or transmitting periodic CSI/L1-RSRP.

    - FFS how to let gNB and UE have the common understanding on where the UE performs measurement.

    - FFS: UE reports its capability on whether its MR and LP-WUR can work simultaneously.

Procedure of UE after receiving LP-WUS targeted to itself (from R1-2409425)

- ZTE

  - Define two capability and describe the UE behavior based on the UE capability

    - Capability1: UE does not allow simultaneous operation between LR and MR

      - Identify the scenarios for behavior clarification.

  - Capability2: UE allow simultaneous operation between LR and MR

- ETRI

  - For the purpose of future discussions, RAN1 assumes at least the following

    - For RRC connected mode,

      - LP-WUR and MR cannot support simultaneous operation at a given time instance.

      - FFS: additional consideration of simultaneous operation between LP-WUR and MR

- LGE

  - Consider the case where LP-WUS monitoring is in active state even during PDCCH monitoring is activated

- DCM

  - For the purpose of future discussions, RAN1 assumes at least the following

    - For RRC connected mode,

      - LP-WUR is NOT able to receive signal(s) during the time where MR transmits/receives signals/channels.

      - FFS: additional consideration of simultaneous operation between LP-WUR and MR

According to the discussion in the last meeting and companies input in this meeting, the main discussion point is whether LR and MR can operate simultaneously for the following cases, and corresponding proposal is made

- Case 1: LP-WUS monitoring vs PDCCH monitoring during C-DRX active time

- Case 2: LP-WUS monitoring vs periodic CSI/L1-RSRP reporting

- Case 3: LP-WUS monitoring vs RRM measurement

[Old]Proposal 3.1-1:

- At least support following for RRC connected mode:

  - UE does not monitor LP-WUS in C-DRX active time

  - FFS: UE does not monitor LP-WUS when performing periodic CSI/L1-RSRP reporting

  - FFS: UE does not monitor LP-WUS when performing RRM measurement

- FFS: additonal consideration of simultaneous operation between LP-WUR and MR

| Company | Y/N | Comments |
| --- | --- | --- |
| Qualcomm | Y |  |
| Xiaomi |  | Is this intened to excluded Option 3? We support Option 3 since it can reduce uncessary PDCCH monitoring during C-DRX active time and is not conlicted with Option 1 or Option 2 at all. It would be a pity to not support Option 3 in current release since the mechanism to suppoort Option2 and Option3 are quite similar |
| CMCC |  | We suggest to seperate discuss Case 1 and Case 2/3. For Case 1, this is related to the specific scheme for LP-WUS, i.e., whether Option 1-2-1/1-2-3 among Optoin 1-2 is supported, and whether option 1-3 is supported. Our preference is that option 1-2-2 is supported since: - Option 1-1 is already supported in the previous meeting. - If C-DRX cycle is configured as a smaller value in Option 1-1, Option 1-1 can achieve the same purpose as Option 1-2-1 to allow more transmission opportunity. For Case 2/3, this is related to whether LR and MR can work simultaneously. We are open for this issue. |
| ZTE, Sanechips | Y |  |
| Panasonic | Y |  |
| Samsung | Y |  |
| Nokia1 | Y |  |
| Spreadtrum | Y |  |
| Apple | N | We think this is related to the discussion for Option 1-3, when the PDCCH monitoring is skipped, the drx-onDurationTimer is still running, it needs to be clarified whether it is still in C-DRX Active Time. |
| ETRI | Y |  |
| Lenovo | Y |  |
| LGE |  | Addition to 3 cases, we think UL transmission such as CG-PUSCH should be considered. For exmaple, a UE not in DRX active time may or may not transmit CG-PUSCH if configured. In this case, whether LP-WUS monitored or not is confusing, so it can be discussed whether CG-PUSCH and LP-WUS monitoring can be operated or not. |
| OPPO |  | With the introduction of LP-WUS, the active time defination may be changed. How could the proposal for Case 1 can work without ambiguity. |
| MTK | Y |  |
| vivo | Y | To address the concern of companies on option 1-3, maybe we can say for option 1-1 and option 1-2, At least support following for RRC connected mode… |

[Old]Proposal 3.1-1a:

Alt1

- At least support following for RRC connected mode:

  - UE does not monitor LP-WUS when in C-DRX active time triggered by LP-WUS [or other timers are running]

  - FFS: UE does not monitor LP-WUS when performing periodic CSI/L1-RSRP reporting

  - FFS: UE does not monitor LP-WUS when performing RRM measurement

  - FFS: CG-PUSCH

- FFS: additonal consideration of simultaneous operation between LP-WUR and MR

Alt2

- For the purpose of future discussion, RAN1 assumes at least the following as baseline.

  - LP-WUR is NOT able to receive signal(s) during the time where MR transmits/receives signals/channels.

[Open]Proposal 3.1-1b:

- At least support following for RRC connected mode:

  - UE does not monitor LP-WUS during the time where MR transmits/receives signals/channels

  - FFS: exact cases when MR transmits/receives signals/channels

- FFS: additonal consideration of simultaneous operation between LP-WUS monitoring and MR transmission/reception

| Company | Y/N | Comments |
| --- | --- | --- |
| Moderator |  | After Monday offline, I had offline disccusion with some companies, and Proposal 3.1-1a is merged into the above to include both UE procedure perspective (while details can be discussed further) and also restriction of UE implementation perspective in high-level, which also avoid detail discussion on DRX acteive time and RAN2 timers for now. |
| Huawei, HiSilicon | Y | According to the analysis in our paper, based on some particular implementation, UE has no ability to recevie any DL signals simultaneously by MR and LR. Therefore, we think the worst case should be the baseline for further discussion. In other words, in all 3 cases spec should support the UE that cannot receive simultaneously by MR and LR. If some companies still think there is possibility to have LP-WUR to work simultaneously with MR, we are also ok for such as ’both cases are considered’, as long as the ’not able to work simultaneously’ case is considered for future discussion. |
| CMCC | Y | Support |
| Qualcomm |  | We do not think we can specify, within Rel-19 timeframe, UE behaviors for LP-WUS monitoring in DRX Active Time only when MR does not transmit/receive signal/channel. This requires careful work on (1) collision handling between LP-WUS monitoring vs MR Tx/Rx; (2) necessary time gap between LP-WUS monitoring vs MR Tx/Rx, etc. It is more realistic and simple to specify Proposal 3.1-1a with a little bit of updates, as follows: - Specification support at least following: - UE does not monitor LP-WUS when in C-DRX active time triggered by LP-WUS [or other timers are running] - FFS: Support the case where UE monitors LP-WUS in C-DRX active time The above is aligned with R-16 DCP specified in 38.213 as follows. |
| OPPO |  | We wonder what will be ’supported’ by the proposal. The actually specification impact is way to be decieded. Even with the C-DRX active time, the specification impact may or may or be. We somehow need indentify the impact. |
| Xiaomi |  | Our understanding of the proposal is form UE capability perspective, UE does not support the case that LR and MR are working simutaneously. We think it is reasonable baseline. And we think it is also OK to support the case that LR and MR are working simutaneously,if UE capability allows. |
| Moderator |  | This proposal was not dsicussed during offline/online in this meeting. Companies are engouraged to fuether check and consider towards next meeting. |

Regarding the FFS whether/how to support both Option 1-1 and Option 1-2 simultaneously configured for the same UE in the above working assumption, following views are provided from companies.

- Support: QC (but only one is active), IDC, SPRD, NEC, Pana, ETRI, LGE, Lenovo

  - QC:

    - Either LP-WUS Option 1-1 or Option 1-2 (or none of them) is active

    - The activation/deactivation of LP-WUS Option 1-1 and Option 1-2 is done by MAC-CE

  - IDC: Introduce a mechanism to properly manage PDCCH monitoring based on Option 1-1 and Option 1-2

  - SPRD

    - if the drx-onDurationTimer is not started by the first LP-WUS, there are three kinds of UE behaviors during the drx-onDurationTimer.

      - Alt 1: UEs do not need to monitor second LP-WUS during the drx-onDurationTimer.

      - Alt 2: UEs need to monitor second LP-WUS during the drx-onDurationTimer.

      - Alt 3: The first LP-WUS indicates whether the UE needs to monitor second LP-WUS during the drx-onDurationTimer.

  - Pana

    - How to handle the LP-WUS monitoring configured by both Option 1-1 and 1-2, including the potential overlapping of LP-WUS MO of Option 1-1 and Option 1-2.

    - Whether/how drx-onDurationTimer triggered by Option 1-1 work together with the new timer -triggered by Option 1-2.

  - ETRI

    - If the new timer triggered by LP-WUS of Option 1-2 is overlapped with legacy C-DRC cycle, the new timer should be stopped before the start of legacy C-DRX cycle in order to prevent the conflict between the new timer and drx-onDurationTimer.

      - FFS: how to handling the collision between the new timer and LP-WUS monitoring for Option 1-1 if Option 1-1 and Option 1-2 are supported simultaneously.

- Not support: vivo, E///, HW/HiSi, CMCC, ZTE, Samsung, CATT, TCL, [DCM], Sharp, MTK

  - Same effect can be achieved by proper configuration of Option 1-2

  - High complexity due to collision handling between Option 1-1 and Option 1-2

Example of simultaneous activation of Option 1-1 and Option 1-2 (from R1-2410399)

Here to avoid misunderstanding, let’s rephrase “simultaneously configured” to “simultaneously activated” based on previous agreement. Then, this allows the case when Option 1-1 and Option 1-2 are simultaneously configured (enabled by RRC signalling) for the same UE while only one of them can be activated, as proposed by some companies.

Agreement For RRC CONNECTED mode, from RAN1 perspective, - PDCCH monitoring triggered by LP-WUS is enabled/disabled by gNB RRC signaling - FFS whether to support UE assistance. - LP-WUS monitoring by UE is known to gNB. - FFS whether implicit/explicit indication from UE is necessary - In case LP-WUS monitoring is enabled, following options are further studied - Option 1: No additional indication/condition are introduced for activation/deactivation of LP-WUS monitoring - Option 2: Activation/deactivation of LP-WUS monitoring by gNB L1/L2 signaling with or without UE assistance. - Option 3: Activation/deactivation of LP-WUS monitoring based on condition(s), such as timer. - Option 4: Activation/deactivation of LP-WUS monitoring based on implicit indication/condition, e.g. UL transmission.

Then, as many companies pointed out, if Option 1-1 and Option 1-2 are simultaneously activated, it causes high complexity due to collision handling between Option 1-1 and Option 1-2. Therefore, following proposal is made

[Close]Proposal 3.1-2:

- For LP-WUS CONNECTED mode operation, it is NOT supported that the case when both Option 1-1 and Option 1-2 are simultaneously activated for the same UE

  - Note: this does not preclude the case when Option 1-1 and Option 1-2 are simultaneously configured for the same UE while only one of them can be activated

| Company | Y/N | Comments |
| --- | --- | --- |
| Qualcomm | Y |  |
| CMCC | Y |  |
| ZTE, Sanechips | Y |  |
| Nokia1 | Y |  |
| Apple |  | We think this can be discussed later after Proposal 3.1-3 becomes clearer. |
| OPPO |  | We are not sure the Option 1-1 and Option 1-2 had decided to support activation/deactivation on ceratin Option. Should we decuss the meaning of the activation? |
| MTK | Y |  |
| vivo |  | In our view, it is also no need to supported that the case when both Option 1-1 and Option 1-2 are simultaneously configuraed for the same UE. And the need and the potential benefit for the Option 1-1 and Option 1-2 to be simultaneously configured for the same UE should be clarified. |
| Sharp |  | The necessity to configuring both mode for a UE and only activing one needs clarification |
| ETRI | Y |  |
| Huawei, HiSilicon | N | We don’t see the necessity to switch between Option 1-1 and Option 1-2 dynamically by e.g., MAC CE. Therefore, we prefer the following change: - For LP-WUS CONNECTED mode operation, it is NOT supported that the case when both Option 1-1 and Option 1-2 are simultaneously activated configured for the same UE Note: this does not preclude the case when Option 1-1 and Option 1-2 are simultaneously configured for the same UE while only one of them can be activated |
| Qualcomm |  | If companies want to decouple the discussion between activation/deactivation and simultaneous option 1-1 + option 1-2, we think following update would work. - For LP-WUS CONNECTED mode operation, it is NOT supported that the case when a UE monitors LP-WUS based on both Option 1-1 and Option 1-2 are simultaneously on a serving cell activated for the same UE - Note: this does not preclude the case when Option 1-1 and Option 1-2 are simultaneously configured for the same UE while only one of them can be activated Suggest to add ”on a serving cell”, since RAN1 already agreed that a UE can be configured with LP-WUS monitoring in different serving cells of different cell-groups in NR-DC. |

[Close]Proposal 3.1-2a:

Alt1

- For LP-WUS CONNECTED mode operation, it is NOT supported that a UE monitors LP-WUS based on Option 1-1 and Option 1-2 simultaneously for a given cell group

Alt2

- For LP-WUS CONNECTED mode operation, it is NOT supported that the case when both Option 1-1 and Option 1-2 are simultaneously configured for the same UE in a given cell group

| Company | Y/N | Comments |
| --- | --- | --- |
| Moderator |  | Following was agreed in Wednesday online: Agreement For LP-WUS CONNECTED mode operation, the following case is not supported: - Both Option 1-1 and Option 1-2 are simultaneously configured for the same UE in a given cell |

Regarding the support of Option 1-3, which has not been agreed yet, a few companies propose to support it while some others propose to deprioritize it. As pointed by some companies, one potential way forward is to support additional indication/condition to activate LP-WUS monitoring inside C-DRX active time on top of Option 1-2, so that it is not necessary to specify the procedure from scratch.

- Support: Apple, Xiaomi

  - Can be achieved by Option 1-2 + additional indication/condition to activate LP-WUS monitoring

- Deprioritize: Nokia

Example of Option 1-2 + additional indication to activate LP-WUS monitoring

[Close]Proposal 3.1-3:

- For Option 1-3 of LP-WUS CONNECTED mode operation, the followings are assumed on top of Option 1-2 from RAN1 perspective.

LP-WUS monitoring configuration based on Option 1-2

LP-WUS monitoring inside C-DRX active time based on additional indication/condition, to trigger PDCCH monitoring.

    - FFS details of the additional indication/condition, based on L1/L2 signaling or timer

    - UE is not required to monitor PDCCH when monitoring LP-WUS in the Active Time

UE is configured with legacy C-DRX configurations as Rel-18

UE behaviors related to C-DRX active time triggered by all legacy DRX timers and the new timer triggered by LP-WUS are not affected unless included in this proposal

UE is expected to be configured with LP-WUS monitoring configuration (periodicity and offset can be different from those from C-DRX configuration)

| Company | Y/N | Comments |
| --- | --- | --- |
| Qualcomm | N (discuss) | We are fine and think it is beneficial to use an explicit indication to stop DRX Active Time with resuming LP-WUS monitoring (to realize something like Option 1-3). However, this indication does not need to be restricted to Option 1-2. Even for Option 1-1, the indication is useful, since the value(s) of drx-onDurationTimer and/or drx-InactivityTimer of C-DRX configuration is/are sometime unnecessarily long. Stop of these timers is useful even for Option 1-1. As for the explicit indication, we think such indication has already been supported in 38.321 as DRX Command MAC CE / Long DRX Command MAC CE since Rel-15. These indications can stop drx-onDurationTimer and drx-InactivityTimer of the on-going DRX Active Time. For Rel-16 DCP, it is possible to use these MAC CE commands to stop DRX Active Time of the current DRX cycle, and resume DCP monitoring for next DRX cycle. Therefore, use of these MAC CE commands for LP-WUS Option 1-1 and Option 1-2 is straightforward approach. The question is, whether the existing MAC CEs (L2 control) are sufficient, or new signalling (e.g., L1 control) is necessary for LP-WUS. We are not yet sure about the necessity and would like to discuss further. Having said that, we suggest following. - For Option 1-1 and Option 1-2, an explicit signalling can indicate stop of DRX Active Time - For Option 1-1, the signalling indicates stop of drx-onDurationTimer and drx-InactivityTimer for the DRX group - For Option 1-2, the signalling indicates stop of the new timer for PDCCH monitoring in Option 1-2, and drx-InactivityTimer for the DRX group - At least DRX Command MAC CE and Long DRX Command MAC CE can be used as the explicit signalling (same as legacy) - FFS: any other new explicit signalling is supported |
| CMCC |  | We should first discuss whether Option 1-3 is supported. |
| ZTE, Sanechips |  | If proposal 3.1-1 is approved, LP-WUS monitoring inside C-DRX active time can not be supported. |
| Nokia1 | N | The continued study of the use of the LP-WUS within the On-Duration itself, Option 1-3, should be deprioritized for the following reasons: • Limited TUs for the WI -- adding option 1-3 is non-trivial, it raises additional issues, such as, (1) need for new minimum gap (2) restricted placement of LPWUS (3) new UE capablity (4) RAN2 TU • Pre-existing mechanisms such as PDCCH Skipping or Search Space Switching, to reduce PDCCH monitoring in CONNECTED mode. |
| Apple | Y (also with Option 1-1) | We agree with QC on the observation that even for Option 1-1, the drx-onDurationTimer and/or drx-InactivityTimer may be unnecessarily too long, therefore, we believe it is important that UE can be indicated to monitor LP-WUS during the Active Time, to save power and resume back to PDCCH monitoring to ensure latency. However, for the proposal to completely stop the drx-onDurationTimer, it should be noted that for Option 1-1, if the signaling indicates stop of drx-onDurationTimer or drx-InactivityTime only, the UE can only be waken up in the next cycle and the latency would be increased. |
| Lenovo |  | We should first discuss whether Option 1-3 is supported. |
| LGE |  | As Qualcomm commented, we think that an explicit signaling to stop DRX active time can be useful. However, which signal can be used for this? If proposal 3.1-1 is agreed, LP-WUS cannot be monitored in DRX active time, so LP-WUS itself cannot be used for signaling to stop DRX active time. |
| OPPO |  | We could consider this 1-3 if we had concluded that the Option 1-1 and 1-2 not configured same time. Some more clarification may be needed e.g. the relation to proposal 3.1-1 |
| MTK |  | Prefer NO LP-WUS monitoring inside C-DRX active time. |
| Huawei, HiSilicon |  | We think this issue include two sub-issues: - Sub-issue#1: Whether to support using L1/L2 signalling to dynamically stop PDCCH monitoring within Active Time. - Sub-issue#2: if the L1/L2 signalling is supported, whether the Active Time is stopped by the L1/L2 signalling. For sub-issue#1, we support it, since L1/L2 signalling is efficient to adapt LP-WUS/PDCCH monitoring to traffic status. For sub-issue#2, we are either fine for the following two ways, which achieve the same effect: - Active Time is stopped by the L1/L2 signalling, and in this case Option 1-3 is not necessarily to be supported. Active Time is not stopped by the L1/L2 signalling, and in this case Option 1-3 should be supported, otherwise LP-WUS cannot be monitored when PDCCH monitoring is stopped. |

[Close]Proposal 3.1-3a:

- For Option 1-1 and Option 1-2 of LP-WUS CONNECTED mode operation, an explicit signalling can indicate stop of DRX Active Time

For Option 1-1, the signalling indicates stop of drx-onDurationTimer and drx-InactivityTimer for the DRX group

For Option 1-2, the signalling indicates stop of the new timer for PDCCH monitoring in Option 1-2, and drx-InactivityTimer for the DRX group

At least existing DRX Command MAC CE and Long DRX Command MAC CE can be used as the explicit signalling (same as legacy)

    - FFS: any other new explicit signalling is supported

Note: This intends not to support Option 1-3 as standalone feature

| Company | Y/N | Comments |
| --- | --- | --- |
| Qualcomm |  | 38.321 already supports DRX Command MAC CE and Long DRX Command MAC CE that indicate stop of drx-onDurationTimer and drx-InactivityTimer for the DRX group. Since this part does not need to be agreed (as already there) we suggest to decouple the cases as follows. (For RAN1 conclusion) - For Option 1-1, RAN1 assumes existing DRX Command MAC CE and Long DRX Command MAC CE can be used to stop drx-onDurationTimer and drx-InactivityTimer for the DRX group When UE is not in C-DRX active time, UE goes back to LPWUS monitoring (For RAN1 agreement) - For Option 1-2, RAN1 assumes existing DRX Command MAC CE and Long DRX Command MAC CE can be used to stop the new timer for PDCCH monitoring in Option 1-2 and drx-InactivityTimer for the DRX group When UE is not in C-DRX active time, UE goes back to LPWUS monitoring [this sub-bullet can be deleted since this was already agreed in RAN2] (For RAN1 agreement) - RAN1 studies if any other additional explicit signalling is supported for Option 1-1 and 1-2 |
| xiaomi |  | We think it is a RAN2 issue on how to stop the active timer. |

[Close]Proposal 3.1-3a:

- For Option 1-1 and Option 1-2 of LP-WUS CONNECTED mode operation, an explicit signalling can indicate stop of DRX Active Time

For Option 1-1, the signalling indicates stop of drx-onDurationTimer and drx-InactivityTimer for the DRX group

For Option 1-2, the signalling indicates stop of the new timer for PDCCH monitoring in Option 1-2, and drx-InactivityTimer for the DRX group

At least existing DRX Command MAC CE and Long DRX Command MAC CE can be used as the explicit signalling (same as legacy)

    - FFS: any other new explicit signalling is supported

Note: This intends not to support Option 1-3 as standalone feature

[Close]Proposed conclusion 3.1-3b:

- For Option 1-1 of LP-WUS CONNECTED mode operation, RAN1 assumes existing DRX Command MAC CE and Long DRX Command MAC CE can be used to stop drx-onDurationTimer and drx-InactivityTimer for the DRX group

When UE is not in C-DRX active time, UE monitors LP-WUS

- Send LS to RAN2 to ask whether RAN2 has same understanding or not.

[Close]Proposal 3.1-3c:

- For Option 1-2 of LP-WUS CONNECTED mode operation, RAN1 considers following two options of explicit L1/L2 signalling to stop the new timer for PDCCH monitoring in Option 1-2 and drx-InactivityTimer for the DRX group

Option 1: existing DRX Command MAC CE and Long DRX Command MAC CE

Option 2: additional explicit L1/L2 signalling

When UE is not in C-DRX active time, UE monitors LP-WUS

- Send LS to RAN2 to ask the feasibility/necessity of these options.

| Company | Y/N | Comments |
| --- | --- | --- |
| Moderator |  | Following was agreed in Wednesday online: Conclusion - For Option 1-1 of LP-WUS CONNECTED mode operation, RAN1 assumes existing DRX Command MAC CE and Long DRX Command MAC CE can be used to stop drx-onDurationTimer and/or drx-InactivityTimer for the DRX group - When UE is not in C-DRX active time, UE monitors LP-WUS - Send LS to RAN2 to ask whether RAN2 has same understanding or not. LS in R1-2410XXX. Agreement - For Option 1-2 of LP-WUS CONNECTED mode operation, for explicit indication to stop the drx-InactivityTimer for the DRX group, RAN1 identified the following potential solution - Use existing DRX Command MAC CE and Long DRX Command MAC CE - FFS: Applicability of this feature for new timer for PDCCH monitoring in Option 1-2 - When UE is not in C-DRX active time, UE monitors LP-WUS - Send LS to RAN2 to ask the feasibility of the above potential solution. LS in R1-2410XXX. Corresponding LS is to be further discussed, and to be approved on Friday. Note that any additional contents on LS can be added, if agreed by the end of this meeting. |

[LS]Question 3.1-3d:

- Companies are invited to review the drat LS, which is available in the following folder, corresponding to the conclusion/agreeement in this meeting, and provide comments, if any.

/RAN/RAN1/Inbox/drafts/9.6(NR_LPWUS)/9.6.3/LS

| Company | Comments |
| --- | --- |
| Moderator | The main body is copied here for reference --- - Overall Description: RAN1 made following agreement in RAN1#116, and later agreed to support Option 1-1 and Option 1-2 from RAN1 perspective. Meanwhile, Option 1-3 has not been agreed whether to support or not. Agreement - For RRC CONNECTED mode, from RAN1 perspective, further study following LP-WUS procedures to trigger PDCCH monitoring: - Case 1: PDCCH monitoring is triggered by LP-WUS with C-DRX configuration - Option 1-1: LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer. - This option may replace DCP functionality - Option 1-2: LP-WUS monitoring outside C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring. - PDCCH monitoring possibly irrespective of drx-onDurationTimer - Option 1-3: LP-WUS monitoring inside C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring. - Case 2: PDCCH monitoring is triggered by LP-WUS without C-DRX configuration. LP-WUS can be monitored at any time according to the LP-WUS monitoring configuration - FFS duty-cycled and/or continuous LP-WUS monitoring - Combination of options in Case 1 and combination of options in Case 1 and Case 2 are not precluded.. In RAN#119, RAN1 assumed Option 1-3 is not supported as standalone feature, while the same purpose can be achieved by using explicit L1/L2 signalling to stop C-DRX related timers and resuming LP-WUS monitoring. Accordingly, following conclusion and agreement were made: Conclusion - For Option 1-1 of LP-WUS CONNECTED mode operation, RAN1 assumes existing DRX Command MAC CE and Long DRX Command MAC CE can be used to stop drx-onDurationTimer and/or drx-InactivityTimer for the DRX group - When UE is not in C-DRX active time, UE monitors LP-WUS - Send LS to RAN2 to ask whether RAN2 has same understanding or not. LS in R1-2410XXX. [Wed]Proposal 3.1-3c: - For Option 1-2 of LP-WUS CONNECTED mode operation, for explicit indication to stop the drx-InactivityTimer for the DRX group, RAN1 identified the following potential solution - Use existing DRX Command MAC CE and Long DRX Command MAC CE - FFS: Applicability of this feature for new timer for PDCCH monitoring in Option 1-2 - When UE is not in C-DRX active time, UE monitors LP-WUS - Send LS to RAN2 to ask the feasibility of the above potential solution. LS in R1-2410XXX.. Regarding the above conclusion on Option 1-1, RAN1 would like to ask RAN2 whether RAN2 has same understanding or not. Regarding the above agreement on Option 1-2, RAN1 would like to ask RAN2 the feasibility of the above potential solution. 2. Actions: To RAN WG2 ACTION: RAN1 respectfully asks RAN2 to reply to the questions above. |
| Apple | We do not agree that the same purpose and benefit of Option 1-3 can be achieved by the RAN1 agreement to use existing MAC CE signaling. In Option 1-3, the active timer can be still running and UE can monitor LP-WUS during the Active Time with smaller periodicity to be able to wake up again in a faster way. However, based on the above RAN1 agreement, the timer is stopped based on the MAC CE command, then UE can only monitor the LP-WUS in the next cycle. For Option 1-1, the UE can only monitoring the MOs before the next C-DRX cycle. Nevertheless, we think that the LS should focus on sending the agreements and questions only. We suggest the following modifications: RAN1 made following agreement in RAN1#116, and later agreed to support Option 1-1 and Option 1-2 from RAN1 perspective. Meanwhile, Option 1-3 has not been agreed whether to support or not. Agreement - For RRC CONNECTED mode, from RAN1 perspective, further study following LP-WUS procedures to trigger PDCCH monitoring: - Case 1: PDCCH monitoring is triggered by LP-WUS with C-DRX configuration - Option 1-1: LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer. - This option may replace DCP functionality - Option 1-2: LP-WUS monitoring outside C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring. - PDCCH monitoring possibly irrespective of drx-onDurationTimer - Option 1-3: LP-WUS monitoring inside C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring. - Case 2: PDCCH monitoring is triggered by LP-WUS without C-DRX configuration. LP-WUS can be monitored at any time according to the LP-WUS monitoring configuration - FFS duty-cycled and/or continuous LP-WUS monitoring - Combination of options in Case 1 and combination of options in Case 1 and Case 2 are not precluded.. In RAN#119, RAN1 assumed Option 1-3 is not supported as standalone feature, while the same purpose can be achieved by using made the following conclusion and agreement to use explicit L1/L2 signalling to stop C-DRX related timers and resuming LP-WUS monitoring. Accordingly, following conclusion and agreement were made: Conclusion - For Option 1-1 of LP-WUS CONNECTED mode operation, RAN1 assumes existing DRX Command MAC CE and Long DRX Command MAC CE can be used to stop drx-onDurationTimer and/or drx-InactivityTimer for the DRX group - When UE is not in C-DRX active time, UE monitors LP-WUS - Send LS to RAN2 to ask whether RAN2 has same understanding or not. LS in R1-2410XXX. [Wed]Proposal 3.1-3c: - For Option 1-2 of LP-WUS CONNECTED mode operation, for explicit indication to stop the drx-InactivityTimer for the DRX group, RAN1 identified the following potential solution - Use existing DRX Command MAC CE and Long DRX Command MAC CE - FFS: Applicability of this feature for new timer for PDCCH monitoring in Option 1-2 - When UE is not in C-DRX active time, UE monitors LP-WUS - Send LS to RAN2 to ask the feasibility of the above potential solution. LS in R1-2410XXX.. Regarding the above conclusion on Option 1-1, RAN1 would like to ask RAN2 whether RAN2 has same understanding or not. Regarding the above agreement on Option 1-2, RAN1 would like to ask RAN2 the feasibility of the above potential solution. |
| Moderator | Based on the coment from Apple, I prepared another version v002 without any background information. Let’s quickly check during Friday offline which version to take. In addition, RAN2 made following agreement in this RAN2 meeting. For option 1-2, if UE receives DRX command MAC CE or Long DRX command MAC CE, UE stops the new timer triggered by LP-WUS. Accordingly, RAN1 can update the previous agreement. |

[Fri]Proposal 3.1-3e:

Update the following agreement in red:

Agreement

- For Option 1-2 of LP-WUS CONNECTED mode operation, for explicit indication to stop the new timer for PDCCH monitoring in Option 1-2 and drx-InactivityTimer for the DRX group, RAN1 identified the following potential solution

  - Use existing DRX Command MAC CE and Long DRX Command MAC CE

    - FFS: Applicability of this feature for new timer for PDCCH monitoring in Option 1-2

  - When UE is not in C-DRX active time, UE monitors LP-WUS

- Send LS to RAN2 to ask the feasibility of the above potential solution. LS in R1-2410XXX.

| Company | Y/N | Comments |
| --- | --- | --- |
| Moderator |  | See the comment above |

[Fri]Proposal 3.1-3f:

Endorse draft LS to RAN2 in R1-2410908, final LS in R1-2410909

| Company | Y/N | Comments |
| --- | --- | --- |
| Moderator |  | Draft LS based on either v001 or v002 in “/RAN/RAN1/Inbox/drafts/9.6(NR_LPWUS)/9.6.3/LS” |
| Moderator |  | Following agreement/conclusion were made in this meeting Conclusion - For Option 1-1 of LP-WUS CONNECTED mode operation, RAN1 assumes existing DRX Command MAC CE and Long DRX Command MAC CE can be used to stop drx-onDurationTimer and/or drx-InactivityTimer for the DRX group - When UE is not in C-DRX active time, UE monitors LP-WUS - Send LS to RAN2 to ask whether RAN2 has same understanding or not. LS in R1-2410909. Agreement - For Option 1-2 of LP-WUS CONNECTED mode operation, for explicit indication to stop the new timer for PDCCH monitoring in Option 1-2 and drx-InactivityTimer for the DRX group, RAN1 identified the following potential solution - Use existing DRX Command MAC CE and Long DRX Command MAC CE - FFS: Applicability of this feature for new timer for PDCCH monitoring in Option 1-2 - When UE is not in C-DRX active time, UE monitors LP-WUS - Send LS to RAN2 to ask the feasibility of the above potential solution. LS in R1-2410909. |

Regarding the periodic CSI/L1-RSRP report, some companies proposed to clarify the UE behaviour and following proposal is made accordingly

[Old]Proposal 3.1-4:

- For LP-WUS CONNECTED mode operation, the periodic CSI/L1-RSRP reporting operation is same as Rel-16 DCP, that is the UE can be configured with a parameter to enable/disable periodic CSI/L1-RSRP reporting, respectively:

  - If the parameter is NOT configured:

    - If the UE is not indicated to wake up by LP-WUS, the periodic CSI/L1-RSRP is not reported.

    - If the UE is indicated to wake up by LP-WUS, the periodic CSI/L1-RSRP is reported during the DRX active time as in legacy specification.

  - If the parameter is configured:

    - If the UE is not indicated to wake up by LP-WUS, the periodic CSI/L1-RSRP is reported during the time given by the configured drx-onDurationTimer in DRX-Config, even if the drx-onDurationTimer does not start.

    - If the UE is indicated to wake up by LP-WUS, the periodic CSI/L1-RSRP is reported during the DRX active time as in legacy specification.

| Company | Y/N | Comments |
| --- | --- | --- |
| Qualcomm | Y |  |
| CMCC | Y |  |
| ZTE, Sanechips |  | For the case if the parameter is not configured, whether DRX active time includes the active time triggered by new timer? |
| Samsung | Partially Yes | We suggest the following changes to be aligned with R16 spec: - For LP-WUS CONNECTED mode operation, the periodic CSI/L1-RSRP reporting operation is same as Rel-16 DCP, that is the UE can be configured with a parameter to enable/disable periodic CSI/L1-RSRP reporting, respectively: - If the parameter is NOT configured: - If the UE is not indicated to wake up by LP-WUS, the periodic CSI/L1-RSRP is not reported during the time given by the configured drx-onDurationTimer in DRX-Config and outside the DRX active time. - If the UE is indicated to wake up by LP-WUS, the periodic CSI/L1-RSRP is reported during the DRX active time as in legacy specification. - If the parameter is configured: - If the UE is not indicated to wake up by LP-WUS, the periodic CSI/L1-RSRP is reported during the time given by the configured drx-onDurationTimer in DRX-Config and outside the DRX active time, even if the drx-onDurationTimer does not start. - If the UE is indicated to wake up by LP-WUS, the periodic CSI/L1-RSRP is reported during the DRX active time as in legacy specification. Note: no RAN1 spec impact is expected. |
| Nokia1 |  | For option 1-2, we would like to clarify what the following text means: ” is reported during the DRX active time as in legacy specification.” We see 2 interpretations …. Option 1: legacy ACTIVE time linked to the legacy ON-duration? Option 2: extends to new ACTIVE times triggered between legacy ON-duration |
| Spreadtrum | Y |  |
| Apple | Y | Samsung’s modified version is OK. |
| ETRI | Y |  |
| Lenovo | Y |  |
| LGE |  | In this proposal, do we assume that one parameter for LP-WUS play a role for two legacy parameters (ps-TransmitPeriodicL1-RSRP, ps-TransmitOtherPeriodicCSI)? |
| OPPO | Y |  |
| MTK | Y | If the parameter is NOT configured: If the UE is not indicated to wake up by LP-WUS, the periodic CSI/L1-RSRP is not reported during the time given by the configured drx-onDurationTimer in DRX-Config and outside the DRX active time. |
| vivo | Y |  |

[Old]Proposal 3.1-4a:

- For LP-WUS CONNECTED mode operation, the periodic CSI/L1-RSRP reporting operation is same as Rel-16 DCP, that is the UE can be configured with a parameter to enable/disable periodic CSI/L1-RSRP reporting, respectively:

  - If the parameter is NOT configured:

    - If the UE is not indicated to wake up by LP-WUS, the periodic CSI/L1-RSRP is not reported.

    - If the UE is indicated to wake up by LP-WUS, the periodic CSI/L1-RSRP is reported during the DRX active time as in legacy specification.

  - If the parameter is configured:

    - If the UE is not indicated to wake up by LP-WUS, the periodic CSI/L1-RSRP is reported during the time given by the configured drx-onDurationTimer in DRX-Config, even if the drx-onDurationTimer does not start.

    - If the UE is indicated to wake up by LP-WUS, the periodic CSI/L1-RSRP is reported during the DRX active time as in legacy specification.

  - Note: no RAN1 spec impact is expected.

| Company | Y/N | Comments |
| --- | --- | --- |
| Moderator |  | Following agreement was made in Monday Online: Agreement - For 1-2, for LP-WUS CONNECTED mode operation, the periodic CSI/L1-RSRP reporting operation is same as Rel-16 DCP, that is the UE can be configured with a parameter to enable/disable periodic CSI/L1-RSRP reporting, respectively: - If the parameter is NOT configured: - If the UE is not indicated to wake up by LP-WUS, the periodic CSI/L1-RSRP is not reported. - If the UE is indicated to wake up by LP-WUS, the periodic CSI/L1-RSRP is reported during the DRX active time. - If the parameter is configured: - If the UE is not indicated to wake up by LP-WUS, the periodic CSI/L1-RSRP is reported during the time given by the configured drx-onDurationTimer in DRX-Config [and outside DRX active time], even if the drx-onDurationTimer does not start. - If the UE is indicated to wake up by LP-WUS, the periodic CSI/L1-RSRP is reported during the DRX active time. - FFS: Whether RAN1 spec impact is required. Regarding ”[and outside DRX active time]”, moderator’s understanding is that this is aligned with TS38.321 procedure as reffered below (expecially highlighted part) and can include any DRX active time. Hence, this can be confirmed |
| Clause 5.7 in TS38.321 | - ps-TransmitOtherPeriodicCSI (optional): the configuration to report periodic CSI that is not L1-RSRP on PUCCH during the time duration indicated by drx-onDurationTimer in case DCP is configured but associated drx-onDurationTimer is not started; - ps-TransmitPeriodicL1-RSRP (optional): the configuration to transmit periodic CSI that is L1-RSRP on PUCCH during the time duration indicated by drx-onDurationTimer in case DCP is configured but associated drx-onDurationTimer is not started; (omitted) 1> if DCP monitoring is configured for the active DL BWP as specified in TS 38.213 [6], clause 10.3; and 1> if the current symbol n occurs within drx-onDurationTimer duration; and 1> if drx-onDurationTimer associated with the current DRX cycle is not started as specified in this clause: 2> if the MAC entity would not be in Active Time considering grants/assignments/DRX Command MAC CE/Long DRX Command MAC CE received and Scheduling Request sent until 4 ms prior to symbol n when evaluating all DRX Active Time conditions as specified in this clause; and 2> if allowCSI-SRS-Tx-MulticastDRX-Active is not configured or, if all multicast DRXes would not be in Active Time considering multicast assignments/DRX Command MAC CE for MBS multicast received until 4 ms prior to symbol n when evaluating all DRX Active Time conditions as specified in Clause 5.7b and all multicast sessions are configured with multicast DRX: 3> not transmit periodic SRS and semi-persistent SRS defined in TS 38.214 [7]; 3> not report semi-persistent CSI configured on PUSCH; 3> if ps-TransmitPeriodicL1-RSRP is not configured with value true: 4> not report periodic CSI that is L1-RSRP on PUCCH. 3> if ps-TransmitOtherPeriodicCSI is not configured with value true: 4> not report periodic CSI that is not L1-RSRP on PUCCH. |  |
| Qualcomm | Regarding [and outside DRX active time], we think the corresponding spec, mentioned during online session by some companies, is following in 38.214 5.2.2.5: In our understanding, the “also outside active time” in the above spec is meant to say “even if drx-onDurationTimer is not started”. This can be understood from the other highlighted part “during the time duration indicated by drx-onDurationTimer in DRX-Config outside DRX active time or in DRX Active Time”. The agreement already includes “even if the drx-onDurationTimer does not start”. Therefore, we think [and outside DRX active time] is not necessary and should be deleted. |  |

[Old]Proposal 3.1-4b:

Update following agreement in red:

Agreement

- For Option 1-2, for LP-WUS CONNECTED mode operation, the periodic CSI/L1-RSRP reporting operation is same as Rel-16 DCP, that is the UE can be configured with a parameter to enable/disable periodic CSI/L1-RSRP reporting, respectively:

  - If the parameter is NOT configured:

    - If the UE is not indicated to wake up by LP-WUS, the periodic CSI/L1-RSRP is not reported.

    - If the UE is indicated to wake up by LP-WUS, the periodic CSI/L1-RSRP is reported during the DRX active time.

  - If the parameter is configured:

    - If the UE is not indicated to wake up by LP-WUS, the periodic CSI/L1-RSRP is reported during the time given by the configured drx-onDurationTimer in DRX-Config [and outside DRX active time], even if the drx-onDurationTimer does not start.

    - If the UE is indicated to wake up by LP-WUS, the periodic CSI/L1-RSRP is reported during the DRX active time.

  - FFS: Whether RAN1 spec impact is required.

| Company | Y/N | Comments |
| --- | --- | --- |
| Moderator |  | Please also see moderator’s comment in Proposal 3.1-4a |
| Huawei, HiSilicon |  | We are generally fine with the proposal. Per our understanding, this proposal is a clarification to previous agreement, which applies to both Option 1-1 and Option 1-2. Not sure why we emphasize Option 1-2 in the main bullet. |
| CMCC | Y |  |
| Qualcomm |  | As we input earlier, we suggest to delete ” and outside DRX active time” as follows. ”and outside DRX active time” implies the UE shall report periodic CSI/L1-RSRP all over the time. - For Option 1-2, for LP-WUS CONNECTED mode operation, the periodic CSI/L1-RSRP reporting operation is same as Rel-16 DCP, that is the UE can be configured with a parameter to enable/disable periodic CSI/L1-RSRP reporting, respectively: - If the parameter is NOT configured: - If the UE is not indicated to wake up by LP-WUS, the periodic CSI/L1-RSRP is not reported. - If the UE is indicated to wake up by LP-WUS, the periodic CSI/L1-RSRP is reported during the DRX active time. - If the parameter is configured: - If the UE is not indicated to wake up by LP-WUS, the periodic CSI/L1-RSRP is reported during the time given by the configured drx-onDurationTimer in DRX-Config [and outside DRX active time], even if the drx-onDurationTimer does not start. - If the UE is indicated to wake up by LP-WUS, the periodic CSI/L1-RSRP is reported during the DRX active time. - FFS: Whether RAN1 spec impact is required. |
| OPPO |  | If eidotrial change needed, suggest to change wht wording accordingly. ” the periodic CSI/L1-RSRP can be reported during the time given by the configured drx-onDurationTimer in DRX-Config [and outside DRX active time]” The last row is also ”can be” This means the report is allowed base on configuration, but not always. |
| xiaomi |  | Share similar view as Qualcomm. |

[Old]Proposal 3.1-4c:

Update following agreement in red:

Agreement

- For Option 1-2, for LP-WUS CONNECTED mode operation, the periodic CSI/L1-RSRP reporting operation is same as Rel-16 DCP, that is the UE can be configured with a parameter to enable/disable periodic CSI/L1-RSRP reporting, respectively:

  - If the parameter is NOT configured:

    - If the UE is not indicated to wake up by LP-WUS, the periodic CSI/L1-RSRP is not reported.

    - If the UE is indicated to wake up by LP-WUS, the periodic CSI/L1-RSRP is reported during the DRX active time.

  - If the parameter is configured:

    - If the UE is not indicated to wake up by LP-WUS, the periodic CSI/L1-RSRP is reported during the time given by the configured drx-onDurationTimer in DRX-Config [and outside DRX active time], even if the drx-onDurationTimer does not start.

    - If the UE is indicated to wake up by LP-WUS, the periodic CSI/L1-RSRP is reported during the DRX active time.

  - FFS: Whether RAN1 spec impact is required.

| Company | Y/N | Comments |
| --- | --- | --- |
| Moderator |  | Updated based on companies comments |

[Fri]Proposal 3.1-4c:

Update following agreement in red:

Agreement

- For Option 1-2, for LP-WUS CONNECTED mode operation, the periodic CSI/L1-RSRP reporting operation is same as Rel-16 DCP, that is the UE can be configured with a parameter to enable/disable periodic CSI/L1-RSRP reporting, respectively:

  - If the parameter is NOT configured:

    - If the UE is not indicated to wake up by LP-WUS, the periodic CSI/L1-RSRP is not reported.

    - If the UE is indicated to wake up by LP-WUS, the periodic CSI/L1-RSRP is reported during the DRX active time.

  - If the parameter is configured:

    - If the UE is not indicated to wake up by LP-WUS, the periodic CSI/L1-RSRP is reported during the time given by the configured drx-onDurationTimer in DRX-Config [and outside DRX active time], even if the drx-onDurationTimer does not start.

    - If the UE is indicated to wake up by LP-WUS, the periodic CSI/L1-RSRP is reported during the DRX active time.

  - FFS: Whether RAN1 spec impact is required.

| Company | Y/N | Comments |
| --- | --- | --- |
| Moderator |  | RAN2 made following agreement in this RAN2 meeting: The drx-onDurationTimer is not started with Option 1-2 LP-WUS. For Option 1-2, network can configure whether UE reports periodic CSI/L1-RSRP during the time given by the configured drx-onDurationTimer, for the case when UE is outside C-DRX active time. Accordingly, the proposal is updated. |
| Moderator |  | Following agreement was made in this meeting Agreement - For Option 1-2, for LP-WUS CONNECTED mode operation, the periodic CSI/L1-RSRP reporting operation is same as Rel-16 DCP, that is the UE can be configured with a parameter to enable/disable periodic CSI/L1-RSRP reporting, respectively: - If the parameter is NOT configured: - If the UE is not indicated to wake up by LP-WUS, the periodic CSI/L1-RSRP is not reported. - If the UE is indicated to wake up by LP-WUS, the periodic CSI/L1-RSRP is reported during the DRX active time. - If the parameter is configured: - If the UE is not indicated to wake up by LP-WUS, the periodic CSI/L1-RSRP is reported during the time given by the configured drx-onDurationTimer in DRX-Config for the case when UE is outside C-DRX active time. - If the UE is indicated to wake up by LP-WUS, the periodic CSI/L1-RSRP is reported during the DRX active time. - FFS: Whether RAN1 spec impact is required. |

3.2	Timeline between LP-WUS reception/detection and PDCCH monitoring

Regarding the timeline LP-WUS reception/detection and PDCCH monitoring, following agreements were made in previous RAN1 meetings:

Agreement For RRC CONNECTED mode, minimum time gap between LP-WUS reception and MR to start PDCCH monitoring is introduced considering at least following - LP-WUS processing time - MR transition time for ramp up - Time/frequency synchronization of MR - FFS whether UE can report supported minimum time gap from candidate values FFS: Whether the minimum time gap values can be more than one Agreement For RRC CONNECTED mode, support UE capability report for determination of minimum time gap between LP-WUS reception and MR to start PDCCH monitoring. - FFS: exact value(s) of the minimum time gap - FFS: support of multiple minimum time gaps - FFS whether the reported value includes the duration for time/frequency synchronization of MR Agreement Select one of the following alternatives in RAN1#118bis Alt 1 For RRC CONNECTED mode, UE reports one value for each SCS from X candidate values for the determination of the minimum time gap between LP-WUS reception and MR to start PDCCH monitoring via UE capability reporting. FFS: X FFS: definition of reported value Alt2 For RRC CONNECTED mode, UE reports multiple values for each SCS from X candidate values for the determination of the minimum time gap between LP-WUS reception and MR to start PDCCH monitoring via UE capability reporting. FFS: X FFS: definition of reported value Different minimum time gaps correspond to different sleep states Companies are encouraged to details on how the reported values are to be used by the network Agreement For RRC CONNECTED mode, UE reports one value for each SCS from X candidate values for the determination of the minimum time gap between LP-WUS reception and MR to start PDCCH monitoring via UE capability reporting. FFS: X FFS: definition of reported candidate value UE can indicate its preference for configured time offset between LP-WUS and PDCCH reception using UAI mechanism. FFS details. UAI is optional. FFS: A single time offset is configured for a UE at a given time

A number of companies provided their view on the FFSs in the agreements:

- FFS: X

  - 2: vivo, E///, Samsung, ETRI, TCL, MTK

  - 3: vivo, Xiaomi, OPPO,

- FFS: definition of reported candidate value

  - 10ms (deep sleep state) for all SCS: QC

  - 20 ms, 6 ms and 2 ms: OPPO

  - The candidate value should differ from different WUR type: ZTE

  - reusing the minimum time gap values of DCI format 2-6 from 38.213 Table 10.3-1: MTK

- FFS whether the reported value includes the duration for time/frequency synchronization of MR

  - Yes: E///, Samsung, Pana, CATT, LGE, Lenovo

    - The duration for time/frequency synchronization of MR should be included for accurate synchronization of MR using SSB before PDCCH MO. The value may depend on the sleep mode of MR, which is further determined by how much clock accuracy is maintained in MR, the time offset from detected LP-WUS to SSB, and the number of sampled SSB bursts used for measurement and tracking

  - No: vivo, HW/HiSi (w/ additional reporting of required number of SSB/TRS for time/synchronization), CMCC, ZTE, DCM,

    - MR receives SSB for RLM/BFD/CSI and RRM measurements periodically

- Reference SCS

  - Carrier where LP-WUS is monitored: vivo

- UAI details

  - Up to RAN2: vivo

  - the value indicated by UAI must be larger than the duration consisting of the one value reported from the X candidate values and the time duration required for synchronization: HW

  - Candidate values for the indicated preferred gap value by UAI shall be same as the candidate values for the UE capability reporting: Samsung

  - UE could additional report another smaller gap than its capability report as UAI: OPPO

- FFS: A single time offset is configured for a UE at a given time

  - Support: vivo, E///, Sony

  - Not support: IDC (more than one)

Examples for the time gap between LP-WUS and PDCCH monitoring (from R1-2409425)

Regarding X and the exact value(s) of the minimum time gap, following can be considered as starting point according to companies input.

[Old]Proposal 3.2-1:

- For RRC CONNECTED mode, the number X of candidate values for the determination of the minimum time gap between LP-WUS reception and MR to start PDCCH monitoring via UE capability reporting is up to 3.

  - The reported candidate values are selected from {2, 6, 10, 20}ms

| Company | Y/N | Comments |
| --- | --- | --- |
| xiaomi | Y | Generally OK. |
| CMCC | Y | Fine with the proposal |
| ZTE, Sanechips | N | For OOK WUR, it could be possible, for OFDM WUR, it should reuse the values for DCP or view that as starting point. Additionally, 3ms means LP-WUS can not be used for XR traffic, since it would cause obvious capacity loss. |
| Panasonic | Y | We are open to discuss the specific values for the reported candidate values. |
| Samsung | N | We didn’t see a need to support additional values comparing to DCP, i.e., 2 is sufficient. |
| Nokia1 |  | From other contributions, we see that the majority favour this ”sleep mode” linked approach. Our only concern with this approach, is whether or not it will be impacted later by factors such as: LP-WUS placement relative to the PDCCH (time for retuning?) The operation of CA/DC (staggered wake of multiple radios?) The mode of operation – mode 1-1 or mode 1-2 (and even 1-3!) Availability/density of sync sources |
| Apple | Y | Specific values can be discussed further |
| ETRI | N | There are more companies supporting 2 instead of 3. We also think 2 is sufficient. |
| LGE |  | We prefer two candidate values same as DCP. Also, different types of LP-WUR should be considered. |
| OPPO | Y | We are OK for the proposal. The values candidate may be added with “[]” for further decision on UE capability. |
| MTK | N | Revised proposal: For RRC CONNECTED mode, the number X of candidate values for the determination of the minimum time gap between LP-WUS reception and MR to start PDCCH monitoring via UE capability reporting is up to 2. |
| vivo | Y | These candidate values can be a starting point. |
| FL |  | DCP capability for reference Table 10.3-1 Minimum time gap value X SCS (kHz); Minimum Time Gap X (slots). Value 1; Value 2. 15; 1; 3. 30; 1; 6. 60; 1; 12. 120; 2; 24. 480; 8; 96. 960; 16; 192. |

[Old]Proposal 3.2-1a:

- For RRC CONNECTED mode, the number X of candidate values for the determination of the minimum time gap between LP-WUS reception and MR to start PDCCH monitoring via UE capability reporting is [2, 3, 4].

  - The reported candidate values are selected from {0.25, 0.5, 1, 2, 3, 6, 10, 20}ms as starting point

    - Note: Other values are not precluded

    - FFS whether different values are selected for OOK-based WUR and OFDM-based WUR

  - FFS whether/how to consider the case of CA

[Old]Proposal 3.2-1b:

- For RRC CONNECTED mode, the number X of candidate values for the determination of the minimum time gap between LP-WUS reception and MR to start PDCCH monitoring via UE capability reporting is [2, 3, 4].

  - The reported candidate values are selected from {0.25, 0.5, 1, 2, 3, 6, 10, 20}ms as starting point

    - Note: Other values are not precluded

    - FFS whether different values are selected for OOK-based WUR and OFDM-based WUR

  - FFS whether/how to consider the case of CA

  - FFS whether the reported value includes the duration for time/frequency synchronization of MR

| Company | Y/N | Comments |
| --- | --- | --- |
| Moderator |  | Proposal 3.2-1a was discussed in Monday online but no consensus was achieved. Another FFS is added based on the comment during online, which can be furtehr discussed in another proposal 3.2-2. |
| Huawei, HiSilicon | Y |  |
| CMCC | Y | Fine with the proposal |
| Qualcomm |  | For the last FFS, we suggest following: - FFS whether the reported value includes the a time duration for time/frequency synchronization of MR is necessary in addition to the reported minimum time gap between LP-WUS reception and MR to start PDCCH monitoring |
| OPPO | Y |  |
| xiaomi | Y |  |

Regarding whether the reported value includes the duration for time/frequency synchronization of MR, companies have divergent views. One of the motivations not to include the duration for time/frequency synchronization of MR would be that, time/frequency synchronization of MR is not related to the UE capability but the duration where sufficient number of sync sources, such as SSB and TRS, can be received after MR wake up. In this case, minimum time gap Xmin between LP-WUS reception and MR to start PDCCH monitoring can be expressed as T1 + T2, where

- T1 is the reported UE capability which includes LP-WUS processing time and MR transition time for ramp up

- T2 is the duration for time/frequency synchronization of MR

Another motivation is that T2 can be 0 since MR does not go to ultra-deep sleep state and hence MR can receive SSB for sync and RRM.

One company proposes that T2 should be decided based on the additional reporting of required number of SSB/TRS for time/synchronization, as explained in the above figure. This can be considered as starting point for further discussion.

[Old]Proposal 3.2-2:

- For LP-WUS in CONNECTED mode, if a UE receives LP-WUS targeting to itself, the UE is not required to start monitoring PDCCH before t0+T1+T2, where:

t0 is the time point corresponding to the end of the LP-WUS.

T1 is the time duration for ramp-up reported as the one value from X candidate values in UE capability reporting.

T2 is the time duration to receive number C of SSB/TRS for time/frequency synchronization of MR, where C is reported as the number of SSB/TRS for time/synchronization in UE capability reporting.

| Company | Y/N | Comments |
| --- | --- | --- |
| Qualcomm |  | For connected mode, the all measurements are supposed to be done by MR during LP-WUS monitoring. We are not sure why time/frequency synchronization using C SSBs/TRSs is necessary for MR after LP-WUS indication of wake-up. |
| CMCC | N | From our understanding, since the monitoring of LP-WUS does not affect measurement and T/F synchronization for MR in CONNECTED mode, there is no need to consider the duration for T/F sync for MR in the reported value. |
| ZTE, Sanechips |  | T2 is not needed. T0 and T1 is not needed to be defined. For simplicity, it could be described as: For LP-WUS in CONNECTED mode, if a UE receives LP-WUS targeting to itself, the UE is not required to start monitoring PDCCH before the minimum wake-up delay |
| Panasonic | Y |  |
| Nokia1 | N | - We are unsure why we need to explicitly breakdown this time into T1 and T2, we feel synchronisation time should be included within this ”min gap” style definition, but should be left to UE implementation - Such a definition of T2 seems to limit UE implementation, as it rules out the possibility of UEs exploiting other means for maintaining high quality sync for longer. (3) The definition of t0 needs more discussion, is it: the end of the LP-WUS transmission the end of the last slot used for LP-WUS transmission the end of window (if agreed?) used for LPWUS Mos |
| Lenovo | N | T2 is not needed, we are fine with proposal update by ZTE |
| LGE |  | T2 is not needed. |
| OPPO |  | We see one time Gap definition would be good enough. Furthe spliting the time need some justification. |
| MTK |  | Revised proposal: For LP-WUS in CONNECTED mode, if a UE receives LP-WUS targeting to itself, the UE is not required to start monitoring PDCCH before t0 plus a single minimum wake-up delay as reported in UE capability reporting. |
| vivo | N | T2 is not neeed. For LP-WUS in CONNECTED mode, if a UE receives LP-WUS targeting to itself, the UE is not required to start monitoring PDCCH before its reported minimum wake-up delay. |
| Huawei, HiSilicon | Y | To answer QC’s question: even in CONNECTED mode, to get enough power saving gain, MR should be in particular sleep state, e.g. micro, light, deep. If MR stays in light/deep sleep, T/F sync is needed after MR wakeup to get good synchronization with gNB for PDCCH/PDSCH repection. Especially for Option 1-2, UE’s MR will not wake up in advance before each MO, since the periodicity of MO for Option 1-2 is usually assumed to be small. Otherwise, UE needs to be in only micro-sleep or stand-by state for most of the time, and thus the power saving gain is quite limited. |

- Support T2

  - Pana, HW

- Not support T2

  - QC, CMCC, ZTE, Nokia, Lenovo, LGE, OPPO, MTK, vivo

[Old]Proposal 3.2-2a:

- For LP-WUS in CONNECTED mode, if a UE receives LP-WUS targeting to itself, the UE is not required to start monitoring PDCCH before t0+T1+T2, where:

t0 is the time point corresponding to the end of the LP-WUS. FFS details, including

    - the end of the LP-WUS transmission

    - the end of the last slot where LP-WUS is transmitted

    - the end of window used for LPWUS MOs (if supported)

T1 is the time duration for ramp-up reported as the one value from X candidate values in UE capability reporting.

T2 is the time duration to receive number C of SSB/TRS for time/frequency synchronization of MR, where C is reported as the number of SSB/TRS for time/synchronization in UE capability reporting.

| Company | Y/N | Comments |
| --- | --- | --- |
| Qualcomm |  | In Rel-16 DCP, RRC parameter ps-offset indicates a time where the UE starts monitoring PDCCH for detection of DCI format 2_6 prior to a slot that drx-onDurationTimer would start. We wonder t0 can be from the start of the LP-WUS transmission/occasion/slot. Or, we can keep the definition of t0 as totally FFS for now. |
| xiaomi |  | If LP WUS repetition is applied, t0 can be the end of the last slot where LP-WUS is transmitted, or the end of window used for LPWUS MOs (if supported). But if LP WUS repetition is not applied, all the three options are OK. |

[Old]Proposal 3.2-2b:

- For LP-WUS in CONNECTED mode, if a UE receives LP-WUS targeting to itself, the UE is not required to start monitoring PDCCH before t0+T1+T2, where:

t0 is the reference time point. Details FFS

T1 is the time duration for ramp-up reported as the one value from X candidate values in UE capability reporting.

T2 is the time duration to receive number C of SSB/TRS for time/frequency synchronization of MR, where C is reported as the number of SSB/TRS for time/synchronization in UE capability reporting.

[Fri]Combined Proposal 3.2-1/3.2-2c:

For future meetings:

Consider the following alternatives for UE capability report on the minimum time gap between LP-WUS reception and MR to start PDCCH monitoring:

- Alt 1: For the X = [2, 3, 4] candidate values for the UE capability report on the minimum time gap, consider {0.25, 0.5, 1, 2, 3, 6, 10, 20}ms as starting point

  - Note: Other values are not precluded

  - FFS whether different values are selected for OOK-based WUR and OFDM-based WUR

  - FFS whether/how to consider the case of CA

- Alt 2: For the UE capability report on the minimum time gap, UE reports a pair of values for main radio ramp up and the number of SSBs needed for synchronization (D, NSSB).

- Alt 3: UE capability report on the minimum time gap includes the main radio ramp up time. The number of SSBs needed for synchronization is defined in specs, and different number of SSBs can be defined for different UE capability on the main radio ramp up time.

- Alt 4: UE capability report on the minimum time gap includes the main radio ramp up time. The time needed for synchronization will be defined by RAN4 requirements later.

| Company | Y/N | Comments |
| --- | --- | --- |
| Moderator |  | Proposal is made based on the similar note for idle/inactive mode made in Wednesday online: For future meetings: Consider the following alternatives for UE capability report on the wake-up delay: - Alt 1: For the 3 candidate values for the wake-up delay capability report, support {[80ms], [500ms] and [900ms]}. The reported values assume SSB periodicity of 20ms, where [80ms] assumes [3] SSBs needed for synchronization, [500ms] and [900ms] assume [5] SSBs needed for synchronization. FFS: translation of wake-up delay for different SSB periodicities - Alt 2: For the wake-up delay capability report, UE reports a pair of values for main radio ramp up and the number of SSBs needed for synchronization (D, NSSB). Support the following 3 candidate capabilities: {[(20ms, 3), (400ms, 5), (800ms, 5)]}. - Alt 3: Modify the previous agreement on UE capability report on the wake-up delay to UE capability report on the main radio ramp up time. The number of SSBs needed for synchronization is defined in specs, and different number of SSBs can be defined for different UE capability on the main radio ramp up time. 3 candidate values for the main radio ramp up time: {[20ms, 400ms, 800ms]} The number of SSBs needed for synchronization is defined as [3, 5, 5] for [20ms, 400ms, 800ms] main radio ramp up time, respectively. - Alt 4: Modify the previous agreement on UE capability report on the wake-up delay to UE capability report on the main radio ramp up time. The time needed for synchronization will be defined by RAN4 requirements later. |
| Moderator |  | Following was captured in Chair’s note Companies are encouraged to consider [Fri]Combined Proposal 3.2-1/3.2-2c of R1-2410906 for further discussions in future meetings. |

For future meetings:

Consider the following alternatives for UE capability report on the minimum time gap between LP-WUS reception and MR to start PDCCH monitoring:

- Alt 1: For the X = [2, 3, 4] candidate values for the UE capability report on the minimum time gap, consider {0.25, 0.5, 1, 2, 3, 6, 10, 20}ms as starting point

  - Note: Other values are not precluded

  - FFS whether different values are selected for OOK-based WUR and OFDM-based WUR

  - FFS whether/how to consider the case of CA

- Alt 2: For the UE capability report on the minimum time gap, UE reports a pair of values for main radio ramp up and the number of SSBs/TRSs needed for synchronization (D, NSSB/NTRS). This may require RAN4 coordination

- Alt 3: UE capability report on the minimum time gap includes the main radio ramp up time. The number of SSBs/TRSs needed for synchronization is defined in specs, and different number of SSBs/TRSs can be defined for different UE capability on the main radio ramp up time. This may require RAN4 coordination

- Alt 4: UE capability report on the minimum time gap includes the main radio ramp up time. The time needed for synchronization will be defined by RAN4 requirements later.

- FFS: For the case of UAI

3.3 LP-WUS monitoring occasions

For LP-WUS monitoring occasions in connected mode, following agreements were made in previous RAN1 meetings:

Agreement LP-WUS monitoring occasions (MOs) are configured by RRC, where UE can monitor for LP-WUS transmission in RRC CONNECTED mode. - FFS whether to define a time window for MOs - It is at least supported that a UE can monitor MOs with a periodicity. - FFS details of the periodicity, e.g. derived from DRX cycle, separately configured Agreement - Regarding the potential restriction for LP-WUS configuration in relation with C-DRX configuration for option 1-2 of LP-WUS CONNECTED mode operation, from RAN1 perspective, - Opt2: At least LP-WUS periodicity is no larger than long C-DRX cycle, FFS other restrictions - Introducing monitoring window is not precluded

Note that RAN2 also made some agreements related to LP-WUS monitoring occasions

- In RRC_CONNECTED mode, RAN2 to further discuss the impacts of LP-WUS operation methods identified in RAN1. - For Option 1-1 (as described in RAN1 agreement), the LP-WUS monitoring occasion locates at a configured time offset before the start of drx-onDurationTimer. The range of time offset can be determined by RAN1. - For Option 1-1, RAN2 assumes the solutions/ operations introduced for DCP mechanism is taken as baseline. - RAN2 assume that legacy DCP and Option 1-1 is not configured simultaneously for a UE. - The LP-WUS related configuration for RRC CONNECTED state UE is provided via dedicated RRC message. For option 1-2, - After LP-WUS triggers the UE to perform PDCCH monitoring, the UE starts one timer. When the timer is running, the UE monitors PDCCH. FFS on the timer (e.g., newly defined timer or legacy timer.) - The timer is started at a time offset after receiving the LP-WUS indication for PDCCH monitoring. The range of time offset is left for RAN1.

Regarding the LP-WUS monitoring configurations, many companies provided views as follows:

- Vivo

  - LP-WUS monitoring occasions (MOs) are defined by a network-configured Periodicity and Offset. The configuration should allow multiple LP-WUS MOs within each periodicity.

    - For LP-WUS operation option 1-1, the LP-WUS periodicity equals to the C-DRX periodicity

    - For LP-WUS operation option 1-2, the LP-WUS periodicity is separately configured from C-DRX periodicity and no larger than long C-DRX cycle

- E///

  - a time window could be configured which can contain multiple LP-WUS monitoring occasions

    - Option 1-1

      - the periodicity of LP-WUS monitoring is same as the C-DRX cycle length

      - wus-offset: an offset indicating a start of a time window where the UE starts monitoring LP-WUS, prior to a slot where the drx-onDurationTimer is to be started.

    - Option 1-2

      - the periodicity of LP-WUS monitoring is separately configured (e.g. can be different from the C-DRX cycle length)

      - wus-offset: an offset indicating a time where the UE starts monitoring PDCCH, after a slot where the UE detects LP-WUS

- HW/HiSi

  - If a LP-WUS triggering a PDCCH monitoring is received by the UE at symbol n, the UE is not expected to monitor a PDCCH before symbol n+Y where Y is the configured time offset.

  - For both Option 1-1 and 1-2, gNB configures a periodicity and an offset for LP-WUS MOs, where the periodicity is no larger than C-DRX cycle.

    - For Option 1-1, gNB additionally configures a window, which is before the time duration indicated by drx-OnDuraitonTimer.

    - For Option 1-2, there is no further restriction on LP-WUS configuration in relation with C-DRX configuration other than that LP-WUS periodicity is no larger than long C-DRX cycle.

      - Continuous monitoring is supported by configuring proper value of monitoring occasion number and monitoring periodicity.

- Nokia

  - option 1-1

    - A time domain window is defined for MOs that the UE is expected to monitor for LP-WUS candidates.

      - Note: The network can select which MO(s) within a time window, it uses to transmit LP-WUS

    - The earliest the UE is expected to start monitoring for PDCCH from receiving a LP-WUS, is the beginning of the slot where the UE would start the drx-onDurationTimer provided that is configured to equal or exceed the configured minimum time gap from the end of the LP-WUS MO time domain window

    - RAN1 discuss the characteristics required by the UE to configure the LP-WUS which may include:

      - Time gap from start of On-Duration to end of LP-WUS time window

      - Time gap from start of On-Duration to start of LP-WUS time window

      - LP-WUS MO Periodicity within time window

      - LP-WUS MO Starting Offset within the window

      - LP-WUS MO Frequency domain location

      - LP-WUS message configuration

    - RAN1 discuss if the physical message characteristics of LP-WUS candidates sent within a time domain window can vary.

  - Option 1-2

    - the time domain window concept is supported

    - the PDCCH monitoring which LP-WUS can trigger monitoring of, are defined using existing mechanisms

  - If the UE cannot meet the single minimum time gap for a given PDCCH monitoring opportunity, it defaults to monitoring the associated PDCCH monitoring opportunity

- CMCC

  - For Option 1-1, a time offset between LP-WUS and the starting point of DRX ON is configured without the configuration of periodicity.

  - For Option 1-2, a time offset and periodicity can be configured similar as search space configuration.

- ZTE

  - For option 1-1, the LP-WUS monitoring periodicity is the same as long C-DRX cycle

  - For option 1-2,

    - An offset relative to legacy C-DRX can be configured

      - For option 1-2-1, offset applies between new/legacy active time and LP-WUS.

      - For option 1-2-2, offset applies between new active time and LP-WUS

    - Using RRC signalling indicates legacy timer could be triggered for PDCCH monitoring or only new timer could be used to trigger PDCCH monitoring

      - In case legacy timer could be triggered for PDCCH monitoring, if UE detects a LP-WUS in the nearest MOs or window relative to C-DRX, the legacy active time is triggered,otherwise, the new timer could be triggered

      - In case only new timer could be triggered for PDCCH monitoring, if UE detect a LP-WUS in MOs or window relative to C-DRX, the new active time is triggered.

  - consider following two alternatives to achieve better scheduling flexibility

    - Alternative 1: multiple LP-WUS monitoring with different offsets

    - Alternative 2: a monitoring window with configurable length is considered for LP-WUS monitoring

- Samsung

  - For configurations of LP-WUS monitoring occasions, support a time domain window for both Option 1-1 and Option 1-2:

    - Periodicity of the window is same as the DRX cycle;

    - Duration of the window is configured by RRC;

    - Offset between the start of the window and the start of DRX cycle is configured by RRC;

    - Interval between neighboring monitoring occasions is configured by RRC;

    - For Option 1-1, the window is located before the start of DRX cycle;

    - For Option 1-2, the window can be located anywhere within the DRX cycle, and the duration of the window can be configured same as the duration of the DRX cycle.

- SPRD

  - For Option 1-1, the periodicity of LP-WUS monitoring can be derived from DRX cycle

  - Proposal 4: For Option 1-2, the periodicity and offset of LP-WUS can be different from those from C-DRX configuration.

- Apple

  - If Option 1-1 is supported, a time window for LP-WUS monitoring can be configured, otherwise no explicit configuration of time window is needed.

- Xiaomi

  - Support multiple repetitions of a LP WUS occasion

  - Support unified LP WUS configuration for Option 1-1/1-2/1-3. A monitoring window before on duration in each C-DRX cycle can be additionally configured for Option 1-1

- CATT

  - The periodicity of LP-WUS MOs can be configured deterministically without considering a time window

- OPPO

  - The LP-WUS monitoring periodicity and offset can be introduced for triggering a PDCCH monitoring timer, in CONNECTED mode for both Options.

    - For option 1-1, the LP-WUS monitoring can be further restricted to a range of time before the DRX-ON.

    - For option 1-2, the LP-WUS monitoring occasion can be defined during at least for inactive time.

  - RAN1 may specify that the monitoring occasion of LP-WUS and associated monitoring occasion of PDCCH should meet the reported minimum gap

- LGE

  - Specify LP-WUS MOs with a periodicity.

    - LP-WUS inside monitoring window triggers PDCCH monitoring by starting drx-onDurationTimer (Option 1-1).

    - LP-WUS outside monitoring window triggers PDCCH monitoring outside legacy DRX Active Time (Option 1-2).

- Lenovo

  - One or more periodical LP-WUS occasions are configured in WUS monitoring window before drx-onDurationTimer to trigger the PDCCH monitoring at least for Option 1-1

- DCM

  - Define LP-WUS monitoring window which includes one or multiple LP-WUS MOs with a periodicity and offset in RRC CONNECTED mode

    - Option 1-1: Periodicity is same as long C-DRX cycle. Offset is configured from the beginning of drx-onDurationTimer

    - Option 1-2: Periodicity is configured to be no larger than long C-DRX cycle. FFS Offset

In general, companies view is well summarized in R1-2410488 (from Qualcomm) for the PDCCH MO and LP-WUS MO configurations for Rel-16 DCP and LP-WUS Option 1-1/1-2, which can be considered as starting point for further discussion

- Rel-16 DCP

  - PDCCH monitoring periodicity, offset, and duration, are configured by monitoringSlotPeriodicityAndOffset, monitoringSymbolWithinSlot, and duration, as part of RRC IE SearchSpace.

  - Among the PDCCH monitoring occasions according to the parameters in the SearchSpace for the DCI format 2_6, the UE actually monitors PDCCH for the DCI format 2_6 in the first Ts slots according to duration starting from the time ps-Offset prior to the start of drx-onDurationTimer.

Rel-16 DCP (from R1-2410488)

- LP-WUS Option 1-1

  - Approach 1:

    - LP-WUS occasions, including periodicity and offset, are configured independently from the C-DRX periodicity/offset by new RRC parameter(s), such as “LPWUS-SlotPeriodicityAndOffset” and “LPWUS-SymbolWithinSlot”.

    - UE selectively monitors LP-WUS in the LP-WUS occasions identified by other parameters such as “LPWUS-Offset” prior to the start of drx-onDurationTimer.

  - Approach 2:

    - LP-WUS occasions, including periodicity and offset, are identified based on the slot that drx-onDurationTimer would start and a new RRC parameter indicating the time offset such as “LPWUS-Offset” until the slot that drx-onDurationTimer would start.

  - FFS: RRC parameter “LPWUS-duration” indicates the duration where the UE monitors LP-WUS consecutively in time

    - the consecutive LP-WUS occasions in time identified by “LPWUS-duration” are associated with a same C-DRX cycle;

    - LP-WUS occasions in time identified by “LPWUS-duration” provides consistent indication for the same C-DRX cycle of the UE, i.e., UE does not expect to detect more than one LP-WUSs with different indications for the UE in the LP-WUS occasions associated with the same C-DRX cycle.

Approach 1: LP-WUS periodicity/offset are configured by RRC

Approach 2: LP-WUS valid occasions are based on C-DRX configuration

Rel-19 LP-WUS option 1-1 procedure (from R1-2410488)

- LP-WUS Option 1-2

  - Approach 1:

    - LP-WUS occasions, including periodicity and offset, are configured independently from the C-DRX periodicity/offset by new RRC parameter(s), such as “LPWUS-SlotPeriodicityAndOffset” and “LPWUS-SymbolWithinSlot”.

    - UE monitors LP-WUS in the LP-WUS occasions and starts a new timer for PDCCH monitoring triggered by LP-WUS, such as “drx-onDurationTimer-LPWUS” after a configured time offset, such as “LPWUS-Offset”.

  - Approach 2:

    - Periodicity/offset of the slot that the new timer for PDCCH monitoring triggered by LP-WUS are configured by RRC parameters such as “drx-CycleStartOffset-LPWUS” and “drx-SlotOffset-LPWUS”.

    - LP-WUS occasions, including periodicity and offset, are identified based on the slot that drx-onDurationTimer-LPWUS would start and a new RRC parameter indicating the time offset such as “LPWUS-Offset” until the slot that drx-onDurationTimer-LPWUS would start.

  - FFS: RRC parameter “LPWUS-duration” indicates the duration where the UE monitors LP-WUS consecutively in time

    - the consecutive LP-WUS occasions in time identified by “LPWUS-duration” are associated with the same slot that the timer drx-onDurationTimer-LPWUS would start;

    - LP-WUS occasions in time identified by “LPWUS-duration” provides consistent indication for the same slot that the timer drx-onDurationTimer-LPWUS would start, i.e., UE does not expect to detect more than one LP-WUSs with different indications for the UE in the LP-WUS occasions associated with the same slot that the timer drx-onDurationTimer-LPWUS would start.

Approach 1: LP-WUS periodicity/offset are configured by RRC

Approach 2: LP-WUS valid occasions are based on the periodicity/offset configuration of the slot that drx-onDurationTimer-LPWUS would start

Rel-19 LP-WUS option 1-2 procedure (from R1-2410488)

[Close]Proposal 3.3-1:

- For LP-WUS monitoring occasions in connected mode for Option 1-1, following is considered further

  - Approach 1:

    - LP-WUS occasions, including periodicity and offset, are configured independently from the C-DRX periodicity/offset by new RRC parameter(s), such as “LPWUS-SlotPeriodicityAndOffset” and “LPWUS-SymbolWithinSlot”.

    - UE selectively monitors LP-WUS in the LP-WUS occasions identified by other parameters such as “LPWUS-Offset” prior to the start of drx-onDurationTimer.

  - Approach 2:

    - LP-WUS occasions, including periodicity and offset, are identified based on the slot that drx-onDurationTimer would start and a new RRC parameter indicating the time offset such as “LPWUS-Offset” until the slot that drx-onDurationTimer would start.

  - FFS: RRC parameter “LPWUS-duration” indicates the duration where the UE monitors LP-WUS consecutively in time

    - the consecutive LP-WUS occasions in time identified by “LPWUS-duration” are associated with a same C-DRX cycle;

    - LP-WUS occasions in time identified by “LPWUS-duration” provides consistent indication for the same C-DRX cycle of the UE, i.e., UE does not expect to detect more than one LP-WUSs with different indications for the UE in the LP-WUS occasions associated with the same C-DRX cycle.

| Company | Y/N | Comments |
| --- | --- | --- |
| Qualcomm | Y | We think both approaches are workable. However, it would be good to make a common RRC framework for both Option 1-1 and Option 1-2. In this sense, we suggest to either (1) or (2) below: (1) if approach 2 (periodicity is not configured for LP-WUS, it is identified by C-DRX cycle) is adopted for Option 1-1, approach 2 (periodicity is not cofnigured for LP-WUS, it is configured to the possible start occasions of PDCCH mointoring triggered by LP-WUS) should be adopted for Option 1-2. (2) if approach 1 (periodicity is configured for LP-WUS) for Option 1-2, then approach 1 (periodicity is configured for LP-WUS, separately from C-DRX cycle) should be adopted for Option 1-1. Regarding LP-WUS monitoring duration, we are OK to keep it FFS for further discussion. |
| CMCC | Y |  |
| ZTE, Sanechips |  | Approach 2 should be considered. Approach 1 pursues a independent periodicity or offset, which actually may cause LP-WUS occasions overlapping with DRX active time and cause the complicated UE behavior or processing. Additionally, we also believe a unified frame work for option 1-1 and option 1-2 should be considered to avoided additional complexity. For example, we do not need to differentiate option 1-1 and option 1-2 by separate feature. If periodicity is the same as C-DRX, then it is option 1-1, and if it is smaller than C-DRX, it is option 1-2. |
| Nokia1 |  | In principle, we support the direction of this proposal. It seems approach 1 provides a more unified framework for both 1-1 and 1-2 and approach 2 provides a more 1-1 optimised approach. One suggestion we have for the FFS, is to remove the word ”consecutively”. If our understanding is correct, this would imply pseudo continious monitoring LPWUS and prohibit the option of a more ”duty cycle” based monitoring within the effective ”LPWUS-offset” window. We would like more time to consider the pros/cons the consecutive approach. |
| Apple |  | Both approaches can work. It is preferred a unified configuration for Option 1-1 and Option 1-2. |
| Lenovo |  | Approach 2 should be considered at least for Option 1-1 since it is straightforward way similar as DCP in Rel.16. |
| LGE |  | We prefer Approach 1 in this proposal. Independently configured LP-WUS occasions can be used for Option 1-1 and Option 1-2. It can achieve unified and simple configuration of LP-WUS occasions. |
| OPPO |  | We also prefer some unified configuration for Option 1-1/1-2 |
| MTK |  | Prefer a unified configuration framework for both Option 1-1 and Option 1-2. |
| vivo |  | Based on the discussion in the previous meetings, the striahtforward method for the configuration of Option 1-1 is to configure LP-WUS MOs based on network-configured monitoring window and Offset to DRX Onduration. The periodicity of the LP-WUS monitoring window is equal to the C-DRX cycle. And each LP-WUS monitoirng window is prior to the associated DRX onduration and can be determined based on the Offset to DRX onduration. Besides, there can be one or mulitple LP-WUS MOs within a LP-WUS monitoring window. Given the above analysis, we think in principle the approach 2 seems to be more suitable. Besides, does the RRC parameter “LPWUS-duration” refer to LP-WUS monitoring window? And if so, our question is why UE is forced to monitor LP-WUS consecutively in time? |
| Sharp | Y |  |
| Huawei, HiSilicon |  | We are OK for further discussion, and our preference is Approach 1, which provide gNB with more flexibility to transmit LP-WUS in any MO within th e window. But different from the figure drawn by QC, in our understanding, for both DCP design and Approach 1 of Option 1-1, the second ’duraiton/LP-WUS duration’ within the first window should be also monitored by UE (see the red circle below). |

[Close]Proposal 3.3-2:

- For LP-WUS monitoring occasions in connected mode for Option 1-2, following is considered further

  - Approach 1:

    - LP-WUS occasions, including periodicity and offset, are configured independently from the C-DRX periodicity/offset by new RRC parameter(s), such as “LPWUS-SlotPeriodicityAndOffset” and “LPWUS-SymbolWithinSlot”.

    - UE monitors LP-WUS in the LP-WUS occasions and starts a new timer for PDCCH monitoring triggered by LP-WUS, such as “drx-onDurationTimer-LPWUS” after a configured time offset, such as “LPWUS-Offset”.

  - Approach 2:

    - Periodicity/offset of the slot that the new timer for PDCCH monitoring triggered by LP-WUS are configured by RRC parameters such as “drx-CycleStartOffset-LPWUS” and “drx-SlotOffset-LPWUS”.

    - LP-WUS occasions, including periodicity and offset, are identified based on the slot that drx-onDurationTimer-LPWUS would start and a new RRC parameter indicating the time offset such as “LPWUS-Offset” until the slot that drx-onDurationTimer-LPWUS would start.

  - FFS: RRC parameter “LPWUS-duration” indicates the duration where the UE monitors LP-WUS consecutively in time

    - the consecutive LP-WUS occasions in time identified by “LPWUS-duration” are associated with the same slot that the timer drx-onDurationTimer-LPWUS would start;

    - LP-WUS occasions in time identified by “LPWUS-duration” provides consistent indication for the same slot that the timer drx-onDurationTimer-LPWUS would start, i.e., UE does not expect to detect more than one LP-WUSs with different indications for the UE in the LP-WUS occasions associated with the same slot that the timer drx-onDurationTimer-LPWUS would start.

| Company | Y/N | Comments |
| --- | --- | --- |
| Qualcomm | Y | Same comment as 3.3-1. |
| CMCC | Y |  |
| ZTE, Sanechips |  | Similar comment as previous. Approach 2 is considered and unified frame work should be pursued. Additionally, to include option 1-2-2, the following update is suggested - LP-WUS occasions, including periodicity and offset, are identified based on the slot that drx-onDurationTimer-LPWUS would start and a new RRC parameter indicating the time offset such as “LPWUS-Offset” until the slot that drx-onDurationTimer-LPWUS or drx-onDurationTimer would start. Regarding the FFS, it should be more general. For example, ‘consecutively in time’ is not clear especially considering the TDD case. |
| Nokia1 |  | Same comment as 3.3-1. |
| Apple | Y | For Option 1-2, we see some benefit if the LP-WUS occasions are defined relative to the start of legacy drx-onDurationTimer, so that the potential new timer triggered by the LP-WUS would be overlapped with legacy drx-onDurationTimer, in this way, PDCCH may be monitored together when UE is performing measurement. |
| LGE |  | We’d like to suggest more flexible approach that LP-WUS occasions and Periodicity/offset of the new timer are independently configured. In Approach 1, LP-WUS occasions can be independently configured but Periodicity/offset for the new timer cannot. Similarly, in Approach 2, Periodicity/offset can but LP-WUS occasions cannot. However, those two can be separately configured respectively so that more flexibilities is provided to gNB. For example, if a UE monitors LP-WUS based on the configuration and can start the new timer in the first slot after configured offset, based on the periodicity and offset configuration of the new timer. - Approach 3: - LP-WUS occasions, including periodicity and offset, are configured independently from the C-DRX periodicity/offset by new RRC parameter(s), such as “LPWUS-SlotPeriodicityAndOffset” and “LPWUS-SymbolWithinSlot”. - Periodicity/offset of the slot that the new timer for PDCCH monitoring triggered by LP-WUS are configured by RRC parameters such as “drx-CycleStartOffset-LPWUS” and “drx-SlotOffset-LPWUS”. UE monitors LP-WUS in the LP-WUS occasions and starts a new timer for PDCCH monitoring triggered by LP-WUS such as “drx-onDurationTimer-LPWUS” in the first slot after a configured time offset, such as “LPWUS-Offset”, based on RRC parameters such as “drx-CycleStartOffset-LPWUS” and “drx-SlotOffset-LPWUS”. |
| OPPO |  | Same comment as 3.3-1. |
| vivo |  | For option 1-2, no need to define LP-WUS MO based on the new timer. That is, UE just monitors in the configured LP-WUS MOs and starts the new timer if triggered by LP-WUS. To follow this logic, the aprroach 1 seems to be a proper one. But we are trying to understand what is the purpose of “LPWUS-SymbolWithinSlot” given that “LPWUS-SlotPeriodicityAndOffset” is already address the periodicity and offset of LP-WUS MO. Besides, does the RRC parameter “LPWUS-duration” refer to LP-WUS monitoring window? And if so, our question is why UE is forced to monitor LP-WUS consecutively in time? |
| Huawei, HiSilicon |  | We don’t see the necessity of Approach 2. Approach 2 is actually achieving the same effect as Approach 1, which introduce an unnecessary and wired ’drx-onDurationTimer-LPWUS’. Thus we suggest to agree on Approach 1 directly. |

[Close]Combined Proposal 3.3-1/2a:

- For LP-WUS MOs in connected mode for Option 1-1, following is considered further

  - Approach 1:

    - LP-WUS MOs, including periodicity and offset, are configured independently from the C-DRX periodicity/offset by new RRC parameter(s), such as “LPWUS-SlotPeriodicityAndOffset” and “LPWUS-SymbolWithinSlot”.

    - UE selectively monitors LP-WUS in the LP-WUS MOs identified by other parameters such as “LPWUS-Offset” prior to the start of drx-onDurationTimer.

  - Approach 2:

    - LP-WUS MOs, including periodicity and offset, are identified based on the slot that drx-onDurationTimer would start and a new RRC parameter indicating the time offset such as “LPWUS-Offset” until the slot that drx-onDurationTimer would start.

  - FFS: RRC parameter “LPWUS-duration” indicates the duration where the UE monitors LP-WUS [consecutively in time]

    - FFS: the consecutive LP-WUS MOs in time identified by “LPWUS-duration” are associated with a same C-DRX cycle;

    - FFS: LP-WUS MOs in time identified by “LPWUS-duration” provides consistent indication for the same C-DRX cycle of the UE, i.e., UE does not expect to detect more than one LP-WUSs with different indications for the UE in the LP-WUS MOs associated with the same C-DRX cycle.

- For LP-WUS MOs in connected mode for Option 1-2, following is considered further

  - Approach 1:

    - LP-WUS MOs, including periodicity and offset, are configured independently from the C-DRX periodicity/offset by new RRC parameter(s), such as “LPWUS-SlotPeriodicityAndOffset” and “LPWUS-SymbolWithinSlot”.

    - UE monitors LP-WUS in the LP-WUS MOs and starts a new timer for PDCCH monitoring triggered by LP-WUS, such as “drx-onDurationTimer-LPWUS” after a configured time offset, such as “LPWUS-Offset”. FFS legacy onduration timer

  - Approach 2:

    - Periodicity/offset of the slot that the new timer for PDCCH monitoring triggered by LP-WUS are configured by RRC parameters such as “drx-CycleStartOffset-LPWUS” and “drx-SlotOffset-LPWUS”.

    - LP-WUS MOs, including periodicity and offset, are identified based on the slot that drx-onDurationTimer-LPWUS would start and a new RRC parameter indicating the time offset such as “LPWUS-Offset” until the slot that drx-onDurationTimer-LPWUS would start.

  - FFS: RRC parameter “LPWUS-duration” indicates the duration where the UE monitors LP-WUS consecutively in time

    - the consecutive LP-WUS MOs in time identified by “LPWUS-duration” are associated with the same slot that the timer drx-onDurationTimer-LPWUS would start;

    - LP-WUS MOs in time identified by “LPWUS-duration” provides consistent indication for the same slot that the timer drx-onDurationTimer-LPWUS would start, i.e., UE does not expect to detect more than one LP-WUSs with different indications for the UE in the LP-WUS MOs associated with the same slot that the timer drx-onDurationTimer-LPWUS would start.

- FFS: For Option 1-1 and Option 1-2, the same apporach among {Approach 1, Approach 2} is down-selected.

| Company | Y/N | Comments |
| --- | --- | --- |
| Qualcomm | Y |  |
| Xiaomi |  | OK with the apporaches, but we doe not think the following restriction is needed, and can be deleted. - For Option 1-1 and Option 1-2, the same apporach among {Approach 1, Approach 2} is down-selected. |

[Close]Combined Proposal 3.3-1/2a:

- For LP-WUS MOs in connected mode for Option 1-1, following is considered further

  - Approach 1:

    - LP-WUS MOs, including periodicity and offset, are configured independently from the C-DRX periodicity/offset by new RRC parameter(s).

    - UE selectively monitors LP-WUS in the LP-WUS MOs identified by other parameters such as time offset prior to the start of drx-onDurationTimer.

  - Approach 2:

    - LP-WUS MOs, including periodicity and offset, are identified based on the slot that drx-onDurationTimer would start and a new RRC parameter indicating the time offset until the slot that drx-onDurationTimer would start.

  - FFS: An RRC parameter indicates the duration where the UE monitors LP-WUS [consecutively in time]

    - FFS: the consecutive LP-WUS MOs in time identified by the RRC parameter are associated with a same C-DRX cycle;

    - FFS: LP-WUS MOs in time identified by the RRC parameter provides consistent indication for the same C-DRX cycle of the UE, i.e., UE does not expect to detect more than one LP-WUSs with different indications for the UE in the LP-WUS MOs associated with the same C-DRX cycle.

- For LP-WUS MOs in connected mode for Option 1-2, following is considered further

  - Approach 1:

    - LP-WUS MOs, including periodicity and offset, are configured independently from the C-DRX periodicity/offset by new RRC parameter(s).

    - UE monitors LP-WUS in the LP-WUS MOs and starts a new timer for PDCCH monitoring triggered by LP-WUS, after a configured time offset. FFS whether LP-WUS triggers drx-onDurationTimer

  - Approach 2:

    - Periodicity/offset of the slot that the new timer for PDCCH monitoring triggered by LP-WUS are configured by RRC parameters.

    - LP-WUS MOs, including periodicity and offset, are identified based on the slot that new timer would start and a new RRC parameter indicating the time offset until the slot that new timer would start.

  - FFS: An RRC parameter indicates the duration where the UE monitors LP-WUS [consecutively in time]

    - FFS: the consecutive LP-WUS MOs in time identified by the RRC parameter are associated with the same slot that the new timer would start;

    - FFS: LP-WUS MOs in time identified by the RRC parameter provides consistent indication for the same slot that the new timer would start, i.e., UE does not expect to detect more than one LP-WUSs with different indications for the UE in the LP-WUS MOs associated with the same slot that the new timer would start.

| Company | Y/N | Comments |
| --- | --- | --- |
| Moderator |  | Following was agreed in Thursday online: Agreement - For LP-WUS MOs in connected mode for Option 1-1, following is considered further discussion and possible down-selection - Approach 1: - LP-WUS MOs, including periodicity and offset, are configured independently from the C-DRX periodicity/offset by new RRC parameter(s). - UE selectively monitors LP-WUS in the LP-WUS MOs identified by other parameters such as time offset prior to the start of drx-onDurationTimer. - Approach 2: - LP-WUS MOs, including periodicity and offset, are identified based on the slot that drx-onDurationTimer would start and a new RRC parameter indicating the time offset until the slot that drx-onDurationTimer would start. - FFS: An RRC parameter indicates the duration where the UE monitors LP-WUS [consecutively in time] - FFS: the consecutive LP-WUS MOs in time identified by the RRC parameter are associated with a same C-DRX cycle; - FFS: LP-WUS MOs in time identified by the RRC parameter provides consistent indication for the same C-DRX cycle of the UE, i.e., UE does not expect to detect more than one LP-WUSs with different indications for the UE in the LP-WUS MOs associated with the same C-DRX cycle. - For LP-WUS MOs in connected mode for Option 1-2, following is considered further - Approach 1: - LP-WUS MOs, including periodicity and offset, are configured independently from the C-DRX periodicity/offset by new RRC parameter(s). - UE monitors LP-WUS in the LP-WUS MOs and starts a new timer for PDCCH monitoring triggered by LP-WUS, after a time offset. - Approach 2: - Periodicity/offset of the slot that the new timer for PDCCH monitoring may be potentially triggered by LP-WUS are configured by RRC parameters. - LP-WUS MOs, including periodicity and offset, are identified based on the slot that new timer would start and a new RRC parameter indicating the time offset until the slot that new timer would start. - FFS: An RRC parameter indicates the duration where the UE monitors LP-WUS [consecutively in time] - FFS: the consecutive LP-WUS MOs in time identified by the RRC parameter are associated with the same slot that the new timer would start; - FFS: LP-WUS MOs in time identified by the RRC parameter provides consistent indication for the same slot that the new timer would start, i.e., UE does not expect to detect more than one LP-WUSs with different indications for the UE in the LP-WUS MOs associated with the same slot that the new timer would start. |

A number of companies provided their view on other aspects related to LP-WUS MOs, to be discussed later.

- FFS other restrictions on Option 1-2 LP-WUS configuration

  - None: vivo, E///?, HW/HiSi

    - to enable more flexible triggering for PDCCH monitoring that is not restricted by C-DRX configuration

    - when the legacy DRX On duration timer is running, the UE should be able to monitor LP-WUS and trigger any associated PDCCH monitoring.

- Frequency location of LP-WUS

  - QC

    - As baseline, LP-WUS on a serving cell is configured within active DL BWP of the serving cell

      - FFS: outside active DL BWP of the serving cell (as optional UE capability)

  - ZTE

    - The LP-WUS is applied for active DL BWP for PDCCH monitoring

      - LP-WUS could be configured outside the active BWP

3.4 QCL/TCI-state for LP-WUS

Regarding the QCL/TCI state for LP-WUS in CONNECTED mode, following agreements were made in previous RAN1 meetings:

Agreement For LP-WUS monitoring in RRC CONNECTED mode, a LP-WUS is QCLed with existing NR signal/channel/CORESET for the TCI state - FFS which existing NR signal/channel/CORESET is the QCL source of LP-WUS - FFS exact definition of QCL relationship between LP-WUS and existing NR signal/channel/CORESET Agreement - For LP-WUS monitoring in RRC CONNECTED mode, SSB and/or CSI-RS can be the QCL source of LP-WUS FFS applicable QCL type(s)

Following views are provide in the contributions:

- FFS applicable QCL type(s)

  - Type A: CMCC, Pana, Xiaomi (LP-SS)

  - Type C: HW/HiSi, ZTE, Pana, Xiaomi (SSB)

    - no channel estimation and channel equalization operations are expected

  - Type D: vivo (FR2), CMCC (FR2)

  - Reuse QCL Type(s) defined for CORESET/PDCCH: QC

- How to define/configure/indicate TCI-state of LP-WUS

  - can be discussed after the progress achieved on LP-WUS monitoring activation/deactivation: HW/HiSi

  - Re-use existing TCI-state framework: Nokia

    - As part of the configuration for LP-WUS: QC, vivo, Samsung

      - QC: The TCI-state in the LP-WUS configuration indicates one or two DL RSs with the corresponding QCL types

    - Reuse unified TCI framework: QC, vivo

      - QC:

        - If a UE is configured with dl-OrJointTCI-StateList, the UE obtains the QCL assumptions from a TCI state within the dl-OrJointTCI-StateList

        - The TCI state from which the UE obtains the QCL assumptions for LP-WUS is the one activated by MAC-CE activation command and is indicated by the codepoint of the DCI field “Transmission Configuration Indication”

    - Reuse TCI-state framework + default TCI state: LGE

    - Reuse TCI-state framework of CORESET: DCM

  - TCI association with CORESET

    - Same TCI state as the CORESET used for PDCCH monitoring triggered by LP-WUS: E///, CATT, OPPO

    - Latest TCI state of CORESET + default TCI state: ETRI

  - multiple LP-WUS monitoring space configuration to have different TCI-state configuration: ZTE

[Close]Proposal 3.4-1:

- For LP-WUS monitoring in RRC CONNECTED mode, SSB and/or CSI-RS can be the QCL source of LP-WUS

QCL type(s) is {Type A or Type C} and Type D, when applicable

| Company | Y/N | Comments |
| --- | --- | --- |
| Qualcomm | Y |  |
| xiaomi | Y |  |
| CMCC | Y |  |
| ZTE, Sanechips |  | Suggest to add FFS to discuss SSB could be referred to CD-SSB or nNCD-SSB. Additionally, type D is for FR2 in our understanding. |
| Panasonic |  | We think the QCL relationship between CSI-RS (other than TRS) and LP-WUS does not need to be defined as LP-WUR would not receive and measure CSI-RS (other than TRS). |
| Samsung | Y |  |
| Nokia1 | Y |  |
| Spreadtrum | Y |  |
| Apple | Y |  |
| ETRI | Y |  |
| Lenovo | Y |  |
| LGE |  | QCL type D for FR1 is supported? |
| OPPO | Y |  |
| MTK | Y |  |
| vivo | Y |  |
| Moderator |  | Following was agreed in Monday online: Agreement - For LP-WUS monitoring in RRC CONNECTED mode, SSB and/or CSI-RS can be the QCL source of LP-WUS - QCL type(s) is {Type A or Type C – for down-selection} and Type D, when applicable |

[Open]Proposal 3.4-2:

- For LP-WUS monitoring in RRC CONNECTED mode, consider following options on how to define/configure/indicate TCI-state of LP-WUS

Opt.1: Reuse TCI-state framework to indicate TCI-state of LP-WUS

Opt.2: TCI-state association between LP-WUS and CORESET

Opt.3: Default TCI-state

Note: Other options are not precluded

| Company | Y/N | Comments |
| --- | --- | --- |
| Qualcomm |  | A bit more clarification on Option 3 is appreciated. |
| xiaomi | Y |  |
| CMCC | Y |  |
| ZTE, Sanechips |  | Pls add clarification for the TCI-state framework definition. For option3, it could be updated as Default/configured TCI-state. That’s to say, the TCI state is predetermined or configured by gNB. |
| Samsung |  | Clarification for Opt 2 and Opt 3 is needed. Our preference is TCI-state is configured by gNB, in the LP-WUS configuration, and it’s in Option 3? |
| Nokia1 | Y | Similar to Qualcomm, when would a default TCI-state be needed? |
| Spreadtrum | Y |  |
| Apple | Y |  |
| ETRI | Y |  |
| Lenovo | Y |  |
| LGE | Y |  |
| OPPO | Y |  |
| vivo | Y | And same comment as Qualcomm. |
| Sharp | Y |  |
| Huawei, HiSilicon |  | The meanings of the Options, and the differences among the options are not clear to us. Could the FL to give more explanation? For example, what is the ‘TCI-state framework’ in Option 1? |
| Moderator |  | This proposal was not dsicussed during offline/online in this meeting. Companies are engouraged to fuether check and consider towards next meeting. |

4	Activation/deactivation of LP-WUS monitoring

This issue has been discussed in previous RAN1 meetings and following agreements were made:

Agreement For RRC CONNECTED mode, from RAN1 perspective, - PDCCH monitoring triggered by LP-WUS is enabled/disabled by gNB RRC signaling - FFS whether to support UE assistance. - LP-WUS monitoring by UE is known to gNB. - FFS whether implicit/explicit indication from UE is necessary - In case LP-WUS monitoring is enabled, following options are further studied - Option 1: No additional indication/condition are introduced for activation/deactivation of LP-WUS monitoring - Option 2: Activation/deactivation of LP-WUS monitoring by gNB L1/L2 signaling with or without UE assistance. - Option 3: Activation/deactivation of LP-WUS monitoring based on condition(s), such as timer. - Option 4: Activation/deactivation of LP-WUS monitoring based on implicit indication/condition, e.g. UL transmission.

Many companies provided their view on FFS and these options

- FFS whether implicit/explicit indication from UE is necessary

  - Support: IDC, Samsung,

    - IDC: Introduce a mechanism to report UE decision of LP-WUS monitoring deactivation potentially with reason for the decision

    - Samsung: support explicit confirmation message(s) from the UE to the gNB for handshake in Option 2

  - Not support: vivo, ETRI

- FFS whether to support UE assistance.

  - Yes: IDC

  - No: CATT

- Option 1: CMCC, CATT, Sony

- Option 2: QC, vivo, HW, Samsung, SPRD, Apple, LGE

  - QC: Introduce MAC-CE for LP-WUS activation/deactivation

    - A UE can be configured with one or multiple LP-WUS configurations

    - The MAC-CE activates one (or none) of the LP-WUS configurations dynamically

  - Vivo: UE switching from PDCCH monitoring to LP-WUS monitoring is supported by gNB L1/L2 signaling

  - HW: LP-WUS monitoring can be triggered explicitly by L1/L2 signaling, e.g. by DCI or MAC CE

  - SPRD: Activation/deactivation of LP-WUS monitoring by gNB L1 signaling (e.g., DCI) can be considered

  - Apple: Support UE switching to LP-WUS monitoring within Active Time, based on L1/L2 signaling or timer

    - It can be further discussed which active timer(s) are considered.

  - LGE: Specify activation/deactivation of LP-WUS monitoring by gNB L1/L2 signaling with or without UE assistance

- Option 3: Nokia, IDC, Samsung, OPPO, TCL

  - Nokia: For both options 1-1 and 1-2, a new timer (option 3) is supported to trigger the deactivation of LP-WUS monitoring

  - IDC:

    - For activating LP-WUS monitoring: level of UE activity (e.g., timer and/or counter associated with UE activity), channel/signal quality, mobility.

    - For deactivating LP-WUS monitoring: level of UE activity (e.g., timer and/or counter associated with UE activity), channel/signal quality, mobility, LP-WUS reception quality

  - OPPO: For the LP-WUS activation and deactivation by pre-configured condition(s), timer-based monitoring of LP-WUS is the baseline

  - TCL: Deactivation of LP-WUS monitoring and resume PDCCH monitoring by timers.

  - Lenovo: LP-WUS monitoring can be activated/deactivated by timer (especially for switching between LP-WUS monitoring and MR PDCCH monitoring after activation).

- Option 4: QC, vivo, HW, TCL

  - QC: UE can trigger recovery from beam failure or radio link failure even when the UE monitors LP-WUS and does not monitor PDCCH. Once the UE triggers the recovery (transmits PRACH), the UE autonomously activate PDCCH monitoring to receive the response to the recovery request

  - Vivo: UL transmissions such as SR, PRACH and CG PUSCH can trigger MR PDCCH monitoring according to the legacy UE behavior.

  - HW:

    - Deactivation of LP-WUS monitoring by the reception of a LP-WUS targeting to the UE

    - Deactivation of LP-WUS monitoring based on UL transmission

    - PDCCH monitoring of a UE is suspended if LP-WUS monitoring of the UE is activated, and vice versa. PDCCH monitoring of a UE is resumed if LP-WUS monitoring of the UE is deactivated.

  - TCL: Deactivation of LP-WUS monitoring and resume PDCCH monitoring by UL signaling including SR, BSR, PRACH, CG PUSCH

Related to Option 3, some companies also propose autonomous fallback to PDCCH monitoring when UE monitors LP-WUS

- Autonomous fallback to PDCCH monitoring when UE monitors LP-WUS

  - Support: QC, vivo, ZTE, Apple, ETRI, DCM

  - Not support: E///

  - Condition to fallback

    - When the channel/beam quality is not satisfactory for successful LP-WUS reception: QC, ETRI, Lenovo

    - When UE does not received LP-WUS for a long time: ZTE

    - Without specified condition (by UE implementation): vivo

  - Fallback to

    - Option 1-1

      - Legacy C-DRX: QC, vivo, Apple, Lenovo, DCM

    - Option 1-2

      - Legacy C-DRX: QC, Lenovo, [DCM]

      - Always-on PDCCH monitoring: vivo

      - PDCCH monitoring according to LP-WUS cycle: Apple, [DCM]

  - How to avoid misalignment between gNB and UE for Option 1-2

    - Network configuration (e.g., configure Option 1-2 such that the time window for drx-onDurationTimer and a time window based on the new timer e.g., “drx-onDurationTimer-LPWUS” overlap in time so that in either case there are at least some time windows that the UE can receive PDCCH): QC

    - new L1/L2 dynamic report of UE status (e.g., indicating whether the UE monitors LP-WUS or the UE fallback to PDCCH monitoring based on C-DRX configuration): QC

Regarding additional indication/condition for activation/deactivation of LP-WUS monitoring, companies have different preference, and further discussion is necessary. As supported by a number of companies, in Option 3, which has the most support, autonomous fallback can be considered as starting point for further discussion

[Old]Proposal 4-1:

- When LP-WUS monitoring is enabled for RRC CONNECTED mode, support UE autonomous fallback to PDCCH monitoring when UE monitors LP-WUS

FFS the condition for the fallback

For LP-WUS operation option 1-1, UE fallbacks to legacy C-DRX behavior

For LP-WUS operation option 1-2,

    - FFS whether UE fallbacks to

      - Alt1: legacy C-DRX behavior

      - Alt2: PDCCH monitoring without legacy C-DRX

      - Alt3: PDCCH monitoring according to LP-WUS cycle

    - FFS whether/how to avoid misalignment between gNB and UE on the UE LP-WUS/PDCCH monitoring

| Company | Y/N | Comments |
| --- | --- | --- |
| Qualcomm | Y | For the FFS of LP-WUS operation option 1-2, we prefer either Alt.2 or Alt.3. |
| CMCC | Y |  |
| ZTE, Sanechips | Y | We prefer Alt1, otherwise, there is misalignment between gNB and UE. Additionally, the condition comprise when the UE does not receive LP-WUS for a long time. |
| Panasonic | Y |  |
| Nokia1 | Y |  |
| Apple | Y |  |
| Lenovo | Y |  |
| LGE | Y |  |
| OPPO | Y |  |
| MTK | Y | Support UE autonomous fallback to PDCCH monitoring when UE monitors LP-WUS. For LP-WUS operation option 1-1 and option 1-2, UE fallbacks to legacy C-DRX behavior. |
| vivo | Y |  |
| Sharp | Y |  |
| Huawei, HiSilicon |  | Before knowing the condition, we cannot agree on falling back operation. |
| Ericsson1 | N | In connected mode LP-WUS monitoring by UE should be known to gNB as already agreed. UE autonomous switching implies the state is not known to gNB. A UE that does not autonomously switch correctly and continues to monitor LP-WUS monitoring becomes unreachable to gNB.This leads to consistent waste of resources on sending signals that UE does not monitor and gNB cannot deactivate/disable LP-WUS monitoring to save resources. |
| xiaomi | Y |  |

[Fri]Proposal 4-1a:

When LP-WUS monitoring is enabled for RRC CONNECTED mode, support UE fallback to PDCCH monitoring when UE monitors LP-WUS

- FFS the condition for the fallback, e,g.,

  - When the channel/beam quality is not satisfactory for successful LP-WUS reception

  - When UE does not received LP-WUS for a long time

  - Without specified condition (by UE implementation)

- For LP-WUS operation option 1-1, UE fallbacks to legacy C-DRX behavior

- For LP-WUS operation option 1-2,

    - FFS what UE fallbacks to

- FFS how to avoid misalignment between gNB and UE on the UE LP-WUS/PDCCH monitoring

| Company | Y/N | Comments |
| --- | --- | --- |
| Moderator |  | Proposal 4-1 was discussed during Wednesday Online but no consensus was achieved. ”FFS the condition for the fallback” is updated to give some examples proposed by companies. |
| Moderator |  | This proposal was not dsicussed during Friday online. Companies are engouraged to fuether check and consider towards next meeting. |

Following proposals can be further considered as second priority:

[Close]Proposal 4-2:

- When LP-WUS monitoring is enabled for RRC CONNECTED mode, following opions are further considered for the activation/deactivation of LP-WUS monitoring by gNB L1/L2 signaling

Opt.2-1: UE can be configured with one LP-WUS configurations. Introduce MAC-CE for LP-WUS activation/deactivation, which activates/deactivates the LP-WUS configurations

Opt.2-2: UE can be configured with one or multiple LP-WUS configurations. Introduce MAC-CE for LP-WUS activation/deactivation, which activates/deactivates one (or none) of the LP-WUS configurations

| Company | Y/N | Comment |
| --- | --- | --- |
| Qualcomm | Y | We are OK with these options for further discussion. |
| CMCC |  | The benefit to should clarified |
| ZTE, Sanechips |  | Why we firstly to consider MAC activation instead of DCI, why we need MAC activation compared with RRC. It is still not clear. But we are OK to consider UE can be configure one or multiple LP-WUS configurations. |
| Nokia1 |  | Wording of Option 2-1 should be clarified, use of plural “configurations” is confusing. Is MAC-CE just being used to activate/deactivate the 1 configuration or somehow enabling switching between configurations (which seems to be opt-2-2?) Also, would like to understand why MAC-CE, as opposed to DCI, and the consequences of missed/incorrect L1/2 indications. |
| Apple |  | This is also related to Proposal 3.3-1 and 3.3-2, it also needs to be clarified what is the definition of an LP-WUS configuration. |
| Lenovo | Y | OK for further discussion. |
| LGE | Y |  |
| OPPO |  | We haven’t see the justification of L1/MAC-CE activation/deactivation. |
| MTK | Y | Okay with MAC-CE with a need of ACK and activation time. |
| vivo |  | We support that the activation/deactivation of LP-WUS monitoring by gNB L1/L2 signaling e.g., using PDCCH skipping indication. But what is the benefit and purpose for the following two options? In our view, there seems no need for a UE to be configured with multiple LP-WUS configurations. Hence, there is no need to introduce MAC-CE for LP-WUS activation/deactivation of the LP-WUS configurations. |
| Sharp |  | In the main bullet, L1/L2 signaling are proposed for activation/deactivation, but, only MAC-CE is mentioned in opt2-1/2-2. As comments by other companys, DCI can also be considerd for activation/deactivation. |
| Huawei, HiSilicon |  | One general suggestion is that we separately discuss activation and deactivation, since different signalling/procedure may be used. Regarding the two options, Opt.2-1 is clear to us, and we are fine with it for activation (we are also OK for DCI based activation). However, Opt.2-2 is not clear to us, more clarification is needed. |

[Close]Proposal 4-2a:

- When LP-WUS monitoring is enabled for RRC CONNECTED mode, following opions are further considered for the activation/deactivation of LP-WUS monitoring by gNB L1/L2 signaling

Opt.2-1: UE can be configured with one LP-WUS configurations. Introduce gNB L1/L2 signaling for LP-WUS activation/deactivation, which activates/deactivates the LP-WUS configurations

Opt.2-2: UE can be configured with one or multiple LP-WUS configurations. Introduce gNB L1/L2 signaling for LP-WUS activation/deactivation, which activates/deactivates one (or none) of the LP-WUS configurations

| Company | Y/N | Comments |
| --- | --- | --- |
| Moderator |  | Following was agreed in Wednesday online. RAN1 can further discuss based on RAN2 reply, if necessary, and hence, no further discussion in this meeting is expected. Agreement For LP-WUS CONNECTED mode operation, the following case is not supported: - Both Option 1-1 and Option 1-2 are simultaneously configured for the same UE in a given cell Conclusion - For Option 1-1 of LP-WUS CONNECTED mode operation, RAN1 assumes existing DRX Command MAC CE and Long DRX Command MAC CE can be used to stop drx-onDurationTimer and/or drx-InactivityTimer for the DRX group - When UE is not in C-DRX active time, UE monitors LP-WUS - Send LS to RAN2 to ask whether RAN2 has same understanding or not. LS in R1-2410XXX. Agreement - For Option 1-2 of LP-WUS CONNECTED mode operation, for explicit indication to stop the drx-InactivityTimer for the DRX group, RAN1 identified the following potential solution - Use existing DRX Command MAC CE and Long DRX Command MAC CE - FFS: Applicability of this feature for new timer for PDCCH monitoring in Option 1-2 - When UE is not in C-DRX active time, UE monitors LP-WUS - Send LS to RAN2 to ask the feasibility of the above potential solution. LS in R1-2410XXX. |

[Open]Proposal 4-3:

- When LP-WUS monitoring is enabled for RRC CONNECTED mode, it is supported that UL transmissions including SR, PRACH and CG-PUSCH triggers MR PDCCH monitoring according to the legacy UE behavior

| Company | Y/N | Comments |
| --- | --- | --- |
| Qualcomm | Y |  |
| CMCC |  | The benefit to should clarified |
| ZTE, Sanechips |  | Try to understand this proposal. If LP-WUS is not enabled, in legacy, the UL transmissions including SR, PRACH and CG-PUSCH can trigger MR PDCCH monitoring. So with this proposal, we are to clarify the UE behavior is not changed? Or why we need this proposal. |
| Nokia1 |  | Is this part of the larger discussion on simultaneous LR/MR operation, [Mon]Proposal 3.1-1? |
| Apple | Y |  |
| LGE | Y | The legacy UE behavior should be supported. As we mentioned in the proposal 3.1-1, CG-PUSCH needs to be clarified. If a UE is configured with CG-PUSCH, the UE may or may not transmit CG-PSUCH in these occasions. So, even though a UE is in CG-PUSCH occasions, the UE does not transit PUSCH actually and can monitor LP-WUS. |
| OPPO |  | They need some further justication. |
| MTK |  | LP-WUS monitoring should not change UL transmissions such as SR, PRACH, and CG PUSCH. |
| vivo | Y |  |
| Huawei, HiSilicon | Y |  |
| Moderator |  | This proposal was not dsicussed during offline/online in this meeting. Companies are engouraged to fuether check and consider towards next meeting. |

5	LP-WUS payload

This issue has been discussed in previous RAN1 meetings, and following agreements were made:

Agreement For RRC CONNECTED mode, maximum number of LP-WUS information bits is up to X bits - FFS value X, which is no more than [8 or 16] Agreement Regarding the LP-WUS information to trigger PDCCH monitoring of RRC connected UEs, at least consider the following: - Option 1: A bitmap with each bit corresponding to [one or more] UEs - Option 2: A codepoint value corresponding to one or part of UE identity, e.g., C-RNTI - Option 3: A codepoint value corresponding to [one or more] UEs - Option 4: Multiple codepoint values with each corresponding to [one or more] UE(s) - Option 5: Multiple bit blocks with each corresponding to [one or more] UE(s) - Combination of above options are not precluded. - FFS how to carry LP-WUS information, e.g, by encoded bits (with/without CRC) and/or by OOK sequence selection for ‘ON-OFF’ pattern for OOK symbols of LP-WUS. - FFS how to carry LP-WUS information by overlaid OFDM sequences. - It doesn’t preclude considering the configuration where a single candidate overlaid OFDM sequence is used - FFS details of LP-WUS information to trigger PDCCH monitoring (e.g. whether above is applicable to one or more serving cells) Agreement Regarding the LP-WUS information to trigger PDCCH monitoring of RRC connected UEs (for non-CA case), select at least one from the following - Option 1: A bitmap with each bit corresponding to [one or more] UEs - Option 3: A codepoint value corresponding to [one or more] UEs FFS details for extension of option 1 and/or 3 when UE is configured with CA

Many companies provided their view on the remaining issues

- Value X (to be discussed in AI 9.6.1)

  - 8: Sony

  - 16: HW/HiSi, Lenovo

- LP-WUS duration (to be discussed in AI 9.6.1)

  - At most 2 slots: E/// (w/ evaluation)

- Bitmap vs codepoint (to be discussed in AI 9.6.1)

  - Option 1 (bitmap): vivo, ZTE, Sharp

  - Option 3 (codepoint): OPPO, DCM

- Other contents

  - Scell dormancy:

    - Yes: IDC, CATT

      - to make LP-WUS monitoring can be used for all the use cases supported by DCP

    - No: vivo, OPPO, Sharp

      - LP-WUS has limited payload

    - SCell dormancy indication can be achieved by other DCI formats

  - SSSG switching: Xiaomi

  - BWP switching: Xiaomi

Regarding the value X, since the above agreement was made in RAN1#116 meeting as a great compromise among companies, moderator assumes we don’t need to further down select the value in this agenda, but it can be further discussed in AI 9.6.1 considering performance, system overhead, etc.

Regarding Bitmap vs codepoint, this is highly related to LP-WUS structure and hence, those can be further discussed in AI 9.6.1.

Regarding the other contents than wake-up indication (PDCCH triggering), following proposal is made

[Close]Proposal 5-1:

- LP-WUS in RRC CONNECTED mode does not include Scell dormancy indication

| Company | Y/N | Comments |
| --- | --- | --- |
| Qualcomm | Y |  |
| xiaomi | Y |  |
| ZTE, Sanechips | Y |  |
| Panasonic | Y |  |
| Samsung | Y |  |
| Nokia1 |  | Before we agree this, we should first clarify/agree on how to support CA and dormant SCell behaviours whilst there are LP-WUS monitoring. |
| Spreadtrum | Y |  |
| Apple | Y |  |
| ETRI | Y |  |
| Lenovo | Y |  |
| LGE | Y |  |
| OPPO | Y |  |
| vivo | Y |  |

[Close]Proposal 5-1a:

- LP-WUS in RRC CONNECTED mode does not include Scell dormancy indication

FFS whether/how to support dormant SCell behaviours

| Company | Y/N | Comments |
| --- | --- | --- |
| Moderator |  | Following was agreed in Monday online: Agreement LP-WUS in RRC CONNECTED mode does not include Scell dormancy indication. - Note: LP-WUS indication does not impact SCell dormancy behavior |

Regarding whether wake-up indication is applicable to one or more serving cells, following agreements were made in previous RAN1 meetings

Agreement Study whether/how LP-WUS works when UE is configured with CA in RRC CONNECTED mode FFS: The cell(s) where PDCCH monitoring triggered by a LP-WUS is applicable Option 1: one or more serving cells based on gNB indication/configuration Option 2: all activated serving cells Note: other options are not precluded Agreement LP-WUS is at least supported for the case where a UE is configured with CA in RRC CONNECTED mode - FFS: DC Agreement LP-WUS is supported when UE is configured with NR-DC in RRC CONNECTED mode - Above is supported for the case PDCCH monitoring is triggered by LP-WUS in the same cell group - FFS: The cell(s) where PDCCH monitoring triggered by a LP-WUS is applicable

Many companies provided their view on the remaining issues:

- FFS: The cell(s) where PDCCH monitoring triggered by a LP-WUS is applicable

  - Option 1: CMCC, ZTE, SPRD (Option 1-2), LGE, lenovo, Sharp, MTK

  - Option 2: QC, vivo, E///, IDC, SPRD (Option 1-1). Apple, Pana, CATT, ETRI, DCM

  - Others

    - Nokia: PDCCH monitoring is started on only the cells where scheduling is supported and no implicit Scell status change.

    - Xiaomi: LP WUS indication can be per cell or per cell group when UE is configured with multiple serving cells.

    - OPPO: One serving cell should be only indicated by the LP-WUS in the same carrier.

- The cell(s) where LP-WUS is monitored

  - DC

    - on a serving cell per cell-group: QC, vivo (depending on UE capability), Nokia, Pana, DCM

    - on PCell only and indicate whether to wake-up PSCell in addition to Pcell:

    - onPCell or SCell in MCG to indicate PDCCH monitoring in both MCG and SCG:

  - CA without dual DRX

    - on a serving cell per cell-group: QC, vivo, Pana. Xiaomi, DCM

    - on Pcell: E///, ZTE (FFS SCell), Sharp

  - CA with dual DRX

    - vivo

      - on a serving cell per cell-group

        - FFS: LP-WUS received in the serving cell of primary DRX group indicates the triggering of PDCCH monitoring in primary DRX group only, or both DRX groups

    - QC

      - on a serving cell per DRX group

        - Opt.1: LP-WUS is configured to be monitored per DRX group

          - For dual DRX groups, LP-WUS monitoring is configured per DRX group

          - LP-WUS monitored on a cell within a DRX group triggers PDCCH monitoring on the cell of the DRX group

        - Opt.2: LP-WUS carries an indication for which DRX group(s) it triggers PDCCH monitoring

    - CMCC

      - LP-WUS should be separately configured for each DRX cell group.

    - DCM

      - For dual DRX groups, further consider following options

        - Option 1: LP-WUS monitoring/indication is applied per DRX group

        - Option 2: LP-WUS monitoring on one of DRX group, LP-WUS indication to either or both DRX group(s)

  - Special handling

    - HW/HiSi

      - For LP-WUS in FR1-FR2 CA, the trigger of PDCCH monitoring of FR2 cells can be separate from that of FR1 cells.

        - If PDCCH monitoring of an SCell is triggered, then PDCCH monitoring of PCell is also triggered, regardless their frequency range.

Regarding the FFS: The cell(s) where PDCCH monitoring triggered by a LP-WUS is applicable, companies views are quite divergent. Basically Option 1 supports finer wake-up indication with the cost of additional overhead, which is the main concern from companies supporting Option 2. Then the question is how much finer indication is necessary for CA/DC case. As pointed out by some companies, one of the main motivations for the separate indications would be dual DRX groups, typically used for FR1-FR2 CA. Hence, following proposal is made as starting point for further discussion.

[Old]Proposal 5-2:

- For the case when UE is configured with NR-DC and/or CA without dual DRX groups in RRC CONNECTED mode,

LP-WUS is configured on a serving cell per cell-group

LP-WUS indication is applicable to all activated serving cells in the cell-group

- For the case when UE is configured with NR-DC and/or CA with dual DRX groups in RRC CONNECTED mode,

Opt.1

    - LP-WUS is configured on a serving cell per cell-group

    - LP-WUS carries an indication for which DRX group(s) it triggers PDCCH monitoring

Opt.2

    - LP-WUS is configured on a serving cell per DRX group per cell-group

    - LP-WUS indication is applicable to all activated serving cells in the DRX group per cell-group

| Company | Y/N | Comments |
| --- | --- | --- |
| Qualcomm | Y | DRX operation in RAN2 is per DRX-group (for CA, if configured) and per-cell group (for DC). So we think the proposal makes sense. |
| CMCC | Y |  |
| ZTE, Sanechips | N | If the second DRX groups can be considered for FR2 cells or FR1 cells differentiation for PSG, then only when one DRX groups is configured, same solution should be pursued. Or we just do not support LP-WUS configured with second DRX group like DCP. |
| Panasonic | Y |  |
| Nokia1 |  | In principle, we are ok with the direction of this proposal, but we would like on FFS/discussion on what the ”LP-WUS indication” triggers, in particular, where it triggers PDCCH monitoring (accounting of CA x-scheduling options and SCell dormancy) |
| Apple | Y |  |
| Lenovo | Y |  |
| OPPO |  | It seems we have agreements that ”LP-WUS indication is applicable to all activated serving cells” and ”LP-WUS indication is applicable to one activated serving cells”. Both a OK for CA. But then for DC, it seem LP-WUS should always active a gourp of Cells. |
| vivo | Y | For the case when UE is configured with NR-DC and/or CA with dual DRX groups in RRC CONNECTED mode, there can also be another option: - LP-WUS is configured on a serving cell per cell-group - LP-WUS indication is applicable to all activated serving cells of all DRX groups in the cell-group. |
| Huawei, HiSilicon |  | We prefer to have separate control of PDCCH monitoring in FR1 and FR2 no matter dual DRX group is configured or not. Since typically for UE MR the FR1 and FR2 uses different hardware component, having separate control for FR1 and FR2 is beneficial for UE power saving. |

[Old]Proposal 5-2:

- For the case when UE is configured with NR-DC and/or CA without dual DRX groups in RRC CONNECTED mode,

LP-WUS is configured on a serving cell per cell-group

LP-WUS indication is applicable to all activated serving cells in the cell-group

- For the case when UE is configured with NR-DC and/or CA with dual DRX groups in RRC CONNECTED mode,

Opt.1

    - LP-WUS is configured on a serving cell per cell-group

    - LP-WUS carries an indication for which all activated serving cells of DRX group(s) it triggers PDCCH monitoring

Opt.2

    - LP-WUS is configured on a serving cell per DRX group per cell-group

    - LP-WUS indication is applicable to all activated serving cells in the DRX group per cell-group

Opt.3

    - LP-WUS is configured on a serving cell per cell-group

    - LP-WUS indication is applicable to all activated serving cells of all DRX groups per cell-group.

| Company | Y/N | Comments |
| --- | --- | --- |
| Qualcomm | Y |  |
| xiaomi | Y |  |

[Fri]Proposal 5-2a:

- For the case when UE is configured with NR-DC and/or CA without dual DRX groups in RRC CONNECTED mode,

- LP-WUS is configured on a serving cell per cell-group

- LP-WUS indication is applicable to all activated serving cells in the cell-group

- For the case when UE is configured with NR-DC and/or CA with dual DRX groups in RRC CONNECTED mode,

  - Opt.1

    - LP-WUS is configured on a serving cell per cell-group

    - LP-WUS carries separate indications for each DRX group(s) to trigger PDCCH monitoring of all activated serving cells

    - Note: This requires RAN2 coordination via LS

  - Opt.2

    - LP-WUS is configured on a serving cell per DRX group per cell-group

    - LP-WUS indication is applicable to all activated serving cells in the DRX group per cell-group

    - Note: This requires RAN2 coordination via LS

  - Opt.3

    - LP-WUS is configured on a serving cell per cell-group

    - LP-WUS indication is applicable to all activated serving cells of all DRX groups per cell-group.

FFS: Whether the serving on which the LP-WUS is configured is only PCell/PSCell or RRC configured cell other than PCell/PSCell

| Company | Y/N | Comments |
| --- | --- | --- |
| Moerator |  | Update based on companies comment during Thrusday online |
| Moderator |  | This proposal was not dsicussed during Friday online. Companies are engouraged to fuether check and consider towards next meeting. |

6	Coexistence with existing UE power saving features

This issue has been discussed in previous RAN1 meetings, and following agreements were made:

Agreement For RRC CONNECTED mode, LP-WUS can be configured without following existing features. Rel-16 DCP Rel-17 PDCCH skipping Rel-17 SSSG switching Rel-18 cell DTX Further study whether/how LP-WUS works with following existing features (PDCCH skipping, SSSG switching, cell DTX)

Note that RAN2 have made following agreements, and hence simultanoeus configuration of LP-WUS and DCP is no longer considered.

- In RRC_CONNECTED mode, RAN2 to further discuss the impacts of LP-WUS operation methods identified in RAN1. - For Option 1-1 (as described in RAN1 agreement), the LP-WUS monitoring occasion locates at a configured time offset before the start of drx-onDurationTimer. The range of time offset can be determined by RAN1. - For Option 1-1, RAN2 assumes the solutions/ operations introduced for DCP mechanism is taken as baseline. - RAN2 assume that legacy DCP and Option 1-1 is not configured simultaneously for a UE. - The LP-WUS related configuration for RRC CONNECTED state UE is provided via dedicated RRC message.

Many companies provided their view on the coexistence with existing features as follows:

- PDCCH skipping

  - HW/HiSi: After LP-WUS monitoring is activated, the UE should stop/suspend the Rel-17 PDCCH adaption mechanism, if configured

  - CMCC: LP-WUS procedure Option 1-1/Option 1-2 and Rel-17 PDCCH skipping/SSSG switching can be configured or activated to one UE simultaneously

  - ZTE: If LP-WUS is monitored outside active time, no spec impact is foreseen for PDCCH skipping and LP-WUS coexistence

  - Vivo: For option 1-2 of LP-WUS CONNECTED mode operation, an explicit indication can be used to terminate the DRX active time and to activate LP-WUS monitoring.

  - OPPO: When PDCCH Skipping is configured, the PDCCH skipping indication can enable the LP-WUS monitoring occasions

  - LGE: Co-existence of Rel-17 PDCCH monitoring adaptation and LP-WUS can be revisited after the LP-WUS procedure about PDCCH monitoring are discussed

  - E///: There is no issue for option 1-1 and option 1-2 on coexistence with existing power saving techniques like PDCCH skipping and SSSG switching.

  - DCM: LP-WUS can be configured with PDCCH skipping. FFS: PDCCH skipping can indicate to stop PDCCH monitoring and start LP-WUS monitoring

- SSSG switching

  - HW/HiSi: After LP-WUS monitoring is activated, the UE should stop/suspend the Rel-17 PDCCH adaption mechanism, if configured

  - Vivo: For a UE configured with both LP-WUS monitoring and Rel-17 SSSG switching, Rel-17 SSSG switching is applied during the time period when MR performs PDCCH monitoring. There should be no impact due to LP-WUS monitoring.

  - CMCC: LP-WUS procedure Option 1-1/Option 1-2 and Rel-17 PDCCH skipping/SSSG switching can be configured or activated to one UE simultaneously

  - ZTE: No much power saving is observed if LP-WUS is used for SSS switching and no issue is observed when LP-WUS coexist with SSSG switching

  - LGE: Co-existence of Rel-17 PDCCH monitoring adaptation and LP-WUS can be revisited after the LP-WUS procedure about PDCCH monitoring are discussed

  - E///: There is no issue for option 1-1 and option 1-2 on coexistence with existing power saving techniques like PDCCH skipping and SSSG switching.

  - DCM: LP-WUS can be configured with SSSG switching. No interaction between SSSG switching and LP-WUS monitoring is assumed

- Cell DTX

  - Vivo:

    - A UE does not expect to monitor the LP-WUS during the non-active periods of cell DTX of a serving cell, if cell DTX is activated for the serving cell with the LP-WUS monitoring

    - As in legacy, UE monitors PDCCH on a cell during the cell-DTX active period in case LP-WUS for the cell is detected.

  - HW/HiSi: LP-WUS monitoring outside legacy cell DTX active time according to the LP-WUS monitoring configuration can trigger the PDCCH monitoring

  - OPPO: LP-WUS is assumed to be disabled during the DTX time

  - ZTE: During Cell-DTX off period, UE does not need to monitor LP-WUS

  - Apple: Further consider whether/how to handle the case when LP-WUS monitoring occasions are all configured within cell DTX non-active period.

  - LGE: Further discuss how LP-WUS (and LP-SS) can be handled or interacted with cell DTX configurations

  - TCL: LP-WUS can be configured together with Cell DTX. When Cell DTX is configured and activated, UE monitors LP-WUS during the active time of cell DTX (i.e., celldtxdrx-onDurationTimer) and does not monitor LP-WUS during the non-active time of Cell DTX

  - DCM: LP-WUS can be configured with Cell DTX

    - During Cell DTX inactive time, the UE is not expected to monitor LP-WUS

    - FFS: whether/how to handle the case when all LP-WUS MOs are configured within cell DTX inactive time

Most all companies assume LP-WUS works with {PDCCH skipping, SSSG switching, cell DTX}. How LP-WUS works with these features can be discussed further.

[Old]Proposal 6-1:

- For RRC CONNECTED mode, LP-WUS can be configured with PDCCH skipping

FFS: PDCCH skipping can indicate to stop PDCCH monitoring and start LP-WUS monitoring

| Company | Y/N | Comments |
| --- | --- | --- |
| Qualcomm |  | We think LP-WUS should be able to work with PDCCH skipping. However, we prefer to delete the sub-bullet. Since the explicit indication will be discussed in Proposal 3.1-3, the proposal 6-1 should focus on the main bullet. |
| ZTE, Sanechips |  | We do not think ‘start LP-WUS monitoring’ is needed currently and should be removed. |
| Nokia1 | Y |  |
| Apple | Y |  |
| LGE |  | Even though a UE is indicated to skip PDCCH, the UE is in DRX active time. So, ‘start LP-WUS monitoring’ should be discussed if the proposal 3.1-1 is agreed. |
| OPPO | Y |  |
| vivo | Y | For the FFS, we think it is also highly related to the activation/deactivation of LP-WUS monitoring by gNB L1/L2 signaling. |
| Huawei, HiSilicon |  | We are generally OK with the main bullet. For the sub-bullet, it can be discussed in Section 4. |

[Open]Proposal 6-1a:

- For RRC CONNECTED mode, LP-WUS can be configured with PDCCH skipping

FFS: PDCCH skipping can indicate to stop PDCCH monitoring and start LP-WUS monitoring

| Company | Y/N | Comments |
| --- | --- | --- |
| xiaomi | Y |  |
| Moderator |  | This proposal was not dsicussed during offline/online in this meeting. Companies are engouraged to fuether check and consider towards next meeting. |

[Old]Proposal 6-2:

- For RRC CONNECTED mode, LP-WUS can be configured with SSSG switching

No interaction between SSSG swiching and LP-WUS monitoring is assumed

| Company | Y/N | Comments |
| --- | --- | --- |
| Qualcomm |  | We prefer to delete the sub-bullet, since unless any other specific agreement is made, the sub-bullet is a natural consequence. |
| CMCC | Y |  |
| ZTE, Sanechips | Y |  |
| Nokia1 | Y |  |
| Apple | Y |  |
| LGE | Y |  |
| OPPO | Y |  |
| vivo | Y |  |
| Huawei, HiSilicon |  | We share the similar view as QC. Main bullet is OK for us, but we still need more time to see whether there is any issue found for interaction with SSSG switching. |

[Open]Proposal 6-2a:

- For RRC CONNECTED mode, LP-WUS can be configured with SSSG switching

No interaction between SSSG swiching and LP-WUS monitoring is assumed

| Company | Y/N | Comments |
| --- | --- | --- |
| OPPO | Y |  |
| xiaomi | Y |  |
| Moderator |  | This proposal was not dsicussed during offline/online in this meeting. Companies are engouraged to fuether check and consider towards next meeting. |

[Open]Proposal 6-3:

- For RRC CONNECTED mode, LP-WUS can be configured with Cell DTX

During Cell DTX inactive time, the UE is not expected to monitor LP-WUS

FFS: whether/how to handle the case when all LP-WUS MOs are configured within cell DTX inactive time

| Company | Y/N | Comments |
| --- | --- | --- |
| Qualcomm | Y |  |
| CMCC |  | Cell DTX is introduced for NES. gNB can choose to not transmit LP-WUS during the non-active periods of cell DTX and UE blind detection may not cause a large additonal energy consumption. Thus, whether UE is expected to monitor LP-WUS can be further studied. |
| ZTE, Sanechips |  | Removing the FFS is better. If the gNB configure all the LP-WUS MOS within DTX inactive time, it also means the gNB does not want the UE to use LP-WUS. |
| Nokia1 |  | If the conclusion of the FFS is that LP-WUS can be monitored in CELL DTX inactive time (something we currently support), then does the first bullet have to be reverted? |
| Apple | Y |  |
| OPPO | Y |  |
| vivo | Y |  |
| Huawei, HiSilicon |  | We are OK with the main bullet. For the sub-bullets, we share the similar view as CMCC. We’d like to point out the fact that in NES topic, DCI format 2_6 is still monitored by UE outside the active time of cell DTX. Per our understanding, this is to provide gNB with more flexibility of scheduling. For example, if emergency traffic arrives and gNB decides to wake up earlier, gNB can use DCI format 2_6 to wake up UE earlier. Following the same principle, UE also needs to monitor LP-WUS outside the active time of cell DTX. Also considering that monitoring LP-WUS by LP-WUR is expected to consume little power, it does almost no harm for UE to monitor LP-WUS outside the active time of cell DTX. |
| Moderator |  | This proposal was not dsicussed during offline/online in this meeting. Companies are engouraged to fuether check and consider towards next meeting. |

7	Conclusions

Agreement

For LP-WUS monitoring in RRC CONNECTED mode, SSB and/or CSI-RS can be the QCL source of LP-WUS QCL type(s) is {Type A or Type C – for down-selection} and Type D, when applicable

Agreement

LP-WUS in RRC CONNECTED mode does not include Scell dormancy indication.

- Note: LP-WUS indication does not impact SCell dormancy behavior

Agreement

For LP-WUS CONNECTED mode operation, the following case is not supported:

- Both Option 1-1 and Option 1-2 are simultaneously configured for the same UE in a given cell

Agreement

- For LP-WUS MOs in connected mode for Option 1-1, following is considered further discussion and possible down-selection

  - Approach 1:

    - LP-WUS MOs, including periodicity and offset, are configured independently from the C-DRX periodicity/offset by new RRC parameter(s).

    - UE selectively monitors LP-WUS in the LP-WUS MOs identified by other parameters such as time offset prior to the start of drx-onDurationTimer.

  - Approach 2:

    - LP-WUS MOs, including periodicity and offset, are identified based on the slot that drx-onDurationTimer would start and a new RRC parameter indicating the time offset until the slot that drx-onDurationTimer would start.

  - FFS: An RRC parameter indicates the duration where the UE monitors LP-WUS [consecutively in time]

    - FFS: the consecutive LP-WUS MOs in time identified by the RRC parameter are associated with a same C-DRX cycle;

    - FFS: LP-WUS MOs in time identified by the RRC parameter provides consistent indication for the same C-DRX cycle of the UE, i.e., UE does not expect to detect more than one LP-WUSs with different indications for the UE in the LP-WUS MOs associated with the same C-DRX cycle.

- For LP-WUS MOs in connected mode for Option 1-2, following is considered further

  - Approach 1:

    - LP-WUS MOs, including periodicity and offset, are configured independently from the C-DRX periodicity/offset by new RRC parameter(s).

    - UE monitors LP-WUS in the LP-WUS MOs and starts a new timer for PDCCH monitoring triggered by LP-WUS, after a time offset.

  - Approach 2:

    - Periodicity/offset of the slot that the new timer for PDCCH monitoring may be potentially triggered by LP-WUS are configured by RRC parameters.

    - LP-WUS MOs, including periodicity and offset, are identified based on the slot that new timer would start and a new RRC parameter indicating the time offset until the slot that new timer would start.

  - FFS: An RRC parameter indicates the duration where the UE monitors LP-WUS [consecutively in time]

    - FFS: the consecutive LP-WUS MOs in time identified by the RRC parameter are associated with the same slot that the new timer would start;

    - FFS: LP-WUS MOs in time identified by the RRC parameter provides consistent indication for the same slot that the new timer would start, i.e., UE does not expect to detect more than one LP-WUSs with different indications for the UE in the LP-WUS MOs associated with the same slot that the new timer would start.

Agreement

- For Option 1-2 of LP-WUS CONNECTED mode operation, for explicit indication to stop the new timer for PDCCH monitoring in Option 1-2 and drx-InactivityTimer for the DRX group, RAN1 identified the following potential solution

  - Use existing DRX Command MAC CE and Long DRX Command MAC CE

  - When UE is not in C-DRX active time, UE monitors LP-WUS

- Send LS to RAN2 to ask the feasibility of the above potential solution. LS in R1-2410909.

Conclusion

- For Option 1-1 of LP-WUS CONNECTED mode operation, RAN1 assumes existing DRX Command MAC CE and Long DRX Command MAC CE can be used to stop drx-onDurationTimer and drx-InactivityTimer for the DRX group

  - When UE is not in C-DRX active time, UE monitors LP-WUS

- Send LS to RAN2 to ask whether RAN2 has same understanding or not. LS in R1-2410909.

Agreement

- For Option 1-2, for LP-WUS CONNECTED mode operation, the periodic CSI/L1-RSRP reporting operation is same as Rel-16 DCP, that is the UE can be configured with a parameter to enable/disable periodic CSI/L1-RSRP reporting, respectively:

  - If the parameter is NOT configured:

    - If the UE is not indicated to wake up by LP-WUS, the periodic CSI/L1-RSRP is not reported.

    - If the UE is indicated to wake up by LP-WUS, the periodic CSI/L1-RSRP is reported during the DRX active time.

  - If the parameter is configured:

    - If the UE is not indicated to wake up by LP-WUS, the periodic CSI/L1-RSRP is reported during the time given by the configured drx-onDurationTimer in DRX-Config for the case when UE is outside C-DRX active time.

    - If the UE is indicated to wake up by LP-WUS, the periodic CSI/L1-RSRP is reported during the DRX active time.

  - FFS: Whether RAN1 spec impact is required.

Companies are encouraged to consider [Fri]Combined Proposal 3.2-1/3.2-2c of R1-2410906 for further discussions in future meetings.

References

| [1] | RP-241824 | Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR) | vivo, NTT DOCOMO |
| --- | --- | --- | --- |
| [2] | R1-2400640 | Work plan for Low-power wake-up signal and receiver for NR | vivo, NTT DOCOMO |
| [3] | R1-2409266 | FL summary #5 on LP-WUS operation in CONNECTED mode | Moderator (NTT DOCOMO) |
| [4] | R1-2409425 | Procedures and functionalities of LP-WUS in CONNECTED mode | Huawei, HiSilicon |
| [5] | R1-2409446 | LP-WUS operation in CONNECTED mode | Nokia |
| [6] | R1-2409458 | Discussion on RRC CONNECTED mode LP-WUS monitoring | InterDigital, Inc. |
| [7] | R1-2409522 | Discussion on LP-WUS operation in CONNECTED mode | CMCC |
| [8] | R1-2409561 | Discussion on LP-WUS operation in CONNECTED mode | ZTE Corporation, Sanechips |
| [9] | R1-2409607 | Discussion on LP-WUS operation in CONNECTED modes | Samsung |
| [10] | R1-2409646 | Discussion on LP-WUS operation in CONNECTED modes | Spreadtrum, UNISOC |
| [11] | R1-2409691 | Discussion on LP-WUS operation in CONNECTED modes | vivo |
| [12] | R1-2409813 | LP-WUS operation in CONNECTED modes | Apple |
| [13] | R1-2409862 | Discussion on LP-WUS operation in RRC CONNECTED mode | NEC |
| [14] | R1-2409875 | Discussion on LP-WUS operation in CONNECTED mode | Panasonic |
| [15] | R1-2409906 | Discussion on LP-WUS operation in Connected mode | Xiaomi |
| [16] | R1-2409951 | System design and procedure of LP-WUS operation for UE in CONNECTED Modes | CATT |
| [17] | R1-2410073 | Further consideration on LP-WUS operation in connected mode | OPPO |
| [18] | R1-2410233 | LP-WUS operation in CONNECTED mode | Sony |
| [19] | R1-2410275 | Discussion on LP-WUS operation in CONNECTED modes | ETRI |
| [20] | R1-2410296 | Discussion on LP-WUS operation in CONNECTED modes | LG Electronics |
| [21] | R1-2410358 | Discussion on LP-WUS procedures in Connected mode | TCL |
| [22] | R1-2410363 | Discussion on LP-WUS operation in CONNECTED modes | Lenovo |
| [23] | R1-2410399 | Discussion on LP-WUS operation in CONNECTED mode | NTT DOCOMO, INC. |
| [24] | R1-2410421 | Discussion on LP-WUS operation in CONNECTED modes | Sharp |
| [25] | R1-2410446 | LP-WUS operation in CONNECTED mode | Ericsson |
| [26] | R1-2410488 | LP-WUR operation in connected mode | Qualcomm Incorporated |
| [27] | R1-2410512 | LP-WUS operation in CONNECTED modes | MediaTek Inc. |

Summary of observations/proposals in contributions

R1-2409425	Procedures and functionalities of LP-WUS in CONNECTED mode	Huawei, HiSilicon

- LP-WUS configuration (periodicity and offset) can be decoupled from C-DRX configuration. - Option 1-2 can achieve the same effect as Option 1-1 if configured properly. - Option 1-1 cannot achieve the same effect as Option 1-2 since shorter C-DRX cycle introduces more measurements and may not always feasible in real deployment. - DCI format 2_6 is monitored outside active time of cell DTX, which leaves gNB the flexibility to wake up UE earlier. - Monitoring LP-WUS also outside active time of cell DTX has marginal impact on power saving gain, since the power consumption for LP-WUS monitoring is expected to be low. - For the purpose of future discussion, RAN1 assumes at least the following as baseline. - LP-WUR is NOT able to receive signal(s) during the time where MR transmits/receives signals/channels. - FFS: additional consideration of simultaneous operation between LP-WUR and MR. - Further discuss the UE behaviour with respect to the LP-WUR monitoring when MR is performing measurement or transmitting periodic CSI/L1-RSRP. - FFS how to let gNB and UE have the common understanding on where the UE performs measurement. - FFS: UE reports its capability on whether its MR and LP-WUR can work simultaneously. - For LP-WUS in CONNECTED mode, if a UE receives LP-WUS targeting to itself, the UE is not required to start monitoring PDCCH before t0+A+B, where: - t0 is the time point corresponding to the end of LP-WUS. - A is the time duration for ramp-up reported as the one value from X candidate values in UE capability reporting. - B is the time duration to receive number C of SSB/TRS for time/frequency synchronization of MR, where C is reported as the number of SSB/TRS for time/synchronization in UE capability reporting. - UE does not expect gNB to configure a time offset that is shorter than the duration consisting of the one value reported from the X candidate values and the time duration required by synchronization: - the value indicated by UAI must be larger than the duration consisting of the one value reported from the X candidate values and the time duration required for synchronization; - If a LP-WUS triggering a PDCCH monitoring is received by the UE at symbol n, the UE is not expected to monitor a PDCCH before symbol n+Y where Y is the configured time offset. - In CONNECTED mode, maximum number of LP-WUS information bits is up to 16 bits. - The impact due to miss-detection/falsely-detection of LP-WUS activation/deactivation signaling should be minimized/avoided for CONNECTED mode UE by e.g. explicit feedback signaling from UE to gNB. - In CONNECTED mode, if LP-WUS monitoring is enabled, at least the following options are further supported. - Activation of LP-WUS monitoring by gNB L1/L2 signaling. - Deactivation of LP-WUS monitoring by the reception of a LP-WUS targeting to the UE. - Deactivation of LP-WUS monitoring based on UL transmission. - Note: Activation of LP-WUS monitoring based on timer has been supported by the agreement for UE behavior Option 1-2 in RAN1#118 and the agreement in RAN2#127bis. - Note: PDCCH monitoring of a UE is suspended if LP-WUS monitoring of the UE is activated, and vice versa. PDCCH monitoring of a UE is resumed if LP-WUS monitoring of the UE is deactivated. - It is not supported that both Option 1-1 and Option 1-2 are simultaneously configured for the same UE. - For the LP-WUS monitoring occasion configuration - For both Option 1-1 and 1-2, gNB configures a periodicity and an offset for LP-WUS MOs, where the periodicity is no larger than C-DRX cycle. - For Option 1-1, gNB additionally configures a window, which is before the time duration indicated by drx-OnDuraitonTimer. - For Option 1-2, there is no further restriction on LP-WUS configuration in relation with C-DRX configuration other than that LP-WUS periodicity is no larger than long C-DRX cycle. - Continuous monitoring is supported by configuring proper value of monitoring occasion number and monitoring periodicity. - If DCP is configured, after LP-WUS monitoring is activated, the UE should not monitor DCP in MR. - After LP-WUS monitoring is activated, the UE should stop/suspend the Rel-17 PDCCH adaption mechanism, if configured. - LP-WUS monitoring outside legacy cell DTX active time according to the LP-WUS monitoring configuration can trigger the PDCCH monitoring. - For LP-WUS in FR1-FR2 CA, the trigger of PDCCH monitoring of FR2 cells can be separate from that of FR1 cells. - If PDCCH monitoring of an SCell is triggered, then PDCCH monitoring of PCell is also triggered, regardless their frequency range. - The TCI state determination can be discussed after the progress achieved on LP-WUS monitoring activation/deactivation. - If LP-WUS monitoring is activated by PDCCH, UE can assume the same TCI state as the CORESET to receive the PDCCH activating LP-WUS monitoring. - If LP-WUS monitoring is activated by timer, the TCI state can be the same as the CORESET to receive the last PDCCH before the timer expires. - The QCL type between LP-WUS and SSB/CSI-RS is typeC. - In CONNECTED mode, monitoring parameters of LP-WUS can be adjusted by MR signaling and/or pre-defined rules at least for different coverage requirement.

R1-2409446	LP-WUS operation in CONNECTED mode	Nokia

Observations: Observation 1: For option 1-2, using the existing PDCCH coreset and search space definitions without the c-DRX On-duration limitation, there are PDCCH monitoring opportunities (coresets) potentially every slot. Observation 2: For option 1-2, the duration of the LP-WUS (e.g. 1 slot) combined with the waking up time of the MR (e.g. 5ms), can easily overlap multiple PDCCH monitoring opportunities (coresets). Observation 3: For option 1-2, the recovery time required for the network to determine a missed LP-WUS can be long and cause a degradation in performance. Observation 4: For option 1-2, a time domain window mechanism, where multiple LP-WUS MOs are available, provides the network with the option to apply repetitions dynamically to improve the reliability of the LP-WUS when needed. Observation 5: From a network perspective, it is preferable to minimise the size, X, of the minimum gap value candidate set. Observation 6: For RRC CONNECTED mode, legacy procedures can be used to vary the activation and dormancy states of SCells during the PDCCH monitoring phase between LP-WUS monitoring periods. Observation 7: For RRC CONNECTED mode, the key difference between an ACTIVATED SCell that is in a DORMANT state and one that is a NON-DORMANT state, is that the NON-DORMANT SCell supports both measurements and PDCCH monitoring whereas the DORMANT SCell supports only a minimal set of measurements. Observation 8: LP-WUS does not impose limitations on existing CA configuration. Observation 9: The existing beam failure recovery procedure means that there is no need to define a default TCI state for the LP-WUS. Observation 10: For CONNECTED mode, LP-WUR time and frequency synchronisation can be provided by the MR. Proposals: Proposal 1: For option 1-1, a time domain window is defined for MOs that the UE is expected to monitor for LP-WUS candidates. Note: The network can select which MO(s) within a time window, it uses to transmit LP-WUS. Proposal 2: For option 1-1, the earliest the UE is expected to start monitoring for PDCCH from receiving a LP-WUS, is the beginning of the slot where the UE would start the drx-onDurationTimer provided that is configured to equal or exceed the configured minimum time gap from the end of the LP-WUS MO time domain window. Proposal 3: For option 1-1, RAN1 discuss the characteristics required by the UE to configure the LP-WUS which may include: - Time gap from start of On-Duration to end of LP-WUS time window - Time gap from start of On-Duration to start of LP-WUS time window - LP-WUS MO Periodicity within time window - LP-WUS MO Starting Offset within the window - LP-WUS MO Frequency domain location - LP-WUS message configuration Proposal 4: For option 1-1, RAN1 discuss if the physical message characteristics of LP-WUS candidates sent within a time domain window can vary. Proposal 5: For connected mode, a LP-WUS used by a group of UEs, can be configured to indicate to individual UEs within that group, when to start PDCCH monitoring. Proposal 6: For option 1-2, the time domain window concept is supported. Proposal 7: For option 1-2, the PDCCH monitoring which LP-WUS can trigger monitoring of, are defined using existing mechanisms. Proposal 8: Option 1-3 is deprioritised. Proposal 9: RAN1 determine the factors that significantly impact the UE’s choice of minimum time gap. Some potential factors are listed below: - Sleep mode - cDRX settings - Mode of operation - Synchronisation sources/UE implementation - BWP switching - DC/CA operation Proposal 10: RAN2 to consider rationalisation of the value X, through early indication of relevant network configured aspects to the UE, before the UE reports a preferred minimum time gap. Proposal 11: When using the UE Assistance Information procedure, the UE is restricted to indicating alternative minimum gap values, that come from the set initially indicated in the UE capability procedure. Proposal 12: If the UE cannot meet the single minimum time gap for a given PDCCH monitoring opportunity, it defaults to monitoring the associated PDCCH monitoring opportunity. Proposal 13: For NR-DC in RRC connected mode, the cell-group (master or secondary) containing the cell where LP-WUS is monitored, is the only cell-group where LP-WUS can trigger PDCCH monitoring. Proposal 14: When LP-WUS is used to trigger PDCCH monitoring with CA configured, PDCCH monitoring is started on only the cells where scheduling is supported and no implicit Scell status change. Proposal 15: RAN1 discuss the handling of SCell dormancy when LP-WUS is in operation. Proposal 16: The Network re-uses the TCI-state framework for the PDCCH Coreset associated with the LP-WUS (option 1), to determine the QCL relationship with LP-WUS. Proposal 17: For both options 1-1 and 1-2, a new timer (option 3) is supported to trigger the deactivation of LP-WUS monitoring. Proposal 18: For both options 1-1 and 1-2, the activation and deactivation of LP-WUS monitoring can be achieved without additional indication from the UE (option 1). FFS: RAN4 to determine the characteristics of the MR measurements required to ensure this “no indication” method is reliable. Proposal 19: For both options 1-1 and 1-2, at least for scenarios where the UE has deactivated LP-WUS monitoring, some form of L1 based UE Assistance indication (Option 2) is provided by the UE. FFS: Whether this is also useful for activation Proposal 20: For CONNECTED mode, LP-SS is not needed during normal LP-WUR operation with LP-WUR. Proposal 21: For CONNECTED mode, there is no change to how existing MR based measurements (RRM/RLM/BFD/CSI) are determined when LP-WUS is enabled.

R1-2409458	Discussion on RRC CONNECTED mode LP-WUS monitoring	InterDigital, Inc.

Observation 1: Supporting only Option 1-2 can cause issues due to limited coverage of LP-WUS. Observation 2: Identifying an option between Option 1-1 and Option 1-2 for triggering PDCCH monitoring is needed when Option 1-1 and Option 1-2 are simultaneously configured. Observation 3: LP-WUS monitoring configuration should be selected to match the conditions UE is experiencing. Observation 4: Higher power saving can be achieved by activating LP-WUS monitoring only when UE is in appropriate conditions. Observation 5: gNB is unable to select all the appropriate LP-WUS monitoring configuration and activate/deactivate LP-WUS monitoring without UE assistance. Observation 6: Limiting PDCCH monitoring triggered by LP-WUS to a subset of serving cells can make LP-WUS monitoring not suitable for users that required fast access to additional bandwidth. Observation 7: A LP-WUS design that can control waking-up of each UE separately can provide higher power saving compared to a LP-WUS design in which multiple UEs share the same wake-up indication. Observation 8: Supporting SCell dormancy indication via LP-WUS is important to make LP-WUS monitoring can be used for all the use cases supported by DCP. Observation 9: Compact LP-WUS payload size is important to support coverage comparable with PUSCH for Msg3. Observation 10: LP-WUS payload can be reduced by implicitly indicating some of information of LP-WUS. Observation 11: Use of sleep states achieving higher power saving can cause increased latency while use of sleep states that supports lower latency can reduce power saving gains of LP-WUS monitoring. Observation 12: Configuration of periodic CSI/L1-RSRP reporting mechanism can be inefficient in case of long inactive durations. Proposal 1: Support simultaneous configuration of Option 1-1 and Option 1-2. Proposal 2: Introduce a mechanism to properly manage PDCCH monitoring based on Option 1-1 and Option 1-2. Proposal 3: PDCCH monitoring triggered by LP-WUS is enabled/disabled or activated/deactivated with UE assistance. Proposal 4: For a LP-WUS monitoring enabled UE, support activating/deactivating LP-WUS monitoring based on following preconfigured conditions. - For activating LP-WUS monitoring: level of UE activity (e.g., timer and/or counter associated with UE activity), channel/signal quality, mobility. - For deactivating LP-WUS monitoring: level of UE activity (e.g., timer and/or counter associated with UE activity), channel/signal quality, mobility, LP-WUS reception quality. Proposal 5: For OFDM based LP-WUR, both NR-SS (existing SSB) and LP-SS can be used to determine activation/deactivation of LP-WUS monitoring. Proposal 6: Introduce a mechanism to report UE decision of LP-WUS monitoring deactivation potentially with reason for the decision. Proposal 7: When UE is configured with CA in RRC CONNECTED mode, PDCCH monitoring triggered by a LP-WUS is applicable to all active cells (Option 2). Proposal 8: Indications supported in DCP, i.e., per UE wake-up indication and SCell dormancy indication, should be supported in LP-WUS. Proposal 9: A subset of LP-WUS information can be informed to UE implicitly using LP-WUS monitoring resource (e.g., LO and MO) association and structure of LP-WUS. Proposal 10: UE can be configured with more than one time offsets at a given time. Proposal 11: Reported minimum time gap between LP-WUS reception to PDCCH monitoring for each SCS defines the maximum time offset between the end point of LP-WUS LO/MO and starting point of associated timer for PDCCH monitoring (starting point of drx-onDurationTimer for Option 1-1 and new timer for Option 1-2). Proposal 12: For CONNECTED mode LP-WUS monitoring, introduce a mechanism to support CSI/L1-RSRP reporting triggered when inactivity periods exceed a preconfigured threshold.

R1-2409522	Discussion on LP-WUS operation in CONNECTED mode	CMCC

Proposal 1. Regarding the specific function of LP-WUS, support: - Option 1-2-2 (i.e. PDCCH monitoring is not triggered by legacy C-DRX cycle and drx-onDurationTimer when monitoring LP-WUS) under Option 1-2 for further study. - Option 1-1 and Option 1-2 are not simultaneously configured for the same UE. - A new timer is introduced for Option 1-2. Proposal 2. LP-WUS procedure Option 1-1 and Rel-16 DCP cannot be configured or activated to one UE simultaneously. Proposal 3. LP-WUS procedure Option 1-1/Option 1-2 and Rel-17 PDCCH skipping/SSSG switching can be configured or activated to one UE simultaneously. Proposal 4. Support the minimum time gap not include the duration for time/frequency sync of MR. - Also note that there is no need to define the specific composition of the candidate values. Proposal 5. Support LP-WUS QCLed with SSB and/or CSI-RS in QCL-Type A for FR1 scenario, and QCL-Type A+D for FR2 scenario. Proposal 6. Support not to introduce additional activation/deactivation mechanism for LP-WUS monitoring. Proposal 7. Regarding the issue on LP-WUS monitoring configuration, support: - For Option 1-1, a time offset between LP-WUS and the starting point of DRX ON is configured without the configuration of periodicity. - For Option 1-2, a time offset and periodicity can be configured similar as search space configuration. Proposal 8. Support LP-WUS applicable to one or more serving cells in a (DRX) cell group in CA/DC scenario. - For the case that a MAC entity has two DRX cell groups, LP-WUS should be separately configured for each DRX cell group.

R1-2409561	Discussion on LP-WUS operation in CONNECTED mode	ZTE Corporation, Sanechips

Observations: Observation 1: Monitoring DCI for LP-WUS activation, compared with RRC activation, causes more power consumption. Observation 2: When LP-WUS periodicity is the same as C-DRX, - There is no difference in terms of PSG and latency between option 1-1 and option 1-2-2. - There is no difference between option 1-1 and option 1-2-1 Observation 3: When LP-WUS periodicity is smaller than C-DRX, - Option 1-1 can not provide additional latency reduction but Option 1-2 can. Observation 4: If LP-WUS is configured in BWP framework, the retuning capability is required and the feasibility for RF-ED architecture may need further studied. Observation 5: - LP-WUS indicating wake-up for all SCells in FR1 or FR2 belongs to option1 - LP-WUS indicating wake-up for all SCells in first DRX group or second DRX group belongs to option1 - Option2 does not differentiate any serving cells group based on DRX or frequency range Observation 6: LP-WUS wake-up all the activated serving cells bring massive unnecessary power consumption. - LP-WUS can not bring PSG but more power consumption Observation 7: For LP-WUS indication method, bitmap method has obvious less system overhead than codepoint method especially for frequent traffic case. Observation 8: For LP-WUS indication method, bitmap method may occupies more MOs, but not obvious difference as codepoint method. Observation 9: For the CA/DC supportted in LP-WUS indication, bitmap method is easier to be extended. Observation 10: UAI information already provides offset information for PSG Observation 11: OFDM WUR has smaller minimum time gap than OOK WUR - OFDM WUR has less processing time than OOK WUR - OFDM WUR has less ramp-up time than OOK WUR Observation 12: If LP-WUS is monitored outside active time, no spec impact is foreseen for PDCCH skipping and LP-WUS coexistence. Observation 13: No much power saving is observed if LP-WUS is used for SSS switching and no issue is observed when LP-WUS coexist with SSSG switching. Proposals: Proposal 1: RRC activation/deactivation is the baseline - DCI deactivation and fallback mechanism based on that UE does not received LP-WUS for a long time can be considered. - The motivation for L2 activation/deactivation should be further clarified if L1 deactivation is introduced. - Option 4 is not considered for activation/deactivation of LP-WUS monitoring. Proposal 2: For option 1-1, the LP-WUS monitoring periodicity is the same as long C-DRX cycle. Proposal 3: For option 1-2, - An offset relative to legacy C-DRX can be configured - For option 1-2-1, offset applies between new/legacy active time and LP-WUS. - For option 1-2-2, offset applies between new active time and LP-WUS - Using RRC signalling indicates legacy timer could be triggered for PDCCH monitoring or only new timer could be used to trigger PDCCH monitoring - In case legacy timer could be triggered for PDCCH monitoring, if UE detects a LP-WUS in the nearest MOs or window relative to C-DRX, the legacy active time is triggered,otherwise, the new timer could be triggered - In case only new timer could be triggered for PDCCH monitoring, if UE detect a LP-WUS in MOs or window relative to C-DRX, the new active time is triggered. Proposal 4: Based on the periodicity configuration, no need to consider support simultaneously configuration of Option 1-1 and Option 1-2. - If the periodicity is the same as long C-DRX cycle, the LP-WUS is used to trigger the starting of the existing timer, e.g., drx-onDurationTimer - If the periodicity is smaller than the long DRX cycle, the LP-WUS is used to trigger the starting of the new timer (option 1-2-2) or existing timer and new timer(option 1-2-1) Proposal 5: For LP-WUS monitoring with option 1-1 or option 1-2, consider following two alternatives to achieve better scheduling flexibility - Alternative 1: multiple LP-WUS monitoring with different offsets - Alternative 2: a monitoring window with configurable length is considered for LP-WUS monitoring Proposal 6: The LP-WUS is applied for active DL BWP for PDCCH monitoring - LP-WUS could be configured outside the active BWP Proposal 7: Outside new or legacy active time, reusing legacy mechanism as DCP for Searchspace monitoring. Proposal 8: The LP-WUS is used to indicate wake-up for PDCCH monitoring - Target all the configured searchspaces in the active DL BWP Proposal 9: In connected mode, support multiple LP-WUS monitoring space configuration to have different TCI-state configuration Proposal 10: The LP-WUS QCL source could be CD-SSB, NCD-SSB or CSI-RS. - At least type C QCL relationship can be supported. Proposal 11: LP-WUS at least can be transmitted in PCell or PScell. - FFS transmitted in SCell which depends on separate band support. Proposal 12: Based on gNB indication/configuration, LP-WUS can indicate wake-up for one or more groups of serving cells, wherein each group includes one or more serving cells Proposal 13: Support Option 1: bitmap corresponding to [one or more] UEs as baseline Proposal 14: The candidate value for minimum time gap should be defined based on UE minimum capability, instead of targeting PSG. Proposal 15: The candidate value is defined based on LP-WUS processing time and ramp-up time from LR to MR - The candidate value should differ from different WUR type Proposal 16: Define two capability and describe the UE behavior based on the UE capability - Capability1: UE does not allow simultaneous operation between LR and MR - Identify the scenarios for behavior clarification. - Capability2: UE allow simultaneous operation between LR and MR

R1-2409607	Discussion on LP-WUS operation in CONNECTED modes	Samsung

Proposal 1: For activation/deactivation of LP-WUS monitoring, prioritize Option 2 and Option 3, and support explicit confirmation message(s) from the UE to the gNB for handshake in Option 2. Proposal 2: UE is only configured one from Option 1-1 and Option 1-2 by the gNB. Proposal 3: For configurations of LP-WUS monitoring occasions, support a time domain window for both Option 1-1 and Option 1-2: - Periodicity of the window is same as the DRX cycle; - Duration of the window is configured by RRC; - Offset between the start of the window and the start of DRX cycle is configured by RRC; - Interval between neighboring monitoring occasions is configured by RRC; - For Option 1-1, the window is located before the start of DRX cycle; - For Option 1-2, the window can be located anywhere within the DRX cycle, and the duration of the window can be configured same as the duration of the DRX cycle. Proposal 4: For the time gap between LP-WUS reception and MR to start PDCCH monitoring: - Support two set of minimum gaps per SCS (i.e., X=2); - UE’s reported value shall include the duration for time/frequency synchronization of MR; - Candidate values for the indicated preferred gap value by UAI shall be same as the candidate values for the UE capability reporting. Proposal 5: The QCL source for LP-WUS can be provided to the UE as part of the configuration for LP-WUS. Proposal 6: RAN1 needs to clarify whether to support the secondary group DRX configuration with LP-WUS.

R1-2409646	Discussion on LP-WUS operation in CONNECTED modes	Spreadtrum, UNISOC

Proposal 1: Both Option 1-1 and Option 1-2-1 simultaneously configured for the same UE should be supported. Proposal 2: When both Option 1-1 and Option 1-2-1 simultaneously configured for the same UE, if the drx-onDurationTimer is not started by the first LP-WUS, there are three kinds of UE behaviors during the drx-onDurationTimer. - Alt 1：UEs do not need to monitor second LP-WUS during the drx-onDurationTimer. - Alt 2: UEs need to monitor second LP-WUS during the drx-onDurationTimer. - Alt 3: The first LP-WUS indicates whether the UE needs to monitor second LP-WUS during the drx-onDurationTimer. Proposal 3: For Option 1-1, the periodicity of LP-WUS monitoring can be derived from DRX cycle Proposal 4: For Option 1-2, the periodicity and offset of LP-WUS can be different from those from C-DRX configuration. Proposal 5: Activation/deactivation of LP-WUS monitoring by gNB L1 signaling (e.g., DCI) can be considered. Proposal 6: For Option 1-2, wake-up indication by LP-WUS can be applied for a group of activated serving cells for triggering PDCCH monitoring.

R1-2409691	Discussion on LP-WUS operation in CONNECTED modes	vivo

Proposal 1: - For option 1-1 of LP-WUS CONNECTED mode operation, a UE does not monitor LP-WUS in DRX Active Time. - For option 1-2 of LP-WUS CONNECTED mode operation, a UE does not monitor LP-WUS in DRX Active Time. An explicit indication can be used to terminate the DRX active time and to activate LP-WUS monitoring. Proposal 2: For RRC connected mode, at least support the case where LP-WUR is NOT able to receive signal(s) during the time period when MR is transmitting or receiving signals/channels. Proposal 3: For option 1-2 of LP-WUS CONNECTED mode operation, the periodic CSI/L1-RSRP reporting operation is same as Rel-16 DCP, that is the UE can be configured with a parameter to enable/disable periodic CSI/L1-RSRP reporting, respectively: - If the parameter is NOT configured: - If the UE is not indicated to wake up by LP-WUS, the periodic CSI/L1-RSRP is not reported. - If the UE is indicated to wake up by LP-WUS, the periodic CSI/L1-RSRP is reported during the DRX active time as in legacy specification. - If the parameter is configured: - If the UE is not indicated to wake up by LP-WUS, the periodic CSI/L1-RSRP is reported during the time given by the configured drx-onDurationTimer in DRX-Config, even if the drx-onDurationTimer does not start. - If the UE is indicated to wake up by LP-WUS, the periodic CSI/L1-RSRP is reported during the DRX active time as in legacy specification. Proposal 4: For LP-WUS operation Option 1-2, no other restrictions for LP-WUS monitoring configuration in relation with C-DRX configuration are needed, except LP-WUS periodicity is no larger than long C-DRX cycle. Proposal 5: The simultaneous configuration of both Option 1-1 and Option 1-2 for the same UE is not supported. Proposal 6: For both LP-WUS operation option 1-1 and option 1-2, LP-WUS does not support SCell dormancy indication. Proposal 7: For both LP-WUS operation option 1-1 and option 1-2, a UE monitors LP-WUS only on one serving cell per MAC entity when UE is configured with CA in RRC CONNECTED mode. - The serving cell where to monitor LP-WUS is configured by the network. - This applies to UEs configured with single and dual DRX groups Proposal 8: For both LP-WUS operation option 1-1 and option 1-2, when UE is configured with CA in RRC CONNECTED mode, the LP-WUS information is applied to all activated serving/scheduling cells. - FFS for the case of dual DRX groups, LP-WUS received in the serving cell of primary DRX group indicates the triggering of PDCCH monitoring in primary DRX group only, or both DRX groups Proposal 9: For a UE configured with DC and for both LP-WUS operation option 1-1 and option 1-2, it can support LP-WUS monitoring on a serving cell per cell-group depending on UE capability. - The serving cell where to monitor LP-WUS per cell group is configured by the network. - The LP-WUS information is applied to all activated serving/scheduling cells within each cell group. Proposal 10: For both LP-WUS operation option 1-1 and option 1-2 in RRC CONNECTED mode, - UE reports one value for each SCS from X, where X = 2 or 3 candidate values for the determination of the minimum time gap between LP-WUS reception and MR to start PDCCH monitoring via UE capability reporting. - The reported value(s) of minimum time gap between LP-WUS reception and MR to start PDCCH monitoring includes LP-WUS processing time and MR transition time for ramp up and does not include the duration for time/frequency synchronization of MR. Proposal 11: Clarify that the SCS for the reported value(s) for the determination of the minimum time gap between LP-WUS reception and MR to start PDCCH monitoring is the respective SCS of the carrier where LP-WUS is monitored. Proposal 12: - UAI details should be decided in RAN2. - A single time offset is configured for a UE at a given time. Proposal 13: In RRC_CONNECTED mode, if the LP-WUS is not detected on any WUS monitoring occasion (MO) of a WUS monitoring window if configured by the UE, - For LP-WUS operation option 1-1, the UE is not required to trigger the starting of the drx-onDurationTimer. - For LP-WUS operation option 1-2, the UE is not required to wake up the MR for PDCCH monitoring for option 1-2. Proposal 14: In RRC_CONNECTED mode, for both LP-WUS operation option 1-1 and option 1-2, the indication from UE to indicate whether it monitors LP-WUS or PDCCH is not needed. Proposal 15: After LP-WUS monitoring is enabled by RRC signalling, the specified conditions used for LP-WUS deactivation due to poor channel condition for LP-WUS monitoring are not needed for both LP-WUS operation option 1-1 and option 1-2. Proposal 16: UE is allowed to fallback to MR PDCCH monitoring by implementation (e.g. due to poor channel condition) without specified condition. - For LP-WUS operation option 1-1, the fallback behavior for PDCCH monitoring is according to the C-DRX configuration - For LP-WUS operation option 1-2, the fallback behavior for PDCCH monitoring is always-on PDCCH monitoring. Proposal 17: For LP-WUS operation option 1-1 and option 1-2 in RRC_CONNECTED mode, UL transmissions such as SR, PRACH and CG PUSCH can trigger MR PDCCH monitoring according to the legacy UE behavior. Proposal 18: For LP-WUS operation option 1-1 and option 1-2 in RRC_CONNECTED mode, UE switching from PDCCH monitoring to LP-WUS monitoring is supported by gNB L1/L2 signaling. Proposal 19: if PDCCH skipping indication is used as LP-WUS activation signalling, enhanced PDCCH skipping indication can be used to indicate UE to terminate the DRX active time (e.g., stop PDCCH monitoring on one or multiple serving cells) and starts LP-WUS monitoring. Proposal 20: For a UE configured with both LP-WUS monitoring and Rel-17 SSSG switching, Rel-17 SSSG switching is applied during the time period when MR performs PDCCH monitoring. There should be no impact due to LP-WUS monitoring. Proposal 21: A UE does not expect to monitor the LP-WUS during the non-active periods of cell DTX of a serving cell, if cell DTX is activated for the serving cell with the LP-WUS monitoring. Proposal 22: As in legacy, UE monitors PDCCH on a cell during the cell-DTX active period in case LP-WUS for the cell is detected. Proposal 23: LP-WUS monitoring occasions (MOs) are defined by a network-configured Periodicity and Offset. The configuration should allow multiple LP-WUS MOs within each periodicity. - For LP-WUS operation option 1-1, the LP-WUS periodicity equals to the C-DRX periodicity - For LP-WUS operation option 1-2, the LP-WUS periodicity is separately configured from C-DRX periodicity and no larger than long C-DRX cycle. Proposal 24: For RRC CONNECTED mode, for LP-WUS monitoring in FR2, QCL Type D is applicable for LP-WUS QCLed with SSB/CSI-RS. - FFS QCL Type A, B and C for FR1 and FR2 Proposal 25: Re-use existing TCI state with or without unified framework to support the TCI state configuration/indication for a LP-WUS resource. Proposal 26: Support at least bitmap for RRC connected mode.

R1-2409813	LP-WUS operation in CONNECTED modes	Apple

Switching to LP-WUS during Active Time Proposal 1: Support UE switching to LP-WUS monitoring within Active Time, based on L1/L2 signaling or timer. - It can be further discussed which active timer(s) are considered. Proposal 2: For option 1-3 of LP-WUS CONNECTED mode operation, the followings are assumed from RAN1 perspective: - For option 1-3 of LP-WUS CONNECTED mode operation, the followings are assumed from RAN1 perspective. LP-WUS monitoring inside at least legacy C-DRX Active Time based on additional indication according to the LP-WUS monitoring configuration to trigger PDCCH monitoring. - FFS details of the additional indication, based on L1/L2 signaling or timer - UE is not required to monitor PDCCH when monitoring LP-WUS in the Active Time UE is configured with legacy C-DRX configurations as Rel-18 UE behaviors related to CDRX active time triggered by all legacy DRX timers and the new timer triggered by LPWUS are not affected unless included in this proposal UE is expected to be configured with LP-WUS monitoring configuration (periodicity and offset can be different from those from C-DRX configuration) LP-WUS monitoring modes Proposal 3: Support the following four monitoring modes of LP-WUS monitoring, and NW configures the corresponding mode for the LP-WUS configuration: - Mode 1: LP-WUS monitoring occasions configured by RRC are always monitored. - Mode 2: LP-WUS monitoring occasions configured by RRC are monitored outside Active Time and not monitored within Active Time. - Mode 3: LP-WUS monitoring occasions configured by RRC are monitored outside Active Time and not monitored within Active Time until L1/L2 signaling indication. - Mode 4: LP-WUS monitoring occasions configured by RRC are not monitored by default and are monitored according to L1/L2 signaling within Active Time. LP-WUS monitoring occasions Proposal 4: If Option 1-1 is supported, a time window for LP-WUS monitoring can be configured, otherwise no explicit configuration of time window is needed. UE fallback behavior Proposal 5: Support UE fall back to PDCCH monitoring: - For Option 1-1, UE falls back to legacy C-DRX behaviors - For Option 1-2, the following alternatives can be considered: - Alt 1: UE falls back to C-DRX behavior, and indicate the fallback state to NW - Alt 2: UE falls back to PDCCH monitoring according to LP-WUS cycle Minimum time gap Proposal 6: For the preferred configured time offset between LP-WUS and PDCCH reception using UAI mechanism, it is up to UE to report a larger or a smaller value than that reported by UE capability. Proposal 7: Time offsets between LP-WUS and start of PDCCH monitoring can be different within or outside C-DRX Active time. LP-WUS operation with CA Proposal 8: Support LP-WUS triggerring all activated serving cells (Option 2), considering impact on UE complexity as well as NW signaling overhead. Coexistence with existing cell DTX Proposal 9: Further consider whether/how to handle the case when LP-WUS monitoring occasions are all configured within cell DTX non-active period.

R1-2409862	Discussion on LP-WUS operation in RRC CONNECTED mode	NEC

Proposal 1: support LP-WUS in combination with DCP to further reduce power consumption without introducing too much overhead. Proposal 2: support Option 1-1 and Option 1-2 simultaneously configured for the same UE.

R1-2409875	Discussion on LP-WUS operation in CONNECTED mode	Panasonic

Proposal 1: To support simultaneous configuration of both Option 1-1 and Option 1-2 for the same UE, at least following aspects should be discussed: - How to handle the LP-WUS monitoring configured by both Option 1-1 and 1-2, including the potential overlapping of LP-WUS MO of Option 1-1 and Option 1-2. - Whether/how drx-onDurationTimer triggered by Option 1-1 work together with the new timer -triggered by Option 1-2. Proposal 2: The reported value in UE capability report should be defined as the summed value of LP-WUS processing delay in LR, ramping up time of MR, the duration for time/frequency synchronization of MR and optionally the required time to change the carrier. Proposal 3: How to jointly use the UE capability report and UAI mechanism should be further discussed. Proposal 4: For supporting CA, LP-WUS can be available in either PCell or SCell and trigger PDCCH monitoring in all activated serving cells (= Option 2 in RAN1 #117 agreement). Proposal 5: For supporting DC, as same as supporting CA, LP-WUS can be available in either PCell or SCell to trigger PDCCH monitoring in all activated serving cells within the same cell group. UE is not required to support two LRs for two Cell groups. Proposal 6: The relation between SSB and LP-WUS is QCL Type A or Type C in RRC CONNECTED mode, as same as QCL configuration in RRC IDLE/INACTIVE mode. To support beam forming, QCL type D should be configurable. Proposal 7: The relation between TRS and LP-WUS is QCL Type A or Type C when TRS is configured. To support beam forming, QCL type D can be configurable. Proposal 8: The relation between CSI-RS other than TRS and LP-SS/LP-WUS is not required to be defined.

R1-2409906	Discussion on LP-WUS operation in Connected mode	Xiaomi

Proposal 1: How to reduce unnecessary PDCCH monitoring within legacy C-DRX active time should be considered for both Option 1-1 and Option 1-2. Proposal 2: LP WUS applied within C-DRX legacy active time to reduce unnecessary PDCCH monitoring should be considered and down-selected, Case 1: LP WUS is used within C-DRX re-transmission timer running duration; Case 2: LP WUS is used within C-DRX active time and within PDCCH skipping duration; Case 3: LP WUS is used within C-DRX active time and outside PDCCH skipping duration. Proposal 3: Support Option 1-3: LP-WUS monitoring within legacy C-DRX active time to reduce unnecessary PDCCH monitoring. Proposal 4: Support multiple repetitions of a LP WUS occasion. Proposal 5: LP WUS can be configured in the same or different carrier/band from where MR operates. Proposal 6: For CA scenario, UE is only required to monitor LP WUS on one carrier. Proposal 7: Support unified LP WUS configuration for Option 1-1/1-2/1-3. A monitoring window before on duration in each C-DRX cycle can be additionally configured for Option 1-1. Proposal 8: LP WUS at least includes bits indicating whether to monitor PDCCH. SSSG switching, BWP switching indication can also be considered. Proposal 9: LP WUS indication can be per cell or per cell group when UE is configured with multiple serving cells. Proposal 10: To support LP WUS configuration in NR-DC, UE should be able to receive LP WUS on two bands simultaneously by LR. Proposal 11: For RRC CONNECTED mode, there are at least 3 candidate values for the determination of the minimum time gap between LP-WUS reception and MR to start PDCCH monitoring via UE capability reporting. Proposal 12: Impact of semi-persistent channels on LP WUS wake-up delay should be considered. Proposal 13: Support CSI report skipped or relaxed during the MR’s non-active period and support CSI report for additional active time triggered by LP WUS. Proposal 14: LP WUS in connected state can be QCLed with LP -SS by QCL type A, and can be QCLed with SSB by QCL type C.

R1-2409951	System design and procedure of LP-WUS operation for UE in CONNECTED Modes	CATT

Proposal 1: For RRC_CONNECTED mode, the LP-WUS could be configured for one or more UEs for the control signaling to indicate the UE wake up in the subsequent DRX ON. Proposal 2: The minimum time gap of LP-WUS needs to take into account the time of MR in preparation of channel tracking for PDCCH coherent demodulation when LP-WUS triggers UE wakeup. Proposal 3: The candidate value for UE reported in the UE capability includes the LP-WUS processing time, MR transition time for ramp up and the time/frequency synchronization of MR. Proposal 4: The periodicity of LP-WUS MOs can be configured deterministically without considering a time window. Proposal 5: One LP-WUS is configured to be QCLed with the RS associated with each of the TCI states of the CORESETs in order to trigger the PDCCH monitoring at the associated CORESETs when LP-WUS is detected. Proposal 6: C-DRX adaptation by LP-WUS in Option 1-1 and Option 1-2 should NOT be configured simultaneously for an UE. Proposal 7: UE can be configured with parameters to enable/disable the periodic CSI and/or L1-RSRP reporting when the C-DRX is not triggered by LP-WUS to wake up for PDCCH monitoring, respectively. - If the related parameter is NOT configured with value true and UE does NOT report the periodic CSI and/or L1-RSRP. - Otherwise, UE reports the periodic CSI and/or L1-RSRP regardless UE is indicated by LP-WUS to wake up or not. Proposal 8: UE reports the periodic CSI/L1-RSRP on PUCCH resource configured at each C-DRX cycle indicated by the legacy drx-onDurationTimer. Proposal 9: UE assistance for PDCCH monitoring triggered by LP-WUS is not supported. Proposal 10: Support Option 1: No additional indication/condition is introduced for activation/deactivation of LP-WUS monitoring. Proposal 11: PDCCH monitoring triggered by a LP-WUS is applicable for all serving cells with SCell dormancy indication up to 5 SCells by LP-WUS.

R1-2410073	Further consideration on LP-WUS operation in connected mode	OPPO

Proposal 1: A UE can be indicated by LP-WUS codepoints, in which each value can be corresponding to one or more UEs in CONNECTED mode. UE in CONNECTED mode is UE-specifically configured with the LP-WUS resources and codepoints. One UE can be configured with up to 2 codepoints for monitoring wake-up indication. Proposal 2: The LP-WUS monitoring periodicity and offset can be introduced for triggering a PDCCH monitoring timer, in CONNECTED mode for both Options. For option 1-1, the LP-WUS monitoring can be further restricted to a range of time before the DRX-ON. For option 1-2, the LP-WUS monitoring occasion can be defined during at least for inactive time. Proposal 3: The LP-WUS should be configured with following legacy schemes in CONNECTED mode: Rel-16 Cross-slot scheduling can be configured together with LP-WUS in CONNECTED mode without specification impact. When PDCCH Skipping/SSSG switching is configured, the PDCCH skipping indication can enable the LP-WUS monitoring occasions. For the Cell DTX, LP-WUS is assumed to be disabled during the DTX time. Proposal 4: When the LP-WUS is configured with CA in CONNECTED mode: One serving cell should be only indicated by the LP-WUS in the same carrier. Only wake-up information is carried by LP-WUS, e.g., no Scell dormancy indication. Proposal 5: For the LP-WUS activation and deactivation by pre-configured condition(s), timer-based monitoring of LP-WUS is the baseline. Proposal 6: RAN1 can assume that MR measurements are one of the UE assistant information for the LP-WUS activation and deactivation. RAN1 discuss the introduction LP-SS measurement in CONNECTED mode as another UE assistant information. Proposal 7: For the LP-WUS the LR/MR activation assumption may be needed. We are fine for conclude the design assumption with clear definition of LR and MR behavior. The specification impact is to be further discussed. Proposal 8: LP-WUS Monitoring Occasions can be associated with CORESETs for the TCI states determination. The activated TCI state of the CORESET is applied for the associated LP-WUS Monitoring Occasion. Proposal 9: In RRC CONNECTED mode, support UE capability report one value among 3 candidate values for minimum time gap between LP-WUS reception and MR to start PDCCH monitoring. 20 ms, 6 ms and 2 ms are the candidature values. UE could additional report another smaller gap than its capability report as UAI. Proposal 10: RAN1 may specify that the monitoring occasion of LP-WUS and associated monitoring occasion of PDCCH should meet the reported minimum gap.

R1-2410233	LP-WUS operation in CONNECTED mode	Sony

Proposal 1: Support LP-WUS in RRC connected mode with multiple minimum time-gaps depending on UE capability. Proposal 2: Support a single time offset (FFS: explicit / implicit) that can be configured for a UE at a given time. Proposal 3: At least for Option 1-2, LP-WUS transmission in RRC Connected mode indicates the UE on the subsequent action to be performed upon the reception of LP-WUS. Proposal 4: For option 1-2, support a UE to monitor multiple sub-LP WUS occasions (sLOs) in which each sLO is associated with one or more DRX parameters (e.g., the starting time of DRX ON). Proposal 4: RAN1 strives for the LP-WUS common design including supporting sequence-based LP-WUS operation in both RRC CONNECTED and RRC idle/inactive mode. Proposal 5: The maximum number of LP-WUS information bits for RRC connected mode is up to 8 bits Proposal 6: The UE operating in RRC connected mode and configured to receive LP-WUS is expected to monitor a given LP-WUS resource (i.e., LP-WUS occasion). Proposal 7: In addition to duty-cycled monitoring mode, continuous monitoring mode for LP-WUS in RRC-connected is also supported. Proposal 8: Enabling/Disabling of LP-WUS monitoring by RRC signalling is sufficient. No additional indication/condition are introduced for activation/deactivation of LP-WUS monitoring (known as Option 1).

R1-2410275	Discussion on LP-WUS operation in CONNECTED modes	ETRI

Proposal 1: In the previous agreement for Option 1-2, the last sub-bullet should be removed as follows: Agreement - For option 1-2 of LP-WUS CONNECTED mode operation, the followings are assumed from RAN1 perspective. LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring. UE is configured with legacy C-DRX configurations as Rel-18 UE is expected to be configured with LP-WUS monitoring configuration (periodicity and offset can be different from those from C-DRX configuration) - FFS potential restriction for LP-WUS configuration in relation with C-DRX configuration LP-WUS triggers the start of a timer during which UE monitors PDCCH - FFS the timer is existing timer and/or new timer UE PDCCH monitoring behaviors related to other legacy DRX timers are not affected - drx-InactivityTimer, drx-RetransmissionTimerDL, drx-RetransmissionTimerUL, drx-HARQ-RTT-TimerDL, drx-HARQ-RTT-TimerUL No impact on RRM/RLM/BFD measurement requirements is assumed For periodic CSI/L1-RSRP reporting, UE can be configured with one of the following (same as Rel-16 DCP and option 1-1) - Periodic CSI/L1-RSRP is not reported if UE is not indicated to wake-up - Periodic CSI/L1-RSRP is periodically reported regardless if UE is indicated to wake-up or not UE PDCCH monitoring is not triggered by legacy C-DRX cycle and drx-onDurationTimer when monitoring LP-WUS. Proposal 2: Both Option 1-1 and Option 1-2 should be supported simultaneously when Option 1-2 is configured. Proposal 3: If the new timer triggered by LP-WUS of Option 1-2 is overlapped with legacy C-DRC cycle, the new timer should be stopped before the start of legacy C-DRX cycle in order to prevent the conflict between the new timer and drx-onDurationTimer. - FFS: how to handling the collision between the new timer and LP-WUS monitoring for Option 1-1 if Option 1-1 and Option 1-2 are supported simultaneously. Proposal 4: It is proposed to focus only duty-cycled monitoring mode for LP-WUS in Rel-19. Proposal 5: For RRC CONNECTED mode, UE reports one value for each SCS from X=2 candidate values for the determination of the minimum time gap between LP-WUS reception and MR to start PDCCH monitoring via UE capability reporting. - FFS: definition of reported candidate values Proposal 6: Regarding the restriction for LP-WUS configuration for Option 1-2, further restriction is necessary. - The periodicity for LP-WUS should be the same as the C-DRX periodicity. Proposal 7: In order for gNB to know that LP-WUS is monitored by a UE correctly, it will be sufficient the implicit indication such as - HARQ-ACK feedback corresponding to scheduled downlink transmission - Scheduled uplink transmission - Other options are not precluded. Proposal 8: In case LP-WUS monitoring is enabled, no additional indication/condition is introduced for activation/deactivation of LP-WUS monitoring. Proposal 9: It should be supported fallback mechanisms which the MR wakes up and switches to legacy operation in case the channel condition of LP-WUS is not good enough. - Details of fallback mechanisms can be discussed further. Proposal 10: For the purpose of future discussions, RAN1 assumes at least the following - For RRC connected mode, - LP-WUR and MR cannot support simultaneous operation at a given time instance. - FFS: additional consideration of simultaneous operation between LP-WUR and MR Proposal 11: Regarding the cell(s) where PDCCH monitoring triggered by a LP-WUS is applicable, Option 2 should be considered for further study. - Option 2: all activated serving cells Proposal 12: For LP-WUS transmission, the latest TCI state for CORESET can be applied. - Default TCI state can be also pre-configured in order to prevent application of outdated TCI state.

R1-2410296	Discussion on LP-WUS operation in CONNECTED modes	LG Electronics

Observation #1: Option 1-1 can be more robust than Option 1-2, if Option 1-1 is configured with the same principle as DCP. Proposal #1: Specify LP-WUS MOs with a periodicity. - LP-WUS inside monitoring window triggers PDCCH monitoring by starting drx-onDurationTimer (Option 1-1). - LP-WUS outside monitoring window triggers PDCCH monitoring outside legacy DRX Active Time (Option 1-2). Proposal #2: Consider candidate starting points of potential DRX Active Time triggered by LP-WUS. Proposal #3: Support simultaneous configuration of Option 1-1 and Option 1-2. Proposal #4: Consider a UE can monitor LP-WUS to trigger the starting of drx-onDurationTimer (i.e., Option 1-1) when the UE is monitoring PDCCH outside legacy DRX Active Time. Proposal #5: Consider the starting of drx-onDurationTimer without PDCCH monitoring periodically based on legacy DRX configuration. Proposal #6: The minimum time gap between LP-WUS reception and MR to start PDCCH monitoring should include the duration required for time/frequency synchronization of MR. Proposal #7: For LP-WUS monitoring in RRC CONNECTED mode, consider followings on TCI-state of LP-WUS - Re-use existing TCI-state framework to indicate TCI state of LP-WUS. - If TCI-state is configured but not indicated, a UE assumes that LP-WUS is QCLed with SSB that the UE recently received for random access or initial access. Proposal #8: Support LP-WUS triggering PDCCH monitoring for the serving cells based on gNB indication/configuration. (i.e., option 1). Proposal #9: Study how SCell of the UE is operated when start of drx-onDurationTimer is triggered by LP-WUS replacing DCP. Proposal #10: Co-existence of Rel-17 PDCCH monitoring adaptation and LP-WUS can be revisited after the LP-WUS procedure about PDCCH monitoring are discussed. Proposal #11: Further discuss how LP-WUS (and LP-SS) can be handled or interacted with cell DTX configurations. Proposal #12: Specify activation/deactivation of LP-WUS monitoring by gNB L1/L2 signaling with or without UE assistance. Proposal #13: Specify activation/deactivation of LP-WUS monitoring based on DRX timer and/or UL transmission at least considering followings, - LP-WUS monitoring is deactivated when drx-onDurationTimer and drx-InactivityTimer is running. - LP-WUS monitoring is activated when drx-RetransmissionTimerDL, drx-RetransmissionTimerUL, drx-RetransmissionTimerSL, ra-ContentionResolutionTimer, or msgB-ResponseWindow is running. Proposal #14: Consider the case where LP-WUS monitoring is in active state even during PDCCH monitoring is activated. Proposal #15: To overcome limited maximum number of LP-WUS information bits, consider the combination of time/frequency resources and LP-WUS payload for UE identification. Proposal #16: For supporting more functionality or information bits, consider the transmission of overlaid OFDM sequence over LP-WUS for CONNECTED modes. Proposal #17: Discuss how to compose the payload and overlaid OFDM sequence of LP-WUS for CONNECTED mode at least as follows, - The essential indication for UE behavior is contained in the payload of LP-WUS. - The additional information for UE behavior is contained in the overlaid OFDM sequence of LP-WUS.

R1-2410358	Discussion on LP-WUS procedures in Connected mode	TCL

Observation 1: The minimum time gap between LP-WUS reception and PDCCH monitoring includes LP-WUS processing time and the MR transition time. The duration used for synchronization by MR does not need to be included in the minimum time gap. Observation 2: When CA is configured for the UE, the LP-WUR may not know the CC which the gNB uses for LP-SS and LP-WUS transmissions. This uncertainty can increase power consumption and complicate the RF filter architecture. Proposal 1: 2 candidate values, i.e., X=2, can be the baseline. Proposal 2: Based on the reported minimum time gap or preference offset, gNB can configure a time offset between LP-WUS reception and PDCCH monitoring. Proposal 3: For option 1-2, the existing timer duration can be used for the additional active time triggered by LP-WUS. Proposal 4: For option 1-2, the existing periodical drx-onDurationTimer can be disabled. Proposal 5: It is preferred that not support simultaneously configuring Option 1-1 and Option 1-2 Proposal 6: The following fallback mechanism should be supported: - Deactivation of LP-WUS monitoring and resume PDCCH monitoring by timers. - Deactivation of LP-WUS monitoring and resume PDCCH monitoring by UL signaling including SR, BSR, PRACH, CG PUSCH. Proposal 7: In RRC connected state, the gNB can explicitly configure the CC for the UE where the gNB transmits the LP-SS and LP-WUS when CA is enabled. Proposal 8: LP-WUS can be configured together with Cell DTX. Proposal 9: When Cell DTX is configured and activated, UE monitors LP-WUS during the active time of cell DTX (i.e., celldtxdrx-onDurationTimer) and does not monitor LP-WUS during the non-active time of Cell DTX.

R1-2410363	Discussion on LP-WUS operation in CONNECTED modes	Lenovo

Proposal 1: Option 1-1 and Option 1-2 can be specified with separate frameworks/configurations. If both frameworks are configured for a UE, the UE operates both simultaneously. Proposal 2: The reported time gap value includes LP-WUS processing time, MR transition time for ramp up, and the duration for time/frequency synchronization of MR. Proposal 3: UE can further indicate its preference configured time gap in addition to the minimal time gap. Proposal 4: UE shall go to the corresponding sleep state (e.g., deep sleep or light sleep) based on the configured time gap and/or synchronization performance. Proposal 5: One or more periodical LP-WUS occasions are configured in WUS monitoring window before drx-onDurationTimer to trigger the PDCCH monitoring at least for Option 1-1. Proposal 6: For CONNECTED mode, LP-WUS monitoring can be activated/deactivated by timer (especially for switching between LP-WUS monitoring and MR PDCCH monitoring after activation). Proposal 7: When channel/beam quality is below a certain value for a certain time in LR, UE is expected to fallback to trigger PDCCH monitoring in MR. Proposal 8: For LP-WUS operation option 1-1 and option 1-2, UE fallbacks to legacy C-DRX behavior. Proposal 9: gNB configures the cell lists/carrier lists for UE PDCCH monitoring if MR is configured with CA. Proposal 10: For RRC CONNECTED mode, maximum number of LP-WUS information bits is up to X=16 bits.

R1-2410399	Discussion on LP-WUS operation in CONNECTED mode	NTT DOCOMO, INC.

Proposal 1: Support Option 1-1 and Option 1-2 as separate UE capabilities Proposal 2: On whether/how to support both Option 1-1 and Option 1-2 simultaneously configured for the same UE, discuss overlap handling between Option 1-1 LP-WUS/PDCCH MOs and Option 1-2 LP-WUS/PDCCH MOs Proposal 3: UE can be configured with a parameter to enable/disable periodic CSI/L1-RSRP reporting, respectively: - When the parameter is NOT configured: - If the UE is not indicated to wake-up by LP-WUS, the periodic CSI/L1-RSRP is not reported. - If the UE is indicated to wake-up by LP-WUS, the periodic CSI/L1-RSRP is reported during the DRX active time. - When the parameter is configured: - If the UE is not indicated to wake-up by LP-WUS, the periodic CSI/L1-RSRP is reported during the time given by the configured drx-onDurationTimer, regardless if UE is indicated to wake-up or not. - If the UE is indicated to wake-up by LP-WUS, the periodic CSI/L1-RSRP is reported during the DRX active time. Proposal 4: For RRC CONNECTED mode, the minimum time gap between LP-WUS reception and MR to start PDCCH monitoring does not include the duration for time/frequency synchronization of MR Proposal 5: Define LP-WUS monitoring window which includes one or multiple LP-WUS MOs with a periodicity and offset in RRC CONNECTED mode - Option 1-1: Periodicity is same as long C-DRX cycle. Offset is configured from the beginning of drx-onDurationTimer - Option 1-2: Periodicity is configured to be no larger than long C-DRX cycle. FFS Offset Proposal 6: Consider TCI framework for CORESET as baseline for LP-WUS: RRC configured K TCI states and MAC-CE activates one of them - FFS value range of K - FFS timeline to apply the activated TCI state Proposal 7: Regarding the LP-WUS information to trigger PDCCH monitoring of RRC connected UEs (for non-CA case), support Option 3: A codepoint value corresponding to one or more UEs - FFS details for extension of Option 3 when UE is configured with CA Proposal 8: When UE is configured with CA in RRC CONNECTED mode, - LP-WUS indication is applicable to all activated serving cells in the same DRX group, if configured - For dual DRX groups, further consider following options - Option 1: LP-WUS monitoring/indication is applied per DRX group - Option 2: LP-WUS monitoring on one of DRX group, LP-WUS indication to either or both DRX group(s) Proposal 9: support UE autonomous fallback to PDCCH monitoring when UE monitors LP-WUS - FFS the condition for the fallback - For LP-WUS operation Option 1-1, UE monitors PDCCH during drx-onDurationTimer - For LP-WUS operation Option 1-2, FFS whether UE monitors PDCCH during drx-onDurationTimer or new timer Proposal 10: For the purpose of future discussions, RAN1 assumes at least the following - For RRC connected mode, - LP-WUR is NOT able to receive signal(s) during the time where MR transmits/receives signals/channels. - FFS: additional consideration of simultaneous operation between LP-WUR and MR Proposal 11: For RRC CONNECTED mode, LP-WUS can be configured with PDCCH skipping - FFS: PDCCH skipping can indicate to stop PDCCH monitoring and start LP-WUS monitoring Proposal 12: For RRC CONNECTED mode, LP-WUS can be configured with SSSG switching - No interaction between SSSG switching and LP-WUS monitoring is assumed Proposal 13: For RRC CONNECTED mode, LP-WUS can be configured with Cell DTX - During Cell DTX inactive time, the UE is not expected to monitor LP-WUS - FFS: whether/how to handle the case when all LP-WUS MOs are configured within cell DTX inactive time

R1-2410421	Discussion on LP-WUS operation in CONNECTED modes	Sharp

Proposal 1: Not support simultaneous configuration with both Option 1-1 and Option 1-2 for the same UE. Proposal 2: It is not necessary to include the dormancy indication in LP-WUS for connected UEs. Proposal 3: Support providing LP-WUS only on the PCell to carry indications for all SCells in CA cases. Proposal 4: Support Option 1, which includes indications for one or more serving cells separately within an LP-WUS. Proposal 5: support bitmap with each bit corresponding to one UE for UE in connected mode for non-CA case.

R1-2410446	LP-WUS operation in CONNECTED mode	Ericsson

Observation 1 OOK-based LP-WUS (with ≤2 slots duration) is less reliable than PDCCH for following cases - PDCCH 1Rx reception for PDCCH aggregation levels 8,16 - PDCCH 2Rx reception for PDCCH aggregation levels 4,8,16 - PDCCH 4Rx reception for PDCCH aggregation levels 2,4,8,16 Observation 2 There is no issue for option 1-1 and option 1-2 on coexistence with existing power saving techniques like PDCCH skipping and SSSG switching. Proposal 1 Considering the need to minimize overhead and latency, LP-WUS duration used for connected mode should be limited to at most 2 slots. Proposal 2 OOK-based LP-WUS in connected mode should only be used for triggering PDCCH monitoring for a limited set of PDCCH CCE aggregation levels. For example, - PDCCH aggregation levels 1,2,4 for PDCCH 1Rx reception - PDCCH aggregation levels 1,2 for PDCCH 2Rx reception - PDCCH aggregation level 1 for PDCCH 4Rx reception Proposal 3 UE PDCCH monitoring of at least certain configured PDCCH search space set(s) with higher ALs (e.g., used for scheduling RRC reconfiguration) should be independent of LP-WUS detection, Proposal 4 Only duty-cycled LP-WUS operation is supported in Rel-19. Proposal 5 Do not support both Option 1-1 and Option 1-2 simultaneously configured for the same UE. Proposal 6 For option 1-2, when the legacy DRX On duration timer is running, the UE should be able to monitor LP-WUS and trigger any associated PDCCH monitoring. Proposal 7 For Option 1-1 and Option 1-2, a time window could be configured which can contain multiple LP-WUS monitoring occasions. Proposal 8 For Option 1-1, the periodicity of LP-WUS monitoring is same as the C-DRX cycle length. Proposal 9 For Option 1-2, the periodicity of LP-WUS monitoring is separately configured (e.g. can be different from the C-DRX cycle length). Proposal 10 The number of candidate values for minimum time gap reporting is no larger than 2. Proposal 11 The reported minimum time gap denotes the gap between LP-WUS reception and start of PDCCH monitoring at the UE i.e. there is no need to explicitly (or separately) consider duration for time/frequency synchronization of MR. Proposal 12 For Option 1-1 and Option 1-2, wus-offset is defined as follows: - wus-offset for Option 1-1: an offset indicating a start of a time window where the UE starts monitoring LP-WUS, prior to a slot where the drx-onDurationTimer is to be started. - wus-offset for Option 1-2: an offset indicating a time where the UE starts monitoring PDCCH, after a slot where the UE detects LP-WUS. Proposal 13 For option 1-2, a single offset (between LP-WUS and start of the associated timer) is configured for a UE at a given time. Proposal 14 Do not support UE autonomous switching between LP-WUS monitoring and no LP-WUS monitoring without gNB knowledge. Proposal 15 With CA (single DRX group), LP-WUS can be configured on the primary cell of the cell group and the detection of LP-WUS will trigger PDCCH monitoring on all the serving cells. Proposal 16 LP-WUS should share the same QCL source with DM-RS antenna port associated with PDCCH receptions in the CORESET. The QCL source for LP-WUS is same as that given by the TCI state of the CORESET where the PDCCH is monitored after receiving LP-WUS.

R1-2410488	LP-WUR operation in connected mode	Qualcomm Incorporated

Proposal 1: - For option 1-1 of LP-WUS CONNECTED mode operation, decide either of the following approaches. - Approach 1: LP-WUS occasions, including periodicity and offset, are configured independently from the C-DRX periodicity/offset by new RRC parameter(s), such as “LPWUS-SlotPeriodicityAndOffset” and “LPWUS-SymbolWithinSlot”. UE selectively monitors LP-WUS in the LP-WUS occasions identified by other parameters such as “LPWUS-Offset” prior to the start of drx-onDurationTimer. - Approach 2: LP-WUS occasions, including periodicity and offset, are identified based on the slot that drx-onDurationTimer would start and a new RRC parameter indicating the time offset such as “LPWUS-Offset” until the slot that drx-onDurationTimer would start. - Discuss the need of RRC parameter “LPWUS-duration” that indicates the duration where the UE monitors LP-WUS consecutively in time - the consecutive LP-WUS occasions in time identified by “LPWUS-duration” are associated with a same C-DRX cycle; - LP-WUS occasions in time identified by “LPWUS-duration” provides consistent indication for the same C-DRX cycle of the UE, i.e., UE does not expect to detect more than one LP-WUSs with different indications for the UE in the LP-WUS occasions associated with the same C-DRX cycle. Proposal 2: - For option 1-2 of LP-WUS CONNECTED mode operation, decide either of the following approaches. - Approach 1: LP-WUS occasions, including periodicity and offset, are configured independently from the C-DRX periodicity/offset by new RRC parameter(s), such as “LPWUS-SlotPeriodicityAndOffset” and “LPWUS-SymbolWithinSlot”. UE monitors LP-WUS in the LP-WUS occasions and starts a new timer for PDCCH monitoring triggered by LP-WUS, such as “drx-onDurationTimer-LPWUS” after a configured time offset, such as “LPWUS-Offset”. - Approach 2: Periodicity/offset of the slot that the new timer for PDCCH monitoring triggered by LP-WUS are configured by RRC parameters such as “drx-CycleStartOffset-LPWUS” and “drx-SlotOffset-LPWUS”. LP-WUS occasions, including periodicity and offset, are identified based on the slot that drx-onDurationTimer-LPWUS would start and a new RRC parameter indicating the time offset such as “LPWUS-Offset” until the slot that drx-onDurationTimer-LPWUS would start. - Discuss the need of “LPWUS-duration” that indicates the duration where the UE monitors LP-WUS consecutively in time - the consecutive LP-WUS occasions in time identified by “LPWUS-duration” are associated with the same slot that the timer drx-onDurationTimer-LPWUS would start; - LP-WUS occasions in time identified by “LPWUS-duration” provides consistent indication for the same slot that the timer drx-onDurationTimer-LPWUS would start, i.e., UE does not expect to detect more than one LP-WUSs with different indications for the UE in the LP-WUS occasions associated with the same slot that the timer drx-onDurationTimer-LPWUS would start. Proposal 3: - At least support following: - UE does not monitor/receive LP-WUS in C-DRX active time. Proposal 4: - Confirm that a UE can trigger recovery from beam failure or radio link failure even when the UE monitors LP-WUS and does not monitor PDCCH - Once the UE triggers the recovery, the UE autonomously activate PDCCH monitoring to receive the response to the recovery request - Additionally, allow a UE to autonomously fallback from LP-WUS monitoring to C-DRX operation when the channel/beam quality is not satisfactory for successful LP-WUS reception - FFS: whether/how to support UE autonomous fallback from LP-WUS monitoring to Rel-16 DCP based DRX adaptation Observation 1: - For Option 1-1, autonomous fallback from LP-WUS monitoring to PDCCH monitoring in C-DRX active time can be enabled without specific handling - It maybe beneficial to consider UE feedback information to let network know whether the UE is in LP-WUS coverage or not, and with which LP-WUS configuration the UE can receive LP-WUS in the present status - For Option 1-2, autonomous fallback from LP-WUS monitoring to PDCCH monitoring in C-DRX active time may need specific handling - The handling can be e.g., based on network configuration, or based on a new L1/L2 dynamic report of UE status (e.g., indicating whether the UE monitors LP-WUS or the UE fallback to PDCCH monitoring based on C-DRX configuration) Proposal 5: - Introduce MAC-CE for LP-WUS activation/deactivation - A UE can be configured with one or multiple LP-WUS configurations - The MAC-CE activates one (or none) of the LP-WUS configurations dynamically Proposal 6: - On the serving cell that the UE monitors LP-WUS in connected mode, - UE can be configured with LP-WUS for Option 1-1 and Option 1-2 simultaneously - Either LP-WUS Option 1-1 or Option 1-2 (or none of them) is active - The activation/deactivation of LP-WUS Option 1-1 and Option 1-2 is done by MAC-CE Proposal 7: - Support at least “10ms” as one of the X candidate values that a UE can report in the UE capability for all the SCSs. - FFS other value(s) smaller than 10ms Proposal 8: - Support TCI-state in the RRC configuration for LP-WUS - The TCI-state in the LP-WUS configuration indicates one or two DL RSs with the corresponding QCL types. - Support unified TCI framework for LP-WUS - If a UE is configured with dl-OrJointTCI-StateList, the UE obtains the QCL assumptions from a TCI state within the dl-OrJointTCI-StateList - The TCI state from which the UE obtains the QCL assumptions for LP-WUS is the one activated by MAC-CE activation command and is indicated by the codepoint of the DCI field “Transmission Configuration Indication” Proposal 9: - QCL Type(s) defined for CORESET/PDCCH is re-used for LP-WUS Proposal 10: - As baseline, LP-WUS on a serving cell is configured within active DL BWP of the serving cell - FFS: outside active DL BWP of the serving cell (as optional UE capability) Proposal 11: - For a UE configured with DC, support LP-WUS monitoring on a serving cell per cell-group - For a UE configured with CA without dual-DRX groups, support LP-WUS monitoring on a serving cell that triggers PDCCH monitoring on all the serving cells - For a UE configured with CA with dual-DRX groups, support LP-WUS monitoring on a serving cell per DRX group - Opt.1: LP-WUS is configured to be monitored per DRX group - For dual DRX groups, LP-WUS monitoring is configured per DRX group - LP-WUS monitored on a cell within a DRX group triggers PDCCH monitoring on the cell of the DRX group - Opt.2: LP-WUS carries an indication for which DRX group(s) it triggers PDCCH monitoring

R1-2410512	LP-WUS operation in CONNECTED modes	MediaTek Inc.

Observation 1: Option 1-1 is suitable for predictable, periodic traffic like VoIP, while Option 1-2 is designed for unpredictable or misaligned periodic traffic such as XR. Proposal 1: Support UE can be only configured either Option 1-1or Option 1-2 for PDCCH monitoring in RRC CONNECTED mode to accommodate different traffic types. Observation 2: Reporting one value per SCS is the same as the design of R16 DCI format 2-6. Proposal 2: For the reporting value, consider reusing the minimum time gap values of DCI format 2-6 from 38.213 Table 10.3-1. Observation 3: Support of CA and NR/DC can provide power saving benefits for UE to turn part of RF components off. Proposal 3: Using a bitmap to indicate the wake-up cell/cell groups in CA and NR/DC cases is beneficial for UE turning part of RF components off.

RAN1 agreements

RAN1#116

Agreement

- For RRC CONNECTED mode, from RAN1 perspective, further study following LP-WUS procedures to trigger PDCCH monitoring:

  - Case 1: PDCCH monitoring is triggered by LP-WUS with C-DRX configuration

    - Option 1-1: LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer.

      - This option may replace DCP functionality

    - Option 1-2: LP-WUS monitoring outside C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring.

      - PDCCH monitoring possibly irrespective of drx-onDurationTimer

    - Option 1-3: LP-WUS monitoring inside C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring.

  - Case 2: PDCCH monitoring is triggered by LP-WUS without C-DRX configuration. LP-WUS can be monitored at any time according to the LP-WUS monitoring configuration

    - FFS duty-cycled and/or continuous LP-WUS monitoring

- Combination of options in Case 1 and combination of options in Case 1 and Case 2 are not precluded.

Agreement

For RRC CONNECTED mode, maximum number of LP-WUS information bits is up to X bits

- FFS value X, which is no more than [8 or 16]

Agreement

For RRC CONNECTED mode, minimum time gap between LP-WUS reception and MR to start PDCCH monitoring is introduced considering at least following

- LP-WUS processing time

- MR transition time for ramp up

- Time/frequency synchronization of MR

- FFS whether UE can report supported minimum time gap from candidate values

FFS: Whether the minimum time gap values can be more than one

Agreement

For RRC CONNECTED mode, from RAN1 perspective,

- PDCCH monitoring triggered by LP-WUS is enabled/disabled by gNB RRC signaling

  - FFS whether to support UE assistance.

- LP-WUS monitoring by UE is known to gNB.

  - FFS whether implicit/explicit indication from UE is necessary

- In case LP-WUS monitoring is enabled, following options are further studied

  - Option 1: No additional indication/condition are introduced for activation/deactivation of LP-WUS monitoring

  - Option 2: Activation/deactivation of LP-WUS monitoring by gNB L1/L2 signaling with or without UE assistance.

  - Option 3: Activation/deactivation of LP-WUS monitoring based on condition(s), such as timer.

  - Option 4: Activation/deactivation of LP-WUS monitoring based on implicit indication/condition, e.g. UL transmission.

RAN1#116bis

Agreement

Update the following agreement in RAN1#116 in red:

Agreement

- For RRC CONNECTED mode, from RAN1 perspective, further study following LP-WUS procedures to trigger PDCCH monitoring:

  - Case 1: PDCCH monitoring is triggered by LP-WUS with C-DRX configuration

    - Option 1-1: LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer.

      - This option may replace DCP functionality

    - Option 1-2: LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring.

      - PDCCH monitoring possibly irrespective of drx-onDurationTimer

        - Option 1-2-1: PDCCH monitoring may be additionally triggered based on legacy C-DRX cycle and drx-onDurationTimer when monitoring LP-WUS

          - If this is adopted, it should be configured together with Option 1-1 to achieve power saving gain compared to legacy C-DRX

        - Option 1-2-2: PDCCH monitoring is not triggered by legacy C-DRX cycle and drx-onDurationTimer when monitoring LP-WUS

    - Option 1-3: LP-WUS monitoring inside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring.

  - Case 2: PDCCH monitoring is triggered by LP-WUS without C-DRX configuration. LP-WUS can be monitored at any time according to the LP-WUS monitoring configuration

    - FFS duty-cycled and/or continuous LP-WUS monitoring

- Combination of options in Case 1 and combination of options in Case 1 and Case 2 are not precluded should be considered.

- RAN1 does not discuss C-DRX related timers other than drx-onDurationTimer, this topic is up to RAN2

- Note: Above does not preclude to support fallback mechanism to trigger PDCCH monitoring, if any

RAN1#117

Agreement

For RRC CONNECTED mode, support UE capability report for determination of minimum time gap between LP-WUS reception and MR to start PDCCH monitoring.

- FFS: exact value(s) of the minimum time gap

- FFS: support of multiple minimum time gaps

- FFS whether the reported value includes the duration for time/frequency synchronization of MR

Agreement

For LP-WUS monitoring in RRC CONNECTED mode, a LP-WUS is QCLed with existing NR signal/channel/CORESET for the TCI state

- FFS which existing NR signal/channel/CORESET is the QCL source of LP-WUS

- FFS exact definition of QCL relationship between LP-WUS and existing NR signal/channel/CORESET

Agreement

LP-WUS monitoring occasions (MOs) are configured by RRC, where UE can monitor for LP-WUS transmission in RRC CONNECTED mode.

- FFS whether to define a time window for MOs

- It is at least supported that a UE can monitor MOs with a periodicity.

  - FFS details of the periodicity, e.g. derived from DRX cycle, separately configured

Agreement

- Study whether/how LP-WUS works when UE is configured with CA in RRC CONNECTED mode

FFS: The cell(s) where PDCCH monitoring triggered by a LP-WUS is applicable

Option 1: one or more serving cells based on gNB indication/configuration

Option 2: all activated serving cells

Note: other options are not precluded

Agreement

For RRC CONNECTED mode, LP-WUS can be configured without following existing features.

Rel-16 DCP

Rel-17 PDCCH skipping

Rel-17 SSSG switching

Rel-18 cell DTX

Further study whether/how LP-WUS works with following existing features (PDCCH skipping, SSSG switching, cell DTX)

RAN1#118

Agreement

- For option 1-2 of LP-WUS CONNECTED mode operation, the followings are assumed from RAN1 perspective.

LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring.

UE is configured with legacy C-DRX configurations as Rel-18

UE is expected to be configured with LP-WUS monitoring configuration (periodicity and offset can be different from those from C-DRX configuration)

    - FFS potential restriction for LP-WUS configuration in relation with C-DRX configuration

LP-WUS triggers the start of a timer during which UE monitors PDCCH

    - FFS the timer is existing timer and/or new timer

UE PDCCH monitoring behaviors related to other legacy DRX timers are not affected

    - drx-InactivityTimer, drx-RetransmissionTimerDL, drx-RetransmissionTimerUL, drx-HARQ-RTT-TimerDL, drx-HARQ-RTT-TimerUL

No impact on RRM/RLM/BFD measurement requirements is assumed

For periodic CSI/L1-RSRP reporting, UE can be configured with one of the following (same as Rel-16 DCP and option 1-1)

    - Periodic CSI/L1-RSRP is not reported if UE is not indicated to wake-up

    - Periodic CSI/L1-RSRP is periodically reported regardless if UE is indicated to wake-up or not

UE PDCCH monitoring is not triggered by legacy C-DRX cycle and drx-onDurationTimer when monitoring LP-WUS

Working assumption

From RAN1 perspective, for RRC CONNECTED mode, PDCCH monitoring is triggered by LP-WUS with C-DRX configuration

Support Option 1-1: LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer.

Support Option 1-2: LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring.

FFS whether/how to support both Option 1-1 and Option 1-2 simultaneously configured for the same UE

Note: Above can be revisited considering RAN2 decisions.

Agreement

Select one of the following alternatives in RAN1#118bis

Alt 1

For RRC CONNECTED mode, UE reports one value for each SCS from X candidate values for the determination of the minimum time gap between LP-WUS reception and MR to start PDCCH monitoring via UE capability reporting.

FFS: X

FFS: definition of reported value

Alt2

For RRC CONNECTED mode, UE reports multiple values for each SCS from X candidate values for the determination of the minimum time gap between LP-WUS reception and MR to start PDCCH monitoring via UE capability reporting.

FFS: X

FFS: definition of reported value

Different minimum time gaps correspond to different sleep states

Companies are encouraged to details on how the reported values are to be used by the network

Agreement

LP-WUS is at least supported for the case where a UE is configured with CA in RRC CONNECTED mode

- FFS: DC

Agreement

- For LP-WUS monitoring in RRC CONNECTED mode, SSB and/or CSI-RS can be the QCL source of LP-WUS

FFS applicable QCL type(s)

RAN1#118bis

Agreement

- For Option 1-1 of LP-WUS CONNECTED mode operation, the followings are assumed from RAN1 perspective.

LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer

UE is configured with legacy C-DRX configurations as Rel-18

UE behaviors related to CDRX active time triggered by all legacy DRX timers are not affected unless included in this proposal

No impact on RRM/RLM/BFD measurement requirements is assumed

For periodic CSI/L1-RSRP reporting, UE can be configured with one of the following (same as Rel-16 DCP)

    - Periodic CSI/L1-RSRP is not reported during the time given by the configured drx-onDurationTimer if UE is not indicated to wake-up

    - Periodic CSI/L1-RSRP is periodically reported during the time given by the configured drx-onDurationTimer regardless if UE is indicated to wake-up or not

      - Note: Periodic CSI/L1-RSRP is reported during CDRX active time as in legacy specification

Agreement

Confirm following working assumption

Working Assumption

From RAN1 perspective, for RRC CONNECTED mode, PDCCH monitoring is triggered by LP-WUS with C-DRX configuration

Support Option 1-1: LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer.

Support Option 1-2: LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring.

FFS whether/how to support both Option 1-1 and Option 1-2 simultaneously configured for the same UE

Note: Above can be revisited considering RAN2 decisions.

Agreement

For RRC CONNECTED mode, UE reports one value for each SCS from X candidate values for the determination of the minimum time gap between LP-WUS reception and MR to start PDCCH monitoring via UE capability reporting.

FFS: X

FFS: definition of reported candidate value

UE can indicate its preference for configured time offset between LP-WUS and PDCCH reception using UAI mechanism. FFS details. UAI is optional.

FFS: A single time offset is configured for a UE at a given time

Agreement

LP-WUS is supported when UE is configured with NR-DC in RRC CONNECTED mode

  - Above is supported for the case PDCCH monitoring is triggered by LP-WUS in the same cell group

  - FFS: The cell(s) where PDCCH monitoring triggered by a LP-WUS is applicable

Agreement

- For LP-WUS CONNECTED mode operation, Option 1-2 can be configured separately from Option 1-1

  - FFS whether/how to support both Option 1-1 and Option 1-2 simultaneously configured for the same UE

Agreement

- Regarding the potential restriction for LP-WUS configuration in relation with C-DRX configuration for option 1-2 of LP-WUS CONNECTED mode operation, from RAN1 perspective,

  - Opt2: At least LP-WUS periodicity is no larger than long C-DRX cycle, FFS other restrictions

  - Introducing monitoring window is not precluded