3GPP TSG-RAN WG1 #72bis      	                                R1-131395

April 15 – 19, 2013

Chicago, IL, USA

Agenda item:    7.2.3.2

Source:             Qualcomm Incorporated

Title:                  Signaling Mechanisms for Reconfiguration

Document for:  Discussion/Decision

- Introduction

In RAN#58, a new work item of LTE_TDD_eIMTA was approved [1].  The objective of this work item is to enable TDD UL-DL reconfiguration for traffic adaptation in small cells, including:

- Agree on the supported time scale together with the necessary signaling mechanism(s) for TDD UL-DL reconfiguration and specify the necessary (if any) enhancements for TDD UL-DL reconfiguration with the agreed time scale and signaling mechanism(s), e.g.

  - HARQ/scheduling timeline,

  - RLM/RRM measurements,

  - CSI reporting;

A number of signalling options have been discussed during the SI phase of the IMTA study, including SIB, paging, RRC, MAC and L1 signalling.  Extensive evaluation results have shown that generally faster reconfiguration provides more gains compared to slower ones.  On the other hand, due to inherent transition issues, implementation of faster reconfiguration increases complexity as well as loss due to boundary effects [2].

In this contribution, we discuss the HARQ timeline for TDD UL-DL reconfiguration and the corresponding signalling mechanisms.

- HARQ Timeline for UL-DL reconfiguration

For UL-DL reconfiguration, the Rel-8 HARQ timing is not applicable since transmission direction of some subframes is dynamically changed. In general, there are two solutions: one is dynamically changing HARQ timing based on the latest reconfigured TDD UL/DL allocation; another is to use a fixed reference configuration for HARQ ACK/NACK and scheduling timing, regardless of how TDD UL/DL configuration changes.

  - HARQ timing dynamically changed with UL-DL reconfiguration

For this HARQ timing solution, every time UE receives a new TDD UL-DL configuration, the UE also obtains a new HARQ timing which can be either derived from the latest TDD UL/DL allocation or as the function of currently configured configuration and the upcoming configuration. The principle is similar to different reference configurations used in TDD inter-band carrier aggregation.

The main issue with this method is the potential misalignment of TDD UL-DL configuration between eNB and UE when the reconfiguration signalling is false alarmed or missed. The misunderstanding could occur frequently if reconfiguration is on the time scale of 10 ms via L1 signalling. Another disadvantage for this method is higher implementation complexity to dynamically adjust HARQ timing on a frame basis. Therefore, the dynamic adjusted HARQ timing based on the latest configuration may not be appropriate for TDD UL-DL reconfiguration in order of 10 ms.

  - HARQ timing based on the fixed reference configuration

In this HARQ timing implementation, a fixed UL-DL reference configuration is defined for TDD UL-DL reconfiguration. For example, DL HARQ operation can be based on UL-DL subframe configuration #5, regardless of the actual UL-DL subframe configuration in use in a frame or half a frame. That is to say, if dynamic UL-DL reconfiguration is enabled, the DL HARQ timing can be always based on the 9:1 UL-DL subframe configuration. At the same time, UL HARQ operation can be based on UL-DL subframe configuration #0, regardless of the actual UL-DL subframe configuration in use in a frame or half a frame. That is, if dynamic UL-DL reconfiguration is enabled, the UL HARQ timing can be always based on the 4:6 UL-DL subframe configuration.

Figure 1	 Example of HARQ and scheduling timing based on fixed reference configuration #5 and #0 for DL and UL separately.

The fixed reference configuration based HARQ timing is simple and provides no or very limited additional overhead since only RRC configuration is needed to activate dynamic UL-DL reconfiguration. Nevertheless, employing fixed extremely UL-heavy or DL-heavy configuration timeline as reference also imposes constraint on performance of flexible traffic adaptation. The 9:1 configuration for DL HARQ timing requires ACK/NAK feedback for 9 DL subframes in one UL subframe, causing large UL control overhead in that subframe, and may also limit the coverage range. Although bundling can be configured, the bundling is associated with some DL throughput loss. Furthermore, ACK/NAK feedback for 9 DL subframes in one UL subframe also requires large PUCCH resource in subframe #2, and will decrease UL throughput. Similarly, the 4:6 configurations for UL HARQ timing requires double PHICH resource reservation and the need to schedule two UL subframes in one DL subframe. As a result, the 2-bit UL scheduling index has to be in UL grants, instead of DAI (downlink assignment index). The lack of DAI may lead to inefficient ACK/NAK feedback on PUSCH, since DAI indicates the total number of DL assignments which helps the UE detect missing DL grants.

  - HARQ timing based on the semi-static reference configuration

Another alternative is to use semi-static configured reference configuration for HARQ timing as proposed in [4]. The eNB will configure two reference configurations for UL and DL HARQ separately. This is similar to the fixed method except now the reference configuration is configured by eNB. An example of HARQ timing based on reference configuration of 2 and 0 for DL and UL is shown in Figure 3.

Figure 3	 HARQ timing based on semi-static reference configuration of 2 and 0 for DL and UL

The main advantages of HARQ timing based on semi-static reference configurations are

- Provide a simple HARQ timing solution to avoid potential misunderstanding between eNB and UE when UL-DL configuration changes

- Provide the flexibility for eNB to control HARQ timing and scheduling grant to avoid strong inter-cell interference and meanwhile improve operation efficiency

- Signaling Design

There is a need for additional signaling to indicate the transmission direction of the non-anchor subframes. Explicit signaling is preferred since UE can be unambiguously notified in advance which subframes will be DL in order to turn off TX and save the power consumption. In this section we discuss different aspects of signaling mechanisms to support efficient eIMTA operation.

  - RRC vs. L1 signaling

By using dedicated RRC signalling, the eNB can indicate the TDD UL-DL configuration to the RRC connected UEs. The corresponding time scale supported by this method is on the order of 200ms. The main advantage of this approach is signalling reliability due to the use of RLC-ARQ mechanism. To support reconfiguration time scale on the order of one or several radio frames L1 signalling shall be used. This approach provides the best traffic adaptation performance over the others, while inter-cell interference management may not perform well with 10ms UL-DL reconfiguration due to fast variation of interference. However, L1 signalling for UL-DL reconfiguration does not necessarily mean dynamic reconfiguration, which depends on implementation. Evaluation results in [3] show that even with fast reconfiguration rate of 10ms, more than 80% of reconfiguration interval is no less than 100ms, e.g. period of system staying in one configuration is typically longer than 100ms. The approach of L1 signalling for reconfiguration provides the capability to track the traffic fluctuation better.

  - Dedicated vs. common signaling

There are several alternatives to implement L1 signalling for UL-DL reconfiguration, such as a common L1 signalling with high reliability to carrier reconfiguration information for Rel-12 UEs. To reduce the overhead, the common L1 signalling can be sent only on the predefined subframes. The problem with this approach is that due to no feedback eNB cannot know whether UE has missed detection or wrong information due to false alarm. Considering a small cell with 5 active UEs, 50% of subframes being non-anchor, the number of missed PDSCH transmissions in one hour can be estimated as 10-3×3.6×106×5×0.5=9000 when missed detection rate for L1 signalling is assumed to be 1e-3. Furthermore, if ACK bundling is used for PDSCH HARQ feedback all DL transmission will be failed as long as there is one DL subframe missed by UE. To improve the reliability, the error rate for common L1 signalling shall be at a level of 1e-5 or 1e-6, which will increase the signalling overhead. Another disadvantage for the common signalling is that all UEs need to monitor and decode the signalling every radio frame to get the latest UL-DL configuration, which is not possible for UE in CDRX mode. In such cases, there is misalignment of the UL-DL configuration between eNB and UE and when UE wakes up it might miss the PDCCH grant in the non-anchor subframe resulting in DL throughput loss.

Another alternative is to use dedicated signalling to transmit the UL-DL configuration to specific UE with traffic to be served in the non-anchor subframes. PDCCH and EPDCCH can be reused in such case. With this mechanism, the spec impact can be very limited with small signalling overhead. Compared to new L1 common control channel this approach has no impact on the legacy UE and can support reconfiguration on radio frame level. Moreover, considering typical eIMTA scenario where only limited number of UEs have traffic to be served, dedicated signalling fits well in such scenario compared to common signalling in terms of overhead and flexibility.

- Conclusions

In this contribution, we discussed HARQ timing and signaling aspects of flexible UL/DL subframe reconfiguration to facilitate efficient eIMTA, and proposed:

- HARQ timing is based on semi-static reference configuration and independent of how TDD UL/DL configuration changes.

- Explicit L1 signaling is used to indicate the transmission direction of the non-anchor subframes of the next radio frame.

References

[1] RP-121722, “New work item proposal for Further Enhancements to LTE TDD for DL-UL Interference Management and Traffic Adaptation”, CATT.

[2] R1-122807, “Methods to Support Different Time Scales for Reconfiguration,” Qualcomm.

[3] R1-130586, “DL-UL Interference mitigation schemes for LTE TDD eIMTA,” Qualcomm.

[4] R1-130130, “HARQ timing in TDD eIMTA,” ZTE.