3GPP TSG RAN WG1 Meeting #72BIS                                R1-131507

Chicago, USA, 15th April – 19th April, 2013

Agenda Item:	7.2.4.4

Source:	CATR

Title:	Simulation results for PUSCH and PDSCH coverage enhancement

Document for:	Discussion

# Introduction

Coverage improvement techniques for low cost MTC have been discussed in RAN1 #72. Simulation assumptions for data channel PUSCH and PDSCH are agreed[1]. According to these simulation assumptions, in this contribution, we provide some simulation results for PUSCH and PDSCH.

# Simulation results

In this section, we will focus on the repetition techniques for PDSCH and PUSCH. And give the simulation results to show how many retransmission times are required to meet the iBLER target under the low SINR constraint.

Figure 1 gives the simulation results for PDSCH when the SINR is set to -19.3dB. The simulation assumptions are listed in table 1 in appendix. We choose realistic 1 subframe channel estimation. Simulation results show that to reach 10% BLER for PDSCH, over 40 times repetition is required.

Figure 1 Repetition times vs. BLER with minimum required SINR -19.3dB for PDSCH.

Figure 2 gives the simulation results for PUSCH when the SINR is set to -24.3dB. The simulation assumptions are listed in table 2. Realistic 1 subframe channel estimation is chosen. Simulation results show that to reach 10% BLER for PDSCH, over 90 times repetition is required.

Figure 2 Repetition times vs. BLER with minimum required SINR -19.3dB for PUSCH.

Observation: To get 10%BLER, the repetition times for PDSCH is over 40, while for PUSCH, the repetition times is over 90.

# Conclusion

In this contribution, we give the simulation results for PDSCH and PUSCH repetition.

Observation: To get 10%BLER, the repetition times for PDSCH is over 40, while for PUSCH, the repetition times is over 90.

Reference

[1] Chairman's Notes RAN1#72 – final

Appendix

Table 1 Simulation assumptions on PDSCH

| Parameter | Value |
| --- | --- |
| System bandwidth | 10MHz |
| Frame structure | FDD |
| Carrier frequency | 2.0 GHz for FDD |
| Antenna configuration | 2x2, low correlation for FDD; |
| Channel model | EPA |
| Doppler spread | 1Hz |
| MCS | 0 Note: We need to satisfy the traffic requirements agreed before |
| Number of DL RBs | 6 Note: We need to satisfy the traffic requirements agreed before |
| Transmission mode | TM2 |
| Frequency tracking error | 100Hz or 20Hz |
| Performance target | 10% iBLER |
| Channel estimation | 1subframe channel estimation |
| The minimum required SINR | -19.3dB for FDD; |
| Output | The amount of repetitions as well as other techniques to achieve performance target At the minimum required SINR |

Table 2 Simulation assumptions on PUSCH

| Parameter | Value |
| --- | --- |
| System bandwidth | 10MHz |
| Frame structure | FDD |
| Carrier frequency | 2.0 GHz for FDD |
| Antenna configuration | 1x2, low correlation for FDD; |
| Channel model | EPA |
| Doppler shift | 1Hz |
| TBS | 42 Note: We need to satisfy the traffic requirements agreed before |
| Number of UL RBs | 1 Note: We need to satisfy the traffic requirements agreed before |
| Transmission mode | TM1 |
| Frequency tracking error | 100Hz or 20Hz |
| Performance target | 10% iBLER |
| Channel estimation | Realistic 1 subframe channel estimation |
| The minimum required SINR | -24.3dB for FDD; |
| Output | The amount of repetitions at the minimum required SINR |