3GPP TSG RAN WG1 Meeting #72BIS	R1-131509

Chicago, USA, 15th April – 19th April, 2013

Agenda Item:	7.2.5.2.3

Source:	CATR

Title:	Simulation results for downlink 256QAM

Document for:	Discussion

# Introduction

In RAN#58, “New Study Item Proposal for Small Cell Enhancements for E-UTRA and E-UTRAN – Physical-layer Aspects” was approved. One object of this SI is “Introduction of a higher order modulation scheme (e.g. 256 QAM) for the downlink [1]”. In RAN1 #72, simulation assumptions were approved by email discussion [2]. In this contribution, we will provide some simulation results for 256QAM.

# Discussion

The design of 256QAM is Gray mapping. Simulation assumptions are listed in table 1 which is given in [2] as scenario #3(sparse). We set the EVM constraint as 4% and 6 coding rate is used.

Table 1 simulation assumptions on 256QAM

| Parameters | Assumption |
| --- | --- |
| Carrier frequency | 3.5 GHz |
| System bandwidth | 10 MHz |
| Distance-dependent path loss | ITU InH [referring to Table B.1.2.1-1 in TR36.814] 3D distance between an eNB and a UE is applied |
| Penetration | 0 |
| Shadowing | ITU InH [referring to Table A.2.1.1.5-1 in TR36.814] |
| Antenna Height: | 6m |
| Antenna gain + connector loss | 5dBi |
| Fast fading channel between eNB and UE | ITU InH |
| Number of UEs | 10 |
| UE dropping | Randomly and uniformly distributed over area per floor |
| Minimum distance (2D distance) | Small cell-UE: 3m |
| Traffic model | FTP model 3 |
| EVM constraint | 4% |
| 256QAM coding rate | 0.7667,0.8,0.8333,0.8667,0.9,0.93 |

Figure 1 shows the geometry of scenario 3(sparse) without EVM constraint. We can see, over 45% users’ geometry are over 20dB.

Figure 1 Geometry of scenario 3(sparse) without EVM constraint

Figure 2 gives the relative gain of 256QAM for cell average throughput and 5% cell edge user throughput. The baseline is system simulation results without 256QAM. The EVM constraint is 4%. From Figure 2, 256QAM will bring about 24% cell average throughput gain comparing with no 256QAM. The benefit of introducing 256QAM is quite small for cell edge users. It brings less than 1% throughput gain for 5% cell edge user.

Figure 2 Relative gain of 256QAM in scenario 3(sparse)

Obeservation 1: 256QAM will bring about 24% cell average throughput gain and less than 1% cell edge user throughput gain with 4% EVM constraint.

# Conclusion

In this contribution, we show the performance of 256QAM in scenario 3(sprase) and get the following observation:

Obeservation 1: 256QAM will bring about 24% cell average throughput gain and less than 1% cell edge user throughput gain with 4% EVM constraint.

Reference

- [1] RP-122032, “New Study Item Proposal for Small Cell Enhancements for E-UTRA and E-UTRAN – Physical-layer Aspects”, 3GPP RAN#58, Barcelona, Spain, Dec. 2012.

[2] R1-130856, “Evaluation assumptions for small cell enhancements-physical layer”, Huawei, HiSilicon