TSG-RAN WG1 #72bis	R1-131595

Chicago, USA, 15th – 19th April 2013

Source:	Orange

Title:	Views on network-assisted interference cancellation and suppression

Agenda Item:	7.2.8

Document for:	Discussion

# Introduction

This contribution presents our views on the most promising receiver structure candidates, as well as where/how network assistance can be useful in order to translate the link-level gains of IC receivers into system-level  gains.

# Identification of LTE receiver structures for Interference Cancellation and Suppression

The candidate LTE receiver structures for interference cancellation and suppression are described below.

## Linear MMSE-IRC

The performance requirements related to this type of receiver, referred to as Type A, are already defined in Release 11 [TR 36.829].

LMMSE-IRC receiver can efficiently suppress interference when the degrees of freedom at the receiver are sufficient, i.e., the number of receiver antennas is higher than that of the number of interfering spatial layers. In practice, the number of interfering layers is typically higher than the number of receive antennas; still, the LMMSE-IRC receiver can reduce the interference level, although not perfectly suppressing it.

This receiver needs an estimation of the covariance matrix of the interference.

Network assistance may benefit LMMSE IRC receivers, e.g. by reducing the rank of the interfering transmission on the resources allocated to the victim, in order to increase the number of degrees of freedom of the receiver to mitigate interference. Nevertheless, such assistance is not necessary for the LMMSE-IRC receiver to provide gains.

## Successive Interference Cancellation (SIC)

For the following types of IC receivers, there is no performance requirements currently defined in RAN4.

### Symbol level Interference Canceller receiver

This type of receiver relies on demodulation of the interferer’s symbols.

It is expected to be efficient when a single interferer is received with a high power. In other cases, it is unclear whether this receiver can provide sufficient gain, due to the lack of processing gain in order to estimate with good reliability the interfering signal. The applicability of this type of receivers seems therefore limited. Moreover, this type of receiver may introduce error propagation phenomenon since no CRC check is possible. The situation where the interference power is much higher than the useful signal one is therefore a favorable situation for the Symbol level SIC receiver.

Note that channel re-estimation of the useful signal can be advantageously performed after interference subtraction.

### Hard Successive Interference Canceller (SIC) receiver

The Hard Successive Interference Canceller (SIC) attempts to detect and decode one by one the interferers of interest, as depicted on Figure 1 in case of MU-MIMO and inter-cell interference cancellation.

Figure 1: Interference Cancellation principle in case of MU-MIMO and inter-cell interference

This type of receiver can take advantage of the Cyclic Redundancy Check attached to each transport block issued by the MAC layer before channel coding: if after channel decoding the CRC check is successful, the block has been correctly decoded and the interfering signal can be perfectly reconstructed (minor the channel estimation errors). After the received signal has been cleaned up from one interferer (generally the one with the highest received power), a second interferer can be processed, and so on.

This receiver requires estimating the channel of the interference and decoding its signal.

The Hard SIC to be efficient needs to find at least one interferer that can be decoded without error (in order to subtract its interference from the useful signal). As a result, the situations where the interference power is much higher than the useful signal power and/or when the interference has a robust MCS are favorable situations where the Hard SIC brings the more gains.

In case the interference and useful signal have similar power, the Hard SIC imposes the constraint that the MCS used by the first interferer be more robust than the MCS used for the signal of interest, as it will need to be decoded under the interference of the latter (UE1 in the example of Figure 1).

Note that channel re-estimation of the useful signal can be advantageously performed after interference subtraction.

### (Soft) Turbo Successive Interference Canceller (SIC) receiver

The (Soft) Turbo Successive Interference Canceller (SIC) receiver is based on the following principle, illustrated in Figure 2, assuming a single interferer and Linear MMSE IC detection for the sake of clarity:

- Soft estimation of UE1’s information bits using LMMSE detection and a soft-in soft-out channel decoder;

- Construction of soft estimates of UE1’s symbols and multiplication by the estimated channel response to obtain a soft estimate of UE1’s signal contribution.

- Subtraction of UE1’s soft signal estimation from the originally received signal;

- Do steps 1-2 for UE2 (interferer) instead of UE1, from the signal obtained in step 3;

- Subtraction of UE2’s soft signal estimation from the originally received signal;

- Soft estimation of UE1’s data, from the signal obtained in step 5;

- Repeat steps 2-6 until successful decoding of UE1’s data (e.g. checked by means of CRC), or after a given number of iterations.

Figure 2: Turbo-SIC architecture [4]

In case of intra-cell MU-MIMO and inter-cell interference, Turbo-SIC receivers need network assistance to inform the victim UE about the transmission parameters of the interferers. Further, it may benefit from assistance in the form of scheduling coordination, as described in the previous section for Hard SIC.

Note: Turbo-SIC naturally includes PIC (Parallel Interference Canceller) when a codeword experiences more than one spatial layer. A more detailed description of Turbo-PIC is provided in [3].

Note that Turbo-SIC can be coupled with turbo channel estimation, that is to say that channel re-estimation can be performed anew after each subtraction step. Data aided channel (re)-estimation (interferer and useful signal) based on the soft reliability of the decoded signal can be performed also within each iteration.

From our perspective, Turbo-SIC receivers based on LMMSE-IC detection offer a good tradeoff in terms performance vs. complexity.

An important parameter of these receivers is the number of turbo-code iterations for each detection and decoding step (see [3] for a specific optimized setting).

In order to evaluate the performance of Turbo-SIC receivers in system level simulations, an appropriate model of the link level performance is needed; such a model can be found in [4].

## Maximal Likelihood (ML)

MMSE-IRC could be enhanced to ML detection taking into account the modulation and channel of the interferer.

Similarly, the MMSE detection can be replaced in all other receivers structure described so far by a ML one.

Enhancing the LMMSE receiver to an ML one would be beneficial in terms performance, particularly, when the number of receive antennas is lower than that of the number of interfering spatial layers. However, for complexity reason degraded ML implementation often need to be considered whose gain is unclear compared to MMSE detection.

The interference parameters needed for this kind of receiver are the same than the ones associated to the symbol level Interference canceller receiver.

## Summary of the candidate receiver structures and assumptions of network assistance

The following table summarizes for each receiver structure the expected potential gains, the complexity aspects as well as the possible need for network assistance.

| IS/IC Receiver structure | Processing | Complexity | Interference type | Network assistance | Potential average user throughput gains |
| --- | --- | --- | --- | --- | --- |
| LMMSE-IRC | Estimation of the covariance matrix of the interferer’s signal | Low | Intra-cell SU-MIMO | Not needed | Baseline |
|  |  |  | Intra-cell MU-MIMO or Inter-cell | Limited or not needed |  |
| Symbol-level IC | Demodulation of the interferer’s symbols | Low/Medium | Intra-cell SU-MIMO | Not needed | Low (significant for high power interferer scenario but limited in the general case) |
|  |  |  | Intra-cell MU-MIMO or Inter-cell | Limited or not needed |  |
| Hard SIC | Estimation of the interferer’s channel and decoding of its signal | Medium Depends on the number of interferers | Intra-cell SU-MIMO | Not needed | Medium |
|  |  |  | Intra-cell MU-MIMO or Inter-cell | Needed | Medium with network assistance |
| Turbo-SIC (or Turbo PIC) | Estimation of the interferer’s channel and decoding of its signal | High Depends on the number of interferers and number of iterations Risk of delay increase in case of high number of iterations | Intra-cell SU-MIMO | Not needed | High |
|  |  |  | Intra-cell MU-MIMO or Inter-cell | Needed | High with network assistance |
| Non iterative ML | Estimation of the interferer’s channel | Intractable for certain configurations (suboptimal ML implementation needed) | Intra-cell SU-MIMO | Not needed | Low (interesting for more interference spatial layers than receive antennas) |
|  |  |  | Intra-cell MU-MIMO or Inter-cell | Limited or not needed |  |
| Iterative ML | Estimation of the interferer’s channel and decoding of its signal | Intractable for certain configurations (suboptimal ML implementation needed) | Intra-cell SU-MIMO | Not needed | Very High (but intractable) |
|  |  |  | Intra-cell MU-MIMO or Inter-cell | Needed | Very High with network assistance (but intractable) |

Table 1: Receiver structures and assumptions of network assistance

# Network assistance

This section discusses where/how network assistance can be useful.

## Interferers’ channel estimation

For all the receivers presented in section 2, except for the linear MMSE-IRC, the channel of each processed interferer has to be reliably estimated, to decode its data and to reconstruct the associated received signal with a good quality. This requires the reference signal (RS) used by the interferer to be known at the victim, and the SINR experienced on the RS to allow a sufficient channel estimation quality.

In the case of DMRS-based demodulation, orthogonal DMRS sequences are available for UEs in the same cell (for MU-MIMO) since Release 10. In addition, the possibility to allocate orthogonal DMRS sequence to UEs in neighbouring cells has been introduced in Release 11. This framework however limits the number of orthogonal sequences to two, which constrains the victim and aggressor UEs to be served with a single MIMO layer if  DMRS interference is to be avoided. Note that the lack of RS orthogonality may be compensated by more powerful signal processing, e.g. by refining the channel estimation at each iteration of the Turbo-SIC process. However, some signalling is required to inform a UE about the DMRS sequence used by another interfering UE.

For CRS, no mechanism facilitating the RS interference handling is available, nevertheless their high density combined with efficient signal processing may allow acceptable channel estimation quality for both CSI feedback and demodulation. Further study is needed to clarify this point. CRS being cell-specific, the victim UE can deduce the interferer’s RS sequence from the knowledge of its serving cell.

Note that, in the case of Successive Interferernce Cancellation receivers, channel re-estimation of the useful signal can be advantageously performed after interference subtraction in IC receivers.

## Informing the UE about the interferer’s parameters

In the case of inter-user interference mitigation (intra-cell MU-MIMO and/or inter-cell interference), and depending on the receiver type, the victim UE may require some interferer’s transmission parameters.

As stated in section 3.1, the channel of the interferer needs to be estimated for all the receivers presented in this contribution, except for the linear MMSE-IRC. For this, in case of DMRS-based demodulation, the DMRS sequence of the interferer is required. In case of CRS-based demodulation, the precoding information is required.

In the case of symbol level IC receiver, the victim UE could be informed about the modulation order of the interferer(s); this may not  be necessary if using blind demodulation detection techniques.

Resource allocation could be coordinated between the victim and its interferer(s) in order to facilitate the interference demodulation. For instance, QPSK could be favoured over higher-order modulations for UEs expected to be interferers, as the former will be easier to demodulate by the victim.

In the case of hard SIC and of Turbo-SIC receiver, the victim UE needs to know the following transmission parameters of the interferers:

- PRB assignments

- MCS

- RNTI

- DMRS sequence, if demodulation is based on DMRS

- Precoding information, if demodulation is based on CRS

Up to Release 11, a UE cannot access any of these pieces of information related to another UE. Some mechanisms (e.g. a new signalling) then need to be introduced into the standard in order to provide this information to the victim UE.

## Interferer’s MCS decodability

If channel decoding of the interferer is used (hard SIC receiver or turbo-SIC receiver), in order to decode the interferer’s data, its modulation and coding scheme (MCS) has to be decodable by the victim. Indeed, if a UE at the cell edge (thus experimenting generally a poor channel quality) is interfered by a signal intended to a UE in the cell center encoded with a high MCS (e.g. 64QAM, and 8/9 code rate) then the victim will most likely not be able to decode the interfering data. A coordinated link adaptation across neighbouring cells (or within the cell for MU-MIMO) taking into account both the UE of interest but also a potential IC-enabled interfered UE is therefore needed to guarantee that IC is feasible. Without such a coordination, IC would be sometimes feasible and sometimes not, making difficult for the base station to select the appropriate MCS for its served UE. This may impose scheduling restrictions, whose impact on the overall performance needs to be analyzed via system level evaluations.

## Resource alignment between victim and interferers

If channel decoding of the interferer is used (hard SIC receiver or turbo-SIC receiver), the resources allocated to both the victim UE and the interferer(s) should be aligned as much as possible in order to reduce the processing complexity. Indeed, the channel decoding operation requires demodulating the full resource allocation of the interferer. For instance, if one interferer’s signal occupies the full system bandwidth whereas the victim’s signal is transmitted only over one third of the bandwidth, then the complexity associated with decoding the interferer’s data is much higher than for the data of interest, which may reduce the interest for employing IC in this case. Ideally, the victim and interferer(s) would occupy the same resources. Nevertheless, having a non-identical but close resource allocation is expected to capture a large part of the IC benefits. The consequence would be that interference may not be cancelled on all the resources (if the interferer’s allocation is smaller than for the victim) or that a small extra complexity would be needed (if the interferer’s allocation slightly exceeds the victim’s one). However, it would yield the great benefit of providing more opportunities to the schedulers to find pairs of victim/aggressor UEs able to take advantage of IC. The resource alignment has to be taken care of by coordinated scheduling between neighbouring cells, in the same way as for CoMP.

In order to minimize the inter-cell IC complexity, it is required to time-synchronize the network, so that the victim and interfering signals can be demodulated via a single FFT.

Note that the two last challenges (PRB alignment and MCS decodability for hard SIC and turbo-SIC) require some coordination among the schedulers. In case of inter-site interference mitigation, this coordination can be limited due to backhaul constraints. Further study is required to evaluate the impact of limited coordination on capacity gain in this case.

# Initial IC receivers gain evaluation

This section provides preliminary IC receivers gain evaluation over baseline LMMSE receivers.

## IC receivers gain evaluation in case of intra-cell SU-MIMO interference cancellation

### Simulation assumptions

The following table provides the link level simulation assumptions.

| - Parameter | - Value |
| --- | --- |
| Bandwidth | 20 MHz |
| Channel Type | ePA 3 |
| Correlation at eNb side | Medium (0.5) |
| Correlation at UE side | Medium (0.2) |
| Transmission Mode | Beamforming (TM6), Closed loop precoding (TM4) |
| UE Reporting Mode | PUSCH Mode 1-2: WideBand CQI, Multiple PMI |
| HARQ | Chase Combining (up to 4 retransmissions) |
| UE Category | 5 [MIMO 4x4] |
| Receiver Type | LMMSE, Hard SIC and Turbo-SIC (ideal) |

### Link-level simulation results

The following figure shows the link level simulations results for LTE category 5 UE with typical transmit and receive antenna spatial correlation of 0.5 and 0.2, respectively.

Figure 2: link level simulations results for LTE receivers: LMMSE, Hard SIC, Turbo SIC (ideal)

Note that ideal Turbo SIC refers to the interference free upper bound. In practice, turbo-SIC implementation will yield performance that is within the Hard-SIC ones and this upper bound.  However, we have witnessed that when the number of spatial layers are below the number of receive antennas, turbo-SIC based on LMMSE-IC receiver performance are able to approach the interference free upper bound in some cases (sufficiently low coding rate).

## Upper bound of SIC receivers gain evaluation in case of perfect inter-cell interference cancellation under network assistance

- This section provides initial system level simulation results for SIC receivers [2].

### Simulation assumptions

- The following table provides the system simulation assumptions.

| - Parameter | - Value |
| --- | --- |
| - Carrier | - FDD DL, 2 GHz |
| - Bandwidth | - 10 MHz |
| - Network layout | - 19 tri-sector macro sites with wrap-around, 500 m ISD |
| - Channel model | - ITU Uma (dense urban), 3 km/h |
| - Penetration loss | - 20 dB for indoor UEs, 0 dB for outdoor UEs |
| - UEs distribution | - Uniform, 75% UEs indoors, 10 UEs/cell for full buffer |
| - Traffic models | - Full buffer |
| - Antennas | - 2 Tx X-pol, 2 Rx X-pol |
| - MIMO scheme | - SU-MIMO with adaptive number of layers |
| - Feedback | - PUSCH 3-1 with 5 ms duty cycle |
| - UE receiver | - MMSE-SIC with IRC for SU-MIMO - w/wo hard inter-cell interference cancellation up to 2 interferers |
| - Interference modelling | - Fully modelled from 57 cells |
| - Channel estimation | - Ideal |
| - Link adaptation | - Realistic, based on CQI feedback |
| - Scheduling | - Proportional Fair, frequency selective |
| - HARQ | - Explicit with 4 transmissions maximum |

- The baseline receiver is an MMSE-SIC modeling realistically the SU-MIMO interference cancellation. Inter-cell SIC is modeled in an idealistic manner: the received interference power associated from the processed interferers is affected by a 0.2 coefficient in the post-receiver SINR formula, which leads to cancelling 80% of the interferer’s received power. The modeled IC receiver have the capability of performing inter-cell IC for the N dominant interferers in the long term, i.e. based on their path loss and shadowing only, with N = 1 or N = 2.

- Note that IC receiver is assumed to be always possible, with no scheduling restriction, which is clearly an idealistic assumption. These results provide therefore an upper bound of the achievable performance.

### Intra-site case: system-level simulation results

| - No IC / IC | - Capacity (Mbps) | - Cell-edge user throughput (Mbps) |
| --- | --- | --- |
| - No IC | - 15.5 (0%) | - 0.38 (0%) |
| - IC, N=1 | - 19.2 (+24%) | - 0.45 (+19%) |
| - IC, N=2 | - 23.2 (+50%) | - 0.47 (+22%) |

- These results show that the intra-site case with 1or 2 interferers case provide already significant cell capacity and cell edge user throughput gains. This suggests that high gains can be expected even in the case of imperfect backhaul.

### Inter-site case: system-level simulation results

| - No IC / IC | - Capacity (Mbps) | - Cell-edge user throughput (Mbps) |
| --- | --- | --- |
| - No IC | 15.5 (0%) | 0.38 (0%) |
| - IC, N=1 | 20.3 (+31%) | 0.49 (+30%) |
| - IC, N=2 | 26.4 (+70%) | 0.63 (+66%) |

- These results show that the inter-site case with 1or 2 interferers case provide very high cell capacity and cell edge user throughput gains.

# Conclusions

This contribution has provided an analysis of the candidate receiver structures in terms of complexity and potential gains. According to the initial evaluation, the turbo-SIC receiver shows high potential for capacity gains, both for intra-cell interference and for inter-cell interference.

Regarding the challenges for interference cancellation receivers, we have identified the following aspects to be further studied:

- Guaranteeing a sufficient quality of the interferer’s channel estimation.

- Informing the UE about the interferer’s parameters.

- Allowing a given level of coordination among the schedulers in the case of inter-cell interference mitigation, (e.g., if channel decoding of the interferer is used, coordination may be required to address the issues of interferer’s MCS decodability and resource alignment between victim and interferers). The impact of limited scheduling coordination due to backhaul constraints needs to be evaluated during the Study Item.

# References

[1] RP-130404, “Study on Network-Assisted Interference Cancellation and Suppression for LTE”, MediaTek, Renesas Mobile Europe, Broadcom Corporation, March 2013

[2] Eric Hardouin, Mohamad Sayed Hassan and Ahmed Saadani, “Downlink Interference Cancellation in LTE: Potential and Challenges”, , WCNC 2013, April 2013.

[3] Artist4G project deliverable D2.2 “Advanced receiver signal processing techniques: evaluation and characterization”, Alexander Tyrrell, Hendrik Schoeneich, https://ict-artist4g.eu/projet/deliverables

[4] R1-090919, “Performance prediction of turbo-SIC receivers for system-level simulations”, Orange, Nokia, Nokia Siemens Networks, and Texas Instrument, Athens, Greece, Feb. 2009

[5] R1-084319, “Considerations on SC-FDMA and OFDMA for LTE-Advanced Uplink,” Nokia Siemens

Networks, Nokia.