3GPP TSG-RAN WG1 #72bis	                                      R1-131632

Chicago, USA, April 15 – April 19, 2013

Agenda item:	7.2.1.1

Source: 	CATR

Title: 	Simulation results of new DM RS pattern design

Document for:	Discussion/Decision

1	Introduction

At RAN1#72 meeting, to avoid collisions with PSS/SSS, and improve performance for data transmission in legacy control region (especially at high speed), small cell related motivations(reduced overhead for very low mobility, improved channel estimation for higher order modulation), a conclusion was made to study further the different alternatives, including studying the benefits of new DM RS pattern on the NCT, study should consider both PDSCH and ePDCCH, any possible impact on CSI-RS, may also take into account hypothetical impact on PBCH. Simulation assumptions were also made and some DM RS patterns were shown in the following email discussion.

In this paper, we evaluate different DM RS patterns based on the agreed simulation assumptions, and give our observation.

2	Different DM RS pattern

The positions of PSS/SSS in TDD system and FDD system are different, in FDD system, PSS/SSS is positioned at two continuous OFDM symbols of both subframe 0 and subframe 5, in TDD system, the PSS is positioned at one OFDM symbol of subframe 1 and subframe 6 (special subframe), and the SSS is positioned at one OFDM symbol of subframe 0 and subframe 5, PSS/SSS have intervals of two OFDM symbols.

Here we simulate several DM RS patterns as shown in figure 1. Figure 1(a),1(c),1(d) can avoid collision with PSS/SSS of both TDD and FDD normal subframes. Figure1(b) cannot avoid collision with PSS/SSS of TDD normal subframes.

- Pattern 1(proposed by LGE)   (b) pattern 2(proposed by Ericsson)

(c) pattern 3(proposed by HW)   (d) pattern 4(proposed by LGE)

Figure 1 different new DM RS patterns

2.1 	Simulation results

Here we give simulation results of PDSCH, to see the BLER and spectrum efficiency performance. From figure 2 and figure 3, we could see the performance of different DM RS patterns is similar with R10 DM RS pattern in scenarios of both EPA 3km/h and ETU 30km/h.

Figure 2 BLER and spectrum efficiency performance of PDSCH（EPA 3km/h）

Figure3 BLER and Spectrum efficiency performance of PDSCH （ETU 30km/h）

Observation :

- Different DM RS patterns have similar performance with R10 DM RS pattern at scenarios of both low and medium speeds.

3	Conclusions

From our simulation results, we observed similar performance of new DM RS patterns with  R10 DM RS pattern.

References

- R1-130835 Proposals on DMRS enhancements for NCT CATT

- R1-130834 Candidate DM RS Patterns for New Carrier Type LGE

- R1-130833 Alternate UE specific RS pattern on NCT for Evaluation Ericsson

- R1-130742 Simulation Assumptions for DM RS pattern in NCT Huawei

Appendix

Table 1 simulation assumptions

| Parameter | Setting |
| --- | --- |
| Carrier frequency | 2 GHz |
| Transmission bandwidth | 5 MHz |
| Channel model | EPA 3km/h ETU 30km/h |
| CP length | Normal CP |
| Location and number of PDSCH RBs | Fixed, 6 PRBs |
| Transmission mode | TM9 |
| Antenna configuration | 2 TX, 2 RX |
| Rank | 1 |
| PDSCH/EPDCCH starting symbol | The first OFDM symbol |
| Overhead assumption | No PSS/SSS/CRS/CSI-RS for all subframes 12 REs/PRB for DMRS |
| Modulation model | QPSK |
| Performance metrics for PDSCH | Spectral effciency |