3GPP TSG RAN WG1 Meeting #77	                                R1-142134

Seoul, Korea, 19th – 23rd May 2014

      - Agenda Item:	6.2.3.1

- Source:	LG Electronics

- Title:	Remaining issues for supporting downlink 256QAM

Document for:	Discussion and decision

Introduction

In last RAN1 #76bis meeting, following agreements or conclusions on 256QAM support were made:

- Switching point of 64QAM and 256QAM should be CQI 15 in the existing table

  - The modulation order of existing CQI 15 is changed to 256QAM

- Working assumption: down-sample low CQI entries by removing 3 QPSK entries, and add 3 new entries for 256QAM region

  - Revisit if problems if significant issues are found

  - The 3 entries to be removed are either {#1, #3, #5} or {#2, #4, #6}

  - The last 4 entries will be for 256QAM, but the actual SE is FFS

- Order the CQI indices in the Rel-12 CQI table according to the spectral efficiencies

- 256QAM is supported for up to 8-layer PDSCH transmissions

- TBS table

  - Define overhead assumption(s) (REs/PRB) for PDSCH

    - Use 120 REs per PRB for all 256QAM spectral efficiencies except for the highest spectral efficiency

    - Use 136 REs per PRB for the highest spectral efficiency

  - Limit the number of new TBS values as much as possible

- DCI format 1A and DCI format 1C are associated with the legacy MCS table, i.e., not supporting 256QAM PDSCH scheduling

- For all other DCI formats scheduling PDSCH, 256QAM can be supported

- 256QAM PDSCH scheduling is only supported for C-RNTI based PDSCH transmissions

- FFS whether or not 256QAM is supported for PMCH transmissions

- MCS Table

  - 7 explicit MCS entries for 256QAM

  - As a working assumption, the # of implicit entries is 4 (for QPSK, 16/64/256QAM re-transmissions)

    - Revisit if significant issues are found

- In TM10, decide in RAN1#77 between the following alternatives:

  - Alt 1: CQI table can be CSI process dependent and MCS table can be PQI dependent

  - Alt 2: CQI table is common for all CSI processes and MCS table is common for all PQI sets

  - Alt 3: CQI table can be CSI process dependent and MCS table is common for all PQI states

- FFS, decide in RAN1#77 between the following two alternatives

  - Alt 1: the use of 256QAM CQI table can be configured for each measurement subframe set

  - Alt 2: the use of 256QAM CQI table can be configured is common for all measurement subframe sets

In this contribution, we discuss remaining issues for 256QAM operation including details of CQI/MCS/TBS tables, 256QAM configurability in eICIC and CoMP scenarios, and some considerations for power backoff for medium class eNB operation.

Further details on CQI/MCS/TBS tables

CQI/MCS table

According to the agreement/working assumptions in last RAN1#76bis meeting, switching point between 64QAM and 256QAM is CQI 15 and 3 new CQI entries should be introduced for 256QAM. Then, it is required to set the spectral efficiency (SE)/coding rate for new CQI entries. In general, it is desirable to minimize the standard effort for CQI table design. Since the switching point between 64QAM and 256QAM was decided to be CQI 15, the spectral efficiency should be kept. Otherwise, the corresponding TBS entries (such as mapping to ITBS = 25 and ITBS = 26) may be changed.

Proposal 1: The SE of CQI entry corresponding to switching point between 64QAM and 256QAM should be kept as current one.

Regarding the maximum SE, it is also desirable to set it assuming current maximum coding rate (e.g., 948/1024) in the legacy CQI table to minimize standard effort.

Proposal 2: The maximum SE should be calculated assuming current maximum coding rate (e.g., 948/1024).

The Table1 shows the candidate of SE/coding rate of 256QAM CQI entries.

Table 1. Candidate 256QAM CQI table.

| CQI index | Modulation | Coding rate*1024 | Spectral efficiency |
| --- | --- | --- | --- |
| 12 | 256 | 711 | 5.5547 |
| 13 | 256 | 791 | 6.1779 |
| 14 | 256 | 876 | 6.8434 |
| 15 | 256 | 948 | 7.4063 |

According to the working assumption in last RAN1#76bis meeting, 3 CQI entries from legacy CQI table should be replaced with entries for 256QAM. The two candidates for removal are as follows:

- Candidate set 1:{CQI 1, CQI 3, CQI 5}

- Candidate set 2: {CQI 2, CQI 4, CQI 6}

The removed entries affect the MCS table since CQI and MCS table were linked for each other. For example, CQI  2 is mapped to MCS 0 and CQI 3 is mapped to MCS 2, respectively. Hence, removal of Candidate set 1 makes the MCS 2 and MCS 6 be removed. On the other hand, removal of Candidate set 2 makes the MCS 0, MCS 4, and MCS 8 be removed. If we remove MCS indices corresponding to overlapped TBS indices, MCS 10 and MCS 17 should be also removed. It is natural to keep the corresponding MCS/TBS indices mapped to survived CQI entries. Then, since 7 new MCS entries should be introduced to support 256QAM, additional 3 and 2 MCS indices should be selected to be removed in the legacy MCS table for each candidate respectively. We evaluate the throughput performance for possible set of combinations of MCS/TBS indices to show the performance impact by removal of CQI entries.  The detailed simulation assumptions are shown in appendix. The Table 2 shows the best performance among possible combinations for two candidates. The removed MCS indices for each Candidate set are {MCS 2, MCS 3, MCS 5, MCS 6, MCS10, MCS12, MCS 17} and {MCS 0, MCS 3, MCS 4, MCS 7, MCS 8, MCS 10, TBS 17}, respectively. As can be seen in Table 2, the removal of Candidate set 1 outperforms the removal of Candidate set 2. Thus, we propose to remove the set of {CQI 1, CQI 3, CQI 5} in the legacy CQI table and the set of {MCS 2, MCS 3, MCS 5, MCS 6, MCS10, MCS12, MCS 17} in the legacy MCS table.

Table 2. Performance comparison by removal of Candidate set 1 and Candidate set 2.

| Config | UeRxTput | UeTput(5%) | UeTput(50%) | UeTput(95%) |
| --- | --- | --- | --- | --- |
| Candiate set 1: RSRP threshold: -80 dBm | 50101 | 5674 | 50000 | 78431 |
| Candiate set 2: RSRP threshold: -80 dBm | 49999 | 5525 | 50000 | 78431 |

Proposal 3: Remove the set of {CQI 1, CQI 3, CQI 5} in the legacy CQI table and the set of {MCS 2, MCS 3, MCS 5, MCS 6, MCS10, MCS12, MCS 17} in the legacy MCS table.

Regarding the ordering of MCS indices in the MCS table, we prefer the increasing ordering of MCS indices according to SE. Since we already agreed to use the legacy CQI/MCS table to support robust fall-back operation, we don’t see the clear benefit of not ordering the MCS indices according to SE.

Proposal 4: MCS indices should be ordered in increasing order of SE in the MCS table.

TBS table

The overhead assumption for TBS determination was agreed in the last RAN1#76bis meeting: 120 RE’s will be assumed except for the highest spectral efficiency and 136 RE’s will be assumed for the highest SE. In the current TBS table, the overhead assumption of TBS 26 is 136 RE’s. Hence, the TBS corresponding to TBS 26 may be also changed in 256QAM transmission. The Table 3 shows the example of SE values for TBS index for 256QAM.

Table 3. Example of SE values for 256QAM TBS index.

| TBS index | 26 | 27 | 28 | 29 | 30 | 31 | 32 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SE | 5.5547 | 5.8663 | 6.17791 | 6.5106 | 6.8434 | 7.1248 | 7.4063 |

The Table 4 and Table 5 show the example of TBS for single layer and multi-layer transmission, respectively.  If no new TBS for single layer transmission is assumed, the maximum TBS could be 97896.

However, the effective coding rate will be 0.904 with 97896 TBS. This value is much less than the maximum supportable coding rate of 0.93. Hence, we can consider the new TBS only for the highest TBS index so that the coding rate can approach to 0.93. For example, we can achieve coding rate of 0.927 with TBS of 99664. Even if this TBS does not exist in the set of legacy TBS tables, we can enhance peak rate for 256QAM transmission with negligible standard impact. Then, if the TBS for 100 RB to 110 RB is 99664, the TBS for multi-layer transmission will be 199824, 299856, and 397776 for 2-layer, 3-layer, and 4-layer transmission as shown in Table 5.

Table 4. Example of TBS for 256QAM for single layer transmission.

Table 5. Example TBS for 256QAM for multi-layer transmission.

| TBS_L1 | TBS_L2 | TBS_L3 | TBS_L4 |
| --- | --- | --- | --- |
| 76208 | 152976 | 230104 | 305976 |
| 78704 | 157432 | 236160 | 314888 |
| 81176 | 161760 | 245648 | 324336 |
| 84760 | 169544 | 254328 | 339112 |
| 87936 | 175600 | 266440 | 351224 |
| 90816 | 181656 | 275376 | 363336 |
| 93800 | 187712 | 284608 | 375448 |
| 97896 | 195816 | 293736 | 391656 |
| 99664 | 199824 | 299856 | 397776 |

256QAM configurability

256QAM configuration per measurement subframe set

When a UE is configured with eICIC and measurement subframe set, it is still expected that some UEs can be serviced with 256QAM when the dominant interferer performs ABS operation. However, it is expected that the same UE may not be able to enjoy 256QAM in non-ABS subframes due to heavy interference from the dominant interferer. Another example would be eIMTA scenario where downlink power may be different in flexible and fixed downlink subframes which could lead different SINR range in two subframe sets. Considering this, we can consider whether to have separate configuration of 256QAM per measurement subframe set or eIMTA flexible/fixed subframes. To show performance difference, we evaluate eICIC scenario with 30% ABS ratio in a small cell scenario #1. The detailed simulation assumptions are found in the appendix. As concluded in the last meeting, we compare two alternatives to handle this case

- Alt 1: the use of 256QAM CQI table can be configured for each measurement subframe set

- Alt 2: the use of 256QAM CQI table can be configured is common for all measurement subframe sets

Table 5. User throughput difference between two options

|  | UeRxTput | UeTput(5%) | UeTput(50%) |
| --- | --- | --- | --- |
| Alt1 | 102% (+2%) | 111%(+11%) | 108%(+8%) |
| Alt2 | 100% | 100% | 100% |

We configure 256QAM to all UEs in small cell layer for the simplicity in this evaluation. As shown in Table 5, Alt1 (different configuration per measurement set) shows potential performance gain over Alt2 in 5%/50% user throughput. Thus, we propose to adopt Alt1.

Proposal 5: The use of 256QAM CQI table can be configured for each measurement subframe set.

Handling CoMP scenario

In a CoMP scenario, two aspects should be considered. First aspect is whether all CoMP transmission points can support 256QAM operation. Particularly, if we consider a CoMP scenario such as scenario 3 where macro and RRH may perform CoMP operation, macro cell may not support 256QAM due to its power limitation or EVM requirement. Thus, separate capability per transmission point should be considered. Second aspect is whether all transmission points will configure 256QAM to a UE if 256QAM is configured. Since a UE has different SINR condition from each TP and also supporting 256QAM by transmission point may require power backoff which results in lower coverage for 256QAM configured UEs, separate configurability of 256QAM per transmission point according to UE SINR conditions should be considered.

Thus, separate configuration of 256QAM per TP or non-zero power CSI-RS resource should be supported.  If this is applied, which table is used to interpret MCS field of DCI and calculate TBS can be determined by PQI where each entry in PQI table can contain the configurability of 256QAM. Furthermore, CQI table can be separately configured for each CSI process for efficient operation.

As concluded in the last meeting, there are three potential options to handle CoMP scenario as the following.

- Alt 1: CQI table can be CSI process dependent and MCS table can be PQI dependent

- Alt 2: CQI table is common for all CSI processes and MCS table is common for all PQI sets

- Alt 3: CQI table can be CSI process dependent and MCS table is common for all PQI states

To illustrate potential SINR difference by coordinated muting, we compare geometry with coordinated muting and not as shown in Figure 1 for UEs with SINR larger than 19.4dB.

(a) Geometry difference between CoMP                      (b) Geometry differences CDF

and non-CoMP scenario

Figure 1. Geometry Difference

As it shows, the SINR difference by coordinated muting and not can be considerable. Thus, we consider Alt1 should be adopted.

Proposal 6: CQI table can be CSI process dependent and MCS table can be PQI dependent.

Power backoff impact

According to LS response from RAN to RAN4 [1], RAN4 receives guidance from RAN to start working on requirements for medium range BS once requirements on local area BS and home BS are finalized. When medium range BS class is considered, it could be that the power backoff range can be increased to meet the EVM requirement. In other words, power difference due to backoff between supporting 256QAM and not may not be negligible. Thus, some considerations on how to apply power backoff needs some further considerations. In terms of applying power backoff, overall the following three approaches can be considered.

- cell-common power backoff: eNB supporting 256QAM reduces the power regardless of 256QAM configuration. This approach would lead coverage shrinkage as the overall power is reduced. Thus, it is expected that this will impact the overall user throughput considerably.

- power backoff on 256QAM configured UEs: instead of reducing the average power, the network may selectively reduce data transmission power to 256QAM configured UEs. More specifically, power on control channels can be kept where power on data channels can be reduced for those UEs. This approach reduces coverage for 256QAM configured UEs. Thus, if a UE is configured with 256QAM yet not scheduled with 256QAM, the overall user throughput for the UE may be degraded due to coverage loss. This may be supported by configuring UE-specific  and accordingly.

- power backoff on 256QAM scheduled UEs: further to reduce the impact from power backoff, power backoff may be achieved only if a UE is scheduled with 256QAM. Since different power backoff values are used per modulation scheme used, this approach would require additional specification work such as introducing additional.

- subframe set based approach where two subframe sets can be used with different power and possible modulation. For example, the first set can be scheduled with 30dBm and non-256QAM modulation whereas the second set can be scheduled with 24dBm with 256QAM modulation.

We evaluate potential user throughput differences between different options. We compare the performance among

- Reference: use power backoff in all subframes with 256QAM configuration to UEs with SINR >=19.4dB

- Option 1: Subframe set approach (with 20% and 40% 24dBm subframes – i.e., 2 out of 10 subframes use 24dBm and 4 out of 10 subframes use 24dBm respectively)

- Option 2:  apply power backoff only to 256QAM scheduled UEs

Table 6. User throughput results with different options

| Config | #UEs (small cell UE) | UeRxTput | UeTput(5%) | UeTput(50%) |
| --- | --- | --- | --- | --- |
| Reference | 1611 | 1 | 1 | 1 |
| Option 1 (20%) | 1611 | 1.03 | 3.23 | 1.18 |
| Option 1 (40%) | 1609 | 0.96 | 1.80 | 1.01 |
| Option 2 | 1604 | 1.22 | 3.60 | 1.5 |

As it can be shown in the results, either Option 1 with 20% or Option 2 offers performance benefits over reference technique where power backoff is applied regardless of 256QAM schedulability. For the reference, 6dB RSRQ bias is used to maintain the similar small cell UE ratio. Particularly, with Option 2, the performance gains in both 5% and 50% are considerably high. Given the potential performance gain, we propose to further consider mechanisms to handle large power backoff when 256QAM is applied to medium power class eNBs.

Proposal 7: Further consideration on power backoff for 256QAM operation of medium class eNB is needed.

UE category

In RAN1#76bis, some discussions on handling of 256QAM capability and UE category occurred. In our view, one new UE category targeting ~4Gbps with 5CC/8 layer MIMO with 256QAM can be introduced as it reflects the ultimate data rate that current technology can offer. In terms of handling 256QAM in other UE categories, we think that UE capability to indicate whether the UE supports downlink 256QAM or not can be introduced as well. In terms of applying 256QAM capability in existing UE categories, as discussed in [2], there could be four options to handle maximum TB size and soft buffer size. As the applicability of 256QAM may be limited to cases where operating SINR range is very high such as sparse indoor scenarios, we consider minimizing hardware impact to introduce 256QAM would be beneficial. Particularly, for low category such as Cat 3/4, we prefer not to change any UE capability other than introducing 256QAM capability signaling. For Cat 5 to Cat 7, we can consider to increase the maximum TB size per transport block without increasing maximum data rate which will have minimum impact on baseband capability. If we consider increasing peak data rate as well, Cat 9/10 may be considered to increase the peak data rate.

Based on these, the following summarizes our proposal.

| UE Category | Maximum number of DL-SCH transport block bits received within a TTI (Note) | Maximum number of bits of a DL-SCH transport block received within a TTI | Total number of soft channel bits | Maximum number of supported layers for spatial multiplexing in DL |
| --- | --- | --- | --- | --- |
| Category 1 | 10296 | 10296 | 250368 | 1 |
| Category 2 | 51024 | 51024 | 1237248 | 2 |
| Category 3 | 102048 | 75376 | 1237248 | 2 |
| Category 4 | 150752 | 75376 | 1827072 | 2 |
| Category 5 | 299552 | 149776 (64QAM 4 layers) 199824 (256QAM 4 layers) | 3667200 | 4 |
| Category 6 | 301504 | 149776 (64QAM 4 layers) 199824 (256QAM 4 layers) 75376(64QAM 2 layers) 99664 (256QAM 2 layers) | 3654144 | 2 or 4 |
| Category 7 | 301504 | 149776 (64QAM 4 layers) 199824 (256QAM 4 layers) 75376 (64QAM 2 layers) 99664 (256QAM 2 layers) | 3654144 | 2 or 4 |
| Category 8 | 2998560 | 299856 | 35982720 | 8 |
| Category 9 | 452256 (64QAM) [597984 (256QAM)] | 149776 (64QAM 4 layers) 199824 (256QAM 4 layers) 75376 (64QAM 2 layers) 99664 (256QAM 2 layers) | 5481216 | 2 or 4 |
| Category 10 | 452256 (64QAM) [597984 (256QAM) ] | 149776 (64QAM 4 layers) 199824 (256QAM 4 layers) 75376 (64QAM 2 layers) 99664 (256QAM 2 layers) | 5481216 | 2 or 4 |
| Category 11 | 3977760 (256QAM 8 layer) | 3977760 (256QAM 8 layer) | 47733120 | 8 |
| NOTE: In carrier aggregation operation, the DL-SCH processing capability can be shared by the UE with that of MCH received from a serving cell. If the total eNB scheduling for DL-SCH and an MCH in one serving cell at a given TTI is larger than the defined processing capability, the prioritization between DL-SCH and MCH is left up to UE implementation. |  |  |  |  |

Conclusions

In this contribution, we discuss the remaining issues for 256QAM support in downlink. We have the following proposals:

Proposal 1: The SE of CQI entry corresponding to switching point between 64QAM and 256QAM should be kept as current one.

Proposal 2: The maximum SE should be calculated assuming current maximum coding (e.g., 948/1024).

Proposal 3: Remove the set of {CQI 1, CQI 3, CQI 5} in the legacy CQI table and the set of {MCS 2, MCS 3, MCS 5, MCS 6, MCS10, MCS12, MCS 17} in the legacy MCS table.

Proposal 4: MCS indices should be ordered in increasing order of SE in the MCS table.

Proposal 5: The use of 256QAM CQI table can be configured for each measurement subframe set.

Proposal 6: CQI table can be CSI process dependent and MCS table can be PQI dependent

Proposal 7: Further consideration on power backoff for 256QAM operation of medium class eNB is needed.

References

[1] R1-141111	“Reply LS to R4-141254 = RP-140024 on BS classes and maximum BS power on 256QAM”, RAN#63

[2] R1-141513, 	“On UE categories for 256QAM support”, Nokia, NSN, RAN1#76bis

- Appendix

- A. SLS parameter assumption

| Parameters | Assumptions |  |  |
| --- | --- | --- | --- |
| Section | 2 | 3 | 4 |
| Scenario | Scenario #2b sparse | Scenario #1 sparse, Scenario #2a sparse | Scenario #2a sparse |
| Number of macro site | 7 |  |  |
| System bandwidth per carrier | 10 MHz |  |  |
| Total Small cell TX Power | 24 dBm | 24 dBm |  |
| Number of clusters per macro cell geographical area | 1 |  |  |
| Number of small cells per cluster | 4 | 2 | 2 |
| Distance-dependent path loss | ITU model as baseline. |  |  |
| UE dropping | 20% UEs are outdoor and 80% UEs are indoor. |  |  |
| Cell association | RSRQ with realistic buffer. | RSRP + bias 6 dB. | RSRQ + bias with realistic buffer. |
| Scheduling | PF |  |  |
| Traffic model | FTP Model 1 as in TR 36.814 |  |  |
| RU | 10% | 50% | 20% |
| UE receiver | MMSE-IRC |  |  |
| Antenna configuration | 2Tx2Rx in DL, Cross-polarized |  |  |
| Tx EVM | 4% | 4% | 4% for Small cell Tx power = 24 dBm, 8% for Small cell Tx power = 30 dBm |