3GPP TSG-RAN1 Meeting #77   	                               R1- 142211

Seoul, Korea, 19th – 23rd May 2014

----------------------------------------------------------------------------------------------------------------

Agenda Item:	6.2.5.1.1

Source:		ETRI

Title:	DMRS Structure for Reducing UE Power Consumption

Document for:	Discussion and decision

---------------------------------------------------------------------------------------------------------------

# 1 Introduction

UE power consumption is one of the major issues in D2D communications. In order to reduce power consumption, previous RAN1 meetings have adopted separation of resource pools for SA and D2D data communication. Also, RAN1 is considering including an ID in the SA to enable UEs to reduce the probability of decoding of data packets which they are not interested in.

This contribution suggests DMRS structure of SA, which provides further reduction of UE power consumption on top of the separation of resource pools for SA and D2D data transmissions.

# 2 Separation of resource pools

In RAN1 #76, there was a discussion that separation of resource pools between SA transmissions and D2D data communications can save UE battery power consumption. As shown in Figure 1, there is a linkage between SA packet and D2D data packet. SA packet contains information of resource allocation and reception parameters for a specific D2D data packet. Therefore, upon decodings of SA packets in the SA resource pool, UE knows how to decode and whether to decode or dismiss D2D data packets in the D2D data resource pool if there are D2D data packets dedicated to itself.

Also, SA packet can include an ID enabling UE to reduce the probability of decoding of D2D data packets which are not interested in.

Figure 1: Separation of resource pools for SA and D2D data.

# 3 Further UE battery savings

This Section suggests further reduction of UE power consumption on top of the separation of resource pools for SA and D2D data transmissions.

Figure 2 shows SA packet structure based on PUSCH. DMRS in SA packet is scrambled by an ID which can reduce the probability of decoding of data packets. In the figure, this ID is used as a seed to generate scrambling sequence for DMRS. The detection of the ID on DMRS enables a UE not to decode even SA packets which are not dedicated to the UE itself. Not fully decoding of SA packet but just detection of DMRS further decreases UE power consumption.

Figure 2: SA packet structure based on PUSCH.

- Proposal 1: DMRS in SA packet should be scrambled.

- Proposal 1a: The seed of scrambling sequence for DMRS in SA should be an ID included in the SA itself.

# 4 Conclusions

In this contribution, we discussed DMRS structure which leads to reducing UE power consumption. From the discussions, following proposals can be obtained.

- Proposal 1: DMRS in SA packet should be scrambled.

- Proposal 1a: The seed of scrambling sequence for DMRS in SA should be an ID included in the SA itself.

References

- Chairman’s Notes RAN1#76.

- Chairman’s Notes RAN1#76bis.

- TR 36.843 v1.2.0 (2014-02), “Study of LTE Device to Device Proximity Services; Radio Aspects.”