3GPP TSG RAN WG1 Meeting #77,	R1-142375

Seoul, Korea, 19th – 23rd May 2014

Source:	Ericsson

Title:	CQI and MCS table design for downlink 256QAM

Agenda Item:	6.2.3.1

Document for:	Discussion and Decision

# Introduction

In RAN1 #76bis meeting, further details on table design for DL 256QAM were discussed and agreements were made with a few items for further study. In this contribution, we discuss the remaining issues and propose CQI and MCS table for DL 256QAM. The TBS table design is discussed in our companion contribution [1].

# CQI table design

The CQI table design was discussed in RAN1 #76bis meeting and agreed working assumption is:

- Working assumption: down-sample low CQI entries by removing 3 QPSK entries, and add 3 new entries for 256QAM region

  - Revisit if significant issues are found

  - The 3 entries to be removed are either {#1, #3, #5} or {#2, #4, #6}

  - The last 4 entries will be for 256QAM, but the actual SE is FFS

System level simulations have been done to verify the two options as shown in Figure 1 based on simulation assumptions in Table 5. The performance difference of the two options is negligible regarding both the number of radio link failures and DL throughput. To support a robust system operation, it is preferred to keepin the MCS table to ensure the same RRC and VoIP robustness as in Rel-8. In order to use , Rel-8 CQI 2 should be retained in the new CQI table for a good mapping. Therefore, it is preferred to remove CQI entries {1, 3, 5}.

It was agreed that the switching point between 64QAM and 256QAM is CQI 15 and the last 4 entries in the new CQI table are used for 256QAM. According to this principle, Rel-8 CQI 15 is modified to be 256QAM at the same spectral efficiency (SE) and three new CQI values are introduced to cover equal SNR spacing between CQI 15 rate and the highest 256QAM rate at 8*0.9258 bps/Hz. The new CQI table is given in Table 2 in Annex.

Proposal:

- Remove CQI entries {1, 3, 5}.

- Rel-8 CQI 15 is modified to be 256QAM entry at same SE and the SE  for 3 new 256QAM entries are 6.1719, 6.7890, 7.4063 bps/Hz.

Figure 1 Performance comparison of the legacy and the new CQI tables (opt1: remove CQI #1,3,5; opt2: remove CQI #2,4,6)

# MCS table design

The agreement on MCS table design from RAN1 #76bis meeting is:

- 7 explicit MCS entries for 256QAM

- As a working assumption, the # of implicit entries is 4 (for QPSK, 16/64/256QAM re-transmissions)

  - Revisit if significant issues are found

Based on RAN1 agreements, we provide two options of MCS table design as given in Table 3 and Table 4 in Annex.

In Option 1, there are 28 explicit MCS entries and 4 reserved entries for re-transmission, aligned with RAN1 working assumption.

- In the low SINR region, MCS 0 is kept to support  for RRC/VoIP robustness and MCS entries {1, 3, 5, 7, 9, 10} are removed from Rel-8 table. The SE overlapping is also removed to save entries for 256QAM.

- In the high SINR region, 7 entries are added according to RAN1 agreement. Four of the MCS entries map to the 256QAM CQI entries in the new CQI table except the SE of last MCS entry which is modified to a higher SE due to a smaller overhead assumption, i.e. 136 REs/RB instead of 120 REs/RB. Three MCS entries are added in between to sample 256QAM with equal spacing. It is noted that MCS 23 in the new table reuse TBS 26 and hence only 6 new TBS rows need to be appended in the TBS table as given in [1].

In Option 2, there are 29 explicit MCS entries and 3 reserved entries for re-transmission. The design of explicit MCS entries is the same as option 1 except that one entry is added back, i.e. Rel-8 MCS 10. With one more entry in the table, the SINR gap between QPSK and 16QAM is decreased, which could achieve a better scheduling performance. In this option, 3 reserved entries MCS {29, 30, 31} are used for adaptive retransmission, as it is very unlikely that QPSK modulated transmission will apply 256QAM for retransmission and vice versa. The retransmission scheme depends on the initial modulation shown as below:

- 2, 4, 6 if initial transmission is based on QPSK or 16QAM

- 4, 6, 8 if initial transmission is based on 64QAM or 256QAM

It can be observed from Table 1 that both options have good VoIP support, i.e., all the special VoIP sizes can still be supported (i.e., the remaining rows without dark grey shading).

Table 1: Rel-8 VoIP TBS

|  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 16 | 32 | 56 | 88 | 120 | 152 | 176 | 208 | 224 | 256 |
| 1 | 24 | 56 | 88 | 144 | 176 | 208 | 224 | 256 | 328 | 344 |
| 2 | 32 | 72 | 144 | 176 | 208 | 256 | 296 | 328 | 376 | 424 |
| 3 | 40 | 104 | 176 | 208 | 256 | 328 | 392 | 440 | 504 | 568 |
| 4 | 56 | 120 | 208 | 256 | 328 | 408 | 488 | 552 | 632 | 696 |
| 5 | 72 | 144 | 224 | 328 | 424 | 504 | 600 | 680 | 776 | 872 |
| 6 | 328 | 176 | 256 | 392 | 504 | 600 | 712 | 808 | 936 | 1032 |
| 7 | 104 | 224 | 328 | 472 | 584 | 712 | 840 | 968 | 1096 | 1224 |
| 8 | 120 | 256 | 392 | 536 | 680 | 808 | 968 | 1096 | 1256 | 1384 |
| 9 | 136 | 296 | 456 | 616 | 776 | 936 | 1096 | 1256 | 1416 | 1544 |
| 10 | 144 | 328 | 504 | 680 | 872 | 1032 | 1224 | 1384 | 1544 | 1736 |
| 11 | 176 | 376 | 584 | 776 | 1000 | 1192 | 1384 | 1608 | 1800 | 2024 |
| 12 | 208 | 440 | 680 | 904 | 1128 | 1352 | 1608 | 1800 | 2024 | 2280 |
| 13 | 224 | 488 | 744 | 1000 | 1256 | 1544 | 1800 | 2024 | 2280 | 2536 |
| 14 | 256 | 552 | 840 | 1128 | 1416 | 1736 | 1992 | 2280 | 2600 | 2856 |
| 15 | 280 | 600 | 904 | 1224 | 1544 | 1800 | 2152 | 2472 | 2728 | 3112 |
| 16 | 328 | 632 | 968 | 1288 | 1608 | 1928 | 2280 | 2600 | 2984 | 3240 |
| 17 | 336 | 696 | 1064 | 1416 | 1800 | 2152 | 2536 | 2856 | 3240 | 3624 |
| 18 | 376 | 776 | 1160 | 1544 | 1992 | 2344 | 2792 | 3112 | 3624 | 4008 |
| 19 | 408 | 840 | 1288 | 1736 | 2152 | 2600 | 2984 | 3496 | 3880 | 4264 |
| 20 | 440 | 904 | 1384 | 1864 | 2344 | 2792 | 3240 | 3752 | 4136 | 4584 |
| 21 | 488 | 1000 | 1480 | 1992 | 2472 | 2984 | 3496 | 4008 | 4584 | 4968 |
| 22 | 520 | 1064 | 1608 | 2152 | 2664 | 3240 | 3752 | 4264 | 4776 | 5352 |
| 23 | 552 | 1128 | 1736 | 2280 | 2856 | 3496 | 4008 | 4584 | 5160 | 5736 |
| 24 | 584 | 1192 | 1800 | 2408 | 2984 | 3624 | 4264 | 4968 | 5544 | 5992 |
| 25 | 616 | 1256 | 1864 | 2536 | 3112 | 3752 | 4392 | 5160 | 5736 | 6200 |
| 26 | 712 | 1480 | 2216 | 2984 | 3752 | 4392 | 5160 | 5992 | 6712 | 7480 |

Observation:

- Both Option 1 and 2 of MCS table design have good VoIP support and fine SINR granularity.

- MCS table design Option 2 has a smaller SINR gap between QPSK and 16QAM, which could achieve a better scheduling performance.

Proposal:

- Retain TBS 0 in Rel-8 MCS table for RRC/VoIP robustness and remove every second MCS entries at QPSK region, i.e., remove MCS {1, 3, 5, 7, 9}.

- Introduce 7 MCS entries for 256QAM

  - 4 MCS entries correspond to CQI 12, 13, 14, and 15 in the new CQI table, except that the SE of MCS entry corresponding to CQI 15 is increased due to smaller overhead assumption for last TBS row.

  - 3 MCS entries are added in between with equal spacing.

- Reuse Rel-8 TBS indices as much as possible

  - Introduce 6 new TBS indices

# Simulation results

Link level simulation has been performed to verify the proposal CQI and MCS table (option 2). The achievable spectral efficiency for different MCS with 10% BLER target is shown in Figure 2 for both AWGN channel and the EPA channel (both rank 1 and rank 2).

Figure 2 Link level simulations using new CQI/MCS table (BLER 0.1)

# Conclusions

In this contribution, we discussed the CQI table and MCS table design for 256QAM. Based on the analysis and simulation results, we believe the CQI and MCS tables are appropriately designed to support downlink 256QAM in Rel-12.

Observation:

- Both option 1 and 2 of MCS table design have good VoIP support and fine SINR granularity.

- MCS table design option 2 has a smaller SINR gap between QPSK and 16QAM, which could achieve a better scheduling performance.

Proposal:

CQI table:

- Remove CQI entries {1, 3, 5}.

- Rel-8 CQI 15 is modified to be 256QAM entry at same SE and the SE  for 3 new 256QAM entries are 6.1719, 6.7890, 7.4063 bps/Hz.

MCS table:

- Retain TBS 0 in Rel-8 MCS table for RRC/VoIP robustness and remove every second MCS entries at QPSK region, i.e., remove MCS {1, 3, 5, 7, 9}.

- Introduce 7 MCS entries for 256QAM

  - 4 MCS entries correspond to CQI 12, 13, 14, and 15 in the new CQI table, except that the SE of MCS entry corresponding to CQI 15 is increased due to smaller overhead assumption for last TBS row.

  - 3 MCS entries are added in between with equal spacing.

- Reuse Rel-8 TBS indices as much as possible

  - Introduce 6 new TBS indices

# References

R1-142376, “TBS table design for 256QAM”, Ericsson

# Annex

Table 2: Rel-8 CQI table & Proposed CQI table for 256QAM support

| (a) Rel-8 CQI Table |  |  |  |  | (b) Proposed CQI Table for 256QAM support |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CQI | Mod | r×1024 | SE |  | CQI | Mod | r×1024 | SE |
| 0 | out of range |  |  |  | 0 | out of range |  |  |
| 1 | 2 | 78 | 0.1523 |  |  |  |  |  |
| 2 | 2 | 120 | 0.2344 |  | 1 | 2 | 120 | 0.2344 |
| 3 | 2 | 193 | 0.3770 |  |  |  |  |  |
| 4 | 2 | 308 | 0.6016 |  | 2 | 2 | 308 | 0.6016 |
| 5 | 2 | 449 | 0.8770 |  |  |  |  |  |
| 6 | 2 | 602 | 1.1758 |  | 3 | 2 | 602 | 1.1758 |
| 7 | 4 | 378 | 1.4766 |  | 4 | 4 | 378 | 1.4766 |
| 8 | 4 | 490 | 1.9141 |  | 5 | 4 | 490 | 1.9141 |
| 9 | 4 | 616 | 2.4063 |  | 6 | 4 | 616 | 2.4063 |
| 10 | 6 | 466 | 2.7305 |  | 7 | 6 | 466 | 2.7305 |
| 11 | 6 | 567 | 3.3223 |  | 8 | 6 | 567 | 3.3223 |
| 12 | 6 | 666 | 3.9023 |  | 9 | 6 | 666 | 3.9023 |
| 13 | 6 | 772 | 4.5234 |  | 10 | 6 | 772 | 4.5234 |
| 14 | 6 | 873 | 5.1152 |  | 11 | 6 | 873 | 5.1152 |
| 15 | 6 | 948 | 5.5547 |  | 12 | 8 | 711 | 5.5547 |
|  |  |  |  |  | 13 | 8 | 790 | 6.1719 |
|  |  |  |  |  | 14 | 8 | 869 | 6.7890 |
|  |  |  |  |  | 15 | 8 | 948 | 7.4063 |

Table 3: Rel-8 MCS table & Proposed MCS table for 256QAM support (Option 1)

| (a) Rel-8 PDSCH MCS Table |  |  |  |  | (b) Proposed PDSCH MCS Table for 256QAM support |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MCS | Mod | TBS | Target r×1024 |  | MCS | Mod | TBS | Target r×1024 |
| 0 | 2 | 0 | 120 |  | 0 | 2 | 0 | 120 |
| 1 | 2 | 1 | 157 |  |  |  |  |  |
| 2 | 2 | 2 | 193 |  | 1 | 2 | 2 | 193 |
| 3 | 2 | 3 | 251 |  |  |  |  |  |
| 4 | 2 | 4 | 308 |  | 2 | 2 | 4 | 308 |
| 5 | 2 | 5 | 379 |  |  |  |  |  |
| 6 | 2 | 6 | 449 |  | 3 | 2 | 6 | 449 |
| 7 | 2 | 7 | 526 |  |  |  |  |  |
| 8 | 2 | 8 | 602 |  | 4 | 2 | 8 | 602 |
| 9 | 2 | 9 | 679 |  |  |  |  |  |
| 10 | 4 | 9 | 340 |  |  |  |  |  |
| 11 | 4 | 10 | 378 |  | 5 | 4 | 10 | 378 |
| 12 | 4 | 11 | 434 |  | 6 | 4 | 11 | 434 |
| 13 | 4 | 12 | 490 |  | 7 | 4 | 12 | 490 |
| 14 | 4 | 13 | 553 |  | 8 | 4 | 13 | 553 |
| 15 | 4 | 14 | 616 |  | 9 | 4 | 14 | 616 |
| 16 | 4 | 15 | 658 |  |  |  |  |  |
| 17 | 6 | 15 | 438 |  | 10 | 6 | 15 | 438 |
| 18 | 6 | 16 | 466 |  | 11 | 6 | 16 | 466 |
| 19 | 6 | 17 | 517 |  | 12 | 6 | 17 | 517 |
| 20 | 6 | 18 | 567 |  | 13 | 6 | 18 | 567 |
| 21 | 6 | 19 | 616 |  | 14 | 6 | 19 | 616 |
| 22 | 6 | 20 | 666 |  | 15 | 6 | 20 | 666 |
| 23 | 6 | 21 | 719 |  | 16 | 6 | 21 | 719 |
| 24 | 6 | 22 | 772 |  | 17 | 6 | 22 | 772 |
| 25 | 6 | 23 | 822 |  | 18 | 6 | 23 | 822 |
| 26 | 6 | 24 | 873 |  | 19 | 6 | 24 | 873 |
| 27 | 6 | 25 | 910 |  | 20 | 6 | 25 | 910 |
| 28 | 6 | 26 | 1054 |  | 21 | 8 | 27 | 711 |
|  |  |  |  |  | 22 | 8 | 28 | 751 |
|  |  |  |  |  | 23 | 8 | 26 | 790 |
|  |  |  |  |  | 24 | 8 | 29 | 830 |
|  |  |  |  |  | 25 | 8 | 30 | 869 |
|  |  |  |  |  | 26 | 8 | 31 | 909 |
|  |  |  |  |  | 27 | 8 | 32 | 1054 |
|  |  |  |  |  | 28 | 2 | reserved |  |
| 29 | 2 | reserved |  |  | 29 | 4 |  |  |
| 30 | 4 |  |  |  | 30 | 6 |  |  |
| 31 | 6 |  |  |  | 31 | 8 |  |  |

Table 4: Rel-8 MCS table & Proposed MCS table for 256QAM support (Option 2)

| (a) Rel-8 PDSCH MCS Table |  |  |  |  | (b) Proposed PDSCH MCS Table for 256QAM support |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MCS | Mod | TBS | Target r×1024 |  | MCS | Mod | TBS | Target r×1024 |
| 0 | 2 | 0 | 120 |  | 0 | 2 | 0 | 120 |
| 1 | 2 | 1 | 157 |  |  |  |  |  |
| 2 | 2 | 2 | 193 |  | 1 | 2 | 2 | 193 |
| 3 | 2 | 3 | 251 |  |  |  |  |  |
| 4 | 2 | 4 | 308 |  | 2 | 2 | 4 | 308 |
| 5 | 2 | 5 | 379 |  |  |  |  |  |
| 6 | 2 | 6 | 449 |  | 3 | 2 | 6 | 449 |
| 7 | 2 | 7 | 526 |  |  |  |  |  |
| 8 | 2 | 8 | 602 |  | 4 | 2 | 8 | 602 |
| 9 | 2 | 9 | 679 |  |  |  |  |  |
| 10 | 4 | 9 | 340 |  | 5 | 4 | 9 | 340 |
| 11 | 4 | 10 | 378 |  | 6 | 4 | 10 | 378 |
| 12 | 4 | 11 | 434 |  | 7 | 4 | 11 | 434 |
| 13 | 4 | 12 | 490 |  | 8 | 4 | 12 | 490 |
| 14 | 4 | 13 | 553 |  | 9 | 4 | 13 | 553 |
| 15 | 4 | 14 | 616 |  | 10 | 4 | 14 | 616 |
| 16 | 4 | 15 | 658 |  |  |  |  |  |
| 17 | 6 | 15 | 438 |  | 11 | 6 | 15 | 438 |
| 18 | 6 | 16 | 466 |  | 12 | 6 | 16 | 466 |
| 19 | 6 | 17 | 517 |  | 13 | 6 | 17 | 517 |
| 20 | 6 | 18 | 567 |  | 14 | 6 | 18 | 567 |
| 21 | 6 | 19 | 616 |  | 15 | 6 | 19 | 616 |
| 22 | 6 | 20 | 666 |  | 16 | 6 | 20 | 666 |
| 23 | 6 | 21 | 719 |  | 17 | 6 | 21 | 719 |
| 24 | 6 | 22 | 772 |  | 18 | 6 | 22 | 772 |
| 25 | 6 | 23 | 822 |  | 19 | 6 | 23 | 822 |
| 26 | 6 | 24 | 873 |  | 20 | 6 | 24 | 873 |
| 27 | 6 | 25 | 910 |  | 21 | 6 | 25 | 910 |
| 28 | 6 | 26 | 1054 |  | 22 | 8 | 27 | 711 |
|  |  |  |  |  | 23 | 8 | 28 | 751 |
|  |  |  |  |  | 24 | 8 | 26 | 790 |
|  |  |  |  |  | 25 | 8 | 29 | 830 |
|  |  |  |  |  | 26 | 8 | 30 | 869 |
|  |  |  |  |  | 27 | 8 | 31 | 909 |
|  |  |  |  |  | 28 | 8 | 32 | 1054 |
| 29 | 2 | reserved |  |  | 29 | 2/4* | reserved |  |
| 30 | 4 |  |  |  | 30 | 4/6* |  |  |
| 31 | 6 |  |  |  | 31 | 6/8* |  |  |

* It depends on modulation scheme of initial transmission.

Table 5 System level simulation assumptions

| Parameters | Assumptions |
| --- | --- |
| Scenario | SCE 2a sparse |
| Number of macro sites | 7 |
| Number of cluster per macro | 1 |
| Number of picos per cluster | 4 |
| UE dropping | 2/3 dropped within the cluster, 1/3 dropped throughout the macro geographical area; 20% outdoor 80% indoor |
| Traffic model | FTP Model 1 as in TR 36.814 |
| Pico PRB usage | Load 1: 4%; load 2: 7%; load 3: 12.5% |
| Macro PRB usage | Load 1: 7%; load 2: 15%; load 3: 32% |
| Radio link monitoring | Q out = -8, Qin = -5, N310 = 20, T310 = 2 |
| MaxRlcRetxThreshold (both DRB and SRB) | 8 |
| Maximum No RachAttempts | 5 |
| Handover A1 events settings | Threshold1/Hysteresis: -105dBm/1dB TimeToTrigger: 320ms |
| Handover A2 events settings | Threshold1/Hysteresis : -105dBm/1dB TimeToTrigger: 320ms |
| Handover A5 events settings | Threshold1/Hysteresis: -108dBm/1dB Threshold2/Hysteresis: -105dBm/1dB TimeToTrigger: 320ms |
| Handover A3 events settings | Threshold1/Hysteresis: 1dB/1dB TimeToTrigger: 320ms |