3GPP TSG RAN WG1 Meeting #77		R1-142472

Seoul, South Korea, 19th – 23th May 2014

Source:	QUALCOMM Incorporated

Title:	HARQ Mechanism for TDM operation

Agenda item:		5.4.2

Document for:	Discussion/Decision

# 1	Introduction

As part of the F-EUL work item, specification changes to improve performance under TDM scheduling are being considered. In particular, a new granting scheme has been agreed on wherein the UE’s grant is de-assigned if the UE detects that an E-AGCH transmission has been made to some other UE. In the current specification, retransmissions get priority, and a UE can make HARQ retransmissions even if its grant has been reduced or de-assigned. To ensure true TDM operation where only one UE transmits in each TTI, these retransmissions have to be taken into account. This contribution proposes a HARQ operation scheme that ensure true TDM operation while minimizing the E-AGCH transmission overhead. We also propose a scheme to allow coexistence of both the legacy-type grants as well as the new TDM-type grants.

# 2	HARQ operation in TDM scheduling

The issue of E-AGCH overhead due to HARQ in TDM scheduling is shown in Figure 1. We see in Figure 1(a) that if legacy HARQ operation is allowed, the HARQ re-transmissions of UE1 would be concurrent with the transmissions of UE2, thus breaking the TDM operation. Preserving TDM operation requires several E-AGCH to keep re-assigning the grant between UE1 and UE2 until all retransmissions of UE1 are complete. Only then can the grant be continuously assigned to UE2. If each UE typically holds the grant for a large number of TTIs, the E-AGCH transmissions are infrequent and the overhead of a few additional E-AGCH to flush out the re-transmissions may not be significant. However, scheduler fairness as well as presence of bursty data users would imply that the typical duration for which one UE is continuously assigned the TDM grant is unlikely to be large enough for this E-AGCH overhead to be negligible.

Figure 1: HARQ scheme for E-AGCH overhead reduction with TDM scheduling

Waiting for all HARQ retransmissions of UE1 to complete before re-assigning the grant to UE2 also does not satisfactorily solve this issue. If the UE has data, it keeps transmitting fresh packets. With 8 HARQ processes all assigned to one UE, the probability that all of them terminate simultaneously reduces even if individually each process has a low probability of re-transmission (eg, based on 10% BLER target after the first HARQ transmission). Thus, while we avoid multiple E-AGCH transmissions, we may have to wait for a long time before de-assigning the grant from UE1, breaking the fairness. On the other hand, if the UE runs out of data to transmit, then the wait time is bounded, but there may be many TTIs in which the system capacity lies unused while waiting to complete all the re-transmissions.

The HARQ scheme of Figure 1(b) aims to mitigate this issue as shown: When a re-transmission would have to be sent according to legacy rules, if a grant is not available for that TTI, the re-transmission is delayed to the next re-transmission opportunity corresponding to that HARQ process. This delay continues to occur (skipping every 8 HARQ processes) until a grant is made available to the UE.

Since the notion of periodic HARQ process is still maintained, no change is needed to existing E-DPCCH signaling. The scheduler ensures that the HARQ re-transmissions are not excessively delayed so that the MAC timers that monitor lost packets are not activated. Thus, it has more flexibility to schedule UEs in a manner that minimizes the E-AGCH overhead.

One possible issue with this HARQ operation is that there may be grants issues by the NodeB in soft handover where the non-serving cell may have decoded and acknowledged the transmission. However, operating at a fairly strict BLER target would minimize this occurance. In any case, this issue is present even in the legacy mdoe of operation.

Proposal 1: A UE with a Rel-12 TDM-grant delays a HARQ re-transmission by 8 TTIs if it does not have a grant at the time of the re-transmission. This action is repeated until the UE has a grant, at which point the re-transmission is made.

# 2	Grant scheme to handle multiple user scheduling

It is desirable even for the Rel-12 UEs that follow the TDM granting scheme to have the ability to also follow the legacy granting scheme since the network may want to schedule multiple users.

This can be accomplished using two E-RNTIs for each Rel-12 UE, one operating in accordance with legacy rules and one in Rel-12 mode. The Rel-12 UE monitors E-AGCH transmissions with both E-RNTIs. If it has a grant, it follows the new Rel-12 procedure and de-assigns it on detecting E-AGCH transmission to other UEs if the most recent E-AGCH received by the UE was scrambled with the Rel-12 E-RNTI. On the other hand, this Rel-12 procedure is not followed if the most recent E-AGCH it received was scrambled with the legacy E-RNTI.

Proposal 2: UE supporting Rel-12 TDM granting scheme shall monitor E-AGCH with two E-RNTIs, applying legacy rules to grants received on one of them and the new Rel-12 rules to grants received on the other one. The mapping of the E-RNTI to the legacy or Rel-12 modes of operation is signalled by the network.

# 3	Implicit Release of grant

One issue with the grant scheme that has been agreed in Rel-12 is that the NodeB cannot remove the grant once it’s been assigned. There could be situations that arise where there are no other UEs present and transmitting in the system and so once a UE completes all transmissions and no data is left in the buffer, there is a need to release the grant and fall back to CPC.

Another aspect to be considered is the interaction with DTX/DRX. In this case, the UE may fall back into DRX immediately after transmission of a packet and so it is unable to detect other E-AGCH transmissions and there is not able to judge whether the grant has been de-assigned or not.

In both these situations, it would be useful to have an implicit release functionality wherein the UE transmits SI=0 (or TEBS=0) and releases the grant. This would for eacmple occur just before the UE falls back to CPC. This is the same mechanism used in EUL in Cell-FACH.

Proposal 3: When the UE does not have any data to transmit, SI=0 is transmitted and the grant is implicitly released.

Proposal 4: When the UE goes into DRX, an SI=0 is transmitted and the UE releases its grant.

# 4	Interpretation of Grant scope bit

With the new granting scheme, the grant scope bit could continue to have its existing interpretation. When set to ‘per-HARQ process’, then UE has a grant only for one HARQ process, and only monitors E-AGCH directed to other users for that HARQ process. If the grant scope bit is set to ‘all HARQ processes’, then UE has the grant for all HARQ processes, and deassigns the grant for all HARQ processes if it detects any E-AGCH directed to another UE. Another alternative is to always use the ‘all HARQ processes’ interpretation, making the grant scope bit irrelevant. Since assigning a contiguous set of TTIs is more link-efficient than assigning only a single TTI, it appears preferable to treat the grant scope bit as always set to ‘all HARQ processes’.

Proposal 5: The grant scope bit for a TDM grant shall always be interpreted as indicating that the grant is valid for all HARQ processes.

# 5	Conclusions

We have proposed improvements to TDM scheduling outlined in the following proposals:

Proposal 1: A UE with a Rel-12 TDM-grant delays a HARQ re-transmission by 8 TTIs if it does not have a grant at the time of the re-transmission. This action is repeated until the UE has a grant, at which point the re-transmission is made.

Proposal 2: UE supporting Rel-12 TDM granting scheme shall monitor E-AGCH with two E-RNTIs, applying legacy rules to grants received on one of them and the new Rel-12 rules to grants received on the other one.

Proposal 3: When the UE does not have any data to transmit, SI=0 is transmitted and the grant is implicitly released.

Proposal 4: When the UE goes into DRX, an SI=0 is transmitted and the UE releases its grant.

Proposal 5: The grant scope bit for a TDM grant shall always be interpreted as indicating that the grant is valid for all HARQ processes.