3GPP TSG RAN WG1 Meeting #77,	R1-142483

Seoul, Korea, 19th – 23rd May 2014

Source:	Kyocera

Title:	        Mode 1 resource allocation for D2D communication

Agenda Item:	6.2.5.2.2

Document for:	Discussion/Decision

Introduction

During RAN1#76bis, Mode 1 resource allocation for D2D communication was discussed [1]. The agreements are as follows:

Agreements: - For Mode 1 transmission, - eNodeB or Rel-10 relay allocates resources to a D2D transmitter for SA and Data using PDCCH or EPDCCH - FFS: Linkage between SA and Data - FFS: Separate grant for Data - Single grant can schedule multiple Data transmission opportunities - The multiple opportunities can be used for the multiple transmissions of a single TB - The multiple opportunities can be used for the transmissions of multiple TBs - FFS: Which entity decides how each transmission opportunity is used - FFS: Single grant can schedule single SA transmission - Single grant can schedule multiple SA transmissions - FFS: Whether the multiple SA transmissions are of the same SA or different SA - FFS: C-RNTI or another UE-specific RNTI is used at least for scrambling of CRC of a D2D grant - eNodeB or Rel-10 relay controls transmission power of SA and Data using PDCCH or EPDCCH

Agreements: - One or more resource patterns for transmission (RPT) of time and/or frequency resources for multiple transmission opportunities of data TBs can be defined - RPT is either implicitly or explicitly signaled by the eNB or Rel-10 Relay for Mode1 - RPT is either implicitly or explicitly signaled in SA - If multiple transmission opportunities of the same SA are supported - FFS whether one or more RPT are defined for (re)-transmissions of SAs

Agreements: - Semi-static pool(s) of resources can be allocated for SA - eNodeB may broadcast the information about the SA resource pool using SIB for D2D UE - Transmission pool for Mode 2 - Reception pool(s) for Mode 1 and Mode 2 - UE is not required to decode neighboring cell SIB

In this contribution we consider further Mode 1 resource allocation details. The detailed SA physical design is discussed in [2].

SA and Data resource pool

In this section we describe the resource SA and D2D broadcast data pool design. To lower the UE complexity it is preferable to have a common PHY design for both mode 1 and mode 2. This will allow the UE receiver to be indifferent to the two types of resource allocation modes. To achieve the above we should define same linkage relation (or mapping) between the SA and the D2D broadcast data for both the modes 1 and 2. Further details are described about the resource pool configuration in [3].

Figure 1: D2D resource pool configuration(10MHz)

D2D grant for SA resource indication

In this section we describe D2D grant sent on DL by the eNB for the SA transmissions. . As described in a separate paper [2] the Data allocations are mapped to the SA allocations. Therefore, the eNB transmits a SA grant on the DL and no separate grant is sent for the D2D data allocations. The UE receiver can derive the locations of the data transmissions after decoding the SAs. The SA grant sent on the DL is n-4th subframes from the SA region’s starting subframe n.

Proposal 1:

The eNB transmits a SA grant on the DL and no separate grant is sent for the D2D data allocations

Figure 3: D2D grant for SA resource indication

Table 2: DCI for SA resource indication

| Item | Num of bits |
| --- | --- |
| SA’s position | Frequency position : 3bits (1 of 6) Time position: 4bits (all patterns of repetition) |
| MCS | 5 bits |
| D2D data power UP/DOWN | 1bits[FFS] |

D2D Power Control

Since D2D operates on the UL carrier, the D2D transmission could cause interference at the eNB receiver and degrade the UL performance. One approach to mitigate this interference at the eNB receiver is to use power control for the D2D transmissions.  We propose open loop maximum transmit power restriction and a closed loop power control using DL control channel PDCCH to deliver power control commands to the D2D UE.

Figure 4: eNB interference calculation

Using the DL path loss an in-coverage UE could calculate the expected interference power it will cause at the eNB UL receiver as shown in Figure 4. Therefore, D2D UE could use the above to restrict itself by lowering its transmitting power.  The following open-loop equation can be used by the D2D UE:

ID2D= PD2D-PL

PD2D_Tx= PD2D  if  ID2D≤ThD2D

PD2D_Tx=PL+ ThD2D   if ID2D> ThD2D

ID2D=Expected Interference on eNB

PD2D=Constant

PD2D_Tx=D2D Transmit power

PL=DL pathloss

ThD2D =D2D Interference threshold

Section 3 of [4] shows the open-loop power control is beneficial for WAN performance improvement. In addition to the open-loop power control, the closed-loop power control can be used if the interference caused by D2D transmissions is over a threshold ThD2D. eNB delivers TPC commands using DL control channel as described in Section 3. For calculating ID2D, in-band emission can be considered.  ThD2D can be set to infinite, that means no open-loop power control.

Proposal 2:

The open-loop and the closed-loop power control should be supported for D2D transmit power control.

Figure 4: power control procedure

Conclusions

Proposal 1:

The eNB transmits a SA grant on the DL and no separate grant is sent for the D2D data allocations

Proposal 2:

The open-loop and the closed-loop power control should be supported for D2D transmit power control.

References

[1] Chairman's Notes RAN1#76bis

[2] R1-142480, Design details of scheduling assignments, Kyocera

[3] R1-142481, Mode 2 resource allocation for D2D communication, Kyocera

[4] R1-142486, Simulation result on WAN impact of D2D communication, Kyocera