3GPP TSG RAN WG1 Meeting #77,	R1-142486

Seoul, Korea, 19th – 23rd May 2014

Source:	Kyocera

Title:	        Simulation result on WAN impact of D2D communication

Agenda Item:	6.2.5.2.4

Document for:	Discussion/Decision

Introduction

During RAN1#76bis, D2D communication impact on the cellular communications are discussed [1][2].

Agreements: - The following work plan is proposed for RAN1: - Companies are invited provide results on cellular impact by discovery and/or communication D2D in RAN1#77 - Discovery and communication should be evaluated separately - Simulations assumptions according to TR 36.843 - Focus on VoIP traffic model for D2D PS communication - In addition to metrics related to D2D performance (as in TR 36.843, section 10), the following metrics related to impact on cellular can be provided : - Mandatory performance metrics are those agreed in the TR, with and without D2D, with and without the potential solutions - Additionally, companies may provide other results (e.g., PUCCH BLER), with and without D2D, with and without the potential solutions - Discuss solutions at RAN1#77 and specify them in case of standard impact - Companies are invited to discuss remaining simulation parameters that need to be harmonized to provide meaningful results.

Results of WAN impact

In this section we study D2D communications impact on the WAN. The simulation assumptions are described in Appendix A. Table 1 provides details for the D2D resource allocations. In each resource allocation type 25% of the total resources are allocated for D2D communication (10 MHz bandwidth). In Table 2 two types of WAN resource allocations are listed. In the non-shared case D2D resources are pre-configured and the eNB can only schedule non-D2D resources. In the shared case eNB schedules both the D2D and the non-D2D resources.

Table 1: Resource allocation configuration (D2D allocation)

| Resource allocation method |  |  |
| --- | --- | --- |
| TDM | Time resource | 1 of every 4 subframes |
|  | Frequency resource | 44 PRBs |
| FDM | Time resource | all subframe |
|  | Frequency resource | 10 PRBs |
| FDM and TDM | Time resource | 2 of every 4 subframes |
|  | Frequency resource | 22 PRBs |

Table 2: Resource allocation configuration (WAN allocation)

| Resource allocation method |  |
| --- | --- |
| Non shared | eNB can allocate resources not reserved for D2D |
| Shared | eNB can allocate all resources |

In Table 3 and Table 4 simulation results are shown. As expected the WAN performance has 25% degradation in most cases except in the shared cases. In the shared cases there is a gain in reusing the resources for both D2D and WAN communications. FDM shared case shows the least impact on the WAN performance.

Table 3: WAN impact result (Tx Power: 23dBm)

| UL Throughput Average [kbps] |  | Degradation from WAN Only [%] |
| --- | --- | --- |
| WAN Only | 683 | 0 |
| FDM | 516 | 24.5 |
| TDM | 519 | 23.9 |
| TDM and FDM | 510 | 25.3 |
| FDM (Shared) | 587 | 14.0 |
| TDM and FDM (Shared) | 551 | 19.3 |

Figure 1: WAN UL throughput impact (Tx Power: 23dBm)

Table 4: WAN impact result (Tx Power: 31dBm)

| UL Throughput Average [kbps] |  | Degradation from WAN Only [%] |
| --- | --- | --- |
| WAN Only | 683 | 0 |
| FDM | 458 | 33 |
| TDM | 495 | 27 |
| TDM and FDM | 469 | 31 |
| FDM (Shared) | 489 | 28 |
| TDM and FDM (Shared) | 437 | 36 |

Figure 2: WAN UL throughput impact (Tx Power: 31dBm)

Proposal 1:

WAN UL resource allocation should eNB implementation and no resource allocation restriction must be standardized.

Power control results

In this section, we discuss the impact of power control to further improve the FDM-shared case. In [5], we proposed a power control algorithm to be used. This power control mitigates interference at the eNB receiver which is caused by the D2D transmissions.  The interference threshold is set to -100dBm (per 1PRB). The result is listed in Table 5 and Table 6 showing there is negligible performance degradation in the WAN performance if power-control is used to control the D2D transmissions.

Proposal 2:

D2D Transmit Power control should be supported.

Table 5: WAN impact result (power control) (Tx Power: 23dBm)

| UL Throughput Average [kbps] |  | Degradation from WAN Only [%] |
| --- | --- | --- |
| WAN Only | 683 | 0 |
| FDM (Shared) with power control | 682 | 0.1 |

Table 6: WAN impact result (power control) (Tx Power: 31dBm)

| UL Throughput Average [kbps] |  | Degradation from WAN Only [%] |
| --- | --- | --- |
| WAN Only | 683 | 0 |
| FDM (Shared) with power control | 648 | 5.0 |

Conclusions

Proposal 1:

WAN UL resource allocation should eNB implementation and no resource allocation restriction must be standardized.

Proposal 2:

D2D Transmit Power control should be supported.

References

[1] Chairman's Notes RAN1#76bis

[2] R1-141163, Discussion on Multiplexing of Cellular and D2D Transmissions from System Perspective, Intel Corporation

[3] R1-141820, Simulation assumptions for impact of D2D on WAN - agreements from [76b-11], Qualcomm Incorporated

[4] 3GPP TR 36.843 “Study on LTE Device to Device Proximity Services; Radio Aspects”

[5] R1-142483, Mode 1 resource allocation for D2D communication, Kyocera

- Appendix A – Evaluation Assumptions

The simulation assumption is basically same as described in [3] [4]. The following is additional information of the simulation.

| Item | Value |  |
| --- | --- | --- |
| UE transmit power | 23 dBm, 31dBm |  |
| Transmission power control | Fixed / Proposed in [5] maximum interference threshold = -100dBm (per 1PRB) |  |
| Number of D2D broadcast transmitters | Option 1 (3) |  |
| WAN traffic | Option 1 |  |
| D2D and WAN multiplexing | D2D resources | For TDM Time resource : 1 of every 4 subframes Frequency resource : 44RBs For FDM Time resource : all subframe Frequency resource : 10RBs For FDM and TDM Time resource : 2 of every 4 subframes Frequency resource : 22RBs |
|  | WAN resources | Scheduling : Proportional Fair For non-shared Resources not reserved for D2D For shared All resources |
| D2D allocation | 1 resource unit is 2RBs Frequency allocation : Random Time allocation : 4 subframe interval No SA’s timing consideration |  |