3GPP TSG-RAN WG1 Meeting #77	R1-142540

Seoul, Korea, 19 - 23 May 2014

Agenda item:		6.2.2.3

Source:	Ericsson, NSN

Title:	Further Enhancements for MTC in LTE Rel-13

Document for:		Information

# 1	Introduction

In Rel-12, a low-cost MTC UE category will be introduced with the bill of material cost expected to be around 40-50% of Category-1 UE. In Rel-13, it is expected that a feature to provide coverage extension of up to 15dB based on previous WID will be introduced. In this contribution, we provide an overview of further Rel-13 enhancements that should be considered for machine-type communications on LTE.

# 2	Further Enhancements for MTC on LTE

The Internet of Things (IoT) refers to interconnection and exchange of data among devices. An estimated 50 billion connected devices will be deployed by 2020 [1]. Machine-type communications integrates information and communication technologies, thus enabling the Internet of Things. Cellular IoT is expected to be a key driver for growth in cellular networks. The following requirements for cellular IoT are envisioned –

- Very low cost devices

- Ubiqitious coverage including locations with high penetration loss such as basements or meter closets

- Low power consumption by the device – e.g. battery life of greater than 10 years with 2 AA batteries

- Support large number devices used for MTC

- Protocols that are optimized for transfer of small size payload with low control plane overhead

To satisfy these requirements, at least the following enhancements should be considered in Rel-13 –

- Device cost and complexity reduction

Further cost reduction should be considered to reduce the bill of material cost of the modem to 20-25% of that of Category-1 UE. This can be achived by reducing the UE receive and transmit bandwidth in the RF to 1.4 MHz, reducing the peak data rates for both downlink and uplink to approximately 200 kbps, and possibly by reducing the maximum transmission power of the UE as shown in Table 1.

Table 1. Feature list comparison for different UE categories.

| Feature | Cat-4 | Cat-1 | Rel-12 Low Complexity UE | Rel-13 Low Complexity UE |
| --- | --- | --- | --- | --- |
| UE RF Bandwidth | 20 MHz | 20 MHz | 20 MHz | 1.4 MHz |
| DL Peak Rate | 150 Mbps | 10 Mbps | 1 Mbps | ~200 kbps |
| Max No of DL Layers | 2 | 1 | 1 | 1 |
| UL Peak Rate | 50 Mbps | 5 Mbps | 1 Mbps | ~200 kbps |
| No of RF Rx chains | 2 | 2 | 1 | 1 |
| Max UE Tx power | 23 dBm | 23 dBm | 23 dBm | ~20 dBm |
| Duplex Mode | Full | Full | Half (optional) | Half (optional) |
| Relative BOM Cost | 125% | 100% | 50% | 20-25% |

Introduction of a new UE power class with a few dB reduced maximum transmission power is intended to enable cheap single chip modem implementations without the need for an external power amplifier.

Note that the bandwidth reduced UE will be able to operate within any LTE system bandwidth up to 20 MHz even though it can only receive and transmit a small portion of the total bandwidth at a time.

Furthermore, device complexity may be reduced to provide further cost saving and shorten development time. This may include simplifying or eliminating some current LTE operations for these UEs.

- Coverage enhancement

In Rel-13, it is expected that a feature to provide coverage extension of up to 15dB will be introduced. However, it has been shown in [2] that up to 20dB coverage extension is feasible. Therefore, up to 20dB may be considered in Rel-13. Potential techniques that may be used to extend coverage include relaxation of requirements, repetition, subframe bundling, HARQ retransmission, and PSD boosting as shown in Table 2.

Table 2. Link budget for coverage enhancement (FDD case).

- Device power saving

In Rel-12, a UE power saving mode will be introduced to increase UE battery life time. Further improvements to increase battery life may be considered in Rel-13. This needs to be further studied by the appropriate working groups.

Past studies on MTC in LTE have demonstrated the feasibility of the above enhancements and provided some potential solutions. Techniques for reducing cost and complexity as well as for extending coverage have been studied in [2]. Protocol and higher-layer enhancements have been studied in [3][4].

# 3		Conclusion

While Rel-12 provides substantial improvements for MTC with respect to device complexity and power consumption, we still see a big potential for further improvements for MTC in Rel-13.

# 4		Reference

- Evans, D., “The Internet of Things, How the Next Evolution of the Internet Is Changing Everything,” Cisco Whitepaper, April 2011.

- TR 36.888, Machine-Type Communications (MTC) User Equipments (UEs) based on LTE, V12.0.0, June 2013.

- TR 37.868, Study on RAN Improvements for Machine-type Communications, V11.0.0, September 2011.

- TR 37.869, Study on Enhancements to Machine-Type Communications (MTC) and other Mobile Data Applications, V12.0.0, September 2013.