3GPP TSG RAN WG1 RAN1#81	R1-153404

Fukuoka, Japan, 25th – 29th May 2015

Agenda item:	6.2.4.1

Source: 	IIT Hyderabad

Title: 	Coexistence Results for LAA DL only and Wi-Fi DL+UL

Document for:	Discussion

- Introduction

This contribution contains the evaluation results for LAA DL only coexisting with Wi-Fi UL and DL when the traffic is ftp.

- Simulation Assumptions

LAA Assumptions

LAA uses LBT Cat 4 [1] channel access mechanism as shown in Figure 1. The contention window is updated based on ACK/NACKs.

- Initial CCA peroid is 34 s

- eCCA slot duration 9 s

- LAA Energy Detection threshold -62 dBm

- Dynamic Exponential Back off [CWmin, CWmax] is [16 , 1024]

- Maximum Channel Occupancy 4 ms

- LAA Burst is not alligned to subframe boundaries.

- Only Unlicecensed Band with one carrier is used. Bandwidth is 20MHz

- Antenna configuration is 1Tx and 1Rx

- DL only

- MCS table from release 12 without 256 QAM

Wi-Fi Assumptions

- MCS table from 802.11ac including 256 QAM

- DIFS time 34 s

- Slot duration 9 s

- Energy Detection Threshold -62 dBm

- Preamble Detection Threshold -82 dBm

- Long GI of 800 ns

- RTS/CTS not applied

- DL to UL ratio 50 % to 50 %

- Antenna Configuration 1Tx 1Rx in DL and UL

Figure 1

- Simulation Results: Coexistence of Wi-Fi and LAA

Simulations are carried out by following the evaluation methodology for indoor scenario defined in [2].

Following Scenarios are simulated and reported in the results section.

Step1: Operator 1 and 2 both deploy Wi-Fi and the performance metrics are recorded.

Step2: Operator 2 replaces Wi-Fi with LAA DL only and the performance metrics are captured.

Each operator serves 20 users at a time.

The 5%-ile, 50%-ile and 95%-ile of the UPT and delay CDFs as well as the corresponding mean values are reported for FTP traffic. Average cell buffer occupancy is reported.

  - Evaluation results for Indoor deployment with one unlicensed carrier for FTP traffic

Table 1  Wi-Fi DL UL and LAA DL only coexistence scenario with FTP traffic

| Reported parameters |  | Low load BO range for Wi-Fi Opt.1 in Step 1: 10%~25% |  |  |  | Medium load BO range for Wi-Fi Opt. 1 in Step 1: 35%~60% |  |  |  | High load BO range for Wi-Fi Opt.1 in Step 1: above 70% |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Wi-Fi Opt.1 in step 1 | Wi-Fi Opt.2 in step 1 | Wi-Fi Opt. 1 in step 2 | LAA Opt.2 in step 2 | Wi-Fi Opt.1 in step 1 | Wi-Fi Opt.2 in step 1 | Wi-Fi Opt. 1 in step 2 | LAA Opt.2 in step 2 | Wi-Fi Opt.1 in step 1 | Wi-Fi Opt.2 in step 1 | Wi-Fi Opt. 1 in step 2 | LAA Opt.2 in step 2 |
| DL: UPT CDF [Mbps] | 5% | 12 | 13.5 | 12 | 12.8 | 0.8 | 1.1 | 2.25 | 4.1 | 0.4 | 0.55 | 0.9 | 0.7 |
|  | 50% | 38 | 38 | 42.5 | 44.2 | 10.4 | 10.8 | 16.2 | 26.5 | 2 | 3 | 4 | 20 |
|  | 95% | 84 | 84 | 84 | 82 | 59 | 51 | 60 | 67 | 30 | 32 | 40 | 51 |
|  | Mean | 44.3 | 45.9 | 46 | 48 | 16.8 | 16.3 | 22 | 28.8 | 6.1 | 8.2 | 11.4 | 20.7 |
| DL: Delay CDF [s] | 5% | 0.048 | 0.048 | 0.048 | 0.05 | 0.06 | 0.07 | 0.066 | 0.058 | 0.12 | 0.18 | 0.09 | 0.08 |
|  | 50% | 0.1 | 0.094 | 0.095 | 0.09 | 0.35 | 0.35 | 0.24 | 0.15 | 1.2 | 1.8 | 0.85 | 0.21 |
|  | 95% | 0.35 | 0.33 | 0.33 | 0.31 | 2.5 | 3.0 | 1.8 | 0.9 | 8 | 10 | 4.1 | 6 |
|  | Mean | 0.138 | 0.129 | 0.13 | 0.12 | 0.762 | 0.693 | 0.45 | 0.28 | 2.8 | 1.9 | 1.3 | 1.1 |
| UL: UPT CDF [Mbps] | 5% | 11.2 | 11.7 | 13.4 | - | 4.3 | 3.9 | 5.6 | - | 1.55 | 1.35 | 1.25 | - |
|  | 50% | 38 | 42 | 43.6 | - | 15.7 | 13 | 20 | - | 5 | 5.5 | 4.5 | - |
|  | 95% | 84 | 84 | 84 | - | 63 | 47 | 78 | - | 38 | 50 | 37 | - |
|  | Mean | 42.1 | 45.4 | 47 | - | 21.2 | 17.8 | 26.4 | - | 10.6 | 12.5 | 10.8 | - |
| UL: Delay CDF [s] | 5% | 0.048 | 0.048 | 0.048 | - | 0.06 | 0.08 | 0.054 | - | 0.1 | 0.07 | 0.1 | - |
|  | 50% | 0.1 | 0.1 | 0.09 | - | 0.245 | 0.296 | 0.2 | - | 0.75 | 0.7 | 0.7 | - |
|  | 95% | 0.32 | 0.28 | 0.3 | - | 0.870 | 0.980 | 0.71 | - | 2.6 | 2.8 | 3.5 | - |
|  | Mean | 0.140 | 0.123 | 0.12 | - | 0.352 | 0.370 | 0.27 | - | 0.9 | 1.0 | 1.0 | - |
| 𝜌DL |  | 0.999 | 0.996 | 1 | 0.99 | 0.95 | 0.96 | 0.99 | 0.99 | 0.56 | 0.59 | 0.76 | 0.78 |
| 𝜌UL |  | 0.996 | 0.992 | 0.99 | - | 0.99 | 0.98 | 0.99 | - | 0.95 | 0.92 | 0.97 | - |
| BO |  | 0.2 | 0.2 | 0.19 | 0.17 | 0.57 | 0.58 | 0.45 | 0.4 | 0.88 | 0.91 | 0.85 | 0.7 |
| 𝜆 |  | 0.37 |  |  |  | 0.52 |  |  |  | 0.7 |  |  |  |
| Company/tdoc: IIT Hyderabad LBT category: 4 Unlicensed only, LAA Sensing Threshold -62dBm, LAA uses 64 QAM and Wi-Fi uses 256 QAM. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Observations

From Table 1 we can see that LAA is not affecting Wi-Fi in any load condition. The Wi-Fi performance in step 2 is improved when compared to step 1. This behavior is expected because of the absence of UL of LAA.

Under medium and high loads LAA is doing better than Wi-Fi and at the same time without degrading Wi-Fi. This is because of the high sensing threshold (-62 dBm) used by LAA.

- Conclusion

The mechanism in Figure 1 is similar to Wi-Fi’s DCF access scheme and this mechanism ensures fairness when LAA DL only is co-located with another Wi-Fi network.

References

- R1-152326 “Discussion on LBT Protocols”

- 3GPP TR 36.889, “Study on Licensed-Assisted Access to Unlicensed Spectrum”