3GPP TSG RAN WG1 Meeting #82		R1-153808

Beijing, China, 24th - 28th August 2015

|  | INTERNATIONAL TELECOMMUNICATION UNION |  |  |  |  |  |  | COM 15 – LS 266 – E |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  |  |  |  |  |  |
|  |  |  |  |  |  | English only Original: English |  |  |
| Question(s): |  | 13/15 |  |  |  |  |  |  |
| Ref.: TD 381 (PLEN/15) Annex I |  |  |  |  |  |  |  |  |
| Source: |  | ITU-T Study Group 15 |  |  |  |  |  |  |
| Title: |  | LS/r on work on time synchronization and future target requirements (reply to 3GPP-RAN-RP-151001) |  |  |  |  |  |  |
| LIAISON STATEMENT |  |  |  |  |  |  |  |  |  |
| For action to: |  |  |  | 3GPP TSG RAN |  |  |  |  |  |
| For comment to: |  |  |  | - |  |  |  |  |  |
| For information to: |  |  |  | RAN1, RAN3, RAN4 |  |  |  |  |  |
| Approval: |  |  |  | ITU-T SG15 meeting (22 June – 3 July 2015) |  |  |  |  |  |
| Deadline: |  |  |  | 7 September 2015 |  |  |  |  |  |
| Contact: |  |  | Stefano Ruffini Rapporteur Q13/15 |  |  |  | Email: stefano.ruffini@ericsson.com |  |  |
| Contact: |  |  | Silvana Rodrigues Associate Rapporteur Q13/15 |  |  |  | Email: silvana.rodrigues@idt.com |  |  |

ITU-T Q13/15 thanks 3GPP for its response to our liaison SG15-LS225.

In your liaison you have indicated the following:

“It is noted that values smaller than 3μs related to the synchronization are applicable as the timing alignment error requirements between serving cells of UE involved in carrier aggregation and in some UE test cases (e.g. CoMP). However, those requirements are not applicable over the whole network.”

Recommendation ITU-T G.8271 [1], developed by ITU-T Q13/15, defines phase and time accuracy requirements based on various application cases, mainly 3GPP wireless services requirements. Up to now, Q13/15 has mainly considered applications with relative phase deviation up to 3 µs [4]. This implies for instance that the time error of the base station relative to UTC must be within the ±1.5 µs range. More recently, Q13/15 has also started to study more stringent requirements as also indicated in the earlier liaison.

Moreover it is planned to distinguish between applications requiring access to an absolute synchronization reference (e.g., an ideal reference such as GPS or UTC), and applications with only relative phase deviation requirement as applicable to a cluster of nodes.

In order to better understand the applicability of this analysis to use cases that may be of interest for 3GPP we would like to get some clarification on the following point.

Our understanding is that the second case may apply to the examples you have indicated (carrier aggregation (CA) and CoMP). For these we noted the following:

- For CA, the time alignment error could be either +/-130 ns or +/- 260 ns, as per section 6.5.3.1 of 3GPP 36.104 [3];

- For CoMP, the time error could be +/-500ns, which is based on table 8.3.2.4.1-3 of 3GPP 36.101 [4], indicating that the time offset between TPs is [-0.5us, 2us].

Based on discussions in ITU-T Q13/15, the CA and CoMP phase/time accuracy requirements in (1) and (2) above are considered local requirements and not network-level requirements. Specifically, consider Figure 1 below.

RRU3

RRU4

Transport Network

BBU1

BBU2

RRU1

RRU2

Figure 1 - Transport Network with BBUs and RRUs

For CA and CoMP, Q13/15’s understanding is that the accuracy specifications are only required for those RRUs that are connected to the same BBU, e.g., RRU1 and RRU2, or RRU3 and RRU4. For example, the phase/time difference between RRU1 and RRU2 should be less than a certain error; however, there are no requirements related to the phase/time difference between RRU1 and RRU3, or between RRU1 and RRU4.

# Actions

Is the phase/time accuracy of CA and CoMP features required only for RRUs that connect to the same BBU, or also for RRUs connecting to different BBUs?

## References

- Recommendation ITU-T G.8271, Time and phase synchronization aspects of packet networks, ITU-T Q13/15, February 2012,
http://www.itu.int/ITU-T/recommendations/rec.aspx?rec=11527

- 3GPP 36.104 version 12.5.0 Release 12, LTE; Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) radio transmission and reception

- 3GPP 36.101 version 11.9.0 Release 11, LTE; Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) radio transmission and reception

- Liaison on work on time synchronization and future target requirements (reply to SG15-LS225) [from 3GPP RAN], ITU-T SG15 TD390/WP3, June 22 – July 3, 2015

_______________________